3GPP TSG RAN WG1 Meeting #96	R1-1903065

Athens, Greece, February 25 – March 1, 2019

Agenda Item:	7.2.3.1

Source:	Huawei, HiSilicon

Title:	Remaining issues on SSB periodicity for IAB

Document for:	Discussion and Decision

Introduction

In RAN1 Ad-Hoc 1901, the following agreements were achieved:

Agreements: - There is no need to introduce any specification change to enable longer RMSI transmission periodicities. Agreements: - For both SA and NSA, for initial cell selection, an IAB-node MT may assume that half frames with SS/PBCH blocks occur with a periodicity of 16 frames. - FFS cell re-selection - Study further the value of assumed SS/PBCH block periodicity for a backhaul link switching to a new parent node when the link to the current parent node fails.

In this contribution, we focus on the remaining issues related to SSB periodicity assumed by IAB-node MT in case of cell re-selection and backhaul link switching.

Discussion

SSB periodicity for cell re-selection

It was agreed that for initial cell selection, an IAB-node MT may assume that half frames with SS/PBCH blocks occur with a periodicity of 16 frames. Therefore, the serving cell may transmit SS/PBCH blocks with a periodicity equal or smaller than 160ms, e.g. 160ms on an NR carrier in NSA deployment.

When performing cell re-selection, the IAB-node MT does not know the potential new serving cell is a NSA cell or an SA cell. To ensure that the IAB node MT can detect and access both NSA and SA cells successfully via cell re-selection, IAB-node MT may assume that half frames with SS/PBCH blocks occur with a periodicity of 16 frames, which is same as initial access cell selection. Otherwise, if the IAB-node MT assumes a shorter SSB periodicity than initial cell selection, it may fail to get access a new cell via re-selection.

Proposal 1: For cell re-selection, an IAB-node MT may assume that half frames with SS/PBCH blocks occur with a periodicity of 16 frames.

SSB periodicity for backhaul link switching

According to TS 38.300 [1] (Section 9.2.7), after RLF is declared, UE shall stay in RRC_CONNECTED and selects a suitable cell and then initiates RRC re-establishment. When current backhaul link fails, an IAB-node MT can follow Rel-15 UE’s behaviors, i.e.:

- stays in RRC_CONNECTED;

- selects a suitable cell and then initiates RRC re-establishment;

- enters RRC_IDLE if a suitable cell wasn't found within a certain time after RLF was declared

Since the IAB-node MT will perform cell selection, the issue becomes similar to cell re-selection, the IAB-node MT may assume that half frames with SS/PBCH blocks occur with a periodicity of 16 frames, same assumption as initial cell selection.

Proposal 2: For backhaul link switching when the link to the current parent node fails, an IAB-node MT may follow the same assumption as initial cell selection on SS/PBCH blocks periodicity.

To achieve fast link recovery/switching after RLF, a shorter SS/PBCH block transmission periodicity can be adopted by network implementation. However, this does not require any changes to IAB-node MT assumption for SS/PBCH detection.

Conclusions

Based on the analysis, we have following proposals:

Proposal 1: For cell re-selection, an IAB-node MT may assume that half frames with SS/PBCH blocks occur with a periodicity of 16 frames.

Proposal 2: For backhaul link switching when the link to the current parent node fails, an IAB-node MT may follow the same assumption as initial cell selection on SS/PBCH blocks periodicity.

- References

[1] 3GPP TS 38.300 V15.4.0 NR; NR and NG-RAN Overall Description; Stage 2