3GPP TSG RAN WG1 Meeting #96	R1-1903200

Athens, Greece, February 25 – March 1, 2019

Agenda Item:	7.2.14

Source:	Huawei, HiSilicon

Title:	Discussion on PAPR in NTN

Document for:	Discussion and decision

Introduction

A new SI was approved by RAN#80 on studying a set of necessary features/adaptations enabling the operation of NR protocol in non-terrestrial networks (NTN), including physical layer, Layer 2 and above []. Specifically, in 3GPP TR 38.811 (V15.0.0) [], it is expressed that:

7.3.7.2.3	NR impact considerations

PAPR reduction techniques of CP-OFDM signal on the downlink would be beneficial to optimize the capacity of non-terrestrial networks and therefore could be considered in future studies.

To enhance the throughput / power ratio, peak-to-average power ratio (PAPR) reduction techniques could be considered in NTN. Consequently, in this contribution we would like to discuss PAPR effects considering the HPA model in NTN, criteria for selection of PAPR reduction techniques, and provide some simulation results of existing typical PAPR reduction techniques as a benchmark for further study.

Discussion on PAPR problem in NTN

As satellite systems are power-limited, the high power amplifier (HPA) of the satellite transmitter prefers to work at or near the saturation region for obtaining sufficient transmit power []. However, CP-OFDM signals with high PAPR will get into the non-linear region of HPA, which will introduce severe in-band non-linear distortions and out-of-band emission. For avoiding this, the HPA requires a large dynamic linear region and large input back off (IBO). Nevertheless, this kind of HPA for satellites is very expensive and has poor power efficiency. Meanwhile, HPA with low power efficiency cannot guarantee the sufficient signal-to-noise ratio (SNR) due to the high propagation path loss in NTN.

Figure 1 Relation between PAPR and SNR

The relation between PAPR and SNR is depicted in Figure 1, and it can be concluded that SNR decreases with increasing PAPR when the satellite uses the same HPA. Therefore, for power-limited satellites the PAPR problem is critical. A better solution is to reduce the CP-OFDM signals PAPR before they are fed into the HPA. Hence, PAPR reduction techniques need to be considered in NTN.

PAPR effects considering the HPA model

There are mainly two kinds of amplifiers in wireless communication systems, i.e., solid state power amplifier (SSPA) and traveling wave tube amplifier (TWTA). SSPAs are usually used in mobile communication systems and TWTAs are generally applied in satellite communication systems when high transmit power is required [,]. Therefore, for evaluating high PAPR effects on transmitted signals in NTN, the TWTA model is used to simulate the CP-OFDM BER performance.

The TWTA can be modelled as [,]

where  is the input signal of the HPA. y(t) is the output signal of the HPA. Amplitude modulation to amplitude modulation (AM/AM) conversion A[r(t)] and amplitude modulation to phase modulation (AM/PM) conversion  can be expressed as

where α and β are real-valued parameters that can be used to tune the model to fit a specific amplifier. Here we use the values in [6] as an example, i.e., , , , .

| Figure 2 Amplitude and phase transfer functions of original TWTA model | Figure 3 Amplitude transfer function of TWTA model with ideal pre-distorter |
| --- | --- |

Figure 2 shows the nonlinear amplitude and phase transfer functions of the original TWTA model without pre-distorter (PD). It can be seen that output signals of the TWTA experience severe amplitude and phase distortions. Hence, distortion compensation for the amplitude and the phase is needed before signals are fed into the TWTA. In Figure 3, the ideal PD [] is used to compensate for the distortion, and the linear range for the TWTA can be obtained.

Table 1 Simulation parameters

| Assumption | Value |
| --- | --- |
| Channel | AWGN |
| Modulation | QPSK |
| PD | Ideal PD |
| IBO (input backoff) | 5 dB |
| Subcarrier number | 512 |

Figure 4 BER performance for CP-OFDM signals with the TWTA model

For evaluating the effects of HPA non-linear distortions on decoding performance at the receiver, some simulations are implemented. Simulation parameters are shown in Table 1. Figure 4 shows that, with QPSK modulation and IBO=5 dB, the BER performance of CP-OFDM signals degrades because of non-linear distortions, compared with the CP-OFDM signals which are not going through the TWTA. Meanwhile, the IBO of input signals reduces the TWTA power efficiency.

Observation 1: High PAPR of CP-OFDM signals would reduce the system BER performance because of non-linear distortions, and the power amplifier efficiency because of the large IBO for power-limited satellites in NTN.

Criteria for selection of PAPR reduction techniques

According to the above analysis, it can be seen that the satellite with the CP-OFDM waveform may need to apply some PAPR reduction techniques to improve its power efficiency and relieve non-linear distortions. Here we list some criteria for selection of PAPR reduction techniques:

- PAPR reduction capability: This is clearly the most important factor for choosing a PAPR reduction technique. The chosen one should be able to reduce the CP-OFDM signals PAPR to an acceptable level.

- BER performance: After applying some PAPR reduction techniques, the original constellation signals may be changed or the signal average transmit power may be increased [], e.g., amplitude clipping, which makes decoding more difficulty. This is also an important factor, which has to be considered.

- Data rate: Some PAPR reduction techniques need to transmit “redundant signals” to the receiver, e.g., side information of partial transmit sequence (PTS) [] and selective mapping (SLM) [], which will reduce the system data rate.

- Implementation complexity: Another important factor, which needs to be considered for PAPR reduction technique selection, is the implementation complexity. Some techniques, such as PTS and SLM, need to search for an optimal or suboptimal solution to reduce PAPR, which need many iterations. It can be generally concluded that higher implementation complexity introduces better PAPR reduction capacity.

Observation 2: Considering the trade-off between PAPR reduction and other performance, some criteria, e.g. PAPR reduction capability, BER performance, data rate and implementation complexity, should be considered before choosing a specific PAPR reduction technique for NTN.

PAPR reduction techniques

In this section, we intend to give some simulation results of existing typical PAPR reduction techniques, which could be a benchmark for further study. Simple distortion techniques and probabilistic technique for reducing PAPR are compared, including clipping and filtering (CAF) [], companding [] and PTS [].

Amplitude clipping removes the peak parts of CP-OFDM signals which are outside the allowed region. However, the clipping would introduce the out-of-band radiation, which can be eliminated by filtering. Companding technique reduces the PAPR in another non-linear way compared with clipping, i.e., the transform formula is changed. Here, a simple example in [12] is applied. PTS partitions a data block into a number of disjoint sub-blocks. Then they are weighted by phase rotation factors separately after the IDFT. The optimal rotation factor combination is selected for minimizing the PAPR of resulting CP-OFDM signals. Meanwhile, the side information of the optimal rotation factors needs to be transmitted to the receiver for decoding the transmitted signals correctly, which may decrease the system data rate.

Figure 5 shows PAPR performance comparisons of these different PAPR reduction techniques. The parameters of these reduction techniques are selected for obtaining similar PAPR performance, which are listed in Table 2, so that we can compare their BER performance fairly.

Figure 5 PAPR performance of CP-OFDM signals with different PAPR reduction techniques

Table 2 Simulation parameters for PAPR reduction techniques

| Assumption | Value |
| --- | --- |
| Channel | AWGN |
| Modulation | QPSK |
| Subcarrier number | 256 |
| Clipping ratio (CAF) | 3.8 |
| k1 (Companding) | 2 |
| V (PTS) | 4 |
| Rotation factors (PTS) | {±1, ±j} |

Figure 6 BER performance of CP-OFDM signals with different PAPR reduction techniques

Table 3 Comparisons of PAPR reduction techniques

| PAPR reduction technique | BER increase | Data rate loss | Implementation complexity |
| --- | --- | --- | --- |
| CAF | Yes | No | Low |
| Companding | Yes | No | Low |
| PTS | No | Yes | High |

From Figure 6, it can be seen that the distortion techniques (CAF and companding) reduce the CP-OFDM BER performance, and the PTS does not change that. It can be concluded that these reduction techniques can improve CP-OFDM signals PAPR performance and the HPA power efficiency with some other performance loss. In Table 3 we summarize these PAPR reduction techniques according to the criteria of section 4.

Observation 3: The HPA power efficiency can be improved with PAPR reduction techniques at the cost of some other performance loss for NTN.

Conclusions

This contribution concludes with the following observations:

Observation 1: High PAPR of CP-OFDM signals would reduce the system BER performance because of non-linear distortions, and the power amplifier efficiency because of the large IBO for power-limited satellites in NTN.

Observation 2: Considering the trade-off between PAPR reduction and other performance, some criteria, e.g. PAPR reduction capability, BER performance, data rate and implementation complexity, should be considered before choosing a specific PAPR reduction technique for NTN.

Observation 3: The HPA power efficiency can be improved with PAPR reduction techniques at the cost of some other performance loss for NTN.

- References

[] RP-181370, “Study on solutions evaluation for NR to support Non-Terrestrial Network”, Thales, RAN#80, La Jolla, USA, June 11-14, 2018

[] 3GPP TR 38.811 V 15.0.0, “Study on New Radio (NR) to support non terrestrial networks (Release 15)”, August 10, 2018

[] K. Kang, S. Kim, D, Ahn, et al. “Efficient PAPR reduction scheme for satellite MC-CDMA systems”, IEE Proceedings-Communications, 2005, 152(5): 697-703.

[] B. M. Lee and R. J. P. de Figueiredo, “Adaptive predistorters for linearization of high-power amplifiers in OFDM wireless communications”, Circuits, Systems and Signal Processing, 2006, 25 (1): 59-80.

[] J. M. Weekley and B. J. Mangus, “TWTA versus SSPA: A comparison of on-orbit reliability data”, IEEE Transactions on Electron Devices, 2005, 52(5): 650-652.

[] A. A. Saleh, “Frequency-independent and frequency-dependent nonlinear models of TWT amplifiers”, IEEE Transactions on communications, 1981, 29(11): 1715-1720.

[] 3GPP TR 38.803 V14.2.0, “Study on new radio access technology: Radio Frequency (RF) and co-existence aspects (Release 14)”, September 2017

[] A. Brajal and A. Chouly, “Compensation of nonlinear distortions for orthogonal multicarrier schemes using predistortion”, 1994 IEEE GLOBECOM. Communications: The Global Bridge, San Francisco, CA, USA, 1994, pp. 1909-1914

[] T. Wattanasuwakull and W. Benjapolakul, “PAPR Reduction for OFDM Transmission by using a method of Tone Reservation and Tone Injection”, 2005 5th International Conference on Information Communications & Signal Processing, Bangkok, 2005, pp. 273-277.

[] S. H. Muller and J. B. Huber, “OFDM with reduced peak-to-average power ratio by optimum combination of partial transmit sequences”, Electronics Letters, 1997, 33(5): 368-369.

[] R. W. Bauml, R. F. H. Fischer, and J. B. Huber, “Reducing the peak-to-average power ratio of multicarrier by selected mapping”, Electronics Letters, 1996, 32(22): 2056-2057.

[] X. Li and L. J. Cimini, “Effects of clipping and filtering on the performance of OFDM”, 1997 IEEE 47th Vehicular Technology Conference. Technology in Motion, Phoenix, AZ, USA, 1997, pp. 1634-1638.

[] R1-1608828, “Overview of PAPR reduction techniques”, Huawei, HiSilicon, RAN1#86bis, Lisbon, Portugal, October 10 - 14, 2016

[] S. H. Muller and J. B. Huber, “OFDM with reduced peak-to-average power ratio by optimum combination of partial transmit sequences”, Electronics Letters, 1997, 33(5): 368-369.