3GPP TSG RAN WG1 Meeting #96																R1-1903485

Athens, Greece, February 25th - March 1st, 2019

Source:	Intel Corporation

Title:	Offline Discussion Outcome on Intermediate Conclusions for NR Positioning Study Item and Remaining Open Aspects

Agenda item:	7.2.10

Document for:	Discussion and Decision

Introduction

At the RAN#81 WG meeting, the final version of study item on NR Positioning was approved [1]. Following the objective of the study item description document RAN1 has achieve the following results during the work on NR positioning study item by RAN WG1:

Discussed and agreed on target requirements for NR RAT dependent positioning solutions

Developed evaluation methodology for NR positioning system level studies

Identified positioning techniques for NR DL, UL, as well as DL&UL based positioning in FR1 and FR2

Identified reference signals for UE and gNB measurements to support various positioning techniques

Conducted comprehensive system level performance evaluation to analyze performance for various solutions

In this contribution, we provide outcome of the offline discussion on conclusions of the NR positioning study item as well as additional list of open items that needs to be concluded in study item.

# E-CID Support for NR Positioning

The following consensus was reached based on offline discussion with respect to E-CID support.

Offline consensus

NR supports E-CID based positioning

  - E-CID downlink measurements are supported based on at least RRM measurements defined in NR Rel. 15

  - Note:

    - Support of NR R16 signals, measurements and procedures for E-CID are to be discussed when specified

    - NR R15 signals may be used to define additional UE/gNB measurements to facilitate E-CID support

# Text Proposal for NR Positioning Conclusions

In the next section, we provide text proposal on intermediate conclusions for NR Positioning study item.

Offline Consensus

Proposal:

The text below is captured in the conclusion section of the 3GPP TR 38.855

-------------------------------------------- Start of Text Proposal ----------------------------------------------------------------

This study focused on analysis of the 3GPP NR RAT dependent positioning technologies and RAT-independent positioning technologies.

Regulatory and commercial target performance requirements for RAT dependent solutions are captured in Section 5.

Three NR positioning evaluation scenarios were identified for system level analysis of RAT-dependent positioning techniques as described in Section 6.1.

Evaluation results have shown that in considered evaluation scenarios it is feasible to achieve regulatory and commercial target performance requirements for horizontal positioning accuracy using RAT-dependent solutions as described in Section 8.4 that provides summary of evaluations results from different sources. The summary in Section 8.4 indicates which RAT-dependent solutions can reach target performance requirements for each evaluation scenario.

Based on conducted analysis it is recommended to define reference signals to be used for UE and gNB measurements to facilitate support of the following positioning techniques:

-	DL based positioning

-	DL-TDOA and DL-AoD based positioning techniques

-	UL based positioning

-	UL-TDOA and UL-AoA based positioning techniques

-	DL and UL based positioning

-	RTT based positioning technique

-	E-CID based positioning technique

At least the following set of measurements was identified as necessary for NR RAT dependent positioning solutions and recommended to be specified:

-	UE measurements based on reference signals from serving and neighbouring gNBs

-	DL RSTD measurements

-	DL RSRP measurements

-	UE RX-TX time difference measurements

-	gNB measurements at serving and neighbouring gNBs

-	UL RTOA measurements

-	UL Angle of Arrival (AoA) measurements (including Azimuth and Zenith Angles)

-	UL RSRP (reference signal received power) measurements

-	gNB RX-TX time difference measurements

It was identified that enhancements are necessary to meet horizontal accuracy requirements in some scenarios. In order to achieve target performance characteristics, it is recommended to introduce new downlink positioning reference signals (DL - PRS). As for UL reference signals, it was identified that UL SRS can be used as a starting point for UL PRS for NR positioning.

It is recommended to proceed with a normative work to support NR positioning.

-------------------------------------------- End of Text Proposal -----------------------------------------------------------------

# Remaining Open Aspects for NR Positioning

At the previous RAN1 WG meeting, the following additional aspects were discussed:

Additional aspects that have been proposed during the study: - Receive signal waveform reporting - UE-based positioning - Multipath measurements - SFN transmissions for positioning - Techniques to lower UE complexity for OTDOA - Broadcast of assistance data - LMUs NR based positioning for UEs in RRC-IDLE and/or RRC-INACTIVE states

In our view, it is desirable to conclude on at least the following open aspects during the study item phase:

Receive signal waveform reporting

UE-based positioning

Multipath measurements

Positioning for UEs in RRC-IDLE and/or RRC-INACTIVE states

Use of NR R15 legacy signals (e.g. PRACH, SSB, CSI-RS(TRS)) for NR positioning

The other aspects are either outside of the RAN1 WG scope or can be resolved during WI phase. Our views on the identified above aspects are provided in the companion contributions [2]-[7].

  - RAN1 is to discuss and try to conclude on at least the following open aspects:

    - Receive signal waveform reporting

    - UE-based positioning

    - Multipath measurements

      - Including LOS detection

    - Positioning for UEs in RRC-IDLE and/or RRC-INACTIVE states

    - Use of PRACH, SSB, CSI-RS(TRS) signals for NR positioning

Conclusions

In this contribution, we have provided text proposal for the 3GPP TR summarizing intermediate conclusions of the NR Positioning Study from RAN1 perspective. In addition we have listed open aspects proposed during the study item where additional conclusion needs to be reached.

References

- RP-182155, “Revised SID: Study on NR positioning support”, Intel Corporation, Ericsson, Gold Coast, Australia, September 2018

- R1-1902511, “Design Aspects for NR DL Positioning”, Intel Corporation, Greece, Athens, February, 2019

- R1-1902512, “Design Aspects for NR UL Positioning”, Intel Corporation, Greece, Athens, February, 2019

- R1-1902513, “Design Aspects for NR DL and UL Positioning”, Intel Corporation, Greece, Athens, February, 2019

- R1-1902514, “NR Positioning Evaluation Results”, Intel Corporation, Greece, Athens, February, 2019

- R1-1902515, “Comparative Analysis of NR Positioning Techniques”, Intel Corporation, Greece, Athens, February, 2019

- R1-1902516, “Reporting of Received Signal Waveform to Boost NR Positioning Performance”, Intel Corporation, Greece, Athens, February, 2019

- 3GPP TR 38.855, “Study on NR positioning support”, v1.0.1