3GPP TSG RAN WG1 96			R1-1903501

Athens, Greece, 25th February – 1st March, 2019

Agenda item:	7.2.8.1

Source: 	Samsung

Title: 	Summary of Offline Email Discussion on MU-MIMO CSI

Document for:	Discussion and Decision

# Introduction

The outcome of offline email discussion on MU-MIMO CSI enhancements is summarized below.

Summary

## LC coefficient quantization

From the FL summary [1], the following observation is given:“ On LCC quantization,

- Alt1, Alt2, and Alt4 receive equal majority support (8 companies per alternative).

- A good number of companies show that the performance of LCC quantization schemes Alt1, Alt2, and/or Alt4 are within the same ballpark.”

It was also agreed that “… Further discuss to select one of the five alternatives later this week (in RAN1#96)

- Any new alternative (including merged/compromise proposals) will not be considered for down selection.”

In addition to the available from the contributions referred in the FL summary [1], new simulation results have been provided in the updated contributions in section 3 (e.g. [2]-[6]) as well as the ones shared on the reflector. After some offline discussion, some companies have revised their positions. A summary of the revised views is given below.

Table 1 LCC quantization: summary of revised companies’ views

| Category | No. companies | Companies |
| --- | --- | --- |
| Alt1 (Rel.15+16PSK) | 2 | LGE, Qualcomm |
| Alt2 (differential per polarization) | 12 | Ericsson, Huawei/HiSi, Intel, MediaTek, MotM/Lenovo, OPPO, Samsung, Spreadtrum, vivo, ZTE |
| Alt3 (ABC) | 3 | Fraunhofer/HHI, NTT Docomo |
| Alt4 (two-part amplitude) | 2 | Nokia/NSB |
| Alt5 (two-part phase) | 1 | CATT |

Observation 1: On LCC quantization, Alt2 (differential per polarization) represents the majority view.

Proposal 1: On LCC quantization, agree on Alt2 (differential per polarization) per the description in R1-1902304

## Subset selection for RI=2

Pending issue for RI=2: Down select among the three alternatives below

- Alt 1A: layer-common FD basis subset selection, layer-common coefficient subset selection

- Alt 1B: layer-common FD basis subset selection, layer-independent coefficient subset selection

- Alt 2: layer-independent FD basis subset selection, layer-independent coefficient subset selection

On the three alternatives, the views from different companies can be summarized below (after email discussion). Following the suggestion from MediaTek during the Tuesday online session, companies’ views on FD basis subset selection and coefficient subset selection are also separately considered below.

Table 2 RI=2 subset selection: summary of revised companies’ views

| Category | No. companies | Companies |
| --- | --- | --- |
| Alt1A (common basis & coefficient) | 1 | MediaTek |
| Alt1B (common basis, independent coefficient) | 2 | Ericsson, MediaTek |
| Alt2 (independent basis & coefficient) | 20 | CATT, Ericsson, Fraunhofer/HHI, Huawei/HiSi, Intel, LGE, OPPO, Qualcomm, MotM/Lenovo, NEC, Nokia/NSB, NTT Docomo, Samsung, Spreadtrum, vivo, ZTE |

Table 3 RI=2 FD basis subset selection: summary of revised companies’ views

| Category | No. companies | Companies |
| --- | --- | --- |
| Layer-common | 2 | Ericsson, MediaTek |
| Layer-independent | 20 | CATT, Ericsson, Fraunhofer/HHI, Huawei/HiSi, Intel, LGE, OPPO, Qualcomm, MotM/Lenovo, NEC, Nokia/NSB, NTT Docomo, Samsung, Spreadtrum, vivo, ZTE |

Table 4 RI=2 coefficient subset selection: summary of revised companies’ views

| Category | No. companies | Companies |
| --- | --- | --- |
| Layer-common | 1 | MediaTek |
| Layer-independent | 21 | CATT, Ericsson, Fraunhofer/HHI, Huawei/HiSi, Intel, LGE, OPPO, Qualcomm, MediaTek, MotM/Lenovo, NEC, Nokia/NSB, NTT Docomo, Samsung, Spreadtrum, vivo, ZTE |

In addition, the majority of companies strongly expressed the need for finalizing both FD basis selection and coefficient selection in this meeting. This is because, e.g. both issues heavily affect the design for RI=3 and 4 extension (which will start in RAN1#96B).

Observation 2: For RI=2,

- All companies support layer-independent coefficient subset selection

- Layer-independent FD basis subset selection represents the super-majority view (20 vs. 2)

- Hence, Alt2 (layer-independent FD basis and coefficient subset selection) represents the super-majority view (20 vs. 2)

Proposal 2: For RI=2, agree on Alt2:

- Layer-independent FD basis subset selection

- Layer-independent coefficient subset selection

# References

- R1-1902304, Samsung, Summary of CSI enhancement for MU-MIMO

- R1-1903434, Samsung, SLS evaluation on MU-MIMO CSI: LC coefficient quantization

- R1-1903505, Motorola Mobility, Lenovo, On MU-MIMO CSI enhancements

- R1-1903343, ZTE, CSI enhancements for MU-MIMO support

- R1-1903569, vivo, Performance evaluation on type II CSI compression

- R1-1903503, MediaTek, Discussion on quantization schemes