3GPP TSG RAN WG1 Meeting #96	R1-1903513

Athens, Greece, Feb 25 – March 1, 2019

Agenda Item:	6.2.2.2

Source:	Huawei, HiSilicon

Title:	Updated feature lead summary of Support for transmission in preconfigured UL resources

Document for:	Discussion and decision

# Introduction

One of the objectives of the Rel-16 WI on additional enhancements for NB-IoT [1] is to specify support for UL transmission in preconfigured resources.

Improved UL transmission efficiency and/or UE power consumption: - Specify support for transmission in preconfigured resources in idle and/or connected mode based on SC-FDMA waveform for UEs with a valid timing advance[RAN1, RAN2, RAN4] - Both shared resources and dedicated resources can be discussed - Note: This is limited to orthogonal (multi) access schemes

Agreements on transmission on UL preconfigured resources in RAN1#94 [2], RAN1#94b [3] and RAN1#95 [4] follow.

RAN1#94

Agreement Idle mode based pre-configured UL resources is supported for UEs in possession of a valid TA - FFS: Validation mechanism for TA - FFS: How the pre-configured UL resources is acquired Agreement For transmission in preconfigured UL resources, the UE may use the latest TA of which its validity can be confirmed Agreement Study both shared and dedicated resource for preconfigured UL resources. If both shared and dedicated resources are supported, strive for commonality in design of both resource types. Agreement HARQ procedures for transmission in preconfigured UL resources should be studied and the following aspects should be considered: - Whether to support HARQ; - If supported, details of HARQ design including the number of HARQ processes; - Whether ACK/NACK is necessary Fallback mechanisms should be considered, e.g. fallback to legacy RACH/EDT procedures.

RAN1#94b

Agreement - In idle mode, the UE will at least consider one or more of the following attributes when validating TA (combination of multiple attributes is allowed): - Serving cell changes (serving cell refers the cell that the UE is camping on) - Time Alignment Timer for idle mode - Serving cell NRSRP changes (serving cell refers the cell that the UE is camping on) - FFS Other attributes: - Neighbour cell NRSRP change - TDOA of >=2 eNBs - TA History - Subscription based UE differentiation - Others not precluded (for example, attributes that need to be considered for high mobility UEs) - Note that UE power consumption should be taken into account for the FFS attributes Agreement For transmission in preconfigured UL resources, an RRC idle UE may use the latest TA that passed the validation criteria Agreement Pre-configured UL resources for transmission of data are indicated by RRC signaling. At least UE-specific RRC signaling is supported. Agreement The resource configuration includes at least the following - Time domain resources including periodicity(s) - Frequency domain resources - TBS(s)/MCS(s) Agreement For UL transmission in preconfigured resource, fallback mechanism to RACH/EDT procedures is supported. Agreement Dedicated preconfigured UL resource is defined as an NPUSCH resource used by a single UE - NPUSCH resource is time-frequency resource - Dedicated PUR is contention-free Contention-free shared preconfigured UL resource (CFS PUR) is defined as an NPUSCH resource simultaneously used by more than one UE - NPUSCH resource is at least time-frequency resource - CFS PUR is contention-free Contention-based shared preconfigured UL resource (CBS PUR) is defined as an NPUSCH resource simultaneously used by more than one UE - NPUSCH resource is at least time-frequency resource - CBS PUR is contention-based (CBS PUR may require contention resolution) Agreement In IDLE mode, HARQ is supported for transmission in dedicated PUR - A single HARQ process is supported, - FFS whether two HARQ processes are supported - FFS: The design of the corresponding NPDCCH search space Agreement In idle mode, dedicated PUR is supported. - Support for CFS PUR is FFS. - Support for CBS PUR is FFS.

RAN1#95

Agreement In idle mode, at least the following TA validation attributes are supported: - Serving cell changes (serving cell refers the cell that the UE is camping on) - Time Alignment Timer for idle mode - Serving cell NRSRP changes (serving cell refers the cell that the UE is camping on) - Based on NRSRP measurement definition in existing Rel-15 TS36.214 Send LS to RAN2, RAN4 to consider in their work. All the agreements with ‘include in LS to RAN2, RAN4’ for NB-IoT and eMTC should be captured in this LS. The LS is endorsed in R1-1813778 Agreement The UE can be configured to use at least these TA validation attributes: - Time Alignment Timer for idle mode - Serving cell NRSRP changes - Note: the configuration shall support disabling of the TA validation attributes Include in LS to RAN2, RAN4. For further study: TA validation attributes: - Subscription based UE differentiation (or Stationary indication in held in subscription) - Cell specific indication where TA is valid within that cell Agreement Include in LS to RAN2, RAN4: RAN1 assumes that a UE transitioning from EDT/connected to idle mode can use the valid TA that was used while in EDT/connected mode. Agreement For dedicated PUR in idle mode, the UE may skip UL transmissions. - FFS: Resource release mechanism - FFS: Whether or not to support mechanism to disable skipping by eNB Agreement In idle mode, only one HARQ process is supported for dedicated PUR Agreement For dedicated PUR in idle mode, UL grant for HARQ retransmission is transmitted in search space - FFS: Details on the search space (for example USS, CSS)

This contribution is the updated feature lead summary to be discussed in the comeback session.

# From Monday online PUR session

Agreement

When the UE is configured to use several TA validation criteria, the TA is valid only when all the configured TA validation criteria are satisfied.

For futher discussion

The value of infinity is supported for both the Time Alignment Timer for idle mode and for the NRSRP change threshold.

For futher discussion

TA value for UE transitioning from connected to idle mode:

- An idle UE can reuse the last TA command in serving cell before going to idle for transmission in PUR if it is still valid according to the configured TA validation attributes

For futher discussion

When the TA is invalid, the UE may obtain TA via legacy RACH or EDT

- FFS other mechanisms for TA update

For futher discussion

Support both UE triggered and eNB triggered resource release mechanism

  - FFS the details

For futher discussion

For dedicated PUR in idle mode, the PUR ACK is at least sent on NPDCCH

- RAN2 can decide if a higher layer PUR ACK is also supported

For futher discussion

After data transmission on PUR, upon unsuccessful decoding by eNB, the UE can expect an UL grant for retransmission on NPDCCH.

  - FSS if the UE can also expect an explicit NACK on NPDCCH.

Agreement

For dedicated PUR in idle mode, the PUR search space configuration shall be included in the PUR configuration.

- PUR search space is the search space where UE monitors for NPDCCH

- FFS: Whether PUR search space is common or UE specific

For further discussion: details on CFS PUR including whether to support it

# Come back - Possible agreements

Agreement#1

In idle mode, the TA validation configuration can include “PUR Time Alignment Timer”

- Where the UE considers the TA as invalid if the (current time – time at last TA update) > the PUR Time Alignment Timer

- Details on how to specify the “PUR Time Alignment Timer” is up to RAN2

Agreement#2

In idle mode, when the UE validates TA, the UE considers the TA for the previous serving cell as invalid if the serving cell changes

- Above applies for the case where the UE is configured to use the serving cell change attribute

Agreement#3

For dedicated PUR in idle mode, the dedicated PUR ACK is at least sent on NPDCCH

- RAN2 can decide if a higher layer PUR ACK is also supported

Agreement#4

When the TA is validated and found to be invalid and the UE has data to send, the UE can obtain a valid TA and may send data via legacy RACH or EDT procedures

- FFS whether only TA is acquired and then data sent on PUR is supported

- FFS other approaches to obtain a valid TA

Agreement#5

For dedicated PUR, in idle mode, the PUR resource configuration includes at least the following

- Time domain resources including periodicity(s)

    - Note: also includes number of repetitions, number of RUs, starting position

- Frequency domain resources

- TBS(s)/MCS(s)

- Power control parameters

Agreement#6

In idle mode, at least the following PUR configurations and PUR parameters may be updated after a PUR transmission:

Timing advance adjustment

UE TX power adjustment

FFS: Repetition adjustment for NPUSCH

FFS: Whether the above update is done in L1 and/or higher layer

Agreement#7

In idle mode, the PUR search space configuration includes at least the following:

NPDCCH repetitions and aggregation levels

NPDCCH starting subframe periodicity (variable G)

Starting subframe position (alpha_offset)

# References

- RP-181451	WID revision: Additional enhancements for NB-IoT, September 2018.

- Chairman’s Notes RAN1#94, August 2018.

- Chairman’s Notes RAN1#94b, October 2018.

- Chairman’s Notes RAN1#95, November 2018.

- R1-1901503	Transmission in preconfigured UL resources	Huawei, HiSilicon

- R1-1901629	NB-IOT Pre-configured UL Resources Design Considerations 	Sierra Wireless, S.A.

- R1-1901733	Early Transmission in Preconfigured UL resources in NB-IoT	MediaTek Inc.

- R1-1901747	Support for transmission in preconfigured UL resources in NB-IoT	Ericsson

- R1-1901866	Support for transmission in preconfigured UL resources for NB-IoT	ZTE (Late submission)

- R1-1901964	Preconfigured Grant for Uplink transmission	Nokia, Nokia Shanghai Bell

- R1-1902064	Discussion on preconfigured UL resources in NB-IoT	LG Electronics

- R1-1902149	UL transmission in preconfigured resources for NBIoT	Lenovo, Motorola Mobility

- R1-1902217	Discussion on transmission in preconfigured UL reosurces for NB-IoT	Samsung

- R1-1902375	Support for transmission in preconfigured UL resources	Qualcomm Incorporated

- R1-1902458	UL transmission in preconfigured resources for NB-IoT	Intel Corporation

- R1-1902553	Consideration for preconfigured uplink resources (PUR)	Sequans Communications

- R1-1902779	Discussion on preconfigured UL resources in NB-IoT	NTT DOCOMO, INC.

- R1-1902933	Discussion on transmission in preconfigured UL resources	III