3GPP TSG-RAN WG1 Meeting #96	DRAFT CR R1-1903683

# Athens, Greece, February 25th - March 1st, 2019

| CR-Form-v11.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.213 | CR | Draft | rev | - | Current version: | 15.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | QCL properties of Msg4 in CONNECTED Mode |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | - |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 28-Feb-2019 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-14 (Release 15) Rel-14 (Release 16) |  |
| Reason for change: |  | The QCL properties of Msg4 in CONNECTED mode addressed to the C-RNTI are missing for PDCCH and PDSCH. In TS38.214, the only case covered is the PDSCH addressed to TC-RNTI. However, a Msg4 PDSCH addressed to the C-RNTI is not covered. Earlier RAN1 agreement didn’t distinguish TC-RNTI and C-RNTI: Agreements: (RAN1-#90bis Prague) - If there is no beam reporting in RACH message 3, UE may assume that the DMRS of PDCCH and the DMRS of PDSCH conveying Msg4 are QCL’ed with the SS block that the UE selected for RACH association and transmission - FFS: Whether to support beam reporting in Msg3 - FFS: If and how beam reporting in RACH message 3 impacts message 4 Tx QCL assumption There is no beam reporting in Msg3 |  |  |  |  |  |  |  |  |
| Summary of change: |  | Msg4 PDCCH and PDSCH are QCLed with the SS/PBCH Block Index selected for preamble assoication and transmission. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Specifications is not aligned with earlier RAN1 agreement which states that the DMRS of PDCCH and PDSCH of Msg4 are QCLed with the SS block that the UE selected for RACH association and transmission. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  | Isolated Impact Analysis: This impacts the case of CBRA in CONNECTED mode when the DCI is addressed to the C-RNTI. If network implements this change, but the UE doesn’t implement the change, the UE may attempt to receive Msg4 with different QCL assumptions and the reception of Msg4 fails. It should be noted that in case of CB-BFR, the attempt of the UE to receive Msg4 on any beam other than that selected for RACH preamble assoication would most likely fail. If the UE implements this change and the network doesn’t implement. A base station with beam reciprocity attempts to transmit Msg4 on the same beam used for reception of Msg3. The Msg3 beam is the beam associated with the selected preamble. In this case, the UE is expected to receive Msg4. |  |  |  |  |  |  |  |  |

## 8.4	PDSCH with UE contention resolution identity

In response to a PUSCH transmission scheduled by a RAR UL grant when a UE has not been provided a C-RNTI, the UE attempts to detect a DCI format 1_0 with CRC scrambled by a corresponding TC-RNTI scheduling a PDSCH that includes a UE contention resolution identity [11, TS 38.321]. In response to the PDSCH reception with the UE contention resolution identity, the UE transmits HARQ-ACK information in a PUCCH. The PUCCH transmission is within a same active UL BWP as the PUSCH transmission. A minimum time between the last symbol of the PDSCH reception and the first symbol of the corresponding PUCCH transmission with the HARQ-ACK information is equal to  msec.  is a time duration of  symbols corresponding to a PDSCH reception time for UE processing capability 1 when additional PDSCH DM-RS is configured.

If a UE detects a DCI in response to a PUSCH transmission scheduled by a RAR UL grant or corresponding PUSCH retransmission, the UE may assume same DM-RS antenna port quasi co-location properties, as described in [6, TS 38.214], as for a SS/PBCH block resource the UE used for PRACH association, as described in Subclause 8.1, regardless of whether or not the UE is provided TCI-State for the CORESET where the UE receives the PDCCH with the DCI.