3GPP TSG RAN WG1 Meeting #97	R1-1907627

Reno, USA, May 13-17, 2019

Agenda Item:	6.2.2.2

Source:	Huawei, HiSilicon

Title:	Further topics for transmission in preconfigured UL resources

Document for:	Discussion and decision

# Introduction

This contribution summarizes topics for discussion in Thursday online session, including comebacks.

From Chairman’s notes v4:

Agreement

- For dedicated PUR in idle mode and for HD-FDD UEs, the start of the PUR SS Window is [x] subframes after the end PUR transmission

FFS: Value of x, and if x is fixed or signaled

FFS: Support for monitoring of PUR SS Window before PUR transmission

Note: The PUR SS Window is the time period where the UE monitors the NPDCCH for at least a time period after a PUR transmission

Possible Agreement

If nothing is received in the time period after a PUR transmission, the UE may fallback to legacy RACH/EDT procedure or may wait for the next PUR resource.

Agreement

NPDCCH candidates are determined by USS like search space

- FFS: Other details on the USS like search space

  - Type2-CSS can also be discussed as part of the FFS

Possible Agreement

After data transmission on PUR, upon unsuccessful decoding by eNB, the UE can expect

  - an UL grant for retransmission on NPDCCH, or

  - an explicit NACK on NPDCCH, or

  - nothing

Possible Agreement

The RAN1 agreement from RAN1#96 is updated as follows:

When the TA is validated and found to be invalid and the UE has data to send, the UE can obtain a valid TA and may send data via legacy RACH or EDT procedures

- FFS whether only TA is acquired and then data sent on PUR is supported

- FFS other approaches to obtain a valid TA

Possible Agreement

CFS PUR is supported with

- CFS PUR is based on MU-MIMO

- UE is configured with a UE-specific DMRS

Conclusion

CBS PUR is not supported in Rel-16

# Release of PUR resources

Table 1

| Source | Proposals |
| --- | --- |
| Lenovo, Motorola Mobility | - Proposal 7: PUR resource can be configured as periodic starting subframe and resource duration for each PUR resource occasion. - Proposal 8: PUR resource is implicitly released by 1) transmission until to the preconfigured number; 2) consecutive multiple missed transmissions. - Proposal 9: eNB may trigger re-configuration/release of PUR configured to a UE by dedicated RRC or DCI. |
| Samsung | Proposal #8: Preconfigured UL resources can be activated/deactivated in system information. |
| NTT DOCOMO, INC. | Proposal 3: Both eNB triggered and UE triggered PUR release command should be supported • FFS: The details of signaling |
| III | Proposal 7: UE releases the preconfigured UL resource when no ACK has been received after m consecutive occasions of preconfigured UL resource. Proposal 8: UE releases the preconfigured UL resource when m retransmission has been scheduled since the last time D-PUR was configured. |

Regarding the resource release mechanism, some companies propose that it should be supported both eNB and UE triggered resource release. It can be considered the following agreement for further discussion:

Potential agreement#1:

For dedicated PUR, release of PUR resources can be triggered by either eNB or UE.

- FFS signaling

# Unused PUR

Table 2

| Source | Proposals |
| --- | --- |
| Huawei, HiSilicon | Observation 5: In legacy RACH/EDT procedures, the eNB can first identify UE activity based on preamble detection, and then decode the data when the eNB detects there are UE transmitting data. Observation 6: If the eNB does not know whether there is transmission in the preconfigured UL resources, the eNB needs to do blind processing on the PUR resources including assuming multiple hypotheses considering all the possible MCS, TBS, repetition numbers, etc. This impacts eNB complexity and power consumption. Proposal 12: RAN1 considers eNB blind decoding complexity and power consumption in the design of PUR mechanisms. Observation 7: UE activity detection can reduce the eNB complexity and power consumption, by helping the eNB to identify whether there is transmission in the preconfigured UL resources. The eNB then performs decoding if needed. Observation 8: For NB-IoT NPUSCH format 1, the DMRS is limited in bandwidth and sparse in time domain. So the UE activity detection performance (e.g., miss detection rate) based on DMRS needs further study. Proposal 13: Consider UE activity detection based on an indication signal. Proposal 14: Consider the impact on eNB scheduling due to preconfigured UL resources, e.g., separating preconfigured resources and normal NPUSCH resources. |
| Sequans Communications | Observation 4: Skipping PUR transmission is resource inefficient. Observation 5: Serving cell is unaware of UE moving out of the cell coverage until after the UE connects to another cell in the network. Observation 6: Assuming a UE will have ‘n’ PUR skips allowed by the network, a UE may wish to reserve some of its ‘n’ skips. Proposal 4: Study the use of small highly detectable signaling to notify the serving cell on unused PUR. Proposal 5: Study at which conditions it is most beneficial to use PURPLE. Observation 7: PURPLE may be used to terminate transmission with better efficiency than RRC message. |

It is suggested to further discuss aspects related to notifying eNB of unused PUR resources.

# Power control

Table 13 Summary of proposals

| Source | Related Proposals & Observations |
| --- | --- |
| Ericsson | Observation 6 The NB-IoT open loop power control seems to be a suitable candidate to be used as power control mechanism for PUR. Proposal 5 The NB-IoT open loop power control is used as power control mechanism for PUR. • FFS whether a slight modification will be need or not to adapt it better to the PUR transmissions. |
| LG Electronics | Proposal 7: Support UL transmission power control scheme for D-PUR initial transmission and retransmission.  The following options are proposed for further consideration o Opt.1 n-bit (n>=1) signaling or n (n>=1) states in UL grant DCI for power control command o Opt.2 UE autonomous power control without explicit signaling of power control command (e.g., UL transmission power ramp-up if there is no explicit ACK after a D-PUR transmission)  FFS whether to reset the TPC for every initial UL transmission in D-PUR e.g., for the case where the D-PUR period is large.  FFS whether to use the UL TPC results when UE falls back to RACH/EDT procedure. |

It is suggested to discuss further. It can be considered to either to agree that NB-IoT open loop power control is used for PUR, or capture on the Chairman’s notes

FFS Potential enhancements of power control mechanisms for PUR. (The baseline is the existing NB-IoT open loop power control.)

# Comebacks

Comeback#1

Possible Agreement

If nothing is received in the time period after a PUR transmission, the UE may fallback to legacy RACH/EDT procedure or may wait for the next PUR resource.

This agreement is about the UE behaviour for the following case:

After a PUR transmission, the UE monitors the NPDCCH for a certain time period, which is one PUR search space window (search space window is defined in one of the agreements at this meeting). If during this time period the UE does not detects any NPDCCH, the UE shall consider the data transmission on PUR not successful and fall back to legacy RACH/EDT procedure. In this case, UE behaviors described in the possible agreement from the chairman’s notes above, and in contributions, are both included. It is suggested to consider the following:

Proposed agreement#1: After a PUR transmission, the UE monitors the NPDCCH for a time period (for example the PUR search space window). If during this time period the UE does not detects any NPDCCH, the UE shall consider the data transmission on PUR not successful and fall back to legacy RACH/EDT procedure.

Comeback#2

Possible Agreement

After data transmission on PUR, upon unsuccessful decoding by eNB, the UE can expect

  - an UL grant for retransmission on NPDCCH, or

  - an explicit NACK on NPDCCH, or

  - nothing

The option “nothing” can correspond to what proposed in proposed agreement#1, i.e. the UE monitors NPDCCH in a time period but it does not receive anything.

Comeback#3

Possible Agreement

The RAN1 agreement from RAN1#96 is updated as follows:

When the TA is validated and found to be invalid and the UE has data to send, the UE can obtain a valid TA and may send data via legacy RACH or EDT procedures

- FFS whether only TA is acquired and then data sent on PUR is supported

- FFS other approaches to obtain a valid TA

Comeback#4

Possible Agreement

CFS PUR is supported with

- CFS PUR is based on MU-MIMO

- UE is configured with a UE-specific DMRS

# References

- RP-190757	WID revision: Additional enhancements for NB-IoT, Huawei, Shenzhen, China, March 2019.

- R1-1905966	Support for transmission in preconfigured UL resources in NB-IoT	Ericsson

- R1-1905972	Transmission in preconfigured UL resources	Huawei, HiSilicon

- R1-1906190	Discussion on preconfigured UL resources in NB-IoT	NTT DOCOMO, INC.

- R1-1906280	UL transmission in preconfigured resources for NBIoT	Lenovo, Motorola Mobility

- R1-1906461	NB-IOT Pre-configured UL Resources Design Considerations	Sierra Wireless, S.A.

- R1-1906506	Support for transmission in preconfigured UL resources for NB-IoT	ZTE

- R1-1906691	Discussion on preconfigured UL resources in NB-IoT	LG Electronics

- R1-1906711	Preconfigured Grant for Uplink transmission	Nokia, Nokia Shanghai Bell

- R1-1906773	UL transmission in preconfigured resources for NB-IoT	Intel Corporation

- R1-1906897	Discussion on transmission in preconfigured UL reosurces for NB-IoT	Samsung

- R1-1907001	Support for transmission in preconfigured UL resources	Qualcomm Incorporated

- R1-1907081	Consideration for preconfigured uplink resources (PUR)	Sequans Communications

- R1-1907358	Discussion on NB-IoT PUR resource configuration in respect to radio condition	ASUSTEK COMPUTER (SHANGHAI)

- R1-1907370	Discussion on transmission in preconfigured UL resources	III