3GPP TSG RAN WG1 #98bis		R1-1910705

Chongqing, CN, October 14th – 20th, 2019

Title:	[DRAFT] LS on PSBCH content

Response to:	-

Release:	Release 16

Work Item:	5G_V2X_NRSL-Core

Source:	Futurewei [RAN WG1]

To:	RAN WG2, RAN WG4

Cc:

Contact Person:

#### Name: Philippe Sartori

E-mail Address:	philippe.sartori@futurewei.com

Attachments:

1. Overall Description:

RAN1 discussed the PSBCH content and endorsed the fields of the table below:

| SL-MIB Content | Notes |
| --- | --- |
| sl_Bandwidth | Needed since the sidelink bandwidth could be configured by the network and different to what is used for out-of-coverage UEs. |
| tdd_ConfigSL | Indicates the TDD configuration |
| directFrameNumber | To obtain Frame Number for synchronization |
| directSlotNumber | To obtain Slot Number for synchronization. Similar field to directSubframeNumber |
| inCoverage | Indicates if the UE is synchronized with an in-network source or not |
| systemFrameNumber | Not needed since frame synchronization for sidelink is already achieved using the fields directFrameNumber and directSlotNumber |
| subCarrierSpacingCommon | Not needed. After detecting the SLSS, the UE has information about the SCS and the CP length. |
| ssb-SubcarrierOffset | Not needed. if a fixed location is selected for the synchronization signals on the sidelink, this parameter is not used. |
| Dmrs-TypeA-Position | Not needed. |
| Pdcch-ConfigSIB1 | Not needed. Already available at the UE from Uu synchronization procedure. |
| cellBarred | For NR Uu, this parameter controls whether the UE is allowed to camp on a cell. For NR SL, similarly in preventing/allowing V2X operation e.g. consideration of system interference from V2X. |
| Reserved |  |

2. Actions:

RAN1 respectfully ask RAN2 to take into account RAN1’s decision.

3. Date of Next TSG-RAN2 Meetings:

3GPP RAN1#99		18 - 22 Nov 2019			Reno, US

3GPP RAN1#100		24 - 28 Feb 2020			Athens, GR