3GPP TSG-RAN WG1 Meeting #98bis	R1-1910982

Chongqing, China, 14th – 20th October 2019

Agenda Item:	7.2.5.3

Source:	Ericsson

Title:	On NTN synchronization, random access, and timing advance

Document for:	Discussion

# 1	Introduction

In RAN1#97 and RAN1#98, several agreements (as cited in Appendix A.1) were made related to NTN synchronization, random access and timing advance. In this contribution, we provide our views on these aspects for further progress.

# NTN downlink synchronization

In RAN1#98, it was concluded that the SSB design in Rel-15 can provide robust initial DL synchronization performance for LEO systems if the DL carrier frequency is pre-compensated for Doppler shift. The performance without pre-compensation was left for further study.

In this section, we provide simulation results that confirm the conclusion from RAN1#98 for the case without pre-compensation. Further, the performance in a LEO system without pre-compensation is investigated.

## 2.1	Simulation setup

The single-shot successful PCID detection rate during initial DL synchronization is simulated in a link simulator. The simulation setup follows the agreements in 38.821 [1] with channel model defined in 38.811 [2]. The most important parameters are summarized in Table 3 in Appendix A.2.

The PSS detection algorithm hypothesizes a number of frequency offsets in steps of 0.5 SCS and performs PSS detection for each hypothesis. The total number of frequency hypotheses has in each case been selected to cover the expected maximum frequency offset relative to the SCS.

The PSS/SSS detection algorithm has been tuned to get a false alarm rate (FAR) of at most 1%.

## 2.2	Simulation results

The probability of correctly detecting the PCID from a single SSB instance versus SNR is shown in Figure 1. In all cases, a detection probability of 90% is achieved at approximately -9.2 dB to -8.8 dB.

Figure 1: One-shot initial cell detection accuracy of PCID.

## 2.3	Discussion

The results confirm the observation from RAN1#98 that the Rel-15 SSB design can provide robust initial DL synchronization performance for LEO systems if the DL carrier frequency is pre-compensated for Doppler shift. Further, even without pre-compensation the performance is found to be robust.

It should be noted that the performance without pre-compensation comes at the cost of increased receiver complexity due to the increased number of frequency hypotheses. E.g., with a subcarrier spacing of 15 kHz the number of frequency hypotheses with and without pre-compensation was about 6 and 18, respectively.

Robust initial DL synchronization performance can be achieved without DL pre-compensation but with a significant increase in receiver complexity.

Comparing to the link budgets presented in R1-1910980 [3] the presented performance is robust and is a good start for facilitating efficient cell acquisition. To make the synchronization procedure even more efficient it is assumed beneficial for a UE to possess knowledge of the satellite ephemeris data. This will allow the UE to point its receiver beam towards the expected location of the satellite. In case of initial cell acquisition and PLMN selection this information can only be made available through pre-programming of the UE, or its uSIM. It is proposed that RAN1 agrees to further study the benefits of assisting the UE with pre-programmed information on the satellite orbit data for improving the initial cell acquisition performance. The importance of a correct initial RX beam orientation is also clearly indicated by the below plotted VSAT normalized antenna diagram. It shows that the gain is expected to drop 20 dB or more if the antenna is off by 2 degrees or more compared to its optimal orientation.

The benefits of pre-programming the UE, or its uSIM, with satellite ephemeris should be studied further by RAN1.

Figure 2: VSAT antenna diagram.

# NTN time and frequency tracking

The simulation results presented in the previous section demonstrate that SSB design in Rel-15 can provide robust performance in NTN networks, even in a challenging LEO NTN without pre-compensation. Nevertheless, it is important that UE can track DL time and frequency after the initial DL synchronization, especially in a LEO NTN where satellites are moving at high speed. The NTN design should facilitate UE tracking the DL time and frequency.

In this section, the Doppler shift of a LEO satellite with beams moving with the satellite (scenario C2/D2) is analyzed in detail. Simulations are run to assess the magnitude of the Doppler shift and Doppler variation rate per beam. The possibility to do Doppler frequency compensation based on statistical knowledge of the Doppler shift is investigated.

## 3.1	Simulation assumptions

The beam layout has been defined as a hexagonal pattern in UV plane with seven tiers of beams (127 cells) of which the six innermost tiers are evaluated while the outermost tier is used only to define the border of the evaluation area (see [3] for related discussions on simulation assumptions). The center beam bore sight direction is at (0,0). The projection on earth of the beam centers at the time instant when the satellite is at latitude=0, longitude=0 is shown in Figure 3. The green crosses correspond to beams in the evaluation area (tiers 1-6) while the red crosses correspond to beams used to define the limit of the evaluation area.

Figure 3: Beam centers on earth surface.

Other simulation parameters are summarized in Table 4 (see Section A.3.1	Simulation Parameters in the appendix). For brevity, only simulation results for S-band at 600 km altitude are discussed here. The simulation results for remaining scenarios are relegated to the appendix (see A.3.2	Simulation results: 1200 km altitude, S-band for S-band, A.3.3	Simulation results: Ka-band for Ka-band and A.3.4	Simulation results: Larger constellations, larger beam widths for impact of larger cell sizes).

## 3.2.	Simulation results: 600 km altitude, S-band

The DL Doppler shift versus UE position on earth surface (with satellite at position latitude=0°, longitude =0°) in the evaluation area is shown in Figure 4. The frequency offset is in the range ±18.5 kHz.

Figure 4: DL Doppler shift.

When DL pre-compensation per beam is applied based on the center of each beam to reduce the Doppler shift, the residual Doppler shift is in the range ±1.8 kHz, as shown in Figure 5.

Figure 5: Residual DL Doppler shift with pre-compensation.

As the satellite coverage area passes the UE, the UE will synchronize to the received DL frequency for each new cell it enters. As the UE moves further into a cell (or rather when the cell moves since moving beams are assumed), the received DL frequency will drift relative to the frequency at the initial synchronization. Assuming the UE synchronizes perfectly to the perceived DL frequency during initial DL synchronization at the time a UE enters each new cell but not performing further tracking of the DL frequency as the UE passes through the cell, the UE DL frequency offset is shown in Figure 6. The offset ranges from 0 Hz to -3.4 kHz. This indicates that continuous frequency tracking will be necessary to mitigate the Doppler shift variation.

Figure 6: Residual DL frequency offset after UE DL synchronization.

A noteworthy observation is that the Doppler shift appears to vary approximately linearly with time in each cell (more detailed plots can be found in Appendix A.3.5). A potential way to manage the Doppler drift induced by the satellite movement is that the network signals to the UE the Doppler rate to apply in a given cell (beam). After initial DL synchronization, the UE gradually reduces its RX frequency by the given rate.

The residual frequency offset after this tracking is shown in Figure 7. Clearly, the linear approximation is sufficient in this scenario. The maximum remaining frequency offset is in the range of 34 Hz.

Figure 7: Residual DL frequency offset after UE DL synchronization with linear frequency drift compensation.

The insights from the simulation results (including those in Appendix A.3) are summarized as follows.

Continuous frequency tracking will be necessary to mitigate the Doppler shift variation in LEO NTN.

Doppler shift appears to vary approximately linearly with time in each cell.

The trends observed for S-band carry over to Ka-band.

The linear approximation of Doppler shift (as a function of time) becomes more accurate as cell size and/or constellation size is reduced.

In the preceding discussion, the UE was assumed to continuously update its local RX frequency to compensate for the Doppler drift. It is of interest to know how frequent this compensation should be done, which depends on the absolute Doppler rate. The Doppler rate for the baseline scenarios is summarized in Table 1.

Table 1 Doppler variation rate.

| Band | S-band |  | Ka-band |  |
| --- | --- | --- | --- | --- |
| Satellite altitude | 600 km | 1200 km | 600 km | 1200 km |
| Maximum Doppler rate in any beam | 580 Hz/s | 250 Hz/s | 5.8 kHz/s | 2.4 kHz/s |

Based on pre-calculated average (or beam center) Doppler rate per beam and linear post-compensation in the UE, the residual Doppler shift of a LEO satellite can be kept at manageable levels under typical conditions. The applicable Doppler rate can be signaled to the UE. The information is not only useful for UE frequency tracking but also for UE timing tracking.

At least LEO NTN should support network to signal Doppler rate to assist with UE frequency and time tracking.

## 3.3	Considerations on UL frequency compensation

So far, only the downlink was considered. However, the observation that the frequency drift is approximately linear (at least for small to medium cell sizes) holds also for uplink.

A UE in connected mode will access a new cell as it enters it. If closed-loop frequency compensation is used during the random access procedure, the network will estimate the frequency offset of the UL signal based on the transmission on PRACH. The frequency offset will be the sum of the DL Doppler shift and UL Doppler shift. The network communicates the frequency offset to the UE that adjusts its UL TX frequency by that amount in subsequent transmissions.

As the beam continues to move, the UL Doppler shift will decrease approximately linearly, just as the DL Doppler shift. It should therefore be considered to apply a linear compensation also to the UL frequency. The rate of this compensation can be derived from the DL Doppler rate signaled by the network.

If a Doppler rate is signaled to the UE according to Proposal 2, it can be used by the UE also for linear Doppler frequency pre-compensation of the UL signal.

# NTN random access

In the last RAN1 meeting, the need of a new PRACH format was discussed and companies are encouraged to provide evaluations to justify their proposed PRACH design. In this section, we provide our views on the need of a new PRACH format, design considerations of a new PRACH format, and preliminary evaluation results on a candidate new PRACH format.

## 4.1	On the need of a new PRACH format

The existing NR PRACH formats design utilize Zadoff-Chu (ZC) sequences and are illustrated in Figure 8. The formats have been designed to facilitate UL timing estimation, with the assumption that the UL frequency offset between the transmitter (UE) and the receiver (gNB) is small after DL synchronization. The assumption of small UL frequency offset between UE and gNB in Msg1 is valid in a terrestrial network, where the gNB is stationary and the frequency offset is mainly determined by the UE’s DL frequency synthesizer error plus the Doppler frequency due to UE movement.

Figure 8: NR PRACH formats

In a LEO NTN, the assumption of small UL frequency offset between UE and gNB in Msg1 is no longer valid due to the fast movement of a LEO satellite that results in high Doppler shift. Indeed, RAN1 now well recognizes the large frequency offset between UE and gNB in the UL of a LEO NTN and has made the following agreement to mitigate the UL frequency misalignment issue.

Agreement:

For UL frequency compensation at least in LEO systems, parameter(s) for frequency correction can be indicated by gNB to UE

- FFS: Signaling details including whether signalling is broadcast or UE specific, and which parameter(s) are signalled.

Specifically, according to TR 38.821, NTN should address

- Max differential delay: 10.3 ms for GEO and 3.18 ms for LEO

- Max Doppler shift: 0.93 ppm for GEO and 24 ppm for LEO (note that these values need to be updated as now the max differential delay is significantly increased due to the increased beam size)

- Max residual Doppler shift: n/a for GEO and 1.91 ppm for LEO (note that these values need to be updated as now the max differential delay is significantly increased due to the increased beam size)

RAN1 to update the applicable max Doppler shift value and max residual Doppler shift value in NTN based on the updated assumptions on beam/cell size (3500 km for GEO and 1000 km for LEO).

For GEO, the large max differential delay cannot be accommodated by even the longest CP in the existing PRACH formats. Note that extending the CP length beyond an OFDM symbol duration is equivalent to increasing the number of repetitions. Having a PRACH format of similar length as the max differential delay of 10.3 ms for GEO is highly inefficient in terms of radio resource usage. A more appropriate approach would be to use implementation-based techniques to deal with the large max differential delay in GEO (with small Doppler). For example, the gNB can have a large PRACH receiving window and use multiple timing hypotheses to detect random-access preamble and estimate TA accordingly. Figure 9 gives an illustration of this method: the receiver can detect the preamble with timing hypothesis 4 and estimate x by processing the received signal, leading to a time-of-arrival estimate equal to 3*(0.8+0.8)+(0.8-x) ms. Note that in Figure 9 we choose the smallest SCS to get the longest symbol duration in PRACH. Using a PRACH format with a larger SCS may be more robust to Doppler, but the number of hypotheses required would scale up with the SCS, leading to increased receiver complexity. The performance of such method deserves further study in RAN1.

Figure 9: PRACH receiver processing for GEO (max differential delay up to 10.3 ms)

Extending the CP length in a PRACH format beyond an OFDM symbol duration is equivalent to increasing the number of repetitions in the PRACH format.

RAN1 to study how to detect PRACH preamble for GEO NTN with large max differential delay (up to 10.3 ms) and identify which Rel-15 PRACH formats (defined by CP, subcarrier spacing, length, etc.) are suitable for use in S-band GEO and Ka-band GEO, respectively.

For LEO, the large max differential delay of 3.18 ms cannot be accommodated by even the longest CP in the existing PRACH formats. Implementation-based techniques using multiple timing hypotheses can be used, similar to the GEO case. However, for LEO, the PRACH receiver needs to handle a large Doppler shift in addition to a large differential delay.

As noted earlier, the applicable max Doppler shift value and max residual Doppler shift value in NTN based on the updated assumptions on beam/cell size (3500 km for GEO and 1000 km for LEO) should be updated accordingly. In the sequel, we use the current (outdated) values for the purpose of discussion.

In the UL, the Doppler shift is about doubled in Msg1, resulting in 48 ppm Doppler. The 48 ppm equals 96 kHz in S band (2 GHz) and 1440 kHz in Ka band (30 GHz). This magnitude of Doppler and the large timing uncertainty make it difficult to detect existing PRACH formats and estimate timing accordingly.

- Long sequence with 1.25 kHz (resp. 5 kHz) subcarrier spacing together with restricted set B can handle Doppler up to 2.5 kHz (resp. 10 kHz), which is much less than 96 kHz in S band and 1440 kHz in Ka band.

- Short sequence with {15 kHz, 30 kHz} in FR1 and {60 kHz, 120 kHz} in FR2 does not have restricted set defined. Even with restricted set, the level of Doppler that can be handled is much less than 96 kHz in S band and 1440 kHz Doppler in Ka band.

Since the magnitude of absolute Doppler shift is too large, it is reasonable to assume that the gNB would apply post-compensation such that the PRACH receiver only needs to handle max residual Doppler shift, which is 3.82 ppm.  The 3.82 ppm equals 7.64 kHz in S band (2 GHz) and 114.6 kHz in Ka band (30 GHz). Note that 2 GHz and 30 GHz are chosen for reference. If NTN needs to support higher frequency in FR1 (410 MHz to 7.125 GHz) and FR2 (up to 52.6 GHz), the 3.82 ppm would imply larger frequency error in higher frequency (e.g. 27.2 kHz at 7.125 GHz and 200.9 kHz at 52.6 GHz). The bottom line is that the magnitude of max residual Doppler, even the current (outdated) value, seems challenging to be handled by the existing PRACH formats. The large timing uncertainty compounds the challenge further.

Thus, a suitable PRACH format for a LEO NTN should facilitate UL timing estimation in the presence of large UL frequency offset between UE and gNB. It would be even more desirable that the PRACH format could facilitate both UL timing estimation and UL frequency estimation. This would allow gNB to send a frequency correction command besides timing advance command in Msg2, which could help to establish the uplink orthogonality before UEs transmit Msg3. Otherwise, uplink orthogonality would be lost in Msg3 transmissions, causing inter-subcarrier interference to other transmissions (not just Msg3) overlapping in time.

The existing NR PRACH formats based on ZC sequences have been designed to facilitate UL timing estimation, with the assumption that the UL frequency offset between UE and gNB is small after DL synchronization. The assumption of small UL frequency offset is not valid in LEO NTN.

At least for LEO NTN, a suitable PRACH format should be designed to facilitate both UL timing estimation and UL frequency estimation.

## 4.2	A new PRACH format for UL time and frequency alignment

To design a suitable PRACH format for both UL timing estimation and UL frequency estimation, it is imperative to first understand why the existing NR PRACH formats based on ZC sequences cannot meet the target.

It is well known that there are several peaks in the ambiguity function of ZC sequences in the Delay-Doppler plane, leading to many timing and Doppler ambiguities [4]. Due to the nature of ZC sequences, both delay and frequency shift cause cyclic shift in the observation window of received ZC sequences at the gNB. As a result, two issues may arise.

- It is difficult if not impossible to separate the two effects (delay and frequency shifts) by observing the composite cyclic shift. Separating them in order to estimate delay and/or frequency shift is needed. This issue exists, even if cyclic shifted ZC sequences with the same root are not used.

- If cyclic shifted ZC sequences are used, the composite shift may make sequence A become sequence B, leading to misdetection. This issue has resulted in the introduction of restricted sets in PRACH formats.

We give a concrete example, illustrating the timing and Doppler ambiguities in PRACH. Assume zero delay and 1.25 kHz frequency offset between transmitter and receiver. The receiver aims to estimate delay and frequency offset by cross correlating the received signal with its reference copy of the transmitted signal. The correlation is performed at multiple hypotheses of frequency offsets that are on the step size of 1.25 kHz. The sampling rate is 30.72 MHz. The cross correlation results are plotted in Figure 10 and Figure 11 for ZC sequences with roots 56 and 714, respectively. The correlation values in each figure are normalized by the maximum correlation value, yielding a maximum value of 0 dB in each figure. It is clear that in either Figure 10 or Figure 11 multiple correlation peaks of the same height are observed. This implies that it is impossible to separate the effects of delay and frequency offset in PRACH in the presence of both large timing and frequency uncertainties, leading to difficulties in timing estimate at the gNB and misdetection of random access preambles.

Figure 10: Time-frequency uncertainty: ZC with root 56.

Figure 11: Time-frequency uncertainty: ZC with root 714.

We can understand the timing and frequency offset ambiguities of ZC sequences by examining their theoretical properties. To this end, we introduce the following notation.

- NZC: the length of a ZC sequence

- u: the root of a ZC sequence, and 0<u<NZC

- p: the inverse modulo NZC of u, i.e., p*u mod NZC=1, and 0<p<NZC

- fSC: the subcarrier spacing of an OFDM signal

- Δf: the frequency offset between transmitted and received signals

- n0: the delay of received signals relative to the transmitted signal

Let us consider the following form of ZC sequences:

xun=exp-jπunn+1NZC, n=0, 1,…, NZC-1.

If NZC is prime, each u is associated with a unique inverse modulo NZC. It can be shown that if k=Δf/fSC (and for simplicity k is assumed to be an integer), the peak of cross correlation of the transmitted and received signals is located at the position of n0+kp mod NZC. Clearly, both delay and frequency shift cause cyclic shift in the received ZC sequences, resulting in a composite cyclic shift from which the effect of delay cannot be separated from the effect of frequency shift.

The above analysis also sheds light on how to design a PRACH format to resolve the timing and frequency offset ambiguities. Intuitively, two equations can be used to solve for two unknowns (delay and frequency offset). In particular, if a transmitter sends two signals based on two ZC sequences (that have different properties), the receiver can resolve the timing and frequency offset ambiguities by processing the two received signals.

For example, for 2 ZC sequences with roots u and -u respectively, the peak of cross correlation of the transmitted and received signals locate at two positions:

- Position 1: s1=n0+kp mod NZC

- Position 2: s2=n0-kp mod NZC

In this case, the second ZC sequence can be treated as the complex conjugate of the first ZC sequence, as shown in Figure 12. With two equations, the delay n0 can be estimated as

n0=s1+s2 mod NZC2

Once the delay is estimated, the frequency offset can then be readily estimated.

Figure 12: PRACH format with 2 ZC sequences, where the second ZC sequence is the complex conjugate of the first ZC sequence.

Note that the PRACH format illustrated in Figure 12 has minimal specification impact. The transmission of the first ZC sequence follows an existing NR PRACH format, and the change would be merely to request one additional transmission of a second ZC sequence that is the complex conjugate of the first ZC sequence.

Two different ZC sequences are needed for UL timing estimation and UL frequency estimation in LEO NTN.

A suitable PRACH format for LEO NTN should be composed of two parts: the first part is a ZC sequence transmitted using an existing NR PRACH format, and the second part is a ZC sequence that is the complex conjugate of the first ZC sequence.

## 4.3	Evaluation results on the proposed new PRACH format

TBA when available.

# NTN timing advance and frequency correction

## 5.1	Timing advance

In RAN1#98, the following agreement about timing advance was made.

Agreement:

Following options can be considered to support TA adjustment for UL transmission:

- Option 1

  - Autonomous acquisition of the TA at UE with known location and satellite ephemeris:

    - FFS: how to compensate the TA, e.g., full TA or only UE-specific differential TA

      - Note: If only UE-specific differential TA is compensated, timing offset between gNB DL and UL frame should be managed by network and acquisition of common TA is needed.

    - FFS: additional TA signalling from BS considering the potential inaccuracy.

- Option 2

  - Indication of common TA to all users within the coverage of the same beam with broadcasting as a baseline for signalling, e.g., via SIB/MIB

    - FFS: additional UE-specific differential TA signalling from BS.

    - FFS: the reference point(s)  for common TA calculation

- Additional enhancements to existing TA signaling in Rel-15 can be considered for TA maintenance

  - Parameters indicated by gNB to enable the TA adjustment

  - Cell/UE-group specific signalling

We note that the two options are not mutually exclusive.

If an NTN network needs to serve UEs with different capabilities, both Option 1 and Option 2 will be needed.

For Option 1, as the UE already knows the UE location and satellite ephemeris, it can autonomously estimate the TA before MSG1 transmission. Then, we need to decide whether the UE applies full TA before MSG1 transmission or only the UE-specific differential TA.

If the UE of Option 1 applies full-TA before MSG1 transmission, the network needs to only deal with the residual timing error.

When full-TA is applied, the residual timing error is small enough to be estimated by the PRACH receiver as done in terrestrial cellular networks. The network can then send any further timing corrections to the UE in MSG2 which the UE will apply for MSG3 transmission.

If the UE of Option 1 applies UE-specific differential TA before MSG1 transmission, the network needs to deal with a frame timing offset as large as the common delay in the cell/beam.

We note that the common delay in the beam/cell varies with time.

If the UE applies UE-specific differential TA, the network needs to cope with a time-varying frame timing offset.

Our view is that Option 2 will be needed for UEs without UE location information or satellite ephemeris. In this case, the UE will have to acquire common TA from system information broadcast by the network. The UE will apply common TA before MSG1 transmission. This means that the network will have to cope with the differential timing offset as large as the maximum differential delay within the cell/beam. In MSG2, the network will send the UE-specific differential TA that the UE will apply for MSG3 transmission.

If the UE of Option 1 applies UE-specific differential TA and that of Option 2 applies common TA before MSG1 transmission, the network needs to cope with drastically different frame timing offsets for the two cases.

To simplify and synergize the TA design for both cases, we support keeping the difference in the frame timing offsets for the two cases as small as possible.

The UE in Option 1 applies full-TA before MSG1 transmission and the UE in Option 2 applies the signaled common-TA before MSG1 transmission.

NTN system will require the network to broadcast the propagation delay for a reference point (common delay) in the cell/beam. Due to satellite motion, the common delay will vary with time as well as the reference location. The choice of reference point is to a large extent up to network configuration.

The reference point for common TA calculation is up to network configuration.

Due to high speed satellite motion, the propagation delay between the UE and gNB is constantly changing. When the propagation delay is large, the TA command sent by the gNB could be stale by the time the UE receives it. For example, with a maximum timing drift of 40 μs/s and a propagation delay of 15 ms, the timing indicated by the TA command can be off by 0.6 μs. Note that this exceeds the CP durations for 120 kHz SCS. A possible solution is that the TA command sent by the gNB at time t conveys the TA value that should be used at time t+tdelay where tdelay is the expected delay before the message reaches the UE.

The network can leverage the timing drift information to send an adjusted TA value that accounts for the expected TA inaccuracy.

The network sends timing advance commands to a UE in connected mode to maintain uplink timing. There are 6 bits in the MAC-CE for indicating an index TA where TA∈{0,1,…,63} to the UE. The UE calculates the new TA value NTA_new using NTA_new=NTA_old+(TA-31)×16×64× 2-μ (in units of Tc where Tc=0.509 ns). This allows a maximum change in the TA value of 32×16×64× 2-μ Tc which is shown in Table 2. To cope with a large timing drift of 40 μs/s, several such commands per second are required.

Table 2 Maximum change in TA value in connected mode for various SCS.

| SCS | 15 kHz | 30 kHz | 60 kHz | 120 kHz |
| --- | --- | --- | --- | --- |
| Max. change in TA per MAC-CE | 16.67 μs | 8.33 μs | 4.16 μs | - μs |

Due to the aforementioned reasons, there is a need to empower the UE to adjust its uplink timing amid large propagation delays and timing drift. It has been argued that different UEs in a cell typically experience similar timing drift, which motivates broadcasting drift information for the UEs. We would like to point out that this may not always be an efficient approach. For example, in certain LEO scenarios, different UEs may experience different timing drifts at a given time. Therefore, the signaling should be UE specific.

MAC CE command for timing advance may come with excessive overhead to cope with the large timing drift in LEO NTN. One alternative would be to transmit timing advance command directly in DCI, e.g. a configurable field in non-fallback DCI format 0_1/1_1 or a group common DCI specifically designed for timing advance command (similar to DCI format 2_2/2_3 designed for the transmissions of TPC commands).

Consider using DCI to transmit timing advance command to cope with the large timing drift in LEO NTN.

## 5.2	UL frequency correction

RAN1 agreed that for UL frequency compensation at least in LEO systems, parameter(s) for frequency correction can be indicated by gNB to UE. In this section, we provide our views on the signaling details.

Similar to timing advance, the purpose of UL frequency correction is to establish uplink orthogonality for OFDMA based NR.

- The transmissions from different UEs in a cell are time-aligned at the gNB to maintain uplink orthogonality. Time alignment is achieved by using different timing advance values at different UEs to compensate for their different propagation delays.

- Similarly, for UL frequency correction, the transmissions from different UEs in a cell should be frequency-aligned at the gNB to maintain uplink orthogonality. Frequency alignment is achieved by using different frequency correction values at different UEs to compensate for their different frequency offset values.

NR supports timing advance methods, including timing advance command in MSG2 and MAC CE based timing adjustment command. Due to the similar purpose of UL frequency correction, it is natural that similar mechanisms should be supported for UL frequency correction, i.e., frequency correction command in MSG2 and MAC CE based frequency correction command. Due to the fast varying frequency offset, it might be beneficial to support DCI based frequency correction command as well, which deserves further study.

UL frequency correction command can be transmitted in MSG2 and MAC CE. FFS DCI based frequency correction command.

# 6.	Conclusions

In the previous sections, we discuss NTN synchronization, random access and timing advance related issues. We made the following observations:

Observation 1	Robust initial DL synchronization performance can be achieved without DL pre-compensation but with a significant increase in receiver complexity.

Observation 2	Continuous frequency tracking will be necessary to mitigate the Doppler shift variation in LEO NTN.

Observation 3	Doppler shift appears to vary approximately linearly with time in each cell.

Observation 4	The trends observed for S-band carry over to Ka-band.

Observation 5	The linear approximation of Doppler shift (as a function of time) becomes more accurate as cell size and/or constellation size is reduced.

Observation 6	If a Doppler rate is signaled to the UE according to Proposal 2, it can be used by the UE also for linear Doppler frequency pre-compensation of the UL signal.

Observation 7	Extending the CP length in a PRACH format beyond an OFDM symbol duration is equivalent to increasing the number of repetitions in the PRACH format.

Observation 8	The existing NR PRACH formats based on ZC sequences have been designed to facilitate UL timing estimation, with the assumption that the UL frequency offset between UE and gNB is small after DL synchronization. The assumption of small UL frequency offset is not valid in LEO NTN.

Observation 9	Two different ZC sequences are needed for UL timing estimation and UL frequency estimation in LEO NTN.

Observation 10	If an NTN network needs to serve UEs with different capabilities, both Option 1 and Option 2 will be needed.

Observation 11	If the UE of Option 1 applies full-TA before MSG1 transmission, the network needs to only deal with the residual timing error.

Observation 12	If the UE of Option 1 applies UE-specific differential TA before MSG1 transmission, the network needs to deal with a frame timing offset as large as the common delay in the cell/beam.

Observation 13	If the UE applies UE-specific differential TA, the network needs to cope with a time-varying frame timing offset.

Observation 14	If the UE of Option 1 applies UE-specific differential TA and that of Option 2 applies common TA before MSG1 transmission, the network needs to cope with drastically different frame timing offsets for the two cases.

Observation 15	The network can leverage the timing drift information to send an adjusted TA value that accounts for the expected TA inaccuracy.

Based on the discussion in the previous sections we propose the following:

Proposal 1	The benefits of pre-programming the UE, or its uSIM, with satellite ephemeris should be studied further by RAN1.

Proposal 2	At least LEO NTN should support network to signal Doppler rate to assist with UE frequency and time tracking.

Proposal 3	RAN1 to update the applicable max Doppler shift value and max residual Doppler shift value in NTN based on the updated assumptions on beam/cell size (3500 km for GEO and 1000 km for LEO).

Proposal 4	RAN1 to study how to detect PRACH preamble for GEO NTN with large max differential delay (up to 10.3 ms) and identify which Rel-15 PRACH formats (defined by CP, subcarrier spacing, length, etc.) are suitable for use in S-band GEO and Ka-band GEO, respectively.

Proposal 5	At least for LEO NTN, a suitable PRACH format should be designed to facilitate both UL timing estimation and UL frequency estimation.

Proposal 6	A suitable PRACH format for LEO NTN should be composed of two parts: the first part is a ZC sequence transmitted using an existing NR PRACH format, and the second part is a ZC sequence that is the complex conjugate of the first ZC sequence.

Proposal 7	The UE in Option 1 applies full-TA before MSG1 transmission and the UE in Option 2 applies the signaled common-TA before MSG1 transmission.

Proposal 8	The reference point for common TA calculation is up to network configuration.

Proposal 9	Consider using DCI to transmit timing advance command to cope with the large timing drift in LEO NTN.

Proposal 10	UL frequency correction command can be transmitted in MSG2 and MAC CE. FFS DCI based frequency correction command.

# References

3GPP TR 38.821, ”Solutions for NR to support non-terrestrial networks (NTN)”

3GPP TR 38.811, “Study on New Radio (NR) to support non terrestrial networks”

R1-1910980, On NTN link level and system level evaluations, Ericsson, RAN1#98bis, Chongqing, China, October 2019.

Y. S. Shinakov, "Ambiguity Functions of Zadoff-Chu Signals for 5-G Synchronization Systems," 2018 Systems of Signal Synchronization, Generating and Processing in Telecommunications (SYNCHROINFO), Minsk, 2018, pp. 1-7.

# Appendix

## A.1	RAN1 agreements

RAN1#97

Agreement:

Performance evaluations of the synchronization for DL are encouraged. For these evaluations,

- For LEO systems, beam specific pre-compensation of the common frequency shift at satellite with respect to the spot beam center can be considered.

Agreement:

For UL frequency compensation at least in LEO systems:

- Both open and closed-loop can be studied

  - Beam specific post-compensation of common frequency offset at gNB can be considered.

    - FFS: Further indication of common frequency offset

    - FFS: Signalling details

    - FFS: Compensation of common frequency offset at UE side

- For Open-loop method:

  - Estimation of UE-specific frequency offset and pre-compensation at UE side can be conducted based on:

    - DL RSs

    - UE location and satellite ephemeris

      - FFS: Determination of UE location

Agreement:

The scenarios where the Rel-15 PRACH design is sufficient and the scenarios where an extended or new PRACH design is required should be identified as part of the study.

RAN1#98

Agreement:

For DL initial synchronization in NTN,

- SSB design in Rel-15 can provide robust performance in the following cases

  - GEO

  - With pre-compensation for LEO

    - Note: The above observation can be revised if proved by other results

- FFS: Whether SSB design in Rel-15 can provide robust performance for LEO without Doppler pre-compensation

  - Factors that need to be considered include at least latency and complexity for SSB detection

Agreement:

Companies are encouraged to evaluate whether Rel-15 mechanisms are sufficient for time/frequency tracking

Agreement:

Companies are encouraged to provide the evaluations based on agreed assumptions for the following cases to justify their proposed PRACH design:

[Table omitted]

Agreement:

For UL frequency compensation at least in LEO systems, parameter(s) for frequency correction can be indicated by gNB to UE

- FFS: Signaling details including whether signalling is broadcast or UE specific, and which parameter(s) are signalled.

Agreement:

Following options can be considered to support TA adjustment for UL transmission:

- Option 1

  - Autonomous acquisition of the TA at UE with known location and satellite ephemeris:

    - FFS: how to compensate the TA, e.g., full TA or only UE-specific differential TA

      - Note: If only UE-specific differential TA is compensated, timing offset between gNB DL and UL frame should be managed by network and acquisition of common TA is needed.

    - FFS: additional TA signalling from BS considering the potential inaccuracy.

- Option 2

  - Indication of common TA to all users within the coverage of the same beam with broadcasting as a baseline for signalling, e.g., via SIB/MIB

    - FFS: additional UE-specific differential TA signalling from BS.

    - FFS: the reference point(s)  for common TA calculation

- Additional enhancements to existing TA signaling in Rel-15 can be considered for TA maintenance

  - Parameters indicated by gNB to enable the TA adjustment

  - Cell/UE-group specific signalling

## A.2	Link level simulation parameters for downlink synchronization simulations

Table 3: Simulation parameters for DL sync simulations.

| Scenario |  | LEO |  |
| --- | --- | --- | --- |
| Channel model |  | NTN TDL D, suburban, 30° elevation angle |  |
| Carrier frequency |  | 2 GHz |  |
| DL frequency offset | Satellite oscillator offset | 0.5 ppm |  |
|  | UE oscillator offset | 10 ppm |  |
|  | Worst-case Doppler shift | 24 ppm (without pre-compensation, 600 km altitude) | 1.91 ppm (with pre-compensation, Set 2, 1200 km altitude) |
|  | Total | 34.5 ppm (69 kHz) | 12.41 ppm (24.82 kHz) |
| Sub-carrier spacing |  | 15 kHz, 30 kHz |  |
| UE speed |  | 3 km/h |  |
| Antenna configuration |  | 1 Tx, 2 Rx |  |
| Phase noise model |  | none |  |

## A.3	Parameters and additional results for Doppler shift simulations

### A.3.1	Simulation Parameters

The parameters for the simulation results in Section 2.3 are tabulated below.

Table 4 Simulation parameters.

| Band | S-band | Ka-band |
| --- | --- | --- |
| DL carrier frequency | 2 GHz | 20 GHz |
| Beam layout | 91 cells in 6 tiers hexagonal grid in UV plane | 91 cells in 6 tiers hexagonal grid in UV plane |
| Adjacent beam spacing in UV plane | 0.0668 | 0.0267 |
| Satellite altitude | 600 km 1200 km | 600 km 1200 km |
| Elevation angle of center beam | 90° | 90° |
| Adjacent beam spacing on earth (center beam vs 1st tier) | 600 km altitude: 40.2 km 1200 km altitude: 80.4 km | 600 km altitude: 16.0 km 1200 km altitude: 32.1 km |
| Satellite orbit | Circular, inclination angle=0 | Circular, inclination angle=0 |
| UE velocity | Not modelled | Not modelled |
| Earth rotation | Not modelled | Not modelled |

### A.3.2	Simulation results: 1200 km altitude, S-band

Performing the same evaluation but with a satellite altitude of 1200 km yields the residual DL frequency offset with linear frequency drift compensation shown in Figure 13. The maximum remaining frequency offset is 37 Hz.

Figure 13: Residual DL frequency offset after UE DL synchronization with linear frequency drift compensation.

### A.3.3	Simulation results: Ka-band

In the Ka-band, the maximum residual frequency offset with linear frequency drift compensation is 18 Hz and 21 Hz at altitude 600 km and 1200 km, respectively. The smaller offset is due to the smaller cell size.

### A.3.4	Simulation results: Larger constellations, larger beam widths

To determine the validity of the linear approximation, additional scenarios have been evaluated.

With more beams in the antenna constellation, higher absolute Doppler shifts and large cells will be experienced at the outer edge of the coverage area. For example, if the number of tiers of cell in the evaluation area is increased to 10, the maximum residual Doppler will be as shown in Table 5.

If fewer but larger cells are used to cover the satellite coverage area, the Doppler shift differences in a cell will be larger and a linear approximation therefore less accurate. This is confirmed in the examples in Table 6, where the beam width (defining the ABS in the UV plane) is doubled compared to the previous examples.

Table 5 Larger constellations.

| Band | S-band |  | Ka-band |  |
| --- | --- | --- | --- | --- |
| Beam layout | 271 cells in 10 tiers |  | 271 cells in 10 tiers |  |
| Adjacent beam spacing in UV plane | 0.0668 |  | 0.0267 |  |
| Satellite altitude | 600 km | 1200 km | 600 km | 1200 km |
| Maximum residual Doppler shift with linear frequency drift compensation | 90 Hz | 96 Hz | 35 Hz | 39 Hz |

Table 6 Doubled beam widths.

| Band | S-band |  | Ka-band |  |
| --- | --- | --- | --- | --- |
| Beam layout | 37 cells in 4 tiers |  | 37 cells in 4 tiers |  |
| Adjacent beam spacing in UV plane | 0.1335 |  | 0.0534 |  |
| Satellite altitude | 600 km | 1200 km | 600 km | 1200 km |
| Maximum residual Doppler shift with linear frequency drift compensation | 191 Hz | 213 Hz | 94 Hz | 101 Hz |

### A.3.5	Detailed view of residual DL frequency offset after UE DL synchronization

Figure 14 zooms in on one cell in Figure 6. Figure 15 shows the same cell but from a different angle. The arrows in Figure 15 indicate the movement of the UEs relative to the cell. The DL frequency offset appears to be approximately linear. Figure 16 shows that the Doppler rate in the same cell is approximately constant, which confirms the observation in Figure 14 and Figure 15.

Figure 14: Residual DL frequency offset after UE DL synchronization.

Figure 15: Residual DL frequency offset after UE DL synchronization, viewed from above.

Figure 16: Doppler rate.

Modelled as a fixed frequency offset in the received signal.