3GPP TSG RAN WG1 #98bis			R1-1911166

Chongqing, China, 14th – 18th Oct. 2019

Source:	NTT DOCOMO, INC.

Title:	Mechanisms for resource multiplexing among backhaul and access links

Agenda Item:	7.2.3.2

Document for: 	Discussion and Decision

- Introduction

At the RAN1#98 meeting, the mechanisms for resource multiplexing among backhaul and access links were discussed and following agreements were made [1].

Agreement: - In Rel-16 for IAB backhaul, only existing slot pattern durations are supported. Agreements: Slot formats with the sequence order UL-Flexible-DL are supported for IAB-node DU resource configurations (F1-AP) and IAB-node MT resource configurations provided in dedicated RRC signaling (e.g. enhancements to the existing TDD-UL-DL-SlotConfig in TDD-UL-DL-ConfigDedicated – details up to RAN2). - FFS: Support of dynamic indication of UL-Flexible-DL Agreements: The H/S/NA attributes for the per-cell DU resource configuration should take into account the associated MT carrier frequency(ies). - Note: RAN1 assumes that this is mainly needed for intra-band cases Agreements: To handle potential misalignment in time of the configured DU and MT resources when determining the validity of H/S/NA at the DU, the following is supported: - H/S/NA is applied/determined relative to the DU resource configuration (D/U/F) slot timing without considering the MT resource configuration or timing - The parent DU should know the information about the misalignment between the MT and DU DL Tx/Rx / UL Tx/Rx timing(s) in order to avoid resource type conflicts - FFS whether any explicit signalling is necessary or not - It is up to the parent IAB node implementation and network configuration to handle any potential resource type conflicts (e.g. MT scheduling restrictions or configuration of guard symbols). The child IAB node should handle any remaining conflicts using the existing conflict resolution rules Agreements: For the explicit indication of DU-IA: - At least DU-IA can be indicated independently of the MT resource configuration: - FFS the DCI indicates DU-IA per-resource type (D/U/F), slot, etc. - FFS: Whether this indication is via a new DCI format or DCI Format 2_0 using existing and/or reserved entries of Table 11.1.1-1 in 38.213 - FFS: Whether for the purpose of signaling optimization a DU/MT resource type can additionally be jointly indicated with DU-IA which overrides the flexible resources in the semi-static DU/MT resource configuration

In this contribution, we discuss further on the mechanisms for resource multiplexing among backhaul and access links.

- Discussion on mechanisms for resource multiplexing among backhaul and access links

2.1 Necessary semi-static DU resource configuration for parent IAB-node

At the RAN1#96bis meeting, it was agreed that an IAB node has the ability to be made aware of the semi-static DU resource configuration (D/U/F/H/S/NA) of all its child IAB nodes, and in the case the full DU resource configuration information is not necessary, only the necessary configuration information is signaled to the IAB node.

Figure 1 describes the resource allocation for radio links (Link_parent or Link_child), according to the agreement in the RAN1 #96bis meeting [2], with different DU resource configuration for an IAB node. First of all, the parent node should recognize the soft DU resource of the IAB node, since it indicates availability of the soft DU resource. Additionally, the parent node should recognize the hard DU resource of the IAB node, since the Link_parent is not available for the IAB node in the case of the hard DU resource configuration, except that cell-specific signal/channel for the IAB node MT is configured. It may implicitly recognize the NA resource of the IAB node, when the soft and hard DU resource is informed to the parent IAB node.

Considering the DU resource configuration of (D/U/F), if an IAB node has capability of receiving signals from a child node and a parent node simultaneously or transmitting signals for a child node and a parent node simultaneously, the information on D/U/F for DU resource may be beneficial for the scheduling at the parent node. However, the reception or transmission timing between DU and MT within an IAB node would be different due to the timing advance (TA), and hence benefit of resource configuration information of (D/U/F) would be limited.

Figure 1: MT and DU resource management with different DU resource configurations for IAB node

Based on the above discussion, we have a following proposal.

Proposal 1: Parent IAB-node has the ability to be made aware of the resource type (H/S/NA) of its child IAB node.

At the RAN1 #97 meeting, it was agreed that if cell-specific signal/channel is configured for an NA or a soft not indicated available DU resource, the resource is treated as if it were a hard DU resource. On the other hands, if the resource is treated as a hard DU resource, IAB node MT will not be available for transmitting/receiving signal on Link_parent although a parent node DU would schedule a resource to the IAB node as shown in Fig. 2 (marked with a blue square). In order to avoid unnecessary resource allocation from parent node to the IAB node, a parent node should know the hard DU resource treated by IAB node as in the semi-static DU resource configuration (H/S/NA).

Figure 2 shows an example of resource types of an IAB node with a semi-static DU resource indication and treated as a hard DU resource. As shown in Fig.2, periodicity of the semi-static DU resource indication is relatively short span, e.g. every slots/subframes/frames, and the same indication may be applied repeatedly. On the other hands, periodicity of the hard DU resource is relatively long span, e.g. every 160msec/320msec/640msec. Therefore, indications of the semi-static DU resource and the hard DU resource should be separated, e.g. the semi-static DU resource is indicated every slots/subrames/frames with applying the repetition, and the hard DU resource is indicated with a hard resource pattern within a slot/subframe/frame, periodicity, and offset.

Figure 2: Example of resource types of IAB node with semi-static DU resource indication and treated as DU H resource.

Proposal 2: Parent IAB-node has the ability to be aware of the hard DU resource treated by its child IAB-node.

Proposal 3: Indications of the semi-static DU resource and the hard DU resource from IAB node to its parent node should be separated.

2.2	Semi-static resource configuration for DU

At the last RAN1 meeting, it was agreed that semi-static configuration of H/S/NA is applied/determined relative to the DU resource configuration (D/U/F) slot timing without considering the MT resource configuration or timing. As shown in Figure 3, there exists 4 cases for potential resource type conflict due to timing misalignment between IAB node MT and DU. Since the gap between DL and UL symbols is already considered in Rel-15, in this contribution, we’d like to mainly discuss the potential resource conflict in Case1 and Case2.

- Case1: MT DL Rx -> DU DL Tx

- Case2: DU UL Rx -> MT UL Tx

- Case3: MT DL Rx -> DU UL Rx

- Case4: DU DL Tx -> MT UL Tx

Figure 3: Examples of potential conflict due to timing misalignment between MT and DU

In order to avoid the conflict in Case1 and Case2, an issue is that whether an explicit signaling to parent node is necessary or not to indicate the information of timing misalignment between IAB node MT and DU so that the conflict can be handled by parent node.

In Case1, as shown in Fig.3, if the resource type of slot n is indicated as MT DL and DU NA while resource type of slot n+1 is indicated as DU hard DL, there is resource overlapping between symbol#13 of slot n and symbol#0 of slot n+1. The overlapping time in this case is the propagation delay between the parent node and the IAB node, which can be known by the parent node. Thus, in this case, no explicit signaling to parent node is needed. As analyzed in the above section, parent node should be aware of DU hard resources and DU treated as hard resources. With the information of propagation delay and the information of DU hard and treated as hard resources, parent node DU can avoid scheduling IAB node MT on the resources overlapped with DU hard or DU treated as hard resources.

In Case2, e.g., if the resource type of slot n is indicated as DU hard UL while resource type of slot n+1 is indicated as MT UL and DU NA, the overlapping time between DU UL and MT UL is Tp+Tg1-Tg2, where Tp is the propagation delay between parent node and IAB node, Tg1 is the time interval between DU UL Rx and DL Tx at parent node and Tg2 is the time interval between DU UL Rx and DL Tx at IAB node. To handle potential conflict in this case, following alternatives can be considered.

- Alt.1 The conflict is handled by parent node

  - Alt.1-1 The conflict is handled by a parent node with the information of Tp, Tg1 and Tg2.

  - Alt.1-2 The conflict is handled by a parent node with only the information of Tp and Tg1.

- Alt.2 The conflict is handled by child node

In Alt.1-1, Tg2 needs to be indicated to the parent node so that the parent node can handle the conflict by avoiding scheduling MT on the overlapping resources. However, indicating Tg2 to the parent node will introduce new signaling design and additional signaling overhead. In Alt.1-2 the conflict can be handled by the parent node with only the information of propagation delay and  Tg1, which is already known by parent node without additional signaling. In this alternative, e.g., parent node can avoid scheduling MT in the time interval of Tp+Tg1 which is long enough to handle the conflict. However, it may lead to lower resource utilization. Another alternative Alt.2 is to let the IAB node handle the potential conflict, i.e. whether the overlapping resources are used by MT or DU is decided by the IAB node as the same procedure of hard resources conflict. Since the timing misalignment is known by an IAB node, the IAB node DU can avoid scheduling its child link on the overlapping resources if the IAB node MT is scheduled on the resources. Therefore, in this case, considering signaling overhead and specification impact, the potential conflict can be handled by a child node and no additional signaling to a parent node is needed.

Proposal 4: Explicit signalling for information of timing misalignment between IAB node MT and IAB node DU to parent node is not necessary.

Proposal 5: For the case of MT DL Rx switching to DU DL Tx, the potential resource type conflict is handled by parent node with the information of propagation delay and the information of DU hard or DU treated as hard resources.

Proposal 6: For the case of DU UL Rx switching to MT UL Tx, whether the overlapping resource is used by MT or DU is decided by IAB node.

2.3	Dynamic indication of the availability of soft resources

2.3.1	Dynamic resource indication for IAB node

For the dynamic resource configuration for an IAB node, it was agreed in last meeting that at least DU-IA can be indicated independently of MT resource type, and for signaling optimization, joint indication of DU-IA with MT resource type or DU resource type were considered. However, necessity of following three indications needs to be considered.

- DU-IA indication for DU soft resource

- D/U/F indication for MT flexible resource

- D/U/F indication for DU flexible resource

Firstly, the availability of DU soft resource should be indicated by a parent node. Secondly, a D/U/F indication for MT flexible resource should also be indicated as for Rel-15 UEs. Finally, considering a D/U/F indication for a DU flexible resource, it can be decided by IAB node DU scheduler according to child link traffic. In addition, centralized management of resources may cause additional overhead and delay, so that a dynamic indication of D/U/F for a DU flexible resource from a parent node may not be necessary. Therefore, we conclude following proposal.

Proposal 7: DU-IA and MT-D/U/F should be dynamically indicated to align with Rel-15 operation and to save signalling overhead.

2.3.2	Signaling design for dynamic indication

In this section we discuss the details for signaling design for the dynamic indication for following information.

- DU-IA indication for DU soft resource

- D/U/F indication for MT flexible resource

Considering dynamic indication of DU-IA, an issue is that whether DU-IA is indicated per slot or per resource type or per symbol. Considering the flexibility of resource sharing between backhaul and access link, soft resource availability indication in symbol level should be supported. For example, as shown in Figure 4, there are several DU soft UL symbols in a slot overlapped with MT flexible and MT UL symbols. A possible case is that the parent node may indicate the DU soft resources overlapped with MT flexible resources as DU-IA, while indicate the DU soft resources overlapped with MT UL resources as DU-INA. Thus, it is necessary to support symbol-level indication of DU-IA for flexible resource multiplexing between backhaul and access.

Proposal 8: Support dynamic indication of DU soft resource availability in symbol level.

Figure 4: Example of symbol-level soft resource availability indication

To support the dynamic indication of DU-IA and MT resource type, both joint indication of two information and separate indication of each information can be considered as possible mechanisms. Alt. 1 of SFI-like indication via DCI format 2_0 can be regarded as a joint indication method. For Alt. 1, there are two sub-options as follows.

- Alt.1: Joint indication of DU-IA and MT resource type: SFI-like indication via DCI Format 2_0

  - Alt.1-1: reuse DCI format 2-0 and use reserved entries (56-254) in current slot format table to indicate the availability of DU soft resource together with link direction for MT resource

  - Alt.1-2: reuse DCI format 2-0 and enhance slotFormats in slotFormatCombination with 2-bit per symbol to directly indicate one out of MT-D/MT-U/MT-F/DU-A

- Alt.2: Separate indication of DU-IA and MT resource type:

  - DCI Format 2_0 for MT link direction and new DCI format for availability of DU soft resource

[Signal design for Alt.1-1: Joint indication of DU-IA and MT resource type (SFI-like indication)]

In Rel-15, slot format is indicated by DCI format 2-0 and the indices from 56 to 254 are reserved. Hence, a possible mechanism of Alt.1, i.e., Alt.1-1, is to reuse DCI format 2_0 and to use those reserved indices to indicate new slot formats including MT-D/MT-U/MT-F/DU-A as shown in Table 1, where MT-D/U/F indicates link direction for MT while DU-A indicates a symbol that will not be used by MT. However, due to limited number of reserved indices, the design of new slot formats should be carefully studied/selected. For example, current existing slot formats should be taken as baseline, and new entries can be added by substituting some symbols with the new type ‘DU-A’. In addition, due to limited number of reserved entries, supporting only some fixed pattern of DU available resources can be considered, e.g. fixed number or fixed location of ‘DU-A’.

Table 1 Example of slot formats for dynamic indication for IAB node

[Signal design for Alt.1-2: Joint indication of DU-IA and MT resource type (enhancement of slotFormats)]

The other possible mechanism of Alt.1-2, is to use 2-bit to directly indicate one out of MT-D/MT-U/MT-F/DU-A for a symbol for slotFormats in slotFormatCombination as shown in Fig.5. Instead of indicating entry index in slot format table, flexible slot format indication can be considered. In this Alt.1-2, higher flexibility can be achieved at the cost of larger RRC signaling overhead. To reduce the overhead, symbol group can be configured additionally and the symbols within a symbol group could share the same configuration of MT-D/MT-U/MT-F/DU-A. In addition, instead of indicating one out of MT-D/MT-U/MT-F/DU-A for all the symbols in a slot using slotFormats, indication only for symbols of MT flexible resource(s) and MT resource(s) overlapped with DU soft resource(s) in a slot using slotFormats can also be considered.

Figure 5: Example of dynamic indication for MT link direction and availability for DU soft resource

[Signal design for Alt.2: Separate indication of DU-IA and MT resource type]

The above mechanisms jointly indicate link direction for MT resource and availability of DU soft resource. To support independent indication of availability for DU soft resource and link direction for MT resource, indication of link direction for MT resource can follow Rel-15 mechanism, while for the indication of the availability of DU soft resource a new DCI format similar as DCI format 2-0 can be defined as follows.

Figure 6: Example of dynamic indication for availability for DU soft resource via new DCI format

As shown in Fig.6, each availability indicator consists of a bitmap with each bit indicating IA/INA for a soft DU symbol. In addition, each availability indicator can indicate DU soft resource availability of one or more slots for a serving cell. However, in that case, an IAB node may need to detect two DCIs, i.e., both legacy DCI format 2-0 and the new DCI format for correct slot format determination. The signaling overhead is higher than joint indication.

To summarize, for explicit indication of soft resources availability, new slot SFI-like indication via DCI format 2-0 to jointly indicate MT’s link direction and DU’s soft resource availability should be supported. In addition, both Alt.1-1 and Alt.1-2 can be considered to indicate the new slot format consisting of MT-D/MT-U/MT-F/DU-A.

Proposal 9: For explicit indication of soft resource availability, SFI-like indication via DCI format 2-0 for joint indication of DU soft resource availability and MT’s link direction is supported. The following sub-options should be considered to indicate the new slot format consisting of MT-D/MT-U/MT-F/DU-A.

- Alt.1-1: reuse DCI format 2-0 and use reserved entries (56-254) in current slot format table to indicate the availability of DU soft resource together with link direction for MT resource

- Alt.1-2: reuse DCI format 2-0 and enhance slotFormats in slotFormatCombination with 2-bit per symbol to directly indicate one out of MT-D/MT-U/MT-F/DU-A

Another issue of explicit indication of availability for DU soft resource is the applicability of the indication. If DCI format 2-0 is reused, a straightforward way is to reuse mechanism in Rel-15. That is, the SFI in DCI format 2-0 indicates a slot format starting from where the IAB node detects the DCI format 2-0. In addition, the number of slots for the indication is determined by the number of slotFormats included in the slotFormatCombination, which is equal to or larger than the number of slots within a PDCCH monitoring periodicity for DCI format 2_0.

2.4 New slot formats for IAB node

It was agreed in last meeting that slot formats with the sequence order UL-Flexible-DL are supported for semi-static IAB-node DU resource configurations and MT resource configurations. For MT resource, considering the flexibility of resource configuration, dynamic indication of the new slot formats should be also supported in addition to semi-static configuration and reserved entries in 38.213 Table 11.1.1-1 can be used. For DU resource, as analyzed in the above section, dynamic indication of DU link direction from parent node to child node is not needed, thus, dynamic indication of the new slot formats is needed only for MT.

Proposal 10: Dynamic indication of slot formats with the sequence order UL-Flexible-DL is supported for IAB-node MT resource configuration.

- Conclusion

In this contribution, we discussed on the mechanisms for resource multiplexing among backhaul and access links. Based on the discussion, we made following proposals.

Proposal 1: Parent IAB-node has the ability to be made aware of the resource type (H/S/NA) of its child IAB node.

Proposal 2: Parent IAB-node has the ability to be aware of the hard DU resource treated by its child IAB-node.

Proposal 3: Indications of the semi-static DU resource and the hard DU resource from IAB node to its parent node should be separated.

Proposal 4: Explicit signalling for information of timing misalignment between IAB node MT and IAB node DU to parent node is not necessary.

Proposal 5: For the case of MT DL Rx switching to DU DL Tx, the potential resource type conflict is handled by parent node with the information of propagation delay and the information of DU hard or DU treated as hard resources.

Proposal 6: For the case of DU UL Rx switching to MT UL Tx, whether the overlapping resources is used by MT or DU is decided by IAB node.

Proposal 7: DU-IA and MT-D/U/F should be dynamically indicated to align with Rel-15 operation and to save signalling overhead.

Proposal 8: Support dynamic indication of DU soft resource availability in symbol level.

Proposal 9: For explicit indication of soft resource availability, SFI-like indication via DCI format 2-0 for joint indication of DU soft resource availability and MT’s link direction is supported. The following sub-options should be considered to indicate the new slot format consisting of MT-D/MT-U/MT-F/DU-A.

- Alt.1-1: reuse DCI format 2-0 and use reserved entries (56-254) in current slot format table to indicate the availability of DU soft resource together with link direction for MT resource

- Alt.1-2: reuse DCI format 2-0 and enhance slotFormats in slotFormatCombination with 2-bit per symbol to directly indicate one out of MT-D/MT-U/MT-F/DU-A

Proposal 10: Dynamic indication of slot formats with the sequence order UL-Flexible-DL is supported for IAB-node MT resource configuration.

References

- 3GPP, RAN1#98, RAN1 Chairman’s Notes, Aug. 2019.

- 3GPP, RAN1#96bis, RAN1 Chairman’s Notes, Apr. 2019.

- R1-1909925, Qualcomm, “Upper layers parameters to support IAB physical layer operation,” Oct. 2019.

Appendix

A.1	Proposal for upper layer parameters

An upper layer parameters proposed in this contribution are captured in the tables for upper layer parameters for the resource multiplexing proposed in [3].

Table A1-1: Upper layer parameters for the resource multiplexing:

Dedicated RRC signaling from CU to IAB node MT

| Sub-feature group | New or existing parameter | Parameter name in specification | Description | Value range | Default value | IAB node specific/IAB nodes common | Specification | Comment |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Resource multiplexing | New or Existing | TDD-UL-DL-Config-IAB-MT | Per-MT D/U/F resource configuration | All patterns, periodicities, and slot configs supported in TDD-UL-DL-Config and TDD-UL-DL-SlotConfig + TBD slot formats with the sequence order UL-Flexible-DL |  | IAB node specific |  | Whether to introduce new IEs or enhancements to existing RRC signaling is up to RAN2 |
| Resource multiplexing | New or Existing | slotFormats-r16 | Enhancement of slotFormats in slotFormatCombination with 2-bit per symbol to directly indicate one out of MT-D/MT-U/MT-F/DU-A | TBD (e.g., one or multiple of 28 bit bitmap) |  | IAB nodes specific |  | Whether to introduce new IEs or enhancements to existing RRC signaling is up to RAN2 |

Table A1-2: Upper layer parameters for the resource multiplexing:

Dedicated signaling from IAB node MT to parent node DU or CU

| Sub-feature group | New or existing parameter | Parameter name in specification | Description | Value range | Default value | IAB node specific/IAB nodes common | Specification | Comment |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Resource multiplexing | New | DU-H symbol position (name in specification to be determined by RAN2/3) | DU-H symbol position in a slot | TBD (e.g., 14 bit bitmap) |  | IAB nodes specific |  | FFS for using RRC signalling or MAC CE If this is reported to CU, CU indicates this to parent node DU |
| Resource multiplexing | New | DU-H duration (name in specification to be determined by RAN2/3) | Duration of DU-H resource | TBD (e.g., number of slots) |  | IAB nodes specific |  | FFS for using RRC signalling or MAC CE If this is reported to CU, CU indicates this to parent node DU |
| Resource multiplexing | New | DU-H periodicity and offset (name in specification to be determined by RAN2/3) | Periodicity and offset of DU-H resource | TBD |  | IAB nodes specific |  | FFS for using RRC signalling or MAC CE If this is reported to CU, CU indicates this to parent node DU |