3GPP TSG RAN WG1 #98bis		                                    R1-1911297

Chongqing, China, October 14th – 20th, 2019

Agenda Item:	7.2.6.3

Source:	InterDigital Inc.

Title:	PUSCH Enhancements for eURLLC

Document for:	Discussion and Decision

# Introduction

This contribution discusses the following issues related to PUSCH enhancements:

- Interpretation and signalling of L and K

- Handling of orphan symbols

- Interaction of enhanced PUSCH with DL/UL direction

- Collision with other signals (SRS/PUCCH/other PUSCH)

# Interpretation and signalling of L and K

In RAN1#97, RAN1 agreed to adopt Option 4 from TR38.824 (with an update for the TDRA field indication).

In RAN1#98, RAN1 agreed to narrow down between two alternatives for the interpretation of L and K:

In terms of how to interpret L and K for all PUSCH transmissions, down-select between the following two: - Alt 1: The time window within which valid symbols are used for transmission is L*K. - FFS the definition of “valid symbols” - Alt 2: The time window within which valid symbols are used for transmission can be longer than L*K symbols, and it is extended at least in case of semi-static DL symbols. - FFS extension of the time window in case of dynamic DL symbols and/or semi-static flexible symbols and/or reserved symbols (if defined) and/or SSB symbols and/or type-0 CSS in CORESET#0 (as indicated by MIB) - FFS the definition of “valid symbols” - FFS whether to define a maximum time window size and if so, details

In Alt. 1, the network may set the value of L*K that results in the needed number of transmitted symbols after removal of the invalid symbols. Therefore, the value of L*K may be larger than the number of transmitted symbols that meets the performance requirement. For configured grant, a single value of L*K is applicable to all periodically recurring instances and may result into different numbers of transmitted symbols between instances depending on the slot format. The network may have to set L*K to ensure that for all instances, the number of transmitted symbols is above a minimum. As a result, there may be excessively large number of transmitted symbols for most instances and resource waste.

In Alt. 2, the value of L*K should in principle correspond to the actual number of transmitted symbols for all instances of a configured grant. The duration of the time window within which symbols are transmitted is implicitly extended when symbols are unavailable for transmission due to the slot structure. This may result in different durations of this time window between configured grant instances, depending on the slot format. One potential drawback is that there may be DMRS misalignment between UEs in some instances. In this case the network may have to configure a larger value of L*K to compensate for performance loss (as in Alt. 1). However, it may also be possible for the network to reduce the severity of this issue by resource management.

Both alternatives may result in some unnecessary transmissions for configured grant. The extent of the resource waste depends on how much variation of slot format can exist between subsequent instances, which is largely under network control. However, there seems to be a bit more flexibility to manage the trade-off between ability to change slot format and resource waste with Alt. 2.

Proposal 1: The time window within which valid symbols are used for transmission can be longer than L*K symbols and it is extended at least in case of semi-static DL symbols (Alt.2).

In RAN1#97, RAN1 agreed that the TDRA field indicates the resource for the first nominal repetition. For dynamic grant, the DCI also needs to signal the values of K and L. Since the values of these parameters are correlated with the resource of the first repetition in practice, it makes sense to signal them from the same field to reduce overhead. We suggest using the same approach for the configured grant type 1 to unify the design.

Proposal 2: The TDRA field in the DCI indicates the values of L and K.

Proposal 3: The TDRA parameter of the type 1 configured grant indicates the values of L and K.

# Handling of orphan symbols

Previous agreement for Option 4 states:

if a "nominal" repetition goes across the slot boundary or DL/UL switching point, this "nominal" repetition is split into multiple PUSCH repetitions, with one PUSCH repetition in each UL period in a slot.

There could be scenarios where a “split” PUSCH repetition consists of a single symbol that would carry e.g. a front-loaded DMRS. In such case transmitting for this repetition may bring very limited benefit, but cause interference. For this reason, we propose that the UE skips this repetition.

Proposal 4: The UE does not transmit a repetition consisting of a single symbol (“orphan symbol”).

In the context of the previous agreement on the interpretation of K and L, the orphan symbol should not be a valid symbol.

Proposal 5: If a repetition consists of a single symbol, this symbol is not a valid symbol.

If Alt. 2 of the previous agreement is adopted, this means that the UE should postpone the corresponding repetition to the next available opportunity.

Proposal 6: When a repetition consists of a single symbol, the UE postpones the repetition to the next available opportunity.

# Interaction of enhanced PUSCH with DL/UL direction

In RAN1#98, RAN1 agreed to consider a set of options to handle the interaction of enhanced PUSCH with DL/UL directions (slot format) when dynamic SFI is configured.

A first issue is whether the UE should use information from the dynamic SFI to determine valid symbols for transmission (Option 1 vs Option 2).

In R15, a group-common DCI carries the dynamic SFI. If the UE fails to receive the DCI carrying SFI, this may affect transmission of PUSCH by configured grant but not by dynamic grant. The UE does not perform an unexpected transmission as a result of failing to receive SFI.

Given that the main purpose of enhanced PUSCH is to guarantee high reliability and low latency, it seems better to not introduce an independent source of error by utilizing the dynamic SFI carried on a separate DCI. This should apply to enhanced PUSCH scheduled by dynamic grant as well as enhanced PUSCH by configured grant.

Proposal 7: Enhanced PUSCH scheduled by dynamic grant does not depend on SFI in DCI format 2_0 (Option 1).

Proposal 8: Enhanced PUSCH transmission by configured grant does not depend on SFI in DCI format 2_0 (Option 1).

For the case of dynamic grant, the Conclusion from RAN1#98 identifies 4 sub-options under Option 1 (1-1 to 1-4):

Option 1-1: Semi-static flexible symbols are used for PUSCH. A potential drawback with this option is that when the network schedules the enhanced PUSCH, subsequent flexible symbols are constrained to be for uplink transmissions and latency-critical downlink transmissions may be delayed.

Option 1-2: Semi-static flexible symbols are not used for PUSCH. A potential drawback with this option is that it may unnecessarily delay the enhanced PUSCH when flexible symbols are not needed for downlink transmissions.

Option 1-3: Dynamic indication in UL grant on which set of semi-static flexible symbols are used for PUSCH. This option offers the most flexibility. To minimize the overhead, the TDRA field can provide the dynamic indication of the set of semi-static flexible symbols used for PUSCH (e.g. using SFI value), jointly with other parameters.

Option 1-4: Pre-defined rules to determine which set of semi-static flexible symbols are used for PUSCH. This option attempts to compromise between Option 1-1 and Option 1-2. However, it does not improve flexibility from network perspective.

Proposal 9: For dynamic grant, DCI indicates which set of semi-static flexible symbols is used of PUSCH (Option 1-3).

For the case of configured grant, the Conclusion from RAN1#98 identifies 2 sub-options under Option 1 (1-2 and 1-4):

Option 1-2: Semi-static DL/flexible symbols are not used for PUSCH.

Option 1-4: Pre-defined rules to determine which set of semi-static flexible symbols are used for PUSCH.

There is no obvious benefit with Option 1-4 since the semi-static flexible symbols that would be used for PUSCH could instead be configured as uplink in the semi-static configuration.

Proposal 10: For configured grant, semi-static downlink/flexible symbols are not used for PUSCH (Option 1-2).

If the UE does not transmit a repetition according to the above rules, it should postpone transmission of the repetition to the next available opportunity, consistent with Alt. 2 from the agreement on the interpretation of L and K.

Proposal 11: The UE postpones a repetition to the next available opportunity when the repetition overlaps with symbols that cannot be used for PUSCH.

# Collision with other signals

The UE may need to avoid other important signals from other UEs and from itself during it PUSCH transmission. To avoid signals such as the SRS, PUCCH, PUSCH from the same or other UEs, the gNB could signal resources that the UE should avoid. The UE could rate match around these resources or puncture its transmission that coincides with these resources. The gNB could signal the usable resources to a specific UE or to a group of UEs. The signaling can be based on a bitfield which can be mapped to the UL repetition resources, explicitly indicating the frequency and time of the resources in RBs and OFDM symbols to be avoided. The granularity for time-frequency resources can be semi-statically configured by a higher layer.

Proposal 12: DCI indicates resources that need to be avoided during PUSCH repetition.

# Conclusion

In this contribution, we provided our views on the PUSCH enhancements for eURLLC. Our proposals are as follows:

Proposal 1: The time window within which valid symbols are used for transmission can be longer than L*K symbols and it is extended at least in case of semi-static DL symbols (Alt.2).

Proposal 2: The TDRA field in the DCI indicates the values of L and K.

Proposal 3: The TDRA parameter of the type 1 configured grant indicates the values of L and K.

Proposal 4: The UE does not transmit a repetition consisting of a single symbol (“orphan symbol”).

Proposal 5: If a repetition consists of a single symbol, this symbol is not a valid symbol.

Proposal 6: When a repetition consists of a single symbol, the UE postpones the repetition to the next available opportunity.

Proposal 7: Enhanced PUSCH scheduled by dynamic grant does not depend on SFI in DCI format 2_0 (Option 1).

Proposal 8: Enhanced PUSCH transmission by configured grant does not depend on SFI in DCI format 2_0 (Option 1).

Proposal 9: For dynamic grant, DCI indicates which set of semi-static flexible symbols is used of PUSCH (Option 1-3).

Proposal 10: For configured grant, semi-static downlink/flexible symbols are not used for PUSCH (Option 1-2).

Proposal 11: The UE postpones a repetition to the next available opportunity when the repetition overlaps with symbols that cannot be used for PUSCH.

Proposal 12: DCI indicates resources that need to be avoided during PUSCH repetition.

# References

- TR38.824, “Study on physical layer enhancements for NR ultra-reliable and low latency case (URLLC)”, v16.0.0.

# Appendix

Agreements from RAN1#97:

Agreements:

- Adopt option 4 with the following update:

- The time domain resource assignment (TDRA) field in the DCI or the TDRA parameter in the type 1 configured grant indicates the resource for the first “nominal” repetition.

  - FFS the detailed interaction with the procedure of UL/DL direction determination

TR38.824 [1]

### 6.3.3	Option 4

One or more actual PUSCH repetitions in one slot, or two or more actual PUSCH repetitions across slot boundary in consecutive available slots, is supported using one UL grant for dynamic PUSCH, and one configured grant configuration for configured grant PUSCH. It further consists of:

-	The number of the repetitions signalled by gNB represents the "nominal" number of repetitions. The actual number of repetitions can be larger than the nominal number.

-	FFS dynamically or semi-statically signalled for dynamic PUSCH and type 2 configured grant PUSCH

-	The time domain resource assignment (TDRA) field in the DCI or the TDRA parameter in the type 1 configured grant indicates the resource for the first "nominal" repetition.

-	The time domain resources for the remaining repetitions are derived based at least on the resources for the first repetition and the UL/DL direction of the symbols.

-	FFS the detailed interaction with the procedure of UL/DL direction determination

-	If a "nominal" repetition goes across the slot boundary or DL/UL switching point, this "nominal" repetition is splitted into multiple PUSCH repetitions, with one PUSCH repetition in each UL period in a slot.

-	Handling of the repetitions under some conditions, e.g., when the duration is too small due to splitting, is to be further investigated in the WI phase.

-	No DMRS sharing across multiple PUSCH repetitions

-	The maximum TBS size is not increased compared to Rel-15.

-	FFS: L > 14

-	S+L can be larger than 14

-	FFS: The bitwidth for TDRA is up to 4 bits.

-	Note: different repetitions may have the same or different RV.

Agreements from RAN1#98:

Agreements:

In terms of how to interpret L and K for all PUSCH transmissions, down-select between the following two:

- Alt 1: The time window within which valid symbols are used for transmission is L*K.

  - FFS the definition of “valid symbols”

- Alt 2: The time window within which valid symbols are used for transmission can be longer than L*K symbols, and it is extended at least in case of semi-static DL symbols.

  - FFS extension of the time window in case of dynamic DL symbols and/or semi-static flexible symbols and/or reserved symbols (if defined) and/or SSB symbols and/or type-0 CSS in CORESET#0 (as indicated by MIB)

  - FFS the definition of “valid symbols”

  - FFS whether to define a maximum time window size and if so, details

Conclusion:

In terms of how to handle the interaction of enhanced PUSCH with DL/UL directions, consider the following options:

- For DG PUSCH

  - If dynamic SFI is not configured,

    - Semi-static flexible symbols are used for PUSCH. Segmentation occurs only around semi-static DL symbols.

  - If dynamic SFI is configured

    - Option 1: behavior not dependent on dynamic SFI

      - Option 1-1: Semi-static flexible symbols are used for PUSCH. Segmentation occurs only around semi-static DL symbols.

        - FFS whether the conflict between dynamic SFI and symbols used for PUSCH transmission is considered as an error case, e.g.

          - Option 1-1a: The UE does not expect any semi-static flexible symbol to be indicated as DL within the PUSCH transmission time window.

          - Option 1-1b: No error case is defined and in general all semi-static flexible symbols are used for PUSCH within the PUSCH transmission time window.

      - Option 1-2: Semi-static DL/flexible symbols are not used for PUSCH. Segmentation occurs around semi-static DL/flexible symbols.

      - Option 1-3: Dynamic indication in UL grant on which set of semi-static flexible symbols are used for PUSCH. Segmentation occurs around semi-static DL and the dynamically indicated invalid symbols.

      - Option 1-4: Pre-defined rules to determine which set of semi-static flexible symbols are used for PUSCH. Segmentation occurs around semi-static DL and the invalid symbols as defined in the rules.

    - Option 2: the UE uses SFI to determine the symbols to transmit

      - In case SFI is configured and received

        - Option 2-1: Segmentation occurs around semi-static DL symbols and dynamic DL/flexible symbols

        - Option 2-2: Dynamic flexible symbols are used for PUSCH. Segmentation occurs around semi-static DL symbols and dynamic DL symbols

        - Option 2-3: Dynamic flexible symbols are used for PUSCH. A repetition is not transmitted if it conflicts with a dynamic DL symbol.

        - Option 2-4: A repetition is not transmitted if it conflicts with a dynamic DL/flexible symbol

      - In case SFI is configured and not received

        - A repetition is not transmitted if it conflicts with a semi-static flexible symbol.

- For CG PUSCH other than the first Type 2 CG PUSCH (including all the repetitions) activated by an UL grant

  - If dynamic SFI is not configured,

    - Semi-static flexible symbols are used for PUSCH. Segmentation occurs only around semi-static DL symbols.

  - If dynamic SFI is configured

    - Option 1: behavior not dependent on dynamic SFI

      - Option 1-1: Semi-static flexible symbols are used for PUSCH. Segmentation occurs only around semi-static DL symbols.

        - This does not seem to make much sense for CG. If semi-static flexible symbols are always used for CG PUSCH, the gNB can essentially configure these symbols as UL in semi-static configuration. – no need for this option?

      - Option 1-2: Semi-static DL/flexible symbols are not used for PUSCH. Segmentation occurs around semi-static DL/flexible symbols.

      - Option 1-3 from DG is not applicable for CG.

      - Option 1-4: Pre-defined rules to determine which set of semi-static flexible symbols are used for PUSCH. Segmentation occurs around semi-static DL and the invalid symbols as defined in the rules.

    - Option 2: the UE uses SFI to determine the symbols to transmit

      - In case SFI is configured and received

        - Option 2-1: Segmentation occurs around semi-static DL symbols and dynamic DL/flexible symbols

        - Option 2-2 does not make sense for CG. (Dynamic flexible symbols are used for PUSCH. Segmentation occurs around semi-static DL symbols and dynamic DL symbols)

        - Option 2-3 does not make sense for CG. (Dynamic flexible symbols are used for PUSCH. A repetition is not transmitted if it conflicts with a dynamic DL symbol.)

        - Option 2-4: a repetition is not transmitted if it conflicts with a semi-static DL symbol and a dynamic DL/flexible symbol

      - In case SFI is configured and not received

        - A repetition is not transmitted if it conflicts with a semi-static flexible symbol.

- For the first Type 2 CG PUSCH (including all the repetitions) activated by an UL grant,

  - Alt 1: same behavior as DG PUSCH

  - Alt 2: same behavior as CG PUSCH without an associated UL grant

  - …

- FFS: in case of a repetition not being transmitted (as in the above bullets), whether a repetition is a nominal repetition or a repetition after segmentation due to semi-static DL symbol(s)/slot boundary

- FFS: whether to postpone or not, and if yes, under what condition(s)

- FFS: whether/how guard period is handled

- Note that segmentation at slot boundary is always performed, even though it is not explicitly mentioned in the bullets above.

- FFS: the handling of conflict with SSB/PRACH symbols, the handling of conflict with semi-statically configured DL reception, etc.

- Other options are not precluded