3GPP TSG-RAN WG1 Meeting #99	        		            R1-1912449

Reno, USA, November 18th – 22nd, 2019

Agenda item:	7.2.2.2.1

Source: 	Samsung

Title: 	Channel access procedures for NR-U

Document for:	Discussion and Decision

# Introduction

In RAN1 #98b meeting [1], the following agreements regarding channel access for NR-U were made:

Agreement:

Sharing of a UE-initiated channel occupancy (either CG-PUSCH or scheduled UL) with gNB is supported, such that the gNB is allowed to transmit control/broadcast signals/channels for any UEs as long as the transmission contains transmissions for the UE that initiated the channel occupancy and/or DL signals/channels (PDSCH, PDCCH, reference signals) meant for the UE that initiated the channel occupancy.

- The ED threshold that the UE applies when initiating a channel occupancy to be shared with the gNB is configured by gNB (RRC signaling)

  - if ED threshold that the UE applies when initiating a channel occupancy to be shared with the gNB is not configured, the transmission of the gNB in UE initiated COT may include only control/broadcast signals/channels transmissions of up to 2/4/8 OFDM symbols in duration for 15/30/60 kHz SCS

  - When absence of WiFi cannot be assumed based on e.g. regulation, the ED threshold that the gNB configures to the UE to apply when initiating the channel occupancy is determined based on the max gNB TX power

- Cat. 2 LBT can be used (for gaps of 16us and 25us).

- Cat. 1 LBT can be used under the following conditions.

  - Gap duration <= 16us

  - For the transmission of the gNB after the first switch between the UE and the gNB if the gNB transmission contains only control/broadcast signals/channels

  - For the transmission of the gNB after the first switch between the UE and the gNB if the gNB transmission has a duration below X ms (X >= 0).

    - FFS: X

  - FFS: For transmissions after the second and subsequent switches between UE and gNB

Agreement:

- For the CP extension prior to at least a dynamically scheduled PUSCH transmission, the CP extension is located in the symbol(s) immediately preceding the PUSCH allocation indicated by SLIV. The supported durations for CP extension at the UE are:

  - 0 (i.e. no CP extension)

  - C1*symbol length – 25 us

  - C2*symbol length – 16 us - TA

  - C3*symbol length – 25 us – TA

- C1=1 for 15 and 30 kHz SCS, C1=2 for 60 kHz SCS

- FFS: Whether C2/C3 is fixed or implicitly derived based on TA for each subcarrier spacing

- The N2 timeline (UL grant to PUSCH delay) needs to be relaxed to take the CP extension into account

- FFS: Whether the limit as per the previous agreement bounding the resulting CP extension to be less than or equal to one symbol for the given subcarrier spacing should be relaxed

- FFS: Applicability of this to other UL transmissions

- FFS: Whether the number of durations for CP extension that the UE can be signalled dynamically can be configured

Agreement:

- For FBE operation, when the gNB operates as an initiating device

- The UE is provided information on the gNB fixed frame period and the starting positions of the fixed frame periods

- For the provision of the above information the following is signalled

  - Indication of the fixed frame period and the starting positions of the fixed frame period (SIBx)

    - FFS: Whether Rel-15 signaling can be reused

- When the UE is not initiating a channel occupancy, UE transmissions within a fixed frame period can occur if DL signals/channels (e.g., PDCCH, SSB, PBCH, RMSI, GC-PDCCH, …) within the fixed frame period are detected.

  - FFS: Extension of GC-PDCCH configuration to idle UEs

- FFS: Signaling for FBE operation, when the UE operates as an initiating device

This contribution discusses the design considerations for FR1 NR-U channel access, including the LBT type/priority class indication method, contention window size adaptation rule for CAT-4 LBT of NR-U data transmissions and transmissions without HARQ/ACK feedbacks, LBT for wide-band operation design, and UE to gNB COT sharing, support for directional LBT, LBT for FBE-based NR-U, LBT with handshake mechanism.

# Signaling support for LBT type/priority indication

Since the LBT type by a UE prior to transmission of the UL burst can include CAT-1 LBT, 16µs CAT-2 LBT, 25 µs CAT-2 LBT, and CAT-4 LBT, a corresponding signaling method of the LBT type needs to be indicated to the UE.

Either an explicit signaling method or an implicit signaling method for the LBT type indication can be considered. With an explicit signaling method, the LBT type for UL burst can be indicated dynamically through the UL grant scheduling the UL transmission. For example, an LBT type field of 2 bits can be included in the UL grant to indicate one of the LBT type of CAT-1 LBT, 16µs CAT-2 LBT, 25 µs CAT-2 LBT, and CAT-4 LBT.

With an implicit signaling method, the UE determines its LBT type implicitly without an explicit LBT type indication through the UL grant. However, the implicit indication method would need the UE to determine or to be indicated with the COT structure and/or gap duration between transmissions within the COT. In addition, the implicit indication method could have ambiguity when the end of previous DL transmission and start of the scheduled UL burst has a gap duration of 16µs, in which case either CAT-1 LBT or 16µs CAT-2 LBT can be applied by the UE.

Compared to the implicit signaling method, the explicit signaling method can provide better flexibility in configuring the UE UL LBT type as well as less ambiguity in indicating the LBT type for the UL burst. As a result, an explicit indication method of the LBT type for UL transmission can be utilized.

The channel access priority class (CAPC) value corresponding to the scheduled UL burst can also be explicitly indicated similar to the LBT type indication. If the UL burst includes multiple UL signals/channels with different CAT-4 LBT priority classes, the highest channel access priority class value can be applied by the UE.

In addition, it has been agreed in [1] that the four durations for the CP extension at the UE can be supported, including: (1) 0; (2) C1 * symbol length – 25us; (3) C2 * symbol length – 16us – TA; and (4) C3 * symbol length – 25us – TA. The value of C1 is agreed to be 1 and 2 for 15/30 kHz SCS and 60 kHz SCS respectively. Similarly, the value of C2 and C3 can be fixed. Considering the typical scenario for NR-U is small cell with cell radius less than 1000 m, it is sufficient to fix C2 and C3 to 1 for 15/30kHz SCS (same as Rel-15 eLAA for 15KHz case), and 2 for 60KHz.

Proposal 1: C2/C3=1 for 15 and 30KHz SCS, and C2/C3=2 for 60KHz SCS.

Since the CP extension duration is closely related to the LBT type of the UL transmission, at least the UL LBT type and CP extension duration can be jointly indicated unless there is significant limitation on a flexibility. For example, CAT1-LBT and 16us CAT-2 LBT can be indicated with C2 * symbol length – 16us – TA while 25us CAT-2 LBT be with C1 * symbol length – 25us or C3 * symbol length – 25us – TA. However, if CAT-4 LBT needs to be applied for all CP extension durations (i.e., not limited to the CP duration of 0), as supported in LTE-eLAA, separate indication of the CP extension duration, the LBT type, and the CAPC is more desirable because the benefits of joint indication is quite marginal.

Proposal 2: If CAT-4 LBT can be used for all CP extension durations, separate indication of the CP extension duration, the LBT type, and the CAPC are preferred. Otherwise, joint indication can be considered.

For single PUSCH scheduling, UE performs LBT according to the LBT type indication in UL grant. For multi-PUSCH scheduling, if UE fails to access the channel for the first PUSCH, whether UE can continue to attempt the next PUSCH transmission according to the indicated LBT type needs to be clarified. Specifically, if UL grant indicates Cat-2 16us LBT, and if UE fails to access the channel for first scheduled PUSCH, the gap between DL ending symbol and next PUSCH start symbol is larger than 16us gap. In that case, UE shall not perform Cat-2 16us LBT. Instead, Cat-2 25us LBT shall be performed. For other LBT types, i.e. Cat-1 LBT, Cat-2 25us LBT and Cat-4 LBT, UE can continue the on-going LBT procedure for next PUSCH transmission.

Proposal 3: For multi-PUSCH scheduling, if Cat-2 16us LBT is indicated by the UL grant, UE can perform Cat-2 16us LBT for the first scheduled PUSCH and switch to Cat-2 25us LBT for next PUSCH transmission if UE fails to access the channel for first scheduled PUSCHs.

# Contention Window Size Adaptation for NR-U Data Transmissions

In [1], the reference duration for NR-U contention window size (CWS) adjustment has been defined for both gNB-initiated COT with unicast PDSCH transmission(s) and UE-initiated COT with PUSCH transmission. A remaining detail for CWS adjustment is the determination of a reference transmission burst. Specifically, for DL CWS adjustment, by the timing that NR-U gNB starts to perform a CAT-4 LBT procedure, gNB needs to identify a set of HARQ-ACK values corresponding to the reference duration from a reference DL transmission burst for the CWS adjust decision. The reference transmission burst can be defined as the most recent transmission burst wherein at least some HARQ-ACK feedbacks for the reference duration within this transmission burst are expected to be available by the time gNB performs CWS adaptation.

An illustration of the timing relation is provided in Figure 1, wherein the gNB attempts a new LBT process and updates the CWS on slot n-3. A corresponding reference downlink transmission burst consists of slot n-5 and n-4, wherein three UEs are scheduled with the PDSCH to HARQ delay of K1 = 1, 5, 4 slots respectively. The reference duration in this example is slot n-5, and the HARQ-ACK value corresponding to the reference duration from UE1 can be expected in a HARQ-ACK slot n-4, while the HARQ-ACK values corresponding to UE2 and UE3 can be expected in a HARQ-ACK slot n. Then, gNB updates CWS for a DL burst starting from slot n-2 based on HARQ-ACK of UE1.

Figure 1. An example of the reference transmission burst.

Proposal 4: A reference DL transmission burst can be the most recent burst wherein at least some HARQ-ACK feedbacks corresponding to the reference duration of this burst is expected to be available when gNB adjusts CWS.

In LTE-LAA, each eNB shall adjust the contention window based on a given reference subframe only once. For NR-U, due to its flexible and configurable timing from PDSCH transmission to corresponding HARQ-ACK feedback (i.e., K1), potential LBT failure for HARQ-ACK and potential HARQ-ACK enhancements, the limit on how gNB utilizes a given reference duration for CW adjustment can be relaxed from LTE-LAA. Specifically, it is possible for different transmission bursts to be corresponding to different PDSCHs in the same reference duration for CWS adaptation, and thus gNB may adjust the CWS based on a given reference duration more than once but based on a given PDSCH only once. An example is provided in Figure 2, wherein DL burst 0, 1, and 2 spans from slot n-5 to n-3, slot n to slot n+2, and slot n+4 to slot n+6 respectively, and K1 represents the PDSCH to HARQ delay in terms of number of NR-U slots. The CW adjust for DL burst 1 can be based on HARQ-ACK feedback of PDSCH transmission(s) to UE0 on DL burst 0; and the CW adjust for DL burst 2 can be based on HARQ-ACK feedback of PDSCH transmission(s) to UE1 on DL burst 0 (HARQ-ACK feedbacks for DL burst 1 is not expected to be received by slot n+3). As a result, gNB adjusts the contention window based on the reference duration of DL burst 0 twice.

Figure 2. An example of the reference duration and reference transmission burst.

Proposal 5: NR-U gNB can adjust CWS based on a reference duration more than once.

In the set of UL resources that are expected to contain the HARQ-ACK feedback(s) corresponding to the reference duration for CW adjustment, HARQ-ACK feedbacks in addition to those corresponding to the reference duration can be included due to flexible PDSCH to HARQ delay. For example, in Figure 2, both UE0 and UE2 are expected to report HARQ-ACK feedbacks at slot n-2. Under such a scenario, only the HARQ-ACK feedback(s) corresponding to the reference duration received by the timing that gNB performs CW adjustment can be utilized for CW adjustment. In the example of Figure 2, only the HARQ-ACK feedback(s) corresponding to PDSCH transmission to UE0 on slot n-5 will be utilized for CW adjustment decision on slot n-1.

Proposal 6: NR-U gNB can adjust CWS only based on the HARQ-ACK feedbacks corresponding to the reference duration.

The CWS adaptation rule for NR-U also needs to be enhanced from LTE-LAA, given that CBG-based HARQ-ACK feedback is supported in addition to TB-based feedback. Specifically, the gNB can increase CWS to next available value if at least 80% of HARQ-ACK values corresponding to the TBs transmitted in the reference duration are determined as NACK. In particular, when CBG-based HARQ-ACK feedbacks are used for a TB, this TB is determined as NACK if the HARQ-ACK value for all transmitted (or re-transmitted) CBGs within this TB is NACK. In addition, by the timing that gNB performs the CWS adaptation, the ACK/NACK corresponding to the reference duration that is missing (e.g., due to failed LBT) or has not become available yet (e.g., due to large K1) can be treated as DTX and not counted towards the current CWS adaptation decision.

Proposal 7: CWS is increased to next available value if at least 80% of the TBs transmitted in the reference duration are determined as NACK; and a TB with CBG-based feedback is determined as NACK if the HARQ-ACK value for all (re)transmitted CBGs within this TB is NACK.

Proposal 8: By the timing that gNB performs the CWS adaptation, missed/unavailable HARQ-ACK of reference duration can be not counted towards the current CWS adaptation decision.

NR-U UL CWS adaptation can also be based on HARQ-ACK feedbacks from gNB, e.g., by DFI-like DCI, or by NDI in UL grant. In LTE-LAA, the minimum latency between the slot in which UE receives UL grant or DFI and the reference duration is defined as 4ms. Similarly, a minimum latency should be defined for NR-U.  In last meeting, RAN1 agreed that gNB configures minimum duration D from the ending symbol of the PUSCH to the starting symbol of the DFI carrying HARQ-ACK for that PUSCH by RRC signaling. The same minimum duration can be reused as the  minimum latency between reference duration/slot and the following UL grant or DFI for CWS adaption.

Proposal 9: Reuse the RRC configured minimum duration D between PUSCH and DFI carrying HARQ-ACK for that PUSCH for the minimum latency between reference duration and the following UL grant or DFI  for CWS adaption.

# Contention Window Size for Transmissions without HARQ-ACK

In addition to the regular data transmission, NR-U can also use CAT-4 LBT to grant transmissions of non-unicast signals/channels without HARQ-ACK feedbacks, such as the DRS, OSI, paging, Msg2, PDCCH, PRACH, PUCCH, etc. The maintenance and adjustment of CAT-4 LBT CWS for transmissions without HARQ-ACK feedback needs to be enhanced from that of the regular NR-U data transmissions.

## A baseline CWS maintenance rule for signals/channels without HARQ-ACK

When CAT-4 LBT is used for signals/channels without HARQ-ACK feedback, the CWS corresponding to the CAT-4 LBT priority class of such signals/channels can be updated according to the procedure defined in Section 3 when they are multiplexed with other unicast data within a same transmission burst. Specifically, the CWS is increased to next available value or reset to the minimum according to the HARQ-ACK feedbacks of the unicast data on the reference duration. Otherwise, if the signals/channels without HARQ-ACK are transmitted alone or multiplexed only with other non-unicast data within a same transmission burst, the CWS corresponding to the CAT-4 LBT priority class of such signals/channels can remain unchanged. For example, the baseline rule can be applied to standalone downlink/uplink transmission such as PDCCH (excluding UL grant), PUCCH, SRS, or PUSCH without UL-SCH.

Proposal 10: If the signals/channels without HARQ-ACK are transmitted alone or multiplexed only with other non-unicast data within a same transmission burst, the CWS corresponding to the CAT-4 LBT priority class of such signals/channels can remain unchanged.

## CWS Adjust for Random Access

The random access procedure is based on a 4-step procedure, wherein PRACH/Msg1, Msg2, Msg3 and Msg4 need to be exchanged between UE and gNB in a sequential order. Therefore, the CWS adjust for the random access procedure can be further enhanced from the baseline CWS maintenance rule for signals/channels without HARQ-ACK.

It has been agreed that CAT-4 is applied for UL transmissions that are part of a RACH procedure that initiate a channel occupancy. Therefore, CAT-4 LBT is used for standalone transmission of NR-U PRACH. A CWS adjust rule corresponding to the CAT-4 LBT of PRACH needs to be designed, which can be based on the reception status of RAR. Specifically, the CWS for PRACH LBT can be increased to the next available value if RAR corresponding to the transmitted PRACH has not been received within the RAR monitoring window; otherwise, the CWS for PRACH LBT can reset to the minimum value.

Proposal 11: CWS for PRACH LBT can increase to the next available value if RAR is not received within the RAR monitoring window; otherwise, the CWS for PRACH LBT can reset to the minimum value.

The LBT for Msg2 can be CAT-2 LBT if the Msg2 can be transmitted within the UE-initiated MCOT obtained by CAT-4 LBT of PRACH; otherwise, LBT for Msg2 can be CAT-4 based LBT potentially with higher LBT priority class for a faster Msg2 transmission. If Msg2 is transmitted alone and is subject to CAT-4 LBT, the CWS adaptation for Msg2 can be based on the reception of Msg3. For example, if Msg3 that corresponds to the Msg2 is received, the Msg2 transmission is considered as successful and CWS for Msg2 LBT can reset to minimum. By contrast, if Msg3 that corresponds to this Msg2 is not received (e.g., after certain duration following Msg2 transmission), this Msg2 transmission can be considered as subject to collision and CWS can be increased to the next available value.

Proposal 12: CWS for Msg2 with CAT-4 LBT can reset to the minimum value if correct Msg3 corresponding to the transmitted Msg2 is received; otherwise CWS for Msg2 increases to the next available value.

When RAR is subject to CAT-4 LBT, Msg3 can be scheduled to be within the MCOT of RAR through UL grant, such that the Msg3 can be transmitted subject to a CAT-2 LBT. When RAR is transmitted following a CAT-2 LBT, or if Msg3 is scheduled to be outside the MCOT of RAR, a CAT-4 LBT potentially with the lowest channel access priority class value can be used for Msg3 transmission.

If CAT-4 LBT is used for Msg3, the CWS adaptation for Msg3 can be based on the reception status of Msg4. For example, if correct Msg4 that corresponds to this Msg3 is received (e.g., containing the correct UE contention resolution identity), the Msg3 transmission is considered as successful and CWS for Msg3 LBT can reset to minimum. On the other hand, if the correct Msg4 that corresponds to this Msg3 is not received (e.g., after certain duration following Msg3 transmission, or a wrong UE contention resolution identity is detected), this Msg3 transmission can be considered as subject to collision and CWS can be increased to the next available value.

Proposal 13: CWS for Msg3 with CAT-4 LBT can reset to the minimum value if correct Msg4 corresponding to the transmitted Msg3 is received; otherwise CWS for Msg3 increases to the next available value.

For 2-step RACH, the CWS for MsgA LBT can be increased to the next available value if MsgB corresponding to the transmitted MsgA has not been received within the RAR monitoring window. If successRAR is received, MsgA transmission is considered as successful and CWS can reset to minimum. If fallbackRAR is received, although Msg A PUSCH transmission fails, it still makes sense to reset CWS or reuse same CWS for MsgA PUSCH retransmission, considering MsgA PRACH transmission is successful and the failure of Msg A PUSCH is less likely caused by hidden node.

Proposal 14: For 2-step RACH, CWS for MsgA PRACH+PUSCH LBT can increase to the next available value if MsgB corresponding to the transmitted MsgA is not received within the RAR monitoring window; CWS for MsgA PUSCH transmission can remain unchanged if fallbackRAR is received and can reset to the minimum value if successRAR is received.

# Contention Window Size for Wideband Operation of NR-U

It has been agreed in RAN1 #97 [1] that multi-carrier channel access schemes of LTE-LAA are applicable for the sub-band LBT of NR-U wideband operations. One remaining issue is how the contention window size (CWS) for each LBT bandwidth is maintained and updated, wherein either each LBT bandwidth can maintain a CWS independently, or a single CWS can be maintained across all the LBT bandwidths.

Since it was concluded in [1] that a UE can receive a PDSCH scheduled within an LBT bandwidth or over multiple LBT bandwidths, thus when UE receives a PDSCH scheduled within multiple LBT bandwidths, a mechanism to calculate HARQ-ACK for each LBT bandwidth that overlaps with the scheduled PDSCH needs to be designed if each LBT bandwidth maintains a CWS independently. For example, such mechanism could depend on the PDSCH mapping scheme over multiple LBT bandwidths.

As a result, a baseline method to maintain CWS for NR-U wideband operation is to maintain a single CWS across all LBT bandwidths similar to type B1 multi-carrier LBT scheme of LTE-LAA. Specifically, a single CWS can be maintained for all LBT bandwidths, and CWS is updated according to HARQ-ACK corresponding to the PDSCH transmissions on reference durations across all LBT bandwidths. Compared to maintaining a CWS for each LBT bandwidth independently, such baseline method works irrespective of the PDSCH mapping scheme, and does not require defining a mechanism to calculate HARQ-ACK for each LBT bandwidth that overlaps with the scheduled PDSCH.

Proposal 15: A single CWS can be maintained across LBT bandwidths for NR-U wideband operation similar to type B1 of LTE-LAA multi-carrier LBT.

A UE can be configured with multiple UL BWPs within a carrier, and only one BWP can be activated for one time. These BWPs may be partially overlapped or non-overlapped as shown in Figure 3. How to manage CWS with UL BWP operation should be clarified. For example, a single CWS can be maintained per UE across UL BWPs, or separate CWS can be maintained for each UL BWP. If a single CWS is maintained, HARQ-ACK of PUSCH on a previous UL BWP can be used for CWS adjustment for current active UL BWP. In the example of Figure 3, when UL BWP switches from BWP2 to BWP3, CWS for BWP3 adjusts according to HARQ-ACK of BWP2. If separate CWS is maintained for each UL BWP, further discussions are needed to determine when a previous UL BWP and current active UL BWP are overlapped, whether HARQ-ACK of a PUSCH in the previous UL BWP can be the reference for CWS adjustment on current active UL BWP. For example, in Figure 3, when UL BWP switches from BWP2 to BWP1, whether HARQ-ACK of PUSCH in BWP2 can be taken into account to adjust CWS for BWP1. The proper mechanism should be defined with the consideration of fair coexistence with Wi-Fi/LTE LAA nodes.

Figure 3. An example of overlapped and non-overlapped BWPs for a UE.

Proposal 16: CWS management for different UL BWPs should be defined.

# UE to gNB COT sharing

Another remaining issue for NR-U LBT design is on the applicability of COT sharing from UE to gNB. It has been agreed in [1] that a UE-initiated COT obtained through CAT-4 LBT can be shared with its serving gNB, as long as the downlink transmission that shares the UE-initiated COT includes DL transmissions intended to the UE that obtains the COT.

One remaining issues is the restriction of the DL transmission duration X ms (X >= 0) that is allowed to follow the CAT-1 LBT after the first switch between the UE and the gNB. The transmission duration X ms should not be limited as long as the downlink transmission is within the remaining of the UE-initiated COT and that the conditions to allow UE to gNB COT sharing are met.

Proposal 17: gNB can transmit in the remaining UE-initiated COT after first switch from UE to gNB with CAT-1 LBT.

There may exist multiple UEs associated with the same gNB and have passed the CAT-4 LBT, wherein these UEs may occupy different LBT BWs or BWPs. Under such scenario, it is worth discussing the gNB behavior for PDCCH transmission in the shared COT(s). Specifically, a baseline option is to allow gNB to transmit PDCCH intended to a UE only within the UE's LBT bandwidth(s) wherein the UE succeeded in CAT-4 LBT. On the other hand, a second option is to allow gNB to transmit PDCCH on any of the LBT bandwidths of the shared COTs to any of the UEs that have passed CAT-4 LBT. The second option provides better scheduling flexibility, and may release DL LBT BWs by transmitting PDCCH intended to multiple UEs in the same DL LBT BW(s). The allowance of the second option for UE to gNB COT sharing can be further discussed.

In addition, it has been agreed in [1] that for UL transmission, UE transmits on the UL only if LBT is successful at the UE in all LBT bandwidths that overlap with the resource allocated for the UL transmission. Therefore, from a frequency-domain perspective for the UE to gNB COT sharing, since the UE has succeed LBT in all LBT bandwidths that overlap with the scheduled UL resources, the gNB can share the UE-initiated COT over all of the LBT bandwidths that overlap with scheduled UL resources (for which the UE CAT-4 LBT is successful) for downlink transmission.

Proposal 18: gNB can share UE-initiated COT over all of the LBT bandwidths that overlap with scheduled UL resources for downlink transmission.

# Channel Access for FBE-based NR-U

Besides the load-based equipment (LBE) operation mode, NR-U can also support the FBE operation mode as discussed in [2]. The FBE operations as in the ETSI BRAN specifications can be the baseline for channel access of FBE-based NR-U, wherein the gNB performs a single-shot LBT immediately before starting transmission on the operating channel at the start of a fixed frame period.

It has been agreed in [1] that gNB can operate as an initiating device for FBE operation, in which case the UE transmission within a fixed frame period (FFP) can occur if DL signals/channels (PDCCH, SSB, PBCH, RMSI, GC-PDCCH,..) within the FFP are detected. For RRC_IDLE FBE UEs, the UE can also determine if gNB has obtained the COT of current FFP through monitoring and detecting for PDCCH with DCI format 1_0 with CRC scrambled by P-RNTI or the associated paging message.

In addition to allowing gNB as the initiating device for FBE operation, UE may also be allowed to be the initiating device for FBE operation under special conditions. For example, UE can operate as the initiating device for FBE operation only if the UE can determine its serving gNB has failed to obtain the current COT, and that non-scheduled UL transmissions are configured (e.g., CG-PUSCH or PRACH) on current COT. When UE operates as the initiating device, the signaling to enable/disable the FBE operation and corresponding parameters for FBE operation (e.g., fixed frame period and starting position) of the UE can be configured explicitly through dedicated RRC parameters.

Proposal 19: FBE UE can be the initiating device for FBE operation only if gNB has failed to obtain the COT and non-scheduled UL transmissions are configured on current COT.

For the baseline FBE channel access scheme, when one or multiple synchronized FBE-based NR-U operators coexist in the operating channel, the channel access can always be successful since the gNBs perform LBT at the same time. As a result, despite the spec impacts for synchronized FBE NR-U are minimal, strong interference may exist between neighboring gNBs during the COT.

For asynchronous FBE NR-U, LBT performed by a gNB may always be blocked by the transmission from a nearby asynchronous gNB. The above scenarios can be alleviated by supporting directional LBT and directional transmissions by the gNB. For example, LBT by a gNB is less likely to be blocked by neighboring asynchronous gNB when their beam directions are not aligned. As a result, spatial reuse can be improved when directional LBT is supported for FBE-based NR-U.

Another enhancement for asynchronous FBE NR-U with potential spec impacts is to introduce a window of L (L>1) observation slots consecutively allocated in time-domain. As shown in Figure 4, gNB 1 and gNB 2 can randomly select one observation slot within the observation slot window, wherein the observation slot window is within the idle period and the device can reserve the channel if LBT passes on its observation slot. The enhancement as shown in Figure 4 is subject to regulation allowance that transmission can start within the idle period; otherwise, the observation slot window can be shifted to the start of the fixed frame period.

Figure 4.  Random CCA slots within idle period.

Alternatively, each gNB chooses a random number M before accessing the unlicensed channel. Then, the maximum number of the fixed frame periods that gNB can grab before entering “muting” periods is set to M. If gNB grabs the channel for M fixed frame periods, it chooses a random number N and will not access the channel for N fixed frame periods. During such muting periods, neighbour gNBs that have failed to grab the channel due to the biased CCA timing can get channel access opportunities so that fairness among multiple gNBs can be improved as shown in Figure 5.

Figure 5. One example of modified FBE for resolving the biased channel access.

Proposal 20: Support directional LBT and/or introduce random observation slot within idle period and/or random muting period for asynchronous FBE NR-U to improve spatial reuse.

# Directional LBT for NR-U

To support the multi-beam operation for NR-U FR1, the directionality is also an important design consideration for the LBT operation. The baseline option to perform LBT omni-directionally by the potential transmitter can provide good coexistence with the incumbent systems, such as when NR-U operates in 5 GHz unlicensed bands or when NR-U coexists with another Wi-Fi network.

Another option to cope with directional transmissions of NR-U and improve spatial reuse is for the potential transmitter to perform a directional LBT over its intended transmit beam direction. This option can be at least used for NR-U and NR-U coexistence, and for the 6 GHz unlicensed bands as well as unlicensed bands with carrier frequency higher than 52.6 GHz.

Compared to the baseline omni-directional LBT, directional LBT can lead to better channel access probability and correspondingly better spatial reuse for NR-U/NR-U coexistence under the same ED threshold. The channel access gain for directional LBT will further outweigh its potential SINR performance loss compared to omni-directional LBT, which correspondingly will lead to a better throughput performance than the omni-directional LBT. In particular, the throughput gain under directional LBT compared to omni-directional LBT is observed to increase as traffic arrival rate increases for NR-U/NR-U coexistence.

Proposal 21: Directional LBT can be supported at least for NR-U/NR-U coexistence.

# LBT with Handshake Mechanism for NR-U

In general, a potential transmission in the unlicensed band can be granted channel access if the following two conditions are met: (1) the receiver(s) of the potential transmission is ready to receive; and (2) the potential transmission will not cause strong interference to the receivers of ongoing transmissions. Condition (1) ensures that the potential transmission can have enough SINR and can avoid the hidden node issue, while condition (2) ensures that the potential transmission can provide fair coexistence with ongoing unlicensed transmissions. LBT schemes that are performed at the potential transmitter side can only approximately satisfy both conditions.

A general approach for NR-U transmissions to satisfy both condition (1) and condition (2) is through supporting a configurable handshake mechanism between gNB and UE, on top of the LBT schemes discussed in Section 2 for data transmissions. It has been discussed in [2] that handshaking between transmitter and receiver is a possible means to reduce the impact of hidden nodes for NR-U, and that detailed solutions for the handshaking can be considered.

The handshake between NR-U gNB and UE can be including an exchange of a channel access request (CARQ) and a channel access response (CARP). Specifically, CARQ and CARP can be message-based, wherein the CARQ can be transmitted by the potential transmitter upon successful LBT for data transmission; and the CARP can be transmitted by the receiver upon receiving CARQ and after successful LBT (e.g., single-shot LBT). The CARQ (CARP) message can include the following information: the indicator for CARQ (CARP), the transmitter (receiver) identity, transmitter (receiver) beam direction information, the maximum channel occupancy time (MCOT), etc. Figure 6 illustrates an example of the LBT with handshake mechanism for NR-U downlink transmission.

Figure 6. Example of LBT with CARQ/CARP exchange for NR-U downlink transmission.

For a potential NR-U transmitter and receiver pair with the handshake mechanism, the transmission can be granted if (1) the LBT for data transmission at the transmitter side is successful; (2) the CARQ/CARP message exchange between the transmitter and receiver is successful; and optionally that (3) through monitoring the CARQ/CARP messages from other NR-U transmitter/receiver pairs, the potential NR-U transmitter can determine that its transmission will not cause strong interference to existing transmissions. Figure 7 illustrates an example where gNB1 is transmitting to UE1; and gNB2 intends to transmit to UE2. Despite gNB2 can succeed in the directional LBT scheme, gNB2 can decide to not transmit to UE2, since UE2 will fail to respond the CARP addressed to gNB2 and/or gNB2 can receive the CARP message from UE1 that is addressed to gNB1. Therefore, the handshake mechanism can be combined with the LBT scheme for NR-U to achieve fair and efficient channel access with the coexisting radio access technologies (RATs), which can be beneficial for both omni-directional and directional LBT schemes.

Figure 7. Example of addressing hidden node issue with handshake mechanism.

Proposal 22: NR-U can support LBT with handshake mechanism through the exchange of a CARQ message transmitted by the potential transmitter upon successful LBT for data transmission; and a CARP message transmitted by the receiver upon receiving CARQ and after a successful LBT.

# Remaining Issues on NR-U channel access

## Max number of CAT-2 LBT attempts in a 5ms DRS transmission window

In LTE-LAA, Cat2 LBT can be applied to the transmission of DRS, with a granularity of starting location as 1 ms. For example, every subframe in the configured DRS transmission window can be utilized for DRS transmission, and Cat2 LBT is performed right before the subframe boundary for DRS.

In NR-U, the granularity of starting position for DRS transmission should be at least the same as the one in LTE-LAA for fair coexistence. If NR-U uses a smaller granularity than LTE-LAA, there are scenarios that DRS in LTE-LAA cannot be transmitted at any time, and the more aggressive channel access mechanism in NR-U does not fair coexist with LTE-LAA on the 5 GHz unlicensed spectrum.

One example for this issue is shown in Figure 8. If both NR-U and LTE-LAA implemented on the same band, NR-U has much higher channel access opportunity for DRS transmission comparing to LTE-LAA, if using a granularity of starting location smaller 1 ms.

Figure 8. Example of unfair coexistence with LTE-LAA when using small granularity for starting location of DRS transmission.

Proposal 23: The granularity of the starting locations for DRS transmission in a DRS transmission window should be 1 ms.

## Max number of CAT-2 LBT attempts for UL burst

One remaining NR-U LBT design consideration is the number of LBT attempts allowed to grant the transmission of an UL burst within a gNB-initiated COT [2]. Specifically, the UL burst transmission can start after a CAT-2 LBT is successful at the UE, otherwise the UE can perform a new CAT-2 LBT attempt. In order to increase the opportunity to transmit the UL burst, such CAT-2 LBT attempt can always be allowed, as long as the UL transmission can start within the COT and the total number of CAT-2 LBT attempts does not exceed a pre-configured number M >= 1. In this case, the maximum number of such CAT-2 LBT attempts can be determined as min(ceil(T/τ), M), wherein T can be the duration of the UL burst, and τ refers to the NR-U OFDM symbol duration.

Proposal 24: CAT-2 LBT can be used for an UL burst within gNB-initiated COT as long as UL burst can start within the COT and the total number of LBT attempts does not exceed a pre-configured number.

# Conclusions

This contribution considered the overall design considerations for channel access mechanisms in NR unlicensed spectrum and proposes the following.

Proposal 1: C2/C3=1 for 15 and 30KHz SCS, and C2/C3=2 for 60KHz SCS.

Proposal 2: If CAT-4 LBT can be used for all CP extension durations, separate indication of the CP extension duration, the LBT type, and the CAPC are preferred. Otherwise, joint indication can be considered.

Proposal 3: For multi-PUSCH scheduling, if Cat-2 16us LBT is indicated by the UL grant, UE can perform Cat-2 16us LBT for the first scheduled PUSCH and switch to Cat-2 25us LBT for next PUSCH transmission if UE fails to access the channel for first scheduled PUSCHs.

Proposal 4: A reference DL transmission burst can be the most recent burst wherein at least some HARQ-ACK feedbacks corresponding to the reference duration of this burst is expected to be available when gNB adjusts CWS.

Proposal 5: NR-U gNB can adjust CWS based on a reference duration more than once.

Proposal 6: NR-U gNB can adjust CWS only based on the HARQ-ACK feedbacks corresponding to the reference duration.

Proposal 7: CWS is increased to next available value if at least 80% of the TBs transmitted in the reference duration are determined as NACK; and a TB with CBG-based feedback is determined as NACK if the HARQ-ACK value for all (re)transmitted CBGs within this TB is NACK.

Proposal 8: By the timing that gNB performs the CWS adaptation, missed/unavailable HARQ-ACK of reference duration can be not counted towards the current CWS adaptation decision.

Proposal 9: Reuse the RRC configured minimum duration D between PUSCH and DFI carrying HARQ-ACK for that PUSCH for the minimum latency between reference duration and the following UL grant or DFI  for CWS adaption.

Proposal 10: If the signals/channels without HARQ-ACK are transmitted alone or multiplexed only with other non-unicast data within a same transmission burst, the CWS corresponding to the CAT-4 LBT priority class of such signals/channels can remain unchanged.

Proposal 11: CWS for PRACH LBT can increase to the next available value if RAR is not received within the RAR monitoring window; otherwise, the CWS for PRACH LBT can reset to the minimum value.

Proposal 12: CWS for Msg2 with CAT-4 LBT can reset to the minimum value if correct Msg3 corresponding to the transmitted Msg2 is received; otherwise CWS for Msg2 increases to the next available value.

Proposal 13: CWS for Msg3 with CAT-4 LBT can reset to the minimum value if correct Msg4 corresponding to the transmitted Msg3 is received; otherwise CWS for Msg3 increases to the next available value.

Proposal 14: For 2-step RACH, CWS for MsgA PRACH+PUSCH LBT can increase to the next available value if MsgB corresponding to the transmitted MsgA is not received within the RAR monitoring window; CWS for MsgA PUSCH transmission can remain unchanged if fallbackRAR is received and can reset to the minimum value if successRAR is received.

Proposal 15: A single CWS can be maintained across LBT bandwidths for NR-U wideband operation similar to type B1 of LTE-LAA multi-carrier LBT.

Proposal 16: CWS management for different UL BWPs should be defined.

Proposal 17: gNB can transmit in the remaining UE-initiated COT after first switch from UE to gNB with CAT-1 LBT.

Proposal 18: gNB can share UE-initiated COT over all of the LBT bandwidths that overlap with scheduled UL resources for downlink transmission.

Proposal 19: FBE UE can be the initiating device for FBE operation only if gNB has failed to obtain the COT and non-scheduled UL transmissions are configured on current COT.

Proposal 20: Support directional LBT and/or introduce random observation slot within idle period and/or random muting period for asynchronous FBE NR-U to improve spatial reuse.

Proposal 21: Directional LBT can be supported at least for NR-U/NR-U coexistence.

Proposal 22: NR-U can support LBT with handshake mechanism through the exchange of a CARQ message transmitted by the potential transmitter upon successful LBT for data transmission; and a CARP message transmitted by the receiver upon receiving CARQ and after a successful LBT.

Proposal 23: The granularity of the starting locations for DRS transmission in a DRS transmission window should be 1 ms.

Proposal 24: CAT-2 LBT can be used for an UL burst within gNB-initiated COT as long as UL burst can start within the COT and the total number of LBT attempts does not exceed a pre-configured number.

# References

[1] RAN1 Chairman’s Note, 3GPP TSG RAN WG1 Meeting #98bis, Chongqing, China, Oct 14th – 20th, 2019.

[2] TR 38.889, Study on NR-based Access to Unlicensed Spectrum (Release 16), Dec. 2018.