3GPP TSG RAN WG1 Meeting #99     																	           R1-1913600

Reno, USA, 18th November– 22nd November 2019

Agenda Item:		7.2.3

Source:				WI Rapporteur (Qualcomm Incorporated)

Title: 					RAN1 agreements for Rel-16 IAB

Document for:		Information

### Introduction

This contribution provides the RAN1 decisions made for the Rel-16 WI on Integrated Access and Backhaul for NR (WI code NR_IAB; WID in RP-192188 [1]), up to and including RAN1#99.

The WI objectives are [1]:

- Specification of SSB/RMSI periodicity for NR initial access assumed by an IAB-node.

- Specification of extensions to Rel. 15 to support the use of SSBs orthogonal to SSBs used for UEs (via TDM and/or FDM), for inter-IAB-node discovery and measurements, including additional SMTC periodicities and time-domain mapping of SSB locations (e.g. enable muting patterns to deal with half-duplex constraint).

- Specification of extension of RACH occasions and periodicities for backhaul RACH resources. w.r.t. access RACH resources, and associated network coordination mechanisms for selection of such parameters (in order to orthogonalize access and BH due to the half-duplex constraints)

- Specification of mechanisms for resource multiplexing among backhaul and access links. This includes:

  - Specification of semi-static configuration for IAB-node/IAB-donor DU resources in case of TDM operation subject to half-duplex constraint. This shall be forward compatible to allow the support of half-duplex scenarios with FDM and SDM resource sharing among backhaul and access links.

  - Specification of time resource types for the DU’s child links: DL hard, DL soft, UL hard, UL soft, Flexible hard, Flexible soft, Not Available

  - Specification of dynamic indication (L1 signalling) of the availability of soft resources for a child IAB-node DU

  - Specification of required transmission/reception rules for IAB-nodes and associated behaviours regarding time resource utilization as discussed in TR 38.874 clause 7.3.3.

- Specification of mechanism to support the “case-1” OTA timing alignment.

### SSB/RMSI periodicity for NR initial access

RAN1 #AH1901 (Taipei, January 2019) [2]

Feature Lead summary R1-1901367

Agreements:

- There is no need to introduce any specification change to enable longer RMSI transmission periodicities.

Feature Lead summary R1-1901400

Agreements:

- For both SA and NSA, for initial cell selection, an IAB-node MT may assume that half frames with SS/PBCH blocks occur with a periodicity of 16 frames.

  - FFS cell re-selection

  - Study further the value of assumed SS/PBCH block periodicity for a backhaul link switching to a new parent node when the link to the current parent node fails.

RAN1 #96 (Athens, February 2019) [3]

Feature Lead summary R1-1903573

Agreements:

- Existing NR mechanisms are used by the network to signal to IAB MTs the SSB periodicity for cell re-selection.

- The IAB-node MT initial access assumption that half frames with SS/PBCH blocks occur with a periodicity of 16 frames does not have an impact on cell re-selection.

Agreements:

- The IAB-node MT initial access assumption that half frames with SS/PBCH blocks occur with a periodicity of 16 frames does not have an impact on backhaul link rerouting to a new parent node in response to a failure in the link to the current parent node.

LS to RAN2/3 capturing agreements from this meeting and RAN1 AdHoc1901 – R1-1903809

### Extensions of SSBs for inter-IAB-node discovery and measurements

RAN1 #AH1901 (Taipei, January 2019) [2]

Feature Lead summary R1-1901369

Agreements:

- SSB transmission configuration information (i.e. the necessary information for the DU to determine when to transmit SSB) and SSB reception configuration information (i.e. the necessary information for the MT to determine when to search for SSB) for IAB inter-node measurements are determined in a centralized manner, i.e. not locally by the IAB node.

- An IAB node is provided with configuration of SSB transmission and reception information for IAB inter-node measurements.

  - FFS details

RAN1 #96 (Athens, February 2019) [3]

Feature Lead summary R1-1903574

Agreements:

- The SSBs for IAB inter-node discovery and measurements are defined with a framework using the characteristics of the Rel 15 SMTC framework with the following enhancements:

  - The maximum number of SMTC windows that can be configured for an IAB node, referred as max( NRX ), is at least 3

    - FFS whether max( NRX ) needs to be greater than 3

    - FFS whether or not to extend existing SMTC window configuration

  - Each SMTC window is provided its own independent configuration (e.g. periodicity, offset, duration)

  - Introduction of SSB transmission configurations (STC) indicating SSB transmission(s)

    - FFS details

  - FFS how to resolve potential collision between SMTC and STC

RAN1 #96bis (Xi’an, April 2019) [4]

Feature Lead summary R1-1905832

Agreements:

- The Rel-15 SSB mapping pattern within a half frame is reused for IAB node discovery and measurements

Agreements:

For IAB node discovery and measurements, at least the following information should be provided in STC

- SSB center frequency

- SSB subcarrier spacing

- SSB transmission periodicity

  - 5, 10, 20, 40, 80, 160, 320, 640 subframes are supported.

- SSB transmission timing offset in half frame(s)

- The index of SSBs to transmit

  - The SSBs to be transmitted in the half frame

- FFS configuration information other than above

- FFS some of the parameters can have default values, e.g. reuse the same value of the cell-defining SSBs.

- Note: The detailed signaling design is up to RAN3.

Agreements:

For IAB node discovery and measurements, an IAB node DU can be configured with zero, one or multiple STCs.

- FFS the maximum number of STCs

- FFS the dependency of multiple STCs

Agreements:

- For IAB node discovery and measurement, the maximum number of SMTC windows that can be configured for an IAB node is 4.

RAN1 #97 (Reno, May 2019) [5]

Feature Lead summary R1-1907834

Agreements:

All parameters in STC for inter-IAB-node discovery and measurement are provided explicitly, i.e. the default values for the parameters in STC will not be discussed further in RAN1.

Agreements:

The configurable values of the parameters in STC for IAB node discovery and measurement are provided in the following

- SSB center frequency:

  - ARFCN-ValueNR

- SSB subcarrier spacing:

  - FR1: 15khz, 30khz

  - FR2: 120khz, 240khz

- SSB transmission periodicity:

  - 5ms, 10ms, 20ms, 40ms, 80ms, 160ms, 320ms, 640ms (agreed in RAN1 #96bis)

- SSB transmission timing offset in half frame(s)

  - [0, … , (number of half frames within SSB transmission periodicity) – 1]

- The index of SSBs to transmit (the SSBs to be transmitted in the half frame)

  - Same as Rel-15

- FFS additional parameter(s) other than above

Agreements:

For IAB node discovery and measurement, the maximum number of STCs that can be configured for an IAB node DU per cell at one frequency location is 4.

- Note: this number does not include the cell-defining SSBs for initial access

Agreements:

Each STC is independently configured when multiple STCs are configured.

- Up to RAN3 to decide that if SSB index is common across two or more STC configurations, whether additional signalling optimization is necessary or not

Agreements:

The configurable values of the parameters in the SSB reception configurations including SMTC for IAB node discovery and measurement are provided in the following

- SMTC window periodicity:

  - 5ms, 10ms, 20ms, 40ms, 80ms, 160ms, 320ms, 640ms, 1280ms

- SMTC window timing offset:

  - [0, ..., (number of subframes within SMTC window periodicity) – 1]

- SMTC window duration:

  - [1, …, 5] (subframes)

    - FFS larger than 5

- List of physical cell IDs to be measured

  - [cell ID 0, …, cell ID M-1 ]

    - FFS value of M

- SSB-ToMeasure

  - Same as Rel-15

RAN1 #98 (Prague, August 2019) [6]

Feature Lead summary R1-1909542

Agreements:

For IAB node discovery and measurements, the Physical cell-ID (0…1007) is provided in STC and the detailed signaling design is up to RAN3.

Agreements:

For IAB node discovery and measurement

  - The maximum SMTC window duration is 5ms.

  - The maximum number of PCIs per SMTC (maxNrofPCIsPerSMTC) is 64.

RAN1 #98bis (Chongqing, October 2019) [7]

Feature Lead summary R1-1911408

Agreements:

For inter-IAB node discovery and measurement:

  - The IAB node does not expect STC and SMTC are overlapped in time

Extension of RACH occasions and periodicities for backhaul RACH resources

RAN1 #AH1901 (Taipei, January 2019) [2]

Feature Lead summary R1-1901370

No decisions.

RAN1 #96 (Athens, February 2019) [3]

Feature Lead summary R1-1903575

Agreements:

- New RACH configurations specific to IAB nodes are derived with extension of existing Rel 15 RACH configurations obtained by:

  - scaling the parameter ‘x’ from the PRACH configuration table, and

  - by adding an offset y_offset to the parameter ‘y’ (frame-based offset) and/or adding an offset to the slot/subframe number (slot/subframe-based offset) from the PRACH configuration table.

- FFS values and ranges for scaling factor applicable to ‘x’ and for y_offset.

- FFS whether a simple extension rule can be generally applied to all existing configurations leaving it up to the network to not use resulting configurations that may not fit other system constraints.

RAN1 #96bis (Xi’an, April 2019) [4]

Feature Lead summary R1-1905738

Agreements:

The periodicity of a backhaul RACH configuration in frames takes the form x_iab = x * λ where:

- x is the periodicity of an existing RACH configuration,

- λ is a scaling factor taking values in {1, 2, 4, 8, 16, 32, 64} subject to the constraint x_iab ≤ 64.

Agreements:

The frame containing backhaul ROs is identified by (nSFN mod x_iab) = ((y + Δy) mod x_iab) where Δy denotes a time offset in frames taking values in the range from 0 to x_iab – 1.

Agreements:

The subframe (slot) number for a RO of a backhaul RACH configuration is identified by (Sn + Δs,) mod L, where:

- Sn is the subframe (slot) number of an existing RACH configuration,

- Δs denotes a time offset in subframes (slots) taking values in the range from 0 to L – 1, where L is the number of subframes (slots) in a frame.

NOTE: The usage of the terms subframe or slot is meant to align to the terminology used in the existing RACH configuration tables in TS 38.211, e.g. subframe in Tables 6.3.3.2-2 and 6.3.3.2-3, and slot in Table 6.3.3.2-4.

Agreements:

The validity of ROs for backhaul RACH configurations is regulated by the rules defined in Rel-15 for existing RACH configurations.

Agreements:

Partial overlap of ROs between RACH configurations used in two adjacent links (upstream towards the parent and downstream towards the children from an IAB node perspective) is allowed.

RAN1 #97 (Reno, May 2019) [5]

Feature Lead summary R1-1907680

Agreements:

The introduction of scaling factor for PRACH configuration period also scales the SSB-RACH association period and association pattern period accordingly.

Assuming the scaling factor to be λ, for IAB nodes table 8.1-1 of 38.213 gets modified to the following:

Mapping between scaled PRACH Configuration Period (x * λ) and SSB to PRACH Occasion Association Period

| Scaled PRACH configuration period (ms) | Association period |
| --- | --- |
| 10 | {1, 2, 4, 8, 16, 32, 64} |
| 20 | {1, 2, 4, 8, 16, 32} |
| 40 | {1, 2, 4, 8, 16} |
| 80 | {1, 2, 4, 8} |
| 160 | {1, 2, 4} |
| 320 | {1, 2} |
| 640 | {1} |

The definition of the SSB-RACH association pattern period for IAB nodes should be updated as follows: An association pattern period includes one or more association periods and is determined so that a pattern between PRACH occasions and SS/PBCH blocks repeats at most every 640 msec.

Agreements:

The signaling design for the IAB specific backhaul RACH configurations is up to RAN2.

RAN1 has the following recommendations:

- The ability to configure the IAB specific backhaul RACH configuration additionally to the Rel 15 RACH configuration for access UEs is supported.

  - If the IAB specific RACH configuration is not provided, RAN1 assumes that IAB node will use the configured Rel 15 RACH configuration for IAB node initial access.

- Capability to configure IAB specific backhaul RACH configurations need to be provided for both CBRA & CFRA.

RAN1 #98 (Prague, August 2019) [6]

Feature Lead summary R1-1909526

Agreements:

IAB-specific backhaul RACH configurations are also available for intial access of IAB nodes.

### Mechanisms for resource multiplexing among backhaul and access links

RAN1 #AH1901 (Taipei, January 2019) [2]

Feature Lead summary R1-1901455

Agreements:

At least existing resource definitions (D/U/F) and semi-static and dynamic signaling methods defined in Rel-15 for access UEs are reused for configuration and indication of MT resources to be used by the backhaul link between the IAB node and its parent.

- FFS details

Agreements:

IAB-node/IAB-donor DU resources are provided by a semi-static configuration which is provided separately from the MT resource indication

- FFS: whether the configuration is per-link or per-DU

- FFS: details for the configuration

Agreements:

Support indication of the dynamic availability of soft resources at an IAB node from a parent

- FFS details, including explicit vs. implicit indication

RAN1 #96 (Athens, February 2019) [3]

Feature Lead summary R1-1903582

Agreements:

- If a resource is configured as not available, the DU cannot assume it can use the resource

- In case of hard DU resources, the DU can assume it can use the resource regardless of the MT’s configuration

  - FFS: Exception cases for specific signals/channels to be transmitted or received by the MT in the same resource (e.g. SS/PBCH blocks, SI reception, RACH)

- In case of soft DU resources:

  - If the soft resource is indicated as available, the DU can assume it can use the resource

  - If the soft resource is not indicated as available, the DU cannot assume it can use the resource

  - The use of soft resources at least corresponds to transmission/reception of specific signals and channels (e.g. PDSCH/PUSCH) at the DU

    - FFS the use of soft resources in case of cell-specific (e.g. SS/PBCH blocks, SI reception, RACH) signals and channels to be potentially transmitted/received at the DU

  - Both implicit and explicit indication of the availability of soft resources at an IAB node is supported

    - In case of implicit indication of DU soft-resource availability, the IAB node knows that the DU resource can be used without impacting the MTs ability to transmit/receive according to its configuration and scheduling based on indirect means. Examples of such means may include:

      - the lack of uplink scheduling grant at the MT

      - no data available for MT transmission

      - the configured MT search space,

      - configured RS measurement occasions (e.g. SSB/CSI-RS)

      - FFS: consider whether the parent should be able to always be aware of/control the outcome of implicit indications at child nodes

    - Explicit indication that a resource is available is based on DCI indication. The following options can be considered:

      - SFI-like indication via DCI Format 2_0

      - Use of 2 SFI indications (e.g. based on multi-slot scheduling mechanism)

      - Define a new DCI format

      - Other options are not precluded

      - FFS: whether an explicit indication that a resource is available always has priority over any implicit determination of the availability of a resource

  - Further consider factors impacting the usage of soft resources at a child DU, including:

    - MT’s decoding delay

    - Information exchange delay between MT and DU

    - DU’s PDSCH preparation time

    - UE PUSCH preparation time

    - Accumulated delay across hops

Agreements:

- Inter-IAB node conflict resolution can be supported by the following options (to be down-selected)

  - Alt1: the parent node is aware of all of the DU resource configurations (D/U/F/H/S/NA) of its child IAB node DUs,

  - Alt2: the parent node may be made aware of a subset of the DU resource configurations (D/U/F/H/S/NA) of its child IAB node DUs

  - FFS: whether the indication of the child DU resources at the parent is via explicit (e.g. F1-AP signaling) or implicit (e.g. based on child MT configuration) means

Agreements:

- Both slot and symbol-level multiplexing of access and backhaul links are supported.

RAN1 #96bis (Xi’an, April 2019) [4]

Feature Lead summary R1-1905739

Agreements:

- In case of Hard or Soft Indicated Available DU resources, no additional exception cases need to be defined for cell specific signals/channels to be transmitted or received by the MT in the same resource (e.g. SS/PBCH blocks, SI reception, RACH).

  - The decision on whether to give priority to the DU or to the MT for the use of the resource (e.g. in case of MT RACH transmission) is left to the IAB node implementation.

    - The IAB shall fulfill its performance requirements in terms of measurement and transmission of cell specific signals / channels.

- In case of NA or Soft not Indicated Available DU resources, for potential conflicting configuration between NA and Soft not Indicated Available DU resources and cell-specific signals/channels (e.g. SSB, SI, PRACH) configured at the DU, the following alternatives are to be considered (to be down-selected):

  - Alt 1): A resource with cell-specific signals/channels configured at DU shall not be configured as type NA or Soft without being Indicated Available.

  - Alt 2): If a DU NA or Soft resource is configured with cell-specific signals/channels, the resource is treated as if it were a Hard DU resource.

- FFS the cell specific signals/channels. The list may include (not necessarily an exhaustive list)

  - SSB transmissions

  - Broadcast system information

  - Configured periodic CSI-RS

  - PRACH resources

  - Resources for scheduling requests

Agreements:

- An IAB node has the ability to be made aware of the semi-static DU resource configuration (D/U/F/H/S/NA) of all its child IAB nodes.

  - in the case the full DU resource configuration information of its child IAB nodes is not necessary, ensure that only the necessary configuration information is signaled to the IAB node.

    - FFS the necessary configuration information.

Agreements:

- An explicit indication of availability of a Soft resource by the parent node makes the resource available to the child node, irrespective of the outcome of any implicit determination of availability by the child node.

- The parent node does not need to be aware of the outcome of an implicit determination of availability of a DU Soft resources at a child node.

RAN1 #97 (Reno, May 2019) [5]

Feature Lead summary R1-1907679

Agreements:

For the semi-static DU resource configuration, the following is supported:

- The resources are configured on a per DU (cell) basis

  - FFS: indication of additional supplemental per-link resource configurations of child DUs

- Indication of D/U/F resources in the semi-static DU resource configuration includes the following:

  - The flexibility to configure all of the slot patterns and formats supported by the existing Rel-15 TDD-UL-DL-Config RRC configurations and slot format table defined in Table 11.1.1-1 in TS38.213

    - FFS: additional pattern durations than supported in Rel-15

    - FFS: default resources or pattern

  - New slot formats defined only for IAB nodes (DU and MTs) which begin with uplink slots, uplink symbols, or flexible symbols.

    - Note: usage of these slot formats should be compatible with Rel-15 access UEs sharing the same link

    - FFS: whether these slot formats also need to be included in the MT RRC configuration and/or SFI carried on DCI Format 2_0

- Indication of H/S/NA for the DU resource configuration is based on one of the following alternatives:

  - Alt. 1: H/S/NA is additionally explicitly indicated per-resource type (D/U/F) in each slot

    - To handle potential misalignment in time of the configured DU and MT resources when determining the validity of H/S/NA at the DU one of the following sub-alternatives need to be supported:

      - 1a: H/S/NA is applied relative to the DU resource configuration (D/U/F) slot timing without considering the MT resource configuration or timing.

        - FFS: definition of additional restrictions on the usage of the semi-static configuration (e.g. guard symbols) based on deployment scenario or DL/UL switching times within an IAB node, etc.

        - FFS: How the CU can get information about the required guard symbols for a given DU configuration if needed

      - 1b: H/S/NA is applied relative to the MT resource configuration (D/U/F) slot timing.

        - FFS: Whether and/or how the CU will know the actual H/S/NA resources at the child DU

        - FFS whether S is explicitly indicated or not

      - 1c: H is applied relative to the DU resource configuration (D/U/F) slot timing. S is not explicitly indicated, but implicitly determined by the DU based on whether the corresponding MT configuration indicates the MT resources is F (DU-S). The remaining resources are assumed to be NA at the child DU.

        - FFS: Whether and/or how the CU will know the actual S/NA resources at the child DU

  - Alt. 2: NA is explicitly indicated as a resource type in each slot for both the DU and MT configuration. H/S is not explicitly indicated, but implicitly determined by the DU based on the corresponding MT configuration

    - FFS: how to handle the case where there is not a 1-1 mapping of DUs and MTs in the child IAB node

- Details of the F1-AP signaling design is up to RAN3

Agreements:

If a DU NA or Soft resource is configured with cell-specific signals/channels, the resource is treated as if it were a Hard DU resource (Alt. 2 from RAN1#96bis).

  - The list of cell-specific signals/channels includes:

- resources for SSB transmission at DU, including both CD-SSB and non-CD-SSB;

- configured RACH occasions for receiving at the DU

- periodic CSI-RS transmission at the DU

- scheduled resource for receving SR at DU

- The parent does not need to be aware of the cell-specific signals/channel configurations of the child DU

RAN1 #98 (Prague, August 2019) [6]

Feature Lead summary R1-1909531

Agreements:

- In Rel-16 for IAB backhaul, only existing slot pattern durations are supported.

Agreements:

Slot formats with the sequence order UL-Flexible-DL are supported for IAB-node DU resource configurations (F1-AP) and IAB-node MT resource configurations provided in dedicated RRC signaling (e.g. enhancements to the existing TDD-UL-DL-SlotConfig in TDD-UL-DL-ConfigDedicated – details up to RAN2).

- FFS: Support of dynamic indication of UL-Flexible-DL

Agreements:

The H/S/NA attributes for the per-cell DU resource configuration should take into account the associated MT carrier frequency(ies).

- Note: RAN1 assumes that this is mainly needed for intra-band cases

Agreements:

To handle potential misalignment in time of the configured DU and MT resources when determining the validity of H/S/NA at the DU, the following is supported:

- H/S/NA is applied/determined relative to the DU resource configuration (D/U/F) slot timing without considering the MT resource configuration or timing

- The parent DU should know the information about the misalignment between the MT and DU DL Tx/Rx / UL Tx/Rx timing(s) in order to avoid resource type conflicts

  - FFS whether any explicit signalling is necessary or not

- It is up to the parent IAB node implementation and network configuration to handle any potential resource type conflicts (e.g. MT scheduling restrictions or configuration of guard symbols). The child IAB node should handle any remaining conflicts using the existing conflict resolution rules

Agreements:

For the explicit indication of DU-IA:

- At least DU-IA can be indicated independently of the MT resource configuration:

  - FFS the DCI indicates DU-IA per-resource type (D/U/F), slot, etc.

- FFS: Whether this indication is via a new DCI format or DCI Format 2_0 using existing and/or reserved entries of Table 11.1.1-1 in 38.213

- FFS: Whether for the purpose of signaling optimization a DU/MT resource type can additionally be jointly indicated with DU-IA which overrides the flexible resources in the semi-static DU/MT resource configuration

RAN1 #98bis (Chongqing, October 2019) [7]

Feature Lead summary R1-1911651

Agreements:

Dynamic indication of UL-Flexible-DL (intended to dynamically update the flexible symbols) is supported for IAB-node MTs

- The dynamic indication of slot formats of UL-Flexible-DL should use DCI format 2_0 and reserved entries in Table 11.1.1-1 in 38.213.

Agreements:

H/S/NA attributes for the per-cell DU resource configuration are explicitly indicated per-resource type (D/U/F) in each slot.

Agreements:

The donor CU and the parent node can be made aware of the multiplexing capability between MT and DU (TDM required, TDM not required) of an IAB node to for any {MT CC, DU cell} pair.

- Signaling details up to RAN2/RAN3.

Agreements:

The behavior for conflict resolution as defined in RAN1 #96bis applies in general and it is not an exception for when the MT is configured with cell specific signals/channels.

Agreements:

A parent IAB node can be made aware of the number of symbols Ng the child IAB node would like the parent IAB node not to use at the edge (beginning or end) of a slot when there is a transition between child MT and child DU. Separately or additionally, the child IAB node can be made aware of the number of guard symbols that the parent IAB node will provide.

- Ng can be provided for each of the [8] possible transitions with potential overlap:

| MT to DU | DL Tx | UL Rx |
| --- | --- | --- |
| DL Rx |  |  |
| UL Tx |  |  |
| DU to MT | DL Rx | UL Tx |
| DL Tx |  |  |
| UL Rx |  |  |

- If Ng is not provided it is assumed to be 0

NOTE: this agreement does not introduce any performance requirement on IAB nodes.

Agreements:

The minimum granularity of explicit indication of DU-IA for IAB DU is per resource type (D/U/F) in a slot.

Agreements:

The explicit indication of DU-IA is provided using the DCI Format 2_0 framework.

- FFS the following alternatives (to be down-selected):

  - Alt 1:  Reuse existing DCI Format 2_0, select/reinterpret current and, possibly, some reserved entries in Table 11.1.1-1 in 38.213 to indicate DU-IA.

  - Alt 2:  Reuse existing DCI Format 2_0, select reserved entries of Table 11.1.1-1 in 38.213 to indicate DU-IA.

  - Alt 3:  New DCI Format.

Agreements:

A DCI following the DCI Format 2_0 structure is used to indicate DU-IA to an IAB node using a new IA-RNTI different from SFI-RNTI.

This DCI contains one or multiple fields (similar to SFI-index fields in DCI Format 2_0), each field value is used as the index in a RRC configured AI (Availability Indicator) AvailabilityCombination table (similar to the SFI

SlotFormatCombination table).

Each entry in the AI AvailabilityCombination table indicates the resource availability for a set of consecutive slots.

Each element of each entry in the AI AvailabilityCombination table indicates the resource availability in a slot.

The resource availability can take 8 values:

| Value | Meaning |
| --- | --- |
| 0 | No resources available |
| 1 | D resources available |
| 2 | U resources available |
| 3 | D and U resources available |
| 4 | F resources available |
| 5 | D and F resources available |
| 6 | U and F resources available |
| 7 | All resources available |

The maximum number of entries in the AI AvailabilityCombination table is 512.

RAN1 #99 (Reno, November 2019) [8]

Feature Lead summary R1-1913503

Agreements:

The indication of the multiplexing capability for the case of no-TDM between IAB MT and IAB DU is additionally provided with respect to each transmission-direction combination (per MT CC/DU cell pair):

- MT-TX/DU-TX

- MT-TX/DU-RX

- MT-RX/DU-TX

- MT-RX/DU-RX

Note: This agreement does not require any additional specification impact in RAN1 in Rel-16, i.e. in Rel-16 the behaviour of the IAB node is only defined for TDM cases. The behaviour for no-TDM is left to IAB-node/network implementation in Rel-16.

Agreements:

A parent IAB node/donor can be provided with the full D/U/F + H/S/NA resource configuration of each child IAB-DU.

- Note: the new parameter for the signalling is already endorsed; the above agreements provide additional details for the signalling

- No additional signaling optimization is required from a RAN1 perspective

Agreements:

A parent IAB node/donor can be provided with cell-specific signals/channels configurations (as listed in the previous agreements copied below) of each child IAB-DU. How/whether to use the information to handle any potential conflict at the parent IAB node/donor is left to network implementation

  - NOTE: This overturns the last sub-bullet of the following RAN1#97 agreement:

Agreements:

If a DU NA or Soft resource is configured with cell-specific signals/channels, the resource is treated as if it were a Hard DU resource (Alt. 2 from RAN1#96bis).

  - The list of cell-specific signals/channels includes:

- resources for SSB transmission at DU, including both CD-SSB and non-CD-SSB;

- configured RACH occasions for receiving at the DU

- periodic CSI-RS transmission at the DU

- scheduled resource for receving SR at DU

- The parent does not need to be aware of the cell-specific signals/channel configurations of the child DU

Agreements:

For IAB-MTs, the following new slot formats are defined and indicated by SFI in DCI Format 2_0:

| U | U | U | U | U | U | U | U | U | U | U | U | U | F |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| U | U | U | U | U | U | U | U | U | U | U | U | F | F |
| U | U | U | U | U | U | U | U | U | U | U | F | F | F |
| U | U | U | U | U | U | U | U | U | U | F | F | F | F |
| U | U | U | U | U | U | U | U | U | F | F | F | F | F |
| U | U | U | U | U | U | U | U | F | F | F | F | F | F |
| U | U | U | U | U | U | U | F | F | F | F | F | F | F |
| U | U | U | U | U | U | F | F | F | F | F | F | F | F |
| U | U | U | U | U | F | F | F | F | F | F | F | F | F |
| U | U | U | U | F | F | F | F | F | F | F | F | F | F |
| U | U | U | F | F | F | F | F | F | F | F | F | F | F |
| U | U | F | F | F | F | F | F | F | F | F | F | F | F |
| U | F | F | F | F | F | F | F | F | F | F | F | F | F |
| U | F | F | F | F | F | F | F | F | F | F | F | F | D |
| U | U | F | F | F | F | F | F | F | F | F | F | F | D |
| U | U | U | F | F | F | F | F | F | F | F | F | F | D |
| U | F | F | F | F | F | F | F | F | F | F | F | D | D |
| U | U | F | F | F | F | F | F | F | F | F | F | D | D |
| U | U | U | F | F | F | F | F | F | F | F | F | D | D |
| U | F | F | F | F | F | F | F | F | F | F | D | D | D |
| U | U | F | F | F | F | F | F | F | F | F | D | D | D |
| U | U | U | F | F | F | F | F | F | F | F | D | D | D |
| U | U | U | U | U | U | U | U | U | U | U | U | F | D |
| U | U | U | U | U | U | U | U | U | U | U | F | F | D |
| U | U | U | U | U | U | U | U | U | U | F | F | F | D |
| U | U | U | U | U | U | U | U | U | U | U | F | D | D |
| U | U | U | U | U | U | U | U | U | U | F | F | D | D |
| U | U | U | U | U | U | U | U | U | F | F | F | D | D |
| U | F | D | D | D | D | D | D | D | D | D | D | D | D |
| U | U | F | D | D | D | D | D | D | D | D | D | D | D |
| U | U | U | F | D | D | D | D | D | D | D | D | D | D |
| U | F | F | D | D | D | D | D | D | D | D | D | D | D |
| U | U | F | F | D | D | D | D | D | D | D | D | D | D |
| U | U | U | F | F | D | D | D | D | D | D | D | D | D |
| U | F | F | F | D | D | D | D | D | D | D | D | D | D |
| U | U | F | F | F | D | D | D | D | D | D | D | D | D |
| U | U | U | F | F | F | D | D | D | D | D | D | D | D |
| U | U | U | U | U | U | U | U | U | F | F | F | F | D |
| U | U | U | U | U | U | F | F | F | F | F | F | D | D |
| U | U | U | U | U | U | F | F | D | D | D | D | D | D |
| U | U | U | U | U | U | U | D | D | D | D | D | D | D |

Agreements:

Update the parameter TDD-UL-DL-Config-IAB-MT to TDD-UL-DL-ConfigDedicated-IAB-MT

If the IAB-MT is additionally provided TDD-UL-DL-ConfigDedicated-IAB-MT, the parameter TDD-UL-DL-ConfigDedicated-IAB-MT overrides all symbols (with a limitation that effectively only flexible symbols can be overwritten in Rel-16) per slot over the number of slots as provided by TDD-UL-DL-ConfigurationCommon.

The TDD-UL-DL-ConfigDedicated-IAB-MT provides

-  a set of slot configurations by slotSpecificConfigurationsToAddModList-IAB-MT

-  for each slot configuration from the set of slot configurations

-  a slot index for a slot provided by slotIndex

-  a set of symbols for a slot by symbols where

-   if symbols = allDownlink, all symbols in the slot are downlink

-   if symbols = allUplink, all symbols in the slot are uplink

-   if symbols = explicit, nrofDownlinkSymbols provides a number of downlink first symbols in the slot and nrofUplinkSymbols provides a number of uplink last symbols in the slot. If nrofDownlinkSymbols is not provided, there are no downlink first symbols in the slot and if nrofUplinkSymbols is not provided, there are no uplink last symbols in the slot. The remaining symbols in the slot are flexible

-   if symbols = explicit-[IAB-MT], nrofUplinkSymbols provides a number of uplink first symbols in the slot and nrofDownlinkSymbols provides a number of downlink last symbols in the slot. If nrofUplinkSymbols is not provided, there are no uplink first symbols in the slot and if nrofDownlinkSymbols is not provided, there are no downlink last symbols in the slot. The remaining symbols in the slot are flexible.

For each slot having a corresponding index provided by slotIndex, the IAB-MT applies a format provided by a corresponding symbols.

Agreements:

Desired Guard Symbols and Provided Guard Symbols are provided per cell and use 3 bits for each of the 8 transitions to indicate the number of guard symbols.

- In Rel-16, a range of 0-4 symbols are supported for each transition. Additional entries are reserved for future use

- A new parameter GuardSymbol-SCS is also provided which indicates the reference SCS (FR1: {15kHz, 30kHz, 60kHz}, FR2: {60kHz, 120kHz}) to be used for the guard symbols.

Agreements:

The minimum granularity of explicit indication of DU-IA is per child IAB-DU resource type (D/U/F) in a slot.

- This is an update to the agreement from RAN1#98bis:

Agreements:

The minimum granularity of explicit indication of DU-IA for IAB DU is per resource type (D/U/F) in a slot.

Agreements:

The resource availability indicated by an AI index field value applies to Soft resources configured for the child IAB-DU in a number of slots, where the number of slots is equal to or larger than a PDCCH monitoring periodicity for DCI format 4_0.

Agreements:

Update the agreement from RAN1#98bis as the following:

The resource availability can take 8 values:

| Value | Meaning |
| --- | --- |
| 0 | D soft resources: no indication of availability U soft resources: no indication of availability F soft resources: no indication of availability |
| 1 | D soft resources: indicated available U soft resources: no indication of availability F soft resources: no indication of availability |
| 2 | D soft resources: no indication of availability U soft resources: indicated available F soft resources: no indication of availability |
| 3 | D soft resources: indicated available U soft resources: indicated available F soft resources: no indication of availability |
| 4 | D soft resources: no indication of availability U soft resources: no indication of availability F soft resources: indicated available |
| 5 | D soft resources: indicated available U soft resources: no indication of availability F soft resources: indicated available |
| 6 | D soft resources: no indication of availability U soft resources: indicated available F soft resources: indicated available |
| 7 | D soft resources: indicated available U soft resources: indicated available F soft resources: indicated available |

Agreements:

Adopt the following regarding the IAB behaviour as it relates to soft DU resources:

The IAB node may use a soft DU resource for transmission and/or reception in line with D/U/F configuration of the resource, if and only if

- the use of the resource does not change the IAB-MT behavior relative to the IAB-MT behavior if the resource was instead configured as unavailable type

or

- the IAB node has received a DCI with DCI format 4_0 indicating that the resource is available

Conclusion:

It is clarified that the IAB-DU-Resource-Configuration-TDD-Config has a periodicity of:

ENUMERATED {ms0p5, ms0p625, ms1, ms1p25, ms2, ms2p5, ms5, ms10}

Agreements:

The IAB-DU-Resource-Configuration-H/S/NA-Config has a periodicity of:

ENUMERATED {ms0p5, ms0p625, ms1, ms1p25, ms2, ms2p5, ms5, ms10, ms20, ms40, ms80, ms160}

Conclusion:

Default values for IAB-DU-Resource-Configuration-TDD-Config and IAB-DU-Resource-Configuration-H/S/NA-Config (if needed) are left to RAN3 to decide (update the Upper Layer Parameters list with this as a note).

Agreements:

If the IAB node receives multiple DCI Format 4_0 indicating resource availability for a given slot, the IAB node does not expect to receive different AI index fields for a given slot in Rel-16.

### Mechanism to support the “case-1” OTA timing alignment

RAN1 #AH1901 (Taipei, January 2019) [2]

Feature Lead summary R1-1901417

Agreements:

An IAB node should set its DL TX timing ahead of its DL Rx timing by TA/2 + T_delta

- T_delta is signalled from the parent node, where the value is intended to account for factors such the offset between parent DL Tx and UL Rx, if any due to factors such as Tx to Rx switching time, HW impairments, etc.

- TA is the timing gap between UL Tx timing and DL Rx timing, which is derived based on existing Rel-15 mechanism

- FFS (not necessarily an exhaustive list):

  - value range and granularity of Tdelta

  - need for aperiodic/periodic updates of Tdelta

  - other timing impairment factors for adjusting IAB node timing to be included in Tdelta

  - timing alignment when the IAB node has multiple parents

  - Note: once the design of the above FFS points is in a good shape, an LS to RAN4 may be necessary to solicit their input

RAN1 #96 (Athens, February 2019) [3]

Feature Lead summary R1-1903583

Agreements:

- T_delta is indicated by a parent to the child node independently from the existing Rel.15 TA indication from the parent node used to set the UL Tx timing of the child IAB node’s MT

  - T_delta is updated on an aperiodic basis determined by the parent node

  - The child IAB node should trigger its DL TX timing adjustment by TA/2 + T_delta after it receives the timing offset T_delta indication from its parent node, if it is using OTA Timing Case 1 to obtain its DL timing.

    - FFS: behavior if TA/2 + T_delta results in an effective negative timing offset

    - FFS: delay between receiving T_delta and application of T_delta at the child node

  - Separate value ranges/granularities may be considered for T_delta in FR1 and T_delta in FR2

- Send LS to RAN4 asking them to determine the exact values and granularity of T_delta and provide confirmation on RAN1’s assumption on the DL timing accuracy requirements for IAB nodes in case of OTA Case 1 timing is applied across multiple hops – Final LS in R1-1903810.

RAN1 #96bis (Xi’an, April 2019) [4]

Feature Lead summary R1-1905778

Agreements:

In order to align the DL TX timing of the IAB node with the DL TX timing of the parent node by setting DL TX timing of the IAB node (TA/2 + T_delta) ahead of its DL Rx timing, T_delta should be set to the (-1/2) of time interval at the parent node between the start of UL RX frame i for the IAB node and the start of DL TX frame i.

- The setting of T_delta is not necessarily specified.

- Note: The above setting of T_delta assumes that, for the same purpose, TA should be the time interval at the IAB node between the start of UL TX frame i and the start of DL RX frame i.

- Send LS to RAN4 for timing clarification. Final LS in R1-1905842.

Agreements:

- In case the calculated TA/2 + T_delta at IAB node is negative, the IAB node should not adjust its DL-Tx timing.

RAN1 #97 (Reno, May 2019) [5]

Feature Lead summary R1-1907667

Agreements:

In Rel-16, an IAB node is not expected to receive T_delta when the IAB node MT is not in RRC_Connected mode.

RAN1 #98 (Prague, August 2019) [6]

Feature Lead summary R1-19098044

Agreements:

- According to RAN1 #96bis agreement, whether T_delta is a “target value” or an “actual value” is up to parent node implementation.

- For the TA and T_delta in (TA/2+T_delta), to down-select:

  - Opt-A: T_delta is given by the latest T_delta signaling, and TA is the current time interval at the IAB node between the start of UL TX frame i and the start of DL RX frame i, which is updated with the received TA command per Rel-15.

  - Opt-B: T_delta is given by the target T_delta signaling, and TA is an average of timing advance intervals (e.g., TA1, TA2, TA3…) updated by a series TA commands.

- Once down-selected, further discuss how to reflect it in RAN1 specs

RAN1 #98bis (Chongqing, October 2019) [7]

Feature Lead summary R1-1911467

Agreements:

- From RAN1 perspective, Rel-16 NR IAB does not introduce signalling of accuracy/quality measure for IAB node DL-Tx timing.

Agreements:

An IAB node with multiple parents treats each parent as a separate synchronization source. The IAB node can also treat RAT-independent sources such as GNSS (if used) as a separate synchronization source.

- It is up to implementation how an IAB node determines its DL-Tx timing from multiple tentative DL-Tx timing, each of which is derived based on one synchronization source.

Agreements:

- For the TA and T_delta in (TA/2+T_delta), Opt-A is adopted with the following update:

  - Opt-A: T_delta is given by the latest T_delta signaling, and TA represents the actual time interval at the IAB node between the start of UL TX frame i and the start of DL RX frame i, which is updated with the received TA command per Rel-15.

    - Note: it is understood that for T_delta, TA/2, and (TA/2+T_delta), they may be either current time interval or filtered over the latest two or more time intervals, up to implementation. If the filtering is applied, the resulting performance is intended to be improved (it doesn’t necessarily mean that there will be the corresponding RAN4 requirements, up to RAN4)  no RAN1 spec impact

Agreements:

- For the signalling to carry T_delta, MAC_CE is used

Send an LS to RAN2 (CCing RAN4) informing the above two agreements –Final LS in R1-1911548.  In the LS, also adding a note:

- There was one company raising concerns of the signalling reliability of using the MAC_CE to signal T_delta (causing misalignment between the parent and the child nodes), comparing with using the RRC approach, although some other companies commented that there are some ways to alleviate the concerns (e.g., by repeating the MAC_CE, by signaling T_delta along with TA command, etc.). There was another company raising concerns whether there is a need for the signaling as frequently as that can be offered by MAC_CE.

### Others

RAN1 #99 (Reno, November 2019) [8]

Feature Lead summary R1-1913316

Agreements:

To be captured in the specification:

- The proposal in the paragraph immediately after the “Conclusion from Wednesday offline session” in R1-1913316 is agreed

Rapporteur’s note: for reference, the above mentioned paragraph from R1-1913316 is:

“If an IAB-node is provided a value Tdelta from a serving cell, the IAB-node may assume that NTA+NTA,offset⋅Tc/2+Tdelta is a time difference between a DU transmission of a signal from the serving cell providing the Tdelta  and a reception of the signal at the IAB-node MT when NTA+NTA,offset⋅Tc/2+Tdelta>0, where NTA and NTA,offset  are obtained as for a “UE” in Subclause 4.2.”

Agreements:

Adding in the specification the following:

The timing difference may be used by an IAB-node in the determination of its DU transmission timing.

Conclusion:

- In the CR stage, check further whether or not there is a need to further clarify in RAN1 spec about the usage of the timing difference based on RAN4 specifications

### References

[1]	RP-192188 - Revised WID: Integrated Access and Backhaul for NR – Qualcomm Incorporated – September 2019

[2]	RAN1 #1901 - Chairman's Notes RAN1 AdHoc 1901 final – January 2019

[3]	RAN1 #96 - Chairman's Notes RAN1 #96 final – February 2019

[4]	RAN1 #96bis - Chairman's Notes RAN1 #96bis final – April 2019

[5]	RAN1 #97 - Chairman's Notes RAN1 #97 final – May 2019

[6]	RAN1 #98 - Chairman's Notes RAN1 #98 final – August 2019

[7]	RAN1 #98bis - Chairman's Notes RAN1 #98bis final – October 2019

[8]	RAN1 #99 - Chairman's Notes RAN1 #99 final – November 2019