3GPP TS 38.331 V15.10.0 (2020-07)

Technical Specification

3rd Generation Partnership Project;

Technical Specification Group Radio Access Network;

NR;

Radio Resource Control (RRC) protocol specification

(Release 15)

The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP.
The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented.
This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification.
Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices.

3GPP

Postal address

3GPP support office address

650 Route des Lucioles - Sophia Antipolis

Valbonne - FRANCE

Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16

Internet

http://www.3gpp.org

Copyright Notification

No part may be reproduced except as authorized by written permission.
The copyright and the foregoing restriction extend to reproduction in all media.

© 2020, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

UMTS™ is a Trade Mark of ETSI registered for the benefit of its members

3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners
LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners

GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	15

1	Scope	16

2	References	16

3	Definitions, symbols and abbreviations	18

3.1	Definitions	18

3.2	Abbreviations	18

4	General	20

4.1	Introduction	20

4.2	Architecture	20

4.2.1	UE states and state transitions including inter RAT	20

4.2.2	Signalling radio bearers	22

4.3	Services	23

4.3.1	Services provided to upper layers	23

4.3.2	Services expected from lower layers	23

4.4	Functions	23

5	Procedures	24

5.1	General	24

5.1.1	Introduction	24

5.1.2	General requirements	24

5.1.3	Requirements for UE in MR-DC	25

5.2	System information	25

5.2.1	Introduction	25

5.2.2	System information acquisition	26

5.2.2.1	General UE requirements	26

5.2.2.2	SIB validity and need to (re)-acquire SIB	26

5.2.2.2.1	SIB validity	26

5.2.2.2.2	SI change indication and PWS notification	27

5.2.2.3	Acquisition of System Information	28

5.2.2.3.1	Acquisition of MIB and SIB1	28

5.2.2.3.2	Acquisition of an SI message	28

5.2.2.3.3	Request for on demand system information	29

5.2.2.3.4	Actions related to transmission of RRCSystemInfoRequest message	30

5.2.2.4	Actions upon receipt of System Information	30

5.2.2.4.1	Actions upon reception of the MIB	30

5.2.2.4.2	Actions upon reception of the SIB1	31

5.2.2.4.3	Actions upon reception of SIB2	33

5.2.2.4.4	Actions upon reception of SIB3	34

5.2.2.4.5	Actions upon reception of SIB4	34

5.2.2.4.6	Actions upon reception of SIB5	35

5.2.2.4.7	Actions upon reception of SIB6	35

5.2.2.4.8	Actions upon reception of SIB7	35

5.2.2.4.9	Actions upon reception of SIB8	36

5.2.2.4.10	Actions upon reception of SIB9	36

5.2.2.5	Essential system information missing	36

5.3	Connection control	37

5.3.1	Introduction	37

5.3.1.1	RRC connection control	37

5.3.1.2	AS Security	38

5.3.2	Paging	39

5.3.2.1	General	39

5.3.2.2	Initiation	39

5.3.2.3	Reception of the Paging message by the UE	39

5.3.3	RRC connection establishment	40

5.3.3.1	General	40

5.3.3.2	Initiation	40

5.3.3.3	Actions related to transmission of RRCSetupRequest message	41

5.3.3.4	Reception of the RRCSetup by the UE	41

5.3.3.5	Reception of the RRCReject by the UE	42

5.3.3.6	Cell re-selection or cell selection while T390, T300 or T302 is running (UE in RRC_IDLE)	42

5.3.3.7	T300 expiry	43

5.3.3.8	Abortion of RRC connection establishment	43

5.3.4	Initial AS security activation	43

5.3.4.1	General	43

5.3.4.2	Initiation	44

5.3.4.3	Reception of the SecurityModeCommand by the UE	44

5.3.5	RRC reconfiguration	45

5.3.5.1	General	45

5.3.5.2	Initiation	45

5.3.5.3	Reception of an RRCReconfiguration by the UE	46

5.3.5.4	Secondary cell group release	49

5.3.5.5	Cell Group configuration	49

5.3.5.5.1	General	49

5.3.5.5.2	Reconfiguration with sync	50

5.3.5.5.3	RLC bearer release	50

5.3.5.5.4	RLC bearer addition/modification	50

5.3.5.5.5	MAC entity configuration	51

5.3.5.5.6	RLF Timers & Constants configuration	51

5.3.5.5.7	SpCell Configuration	52

5.3.5.5.8	SCell Release	52

5.3.5.5.9	SCell Addition/Modification	52

5.3.5.6	Radio Bearer configuration	53

5.3.5.6.1	General	53

5.3.5.6.2	SRB release	53

5.3.5.6.3	SRB addition/modification	53

5.3.5.6.4	DRB release	55

5.3.5.6.5	DRB addition/modification	55

5.3.5.7	AS Security key update	57

5.3.5.8	Reconfiguration failure	58

5.3.5.8.1	Void	58

5.3.5.8.2	Inability to comply with RRCReconfiguration	58

5.3.5.8.3	T304 expiry (Reconfiguration with sync Failure)	59

5.3.5.9	Other configuration	60

5.3.5.10	MR-DC release	60

5.3.5.11	Full configuration	60

5.3.6	Counter check	62

5.3.6.1	General	62

5.3.6.2	Initiation	62

5.3.6.3	Reception of the CounterCheck message by the UE	62

5.3.7	RRC connection re-establishment	63

5.3.7.1	General	63

5.3.7.2	Initiation	63

5.3.7.3	Actions following cell selection while T311 is running	64

5.3.7.4	Actions related to transmission of RRCReestablishmentRequest message	65

5.3.7.5	Reception of the RRCReestablishment by the UE	65

5.3.7.6	T311 expiry	66

5.3.7.7	T301 expiry or selected cell no longer suitable	66

5.3.7.8	Reception of the RRCSetup by the UE	66

5.3.8	RRC connection release	66

5.3.8.1	General	66

5.3.8.2	Initiation	67

5.3.8.3	Reception of the RRCRelease by the UE	67

5.3.8.4	T320 expiry	68

5.3.8.5	UE actions upon the expiry of DataInactivityTimer	68

5.3.9	RRC connection release requested by upper layers	69

5.3.9.1	General	69

5.3.9.2	Initiation	69

5.3.10	Radio link failure related actions	69

5.3.10.1	Detection of physical layer problems in RRC_CONNECTED	69

5.3.10.2	Recovery of physical layer problems	69

5.3.10.3	Detection of radio link failure	69

5.3.11	UE actions upon going to RRC_IDLE	70

5.3.12	UE actions upon PUCCH/SRS release request	71

5.3.13	RRC connection resume	71

5.3.13.1	General	71

5.3.13.2	Initiation	72

5.3.13.3	Actions related to transmission of RRCResumeRequest or RRCResumeRequest1 message	74

5.3.13.4	Reception of the RRCResume by the UE	74

5.3.13.5	T319 expiry or Integrity check failure from lower layers while T319 is running	76

5.3.13.6	Cell re-selection or cell selection while T390, T319 or T302 is running (UE in RRC_INACTIVE)	76

5.3.13.7	Reception of the RRCSetup by the UE	76

5.3.13.8	RNA update	76

5.3.13.9	Reception of the RRCRelease by the UE	77

5.3.13.10	Reception of the RRCReject by the UE	77

5.3.13.11	Inability to comply with RRCResume	77

5.3.13.12	Inter RAT cell reselection	77

5.3.14	Unified Access Control	77

5.3.14.1	General	77

5.3.14.2	Initiation	77

5.3.14.3	Void	79

5.3.14.4	T302, T390 expiry or stop (Barring alleviation)	79

5.3.14.5	Access barring check	79

5.3.15	RRC connection reject	80

5.3.15.1	Initiation	80

5.3.15.2	Reception of the RRCReject by the UE	80

5.4	Inter-RAT mobility	81

5.4.1	Introduction	81

5.4.2	Handover to NR	81

5.4.2.1	General	81

5.4.2.2	Initiation	81

5.4.2.3	Reception of the RRCReconfiguration by the UE	81

5.4.3	Mobility from NR	82

5.4.3.1	General	82

5.4.3.2	Initiation	82

5.4.3.3	Reception of the MobilityFromNRCommand by the UE	82

5.4.3.4	Successful completion of the mobility from NR	82

5.4.3.5	Mobility from NR failure	83

5.5	Measurements	83

5.5.1	Introduction	83

5.5.2	Measurement configuration	85

5.5.2.1	General	85

5.5.2.2	Measurement identity removal	86

5.5.2.3	Measurement identity addition/modification	86

5.5.2.4	Measurement object removal	87

5.5.2.5	Measurement object addition/modification	88

5.5.2.6	Reporting configuration removal	89

5.5.2.7	Reporting configuration addition/modification	89

5.5.2.8	Quantity configuration	89

5.5.2.9	Measurement gap configuration	90

5.5.2.10	Reference signal measurement timing configuration	91

5.5.2.11	Measurement gap sharing configuration	91

5.5.3	Performing measurements	92

5.5.3.1	General	92

5.5.3.2	Layer 3 filtering	94

5.5.3.3	Derivation of cell measurement results	95

5.5.3.3a	Derivation of layer 3 beam filtered measurement	96

5.5.4	Measurement report triggering	96

5.5.4.1	General	96

5.5.4.2	Event A1 (Serving becomes better than threshold)	99

5.5.4.3	Event A2 (Serving becomes worse than threshold)	99

5.5.4.4	Event A3 (Neighbour becomes offset better than SpCell)	100

5.5.4.5	Event A4 (Neighbour becomes better than threshold)	100

5.5.4.6	Event A5 (SpCell becomes worse than threshold1 and neighbour becomes better than threshold2)	101

5.5.4.7	Event A6 (Neighbour becomes offset better than SCell)	102

5.5.4.8	Event B1 (Inter RAT neighbour becomes better than threshold)	102

5.5.4.9	Event B2 (PCell becomes worse than threshold1 and inter RAT neighbour becomes better than threshold2)	103

5.5.5	Measurement reporting	104

5.5.5.1	General	104

5.5.5.2	Reporting of beam measurement information	109

5.5.5.3	Sorting of cell measurement results	110

5.5.6	Location measurement indication	111

5.5.6.1	General	111

5.5.6.2	Initiation	111

5.5.6.3	Actions related to transmission of LocationMeasurementIndication message	111

5.6	UE capabilities	112

5.6.1	UE capability transfer	112

5.6.1.1	General	112

5.6.1.2	Initiation	112

5.6.1.3	Reception of the UECapabilityEnquiry by the UE	112

5.6.1.4	Setting band combinations, feature set combinations and feature sets supported by the UE	113

5.6.1.5	Void	115

5.7	Other	115

5.7.1	DL information transfer	115

5.7.1.1	General	115

5.7.1.2	Initiation	115

5.7.1.3	Reception of the DLInformationTransfer by the UE	115

5.7.2	UL information transfer	115

5.7.2.1	General	115

5.7.2.2	Initiation	115

5.7.2.3	Actions related to transmission of ULInformationTransfer message	115

5.7.2.4	Failure to deliver ULInformationTransfer message	116

5.7.2a	UL information transfer for MR-DC	116

5.7.2a.1	General	116

5.7.2a.2	Initiation	116

5.7.2a.3	Actions related to transmission of ULInformationTransferMRDC message	116

5.7.3	SCG failure information	117

5.7.3.1	General	117

5.7.3.2	Initiation	117

5.7.3.3	Failure type determination for (NG)EN-DC	117

5.7.3.4	Setting the contents of MeasResultSCG-Failure	118

5.7.3.5	Actions related to transmission of SCGFailureInformation message	118

5.7.3a	EUTRA SCG failure information	120

5.7.3a.1	General	120

5.7.3a.2	Initiation	120

5.7.3a.3	Actions related to transmission of SCGFailureInformationEUTRA message	120

5.7.4	UE Assistance Information	121

5.7.4.1	General	121

5.7.4.2	Initiation	121

5.7.4.3	Actions related to transmission of UEAssistanceInformation message	121

5.7.5	Failure information	123

5.7.5.1	General	123

5.7.5.2	Initiation	123

5.7.5.3	Actions related to transmission of FailureInformation message	123

6	Protocol data units, formats and parameters (ASN.1)	124

6.1	General	124

6.1.1	Introduction	124

6.1.2	Need codes and conditions for optional downlink fields	124

6.1.3	General rules	127

6.2	RRC messages	127

6.2.1	General message structure	127

–	NR-RRC-Definitions	127

–	BCCH-BCH-Message	127

–	BCCH-DL-SCH-Message	128

–	DL-CCCH-Message	128

–	DL-DCCH-Message	129

–	PCCH-Message	129

–	UL-CCCH-Message	130

–	UL-CCCH1-Message	130

–	UL-DCCH-Message	131

6.2.2	Message definitions	133

–	CounterCheck	133

–	CounterCheckResponse	134

–	DLInformationTransfer	135

–	FailureInformation	136

–	LocationMeasurementIndication	137

–	MeasurementReport	137

–	MIB	138

–	MobilityFromNRCommand	139

–	Paging	141

–	RRCReestablishment	142

–	RRCReestablishmentComplete	142

–	RRCReestablishmentRequest	143

–	RRCReconfiguration	144

–	RRCReconfigurationComplete	148

–	RRCReject	149

–	RRCRelease	150

–	RRCResume	154

–	RRCResumeComplete	155

–	RRCResumeRequest	156

–	RRCResumeRequest1	157

–	RRCSetup	158

–	RRCSetupComplete	159

–	RRCSetupRequest	160

–	RRCSystemInfoRequest	161

–	SCGFailureInformation	162

–	SCGFailureInformationEUTRA	163

–	SecurityModeCommand	164

–	SecurityModeComplete	165

–	SecurityModeFailure	166

–	SIB1	167

–	SystemInformation	169

–	UEAssistanceInformation	169

–	UECapabilityEnquiry	173

–	UECapabilityInformation	173

–	ULInformationTransfer	174

–	ULInformationTransferMRDC	175

6.3	RRC information elements	176

6.3.0	Parameterized types	176

–	SetupRelease	176

6.3.1	System information blocks	176

–	SIB2	176

–	SIB3	179

–	SIB4	180

–	SIB5	183

–	SIB6	185

–	SIB7	186

–	SIB8	187

–	SIB9	188

6.3.2	Radio resource control information elements	189

–	AdditionalSpectrumEmission	189

–	Alpha	190

–	AMF-Identifier	190

–	ARFCN-ValueEUTRA	190

–	ARFCN-ValueNR	191

–	BeamFailureRecoveryConfig	191

–	BetaOffsets	193

–	BSR-Config	194

–	BWP	195

–	BWP-Downlink	196

–	BWP-DownlinkCommon	197

–	BWP-DownlinkDedicated	197

–	BWP-Id	198

–	BWP-Uplink	198

–	BWP-UplinkCommon	199

–	BWP-UplinkDedicated	200

–	CellAccessRelatedInfo	201

–	CellAccessRelatedInfo-EUTRA-5GC	202

–	CellAccessRelatedInfo-EUTRA-EPC	202

–	CellGroupConfig	203

–	CellGroupId	206

–	CellIdentity	206

–	CellReselectionPriority	207

–	CellReselectionSubPriority	207

–	CGI-InfoEUTRA	207

–	CGI-InfoNR	208

–	CodebookConfig	208

–	ConfiguredGrantConfig	211

–	ConnEstFailureControl	214

–	ControlResourceSet	215

–	ControlResourceSetId	216

–	ControlResourceSetZero	217

–	CrossCarrierSchedulingConfig	217

–	CSI-AperiodicTriggerStateList	218

–	CSI-FrequencyOccupation	220

–	CSI-IM-Resource	220

–	CSI-IM-ResourceId	221

–	CSI-IM-ResourceSet	222

–	CSI-IM-ResourceSetId	222

–	CSI-MeasConfig	222

–	CSI-ReportConfig	224

–	CSI-ReportConfigId	230

–	CSI-ResourceConfig	230

–	CSI-ResourceConfigId	231

–	CSI-ResourcePeriodicityAndOffset	232

–	CSI-RS-ResourceConfigMobility	232

–	CSI-RS-ResourceMapping	235

–	CSI-SemiPersistentOnPUSCH-TriggerStateList	236

–	CSI-SSB-ResourceSet	237

–	CSI-SSB-ResourceSetId	237

–	DedicatedNAS-Message	237

–	DMRS-DownlinkConfig	238

–	DMRS-UplinkConfig	239

–	DownlinkConfigCommon	240

–	DownlinkConfigCommonSIB	241

–	DownlinkPreemption	243

–	DRB-Identity	244

–	DRX-Config	244

–	FilterCoefficient	246

–	FreqBandIndicatorNR	246

–	FrequencyInfoDL	247

–	FrequencyInfoDL-SIB	248

–	FrequencyInfoUL	248

–	FrequencyInfoUL-SIB	249

–	Hysteresis	250

–	I-RNTI-Value	251

–	LocationMeasurementInfo	251

–	LogicalChannelConfig	252

–	LogicalChannelIdentity	254

–	MAC-CellGroupConfig	254

–	MeasConfig	255

–	MeasGapConfig	256

–	MeasGapSharingConfig	258

–	MeasId	259

–	MeasIdToAddModList	259

–	MeasObjectEUTRA	259

–	MeasObjectId	261

–	MeasObjectNR	261

–	MeasObjectToAddModList	265

–	MeasResultCellListSFTD-NR	266

–	MeasResultCellListSFTD-EUTRA	266

–	MeasResults	267

–	MeasResult2EUTRA	271

–	MeasResult2NR	271

–	MeasResultSCG-Failure	271

–	MeasTriggerQuantityEUTRA	272

–	MobilityStateParameters	272

–	MultiFrequencyBandListNR	273

–	MultiFrequencyBandListNR-SIB	273

–	NextHopChainingCount	274

–	NG-5G-S-TMSI	274

–	NR-NS-PmaxList	275

–	NZP-CSI-RS-Resource	275

–	NZP-CSI-RS-ResourceId	276

–	NZP-CSI-RS-ResourceSet	277

–	NZP-CSI-RS-ResourceSetId	277

–	P-Max	278

–	PCI-List	278

–	PCI-Range	279

–	PCI-RangeElement	279

–	PCI-RangeIndex	280

–	PCI-RangeIndexList	280

–	PDCCH-Config	280

–	PDCCH-ConfigCommon	281

–	PDCCH-ConfigSIB1	283

–	PDCCH-ServingCellConfig	284

–	PDCP-Config	284

–	PDSCH-Config	288

–	PDSCH-ConfigCommon	291

–	PDSCH-ServingCellConfig	291

–	PDSCH-TimeDomainResourceAllocationList	293

–	PHR-Config	293

–	PhysCellId	294

–	PhysicalCellGroupConfig	295

–	PLMN-Identity	297

–	PLMN-IdentityInfoList	297

–	PRB-Id	298

–	PTRS-DownlinkConfig	298

–	PTRS-UplinkConfig	299

–	PUCCH-Config	300

–	PUCCH-ConfigCommon	305

–	PUCCH-PathlossReferenceRS-Id	305

–	PUCCH-PowerControl	306

–	PUCCH-SpatialRelationInfo	307

–	PUCCH-TPC-CommandConfig	308

–	PUSCH-Config	309

–	PUSCH-ConfigCommon	312

–	PUSCH-PowerControl	313

–	PUSCH-ServingCellConfig	315

–	PUSCH-TimeDomainResourceAllocationList	316

–	PUSCH-TPC-CommandConfig	317

–	Q-OffsetRange	318

–	Q-QualMin	318

–	Q-RxLevMin	319

–	QuantityConfig	319

–	RACH-ConfigCommon	320

–	RACH-ConfigDedicated	323

–	RACH-ConfigGeneric	325

–	RA-Prioritization	326

–	RadioBearerConfig	327

–	RadioLinkMonitoringConfig	330

–	RadioLinkMonitoringRS-Id	331

–	RAN-AreaCode	332

–	RateMatchPattern	332

–	RateMatchPatternId	334

–	RateMatchPatternLTE-CRS	334

–	RejectWaitTime	335

–	ReportConfigId	335

–	ReportConfigInterRAT	336

–	ReportConfigNR	338

–	ReportConfigToAddModList	343

–	ReportInterval	343

–	ReselectionThreshold	344

–	ReselectionThresholdQ	344

–	ResumeCause	344

–	RLC-BearerConfig	345

–	RLC-Config	346

–	RLF-TimersAndConstants	349

–	RNTI-Value	349

–	RSRP-Range	350

–	RSRQ-Range	350

–	SCellIndex	350

–	SchedulingRequestConfig	351

–	SchedulingRequestId	352

–	SchedulingRequestResourceConfig	352

–	SchedulingRequestResourceId	353

–	ScramblingId	353

–	SCS-SpecificCarrier	354

–	SDAP-Config	354

–	SearchSpace	355

–	SearchSpaceId	359

–	SearchSpaceZero	359

–	SecurityAlgorithmConfig	360

–	ServCellIndex	360

–	ServingCellConfig	361

–	ServingCellConfigCommon	365

–	ServingCellConfigCommonSIB	367

–	ShortI-RNTI-Value	368

–	ShortMAC-I	368

–	SINR-Range	369

–	SI-SchedulingInfo	369

–	SK-Counter	372

–	SlotFormatCombinationsPerCell	372

–	SlotFormatIndicator	373

–	S-NSSAI	374

–	SpeedStateScaleFactors	374

–	SPS-Config	375

–	SRB-Identity	376

–	SRS-CarrierSwitching	376

–	SRS-Config	378

–	SRS-TPC-CommandConfig	382

–	SSB-Index	383

–	SSB-MTC	384

–	SSB-ToMeasure	384

–	SS-RSSI-Measurement	385

–	SubcarrierSpacing	386

–	TAG-Config	386

–	TCI-State	387

–	TCI-StateId	388

–	TDD-UL-DL-ConfigCommon	388

–	TDD-UL-DL-ConfigDedicated	389

–	TrackingAreaCode	390

–	T-Reselection	391

–	TimeToTrigger	391

–	UAC-BarringInfoSetIndex	391

–	UAC-BarringInfoSetList	392

–	UAC-BarringPerCatList	393

–	UAC-BarringPerPLMN-List	393

–	UE-TimersAndConstants	394

–	UplinkConfigCommon	394

–	UplinkConfigCommonSIB	395

–	UplinkTxDirectCurrentList	395

–	ZP-CSI-RS-Resource	397

–	ZP-CSI-RS-ResourceSet	397

–	ZP-CSI-RS-ResourceSetId	398

6.3.3	UE capability information elements	398

–	AccessStratumRelease	398

–	BandCombinationList	399

–	CA-BandwidthClassEUTRA	401

–	CA-BandwidthClassNR	401

–	CA-ParametersEUTRA	402

–	CA-ParametersNR	402

–	CA-ParametersNRDC	403

–	CodebookParameters	404

–	FeatureSetCombination	405

–	FeatureSetCombinationId	406

–	FeatureSetDownlink	406

–	FeatureSetDownlinkId	409

–	FeatureSetDownlinkPerCC	409

–	FeatureSetDownlinkPerCC-Id	410

–	FeatureSetEUTRA-DownlinkId	410

–	FeatureSetEUTRA-UplinkId	410

–	FeatureSets	411

–	FeatureSetUplink	412

–	FeatureSetUplinkId	413

–	FeatureSetUplinkPerCC	413

–	FeatureSetUplinkPerCC-Id	414

–	FreqBandIndicatorEUTRA	414

–	FreqBandList	415

–	FreqSeparationClass	415

–	IMS-Parameters	416

–	InterRAT-Parameters	416

–	MAC-Parameters	417

–	MeasAndMobParameters	418

–	MeasAndMobParametersMRDC	419

–	MIMO-Layers	420

–	MIMO-ParametersPerBand	421

–	ModulationOrder	424

–	MRDC-Parameters	424

–	NRDC-Parameters	425

–	PDCP-Parameters	426

–	PDCP-ParametersMRDC	426

–	Phy-Parameters	427

–	Phy-ParametersMRDC	430

–	ProcessingParameters	431

–	RAT-Type	431

–	RF-Parameters	432

–	RF-ParametersMRDC	434

–	RLC-Parameters	435

–	SDAP-Parameters	435

–	SRS-SwitchingTimeNR	436

–	SRS-SwitchingTimeEUTRA	436

–	SupportedBandwidth	436

–	UE-CapabilityRAT-ContainerList	437

–	UE-CapabilityRAT-RequestList	437

–	UE-CapabilityRequestFilterCommon	438

–	UE-CapabilityRequestFilterNR	439

–	UE-MRDC-Capability	439

–	UE-NR-Capability	441

6.3.4	Other information elements	442

–	EUTRA-AllowedMeasBandwidth	442

–	EUTRA-MBSFN-SubframeConfigList	443

–	EUTRA-MultiBandInfoList	444

–	EUTRA-NS-PmaxList	444

–	EUTRA-PhysCellId	445

–	EUTRA-PhysCellIdRange	445

–	EUTRA-PresenceAntennaPort1	445

–	EUTRA-Q-OffsetRange	446

–	OtherConfig	446

–	RRC-TransactionIdentifier	447

6.4	RRC multiplicity and type constraint values	447

–	Multiplicity and type constraint definitions	447

–	End of NR-RRC-Definitions	451

6.5	Short Message	451

7	Variables and constants	452

7.1	Timers	452

7.1.1	Timers (Informative)	452

7.1.2	Timer handling	455

7.2	Counters	456

7.3	Constants	456

7.4	UE variables	456

–	NR-UE-Variables	456

–	VarPendingRNA-Update	457

–	VarMeasConfig	457

–	VarMeasReportList	458

–	VarResumeMAC-Input	459

–	VarShortMAC-Input	459

–	End of NR-UE-Variables	460

8	Protocol data unit abstract syntax	461

8.1	General	461

8.2	Structure of encoded RRC messages	461

8.3	Basic production	461

8.4	Extension	461

8.5	Padding	462

9	Specified and default radio configurations	462

9.1	Specified configurations	462

9.1.1	Logical channel configurations	462

9.1.1.1	BCCH configuration	462

9.1.1.2	CCCH configuration	463

9.1.1.3	PCCH configuration	463

9.1.2	Void	463

9.2	Default radio configurations	463

9.2.1	Default SRB configurations	463

9.2.2	Default MAC Cell Group configuration	464

9.2.3	Default values timers and constants	464

10	Generic error handling	464

10.1	General	464

10.2	ASN.1 violation or encoding error	465

10.3	Field set to a not comprehended value	465

10.4	Mandatory field missing	465

10.5	Not comprehended field	466

11	Radio information related interactions between network nodes	467

11.1	General	467

11.2	Inter-node RRC messages	467

11.2.1	General	467

11.2.2	Message definitions	468

–	HandoverCommand	468

–	HandoverPreparationInformation	469

–	CG-Config	472

–	CG-ConfigInfo	478

–	MeasurementTimingConfiguration	486

–	UERadioPagingInformation	487

–	UERadioAccessCapabilityInformation	488

11.2.3	Mandatory information in inter-node RRC messages	489

11.3	Inter-node RRC information element definitions	490

11.4	Inter-node RRC multiplicity and type constraint values	490

–	Multiplicity and type constraints definitions	490

–	End of NR-InterNodeDefinitions	491

12	Processing delay requirements for RRC procedures	492

Annex A (informative):	Guidelines, mainly on use of ASN.1	495

A.1	Introduction	495

A.2	Procedural specification	495

A.2.1	General principles	495

A.2.2	More detailed aspects	495

A.3	PDU specification	496

A.3.1	General principles	496

A.3.1.1	ASN.1 sections	496

A.3.1.2	ASN.1 identifier naming conventions	497

A.3.1.3	Text references using ASN.1 identifiers	498

A.3.2	High-level message structure	499

A.3.3	Message definition	500

A.3.4	Information elements	502

A.3.5	Fields with optional presence	503

A.3.6	Fields with conditional presence	504

A.3.7	Guidelines on use of lists with elements of SEQUENCE type	505

A.3.8	Guidelines on use of parameterised SetupRelease type	505

A.3.9	Guidelines on use of ToAddModList and ToReleaseList	507

A.3.10	Guidelines on use of of lists (without ToAddModList and ToReleaseList)	508

A.4	Extension of the PDU specifications	509

A.4.1	General principles to ensure compatibility	509

A.4.2	Critical extension of messages and fields	509

A.4.3	Non-critical extension of messages	511

A.4.3.1	General principles	511

A.4.3.2	Further guidelines	512

A.4.3.3	Typical example of evolution of IE with local extensions	513

A.4.3.4	Typical examples of non critical extension at the end of a message	514

A.4.3.5	Examples of non-critical extensions not placed at the default extension location	515

–	ParentIE-WithEM	515

–	ChildIE1-WithoutEM	516

–	ChildIE2-WithoutEM	517

A.5	Guidelines regarding inclusion of transaction identifiers in RRC messages	517

A.6	Guidelines regarding use of need codes	518

A.7	Guidelines regarding use of conditions	518

A.8	Miscellaneous	519

Annex B (informative):	RRC Information	520

B.1	Protection of RRC messages	520

B.2	Description of BWP configuration options	522

Annex C (informative):	Change history	524

# Foreword

This Technical Specification has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

# 1	Scope

The present document specifies the Radio Resource Control protocol for the radio interface between UE and NG-RAN.

The scope of the present document also includes:

-	the radio related information transported in a transparent container between source gNB and target gNB upon inter gNB handover;

-	the radio related information transported in a transparent container between a source or target gNB and another system upon inter RAT handover.

-	the radio related information transported in a transparent container between a source eNB and target gNB during E-UTRA-NR Dual Connectivity.

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 38.300: "NR; Overall description; Stage 2".

[3]	3GPP TS 38.321: "NR; Medium Access Control (MAC); Protocol specification".

[4]	3GPP TS 38.322: "NR; Radio Link Control (RLC) protocol specification".

[5]	3GPP TS 38.323: "NR; Packet Data Convergence Protocol (PDCP) protocol specification".

[6]	ITU-T Recommendation X.680 (08/2015) "Information Technology – Abstract Syntax Notation One (ASN.1): Specification of basic notation" (Same as the ISO/IEC International Standard 8824-1).

[7]	ITU-T Recommendation X.681 (08/2015) "Information Technology – Abstract Syntax Notation One (ASN.1): Information object specification" (Same as the ISO/IEC International Standard 8824-2).

[8]	ITU-T Recommendation X.691 (08/2015) "Information technology – ASN.1 encoding rules: Specification of Packed Encoding Rules (PER)" (Same as the ISO/IEC International Standard 8825-2).

[9]	3GPP TS 38.215: "NR; Physical layer measurements".

[10]	3GPP TS 36.331: "Evolved Universal Terrestrial Radio Access (E-UTRA) Radio Resource Control (RRC); Protocol Specification".

[11]	3GPP TS 33.501: "Security Architecture and Procedures for 5G System".

[12]	3GPP TS 38.104: "NR; Base Station (BS) radio transmission and reception".

[13]	3GPP TS 38.213: "NR; Physical layer procedures for control".

[14]	3GPP TS 38.133: "NR; Requirements for support of radio resource management".

[15]	3GPP TS 38.101-1: "NR; User Equipment (UE) radio transmission and reception; Part 1: Range 1 Standalone".

[16]	3GPP TS 38.211: "NR; Physical channels and modulation".

[17]	3GPP TS 38.212: "NR; Multiplexing and channel coding".

[18]	ITU-T Recommendation X.683 (08/2015) "Information Technology – Abstract Syntax Notation One (ASN.1): Parameterization of ASN.1 specifications" (Same as the ISO/IEC International Standard 8824-4).

[19]	3GPP TS 38.214: "NR; Physical layer procedures for data".

[20]	3GPP TS 38.304: "NR; User Equipment (UE) procedures in Idle mode and RRC Inactive state".

[21]	3GPP TS 23.003: "Numbering, addressing and identification".

[22]	3GPP TS 36.101: "E-UTRA; User Equipment (UE) radio transmission and reception".

[23]	3GPP TS 24.501: "Non-Access-Stratum (NAS) protocol for 5G System (5GS); Stage 3".

[24]	3GPP TS 37.324: "Service Data Adaptation Protocol (SDAP) specification".

[25]	3GPP TS 22.261: "Service requirements for the 5G System".

[26]	3GPP TS 38.306: "User Equipment (UE) radio access capabilities".

[27]	3GPP TS 36.304: "E-UTRA; User Equipment (UE) procedures in idle mode".

[28]	ATIS 0700041: "WEA 3.0: Device-Based Geo-Fencing".

[29]	3GPP TS 23.041: "Technical realization of Cell Broadcast Service (CBS)".

[30]	3GPP TS 33.401: "3GPP System Architecture Evolution (SAE); Security architecture".

[31]	3GPP TS 36.211: "E-UTRA; Physical channels and modulation".

[32]	3GPP TS 23.501: "System Architecture for the 5G System; Stage 2".

[33]	3GPP TS 36.104:"E-UTRA; Base Station (BS) radio transmission and reception".

[34]	3GPP TS 38.101-3 "NR; User Equipment (UE) radio transmission and reception; Part 3: Range 1 and Range 2 Interworking operation with other radios".

[35]	3GPP TS 38.423: "NG-RAN, Xn application protocol (XnAP)".

[36]	3GPP TS 38.473: "NG-RAN; F1 application protocol (F1AP)".

[37]	3GPP TS 36.423: "E-UTRA; X2 application protocol (X2AP)".

[38]	3GPP TS 24.008: "Mobile radio interface layer 3 specification; Core network protocols; Stage 3".

[39]	3GPP TS 38.101-2 "NR; User Equipment (UE) radio transmission and reception; Part 2: Range 2 Standalone".

[40]	3GPP TS 36.133:"E-UTRA; Requirements for support of radio resource management".

[41]	3GPP TS 37.340: "E-UTRA and NR; Multi-connectivity; Stage 2".

[42]	3GPP TS 38.413: "NG-RAN, NG Application Protocol (NGAP)".

# 3	Definitions, symbols and abbreviations

## 3.1	Definitions

For the purposes of the present document, the terms and definitions given in TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1].

CEIL: Mathematical function used to 'round up' i.e. to the nearest integer having a higher or equal value.

Dedicated signalling: Signalling sent on DCCH logical channel between the network and a single UE.

Field: The individual contents of an information element are referred to as fields.

FLOOR: Mathematical function used to 'round down' i.e. to the nearest integer having a lower or equal value.

Information element: A structural element containing single or multiple fields is referred as information element.

Primary Cell: The MCG cell, operating on the primary frequency, in which the UE either performs the initial connection establishment procedure or initiates the connection re-establishment procedure.

Primary SCG Cell: For dual connectivity operation, the SCG cell in which the UE performs random access when performing the Reconfiguration with Sync procedure.

Primary Timing Advance Group: Timing Advance Group containing the SpCell.

PUCCH SCell: An SCell configured with PUCCH.

PUSCH-Less SCell: An SCell configured without PUSCH.

RLC bearer configuration: The lower layer part of the radio bearer configuration comprising the RLC and logical channel configurations.

Secondary Cell: For a UE configured with CA, a cell providing additional radio resources on top of Special Cell.

Secondary Cell Group: For a UE configured with dual connectivity, the subset of serving cells comprising of the PSCell and zero or more secondary cells.

Serving Cell: For a UE in RRC_CONNECTED not configured with CA/DC there is only one serving cell comprising of the primary cell. For a UE in RRC_CONNECTED configured with CA/ DC the term 'serving cells' is used to denote the set of cells comprising of the Special Cell(s) and all secondary cells.

Special Cell: For Dual Connectivity operation the term Special Cell refers to the PCell of the MCG or the PSCell of the SCG, otherwise the term Special Cell refers to the PCell.

Split SRB: In MR-DC, an SRB that supports transmission via MCG and SCG as well as duplication of RRC PDUs as defined in TS 37.340 [41].

SSB Frequency: Frequency referring to the position of resource element RE=#0 (subcarrier #0) of resource block RB#10 of the SS block.

UE Inactive AS Context: UE Inactive AS Context is stored when the connection is suspended and restored when the connection is resumed. It includes information as defined in clause 5.3.8.3.

## 3.2	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1].

5GC	5G Core Network

ACK	Acknowledgement

AM	Acknowledged Mode

ARQ	Automatic Repeat Request

AS	Access Stratum

ASN.1	Abstract Syntax Notation One

BLER	Block Error Rate

BWP	Bandwidth Part

CA	Carrier Aggregation

CCCH	Common Control Channel

CG	Cell Group

CMAS	Commercial Mobile Alert Service

CP	Control Plane

C-RNTI	Cell RNTI

CSI	Channel State Information

DC	Dual Connectivity

DCCH	Dedicated Control Channel

DCI	Downlink Control Information

DL	Downlink

DL-SCH	Downlink Shared Channel

DM-RS	Demodulation Reference Signal

DRB	(user) Data Radio Bearer

DRX	Discontinuous Reception

DTCH	Dedicated Traffic Channel

EN-DC	E-UTRA NR Dual Connectivity with E-UTRA connected to EPC

EPC	Evolved Packet Core

EPS	Evolved Packet System

ETWS	Earthquake and Tsunami Warning System

E-UTRA	Evolved Universal Terrestrial Radio Access

E-UTRA/5GC	E-UTRA connected to 5GC

E-UTRA/EPC	E-UTRA connected to EPC

E-UTRAN	Evolved Universal Terrestrial Radio Access Network

FDD	Frequency Division Duplex

FFS	For Further Study

GERAN	GSM/EDGE Radio Access Network

GNSS	Global Navigation Satellite System

GSM	Global System for Mobile Communications

HARQ	Hybrid Automatic Repeat Request

IE	Information element

IMSI	International Mobile Subscriber Identity

kB	Kilobyte (1000 bytes)

L1	Layer 1

L2	Layer 2

L3	Layer 3

MAC	Medium Access Control

MCG	Master Cell Group

MIB	Master Information Block

MR-DC	Multi-Radio Dual Connectivity

N/A	Not Applicable

NE-DC	NR E-UTRA Dual Connectivity

(NG)EN-DC	E-UTRA NR Dual Connectivity (covering E-UTRA connected to EPC or 5GC)

NGEN-DC	E-UTRA NR Dual Connectivity with E-UTRA connected to 5GC

NR-DC	NR-NR Dual Connectivity

NR/5GC	NR connected to 5GC

PCell	Primary Cell

PDCP	Packet Data Convergence Protocol

PDU	Protocol Data Unit

PLMN	Public Land Mobile Network

PSCell	Primary SCG Cell

PWS	Public Warning System

QoS	Quality of Service

RAN	Radio Access Network

RAT	Radio Access Technology

RLC	Radio Link Control

RNA	RAN-based Notification Area

RNTI	Radio Network Temporary Identifier

ROHC	Robust Header Compression

RRC	Radio Resource Control

RS	Reference Signal

SCell	Secondary Cell

SCG	Secondary Cell Group

SFN	System Frame Number

SFTD	SFN and Frame Timing Difference

SI	System Information

SIB	System Information Block

SpCell	Special Cell

SRB	Signalling Radio Bearer

SSB	Synchronization Signal Block

TAG	Timing Advance Group

TDD	Time Division Duplex

TM	Transparent Mode

UE	User Equipment

UL	Uplink

UM	Unacknowledged Mode

UP	User Plane

In the ASN.1, lower case may be used for some (parts) of the above abbreviations e.g. c-RNTI.

# 4	General

## 4.1	Introduction

This specification is organised as follows:

-	clause 4.2 describes the RRC protocol model;

-	clause 4.3 specifies the services provided to upper layers as well as the services expected from lower layers;

-	clause 4.4 lists the RRC functions;

-	clause 5 specifies RRC procedures, including UE state transitions;

-	clause 6 specifies the RRC messages in ASN.1 and description;

-	clause 7 specifies the variables (including protocol timers and constants) and counters to be used by the UE;

-	clause 8 specifies the encoding of the RRC messages;

-	clause 9 specifies the specified and default radio configurations;

-	clause 10 specifies generic error handling;

-	clause 11 specifies the RRC messages transferred across network nodes;

-	clause 12 specifies the UE capability related constraints and performance requirements.

## 4.2	Architecture

### 4.2.1	UE states and state transitions including inter RAT

A UE is either in RRC_CONNECTED state or in RRC_INACTIVE state when an RRC connection has been established. If this is not the case, i.e. no RRC connection is established, the UE is in RRC_IDLE state. The RRC states can further be characterised as follows:

-	RRC_IDLE:

-	A UE specific DRX may be configured by upper layers;

-	UE controlled mobility based on network configuration;

-	The UE:

-	Monitors Short Messages transmitted with P-RNTI over DCI (see clause 6.5);

-	Monitors a Paging channel for CN paging using 5G-S-TMSI;

-	Performs neighbouring cell measurements and cell (re-)selection;

-	Acquires system information and can send SI request (if configured).

-	RRC_INACTIVE:

-	A UE specific DRX may be configured by upper layers or by RRC layer;

-	UE controlled mobility based on network configuration;

-	The UE stores the UE Inactive AS context;

-	A RAN-based notification area is configured by RRC layer;

The UE:

-	Monitors Short Messages transmitted with P-RNTI over DCI (see clause 6.5);

-	Monitors a Paging channel for CN paging using 5G-S-TMSI and RAN paging using fullI-RNTI;

-	Performs neighbouring cell measurements and cell (re-)selection;

-	Performs RAN-based notification area updates periodically and when moving outside the configured RAN-based notification area;

-	Acquires system information and can send SI request (if configured).

-	RRC_CONNECTED:

-	The UE stores the AS context;

-	Transfer of unicast data to/from UE;

-	At lower layers, the UE may be configured with a UE specific DRX;

-	For UEs supporting CA, use of one or more SCells, aggregated with the SpCell, for increased bandwidth;

-	For UEs supporting DC, use of one SCG, aggregated with the MCG, for increased bandwidth;

-	Network controlled mobility within NR and to/from E-UTRA;

-	The UE:

-	Monitors Short Messages transmitted with P-RNTI over DCI (see clause 6.5), if configured;

-	Monitors control channels associated with the shared data channel to determine if data is scheduled for it;

-	Provides channel quality and feedback information;

-	Performs neighbouring cell measurements and measurement reporting;

-	Acquires system information.

Figure 4.2.1-1 illustrates an overview of UE RRC state machine and state transitions in NR. A UE has only one RRC state in NR at one time.

Figure 4.2.1-1:	UE state machine and state transitions in NR

Figure 4.2.1-2 illustrates an overview of UE state machine and state transitions in NR as well as the mobility procedures supported between NR/5GC E-UTRA/EPC and E-UTRA/5GC.

Figure 4.2.1-2:	UE state machine and state transitions between NR/5GC, E-UTRA/EPC and E-UTRA/5GC

### 4.2.2	Signalling radio bearers

"Signalling Radio Bearers" (SRBs) are defined as Radio Bearers (RBs) that are used only for the transmission of RRC and NAS messages. More specifically, the following SRBs are defined:

-	SRB0 is for RRC messages using the CCCH logical channel;

-	SRB1 is for RRC messages (which may include a piggybacked NAS message) as well as for NAS messages prior to the establishment of SRB2, all using DCCH logical channel;

-	SRB2 is for NAS messages, all using DCCH logical channel. SRB2 has a lower priority than SRB1 and may be configured by the network after AS security activation;

-	SRB3 is for specific RRC messages when UE is in (NG)EN-DC or NR-DC, all using DCCH logical channel.

In downlink, piggybacking of NAS messages is used only for one dependant (i.e. with joint success/failure) procedure: bearer establishment/modification/release. In uplink piggybacking of NAS message is used only for transferring the initial NAS message during connection setup and connection resume.

NOTE 1:	The NAS messages transferred via SRB2 are also contained in RRC messages, which however do not include any RRC protocol control information.

Once AS security is activated, all RRC messages on SRB1, SRB2 and SRB3, including those containing NAS messages, are integrity protected and ciphered by PDCP. NAS independently applies integrity protection and ciphering to the NAS messages, see TS 24.501 [23].

Split SRB is supported for all the MR-DC options in both SRB1 and SRB2 (split SRB is not supported for SRB0 and SRB3).

## 4.3	Services

### 4.3.1	Services provided to upper layers

The RRC protocol offers the following services to upper layers:

-	Broadcast of common control information;

-	Notification of UEs in RRC_IDLE, e.g. about a mobile terminating call;

-	Notification of UEs about ETWS and/or CMAS

-	Transfer of dedicated signalling.

### 4.3.2	Services expected from lower layers

In brief, the following are the main services that RRC expects from lower layers:

-	Integrity protection, ciphering and loss-less in-sequence delivery of information without duplication;

## 4.4	Functions

The RRC protocol includes the following main functions:

-	Broadcast of system information:

-	Including NAS common information;

-	Information applicable for UEs in RRC_IDLE and RRC_INACTIVE (e.g. cell (re-)selection parameters, neighbouring cell information) and information (also) applicable for UEs in RRC_CONNECTED (e.g. common channel configuration information);

-	Including ETWS notification, CMAS notification.

-	RRC connection control:

-	Paging;

-	Establishment/modification/suspension/resumption/release of RRC connection, including e.g. assignment/modification of UE identity (C-RNTI, fullI-RNTI, etc.), establishment/modification/suspension/resumption/release of SRBs (except for SRB0);

-	Access barring;

-	Initial AS security activation, i.e. initial configuration of AS integrity protection (SRBs, DRBs) and AS ciphering (SRBs, DRBs);

-	RRC connection mobility including e.g. intra-frequency and inter-frequency handover, associated AS security handling, i.e. key/algorithm change, specification of RRC context information transferred between network nodes;

-	Establishment/modification/suspension/resumption/release of RBs carrying user data (DRBs);

-	Radio configuration control including e.g. assignment/modification of ARQ configuration, HARQ configuration, DRX configuration;

-	In case of DC, cell management including e.g. change of PSCell, addition/modification/release of SCG cell(s);

-	In case of CA, cell management including e.g. addition/modification/release of SCell(s);

-	QoS control including assignment/ modification of semi-persistent scheduling (SPS) configuration and configured grant configuration for DL and UL respectively, assignment/ modification of parameters for UL rate control in the UE, i.e. allocation of a priority and a prioritised bit rate (PBR) for each RB.

-	Recovery from radio link failure.

-	Inter-RAT mobility including e.g. AS security activation, transfer of RRC context information;

-	Measurement configuration and reporting:

-	Establishment/modification/release of measurement configuration (e.g. intra-frequency, inter-frequency and inter- RAT measurements);

-	Setup and release of measurement gaps;

-	Measurement reporting.

-	Other functions including e.g. generic protocol error handling, transfer of dedicated NAS information, transfer of UE radio access capability information.

# 5	Procedures

## 5.1	General

### 5.1.1	Introduction

This clause covers the general requirements.

### 5.1.2	General requirements

The UE shall:

1>	process the received messages in order of reception by RRC, i.e. the processing of a message shall be completed before starting the processing of a subsequent message;

NOTE:	Network may initiate a subsequent procedure prior to receiving the UE's response of a previously initiated procedure.

1>	within a sub-clause execute the steps according to the order specified in the procedural description;

1>	consider the term 'radio bearer' (RB) to cover SRBs and DRBs unless explicitly stated otherwise;

1>	set the rrc-TransactionIdentifier in the response message, if included, to the same value as included in the message received from the network that triggered the response message;

1>	upon receiving a choice value set to setup:

2>	apply the corresponding received configuration and start using the associated resources, unless explicitly specified otherwise;

1>	upon receiving a choice value set to release:

2>	clear the corresponding configuration and stop using the associated resources;

1>	in case the size of a list is extended, upon receiving an extension field comprising the entries in addition to the ones carried by the original field (regardless of whether the network signals more entries in total); apply the following generic behaviour unless explicitly stated otherwise:

2>	create a combined list by concatenating the additional entries included in the extension field to the original field while maintaining the order among both the original and the additional entries;

2>	for the combined list, created according to the previous, apply the same behaviour as defined for the original field.

### 5.1.3	Requirements for UE in MR-DC

In this specification, the UE considers itself to be in:

-	EN-DC, if and only if it is configured with nr-SecondaryCellGroupConfig according to TS 36.331[10], and it is connected to EPC,

-	NGEN-DC, if and only if it is configured with nr-SecondaryCellGroupConfig according to TS 36.331[10], and it is connected to 5GC,

-	NE-DC, if and only if it is configured with mrdc-SecondaryCellGroup set to eutra-SCG,

-	NR-DC, if and only if it is configured with mrdc-SecondaryCellGroup set to nr-SCG,

-	MR-DC, if and only if it is in (NG)EN-DC, NE-DC or NR-DC.

NOTE:	This use of these terms deviates from the definition in TS 37.340 [41] and other specifications. In TS 37.340, these terms include also the case where the UE is configured with E-UTRA or NR MCG only (i.e. no NR or E-UTRA SCG) but with one or more bearers terminated in a secondary node (i.e. using NR PDCP).

The UE in (NG)EN-DC only executes a sub-clause of clause 5 in this specification when the subclause:

-	is referred to from a subclause under execution, either in this specification or in TS 36.331 [10]; or

-	applies to a message received on SRB3 (if SRB3 is established); or

-	applies to field(s), IE(s), UE variable(s) or timer(s) in this specification that the UE is configured with.

When executing a subclause of clause 5 in this specification, the UE follows the requirements in clause 5.1.2 and in all subclauses of this specification applicable to the messages (including processing time requirements), fields, IEs, timers and UE variables indicated in the subclause under execution.

## 5.2	System information

### 5.2.1	Introduction

System Information (SI) is divided into the MIB and a number of SIBs where:

-	the MIB is always transmitted on the BCH with a periodicity of 80 ms and repetitions made within 80 ms (TS 38.212 [17], clause 7.1) and it includes parameters that are needed to acquire SIB1 from the cell. The first transmission of the MIB is scheduled in subframes as defined in TS 38.213 [13], clause 4.1 and repetitions are scheduled according to the period of SSB;

-	the SIB1 is transmitted on the DL-SCH with a periodicity of 160 ms and variable transmission repetition periodicity within 160 ms as specified in TS 38.213 [13], clause 13. The default transmission repetition periodicity of SIB1 is 20 ms but the actual transmission repetition periodicity is up to network implementation. For SSB and CORESET multiplexing pattern 1, SIB1 repetition transmission period is 20 ms. For SSB and CORESET multiplexing pattern 2/3, SIB1 transmission repetition period is the same as the SSB period (TS 38.213 [13], clause 13). SIB1 includes information regarding the availability and scheduling (e.g. mapping of SIBs to SI message, periodicity, SI-window size) of other SIBs with an indication whether one or more SIBs are only provided on-demand and, in that case, the configuration needed by the UE to perform the SI request. SIB1 is cell-specific SIB;

-	SIBs other than SIB1 are carried in SystemInformation (SI) messages, which are transmitted on the DL-SCH. Only SIBs having the same periodicity can be mapped to the same SI message. Each SI message is transmitted within periodically occurring time domain windows (referred to as SI-windows with same length for all SI messages). Each SI message is associated with an SI-window and the SI-windows of different SI messages do not overlap. That is, within one SI-window only the corresponding SI message is transmitted. An SI message may be transmitted a number of times within the SI-window. Any SIB except SIB1 can be configured to be cell specific or area specific, using an indication in SIB1. The cell specific SIB is applicable only within a cell that provides the SIB while the area specific SIB is applicable within an area referred to as SI area, which consists of one or several cells and is identified by systemInformationAreaID;

-	For a UE in RRC_CONNECTED, the network can provide system information through dedicated signalling using the RRCReconfiguration message, e.g. if the UE has an active BWP with no common search space configured to monitor system information or paging.

-	For PSCell and SCells, the network provides the required SI by dedicated signalling, i.e. within an RRCReconfiguration message. Nevertheless, the UE shall acquire MIB of the PSCell to get SFN timing of the SCG (which may be different from MCG). Upon change of relevant SI for SCell, the network releases and adds the concerned SCell. For PSCell, the required SI can only be changed with Reconfiguration with Sync.

NOTE:	The physical layer imposes a limit to the maximum size a SIB can take. The maximum SIB1 or SI message size is 2976 bits.

### 5.2.2	System information acquisition

#### 5.2.2.1	General UE requirements

Figure 5.2.2.1-1: System information acquisition

The UE applies the SI acquisition procedure to acquire the AS- and NAS information. The procedure applies to UEs in RRC_IDLE, in RRC_INACTIVE and in RRC_CONNECTED.

The UE in RRC_IDLE and RRC_INACTIVE shall ensure having a valid version of (at least) the MIB, SIB1 through SIB4 and SIB5 (if the UE supports E-UTRA).

#### 5.2.2.2	SIB validity and need to (re)-acquire SIB

##### 5.2.2.2.1	SIB validity

The UE shall apply the SI acquisition procedure as defined in clause 5.2.2.3 upon cell selection (e.g. upon power on), cell-reselection, return from out of coverage, after reconfiguration with sync completion, after entering the network from another RAT, upon receiving an indication that the system information has changed, upon receiving a PWS notification; and whenever the UE does not have a valid version of a stored SIB.

When the UE acquires a MIB or a SIB1 or an SI message in a serving cell as described in clause 5.2.2.3, and if the UE stores the acquired SIB, then the UE shall store the associated areaScope, if present, the first PLMN-Identity in the PLMN-IdentityInfoList, the cellIdentity, the systemInformationAreaID, if present, and the valueTag, if present, as indicated in the si-SchedulingInfo for the SIB. The UE may use a valid stored version of the SI except MIB, SIB1, SIB6, SIB7 or SIB8 e.g. after cell re-selection, upon return from out of coverage or after the reception of SI change indication.

NOTE:	The storage and management of the stored SIBs in addition to the SIBs valid for the current serving cell is left to UE implementation.

The UE shall:

1>	delete any stored version of a SIB after 3 hours from the moment it was successfully confirmed as valid;

1>	for each stored version of a SIB:

2>	if the areaScope is associated and its value for the stored version of the SIB is the same as the value received in the si-SchedulingInfo for that SIB from the serving cell:

3>	if the first PLMN-Identity included in the PLMN-IdentityInfoList, the systemInformationAreaID and the valueTag that are included in the si-SchedulingInfo for the SIB received from the serving cell are identical to the PLMN-Identity, the systemInformationAreaID and the valueTag associated with the stored version of that SIB:

4>	consider the stored SIB as valid for the cell;

2>	if the areaScope is not present for the stored version of the SIB and the areaScope value is not included in the si-SchedulingInfo for that SIB from the serving cell:

3>	if the first PLMN-Identity in the PLMN-IdentityInfoList, the cellIdentity and valueTag that are included in the si-SchedulingInfo for the SIB received from the serving cell are identical to the PLMN-Identity, the cellIdentity and the valueTag associated with the stored version of that SIB:

4>	consider the stored SIB as valid for the cell;

##### 5.2.2.2.2	SI change indication and PWS notification

A modification period is used, i.e. updated SI (other than for ETWS and CMAS) is broadcasted in the modification period following the one where SI change indication is transmitted. The modification period boundaries are defined by SFN values for which SFN mod m = 0, where m is the number of radio frames comprising the modification period. The modification period is configured by system information. The UE receives indications about SI modifications and/or PWS notifications using Short Message transmitted with P-RNTI over DCI (see clause 6.5). Repetitions of SI change indication may occur within preceding modification period.

UEs in RRC_IDLE or in RRC_INACTIVE shall monitor for SI change indication in its own paging occasion every DRX cycle. UEs in RRC_CONNECTED shall monitor for SI change indication in any paging occasion at least once per modification period if the UE is provided with common search space on the active BWP to monitor paging, as specified in TS 38.213 [13], clause 13.

ETWS or CMAS capable UEs in RRC_IDLE or in RRC_INACTIVE shall monitor for indications about PWS notification in its own paging occasion every DRX cycle. ETWS or CMAS capable UEs in RRC_CONNECTED shall monitor for indication about PWS notification in any paging occasion at least once every defaultPagingCycle if the UE is provided with common search space on the active BWP to monitor paging.

For Short Message reception in a paging occasion, the UE monitors the PDCCH monitoring occasion(s) for paging as specified in TS 38.304 [20] and TS 38.213 [13].

If the UE receives a Short Message, the UE shall:

1>	if the UE is ETWS capable or CMAS capable, the etwsAndCmasIndication bit of Short Message is set, and the UE is provided with searchSpaceOtherSystemInformation on the active BWP or the initial BWP:

2> immediately re-acquire the SIB1;

2>	if the UE is ETWS capable and si-SchedulingInfo includes scheduling information for SIB6:

3>	acquire SIB6, as specified in sub-clause 5.2.2.3.2, immediately;

2>	if the UE is ETWS capable and si-SchedulingInfo includes scheduling information for SIB7:

3>	acquire SIB7, as specified in sub-clause 5.2.2.3.2, immediately;

2>	if the UE is CMAS capable and si-SchedulingInfo includes scheduling information for SIB8:

3>	acquire SIB8, as specified in sub-clause 5.2.2.3.2, immediately;

NOTE:	In case SIB6, SIB7, or SIB8 overlap with a measurement gap it is left to UE implementation how to immediately acquire SIB6, SIB7, or SIB8.

1> if the systemInfoModification bit of Short Message is set:

2>	apply the SI acquisition procedure as defined in sub-clause 5.2.2.3 from the start of the next modification period.

#### 5.2.2.3	Acquisition of System Information

##### 5.2.2.3.1	Acquisition of MIB and SIB1

The UE shall:

1>	apply the specified BCCH configuration defined in 9.1.1.1;

1>	if the UE is in RRC_IDLE or in RRC_INACTIVE; or

1>	if the UE is in RRC_CONNECTED while T311 is running:

2>	acquire the MIB, which is scheduled as specified in TS 38.213 [13];

2>	if the UE is unable to acquire the MIB;

3>	perform the actions as specified in clause 5.2.2.5;

2>	else:

3>	perform the actions specified in clause 5.2.2.4.1.

1>	if the UE is in RRC_CONNECTED with an active BWP with common search space configured by searchSpaceSIB1 and pagingSearchSpace and has received an indication about change of system information; or

1>	if the UE is in RRC_IDLE or in RRC_INACTIVE; or

1>	if the UE is in RRC_CONNECTED while T311 is running:

2>	if ssb-SubcarrierOffset indicates SIB1 is transmitted in the cell (TS 38.213 [13]) and if SIB1 acquisition is required for the UE:

3>	acquire the SIB1, which is scheduled as specified in TS 38.213 [13];

3>	if the UE is unable to acquire the SIB1:

4>	perform the actions as specified in clause 5.2.2.5;

3>	else:

4>	upon acquiring SIB1, perform the actions specified in clause 5.2.2.4.2.

2>	else if SIB1 acquisition is required for the UE and ssb-SubcarrierOffset indicates that SIB1 is not scheduled in the cell:

3>	perform the actions as specified in clause 5.2.2.5.

NOTE:	The UE in RRC_CONNECTED is only required to acquire broadcasted SIB1 if the UE can acquire it without disrupting unicast data reception, i.e. the broadcast and unicast beams are quasi co-located.

##### 5.2.2.3.2	Acquisition of an SI message

For SI message acquisition PDCCH monitoring occasion(s) are determined according to searchSpaceOtherSystemInformation. If searchSpaceOtherSystemInformation is set to zero, PDCCH monitoring occasions for SI message reception in SI-window are same as PDCCH monitoring occasions for SIB1 where the mapping between PDCCH monitoring occasions and SSBs is specified in TS 38.213[13]. If searchSpaceOtherSystemInformation is not set to zero, PDCCH monitoring occasions for SI message are determined based on search space indicated by searchSpaceOtherSystemInformation. PDCCH monitoring occasions for SI message which are not overlapping with UL symbols (determined according to tdd-UL-DL-ConfigurationCommon) are sequentially numbered from one in the SI window. The [x×N+K]th PDCCH monitoring occasion (s) for SI message in SI-window corresponds to the Kth transmitted SSB, where x = 0, 1, ...X-1, K = 1, 2, …N, N is the number of actual transmitted SSBs determined according to ssb-PositionsInBurst in SIB1 and X is equal to CEIL(number of PDCCH monitoring occasions in SI-window/N). The actual transmitted SSBs are sequentially numbered from one in ascending order of their SSB indexes. The UE assumes that, in the SI window, PDCCH for an SI message is transmitted in at least one PDCCH monitoring occasion corresponding to each transmitted SSB and thus the selection of SSB for the reception SI messages is up to UE implementation.

When acquiring an SI message, the UE shall:

1>	determine the start of the SI-window for the concerned SI message as follows:

2>	for the concerned SI message, determine the number n which corresponds to the order of entry in the list of SI messages configured by schedulingInfoList in si-SchedulingInfo in SIB1;

2>	determine the integer value x = (n – 1) × w, where w is the si-WindowLength;

2>	the SI-window starts at the slot #a, where a = x mod N, in the radio frame for which SFN mod T = FLOOR(x/N), where T is the si-Periodicity of the concerned SI message and N is the number of slots in a radio frame as specified in TS 38.213 [13];

1>	receive the PDCCH containing the scheduling RNTI, i.e. SI-RNTI in the PDCCH monitoring occasion(s) for SI message acquisition, from the start of the SI-window and continue until the end of the SI-window whose absolute length in time is given by si-WindowLength, or until the SI message was received;

1>	if the SI message was not received by the end of the SI-window, repeat reception at the next SI-window occasion for the concerned SI message in the current modification period;

NOTE 1:	The UE is only required to acquire broadcasted SI message if the UE can acquire it without disrupting unicast data reception, i.e. the broadcast and unicast beams are quasi co-located.

NOTE 2:	The UE is not required to monitor PDCCH monitoring occasion(s) corresponding to each transmitted SSB in SI-window.

NOTE 3:	If the concerned SI message was not received in the current modification period, handling of SI message acquisition is left to UE implementation.

1>	perform the actions for the acquired SI message as specified in sub-clause 5.2.2.4.

##### 5.2.2.3.3	Request for on demand system information

The UE shall:

1>	if SIB1 includes si-SchedulingInfo containing si-RequestConfigSUL and criteria to select supplementary uplink as defined in TS 38.321[13], clause 5.1.1 is met:

2>	trigger the lower layer to initiate the Random Access procedure on supplementary uplink in accordance with [3] using the PRACH preamble(s) and PRACH resource(s) in si-RequestConfigSUL corresponding to the SI message(s) that the UE requires to operate within the cell, and for which si-BroadcastStatus is set to notBroadcasting;

2>	if acknowledgement for SI request is received from lower layers:

3>	acquire the requested SI message(s) as defined in sub-clause 5.2.2.3.2, immediately;

1>	else if SIB1 includes si-SchedulingInfo containing si-RequestConfig and criteria to select normal uplink as defined in TS 38.321[13], clause 5.1.1 is met:

2>	trigger the lower layer to initiate the random access procedure on normal uplink in accordance with TS 38.321 [3] using the PRACH preamble(s) and PRACH resource(s) in si-RequestConfig corresponding to the SI message(s) that the UE requires to operate within the cell, and for which si-BroadcastStatus is set to notBroadcasting;

2>	if acknowledgement for SI request is received from lower layers:

3>	acquire the requested SI message(s) as defined in sub-clause 5.2.2.3.2, immediately;

1>	else:

2>	apply the default L1 parameter values as specified in corresponding physical layer specifications except for the parameters for which values are provided in SIB1;

2>	apply the default MAC Cell Group configuration as specified in 9.2.2;

2>	apply the timeAlignmentTimerCommon included in SIB1;

2>	apply the CCCH configuration as specified in 9.1.1.2;

2>	initiate transmission of the RRCSystemInfoRequest message in accordance with 5.2.2.3.4;

2>	if acknowledgement for RRCSystemInfoRequest message is received from lower layers:

3>	acquire the requested SI message(s) as defined in sub-clause 5.2.2.3.2, immediately;

1>	if cell reselection occurs while waiting for the acknowledgment for SI request from lower layers:

2>	reset MAC;

2>	if SI request is based on RRCSystemInfoRequest message:

3>	release RLC entity for SRB0.

NOTE:	After RACH failure for SI request it is up to UE implementation when to retry the SI request.

##### 5.2.2.3.4	Actions related to transmission of RRCSystemInfoRequest message

The UE shall set the contents of RRCSystemInfoRequest message as follows:

1>	set the requested-SI-List to indicate the SI message(s) that the UE requires to operate within the cell, and for which si-BroadcastStatus is set to notBroadcasting.

The UE shall submit the RRCSystemInfoRequest message to lower layers for transmission.

#### 5.2.2.4	Actions upon receipt of System Information

##### 5.2.2.4.1	Actions upon reception of the MIB

Upon receiving the MIB the UE shall:

1>	store the acquired MIB;

1>	if the UE is in RRC_IDLE or in RRC_INACTIVE, or if the UE is in RRC_CONNECTED while T311 is running:

2>	if the cellBarred in the acquired MIB is set to barred:

3>	consider the cell as barred in accordance with TS 38.304 [20];

3>	if intraFreqReselection is set to notAllowed:

4>	consider cell re-selection to other cells on the same frequency as the barred cell as not allowed, as specified in TS 38.304 [20].

3>	else:

4>	consider cell re-selection to other cells on the same frequency as the barred cell as allowed, as specified in TS 38.304 [20].

2>	else:

3>	apply the received systemFrameNumber, pdcch-ConfigSIB1, subCarrierSpacingCommon, ssb-SubcarrierOffset and dmrs-TypeA-Position.

##### 5.2.2.4.2	Actions upon reception of the SIB1

Upon receiving the SIB1 the UE shall:

1>	store the acquired SIB1;

1>	if the cellAccessRelatedInfo contains an entry with the PLMN-Identity of the selected PLMN:

2>	in the remainder of the procedures use plmn-IdentityList, trackingAreaCode, and cellIdentity for the cell as received in the corresponding PLMN-IdentityInfo containing the selected PLMN;

1>	if in RRC_CONNECTED while T311 is not running:

2>	disregard the frequencyBandList, if received, while in RRC_CONNECTED;

2>	forward the cellIdentity to upper layers;

2>	forward the trackingAreaCode to upper layers;

2>	apply the configuration included in the servingCellConfigCommon;

1>	else:

2>	if the UE supports one or more of the frequency bands indicated in the frequencyBandList for downlink for TDD, or one or more of the frequency bands indicated in the frequencyBandList for uplink for FDD, and they are not downlink only bands, and

2>	if the UE supports at least one additionalSpectrumEmission in the NR-NS-PmaxList for a supported band in the downlink for TDD, or a supported band in uplink for FDD, and

2>	if the UE supports an uplink channel bandwidth with a maximum transmission bandwidth configuration (see TS 38.101-1 [15] and TS 38.101-2 [39]) which

-	is smaller than or equal to the carrierBandwidth (indicated in uplinkConfigCommon for the SCS of the initial uplink BWP), and which

-	is wider than or equal to the bandwidth of the initial uplink BWP, and

2>	if the UE supports a downlink channel bandwidth with a maximum transmission bandwidth configuration (see TS 38.101-1 [15] and TS 38.101-2 [39]) which

-	is smaller than or equal to the carrierBandwidth (indicated in downlinkConfigCommon for the SCS of the initial downlink BWP), and which

-	is wider than or equal to the bandwidth of the initial downlink BWP:

3>	if trackingAreaCode is not provided for the selected PLMN nor the registered PLMN nor PLMN of the equivalent PLMN list:

4>	consider the cell as barred in accordance with TS 38.304 [20];

4>	if intraFreqReselection is set to notAllowed:

5>	consider cell re-selection to other cells on the same frequency as the barred cell as not allowed, as specified in TS 38.304 [20];

4>	else:

5>	consider cell re-selection to other cells on the same frequency as the barred cell as allowed, as specified in TS 38.304 [20];

3>	else:

4>	apply a supported uplink channel bandwidth with a maximum transmission bandwidth which

-	is contained within the carrierBandwidth indicated in uplinkConfigCommon for the SCS of the initial uplink BWP, and which

-	is wider than or equal to the bandwidth of the initial BWP for the uplink;

4>	apply a supported downlink channel bandwidth with a maximum transmission bandwidth which

- is contained within the carrierBandwidth indicated in downlinkConfigCommon for the SCS of the initial downlink BWP, and which

- is wider than or equal to the bandwidth of the initial BWP for the downlink;

4>	select the first frequency band in the frequencyBandList, for FDD from frequencyBandList for uplink, or for TDD from frequencyBandList for downlink, which the UE supports and for which the UE supports at least one of the additionalSpectrumEmission values in nr-NS-PmaxList, if present;

4>	forward the cellIdentity to upper layers;

4>	forward the trackingAreaCode to upper layers;

4>	forward the PLMN identity to upper layers;

4>	if in RRC_INACTIVE and the forwarded information does not trigger message transmission by upper layers:

5>	if the serving cell does not belong to the configured ran-NotificationAreaInfo:

6>	initiate an RNA update as specified in 5.3.13.8;

4>	forward the ims-EmergencySupport to upper layers, if present;

4>	forward the uac-AccessCategory1-SelectionAssistanceInfo to upper layers, if present;

4>	apply the configuration included in the servingCellConfigCommon;

4>	apply the specified PCCH configuration defined in 9.1.1.3;

4>	if the UE has a stored valid version of a SIB, in accordance with sub-clause 5.2.2.2.1, that the UE requires to operate within the cell in accordance with sub-clause 5.2.2.1:

5>	use the stored version of the required SIB;

4>	if the UE has not stored a valid version of a SIB, in accordance with sub-clause 5.2.2.2.1, of one or several required SIB(s), in accordance with sub-clause 5.2.2.1:

5>	for the SI message(s) that, according to the si-SchedulingInfo, contain at least one required SIB and for which si-BroadcastStatus is set to broadcasting:

6>	acquire the SI message(s) as defined in sub-clause 5.2.2.3.2;

5>	for the SI message(s) that, according to the si-SchedulingInfo, contain at least one required SIB and for which si-BroadcastStatus is set to notBroadcasting:

6>	trigger a request to acquire the SI message(s) as defined in sub-clause 5.2.2.3.3;

4>	apply the first listed additionalSpectrumEmission which it supports among the values included in NR-NS-PmaxList within frequencyBandList in uplinkConfigCommon for FDD or in downlinkConfigCommon for TDD;

4>	if the additionalPmax is present in the same entry of the selected additionalSpectrumEmission within NR-NS-PmaxList:

5>	apply the additionalPmax for UL;

4>	else:

5>	apply the p-Max in uplinkConfigCommon for UL;

4>	if supplementaryUplink is present in servingCellConfigCommon; and

4>	if the UE supports one or more of the frequency bands indicated in the frequencyBandList of supplementary uplink; and

4>	if the UE supports at least one additionalSpectrumEmission in the NR-NS-PmaxList for a supported supplementary uplink band; and

4>	if the UE supports an uplink channel bandwidth with a maximum transmission bandwith configuration (see TS 38.101-1 [15] and TS 38.101-2 [39]) which

-	is smaller than or equal to the carrierBandwidth (indicated in supplementaryUplink for the SCS of the initial uplink BWP), and which

-	is wider than or equal to the bandwidth of the initial uplink BWP of the SUL:

5>	consider supplementary uplink as configured in the serving cell;

5>	select the first frequency band in the frequencyBandList of supplementary uplink which the UE supports and for which the UE supports at least one of the additionalSpectrumEmission values in nr-NS-PmaxList, if present;

5>	apply a supported supplementary uplink channel bandwidth with a maximum transmission bandwidth which

-	is contained withn the carrierBandwidth (indicated in supplementaryUplink for the SCS of the initial uplink BWP), and which

-	is wider than or equal to the bandwidth of the initial BWP of the SUL;

5>	apply the first listed additionalSpectrumEmission which it supports among the values included in NR-NS-PmaxList within frequencyBandList for the supplementaryUplink;

5>	if the additionalPmax is present in the same entry of the selected additionalSpectrumEmission within NR-NS-PmaxList for the supplementaryUplink:

6>	apply the additionalPmax in supplementaryUplink for SUL;

5>	else:

6>	apply the p-Max in supplementaryUplink for SUL;

2>	else:

3>	consider the cell as barred in accordance with TS 38.304 [20]; and

3>	perform barring as if intraFreqReselection is set to notAllowed;

##### 5.2.2.4.3	Actions upon reception of SIB2

Upon receiving SIB2, the UE shall:

1>	if in RRC_IDLE or in RRC_INACTIVE or in RRC_CONNECTED while T311 is running:

2>	if, for the entry in frequencyBandList with the same index as the frequency band selected in clause 5.2.2.4.2, the UE supports at least one additionalSpectrumEmission in the NR-NS-PmaxList within the frequencyBandList:

3>	apply the first listed additionalSpectrumEmission which it supports among the values included in NR-NS-PmaxList within frequencyBandList;

3>	if the additionalPmax is present in the same entry of the selected additionalSpectrumEmission within NR-NS-PmaxList:

4>	apply the additionalPmax;

3>	else:

4>	apply the p-Max;

3>	if the UE selects a frequency band (from the procedure in clause 5.2.2.4.2) for the supplementary uplink:

4>	if, for the entry in frequencyBandListSUL with the same index as the frequency band selected in clause 5.2.2.4.2, the UE supports at least one additionalSpectrumEmission in the NR-NS-PmaxList within the frequencyBandListSUL:

5>	apply the first listed additionalSpectrumEmission which it supports among the values included in NR-NS-PmaxList within frequencyBandListSUL;

5>	if the additionalPmax is present in the same entry of the selected additionalSpectrumEmission within NR-NS-PmaxListSUL:

6>	apply the additionalPmax;

5>	else:

6>	apply the p-Max;

4>	else:

5>	apply the p-Max.

2>	else:

3>	apply the p-Max;

##### 5.2.2.4.4	Actions upon reception of SIB3

No UE requirements related to the contents of this SIB3 apply other than those specified elsewhere e.g. within procedures using the concerned system information, and/ or within the corresponding field descriptions.

##### 5.2.2.4.5	Actions upon reception of SIB4

Upon receiving SIB4 the UE shall:

1>	if in RRC_IDLE, or in RRC_INACTIVE or in RRC_CONNECTED while T311 is running:

2>	for each entry in the interFreqCarrierFreqList:

3>	select the first frequency band in the frequencyBandList, and frequencyBandListSUL, if present, which the UE supports and for which the UE supports at least one of the additionalSpectrumEmission values in NR-NS-PmaxList, if present:

3>	if, the frequency band selected by the UE in frequencyBandList to represent a non-serving NR carrier frequency is not a downlink only band:

4>	if, for the selected frequency band, the UE supports at least one additionalSpectrumEmission in the NR-NS-PmaxList within the frequencyBandList:

5>	apply the first listed additionalSpectrumEmission which it supports among the values included in NR-NS-PmaxList within frequencyBandList;

5>	if the additionalPmax is present in the same entry of the selected additionalSpectrumEmission within NR-NS-PmaxList:

6>	apply the additionalPmax;

5>	else:

6>	apply the p-Max;

5>	if frequencyBandListSUL is present in SIB4 and, for the frequency band selected in frequencyBandListSUL, the UE supports at least one additionalSpectrumEmission in the NR-NS-PmaxList within FrequencyBandListSUL:

6>	apply the first listed additionalSpectrumEmission which it supports among the values inlcuded in NR-NS-PmaxList within frequencyBandListSUL;

6>	if the additionalPmax is present in the same entry of the selected additionalSpectrumEmission within NR-NS-PmaxList:

7>	apply the additionalPmax;

6>	else:

7>	apply the p-Max;

5>	else:

6>	apply the p-Max;

4>	else:

5>	apply the p-Max;

##### 5.2.2.4.6	Actions upon reception of SIB5

No UE requirements related to the contents of this SIB5 apply other than those specified elsewhere e.g. within procedures using the concerned system information, and/ or within the corresponding field descriptions.

##### 5.2.2.4.7	Actions upon reception of SIB6

Upon receiving the SIB6 the UE shall:

1>	forward the received warningType, messageIdentifier and serialNumber to upper layers;

##### 5.2.2.4.8	Actions upon reception of SIB7

Upon receiving the SIB7 the UE shall:

1>	if there is no current value for messageIdentifier and serialNumber for SIB7; or

1>	if either the received value of messageIdentifier or of serialNumber, or of both messageIdentifier and serialNumber are different from the current values of messageIdentifier and serialNumber for SIB7:

2>	use the received values of messageIdentifier and serialNumber for SIB7 as the current values of messageIdentifier and serialNumber for SIB7;

2>	discard any previously buffered warningMessageSegment;

2>	if all segments of a warning message have been received:

3>	assemble the warning message from the received warningMessageSegment(s);

3>	forward the received warning message, messageIdentifier, serialNumber and dataCodingScheme to upper layers;

3>	stop reception of SIB7;

3>	discard the current values of messageIdentifier and serialNumber for SIB7;

2>	else:

3>	store the received warningMessageSegment;

3>	continue reception of SIB7;

1>	else if all segments of a warning message have been received:

2>	assemble the warning message from the received warningMessageSegment(s);

2>	forward the received complete warning message, messageIdentifier, serialNumber and dataCodingScheme to upper layers;

2>	stop reception of SIB7;

2>	discard the current values of messageIdentifier and serialNumber for SIB7;

1>	else:

2>	store the received warningMessageSegment;

2>	continue reception of SIB7;

The UE should discard any stored warningMessageSegment and the current value of messageIdentifier and serialNumber for SIB7 if the complete warning message has not been assembled within a period of 3 hours.

##### 5.2.2.4.9	Actions upon reception of SIB8

Upon receiving the SIB8 the UE shall:

1>	if the SIB8 contains a complete warning message and the complete geographical area coordinates (if any):

2>	forward the received warning message, messageIdentifier, serialNumber, dataCodingScheme and the geographical area coordinates (if any) to upper layers;

2>	continue reception of SIB8;

1>	else:

2>	if the received values of messageIdentifier and serialNumber are the same (each value is the same) as a pair for which a warning message and the geographical area coordinates (if any) are currently being assembled:

3>	store the received warningMessageSegment;

3>	store the received warningAreaCoordinatesSegment (if any);

3>	if all segments of a warning message and geographical area coordinates (if any) have been received:

4>	assemble the warning message from the received warningMessageSegment;

4>	assemble the geographical area coordinates from the received warningAreaCoordinatesSegment (if any);

4>	forward the received warning message, messageIdentifier, serialNumber, dataCodingScheme and geographical area coordinates (if any) to upper layers;

4>	stop assembling a warning message and geographical area coordinates (if any) for this messageIdentifier and serialNumber and delete all stored information held for it;

3>	continue reception of SIB8;

2>	else if the received values of messageIdentifier and/or serialNumber are not the same as any of the pairs for which a warning message is currently being assembled:

3>	start assembling a warning message for this messageIdentifier and serialNumber pair;

3>	start assembling the geographical area coordinates (if any) for this messageIdentifier and serialNumber pair;

3>	store the received warningMessageSegment;

3>	store the received warningAreaCoordinatesSegment (if any);

3>	continue reception of SIB8;

The UE should discard warningMessageSegment and warningAreaCoordinatesSegment (if any) and the associated values of messageIdentifier and serialNumber for SIB8 if the complete warning message and the geographical area coordinates (if any) have not been assembled within a period of 3 hours.

NOTE:	The number of warning messages that a UE can re-assemble simultaneously is a function of UE implementation.

##### 5.2.2.4.10	Actions upon reception of SIB9

No UE requirements related to the contents of this SIB9 apply other than those specified elsewhere e.g. within procedures using the concerned system information, and/ or within the corresponding field descriptions.

#### 5.2.2.5	Essential system information missing

The UE shall:

1>	if in RRC_IDLE or in RRC_INACTIVE or in RRC_CONNECTED while T311 is running:

2>	if the UE is unable to acquire the MIB:

3>	consider the cell as barred in accordance with TS 38.304 [20]; and

3>	perform barring as if intraFreqReselection is set to allowed;

2>	else if the UE is unable to acquire the SIB1:

3>	consider the cell as barred in accordance with TS 38.304 [20].

3>	if intraFreqReselection in MIB is set to notAllowed:

4>	consider cell re-selection to other cells on the same frequency as the barred cell as not allowed, as specified in TS 38.304 [20].

3>	else:

4>	consider cell re-selection to other cells on the same frequency as the barred cell as allowed, as specified in TS 38.304 [20].

## 5.3	Connection control

### 5.3.1	Introduction

#### 5.3.1.1	RRC connection control

RRC connection establishment involves the establishment of SRB1. The network completes RRC connection establishment prior to completing the establishment of the NG connection, i.e. prior to receiving the UE context information from the 5GC. Consequently, AS security is not activated during the initial phase of the RRC connection. During this initial phase of the RRC connection, the network may configure the UE to perform measurement reporting, but the UE only sends the corresponding measurement reports after successful AS security activation. However, the UE only accepts a re-configuration with sync message when AS security has been activated.

Upon receiving the UE context from the 5GC, the RAN activates AS security (both ciphering and integrity protection) using the initial AS security activation procedure. The RRC messages to activate AS security (command and successful response) are integrity protected, while ciphering is started only after completion of the procedure. That is, the response to the message used to activate AS security is not ciphered, while the subsequent messages (e.g. used to establish SRB2 and DRBs) are both integrity protected and ciphered. After having initiated the initial AS security activation procedure, the network may initiate the establishment of SRB2 and DRBs, i.e. the network may do this prior to receiving the confirmation of the initial AS security activation from the UE. In any case, the network will apply both ciphering and integrity protection for the RRC reconfiguration messages used to establish SRB2 and DRBs. The network should release the RRC connection if the initial AS security activation and/ or the radio bearer establishment fails. A configuration with SRB2 without DRB or with DRB without SRB2 is not supported (i.e., SRB2 and at least one DRB must be configured in the same RRC Reconfiguration message, and it is not allowed to release all the DRBs without releasing the RRC Connection).

The release of the RRC connection normally is initiated by the network. The procedure may be used to re-direct the UE to an NR frequency or an E-UTRA carrier frequency.

The suspension of the RRC connection is initiated by the network. When the RRC connection is suspended, the UE stores the UE Inactive AS context and any configuration received from the network, and transits to RRC_INACTIVE state. If the UE is configured with SCG, the UE releases the SCG configuration upon initiating a RRC Connection Resume procedure. The RRC message to suspend the RRC connection is integrity protected and ciphered.

The resumption of a suspended RRC connection is initiated by upper layers when the UE needs to transit from RRC_INACTIVE state to RRC_CONNECTED state or by RRC layer to perform a RNA update or by RAN paging from NG-RAN. When the RRC connection is resumed, network configures the UE according to the RRC connection resume procedure based on the stored UE Inactive AS context and any RRC configuration received from the network. The RRC connection resume procedure re-activates AS security and re-establishes SRB(s) and DRB(s).

In response to a request to resume the RRC connection, the network may resume the suspended RRC connection and send UE to RRC_CONNECTED, or reject the request to resume and send UE to RRC_INACTIVE (with a wait timer), or directly re-suspend the RRC connection and send UE to RRC_INACTIVE, or directly release the RRC connection and send UE to RRC_IDLE, or instruct the UE to initiate NAS level recovery (in this case the network sends an RRC setup message).

#### 5.3.1.2	AS Security

AS security comprises of the integrity protection and ciphering of RRC signalling (SRBs) and user data (DRBs).

RRC handles the configuration of the AS security parameters which are part of the AS configuration: the integrity protection algorithm, the ciphering algorithm, if integrity protection and/or ciphering is enabled for a DRB and two parameters, namely the keySetChangeIndicator and the nextHopChainingCount, which are used by the UE to determine the AS security keys upon reconfiguration with sync (with key change), connection re-establishment and/or connection resume.

The integrity protection algorithm is common for SRB1, SRB2, SRB3 (if configured) and DRBs configured with integrity protection, with the same keyToUse value. The ciphering algorithm is common for SRB1, SRB2, SRB3 (if configured) and DRBs configured with the same keyToUse value. For MR-DC, integrity protection is not enabled for DRBs terminated in eNB. Neither integrity protection nor ciphering applies for SRB0.

NOTE 0:	All DRBs related to the same PDU session have the same enable/disable setting for ciphering and the same enable/disable setting for integrity protection, as specified in TS 33.501 [11].

RRC integrity protection and ciphering are always activated together, i.e. in one message/procedure. RRC integrity protection and ciphering for SRBs are never de-activated. However, it is possible to switch to a 'NULL' ciphering algorithm (nea0).

The 'NULL' integrity protection algorithm (nia0) is used only for SRBs and for the UE in limited service mode, see TS 33.501 [11] and when used for SRBs, integrity protection is disabled for DRBs. In case the ′NULL' integrity protection algorithm is used, 'NULL' ciphering algorithm is also used.

NOTE 1:	Lower layers discard RRC messages for which the integrity protection check has failed and indicate the integrity protection verification check failure to RRC.

The AS applies four different security keys: one for the integrity protection of RRC signalling (KRRCint), one for the ciphering of RRC signalling (KRRCenc), one for integrity protection of user data (KUPint) and one for the ciphering of user data (KUPenc). All four AS keys are derived from the KgNB key. The KgNB key is based on the KAMF key (as specified in TS 33.501 [11]), which is handled by upper layers.

The integrity protection and ciphering algorithms can only be changed with reconfiguration with sync. The AS keys (KgNB, KRRCint, KRRCenc, KUPint and KUPenc) change upon reconfiguration with sync (if masterKeyUpdate is included), and upon connection re-establishment and connection resume.

For each radio bearer an independent counter (COUNT, as specified in TS 38.323 [5]) is maintained for each direction. For each radio bearer, the COUNT is used as input for ciphering and integrity protection.

It is not allowed to use the same COUNT value more than once for a given security key. As specified in TS 33.501 subclause 6.9.4.1 [11], the network is responsible for avoiding reuse of the COUNT with the same RB identity and with the same key, e.g. due to the transfer of large volumes of data, release and establishment of new RBs, and multiple termination point changes for RLC-UM bearers and multiple termination point changes for RLC-AM bearer with SN terminated PDCP re-establishment (COUNT reset) due to SN only full configuration whilst the key stream inputs (i.e. bearer ID, security key) at MN have not been updated. In order to avoid such re-use, the network may e.g. use different RB identities for RB establishments, change the AS security key, or an RRC_CONNECTED to RRC_IDLE/RRC_INACTIVE and then to RRC_CONNECTED transition.

In order to limit the signalling overhead, individual messages/ packets include a short sequence number (PDCP SN, as specified in TS 38.323 [5]). In addition, an overflow counter mechanism is used: the hyper frame number (HFN, as specified in TS 38.323 [5]). The HFN needs to be synchronized between the UE and the network.

For each SRB, the value provided by RRC to lower layers to derive the 5-bit BEARER parameter used as input for ciphering and for integrity protection is the value of the corresponding srb-Identity with the MSBs padded with zeroes.

For a UE provided with an sk-counter, keyToUse indicates whether the UE uses the master key (KgNB) or the secondary key (S-KeNB or S-KgNB) for a particular DRB. The secondary key is derived from the master key and sk-Counter, as defined in 33.501[86]. Whenever there is a need to refresh the secondary key, e.g. upon change of MN with KgNB change or to avoid COUNT wrap around, the security key update is used (see 5.3.5.7). When the UE is in NR-DC, the network may provide a UE configured with an SCG with an sk-Counter even when no DRB is setup using the secondary key (S-KgNB) in order to allow the configuration of SRB3. The network can also provide the UE with an sk-Counter, even if no SCG is configured, when using SN terminated MCG bearers.

### 5.3.2	Paging

#### 5.3.2.1	General

Figure 5.3.2.1-1: Paging

The purpose of this procedure is:

-	to transmit paging information to a UE in RRC_IDLE or RRC_INACTIVE.

#### 5.3.2.2	Initiation

The network initiates the paging procedure by transmitting the Paging message at the UE's paging occasion as specified in TS 38.304 [20]. The network may address multiple UEs within a Paging message by including one PagingRecord for each UE.

#### 5.3.2.3	Reception of the Paging message by the UE

Upon receiving the Paging message, the UE shall:

1>	if in RRC_IDLE, for each of the PagingRecord, if any, included in the Paging message:

2>	if the ue-Identity included in the PagingRecord matches the UE identity allocated by upper layers:

3>	forward the ue-Identity and accessType (if present) to the upper layers;

1>	if in RRC_INACTIVE, for each of the PagingRecord, if any, included in the Paging message:

2>	if the ue-Identity included in the PagingRecord matches the UE's stored fullI-RNTI:

3>	if the UE is configured by upper layers with Access Identity 1:

4>	initiate the RRC connection resumption procedure according to 5.3.13 with resumeCause set to mps-PriorityAccess;

3>	else if the UE is configured by upper layers with Access Identity 2:

4>	initiate the RRC connection resumption procedure according to 5.3.13 with resumeCause set to mcs-PriorityAccess;

3>	else if the UE is configured by upper layers with one or more Access Identities equal to 11-15:

4>	initiate the RRC connection resumption procedure according to 5.3.13 with resumeCause set to highPriorityAccess;

3>	else:

4>	initiate the RRC connection resumption procedure according to 5.3.13 with resumeCause set to mt-Access;

2>	else if the ue-Identity included in the PagingRecord matches the UE identity allocated by upper layers:

3>	forward the ue-Identity to upper layers and accessType (if present) to the upper layers;

3>	perform the actions upon going to RRC_IDLE as specified in 5.3.11 with release cause 'other'.

### 5.3.3	RRC connection establishment

#### 5.3.3.1	General

Figure 5.3.3.1-1: RRC connection establishment, successful

Figure 5.3.3.1-2: RRC connection establishment, network reject

The purpose of this procedure is to establish an RRC connection. RRC connection establishment involves SRB1 establishment. The procedure is also used to transfer the initial NAS dedicated information/ message from the UE to the network.

The network applies the procedure e.g.as follows:

-	When establishing an RRC connection;

-	When UE is resuming or re-establishing an RRC connection, and the network is not able to retrieve or verify the UE context. In this case, UE receives RRCSetup and responds with RRCSetupComplete.

#### 5.3.3.2	Initiation

The UE initiates the procedure when upper layers request establishment of an RRC connection while the UE is in RRC_IDLE and it has acquired essential system information as described in 5.2.2.1.

The UE shall ensure having valid and up to date essential system information as specified in clause 5.2.2.2 before initiating this procedure.

Upon initiation of the procedure, the UE shall:

1>	if the upper layers provide an Access Category and one or more Access Identities upon requesting establishment of an RRC connection:

2>	perform the unified access control procedure as specified in 5.3.14 using the Access Category and Access Identities provided by upper layers;

3>	if the access attempt is barred, the procedure ends;

1>	apply the default L1 parameter values as specified in corresponding physical layer specifications except for the parameters for which values are provided in SIB1;

1>	apply the default MAC Cell Group configuration as specified in 9.2.2;

1>	apply the CCCH configuration as specified in 9.1.1.2;

1>	apply the timeAlignmentTimerCommon included in SIB1;

1>	start timer T300;

1>	initiate transmission of the RRCSetupRequest message in accordance with 5.3.3.3;

#### 5.3.3.3	Actions related to transmission of RRCSetupRequest message

The UE shall set the contents of RRCSetupRequest message as follows:

1>	set the ue-Identity as follows:

2>	if upper layers provide a 5G-S-TMSI:

3>	set the ue-Identity to ng-5G-S-TMSI-Part1;

2>	else:

3>	draw a 39-bit random value in the range 0..239-1 and set the ue-Identity to this value;

NOTE 1:	Upper layers provide the 5G-S-TMSI if the UE is registered in the TA of the current cell.

1>	set the establishmentCause in accordance with the information received from upper layers;

The UE shall submit the RRCSetupRequest message to lower layers for transmission.

The UE shall continue cell re-selection related measurements as well as cell re-selection evaluation. If the conditions for cell re-selection are fulfilled, the UE shall perform cell re-selection as specified in 5.3.3.6.

#### 5.3.3.4	Reception of the RRCSetup by the UE

The UE shall perform the following actions upon reception of the RRCSetup:

1>	if the RRCSetup is received in response to an RRCReestablishmentRequest; or

1>	if the RRCSetup is received in response to an RRCResumeRequest or RRCResumeRequest1:

2>	discard any stored UE Inactive AS context and suspendConfig;

2>	discard any current AS security context including the KRRCenc key, the KRRCint key, the KUPint key and the KUPenc key;

2>	release radio resources for all established RBs except SRB0, including release of the RLC entities, of the associated PDCP entities and of SDAP;

2>	release the RRC configuration except for the default L1 parameter values, default MAC Cell Group configuration and CCCH configuration;

2>	indicate to upper layers fallback of the RRC connection;

2>	stop timer T380, if running;

1>	perform the cell group configuration procedure in accordance with the received masterCellGroup and as specified in 5.3.5.5;

1>	perform the radio bearer configuration procedure in accordance with the received radioBearerConfig and as specified in 5.3.5.6;

1>	if stored, discard the cell reselection priority information provided by the cellReselectionPriorities or inherited from another RAT;

1>	stop timer T300, T301 or T319 if running;

1>	if T390 is running:

2>	stop timer T390 for all access categories;

2>	perform the actions as specified in 5.3.14.4;

1>	if T302 is running:

2>	stop timer T302;

2>	perform the actions as specified in 5.3.14.4;

1>	stop timer T320, if running;

1>	if the RRCSetup is received in response to an RRCResumeRequest, RRCResumeRequest1 or RRCSetupRequest:

2>	enter RRC_CONNECTED;

2>	stop the cell re-selection procedure;

1>	consider the current cell to be the PCell;

1>	set the content of RRCSetupComplete message as follows:

2>	if upper layers provide a 5G-S-TMSI:

3>	if the RRCSetup is received in response to an RRCSetupRequest:

4>	set the ng-5G-S-TMSI-Value to ng-5G-S-TMSI-Part2;

3>	else:

4>	set the ng-5G-S-TMSI-Value to ng-5G-S-TMSI;

2>	set the selectedPLMN-Identity to the PLMN selected by upper layers (TS 24.501 [23]) from the PLMN(s) included in the plmn-IdentityList in SIB1;

2>	if upper layers provide the 'Registered AMF':

3>	include and set the registeredAMF as follows:

4>	if the PLMN identity of the 'Registered AMF' is different from the PLMN selected by the upper layers:

5>	include the plmnIdentity in the registeredAMF and set it to the value of the PLMN identity in the 'Registered AMF' received from upper layers;

4>	set the amf-Identifier to the value received from upper layers;

3>	include and set the guami-Type to the value provided by the upper layers;

2>	if upper layers provide one or more S-NSSAI (see TS 23.003 [21]):

3>	include the s-NSSAI-List and set the content to the values provided by the upper layers;

2>	set the dedicatedNAS-Message to include the information received from upper layers;

1>	submit the RRCSetupComplete message to lower layers for transmission, upon which the procedure ends.

#### 5.3.3.5	Reception of the RRCReject by the UE

The UE shall:

1>	perform the actions as specified in 5.3.15;

#### 5.3.3.6	Cell re-selection or cell selection while T390, T300 or T302 is running (UE in RRC_IDLE)

The UE shall:

1>	if cell reselection occurs while T300 or T302 is running:

2>	perform the actions upon going to RRC_IDLE as specified in 5.3.11 with release cause 'RRC connection failure';

1>	else if cell selection or reselection occurs while T390 is running:

2>	stop T390 for all access categories;

2>	perform the actions as specified in 5.3.14.4.

#### 5.3.3.7	T300 expiry

The UE shall:

1>	if timer T300 expires:

2>	reset MAC, release the MAC configuration and re-establish RLC for all RBs that are established;

2>	if the T300 has expired a consecutive connEstFailCount times on the same cell for which connEstFailureControl is included in SIB1:

3>	for a period as indicated by connEstFailOffsetValidity:

4>	use connEstFailOffset for the parameter Qoffsettemp for the concerned cell when performing cell selection and reselection according to TS 38.304 [20] and TS 36.304 [27];

NOTE:	When performing cell selection, if no suitable or acceptable cell can be found, it is up to UE implementation whether to stop using connEstFailOffset for the parameter Qoffsettemp during connEstFailOffsetValidity for the concerned cell.

2>	inform upper layers about the failure to establish the RRC connection, upon which the procedure ends;

#### 5.3.3.8	Abortion of RRC connection establishment

If upper layers abort the RRC connection establishment procedure, due to a NAS procedure being aborted as specified in TS 24.501 [23], while the UE has not yet entered RRC_CONNECTED, the UE shall:

1>	stop timer T300, if running;

1>	reset MAC, release the MAC configuration and re-establish RLC for all RBs that are established;

### 5.3.4	Initial AS security activation

#### 5.3.4.1	General

Figure 5.3.4.1-1: Security mode command, successful

Figure 5.3.4.1-2: Security mode command, failure

The purpose of this procedure is to activate AS security upon RRC connection establishment.

#### 5.3.4.2	Initiation

The network initiates the security mode command procedure to a UE in RRC_CONNECTED. Moreover, the network applies the procedure as follows:

-	when only SRB1 is established, i.e. prior to establishment of SRB2 and/ or DRBs.

#### 5.3.4.3	Reception of the SecurityModeCommand by the UE

The UE shall:

1>	derive the KgNB key, as specified in TS 33.501 [11];

1>	derive the KRRCint key associated with the integrityProtAlgorithm indicated in the SecurityModeCommand message, as specified in TS 33.501 [11];

1>	request lower layers to verify the integrity protection of the SecurityModeCommand message, using the algorithm indicated by the integrityProtAlgorithm as included in the SecurityModeCommand message and the KRRCint key;

1>	if the SecurityModeCommand message passes the integrity protection check:

2>	derive the KRRCenc key and the KUPenc key associated with the cipheringAlgorithm indicated in the SecurityModeCommand message, as specified in TS 33.501 [11];

2>	derive the KUPint key associated with the integrityProtAlgorithm indicated in the SecurityModeCommand message, as specified in TS 33.501 [11];

2>	configure lower layers to apply SRB integrity protection using the indicated algorithm and the KRRCint key immediately, i.e. integrity protection shall be applied to all subsequent messages received and sent by the UE, including the SecurityModeComplete message;

2>	configure lower layers to apply SRB ciphering using the indicated algorithm, the KRRCenc keyafter completing the procedure, i.e. ciphering shall be applied to all subsequent messages received and sent by the UE, except for the SecurityModeComplete message which is sent unciphered;

2>	consider AS security to be activated;

2>	submit the SecurityModeComplete message to lower layers for transmission, upon which the procedure ends;

1>	else:

2>	continue using the configuration used prior to the reception of the SecurityModeCommand message, i.e. neither apply integrity protection nor ciphering.

2>	submit the SecurityModeFailure message to lower layers for transmission, upon which the procedure ends.

### 5.3.5	RRC reconfiguration

#### 5.3.5.1	General

Figure 5.3.5.1-1: RRC reconfiguration, successful

Figure 5.3.5.1-2: RRC reconfiguration, failure

The purpose of this procedure is to modify an RRC connection, e.g. to establish/modify/release RBs, to perform reconfiguration with sync, to setup/modify/release measurements, to add/modify/release SCells and cell groups. As part of the procedure, NAS dedicated information may be transferred from the Network to the UE.

RRC reconfiguration to perform reconfiguration with sync includes, but is not limited to, the following cases:

-	reconfiguration with sync and security key refresh, involving RA to the PCell/PSCell, MAC reset, refresh of security and re-establishment of RLC and PDCP triggered by explicit L2 indicators;

-	reconfiguration with sync but without security key refresh, involving RA to the PCell/PSCell, MAC reset and RLC re-establishment and PDCP data recovery (for AM DRB) triggered by explicit L2 indicators.

In (NG)EN-DC and NR-DC, SRB3 can be used for measurement configuration and reporting, to (re-)configure MAC, RLC, physical layer and RLF timers and constants of the SCG configuration, and to reconfigure PDCP for DRBs associated with the S-KgNB or SRB3, and to reconfigure SDAP for DRBs associated with S-KgNB in NGEN-DC and NR-DC, provided that the (re-)configuration does not require any MN involvement. In (NG)EN-DC and NR-DC, only measConfig, radioBearerConfig and/or secondaryCellGroup are included in RRCReconfiguration received via SRB3.

#### 5.3.5.2	Initiation

The Network may initiate the RRC reconfiguration procedure to a UE in RRC_CONNECTED. The Network applies the procedure as follows:

-	the establishment of RBs (other than SRB1, that is established during RRC connection establishment) is performed only when AS security has been activated;

-	the addition of Secondary Cell Group and SCells is performed only when AS security has been activated;

-	the reconfigurationWithSync is included in secondaryCellGroup only when at least one RLC bearer is setup in SCG;

-	the reconfigurationWithSync is included in masterCellGroup only when AS security has been activated, and SRB2 with at least one DRB are setup and not suspended.

#### 5.3.5.3	Reception of an RRCReconfiguration by the UE

The UE shall perform the following actions upon reception of the RRCReconfiguration:

1>	if the RRCReconfiguration is received via other RAT (i.e., inter-RAT handover to NR):

2>	if the RRCReconfiguration does not include the fullConfig and the UE is connected to 5GC (i.e., delta signalling during intra 5GC handover):

3>	re-use the source RAT SDAP and PDCP configurations if available (i.e., current SDAP/PDCP configurations for all RBs from source E-UTRA RAT prior to the reception of the inter-RAT HO RRCReconfiguration message);

1>	else:

2>	if the RRCReconfiguration includes the fullConfig:

3>	perform the full configuration procedure as specified in 5.3.5.11;

1>	if the RRCReconfiguration includes the masterCellGroup:

2>	perform the cell group configuration for the received masterCellGroup according to 5.3.5.5;

1>	if the RRCReconfiguration includes the masterKeyUpdate:

2>	perform AS security key update procedure as specified in 5.3.5.7;

1>	if the RRCReconfiguration includes the sk-Counter:

2>	perform security key update procedure as specified in 5.3.5.7;

1>	if the RRCReconfiguration includes the secondaryCellGroup:

2>	perform the cell group configuration for the SCG according to 5.3.5.5;

1>	if the RRCReconfiguration includes the mrdc-SecondaryCellGroupConfig:

2>	if the mrdc-SecondaryCellGroupConfig is set to setup:

3>	if the mrdc-SecondaryCellGroupConfig includes mrdc-ReleaseAndAdd:

4>	perform MR-DC release as specified in clause 5.3.5.10;

3>	if the received mrdc-SecondaryCellGroup is set to nr-SCG:

4>	perform the RRC reconfiguration according to 5.3.5.3 for the RRCReconfiguration message included in nr-SCG;

3>	if the received mrdc-SecondaryCellGroup is set to eutra-SCG:

4>	perform the RRC connection reconfiguration as specified in TS 36.331 [10], clause 5.3.5.3 for the RRCConnectionReconfiguration message included in eutra-SCG;

2>	else (mrdc-SecondaryCellGroupConfig is set to release):

3>	perform MR-DC release as specified in clause 5.3.5.10;

1>	if the RRCReconfiguration message includes the radioBearerConfig:

2>	perform the radio bearer configuration according to 5.3.5.6;

1>	if the RRCReconfiguration message includes the radioBearerConfig2:

2>	perform the radio bearer configuration according to 5.3.5.6;

1>	if the RRCReconfiguration message includes the measConfig:

2>	perform the measurement configuration procedure as specified in 5.5.2;

1>	if the RRCReconfiguration message includes the dedicatedNAS-MessageList:

2>	forward each element of the dedicatedNAS-MessageList to upper layers in the same order as listed;

1>	if the RRCReconfiguration message includes the dedicatedSIB1-Delivery:

2>	perform the action upon reception of SIB1 as specified in 5.2.2.4.2;

1>	if the RRCReconfiguration message includes the dedicatedSystemInformationDelivery:

2>	perform the action upon reception of System Information as specified in 5.2.2.4;

1>	if the RRCReconfiguration message includes the otherConfig:

2>	perform the other configuration procedure as specified in 5.3.5.9;

1>	set the content of the RRCReconfigurationComplete message as follows:

2>	if the RRCReconfiguration includes the masterCellGroup containing the reportUplinkTxDirectCurrent:

3>	include the uplinkTxDirectCurrentList for each MCG serving cell with UL;

3>	include uplinkDirectCurrentBWP-SUL for each MCG serving cell configured with SUL carrier, if any, within the uplinkTxDirectCurrentList;

2>	if the RRCReconfiguration includes the secondaryCellGroup containing the reportUplinkTxDirectCurrent:

3>	include the uplinkTxDirectCurrentList for each SCG serving cell with UL;

3>	include uplinkDirectCurrentBWP-SUL for each SCG serving cell configured with SUL carrier, if any, within the uplinkTxDirectCurrentList;

2>	if the RRCReconfiguration message includes the mrdc-SecondaryCellGroupConfig with mrdc-SecondaryCellGroup set to eutra-SCG:

3>	include in the eutra-SCG-Response the E-UTRA RRCConnectionReconfigurationComplete message in accordance with TS 36.331 [10] clause 5.3.5.3;

2> if the RRCReconfiguration message includes the mrdc-SecondaryCellGroupConfig with mrdc-SecondaryCellGroup set to nr-SCG:

3>	include in the nr-SCG-Response the RRCReconfigurationComplete message;

1>	if the UE is configured with E-UTRA nr-SecondaryCellGroupConfig (UE in (NG)EN-DC):

2>	if the RRCReconfiguration message was received via E-UTRA SRB1 as specified in TS 36.331 [10]:

3>	submit the RRCReconfigurationComplete via E-UTRA embedded in E-UTRA RRC message RRCConnectionReconfigurationComplete as specified in TS 36.331 [10];

3>	if reconfigurationWithSync was included in spCellConfig of an SCG:

4>	initiate the Random Access procedure on the SpCell, as specified in TS 38.321 [3];

3>	else:

4>	the procedure ends;

NOTE 1:	The order the UE sends the RRCConnectionReconfigurationComplete message and performs the Random Access procedure towards the SCG is left to UE implementation.

2>	else (RRCReconfiguration was received via SRB3):

3>	submit the RRCReconfigurationComplete message via SRB3 to lower layers for transmission using the new configuration;

NOTE 2:	In (NG)EN-DC and NR-DC, in the case RRCReconfiguration is received via SRB1, the random access is triggered by RRC layer itself as there is not necessarily other UL transmission. In the case RRCReconfiguration is received via SRB3, the random access is triggered by the MAC layer due to arrival of RRCReconfigurationComplete.

1>	else if the RRCReconfiguration message was received within the nr-SCG within mrdc-SecondaryCellGroup (UE in NR-DC, mrdc-SecondaryCellGroup was received in RRCReconfiguration via SRB1):

2>	if reconfigurationWithSync was included in spCellConfig in nr-SCG:

3>	initiate the Random Access procedure on the PSCell, as specified in TS 38.321 [3];

2>	else

3>	the procedure ends;

NOTE 2a:	The order in which the UE sends the RRCReconfigurationComplete message and performs the Random Access procedure towards the SCG is left to UE implementation.

1>	else if the RRCReconfiguration message was received via SRB3 (UE in NR-DC):

2>	submit the RRCReconfigurationComplete message via SRB3 to lower layers for transmission using the new configuration;

1>	else (RRCReconfiguration was received via SRB1):

2>	submit the RRCReconfigurationComplete message via SRB1 to lower layers for transmission using the new configuration;

2>	if this is the first RRCReconfiguration message after successful completion of the RRC re-establishment procedure:

3>	resume SRB2 and DRBs that are suspended;

1>	if reconfigurationWithSync was included in spCellConfig of an MCG or SCG, and when MAC of an NR cell group successfully completes a Random Access procedure triggered above;

2>	stop timer T304 for that cell group;

2>	apply the parts of the CSI reporting configuration, the scheduling request configuration and the sounding RS configuration that do not require the UE to know the SFN of the respective target SpCell, if any;

2>	apply the parts of the measurement and the radio resource configuration that require the UE to know the SFN of the respective target SpCell (e.g. measurement gaps, periodic CQI reporting, scheduling request configuration, sounding RS configuration), if any, upon acquiring the SFN of that target SpCell;

2>	if the reconfigurationWithSync was included in spCellConfig of an MCG:

3>	if T390 is running:

4>	stop timer T390 for all access categories;

4>	perform the actions as specified in 5.3.14.4.

3>	if RRCReconfiguration does not include dedicatedSIB1-Delivery and

3>	if the active downlink BWP, which is indicated by the firstActiveDownlinkBWP-Id for the target SpCell of the MCG, has a common search space configured by searchSpaceSIB1:

4>	acquire the SIB1, which is scheduled as specified in TS 38.213 [13], of the target SpCell of the MCG;

4>	upon acquiring SIB1, perform the actions specified in clause 5.2.2.4.2;

2>	if reconfigurationWithSync was included in masterCellGroup; and

2>	if the UE transmitted a UEAssistanceInformation message during the last 1 second, and the UE is still configured to provide UE assistance information:

3>	initiate transmission of a UEAssistanceInformation message in accordance with section 5.7.4.3;

2>	the procedure ends.

NOTE 3:	The UE is only required to acquire broadcasted SIB1 if the UE can acquire it without disrupting unicast data reception, i.e. the broadcast and unicast beams are quasi co-located.

#### 5.3.5.4	Secondary cell group release

The UE shall:

1>	as a result of SCG release triggered by E-UTRA (i.e. (NG)EN-DC case) or NR (i.e. NR-DC case):

2>	reset SCG MAC, if configured;

2>	for each RLC bearer that is part of the SCG configuration:

3>	perform RLC bearer release procedure as specified in 5.3.5.5.3;

2>	release the SCG configuration;

2>	stop timer T310 for the corresponding SpCell, if running;

2>	stop timer T304 for the corresponding SpCell, if running.

NOTE:	Release of cell group means only release of the lower layer configuration of the cell group but the RadioBearerConfig may not be released.

#### 5.3.5.5	Cell Group configuration

##### 5.3.5.5.1	General

The network configures the UE with Master Cell Group (MCG), and zero or one Secondary Cell Group (SCG). In (NG)EN-DC, the MCG is configured as specified in TS 36.331 [10], and for NE-DC, the SCG is configured as specified in TS 36.331 [10]. The network provides the configuration parameters for a cell group in the CellGroupConfig IE.

The UE performs the following actions based on a received CellGroupConfig IE:

1>	if the CellGroupConfig contains the spCellConfig with reconfigurationWithSync:

2>	perform Reconfiguration with sync according to 5.3.5.5.2;

2>	resume all suspended radio bearers and resume SCG transmission for all radio bearers, if suspended;

1>	if the CellGroupConfig contains the rlc-BearerToReleaseList:

2>	perform RLC bearer release as specified in 5.3.5.5.3;

1>	if the CellGroupConfig contains the rlc-BearerToAddModList:

2>	perform the RLC bearer addition/modification as specified in 5.3.5.5.4;

1>	if the CellGroupConfig contains the mac-CellGroupConfig:

2>	configure the MAC entity of this cell group as specified in 5.3.5.5.5;

1>	if the CellGroupConfig contains the sCellToReleaseList:

2>	perform SCell release as specified in 5.3.5.5.8;

1>	if the CellGroupConfig contains the spCellConfig:

2>	configure the SpCell as specified in 5.3.5.5.7;

1>	if the CellGroupConfig contains the sCellToAddModList:

2>	perform SCell addition/modification as specified in 5.3.5.5.9.

##### 5.3.5.5.2	Reconfiguration with sync

The UE shall perform the following actions to execute a reconfiguration with sync.

1>	if the AS security is not activated, perform the actions upon going to RRC_IDLE as specified in 5.3.11 with the release cause 'other' upon which the procedure ends;

1>	stop timer T310 for the corresponding SpCell, if running;

1>	start timer T304 for the corresponding SpCell with the timer value set to t304, as included in the reconfigurationWithSync;

1>	if the frequencyInfoDL is included:

2>	consider the target SpCell to be one on the SSB frequency indicated by the frequencyInfoDL with a physical cell identity indicated by the physCellId;

1>	else:

2>	consider the target SpCell to be one on the SSB frequency of the source SpCell with a physical cell identity indicated by the physCellId;

1>	start synchronising to the DL of the target SpCell;

1>	apply the specified BCCH configuration defined in 9.1.1.1;

1>	acquire the MIB, which is scheduled as specified in TS 38.213 [13];

NOTE 1:	The UE should perform the reconfiguration with sync as soon as possible following the reception of the RRC message triggering the reconfiguration with sync, which could be before confirming successful reception (HARQ and ARQ) of this message.

NOTE 2:	The UE may omit reading the MIB if the UE already has the required timing information, or the timing information is not needed for random access.

1>	reset the MAC entity of this cell group;

1>	consider the SCell(s) of this cell group, if configured, to be in deactivated state;

1>	apply the value of the newUE-Identity as the C-RNTI for this cell group;

1>	configure lower layers in accordance with the received spCellConfigCommon;

1>	configure lower layers in accordance with any additional fields, not covered in the previous, if included in the received reconfigurationWithSync.

##### 5.3.5.5.3	RLC bearer release

The UE shall:

1>	for each logicalChannelIdentity value included in the rlc-BearerToReleaseList that is part of the current UE configuration within the same cell group (LCH release); or

1>	for each logicalChannelIdentity value that is to be released as the result of an SCG release according to 5.3.5.4:

2>	release the RLC entity or entities as specified in TS 38.322 [4], clause 5.1.3;

2>	release the corresponding logical channel.

##### 5.3.5.5.4	RLC bearer addition/modification

For each RLC-BearerConfig received in the rlc-BearerToAddModList IE the UE shall:

1>	if the UE's current configuration contains an RLC bearer with the received logicalChannelIdentity within the same cell group:

2>	if reestablishRLC is received:

3>	re-establish the RLC entity as specified in TS 38.322 [4];

2>	reconfigure the RLC entity or entities in accordance with the received rlc-Config;

2>	reconfigure the logical channel in accordance with the received mac-LogicalChannelConfig;

NOTE:	The network does not re-associate an already configured logical channel with another radio bearer. Hence servedRadioBearer is not present in this case.

1>	else (a logical channel with the given logicalChannelIdentity is not configured within the same cell group, including the case when full configuration option is used):

2>	if the servedRadioBearer associates the logical channel with an SRB and rlc-Config is not included:

3>	establish an RLC entity in accordance with the default configuration defined in 9.2 for the corresponding SRB;

2>	else:

3>	establish an RLC entity in accordance with the received rlc-Config;

2>	if the servedRadioBearer associates the logical channel with an SRB and if mac-LogicalChannelConfig is not included:

3>	configure this MAC entity with a logical channel in accordance to the default configuration defined in 9.2 for the corresponding SRB;

2>	else:

3>	configure this MAC entity with a logical channel in accordance to the received mac-LogicalChannelConfig;

2>	associate this logical channel with the PDCP entity identified by servedRadioBearer.

##### 5.3.5.5.5	MAC entity configuration

The UE shall:

1>	if SCG MAC is not part of the current UE configuration (i.e. SCG establishment):

2>	create an SCG MAC entity;

1>	reconfigure the MAC main configuration of the cell group in accordance with the received mac-CellGroupConfig excluding tag-ToReleaseList and tag-ToAddModList;

1>	if the received mac-CellGroupConfig includes the tag-ToReleaseList:

2>	for each TAG-Id value included in the tag-ToReleaseList that is part of the current UE configuration:

3>	release the TAG indicated by TAG-Id;

1>	if the received mac-CellGroupConfig includes the tag-ToAddModList:

2>	for each tag-Id value included in tag-ToAddModList that is not part of the current UE configuration (TAG addition):

3>	add the TAG, corresponding to the tag-Id, in accordance with the received timeAlignmentTimer;

2>	for each tag-Id value included in tag-ToAddModList that is part of the current UE configuration (TAG modification):

3>	reconfigure the TAG, corresponding to the tag-Id, in accordance with the received timeAlignmentTimer.

##### 5.3.5.5.6	RLF Timers & Constants configuration

The UE shall:

1>	if the received rlf-TimersAndConstants is set to release:

2>	use values for timers T301, T310, T311 and constants N310, N311, as included in ue-TimersAndConstants received in SIB1;

1>	else:

2>	(re-)configure the value of timers and constants in accordance with received rlf-TimersAndConstants;

2>	stop timer T310 for this cell group, if running;

2>	reset the counters N310 and N311.

##### 5.3.5.5.7	SpCell Configuration

The UE shall:

1>	if the SpCellConfig contains the rlf-TimersAndConstants:

2>	configure the RLF timers and constants for this cell group as specified in 5.3.5.5.6;

1>	else if rlf-TimersAndConstants is not configured for this cell group:

2>	use values for timers T301, T310, T311 and constants N310, N311, as included in ue-TimersAndConstants received in SIB1;

1>	if the SpCellConfig contains spCellConfigDedicated:

2>	configure the SpCell in accordance with the spCellConfigDedicated;

2>	consider the bandwidth part indicated in firstActiveUplinkBWP-Id if configured to be the active uplink bandwidth part;

2>	consider the bandwidth part indicated in firstActiveDownlinkBWP-Id if configured to be the active downlink bandwidth part;

2>	if any of the reference signal(s) that are used for radio link monitoring are reconfigured by the received spCellConfigDedicated:

3>	stop timer T310 for the corresponding SpCell, if running;

3>	reset the counters N310 and N311.

##### 5.3.5.5.8	SCell Release

The UE shall:

1>	if the release is triggered by reception of the sCellToReleaseList:

2>	for each sCellIndex value included in the sCellToReleaseList:

3>	if the current UE configuration includes an SCell with value sCellIndex:

4>	release the SCell.

##### 5.3.5.5.9	SCell Addition/Modification

The UE shall:

1>	for each sCellIndex value included in the sCellToAddModList that is not part of the current UE configuration (SCell addition):

2>	add the SCell, corresponding to the sCellIndex, in accordance with the sCellConfigCommon and sCellConfigDedicated;

2>	configure lower layers to consider the SCell to be in deactivated state;

2>	for each measId included in the measIdList within VarMeasConfig:

3>	if SCells are not applicable for the associated measurement; and

3>	if the concerned SCell is included in cellsTriggeredList defined within the VarMeasReportList for this measId:

4>	remove the concerned SCell from cellsTriggeredList defined within the VarMeasReportList for this measId;

1>	for each sCellIndex value included in the sCellToAddModList that is part of the current UE configuration (SCell modification):

2>	modify the SCell configuration in accordance with the sCellConfigDedicated.

#### 5.3.5.6	Radio Bearer configuration

##### 5.3.5.6.1	General

The UE shall perform the following actions based on a received RadioBearerConfig IE:

1>	if the RadioBearerConfig includes the srb3-ToRelease:

2>	perform the SRB release as specified in 5.3.5.6.2;

1>	if the RadioBearerConfig includes the srb-ToAddModList:

2>	perform the SRB addition or reconfiguration as specified in 5.3.5.6.3;

1>	if the RadioBearerConfig includes the drb-ToReleaseList:

2>	perform DRB release as specified in 5.3.5.6.4;

1>	if the RadioBearerConfig includes the drb-ToAddModList:

2>	perform DRB addition or reconfiguration as specified in 5.3.5.6.5.

1>	release all SDAP entities, if any, that have no associated DRB as specified in TS 37.324 [24] clause 5.1.2, and indicate the release of the user plane resources for PDU Sessions associated with the released SDAP entities to upper layers.

##### 5.3.5.6.2	SRB release

The UE shall:

1>	release the PDCP entity and the srb-Identity of the SRB3.

##### 5.3.5.6.3	SRB addition/modification

The UE shall:

1>	for each srb-Identity value included in the srb-ToAddModList that is not part of the current UE configuration (SRB establishment or reconfiguration from E-UTRA PDCP to NR PDCP):

2>	establish a PDCP entity;

2>	if AS security has been activated:

3>	if target RAT of handover is E-UTRA/5GC; or

3>	if the UE is connected to E-UTRA/5GC:

4>	if the UE is capable of E-UTRA/5GC, but not capable of NGEN-DC:

5>	configure the PDCP entity with the security algorithms and keys (KRRCenc and KRRCint) configured/derived as specified in TS 36.331 [10];

4>	else (i.e., UE capable of NGEN-DC):

5>	configure the PDCP entity with the security algorithms according to securityConfig and apply the keys (KRRCenc and KRRCint) associated with the master key (KeNB) or secondary key (S-KgNB) as indicated in keyToUse, if applicable;

3>	else (i.e., UE connected to NR or UE in EN-DC):

4>	configure the PDCP entity with the security algorithms according to securityConfig and apply the keys (KRRCenc and KRRCint) associated with the master key (KeNB/ KgNB) or secondary key (S-KgNB) as indicated in keyToUse, if applicable;

2>	if the current UE configuration as configured by E-UTRA in TS 36.331 [10] includes an SRB identified with the same srb-Identity value:

3>	associate the E-UTRA RLC entity and DCCH of this SRB with the NR PDCP entity;

3>	release the E-UTRA PDCP entity of this SRB;

2>	if the pdcp-Config is included:

3>	configure the PDCP entity in accordance with the received pdcp-Config;

2>	else:

3>	configure the PDCP entity in accordance with the default configuration defined in 9.2.1 for the corresponding SRB;

1>	for each srb-Identity value included in the srb-ToAddModList that is part of the current UE configuration:

2>	if the reestablishPDCP is set:

3>	if target RAT of handover is E-UTRA/5GC; or

3>	if the UE is connected to E-UTRA/5GC:

4>	if the UE is capable of E-UTRA/5GC, but not capable of NGEN-DC:

5>	configure the PDCP entity to apply the integrity protection algorithm and KRRCint key configured/derived as specified in TS 36.331 [10], i.e. the integrity protection configuration shall be applied to all subsequent messages received and sent by the UE, including the message used to indicate the successful completion of the procedure;

5>	configure the PDCP entity to apply the ciphering algorithm and KRRCenc key configured/derived as specified in TS 36.331 [10], i.e. the ciphering configuration shall be applied to all subsequent messages received and sent by the UE, including the message used to indicate the successful completion of the procedure;

4>	else (i.e., a UE capable of NGEN-DC):

5>	configure the PDCP entity to apply the integrity protection algorithm and KRRCint key associated with the master key (KeNB) or secondary key (S-KgNB), as indicated in keyToUse, i.e. the integrity protection configuration shall be applied to all subsequent messages received and sent by the UE, including the message used to indicate the successful completion of the procedure;

5>	configure the PDCP entity to apply the ciphering algorithm and KRRCenc key associated with the master key (KeNB) or secondary key (S-KgNB) as indicated in keyToUse, i.e. the ciphering configuration shall be applied to all subsequent messages received and sent by the UE, including the message used to indicate the successful completion of the procedure;

3>	else (i.e., UE connected to NR or UE in EN-DC):

4>	configure the PDCP entity to apply the integrity protection algorithm and KRRCint key associated with the master key (KeNB/KgNB) or secondary key (S-KgNB), as indicated in keyToUse , i.e. the integrity protection configuration shall be applied to all subsequent messages received and sent by the UE, including the message used to indicate the successful completion of the procedure;

4>	configure the PDCP entity to apply the ciphering algorithm and KRRCenc key associated with the master key (KeNB/KgNB) or secondary key (S-KgNB) as indicated in keyToUse, i.e. the ciphering configuration shall be applied to all subsequent messages received and sent by the UE, including the message used to indicate the successful completion of the procedure;

3>	re-establish the PDCP entity of this SRB as specified in TS 38.323 [5];

2>	else, if the discardOnPDCP is set:

3>	trigger the PDCP entity to perform SDU discard as specified in TS 38.323 [5];

2>	if the pdcp-Config is included:

3>	reconfigure the PDCP entity in accordance with the received pdcp-Config.

##### 5.3.5.6.4	DRB release

The UE shall:

1>	for each drb-Identity value included in the drb-ToReleaseList that is part of the current UE configuration; or

1>	for each drb-Identity value that is to be released as the result of full configuration according to 5.3.5.11:

2>	release the PDCP entity and the drb-Identity;

2>	if SDAP entity associated with this DRB is configured:

3>	indicate the release of the DRB to SDAP entity associated with this DRB (TS 37.324 [24], clause 5.3.3);

2>	if the DRB is associated with an eps-BearerIdentity:

3>	if a new bearer is not added either with NR or E-UTRA with same eps-BearerIdentity:

4>	indicate the release of the DRB and the eps-BearerIdentity of the released DRB to upper layers.

NOTE 1:	The UE does not consider the message as erroneous if the drb-ToReleaseList includes any drb-Identity value that is not part of the current UE configuration.

NOTE 2:	Whether or not the RLC and MAC entities associated with this PDCP entity are reset or released is determined by the CellGroupConfig.

##### 5.3.5.6.5	DRB addition/modification

The UE shall:

1>	for each drb-Identity value included in the drb-ToAddModList that is not part of the current UE configuration (DRB establishment including the case when full configuration option is used):

2>	establish a PDCP entity and configure it in accordance with the received pdcp-Config;

2>	if the PDCP entity of this DRB is not configured with cipheringDisabled:

3>	if target RAT of handover is E-UTRA/5GC; or

3>	if the UE is connected to E-UTRA/5GC:

4>	if the UE is capable of E-UTRA/5GC but not capable of NGEN-DC:

5>	configure the PDCP entity with the ciphering algorithm and KUPenc key configured/derived as specified in TS 36.331 [10];

4>	else (i.e., a UE capable of NGEN-DC):

5>	configure the PDCP entity with the ciphering algorithms according to securityConfig and apply the key (KUPenc) associated with the master key (KeNB) or secondary key (S-KgNB) as indicated in keyToUse, if applicable;

3>	else (i.e., UE connected to NR or UE connected to E-UTRA/EPC):

4>	configure the PDCP entity with the ciphering algorithms according to securityConfig and apply the KUPenc key associated with the master key (KeNB/KgNB) or the secondary key (S-KgNB/S-KeNB) as indicated in keyToUse;

2>	if the PDCP entity of this DRB is configured with integrityProtection:

3>	configure the PDCP entity with the integrity protection algorithms according to securityConfig and apply the KUPint key associated with the master (KgNB) or the secondary key (S-KgNB) as indicated in keyToUse;

2>	if an sdap-Config is included:

3>	if an SDAP entity with the received pdu-Session does not exist:

4>	establish an SDAP entity as specified in TS 37.324 [24] clause 5.1.1;

4>	if an SDAP entity with the received pdu-Session did not exist prior to receiving this reconfiguration:

5>	indicate the establishment of the user plane resources for the pdu-Session to upper layers;

3>	configure the SDAP entity in accordance with the received sdap-Config as specified in TS 37.324 [24] and associate the DRB with the SDAP entity;

2>	if the DRB is associated with an eps-BearerIdentity:

3>	if the DRB was configured with the same eps-BearerIdentity either by NR or E-UTRA prior to receiving this reconfiguration:

4>	associate the established DRB with the corresponding eps-BearerIdentity;

3>	else:

4>	indicate the establishment of the DRB(s) and the eps-BearerIdentity of the established DRB(s) to upper layers;

1>	for each drb-Identity value included in the drb-ToAddModList that is part of the current UE configuration:

2>	if the reestablishPDCP is set:

3>	if target RAT of handover is E-UTRA/5GC; or

3>	if the UE is connected to E-UTRA/5GC:

4>	if the UE is capable of E-UTRA/5GC but not capable of NGEN-DC:

5>	if the PDCP entity of this DRB is not configured with cipheringDisabled:

6>	configure the PDCP entity with the ciphering algorithm and KUPenc key configured/derived as specified in TS 36.331 [10], clause 5.4.2.3, i.e. the ciphering configuration shall be applied to all subsequent PDCP PDUs received and sent by the UE;

4>	else (i.e., a UE capable of NGEN-DC):

5>	if the PDCP entity of this DRB is not configured with cipheringDisabled:

6>	configure the PDCP entity with the ciphering algorithm and KUPenc key associated with the master key (KeNB) or the secondary key (S-KgNB), as indicated in keyToUse, i.e. the ciphering configuration shall be applied to all subsequent PDCP PDUs received and sent by the UE;

3>	else (i.e., UE connected to NR or UE in EN-DC):

4>	if the PDCP entity of this DRB is not configured with cipheringDisabled:

5>	configure the PDCP entity with the ciphering algorithm and KUPenc key associated with the master key (KeNB/ KgNB) or the secondary key (S-KgNB/S-KeNB), as indicated in keyToUse, i.e. the ciphering configuration shall be applied to all subsequent PDCP PDUs received and sent by the UE;

4>	if the PDCP entity of this DRB is configured with integrityProtection:

5>	configure the PDCP entity with the integrity protection algorithms according to securityConfig and apply the KUPint key associated with the master key (KgNB) or the secondary key (S-KgNB) as indicated in keyToUse;

3>	if drb-ContinueROHC is included in pdcp-Config:

4>	indicate to lower layer that drb-ContinueROHC is configured;

3>	re-establish the PDCP entity of this DRB as specified in TS 38.323 [5], clause 5.1.2;

2>	else, if the recoverPDCP is set:

3>	trigger the PDCP entity of this DRB to perform data recovery as specified in TS 38.323 [5];

2>	if the pdcp-Config is included:

3>	reconfigure the PDCP entity in accordance with the received pdcp-Config.

2>	if the sdap-Config is included:

3>	reconfigure the SDAP entity in accordance with the received sdap-Config as specified in TS37.324 [24];

3>	for each QFI value added in mappedQoS-FlowsToAdd, if the QFI value is previously configured, the QFI value is released from the old DRB;

NOTE 1:	Void.

NOTE 2:	When determining whether a drb-Identity value is part of the current UE configuration, the UE does not distinguish which RadioBearerConfig and DRB-ToAddModList that DRB was originally configured in. To re-associate a DRB with a different key (KeNB to S-KgNB, KgNB to S-KeNB, KgNB to S-KgNB, or vice versa), the network provides the drb-Identity value in the (target) drb-ToAddModList and sets the reestablishPDCP flag. The network does not list the drb-Identity in the (source) drb-ToReleaseList.

NOTE 3:	When setting the reestablishPDCP flag for a radio bearer, the network ensures that the RLC receiver entities do not deliver old PDCP PDUs to the re-established PDCP entity. It does that e.g. by triggering a reconfiguration with sync of the cell group hosting the old RLC entity or by releasing the old RLC entity.

NOTE 4:	In this specification, UE configuration refers to the parameters configured by NR RRC unless otherwise stated.

NOTE 5: Ciphering and integrity protection can be enabled or disabled for a DRB. The enabling/disabling of ciphering or integrity protection can be changed only by releasing and adding the DRB.

#### 5.3.5.7	AS Security key update

The UE shall:

1>	if UE is connected to E-UTRA/EPC or E-UTRA/5GC:

2>	upon reception of sk-Counter as specified in TS 36.331 [10]:

3>	update the S-KgNB key based on the KeNB key and using the received sk-Counter value, as specified in TS 33.401 [30] for EN-DC, or TS 33.501 [11] for NGEN-DC;

3>	derive the KRRCenc and KUPenc keys as specified in TS 33.401 [30] for EN-DC, or TS 33.501 [11] for NGEN-DC;

3>	derive the KRRCint and KUPint keys as specified in TS 33.401 [30] for EN-DC or TS 33.501 [11] for NGEN-DC.

1>	else if this procedure was initiated due to reception of the masterKeyUpdate:

2>	if the nas-Container is included in the received masterKeyUpdate:

3>	forward the nas-Container to the upper layers;

2>	if the keySetChangeIndicator is set to true:

3>	derive or update the KgNB key based on the KAMF key, as specified in TS 33.501 [11];

2>	else:

3>	derive or update the KgNB key based on the current KgNB key or the NH, using the nextHopChainingCount value indicated in the received masterKeyUpdate, as specified in TS 33.501 [11];

2>	store the nextHopChainingCount value;

2>	derive the keys associated with the KgNB key as follows:

3>	if the securityAlgorithmConfig is included in SecurityConfig:

4>	derive the KRRCenc and KUPenc keys associated with the cipheringAlgorithm indicated in the securityAlgorithmConfig, as specified in TS 33.501 [11];

4>	derive the KRRCint and KUPint keys associated with the integrityProtAlgorithm indicated in the securityAlgorithmConfig, as specified in TS 33.501 [11];

3>	else:

4>	derive the KRRCenc and KUPenc keys associated with the current cipheringAlgorithm, as specified in TS 33.501 [11];

4>	derive the KRRCint and KUPint keys associated with the current integrityProtAlgorithm, as specified in TS 33.501 [11].

NOTE 1:	Ciphering and integrity protection are optional to configure for the DRBs.

1>	else if this procedure was initiated due to reception of the sk-Counter (UE is in NE-DC, or NR-DC, or is configured with SN terminated bearer(s)):

2>	derive or update the secondary key (S-KgNB or S-KeNB) based on the KgNB key and using the received sk-Counter value, as specified in TS 33.501 [11];

2>	derive the KRRCenc key and the KUPenc key as specified in TS 33.501 [11] using the ciphering algorithms indicated in the RadioBearerConfig associated with the secondary key (S-KgNB or S-KeNB) as indicated by keyToUse;

2>	derive the KRRCint key and the KUPint key as specified in TS 33.501 [11] using the integrity protection algorithms indicated in the RadioBearerConfig associated with the secondary key (S-KgNB or S-KeNB) as indicated by keyToUse;

NOTE 2:	If the UE has no radio bearer configured with keyToUse set to secondary and receives the sk-Counter without any RadioBearerConfig with keyToUse set to secondary, the UE does not consider it as an invalid reconfiguration.

#### 5.3.5.8	Reconfiguration failure

##### 5.3.5.8.1	Void

##### 5.3.5.8.2	Inability to comply with RRCReconfiguration

The UE shall:

1>	if the UE is in (NG)EN-DC:

2>	if the UE is unable to comply with (part of) the configuration included in the RRCReconfiguration message received over SRB3;

3>	continue using the configuration used prior to the reception of RRCReconfiguration message;

3>	initiate the SCG failure information procedure as specified in subclause 5.7.3 to report SCG reconfiguration error, upon which the connection reconfiguration procedure ends;

2>	else, if the UE is unable to comply with (part of) the configuration included in the RRCReconfiguration message received over SRB1;

3>	continue using the configuration used prior to the reception of RRCReconfiguration message;

3>	initiate the connection re-establishment procedure as specified in TS 36.331 [10], clause 5.3.7, upon which the connection reconfiguration procedure ends.

1>	else if RRCReconfiguration is received via NR (i.e., NR standalone, NE-DC, or NR-DC):

2>	if the UE is unable to comply with (part of) the configuration included in the RRCReconfiguration message received over SRB3;

NOTE 0:	This case does not apply in NE-DC.

3>	continue using the configuration used prior to the reception of RRCReconfiguration message;

3>	initiate the SCG failure information procedure as specified in subclause 5.7.3 to report SCG reconfiguration error, upon which the connection reconfiguration procedure ends;

2>	else if the UE is unable to comply with (part of) the configuration included in the RRCReconfiguration message received over the SRB1;

NOTE 0a:	The compliance also covers the SCG configuration carried within octet strings e.g. field mrdc-SecondaryCellGroupConfig. I.e. the failure behaviour defined also applies in case the UE cannot comply with the embedded SCG configuration or with the combination of (parts of) the MCG and SCG configurations.

3>	continue using the configuration used prior to the reception of RRCReconfiguration message;

3>	if AS security has not been activated:

4>	perform the actions upon going to RRC_IDLE as specified in 5.3.11, with release cause 'other'

3>	else if AS security has been activated but SRB2 and at least one DRB have not been setup:

4>	perform the actions upon going to RRC_IDLE as specified in 5.3.11, with release cause 'RRC connection failure';

3>	else:

4>	initiate the connection re-establishment procedure as specified in 5.3.7, upon which the reconfiguration procedure ends;

1>	else if RRCReconfiguration is received via other RAT (Handover to NR failure):

2>	if the UE is unable to comply with any part of the configuration included in the RRCReconfiguration message:

3>	perform the actions defined for this failure case as defined in the specifications applicable for the other RAT.

NOTE 1:	The UE may apply above failure handling also in case the RRCReconfiguration message causes a protocol error for which the generic error handling as defined in clause 10 specifies that the UE shall ignore the message.

NOTE 2:	If the UE is unable to comply with part of the configuration, it does not apply any part of the configuration, i.e. there is no partial success/failure.

##### 5.3.5.8.3	T304 expiry (Reconfiguration with sync Failure)

The UE shall:

1>	if T304 of the MCG expires:

2>	release dedicated preambles provided in rach-ConfigDedicated if configured;

2>	revert back to the UE configuration used in the source PCell;

2>	initiate the connection re-establishment procedure as specified in subclause 5.3.7.

NOTE 1:	In the context above, "the UE configuration" includes state variables and parameters of each radio bearer.

1>	else if T304 of a secondary cell group expires:

2>	release dedicated preambles provided in rach-ConfigDedicated, if configured;

2>	initiate the SCG failure information procedure as specified in subclause 5.7.3 to report SCG reconfiguration with sync failure, upon which the RRC reconfiguration procedure ends;

1>	else if T304 expires when RRCReconfiguration is received via other RAT (HO to NR failure):

2>	reset MAC;

2>	perform the actions defined for this failure case as defined in the specifications applicable for the other RAT.

#### 5.3.5.9	Other configuration

The UE shall:

1>	if the received otherConfig includes the delayBudgetReportingConfig:

2>	if delayBudgetReportingConfig is set to setup:

3>	consider itself to be configured to send delay budget reports in accordance with 5.7.4;

2>	else:

3>	consider itself not to be configured to send delay budget reports and stop timer T342, if running.

1>	if the received otherConfig includes the overheatingAssistanceConfig:

2>	if overheatingAssistanceConfig is set to setup:

3>	consider itself to be configured to provide overheating assistance information in accordance with 5.7.4;

2>	else:

3>	consider itself not to be configured to provide overheating assistance information and stop timer T345, if running;

#### 5.3.5.10	MR-DC release

The UE shall:

1>	as a result of MR-DC release triggered by E-UTRA or NR:

2>	release SRB3, if established, as specified in 5.3.5.6.2;

2>	release measConfig associated with SCG;

2>	if the UE is configured with NR SCG:

3>	release the SCG configuration as specified in clause 5.3.5.4;

2>	else if the UE is configured with E-UTRA SCG:

3>	release the SCG configuration as specified in TS 36.331 [10], clause 5.3.10.19 to release the E-UTRA SCG;

#### 5.3.5.11	Full configuration

The UE shall:

1>	release/ clear all current dedicated radio configurations except for the following:

-	the MCG C-RNTI;

-	the AS security configurations associated with the master key;

NOTE 1:	Radio configuration is not just the resource configuration but includes other configurations like MeasConfig. In case NR-DC or NE-DC is configured, this also includes the entire NR or E-UTRA SCG configuration which are released according to the MR-DC release procedure as specified in 5.3.5.10. The radio configuration does not include SRB1/SRB2 configurations and DRB configurations as configured by radioBearerConfig or radioBearerConfig2.

1>	if the spCellConfig in the masterCellGroup includes the reconfigurationWithSync (i.e., SpCell change):

2>	release/ clear all current common radio configurations;

2>	use the default values specified in 9.2.3 for timers T310, T311 and constants N310, N311;

1>	else (full configuration after re-establishment or during RRC resume):

2>	use values for timers T301, T310, T311 and constants N310, N311, as included in ue-TimersAndConstants received in SIB1;

1>	apply the default L1 parameter values as specified in corresponding physical layer specifications except for the following:

-	parameters for which values are provided in SIB1;

1>	apply the default MAC Cell Group configuration as specified in 9.2.2;

1>	for each srb-Identity value included in the srb-ToAddModList (SRB reconfiguration):

2>	apply the default SRB configuration defined in 9.2.1 for the corresponding SRB;

NOTE 2:	This is to get the SRBs (SRB1 and SRB2 for reconfiguration with sync and SRB2 for reconfiguration after re-establishment) to a known state from which the reconfiguration message can do further configuration.

1>	for each pdu-Session that is part of the current UE configuration:

2>	release the SDAP entity (clause 5.1.2 in TS 37.324 [24]);

2>	release each DRB associated to the pdu-Session as specified in 5.3.5.6.4;

NOTE 3:	This will retain the pdu-Session but remove the DRBs including drb-identity of these bearers from the current UE configuration. Setup of the DRBs within the AS is described in clause 5.3.5.6.5 using the new configuration. The pdu-Session acts as the anchor for associating the released and re-setup DRB. In the AS the DRB re-setup is equivalent with a new DRB setup (including new PDCP and logical channel configurations).

1>	for each pdu-Session that is part of the current UE configuration but not added with same pdu-Session in the drb-ToAddModList:

2>	if the procedure was triggered due to reconfiguration with sync:

3>	indicate the release of the user plane resources for the pdu-Session to upper layers after successful reconfiguration with sync;

2>	else:

3>	indicate the release of the user plane resources for the pdu-Session to upper layers immediately;

### 5.3.6	Counter check

#### 5.3.6.1	General

Figure 5.3.6.1-1: Counter check procedure

The counter check procedure is used by the network to request the UE to verify the amount of data sent/ received on each DRB. More specifically, the UE is requested to check if, for each DRB, the most significant bits of the COUNT match with the values indicated by the network.

NOTE:	The procedure enables the network to detect packet insertion by an intruder (a 'man in the middle').

#### 5.3.6.2	Initiation

The network initiates the procedure by sending a CounterCheck message.

NOTE:	The network may initiate the procedure when any of the COUNT values reaches a specific value.

#### 5.3.6.3	Reception of the CounterCheck message by the UE

Upon receiving the CounterCheck message, the UE shall:

1>	for each DRB that is established:

2>	if no COUNT exists for a given direction (uplink or downlink) because it is a uni-directional bearer configured only for the other direction:

3>	assume the COUNT value to be 0 for the unused direction;

2>	if the drb-Identity is not included in the drb-CountMSB-InfoList:

3>	include the DRB in the drb-CountInfoList in the CounterCheckResponse message by including the drb-Identity, the count-Uplink and the count-Downlink set to the value of TX_NEXT – 1 and RX_NEXT – 1 (specified in TS 38.323 [5]), respectively;

2>	else if, for at least one direction, the most significant bits of the COUNT are different from the value indicated in the drb-CountMSB-InfoList:

3>	include the DRB in the drb-CountInfoList in the CounterCheckResponse message by including the drb-Identity, the count-Uplink and the count-Downlink set to the value of TX_NEXT – 1 and RX_NEXT – 1 (specified in TS 38.323 [5]), respectively;

1>	for each DRB that is included in the drb-CountMSB-InfoList in the CounterCheck message that is not established:

2>	include the DRB in the drb-CountInfoList in the CounterCheckResponse message by including the drb-Identity, the count-Uplink and the count-Downlink with the most significant bits set identical to the corresponding values in the drb-CountMSB-InfoList and the least significant bits set to zero;

1>	submit the CounterCheckResponse message to lower layers for transmission upon which the procedure ends.

### 5.3.7	RRC connection re-establishment

#### 5.3.7.1	General

Figure 5.3.7.1-1: RRC connection re-establishment, successful

Figure 5.3.7.1-2: RRC re-establishment, fallback to RRC establishment, successful

The purpose of this procedure is to re-establish the RRC connection. A UE in RRC_CONNECTED, for which AS security has been activated with SRB2 and at least one DRB setup, may initiate the procedure in order to continue the RRC connection. The connection re-establishment succeeds if the network is able to find and verify a valid UE context or, if the UE context cannot be retrieved, and the network responds with an RRCSetup according to clause 5.3.3.4.

The network applies the procedure e.g as follows:

-	When AS security has been activated and the network retrieves or verifies the UE context:

-	to re-activate AS security without changing algorithms;

-	to re-establish and resume the SRB1;

-	When UE is re-establishing an RRC connection, and the network is not able to retrieve or verify the UE context:

-	to discard the stored AS Context and release all RBs;

-	to fallback to establish a new RRC connection.

If AS security has not been activated, the UE shall not initiate the procedure but instead moves to RRC_IDLE directly, with release cause 'other'. If AS security has been activated, but SRB2 and at least one DRB are not setup, the UE does not initiate the procedure but instead moves to RRC_IDLE directly, with release cause 'RRC connection failure'.

#### 5.3.7.2	Initiation

The UE initiates the procedure when one of the following conditions is met:

1>	upon detecting radio link failure of the MCG, in accordance with 5.3.10; or

1>	upon re-configuration with sync failure of the MCG, in accordance with sub-clause 5.3.5.8.3; or

1>	upon mobility from NR failure, in accordance with sub-clause 5.4.3.5; or

1>	upon integrity check failure indication from lower layers concerning SRB1 or SRB2, except if the integrity check failure is detected on the RRCReestablishment message; or

1>	upon an RRC connection reconfiguration failure, in accordance with sub-clause 5.3.5.8.2.

Upon initiation of the procedure, the UE shall:

1>	stop timer T310, if running;

1>	stop timer T304, if running;

1>	start timer T311;

1>	suspend all RBs, except SRB0;

1>	reset MAC;

1>	release the MCG SCell(s), if configured;

1>	release spCellConfig, if configured;

1>	if MR-DC is configured:

2>	perform MR-DC release, as specified in clause 5.3.5.10;

1>	release delayBudgetReportingConfig, if configured, and stop timer T342, if running;

1>	release overheatingAssistanceConfig, if configured, and stop timer T345, if running;

1>	perform cell selection in accordance with the cell selection process as specified in TS 38.304 [20], clause 5.2.6.

#### 5.3.7.3	Actions following cell selection while T311 is running

Upon selecting a suitable NR cell, the UE shall:

1>	ensure having valid and up to date essential system information as specified in clause 5.2.2.2;

1>	stop timer T311;

1>	start timer T301;

1>	if T390 is running:

2>	stop timer T390 for all access categories;

2>	perform the actions as specified in 5.3.14.4;

1>	apply the default L1 parameter values as specified in corresponding physical layer specifications except for the parameters for which values are provided in SIB1;

1>	apply the default MAC Cell Group configuration as specified in 9.2.2;

1>	apply the CCCH configuration as specified in 9.1.1.2;

1>	apply the timeAlignmentTimerCommon included in SIB1;

1>	initiate transmission of the RRCReestablishmentRequest message in accordance with 5.3.7.4;

NOTE:	This procedure applies also if the UE returns to the source PCell.

Upon selecting an inter-RAT cell, the UE shall:

1>	perform the actions upon going to RRC_IDLE as specified in 5.3.11, with release cause 'RRC connection failure'.

#### 5.3.7.4	Actions related to transmission of RRCReestablishmentRequest message

The UE shall set the contents of RRCReestablishmentRequest message as follows:

1>	set the ue-Identity as follows:

2>	set the c-RNTI to the C-RNTI used in the source PCell (reconfiguration with sync or mobility from NR failure) or used in the PCell in which the trigger for the re-establishment occurred (other cases);

2>	set the physCellId to the physical cell identity of the source PCell (reconfiguration with sync or mobility from NR failure) or of the PCell in which the trigger for the re-establishment occurred (other cases);

2>	set the shortMAC-I to the 16 least significant bits of the MAC-I calculated:

3>	over the ASN.1 encoded as per clause 8 (i.e., a multiple of 8 bits) VarShortMAC-Input;

3>	with the KRRCint key and integrity protection algorithm that was used in the source PCell (reconfiguration with sync or mobility from NR failure) or of the PCell in which the trigger for the re-establishment occurred (other cases); and

3>	with all input bits for COUNT, BEARER and DIRECTION set to binary ones;

1>	set the reestablishmentCause as follows:

2>	if the re-establishment procedure was initiated due to reconfiguration failure as specified in 5.3.5.8.2:

3>	set the reestablishmentCause to the value reconfigurationFailure;

2>	else if the re-establishment procedure was initiated due to reconfiguration with sync failure as specified in 5.3.5.8.3 (intra-NR handover failure) or 5.4.3.5 (inter-RAT mobility from NR failure):

3>	set the reestablishmentCause to the value handoverFailure;

2>	else:

3>	set the reestablishmentCause to the value otherFailure;

1>	re-establish PDCP for SRB1;

1>	re-establish RLC for SRB1;

1>	apply the specified configuration defined in 9.2.1 for SRB1;

1>	configure lower layers to suspend integrity protection and ciphering for SRB1;

NOTE:	Ciphering is not applied for the subsequent RRCReestablishment message used to resume the connection. An integrity check is performed by lower layers, but merely upon request from RRC.

1>	resume SRB1;

1>	submit the RRCReestablishmentRequest message to lower layers for transmission.

#### 5.3.7.5	Reception of the RRCReestablishment by the UE

The UE shall:

1>	stop timer T301;

1>	consider the current cell to be the PCell;

1>	store the nextHopChainingCount value indicated in the RRCReestablishment message;

1>	update the KgNB key based on the current KgNB key or the NH, using the stored nextHopChainingCount value, as specified in TS 33.501 [11];

1>	derive the KRRCenc and KUPenc keys associated with the previously configured cipheringAlgorithm, as specified in TS 33.501 [11];

1>	derive the KRRCint and KUPint keys associated with the previously configured integrityProtAlgorithm, as specified in TS 33.501 [11].

1>	request lower layers to verify the integrity protection of the RRCReestablishment message, using the previously configured algorithm and the KRRCint key;

1>	if the integrity protection check of the RRCReestablishment message fails:

2>	perform the actions upon going to RRC_IDLE as specified in 5.3.11, with release cause 'RRC connection failure', upon which the procedure ends;

1>	configure lower layers to resume integrity protection for SRB1 using the previously configured algorithm and the KRRCint key immediately, i.e., integrity protection shall be applied to all subsequent messages received and sent by the UE, including the message used to indicate the successful completion of the procedure;

1>	configure lower layers to resume ciphering for SRB1 using the previously configured algorithm and, the KRRCenc key immediately, i.e., ciphering shall be applied to all subsequent messages received and sent by the UE, including the message used to indicate the successful completion of the procedure;

1>	release the measurement gap configuration indicated by the measGapConfig, if configured;

1>	submit the RRCReestablishmentComplete message to lower layers for transmission;

1>	the procedure ends.

#### 5.3.7.6	T311 expiry

Upon T311 expiry, the UE shall:

1>	perform the actions upon going to RRC_IDLE as specified in 5.3.11, with release cause 'RRC connection failure'.

#### 5.3.7.7	T301 expiry or selected cell no longer suitable

The UE shall:

1>	if timer T301 expires; or

1>	if the selected cell becomes no longer suitable according to the cell selection criteria as specified in TS 38.304 [20]:

2>	perform the actions upon going to RRC_IDLE as specified in 5.3.11, with release cause 'RRC connection failure'.

#### 5.3.7.8	Reception of the RRCSetup by the UE

The UE shall:

1>	perform the RRC connection establishment procedure as specified in 5.3.3.4.

### 5.3.8	RRC connection release

#### 5.3.8.1	General

Figure 5.3.8.1-1: RRC connection release, successful

The purpose of this procedure is:

-	to release the RRC connection, which includes the release of the established radio bearers as well as all radio resources; or

-	to suspend the RRC connection only if SRB2 and at least one DRB are setup, which includes the suspension of the established radio bearers.

#### 5.3.8.2	Initiation

The network initiates the RRC connection release procedure to transit a UE in RRC_CONNECTED to RRC_IDLE; or to transit a UE in RRC_CONNECTED to RRC_INACTIVE only if SRB2 and at least one DRB is setup in RRC_CONNECTED; or to transit a UE in RRC_INACTIVE back to RRC_INACTIVE when the UE tries to resume; or to transit a UE in RRC_INACTIVE to RRC_IDLE when the UE tries to resume. The procedure can also be used to release and redirect a UE to another frequency.

#### 5.3.8.3	Reception of the RRCRelease by the UE

The UE shall:

1>	delay the following actions defined in this sub-clause 60 ms from the moment the RRCRelease message was received or optionally when lower layers indicate that the receipt of the RRCRelease message has been successfully acknowledged, whichever is earlier;

1>	stop timer T380, if running;

1>	stop timer T320, if running;

1>	if the AS security is not activated:

2>	ignore any field included in RRCRelease message except waitTime;

2>	perform the actions upon going to RRC_IDLE as specified in 5.3.11 with the release cause 'other' upon which the procedure ends;

1>	if the RRCRelease message includes redirectedCarrierInfo indicating redirection to eutra:

2>	if cnType is included:

3>	after the cell selection, indicate the available CN Type(s) and the received cnType to upper layers;

NOTE:	Handling the case if the E-UTRA cell selected after the redirection does not support the core network type specified by the cnType, is up to UE implementation.

1>	if the RRCRelease message includes the cellReselectionPriorities:

2>	store the cell reselection priority information provided by the cellReselectionPriorities;

2>	if the t320 is included:

3>	start timer T320, with the timer value set according to the value of t320;

1>	else:

2>	apply the cell reselection priority information broadcast in the system information;

1>	if deprioritisationReq is included:

2>	start or restart timer T325 with the timer value set to the deprioritisationTimer signalled;

2>	store the deprioritisationReq until T325 expiry;

1>	if the RRCRelease includes suspendConfig:

2>	apply the received suspendConfig;

2>	reset MAC and release the default MAC Cell Group configuration, if any;

2>	re-establish RLC entities for SRB1;

2>	if the RRCRelease message with suspendConfig was received in response to an RRCResumeRequest or an RRCResumeRequest1:

3>	stop the timer T319 if running;

3>	in the stored UE Inactive AS context:

4>	replace the KgNB and KRRCint keys with the current KgNB and KRRCint keys;

4>	replace the C-RNTI with the temporary C-RNTI in the cell the UE has received the RRCRelease message;

4>	replace the cellIdentity with the cellIdentity of the cell the UE has received the RRCRelease message;

4>	replace the physical cell identity with the physical cell identity of the cell the UE has received the RRCRelease message;

2>	else:

3>	store in the UE Inactive AS Context the current KgNB and KRRCint keys, the ROHC state, the stored QoS flow to DRB mapping rules, the C-RNTI used in the source PCell, the cellIdentity and the physical cell identity of the source PCell, and all other parameters configured except for the ones within ReconfigurationWithSync and servingCellConfigCommonSIB;

2>	suspend all SRB(s) and DRB(s), except SRB0;

2>	indicate PDCP suspend to lower layers of all DRBs;

2>	if the t380 is included:

3>	start timer T380, with the timer value set to t380;

2>	if the RRCRelease message is including the waitTime:

3>	start timer T302 with the value set to the waitTime;

3>	inform upper layers that access barring is applicable for all access categories except categories '0' and '2';

2>	if T390 is running:

3>	stop timer T390 for all access categories;

3>	perform the actions as specified in 5.3.14.4;

2>	indicate the suspension of the RRC connection to upper layers;

2>	enter RRC_INACTIVE and perform cell selection as specified in TS 38.304 [20];

1>	else

2>	perform the actions upon going to RRC_IDLE as specified in 5.3.11, with the release cause 'other'.

#### 5.3.8.4	T320 expiry

The UE shall:

1>	if T320 expires:

2>	if stored, discard the cell reselection priority information provided by the cellReselectionPriorities or inherited from another RAT;

2>	apply the cell reselection priority information broadcast in the system information.

#### 5.3.8.5	UE actions upon the expiry of DataInactivityTimer

Upon receiving the expiry of DataInactivityTimer from lower layers while in RRC_CONNECTED, the UE shall:

1>	perform the actions upon going to RRC_IDLE as specified in 5.3.11, with release cause 'RRC connection failure'.

### 5.3.9	RRC connection release requested by upper layers

#### 5.3.9.1	General

The purpose of this procedure is to release the RRC connection. Access to the current PCell may be barred as a result of this procedure.

#### 5.3.9.2	Initiation

The UE initiates the procedure when upper layers request the release of the RRC connection as specified in TS 24.501 [23]. The UE shall not initiate the procedure for power saving purposes.

The UE shall:

1>	if the upper layers indicate barring of the PCell:

2>	treat the PCell used prior to entering RRC_IDLE as barred according to TS 38.304 [20];

1>	perform the actions upon going to RRC_IDLE as specified in 5.3.11, with release cause 'other'.

### 5.3.10	Radio link failure related actions

#### 5.3.10.1	Detection of physical layer problems in RRC_CONNECTED

The UE shall:

1>	upon receiving N310 consecutive "out-of-sync" indications for the SpCell from lower layers while neither T300, T301, T304, T311 nor T319 are running:

2>	start timer T310 for the corresponding SpCell.

#### 5.3.10.2	Recovery of physical layer problems

Upon receiving N311 consecutive "in-sync" indications for the SpCell from lower layers while T310 is running, the UE shall:

1>	stop timer T310 for the corresponding SpCell.

NOTE 1:	In this case, the UE maintains the RRC connection without explicit signalling, i.e. the UE maintains the entire radio resource configuration.

NOTE 2:	Periods in time where neither "in-sync" nor "out-of-sync" is reported by L1 do not affect the evaluation of the number of consecutive "in-sync" or "out-of-sync" indications.

#### 5.3.10.3	Detection of radio link failure

The UE shall:

1>	upon T310 expiry in PCell; or

1>	upon random access problem indication from MCG MAC while neither T300, T301, T304, T311 nor T319 are running; or

1>	upon indication from MCG RLC that the maximum number of retransmissions has been reached:

2>	if the indication is from MCG RLC and CA duplication is configured and activated, and for the corresponding logical channel allowedServingCells only includes SCell(s):

3>	initiate the failure information procedure as specified in 5.7.5 to report RLC failure.

2>	else:

3>	consider radio link failure to be detected for the MCG i.e. RLF;

3>	if AS security has not been activated:

4>	perform the actions upon going to RRC_IDLE as specified in 5.3.11, with release cause 'other';-

3>	else if AS security has been activated but SRB2 and at least one DRB have not been setup:

4>	perform the actions upon going to RRC_IDLE as specified in 5.3.11, with release cause 'RRC connection failure';

3>	else:

4>	initiate the connection re-establishment procedure as specified in 5.3.7.

The UE shall:

1>	upon T310 expiry in PSCell; or

1>	upon random access problem indication from SCG MAC; or

1>	upon indication from SCG RLC that the maximum number of retransmissions has been reached:

2>	if the indication is from SCG RLC and CA duplication is configured and activated; and for the corresponding logical channel allowedServingCells only includes SCell(s):

3>	initiate the failure information procedure as specified in 5.7.5 to report RLC failure.

2>	else:

3>	consider radio link failure to be detected for the SCG, i.e. SCG RLF;

3>	initiate the SCG failure information procedure as specified in 5.7.3 to report SCG radio link failure.

### 5.3.11	UE actions upon going to RRC_IDLE

The UE shall:

1>	reset MAC;

1>	set the variable pendingRNA-Update to false, if that is set to true;

1>	if going to RRC_IDLE was triggered by reception of the RRCRelease message including a waitTime:

2>	if T302 is running:

3>	stop timer T302;

2>	start timer T302 with the value set to the waitTime;

2>	inform upper layers that access barring is applicable for all access categories except categories '0' and '2'.

1>	else:

2>	if T302 is running:

3>	stop timer T302;

3>	perform the actions as specified in 5.3.14.4;

1>	if T390 is running:

2>	stop timer T390 for all access categories;

2>	perform the actions as specified in 5.3.14.4;

1>	if the UE is leaving RRC_INACTIVE:

2>	if going to RRC_IDLE was not triggered by reception of the RRCRelease message:

3>	if stored, discard the cell reselection priority information provided by the cellReselectionPriorities;

3>	stop the timer T320, if running;

1>	stop all timers that are running except T302, T320 and T325;

1>	discard the UE Inactive AS context, if any;

1>	release the suspendConfig, if configured;

1>	discard the KgNB key, the S-KgNB key, the S-KeNB key, the KRRCenc key, the KRRCint key, the KUPint key and the KUPenc key, if any;

1>	release all radio resources, including release of the RLC entity, the MAC configuration and the associated PDCP entity and SDAP for all established RBs;

1>	indicate the release of the RRC connection to upper layers together with the release cause;

1>	except if going to RRC_IDLE was triggered by inter-RAT cell reselection while the UE is in RRC_INACTIVE or RRC_IDLE or when selecting an inter-RAT cell while T311 was running:

2>	enter RRC_IDLE and perform cell selection as specified in TS 38.304 [20];

### 5.3.12	UE actions upon PUCCH/SRS release request

Upon receiving a PUCCH release request from lower layers, for all bandwidth parts of an indicated serving cell the UE shall:

1>	release PUCCH-CSI-Resources configured in CSI-ReportConfig;

1>	release SchedulingRequestResourceConfig instances configured in PUCCH-Config.

Upon receiving an SRS release request from lower layers, for all bandwidth parts of an indicated serving cell the UE shall:

1>	release SRS-Resource instances configured in SRS-Config.

### 5.3.13	RRC connection resume

#### 5.3.13.1	General

Figure 5.3.13.1-1: RRC connection resume, successful

Figure 5.3.13.1-2: RRC connection resume fallback to RRC connection establishment, successful

Figure 5.3.13.1-3: RRC connection resume followed by network release, successful

Figure 5.3.13.1-4: RRC connection resume followed by network suspend, successful

Figure 5.3.13.1-5: RRC connection resume, network reject

The purpose of this procedure is to resume a suspended RRC connection, including resuming SRB(s) and DRB(s) or perform an RNA update.

#### 5.3.13.2	Initiation

The UE initiates the procedure when upper layers or AS (when responding to RAN paging or upon triggering RNA updates while the UE is in RRC_INACTIVE) requests the resume of a suspended RRC connection.

The UE shall ensure having valid and up to date essential system information as specified in clause 5.2.2.2 before initiating this procedure.

Upon initiation of the procedure, the UE shall:

1>	if the resumption of the RRC connection is triggered by response to NG-RAN paging:

2>	select '0' as the Access Category;

2>	perform the unified access control procedure as specified in 5.3.14 using the selected Access Category and one or more Access Identities provided by upper layers;

3>	if the access attempt is barred, the procedure ends;

1>	else if the resumption of the RRC connection is triggered by upper layers:

2>	if the upper layers provide an Access Category and one or more Access Identities:

3>	perform the unified access control procedure as specified in 5.3.14 using the Access Category and Access Identities provided by upper layers;

4>	if the access attempt is barred, the procedure ends;

2>	set the resumeCause in accordance with the information received from upper layers;

1>	else if the resumption of the RRC connection is triggered due to an RNA update as specified in 5.3.13.8:

2>	if an emergency service is ongoing:

NOTE:	How the RRC layer in the UE is aware of an ongoing emergency service is up to UE implementation.

3>	select '2' as the Access Category;

3>	set the resumeCause to emergency;

2>	else:

3>	select '8' as the Access Category;

2>	perform the unified access control procedure as specified in 5.3.14 using the selected Access Category and one or more Access Identities to be applied as specified in TS 24.501 [23];

3>	if the access attempt is barred:

4>	set the variable pendingRNA-Update to true;

4>	the procedure ends;

1>	if the UE is in NE-DC or NR-DC:

2>	release the MR-DC related configurations (i.e., as specified in 5.3.5.10) from the UE Inactive AS context, if stored;

1>	release the MCG SCell(s) from the UE Inactive AS context, if stored;

1>	apply the default L1 parameter values as specified in corresponding physical layer specifications, except for the parameters for which values are provided in SIB1;

1>	apply the default SRB1 configuration as specified in 9.2.1;

1>	apply the default MAC Cell Group configuration as specified in 9.2.2;

1>	release delayBudgetReportingConfig from the UE Inactive AS context, if stored;

1>	stop timer T342, if running;

1>	release overheatingAssistanceConfig from the UE Inactive AS context, if stored;

1>	stop timer T345, if running;

1>	apply the CCCH configuration as specified in 9.1.1.2;

1>	apply the timeAlignmentTimerCommon included in SIB1;

1>	start timer T319;

1>	set the variable pendingRNA-Update to false;

1>	initiate transmission of the RRCResumeRequest message or RRCResumeRequest1 in accordance with 5.3.13.3.

#### 5.3.13.3	Actions related to transmission of RRCResumeRequest or RRCResumeRequest1 message

The UE shall set the contents of RRCResumeRequest or RRCResumeRequest1 message as follows:

1>	if field useFullResumeID is signalled in SIB1:

2>	select RRCResumeRequest1 as the message to use;

2>	set the resumeIdentity to the stored fullI-RNTI value;

1>	else:

2>	select RRCResumeRequest as the message to use;

2>	set the resumeIdentity to the stored shortI-RNTI value;

1>	restore the RRC configuration, RoHC state, the stored QoS flow to DRB mapping rules and the KgNB and KRRCint keys from the stored UE Inactive AS context except the masterCellGroup and pdcp-Config;

1>	set the resumeMAC-I to the 16 least significant bits of the MAC-I calculated:

2>	over the ASN.1 encoded as per clause 8 (i.e., a multiple of 8 bits) VarResumeMAC-Input;

2>	with the KRRCint key in the UE Inactive AS Context and the previously configured integrity protection algorithm; and

2>	with all input bits for COUNT, BEARER and DIRECTION set to binary ones;

1>	derive the KgNB key based on the current KgNB key or the NH, using the stored nextHopChainingCount value, as specified in TS 33.501 [11];

1>	derive the KRRCenc key, the KRRCint key, the KUPint key and the KUPenc key;

1>	configure lower layers to apply integrity protection for all radio bearers except SRB0 using the configured algorithm and the KRRCint key and KUPint key derived in this subclause immediately, i.e., integrity protection shall be applied to all subsequent messages received and sent by the UE;

NOTE 1:	Only DRBs with previously configured UP integrity protection shall resume integrity protection.

1>	configure lower layers to apply ciphering for all radio bearers except SRB0 and to apply the configured ciphering algorithm, the KRRCenc key and the KUPenc key derived in this subclause, i.e. the ciphering configuration shall be applied to all subsequent messages received and sent by the UE;

1>	re-establish PDCP entities for SRB1;

1>	resume SRB1;

1>	submit the selected message RRCResumeRequest or RRCResumeRequest1 for transmission to lower layers.

NOTE 2:	Only DRBs with previously configured UP ciphering shall resume ciphering.

If lower layers indicate an integrity check failure while T319 is running, perform actions specified in 5.3.13.5.

The UE shall continue cell re-selection related measurements as well as cell re-selection evaluation. If the conditions for cell re-selection are fulfilled, the UE shall perform cell re-selection as specified in 5.3.13.6.

#### 5.3.13.4	Reception of the RRCResume by the UE

The UE shall:

1>	stop timer T319;

1>	stop timer T380, if running;

1>	if the RRCResume includes the fullConfig:

2>	perform the full configuration procedure as specified in 5.3.5.11;

1>	else:

2>	restore the masterCellGroup and pdcp-Config from the UE Inactive AS context;

1>	discard the UE Inactive AS context;

1>	release the suspendConfig except the ran-NotificationAreaInfo;

1>	if the RRCResume includes the masterCellGroup:

2>	perform the cell group configuration for the received masterCellGroup according to 5.3.5.5;

1>	if the RRCResume includes the radioBearerConfig:

2>	perform the radio bearer configuration according to 5.3.5.6;

1>	if the RRCResume message includes the sk-Counter:

2>	perform security key update procedure as specified in 5.3.5.7;

1>	if the RRCResume message includes the radioBearerConfig2:

2>	perform the radio bearer configuration according to 5.3.5.6;

1>	resume SRB2 and all DRBs;

1>	if stored, discard the cell reselection priority information provided by the cellReselectionPriorities or inherited from another RAT;

1>	stop timer T320, if running;

1>	if the RRCResume message includes the measConfig:

2>	perform the measurement configuration procedure as specified in 5.5.2;

1>	resume measurements if suspended;

1>	if T390 is running:

2>	stop timer T390 for all access categories;

2>	perform the actions as specified in 5.3.14.4;

1>	if T302 is running:

2>	stop timer T302;

2>	perform the actions as specified in 5.3.14.4;

1>	enter RRC_CONNECTED;

1>	indicate to upper layers that the suspended RRC connection has been resumed;

1>	stop the cell re-selection procedure;

1>	consider the current cell to be the PCell;

1>	set the content of the of RRCResumeComplete message as follows:

2>	if the upper layer provides NAS PDU, set the dedicatedNAS-Message to include the information received from upper layers;

2>	if the upper layer provides a PLMN, set the selectedPLMN-Identity to PLMN selected by upper layers (TS 24.501 [23]) from the PLMN(s) included in the plmn-IdentityList in SIB1;

2>	if the masterCellGroup contains the reportUplinkTxDirectCurrent:

3>	include the uplinkTxDirectCurrentList for each MCG serving cell with UL;

3>	include uplinkDirectCurrentBWP-SUL for each MCG serving cell configured with SUL carrier, if any, within the uplinkTxDirectCurrentList;

1>	submit the RRCResumeComplete message to lower layers for transmission;

1>	the procedure ends.

#### 5.3.13.5	T319 expiry or Integrity check failure from lower layers while T319 is running

The UE shall:

1>	if timer T319 expires or upon receiving Integrity check failure indication from lower layers while T319 is running:

2>	perform the actions upon going to RRC_IDLE as specified in 5.3.11 with release cause 'RRC Resume failure'.

#### 5.3.13.6	Cell re-selection or cell selection while T390, T319 or T302 is running (UE in RRC_INACTIVE)

The UE shall:

1>	if cell reselection occurs while T319 or T302 is running:

2>	perform the actions upon going to RRC_IDLE as specified in 5.3.11 with release cause 'RRC Resume failure';

1>	else if cell selection or reselection occurs while T390 is running:

2>	stop T390 for all access categories;

2>	perform the actions as specified in 5.3.14.4.

#### 5.3.13.7	Reception of the RRCSetup by the UE

The UE shall:

1>	perform the RRC connection setup procedure as specified in 5.3.3.4.

#### 5.3.13.8	RNA update

In RRC_INACTIVE state, the UE shall:

1>	if T380 expires; or

1>	if RNA Update is triggered at reception of SIB1, as specified in 5.2.2.4.2:

2>	initiate RRC connection resume procedure in 5.3.13.2 with resumeCause set to rna-Update;

1>	if barring is alleviated for Access Category '8', as specified in 5.3.14.4:

2>	if upper layers do not request RRC the resumption of an RRC connection, and

2>	if the variable pendingRNA-Update is set to true:

3>	initiate RRC connection resume procedure in 5.3.13.2 with resumeCause value set to rna-Update.

If the UE in RRC_INACTIVE state fails to find a suitable cell and camps on the acceptable cell to obtain limited service as defined in TS 38.304 [20], the UE shall:

1>	perform the actions upon going to RRC_IDLE as specified in 5.3.11 with release cause 'other'.

NOTE:	It is left to UE implementation how to behave when T380 expires while the UE is camped neither on a suitable nor on an acceptable cell.

#### 5.3.13.9	Reception of the RRCRelease by the UE

The UE shall:

1>	perform the actions as specified in 5.3.8.

#### 5.3.13.10	Reception of the RRCReject by the UE

The UE shall:

1>	perform the actions as specified in 5.3.15.

#### 5.3.13.11	Inability to comply with RRCResume

The UE shall:

1>	if the UE is unable to comply with (part of) the configuration included in the RRCResume message;

2>	perform the actions upon going to RRC_IDLE as specified in 5.3.11 with release cause ′RRC Resume failure′.

NOTE 1:	The UE may apply above failure handling also in case the RRCResume message causes a protocol error for which the generic error handling as defined in 10 specifies that the UE shall ignore the message.

NOTE 2:	If the UE is unable to comply with part of the configuration, it does not apply any part of the configuration, i.e. there is no partial success/failure.

#### 5.3.13.12	Inter RAT cell reselection

Upon reselecting to an inter-RAT cell, the UE shall:

1>	perform the actions upon going to RRC_IDLE as specified in 5.3.11, with release cause 'other'.

### 5.3.14	Unified Access Control

#### 5.3.14.1	General

The purpose of this procedure is to perform access barring check for an access attempt associated with a given Access Category and one or more Access Identities upon request from upper layers according to TS 24.501 [23] or the RRC layer.

After a PCell change in RRC_CONNECTED the UE shall defer access barring checks until it has obtained SIB1 (as specified in 5.2.2.2) from the target cell.

#### 5.3.14.2	Initiation

Upon initiation of the procedure, the UE shall:

1>	if timer T390 is running for the Access Category:

2>	consider the access attempt as barred;

1>	else if timer T302 is running and the Access Category is neither '2' nor '0':

2>	consider the access attempt as barred;

1>	else:

2>	if the Access Category is '0':

3>	consider the access attempt as allowed;

2>	else:

3>	if SIB1 includes uac-BarringPerPLMN-List and the uac-BarringPerPLMN-List contains an UAC-BarringPerPLMN entry with the plmn-IdentityIndex corresponding to the PLMN selected by upper layers (see TS 24.501 [23]):

4>	select the UAC-BarringPerPLMN entry with the plmn-IdentityIndex corresponding to the PLMN selected by upper layers;

4>	in the remainder of this procedure, use the selected UAC-BarringPerPLMN entry (i.e. presence or absence of access barring parameters in this entry) irrespective of the uac-BarringForCommon included in SIB1;

3>	else if SIB1 includes uac-BarringForCommon:

4>	in the remainder of this procedure use the uac-BarringForCommon (i.e. presence or absence of these parameters) included in SIB1;

3>	else:

4>	consider the access attempt as allowed;

3>	if uac-BarringForCommon is applicable or the uac-ACBarringListType indicates that uac-ExplicitACBarringList is used:

4>	if the corresponding UAC-BarringPerCatList contains a UAC-BarringPerCat entry corresponding to the Access Category:

5>	select the UAC-BarringPerCat entry;

5>	if the uac-BarringInfoSetList contains a UAC-BarringInfoSet entry corresponding to the selected uac-barringInfoSetIndex in the UAC-BarringPerCat:

6>	select the UAC-BarringInfoSet entry;

6>	perform access barring check for the Access Category as specified in 5.3.14.5, using the selected UAC-BarringInfoSet as "UAC barring parameter";

5>	else:

6>	consider the access attempt as allowed;

4>	else:

5>	consider the access attempt as allowed;

3>	else if the uac-ACBarringListType indicates that uac-ImplicitACBarringList is used:

4>	select the uac-BarringInfoSetIndex corresponding to the Access Category in the uac-ImplicitACBarringList;

4>	if the uac-BarringInfoSetList contains the UAC-BarringInfoSet entry corresponding to the selected uac-BarringInfoSetIndex:

5>	select the UAC-BarringInfoSet entry;

5>	perform access barring check for the Access Category as specified in 5.3.14.5, using the selected UAC-BarringInfoSet as "UAC barring parameter";

4>	else:

5>	consider the access attempt as allowed;

3>	else:

4>	consider the access attempt as allowed;

1>	if the access barring check was requested by upper layers:

2>	if the access attempt is considered as barred:

3>	if timer T302 is running:

4>	if timer T390 is running for Access Category '2':

5>	inform the upper layer that access barring is applicable for all access categories except categories '0', upon which the procedure ends;

4>	else

5>	inform the upper layer that access barring is applicable for all access categories except categories '0' and '2', upon which the procedure ends;

3>	else:

4>	inform upper layers that the access attempt for the Access Category is barred, upon which the procedure ends;

2>	else:

3>	inform upper layers that the access attempt for the Access Category is allowed, upon which the procedure ends;

1>	else:

2>	the procedure ends.

#### 5.3.14.3	Void

#### 5.3.14.4	T302, T390 expiry or stop (Barring alleviation)

The UE shall:

1>	if timer T302 expires or is stopped:

2>	for each Access Category for which T390 is not running:

3>	consider the barring for this Access Category to be alleviated:

1>	else if timer T390 corresponding to an Access Category other than '2' expires or is stopped, and if timer T302 is not running:

2>	consider the barring for this Access Category to be alleviated;

1>	else if timer T390 corresponding to the Access Category '2' expires or is stopped:

2>	consider the barring for this Access Category to be alleviated;

1>	when barring for an Access Category is considered being alleviated:

2>	if the Access Category was informed to upper layers as barred:

3>	inform upper layers about barring alleviation for the Access Category.

2>	if barring is alleviated for Access Category '8':

3>	perform actions specified in 5.3.13.8;

#### 5.3.14.5	Access barring check

The UE shall:

1>	if one or more Access Identities are indicated according to TS 24.501 [23], and

1>	if for at least one of these Access Identities the corresponding bit in the uac-BarringForAccessIdentity contained in "UAC barring parameter" is set to zero:

2>	consider the access attempt as allowed;

1>	else:

2>	draw a random number 'rand' uniformly distributed in the range: 0 ≤ rand < 1;

2>	if 'rand' is lower than the value indicated by uac-BarringFactor included in "UAC barring parameter":

3>	consider the access attempt as allowed;

2>	else:

3>	consider the access attempt as barred;

1>	if the access attempt is considered as barred:

2>	draw a random number 'rand' that is uniformly distributed in the range 0 ≤ rand < 1;

2>	start timer T390 for the Access Category with the timer value calculated as follows, using the uac-BarringTime included in "AC barring parameter":

T390 = (0.7+ 0.6 * rand) * uac-BarringTime.

### 5.3.15	RRC connection reject

#### 5.3.15.1	Initiation

The UE initiates the procedure upon the reception of RRCReject when the UE tries to establish or resume an RRC connection.

#### 5.3.15.2	Reception of the RRCReject by the UE

The UE shall:

1>	stop timer T300, if running;

1>	stop timer T319, if running;

1>	stop timer T302, if running;

1>	reset MAC and release the default MAC Cell Group configuration;

1>	if waitTime is configured in the RRCReject:

2>	start timer T302, with the timer value set to the waitTime;

1>	if RRCReject is received in response to a request from upper layers:

2>	inform the upper layer that access barring is applicable for all access categories except categories '0' and '2';

1>	if RRCReject is received in response to an RRCSetupRequest:

2>	inform upper layers about the failure to setup the RRC connection, upon which the procedure ends;

1>	else if RRCReject is received in response to an RRCResumeRequest or an RRCResumeRequest1:

2>	if resume is triggered by upper layers:

3>	inform upper layers about the failure to resume the RRC connection;

2>	if resume is triggered due to an RNA update:

3>	set the variable pendingRNA-Update to true;

2>	discard the current KgNB key, the KRRCenc key, the KRRCint key, the KUPint key and the KUPenc key derived in accordance with 5.3.13.3;

2>	suspend SRB1, upon which the procedure ends;

The RRC_INACTIVE UE shall continue to monitor paging while the timer T302 is running.

## 5.4	Inter-RAT mobility

### 5.4.1	Introduction

Network controlled inter-RAT mobility between NR and E-UTRA is supported, where E-UTRA can be connected to either EPC or 5GC.

### 5.4.2	Handover to NR

#### 5.4.2.1	General

Figure 5.4.2.1-1: Handover to NR, successful

The purpose of this procedure is to, under the control of the network, transfer a connection between the UE and another Radio Access Network (e.g. E-UTRAN) to NR.

The handover to NR procedure applies when SRBs, possibly in combination with DRBs, are established in another RAT. Handover from E-UTRA to NR applies only after integrity has been activated in E-UTRA.

#### 5.4.2.2	Initiation

The RAN using another RAT initiates the handover to NR procedure, in accordance with the specifications applicable for the other RAT, by sending the RRCReconfiguration message via the radio access technology from which the inter-RAT handover is performed.

The network applies the procedure as follows:

-	to activate ciphering, possibly using NULL algorithm, if not yet activated in the other RAT;

-	to re-establish SRBs and one or more DRBs;

#### 5.4.2.3	Reception of the RRCReconfiguration by the UE

The UE shall:

1>	apply the default L1 parameter values as specified in corresponding physical layer specifications except for the parameters for which values are provided in SIB1;

1>	apply the default MAC Cell Group configuration as specified in 9.2.2;

1>	perform RRC reconfiguration procedure as specified in 5.3.5;

NOTE:	If the UE is connected to 5GC of the source E-UTRA cell, the delta configuration for PDCP and SDAP can be used for intra-system inter-RAT handover. For other cases, source RAT configuration is not considered when the UE applies the reconfiguration message of target RAT.

### 5.4.3	Mobility from NR

#### 5.4.3.1	General

Figure 5.4.3.1-1: Mobility from NR, successful

Figure 5.4.3.1-2: Mobility from NR, failure

The purpose of this procedure is to move a UE in RRC_CONNECTED to a cell using other RAT, e.g. E-UTRA. The mobility from NR procedure covers the following type of mobility:

-	handover, i.e. the MobilityFromNRCommand message includes radio resources that have been allocated for the UE in the target cell;

#### 5.4.3.2	Initiation

The network initiates the mobility from NR procedure to a UE in RRC_CONNECTED, possibly in response to a MeasurementReport message, by sending a MobilityFromNRCommand message. The network applies the procedure as follows:

-	the procedure is initiated only when AS security has been activated, and SRB2 with at least one DRB are setup and not suspended.

#### 5.4.3.3	Reception of the MobilityFromNRCommand by the UE

The UE shall:

1>	stop timer T310, if running;

1>	if T390 is running:

2>	stop timer T390 for all access categories;

2>	perform the actions as specified in 5.3.14.4;

1>	if the targetRAT-Type is set to eutra:

2>	consider inter-RAT mobility as initiated towards E-UTRA;

2>	forward the nas-SecurityParamFromNR to the upper layers, if included;

1>	access the target cell indicated in the inter-RAT message in accordance with the specifications of the target RAT.

#### 5.4.3.4	Successful completion of the mobility from NR

Upon successfully completing the handover, at the source side the UE shall:

1>	reset MAC;

1>	stop all timers that are running;

1>	release ran-NotificationAreaInfo, if stored;

1>	release the AS security context including the KRRCenc key, the KRRCint key, the KUPint key and the KUPenc key, if stored;

1>	release all radio resources, including release of the RLC entity and the MAC configuration;

1>	release the associated PDCP entity and SDAP entity for all established RBs;

NOTE :	PDCP and SDAP configured by the source RAT prior to the handover that are reconfigured and re-used by target RAT when delta signalling (i.e., during inter-RAT intra-system handover when fullConfig is not present) is used, are not released as part of this procedure.

1>	if the targetRAT-Type is set to eutra and the nas-SecurityParamFromNR is included:

2>	indicate the release of the RRC connection to upper layers together with the release cause 'other'.

#### 5.4.3.5	Mobility from NR failure

The UE shall:

1>	if the UE does not succeed in establishing the connection to the target radio access technology; or

1>	if the UE is unable to comply with any part of the configuration included in the MobilityFromNRCommand message; or

1>	if there is a protocol error in the inter RAT information included in the MobilityFromNRCommand message, causing the UE to fail the procedure according to the specifications applicable for the target RAT:

2>	revert back to the configuration used in the source PCell;

2>	initiate the connection re-establishment procedure as specified in subclause 5.3.7.

## 5.5	Measurements

### 5.5.1	Introduction

The network may configure an RRC_CONNECTED UE to perform measurements and report them in accordance with the measurement configuration. The measurement configuration is provided by means of dedicated signalling i.e. using the RRCReconfiguration or RRCResume.

The network may configure the UE to perform the following types of measurements:

-	NR measurements;

-	Inter-RAT measurements of E-UTRA frequencies.

The network may configure the UE to report the following measurement information based on SS/PBCH block(s):

-	Measurement results per SS/PBCH block;

-	Measurement results per cell based on SS/PBCH block(s);

-	SS/PBCH block(s) indexes.

The network may configure the UE to report the following measurement information based on CSI-RS resources:

-	Measurement results per CSI-RS resource;

-	Measurement results per cell based on CSI-RS resource(s);

-	CSI-RS resource measurement identifiers.

The measurement configuration includes the following parameters:

1.	Measurement objects: A list of objects on which the UE shall perform the measurements.

-	For intra-frequency and inter-frequency measurements a measurement object indicates the frequency/time location and subcarrier spacing of reference signals to be measured. Associated with this measurement object, the network may configure a list of cell specific offsets, a list of 'blacklisted' cells and a list of 'whitelisted' cells. Blacklisted cells are not applicable in event evaluation or measurement reporting. Whitelisted cells are the only ones applicable in event evaluation or measurement reporting.

-	The measObjectId of the MO which corresponds to each serving cell is indicated by servingCellMO within the serving cell configuration.

-	For inter-RAT E-UTRA measurements a measurement object is a single E-UTRA carrier frequency. Associated with this E-UTRA carrier frequency, the network can configure a list of cell specific offsets, a list of 'blacklisted' cells and a list of 'whitelisted' cells. Blacklisted cells are not applicable in event evaluation or measurement reporting. Whitelisted cells are the only ones applicable in event evaluation or measurement reporting.

2.	Reporting configurations: A list of reporting configurations where there can be one or multiple reporting configurations per measurement object. Each reporting configuration consists of the following:

-	Reporting criterion: The criterion that triggers the UE to send a measurement report. This can either be periodical or a single event description.

-	RS type: The RS that the UE uses for beam and cell measurement results (SS/PBCH block or CSI-RS).

-	Reporting format: The quantities per cell and per beam that the UE includes in the measurement report (e.g. RSRP) and other associated information such as the maximum number of cells and the maximum number beams per cell to report.

3.	Measurement identities: A list of measurement identities where each measurement identity links one measurement object with one reporting configuration. By configuring multiple measurement identities, it is possible to link more than one measurement object to the same reporting configuration, as well as to link more than one reporting configuration to the same measurement object. The measurement identity is also included in the measurement report that triggered the reporting, serving as a reference to the network.

4.	Quantity configurations: The quantity configuration defines the measurement filtering configuration used for all event evaluation and related reporting, and for periodical reporting of that measurement. For NR measurements, the network may configure up to 2 quantity configurations with a reference in the NR measurement object to the configuration that is to be used. In each configuration, different filter coefficients can be configured for different measurement quantities, for different RS types, and for measurements per cell and per beam.

5.	Measurement gaps: Periods that the UE may use to perform measurements.

A UE in RRC_CONNECTED maintains a measurement object list, a reporting configuration list, and a measurement identities list according to signalling and procedures in this specification. The measurement object list possibly includes NR measurement object(s) and inter-RAT objects. Similarly, the reporting configuration list includes NR and inter-RAT reporting configurations. Any measurement object can be linked to any reporting configuration of the same RAT type. Some reporting configurations may not be linked to a measurement object. Likewise, some measurement objects may not be linked to a reporting configuration.

The measurement procedures distinguish the following types of cells:

1.	The NR serving cell(s) – these are the SpCell and one or more SCells.

2.	Listed cells – these are cells listed within the measurement object(s).

3.	Detected cells – these are cells that are not listed within the measurement object(s) but are detected by the UE on the SSB frequency(ies) and subcarrier spacing(s) indicated by the measurement object(s).

For NR measurement object(s), the UE measures and reports on the serving cell(s), listed cells and/or detected cells. For inter-RAT measurements object(s) of E-UTRA, the UE measures and reports on listed cells and detected cells.

Whenever the procedural specification, other than contained in sub-clause 5.5.2, refers to a field it concerns a field included in the VarMeasConfig unless explicitly stated otherwise i.e. only the measurement configuration procedure covers the direct UE action related to the received measConfig.

In NR-DC, the UE may receive two independent measConfig:

-	a measConfig, associated with MCG, that is included in the RRCReconfiguration message received via SRB1; and

-	a measConfig, associated with SCG, that is included in the RRCReconfiguration message received via SRB3, or, alternatively, included within a RRCReconfiguration message embedded in a RRCReconfiguration message received via SRB1.

In this case, the UE maintains two independent VarMeasConfig and VarMeasReportList, one associated with each measConfig, and independently performs all the procedures in clause 5.5 for each measConfig and the associated VarMeasConfig and VarMeasReportList, unless explicitly stated otherwise.

### 5.5.2	Measurement configuration

#### 5.5.2.1	General

The network applies the procedure as follows:

-	to ensure that, whenever the UE has a measConfig associated with a CG, it includes a measObject for the SpCell and for each NR SCell of the CG to be measured;

-	to configure at most one measurement identity across all CGs using a reporting configuration with the reportType set to reportCGI;

-	to ensure that, in the measConfig associated with a CG:

-	for all SSB based measurements there is at most one measurement object with the same ssbFrequency;

-	an smtc1 included in any measurement object with the same ssbFrequency has the same value and that an smtc2 included in any measurement object with the same ssbFrequency has the same value;

-	to ensure that all measurement objects configured in this specification and in TS 36.331 [10] with the same ssbFrequency have the same ssbSubcarrierSpacing;

-	to ensure that, if a measurement object associated with the MCG has the same ssbFrequency as a measurement object associated with the SCG:

-	for that ssbFrequency, the measurement window according to the smtc1 configured by the MCG includes the measurement window according to the smtc1 configured by the SCG, or vice-versa, with an accuracy of the maximum receive timing difference specified in TS 38.133 [14].

-	if both measurement objects are used for RSSI measurements, bits in measurementSlots in both objects corresponding to the same slot are set to the same value. Also, the endSymbol is the same in both objects.

-	to ensure that, if a measurement object has the same ssbFrequency as a measurement object configured in TS 36.331 [10]:

-	for that ssbFrequency, the measurement window according to the smtc configured in TS 36.331 [10] includes the measurement window according to the smtc1 configured in TS 38.331, or vice-versa, with an accuracy of the maximum receive timing difference specified in TS 38.133 [14].

-	if both measurement objects are used for RSSI measurements, bits in measurementSlots in both objects corresponding to the same slot are set to the same value. Also, the endSymbol is the same in both objects.

-	when the UE is in NE-DC, NR-DC, or NR standalone, to configure at most one measurement identity across all CGs using a reporting configuration with the reportType set to reportSFTD;

For CSI-RS resources, the network applies the procedure as follows:

-	to ensure that all CSI-RS resources configured in each measurement object have the same center frequency, (startPRB+floor(nrofPRBs/2))

The UE shall:

1>	if the received measConfig includes the measObjectToRemoveList:

2>	perform the measurement object removal procedure as specified in 5.5.2.4;

1>	if the received measConfig includes the measObjectToAddModList:

2>	perform the measurement object addition/modification procedure as specified in 5.5.2.5;

1>	if the received measConfig includes the reportConfigToRemoveList:

2>	perform the reporting configuration removal procedure as specified in 5.5.2.6;

1>	if the received measConfig includes the reportConfigToAddModList:

2>	perform the reporting configuration addition/modification procedure as specified in 5.5.2.7;

1>	if the received measConfig includes the quantityConfig:

2>	perform the quantity configuration procedure as specified in 5.5.2.8;

1>	if the received measConfig includes the measIdToRemoveList:

2>	perform the measurement identity removal procedure as specified in 5.5.2.2;

1>	if the received measConfig includes the measIdToAddModList:

2>	perform the measurement identity addition/modification procedure as specified in 5.5.2.3;

1>	if the received measConfig includes the measGapConfig:

2>	perform the measurement gap configuration procedure as specified in 5.5.2.9;

1>	if the received measConfig includes the measGapSharingConfig:

2>	perform the measurement gap sharing configuration procedure as specified in 5.5.2.11;

1>	if the received measConfig includes the s-MeasureConfig:

2>	if s-MeasureConfig is set to ssb-RSRP, set parameter ssb-RSRP of s-MeasureConfig within VarMeasConfig to the lowest value of the RSRP ranges indicated by the received value of s-MeasureConfig;

2>	else, set parameter csi-RSRP of s-MeasureConfig within VarMeasConfig to the lowest value of the RSRP ranges indicated by the received value of s-MeasureConfig.

#### 5.5.2.2	Measurement identity removal

The UE shall:

1>	for each measId included in the received measIdToRemoveList that is part of the current UE configuration in VarMeasConfig:

2>	remove the entry with the matching measId from the measIdList within the VarMeasConfig;

2>	remove the measurement reporting entry for this measId from the VarMeasReportList, if included;

2>	stop the periodical reporting timer or timer T321 or timer T322, whichever one is running, and reset the associated information (e.g. timeToTrigger) for this measId.

NOTE:	The UE does not consider the message as erroneous if the measIdToRemoveList includes any measId value that is not part of the current UE configuration.

#### 5.5.2.3	Measurement identity addition/modification

The network applies the procedure as follows:

-	configure a measId only if the corresponding measurement object, the corresponding reporting configuration and the corresponding quantity configuration, are configured.

The UE shall:

1>	for each measId included in the received measIdToAddModList:

2>	if an entry with the matching measId exists in the measIdList within the VarMeasConfig:

3>	replace the entry with the value received for this measId;

2>	else:

3>	add a new entry for this measId within the VarMeasConfig;

2>	remove the measurement reporting entry for this measId from the VarMeasReportList, if included;

2>	stop the periodical reporting timer or timer T321 or timer T322, whichever one is running, and reset the associated information (e.g. timeToTrigger) for this measId;

2>	if the reportType is set to reportCGI in the reportConfig associated with this measId:

3>	if the measObject associated with this measId concerns E-UTRA:

4>	start timer T321 with the timer value set to 1 second for this measId;

3>	if the measObject associated with this measId concerns NR:

4>	if the measObject associated with this measId concerns FR1:

5>	start timer T321 with the timer value set to 2 seconds for this measId;

4>	if the measObject associated with this measId concerns FR2:

5>	start timer T321 with the timer value set to 16 seconds for this measId.

2>	if the reportType is set to reportSFTD in the reportConfigNR associated with this measId and the drx-SFTD-NeighMeas is included:

3>	if the measObject associated with this measId concerns FR1:

4>	start timer T322 with the timer value set to 3 seconds for this measId;

3>	if the measObject associated with this measId concerns FR2:

4>	start timer T322 with the timer value set to 24 seconds for this measId.

#### 5.5.2.4	Measurement object removal

The UE shall:

1>	for each measObjectId included in the received measObjectToRemoveList that is part of measObjectList in VarMeasConfig:

2>	remove the entry with the matching measObjectId from the measObjectList within the VarMeasConfig;

2>	remove all measId associated with this measObjectId from the measIdList within the VarMeasConfig, if any;

2>	if a measId is removed from the measIdList:

3>	remove the measurement reporting entry for this measId from the VarMeasReportList, if included;

3>	stop the periodical reporting timer or timer T321 or timer T322, whichever is running, and reset the associated information (e.g. timeToTrigger) for this measId.

NOTE:	The UE does not consider the message as erroneous if the measObjectToRemoveList includes any measObjectId value that is not part of the current UE configuration.

#### 5.5.2.5	Measurement object addition/modification

The UE shall:

1>	for each measObjectId included in the received measObjectToAddModList:

2>	if an entry with the matching measObjectId exists in the measObjectList within the VarMeasConfig, for this entry:

3>	reconfigure the entry with the value received for this measObject, except for the fields cellsToAddModList, blackCellsToAddModList, whiteCellsToAddModList, cellsToRemoveList, blackCellsToRemoveList and whiteCellsToRemoveList;

3>	if the received measObject includes the cellsToRemoveList:

4>	for each physCellId included in the cellsToRemoveList:

5>	remove the entry with the matching physCellId from the cellsToAddModList;

3>	if the received measObject includes the cellsToAddModList:

4>	for each physCellId value included in the cellsToAddModList:

5>	if an entry with the matching physCellId exists in the cellsToAddModList:

6>	replace the entry with the value received for this physCellId;

5>	else:

6>	add a new entry for the received physCellId to the cellsToAddModList;

3>	if the received measObject includes the blackCellsToRemoveList:

4>	for each pci-RangeIndex included in the blackCellsToRemoveList:

5>	remove the entry with the matching pci-RangeIndex from the blackCellsToAddModList;

NOTE 1:	For each pci-RangeIndex included in the blackCellsToRemoveList that concerns overlapping ranges of cells, a cell is removed from the blacklist of cells only if all PCI ranges containing it are removed.

3>	if the received measObject includes the blackCellsToAddModList:

4>	for each pci-RangeIndex included in the blackCellsToAddModList:

5>	if an entry with the matching pci-RangeIndex is included in the blackCellsToAddModList:

6>	replace the entry with the value received for this pci-RangeIndex;

5>	else:

6>	add a new entry for the received pci-RangeIndex to the blackCellsToAddModList;

3>	if the received measObject includes the whiteCellsToRemoveList:

4>	for each pci-RangeIndex included in the whiteCellsToRemoveList:

5>	remove the entry with the matching pci-RangeIndex from the whiteCellsToAddModList;

NOTE2:	For each pci-RangeIndex included in the whiteCellsToRemoveList that concerns overlapping ranges of cells, a cell is removed from the whitelist of cells only if all PCI ranges containing it are removed.

3>	if the received measObject includes the whiteCellsToAddModList:

4>	for each pci-RangeIndex included in the whiteCellsToAddModList:

5>	if an entry with the matching pci-RangeIndex is included in the whiteCellsToAddModList:

6>	replace the entry with the value received for this pci-RangeIndex;

5>	else:

6>	add a new entry for the received pci-RangeIndex to the whiteCellsToAddModList

3>	for each measId associated with this measObjectId in the measIdList within the VarMeasConfig, if any:

4>	remove the measurement reporting entry for this measId from the VarMeasReportList, if included;

4>	stop the periodical reporting timer or timer T321 or timer T322, whichever one is running, and reset the associated information (e.g. timeToTrigger) for this measId;

2>	else:

3>	add a new entry for the received measObject to the measObjectList within VarMeasConfig.

#### 5.5.2.6	Reporting configuration removal

The UE shall:

1>	for each reportConfigId included in the received reportConfigToRemoveList that is part of the current UE configuration in VarMeasConfig:

2>	remove the entry with the matching reportConfigId from the reportConfigList within the VarMeasConfig;

2>	remove all measId associated with the reportConfigId from the measIdList within the VarMeasConfig, if any;

2>	if a measId is removed from the measIdList:

3>	remove the measurement reporting entry for this measId from the VarMeasReportList, if included;

3>	stop the periodical reporting timer or timer T321 or timer T322, whichever one is running, and reset the associated information (e.g. timeToTrigger) for this measId.

NOTE:	The UE does not consider the message as erroneous if the reportConfigToRemoveList includes any reportConfigId value that is not part of the current UE configuration.

#### 5.5.2.7	Reporting configuration addition/modification

The UE shall:

1>	for each reportConfigId included in the received reportConfigToAddModList:

2>	if an entry with the matching reportConfigId exists in the reportConfigList within the VarMeasConfig, for this entry:

3>	reconfigure the entry with the value received for this reportConfig;

3>	for each measId associated with this reportConfigId included in the measIdList within the VarMeasConfig, if any:

4>	remove the measurement reporting entry for this measId from the VarMeasReportList, if included;

4>	stop the periodical reporting timer or timer T321 or timer T322, whichever one is running, and reset the associated information (e.g. timeToTrigger) for this measId;

2>	else:

3>	add a new entry for the received reportConfig to the reportConfigList within the VarMeasConfig.

#### 5.5.2.8	Quantity configuration

The UE shall:

1>	for each RAT for which the received quantityConfig includes parameter(s):

2>	set the corresponding parameter(s) in quantityConfig within VarMeasConfig to the value of the received quantityConfig parameter(s);

1>	for each measId included in the measIdList within VarMeasConfig:

2>	remove the measurement reporting entry for this measId from the VarMeasReportList, if included;

2>	stop the periodical reporting timer or timer T321 or timer T322, whichever one is running, and reset the associated information (e.g. timeToTrigger) for this measId.

#### 5.5.2.9	Measurement gap configuration

The UE shall:

1>	if gapFR1 is set to setup:

2>	if an FR1 measurement gap configuration is already setup, release the FR1 measurement gap configuration;

2>	setup the FR1 measurement gap configuration indicated by the measGapConfig in accordance with the received gapOffset, i.e., the first subframe of each gap occurs at an SFN and subframe meeting the following condition:

SFN mod T = FLOOR(gapOffset/10);

subframe = gapOffset mod 10;

with T = MGRP/10 as defined in TS 38.133 [14];

2>	apply the specified timing advance mgta to the gap occurrences calculated above (i.e. the UE starts the measurement mgta ms before the gap subframe occurrences);

1>	else if gapFR1 is set to release:

2>	release the FR1 measurement gap configuration;

1>	if gapFR2 is set to setup:

2>	if an FR2 measurement gap configuration is already setup, release the FR2 measurement gap configuration;

2>	setup the FR2 measurement gap configuration indicated by the measGapConfig in accordance with the received gapOffset, i.e., the first subframe of each gap occurs at an SFN and subframe meeting the following condition:

SFN mod T = FLOOR(gapOffset/10);

subframe = gapOffset mod 10;

with T = MGRP/10 as defined in TS 38.133 [14];

2>	apply the specified timing advance mgta to the gap occurrences calculated above (i.e. the UE starts the measurement mgta ms before the gap subframe occurrences);

1>	else if gapFR2 is set to release:

2>	release the FR2 measurement gap configuration;

1>	if gapUE is set to setup:

2>	if a per UE measurement gap configuration is already setup, release the per UE measurement gap configuration;

2>	setup the per UE measurement gap configuration indicated by the measGapConfig in accordance with the received gapOffset, i.e., the first subframe of each gap occurs at an SFN and subframe meeting the following condition:

SFN mod T = FLOOR(gapOffset/10);

subframe = gapOffset mod 10;

with T = MGRP/10 as defined in TS 38.133 [14];

2>	apply the specified timing advance mgta to the gap occurrences calculated above (i.e. the UE starts the measurement mgta ms before the gap subframe occurrences);

1>	else if gapUE is set to release:

2>	release the per UE measurement gap configuration.

NOTE 1: For gapFR2 configuration, for the UE in NE-DC or NR-DC, the SFN and subframe of the serving cell indicated by the refServCellIndicator in gapFR2 is used in the gap calculation. Otherwise, the SFN and subframe of a serving cell on FR2 frequency is used in the gap calculation

NOTE 2: For gapFR1 or gapUE configuration, for the UE in NE-DC or NR-DC, the SFN and subframe of the serving cell indicated by the refServCellIndicator in corresponding gapFR1 or gapUE is used in the gap calculation. Otherwise, the SFN and subframe of the PCell is used in the gap calculation.

#### 5.5.2.10	Reference signal measurement timing configuration

The UE shall setup the first SS/PBCH block measurement timing configuration (SMTC) in accordance with the received periodicityAndOffset parameter (providing Periodicity and Offset value for the following condition) in the smtc1 configuration. The first subframe of each SMTC occasion occurs at an SFN and subframe of the NR SpCell meeting the following condition:

SFN mod T = (FLOOR (Offset/10));

if the Periodicity is larger than sf5:

subframe = Offset mod 10;

else:

subframe = Offset or (Offset +5);

with T = CEIL(Periodicity/10).

If smtc2 is present, for cells indicated in the pci-List parameter in smtc2 in the same MeasObjectNR, the UE shall setup an additional SS/PBCH block measurement timing configuration (SMTC) in accordance with the received periodicity parameter in the smtc2 configuration and use the Offset (derived from parameter periodicityAndOffset) and duration parameter from the smtc1 configuration. The first subframe of each SMTC occasion occurs at an SFN and subframe of the NR SpCell meeting the above condition.

On the indicated ssbFrequency, the UE shall not consider SS/PBCH block transmission in subframes outside the SMTC occasion for RRM measurements based on SS/PBCH blocks and for RRM measurements based on CSI-RS except for SFTD measurement (see TS 38.133 [14], subclause 9.3.8).

#### 5.5.2.11	Measurement gap sharing configuration

The UE shall:

1>	if gapSharingFR1 is set to setup:

2>	if an FR1 measurement gap sharing configuration is already setup:

3>	release the FR1 measurement gap sharing configuration;

2>	setup the FR1 measurement gap sharing configuration indicated by the measGapSharingConfig in accordance with the received gapSharingFR1 as defined in TS 38.133 [14];

1>	else if gapSharingFR1 is set to release:

2>	release the FR1 measurement gap sharing configuration;

1>	if gapSharingFR2 is set to setup:

2>	if an FR2 measurement gap sharing configuration is already setup:

3>	release the FR2 measurement gap sharing configuration;

2>	setup the FR2 measurement gap sharing configuration indicated by the measGapSharingConfig in accordance with the received gapSharingFR2 as defined in TS 38.133 [14];

1>	else if gapSharingFR2 is set to release:

2>	release the FR2 measurement gap sharing configuration.

1>	if gapSharingUE is set to setup:

2>	if a per UE measurement gap sharing configuration is already setup:

3>	release the per UE measurement gap sharing configuration;

2>	setup the per UE measurement gap sharing configuration indicated by the measGapSharingConfig in accordance with the received gapSharingUE as defined in TS 38.133 [14];

1>	else if gapSharingUE is set to release:

2>	release the per UE measurement gap sharing configuration.

### 5.5.3	Performing measurements

#### 5.5.3.1	General

An RRC_CONNECTED UE shall derive cell measurement results by measuring one or multiple beams associated per cell as configured by the network, as described in 5.5.3.3. For all cell measurement results in RRC_CONNECTED the UE applies the layer 3 filtering as specified in 5.5.3.2, before using the measured results for evaluation of reporting criteria and measurement reporting. For cell measurements, the network can configure RSRP, RSRQ or SINR as trigger quantity. Reporting quantities can be any combination of quantities (i.e. only RSRP; only RSRQ; only SINR; RSRP and RSRQ; RSRP and SINR; RSRQ and SINR; RSRP, RSRQ and SINR), irrespective of the trigger quantity.

The network may also configure the UE to report measurement information per beam (which can either be measurement results per beam with respective beam identifier(s) or only beam identifier(s)), derived as described in 5.5.3.3a. If beam measurement information is configured to be included in measurement reports, the UE applies the layer 3 beam filtering as specified in 5.5.3.2. On the other hand, the exact L1 filtering of beam measurements used to derive cell measurement results is implementation dependent.

The UE shall:

1>	whenever the UE has a measConfig, perform RSRP and RSRQ measurements for each serving cell for which servingCellMO is configured as follows:

2>	if the reportConfig associated with at least one measId included in the measIdList within VarMeasConfig contains an rsType set to ssb and ssb-ConfigMobility is configured in the measObject indicated by the servingCellMO:

3>	if the reportConfig associated with at least one measId included in the measIdList within VarMeasConfig contains a reportQuantityRS-Indexes and maxNrofRS-IndexesToReport and contains an rsType set to ssb:

4>	derive layer 3 filtered RSRP and RSRQ per beam for the serving cell based on SS/PBCH block, as described in 5.5.3.3a;

3>	derive serving cell measurement results based on SS/PBCH block, as described in 5.5.3.3;

2>	if the reportConfig associated with at least one measId included in the measIdList within VarMeasConfig contains an rsType set to csi-rs and CSI-RS-ResourceConfigMobility is configured in the measObject indicated by the servingCellMO:

3>	if the reportConfig associated with at least one measId included in the measIdList within VarMeasConfig contains a reportQuantityRS-Indexes and maxNrofRS-IndexesToReport and contains an rsType set to csi-rs:

4>	derive layer 3 filtered RSRP and RSRQ per beam for the serving cell based on CSI-RS, as described in 5.5.3.3a;

3>	derive serving cell measurement results based on CSI-RS, as described in 5.5.3.3;

1>	for each serving cell for which servingCellMO is configured, if the reportConfig associated with at least one measId included in the measIdList within VarMeasConfig contains SINR as trigger quantity and/or reporting quantity:

2>	if the reportConfig contains rsType set to ssb and ssb-ConfigMobility is configured in the servingCellMO:

3>	if the reportConfigcontains a reportQuantityRS-Indexes and maxNrofRS-IndexesToReport:

4>	derive layer 3 filtered SINR per beam for the serving cell based on SS/PBCH block, as described in 5.5.3.3a;

3>	derive serving cell SINR based on SS/PBCH block, as described in 5.5.3.3;

2>	if the reportConfig contains rsType set to csi-rs and CSI-RS-ResourceConfigMobility is configured in the servingCellMO:

3>	if the reportConfigcontains a reportQuantityRS-Indexes and maxNrofRS-IndexesToReport:

4>	derive layer 3 filtered SINR per beam for the serving cell based on CSI-RS, as described in 5.5.3.3a;

3>	derive serving cell SINR based on CSI-RS, as described in 5.5.3.3;

1>	for each measId included in the measIdList within VarMeasConfig:

2>	if the reportType for the associated reportConfig is set to reportCGI and timer T321 is running:

3>	perform the corresponding measurements on the frequency and RAT indicated in the associated measObject using available idle periods;

3>	if the cell indicated by reportCGI field for the associated measObject is an NR cell and that indicated cell is broadcasting SIB1 (see TS 38.213 [13], clause 13):

4>	try to acquire SIB1 in the concerned cell;

3>	if the cell indicated by reportCGI field is an E-UTRA cell:

4>	try to acquire SystemInformationBlockType1 in the concerned cell;

2>	if the reportType for the associated reportConfig is periodical or eventTriggered:

3>	if a measurement gap configuration is setup, or

3>	if the UE does not require measurement gaps to perform the concerned measurements:

4>	if s-MeasureConfig is not configured, or

4>	if s-MeasureConfig is set to ssb-RSRP and the NR SpCell RSRP based on SS/PBCH block, after layer 3 filtering, is lower than ssb-RSRP, or

4>	if s-MeasureConfig is set to csi-RSRP and the NR SpCell RSRP based on CSI-RS, after layer 3 filtering, is lower than csi-RSRP:

5>	if the measObject is associated to NR and the rsType is set to csi-rs:

6>	if reportQuantityRS-Indexes and maxNrofRS-IndexesToReport for the associated reportConfig are configured:

7>	derive layer 3 filtered beam measurements only based on CSI-RS for each measurement quantity indicated in reportQuantityRS-Indexes, as described in 5.5.3.3a;

6>	derive cell measurement results based on CSI-RS for the trigger quantity and each measurement quantity indicated in reportQuantityCell using parameters from the associated measObject, as described in 5.5.3.3;

5>	if the measObject is associated to NR and the rsType is set to ssb:

6>	if reportQuantityRS-Indexes and maxNrofRS-IndexesToReport for the associated reportConfig are configured:

7>	derive layer 3 beam measurements only based on SS/PBCH block for each measurement quantity indicated in reportQuantityRS-Indexes, as described in 5.5.3.3a;

6>	derive cell measurement results based on SS/PBCH block for the trigger quantity and each measurement quantity indicated in reportQuantityCell using parameters from the associated measObject, as described in 5.5.3.3;

5>	if the measObject is associated to E-UTRA:

6>	perform the corresponding measurements associated to neighbouring cells on the frequencies indicated in the concerned measObject, as described in 5.5.3.2;

2>	if the reportType for the associated reportConfig is set to reportSFTD and the numberOfReportsSent as defined within the VarMeasReportList for this measId is less than one:

3>	if the reportSFTD-Meas is set to true:

4>	if the measObject is associated to E-UTRA:

5>	perform SFTD measurements between the PCell and the E-UTRA PSCell;

5>	if the reportRSRP is set to true;

6>	perform RSRP measurements for the E-UTRA PSCell;

4>	else if the measObject is associated to NR:

5>	perform SFTD measurements between the PCell and the NR PSCell;

5>	if the reportRSRP is set to true;

6>	perform RSRP measurements for the NR PSCell based on SSB;

3>	else if the reportSFTD-NeighMeas is included:

4>	if the measObject is associated to NR:

5>	if the drx-SFTD-NeighMeas is included:

6>	perform SFTD measurements between the PCell and the NR neighbouring cell(s) detected based on parameters in the associated measObject using available idle periods;

5>	else:

6>	perform SFTD measurements between the PCell and the NR neighbouring cell(s) detected based on parameters in the associated measObject;

5>	if the reportRSRP is set to true:

6>	perform RSRP measurements based on SSB for the NR neighbouring cell(s) detected based on parameters in the associated measObject;

2>	perform the evaluation of reporting criteria as specified in 5.5.4.

#### 5.5.3.2	Layer 3 filtering

The UE shall:

1>	for each cell measurement quantity and for each beam measurement quantity that the UE performs measurements according to 5.5.3.1:

2>	filter the measured result, before using for evaluation of reporting criteria or for measurement reporting, by the following formula:

Fn = (1 – a)*Fn-1 + a*Mn

where

Mn is the latest received measurement result from the physical layer;

Fn is the updated filtered measurement result, that is used for evaluation of reporting criteria or for measurement reporting;

Fn-1 is the old filtered measurement result, where F0 is set to M1 when the first measurement result from the physical layer is received; and for NR, a = 1/2(ki/4), where ki is the filterCoefficient for the corresponding measurement quantity of the i:th QuantityConfigNR in quantityConfigNR-List, and i is indicated by quantityConfigIndex in MeasObjectNR; for E-UTRA, a = 1/2(k/4), where k is the filterCoefficient for the corresponding measurement quantity received by quantityConfigEUTRA in the quantityConfig;

2>	adapt the filter such that the time characteristics of the filter are preserved at different input rates, observing that the filterCoefficient k assumes a sample rate equal to X ms; The value of X is equivalent to one intra-frequency L1 measurement period as defined in TS 38.133 [14] assuming non-DRX operation, and depends on frequency range.

NOTE 1:	If k is set to 0, no layer 3 filtering is applicable.

NOTE 2:	The filtering is performed in the same domain as used for evaluation of reporting criteria or for measurement reporting, i.e., logarithmic filtering for logarithmic measurements.

NOTE 3:	The filter input rate is implementation dependent, to fulfil the performance requirements set in TS 38.133 [14]. For further details about the physical layer measurements, see TS 38.133 [14].

#### 5.5.3.3	Derivation of cell measurement results

The network may configure the UE to derive RSRP, RSRQ and SINR measurement results per cell associated to NR measurement objects based on parameters configured in the measObject (e.g. maximum number of beams to be averaged and beam consolidation thresholds) and in the reportConfig (rsType to be measured, SS/PBCH block or CSI-RS).

The UE shall:

1>	for each cell measurement quantity to be derived based on SS/PBCH block:

2>	if nrofSS-BlocksToAverage in the associated measObject is not configured; or

2>	if absThreshSS-BlocksConsolidation in the associated measObject is not configured; or

2>	if the highest beam measurement quantity value is below or equal to absThreshSS-BlocksConsolidation:

3>	derive each cell measurement quantity based on SS/PBCH block as the highest beam measurement quantity value, where each beam measurement quantity is described in TS 38.215 [9];

2>	else:

3>	derive each cell measurement quantity based on SS/PBCH block as the linear power scale average of the highest beam measurement quantity values above absThreshSS-BlocksConsolidation where the total number of averaged beams shall not exceed nrofSS-BlocksToAverage;

2>	apply layer 3 cell filtering as described in 5.5.3.2;

1>	for each cell measurement quantity to be derived based on CSI-RS:

2>	consider a CSI-RS resource to be applicable for deriving cell measurements when the concerned CSI-RS resource is included in the csi-rs-CellMobility including the physCellId of the cell in theCSI-RS-ResourceConfigMobility in the associated measObject;

2>	if nrofCSI-RS-ResourcesToAverage in the associated measObject is not configured; or

2>	if absThreshCSI-RS-Consolidation in the associated measObject is not configured; or

2>	if the highest beam measurement quantity value is below or equal to absThreshCSI-RS-Consolidation:

3>	derive each cell measurement quantity based on applicable CSI-RS resources for the cell as the highest beam measurement quantity value, where each beam measurement quantity is described in TS 38.215 [9];

2>	else:

3>	derive each cell measurement quantity based on CSI-RS as the linear power scale average of the highest beam measurement quantity values above absThreshCSI-RS-Consolidation where the total number of averaged beams shall not exceed nrofCSI-RS-ResourcesToAverage;

2>	apply layer 3 cell filtering as described in 5.5.3.2.

#### 5.5.3.3a	Derivation of layer 3 beam filtered measurement

The UE shall:

1>	for each layer 3 beam filtered measurement quantity to be derived based on SS/PBCH block;

2>	derive each configured beam measurement quantity based on SS/PBCH block as described in TS 38.215[9], and apply layer 3 beam filtering as described in 5.5.3.2;

1>	for each layer 3 beam filtered measurement quantity to be derived based on CSI-RS;

2>	derive each configured beam measurement quantity based on CSI-RS as described in TS 38.215 [9], and apply layer 3 beam filtering as described in 5.5.3.2.

### 5.5.4	Measurement report triggering

#### 5.5.4.1	General

If AS security has been activated successfully, the UE shall:

1>	for each measId included in the measIdList within VarMeasConfig:

2>	if the corresponding reportConfig includes a reportType set to eventTriggered or periodical:

3>	if the corresponding measObject concerns NR:

4>	if the eventA1 or eventA2 is configured in the corresponding reportConfig:

5>	consider only the serving cell to be applicable;

4>	if the eventA3 or eventA5 is configured in the corresponding reportConfig:

5>	if a serving cell is associated with a measObjectNR and neighbours are associated with another measObjectNR, consider any serving cell associated with the other measObjectNR to be a neighbouring cell as well;

4>	if corresponding reportConfig includes reportType set to periodical; or

4>	for measurement events other than eventA1 or eventA2:

5>	if useWhiteCellList is set to true:

6>	consider any neighbouring cell detected based on parameters in the associated measObjectNR to be applicable when the concerned cell is included in the whiteCellsToAddModList defined within the VarMeasConfig for this measId;

5>	else:

6>	consider any neighbouring cell detected based on parameters in the associated measObjectNR to be applicable when the concerned cell is not included in the blackCellsToAddModList defined within the VarMeasConfig for this measId;

3>	else if the corresponding measObject concerns E-UTRA:

4>	if eventB1 or eventB2 is configured in the corresponding reportConfig:

5>	consider a serving cell, if any, on the associated E-UTRA frequency as neighbour cell;

4>	else:

5>	consider any neighbouring cell detected on the associated frequency to be applicable when the concerned cell is not included in the blackCellsToAddModListEUTRAN defined within the VarMeasConfig for this measId;

2>	else if the corresponding reportConfig includes a reportType set to reportCGI:

3>	consider the cell detected on the associated measObject which has a physical cell identity matching the value of the cellForWhichToReportCGI included in the corresponding reportConfig within the VarMeasConfig to be applicable;

2>	else if the corresponding reportConfig includes a reportType set to reportSFTD:

3>	if the corresponding measObject concerns NR:

4>	if the reportSFTD-Meas is set to true:

5>	consider the NR PSCell to be applicable;

4>	else if the reportSFTD-NeighMeas is included:

5>	if cellsForWhichToReportSFTD is configured in the corresponding reportConfig:

6>	consider any NR neighbouring cell detected on the associated measObjectNR which has a physical cell identity that is included in the cellsForWhichToReportSFTD to be applicable;

5>	else:

6>	consider up to 3 strongest NR neighbouring cells detected based on parameters in the associated measObjectNR to be applicable when the concerned cells are not included in the blackCellsToAddModList defined within the VarMeasConfig for this measId;

3>	else if the corresponding measObject concerns E-UTRA:

4>	if the reportSFTD-Meas is set to true:

5>	consider the E-UTRA PSCell to be applicable;

2>	if the reportType is set to eventTriggered and if the entry condition applicable for this event, i.e. the event corresponding with the eventId of the corresponding reportConfig within VarMeasConfig, is fulfilled for one or more applicable cells for all measurements after layer 3 filtering taken during timeToTrigger defined for this event within the VarMeasConfig, while the VarMeasReportList does not include a measurement reporting entry for this measId (a first cell triggers the event):

3>	include a measurement reporting entry within the VarMeasReportList for this measId;

3>	set the numberOfReportsSent defined within the VarMeasReportList for this measId to 0;

3>	include the concerned cell(s) in the cellsTriggeredList defined within the VarMeasReportList for this measId;

3>	initiate the measurement reporting procedure, as specified in 5.5.5;

2>	else if the reportType is set to eventTriggered and if the entry condition applicable for this event, i.e. the event corresponding with the eventId of the corresponding reportConfig within VarMeasConfig, is fulfilled for one or more applicable cells not included in the cellsTriggeredList for all measurements after layer 3 filtering taken during timeToTrigger defined for this event within the VarMeasConfig (a subsequent cell triggers the event):

3>	set the numberOfReportsSent defined within the VarMeasReportList for this measId to 0;

3>	include the concerned cell(s) in the cellsTriggeredList defined within the VarMeasReportList for this measId;

3>	initiate the measurement reporting procedure, as specified in 5.5.5;

2>	else if the reportType is set to eventTriggered and if the leaving condition applicable for this event is fulfilled for one or more of the cells included in the cellsTriggeredList defined within the VarMeasReportList for this measId for all measurements after layer 3 filtering taken during timeToTrigger defined within the VarMeasConfig for this event:

3>	remove the concerned cell(s) in the cellsTriggeredList defined within the VarMeasReportList for this measId;

3>	if reportOnLeave is set to true for the corresponding reporting configuration:

4>	initiate the measurement reporting procedure, as specified in 5.5.5;

3>	if the cellsTriggeredList defined within the VarMeasReportList for this measId is empty:

4>	remove the measurement reporting entry within the VarMeasReportList for this measId;

4>	stop the periodical reporting timer for this measId, if running;

2>	if reportType is set to periodical and if a (first) measurement result is available:

3>	include a measurement reporting entry within the VarMeasReportList for this measId;

3>	set the numberOfReportsSent defined within the VarMeasReportList for this measId to 0;

3>	if the reportAmount exceeds 1:

4>	initiate the measurement reporting procedure, as specified in 5.5.5, immediately after the quantity to be reported becomes available for the NR SpCell;

3>	else (i.e. the reportAmount is equal to 1):

4>	initiate the measurement reporting procedure, as specified in 5.5.5, immediately after the quantity to be reported becomes available for the NR SpCell and for the strongest cell among the applicable cells;

2>	upon expiry of the periodical reporting timer for this measId:

3>	initiate the measurement reporting procedure, as specified in 5.5.5.

2>	if the corresponding reportConfig includes a reportType is set to reportSFTD:

3>	if the corresponding measObject concerns NR:

4>	if the drx-SFTD-NeighMeas is included:

5>	if the quantity to be reported becomes available for each requested pair of PCell and NR cell:

6>	stop timer T322;

6>	initiate the measurement reporting procedure, as specified in 5.5.5;

4>	else

5>	initiate the measurement reporting procedure, as specified in 5.5.5, immediately after the quantity to be reported becomes available for each requested pair of PCell and NR cell or the maximal measurement reporting delay as specified in TS 38.133 [14];

3>	else if the corresponding measObject concerns E-UTRA:

4>	initiate the measurement reporting procedure, as specified in 5.5.5, immediately after the quantity to be reported becomes available for the pair of PCell and E-UTRA PSCell or the maximal measurement reporting delay as specified in TS 38.133 [14];

2>	if reportType is set to reportCGI:

3>	if the UE acquired the SIB1 or SystemInformationBlockType1 for the requested cell; or

3>	if the UE detects that the requested NR cell is not transmitting SIB1 (see TS 38.213 [13], clause 13):

4>	stop timer T321;

4>	include a measurement reporting entry within the VarMeasReportList for this measId;

4>	set the numberOfReportsSent defined within the VarMeasReportList for this measId to 0;

4>	initiate the measurement reporting procedure, as specified in 5.5.5;

2>	upon the expiry of T321 for this measId:

3>	include a measurement reporting entry within the VarMeasReportList for this measId;

3>	set the numberOfReportsSent defined within the VarMeasReportList for this measId to 0;

3>	initiate the measurement reporting procedure, as specified in 5.5.5.

2>	upon the expiry of T322 for this measId:

3>	initiate the measurement reporting procedure, as specified in 5.5.5;

#### 5.5.4.2	Event A1 (Serving becomes better than threshold)

The UE shall:

1>	consider the entering condition for this event to be satisfied when condition A1-1, as specified below, is fulfilled;

1>	consider the leaving condition for this event to be satisfied when condition A1-2, as specified below, is fulfilled;

1>	for this measurement, consider the NR serving cell corresponding to the associated measObjectNR associated with this event.

Inequality A1-1 (Entering condition)

Ms – Hys > Thresh

Inequality A1-2 (Leaving condition)

Ms + Hys < Thresh

The variables in the formula are defined as follows:

Ms is the measurement result of the serving cell, not taking into account any offsets.

Hys is the hysteresis parameter for this event (i.e. hysteresis as defined within reportConfigNR for this event).

Thresh is the threshold parameter for this event (i.e. a1-Threshold as defined within reportConfigNR for this event).

Ms is expressed in dBm in case of RSRP, or in dB in case of RSRQ and RS-SINR.

Hys is expressed in dB.

Thresh is expressed in the same unit as Ms.

#### 5.5.4.3	Event A2 (Serving becomes worse than threshold)

The UE shall:

1>	consider the entering condition for this event to be satisfied when condition A2-1, as specified below, is fulfilled;

1>	consider the leaving condition for this event to be satisfied when condition A2-2, as specified below, is fulfilled;

1>	for this measurement, consider the serving cell indicated by the measObjectNR associated to this event.

Inequality A2-1 (Entering condition)

Ms + Hys < Thresh

Inequality A2-2 (Leaving condition)

Ms – Hys > Thresh

The variables in the formula are defined as follows:

Ms is the measurement result of the serving cell, not taking into account any offsets.

Hys is the hysteresis parameter for this event (i.e. hysteresis as defined within reportConfigNR for this event).

Thresh is the threshold parameter for this event (i.e. a2-Threshold as defined within reportConfigNR for this event).

Ms is expressed in dBm in case of RSRP, or in dB in case of RSRQ and RS-SINR.

Hys is expressed in dB.

Thresh is expressed in the same unit as Ms.

#### 5.5.4.4	Event A3 (Neighbour becomes offset better than SpCell)

The UE shall:

1>	consider the entering condition for this event to be satisfied when condition A3-1, as specified below, is fulfilled;

1>	consider the leaving condition for this event to be satisfied when condition A3-2, as specified below, is fulfilled;

1>	use the SpCell for Mp, Ofp and Ocp.

NOTE	The cell(s) that triggers the event has reference signals indicated in the measObjectNR associated to this event which may be different from the NR SpCell measObjectNR.

Inequality A3-1 (Entering condition)

Mn + Ofn + Ocn – Hys > Mp + Ofp + Ocp + Off

Inequality A3-2 (Leaving condition)

Mn + Ofn + Ocn + Hys < Mp + Ofp + Ocp + Off

The variables in the formula are defined as follows:

Mn is the measurement result of the neighbouring cell, not taking into account any offsets.

Ofn is the measurement object specific offset of the reference signal of the neighbour cell (i.e. offsetMO as defined within measObjectNR corresponding to the neighbour cell).

Ocn is the cell specific offset of the neighbour cell (i.e. cellIndividualOffset as defined within measObjectNR corresponding to the frequency of the neighbour cell), and set to zero if not configured for the neighbour cell.

Mp is the measurement result of the SpCell, not taking into account any offsets.

Ofp is the measurement object specific offset of the SpCell (i.e. offsetMO as defined within measObjectNR corresponding to the SpCell).

Ocp is the cell specific offset of the SpCell (i.e. cellIndividualOffset as defined within measObjectNR corresponding to the SpCell), and is set to zero if not configured for the SpCell.

Hys is the hysteresis parameter for this event (i.e. hysteresis as defined within reportConfigNR for this event).

Off is the offset parameter for this event (i.e. a3-Offset as defined within reportConfigNR for this event).

Mn, Mp are expressed in dBm in case of RSRP, or in dB in case of RSRQ and RS-SINR.

Ofn, Ocn, Ofp, Ocp, Hys, Off are expressed in dB.

#### 5.5.4.5	Event A4 (Neighbour becomes better than threshold)

The UE shall:

1>	consider the entering condition for this event to be satisfied when condition A4-1, as specified below, is fulfilled;

1>	consider the leaving condition for this event to be satisfied when condition A4-2, as specified below, is fulfilled.

Inequality A4-1 (Entering condition)

Mn + Ofn + Ocn – Hys > Thresh

Inequality A4-2 (Leaving condition)

Mn + Ofn + Ocn + Hys < Thresh

The variables in the formula are defined as follows:

Mn is the measurement result of the neighbouring cell, not taking into account any offsets.

Ofn is the measurement object specific offset of the neighbour cell (i.e. offsetMO as defined within measObjectNR corresponding to the neighbour cell).

Ocn is the measurement object specific offset of the neighbour cell (i.e. cellIndividualOffset as defined within measObjectNR corresponding to the neighbour cell), and set to zero if not configured for the neighbour cell.

Hys is the hysteresis parameter for this event (i.e. hysteresis as defined within reportConfigNR for this event).

Thresh is the threshold parameter for this event (i.e. a4-Threshold as defined within reportConfigNR for this event).

Mn is expressed in dBm in case of RSRP, or in dB in case of RSRQ and RS-SINR.

Ofn, Ocn, Hys are expressed in dB.

Thresh is expressed in the same unit as Mn.

#### 5.5.4.6	Event A5 (SpCell becomes worse than threshold1 and neighbour becomes better than threshold2)

The UE shall:

1>	consider the entering condition for this event to be satisfied when both condition A5-1 and condition A5-2, as specified below, are fulfilled;

1>	consider the leaving condition for this event to be satisfied when condition A5-3 or condition A5-4, i.e. at least one of the two, as specified below, is fulfilled;

1>	use the SpCell for Mp.

NOTE:	The parameters of the reference signal(s) of the cell(s) that triggers the event are indicated in the measObjectNR associated to the event which may be different from the measObjectNR of the NR SpCell.

Inequality A5-1 (Entering condition 1)

Mp + Hys < Thresh1

Inequality A5-2 (Entering condition 2)

Mn + Ofn + Ocn – Hys > Thresh2

Inequality A5-3 (Leaving condition 1)

Mp – Hys > Thresh1

Inequality A5-4 (Leaving condition 2)

Mn + Ofn + Ocn + Hys < Thresh2

The variables in the formula are defined as follows:

Mp is the measurement result of the NR SpCell, not taking into account any offsets.

Mn is the measurement result of the neighbouring cell, not taking into account any offsets.

Ofn is the measurement object specific offset of the neighbour cell (i.e. offsetMO as defined within measObjectNR corresponding to the neighbour cell).

Ocn is the cell specific offset of the neighbour cell (i.e. cellIndividualOffset as defined within measObjectNR corresponding to the neighbour cell), and set to zero if not configured for the neighbour cell.

Hys is the hysteresis parameter for this event (i.e. hysteresis as defined within reportConfigNR for this event).

Thresh1 is the threshold parameter for this event (i.e. a5-Threshold1 as defined within reportConfigNR for this event).

Thresh2 is the threshold parameter for this event (i.e. a5-Threshold2 as defined within reportConfigNR for this event).

Mn, Mp are expressed in dBm in case of RSRP, or in dB in case of RSRQ and RS-SINR.

Ofn, Ocn, Hys are expressed in dB.

Thresh1is expressed in the same unit as Mp.

Thresh2 is expressed in the same unit as Mn.

#### 5.5.4.7	Event A6 (Neighbour becomes offset better than SCell)

The UE shall:

1>	consider the entering condition for this event to be satisfied when condition A6-1, as specified below, is fulfilled;

1>	consider the leaving condition for this event to be satisfied when condition A6-2, as specified below, is fulfilled;

1>	for this measurement, consider the (secondary) cell corresponding to the measObjectNR associated to this event to be the serving cell.

NOTE:	The reference signal(s) of the neighbour(s) and the reference signal(s) of the SCell are both indicated in the associated measObjectNR.

Inequality A6-1 (Entering condition)

Mn + Ocn – Hys > Ms + Ocs + Off

Inequality A6-2 (Leaving condition)

Mn + Ocn + Hys < Ms + Ocs + Off

The variables in the formula are defined as follows:

Mn is the measurement result of the neighbouring cell, not taking into account any offsets.

Ocn is the cell specific offset of the neighbour cell (i.e. cellIndividualOffset as defined within the associated measObjectNR), and set to zero if not configured for the neighbour cell.

Ms is the measurement result of the serving cell, not taking into account any offsets.

Ocs is the cell specific offset of the serving cell (i.e. cellIndividualOffset as defined within the associated measObjectNR), and is set to zero if not configured for the serving cell.

Hys is the hysteresis parameter for this event (i.e. hysteresis as defined within reportConfigNR for this event).

Off is the offset parameter for this event (i.e. a6-Offset as defined within reportConfigNR for this event).

Mn, Ms are expressed in dBm in case of RSRP, or in dB in case of RSRQ and RS-SINR.

Ocn, Ocs, Hys, Off are expressed in dB.

#### 5.5.4.8	Event B1 (Inter RAT neighbour becomes better than threshold)

The UE shall:

1>	consider the entering condition for this event to be satisfied when condition B1-1, as specified below, is fulfilled;

1>	consider the leaving condition for this event to be satisfied when condition B1-2, as specified below, is fulfilled.

Inequality B1-1 (Entering condition)

Mn + Ofn + Ocn – Hys > Thresh

Inequality B1-2 (Leaving condition)

Mn + Ofn + Ocn + Hys < Thresh

The variables in the formula are defined as follows:

Mn is the measurement result of the inter-RAT neighbour cell, not taking into account any offsets.

Ofn is the measurement object specific offset of the frequency of the inter-RAT neighbour cell (i.e. eutra-Q-OffsetRange as defined within the measObjectEUTRA corresponding to the frequency of the neighbour inter-RAT cell).

Ocn is the cell specific offset of the inter-RAT neighbour cell (i.e. cellIndividualOffset as defined within the measObjectEUTRA corresponding to the neighbour inter-RAT cell), and set to zero if not configured for the neighbour cell.

Hys is the hysteresis parameter for this event (i.e. hysteresis as defined within reportConfigInterRAT for this event).

Thresh is the threshold parameter for this event (i.e. b1-ThresholdEUTRA as defined within reportConfigInterRAT for this event).

Mn is expressed in dBm or in dB, depending on the measurement quantity of the inter-RAT neighbour cell.

Ofn, Ocn, Hys are expressed in dB.

Thresh is expressed in the same unit as Mn.

#### 5.5.4.9	Event B2 (PCell becomes worse than threshold1 and inter RAT neighbour becomes better than threshold2)

The UE shall:

1>	consider the entering condition for this event to be satisfied when both condition B2-1 and condition B2-2, as specified below, are fulfilled;

1>	consider the leaving condition for this event to be satisfied when condition B2-3 or condition B2-4, i.e. at least one of the two, as specified below, is fulfilled;

Inequality B2-1 (Entering condition 1)

Mp + Hys < Thresh1

Inequality B2-2 (Entering condition 2)

Mn + Ofn + Ocn – Hys > Thresh2

Inequality B2-3 (Leaving condition 1)

Mp – Hys > Thresh1

Inequality B2-4 (Leaving condition 2)

Mn + Ofn + Ocn + Hys < Thresh2

The variables in the formula are defined as follows:

Mp is the measurement result of the PCell, not taking into account any offsets.

Mn is the measurement result of the inter-RAT neighbour cell, not taking into account any offsets.

Ofn is the measurement object specific offset of the frequency of the inter-RAT neighbour cell (i.e. eutra-Q-OffsetRange as defined within the measObjectEUTRA corresponding to the frequency of the inter-RAT neighbour cell).

Ocn is the cell specific offset of the inter-RAT neighbour cell (i.e. cellIndividualOffset as defined within the measObjectEUTRA corresponding to the neighbour inter-RAT cell), and set to zero if not configured for the neighbour cell.

Hys is the hysteresis parameter for this event (i.e. hysteresis as defined within reportConfigInterRAT for this event).

Thresh1 is the threshold parameter for this event (i.e. b2-Threshold1 as defined within reportConfigInterRAT for this event).

Thresh2 is the threshold parameter for this event (i.e. b2-Threshold2EUTRA as defined within reportConfigInterRAT for this event).

Mp is expressed in dBm in case of RSRP, or in dB in case of RSRQ and SINR.

Mn is expressed in dBm or dB, depending on the measurement quantity of the inter-RAT neighbour cell.

Ofn, Ocn, Hys are expressed in dB.

Thresh1 is expressed in the same unit as Mp.

Thresh2 is expressed in the same unit as Mn.

### 5.5.5	Measurement reporting

#### 5.5.5.1	General

Figure 5.5.5.1-1: Measurement reporting

The purpose of this procedure is to transfer measurement results from the UE to the network. The UE shall initiate this procedure only after successful AS security activation.

For the measId for which the measurement reporting procedure was triggered, the UE shall set the measResults within the MeasurementReport message as follows:

1>	set the measId to the measurement identity that triggered the measurement reporting;

1>	for each serving cell configured with servingCellMO:

2>	if the reportConfig associated with the measId that triggered the measurement reporting includes rsType:

3>	if the serving cell measurements based on the rsType included in the reportConfig that triggered the measurement report are available:

4>	set the measResultServingCell within measResultServingMOList to include RSRP, RSRQ and the available SINR of the serving cell, derived based on the rsType included in the reportConfig that triggered the measurement report;

2>	else:

3>	if SSB based serving cell measurements are available:

4>	set the measResultServingCell within measResultServingMOList to include RSRP, RSRQ and the available SINR of the serving cell, derived based on SSB;

3>	else if CSI-RS based serving cell measurements are available:

4>	set the measResultServingCell within measResultServingMOList to include RSRP, RSRQ and the available SINR of the serving cell, derived based on CSI-RS;

1>	set the servCellId within measResultServingMOList to include each NR serving cell that is configured with servingCellMO, if any;

1>	if the reportConfig associated with the measId that triggered the measurement reporting includes reportQuantityRS-Indexes and maxNrofRS-IndexesToReport:

2>	for each serving cell configured with servingCellMO, include beam measurement information according to the associated reportConfig as described in 5.5.5.2;

1>	if the reportConfig associated with the measId that triggered the measurement reporting includes reportAddNeighMeas:

2>	for each measObjectId referenced in the measIdList which is also referenced with servingCellMO, other than the measObjectId corresponding with the measId that triggered the measurement reporting:

3>	if the measObjectNR indicated by the servingCellMO includes the RS resource configuration corresponding to the rsType indicated in the reportConfig:

4>	set the measResultBestNeighCell within measResultServingMOList to include the physCellId and the available measurement quantities based on the reportQuantityCell and rsType indicated in reportConfig of the non-serving cell corresponding to the concerned measObjectNR with the highest measured RSRP if RSRP measurement results are available for cells corresponding to this measObjectNR, otherwise with the highest measured RSRQ if RSRQ measurement results are available for cells corresponding to this measObjectNR, otherwise with the highest measured SINR;

4>	if the reportConfig associated with the measId that triggered the measurement reporting includes reportQuantityRS-Indexes and maxNrofRS-IndexesToReport:

5>	for each best non-serving cell included in the measurement report:

6>	include beam measurement information according to the associated reportConfig as described in 5.5.5.2;

1>	if the reportConfig associated with the measId that triggered the measurement reporting is set to eventTriggered and eventID is set to eventA3, or eventA4, or eventA5, or eventB1, or eventB2:

2>	if the UE is in NE-DC and the measurement configuration that triggered this measurement report is associated with the MCG:

3>	set the measResultServFreqListEUTRA-SCG to include an entry for each E-UTRA SCG serving frequency with the following:

4>	include carrierFreq of the E-UTRA serving frequency;

4>	set the measResultServingCell to include the available measurement quantities that the UE is configured to measure by the measurement configuration associated with the SCG;

4>	if reportConfig associated with the measId that triggered the measurement reporting includes reportAddNeighMeas:

5>	set the measResultServFreqListEUTRA-SCG to include within measResultBestNeighCell the quantities of the best non-serving cell, based on RSRP, on the concerned serving frequency;

1>	if reportConfig associated with the measId that triggered the measurement reporting is set to eventTriggered and eventID is set to eventA3, or eventA4, or eventA5:

2>	if the UE is in NR-DC and the measurement configuration that triggered this measurement report is associated with the MCG:

3>	set the measResultServFreqListNR-SCG to include for each NR SCG serving cell that is configured with servingCellMO, if any, the following:

4>	if the reportConfig associated with the measId that triggered the measurement reporting includes rsType:

5>	if the serving cell measurements based on the rsType included in the reportConfig that triggered the measurement report are available according to the measurement configuration associated with the SCG:

6>	set the measResultServingCell within measResultServFreqListNR-SCG to include RSRP, RSRQ and the available SINR of the serving cell, derived based on the rsType included in the reportConfig that triggered the measurement report;

4>	else:

5>	if SSB based serving cell measurements are available according to the measurement configuration associated with the SCG:

6>	set the measResultServingCell within measResultServFreqListNR-SCG to include RSRP, RSRQ and the available SINR of the serving cell, derived based on SSB;

5>	else if CSI-RS based serving cell measurements are available according to the measurement configuration associated with the SCG:

6>	set the measResultServingCell within measResultServFreqListNR-SCG to include RSRP, RSRQ and the available SINR of the serving cell, derived based on CSI-RS;

4>	if results for the serving cell derived based on SSB are included:

5>	include the ssbFrequency to the value indicated by ssbFrequency as included in the MeasObjectNR of the serving cell;

4>	if results for the serving cell derived based on CSI-RS are included:

5>	include the refFreqCSI-RS to the value indicated by refFreqCSI-RS as included in the MeasObjectNR of the serving cell;

4>	if the reportConfig associated with the measId that triggered the measurement reporting includes reportQuantityRS-Indexes and maxNrofRS-IndexesToReport:

5>	for each serving cell configured with servingCellMO, include beam measurement information according to the associated reportConfig as described in 5.5.5.2, where availability is considered according to the measurement configuration associated with the SCG;

4>	if reportConfig associated with the measId that triggered the measurement reporting includes reportAddNeighMeas:

5>	if the measObjectNR indicated by the servingCellMO includes the RS resource configuration corresponding to the rsType indicated in the reportConfig:

6>	set the measResultBestNeighCellListNR within measResultServFreqListNR-SCG to include one entry with the physCellId and the available measurement quantities based on the reportQuantityCell and rsType indicated in reportConfig of the non-serving cell corresponding to the concerned measObjectNR with the highest measured RSRP if RSRP measurement results are available for cells corresponding to this measObjectNR, otherwise with the highest measured RSRQ if RSRQ measurement results are available for cells corresponding to this measObjectNR, otherwise with the highest measured SINR, where availability is considered according to the measurement configuration associated with the SCG;

7>	if the reportConfig associated with the measId that triggered the measurement reporting includes reportQuantityRS-Indexes and maxNrofRS-IndexesToReport:

8>	for each best non-serving cell included in the measurement report:

9>	include beam measurement information according to the associated reportConfig as described in 5.5.5.2, where availability is considered according to the measurement configuration associated with the SCG;

1>	if there is at least one applicable neighbouring cell to report:

2>	if the reportType is set to eventTriggered or periodical:

3>	set the measResultNeighCells to include the best neighbouring cells up to maxReportCells in accordance with the following:

4>	if the reportType is set to eventTriggered:

5>	include the cells included in the cellsTriggeredList as defined within the VarMeasReportList for this measId;

4>	else:

5>	include the applicable cells for which the new measurement results became available since the last periodical reporting or since the measurement was initiated or reset;

4>	for each cell that is included in the measResultNeighCells, include the physCellId;

4>	if the reportType is set to eventTriggered or periodical:

5>	for each included cell, include the layer 3 filtered measured results in accordance with the reportConfig for this measId, ordered as follows:

6>	if the measObject associated with this measId concerns NR:

7>	if rsType in the associated reportConfig is set to ssb:

8>	set resultsSSB-Cell within the measResult to include the SS/PBCH block based quantity(ies) indicated in the reportQuantityCell within the concerned reportConfig, in decreasing order of the sorting quantity, determined as specified in 5.5.5.3, i.e. the best cell is included first;

8>	if reportQuantityRS-Indexes and maxNrofRS-IndexesToReport are configured, include beam measurement information as described in 5.5.5.2;

7>	else if rsType in the associated reportConfig is set to csi-rs:

8>	set resultsCSI-RS-Cell within the measResult to include the CSI-RS based quantity(ies) indicated in the reportQuantityCell within the concerned reportConfig, in decreasing order of the sorting quantity, determined as specified in 5.5.5.3, i.e. the best cell is included first;

8>	if reportQuantityRS-Indexes and maxNrofRS-IndexesToReport are configured, include beam measurement information as described in 5.5.5.2;

6>	if the measObject associated with this measId concerns E-UTRA:

7>	set the measResult to include the quantity(ies) indicated in the reportQuantity within the concerned reportConfigInterRAT in decreasing order of the sorting quantity, determined as specified in 5.5.5.3, i.e. the best cell is included first;

2>	else:

3>	if the cell indicated by cellForWhichToReportCGI is an NR cell:

4>	if plmn-IdentityInfoList of the cgi-Info for the concerned cell has been obtained:

5>	include the plmn-IdentityInfoList including plmn-IdentityList, trackingAreaCode (if available), ranac (if available), cellIdentity and cellReservedForOperatorUse for each entry of the plmn-IdentityInfoList;

5>	include frequencyBandList if available;

4>	else if MIB indicates the SIB1 is not broadcast:

5>	include the noSIB1 including the ssb-SubcarrierOffset and pdcch-ConfigSIB1 obtained from MIB of the concerned cell;

3>	if the cell indicated by cellForWhichToReportCGI is an E-UTRA cell:

4>	if all mandatory fields of the cgi-Info-EPC for the concerned cell have been obtained:

5>	include in the cgi-Info-EPC the fields broadcasted in E-UTRA SystemInformationBlockType1 associated to EPC;

4>	if the UE is E-UTRA/5GC capable and all mandatory fields of the cgi-Info-5GC for the concerned cell have been obtained:

5>	include in the cgi-Info-5GC the fields broadcasted in E-UTRA SystemInformationBlockType1 associated to 5GC;

4>	if the mandatory present fields of the cgi-Info for the cell indicated by the cellForWhichToReportCGI in the associated measObject have been obtained:

5>	include the freqBandIndicator;

5>	if the cell broadcasts the multiBandInfoList, include the multiBandInfoList;

5>	if the cell broadcasts the freqBandIndicatorPriority, include the freqBandIndicatorPriority;

1>	if the corresponding measObject concerns NR:

2>	if the reportSFTD-Meas is set to true within the corresponding reportConfigNR for this measId:

3>	set the measResultSFTD-NR in accordance with the following:

4>	set sfn-OffsetResult and frameBoundaryOffsetResult to the measurement results provided by lower layers;

4>	if the reportRSRP is set to true;

5>	set rsrp-Result to the RSRP of the NR PSCell derived based on SSB;

2>	else if the reportSFTD-NeighMeas is included within the corresponding reportConfigNR for this measId:

3>	for each applicable cell which measurement results are available, include an entry in the measResultCellListSFTD-NR and set the contents as follows:

4>	set physCellId to the physical cell identity of the concered NR neighbour cell.

4>	set sfn-OffsetResult and frameBoundaryOffsetResult to the measurement results provided by lower layers;

4>	if the reportRSRP is set to true:

5>	set rsrp-Result to the RSRP of the concerned cell derived based on SSB;

1>	else if the corresponding measObject concerns E-UTRA:

2>	if the reportSFTD-Meas is set to true within the corresponding reportConfigInterRAT for this measId:

3>	set the measResultSFTD-EUTRA in accordance with the following:

4>	set sfn-OffsetResult and frameBoundaryOffsetResult to the measurement results provided by lower layers;

4>	if the reportRSRP is set to true;

5>	set rsrpResult-EUTRA to the RSRP of the EUTRA PSCell;

1>	increment the numberOfReportsSent as defined within the VarMeasReportList for this measId by 1;

1>	stop the periodical reporting timer, if running;

1>	if the numberOfReportsSent as defined within the VarMeasReportList for this measId is less than the reportAmount as defined within the corresponding reportConfig for this measId:

2>	start the periodical reporting timer with the value of reportInterval as defined within the corresponding reportConfig for this measId;

1>	else:

2>	if the reportType is set to periodical:

3>	remove the entry within the VarMeasReportList for this measId;

3>	remove this measId from the measIdList within VarMeasConfig;

1>	if the UE is in (NG)EN-DC:

2>	if SRB3 is configured:

3>	submit the MeasurementReport message via SRB3 to lower layers for transmission, upon which the procedure ends;

2>	else:

3>	submit the MeasurementReport message via E-UTRA embedded in E-UTRA RRC message ULInformationTransferMRDC as specified in TS 36.331 [10].

1>	else if the UE is in NR-DC:

2>	if the measurement configuration that triggered this measurement report is associated with the SCG:

3>	if SRB3 is configured:

4>	submit the MeasurementReport message via SRB3 to lower layers for transmission, upon which the procedure ends;

3>	else:

4>	submit the MeasurementReport message via SRB1 embedded in NR RRC message ULInformationTransferMRDC as specified in 5.7.2a.3;

2>	else:

3>	submit the MeasurementReport message via SRB1 to lower layers for transmission, upon which the procedure ends;

1>	else:

2>	submit the MeasurementReport message to lower layers for transmission, upon which the procedure ends.

#### 5.5.5.2	Reporting of beam measurement information

For beam measurement information to be included in a measurement report the UE shall:

1>	if reportType is set to eventTriggered:

2>	consider the trigger quantity as the sorting quantity if available, otherwise RSRP as sorting quantity if available, otherwise RSRQ as sorting quantity if available, otherwise SINR as sorting quantity;

1>	if reportType is set to periodical:

2>	if a single reporting quantity is set to true in reportQuantityRS-Indexes;

3>	consider the configured single quantity as the sorting quantity;

2>	else:

3>	if rsrp is set to true;

4>	consider RSRP as the sorting quantity;

3>	else:

4>	consider RSRQ as the sorting quantity;

1>	set rsIndexResults to include up to maxNrofRS-IndexesToReport SS/PBCH block indexes or CSI-RS indexes in order of decreasing sorting quantity as follows:

2>	if the measurement information to be included is based on SS/PBCH block:

3>	include within resultsSSB-Indexes the index associated to the best beam for that SS/PBCH block sorting quantity and if absThreshSS-BlocksConsolidation is included in the VarMeasConfig for the measObject associated to the cell for which beams are to be reported, the remaining beams whose sorting quantity is above absThreshSS-BlocksConsolidation;

3>	if includeBeamMeasurements is configured, include the SS/PBCH based measurement results for the quantities in reportQuantityRS-Indexes set to true for each SS/PBCH block index;

2>	else if the beam measurement information to be included is based on CSI-RS:

3>	include within resultsCSI-RS-Indexes the index associated to the best beam for that CSI-RS sorting quantity and, if absThreshCSI-RS-Consolidation is included in the VarMeasConfig for the measObject associated to the cell for which beams are to be reported, the remaining beams whose sorting quantity is above absThreshCSI-RS-Consolidation;

3>	if includeBeamMeasurements is configured, include the CSI-RS based measurement results for the quantities in reportQuantityRS-Indexes set to true for each CSI-RS index.

#### 5.5.5.3	Sorting of cell measurement results

The UE shall determine the sorting quantity according to parameters of the reportConfig associated with the measId that triggered the reporting:

1>	if the reportType is set to eventTriggered:

2>	for an NR cell, consider the quantity used in the aN-Threshold (for eventA1, eventA2 and eventA4) or in the a5-Threshold2 (for eventA5) or in the aN-Offset (for eventA3 and eventA6) as the sorting quantity;

2>	for an E-UTRA cell, consider the quantity used in the bN-ThresholdEUTRA as the sorting quantity;

1>	if the reportType is set to periodical:

2>	determine the sorting quantity according to reportQuantityCell for an NR cell, and according to reportQuantity for an E-UTRA cell, as below:

3>	if a single quantity is set to true:

4>	consider this quantity as the sorting quantity;

3>	else:

4>	if rsrp is set to true;

5>	consider RSRP as the sorting quantity;

4>	else:

5>	consider RSRQ as the sorting quantity.

### 5.5.6	Location measurement indication

#### 5.5.6.1	General

Figure 5.5.5.1-1: Location measurement indication

The purpose of this procedure is to indicate to the network that the UE is going to start/stop location related measurements (eutra-RSTD) which require measurement gaps or start/stop subframe and slot timing detection towards E-UTRA (eutra-FineTimingDetection) which requires measurement gaps. UE shall initiate this procedure only after successful AS security activation.

NOTE:	It is a network decision to configure the measurement gap.

#### 5.5.6.2	Initiation

The UE shall:

1>	if and only if upper layers indicate to start performing location measurements or start subframe and slot timing detection towards E-UTRA, and the UE requires measurement gaps for these operations while measurement gaps are either not configured or not sufficient:

2>	initiate the procedure to indicate start;

NOTE 1:	The UE verifies the measurement gap situation only upon receiving the indication from upper layers. If at this point in time sufficient gaps are available, the UE does not initiate the procedure. Unless it receives a new indication from upper layers, the UE is only allowed to further repeat the procedure in the same PCell once per frequency of the target RAT if the provided measurement gaps are insufficient.

1>	if and only if upper layers indicate to stop performing location measurements or stop subframe and slot timing detection towards E-UTRA:

2>	initiate the procedure to indicate stop.

NOTE 2:	The UE may initiate the procedure to indicate stop even if it did not previously initiate the procedure to indicate start.

#### 5.5.6.3	Actions related to transmission of LocationMeasurementIndication message

The UE shall set the contents of LocationMeasurementIndication message as follows:

1>	if the procedure is initiated to indicate start of location related measurements:

2>	if the procedure is initiated for RSTD measurements towards E-UTRA:

3>	set the measurementIndication to the eutra-RSTD according to the information received from upper layers;

1>	else if the procedure is initiated to indicate stop of location related measurements:

2>	set the measurementIndication to the value release;

1>	if the procedure is initiated to indicate start of subframe and slot timing detection towards E-UTRA:

2>	set the measurementIndication to the value eutra-FineTimingDetection;

1>	else if the procedure is initiated to indicate stop of subframe and slot timing detection towards E-UTRA:

2>	set the measurementIndication to the value release;

1>	submit the LocationMeasurementIndication message to lower layers for transmission, upon which the procedure ends.

## 5.6	UE capabilities

### 5.6.1	UE capability transfer

#### 5.6.1.1	General

This clause describes how the UE compiles and transfers its UE capability information upon receiving a UECapabilityEnquiry from the network.

Figure 5.6.1.1-1: UE capability transfer

#### 5.6.1.2	Initiation

The network initiates the procedure to a UE in RRC_CONNECTED when it needs (additional) UE radio access capability information. The network should retrieve UE capabilities only after AS security activation. Network does not forward UE capabilities that were retrieved before AS security activation to the CN.

#### 5.6.1.3	Reception of the UECapabilityEnquiry by the UE

The UE shall set the contents of UECapabilityInformation message as follows:

1>	if the ue-CapabilityRAT-RequestList contains a UE-CapabilityRAT-Request with rat-Type set to nr:

2>	include in the ue-CapabilityRAT-ContainerList a UE-CapabilityRAT-Container of the type UE-NR-Capability and with the rat-Type set to nr;

2>	include the supportedBandCombinationList, featureSets and featureSetCombinations as specified in clause 5.6.1.4;

1>	if the ue-CapabilityRAT-RequestList contains a UE-CapabilityRAT-Request with rat-Type set to eutra-nr:

2> if the UE supports (NG)EN-DC or NE-DC:

3>	include in the ue-CapabilityRAT-ContainerList a UE-CapabilityRAT-Container of the type UE-MRDC-Capability and with the rat-Type set to eutra-nr;

3>	include the supportedBandCombinationList and featureSetCombinations as specified in clause 5.6.1.4;

1>	if the ue-CapabilityRAT-RequestList contains a UE-CapabilityRAT-Request with rat-Type set to eutra:

2>	if the UE supports E-UTRA:

3>	include in the ue-CapabilityRAT-ContainerList a ue-CapabilityRAT-Container of the type UE-EUTRA-Capability and with the rat-Type set to eutra as specified in TS 36.331 [10], clause 5.6.3.3, according to the capabilityRequestFilter, if received;

1>	submit the UECapabilityInformation message to lower layers for transmission, upon which the procedure ends.

#### 5.6.1.4	Setting band combinations, feature set combinations and feature sets supported by the UE

The UE invokes the procedures in this clause if the NR or E-UTRA network requests UE capabilities for nr, eutra-nr or eutra. This procedure is invoked once per requested rat-Type (see clause 5.6.1.3 for capability enquiry by the NR network; see TS 36.331 [10], clause 5.6.3.3 for capability enquiry by the E-UTRA network). The UE shall ensure that the feature set IDs are consistent across feature sets, feature set combinations and band combinations in all three UE capability containers that the network queries with the same fields with the same values, i.e. UE-CapabilityRequestFilterNR, UE-CapabilityRequestFilterCommon and fields in UECapabilityEnquiry message (i.e. requestedFreqBandsNR-MRDC, requestedCapabilityNR, eutra-nr-only flag, and requestedCapabilityCommon) as defined in TS 36.331, where applicable.

NOTE 1:	Capability enquiry without frequencyBandListFilter is not supported.

NOTE 2:	In EN-DC, the gNB needs the capabilities for RAT types nr and eutra-nr and it uses the featureSets in the UE-NR-Capability together with the featureSetCombinations in the UE-MRDC-Capability to determine the NR UE capabilities for the supported MRDC band combinations. Similarly, the eNB needs the capabilities for RAT types eutra and eutra-nr and it uses the featureSetsEUTRA in the UE-EUTRA-Capability together with the featureSetCombinations in the UE-MRDC-Capability to determine the E-UTRA UE capabilities for the supported MRDC band combinations. Hence, the IDs used in the featureSets must match the IDs referred to in featureSetCombinations across all three containers. The requirement on consistency implies that there are no undefined feature sets and feature set combinations.

NOTE 3:	If the UE cannot include all feature sets and feature set combinations due to message size or list size constraints, it is up to UE implementation which feature sets and feature set combinations it prioritizes.

The UE shall:

1>	compile a list of "candidate band combinations" according to the filter criteria in capabilityRequestFilterCommon (if included), only consisting of bands included in frequencyBandListFilter, and prioritized in the order of frequencyBandListFilter (i.e. first include band combinations containing the first-listed band, then include remaining band combinations containing the second-listed band, and so on), where for each band in the band combination, the parameters of the band do not exceed maxBandwidthRequestedDL, maxBandwidthRequestedUL, maxCarriersRequestedDL, maxCarriersRequestedUL, ca-BandwidthClassDL-EUTRA or ca-BandwidthClassUL-EUTRA, whichever are received;

1>	for each band combination included in the list of "candidate band combinations":

2>	if the network (E-UTRA) included the eutra-nr-only field, or

2>	if the requested rat-Type is eutra:

3>	remove the NR-only band combination from the list of "candidate band combinations";

NOTE 4:	The (E-UTRA) network may request capabilities for nr but indicate with the eutra-nr-only flag that the UE shall not include any NR band combinations in the UE-NR-Capability. In this case the procedural text above removes all NR-only band combinations from the candidate list and thereby also avoids inclusion of corresponding feature set combinations and feature sets below.

2>	if it is regarded as a fallback band combination with the same capabilities of another band combination included in the list of "candidate band combinations", and

2>	if this fallback band combination is generated by releasing at least one SCell or uplink configuration of SCell according to TS 38.306 [26]:

3>	remove the band combination from the list of "candidate band combinations";

NOTE 5:	Even if the network requests (only) capabilities for nr, it may include E-UTRA band numbers in the frequencyBandListFilter to ensure that the UE includes all necessary feature sets needed for subsequently requested eutra-nr capabilities. At this point of the procedure the list of "candidate band combinations" contains all NR- and/or E-UTRA-NR band combinations that match the filter (frequencyBandListFilter) provided by the NW and that match the eutra-nr-only flag (if RAT-Type nr is requested by E-UTRA). In the following, this candidate list is used to derive the band combinations, feature set combinations and feature sets to be reported in the requested capability container.

1>	if the requested rat-Type is nr:

2>	include into supportedBandCombinationList as many NR-only band combinations as possible from the list of "candidate band combinations", starting from the first entry;

3>	if srs-SwitchingTimeRequest is received:

4>	if SRS carrier switching is supported;

5>	include srs-SwitchingTimesListNR for each band combination;

4>	set srs-SwitchingTimeRequested to true;

2>	include, into featureSetCombinations, the feature set combinations referenced from the supported band combinations as included in supportedBandCombinationList according to the previous;

2>	compile a list of "candidate feature set combinations" referenced from the list of "candidate band combinations" excluding entries (rows in feature set combinations) for fallback band combinations with same or lower capabilities;

NOTE 6:	This list of "candidate feature set combinations" contains the feature set combinations used for NR-only as well as E-UTRA-NR band combinations. It is used to derive a list of NR feature sets referred to from the feature set combinations in the UE-NR-Capability and from the feature set combinations in a UE-MRDC-Capability container.

2>	include into featureSets the feature sets referenced from the "candidate feature set combinations" excluding entries (feature sets per CC) for fallback band combinations with same or lower capabilities and may exclude the feature sets with the parameters that exceed any of maxBandwidthRequestedDL, maxBandwidthRequestedUL, maxCarriersRequestedDL or maxCarriersRequestedUL, whichever are received;

1>	else, if the requested rat-Type is eutra-nr:

2>	include into supportedBandCombinationList and/or supportedBandCombinationListNEDC-Only as many E-UTRA-NR band combinations as possible from the list of "candidate band combinations", starting from the first entry;

3>	if srs-SwitchingTimeRequest is received:

4>	if SRS carrier switching is supported;

5>	include srs-SwitchingTimesListNR and srs-SwitchingTimesListEUTRA for each band combination;

4>	set srs-SwitchingTimeRequested to true;

2>	include, into featureSetCombinations, the feature set combinations referenced from the supported band combinations as included in supportedBandCombinationList according to the previous;

1>	else (if the requested rat-Type is eutra):

2>	compile a list of "candidate feature set combinations" referenced from the list of "candidate band combinations" excluding entries (rows in feature set combinations) for fallback band combinations with same or lower capabilities;

NOTE 7:	This list of "candidate feature set combinations" contains the feature set combinations used for E-UTRA-NR band combinations. It is used to derive a list of E-UTRA feature sets referred to from the feature set combinations in a UE-MRDC-Capability container.

2>	include into featureSetsEUTRA (in the UE-EUTRA-Capability) the feature sets referenced from the "candidate feature set combinations" excluding entries (feature sets per CC) for fallback band combinations with same or lower capabilities and may exclude the feature sets with the parameters that exceed ca-BandwidthClassDL-EUTRA or ca-BandwidthClassUL-EUTRA, whichever are received;

1>	include the received frequencyBandListFilter in the field appliedFreqBandListFilter of the requested UE capability, except if the requested rat-Type is nr and the network included the eutra-nr-only field;

1>	if the network included ue-CapabilityEnquiryExt:

2>	include the received ue-CapabilityEnquiryExt in the field receivedFilters;

#### 5.6.1.5	Void

## 5.7	Other

### 5.7.1	DL information transfer

#### 5.7.1.1	General

Figure 5.7.1.1-1: DL information transfer

The purpose of this procedure is to transfer NAS dedicated information from NG-RAN to a UE in RRC_CONNECTED.

#### 5.7.1.2	Initiation

The network initiates the DL information transfer procedure whenever there is a need to transfer NAS dedicated information. The network initiates the DL information transfer procedure by sending the DLInformationTransfer message.

#### 5.7.1.3	Reception of the DLInformationTransfer by the UE

Upon receiving DLInformationTransfer message, the UE shall:

1>	if dedicatedNAS-Message is included:

2>	forward dedicatedNAS-Message to upper layers.

### 5.7.2	UL information transfer

#### 5.7.2.1	General

Figure 5.7.2.1-1: UL information transfer

The purpose of this procedure is to transfer NAS dedicated information from the UE to the network.

#### 5.7.2.2	Initiation

A UE in RRC_CONNECTED initiates the UL information transfer procedure whenever there is a need to transfer NAS dedicated information. The UE initiates the UL information transfer procedure by sending the ULInformationTransfer message.

#### 5.7.2.3	Actions related to transmission of ULInformationTransfer message

The UE shall set the contents of the ULInformationTransfer message as follows:

1>	if the upper layer provides NAS PDU:

2>	set the dedicatedNAS-Message to include the information received from upper layers

1>	submit the ULInformationTransfer message to lower layers for transmission, upon which the procedure ends.

#### 5.7.2.4	Failure to deliver ULInformationTransfer message

The UE shall:

1>	if AS security is not started and radio link failure occurs before the successful delivery of ULInformationTransfer messages has been confirmed by lower layers; or

1>	if PDCP re-establishment or release/addition (e.g due to key refresh upon PCell or PSCell change, or RRC connection re-establishment) occurs on an SRB on which ULInformationTransfer messages were submitted for transmission but successful delivery of these messages was not confirmed by lower layers:

2>	inform upper layers about the possible failure to deliver the information contained in the concerned ULInformationTransfer messages.

### 5.7.2a	UL information transfer for MR-DC

#### 5.7.2a.1	General

Figure 5.7.2a.1-1: UL information transfer MR-DC

The purpose of this procedure is to transfer MR-DC dedicated information from the UE to the network e.g. the NR or E-UTRA RRC MeasurementReport and FailureInformation message.

#### 5.7.2a.2	Initiation

A UE in RRC_CONNECTED initiates the UL information transfer for MR-DC procedure whenever there is a need to transfer MR-DC dedicated information. I.e. the procedure is not used during an RRC connection reconfiguration involving NR or E-UTRA connection reconfiguration, in which case the MR DC information is piggybacked to the RRCReconfigurationComplete message.

#### 5.7.2a.3	Actions related to transmission of ULInformationTransferMRDC message

The UE shall set the contents of the ULInformationTransferMRDC message as follows:

1>	if there is a need to transfer MR-DC dedicated information related to NR:

2>	set the ul-DCCH-MessageNR to include the NR MR-DC dedicated information to be transferred (e.g., NR RRC MeasurementReport and FailureInformation message);

1>	else if there is a need to tranfer MR-DC dedicated information related to E-UTRA:

2>	set the ul-DCCH-MessageEUTRA to include the E-UTRA MR-DC dedicated information to be transferred (e.g., E-UTRA RRC MeasurementReport);

1>	submit the ULInformationTransferMRDC message to lower layers for transmission, upon which the procedure ends;

### 5.7.3	SCG failure information

#### 5.7.3.1	General

Figure 5.7.3.1-1: SCG failure information

The purpose of this procedure is to inform E-UTRAN or NR MN about an SCG failure the UE has experienced i.e. SCG radio link failure, failure of SCG reconfiguration with sync, SCG configuration failure for RRC message on SRB3 and SCG integrity check failure.

#### 5.7.3.2	Initiation

A UE initiates the procedure to report SCG failures when SCG transmission is not suspended and when one of the following conditions is met:

1>	upon detecting radio link failure for the SCG, in accordance with subclause 5.3.10.3;

1>	upon reconfiguration with sync failure of the SCG, in accordance with subclause 5.3.5.8.3;

1>	upon SCG configuration failure, in accordance with subclause 5.3.5.8.2;

1>	upon integrity check failure indication from SCG lower layers concerning SRB3.

Upon initiating the procedure, the UE shall:

1>	suspend SCG transmission for all SRBs and DRBs;

1>	reset SCG MAC;

1>	stop T304 for the SCG, if running;

1>	if the UE is in (NG)EN-DC:

2>	initiate transmission of the SCGFailureInformationNR message as specified in TS 36.331 [10], clause 5.6.13a.

1>	else:

2>	initiate transmission of the SCGFailureInformation message in accordance with 5.7.3.5.

#### 5.7.3.3	Failure type determination for (NG)EN-DC

The UE shall set the SCG failure type as follows:

1>	if the UE initiates transmission of the SCGFailureInformationNR message due to T310 expiry:

2>	set the failureType as t310-Expiry;

1>	else if the UE initiates transmission of the SCGFailureInformationNR message to provide reconfiguration with sync failure information for an SCG:

2>	set the failureType as synchReconfigFailure-SCG;

1>	else if the UE initiates transmission of the SCGFailureInformationNR message to provide random access problem indication from SCG MAC:

2>	set the failureType as randomAccessProblem;

1>	else if the UE initiates transmission of the SCGFailureInformationNR message to provide indication from SCG RLC that the maximum number of retransmissions has been reached:

2>	set the failureType as rlc-MaxNumRetx;

1>	else if the UE initiates transmission of the SCGFailureInformationNR message due to SRB3 integrity check failure:

2>	set the failureType as srb3-IntegrityFailure;

1>	else if the UE initiates transmission of the SCGFailureInformationNR message due to Reconfiguration failure of NR RRC reconfiguration message:

2>	set the failureType as scg-reconfigFailure.

#### 5.7.3.4	Setting the contents of MeasResultSCG-Failure

The UE shall set the contents of the MeasResultSCG-Failureas follows:

1>	for each MeasObjectNR configured on NR SCG for which a measId is configured and measurement results are available:

2>	include an entry in measResultPerMOList;

2>	if there is a measId configured with the MeasObjectNR and a reportConfig which has rsType set to ssb:

3>	set ssbFrequency to the value indicated by ssbFrequency as included in the MeasObjectNR;

2>	if there is a measId configured with the MeasObjectNR and a reportConfig which has rsType set to csi-rs:

3>	set refFreqCSI-RS to the value indicated by refFreqCSI-RS as included in the associated measurement object;

2>	if a serving cell is associated with the MeasObjectNR:

3>	set measResultServingCell to include the available quantities of the concerned cell and in accordance with the performance requirements in TS 38.133 [14];

2>	set the measResultNeighCellList to include the best measured cells, ordered such that the best cell is listed first, and based on measurements collected up to the moment the UE detected the failure, and set its fields as follows;

3>	ordering the cells with sorting as follows:

4>	based on SS/PBCH block if SS/PBCH block measurement results are available and otherwise based on CSI-RS;

4>	using RSRP if RSRP measurement results are available, otherwise using RSRQ if RSRQ measurement results are available, otherwise using SINR;

3>	for each neighbour cell included:

4>	include the optional fields that are available.

NOTE:	The measured quantities are filtered by the L3 filter as configured in the mobility measurement configuration. The measurements are based on the time domain measurement resource restriction, if configured. Blacklisted cells are not required to be reported.

#### 5.7.3.5	Actions related to transmission of SCGFailureInformation message

The UE shall set the contents of the SCGFailureInformation message as follows:

1>	if the UE initiates transmission of the SCGFailureInformation message due to T310 expiry:

2>	set the failureType as t310-Expiry;

1>	else if the UE initiates transmission of the SCGFailureInformation message to provide reconfiguration with sync failure information for an SCG:

2>	set the failureType as synchReconfigFailure-SCG;

1>	else if the UE initiates transmission of the SCGFailureInformation message to provide random access problem indication from SCG MAC:

2>	set the failureType as randomAccessProblem;

1>	else if the UE initiates transmission of the SCGFailureInformation message to provide indication from SCG RLC that the maximum number of retransmissions has been reached:

2>	set the failureType as rlc-MaxNumRetx;

1>	else if the UE initiates transmission of the SCGFailureInformation message due to SRB3 IP check failure:

2>	set the failureType as srb3-IntegrityFailure;

1>	else if the UE initiates transmission of the SCGFailureInformation message due to Reconfiguration failure of NR RRC reconfiguration message:

2>	set the failureType as scg-reconfigFailure.

1> include and set MeasResultSCG-Failure in accordance with 5.7.3.4;

1>	for each MeasObjectNR configured by a MeasConfig associated with the MCG, and for which measurement results are available:

2>	include an entry in measResultFreqList;

2>	if there is a measId configured with the MeasObjectNR and a reportConfig which has rsType set to ssb:

3>	set ssbFrequency in measResultFreqList to the value indicated by ssbFrequency as included in the MeasObjectNR;

2>	if there is a measId configured with the MeasObjectNR and a reportConfig which has rsType set to csi-rs:

3>	set refFreqCSI-RS in measResultFreqList to the value indicated by refFreqCSI-RS as included in the associated measurement object;

2>	if a serving cell is associated with the MeasObjectNR:

3>	set measResultServingCell in measResultFreqList to include the available quantities of the concerned cell and in accordance with the performance requirements in TS 38.133 [14];

2>	set the measResultNeighCellList in measResultFreqList to include the best measured cells, ordered such that the best cell is listed first, and based on measurements collected up to the moment the UE detected the failure, and set its fields as follows;

3>	ordering the cells with sorting as follows:

4>	based on SS/PBCH block if SS/PBCH block measurement results are available and otherwise based on CSI-RS;

4>	using RSRP if RSRP measurement results are available, otherwise using RSRQ if RSRQ measurement results are available, otherwise using SINR;

3>	for each neighbour cell included:

4>	include the optional fields that are available.

NOTE 1:	The measured quantities are filtered by the L3 filter as configured in the mobility measurement configuration. The measurements are based on the time domain measurement resource restriction, if configured. Blacklisted cells are not required to be reported.

NOTE 2:	Field measResultSCG-Failure is used to report available results for NR frequencies the UE is configured to measure by SCG RRC signalling.

The UE shall submit the SCGFailureInformation message to lower layers for transmission.

### 5.7.3a	EUTRA SCG failure information

#### 5.7.3a.1	General

Figure 5.7.3a.1-1: EUTRA SCG failure information

The purpose of this procedure is to inform NR MN about an SCG failure on E-UTRA SN the UE has experienced (e.g. SCG radio link failure, SCG change failure), as specified in TS 36.331 [10] clause 5.6.13.2.

#### 5.7.3a.2	Initiation

A UE initiates the procedure to report EUTRA SCG failures when EUTRA SCG transmission is not suspended and in accordance with TS 36.331 [10] clause 5.6.13.2. Actions the UE shall perform upon initiating the procedure, other than related to the transmission of the SCGFailureInformationEUTRA message are specified in TS 36.331 [10] clause 5.6.13.2.

#### 5.7.3a.3	Actions related to transmission of SCGFailureInformationEUTRA message

The UE shall set the contents of the SCGFailureInformationEUTRA message as follows:

1>	include failureType within failureReportSCG-EUTRA and set it to indicate the SCG failure in accordance with TS 36.331 [10] clause 5.6.13.4;

1>	include and set measResultSCG-FailureMRDC in accordance with TS 36.331 [10] clause 5.6.13.5;

1>	for each EUTRA frequency the UE is configured to measure by measConfig for which measurement results are available:

2>	set the measResultFreqListMRDC to include the best measured cells, ordered such that the best cell is listed first using RSRP to order if RSRP measurement results are available for cells on this frequency, otherwise using RSRQ to order if RSRQ measurement results are available for cells on this frequency, otherwise using SINR to order, and based on measurements collected up to the moment the UE detected the failure, and for each cell that is included, include the optional fields that are available;

NOTE:	Field measResultSCG-FailureMRDC is used to report available results for E-UTRAN frequencies the UE is configured to measure by E-UTRA RRC signalling.

The UE shall submit the SCGFailureInformationEUTRA message to lower layers for transmission.

### 5.7.4	UE Assistance Information

#### 5.7.4.1	General

Figure 5.7.4.1-1: UE Assistance Information

The purpose of this procedure is to inform the network of the UE's delay budget report carrying desired increment/decrement in the connected mode DRX cycle length, or overheating assistance information.

#### 5.7.4.2	Initiation

A UE capable of providing delay budget report in RRC_CONNECTED may initiate the procedure in several cases, including upon being configured to provide delay budget report and upon change of delay budget preference.

A UE capable of providing overheating assistance information in RRC_CONNECTED may initiate the procedure if it was configured to do so, upon detecting internal overheating, or upon detecting that it is no longer experiencing an overheating condition.

Upon initiating the procedure, the UE shall:

1>	if configured to provide delay budget report:

2>	if the UE did not transmit a UEAssistanceInformation message with delayBudgetReport since it was configured to provide delay budget report; or

2>	if the current delay budget is different from the one indicated in the last transmission of the UEAssistanceInformation message including delayBudgetReport and timer T342 is not running:

3>	start or restart timer T342 with the timer value set to the delayBudgetReportingProhibitTimer;

3>	initiate transmission of the UEAssistanceInformation message in accordance with 5.7.4.3 to provide a delay budget report;

1>	if configured to provide overheating assistance information:

2>	if the overheating condition has been detected and T345 is not running; or

2>	if the current overheating assistance information is different from the one indicated in the last transmission of the UEAssistanceInformation message including overheatingAssistance and timer T345 is not running:

3>	start timer T345 with the timer value set to the overheatingIndicationProhibitTimer;

3>	initiate transmission of the UEAssistanceInformation message in accordance with 5.7.4.3 to provide overheating assistance information;

#### 5.7.4.3	Actions related to transmission of UEAssistanceInformation message

The UE shall set the contents of the UEAssistanceInformation message as follows:

1>	if transmission of the UEAssistanceInformation message is initiated to provide a delay budget report according to 5.7.4.2;

2>	set delayBudgetReport to type1 according to a desired value;

1>	if transmission of the UEAssistanceInformation message is initiated to provide overheating assistance information according to 5.7.4.2;

2>	if the UE experiences internal overheating:

3>	if the UE prefers to temporarily reduce the number of maximum secondary component carriers:

4>	include reducedMaxCCs in the OverheatingAssistance IE;

4>	set reducedCCsDL to the number of maximum SCells the UE prefers to be temporarily configured in downlink;

4>	set reducedCCsUL to the number of maximum SCells the UE prefers to be temporarily configured in uplink;

3>	if the UE prefers to temporarily reduce maximum aggregated bandwidth of FR1:

4>	include reducedMaxBW-FR1 in the OverheatingAssistance IE;

4>	set reducedBW-FR1-DL to the maximum aggregated bandwidth the UE prefers to be temporarily configured across all downlink carriers of FR1;

4>	set reducedBW-FR1-UL to the maximum aggregated bandwidth the UE prefers to be temporarily configured across all uplink carriers of FR1;

3>	if the UE prefers to temporarily reduce maximum aggregated bandwidth of FR2:

4>	include reducedMaxBW-FR2 in the OverheatingAssistance IE;

4>	set reducedBW-FR2-DL to the maximum aggregated bandwidth the UE prefers to be temporarily configured across all downlink carriers of FR2;

4>	set reducedBW-FR2-UL to the maximum aggregated bandwidth the UE prefers to be temporarily configured across all uplink carriers of FR2;

3>	if the UE prefers to temporarily reduce the number of maximum MIMO layers of each serving cell operating on FR1:

4>	include reducedMaxMIMO-LayersFR1 in the OverheatingAssistance IE;

4>	set reducedMIMO-LayersFR1-DL to the number of maximum MIMO layers of each serving cell operating on FR1 the UE prefers to be temporarily configured in downlink;

4>	set reducedMIMO-LayersFR1-UL to the number of maximum MIMO layers of each serving cell operating on FR1 the UE prefers to be temporarily configured in uplink;

3>	if the UE prefers to temporarily reduce the number of maximum MIMO layers of each serving cell operating on FR2:

4>	include reducedMaxMIMO-LayersFR2 in the OverheatingAssistance IE;

4>	set reducedMIMO-LayersFR2-DL to the number of maximum MIMO layers of each serving cell operating on FR2 the UE prefers to be temporarily configured in downlink;

4>	set reducedMIMO-LayersFR2-UL to the number of maximum MIMO layers of each serving cell operating on FR2 the UE prefers to be temporarily configured in uplink;

2>	else (if the UE no longer experiences an overheating condition):

3>	do not include reducedMaxCCs, reducedMaxBW-FR1, reducedMaxBW-FR2, reducedMaxMIMO-LayersFR1 and reducedMaxMIMO-LayersFR2 in OverheatingAssistance IE;

The UE shall submit the UEAssistanceInformation message to lower layers for transmission.

### 5.7.5	Failure information

#### 5.7.5.1	General

Figure 5.7.5.1-1: Failure information

The purpose of this procedure is to inform the network about a failure detected by the UE.

#### 5.7.5.2	Initiation

A UE initiates the procedure when there is a need inform the network about a failure detected by the UE. In particular, the UE initiates the procedure when the following condition is met:

1>	upon detecting failure for an RLC bearer, in accordance with 5.3.10.3;

Upon initiating the procedure, the UE shall:

1>	initiate transmission of the FailureInformation message as specified in 5.7.5.3;

#### 5.7.5.3	Actions related to transmission of FailureInformation message

The UE shall:

1>	if initiated to provide RLC failure information, set FailureInfoRLC-Bearer as follows:

2>	set logicalChannelIdentity to the logical channel identity of the failing RLC bearer;

2>	set cellGroupId to the cell group identity of the failing RLC bearer;

2>	set the failureType as rlc-failure;

1>	if used to inform the network about a failure for an MCG RLC bearer:

2>	submit the FailureInformation message to lower layers for transmission via SRB1;

1>	else if used to inform the network about a failure for an SCG RLC bearer:

2>	if SRB3 is configured;

3>	submit the FailureInformation message to lower layers for transmission via SRB3;

2>	else;

3>	if the UE is in (NG)EN-DC:

4>	submit the FailureInformation message via E-UTRA embedded in E-UTRA RRC message ULInformationTransferMRDC as specified in TS 36.331 [10].

3>	else if the UE is in NR-DC:

4>	submit the FailureInformation message via SRB1 embedded in NR RRC message ULInformationTransferMRDC as specified in clause 5.7.2a.3.

# 6	Protocol data units, formats and parameters (ASN.1)

## 6.1	General

### 6.1.1	Introduction

The contents of each RRC message is specified in sub-clause 6.2 using ASN.1 to specify the message syntax and using tables when needed to provide further detailed information about the fields specified in the message syntax. The syntax of the information elements that are defined as stand-alone abstract types is further specified in a similar manner in sub-clause 6.3.

Usage of the text "Network always configures the UE with a value for this field" in the field description indicates that the network has to provide a value for the field in this or in a previous message based on delta configuration (for an optional field with Need M). It does not imply a mandatory presence of the field.

### 6.1.2	Need codes and conditions for optional downlink fields

The need for fields to be present in a message or an abstract type, i.e., the ASN.1 fields that are specified as OPTIONAL in the abstract notation (ASN.1), is specified by means of comment text tags attached to the OPTIONAL statement in the abstract syntax. All comment text tags are available for use in the downlink direction only. The meaning of each tag is specified in table 6.1.2-1.

If conditions are used, a conditional presence table is provided for the message or information element specifying the need of the field for each condition case. The table also specifies whether UE maintains or releases the value in case the field is absent. The conditions clarify what the UE may expect regarding the setting of the message by the network. Violation of conditions is regarded as invalid network behaviour, which the UE is not required to cope with. Hence the general error handling defined in 10.4 does not apply in case a field is absent although it is mandatory according to the CondC or CondM condition.

For guidelines on the use of need codes and conditions, see Annex A.6 and A.7.

Table 6.1.2-1: Meaning of abbreviations used to specify the need for fields to be present

| Abbreviation | Meaning |
| --- | --- |
| Cond conditionTag | Conditionally present Presence of the field is specified in a tabular form following the ASN.1 segment. |
| CondC conditionTag | Configuration condition Presence of the field is conditional to other configuration settings. |
| CondM conditionTag | Message condition Presence of the field is conditional to other fields included in the message. |
| Need S | Specified Used for (configuration) fields, whose field description or procedure specifies the UE behavior performed upon receiving a message with the field absent (and not if field description or procedure specifies the UE behavior when field is not configured). |
| Need M | Maintain Used for (configuration) fields that are stored by the UE i.e. not one-shot. Upon receiving a message with the field absent, the UE maintains the current value. |
| Need N | No action (one-shot configuration that is not maintained) Used for (configuration) fields that are not stored and whose presence causes a one-time action by the UE. Upon receiving message with the field absent, the UE takes no action. |
| Need R | Release Used for (configuration) fields that are stored by the UE i.e. not one-shot. Upon receiving a message with the field absent, the UE releases the current value. |

NOTE:	In this version of the specification, the condition tags CondC and CondM are not used.

Any field with Need M or Need N in system information shall be interpreted as Need R.

The need code used within a CondX definition only applies for the case (part of the condition) where it is defined: A condition may have different need codes for different parts of the condition. In particular, the CondX definition may contain the following "otherwise the field is absent" parts:

-	"Otherwise, the field is absent": The field is not relevant or should not be configured when this part of the condition applies. In particular, the UE behaviour is not defined when the field is configured via another part of the condition and is reconfigured to this part of the condition.  A need code is not provided when the transition from another part of the condition to this part of the condition is not supported, when the field clearly is a one-shot or there is no difference whether UE maintains or releases the value (e.g., in case the field is mandatory present according to the other part of the condition).

-	"Otherwise, the field is absent, Need R": The field is released if absent when this part of the condition applies. This handles UE behaviour in case the field is configured via another part of the condition and this part of the condition applies (which means that network can assume UE releases the field if this part of the condition is valid).

-	"Otherwise, the field is absent, Need M": The UE retains the field if it was already configured when this part of the condition applies. This means the network cannot release the field , but UE retains the previously configured value.

Use of different Need codes in different parts of a condition should be avoided.

For downlink messages, the need codes, conditions and ASN.1 defaults specified for a particular (child) field only apply in case the (parent) field including the particular field is present. Thus, if the parent is absent the UE shall not release the field unless the absence of the parent field implies that.

For (parent) fields without need codes in downlink messages, if the parent field is absent, UE shall follow the need codes of the child fields. Thus, if parent field is absent, the need code of each child field is followed (i.e. Need R child fields are released, Need M child fields are not modified and the actions for Need S child fields depend on the specified conditions of each field). Examples of (parent) fields in downlink messages without need codes where this rule applies are:

-	nonCriticalExtension fields at the end of a message using empty SEQUENCE extension mechanism,

-	groups of non-critical extensions using double brackets (referred to as extension groups), and

-	non-critical extensions at the end of a message or at the end of a structure, contained in a BIT STRING or OCTET STRING (referred to as parent extension fields).

The handling of need codes as specified in the previous is illustrated by means of an example, as shown in the following ASN.1.

-- /example/ ASN1START

RRCMessage-IEs ::=                SEQUENCE {

field1                            InformationElement1            OPTIONAL,  -- Need M

field2                            InformationElement2            OPTIONAL,  -- Need R

nonCriticalExtension              RRCMessage-v1570-IEs           OPTIONAL

}

RRCMessage-1570-IEs ::=           SEQUENCE {

field3                            InformationElement3            OPTIONAL,  -- Need M

nonCriticalExtension              RRCMessage-v1640-IEs           OPTIONAL

}

RRCMessage-v1640-IEs ::=          SEQUENCE {

field4                            InformationElement4            OPTIONAL,  -- Need R

nonCriticalExtension              SEQUENCE {}                    OPTIONAL

}

InformationElement1 ::=           SEQUENCE {

field11                           InformationElement11           OPTIONAL,  -- Need M

field12                           InformationElement12           OPTIONAL,  -- Need R

...,

[[

field13                           InformationElement13           OPTIONAL,  -- Need R

field14                           InformationElement14           OPTIONAL   -- Need M

]]

}

InformationElement2 ::=           SEQUENCE {

field21                           InformationElement11           OPTIONAL,  -- Need M

...

}

-- ASN1STOP

The handling of need codes as specified in the previous implies that:

-	if field1 in RRCMessage-IEs is absent, UE does not modify any child fields configured within field1 (regardless of their need codes);

-	if field2 in RRCMessage-IEs is absent, UE releases the field2 (and also its child field field21);

-	if field1 or field2 in RRCMessage-IEs is present, UE retains or releases their child fields according to the child field presence conditions;

-	if field1 in RRCMessage-IEs is present but the extension group containing field13 and field14 is absent, the UE releases field13 but does not modify field14;

-	if nonCriticalExtension defined by IE RRCMessage-v1570-IEs is absent, the UE does not modify field3 but releases field4;

### 6.1.3	General rules

In the ASN.1 of this specification, the first bit of a bit string refers to the leftmost bit, unless stated otherwise.

Upon reception of a list not using ToAddModList and ToReleaseList structure, the UE shall delete all entries of the list currently in the UE configuration before applying the received list and shall consider each entry as newly created. This applies also to lists whose size is extended (i.e. with a second list structure in the ASN.1 comprising additional entries). This implies that Need M should not be not used for fields in the entries of these lists; if used, UE will handle such fields equivalent to a Need R.

## 6.2	RRC messages

### 6.2.1	General message structure

#### –	NR-RRC-Definitions

This ASN.1 segment is the start of the NR RRC PDU definitions.

-- ASN1START

-- TAG-NR-RRC-DEFINITIONS-START

NR-RRC-Definitions DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

-- TAG-NR-RRC-DEFINITIONS-STOP

-- ASN1STOP

#### –	BCCH-BCH-Message

The BCCH-BCH-Message class is the set of RRC messages that may be sent from the network to the UE via BCH on the BCCH logical channel.

-- ASN1START

-- TAG-BCCH-BCH-MESSAGE-START

BCCH-BCH-Message ::=            SEQUENCE {

message                         BCCH-BCH-MessageType

}

BCCH-BCH-MessageType ::=        CHOICE {

mib                             MIB,

messageClassExtension           SEQUENCE {}

}

-- TAG-BCCH-BCH-MESSAGE-STOP

-- ASN1STOP

#### –	BCCH-DL-SCH-Message

The BCCH-DL-SCH-Message class is the set of RRC messages that may be sent from the network to the UE via DL-SCH on the BCCH logical channel.

-- ASN1START

-- TAG-BCCH-DL-SCH-MESSAGE-START

BCCH-DL-SCH-Message ::=         SEQUENCE {

message                         BCCH-DL-SCH-MessageType

}

BCCH-DL-SCH-MessageType ::=     CHOICE {

c1                              CHOICE {

systemInformation               SystemInformation,

systemInformationBlockType1     SIB1

},

messageClassExtension           SEQUENCE {}

}

-- TAG-BCCH-DL-SCH-MESSAGE-STOP

-- ASN1STOP

#### –	DL-CCCH-Message

The DL-CCCH-Message class is the set of RRC messages that may be sent from the Network to the UE on the downlink CCCH logical channel.

-- ASN1START

-- TAG-DL-CCCH-MESSAGE-START

DL-CCCH-Message ::=             SEQUENCE {

message                         DL-CCCH-MessageType

}

DL-CCCH-MessageType ::=         CHOICE {

c1                              CHOICE {

rrcReject                       RRCReject,

rrcSetup                        RRCSetup,

spare2                          NULL,

spare1                          NULL

},

messageClassExtension           SEQUENCE {}

}

-- TAG-DL-CCCH-MESSAGE-STOP

-- ASN1STOP

#### –	DL-DCCH-Message

The DL-DCCH-Message class is the set of RRC messages that may be sent from the network to the UE on the downlink DCCH logical channel.

-- ASN1START

-- TAG-DL-DCCH-MESSAGE-START

DL-DCCH-Message ::=              SEQUENCE {

message                         DL-DCCH-MessageType

}

DL-DCCH-MessageType ::=         CHOICE {

c1                              CHOICE {

rrcReconfiguration              RRCReconfiguration,

rrcResume                       RRCResume,

rrcRelease                      RRCRelease,

rrcReestablishment              RRCReestablishment,

securityModeCommand             SecurityModeCommand,

dlInformationTransfer           DLInformationTransfer,

ueCapabilityEnquiry             UECapabilityEnquiry,

counterCheck                    CounterCheck,

mobilityFromNRCommand           MobilityFromNRCommand,

spare7 NULL,

spare6 NULL, spare5 NULL, spare4 NULL,

spare3 NULL, spare2 NULL, spare1 NULL

},

messageClassExtension   SEQUENCE {}

}

-- TAG-DL-DCCH-MESSAGE-STOP

-- ASN1STOP

#### –	PCCH-Message

The PCCH-Message class is the set of RRC messages that may be sent from the Network to the UE on the PCCH logical channel.

-- ASN1START

-- TAG-PCCH-PCH-MESSAGE-START

PCCH-Message ::=                SEQUENCE {

message                         PCCH-MessageType

}

PCCH-MessageType ::=            CHOICE {

c1                              CHOICE {

paging                          Paging,

spare1  NULL

},

messageClassExtension       SEQUENCE {}

}

-- TAG-PCCH-PCH-MESSAGE-STOP

-- ASN1STOP

#### –	UL-CCCH-Message

The UL-CCCH-Message class is the set of 48-bits RRC messages that may be sent from the UE to the Network on the uplink CCCH logical channel.

-- ASN1START

-- TAG-UL-CCCH-MESSAGE-START

UL-CCCH-Message ::=             SEQUENCE {

message                         UL-CCCH-MessageType

}

UL-CCCH-MessageType ::=         CHOICE {

c1                              CHOICE {

rrcSetupRequest                 RRCSetupRequest,

rrcResumeRequest                RRCResumeRequest,

rrcReestablishmentRequest       RRCReestablishmentRequest,

rrcSystemInfoRequest            RRCSystemInfoRequest

},

messageClassExtension           SEQUENCE {}

}

-- TAG-UL-CCCH-MESSAGE-STOP

-- ASN1STOP

#### –	UL-CCCH1-Message

The UL-CCCH1-Message class is the set of 64-bits RRC messages that may be sent from the UE to the Network on the uplink CCCH1 logical channel.

-- ASN1START

-- TAG-UL-CCCH1-MESSAGE-START

UL-CCCH1-Message ::=            SEQUENCE {

message                         UL-CCCH1-MessageType

}

UL-CCCH1-MessageType ::=        CHOICE {

c1                              CHOICE {

rrcResumeRequest1               RRCResumeRequest1,

spare3 NULL,

spare2 NULL,

spare1 NULL

},

messageClassExtension SEQUENCE {}

}

-- TAG-UL-CCCH1-MESSAGE-STOP

-- ASN1STOP

#### –	UL-DCCH-Message

The UL-DCCH-Message class is the set of RRC messages that may be sent from the UE to the network on the uplink DCCH logical channel.

-- ASN1START

-- TAG-UL-DCCH-MESSAGE-START

UL-DCCH-Message ::=             SEQUENCE {

message                         UL-DCCH-MessageType

}

UL-DCCH-MessageType ::=         CHOICE {

c1                              CHOICE {

measurementReport               MeasurementReport,

rrcReconfigurationComplete      RRCReconfigurationComplete,

rrcSetupComplete                RRCSetupComplete,

rrcReestablishmentComplete      RRCReestablishmentComplete,

rrcResumeComplete               RRCResumeComplete,

securityModeComplete            SecurityModeComplete,

securityModeFailure             SecurityModeFailure,

ulInformationTransfer           ULInformationTransfer,

locationMeasurementIndication   LocationMeasurementIndication,

ueCapabilityInformation         UECapabilityInformation,

counterCheckResponse            CounterCheckResponse,

ueAssistanceInformation         UEAssistanceInformation,

failureInformation              FailureInformation,

ulInformationTransferMRDC       ULInformationTransferMRDC,

scgFailureInformation           SCGFailureInformation,

scgFailureInformationEUTRA      SCGFailureInformationEUTRA

},

messageClassExtension           SEQUENCE {}

}

-- TAG-UL-DCCH-MESSAGE-STOP

-- ASN1STOP

### 6.2.2	Message definitions

#### –	CounterCheck

The CounterCheck message is used by the network to indicate the current COUNT MSB values associated to each DRB and to request the UE to compare these to its COUNT MSB values and to report the comparison results to the network.

Signalling radio bearer: SRB1

RLC-SAP: AM

Logical channel: DCCH

Direction: Network to UE

CounterCheck message

-- ASN1START

-- TAG-COUNTERCHECK-START

CounterCheck ::=                SEQUENCE {

rrc-TransactionIdentifier       RRC-TransactionIdentifier,

criticalExtensions              CHOICE {

counterCheck                    CounterCheck-IEs,

criticalExtensionsFuture        SEQUENCE {}

}

}

CounterCheck-IEs ::=            SEQUENCE {

drb-CountMSB-InfoList           DRB-CountMSB-InfoList,

lateNonCriticalExtension        OCTET STRING                        OPTIONAL,

nonCriticalExtension            SEQUENCE {}                         OPTIONAL

}

DRB-CountMSB-InfoList ::=       SEQUENCE (SIZE (1..maxDRB)) OF DRB-CountMSB-Info

DRB-CountMSB-Info ::=           SEQUENCE {

drb-Identity                    DRB-Identity,

countMSB-Uplink                 INTEGER(0..33554431),

countMSB-Downlink               INTEGER(0..33554431)

}

-- TAG-COUNTERCHECK-STOP

-- ASN1STOP

CounterCheck-IEs field descriptions

drb-CountMSB-InfoList Indicates the MSBs of the COUNT values of the DRBs.

DRB-CountMSB-Info field descriptions

countMSB-Downlink Indicates the value of 25 MSBs from RX_NEXT – 1 (specified in TS 38.323 [5]) associated to this DRB.

countMSB-Uplink Indicates the value of 25 MSBs from TX_NEXT – 1 (specified in TS 38.323 [5]) associated to this DRB.

#### –	CounterCheckResponse

The CounterCheckResponse message is used by the UE to respond to a CounterCheck message.

Signalling radio bearer: SRB1

RLC-SAP: AM

Logical channel: DCCH

Direction: UE to Network

CounterCheckResponse message

-- ASN1START

-- TAG-COUNTERCHECKRESPONSE-START

CounterCheckResponse ::=        SEQUENCE {

rrc-TransactionIdentifier       RRC-TransactionIdentifier,

criticalExtensions              CHOICE {

counterCheckResponse            CounterCheckResponse-IEs,

criticalExtensionsFuture        SEQUENCE {}

}

}

CounterCheckResponse-IEs ::=    SEQUENCE {

drb-CountInfoList               DRB-CountInfoList,

lateNonCriticalExtension        OCTET STRING                        OPTIONAL,

nonCriticalExtension            SEQUENCE {}                         OPTIONAL

}

DRB-CountInfoList ::=           SEQUENCE (SIZE (0..maxDRB)) OF DRB-CountInfo

DRB-CountInfo ::=               SEQUENCE {

drb-Identity                    DRB-Identity,

count-Uplink                    INTEGER(0..4294967295),

count-Downlink                  INTEGER(0..4294967295)

}

-- TAG-COUNTERCHECKRESPONSE-STOP

-- ASN1STOP

CounterCheckResponse-IEs field descriptions

drb-CountInfoList Indicates the COUNT values of the DRBs.

DRB-CountInfo field descriptions

count-Downlink Indicates the value of RX_NEXT – 1 (specified in TS 38.323 [5]) associated to this DRB.

count-Uplink Indicates the value of TX_NEXT – 1 (specified in TS 38.323 [5]) associated to this DRB.

#### –	DLInformationTransfer

The DLInformationTransfer message is used for the downlink transfer of NAS dedicated information.

Signalling radio bearer: SRB2 or SRB1 (only if SRB2 not established yet. If SRB2 is suspended, the network does not send this message until SRB2 is resumed.)

RLC-SAP: AM

Logical channel: DCCH

Direction: Network to UE

DLInformationTransfer message

-- ASN1START

-- TAG-DLINFORMATIONTRANSFER-START

DLInformationTransfer ::=           SEQUENCE {

rrc-TransactionIdentifier           RRC-TransactionIdentifier,

criticalExtensions                  CHOICE {

dlInformationTransfer           DLInformationTransfer-IEs,

criticalExtensionsFuture            SEQUENCE {}

}

}

DLInformationTransfer-IEs ::=   SEQUENCE {

dedicatedNAS-Message                DedicatedNAS-Message                OPTIONAL,   -- Need N

lateNonCriticalExtension            OCTET STRING                        OPTIONAL,

nonCriticalExtension                SEQUENCE {} OPTIONAL

}

-- TAG-DLINFORMATIONTRANSFER-STOP

-- ASN1STOP

#### –	FailureInformation

The FailureInformation message is used to inform the network about a failure detected by the UE.

Signalling radio bearer: SRB1 or SRB3

RLC-SAP: AM

Logical channel: DCCH

Direction: UE to network

FailureInformation message

-- ASN1START

-- TAG-FAILUREINFORMATION-START

FailureInformation ::=         SEQUENCE {

criticalExtensions             CHOICE {

failureInformation             FailureInformation-IEs,

criticalExtensionsFuture       SEQUENCE {}

}

}

FailureInformation-IEs ::=     SEQUENCE {

failureInfoRLC-Bearer          FailureInfoRLC-Bearer    OPTIONAL,

lateNonCriticalExtension       OCTET STRING             OPTIONAL,

nonCriticalExtension           SEQUENCE {}              OPTIONAL

}

FailureInfoRLC-Bearer ::=      SEQUENCE {

cellGroupId                    CellGroupId,

logicalChannelIdentity         LogicalChannelIdentity,

failureType                    ENUMERATED {rlc-failure, spare3, spare2, spare1}

}

-- TAG-FAILUREINFORMATION-STOP

-- ASN1STOP

#### –	LocationMeasurementIndication

The LocationMeasurementIndication message is used to indicate that the UE is going to either start or stop location related measurement which requires measurement gaps.

Signalling radio bearer: SRB1

RLC-SAP: AM

Logical channel: DCCH

Direction: UE to Network

LocationMeasurementIndication message

-- ASN1START

-- TAG-LOCATIONMEASUREMENTINDICATION-START

LocationMeasurementIndication ::=   SEQUENCE {

criticalExtensions                      CHOICE {

locationMeasurementIndication               LocationMeasurementIndication-IEs,

criticalExtensionsFuture                    SEQUENCE {}

}

}

LocationMeasurementIndication-IEs ::=       SEQUENCE {

measurementIndication                       SetupRelease {LocationMeasurementInfo},

lateNonCriticalExtension                    OCTET STRING                                                            OPTIONAL,

nonCriticalExtension                        SEQUENCE{}                                                              OPTIONAL

}

-- TAG-LOCATIONMEASUREMENTINDICATION-STOP

-- ASN1STOP

#### –	MeasurementReport

The MeasurementReport message is used for the indication of measurement results.

Signalling radio bearer: SRB1, SRB3

RLC-SAP: AM

Logical channel: DCCH

Direction: UE to Network

MeasurementReport message

-- ASN1START

-- TAG-MEASUREMENTREPORT-START

MeasurementReport ::=               SEQUENCE {

criticalExtensions                  CHOICE {

measurementReport                   MeasurementReport-IEs,

criticalExtensionsFuture            SEQUENCE {}

}

}

MeasurementReport-IEs ::=           SEQUENCE {

measResults                         MeasResults,

lateNonCriticalExtension                OCTET STRING                                                            OPTIONAL,

nonCriticalExtension                    SEQUENCE{}                                                              OPTIONAL

}

-- TAG-MEASUREMENTREPORT-STOP

-- ASN1STOP

#### –	MIB

The MIB includes the system information transmitted on BCH.

Signalling radio bearer: N/A

RLC-SAP: TM

Logical channel: BCCH

Direction: Network to UE

MIB

-- ASN1START

-- TAG-MIB-START

MIB ::=                             SEQUENCE {

systemFrameNumber                   BIT STRING (SIZE (6)),

subCarrierSpacingCommon             ENUMERATED {scs15or60, scs30or120},

ssb-SubcarrierOffset                INTEGER (0..15),

dmrs-TypeA-Position                 ENUMERATED {pos2, pos3},

pdcch-ConfigSIB1                    PDCCH-ConfigSIB1,

cellBarred                          ENUMERATED {barred, notBarred},

intraFreqReselection                ENUMERATED {allowed, notAllowed},

spare                               BIT STRING (SIZE (1))

}

-- TAG-MIB-STOP

-- ASN1STOP

MIB field descriptions

cellBarred Value barred means that the cell is barred, as defined in TS 38.304 [20].

dmrs-TypeA-Position Position of (first) DM-RS for downlink (see TS 38.211 [16], clause 7.4.1.1.2) and uplink (see TS 38.211 [16], clause 6.4.1.1.3).

intraFreqReselection Controls cell selection/reselection to intra-frequency cells when the highest ranked cell is barred, or treated as barred by the UE, as specified in TS 38.304 [20].

pdcch-ConfigSIB1 Determines a common ControlResourceSet (CORESET), a common search space and necessary PDCCH parameters. If the field ssb-SubcarrierOffset indicates that SIB1 is absent, the field pdcch-ConfigSIB1 indicates the frequency positions where the UE may find SS/PBCH block with SIB1 or the frequency range where the network does not provide SS/PBCH block with SIB1 (see TS 38.213 [13], clause 13).

ssb-SubcarrierOffset Corresponds to kSSB (see TS 38.213 [13]), which is the frequency domain offset between SSB and the overall resource block grid in number of subcarriers. (See TS 38.211 [16], clause 7.4.3.1). The value range of this field may be extended by an additional most significant bit encoded within PBCH as specified in TS 38.213 [13]. This field may indicate that this cell does not provide SIB1 and that there is hence no CORESET#0 configured in MIB (see TS 38.213 [13], clause 13). In this case, the field pdcch-ConfigSIB1 may indicate the frequency positions where the UE may (not) find a SS/PBCH with a control resource set and search space for SIB1 (see TS 38.213 [13], clause 13).

subCarrierSpacingCommon Subcarrier spacing for SIB1, Msg.2/4 for initial access, paging and broadcast SI-messages. If the UE acquires this MIB on an FR1 carrier frequency, the value scs15or60 corresponds to 15 kHz and the value scs30or120 corresponds to 30 kHz. If the UE acquires this MIB on an FR2 carrier frequency, the value scs15or60 corresponds to 60 kHz and the value scs30or120 corresponds to 120 kHz.

systemFrameNumber The 6 most significant bits (MSB) of the 10-bit System Frame Number (SFN). The 4 LSB of the SFN are conveyed in the PBCH transport block as part of channel coding (i.e. outside the MIB encoding), as defined in clause 7.1 in TS 38.212 [17].

#### –	MobilityFromNRCommand

The MobilityFromNRCommand message is used to command handover from NR to E-UTRA/EPC or E-UTRA/5GC.

Signalling radio bearer: SRB1

RLC-SAP: AM

Logical channel: DCCH

Direction: Network to UE

MobilityFromNRCommand message

-- ASN1START

-- TAG-MOBILITYFROMNRCOMMAND-START

MobilityFromNRCommand ::=       SEQUENCE {

rrc-TransactionIdentifier           RRC-TransactionIdentifier,

criticalExtensions                  CHOICE {

mobilityFromNRCommand               MobilityFromNRCommand-IEs,

criticalExtensionsFuture                SEQUENCE {}

}

}

MobilityFromNRCommand-IEs ::=   SEQUENCE {

targetRAT-Type                          ENUMERATED { eutra, spare3, spare2, spare1, ...},

targetRAT-MessageContainer              OCTET STRING,

nas-SecurityParamFromNR                 OCTET STRING                                OPTIONAL,   -- Cond HO-ToEPC

lateNonCriticalExtension                OCTET STRING                                OPTIONAL,

nonCriticalExtension                    SEQUENCE {}                                 OPTIONAL

}

-- TAG-MOBILITYFROMNRCOMMAND-STOP

-- ASN1STOP

MobilityFromNRCommand-IEs field descriptions

nas-SecurityParamFromNR This field is used to deliver the key synchronisation and Key freshness for the NR to LTE/EPC handovers and a part of the downlink NAS COUNT as specified in TS 33.501 [11].

targetRAT-MessageContainer The field contains a message specified in another standard, as indicated by the targetRAT-Type, and carries information about the target cell identifier(s) and radio parameters relevant for the target radio access technology. A complete message is included, as specified in the other standard. See NOTE 1

targetRAT-Type Indicates the target RAT type.

NOTE 1:	The correspondence between the value of the targetRAT-Type, the standard to apply, and the message contained within the targetRAT-MessageContainer is shown in the table below:

| targetRAT-Type | Standard to apply | targetRAT-MessageContainer |
| --- | --- | --- |
| eutra | TS 36.331 [10] (clause 5.4.2) | DL-DCCH-Message including the RRCConnectionReconfiguration |

| Conditional Presence | Explanation |
| --- | --- |
| HO-ToEPC | This field is mandatory present in case of inter system handover. Otherwise it is absent. |

#### –	Paging

The Paging message is used for the notification of one or more UEs.

Signalling radio bearer: N/A

RLC-SAP: TM

Logical channel: PCCH

Direction: Network to UE

Paging message

-- ASN1START

-- TAG-PAGING-START

Paging ::=                          SEQUENCE {

pagingRecordList                        PagingRecordList                                                        OPTIONAL, -- Need N

lateNonCriticalExtension                OCTET STRING                                                            OPTIONAL,

nonCriticalExtension                    SEQUENCE{}                                                              OPTIONAL

}

PagingRecordList ::=                SEQUENCE (SIZE(1..maxNrofPageRec)) OF PagingRecord

PagingRecord ::=                    SEQUENCE {

ue-Identity                         PagingUE-Identity,

accessType                          ENUMERATED {non3GPP}    OPTIONAL,   -- Need N

...

}

PagingUE-Identity ::=               CHOICE {

ng-5G-S-TMSI                        NG-5G-S-TMSI,

fullI-RNTI                          I-RNTI-Value,

...

}

-- TAG-PAGING-STOP

-- ASN1STOP

PagingRecord field descriptions

accessType Indicates whether the Paging message is originated due to the PDU sessions from the non-3GPP access.

#### –	RRCReestablishment

The RRCReestablishment message is used to re-establish SRB1.

Signalling radio bearer: SRB1

RLC-SAP: AM

Logical channel: DCCH

Direction: Network to UE

RRCReestablishment message

-- ASN1START

-- TAG-RRCREESTABLISHMENT-START

RRCReestablishment ::=              SEQUENCE {

rrc-TransactionIdentifier           RRC-TransactionIdentifier,

criticalExtensions                  CHOICE {

rrcReestablishment                  RRCReestablishment-IEs,

criticalExtensionsFuture            SEQUENCE {}

}

}

RRCReestablishment-IEs ::=          SEQUENCE {

nextHopChainingCount                NextHopChainingCount,

lateNonCriticalExtension            OCTET STRING                        OPTIONAL,

nonCriticalExtension                SEQUENCE {}                         OPTIONAL

}

-- TAG-RRCREESTABLISHMENT-STOP

-- ASN1STOP

#### –	RRCReestablishmentComplete

The RRCReestablishmentComplete message is used to confirm the successful completion of an RRC connection re-establishment.

Signalling radio bearer: SRB1

RLC-SAP: AM

Logical channel: DCCH

Direction: UE to Network

RRCReestablishmentComplete message

-- ASN1START

-- TAG-RRCREESTABLISHMENTCOMPLETE-START

RRCReestablishmentComplete ::=      SEQUENCE {

rrc-TransactionIdentifier           RRC-TransactionIdentifier,

criticalExtensions                  CHOICE {

rrcReestablishmentComplete          RRCReestablishmentComplete-IEs,

criticalExtensionsFuture            SEQUENCE {}

}

}

RRCReestablishmentComplete-IEs ::=  SEQUENCE {

lateNonCriticalExtension            OCTET STRING                    OPTIONAL,

nonCriticalExtension                SEQUENCE {}                     OPTIONAL

}

-- TAG-RRCREESTABLISHMENTCOMPLETE-STOP

-- ASN1STOP

#### –	RRCReestablishmentRequest

The RRCReestablishmentRequest message is used to request the reestablishment of an RRC connection.

Signalling radio bearer: SRB0

RLC-SAP: TM

Logical channel: CCCH

Direction: UE to Network

RRCReestablishmentRequest message

-- ASN1START

-- TAG-RRCREESTABLISHMENTREQUEST-START

RRCReestablishmentRequest ::=       SEQUENCE {

rrcReestablishmentRequest           RRCReestablishmentRequest-IEs

}

RRCReestablishmentRequest-IEs ::=   SEQUENCE {

ue-Identity                         ReestabUE-Identity,

reestablishmentCause                ReestablishmentCause,

spare                               BIT STRING (SIZE (1))

}

ReestabUE-Identity ::=              SEQUENCE {

c-RNTI                              RNTI-Value,

physCellId                          PhysCellId,

shortMAC-I                          ShortMAC-I

}

ReestablishmentCause ::=            ENUMERATED {reconfigurationFailure, handoverFailure, otherFailure, spare1}

-- TAG-RRCREESTABLISHMENTREQUEST-STOP

-- ASN1STOP

ReestabUE-Identity field descriptions

physCellId The Physical Cell Identity of the PCell the UE was connected to prior to the failure.

RRCReestablishmentRequest-IEs field descriptions

reestablishmentCause Indicates the failure cause that triggered the re-establishment procedure. gNB is not expected to reject a RRCReestablishmentRequest due to unknown cause value being used by the UE.

ue-Identity UE identity included to retrieve UE context and to facilitate contention resolution by lower layers.

#### –	RRCReconfiguration

The RRCReconfiguration message is the command to modify an RRC connection. It may convey information for measurement configuration, mobility control, radio resource configuration (including RBs, MAC main configuration and physical channel configuration) and AS security configuration.

Signalling radio bearer: SRB1 or SRB3

RLC-SAP: AM

Logical channel: DCCH

Direction: Network to UE

RRCReconfiguration message

-- ASN1START

-- TAG-RRCRECONFIGURATION-START

RRCReconfiguration ::=              SEQUENCE {

rrc-TransactionIdentifier           RRC-TransactionIdentifier,

criticalExtensions                  CHOICE {

rrcReconfiguration                  RRCReconfiguration-IEs,

criticalExtensionsFuture            SEQUENCE {}

}

}

RRCReconfiguration-IEs ::=          SEQUENCE {

radioBearerConfig                       RadioBearerConfig                                                      OPTIONAL, -- Need M

secondaryCellGroup                      OCTET STRING (CONTAINING CellGroupConfig)                              OPTIONAL, -- Need M

measConfig                              MeasConfig                                                             OPTIONAL, -- Need M

lateNonCriticalExtension                OCTET STRING                                                           OPTIONAL,

nonCriticalExtension                    RRCReconfiguration-v1530-IEs                                           OPTIONAL

}

RRCReconfiguration-v1530-IEs ::=            SEQUENCE {

masterCellGroup                         OCTET STRING (CONTAINING CellGroupConfig)                              OPTIONAL, -- Need M

fullConfig                              ENUMERATED {true}                                                      OPTIONAL, -- Cond FullConfig

dedicatedNAS-MessageList                SEQUENCE (SIZE(1..maxDRB)) OF DedicatedNAS-Message                     OPTIONAL, -- Cond nonHO

masterKeyUpdate                         MasterKeyUpdate                                                        OPTIONAL, -- Cond MasterKeyChange

dedicatedSIB1-Delivery                  OCTET STRING (CONTAINING SIB1)                                         OPTIONAL, -- Need N

dedicatedSystemInformationDelivery      OCTET STRING (CONTAINING SystemInformation)                            OPTIONAL, -- Need N

otherConfig                             OtherConfig                                                            OPTIONAL, -- Need M

nonCriticalExtension                    RRCReconfiguration-v1540-IEs                                           OPTIONAL

}

RRCReconfiguration-v1540-IEs ::=        SEQUENCE {

otherConfig-v1540                       OtherConfig-v1540                      OPTIONAL, -- Need M

nonCriticalExtension                    RRCReconfiguration-v1560-IEs           OPTIONAL

}

RRCReconfiguration-v1560-IEs ::=            SEQUENCE {

mrdc-SecondaryCellGroupConfig               SetupRelease { MRDC-SecondaryCellGroupConfig }                    OPTIONAL,   -- Need M

radioBearerConfig2                          OCTET STRING (CONTAINING RadioBearerConfig)                       OPTIONAL,   -- Need M

sk-Counter                                  SK-Counter                                                        OPTIONAL,   -- Need N

nonCriticalExtension                        SEQUENCE {}                                                       OPTIONAL

}

MRDC-SecondaryCellGroupConfig ::=       SEQUENCE {

mrdc-ReleaseAndAdd                  ENUMERATED {true}                                                         OPTIONAL,   -- Need N

mrdc-SecondaryCellGroup             CHOICE {

nr-SCG                              OCTET STRING  (CONTAINING RRCReconfiguration),

eutra-SCG                           OCTET STRING

}

}

MasterKeyUpdate ::=                 SEQUENCE {

keySetChangeIndicator           BOOLEAN,

nextHopChainingCount            NextHopChainingCount,

nas-Container                   OCTET STRING                                                     OPTIONAL,    -- Cond securityNASC

...

}

-- TAG-RRCRECONFIGURATION-STOP

-- ASN1STOP

RRCReconfiguration-IEs field descriptions

dedicatedNAS-MessageList This field is used to transfer UE specific NAS layer information between the network and the UE. The RRC layer is transparent for each PDU in the list.

dedicatedSIB1-Delivery This field is used to transfer SIB1 to the UE. The field has the same values as the corresponding configuration in servingCellConfigCommon.

dedicatedSystemInformationDelivery This field is used to transfer SIB6, SIB7, SIB8 to the UE.

fullConfig Indicates that the full configuration option is applicable for the RRCReconfiguration message for intra-system intra-RAT HO. For inter-RAT HO from E-UTRA to NR, fullConfig indicates whether or not delta signalling of SDAP/PDCP from source RAT is applicable. This field is absent when the RRCReconfiguration message is transmitted on SRB3, and in an RRCReconfiguration message contained in another RRCReconfiguration message (or RRCConnectionReconfiguration message, see TS 36.331 [10]) transmitted on SRB1.

keySetChangeIndicator Indicates whether UE shall derive a new KgNB. If reconfigurationWithSync is included, value true indicates that a KgNB key is derived from a KAMF key taken into use through the latest successful NAS SMC procedure, or N2 handover procedure with KAMF change, as described in TS 33.501 [11] for KgNB re-keying. Value false indicates that the new KgNB key is obtained from the current KgNB key or from the NH as described in TS 33.501 [11].

masterCellGroup Configuration of master cell group.

mrdc-ReleaseAndAdd This field indicates that the current SCG configuration is released and a new SCG is added at the same time.

mrdc-SecondaryCellGroup Includes an RRC message for SCG configuration in NR-DC or NE-DC. For NR-DC (nr-SCG), mrdc-SecondaryCellGroup contains the RRCReconfiguration message as generated (entirely) by SN gNB. In this version of the specification, the RRC message can only include fields secondaryCellGroup and measConfig. For NE-DC (eutra-SCG), mrdc-SecondaryCellGroup includes the E-UTRA RRCConnectionReconfiguration message as specified in TS 36.331 [10]. In this version of the specification, the E-UTRA RRC message can only include the field scg-Configuration.

nas-Container This field is used to transfer UE specific NAS layer information between the network and the UE. The RRC layer is transparent for this field, although it affects activation of AS security after inter-system handover to NR. The content is defined in TS 24.501 [23].

nextHopChainingCount Parameter NCC: See TS 33.501 [11]

otherConfig Contains configuration related to other configurations.

radioBearerConfig Configuration of Radio Bearers (DRBs, SRBs) including SDAP/PDCP. In EN-DC this field may only be present if the RRCReconfiguration is transmitted over SRB3.

radioBearerConfig2 Configuration of Radio Bearers (DRBs, SRBs) including SDAP/PDCP. This field can only be used if the UE supports NR-DC or NE-DC.

secondaryCellGroup Configuration of secondary cell group ((NG)EN-DC or NR-DC). This field can only be present in an RRCReconfiguration message is transmitted on SRB3, and in an RRCReconfiguration message contained in another RRCReconfiguration message (or RRCConnectionReconfiguration message, see TS 36.331 [10]) transmitted on SRB1.

sk-Counter A counter used upon initial configuration of S-KgNB or S-KeNB, as well as upon refresh of S-KgNB or S-KeNB. This field is always included either upon initial configuration of an NR SCG or upon configuration of the first RB with keyToUse set to secondary, whichever happens first. This field is absent if there is neither any NR SCG nor any RB with keyToUse set to secondary.

| Conditional Presence | Explanation |
| --- | --- |
| nonHO | The field is absent in case of reconfiguration with sync within NR or to NR; otherwise it is optionally present, need N. |
| securityNASC | This field is mandatory present in case of inter system handover. Otherwise the field is optionally present, need N. |
| MasterKeyChange | This field is mandatory present in case masterCellGroup includes ReconfigurationWithSync and RadioBearerConfig includes SecurityConfig with SecurityAlgorithmConfig, indicating a change of the AS security algorithms associated to the master key. If ReconfigurationWithSync is included for other cases, this field is optionally present, need N. Otherwise the field is absent. |
| FullConfig | The field is mandatory present in case of inter-system handover from E-UTRA/EPC to NR. It is optionally present, Need N, during reconfiguration with sync and also in first reconfiguration after reestablishment; or for intra-system handover from E-UTRA/5GC to NR. It is absent otherwise. |

#### –	RRCReconfigurationComplete

The RRCReconfigurationComplete message is used to confirm the successful completion of an RRC connection reconfiguration.

Signalling radio bearer: SRB1 or SRB3

RLC-SAP: AM

Logical channel: DCCH

Direction: UE to Network

RRCReconfigurationComplete message

-- ASN1START

-- TAG-RRCRECONFIGURATIONCOMPLETE-START

RRCReconfigurationComplete ::=              SEQUENCE {

rrc-TransactionIdentifier                   RRC-TransactionIdentifier,

criticalExtensions                          CHOICE {

rrcReconfigurationComplete                  RRCReconfigurationComplete-IEs,

criticalExtensionsFuture                    SEQUENCE {}

}

}

RRCReconfigurationComplete-IEs ::=          SEQUENCE {

lateNonCriticalExtension                    OCTET STRING                                                            OPTIONAL,

nonCriticalExtension                        RRCReconfigurationComplete-v1530-IEs                                    OPTIONAL

}

RRCReconfigurationComplete-v1530-IEs ::=    SEQUENCE {

uplinkTxDirectCurrentList                   UplinkTxDirectCurrentList                                               OPTIONAL,

nonCriticalExtension                        RRCReconfigurationComplete-v1560-IEs                                    OPTIONAL

}

RRCReconfigurationComplete-v1560-IEs ::=    SEQUENCE {

scg-Response                                CHOICE {

nr-SCG-Response                                 OCTET STRING (CONTAINING RRCReconfigurationComplete),

eutra-SCG-Response                              OCTET STRING

}                                                                                                                   OPTIONAL,

nonCriticalExtension                        SEQUENCE {}                                                             OPTIONAL

}

-- TAG-RRCRECONFIGURATIONCOMPLETE-STOP

-- ASN1STOP

RRCReconfigurationComplete-IEs field descriptions

scg-Response In case of NR-DC (nr-SCG-Response), this field includes the RRCReconfigurationComplete message. In case of NE-DC (eutra-SCG-Response), this field includes the E-UTRA RRCConnectionReconfigurationComplete message as specified in TS 36.331 [10].

uplinkTxDirectCurrentList The Tx Direct Current locations for the configured serving cells and BWPs if requested by the NW (see reportUplinkTxDirectCurrent in CellGroupConfig).

#### –	RRCReject

The RRCReject message is used to reject an RRC connection establishment or an RRC connection resumption.

Signalling radio bearer: SRB0

RLC-SAP: TM

Logical channel: CCCH

Direction: Network to UE

RRCReject message

-- ASN1START

-- TAG-RRCREJECT-START

RRCReject ::=                       SEQUENCE {

criticalExtensions                  CHOICE {

rrcReject                           RRCReject-IEs,

criticalExtensionsFuture            SEQUENCE {}

}

}

RRCReject-IEs ::=                   SEQUENCE {

waitTime                            RejectWaitTime                                                          OPTIONAL,   -- Need N

lateNonCriticalExtension            OCTET STRING                                                            OPTIONAL,

nonCriticalExtension                SEQUENCE{}                                                              OPTIONAL

}

-- TAG-RRCREJECT-STOP

-- ASN1STOP

RRCReject-IEs field descriptions

waitTime Wait time value in seconds. The field is always included.

#### –	RRCRelease

The RRCRelease message is used to command the release of an RRC connection or the suspension of the RRC connection.

Signalling radio bearer: SRB1

RLC-SAP: AM

Logical channel: DCCH

Direction: Network to UE

RRCRelease message

-- ASN1START

-- TAG-RRCRELEASE-START

RRCRelease ::=                      SEQUENCE {

rrc-TransactionIdentifier           RRC-TransactionIdentifier,

criticalExtensions                  CHOICE {

rrcRelease                          RRCRelease-IEs,

criticalExtensionsFuture            SEQUENCE {}

}

}

RRCRelease-IEs ::=                  SEQUENCE {

redirectedCarrierInfo               RedirectedCarrierInfo                                                   OPTIONAL,   -- Need N

cellReselectionPriorities           CellReselectionPriorities                                               OPTIONAL,   -- Need R

suspendConfig                       SuspendConfig                                                           OPTIONAL,   -- Need R

deprioritisationReq                 SEQUENCE {

deprioritisationType                ENUMERATED {frequency, nr},

deprioritisationTimer               ENUMERATED {min5, min10, min15, min30}

}                                                                                                           OPTIONAL,   -- Need N

lateNonCriticalExtension                OCTET STRING                                                        OPTIONAL,

nonCriticalExtension                    RRCRelease-v1540-IEs                                                OPTIONAL

}

RRCRelease-v1540-IEs ::=            SEQUENCE {

waitTime                           RejectWaitTime                OPTIONAL, -- Need N

nonCriticalExtension               SEQUENCE {}                   OPTIONAL

}

RedirectedCarrierInfo ::=           CHOICE {

nr                                  CarrierInfoNR,

eutra                               RedirectedCarrierInfo-EUTRA,

...

}

RedirectedCarrierInfo-EUTRA ::=     SEQUENCE {

eutraFrequency                          ARFCN-ValueEUTRA,

cnType                                  ENUMERATED {epc,fiveGC}                                             OPTIONAL    -- Need N

}

CarrierInfoNR ::=                   SEQUENCE {

carrierFreq                         ARFCN-ValueNR,

ssbSubcarrierSpacing                SubcarrierSpacing,

smtc                                SSB-MTC                                                                 OPTIONAL,      -- Need S

...

}

SuspendConfig ::=                   SEQUENCE {

fullI-RNTI                          I-RNTI-Value,

shortI-RNTI                         ShortI-RNTI-Value,

ran-PagingCycle                     PagingCycle,

ran-NotificationAreaInfo            RAN-NotificationAreaInfo                                                OPTIONAL,   -- Need M

t380                                PeriodicRNAU-TimerValue                                                 OPTIONAL,   -- Need R

nextHopChainingCount                NextHopChainingCount,

...

}

PeriodicRNAU-TimerValue ::=         ENUMERATED { min5, min10, min20, min30, min60, min120, min360, min720}

CellReselectionPriorities ::=       SEQUENCE {

freqPriorityListEUTRA               FreqPriorityListEUTRA                                                   OPTIONAL,       -- Need M

freqPriorityListNR                  FreqPriorityListNR                                                      OPTIONAL,       -- Need M

t320                                ENUMERATED {min5, min10, min20, min30, min60, min120, min180, spare1}   OPTIONAL,       -- Need R

...

}

PagingCycle ::=                     ENUMERATED {rf32, rf64, rf128, rf256}

FreqPriorityListEUTRA ::=           SEQUENCE (SIZE (1..maxFreq)) OF FreqPriorityEUTRA

FreqPriorityListNR ::=              SEQUENCE (SIZE (1..maxFreq)) OF FreqPriorityNR

FreqPriorityEUTRA ::=               SEQUENCE {

carrierFreq                         ARFCN-ValueEUTRA,

cellReselectionPriority             CellReselectionPriority,

cellReselectionSubPriority          CellReselectionSubPriority                                              OPTIONAL        -- Need R

}

FreqPriorityNR ::=                  SEQUENCE {

carrierFreq                         ARFCN-ValueNR,

cellReselectionPriority             CellReselectionPriority,

cellReselectionSubPriority          CellReselectionSubPriority                                              OPTIONAL        -- Need R

}

RAN-NotificationAreaInfo ::=        CHOICE {

cellList                            PLMN-RAN-AreaCellList,

ran-AreaConfigList                  PLMN-RAN-AreaConfigList,

...

}

PLMN-RAN-AreaCellList ::=           SEQUENCE (SIZE (1.. maxPLMNIdentities)) OF PLMN-RAN-AreaCell

PLMN-RAN-AreaCell ::=               SEQUENCE {

plmn-Identity                       PLMN-Identity                                                           OPTIONAL,   -- Need S

ran-AreaCells                       SEQUENCE (SIZE (1..32)) OF  CellIdentity

}

PLMN-RAN-AreaConfigList ::=         SEQUENCE (SIZE (1..maxPLMNIdentities)) OF PLMN-RAN-AreaConfig

PLMN-RAN-AreaConfig ::=             SEQUENCE {

plmn-Identity                       PLMN-Identity                                                           OPTIONAL,   -- Need S

ran-Area                            SEQUENCE (SIZE (1..16)) OF  RAN-AreaConfig

}

RAN-AreaConfig ::=                  SEQUENCE {

trackingAreaCode            TrackingAreaCode,

ran-AreaCodeList            SEQUENCE (SIZE (1..32)) OF  RAN-AreaCode        OPTIONAL    -- Need R

}

-- TAG-RRCRELEASE-STOP

-- ASN1STOP

RRCRelease-IEs field descriptions

cnType Indicate that the UE is redirected to EPC or 5GC.

deprioritisationReq Indicates whether the current frequency or RAT is to be de-prioritised.

deprioritisationTimer Indicates the period for which either the current carrier frequency or NR is deprioritised. Value minN corresponds to N minutes.

suspendConfig Indicates configuration for the RRC_INACTIVE state. The network does not configure suspendConfig when the network redirect the UE to an inter-RAT carrier frequency.

redirectedCarrierInfo Indicates a carrier frequency (downlink for FDD) and is used to redirect the UE to an NR or an inter-RAT carrier frequency, by means of cell selection at transition to RRC_IDLE or RRC_INACTIVE as specified in TS 38.304 [20]. In this release of specification, redirectedCarrierInfo is not included in an RRCRelease message with suspendConfig if this message is in response to an RRCResumeRequest or an RRCResumeRequest1 which is triggered by the NAS layer.

CarrierInfoNR field descriptions

carrierFreq Indicates the redirected NR frequency.

ssbSubcarrierSpacing Subcarrier spacing of SSB in the redirected SSB frequency. Only the values 15 kHz or 30 kHz (FR1), and 120 kHz or 240 kHz (FR2) are applicable.

smtc The SSB periodicity/offset/duration configuration for the redirected SSB frequency. It is based on timing reference of PCell. If the field is absent, the UE uses the SMTC configured in the measObjectNR having the same SSB frequency and subcarrier spacing.

RAN-NotificationAreaInfo field descriptions

cellList A list of cells configured as RAN area.

ran-AreaConfigList A list of RAN area codes or RA code(s) as RAN area.

PLMN-RAN-AreaConfig field descriptions

plmn-Identity PLMN Identity to which the cells in ran-Area belong. If the field is absent the UE uses the ID of the registered PLMN.

ran-AreaCodeList The total number of RAN-AreaCodes of all PLMNs does not exceed 32.

ran-Area Indicates whether TA code(s) or RAN area code(s) are used for the RAN notification area. The network uses only TA code(s) or both TA code(s) and RAN area code(s) to configure a UE. The total number of TACs across all PLMNs does not exceed 16.

PLMN-RAN-AreaCell field descriptions

plmn-Identity PLMN Identity to which the cells in ran-AreaCells belong. If the field is absent the UE uses the ID of the registered PLMN.

ran-AreaCells The total number of cells of all PLMNs does not exceed 32.

SuspendConfig field descriptions

ran-NotificationAreaInfo Network ensures that the UE in RRC_INACTIVE always has a valid ran-NotificationAreaInfo.

ran-PagingCycle Refers to the UE specific cycle for RAN-initiated paging. Value rf32 corresponds to 32 radio frames, value rf64 corresponds to 64 radio frames and so on.

t380 Refers to the timer that triggers the periodic RNAU procedure in UE. Value min5 corresponds to 5 minutes, value min10 corresponds to 10 minutes and so on.

#### –	RRCResume

The RRCResume message is used to resume the suspended RRC connection.

Signalling radio bearer: SRB1

RLC-SAP: AM

Logical channel: DCCH

Direction: Network to UE

RRCResume message

-- ASN1START

-- TAG-RRCRESUME-START

RRCResume ::=                       SEQUENCE {

rrc-TransactionIdentifier           RRC-TransactionIdentifier,

criticalExtensions                  CHOICE {

rrcResume                           RRCResume-IEs,

criticalExtensionsFuture            SEQUENCE {}

}

}

RRCResume-IEs ::=                   SEQUENCE {

radioBearerConfig                   RadioBearerConfig                                                       OPTIONAL, -- Need M

masterCellGroup                     OCTET STRING (CONTAINING CellGroupConfig)                               OPTIONAL, -- Need M

measConfig                          MeasConfig                                                              OPTIONAL, -- Need M

fullConfig                          ENUMERATED {true}                                                       OPTIONAL, -- Need N

lateNonCriticalExtension            OCTET STRING                                                            OPTIONAL,

nonCriticalExtension                RRCResume-v1560-IEs                                                     OPTIONAL

}

RRCResume-v1560-IEs ::=             SEQUENCE {

radioBearerConfig2                  OCTET STRING (CONTAINING RadioBearerConfig)                             OPTIONAL, -- Need M

sk-Counter                          SK-Counter                                                              OPTIONAL, -- Need N

nonCriticalExtension                SEQUENCE{}                                                              OPTIONAL

}

-- TAG-RRCRESUME-STOP

-- ASN1STOP

RRCResume-IEs field descriptions

masterCellGroup Configuration of the master cell group.

radioBearerConfig Configuration of Radio Bearers (DRBs, SRBs) including SDAP/PDCP.

radioBearerConfig2 Configuration of Radio Bearers (DRBs, SRBs) including SDAP/PDCP. This field can only be used if the UE supports NR-DC or NE-DC.

sk-Counter A counter used to derive S-KgNB or S-KeNB based on the newly derived KgNB during RRC Resume. The field is only included when there is one or more RB with keyToUse set to secondary.

#### –	RRCResumeComplete

The RRCResumeComplete message is used to confirm the successful completion of an RRC connection resumption.

Signalling radio bearer: SRB1

RLC-SAP: AM

Logical channel: DCCH

Direction: UE to Network

RRCResumeComplete message

-- ASN1START

-- TAG-RRCRESUMECOMPLETE-START

RRCResumeComplete ::=                   SEQUENCE {

rrc-TransactionIdentifier               RRC-TransactionIdentifier,

criticalExtensions                      CHOICE {

rrcResumeComplete                       RRCResumeComplete-IEs,

criticalExtensionsFuture                SEQUENCE {}

}

}

RRCResumeComplete-IEs ::=               SEQUENCE {

dedicatedNAS-Message                    DedicatedNAS-Message                                                    OPTIONAL,

selectedPLMN-Identity                   INTEGER (1..maxPLMN)                                                    OPTIONAL,

uplinkTxDirectCurrentList               UplinkTxDirectCurrentList                                               OPTIONAL,

lateNonCriticalExtension                OCTET STRING                                                            OPTIONAL,

nonCriticalExtension                    SEQUENCE{}                                                              OPTIONAL

}

-- TAG-RRCRESUMECOMPLETE-STOP

-- ASN1STOP

RRCResumeComplete-IEs field descriptions

selectedPLMN-Identity Index of the PLMN selected by the UE from the plmn-IdentityList fields included in SIB1.

uplinkTxDirectCurrentList The Tx Direct Current locations for the configured serving cells and BWPs if requested by the NW (see reportUplinkTxDirectCurrent in CellGroupConfig).

#### –	RRCResumeRequest

The RRCResumeRequest message is used to request the resumption of a suspended RRC connection or perform an RNA update.

Signalling radio bearer: SRB0

RLC-SAP: TM

Logical channel: CCCH

Direction: UE to Network

RRCResumeRequest message

-- ASN1START

-- TAG-RRCRESUMEREQUEST-START

RRCResumeRequest ::=            SEQUENCE {

rrcResumeRequest            RRCResumeRequest-IEs

}

RRCResumeRequest-IEs ::=        SEQUENCE {

resumeIdentity                  ShortI-RNTI-Value,

resumeMAC-I                     BIT STRING (SIZE (16)),

resumeCause                     ResumeCause,

spare                           BIT STRING (SIZE (1))

}

-- TAG-RRCRESUMEREQUEST-STOP

-- ASN1STOP

RRCResumeRequest-IEs field descriptions

resumeCause Provides the resume cause for the RRC connection resume request as provided by the upper layers or RRC. The network is not expected to reject an RRCResumeRequest due to unknown cause value being used by the UE.

resumeIdentity UE identity to facilitate UE context retrieval at gNB.

resumeMAC-I Authentication token to facilitate UE authentication at gNB. The 16 least significant bits of the MAC-I calculated using the AS security configuration as specified in 5.3.13.3.

#### –	RRCResumeRequest1

The RRCResumeRequest1 message is used to request the resumption of a suspended RRC connection or perform an RNA update.

Signalling radio bearer: SRB0

RLC-SAP: TM

Logical channel: CCCH1

Direction: UE to Network

RRCResumeRequest1 message

-- ASN1START

-- TAG-RRCRESUMEREQUEST1-START

RRCResumeRequest1 ::= SEQUENCE {

rrcResumeRequest1      RRCResumeRequest1-IEs

}

RRCResumeRequest1-IEs ::=    SEQUENCE {

resumeIdentity               I-RNTI-Value,

resumeMAC-I                  BIT STRING (SIZE (16)),

resumeCause                  ResumeCause,

spare                        BIT STRING (SIZE (1))

}

-- TAG-RRCRESUMEREQUEST1-STOP

-- ASN1STOP

RRCResumeRequest1-IEs field descriptions

resumeCause Provides the resume cause for the RRCResumeRequest1 as provided by the upper layers or RRC. A gNB is not expected to reject an RRCResumeRequest1 due to unknown cause value being used by the UE.

resumeIdentity UE identity to facilitate UE context retrieval at gNB.

resumeMAC-I Authentication token to facilitate UE authentication at gNB. The 16 least significant bits of the MAC-I calculated using the AS security configuration as specified in 5.3.13.3.

#### –	RRCSetup

The RRCSetup message is used to establish SRB1.

Signalling radio bearer: SRB0

RLC-SAP: TM

Logical channel: CCCH

Direction: Network to UE

RRCSetup message

-- ASN1START

-- TAG-RRCSETUP-START

RRCSetup ::=                        SEQUENCE {

rrc-TransactionIdentifier           RRC-TransactionIdentifier,

criticalExtensions                  CHOICE {

rrcSetup                            RRCSetup-IEs,

criticalExtensionsFuture            SEQUENCE {}

}

}

RRCSetup-IEs ::=                    SEQUENCE {

radioBearerConfig                   RadioBearerConfig,

masterCellGroup                     OCTET STRING (CONTAINING CellGroupConfig),

lateNonCriticalExtension            OCTET STRING                                                            OPTIONAL,

nonCriticalExtension                SEQUENCE{}                                                              OPTIONAL

}

-- TAG-RRCSETUP-STOP

-- ASN1STOP

RRCSetup-IEs field descriptions

masterCellGroup The network configures only the RLC bearer for the SRB1, mac-CellGroupConfig, physicalCellGroupConfig and spCellConfig.

radioBearerConfig Only SRB1 can be configured in RRC setup.

#### –	RRCSetupComplete

The RRCSetupComplete message is used to confirm the successful completion of an RRC connection establishment.

Signalling radio bearer: SRB1

RLC-SAP: AM

Logical channel: DCCH

Direction: UE to Network

RRCSetupComplete message

-- ASN1START

-- TAG-RRCSETUPCOMPLETE-START

RRCSetupComplete ::=                SEQUENCE {

rrc-TransactionIdentifier           RRC-TransactionIdentifier,

criticalExtensions                  CHOICE {

rrcSetupComplete                    RRCSetupComplete-IEs,

criticalExtensionsFuture            SEQUENCE {}

}

}

RRCSetupComplete-IEs ::=            SEQUENCE {

selectedPLMN-Identity               INTEGER (1..maxPLMN),

registeredAMF                       RegisteredAMF                                   OPTIONAL,

guami-Type                          ENUMERATED {native, mapped}                     OPTIONAL,

s-NSSAI-List                        SEQUENCE (SIZE (1..maxNrofS-NSSAI)) OF S-NSSAI  OPTIONAL,

dedicatedNAS-Message                DedicatedNAS-Message,

ng-5G-S-TMSI-Value                  CHOICE {

ng-5G-S-TMSI                        NG-5G-S-TMSI,

ng-5G-S-TMSI-Part2                  BIT STRING (SIZE (9))

}                                                                                   OPTIONAL,

lateNonCriticalExtension            OCTET STRING                                    OPTIONAL,

nonCriticalExtension                SEQUENCE{}                                      OPTIONAL

}

RegisteredAMF ::=                   SEQUENCE {

plmn-Identity                       PLMN-Identity                                   OPTIONAL,

amf-Identifier                      AMF-Identifier

}

-- TAG-RRCSETUPCOMPLETE-STOP

-- ASN1STOP

RRCSetupComplete-IEs field descriptions

guami-Type This field is used to indicate whether the GUAMI included is native (derived from native 5G-GUTI) or mapped (from EPS, derived from EPS GUTI) as specified in TS 24.501 [23].

ng-5G-S-TMSI-Part2 The leftmost 9 bits of 5G-S-TMSI.

registeredAMF This field is used to transfer the GUAMI of the AMF where the UE is registered, as provided by upper layers, see TS 23.003 [21].

selectedPLMN-Identity Index of the PLMN selected by the UE from the plmn-IdentityList fields included in SIB1.

#### –	RRCSetupRequest

The RRCSetupRequest message is used to request the establishment of an RRC connection.

Signalling radio bearer: SRB0

RLC-SAP: TM

Logical channel: CCCH

Direction: UE to Network

RRCSetupRequest message

-- ASN1START

-- TAG-RRCSETUPREQUEST-START

RRCSetupRequest ::=                 SEQUENCE {

rrcSetupRequest                     RRCSetupRequest-IEs

}

RRCSetupRequest-IEs ::=             SEQUENCE {

ue-Identity                         InitialUE-Identity,

establishmentCause                  EstablishmentCause,

spare                               BIT STRING (SIZE (1))

}

InitialUE-Identity ::=              CHOICE {

ng-5G-S-TMSI-Part1                  BIT STRING (SIZE (39)),

randomValue                         BIT STRING (SIZE (39))

}

EstablishmentCause ::=              ENUMERATED {

emergency, highPriorityAccess, mt-Access, mo-Signalling,

mo-Data, mo-VoiceCall, mo-VideoCall, mo-SMS, mps-PriorityAccess, mcs-PriorityAccess,

spare6, spare5, spare4, spare3, spare2, spare1}

-- TAG-RRCSETUPREQUEST-STOP

-- ASN1STOP

RRCSetupRequest-IEs field descriptions

establishmentCause Provides the establishment cause for the RRCSetupRequest in accordance with the information received from upper layers. gNB is not expected to reject an RRCSetupRequest due to unknown cause value being used by the UE.

ue-Identity UE identity included to facilitate contention resolution by lower layers.

InitialUE-Identity field descriptions

ng-5G-S-TMSI-Part1 The rightmost 39 bits of 5G-S-TMSI.

randomValue Integer value in the range 0 to 239 – 1.

#### –	RRCSystemInfoRequest

The RRCSystemInfoRequest message is used to request SI message(s) required by the UE as specified in clause 5.2.2.3.3.

Signalling radio bearer: SRB0

RLC-SAP: TM

Logical channel: CCCH

Direction: UE to Network

RRCSystemInfoRequest message

-- ASN1START

-- TAG-RRCSYSTEMINFOREQUEST-START

RRCSystemInfoRequest ::=            SEQUENCE {

criticalExtensions                  CHOICE {

rrcSystemInfoRequest                RRCSystemInfoRequest-IEs,

criticalExtensionsFuture            SEQUENCE {}

}

}

RRCSystemInfoRequest-IEs ::=    SEQUENCE {

requested-SI-List                   BIT STRING (SIZE (maxSI-Message)),  --32bits

spare                               BIT STRING (SIZE (12))

}

-- TAG-RRCSYSTEMINFOREQUEST-STOP

-- ASN1STOP

RRCSystemInfoRequest-IEs field descriptions

requested-SI-List Contains a list of requested SI messages. According to the order of entry in the list of SI messages configured by schedulingInfoList in si-SchedulingInfo in SIB1, first bit corresponds to first/leftmost listed SI message, second bit corresponds to second listed SI message, and so on.

#### –	SCGFailureInformation

The SCGFailureInformation message is used to provide information regarding NR SCG failures detected by the UE.

Signalling radio bearer: SRB1

RLC-SAP: AM

Logical channel: DCCH

Direction: UE to Network

SCGFailureInformation message

-- ASN1START

-- TAG-SCGFAILUREINFORMATION-START

SCGFailureInformation ::=                 SEQUENCE {

criticalExtensions                         CHOICE {

scgFailureInformation                     SCGFailureInformation-IEs,

criticalExtensionsFuture                 SEQUENCE {}

}

}

SCGFailureInformation-IEs ::=             SEQUENCE {

failureReportSCG                              FailureReportSCG                       OPTIONAL,

nonCriticalExtension                         SCGFailureInformation-v1590-IEs   OPTIONAL

}

SCGFailureInformation-v1590-IEs ::=  SEQUENCE {

lateNonCriticalExtension              OCTET STRING              OPTIONAL,

nonCriticalExtension                  SEQUENCE {}               OPTIONAL

}

FailureReportSCG ::=                       SEQUENCE {

failureType                                    ENUMERATED {

t310-Expiry, randomAccessProblem,

rlc-MaxNumRetx,

synchReconfigFailureSCG, scg-ReconfigFailure,

srb3-IntegrityFailure,  spare2, spare1},

measResultFreqList                           MeasResultFreqList                       OPTIONAL,

measResultSCG-Failure                       OCTET STRING (CONTAINING MeasResultSCG-Failure)                              OPTIONAL,

...

}

MeasResultFreqList ::=               SEQUENCE (SIZE (1..maxFreq)) OF MeasResult2NR

-- TAG-SCGFAILUREINFORMATION-STOP

-- ASN1STOP

SCGFailureInformation field descriptions

measResultFreqList The field contains available results of measurements on NR frequencies the UE is configured to measure by measConfig.

measResultSCG-Failure The field contains the MeasResultSCG-Failure IE which includes available results of measurements on NR frequencies the UE is configured to measure by the NR SCG RRCReconfiguration message.

#### –	SCGFailureInformationEUTRA

The SCGFailureInformationEUTRA message is used to provide information regarding E-UTRA SCG failures detected by the UE.

Signalling radio bearer: SRB1

RLC-SAP: AM

Logical channel: DCCH

Direction: UE to Network

SCGFailureInformationEUTRA message

-- ASN1START

-- TAG-SCGFAILUREINFORMATIONEUTRA-START

SCGFailureInformationEUTRA ::=                SEQUENCE {

criticalExtensions                                CHOICE {

scgFailureInformationEUTRA                       SCGFailureInformationEUTRA-IEs,

criticalExtensionsFuture                          SEQUENCE {}

}

}

SCGFailureInformationEUTRA-IEs ::=           SEQUENCE {

failureReportSCG-EUTRA                           FailureReportSCG-EUTRA                      OPTIONAL,

nonCriticalExtension                              SCGFailureInformationEUTRA-v1590-IEs   OPTIONAL

}

SCGFailureInformationEUTRA-v1590-IEs ::=  SEQUENCE {

lateNonCriticalExtension                  OCTET STRING            OPTIONAL,

nonCriticalExtension                      SEQUENCE {}             OPTIONAL

}

FailureReportSCG-EUTRA ::=                     SEQUENCE {

failureType                                          ENUMERATED {

t313-Expiry, randomAccessProblem,

rlc-MaxNumRetx, scg-ChangeFailure, spare4,

spare3, spare2, spare1},

measResultFreqListMRDC                            MeasResultFreqListFailMRDC                OPTIONAL,

measResultSCG-FailureMRDC                        OCTET STRING                          OPTIONAL,

...

}

MeasResultFreqListFailMRDC ::=      SEQUENCE (SIZE (1.. maxFreq)) OF MeasResult2EUTRA

-- TAG-SCGFAILUREINFORMATIONEUTRA-STOP

-- ASN1STOP

SCGFailureInformationEUTRA field descriptions

measResultFreqListMRDC The field contains available results of measurements on E-UTRA frequencies the UE is configured to measure by measConfig.

measResultSCG-FailureMRDC Includes the E-UTRA MeasResultSCG-FailureMRDC IE as specified in TS 36.331 [10]. The field contains available results of measurements on E-UTRA frequencies the UE is configured to measure by the E-UTRA RRCConnectionReconfiguration message.

#### –	SecurityModeCommand

The SecurityModeCommand message is used to command the activation of AS security.

Signalling radio bearer: SRB1

RLC-SAP: AM

Logical channel: DCCH

Direction: Network to UE

SecurityModeCommand message

-- ASN1START

-- TAG-SECURITYMODECOMMAND-START

SecurityModeCommand ::=             SEQUENCE {

rrc-TransactionIdentifier           RRC-TransactionIdentifier,

criticalExtensions                  CHOICE {

securityModeCommand                 SecurityModeCommand-IEs,

criticalExtensionsFuture            SEQUENCE {}

}

}

SecurityModeCommand-IEs ::=         SEQUENCE {

securityConfigSMC                   SecurityConfigSMC,

lateNonCriticalExtension            OCTET STRING                                                            OPTIONAL,

nonCriticalExtension                SEQUENCE{}                                                              OPTIONAL

}

SecurityConfigSMC ::=               SEQUENCE {

securityAlgorithmConfig             SecurityAlgorithmConfig,

...

}

-- TAG-SECURITYMODECOMMAND-STOP

-- ASN1STOP

#### –	SecurityModeComplete

The SecurityModeComplete message is used to confirm the successful completion of a security mode command.

Signalling radio bearer: SRB1

RLC-SAP: AM

Logical channel: DCCH

Direction: UE to Network

SecurityModeComplete message

-- ASN1START

-- TAG-SECURITYMODECOMPLETE-START

SecurityModeComplete ::=            SEQUENCE {

rrc-TransactionIdentifier           RRC-TransactionIdentifier,

criticalExtensions                  CHOICE {

securityModeComplete                SecurityModeComplete-IEs,

criticalExtensionsFuture            SEQUENCE {}

}

}

SecurityModeComplete-IEs ::=        SEQUENCE {

lateNonCriticalExtension            OCTET STRING                                                            OPTIONAL,

nonCriticalExtension                SEQUENCE{}                                                              OPTIONAL

}

-- TAG-SECURITYMODECOMPLETE-STOP

-- ASN1STOP

#### –	SecurityModeFailure

The SecurityModeFailure message is used to indicate an unsuccessful completion of a security mode command.

Signalling radio bearer: SRB1

RLC-SAP: AM

Logical channel: DCCH

Direction: UE to Network

SecurityModeFailure message

-- ASN1START

-- TAG-SECURITYMODEFAILURE-START

SecurityModeFailure ::=             SEQUENCE {

rrc-TransactionIdentifier           RRC-TransactionIdentifier,

criticalExtensions                  CHOICE {

securityModeFailure                 SecurityModeFailure-IEs,

criticalExtensionsFuture            SEQUENCE {}

}

}

SecurityModeFailure-IEs ::=         SEQUENCE {

lateNonCriticalExtension            OCTET STRING                                                            OPTIONAL,

nonCriticalExtension                SEQUENCE{}                                                              OPTIONAL

}

-- TAG-SECURITYMODEFAILURE-STOP

-- ASN1STOP

#### –	SIB1

SIB1 contains information relevant when evaluating if a UE is allowed to access a cell and defines the scheduling of other system information. It also contains radio resource configuration information that is common for all UEs and barring information applied to the unified access control.

Signalling radio bearer: N/A

RLC-SAP: TM

Logical channels: BCCH

Direction: Network to UE

SIB1 message

-- ASN1START

-- TAG-SIB1-START

SIB1 ::=        SEQUENCE {

cellSelectionInfo                   SEQUENCE {

q-RxLevMin                          Q-RxLevMin,

q-RxLevMinOffset                    INTEGER (1..8)                                              OPTIONAL,   -- Need S

q-RxLevMinSUL                       Q-RxLevMin                                                  OPTIONAL,   -- Need R

q-QualMin                           Q-QualMin                                                   OPTIONAL,   -- Need S

q-QualMinOffset                     INTEGER (1..8)                                              OPTIONAL    -- Need S

}                                                                                                   OPTIONAL,   -- Cond Standalone

cellAccessRelatedInfo               CellAccessRelatedInfo,

connEstFailureControl               ConnEstFailureControl                                           OPTIONAL,   -- Need R

si-SchedulingInfo                   SI-SchedulingInfo                                               OPTIONAL,   -- Need R

servingCellConfigCommon             ServingCellConfigCommonSIB                                      OPTIONAL,   -- Need R

ims-EmergencySupport                ENUMERATED {true}                                               OPTIONAL,   -- Need R

eCallOverIMS-Support                ENUMERATED {true}                                               OPTIONAL,   -- Cond Absent

ue-TimersAndConstants               UE-TimersAndConstants                                           OPTIONAL,   -- Need R

uac-BarringInfo                     SEQUENCE {

uac-BarringForCommon                UAC-BarringPerCatList                                       OPTIONAL,   -- Need S

uac-BarringPerPLMN-List             UAC-BarringPerPLMN-List                                     OPTIONAL,   -- Need S

uac-BarringInfoSetList              UAC-BarringInfoSetList,

uac-AccessCategory1-SelectionAssistanceInfo CHOICE {

plmnCommon                           UAC-AccessCategory1-SelectionAssistanceInfo,

individualPLMNList                   SEQUENCE (SIZE (2..maxPLMN)) OF UAC-AccessCategory1-SelectionAssistanceInfo

}                                                                                               OPTIONAL    -- Need S

}                                                                                                   OPTIONAL,   -- Need R

useFullResumeID                     ENUMERATED {true}                                               OPTIONAL,   -- Need R

lateNonCriticalExtension            OCTET STRING                                                    OPTIONAL,

nonCriticalExtension                SEQUENCE{}                                                      OPTIONAL

}

UAC-AccessCategory1-SelectionAssistanceInfo ::=    ENUMERATED {a, b, c}

-- TAG-SIB1-STOP

-- ASN1STOP

SIB1 field descriptions

cellSelectionInfo Parameters for cell selection related to the serving cell.

ims-EmergencySupport Indicates whether the cell supports IMS emergency bearer services for UEs in limited service mode. If absent, IMS emergency call is not supported by the network in the cell for UEs in limited service mode.

q-QualMin Parameter "Qqualmin" in TS 38.304 [20], applicable for serving cell. If the field is absent, the UE applies the (default) value of negative infinity for Qqualmin.

q-QualMinOffset Parameter "Qqualminoffset" in TS 38.304 [20]. Actual value Qqualminoffset = field value [dB]. If the field is absent, the UE applies the (default) value of 0 dB for Qqualminoffset. Affects the minimum required quality level in the cell.

q-RxLevMin Parameter "Qrxlevmin" in TS 38.304 [20], applicable for serving cell.

q-RxLevMinOffset Parameter "Qrxlevminoffset" in TS 38.304 [20]. Actual value Qrxlevminoffset = field value * 2 [dB]. If absent, the UE applies the (default) value of 0 dB for Qrxlevminoffset. Affects the minimum required Rx level in the cell.

q-RxLevMinSUL Parameter "Qrxlevmin" in TS 38.304 [20], applicable for serving cell.

servingCellConfigCommon Configuration of the serving cell.

uac-AccessCategory1-SelectionAssistanceInfo Information used to determine whether Access Category 1 applies to the UE, as defined in TS 22.261 [25].

uac-BarringForCommon Common access control parameters for each access category. Common values are used for all PLMNs, unless overwritten by the PLMN specific configuration provided in uac-BarringPerPLMN-List. The parameters are specified by providing an index to the set of configurations (uac-BarringInfoSetList). UE behaviour upon absence of this field is specified in clause 5.3.14.2.

ue-TimersAndConstants Timer and constant values to be used by the UE. The cell operating as PCell always provides this field.

useFullResumeID Indicates which resume identifier and Resume request message should be used. UE uses fullI-RNTI and RRCResumeRequest1 if the field is present, or shortI-RNTI and RRCResumeRequest if the field is absent.

| Conditional Presence | Explanation |
| --- | --- |
| Absent | The field is not used in this version of the specification, if received the UE shall ignore. |
| Standalone | The field is mandatory present in a cell that supports standalone operation, otherwise it is absent. |

#### –	SystemInformation

The SystemInformation message is used to convey one or more System Information Blocks. All the SIBs included are transmitted with the same periodicity.

Signalling radio bearer: N/A

RLC-SAP: TM

Logical channels: BCCH

Direction: Network to UE

SystemInformation message

-- ASN1START

-- TAG-SYSTEMINFORMATION-START

SystemInformation ::=               SEQUENCE {

criticalExtensions                  CHOICE {

systemInformation                   SystemInformation-IEs,

criticalExtensionsFuture            SEQUENCE {}

}

}

SystemInformation-IEs ::=           SEQUENCE {

sib-TypeAndInfo                     SEQUENCE (SIZE (1..maxSIB)) OF CHOICE {

sib2                                SIB2,

sib3                                SIB3,

sib4                                SIB4,

sib5                                SIB5,

sib6                                SIB6,

sib7                                SIB7,

sib8                                SIB8,

sib9                                SIB9,

...

},

lateNonCriticalExtension            OCTET STRING                        OPTIONAL,

nonCriticalExtension                SEQUENCE {}                         OPTIONAL

}

-- TAG-SYSTEMINFORMATION-STOP

-- ASN1STOP

#### –	UEAssistanceInformation

The UEAssistanceInformation message is used for the indication of UE assistance information to the network.

Signalling radio bearer: SRB1

RLC-SAP: AM

Logical channel: DCCH

Direction: UE to Network

UEAssistanceInformation message

-- ASN1START

-- TAG-UEASSISTANCEINFORMATION-START

UEAssistanceInformation ::=         SEQUENCE {

criticalExtensions                  CHOICE {

ueAssistanceInformation             UEAssistanceInformation-IEs,

criticalExtensionsFuture            SEQUENCE {}

}

}

UEAssistanceInformation-IEs ::=     SEQUENCE {

delayBudgetReport                   DelayBudgetReport                   OPTIONAL,

lateNonCriticalExtension            OCTET STRING                        OPTIONAL,

nonCriticalExtension                UEAssistanceInformation-v1540-IEs   OPTIONAL

}

DelayBudgetReport::=                CHOICE {

type1                               ENUMERATED {

msMinus1280, msMinus640, msMinus320, msMinus160,msMinus80, msMinus60, msMinus40,

msMinus20, ms0, ms20,ms40, ms60, ms80, ms160, ms320, ms640, ms1280},

...

}

UEAssistanceInformation-v1540-IEs ::= SEQUENCE {

overheatingAssistance               OverheatingAssistance               OPTIONAL,

nonCriticalExtension                SEQUENCE {}                         OPTIONAL

}

OverheatingAssistance ::=           SEQUENCE {

reducedMaxCCs                       SEQUENCE {

reducedCCsDL                        INTEGER (0..31),

reducedCCsUL                        INTEGER (0..31)

} OPTIONAL,

reducedMaxBW-FR1                    SEQUENCE {

reducedBW-FR1-DL                    ReducedAggregatedBandwidth,

reducedBW-FR1-UL                    ReducedAggregatedBandwidth

} OPTIONAL,

reducedMaxBW-FR2                    SEQUENCE {

reducedBW-FR2-DL                    ReducedAggregatedBandwidth,

reducedBW-FR2-UL                    ReducedAggregatedBandwidth

} OPTIONAL,

reducedMaxMIMO-LayersFR1            SEQUENCE {

reducedMIMO-LayersFR1-DL            MIMO-LayersDL,

reducedMIMO-LayersFR1-UL            MIMO-LayersUL

} OPTIONAL,

reducedMaxMIMO-LayersFR2            SEQUENCE {

reducedMIMO-LayersFR2-DL            MIMO-LayersDL,

reducedMIMO-LayersFR2-UL            MIMO-LayersUL

} OPTIONAL

}

ReducedAggregatedBandwidth ::= ENUMERATED {mhz0, mhz10, mhz20, mhz30, mhz40, mhz50, mhz60, mhz80, mhz100, mhz200, mhz300, mhz400}

-- TAG-UEASSISTANCEINFORMATION-STOP

-- ASN1STOP

UEAssistanceInformation field descriptions

delayBudgetReport Indicates the UE-preferred adjustment to connected mode DRX.

reducedBW-FR1-DL Indicates the UE's preference on reduced configuration corresponding to the maximum aggregated bandwidth across all downlink carrier(s) of FR1 indicated by the field, to address overheating. This field is allowed to be reported only when UE is configured with serving cell(s) operating on FR1. This maximum aggregated bandwidth includes downlink carrier(s) of FR1 of both the MCG and the SCG. Value mhz0 is not used. The aggregated bandwidth across all downlink carrier(s) of FR1 is the sum of bandwidth of active downlink BWP(s) across all activated downlink carrier(s) of FR1.

reducedBW-FR1-UL Indicates the UE's preference on reduced configuration corresponding to the maximum aggregated bandwidth across all uplink carrier(s) of FR1 indicated by the field, to address overheating. This field is allowed to be reported only when UE is configured with serving cell(s) operating on FR1. This maximum aggregated bandwidth includes uplink carrier(s) of FR1 of both the MCG and the SCG. Value mhz0 is not used. The aggregated bandwidth across all uplink carrier(s) of FR1 is the sum of bandwidth of active uplink BWP(s) across all activated uplink carrier(s) of FR1.

reducedBW-FR2-DL Indicates the UE's preference on reduced configuration corresponding to the maximum aggregated bandwidth across all downlink carrier(s) of FR2 indicated by the field, to address overheating. This field is allowed to be reported only when UE is configured with serving cell(s) operating on FR2. This maximum aggregated bandwidth includes downlink carrier(s) of FR2 of both the MCG and the NR SCG. The aggregated bandwidth across all downlink carrier(s) of FR2 is the sum of bandwidth of active downlink BWP(s) across all activated downlink carrier(s) of FR2.

reducedBW-FR2-UL Indicates the UE's preference on reduced configuration corresponding to the maximum aggregated bandwidth across all uplink carrier(s) of FR2 indicated by the field, to address overheating. This field is allowed to be reported only when UE is configured with serving cell(s) operating on FR2. This maximum aggregated bandwidth includes uplink carrier(s) of FR2 of both the MCG and the NR SCG. The aggregated bandwidth across all uplink carrier(s) of FR2 is the sum of bandwidth of active uplink BWP(s) across all activated uplink carrier(s) of FR2.

reducedCCsDL Indicates the UE's preference on reduced configuration corresponding to the maximum number of downlink SCells indicated by the field, to address overheating. This maximum number includes both SCells of the MCG and PSCell/SCells of the SCG.

reducedCCsUL Indicates the UE's preference on reduced configuration corresponding to the maximum number of uplink SCells indicated by the field, to address overheating. This maximum number includes both SCells of the MCG and PSCell/SCells of the SCG.

reducedMIMO-LayersFR1-DL Indicates the UE's preference on reduced configuration corresponding to the maximum number of downlink MIMO layers of each serving cell operating on FR1 indicated by the field, to address overheating. This field is allowed to be reported only when UE is configured with serving cells operating on FR1.

reducedMIMO-LayersFR1-UL Indicates the UE's preference on reduced configuration corresponding to the maximum number of uplink MIMO layers of each serving cell operating on FR1 indicated by the field, to address overheating. This field is allowed to be reported only when UE is configured with serving cells operating on FR1.

reducedMIMO-LayersFR2-DL Indicates the UE's preference on reduced configuration corresponding to the maximum number of downlink MIMO layers of each serving cell operating on FR2 indicated by the field, to address overheating. This field is allowed to be reported only when UE is configured with serving cells operating on FR2.

reducedMIMO-LayersFR2-UL Indicates the UE's preference on reduced configuration corresponding to the maximum number of uplink MIMO layers of each serving cell operating on FR2 indicated by the field, to address overheating. This field is allowed to be reported only when UE is configured with serving cells operating on FR2.

type1 Indicates the preferred amount of increment/decrement to the long DRX cycle length with respect to the current configuration. Value in number of milliseconds. Value ms40 corresponds to 40 milliseconds, msMinus40 corresponds to -40 milliseconds and so on.

#### –	UECapabilityEnquiry

The UECapabilityEnquiry message is used to request UE radio access capabilities for NR as well as for other RATs.

Signalling radio bearer: SRB1

RLC-SAP: AM

Logical channel: DCCH

Direction: Network to UE

UECapabilityEnquiry information element

-- ASN1START

-- TAG-UECAPABILITYENQUIRY-START

UECapabilityEnquiry ::=         SEQUENCE {

rrc-TransactionIdentifier           RRC-TransactionIdentifier,

criticalExtensions                  CHOICE {

ueCapabilityEnquiry                 UECapabilityEnquiry-IEs,

criticalExtensionsFuture            SEQUENCE {}

}

}

UECapabilityEnquiry-IEs ::=     SEQUENCE {

ue-CapabilityRAT-RequestList        UE-CapabilityRAT-RequestList,

lateNonCriticalExtension            OCTET STRING                                                            OPTIONAL,

ue-CapabilityEnquiryExt             OCTET STRING (CONTAINING UECapabilityEnquiry-v1560-IEs)                 OPTIONAL

}

UECapabilityEnquiry-v1560-IEs ::=   SEQUENCE {

capabilityRequestFilterCommon       UE-CapabilityRequestFilterCommon                                        OPTIONAL, -- Need N

nonCriticalExtension                SEQUENCE{}                                                              OPTIONAL

}

-- TAG-UECAPABILITYENQUIRY-STOP

-- ASN1STOP

#### –	UECapabilityInformation

The IE UECapabilityInformation message is used to transfer UE radio access capabilities requested by the network.

Signalling radio bearer: SRB1

RLC-SAP: AM

Logical channel: DCCH

Direction: UE to Network

UECapabilityInformation information element

-- ASN1START

-- TAG-UECAPABILITYINFORMATION-START

UECapabilityInformation ::=         SEQUENCE {

rrc-TransactionIdentifier           RRC-TransactionIdentifier,

criticalExtensions                  CHOICE {

ueCapabilityInformation             UECapabilityInformation-IEs,

criticalExtensionsFuture            SEQUENCE {}

}

}

UECapabilityInformation-IEs ::=     SEQUENCE {

ue-CapabilityRAT-ContainerList      UE-CapabilityRAT-ContainerList                                          OPTIONAL,

lateNonCriticalExtension            OCTET STRING                                                            OPTIONAL,

nonCriticalExtension                SEQUENCE{}                                                              OPTIONAL

}

-- TAG-UECAPABILITYINFORMATION-STOP

-- ASN1STOP

#### –	ULInformationTransfer

The ULInformationTransfer message is used for the uplink transfer of NAS or non-3GPP dedicated information.

Signalling radio bearer: SRB2 or SRB1 (only if SRB2 not established yet). If SRB2 is suspended, the UE does not send this message until SRB2 is resumed

RLC-SAP: AM

Logical channel: DCCH

Direction: UE to network

ULInformationTransfer message

-- ASN1START

-- TAG-ULINFORMATIONTRANSFER-START

ULInformationTransfer ::=           SEQUENCE {

criticalExtensions                  CHOICE {

ulInformationTransfer               ULInformationTransfer-IEs,

criticalExtensionsFuture            SEQUENCE {}

}

}

ULInformationTransfer-IEs ::=       SEQUENCE {

dedicatedNAS-Message                DedicatedNAS-Message                OPTIONAL,

lateNonCriticalExtension            OCTET STRING                        OPTIONAL,

nonCriticalExtension                SEQUENCE {}                         OPTIONAL

}

-- TAG-ULINFORMATIONTRANSFER-STOP

-- ASN1STOP

#### –	ULInformationTransferMRDC

The ULInformationTransferMRDC message is used for the uplink transfer of MR-DC dedicated information (e.g. for transferring the NR or E-UTRA RRC MeasurementReport message or the FailureInformation message).

Signalling radio bearer: SRB1

RLC-SAP: AM

Logical channel: DCCH

Direction: UE to Network

ULInformationTransferMRDC message

-- ASN1START

-- TAG-ULINFORMATIONTRANSFERMRDC-START

ULInformationTransferMRDC ::=               SEQUENCE {

criticalExtensions                          CHOICE {

c1                                          CHOICE {

ulInformationTransferMRDC                   ULInformationTransferMRDC-IEs,

spare3 NULL, spare2 NULL, spare1 NULL

},

criticalExtensionsFuture            SEQUENCE {}

}

}

ULInformationTransferMRDC-IEs::=           SEQUENCE {

ul-DCCH-MessageNR                           OCTET STRING                    OPTIONAL,

ul-DCCH-MessageEUTRA                        OCTET STRING                    OPTIONAL,

lateNonCriticalExtension                    OCTET STRING                    OPTIONAL,

nonCriticalExtension                        SEQUENCE {}                     OPTIONAL

}

-- TAG-ULINFORMATIONTRANSFERMRDC-STOP

-- ASN1STOP

ULInformationTransferMRDC field descriptions

ul-DCCH-MessageNR Includes the UL-DCCH-Message. In this version of the specification, the field is only used to transfer the NR RRC MeasurementReport and FailureInformation messages.

ul-DCCH-MessageEUTRA Includes the UL-DCCH-Message. In this version of the specification, the field is only used to transfer the E-UTRA RRC MeasurementReport message.

## 6.3	RRC information elements

### 6.3.0	Parameterized types

#### –	SetupRelease

SetupRelease allows the ElementTypeParam to be used as the referenced data type for the setup and release entries. See A.3.8 for guidelines.

-- ASN1START

-- TAG-SETUPRELEASE-START

SetupRelease { ElementTypeParam } ::= CHOICE {

release         NULL,

setup           ElementTypeParam

}

-- TAG-SETUPRELEASE-STOP

-- ASN1STOP

### 6.3.1	System information blocks

#### –	SIB2

SIB2 contains cell re-selection information common for intra-frequency, inter-frequency and/or inter-RAT cell re-selection (i.e. applicable for more than one type of cell re-selection but not necessarily all) as well as intra-frequency cell re-selection information other than neighbouring cell related.

SIB2 information element

-- ASN1START

-- TAG-SIB2-START

SIB2 ::=                            SEQUENCE {

cellReselectionInfoCommon           SEQUENCE {

nrofSS-BlocksToAverage              INTEGER (2..maxNrofSS-BlocksToAverage)          OPTIONAL,       -- Need S

absThreshSS-BlocksConsolidation     ThresholdNR                                     OPTIONAL,       -- Need S

rangeToBestCell                     RangeToBestCell                                 OPTIONAL,       -- Need R

q-Hyst                              ENUMERATED {

dB0, dB1, dB2, dB3, dB4, dB5, dB6, dB8, dB10,

dB12, dB14, dB16, dB18, dB20, dB22, dB24},

speedStateReselectionPars           SEQUENCE {

mobilityStateParameters             MobilityStateParameters,

q-HystSF                        SEQUENCE {

sf-Medium                       ENUMERATED {dB-6, dB-4, dB-2, dB0},

sf-High                         ENUMERATED {dB-6, dB-4, dB-2, dB0}

}

}                                                                                   OPTIONAL,       -- Need R

...

},

cellReselectionServingFreqInfo      SEQUENCE {

s-NonIntraSearchP                   ReselectionThreshold                            OPTIONAL,       -- Need S

s-NonIntraSearchQ                   ReselectionThresholdQ                           OPTIONAL,       -- Need S

threshServingLowP                   ReselectionThreshold,

threshServingLowQ                   ReselectionThresholdQ                           OPTIONAL,       -- Need R

cellReselectionPriority             CellReselectionPriority,

cellReselectionSubPriority          CellReselectionSubPriority                      OPTIONAL,       -- Need R

...

},

intraFreqCellReselectionInfo        SEQUENCE {

q-RxLevMin                          Q-RxLevMin,

q-RxLevMinSUL                       Q-RxLevMin                                      OPTIONAL,       -- Need R

q-QualMin                           Q-QualMin                                       OPTIONAL,       -- Need S

s-IntraSearchP                      ReselectionThreshold,

s-IntraSearchQ                      ReselectionThresholdQ                           OPTIONAL,       -- Need S

t-ReselectionNR                     T-Reselection,

frequencyBandList                   MultiFrequencyBandListNR-SIB                    OPTIONAL,       -- Need S

frequencyBandListSUL                MultiFrequencyBandListNR-SIB                    OPTIONAL,       -- Need R

p-Max                               P-Max                                           OPTIONAL,       -- Need S

smtc                                SSB-MTC                                         OPTIONAL,       -- Need S

ss-RSSI-Measurement                 SS-RSSI-Measurement                             OPTIONAL,       -- Need R

ssb-ToMeasure                       SSB-ToMeasure                                   OPTIONAL,       -- Need S

deriveSSB-IndexFromCell             BOOLEAN,

...,

[[

t-ReselectionNR-SF                  SpeedStateScaleFactors                          OPTIONAL        -- Need N

]]

},

...

}

RangeToBestCell    ::= Q-OffsetRange

-- TAG-SIB2-STOP

-- ASN1STOP

SIB2 field descriptions

absThreshSS-BlocksConsolidation Threshold for consolidation of L1 measurements per RS index. If the field is absent, the UE uses the measurement quantity as specified in TS 38.304 [20].

cellReselectionInfoCommon Cell re-selection information common for intra-frequency, inter-frequency and/ or inter-RAT cell re-selection.

cellReselectionServingFreqInfo Information common for non-intra-frequency cell re-selection i.e. cell re-selection to inter-frequency and inter-RAT cells.

deriveSSB-IndexFromCell This field indicates whether the UE can utilize serving cell timing to derive the index of SS block transmitted by neighbour cell. If this field is set to true, the UE assumes SFN and frame boundary alignment across cells on the serving frequency as specified in TS 38.133 [14].

frequencyBandList Indicates the list of frequency bands for which the NR cell reselection parameters apply. The UE behaviour in case the field is absent is described in subclause 5.2.2.4.3.

intraFreqCellReselectionInfo Cell re-selection information common for intra-frequency cells.

nrofSS-BlocksToAverage Number of SS blocks to average for cell measurement derivation. If the field is absent the UE uses the measurement quantity as specified in TS 38.304 [20].

p-Max Value in dBm applicable for the intra-frequency neighbouring NR cells. If absent the UE applies the maximum power according to TS 38.101-1 [15] in case of an FR1 cell or TS 38.101-2 [39] in case of an FR2 cell. In this release of the specification, if p-Max is present on a carrier frequency in FR2, the UE shall ignore the field and applies the maximum power according to TS 38.101-2 [39].

q-Hyst Parameter "Qhyst" in TS 38.304 [20], Value in dB. Value dB1 corresponds to 1 dB, dB2 corresponds to 2 dB and so on.

q-HystSF Parameter "Speed dependent ScalingFactor for Qhyst" in TS 38.304 [20]. The sf-Medium and sf-High concern the additional hysteresis to be applied, in Medium and High Mobility state respectively, to Qhyst as defined in TS 38.304 [20]. In dB. Value dB-6 corresponds to -6dB, dB-4 corresponds to -4dB and so on.

q-QualMin Parameter "Qqualmin" in TS 38.304 [20], applicable for intra-frequency neighbour cells. If the field is absent, the UE applies the (default) value of negative infinity for Qqualmin.

q-RxLevMin Parameter "Qrxlevmin" in TS 38.304 [20], applicable for intra-frequency neighbour cells.

q-RxLevMinSUL Parameter "Qrxlevmin" in TS 38.304 [20], applicable for intra-frequency neighbour cells.

rangeToBestCell Parameter "rangeToBestCell" in TS 38.304 [20]. The network configures only non-negative (in dB) values.

s-IntraSearchP Parameter "SIntraSearchP" in TS 38.304 [20].

s-IntraSearchQ Parameter "SIntraSearchQ" in TS 38.304 [20]. If the field is absent, the UE applies the (default) value of 0 dB for SIntraSearchQ.

s-NonIntraSearchP Parameter "SnonIntraSearchP" in TS 38.304 [20]. If this field is absent, the UE applies the (default) value of infinity for SnonIntraSearchP.

s-NonIntraSearchQ Parameter "SnonIntraSearchQ" in TS 38.304 [20]. If the field is absent, the UE applies the (default) value of 0 dB for SnonIntraSearchQ.

smtc Measurement timing configuration for intra-frequency measurement. If this field is absent, the UE assumes that SSB periodicity is 5 ms for the intra-frequnecy cells.

ssb-ToMeasure The set of SS blocks to be measured within the SMTC measurement duration (see TS 38.215 [9]). When the field is absent the UE measures on all SS-blocks.

t-ReselectionNR Parameter "TreselectionNR" in TS 38.304 [20].

t-ReselectionNR-SF Parameter "Speed dependent ScalingFactor for TreselectionNR" in TS 38.304 [20]. If the field is absent, the UE behaviour is specified in TS 38.304 [20].

threshServingLowP Parameter "ThreshServing, LowP" in TS 38.304 [20].

threshServingLowQ Parameter "ThreshServing, LowQ" in TS 38.304 [20].

#### –	SIB3

SIB3 contains neighbouring cell related information relevant only for intra-frequency cell re-selection. The IE includes cells with specific re-selection parameters as well as blacklisted cells.

SIB3 information element

-- ASN1START

-- TAG-SIB3-START

SIB3 ::=                            SEQUENCE {

intraFreqNeighCellList              IntraFreqNeighCellList      OPTIONAL,   -- Need R

intraFreqBlackCellList              IntraFreqBlackCellList      OPTIONAL,   -- Need R

lateNonCriticalExtension            OCTET STRING                OPTIONAL,

...

}

IntraFreqNeighCellList ::=          SEQUENCE (SIZE (1..maxCellIntra)) OF IntraFreqNeighCellInfo

IntraFreqNeighCellInfo ::=          SEQUENCE {

physCellId                          PhysCellId,

q-OffsetCell                        Q-OffsetRange,

q-RxLevMinOffsetCell                INTEGER (1..8)              OPTIONAL,   -- Need R

q-RxLevMinOffsetCellSUL             INTEGER (1..8)              OPTIONAL,   -- Need R

q-QualMinOffsetCell                 INTEGER (1..8)              OPTIONAL,   -- Need R

...

}

IntraFreqBlackCellList ::=          SEQUENCE (SIZE (1..maxCellBlack)) OF PCI-Range

-- TAG-SIB3-STOP

-- ASN1STOP

SIB3 field descriptions

intraFreqBlackCellList List of blacklisted intra-frequency neighbouring cells.

intraFreqNeighCellList List of intra-frequency neighbouring cells with specific cell re-selection parameters.

q-OffsetCell Parameter "Qoffsets,n" in TS 38.304 [20].

q-QualMinOffsetCell Parameter "Qqualminoffsetcell" in TS 38.304 [20]. Actual value Qqualminoffsetcell = field value [dB].

q-RxLevMinOffsetCell Parameter "Qrxlevminoffsetcell" in TS 38.304 [20]. Actual value Qrxlevminoffsetcell = field value * 2 [dB].

q-RxLevMinOffsetCellSUL Parameter "QrxlevminoffsetcellSUL" in TS 38.304 [20]. Actual value QrxlevminoffsetcellSUL = field value * 2 [dB].

#### –	SIB4

SIB4 contains information relevant only for inter-frequency cell re-selection i.e. information about other NR frequencies and inter-frequency neighbouring cells relevant for cell re-selection. The IE includes cell re-selection parameters common for a frequency as well as cell specific re-selection parameters.

SIB4 information element

-- ASN1START

-- TAG-SIB4-START

SIB4 ::=                            SEQUENCE {

interFreqCarrierFreqList            InterFreqCarrierFreqList,

lateNonCriticalExtension            OCTET STRING                                OPTIONAL,

...

}

InterFreqCarrierFreqList ::=        SEQUENCE (SIZE (1..maxFreq)) OF InterFreqCarrierFreqInfo

InterFreqCarrierFreqInfo ::=        SEQUENCE {

dl-CarrierFreq                      ARFCN-ValueNR,

frequencyBandList                   MultiFrequencyBandListNR-SIB                OPTIONAL,   -- Cond Mandatory

frequencyBandListSUL                MultiFrequencyBandListNR-SIB                OPTIONAL,   -- Need R

nrofSS-BlocksToAverage              INTEGER (2..maxNrofSS-BlocksToAverage)      OPTIONAL,   -- Need S

absThreshSS-BlocksConsolidation     ThresholdNR                                 OPTIONAL,   -- Need S

smtc                                SSB-MTC                                     OPTIONAL,   -- Need S

ssbSubcarrierSpacing                SubcarrierSpacing,

ssb-ToMeasure                       SSB-ToMeasure                               OPTIONAL,   -- Need S

deriveSSB-IndexFromCell             BOOLEAN,

ss-RSSI-Measurement                 SS-RSSI-Measurement                         OPTIONAL,

q-RxLevMin                          Q-RxLevMin,

q-RxLevMinSUL                       Q-RxLevMin                                  OPTIONAL,   -- Need R

q-QualMin                           Q-QualMin                                   OPTIONAL,   -- Need S

p-Max                               P-Max                                       OPTIONAL,   -- Need S

t-ReselectionNR                     T-Reselection,

t-ReselectionNR-SF                  SpeedStateScaleFactors                      OPTIONAL,   -- Need S

threshX-HighP                       ReselectionThreshold,

threshX-LowP                        ReselectionThreshold,

threshX-Q                           SEQUENCE {

threshX-HighQ                       ReselectionThresholdQ,

threshX-LowQ                        ReselectionThresholdQ

}                                                                               OPTIONAL,   -- Cond RSRQ

cellReselectionPriority             CellReselectionPriority                     OPTIONAL,   -- Need R

cellReselectionSubPriority          CellReselectionSubPriority                  OPTIONAL,   -- Need R

q-OffsetFreq                        Q-OffsetRange                               DEFAULT dB0,

interFreqNeighCellList              InterFreqNeighCellList                      OPTIONAL,   -- Need R

interFreqBlackCellList              InterFreqBlackCellList                      OPTIONAL,   -- Need R

...

}

InterFreqNeighCellList ::=          SEQUENCE (SIZE (1..maxCellInter)) OF InterFreqNeighCellInfo

InterFreqNeighCellInfo ::=          SEQUENCE {

physCellId                          PhysCellId,

q-OffsetCell                        Q-OffsetRange,

q-RxLevMinOffsetCell                INTEGER (1..8)                              OPTIONAL,   -- Need R

q-RxLevMinOffsetCellSUL             INTEGER (1..8)                              OPTIONAL,   -- Need R

q-QualMinOffsetCell                 INTEGER (1..8)                              OPTIONAL,   -- Need R

...

}

InterFreqBlackCellList ::=          SEQUENCE (SIZE (1..maxCellBlack)) OF PCI-Range

-- TAG-SIB4-STOP

-- ASN1STOP

SIB4 field descriptions

absThreshSS-BlocksConsolidation Threshold for consolidation of L1 measurements per RS index. If the field is absent, the UE uses the measurement quantity as specified in TS 38.304 [20].

deriveSSB-IndexFromCell This field indicates whether the UE may use the timing of any detected cell on that frequency to derive the SSB index of all neighbour cells on that frequency. If this field is set to true, the UE assumes SFN and frame boundary alignment across cells on the neighbor frequency as specified in TS 38.133 [14].

dl-CarrierFreq This field indicates center frequency of the SS block of the neighbour cells, where the frequency corresponds to a GSCN value as specified in TS 38.101-1 [15].

frequencyBandList Indicates the list of frequency bands for which the NR cell reselection parameters apply.

interFreqBlackCellList List of blacklisted inter-frequency neighbouring cells.

interFreqCarrierFreqList List of neighbouring carrier frequencies and frequency specific cell re-selection information.

interFreqNeighCellList List of inter-frequency neighbouring cells with specific cell re-selection parameters.

nrofSS-BlocksToAverage Number of SS blocks to average for cell measurement derivation. If the field is absent, the UE uses the measurement quantity as specified in TS 38.304 [20].

p-Max Value in dBm applicable for the neighbouring NR cells on this carrier frequency. If absent the UE applies the maximum power according to TS 38.101-1 [15] in case of an FR1 cell or TS 38.101-2 [39] in case of an FR2 cell. In this release of the specification, if p-Max is present on a carrier frequency in FR2, the UE shall ignore the field and applies the maximum power according to TS 38.101-2 [39].

q-OffsetCell Parameter "Qoffsets,n" in TS 38.304 [20].

q-OffsetFreq Parameter "Qoffsetfrequency" in TS 38.304 [20].

q-QualMin Parameter "Qqualmin" in TS 38.304 [20]. If the field is absent, the UE applies the (default) value of negative infinity for Qqualmin.

q-QualMinOffsetCell Parameter "Qqualminoffsetcell" in TS 38.304 [20]. Actual value Qqualminoffsetcell = field value [dB].

q-RxLevMin Parameter "Qrxlevmin" in TS 38.304 [20].

q-RxLevMinOffsetCell Parameter "Qrxlevminoffsetcell" in TS 38.304 [20]. Actual value Qrxlevminoffsetcell = field value * 2 [dB].

q-RxLevMinOffsetCellSUL Parameter "QrxlevminoffsetcellSUL" in TS 38.304 [20]. Actual value QrxlevminoffsetcellSUL = field value * 2 [dB].

q-RxLevMinSUL Parameter "Qrxlevmin" in TS 38.304 [20].

smtc Measurement timing configuration for inter-frequency measurement. If this field is absent, the UE assumes that SSB periodicity is 5 ms in this frequency.

ssb-ToMeasure The set of SS blocks to be measured within the SMTC measurement duration (see TS 38.215 [9]). When the field is absent the UE measures on all SS-blocks.

ssbSubcarrierSpacing Subcarrier spacing of SSB. Only the values 15 kHz or 30 kHz (FR1), and 120 kHz or 240 kHz (FR2) are applicable.

threshX-HighP Parameter "ThreshX, HighP" in TS 38.304 [20].

threshX-HighQ Parameter "ThreshX, HighQ" in TS 38.304 [20].

threshX-LowP Parameter "ThreshX, LowP" in TS 38.304 [20].

threshX-LowQ Parameter "ThreshX, LowQ" in TS 38.304 [20].

t-ReselectionNR Parameter "TreselectionNR" in TS 38.304 [20].

t-ReselectionNR-SF Parameter "Speed dependent ScalingFactor for TreselectionNR" in TS 38.304 [20]. If the field is absent, the UE behaviour is specified in TS 38.304 [20].

| Conditional Presence | Explanation |
| --- | --- |
| Mandatory | The field is mandatory present in SIB4. |
| RSRQ | The field is mandatory present if threshServingLowQ is present in SIB2; otherwise it is absent. |

#### –	SIB5

SIB5 contains information relevant only for inter-RAT cell re-selection i.e. information about E-UTRA frequencies and E-UTRAs neighbouring cells relevant for cell re-selection. The IE includes cell re-selection parameters common for a frequency.

SIB5 information element

-- ASN1START

-- TAG-SIB5-START

SIB5 ::=                            SEQUENCE {

carrierFreqListEUTRA                CarrierFreqListEUTRA                        OPTIONAL,       -- Need R

t-ReselectionEUTRA                  T-Reselection,

t-ReselectionEUTRA-SF               SpeedStateScaleFactors                      OPTIONAL,       -- Need S

lateNonCriticalExtension            OCTET STRING                                OPTIONAL,

...

}

CarrierFreqListEUTRA ::=            SEQUENCE (SIZE (1..maxEUTRA-Carrier)) OF CarrierFreqEUTRA

CarrierFreqEUTRA ::=                SEQUENCE {

carrierFreq                         ARFCN-ValueEUTRA,

eutra-multiBandInfoList             EUTRA-MultiBandInfoList                     OPTIONAL,       -- Need R

eutra-FreqNeighCellList             EUTRA-FreqNeighCellList                     OPTIONAL,       -- Need R

eutra-BlackCellList                 EUTRA-FreqBlackCellList                     OPTIONAL,       -- Need R

allowedMeasBandwidth                EUTRA-AllowedMeasBandwidth,

presenceAntennaPort1                EUTRA-PresenceAntennaPort1,

cellReselectionPriority             CellReselectionPriority                     OPTIONAL,       -- Need R

cellReselectionSubPriority          CellReselectionSubPriority                  OPTIONAL,       -- Need R

threshX-High                        ReselectionThreshold,

threshX-Low                         ReselectionThreshold,

q-RxLevMin                          INTEGER (-70..-22),

q-QualMin                           INTEGER (-34..-3),

p-MaxEUTRA                          INTEGER (-30..33),

threshX-Q                           SEQUENCE {

threshX-HighQ                       ReselectionThresholdQ,

threshX-LowQ                        ReselectionThresholdQ

}                                                                               OPTIONAL        -- Cond RSRQ

}

EUTRA-FreqBlackCellList ::=         SEQUENCE (SIZE (1..maxEUTRA-CellBlack)) OF EUTRA-PhysCellIdRange

EUTRA-FreqNeighCellList ::=         SEQUENCE (SIZE (1..maxCellEUTRA)) OF EUTRA-FreqNeighCellInfo

EUTRA-FreqNeighCellInfo ::=         SEQUENCE {

physCellId                          EUTRA-PhysCellId,

dummy                               EUTRA-Q-OffsetRange,

q-RxLevMinOffsetCell                INTEGER (1..8)                              OPTIONAL,       -- Need R

q-QualMinOffsetCell                 INTEGER (1..8)                              OPTIONAL        -- Need R

}

-- TAG-SIB5-STOP

-- ASN1STOP

SIB5 field descriptions

carrierFreqListEUTRA List of carrier frequencies of E-UTRA.

dummy This field is not used in the specification. If received it shall be ignored by the UE.

eutra-BlackCellList List of blacklisted E-UTRA neighbouring cells.

eutra-multiBandInfoList Indicates the list of frequency bands in addition to the band represented by carrierFreq for which cell reselection parameters are common, and a list of additionalPmax and additionalSpectrumEmission values, as defined in TS 36.101 [22], table 6.2.4-1, for the frequency bands in eutra-multiBandInfoList

p-MaxEUTRA The maximum allowed transmission power in dBm on the (uplink) carrier frequency, see TS 36.304 [27].

q-QualMin Parameter "Qqualmin" in TS 36.304 [27]. Actual value Qqualmin = field value [dB].

q-QualMinOffsetCell Parameter "Qqualminoffsetcell" in TS 38.304 [20]. Actual value Qqualminoffsetcell = field value [dB].

q-RxLevMin Parameter "Qrxlevmin" in TS 36.304 [27]. Actual value Qrxlevmin = field value * 2 [dBm].

q-RxLevMinOffsetCell Parameter "Qrxlevminoffsetcell" in TS 38.304 [20]. Actual value Qrxlevminoffsetcell = field value * 2 [dB].

t-ReselectionEUTRA Parameter "TreselectionEUTRA" in TS 38.304 [20].

threshX-High Parameter "ThreshX, HighP" in TS 38.304 [20].

threshX-HighQ Parameter "ThreshX, HighQ" in TS 38.304 [20].

threshX-Low Parameter "ThreshX, LowP" in TS 38.304 [20].

threshX-LowQ Parameter "ThreshX, LowQ" in TS 38.304 [20].

t-ReselectionEUTRA-SF Parameter "Speed dependent ScalingFactor for TreselectionEUTRA" in TS 38.304 [20]. If the field is absent, the UE behaviour is specified in TS 38.304 [20].

| Conditional Presence | Explanation |
| --- | --- |
| RSRQ | The field is mandatory present if the threshServingLowQ is present in SIB2; otherwise it is absent. |

#### –	SIB6

SIB6 contains an ETWS primary notification.

SIB6 information element

-- ASN1START

-- TAG-SIB6-START

SIB6 ::=                            SEQUENCE {

messageIdentifier                   BIT STRING (SIZE (16)),

serialNumber                        BIT STRING (SIZE (16)),

warningType                         OCTET STRING (SIZE (2)),

lateNonCriticalExtension            OCTET STRING                                OPTIONAL,

...

}

-- TAG-SIB6-STOP

-- ASN1STOP

SIB6 field descriptions

messageIdentifier Identifies the source and type of ETWS notification.

serialNumber Identifies variations of an ETWS notification.

warningType Identifies the warning type of the ETWS primary notification and provides information on emergency user alert and UE popup.

#### –	SIB7

SIB7 contains an ETWS secondary notification.

SIB7 information element

-- ASN1START

-- TAG-SIB7-START

SIB7 ::=                            SEQUENCE {

messageIdentifier                   BIT STRING (SIZE (16)),

serialNumber                        BIT STRING (SIZE (16)),

warningMessageSegmentType           ENUMERATED {notLastSegment, lastSegment},

warningMessageSegmentNumber         INTEGER (0..63),

warningMessageSegment               OCTET STRING,

dataCodingScheme                    OCTET STRING (SIZE (1))                     OPTIONAL,   -- Cond Segment1

lateNonCriticalExtension            OCTET STRING                                OPTIONAL,

...

}

-- TAG-SIB7-STOP

-- ASN1STOP

SIB7 field descriptions

dataCodingScheme Identifies the alphabet/coding and the language applied variations of an ETWS notification.

messageIdentifier Identifies the source and type of ETWS notification.

serialNumber Identifies variations of an ETWS notification.

warningMessageSegment Carries a segment of the Warning Message Contents IE.

warningMessageSegmentNumber Segment number of the ETWS warning message segment contained in the SIB. A segment number of zero corresponds to the first segment, A segment number of one corresponds to the second segment, and so on.

warningMessageSegmentType Indicates whether the included ETWS warning message segment is the last segment or not.

| Conditional Presence | Explanation |
| --- | --- |
| Segment1 | The field is mandatory present in the first segment of SIB7, otherwise it is absent. |

#### –	SIB8

SIB8 contains a CMAS notification.

SIB8 information element

-- ASN1START

-- TAG-SIB8-START

SIB8 ::=                        SEQUENCE {

messageIdentifier               BIT STRING (SIZE (16)),

serialNumber                    BIT STRING (SIZE (16)),

warningMessageSegmentType       ENUMERATED {notLastSegment, lastSegment},

warningMessageSegmentNumber     INTEGER (0..63),

warningMessageSegment           OCTET STRING,

dataCodingScheme                OCTET STRING (SIZE (1))                         OPTIONAL,   -- Cond Segment1

warningAreaCoordinatesSegment   OCTET STRING                                    OPTIONAL,   -- Need R

lateNonCriticalExtension        OCTET STRING                                    OPTIONAL,

...

}

-- TAG-SIB8-STOP

-- ASN1STOP

SIB8 field descriptions

dataCodingScheme Identifies the alphabet/coding and the language applied variations of a CMAS notification.

messageIdentifier Identifies the source and type of CMAS notification.

serialNumber Identifies variations of a CMAS notification.

warningAreaCoordinatesSegment If present, carries a segment, with one or more octets, of the geographical area where the CMAS warning message is valid as defined in [28]. The first octet of the first warningAreaCoordinatesSegment is equivalent to the first octet of Warning Area Coordinates IE defined in and encoded according to TS 23.041 [29] and so on.

warningMessageSegment Carries a segment, with one or more octets, of the Warning Message Contents IE defined in TS 38.413 [42]. The first octet of the Warning Message Contents IE is equivalent to the first octet of the CB data IE defined in and encoded according to TS 23.041 [29], clause 9.4.2.2.5, and so on.

warningMessageSegmentNumber Segment number of the CMAS warning message segment contained in the SIB. A segment number of zero corresponds to the first segment, one corresponds to the second segment, and so on. If warning area coordinates are provided for the warning message, then this field applies to both warning message segment and warning area coordinates segment.

warningMessageSegmentType Indicates whether the included CMAS warning message segment is the last segment or not. If warning area coordinates are provided for the warning message, then this field applies to both warning message segment and warning area coordinates segment.

| Conditional Presence | Explanation |
| --- | --- |
| Segment1 | The field is mandatory present in the first segment of SIB8, otherwise it is absent. |

#### –	SIB9

SIB9 contains information related to GPS time and Coordinated Universal Time (UTC). The UE may use the parameters provided in this system information block to obtain the UTC, the GPS and the local time.

NOTE:	The UE may use the time information for numerous purposes, possibly involving upper layers e.g. to assist GPS initialisation, to synchronise the UE clock.

SIB9 information element

-- ASN1START

-- TAG-SIB9-START

SIB9 ::=                            SEQUENCE {

timeInfo                            SEQUENCE {

timeInfoUTC                         INTEGER (0..549755813887),

dayLightSavingTime                  BIT STRING (SIZE (2))                   OPTIONAL,   -- Need R

leapSeconds                         INTEGER (-127..128)                     OPTIONAL,   -- Need R

localTimeOffset                     INTEGER (-63..64)                       OPTIONAL    -- Need R

}                                                                               OPTIONAL,   -- Need R

lateNonCriticalExtension            OCTET STRING                                OPTIONAL,

...

}

-- TAG-SIB9-STOP

-- ASN1STOP

SIB9 field descriptions

dayLightSavingTime Indicates if and how daylight-saving time (DST) is applied to obtain the local time. The semantics are the same as the semantics of the Daylight Saving Time IE in TS 24.501 [23] and TS 24.008 [38]. The first/leftmost bit of the bit string contains the b2 of octet 3 and the second bit of the bit string contains b1 of octet 3 in the value part of the Daylight Saving Time IE in TS 24.008 [38].

leapSeconds Number of leap seconds offset between GPS Time and UTC. UTC and GPS time are related i.e. GPS time -leapSeconds = UTC time.

localTimeOffset Offset between UTC and local time in units of 15 minutes. Actual value = field value * 15 minutes. Local time of the day is calculated as UTC time + localTimeOffset.

timeInfoUTC Coordinated Universal Time corresponding to the SFN boundary at or immediately after the ending boundary of the SI-window in which SIB9 is transmitted. The field counts the number of UTC seconds in 10 ms units since 00:00:00 on Gregorian calendar date 1 January, 1900 (midnight between Sunday, December 31, 1899 and Monday, January 1, 1900). See NOTE 1. This field is excluded when determining changes in system information, i.e. changes of timeInfoUTC should neither result in system information change notifications nor in a modification of valueTag in SIB1.

NOTE 1:	The UE may use this field together with the leapSeconds field to obtain GPS time as follows: GPS Time (in seconds) = timeInfoUTC (in seconds) -  2,524,953,600 (seconds) + leapSeconds, where 2,524,953,600 is the number of seconds between 00:00:00 on Gregorian calendar date 1 January, 1900 and 00:00:00 on Gregorian calendar date 6 January, 1980 (start of GPS time).

### 6.3.2	Radio resource control information elements

#### –	AdditionalSpectrumEmission

The IE AdditionalSpectrumEmission is used to indicate emission requirements to be fulfilled by the UE (see TS 38.101-1 [15], clause 6.2.3, and TS 38.101-2 [39], clause 6.2.3).

AdditionalSpectrumEmission information element

-- ASN1START

-- TAG-ADDITIONALSPECTRUMEMISSION-START

AdditionalSpectrumEmission ::=              INTEGER (0..7)

-- TAG-ADDITIONALSPECTRUMEMISSION-STOP

-- ASN1STOP

#### –	Alpha

The IE Alpha defines possible values of a the pathloss compensation coefficient for uplink power control. Value alpha0 corresponds to the value 0, Value alpha04 corresponds to the value 0.4, Value alpha05 corresponds to the value 0.5 and so on. Value alpha1 corresponds to value 1. See also clause 7.1 of TS 38.213 [13].

-- ASN1START

-- TAG-ALPHA-START

Alpha ::=                       ENUMERATED {alpha0, alpha04, alpha05, alpha06, alpha07, alpha08, alpha09, alpha1}

-- TAG-ALPHA-STOP

-- ASN1STOP

#### –	AMF-Identifier

The IE AMF-Identifier (AMFI) comprises of an AMF Region ID, an AMF Set ID and an AMF Pointer as specified in TS 23.003 [21], clause 2.10.1.

AMF-Identifier information element

-- ASN1START

-- TAG-AMF-IDENTIFIER-START

AMF-Identifier ::=                      BIT STRING (SIZE (24))

-- TAG-AMF-IDENTIFIER-STOP

-- ASN1STOP

#### –	ARFCN-ValueEUTRA

The IE ARFCN-ValueEUTRA is used to indicate the ARFCN applicable for a downlink, uplink or bi-directional (TDD) E-UTRA carrier frequency, as defined in TS 36.101 [22].

ARFCN-ValueEUTRA information element

-- ASN1START

-- TAG-ARFCN-VALUEEUTRA-START

ARFCN-ValueEUTRA ::=                INTEGER (0..maxEARFCN)

-- TAG-ARFCN-VALUEEUTRA-STOP

-- ASN1STOP

#### –	ARFCN-ValueNR

The IE ARFCN-ValueNR is used to indicate the ARFCN applicable for a downlink, uplink or bi-directional (TDD) NR global frequency raster, as defined in TS 38.101-1 [15] and TS 38.101-2 [39], clause 5.4.2.

-- ASN1START

-- TAG-ARFCN-VALUENR-START

ARFCN-ValueNR ::=               INTEGER (0..maxNARFCN)

-- TAG-ARFCN-VALUENR-STOP

-- ASN1STOP

#### –	BeamFailureRecoveryConfig

The IE BeamFailureRecoveryConfig is used to configure the UE with RACH resources and candidate beams for beam failure recovery in case of beam failure detection. See also TS 38.321 [3], clause 5.1.1.

BeamFailureRecoveryConfig information element

-- ASN1START

-- TAG-BEAMFAILURERECOVERYCONFIG-START

BeamFailureRecoveryConfig ::=       SEQUENCE {

rootSequenceIndex-BFR               INTEGER (0..137)                                                          OPTIONAL, -- Need M

rach-ConfigBFR                      RACH-ConfigGeneric                                                        OPTIONAL, -- Need M

rsrp-ThresholdSSB                   RSRP-Range                                                                OPTIONAL, -- Need M

candidateBeamRSList                 SEQUENCE (SIZE(1..maxNrofCandidateBeams)) OF PRACH-ResourceDedicatedBFR   OPTIONAL, -- Need M

ssb-perRACH-Occasion                ENUMERATED {oneEighth, oneFourth, oneHalf, one, two,

four, eight, sixteen}                                      OPTIONAL, -- Need M

ra-ssb-OccasionMaskIndex            INTEGER (0..15)                                                           OPTIONAL, -- Need M

recoverySearchSpaceId               SearchSpaceId                                                             OPTIONAL, -- Need R

ra-Prioritization                   RA-Prioritization                                                         OPTIONAL, -- Need R

beamFailureRecoveryTimer            ENUMERATED {ms10, ms20, ms40, ms60, ms80, ms100, ms150, ms200}            OPTIONAL, -- Need M

...,

[[

msg1-SubcarrierSpacing              SubcarrierSpacing                                                         OPTIONAL  -- Need M

]]

}

PRACH-ResourceDedicatedBFR ::=      CHOICE {

ssb                                 BFR-SSB-Resource,

csi-RS                              BFR-CSIRS-Resource

}

BFR-SSB-Resource ::=                SEQUENCE {

ssb                                 SSB-Index,

ra-PreambleIndex                    INTEGER (0..63),

...

}

BFR-CSIRS-Resource ::=              SEQUENCE {

csi-RS                              NZP-CSI-RS-ResourceId,

ra-OccasionList                     SEQUENCE (SIZE(1..maxRA-OccasionsPerCSIRS)) OF INTEGER (0..maxRA-Occasions-1)   OPTIONAL,   -- Need R

ra-PreambleIndex                    INTEGER (0..63)                                                                 OPTIONAL,   -- Need R

...

}

-- TAG-BEAMFAILURERECOVERYCONFIG-STOP

-- ASN1STOP

BeamFailureRecoveryConfig field descriptions

beamFailureRecoveryTimer Timer for beam failure recovery timer. Upon expiration of the timer the UE does not use CFRA for BFR. Value in ms. Value ms10 corresponds to 10 ms, value ms20 corresponds to 20 ms, and so on.

candidateBeamRSList A list of reference signals (CSI-RS and/or SSB) identifying the candidate beams for recovery and the associated RA parameters. The network configures these reference signals to be within the linked DL BWP (i.e., within the DL BWP with the same bwp-Id) of the UL BWP in which the BeamFailureRecoveryConfig is provided.

msg1-SubcarrierSpacing Subcarrier spacing for contention free beam failure recovery. Only the values 15 kHz or 30 kHz (FR1), and 60 kHz or 120 kHz (FR2) are applicable. See TS 38.211 [16], clause 5.3.2.

rsrp-ThresholdSSB L1-RSRP threshold used for determining whether a candidate beam may be used by the UE to attempt contention free random access to recover from beam failure (see TS 38.213 [13], clause 6).

ra-prioritization Parameters which apply for prioritized random access procedure for BFR (see TS 38.321 [3], clause 5.1.1).

ra-ssb-OccasionMaskIndex Explicitly signalled PRACH Mask Index for RA Resource selection in TS 38.321 [3]. The mask is valid for all SSB resources.

rach-ConfigBFR Configuration of contention free random access occasions for BFR.

recoverySearchSpaceId Search space to use for BFR RAR. The network configures this search space to be within the linked DL BWP (i.e., within the DL BWP with the same bwp-Id) of the UL BWP in which the BeamFailureRecoveryConfig is provided. The CORESET associated with the recovery search space cannot be associated with another search space. Network always configures the UE with a value for this field when contention free random access resources for BFR are configured.

rootSequenceIndex-BFR PRACH root sequence index (see TS 38.211 [16], clause 6.3.3.1) for beam failure recovery.

ssb-perRACH-Occasion Number of SSBs per RACH occasion for CF-BFR, see TS 38.213 [13], clause 8.1.

BFR-CSIRS-Resource field descriptions

csi-RS The ID of a NZP-CSI-RS-Resource configured in the CSI-MeasConfig of this serving cell. This reference signal determines a candidate beam for beam failure recovery (BFR).

ra-OccasionList RA occasions that the UE shall use when performing BFR upon selecting the candidate beam identified by this CSI-RS. The network ensures that the RA occasion indexes provided herein are also configured by prach-ConfigurationIndex and msg1-FDM. Each RACH occasion is sequentially numbered, first, in increasing order of frequency resource indexes for frequency multiplexed PRACH occasions; second, in increasing order of time resource indexes for time multiplexed PRACH occasions within a PRACH slot and Third, in increasing order of indexes for PRACH slots. If the field is absent the UE uses the RA occasion associated with the SSB that is QCLed with this CSI-RS.

ra-PreambleIndex The RA preamble index to use in the RA occasions associated with this CSI-RS. If the field is absent, the UE uses the preamble index associated with the SSB that is QCLed with this CSI-RS.

BFR-SSB-Resource field descriptions

ra-PreambleIndex The preamble index that the UE shall use when performing BFR upon selecting the candidate beams identified by this SSB.

ssb The ID of an SSB transmitted by this serving cell. It determines a candidate beam for beam failure recovery (BFR).

#### –	BetaOffsets

The IE BetaOffsets is used to configure beta-offset values, see TS 38.213 [13], clause 9.3.

BetaOffsets information element

-- ASN1START

-- TAG-BETAOFFSETS-START

BetaOffsets ::=                     SEQUENCE {

betaOffsetACK-Index1                INTEGER(0..31)                                                          OPTIONAL, -- Need S

betaOffsetACK-Index2                INTEGER(0..31)                                                          OPTIONAL, -- Need S

betaOffsetACK-Index3                INTEGER(0..31)                                                          OPTIONAL, -- Need S

betaOffsetCSI-Part1-Index1          INTEGER(0..31)                                                          OPTIONAL, -- Need S

betaOffsetCSI-Part1-Index2          INTEGER(0..31)                                                          OPTIONAL, -- Need S

betaOffsetCSI-Part2-Index1          INTEGER(0..31)                                                          OPTIONAL, -- Need S

betaOffsetCSI-Part2-Index2          INTEGER(0..31)                                                          OPTIONAL  -- Need S

}

-- TAG-BETAOFFSETS-STOP

-- ASN1STOP

BetaOffsets field descriptions

betaOffsetACK-Index1 Up to 2 bits HARQ-ACK (see TS 38.213 [13], clause 9.3). When the field is absent the UE applies the value 11.

betaOffsetACK-Index2 Up to 11 bits HARQ-ACK (see TS 38.213 [13], clause 9.3). When the field is absent the UE applies the value 11.

betaOffsetACK-Index3 Above 11 bits HARQ-ACK (see TS 38.213 [13], clause 9.3). When the field is absent the UE applies the value 11.

betaOffsetCSI-Part1-Index1 Up to 11 bits of CSI part 1 bits (see TS 38.213 [13], clause 9.3). When the field is absent the UE applies the value 13.

betaOffsetCSI-Part1-Index2 Above 11 bits of CSI part 1 bits (see TS 38.213 [13], clause 9.3). When the field is absent the UE applies the value 13.

betaOffsetCSI-Part2-Index1 Up to 11 bits of CSI part 2 bits (see TS 38.213 [13], clause 9.3). When the field is absent the UE applies the value 13.

betaOffsetCSI-Part2-Index2 Above 11 bits of CSI part 2 bits (see TS 38.213 [13], clause 9.3). When the field is absent the UE applies the value 13.

#### –	BSR-Config

The IE BSR-Config is used to configure buffer status reporting.

BSR-Config information element

-- ASN1START

-- TAG-BSR-CONFIG-START

BSR-Config ::=                      SEQUENCE {

periodicBSR-Timer                   ENUMERATED { sf1, sf5, sf10, sf16, sf20, sf32, sf40, sf64,

sf80, sf128, sf160, sf320, sf640, sf1280, sf2560, infinity },

retxBSR-Timer                       ENUMERATED { sf10, sf20, sf40, sf80, sf160, sf320, sf640, sf1280, sf2560,

sf5120, sf10240, spare5, spare4, spare3, spare2, spare1},

logicalChannelSR-DelayTimer         ENUMERATED { sf20, sf40, sf64, sf128, sf512, sf1024, sf2560, spare1}    OPTIONAL, -- Need R

...

}

-- TAG-BSR-CONFIG-STOP

-- ASN1STOP

BSR-Config field descriptions

logicalChannelSR-DelayTimer Value in number of subframes. Value sf20 corresponds to 20 subframes, sf40 corresponds to 40 subframes, and so on.

periodicBSR-Timer Value in number of subframes. Value sf1 corresponds to 1 subframe, value sf5 corresponds to 5 subframes and so on.

retxBSR-Timer Value in number of subframes. Value sf10 corresponds to 10 subframes, value sf20 corresponds to 20 subframes and so on.

#### –	BWP

The IE BWP is used to configure generic parameters of a bandwidth part as defined in TS 38.211 [16], clause 4.5, and TS 38.213 [13], clause 12.

For each serving cell the network configures at least an initial downlink bandwidth part and one (if the serving cell is configured with an uplink) or two (if using supplementary uplink (SUL)) initial uplink bandwidth parts. Furthermore, the network may configure additional uplink and downlink bandwidth parts for a serving cell.

The uplink and downlink bandwidth part configurations are divided into common and dedicated parameters.

BWP information element

-- ASN1START

-- TAG-BWP-START

BWP ::=                             SEQUENCE {

locationAndBandwidth                INTEGER (0..37949),

subcarrierSpacing                   SubcarrierSpacing,

cyclicPrefix                        ENUMERATED { extended }                                                 OPTIONAL    -- Need R

}

-- TAG-BWP-STOP

-- ASN1STOP

BWP field descriptions

cyclicPrefix Indicates whether to use the extended cyclic prefix for this bandwidth part. If not set, the UE uses the normal cyclic prefix. Normal CP is supported for all subcarrier spacings and slot formats. Extended CP is supported only for 60 kHz subcarrier spacing. (see TS 38.211 [16], clause 4.2)

locationAndBandwidth Frequency domain location and bandwidth of this bandwidth part. The value of the field shall be interpreted as resource indicator value (RIV) as defined TS 38.214 [19] with assumptions as described in TS 38.213 [13], clause 12, i.e. setting =275. The first PRB is a PRB determined by subcarrierSpacing of this BWP and offsetToCarrier (configured in SCS-SpecificCarrier contained within FrequencyInfoDL / FrequencyInfoUL / FrequencyInfoUL-SIB / FrequencyInfoDL-SIB within ServingCellConfigCommon / ServingCellConfigCommonSIB) corresponding to this subcarrier spacing. In case of TDD, a BWP-pair (UL BWP and DL BWP with the same bwp-Id) must have the same center frequency (see TS 38.213 [13], clause 12)

subcarrierSpacing Subcarrier spacing to be used in this BWP for all channels and reference signals unless explicitly configured elsewhere. Corresponds to subcarrier spacing according to TS 38.211 [16], table 4.2-1. The value kHz15 corresponds to µ=0, value kHz30 corresponds to µ=1, and so on. Only the values 15 kHz, 30 kHz, or 60 kHz (FR1), and 60 kHz or 120 kHz (FR2) are applicable. For the initial DL BWP this field has the same value as the field subCarrierSpacingCommon in MIB of the same serving cell.

#### –	BWP-Downlink

The IE BWP-Downlink is used to configure an additional downlink bandwidth part (not for the initial BWP).

BWP-Downlink information element

-- ASN1START

-- TAG-BWP-DOWNLINK-START

BWP-Downlink ::=                    SEQUENCE {

bwp-Id                              BWP-Id,

bwp-Common                          BWP-DownlinkCommon                                         OPTIONAL,   -- Cond SetupOtherBWP

bwp-Dedicated                       BWP-DownlinkDedicated                                      OPTIONAL,   -- Cond SetupOtherBWP

...

}

-- TAG-BWP-DOWNLINK-STOP

-- ASN1STOP

BWP-Downlink field descriptions

bwp-Id An identifier for this bandwidth part. Other parts of the RRC configuration use the BWP-Id to associate themselves with a particular bandwidth part. The network configures the BWPs with consecutive IDs from 1. The Network does not include the value 0, since value 0 is reserved for the initial BWP.

| Conditional Presence | Explanation |
| --- | --- |
| SetupOtherBWP | The field is mandatory present upon configuration of a new DL BWP. The field is optionally present, Need M, otherwise. |

#### –	BWP-DownlinkCommon

The IE BWP-DownlinkCommon is used to configure the common parameters of a downlink BWP. They are "cell specific" and the network ensures the necessary alignment with corresponding parameters of other UEs. The common parameters of the initial bandwidth part of the PCell are also provided via system information. For all other serving cells, the network provides the common parameters via dedicated signalling.

BWP-DownlinkCommon information element

-- ASN1START

-- TAG-BWP-DOWNLINKCOMMON-START

BWP-DownlinkCommon ::=              SEQUENCE {

genericParameters                   BWP,

pdcch-ConfigCommon                  SetupRelease { PDCCH-ConfigCommon }                                     OPTIONAL,   -- Need M

pdsch-ConfigCommon                  SetupRelease { PDSCH-ConfigCommon }                                     OPTIONAL,   -- Need M

...

}

-- TAG-BWP-DOWNLINKCOMMON-STOP

-- ASN1STOP

BWP-DownlinkCommon field descriptions

pdcch-ConfigCommon Cell specific parameters for the PDCCH of this BWP.

pdsch-ConfigCommon Cell specific parameters for the PDSCH of this BWP.

#### –	BWP-DownlinkDedicated

The IE BWP-DownlinkDedicated is used to configure the dedicated (UE specific) parameters of a downlink BWP.

BWP-DownlinkDedicated information element

-- ASN1START

-- TAG-BWP-DOWNLINKDEDICATED-START

BWP-DownlinkDedicated ::=           SEQUENCE {

pdcch-Config                        SetupRelease { PDCCH-Config }                                           OPTIONAL,   -- Need M

pdsch-Config                        SetupRelease { PDSCH-Config }                                           OPTIONAL,   -- Need M

sps-Config                          SetupRelease { SPS-Config }                                             OPTIONAL,   -- Need M

radioLinkMonitoringConfig           SetupRelease { RadioLinkMonitoringConfig }                              OPTIONAL,   -- Need M

...

}

-- TAG-BWP-DOWNLINKDEDICATED-STOP

-- ASN1STOP

BWP-DownlinkDedicated field descriptions

pdcch-Config UE specific PDCCH configuration for one BWP.

pdsch-Config UE specific PDSCH configuration for one BWP.

sps-Config UE specific SPS (Semi-Persistent Scheduling) configuration for one BWP. Except for reconfiguration with sync, the NW does not reconfigure sps-Config when there is an active configured downlink assignment (see TS 38.321 [3]). However, the NW may release the sps-Config at any time.

radioLinkMonitoringConfig UE specific configuration of radio link monitoring for detecting cell- and beam radio link failure occasions. The maximum number of failure detection resources should be limited up to 8 for both cell and beam radio link failure detection in Rel-15.

#### –	BWP-Id

The IE BWP-Id is used to refer to Bandwidth Parts (BWP). The initial BWP is referred to by BWP-Id 0. The other BWPs are referred to by BWP-Id 1 to maxNrofBWPs.

BWP-Id information element

-- ASN1START

-- TAG-BWP-ID-START

BWP-Id ::=                          INTEGER (0..maxNrofBWPs)

-- TAG-BWP-ID-STOP

-- ASN1STOP

#### –	BWP-Uplink

The IE BWP-Uplink is used to configure an additional uplink bandwidth part (not for the initial BWP).

BWP-Uplink information element

-- ASN1START

-- TAG-BWP-UPLINK-START

BWP-Uplink ::=                      SEQUENCE {

bwp-Id                              BWP-Id,

bwp-Common                          BWP-UplinkCommon                                            OPTIONAL,   -- Cond SetupOtherBWP

bwp-Dedicated                       BWP-UplinkDedicated                                         OPTIONAL,   -- Cond SetupOtherBWP

...

}

-- TAG-BWP-UPLINK-STOP

-- ASN1STOP

BWP-Uplink field descriptions

bwp-Id An identifier for this bandwidth part. Other parts of the RRC configuration use the BWP-Id to associate themselves with a particular bandwidth part. The network configures the BWPs with consecutive IDs from 1. The Network does not include the value 0, since value 0 is reserved for the initial BWP.

| Conditional Presence | Explanation |
| --- | --- |
| SetupOtherBWP | The field is mandatory present upon configuration of a new UL BWP. The field is optionally present, Need M, otherwise. |

#### –	BWP-UplinkCommon

The IE BWP-UplinkCommon is used to configure the common parameters of an uplink BWP. They are "cell specific" and the network ensures the necessary alignment with corresponding parameters of other UEs. The common parameters of the initial bandwidth part of the PCell are also provided via system information. For all other serving cells, the network provides the common parameters via dedicated signalling.

BWP-UplinkCommon information element

-- ASN1START

-- TAG-BWP-UPLINKCOMMON-START

BWP-UplinkCommon ::=                SEQUENCE {

genericParameters                   BWP,

rach-ConfigCommon                   SetupRelease { RACH-ConfigCommon }                                      OPTIONAL,   -- Need M

pusch-ConfigCommon                  SetupRelease { PUSCH-ConfigCommon }                                     OPTIONAL,   -- Need M

pucch-ConfigCommon                  SetupRelease { PUCCH-ConfigCommon }                                     OPTIONAL,   -- Need M

...

}

-- TAG-BWP-UPLINKCOMMON-STOP

-- ASN1STOP

BWP-UplinkCommon field descriptions

pucch-ConfigCommon Cell specific parameters for the PUCCH of this BWP.

pusch-ConfigCommon Cell specific parameters for the PUSCH of this BWP.

rach-ConfigCommon Configuration of cell specific random access parameters which the UE uses for contention based and contention free random access as well as for contention based beam failure recovery in this BWP. The NW configures SSB-based RA (and hence RACH-ConfigCommon) only for UL BWPs if the linked DL BWPs (same bwp-Id as UL-BWP) are the initial DL BWPs or DL BWPs containing the SSB associated to the initial DL BWP. The network configures rach-ConfigCommon, whenever it configures contention free random access (for reconfiguration with sync or for beam failure recovery).

#### –	BWP-UplinkDedicated

The IE BWP-UplinkDedicated is used to configure the dedicated (UE specific) parameters of an uplink BWP.

BWP-UplinkDedicated information element

-- ASN1START

-- TAG-BWP-UPLINKDEDICATED-START

BWP-UplinkDedicated ::=             SEQUENCE {

pucch-Config                        SetupRelease { PUCCH-Config }                                   OPTIONAL,   -- Need M

pusch-Config                        SetupRelease { PUSCH-Config }                                   OPTIONAL,   -- Need M

configuredGrantConfig               SetupRelease { ConfiguredGrantConfig }                          OPTIONAL,   -- Need M

srs-Config                          SetupRelease { SRS-Config }                                     OPTIONAL,   -- Need M

beamFailureRecoveryConfig           SetupRelease { BeamFailureRecoveryConfig }                      OPTIONAL,   -- Cond SpCellOnly

...

}

-- TAG-BWP-UPLINKDEDICATED-STOP

-- ASN1STOP

BWP-UplinkDedicated field descriptions

beamFailureRecoveryConfig Configuration of beam failure recovery. If supplementaryUplink is present, the field is present only in one of the uplink carriers, either UL or SUL.

configuredGrantConfig A Configured-Grant of type1 or type2. It may be configured for UL or SUL but in case of type1 not for both at a time. Except for reconfiguration with sync, the NW does not reconfigure configuredGrantConfig when there is an active configured uplink grant Type 2 (see TS 38.321 [3]). However, the NW may release the configuredGrantConfig at any time.

pucch-Config PUCCH configuration for one BWP of the normal UL or SUL of a serving cell. If the UE is configured with SUL, the network configures PUCCH only on the BWPs of one of the uplinks (normal UL or SUL). The network configures PUCCH-Config at least on non-initial BWP(s) for SpCell and PUCCH SCell. If supported by the UE, the network may configure at most one additional SCell of a cell group with PUCCH-Config (i.e. PUCCH SCell). In (NG)EN-DC and NE-DC, the NW configures at most one serving cell per frequency range with PUCCH. Iin (NG)EN-DC and NE-DC, if two PUCCH groups are configured, the serving cells of the NR PUCCH group in FR2 use the same numerology. For NR-DC, the maximum number of PUCCH groups in each cell group is one, and only the same numerology is supported for the cell group with carriers only in FR2. The NW may configure PUCCH for a BWP when setting up the BWP. The network may also add/remove the pucch-Config in an RRCReconfiguration with reconfigurationWithSync (for SpCell or PUCCH SCell) or with SCell release and add (for PUCCH SCell) to move the PUCCH between the UL and SUL carrier of one serving cell. In other cases, only modifications of a previously configured pucch-Config are allowed. If one (S)UL BWP of a serving cell is configured with PUCCH, all other (S)UL BWPs must be configured with PUCCH, too.

pusch-Config PUSCH configuration for one BWP of the normal UL or SUL of a serving cell. If the UE is configured with SUL and if it has a PUSCH-Config for both UL and SUL, an UL/SUL indicator field in DCI indicates which of the two to use. See TS 38.212 [17], clause 7.3.1.

srs-Config Uplink sounding reference signal configuration.

| Conditional Presence | Explanation |
| --- | --- |
| SpCellOnly | The field is optionally present, Need M, in the BWP-UplinkDedicated of an SpCell. It is absent otherwise. |

#### –	CellAccessRelatedInfo

The IE CellAccessRelatedInfo indicates cell access related information for this cell.

CellAccessRelatedInfo information element

-- ASN1START

-- TAG-CELLACCESSRELATEDINFO-START

CellAccessRelatedInfo   ::=         SEQUENCE {

plmn-IdentityList                   PLMN-IdentityInfoList,

cellReservedForOtherUse             ENUMERATED {true}  OPTIONAL,            -- Need R

...

}

-- TAG-CELLACCESSRELATEDINFO-STOP

-- ASN1STOP

CellAccessRelatedInfo field descriptions

cellReservedForOtherUse Indicates whether the cell is reserved, as defined in 38.304 [20]. The field is applicable to all PLMNs.

plmn-IdentityList The plmn-IdentityList is used to configure a set of PLMN-IdentityInfoList elements. Each of those elements contains a list of one or more PLMN Identities and additional information associated with those PLMNs. The total number of PLMNs in the PLMN-IdentityInfoList does not exceed 12. A PLMN-identity can be included only once, and in only one entry of the PLMN-IdentityInfoList. The PLMN index is defined as b1+b2+…+b(n-1)+i for the PLMN included at the n-th entry of PLMN-IdentityInfoList and the i-th entry of its corresponding PLMN-IdentityInfo, where b(j) is the number of PLMN-Identity entries in each PLMN-IdentityInfo, respectively.

#### –	CellAccessRelatedInfo-EUTRA-5GC

The IE CellAccessRelatedInfo-EUTRA-5GC indicates cell access related information for an LTE cell connected to 5GC.

CellAccessRelatedInfo-EUTRA-5GC information element

-- ASN1START

-- TAG-CELLACCESSRELATEDINFOEUTRA-5GC-START

CellAccessRelatedInfo-EUTRA-5GC  ::=    SEQUENCE {

plmn-IdentityList-eutra-5gc             PLMN-IdentityList-EUTRA-5GC,

trackingAreaCode-eutra-5gc              TrackingAreaCode,

ranac-5gc                               RAN-AreaCode                                OPTIONAL,

cellIdentity-eutra-5gc                  CellIdentity-EUTRA-5GC

}

PLMN-IdentityList-EUTRA-5GC::=          SEQUENCE (SIZE (1..maxPLMN)) OF PLMN-Identity-EUTRA-5GC

PLMN-Identity-EUTRA-5GC ::=             CHOICE {

plmn-Identity-EUTRA-5GC                 PLMN-Identity,

plmn-index                              INTEGER (1..maxPLMN)

}

CellIdentity-EUTRA-5GC ::=              CHOICE {

cellIdentity-EUTRA                      BIT STRING (SIZE (28)),

cellId-index                            INTEGER (1..maxPLMN)

}

-- TAG-CELLACCESSRELATEDINFOEUTRA-5GC-STOP

-- ASN1STOP

#### –	CellAccessRelatedInfo-EUTRA-EPC

The IE CellAccessRelatedInfo-EUTRA-EPC indicates cell access related information for an LTE cell connected to EPC.

CellAccessRelatedInfo-EUTRA-EPC information element

-- ASN1START

-- TAG-CELLACCESSRELATEDINFOEUTRA-EPC-START

CellAccessRelatedInfo-EUTRA-EPC  ::=    SEQUENCE {

plmn-IdentityList-eutra-epc             PLMN-IdentityList-EUTRA-EPC,

trackingAreaCode-eutra-epc              BIT STRING (SIZE (16)),

cellIdentity-eutra-epc                  BIT STRING (SIZE (28))

}

PLMN-IdentityList-EUTRA-EPC::=          SEQUENCE (SIZE (1..maxPLMN)) OF PLMN-Identity

-- TAG-CELLACCESSRELATEDINFOEUTRA-EPC-STOP

-- ASN1STOP

#### –	CellGroupConfig

The CellGroupConfig IE is used to configure a master cell group (MCG) or secondary cell group (SCG). A cell group comprises of one MAC entity, a set of logical channels with associated RLC entities and of a primary cell (SpCell) and one or more secondary cells (SCells).

CellGroupConfig information element

-- ASN1START

-- TAG-CELLGROUPCONFIG-START

-- Configuration of one Cell-Group:

CellGroupConfig ::=                         SEQUENCE {

cellGroupId                                 CellGroupId,

rlc-BearerToAddModList                      SEQUENCE (SIZE(1..maxLC-ID)) OF RLC-BearerConfig            OPTIONAL,   -- Need N

rlc-BearerToReleaseList                     SEQUENCE (SIZE(1..maxLC-ID)) OF LogicalChannelIdentity      OPTIONAL,   -- Need N

mac-CellGroupConfig                         MAC-CellGroupConfig                                         OPTIONAL,   -- Need M

physicalCellGroupConfig                     PhysicalCellGroupConfig                                     OPTIONAL,   -- Need M

spCellConfig                                SpCellConfig                                                OPTIONAL,   -- Need M

sCellToAddModList                           SEQUENCE (SIZE (1..maxNrofSCells)) OF SCellConfig           OPTIONAL,   -- Need N

sCellToReleaseList                          SEQUENCE (SIZE (1..maxNrofSCells)) OF SCellIndex            OPTIONAL,   -- Need N

...,

[[

reportUplinkTxDirectCurrent                 ENUMERATED {true}                                           OPTIONAL    -- Cond BWP-Reconfig

]]

}

-- Serving cell specific MAC and PHY parameters for a SpCell:

SpCellConfig ::=                        SEQUENCE {

servCellIndex                       ServCellIndex                                               OPTIONAL,   -- Cond SCG

reconfigurationWithSync             ReconfigurationWithSync                                     OPTIONAL,   -- Cond ReconfWithSync

rlf-TimersAndConstants              SetupRelease { RLF-TimersAndConstants }                     OPTIONAL,   -- Need M

rlmInSyncOutOfSyncThreshold         ENUMERATED {n1}                                             OPTIONAL,   -- Need S

spCellConfigDedicated               ServingCellConfig                                           OPTIONAL,   -- Need M

...

}

ReconfigurationWithSync ::=         SEQUENCE {

spCellConfigCommon                  ServingCellConfigCommon                                         OPTIONAL,   -- Need M

newUE-Identity                      RNTI-Value,

t304                                ENUMERATED {ms50, ms100, ms150, ms200, ms500, ms1000, ms2000, ms10000},

rach-ConfigDedicated                CHOICE {

uplink                              RACH-ConfigDedicated,

supplementaryUplink                 RACH-ConfigDedicated

}                                                                                               OPTIONAL,   -- Need N

...,

[[

smtc                                SSB-MTC                                                     OPTIONAL    -- Need S

]]

}

SCellConfig ::=                     SEQUENCE {

sCellIndex                          SCellIndex,

sCellConfigCommon                   ServingCellConfigCommon                                     OPTIONAL,   -- Cond SCellAdd

sCellConfigDedicated                ServingCellConfig                                           OPTIONAL,   -- Cond SCellAddMod

...,

[[

smtc                                SSB-MTC                                                     OPTIONAL    -- Need S

]]

}

-- TAG-CELLGROUPCONFIG-STOP

-- ASN1STOP

CellGroupConfig field descriptions

mac-CellGroupConfig MAC parameters applicable for the entire cell group.

rlc-BearerToAddModList Configuration of the MAC Logical Channel, the corresponding RLC entities and association with radio bearers.

reportUplinkTxDirectCurrent Enables reporting of uplink and supplementary uplink Direct Current location information upon BWP configuration and reconfiguration. This field is only present when the BWP configuration is modified or any serving cell is added or removed. This field is absent in the IE CellGroupConfig when provided as part of RRCSetup message. If UE is configured with SUL carrier, UE reports both UL and SUL Direct Current locations.

rlmInSyncOutOfSyncThreshold BLER threshold pair index for IS/OOS indication generation, see TS 38.133 [14], table 8.1.1-1. n1 corresponds to the value 1. When the field is absent, the UE applies the value 0. Whenever this is reconfigured, UE resets N310 and N311, and stops T310, if running. Network does not include this field.

sCellToAddModList List of seconary serving cells (SCells) to be added or modified.

sCellToReleaseList List of secondary serving cells (SCells) to be released.

spCellConfig Parameters for the SpCell of this cell group (PCell of MCG or PSCell of SCG).

ReconfigurationWithSync field descriptions

rach-ConfigDedicated Random access configuration to be used for the reconfiguration with sync (e.g. handover). The UE performs the RA according to these parameters in the firstActiveUplinkBWP (see UplinkConfig).

smtc The SSB periodicity/offset/duration configuration of target cell for NR PSCell change, NR PCell change, and NR PSCell addition. The network sets the periodicityAndOffset to indicate the same periodicity as ssb-periodicityServingCell in spCellConfigCommon. For case of NR PCell change and NR PSCell addition, the smtc is based on the timing reference of (source) PCell. For case of NR PSCell change, it is based on the timing reference of source PSCell. If the field is absent, the UE uses the SMTC in the measObjectNR having the same SSB frequency and subcarrier spacing, as configured before the reception of the RRC message.

SCellConfig field descriptions

smtc The SSB periodicity/offset/duration configuration of target cell for NR SCell addition. The network sets the periodicityAndOffset to indicate the same periodicity as ssb-periodicityServingCell in sCellConfigCommon. The smtc is based on the timing of the SpCell of associated cell group. In case of inter-RAT handover to NR, the timing reference is the NR PCell. In case of intra-NR PCell change (standalone NR) or NR PSCell change (EN-DC), the timing reference is the target SpCell. If the field is absent, the UE uses the SMTC in the measObjectNR having the same SSB frequency and subcarrier spacing, as configured before the reception of the RRC message.

SpCellConfig field descriptions

reconfigurationWithSync Parameters for the synchronous reconfiguration to the target SpCell.

rlf-TimersAndConstants Timers and constants for detecting and triggering cell-level radio link failure. For the SCG, rlf-TimersAndConstants can only be set to setup and is always included at SCG addition.

servCellIndex Serving cell ID of a PSCell. The PCell of the Master Cell Group uses ID = 0.

| Conditional Presence | Explanation |
| --- | --- |
| BWP-Reconfig | The field is optionally present, Need N, if the BWPs are reconfigured or if serving cells are added or removed. Otherwise it is absent. |
| ReconfWithSync | The field is mandatory present in the RRCReconfiguration message: - in each configured CellGroupConfig for which the SpCell changes, - in the masterCellGroup at change of AS security key derived from KgNB, - in the secondaryCellGroup at: - PSCell addition, - update of required SI for PSCell, - change of AS security key derived from S-KgNB while the UE is configured with at least one radio bearer with keyToUse set to secondary and that is not released by this RRCReconfiguration message. Otherwise it is optionally present, need M. The field is absent in RRCResume or RRCSetup messages. |
| SCellAdd | The field is mandatory present upon SCell addition; otherwise it is absent, Need M. |
| SCellAddMod | The field is mandatory present upon SCell addition; otherwise it is optionally present, need M. |
| SCG | The field is mandatory present in an SpCellConfig for the PSCell. It is absent otherwise. |

NOTE:	In case of change of AS security key derived from S-KgNB/S-KeNB, if reconfigurationWithSync is not included in the masterCellGroup, the network releases all existing MCG RLC bearers associated with a radio bearer with keyToUse set to secondary. In case of change of AS security key derived from KgNB/KeNB, if reconfigurationWithSync is not included in the secondaryCellGroup, the network releases all existing SCG RLC bearers associated with a radio bearer with keyToUse set to primary.

#### –	CellGroupId

The IE CellGroupId is used to identify a cell group. Value 0 identifies the master cell group. Other values identify secondary cell groups. In this version of the specification only values 0 and 1 are supported.

CellGroupId information element

-- ASN1START

-- TAG-CELLGROUPID-START

CellGroupId ::=                             INTEGER (0.. maxSecondaryCellGroups)

-- TAG-CELLGROUPID-STOP

-- ASN1STOP

#### –	CellIdentity

The IE CellIdentity is used to unambiguously identify a cell within a PLMN.

CellIdentity information element

-- ASN1START

-- TAG-CELLIDENTITY-START

CellIdentity ::=                         BIT STRING (SIZE (36))

-- TAG-CELLIDENTITY-STOP

-- ASN1STOP

#### –	CellReselectionPriority

The IE CellReselectionPriority concerns the absolute priority of the concerned carrier frequency, as used by the cell reselection procedure. Corresponds to parameter "priority" in TS 38.304 [20]. Value 0 means lowest priority. The UE behaviour for the case the field is absent, if applicable, is specified in TS 38.304 [20].

CellReselectionPriority information element

-- ASN1START

-- TAG-CELLRESELECTIONPRIORITY-START

CellReselectionPriority ::=             INTEGER (0..7)

-- TAG-CELLRESELECTIONPRIORITY-STOP

-- ASN1STOP

#### –	CellReselectionSubPriority

The IE CellReselectionSubPriority indicates a fractional value to be added to the value of cellReselectionPriority to obtain the absolute priority of the concerned carrier frequency for E-UTRA and NR. Value oDot2 corresponds to 0.2, value oDot4 corresponds to 0.4 and so on.

CellReselectionSubPriority information element

-- ASN1START

-- TAG-CELLRESELECTIONSUBPRIORITY-START

CellReselectionSubPriority ::=          ENUMERATED {oDot2, oDot4, oDot6, oDot8}

-- TAG-CELLRESELECTIONSUBPRIORITY-STOP

-- ASN1STOP

#### –	CGI-InfoEUTRA

The IE CGI-InfoEUTRA indicates EUTRA cell access related information, which is reported by the UE as part of E-UTRA report CGI procedure.

CGI-InfoEUTRA information element

-- ASN1START

-- TAG-CGI-INFOEUTRA-START

CGI-InfoEUTRA ::=                    SEQUENCE {

cgi-info-EPC                            SEQUENCE {

cgi-info-EPC-legacy                     CellAccessRelatedInfo-EUTRA-EPC,

cgi-info-EPC-list                       SEQUENCE (SIZE (1..maxPLMN)) OF CellAccessRelatedInfo-EUTRA-EPC     OPTIONAL

}                                                                                                                   OPTIONAL,

cgi-info-5GC                            SEQUENCE (SIZE (1..maxPLMN)) OF CellAccessRelatedInfo-EUTRA-5GC             OPTIONAL,

freqBandIndicator                       FreqBandIndicatorEUTRA,

multiBandInfoList                       MultiBandInfoListEUTRA                                                      OPTIONAL,

freqBandIndicatorPriority               ENUMERATED {true}                                                           OPTIONAL

}

-- TAG-CGI-INFOEUTRA-STOP

-- ASN1STOP

#### –	CGI-InfoNR

The IE CGI-InfoNR indicates cell access related information, which is reported by the UE as part of report CGI procedure.

CGI-InfoNR information element

-- ASN1START

-- TAG-CGI-INFO-NR-START

CGI-InfoNR ::=                    SEQUENCE {

plmn-IdentityInfoList               PLMN-IdentityInfoList               OPTIONAL,

frequencyBandList                   MultiFrequencyBandListNR            OPTIONAL,

noSIB1                              SEQUENCE {

ssb-SubcarrierOffset                INTEGER (0..15),

pdcch-ConfigSIB1                    PDCCH-ConfigSIB1

}                                                                       OPTIONAL,

...

}

-- TAG-CGI-INFO-NR-STOP

-- ASN1STOP

CGI-InfoNR field descriptions

noSIB1 Contains ssb-SubcarrierOffset and pdcch-ConfigSIB1 fields acquired by the UE from MIB of the cell for which report CGI procedure was requested by the network in case SIB1 was not broadcast by the cell.

#### –	CodebookConfig

The IE CodebookConfig is used to configure codebooks of Type-I and Type-II (see TS 38.214 [19], clause 5.2.2.2)

CodebookConfig information element

-- ASN1START

-- TAG-CODEBOOKCONFIG-START

CodebookConfig ::=                                  SEQUENCE {

codebookType                                        CHOICE {

type1                                               SEQUENCE {

subType                                             CHOICE {

typeI-SinglePanel                                   SEQUENCE {

nrOfAntennaPorts                                    CHOICE {

two                                                 SEQUENCE {

twoTX-CodebookSubsetRestriction                     BIT STRING (SIZE (6))

},

moreThanTwo                                         SEQUENCE {

n1-n2                                               CHOICE {

two-one-TypeI-SinglePanel-Restriction               BIT STRING (SIZE (8)),

two-two-TypeI-SinglePanel-Restriction               BIT STRING (SIZE (64)),

four-one-TypeI-SinglePanel-Restriction              BIT STRING (SIZE (16)),

three-two-TypeI-SinglePanel-Restriction             BIT STRING (SIZE (96)),

six-one-TypeI-SinglePanel-Restriction               BIT STRING (SIZE (24)),

four-two-TypeI-SinglePanel-Restriction              BIT STRING (SIZE (128)),

eight-one-TypeI-SinglePanel-Restriction             BIT STRING (SIZE (32)),

four-three-TypeI-SinglePanel-Restriction            BIT STRING (SIZE (192)),

six-two-TypeI-SinglePanel-Restriction               BIT STRING (SIZE (192)),

twelve-one-TypeI-SinglePanel-Restriction            BIT STRING (SIZE (48)),

four-four-TypeI-SinglePanel-Restriction             BIT STRING (SIZE (256)),

eight-two-TypeI-SinglePanel-Restriction             BIT STRING (SIZE (256)),

sixteen-one-TypeI-SinglePanel-Restriction           BIT STRING (SIZE (64))

},

typeI-SinglePanel-codebookSubsetRestriction-i2      BIT STRING (SIZE (16))        OPTIONAL    -- Need R

}

},

typeI-SinglePanel-ri-Restriction                    BIT STRING (SIZE (8))

},

typeI-MultiPanel                                    SEQUENCE {

ng-n1-n2                                                CHOICE {

two-two-one-TypeI-MultiPanel-Restriction                BIT STRING (SIZE (8)),

two-four-one-TypeI-MultiPanel-Restriction               BIT STRING (SIZE (16)),

four-two-one-TypeI-MultiPanel-Restriction               BIT STRING (SIZE (8)),

two-two-two-TypeI-MultiPanel-Restriction                BIT STRING (SIZE (64)),

two-eight-one-TypeI-MultiPanel-Restriction              BIT STRING (SIZE (32)),

four-four-one-TypeI-MultiPanel-Restriction              BIT STRING (SIZE (16)),

two-four-two-TypeI-MultiPanel-Restriction               BIT STRING (SIZE (128)),

four-two-two-TypeI-MultiPanel-Restriction               BIT STRING (SIZE (64))

},

ri-Restriction                          BIT STRING (SIZE (4))

}

},

codebookMode                                        INTEGER (1..2)

},

type2                                   SEQUENCE {

subType                                 CHOICE {

typeII                                  SEQUENCE {

n1-n2-codebookSubsetRestriction         CHOICE {

two-one                                 BIT STRING (SIZE (16)),

two-two                                 BIT STRING (SIZE (43)),

four-one                                BIT STRING (SIZE (32)),

three-two                               BIT STRING (SIZE (59)),

six-one                                 BIT STRING (SIZE (48)),

four-two                                BIT STRING (SIZE (75)),

eight-one                               BIT STRING (SIZE (64)),

four-three                              BIT STRING (SIZE (107)),

six-two                                 BIT STRING (SIZE (107)),

twelve-one                              BIT STRING (SIZE (96)),

four-four                               BIT STRING (SIZE (139)),

eight-two                               BIT STRING (SIZE (139)),

sixteen-one                             BIT STRING (SIZE (128))

},

typeII-RI-Restriction                   BIT STRING (SIZE (2))

},

typeII-PortSelection                    SEQUENCE {

portSelectionSamplingSize               ENUMERATED {n1, n2, n3, n4}                   OPTIONAL,       -- Need R

typeII-PortSelectionRI-Restriction      BIT STRING (SIZE (2))

}

},

phaseAlphabetSize                       ENUMERATED {n4, n8},

subbandAmplitude                        BOOLEAN,

numberOfBeams                           ENUMERATED {two, three, four}

}

}

}

-- TAG-CODEBOOKCONFIG-STOP

-- ASN1STOP

CodebookConfig field descriptions

codebookMode CodebookMode as specified in TS 38.214 [19], clause 5.2.2.2.2.

codebookType CodebookType including possibly sub-types and the corresponding parameters for each (see TS 38.214 [19], clause 5.2.2.2).

n1-n2-codebookSubsetRestriction Number of antenna ports in first (n1) and second (n2) dimension and codebook subset restriction (see TS 38.214 [19] clause 5.2.2.2.3). Number of bits for codebook subset restriction is CEIL(log2(nchoosek(O1*O2,4)))+8*n1*n2 where nchoosek(a,b) = a!/(b!(a-b)!).

n1-n2 Number of antenna ports in first (n1) and second (n2) dimension and codebook subset restriction (see TS 38.214 [19] clause 5.2.2.2.1).

ng-n1-n2 Codebook subset restriction for Type I Multi-panel codebook (see TS 38.214 [19], clause 5.2.2.2.2).

numberOfBeams Number of beams, L, used for linear combination.

phaseAlphabetSize The size of the PSK alphabet, QPSK or 8-PSK.

portSelectionSamplingSize The size of the port selection codebook (parameter d).

ri-Restriction Restriction for RI for TypeI-MultiPanel-RI-Restriction (see TS 38.214 [19], clause 5.2.2.2.2).

subbandAmplitude If subband amplitude reporting is activated (true).

twoTX-CodebookSubsetRestriction Codebook subset restriction for 2TX codebook (see TS 38.214 [19] clause 5.2.2.2.1).

typeI-SinglePanel-codebookSubsetRestriction-i2 i2 codebook subset restriction for Type I Single-panel codebook used when reportQuantity is CRI/Ri/i1/CQI (see TS 38.214 [19] clause 5.2.2.2.1).

typeI-SinglePanel-ri-Restriction Restriction for RI for TypeI-SinglePanel-RI-Restriction (see TS 38.214 [19], clause 5.2.2.2.1).

typeII-PortSelectionRI-Restriction Restriction for RI for TypeII-PortSelection-RI-Restriction (see TS 38.214 [19], clause 5.2.2.4).

typeII-RI-Restriction Restriction for RI for TypeII-RI-Restriction (see TS 38.214 [19], clause 5.2.2.2.3).

#### –	ConfiguredGrantConfig

The IE ConfiguredGrantConfig is used to configure uplink transmission without dynamic grant according to two possible schemes. The actual uplink grant may either be configured via RRC (type1) or provided via the PDCCH (addressed to CS-RNTI) (type2).

ConfiguredGrantConfig information element

-- ASN1START

-- TAG-CONFIGUREDGRANTCONFIG-START

ConfiguredGrantConfig ::=           SEQUENCE {

frequencyHopping                    ENUMERATED {intraSlot, interSlot}                                       OPTIONAL,   -- Need S

cg-DMRS-Configuration               DMRS-UplinkConfig,

mcs-Table                           ENUMERATED {qam256, qam64LowSE}                                         OPTIONAL,   -- Need S

mcs-TableTransformPrecoder          ENUMERATED {qam256, qam64LowSE}                                         OPTIONAL,   -- Need S

uci-OnPUSCH                         SetupRelease { CG-UCI-OnPUSCH }                                         OPTIONAL,   -- Need M

resourceAllocation                  ENUMERATED { resourceAllocationType0, resourceAllocationType1, dynamicSwitch },

rbg-Size                            ENUMERATED {config2}                                                    OPTIONAL,   -- Need S

powerControlLoopToUse               ENUMERATED {n0, n1},

p0-PUSCH-Alpha                      P0-PUSCH-AlphaSetId,

transformPrecoder                   ENUMERATED {enabled, disabled}                                          OPTIONAL,   -- Need S

nrofHARQ-Processes                  INTEGER(1..16),

repK                                ENUMERATED {n1, n2, n4, n8},

repK-RV                             ENUMERATED {s1-0231, s2-0303, s3-0000}                                  OPTIONAL,   -- Need R

periodicity                         ENUMERATED {

sym2, sym7, sym1x14, sym2x14, sym4x14, sym5x14, sym8x14, sym10x14, sym16x14, sym20x14,

sym32x14, sym40x14, sym64x14, sym80x14, sym128x14, sym160x14, sym256x14, sym320x14, sym512x14,

sym640x14, sym1024x14, sym1280x14, sym2560x14, sym5120x14,

sym6, sym1x12, sym2x12, sym4x12, sym5x12, sym8x12, sym10x12, sym16x12, sym20x12, sym32x12,

sym40x12, sym64x12, sym80x12, sym128x12, sym160x12, sym256x12, sym320x12, sym512x12, sym640x12,

sym1280x12, sym2560x12

},

configuredGrantTimer                    INTEGER (1..64)                                                     OPTIONAL,   -- Need R

rrc-ConfiguredUplinkGrant               SEQUENCE {

timeDomainOffset                        INTEGER (0..5119),

timeDomainAllocation                    INTEGER  (0..15),

frequencyDomainAllocation               BIT STRING (SIZE(18)),

antennaPort                             INTEGER (0..31),

dmrs-SeqInitialization                  INTEGER (0..1)                                                  OPTIONAL,   -- Need R

precodingAndNumberOfLayers              INTEGER (0..63),

srs-ResourceIndicator                   INTEGER (0..15)                                                 OPTIONAL,   -- Need R

mcsAndTBS                               INTEGER (0..31),

frequencyHoppingOffset                  INTEGER (1.. maxNrofPhysicalResourceBlocks-1)                   OPTIONAL,   -- Need R

pathlossReferenceIndex                  INTEGER (0..maxNrofPUSCH-PathlossReferenceRSs-1),

...

}                                                                                                           OPTIONAL,   -- Need R

...

}

CG-UCI-OnPUSCH ::= CHOICE {

dynamic                                 SEQUENCE (SIZE (1..4)) OF BetaOffsets,

semiStatic                              BetaOffsets

}

-- TAG-CONFIGUREDGRANTCONFIG-STOP

-- ASN1STOP

ConfiguredGrantConfig field descriptions

antennaPort Indicates the antenna port(s) to be used for this configuration, and the maximum bitwidth is 5. See TS 38.214 [19], clause 6.1.2, and TS 38.212 [17], clause 7.3.1.

cg-DMRS-Configuration DMRS configuration (see TS 38.214 [19], clause 6.1.2.3).

configuredGrantTimer Indicates the initial value of the configured grant timer (see TS 38.321 [3]) in multiples of periodicity.

dmrs-SeqInitialization The network configures this field if transformPrecoder is disabled. Otherwise the field is absent.

frequencyDomainAllocation Indicates the frequency domain resource allocation, see TS 38.214 [19], clause 6.1.2, and TS 38.212 [17], clause 7.3.1).

frequencyHopping The value intraSlot enables 'Intra-slot frequency hopping' and the value interSlot enables 'Inter-slot frequency hopping'. If the field is absent, frequency hopping is not configured.

frequencyHoppingOffset Frequency hopping offset used when frequency hopping is enabled (see TS 38.214 [19], clause 6.1.2 and clause 6.3).

mcs-Table Indicates the MCS table the UE shall use for PUSCH without transform precoding. If the field is absent the UE applies the value qam64.

mcs-TableTransformPrecoder Indicates the MCS table the UE shall use for PUSCH with transform precoding. If the field is absent the UE applies the value qam64.

mcsAndTBS The modulation order, target code rate and TB size (see TS 38.214 [19], clause 6.1.2). The NW does not configure the values 28~31 in this version of the specification.

nrofHARQ-Processes The number of HARQ processes configured. It applies for both Type 1 and Type 2. See TS 38.321 [3], clause 5.4.1.

p0-PUSCH-Alpha Index of the P0-PUSCH-AlphaSet to be used for this configuration.

periodicity Periodicity for UL transmission without UL grant for type 1 and type 2 (see TS 38.321 [3], clause 5.8.2). The following periodicities are supported depending on the configured subcarrier spacing [symbols]: 15 kHz: 2, 7, n*14, where n={1, 2, 4, 5, 8, 10, 16, 20, 32, 40, 64, 80, 128, 160, 320, 640} 30 kHz: 2, 7, n*14, where n={1, 2, 4, 5, 8, 10, 16, 20, 32, 40, 64, 80, 128, 160, 256, 320, 640, 1280} 60 kHz with normal CP 2, 7, n*14, where n={1, 2, 4, 5, 8, 10, 16, 20, 32, 40, 64, 80, 128, 160, 256, 320, 512, 640, 1280, 2560} 60 kHz with ECP: 2, 6, n*12, where n={1, 2, 4, 5, 8, 10, 16, 20, 32, 40, 64, 80, 128, 160, 256, 320, 512, 640, 1280, 2560} 120 kHz: 2, 7, n*14, where n={1, 2, 4, 5, 8, 10, 16, 20, 32, 40, 64, 80, 128, 160, 256, 320, 512, 640, 1024, 1280, 2560, 5120}

powerControlLoopToUse Closed control loop to apply (see TS 38.213 [13], clause 7.1.1).

rbg-Size Selection between configuration 1 and configuration 2 for RBG size for PUSCH. The UE does not apply this field if resourceAllocation is set to resourceAllocationType1. Otherwise, the UE applies the value config1 when the field is absent. Note: rbg-Size is used when the transformPrecoder parameter is disabled.

repK-RV The redundancy version (RV) sequence to use. See TS 38.214 [19], clause 6.1.2. The network configures this field if repetitions are used, i.e., if repK is set to n2, n4 or n8. Otherwise, the field is absent.

repK The number of repetitions of K.

resourceAllocation Configuration of resource allocation type 0 and resource allocation type 1. For Type 1 UL data transmission without grant, resourceAllocation should be resourceAllocationType0 or resourceAllocationType1.

rrc-ConfiguredUplinkGrant Configuration for "configured grant" transmission with fully RRC-configured UL grant (Type1). If this field is absent the UE uses UL grant configured by DCI addressed to CS-RNTI (Type2). Type 1 configured grant may be configured for UL or SUL, but not for both simultaneously.

srs-ResourceIndicator Indicates the SRS resource to be used.

timeDomainAllocation Indicates a combination of start symbol and length and PUSCH mapping type, see TS 38.214 [19], clause 6.1.2 and TS 38.212 [17], clause 7.3.1.

timeDomainOffset Offset related to SFN=0, see TS 38.321 [3], clause 5.8.2.

transformPrecoder Enables or disables transform precoding for type1 and type2. If the field is absent, the UE enables or disables transform precoding in accordance with the field msg3-transformPrecoder in RACH-ConfigCommon, see TS 38.214 [19], clause 6.1.3.

uci-OnPUSCH Selection between and configuration of dynamic and semi-static beta-offset. For Type 1 UL data transmission without grant, uci-OnPUSCH should be set to semiStatic.

#### –	ConnEstFailureControl

The IE ConnEstFailureControl is used to configure parameters for connection establishment failure control.

ConnEstFailureControl information element

-- ASN1START

-- TAG-CONNESTFAILURECONTROL-START

ConnEstFailureControl ::=   SEQUENCE {

connEstFailCount                    ENUMERATED {n1, n2, n3, n4},

connEstFailOffsetValidity           ENUMERATED {s30, s60, s120, s240, s300, s420, s600, s900},

connEstFailOffset                   INTEGER (0..15)                                                         OPTIONAL    -- Need S

}

-- TAG-CONNESTFAILURECONTROL-STOP

-- ASN1STOP

ConnEstFailureControl field descriptions

connEstFailCount Number of times that the UE detects T300 expiry on the same cell before applying connEstFailOffset.

connEstFailOffset Parameter "Qoffsettemp" in TS 38.304 [20]. If the field is absent, the value of infinity shall be used for "Qoffsettemp".

connEstFailOffsetValidity Amount of time that the UE applies connEstFailOffset before removing the offset from evaluation of the cell. Value s30 corresponds to 30 seconds, value s60 corresponds to 60 seconds, and so on.

#### –	ControlResourceSet

The IE ControlResourceSet is used to configure a time/frequency control resource set (CORESET) in which to search for downlink control information (see TS 38.213 [13], clause 10.1).

ControlResourceSet information element

-- ASN1START

-- TAG-CONTROLRESOURCESET-START

ControlResourceSet ::=              SEQUENCE {

controlResourceSetId                ControlResourceSetId,

frequencyDomainResources            BIT STRING (SIZE (45)),

duration                            INTEGER (1..maxCoReSetDuration),

cce-REG-MappingType                 CHOICE {

interleaved                         SEQUENCE {

reg-BundleSize                      ENUMERATED {n2, n3, n6},

interleaverSize                     ENUMERATED {n2, n3, n6},

shiftIndex                          INTEGER(0..maxNrofPhysicalResourceBlocks-1)       OPTIONAL -- Need S

},

nonInterleaved                      NULL

},

precoderGranularity                 ENUMERATED {sameAsREG-bundle, allContiguousRBs},

tci-StatesPDCCH-ToAddList           SEQUENCE(SIZE (1..maxNrofTCI-StatesPDCCH)) OF TCI-StateId OPTIONAL, -- Cond NotSIB1-initialBWP

tci-StatesPDCCH-ToReleaseList       SEQUENCE(SIZE (1..maxNrofTCI-StatesPDCCH)) OF TCI-StateId OPTIONAL, -- Cond NotSIB1-initialBWP

tci-PresentInDCI                        ENUMERATED {enabled}                                  OPTIONAL, -- Need S

pdcch-DMRS-ScramblingID                 INTEGER (0..65535)                                    OPTIONAL, -- Need S

...

}

-- TAG-CONTROLRESOURCESET-STOP

-- ASN1STOP

ControlResourceSet field descriptions

cce-REG-MappingType Mapping of Control Channel Elements (CCE) to Resource Element Groups (REG) (see TS 38.211 [16], clauses 7.3.2.2 and 7.4.1.3.2).

controlResourceSetId Value 0 identifies the common CORESET configured in MIB and in ServingCellConfigCommon (controlResourceSetZero) and is hence not used here in the ControlResourceSet IE. Values 1..maxNrofControlResourceSets-1 identify CORESETs configured by dedicated signalling or in SIB1. The controlResourceSetId is unique among the BWPs of a serving cell.

duration Contiguous time duration of the CORESET in number of symbols (see TS 38.211 [16], clause 7.3.2.2).

frequencyDomainResources Frequency domain resources for the CORESET. Each bit corresponds a group of 6 RBs, with grouping starting from the first RB group (see TS 38.213 [13], clause 10.1) in the BWP. The first (left-most / most significant) bit corresponds to the first RB group in the BWP, and so on. A bit that is set to 1 indicates that this RB group belongs to the frequency domain resource of this CORESET. Bits corresponding to a group of RBs not fully contained in the bandwidth part within which the CORESET is configured are set to zero (see TS 38.211 [16], clause 7.3.2.2).

interleaverSize Interleaver-size (see TS 38.211 [16], clause 7.3.2.2).

pdcch-DMRS-ScramblingID PDCCH DMRS scrambling initialization (see TS 38.211 [16], clause 7.4.1.3.1). When the field is absent the UE applies the value of the physCellId configured for this serving cell.

precoderGranularity Precoder granularity in frequency domain (see TS 38.211 [16], clauses 7.3.2.2 and 7.4.1.3.2).

reg-BundleSize Resource Element Groups (REGs) can be bundled to create REG bundles. This parameter defines the size of such bundles (see TS 38.211 [16], clause 7.3.2.2).

shiftIndex When the field is absent the UE applies the value of the physCellIdconfigured for this serving cell (see TS 38.211 [16], clause 7.3.2.2).

tci-PresentInDCI This field indicates if TCI field is present or absent in DL-related DCI. When the field is absent the UE considers the TCI to be absent/disabled. In case of cross carrier scheduling, the network sets this field to enabled for the ControlResourceSet used for cross carrier scheduling in the scheduling cell (see TS 38.214 [19], clause 5.1.5).

tci-StatesPDCCH-ToAddList A subset of the TCI states defined in pdsch-Config included in the BWP-DownlinkDedicated corresponding to the serving cell and to the DL BWP to which the ControlResourceSet belong to. They are used for providing QCL relationships between the DL RS(s) in one RS Set (TCI-State) and the PDCCH DMRS ports (see TS 38.213 [13], clause 6.). The network configures at most maxNrofTCI-StatesPDCCH entries.

| Conditional Presence | Explanation |
| --- | --- |
| NotSIB1-initialBWP | The field is absent in SIB1 and in the PDCCH-ConfigCommon of the initial BWP in ServingCellConfigCommon, if SIB1 is broadcasted. Otherwise, it is optionally present, Need N. |

#### –	ControlResourceSetId

The ControlResourceSetId IE concerns a short identity, used to identify a control resource set within a serving cell. The ControlResourceSetId = 0 identifies the ControlResourceSet#0 configured via PBCH (MIB) and in controlResourceSetZero (ServingCellConfigCommon). The ID space is used across the BWPs of a Serving Cell. The number of CORESETs per BWP is limited to 3 (including common and UE-specific CORESETs).

ControlResourceSetId information element

-- ASN1START

-- TAG-CONTROLRESOURCESETID-START

ControlResourceSetId ::=                INTEGER (0..maxNrofControlResourceSets-1)

-- TAG-CONTROLRESOURCESETID-STOP

-- ASN1STOP

#### –	ControlResourceSetZero

The IE ControlResourceSetZero is used to configure CORESET#0 of the initial BWP (see TS 38.213 [13], clause 13).

ControlResourceSetZero information element

-- ASN1START

-- TAG-CONTROLRESOURCESETZERO-START

ControlResourceSetZero ::=                  INTEGER (0..15)

-- TAG-CONTROLRESOURCESETZERO-STOP

-- ASN1STOP

#### –	CrossCarrierSchedulingConfig

The IE CrossCarrierSchedulingConfig is used to specify the configuration when the cross-carrier scheduling is used in a cell.

CrossCarrierSchedulingConfig information element

-- ASN1START

-- TAG-CrossCarrierSchedulingConfig-START

CrossCarrierSchedulingConfig ::=        SEQUENCE {

schedulingCellInfo                      CHOICE {

own                                     SEQUENCE {                  -- Cross carrier scheduling: scheduling cell

cif-Presence                            BOOLEAN

},

other                                   SEQUENCE {                  -- Cross carrier scheduling: scheduled cell

schedulingCellId                        ServCellIndex,

cif-InSchedulingCell                    INTEGER (1..7)

}

},

...

}

-- TAG-CrossCarrierSchedulingConfig-STOP

-- ASN1STOP

CrossCarrierSchedulingConfig field descriptions

cif-Presence The field is used to indicate whether carrier indicator field is present (value true) or not (value false) in PDCCH DCI formats, see TS 38.213 [13]. If cif-Presence is set to true, the CIF value indicating a grant or assignment for this cell is 0.

cif-InSchedulingCell The field indicates the CIF value used in the scheduling cell to indicate a grant or assignment applicable for this cell, see TS 38.213 [13].

other Parameters for cross-carrier scheduling, i.e., a serving cell is scheduled by a PDCCH on another (scheduling) cell. The network configures this field only for SCells.

own Parameters for self-scheduling, i.e., a serving cell is scheduled by its own PDCCH.

schedulingCellId Indicates which cell signals the downlink allocations and uplink grants, if applicable, for the concerned SCell. In case the UE is configured with DC, the scheduling cell is part of the same cell group (i.e. MCG or SCG) as the scheduled cell.

#### –	CSI-AperiodicTriggerStateList

The CSI-AperiodicTriggerStateList IE is used to configure the UE with a list of aperiodic trigger states. Each codepoint of the DCI field "CSI request" is associated with one trigger state (see TS 38.321 [3], clause 6.1.3.13). Upon reception of the value associated with a trigger state, the UE will perform measurement of CSI-RS, CSI-IM and/or SSB (reference signals) and aperiodic reporting on L1 according to all entries in the associatedReportConfigInfoList for that trigger state.

CSI-AperiodicTriggerStateList information element

-- ASN1START

-- TAG-CSI-APERIODICTRIGGERSTATELIST-START

CSI-AperiodicTriggerStateList ::=   SEQUENCE (SIZE (1..maxNrOfCSI-AperiodicTriggers)) OF CSI-AperiodicTriggerState

CSI-AperiodicTriggerState ::=       SEQUENCE {

associatedReportConfigInfoList      SEQUENCE (SIZE(1..maxNrofReportConfigPerAperiodicTrigger)) OF CSI-AssociatedReportConfigInfo,

...

}

CSI-AssociatedReportConfigInfo ::=  SEQUENCE {

reportConfigId                      CSI-ReportConfigId,

resourcesForChannel                 CHOICE {

nzp-CSI-RS                          SEQUENCE {

resourceSet                         INTEGER (1..maxNrofNZP-CSI-RS-ResourceSetsPerConfig),

qcl-info                            SEQUENCE (SIZE(1..maxNrofAP-CSI-RS-ResourcesPerSet)) OF TCI-StateId

OPTIONAL  -- Cond Aperiodic

},

csi-SSB-ResourceSet                 INTEGER (1..maxNrofCSI-SSB-ResourceSetsPerConfig)

},

csi-IM-ResourcesForInterference     INTEGER(1..maxNrofCSI-IM-ResourceSetsPerConfig)

OPTIONAL, -- Cond CSI-IM-ForInterference

nzp-CSI-RS-ResourcesForInterference INTEGER (1..maxNrofNZP-CSI-RS-ResourceSetsPerConfig)

OPTIONAL, -- Cond NZP-CSI-RS-ForInterference

...

}

-- TAG-CSI-APERIODICTRIGGERSTATELIST-STOP

-- ASN1STOP

CSI-AssociatedReportConfigInfo field descriptions

csi-IM-ResourcesForInterference CSI-IM-ResourceSet for interference measurement. Entry number in csi-IM-ResourceSetList in the CSI-ResourceConfig indicated by csi-IM-ResourcesForInterference in the CSI-ReportConfig indicated by reportConfigId above (value 1 corresponds to the first entry, value 2 to the second entry, and so on). The indicated CSI-IM-ResourceSet should have exactly the same number of resources like the NZP-CSI-RS-ResourceSet indicated in nzp-CSI-RS-ResourcesforChannel.

csi-SSB-ResourceSet CSI-SSB-ResourceSet for channel measurements. Entry number in csi-SSB-ResourceSetList in the CSI-ResourceConfig indicated by resourcesForChannelMeasurement in the CSI-ReportConfig indicated by reportConfigId above (value 1 corresponds to the first entry, value 2 to the second entry, and so on).

nzp-CSI-RS-ResourcesForInterference NZP-CSI-RS-ResourceSet for interference measurement. Entry number in nzp-CSI-RS-ResourceSetList in the CSI-ResourceConfig indicated by nzp-CSI-RS-ResourcesForInterference in the CSI-ReportConfig indicated by reportConfigId above (value 1 corresponds to the first entry, value 2 to the second entry, and so on).

qcl-info List of references to TCI-States for providing the QCL source and QCL type for each NZP-CSI-RS-Resource listed in nzp-CSI-RS-Resources of the NZP-CSI-RS-ResourceSet indicated by nzp-CSI-RS-ResourcesforChannel. Each TCI-StateId refers to the TCI-State which has this value for tci-StateId and is defined in tci-StatesToAddModList in the PDSCH-Config included in the BWP-Downlink corresponding to the serving cell and to the DL BWP to which the resourcesForChannelMeasurement (in the CSI-ReportConfig indicated by reportConfigId above) belong to. First entry in qcl-info-forChannel corresponds to first entry in nzp-CSI-RS-Resources of that NZP-CSI-RS-ResourceSet, second entry in qcl-info-forChannel corresponds to second entry in nzp-CSI-RS-Resources, and so on (see TS 38.214 [19], clause 5.2.1.5.1)

reportConfigId The reportConfigId of one of the CSI-ReportConfigToAddMod configured in CSI-MeasConfig

resourceSet NZP-CSI-RS-ResourceSet for channel measurements. Entry number in nzp-CSI-RS-ResourceSetList in the CSI-ResourceConfig indicated by resourcesForChannelMeasurement in the CSI-ReportConfig indicated by reportConfigId above (value 1 corresponds to the first entry, value 2 to thesecond entry, and so on).

| Conditional Presence | Explanation |
| --- | --- |
| Aperiodic | The field is mandatory present if the NZP-CSI-RS-Resources in the associated resourceSet have the resourceType aperiodic. The field is absent otherwise. |
| CSI-IM-ForInterference | This field is mandatory present if the CSI-ReportConfig identified by reportConfigId is configured with csi-IM-ResourcesForInterference; otherwise it is absent. |
| NZP-CSI-RS-ForInterference | This field is mandatory present if the CSI-ReportConfig identified by reportConfigId is configured with nzp-CSI-RS-ResourcesForInterference; otherwise it is absent. |

#### –	CSI-FrequencyOccupation

The IE CSI-FrequencyOccupation is used to configure the frequency domain occupation of a channel state information measurement resource (e.g. NZP-CSI-RS-Resource, CSI-IM-Resource).

CSI-FrequencyOccupation information element

-- ASN1START

-- TAG-CSI-FREQUENCYOCCUPATION-START

CSI-FrequencyOccupation ::=         SEQUENCE {

startingRB                          INTEGER (0..maxNrofPhysicalResourceBlocks-1),

nrofRBs                             INTEGER (24..maxNrofPhysicalResourceBlocksPlus1),

...

}

-- TAG-CSI-FREQUENCYOCCUPATION-STOP

-- ASN1STOP

CSI-FrequencyOccupation field descriptions

nrofRBs Number of PRBs across which this CSI resource spans. Only multiples of 4 are allowed. The smallest configurable number is the minimum of 24 and the width of the associated BWP. If the configured value is larger than the width of the corresponding BWP, the UE shall assume that the actual CSI-RS bandwidth is equal to the width of the BWP.

startingRB PRB where this CSI resource starts in relation to common resource block #0 (CRB#0) on the common resource block grid. Only multiples of 4 are allowed (0, 4, ...)

#### –	CSI-IM-Resource

The IE CSI-IM-Resource is used to configure one CSI Interference Management (IM) resource.

CSI-IM-Resource information element

-- ASN1START

-- TAG-CSI-IM-RESOURCE-START

CSI-IM-Resource ::=                 SEQUENCE {

csi-IM-ResourceId                   CSI-IM-ResourceId,

csi-IM-ResourceElementPattern           CHOICE {

pattern0                                SEQUENCE {

subcarrierLocation-p0                   ENUMERATED { s0, s2, s4, s6, s8, s10 },

symbolLocation-p0                       INTEGER (0..12)

},

pattern1                                SEQUENCE {

subcarrierLocation-p1                   ENUMERATED { s0, s4, s8 },

symbolLocation-p1                       INTEGER (0..13)

}

}                                                                                   OPTIONAL,   -- Need M

freqBand                            CSI-FrequencyOccupation                         OPTIONAL,   -- Need M

periodicityAndOffset                CSI-ResourcePeriodicityAndOffset                OPTIONAL,   -- Cond PeriodicOrSemiPersistent

...

}

-- TAG-CSI-IM-RESOURCE-STOP

-- ASN1STOP

CSI-IM-Resource field descriptions

csi-IM-ResourceElementPattern The resource element pattern (Pattern0 (2,2) or Pattern1 (4,1)) with corresponding parameters (see TS 38.214 [19], clause 5.2.2.4)

freqBand Frequency-occupancy of CSI-IM (see TS 38.214 [19], clause 5.2.2.4)

periodicityAndOffset Periodicity and slot offset for periodic/semi-persistent CSI-IM. Network always configures the UE with a value for this field for periodic and semi-persistent CSI-IM-Resources (as indicated in CSI-ResourceConfig). A change of configuration between periodic or semi-persistent and aperiodic for a CSI-IM-Resource is not supported without a release and add.

subcarrierLocation-p0 OFDM subcarrier occupancy of the CSI-IM resource for Pattern0 (see TS 38.214 [19], clause 5.2.2.4)

subcarrierLocation-p1 OFDM subcarrier occupancy of the CSI-IM resource for Pattern1 (see TS 38.214 [19], clause 5.2.2.4)

symbolLocation-p0 OFDM symbol location of the CSI-IM resource for Pattern0 (see TS 38.214 [19], clause 5.2.2.4)

symbolLocation-p1 OFDM symbol location of the CSI-IM resource for Pattern1 (see TS 38.214 [19], clause 5.2.2.4)

| Conditional Presence | Explanation |
| --- | --- |
| PeriodicOrSemiPersistent | The field is optionally present, Need M, for periodic and semi-persistent CSI-IM-Resources (as indicated in CSI-ResourceConfig). The field is absent otherwise. |

#### –	CSI-IM-ResourceId

The IE CSI-IM-ResourceId is used to identify one CSI-IM-Resource.

CSI-IM-ResourceId information element

-- ASN1START

-- TAG-CSI-IM-RESOURCEID-START

CSI-IM-ResourceId ::=               INTEGER (0..maxNrofCSI-IM-Resources-1)

-- TAG-CSI-IM-RESOURCEID-STOP

-- ASN1STOP

#### –	CSI-IM-ResourceSet

The IE CSI-IM-ResourceSet is used to configure a set of one or more CSI Interference Management (IM) resources (their IDs) and set-specific parameters.

CSI-IM-ResourceSet information element

-- ASN1START

-- TAG-CSI-IM-RESOURCESET-START

CSI-IM-ResourceSet ::=              SEQUENCE {

csi-IM-ResourceSetId                CSI-IM-ResourceSetId,

csi-IM-Resources                    SEQUENCE (SIZE(1..maxNrofCSI-IM-ResourcesPerSet)) OF CSI-IM-ResourceId,

...

}

-- TAG-CSI-IM-RESOURCESET-STOP

-- ASN1STOP

CSI-IM-ResourceSet field descriptions

csi-IM-Resources CSI-IM-Resources associated with this CSI-IM-ResourceSet (see TS 38.214 [19], clause 5.2).

#### –	CSI-IM-ResourceSetId

The IE CSI-IM-ResourceSetId is used to identify CSI-IM-ResourceSets.

CSI-IM-ResourceSetId information element

-- ASN1START

-- TAG-CSI-IM-RESOURCESETID-START

CSI-IM-ResourceSetId ::=            INTEGER (0..maxNrofCSI-IM-ResourceSets-1)

-- TAG-CSI-IM-RESOURCESETID-STOP

-- ASN1STOP

#### –	CSI-MeasConfig

The IE CSI-MeasConfig is used to configure CSI-RS (reference signals) belonging to the serving cell in which CSI-MeasConfig is included, channel state information reports to be transmitted on PUCCH on the serving cell in which CSI-MeasConfig is included and channel state information reports on PUSCH triggered by DCI received on the serving cell in which CSI-MeasConfig is included. See also TS 38.214 [19], clause 5.2.

CSI-MeasConfig information element

-- ASN1START

-- TAG-CSI-MEASCONFIG-START

CSI-MeasConfig ::=                  SEQUENCE {

nzp-CSI-RS-ResourceToAddModList     SEQUENCE (SIZE (1..maxNrofNZP-CSI-RS-Resources)) OF NZP-CSI-RS-Resource   OPTIONAL, -- Need N

nzp-CSI-RS-ResourceToReleaseList    SEQUENCE (SIZE (1..maxNrofNZP-CSI-RS-Resources)) OF NZP-CSI-RS-ResourceId OPTIONAL, -- Need N

nzp-CSI-RS-ResourceSetToAddModList  SEQUENCE (SIZE (1..maxNrofNZP-CSI-RS-ResourceSets)) OF NZP-CSI-RS-ResourceSet

OPTIONAL, -- Need N

nzp-CSI-RS-ResourceSetToReleaseList SEQUENCE (SIZE (1..maxNrofNZP-CSI-RS-ResourceSets)) OF NZP-CSI-RS-ResourceSetId

OPTIONAL, -- Need N

csi-IM-ResourceToAddModList         SEQUENCE (SIZE (1..maxNrofCSI-IM-Resources)) OF CSI-IM-Resource           OPTIONAL, -- Need N

csi-IM-ResourceToReleaseList        SEQUENCE (SIZE (1..maxNrofCSI-IM-Resources)) OF CSI-IM-ResourceId         OPTIONAL, -- Need N

csi-IM-ResourceSetToAddModList      SEQUENCE (SIZE (1..maxNrofCSI-IM-ResourceSets)) OF CSI-IM-ResourceSet     OPTIONAL, -- Need N

csi-IM-ResourceSetToReleaseList     SEQUENCE (SIZE (1..maxNrofCSI-IM-ResourceSets)) OF CSI-IM-ResourceSetId   OPTIONAL, -- Need N

csi-SSB-ResourceSetToAddModList     SEQUENCE (SIZE (1..maxNrofCSI-SSB-ResourceSets)) OF CSI-SSB-ResourceSet   OPTIONAL, -- Need N

csi-SSB-ResourceSetToReleaseList    SEQUENCE (SIZE (1..maxNrofCSI-SSB-ResourceSets)) OF CSI-SSB-ResourceSetId OPTIONAL, -- Need N

csi-ResourceConfigToAddModList      SEQUENCE (SIZE (1..maxNrofCSI-ResourceConfigurations)) OF CSI-ResourceConfig

OPTIONAL, -- Need N

csi-ResourceConfigToReleaseList     SEQUENCE (SIZE (1..maxNrofCSI-ResourceConfigurations)) OF CSI-ResourceConfigId

OPTIONAL, -- Need N

csi-ReportConfigToAddModList        SEQUENCE (SIZE (1..maxNrofCSI-ReportConfigurations)) OF CSI-ReportConfig  OPTIONAL, -- Need N

csi-ReportConfigToReleaseList       SEQUENCE (SIZE (1..maxNrofCSI-ReportConfigurations)) OF CSI-ReportConfigId

OPTIONAL, -- Need N

reportTriggerSize                   INTEGER (0..6)                                                            OPTIONAL, -- Need M

aperiodicTriggerStateList           SetupRelease { CSI-AperiodicTriggerStateList }                            OPTIONAL, -- Need M

semiPersistentOnPUSCH-TriggerStateList    SetupRelease { CSI-SemiPersistentOnPUSCH-TriggerStateList }         OPTIONAL, -- Need M

...

}

-- TAG-CSI-MEASCONFIG-STOP

-- ASN1STOP

CSI-MeasConfig field descriptions

aperiodicTriggerStateList Contains trigger states for dynamically selecting one or more aperiodic and semi-persistent reporting configurations and/or triggering one or more aperiodic CSI-RS resource sets for channel and/or interference measurement.

csi-IM-ResourceSetToAddModList Pool of CSI-IM-ResourceSet which can be referred to from CSI-ResourceConfig or from MAC CEs.

csi-IM-ResourceToAddModList Pool of CSI-IM-Resource which can be referred to from CSI-IM-ResourceSet.

csi-ReportConfigToAddModList Configured CSI report settings as specified in TS 38.214 [19] clause 5.2.1.1.

csi-ResourceConfigToAddModList Configured CSI resource settings as specified in TS 38.214 [19] clause 5.2.1.2.

csi-SSB-ResourceSetToAddModList Pool of CSI-SSB-ResourceSet which can be referred to from CSI-ResourceConfig.

nzp-CSI-RS-ResourceSetToAddModList Pool of NZP-CSI-RS-ResourceSet which can be referred to from CSI-ResourceConfig or from MAC CEs.

nzp-CSI-RS-ResourceToAddModList Pool of NZP-CSI-RS-Resource which can be referred to from NZP-CSI-RS-ResourceSet.

reportTriggerSize Size of CSI request field in DCI (bits) (see TS 38.214 [19], clause 5.2.1.5.1).

#### –	CSI-ReportConfig

The IE CSI-ReportConfig is used to configure a periodic or semi-persistent report sent on PUCCH on the cell in which the CSI-ReportConfig is included, or to configure a semi-persistent or aperiodic report sent on PUSCH triggered by DCI received on the cell in which the CSI-ReportConfig is included (in this case, the cell on which the report is sent is determined by the received DCI). See TS 38.214 [19], clause 5.2.1.

CSI-ReportConfig information element

-- ASN1START

-- TAG-CSI-REPORTCONFIG-START

CSI-ReportConfig ::=                SEQUENCE {

reportConfigId                          CSI-ReportConfigId,

carrier                                 ServCellIndex                   OPTIONAL,   -- Need S

resourcesForChannelMeasurement          CSI-ResourceConfigId,

csi-IM-ResourcesForInterference         CSI-ResourceConfigId            OPTIONAL,   -- Need R

nzp-CSI-RS-ResourcesForInterference     CSI-ResourceConfigId            OPTIONAL,   -- Need R

reportConfigType                        CHOICE {

periodic                                SEQUENCE {

reportSlotConfig                        CSI-ReportPeriodicityAndOffset,

pucch-CSI-ResourceList                  SEQUENCE (SIZE (1..maxNrofBWPs)) OF PUCCH-CSI-Resource

},

semiPersistentOnPUCCH                   SEQUENCE {

reportSlotConfig                        CSI-ReportPeriodicityAndOffset,

pucch-CSI-ResourceList                  SEQUENCE (SIZE (1..maxNrofBWPs)) OF PUCCH-CSI-Resource

},

semiPersistentOnPUSCH                   SEQUENCE {

reportSlotConfig                        ENUMERATED {sl5, sl10, sl20, sl40, sl80, sl160, sl320},

reportSlotOffsetList                SEQUENCE (SIZE (1.. maxNrofUL-Allocations)) OF INTEGER(0..32),

p0alpha                                 P0-PUSCH-AlphaSetId

},

aperiodic                               SEQUENCE {

reportSlotOffsetList                SEQUENCE (SIZE (1..maxNrofUL-Allocations)) OF INTEGER(0..32)

}

},

reportQuantity                          CHOICE {

none                                    NULL,

cri-RI-PMI-CQI                          NULL,

cri-RI-i1                               NULL,

cri-RI-i1-CQI                           SEQUENCE {

pdsch-BundleSizeForCSI                  ENUMERATED {n2, n4}                                         OPTIONAL    -- Need S

},

cri-RI-CQI                              NULL,

cri-RSRP                                NULL,

ssb-Index-RSRP                          NULL,

cri-RI-LI-PMI-CQI                       NULL

},

reportFreqConfiguration                 SEQUENCE {

cqi-FormatIndicator                     ENUMERATED { widebandCQI, subbandCQI }                          OPTIONAL,   -- Need R

pmi-FormatIndicator                     ENUMERATED { widebandPMI, subbandPMI }                          OPTIONAL,   -- Need R

csi-ReportingBand                       CHOICE {

subbands3                               BIT STRING(SIZE(3)),

subbands4                               BIT STRING(SIZE(4)),

subbands5                               BIT STRING(SIZE(5)),

subbands6                               BIT STRING(SIZE(6)),

subbands7                               BIT STRING(SIZE(7)),

subbands8                               BIT STRING(SIZE(8)),

subbands9                               BIT STRING(SIZE(9)),

subbands10                              BIT STRING(SIZE(10)),

subbands11                              BIT STRING(SIZE(11)),

subbands12                              BIT STRING(SIZE(12)),

subbands13                              BIT STRING(SIZE(13)),

subbands14                              BIT STRING(SIZE(14)),

subbands15                              BIT STRING(SIZE(15)),

subbands16                              BIT STRING(SIZE(16)),

subbands17                              BIT STRING(SIZE(17)),

subbands18                              BIT STRING(SIZE(18)),

...,

subbands19-v1530                        BIT STRING(SIZE(19))

}   OPTIONAL    -- Need S

}                                                                                                           OPTIONAL,   -- Need R

timeRestrictionForChannelMeasurements           ENUMERATED {configured, notConfigured},

timeRestrictionForInterferenceMeasurements      ENUMERATED {configured, notConfigured},

codebookConfig                                  CodebookConfig                                              OPTIONAL,   -- Need R

dummy                                           ENUMERATED {n1, n2}                                         OPTIONAL,   -- Need R

groupBasedBeamReporting                     CHOICE {

enabled                                     NULL,

disabled                                    SEQUENCE {

nrofReportedRS                          ENUMERATED {n1, n2, n3, n4}                                 OPTIONAL    -- Need S

}

},

cqi-Table                   ENUMERATED {table1, table2, table3, spare1}                                     OPTIONAL,   -- Need R

subbandSize                 ENUMERATED {value1, value2},

non-PMI-PortIndication      SEQUENCE (SIZE (1..maxNrofNZP-CSI-RS-ResourcesPerConfig)) OF PortIndexFor8Ranks OPTIONAL,   -- Need R

...,

[[

semiPersistentOnPUSCH-v1530         SEQUENCE {

reportSlotConfig-v1530              ENUMERATED {sl4, sl8, sl16}

}                                                                                                           OPTIONAL    -- Need R

]]

}

CSI-ReportPeriodicityAndOffset ::=  CHOICE {

slots4                              INTEGER(0..3),

slots5                              INTEGER(0..4),

slots8                              INTEGER(0..7),

slots10                             INTEGER(0..9),

slots16                             INTEGER(0..15),

slots20                             INTEGER(0..19),

slots40                             INTEGER(0..39),

slots80                             INTEGER(0..79),

slots160                            INTEGER(0..159),

slots320                            INTEGER(0..319)

}

PUCCH-CSI-Resource ::=              SEQUENCE {

uplinkBandwidthPartId               BWP-Id,

pucch-Resource                      PUCCH-ResourceId

}

PortIndexFor8Ranks ::=              CHOICE {

portIndex8                          SEQUENCE{

rank1-8                             PortIndex8                                                      OPTIONAL,   -- Need R

rank2-8                             SEQUENCE(SIZE(2)) OF PortIndex8                                 OPTIONAL,   -- Need R

rank3-8                             SEQUENCE(SIZE(3)) OF PortIndex8                                 OPTIONAL,   -- Need R

rank4-8                             SEQUENCE(SIZE(4)) OF PortIndex8                                 OPTIONAL,   -- Need R

rank5-8                             SEQUENCE(SIZE(5)) OF PortIndex8                                 OPTIONAL,   -- Need R

rank6-8                             SEQUENCE(SIZE(6)) OF PortIndex8                                 OPTIONAL,   -- Need R

rank7-8                             SEQUENCE(SIZE(7)) OF PortIndex8                                 OPTIONAL,   -- Need R

rank8-8                             SEQUENCE(SIZE(8)) OF PortIndex8                                 OPTIONAL    -- Need R

},

portIndex4                          SEQUENCE{

rank1-4                             PortIndex4                                                      OPTIONAL,   -- Need R

rank2-4                             SEQUENCE(SIZE(2)) OF PortIndex4                                 OPTIONAL,   -- Need R

rank3-4                             SEQUENCE(SIZE(3)) OF PortIndex4                                 OPTIONAL,   -- Need R

rank4-4                             SEQUENCE(SIZE(4)) OF PortIndex4                                 OPTIONAL    -- Need R

},

portIndex2                          SEQUENCE{

rank1-2                             PortIndex2                                                      OPTIONAL,   -- Need R

rank2-2                             SEQUENCE(SIZE(2)) OF PortIndex2                                 OPTIONAL    -- Need R

},

portIndex1                          NULL

}

PortIndex8::=                       INTEGER (0..7)

PortIndex4::=                       INTEGER (0..3)

PortIndex2::=                       INTEGER (0..1)

-- TAG-CSI-REPORTCONFIG-STOP

-- ASN1STOP

CSI-ReportConfig field descriptions

carrier Indicates in which serving cell the CSI-ResourceConfig indicated below are to be found. If the field is absent, the resources are on the same serving cell as this report configuration.

codebookConfig Codebook configuration for Type-1 or Type-2 including codebook subset restriction.

cqi-FormatIndicator Indicates whether the UE shall report a single (wideband) or multiple (subband) CQI. (see TS 38.214 [19], clause 5.2.1.4).

cqi-Table Which CQI table to use for CQI calculation (see TS 38.214 [19], clause 5.2.2.1).

csi-IM-ResourcesForInterference CSI IM resources for interference measurement. csi-ResourceConfigId of a CSI-ResourceConfig included in the configuration of the serving cell indicated with the field "carrier" above. The CSI-ResourceConfig indicated here contains only CSI-IM resources. The bwp-Id in that CSI-ResourceConfig is the same value as the bwp-Id in the CSI-ResourceConfig indicated by resourcesForChannelMeasurement.

csi-ReportingBand Indicates a contiguous or non-contiguous subset of subbands in the bandwidth part which CSI shall be reported for. Each bit in the bit-string represents one subband. The right-most bit in the bit string represents the lowest subband in the BWP. The choice determines the number of subbands (subbands3 for 3 subbands, subbands4 for 4 subbands, and so on) (see TS 38.214 [19], clause 5.2.1.4). This field is absent if there are less than 24 PRBs (no sub band) and present otherwise, the number of sub bands can be from 3 (24 PRBs, sub band size 8) to 18 (72 PRBs, sub band size 4).

dummy This field is not used in the specification. If received it shall be ignored by the UE.

groupBasedBeamReporting Turning on/off group beam based reporting (see TS 38.214 [19], clause 5.2.1.4).

non-PMI-PortIndication Port indication for RI/CQI calculation. For each CSI-RS resource in the linked ResourceConfig for channel measurement, a port indication for each rank R, indicating which R ports to use. Applicable only for non-PMI feedback (see TS 38.214 [19], clause 5.2.1.4.2). The first entry in non-PMI-PortIndication corresponds to the NZP-CSI-RS-Resource indicated by the first entry in nzp-CSI-RS-Resources in the NZP-CSI-RS-ResourceSet indicated in the first entry of nzp-CSI-RS-ResourceSetList of the CSI-ResourceConfig whose CSI-ResourceConfigId is indicated in a CSI-MeasId together with the above CSI-ReportConfigId; the second entry in non-PMI-PortIndication corresponds to the NZP-CSI-RS-Resource indicated by the second entry in nzp-CSI-RS-Resources in the NZP-CSI-RS-ResourceSet indicated in the first entry of nzp-CSI-RS-ResourceSetList of the same CSI-ResourceConfig, and so on until the NZP-CSI-RS-Resource indicated by the last entry in nzp-CSI-RS-Resources in the in the NZP-CSI-RS-ResourceSet indicated in the first entry of nzp-CSI-RS-ResourceSetList of the same CSI-ResourceConfig. Then the next entry corresponds to the NZP-CSI-RS-Resource indicated by the first entry in nzp-CSI-RS-Resources in the NZP-CSI-RS-ResourceSet indicated in the second entry of nzp-CSI-RS-ResourceSetList of the same CSI-ResourceConfig and so on.

nrofReportedRS The number (N) of measured RS resources to be reported per report setting in a non-group-based report. N <= N_max, where N_max is either 2 or 4 depending on UE capability. (see TS 38.214 [19], clause 5.2.1.4) When the field is absent the UE applies the value 1.

nzp-CSI-RS-ResourcesForInterference NZP CSI RS resources for interference measurement. csi-ResourceConfigId of a CSI-ResourceConfig included in the configuration of the serving cell indicated with the field "carrier" above. The CSI-ResourceConfig indicated here contains only NZP-CSI-RS resources. The bwp-Id in that CSI-ResourceConfig is the same value as the bwp-Id in the CSI-ResourceConfig indicated by resourcesForChannelMeasurement.

p0alpha Index of the p0-alpha set determining the power control for this CSI report transmission (see TS 38.214 [19], clause 6.2.1.2).

pdsch-BundleSizeForCSI PRB bundling size to assume for CQI calculation when reportQuantity is CRI/RI/i1/CQI. If the field is absent, the UE assumes that no PRB bundling is applied (see TS 38.214 [19], clause 5.2.1.4.2).

pmi-FormatIndicator Indicates whether the UE shall report a single (wideband) or multiple (subband) PMI. (see TS 38.214 [19], clause 5.2.1.4).

pucch-CSI-ResourceList Indicates which PUCCH resource to use for reporting on PUCCH.

reportConfigType Time domain behavior of reporting configuration.

reportFreqConfiguration Reporting configuration in the frequency domain. (see TS 38.214 [19], clause 5.2.1.4).

reportQuantity The CSI related quantities to report. see TS 38.214 [19], clause 5.2.1.

reportSlotConfig Periodicity and slot offset (see TS 38.214 [19], clause 5.2.1.4). If the field reportSlotConfig-v1530 is present, the UE shall ignore the value provided in reportSlotConfig (without suffix).

reportSlotOffsetList Timing offset Y for semi persistent reporting using PUSCH. This field lists the allowed offset values. This list must have the same number of entries as the pusch-TimeDomainAllocationList in PUSCH-Config. A particular value is indicated in DCI. The network indicates in the DCI field of the UL grant, which of the configured report slot offsets the UE shall apply. The DCI value 0 corresponds to the first report slot offset in this list, the DCI value 1 corresponds to the second report slot offset in this list, and so on. The first report is transmitted in slot n+Y, second report in n+Y+P, where P is the configured periodicity. Timing offset Y for aperiodic reporting using PUSCH. This field lists the allowed offset values. This list must have the same number of entries as the pusch-TimeDomainAllocationList in PUSCH-Config. A particular value is indicated in DCI. The network indicates in the DCI field of the UL grant, which of the configured report slot offsets the UE shall apply. The DCI value 0 corresponds to the first report slot offset in this list, the DCI value 1 corresponds to the second report slot offset in this list, and so on (see TS 38.214 [19], clause 6.1.2.1).

resourcesForChannelMeasurement Resources for channel measurement. csi-ResourceConfigId of a CSI-ResourceConfig included in the configuration of the serving cell indicated with the field "carrier" above. The CSI-ResourceConfig indicated here contains only NZP-CSI-RS resources and/or SSB resources. This CSI-ReportConfig is associated with the DL BWP indicated by bwp-Id in that CSI-ResourceConfig.

subbandSize Indicates one out of two possible BWP-dependent values for the subband size as indicated in TS 38.214 [19], table 5.2.1.4-2 . If csi-ReportingBand is absent, the UE shall ignore this field.

timeRestrictionForChannelMeasurements Time domain measurement restriction for the channel (signal) measurements (see TS 38.214 [19], clause 5.2.1.1).

timeRestrictionForInterferenceMeasurements Time domain measurement restriction for interference measurements (see TS 38.214 [19], clause 5.2.1.1).

PortIndexFor8Ranks field descriptions

portIndex8 Port-Index configuration for up to rank 8. If present, the network configures port indexes for at least one of the ranks.

portIndex4 Port-Index configuration for up to rank 4. If present, the network configures port indexes for at least one of the ranks.

portIndex2 Port-Index configuration for up to rank 2. If present, the network configures port indexes for at least one of the ranks.

portIndex1 Port-Index configuration for rank 1.

PUCCH-CSI-Resource field descriptions

pucch-Resource PUCCH resource for the associated uplink BWP. Only PUCCH-Resource of format 2, 3 and 4 is supported. The actual PUCCH-Resource is configured in PUCCH-Config and referred to by its ID.

#### –	CSI-ReportConfigId

The IE CSI-ReportConfigId is used to identify one CSI-ReportConfig.

CSI-ReportConfigId information element

-- ASN1START

-- TAG-CSI-REPORTCONFIGID-START

CSI-ReportConfigId ::=              INTEGER (0..maxNrofCSI-ReportConfigurations-1)

-- TAG-CSI-REPORTCONFIGID-STOP

-- ASN1STOP

#### –	CSI-ResourceConfig

The IE CSI-ResourceConfig defines a group of one or more NZP-CSI-RS-ResourceSet, CSI-IM-ResourceSet and/or CSI-SSB-ResourceSet.

CSI-ResourceConfig information element

-- ASN1START

-- TAG-CSI-RESOURCECONFIG-START

CSI-ResourceConfig ::=      SEQUENCE {

csi-ResourceConfigId        CSI-ResourceConfigId,

csi-RS-ResourceSetList      CHOICE {

nzp-CSI-RS-SSB              SEQUENCE {

nzp-CSI-RS-ResourceSetList  SEQUENCE (SIZE (1..maxNrofNZP-CSI-RS-ResourceSetsPerConfig)) OF NZP-CSI-RS-ResourceSetId

OPTIONAL, -- Need R

csi-SSB-ResourceSetList     SEQUENCE (SIZE (1..maxNrofCSI-SSB-ResourceSetsPerConfig)) OF CSI-SSB-ResourceSetId

OPTIONAL  -- Need R

},

csi-IM-ResourceSetList      SEQUENCE (SIZE (1..maxNrofCSI-IM-ResourceSetsPerConfig)) OF CSI-IM-ResourceSetId

},

bwp-Id                      BWP-Id,

resourceType                ENUMERATED { aperiodic, semiPersistent, periodic },

...

}

-- TAG-CSI-RESOURCECONFIG-STOP

-- ASN1STOP

CSI-ResourceConfig field descriptions

bwp-Id The DL BWP which the CSI-RS associated with this CSI-ResourceConfig are located in (see TS 38.214 [19], clause 5.2.1.2.

csi-IM-ResourceSetList List of references to CSI-IM resources used for beam measurement and reporting in a CSI-RS resource set. Contains up to maxNrofCSI-IM-ResourceSetsPerConfig resource sets if resourceType is 'aperiodic' and 1 otherwise (see TS 38.214 [19], clause 5.2.1.2).

csi-ResourceConfigId Used in CSI-ReportConfig to refer to an instance of CSI-ResourceConfig.

csi-SSB-ResourceSetList List of references to SSB resources used for beam measurement and reporting in a CSI-RS resource set (see TS 38.214 [19], clause 5.2.1.2).

nzp-CSI-RS-ResourceSetList List of references to NZP CSI-RS resources used for beam measurement and reporting in a CSI-RS resource set. Contains up to maxNrofNZP-CSI-RS-ResourceSetsPerConfig resource sets if resourceType is 'aperiodic' and 1 otherwise (see TS 38.214 [19], clause 5.2.1.2).

resourceType Time domain behavior of resource configuration (see TS 38.214 [19], clause 5.2.1.2). It does not apply to resources provided in the csi-SSB-ResourceSetList.

#### –	CSI-ResourceConfigId

The IE CSI-ResourceConfigId is used to identify a CSI-ResourceConfig.

CSI-ResourceConfigId information element

-- ASN1START

-- TAG-CSI-RESOURCECONFIGID-START

CSI-ResourceConfigId ::=            INTEGER (0..maxNrofCSI-ResourceConfigurations-1)

-- TAG-CSI-RESOURCECONFIGID-STOP

-- ASN1STOP

#### –	CSI-ResourcePeriodicityAndOffset

The IE CSI-ResourcePeriodicityAndOffset is used to configure a periodicity and a corresponding offset for periodic and semi-persistent CSI resources, and for periodic and semi-persistent reporting on PUCCH. both, the periodicity and the offset are given in number of slots. The periodicity value slots4 corresponds to 4 slots, value slots5 corresponds to 5 slots, and so on.

CSI-ResourcePeriodicityAndOffset information element

-- ASN1START

-- TAG-CSI-RESOURCEPERIODICITYANDOFFSET-START

CSI-ResourcePeriodicityAndOffset ::=    CHOICE {

slots4                              INTEGER (0..3),

slots5                              INTEGER (0..4),

slots8                              INTEGER (0..7),

slots10                             INTEGER (0..9),

slots16                             INTEGER (0..15),

slots20                             INTEGER (0..19),

slots32                             INTEGER (0..31),

slots40                             INTEGER (0..39),

slots64                             INTEGER (0..63),

slots80                             INTEGER (0..79),

slots160                            INTEGER (0..159),

slots320                            INTEGER (0..319),

slots640                            INTEGER (0..639)

}

-- TAG-CSI-RESOURCEPERIODICITYANDOFFSET-STOP

-- ASN1STOP

#### –	CSI-RS-ResourceConfigMobility

The IE CSI-RS-ResourceConfigMobility is used to configure CSI-RS based RRM measurements.

CSI-RS-ResourceConfigMobility information element

-- ASN1START

-- TAG-CSI-RS-RESOURCECONFIGMOBILITY-START

CSI-RS-ResourceConfigMobility ::=   SEQUENCE {

subcarrierSpacing                   SubcarrierSpacing,

csi-RS-CellList-Mobility            SEQUENCE (SIZE (1..maxNrofCSI-RS-CellsRRM)) OF CSI-RS-CellMobility,

...,

[[

refServCellIndex                    ServCellIndex                                                           OPTIONAL    -- Need S

]]

}

CSI-RS-CellMobility ::=             SEQUENCE {

cellId                              PhysCellId,

csi-rs-MeasurementBW                SEQUENCE {

nrofPRBs                            ENUMERATED { size24, size48, size96, size192, size264},

startPRB                            INTEGER(0..2169)

},

density                             ENUMERATED {d1,d3}                                                      OPTIONAL,   -- Need R

csi-rs-ResourceList-Mobility        SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesRRM)) OF CSI-RS-Resource-Mobility

}

CSI-RS-Resource-Mobility ::=        SEQUENCE {

csi-RS-Index                        CSI-RS-Index,

slotConfig                          CHOICE {

ms4                                 INTEGER (0..31),

ms5                                 INTEGER (0..39),

ms10                                INTEGER (0..79),

ms20                                INTEGER (0..159),

ms40                                INTEGER (0..319)

},

associatedSSB                       SEQUENCE {

ssb-Index                           SSB-Index,

isQuasiColocated                    BOOLEAN

}                                                                                                           OPTIONAL, -- Need R

frequencyDomainAllocation           CHOICE {

row1                                BIT STRING (SIZE (4)),

row2                                BIT STRING (SIZE (12))

},

firstOFDMSymbolInTimeDomain         INTEGER (0..13),

sequenceGenerationConfig            INTEGER (0..1023),

...

}

CSI-RS-Index ::=                    INTEGER (0..maxNrofCSI-RS-ResourcesRRM-1)

-- TAG-CSI-RS-RESOURCECONFIGMOBILITY-STOP

-- ASN1STOP

CSI-RS-CellMobility field descriptions

csi-rs-ResourceList-Mobility List of CSI-RS resources for mobility. The maximum number of CSI-RS resources that can be configured per measObjectNR depends on the configuration of associatedSSB (see TS 38.214 [19], clause 5.1.6.1.3).

density Frequency domain density for the 1-port CSI-RS for L3 mobility. See TS 38.211 [16], clause 7.4.1.

nrofPRBs Allowed size of the measurement BW in PRBs. See TS 38.211 [16], clause 7.4.1.

startPRB Starting PRB index of the measurement bandwidth. See TS 38.211 [16], clause 7.4.1.

CSI-RS-ResourceConfigMobility field descriptions

csi-RS-CellList-Mobility List of cells for CSI-RS based RRM measurements.

refServCellIndex Indicates the serving cell providing the timing reference for CSI-RS resources without associatedSSB. The field may be present only if there is at least one CSI-RS resource configured without associatedSSB. If this field is absent, the UE shall use the timing of the PCell for measurements on the CSI-RS resources without associatedSSB. The CSI-RS resources and the serving cell indicated by refServCellIndex for timing reference should be located in the same band.

subcarrierSpacing Subcarrier spacing of CSI-RS. Only the values 15, 30 kHz or 60 kHz (FR1), and 60 or 120 kHz (FR2) are applicable.

CSI-RS-Resource-Mobility field descriptions

associatedSSB If this field is present, the UE may base the timing of the CSI-RS resource indicated in CSI-RS-Resource-Mobility on the timing of the cell indicated by the cellId in the CSI-RS-CellMobility. In this case, the UE is not required to monitor that CSI-RS resource if the UE cannot detect the SS/PBCH block indicated by this associatedSSB and cellId. If this field is absent, the UE shall base the timing of the CSI-RS resource indicated in CSI-RS-Resource-Mobility on the timing of the serving cell indicated by refServCellIndex. In this case, the UE is required to measure the CSI-RS resource even if SS/PBCH block(s) with cellId in the CSI-RS-CellMobility are not detected. CSI-RS resources with and without associatedSSB may be configured in accordance with the rules in TS 38.214 [19], clause 5.1.6.1.3.

csi-RS-Index CSI-RS resource index associated to the CSI-RS resource to be measured (and used for reporting).

firstOFDMSymbolInTimeDomain Time domain allocation within a physical resource block. The field indicates the first OFDM symbol in the PRB used for CSI-RS, see TS 38.211 [16], clause 7.4.1.5.3. Value 2 is supported only when dmrs-TypeA-Position equals pos3.

frequencyDomainAllocation Frequency domain allocation within a physical resource block in accordance with TS 38.211 [16], clause 7.4.1.5.3 including table 7.4.1.5.2-1. The number of bits that may be set to one depend on the chosen row in that table.

isQuasiColocated Indicates that the CSI-RS resource is quasi co-located with the associated SS/PBCH block, see TS 38.214 [19], clause 5.1.6.1.3.

sequenceGenerationConfig Scrambling ID for CSI-RS (see TS 38.211 [16], clause 7.4.1.5.2).

slotConfig Indicates the CSI-RS periodicity (in milliseconds) and for each periodicity the offset (in number of slots). When subcarrierSpacingCSI-RS is set to kHz15, the maximum offset values for periodicities ms4/ms5/ms10/ms20/ms40 are 3/4/9/19/39 slots. When subcarrierSpacingCSI-RS is set to kHz30, the maximum offset values for periodicities ms4/ms5/ms10/ms20/ms40 are 7/9/19/39/79 slots. When subcarrierSpacingCSI-RS is set to kHz60, the maximum offset values for periodicities ms4/ms5/ms10/ms20/ms40 are 15/19/39/79/159 slots. When subcarrierSpacingCSI-RS is set kHz120, the maximum offset values for periodicities ms4/ms5/ms10/ms20/ms40 are 31/39/79/159/319 slots.

#### –	CSI-RS-ResourceMapping

The IE CSI-RS-ResourceMapping is used to configure the resource element mapping of a CSI-RS resource in time- and frequency domain.

CSI-RS-ResourceMapping information element

-- ASN1START

-- TAG-CSI-RS-RESOURCEMAPPING-START

CSI-RS-ResourceMapping ::=          SEQUENCE {

frequencyDomainAllocation           CHOICE {

row1                                BIT STRING (SIZE (4)),

row2                                BIT STRING (SIZE (12)),

row4                                BIT STRING (SIZE (3)),

other                               BIT STRING (SIZE (6))

},

nrofPorts                           ENUMERATED {p1,p2,p4,p8,p12,p16,p24,p32},

firstOFDMSymbolInTimeDomain         INTEGER (0..13),

firstOFDMSymbolInTimeDomain2        INTEGER (2..12)                                                         OPTIONAL,   -- Need R

cdm-Type                            ENUMERATED {noCDM, fd-CDM2, cdm4-FD2-TD2, cdm8-FD2-TD4},

density                             CHOICE {

dot5                                ENUMERATED {evenPRBs, oddPRBs},

one                                 NULL,

three                               NULL,

spare                               NULL

},

freqBand                            CSI-FrequencyOccupation,

...

}

-- TAG-CSI-RS-RESOURCEMAPPING-STOP

-- ASN1STOP

CSI-RS-ResourceMapping field descriptions

cdm-Type CDM type (see TS 38.214 [19], clause 5.2.2.3.1).

density Density of CSI-RS resource measured in RE/port/PRB (see TS 38.211 [16], clause 7.4.1.5.3). Values 0.5 (dot5), 1 (one) and 3 (three) are allowed for X=1, values 0.5 (dot5) and 1 (one) are allowed for X=2, 16, 24 and 32, value 1 (one) is allowed for X=4, 8, 12. For density = 1/2, includes 1-bit indication for RB level comb offset indicating whether odd or even RBs are occupied by CSI-RS.

firstOFDMSymbolInTimeDomain2 Time domain allocation within a physical resource block. See TS 38.211 [16], clause 7.4.1.5.3.

firstOFDMSymbolInTimeDomain Time domain allocation within a physical resource block. The field indicates the first OFDM symbol in the PRB used for CSI-RS. See TS 38.211 [16], clause 7.4.1.5.3. Value 2 is supported only when dmrs-TypeA-Position equals pos3.

freqBand Wideband or partial band CSI-RS, (see TS 38.214 [19], clause 5.2.2.3.1).

frequencyDomainAllocation Frequency domain allocation within a physical resource block in accordance with TS 38.211 [16], clause 7.4.1.5.3. The applicable row number in table 7.4.1.5.3-1 is determined by the frequencyDomainAllocation for rows 1, 2 and 4, and for other rows by matching the values in the column Ports, Density and CDMtype in table 7.4.1.5.3-1 with the values of nrofPorts, cdm-Type and density below and, when more than one row has the 3 values matching, by selecting the row where the column (k bar, l bar) in table 7.4.1.5.3-1 has indexes for k ranging from 0 to 2*n-1 where n is the number of bits set to 1 in frequencyDomainAllocation.

nrofPorts Number of ports (see TS 38.214 [19], clause 5.2.2.3.1).

#### –	CSI-SemiPersistentOnPUSCH-TriggerStateList

The CSI-SemiPersistentOnPUSCH-TriggerStateList IE is used to configure the UE with list of trigger states for semi-persistent reporting of channel state information on L1. See also TS 38.214 [19], clause 5.2.

CSI-SemiPersistentOnPUSCH-TriggerStateList information element

-- ASN1START

-- TAG-CSI-SEMIPERSISTENTONPUSCHTRIGGERSTATELIST-START

CSI-SemiPersistentOnPUSCH-TriggerStateList ::=

SEQUENCE(SIZE (1..maxNrOfSemiPersistentPUSCH-Triggers)) OF CSI-SemiPersistentOnPUSCH-TriggerState

CSI-SemiPersistentOnPUSCH-TriggerState ::=      SEQUENCE {

associatedReportConfigInfo                      CSI-ReportConfigId,

...

}

-- TAG-CSI-SEMIPERSISTENTONPUSCHTRIGGERSTATELIST-STOP

-- ASN1STOP

#### –	CSI-SSB-ResourceSet

The IE CSI-SSB-ResourceSet is used to configure one SS/PBCH block resource set which refers to SS/PBCH as indicated in ServingCellConfigCommon.

CSI-SSB-ResourceSet information element

-- ASN1START

-- TAG-CSI-SSB-RESOURCESET-START

CSI-SSB-ResourceSet ::=             SEQUENCE {

csi-SSB-ResourceSetId               CSI-SSB-ResourceSetId,

csi-SSB-ResourceList                SEQUENCE (SIZE(1..maxNrofCSI-SSB-ResourcePerSet)) OF SSB-Index,

...

}

-- TAG-CSI-SSB-RESOURCESET-STOP

-- ASN1STOP

#### –	CSI-SSB-ResourceSetId

The IE CSI-SSB-ResourceSetId is used to identify one SS/PBCH block resource set.

CSI-SSB-ResourceId information element

-- ASN1START

-- TAG-CSI-SSB-RESOURCESETID-START

CSI-SSB-ResourceSetId ::=           INTEGER (0..maxNrofCSI-SSB-ResourceSets-1)

-- TAG-CSI-SSB-RESOURCESETID-STOP

-- ASN1STOP

#### –	DedicatedNAS-Message

The IE DedicatedNAS-Message is used to transfer UE specific NAS layer information between the 5GC CN and the UE. The RRC layer is transparent for this information.

DedicatedNAS-Message information element

-- ASN1START

-- TAG-DEDICATED-NAS-MESSAGE-START

DedicatedNAS-Message ::=        OCTET STRING

-- TAG-DEDICATED-NAS-MESSAGE-STOP

-- ASN1STOP

#### –	DMRS-DownlinkConfig

The IE DMRS-DownlinkConfig is used to configure downlink demodulation reference signals for PDSCH.

DMRS-DownlinkConfig information element

-- ASN1START

-- TAG-DMRS-DOWNLINKCONFIG-START

DMRS-DownlinkConfig ::=             SEQUENCE {

dmrs-Type                           ENUMERATED {type2}                                                      OPTIONAL,   -- Need S

dmrs-AdditionalPosition             ENUMERATED {pos0, pos1, pos3}                                           OPTIONAL,   -- Need S

maxLength                           ENUMERATED {len2}                                                       OPTIONAL,   -- Need S

scramblingID0                       INTEGER (0..65535)                                                      OPTIONAL,   -- Need S

scramblingID1                       INTEGER (0..65535)                                                      OPTIONAL,   -- Need S

phaseTrackingRS                     SetupRelease { PTRS-DownlinkConfig  }                                   OPTIONAL,   -- Need M

...

}

-- TAG-DMRS-DOWNLINKCONFIG-STOP

-- ASN1STOP

DMRS-DownlinkConfig field descriptions

dmrs-AdditionalPosition Position for additional DM-RS in DL, see Tables 7.4.1.1.2-3 and 7.4.1.1.2-4 in TS 38.211 [16]. If the field is absent, the UE applies the value pos2. See also clause 7.4.1.1.2 for additional constraints on how the network may set this field depending on the setting of other fields.

dmrs-Type Selection of the DMRS type to be used for DL (see TS 38.211 [16], clause 7.4.1.1.1). If the field is absent, the UE uses DMRS type 1.

maxLength The maximum number of OFDM symbols for DL front loaded DMRS. len1 corresponds to value 1. len2 corresponds to value 2. If the field is absent, the UE applies value len1. If set to len2, the UE determines the actual number of DM-RS symbols by the associated DCI. (see TS 38.211 [16], clause 7.4.1.1.2).

phaseTrackingRS Configures downlink PTRS. If the field is not configured, the UE assumes that downlink PTRS are absent. See TS 38.214 [19] clause 5.1.6.3.

scramblingID0 DL DMRS scrambling initialization (see TS 38.211 [16], clause 7.4.1.1.1). When the field is absent the UE applies the value physCellId configured for this serving cell.

scramblingID1 DL DMRS scrambling initialization (see TS 38.211 [16], clause 7.4.1.1.1). When the field is absent the UE applies the value physCellId configured for this serving cell.

#### –	DMRS-UplinkConfig

The IE DMRS-UplinkConfig is used to configure uplink demodulation reference signals for PUSCH.

DMRS-UplinkConfig information element

-- ASN1START

-- TAG-DMRS-UPLINKCONFIG-START

DMRS-UplinkConfig ::=               SEQUENCE {

dmrs-Type                           ENUMERATED {type2}                                                  OPTIONAL,   -- Need S

dmrs-AdditionalPosition             ENUMERATED {pos0, pos1, pos3}                                       OPTIONAL,   -- Need S

phaseTrackingRS                     SetupRelease { PTRS-UplinkConfig }                                  OPTIONAL,   -- Need M

maxLength                           ENUMERATED {len2}                                                   OPTIONAL,   -- Need S

transformPrecodingDisabled          SEQUENCE {

scramblingID0                       INTEGER (0..65535)                                              OPTIONAL,   -- Need S

scramblingID1                       INTEGER (0..65535)                                              OPTIONAL,   -- Need S

...

}                                                                                                       OPTIONAL,   -- Need R

transformPrecodingEnabled           SEQUENCE {

nPUSCH-Identity                     INTEGER(0..1007)                                                OPTIONAL,   -- Need S

sequenceGroupHopping                ENUMERATED {disabled}                                           OPTIONAL,   -- Need S

sequenceHopping                     ENUMERATED {enabled}                                            OPTIONAL,   -- Need S

...

}                                                                                                       OPTIONAL,   -- Need R

...

}

-- TAG-DMRS-UPLINKCONFIG-STOP

-- ASN1STOP

DMRS-UplinkConfig field descriptions

dmrs-AdditionalPosition Position for additional DM-RS in UL (see TS 38.211 [16], clause 6.4.1.1.3). If the field is absent, the UE applies the value pos2. See also clause 6.4.1.1.3 for additional constraints on how the network may set this field depending on the setting of other fields.

dmrs-Type Selection of the DMRS type to be used for UL (see TS 38.211 [16], clause 6.4.1.1.3) If the field is absent, the UE uses DMRS type 1.

maxLength The maximum number of OFDM symbols for UL front loaded DMRS. len1 corresponds to value 1. len2 corresponds to value 2. If the field is absent, the UE applies value len1. If set to len2, the UE determines the actual number of DM-RS symbols by the associated DCI. (see TS 38.211 [16], clause 6.4.1.1.3).

nPUSCH-Identity Parameter: N_ID^(PUSCH) for DFT-s-OFDM DMRS. If the value is absent or released, the UE uses the value Physical cell ID (physCellId). See TS 38.211 [16].

phaseTrackingRS Configures uplink PTRS (see TS 38.211 [16]).

scramblingID0 UL DMRS scrambling initialization for CP-OFDM (see TS 38.211 [16], clause 6.4.1.1.1.1). When the field is absent the UE applies the value Physical cell ID (physCellId).

scramblingID1 UL DMRS scrambling initialization for CP-OFDM. (see TS 38.211 [16], clause 6.4.1.1.1.1). When the field is absent the UE applies the value Physical cell ID (physCellId).

sequenceGroupHopping For DMRS transmission with transform precoder the NW may configure group hopping by the cell-specific parameter groupHoppingEnabledTransformPrecoding in PUSCH-ConfigCommon. In this case, the NW may include this UE specific field to disable group hopping for PUSCH transmission except for Msg3, i.e., to override the configuration in PUSCH-ConfigCommon (see TS 38.211 [16]). If the field is absent, the UE uses the same hopping mode as for Msg3.

sequenceHopping Determines if sequence hopping is enabled for DMRS transmission with transform precoder for PUSCH transmission other than Msg3 (sequence hopping is always disabled for Msg3). If the field is absent, the UE uses the same hopping mode as for msg3. The network does not configure simultaneous group hopping and sequence hopping. See TS 38.211 [16], clause 6.4.1.1.1.2.

transformPrecodingDisabled DMRS related parameters for Cyclic Prefix OFDM.

transformPrecodingEnabled DMRS related parameters for DFT-s-OFDM (Transform Precoding).

#### –	DownlinkConfigCommon

The IE DownlinkConfigCommon provides common downlink parameters of a cell.

DownlinkConfigCommon information element

-- ASN1START

-- TAG-DOWNLINKCONFIGCOMMON-START

DownlinkConfigCommon ::=        SEQUENCE {

frequencyInfoDL                 FrequencyInfoDL                                 OPTIONAL,   -- Cond InterFreqHOAndServCellAdd

initialDownlinkBWP              BWP-DownlinkCommon                              OPTIONAL,   -- Cond ServCellAdd

...

}

-- TAG-DOWNLINKCONFIGCOMMON-STOP

-- ASN1STOP

DownlinkConfigCommon field descriptions

frequencyInfoDL Basic parameters of a downlink carrier and transmission thereon.

initialDownlinkBWP The initial downlink BWP configuration for a serving cell.The network configures the locationAndBandwidth so that the initial downlink BWP contains the entire CORESET#0 of this serving cell in the frequency domain.

| Conditional Presence | Explanation |
| --- | --- |
| InterFreqHOAndServCellAdd | This field is mandatory present for inter-frequency handover, and upon serving cell (PSCell/SCell) addition. Otherwise, the field is optionally present, Need M. |
| ServCellAdd | This field is mandatory present upon serving cell addition (for PSCell and SCell) and upon handover from E-UTRA to NR. It is optionally present, Need M otherwise. |

#### –	DownlinkConfigCommonSIB

The IE DownlinkConfigCommonSIB provides common downlink parameters of a cell.

DownlinkConfigCommonSIB information element

-- ASN1START

-- TAG-DOWNLINKCONFIGCOMMONSIB-START

DownlinkConfigCommonSIB ::=     SEQUENCE {

frequencyInfoDL                 FrequencyInfoDL-SIB,

initialDownlinkBWP              BWP-DownlinkCommon,

bcch-Config                         BCCH-Config,

pcch-Config                         PCCH-Config,

...

}

BCCH-Config ::=                 SEQUENCE {

modificationPeriodCoeff         ENUMERATED {n2, n4, n8, n16},

...

}

PCCH-Config ::=             SEQUENCE {

defaultPagingCycle                  PagingCycle,

nAndPagingFrameOffset               CHOICE {

oneT                                NULL,

halfT                               INTEGER (0..1),

quarterT                            INTEGER (0..3),

oneEighthT                          INTEGER (0..7),

oneSixteenthT                       INTEGER (0..15)

},

ns                                  ENUMERATED {four, two, one},

firstPDCCH-MonitoringOccasionOfPO   CHOICE {

sCS15KHZoneT                                                            SEQUENCE (SIZE (1..maxPO-perPF)) OF INTEGER (0..139),

sCS30KHZoneT-SCS15KHZhalfT                                              SEQUENCE (SIZE (1..maxPO-perPF)) OF INTEGER (0..279),

sCS60KHZoneT-SCS30KHZhalfT-SCS15KHZquarterT                             SEQUENCE (SIZE (1..maxPO-perPF)) OF INTEGER (0..559),

sCS120KHZoneT-SCS60KHZhalfT-SCS30KHZquarterT-SCS15KHZoneEighthT         SEQUENCE (SIZE (1..maxPO-perPF)) OF INTEGER (0..1119),

sCS120KHZhalfT-SCS60KHZquarterT-SCS30KHZoneEighthT-SCS15KHZoneSixteenthT

SEQUENCE (SIZE (1..maxPO-perPF)) OF INTEGER (0..2239),

sCS120KHZquarterT-SCS60KHZoneEighthT-SCS30KHZoneSixteenthT              SEQUENCE (SIZE (1..maxPO-perPF)) OF INTEGER (0..4479),

sCS120KHZoneEighthT-SCS60KHZoneSixteenthT                               SEQUENCE (SIZE (1..maxPO-perPF)) OF INTEGER (0..8959),

sCS120KHZoneSixteenthT                                                  SEQUENCE (SIZE (1..maxPO-perPF)) OF INTEGER (0..17919)

}   OPTIONAL,           -- Need R

...

}

-- TAG-DOWNLINKCONFIGCOMMONSIB-STOP

-- ASN1STOP

DownlinkConfigCommonSIB field descriptions

frequencyInfoDL-SIB Basic parameters of a downlink carrier and transmission thereon.

initialDownlinkBWP The initial downlink BWP configuration for a PCell. The network configures the locationAndBandwidth so that the initial downlink BWP contains the entire CORESET#0 of this serving cell in the frequency domain. The UE applies the locationAndBandwidth upon reception of this field (e.g. to determine the frequency position of signals described in relation to this locationAndBandwidth) but it keeps CORESET#0 until after reception of RRCSetup/RRCResume/RRCReestablishment.

bcch-Config The modification period related configuration.

pcch-Config The paging related configuration.

BCCH-Config field descriptions

modificationPeriodCoeff Actual modification period, expressed in number of radio frames m = modificationPeriodCoeff * defaultPagingCycle, see clause 5.2.2.2.2. n2 corresponds to value 2, n4 corresponds to value 4, and so on.

PCCH-Config field descriptions

defaultPagingCycle Default paging cycle, used to derive 'T' in TS 38.304 [20]. Value rf32 corresponds to 32 radio frames, value rf64 corresponds to 64 radio frames and so on.

firstPDCCH-MonitoringOccasionOfPO Points out the first PDCCH monitoring occasion for paging of each PO of the PF, see TS 38.304 [20].

nAndPagingFrameOffset Used to derive the number of total paging frames in T (corresponding to parameter N in TS 38.304 [20]) and paging frame offset (corresponding to parameter PF_offset in TS 38.304 [20]). A value of oneSixteenthT corresponds to T / 16, a value of oneEighthT corresponds to T / 8, and so on. If pagingSearchSpace is set to zero and if SS/PBCH block and CORESET multiplexing pattern is 2 or 3 (as specified in TS 38.213 [13]): - for ssb-periodicityServingCell of 5 or 10 ms, N can be set to one of {oneT, halfT, quarterT, oneEighthT, oneSixteenthT} - for ssb-periodicityServingCell of 20 ms, N can be set to one of {halfT, quarterT, oneEighthT, oneSixteenthT} - for ssb-periodicityServingCell of 40 ms, N can be set to one of {quarterT, oneEighthT, oneSixteenthT} - for ssb-periodicityServingCell of 80 ms, N can be set to one of {oneEighthT, oneSixteenthT} - for ssb-periodicityServingCell of 160 ms, N can be set to oneSixteenthT If pagingSearchSpace is set to zero and if SS/PBCH block and CORESET multiplexing pattern is 1 (as specified in TS 38.213 [13]), N can be set to one of {halfT, quarterT, oneEighthT, oneSixteenthT} If pagingSearchSpace is not set to zero, N can be configured to one of {oneT, halfT, quarterT, oneEighthT, oneSixteenthT}

ns Number of paging occasions per paging frame.

#### –	DownlinkPreemption

The IE DownlinkPreemption is used to configure the UE to monitor PDCCH for the INT-RNTI (interruption).

DownlinkPreemption information element

-- ASN1START

-- TAG-DOWNLINKPREEMPTION-START

DownlinkPreemption ::=              SEQUENCE {

int-RNTI                            RNTI-Value,

timeFrequencySet                    ENUMERATED {set0, set1},

dci-PayloadSize                     INTEGER (0..maxINT-DCI-PayloadSize),

int-ConfigurationPerServingCell     SEQUENCE (SIZE (1..maxNrofServingCells)) OF INT-ConfigurationPerServingCell,

...

}

INT-ConfigurationPerServingCell ::= SEQUENCE {

servingCellId                       ServCellIndex,

positionInDCI                       INTEGER (0..maxINT-DCI-PayloadSize-1)

}

-- TAG-DOWNLINKPREEMPTION-STOP

-- ASN1STOP

DownlinkPreemption field descriptions

dci-PayloadSize Total length of the DCI payload scrambled with INT-RNTI (see TS 38.213 [13], clause 11.2).

int-ConfigurationPerServingCell Indicates (per serving cell) the position of the 14 bit INT values inside the DCI payload (see TS 38.213 [13], clause 11.2).

int-RNTI RNTI used for indication pre-emption in DL (see TS 38.213 [13], clause 10).

timeFrequencySet Set selection for DL-preemption indication (see TS 38.213 [13], clause 11.2) The set determines how the UE interprets the DL preemption DCI payload.

INT-ConfigurationPerServingCell field descriptions

positionInDCI Starting position (in number of bit) of the 14 bit INT value applicable for this serving cell (servingCellId) within the DCI payload (see TS 38.213 [13], clause 11.2). Must be multiples of 14 (bit).

#### –	DRB-Identity

The IE DRB-Identity is used to identify a DRB used by a UE.

DRB-Identity information element

-- ASN1START

-- TAG-DRB-IDENTITY-START

DRB-Identity ::=                    INTEGER (1..32)

-- TAG-DRB-IDENTITY-STOP

-- ASN1STOP

#### –	DRX-Config

The IE DRX-Config is used to configure DRX related parameters.

DRX-Config information element

-- ASN1START

-- TAG-DRX-CONFIG-START

DRX-Config ::=                      SEQUENCE {

drx-onDurationTimer                 CHOICE {

subMilliSeconds INTEGER (1..31),

milliSeconds    ENUMERATED {

ms1, ms2, ms3, ms4, ms5, ms6, ms8, ms10, ms20, ms30, ms40, ms50, ms60,

ms80, ms100, ms200, ms300, ms400, ms500, ms600, ms800, ms1000, ms1200,

ms1600, spare8, spare7, spare6, spare5, spare4, spare3, spare2, spare1 }

},

drx-InactivityTimer                 ENUMERATED {

ms0, ms1, ms2, ms3, ms4, ms5, ms6, ms8, ms10, ms20, ms30, ms40, ms50, ms60, ms80,

ms100, ms200, ms300, ms500, ms750, ms1280, ms1920, ms2560, spare9, spare8,

spare7, spare6, spare5, spare4, spare3, spare2, spare1},

drx-HARQ-RTT-TimerDL                INTEGER (0..56),

drx-HARQ-RTT-TimerUL                INTEGER (0..56),

drx-RetransmissionTimerDL           ENUMERATED {

sl0, sl1, sl2, sl4, sl6, sl8, sl16, sl24, sl33, sl40, sl64, sl80, sl96, sl112, sl128,

sl160, sl320, spare15, spare14, spare13, spare12, spare11, spare10, spare9,

spare8, spare7, spare6, spare5, spare4, spare3, spare2, spare1},

drx-RetransmissionTimerUL           ENUMERATED {

sl0, sl1, sl2, sl4, sl6, sl8, sl16, sl24, sl33, sl40, sl64, sl80, sl96, sl112, sl128,

sl160, sl320, spare15, spare14, spare13, spare12, spare11, spare10, spare9,

spare8, spare7, spare6, spare5, spare4, spare3, spare2, spare1 },

drx-LongCycleStartOffset            CHOICE {

ms10                                INTEGER(0..9),

ms20                                INTEGER(0..19),

ms32                                INTEGER(0..31),

ms40                                INTEGER(0..39),

ms60                                INTEGER(0..59),

ms64                                INTEGER(0..63),

ms70                                INTEGER(0..69),

ms80                                INTEGER(0..79),

ms128                               INTEGER(0..127),

ms160                               INTEGER(0..159),

ms256                               INTEGER(0..255),

ms320                               INTEGER(0..319),

ms512                               INTEGER(0..511),

ms640                               INTEGER(0..639),

ms1024                              INTEGER(0..1023),

ms1280                              INTEGER(0..1279),

ms2048                              INTEGER(0..2047),

ms2560                              INTEGER(0..2559),

ms5120                              INTEGER(0..5119),

ms10240                             INTEGER(0..10239)

},

shortDRX                            SEQUENCE {

drx-ShortCycle                      ENUMERATED  {

ms2, ms3, ms4, ms5, ms6, ms7, ms8, ms10, ms14, ms16, ms20, ms30, ms32,

ms35, ms40, ms64, ms80, ms128, ms160, ms256, ms320, ms512, ms640, spare9,

spare8, spare7, spare6, spare5, spare4, spare3, spare2, spare1 },

drx-ShortCycleTimer                 INTEGER (1..16)

}                                                                                                           OPTIONAL,   -- Need R

drx-SlotOffset                      INTEGER (0..31)

}

-- TAG-DRX-CONFIG-STOP

-- ASN1STOP

DRX-Config field descriptions

drx-HARQ-RTT-TimerDL Value in number of symbols of the BWP where the transport block was received.

drx-HARQ-RTT-TimerUL Value in number of symbols of the BWP where the transport block was transmitted.

drx-InactivityTimer Value in multiple integers of 1 ms. ms0 corresponds to 0, ms1 corresponds to 1 ms, ms2 corresponds to 2 ms, and so on.

drx-LongCycleStartOffset drx-LongCycle in ms and drx-StartOffset in multiples of 1 ms. If drx-ShortCycle is configured, the value of drx-LongCycle shall be a multiple of the drx-ShortCycle value.

drx-onDurationTimer Value in multiples of 1/32 ms (subMilliSeconds) or in ms (milliSecond). For the latter, value ms1 corresponds to 1 ms, value ms2 corresponds to 2 ms, and so on.

drx-RetransmissionTimerDL Value in number of slot lengths of the BWP where the transport block was received. value sl0 corresponds to 0 slots, sl1 corresponds to 1 slot, sl2 corresponds to 2 slots, and so on.

drx-RetransmissionTimerUL Value in number of slot lengths of the BWP where the transport block was transmitted. sl0 corresponds to 0 slots, sl1 corresponds to 1 slot, sl2 corresponds to 2 slots, and so on.

drx-ShortCycleTimer Value in multiples of drx-ShortCycle. A value of 1 corresponds to drx-ShortCycle, a value of 2 corresponds to 2 * drx-ShortCycle and so on.

drx-ShortCycle Value in ms. ms1 corresponds to 1 ms, ms2 corresponds to 2 ms, and so on.

drx-SlotOffset Value in 1/32 ms. Value 0 corresponds to 0 ms, value 1 corresponds to 1/32 ms, value 2 corresponds to 2/32 ms, and so on.

#### –	FilterCoefficient

The IE FilterCoefficient specifies the measurement filtering coefficient. Value fc0 corresponds to k = 0, fc1 corresponds to k = 1, and so on.

FilterCoefficient information element

-- ASN1START

-- TAG-FILTERCOEFFICIENT-START

FilterCoefficient ::=       ENUMERATED { fc0, fc1, fc2, fc3, fc4, fc5, fc6, fc7, fc8, fc9, fc11, fc13, fc15, fc17, fc19, spare1, ...}

-- TAG-FILTERCOEFFICIENT-STOP

-- ASN1STOP

#### –	FreqBandIndicatorNR

The IE FreqBandIndicatorNR is used to convey an NR frequency band number as defined in TS 38.101-1 [15] and TS 38.101-2 [39].

FreqBandIndicatorNR information element

-- ASN1START

-- TAG-FREQBANDINDICATORNR-START

FreqBandIndicatorNR ::=             INTEGER (1..1024)

-- TAG-FREQBANDINDICATORNR-STOP

-- ASN1STOP

#### –	FrequencyInfoDL

The IE FrequencyInfoDL provides basic parameters of a downlink carrier and transmission thereon.

FrequencyInfoDL information element

-- ASN1START

-- TAG-FREQUENCYINFODL-START

FrequencyInfoDL ::=                 SEQUENCE {

absoluteFrequencySSB                ARFCN-ValueNR                                                   OPTIONAL,   -- Cond SpCellAdd

frequencyBandList                   MultiFrequencyBandListNR,

absoluteFrequencyPointA             ARFCN-ValueNR,

scs-SpecificCarrierList             SEQUENCE (SIZE (1..maxSCSs)) OF SCS-SpecificCarrier,

...

}

-- TAG-FREQUENCYINFODL-STOP

-- ASN1STOP

FrequencyInfoDL field descriptions

absoluteFrequencyPointA Absolute frequency position of the reference resource block (Common RB 0). Its lowest subcarrier is also known as Point A (see TS 38.211 [16], clause 4.4.4.2). Note that the lower edge of the actual carrier is not defined by this field but rather in the scs-SpecificCarrierList.

absoluteFrequencySSB Frequency of the SSB to be used for this serving cell. SSB related parameters (e.g. SSB index) provided for a serving cell refer to this SSB frequency unless mentioned otherwise. The cell-defining SSB of the PCell is always on the sync raster. Frequencies are considered to be on the sync raster if they are also identifiable with a GSCN value (see TS 38.101-1 [15]). If the field is absent, the SSB related parameters should be absent, e.g. ssb-PositionsInBurst, ssb-periodicityServingCell and subcarrierSpacing in ServingCellConfigCommon IE. If the field is absent, the UE obtains timing reference from the SpCell. This is only supported in case the SCell is in the same frequency band as the SpCell.

frequencyBandList List containing only one frequency band to which this carrier(s) belongs. Multiple values are not supported.

scs-SpecificCarrierList A set of carriers for different subcarrier spacings (numerologies). Defined in relation to Point A. The network configures a scs-SpecificCarrier at least for each numerology (SCS) that is used e.g. in a BWP (see TS 38.211 [16], clause 5.3).

| Conditional Presence | Explanation |
| --- | --- |
| SpCellAdd | The field is mandatory present if this FrequencyInfoDL is for SpCell. Otherwise the field is optionally present, Need S. |

#### –	FrequencyInfoDL-SIB

The IE FrequencyInfoDL-SIB provides basic parameters of a downlink carrier and transmission thereon.

FrequencyInfoDL-SIB information element

-- ASN1START

-- TAG-FREQUENCYINFODL-SIB-START

FrequencyInfoDL-SIB ::=             SEQUENCE {

frequencyBandList                   MultiFrequencyBandListNR-SIB,

offsetToPointA                      INTEGER (0..2199),

scs-SpecificCarrierList             SEQUENCE (SIZE (1..maxSCSs)) OF SCS-SpecificCarrier

}

-- TAG-FREQUENCYINFODL-SIB-STOP

-- ASN1STOP

FrequencyInfoDL-SIB field descriptions

offsetToPointA Represents the offset to Point A as defined in TS 38.211 [16], clause 4.4.4.2.

frequencyBandList List of one or multiple frequency bands to which this carrier(s) belongs.

scs-SpecificCarrierList A set of carriers for different subcarrier spacings (numerologies). Defined in relation to Point A (see TS 38.211 [16], clause 5.3). The network configures this for all SCSs that are used in DL BWPs in this serving cell.

#### –	FrequencyInfoUL

The IE FrequencyInfoUL provides basic parameters of an uplink carrier and transmission thereon.

FrequencyInfoUL information element

-- ASN1START

-- TAG-FREQUENCYINFOUL-START

FrequencyInfoUL ::=                 SEQUENCE {

frequencyBandList                   MultiFrequencyBandListNR                                OPTIONAL,   -- Cond FDD-OrSUL

absoluteFrequencyPointA             ARFCN-ValueNR                                           OPTIONAL,   -- Cond FDD-OrSUL

scs-SpecificCarrierList             SEQUENCE (SIZE (1..maxSCSs)) OF SCS-SpecificCarrier,

additionalSpectrumEmission          AdditionalSpectrumEmission                              OPTIONAL,   -- Need S

p-Max                               P-Max                                                   OPTIONAL,   -- Need S

frequencyShift7p5khz                ENUMERATED {true}                                       OPTIONAL,   -- Cond FDD-TDD-OrSUL-Optional

...

}

-- TAG-FREQUENCYINFOUL-STOP

-- ASN1STOP

FrequencyInfoUL field descriptions

absoluteFrequencyPointA Absolute frequency of the reference resource block (Common RB 0). Its lowest subcarrier is also known as Point A. Note that the lower edge of the actual carrier is not defined by this field but rather in the scs-SpecificCarrierList (see TS 38.211 [16], clause 4.4.4.2).

additionalSpectrumEmission The additional spectrum emission requirements to be applied by the UE on this uplink. If the field is absent, the UE uses value 0 for the additionalSpectrumEmission (see TS 38.101-1 [15], table 6.2.3.1-1A, and TS 38.101-2 [39], table 6.2.3.1-2).

frequencyBandList List containing only one frequency band to which this carrier(s) belongs. Multiple values are not supported.

frequencyShift7p5khz Enable the NR UL transmission with a 7.5 kHz shift to the LTE raster. If the field is absent, the frequency shift is disabled.

p-Max Maximum transmit power allowed in this serving cell. The maximum transmit power that the UE may use on this serving cell may be additionally limited by p-NR-FR1 (configured for the cell group) and by p-UE-FR1 (configured total for all serving cells operating on FR1). If absent, the UE applies the maximum power according to TS 38.101-1 [15]. Value in dBm.

scs-SpecificCarrierList A set of carriers for different subcarrier spacings (numerologies). Defined in relation to Point A. The network configures a scs-SpecificCarrier at least for each numerology (SCS) that is used e.g. in a BWP (see TS 38.211 [16], clause 5.3).

| Conditional Presence | Explanation |
| --- | --- |
| FDD-OrSUL | The field is mandatory present if this FrequencyInfoUL is for the paired UL for a DL (defined in a FrequencyInfoDL) or if this FrequencyInfoUL is for a supplementary uplink (SUL). It is absent, Need R, otherwise (if this FrequencyInfoUL is for an unpaired UL (TDD). |
| FDD-TDD-OrSUL-Optional | The field is optionally present, Need R, if this FrequencyInfoUL is for the paired UL for a DL (defined in a FrequencyInfoDL), or if this FrequencyInfoUL is for an unpaired UL (TDD) in certain bands (as defined in clause 5.4.2.1 of TS 38.101-1 and in clause 5.4.2.1 of TS 38.104 [12]), or if this FrequencyInfoUL is for a supplementary uplink (SUL). It is absent, Need R, otherwise. |

#### –	FrequencyInfoUL-SIB

The IE FrequencyInfoUL-SIB provides basic parameters of an uplink carrier and transmission thereon.

FrequencyInfoUL-SIB information element

-- ASN1START

-- TAG-FREQUENCYINFOUL-SIB-START

FrequencyInfoUL-SIB ::=                 SEQUENCE {

frequencyBandList                   MultiFrequencyBandListNR-SIB                            OPTIONAL,   -- Cond FDD-OrSUL

absoluteFrequencyPointA             ARFCN-ValueNR                                           OPTIONAL,   -- Cond FDD-OrSUL

scs-SpecificCarrierList             SEQUENCE (SIZE (1..maxSCSs)) OF SCS-SpecificCarrier,

p-Max                               P-Max                                                   OPTIONAL,   -- Need S

frequencyShift7p5khz                ENUMERATED {true}                                       OPTIONAL,   -- Cond FDD-TDD-OrSUL-Optional

...

}

-- TAG-FREQUENCYINFOUL-SIB-STOP

-- ASN1STOP

FrequencyInfoUL-SIB field descriptions

absoluteFrequencyPointA Absolute frequency of the reference resource block (Common RB 0). Its lowest subcarrier is also known as Point A. Note that the lower edge of the actual carrier is not defined by this field but rather in the scs-SpecificCarrierList (see TS 38.211 [16], clause 4.4.4.2).

frequencyBandList Provides the frequency band indicator and a list of additionalPmax and additionalSpectrumEmission values as defined in TS 38.101-1 [15], table 6.2.3.1-1, and TS 38.101-2 [39], table 6.2.3.1-2. The UE shall apply the first listed band which it supports in the frequencyBandList field.

frequencyShift7p5khz Enable the NR UL transmission with a 7.5 kHz shift to the LTE raster. If the field is absent, the frequency shift is disabled.

p-Max Value in dBm applicable for the cell. If absent the UE applies the maximum power according to TS 38.101-1 [15] in case of an FR1 cell or TS 38.101-2 [39] in case of an FR2 cell. In this release of the specification, if p-Max is present on a carrier frequency in FR2, the UE shall ignore the field and applies the maximum power according to TS 38.101-2 [39].

scs-SpecificCarrierList A set of carriers for different subcarrier spacings (numerologies). Defined in relation to Point A (see TS 38.211 [16], clause 5.3). The network configures this for all SCSs that are used in UL BWPs configured in this serving cell.

| Conditional Presence | Explanation |
| --- | --- |
| FDD-OrSUL | The field is mandatory present if this FrequencyInfoUL-SIB is for the paired UL for a DL (defined in a FrequencyInfoDL-SIB) or if this FrequencyInfoUL-SIB is for a supplementary uplink (SUL). It is absent otherwise (if this FrequencyInfoUL-SIB is for an unpaired UL (TDD). |
| FDD-TDD-OrSUL-Optional | The field is optionally present, Need R, if this FrequencyInfoUL-SIB is for the paired UL for a DL (defined in a FrequencyInfoDL-SIB), or if this FrequencyInfoUL-SIB is for an unpaired UL (TDD) in certain bands (as defined in clause 5.4.2.1 of TS 38.101-1 and in clause 5.4.2.1 of TS 38.104 [12]), or if this FrequencyInfoUL-SIB is for a supplementary uplink (SUL). It is absent otherwise. |

#### –	Hysteresis

The IE Hysteresis is a parameter used within the entry and leave condition of an event triggered reporting condition. The actual value is field value * 0.5 dB.

Hysteresis information element

-- ASN1START

-- TAG-HYSTERESIS-START

Hysteresis ::=                      INTEGER (0..30)

-- TAG-HYSTERESIS-STOP

-- ASN1STOP

#### –	I-RNTI-Value

The IE I-RNTI-Value is used to identify the suspended UE context of a UE in RRC_INACTIVE.

I-RNTI-Value information element

-- ASN1START

-- TAG-I-RNTI-VALUE-START

I-RNTI-Value ::=                        BIT STRING (SIZE(40))

-- TAG-I-RNTI-VALUE-STOP

-- ASN1STOP

#### –	LocationMeasurementInfo

The IE LocationMeasurementInfo defines the information sent by the UE to the network to assist with the configuration of measurement gaps for location related measurements.

LocationMeasurementInfo information element

-- ASN1START

-- TAG-LOCATIONMEASUREMENTINFO-START

LocationMeasurementInfo ::=     CHOICE {

eutra-RSTD                  EUTRA-RSTD-InfoList,

...,

eutra-FineTimingDetection    NULL

}

EUTRA-RSTD-InfoList ::= SEQUENCE (SIZE (1..maxInterRAT-RSTD-Freq)) OF EUTRA-RSTD-Info

EUTRA-RSTD-Info ::= SEQUENCE {

carrierFreq                 ARFCN-ValueEUTRA,

measPRS-Offset              INTEGER (0..39),

...

}

-- TAG-LOCATIONMEASUREMENTINFO-STOP

-- ASN1STOP

LocationMeasurementInfo field descriptions

carrierFreq The EARFCN value of the carrier received from upper layers for which the UE needs to perform the inter-RAT RSTD measurements.

measPRS-Offset Indicates the requested gap offset for performing RSTD measurements towards E-UTRA. It is the smallest subframe offset from the beginning of subframe 0 of SFN=0 of the serving cell of the requested gap for measuring PRS positioning occasions in the carrier frequency carrierFreq for which the UE needs to perform the inter-RAT RSTD measurements. The PRS positioning occasion information is received from upper layers. The value of measPRS-Offset is obtained by mapping the starting subframe of the PRS positioning occasion in the measured cell onto the corresponding subframe in the serving cell and is calculated as the serving cell's number of subframes from SFN=0 mod 40. The UE shall take into account any additional time required by the UE to start PRS measurements on the other carrier when it does this mapping for determining the measPRS-Offset. NOTE: Figure 6.2.2-1 in TS 36.331[10] illustrates the measPRS-Offset field.

#### –	LogicalChannelConfig

The IE LogicalChannelConfig is used to configure the logical channel parameters.

LogicalChannelConfig information element

-- ASN1START

-- TAG-LOGICALCHANNELCONFIG-START

LogicalChannelConfig ::=            SEQUENCE {

ul-SpecificParameters               SEQUENCE {

priority                            INTEGER (1..16),

prioritisedBitRate                  ENUMERATED {kBps0, kBps8, kBps16, kBps32, kBps64, kBps128, kBps256, kBps512,

kBps1024, kBps2048, kBps4096, kBps8192, kBps16384, kBps32768, kBps65536, infinity},

bucketSizeDuration                  ENUMERATED {ms5, ms10, ms20, ms50, ms100, ms150, ms300, ms500, ms1000,

spare7, spare6, spare5, spare4, spare3,spare2, spare1},

allowedServingCells                 SEQUENCE (SIZE (1..maxNrofServingCells-1)) OF ServCellIndex

OPTIONAL,   -- PDCP-CADuplication

allowedSCS-List                     SEQUENCE (SIZE (1..maxSCSs)) OF SubcarrierSpacing       OPTIONAL,   -- Need R

maxPUSCH-Duration                   ENUMERATED {ms0p02, ms0p04, ms0p0625, ms0p125, ms0p25, ms0p5, spare2, spare1}

OPTIONAL,   -- Need R

configuredGrantType1Allowed         ENUMERATED {true}                                       OPTIONAL,   -- Need R

logicalChannelGroup                 INTEGER (0..maxLCG-ID)                                  OPTIONAL,   -- Need R

schedulingRequestID                 SchedulingRequestId                                     OPTIONAL,   -- Need R

logicalChannelSR-Mask               BOOLEAN,

logicalChannelSR-DelayTimerApplied  BOOLEAN,

...,

bitRateQueryProhibitTimer       ENUMERATED { s0, s0dot4, s0dot8, s1dot6, s3, s6, s12,s30}   OPTIONAL    -- Need R

}                                                                                               OPTIONAL,   -- Cond UL

...

}

-- TAG-LOGICALCHANNELCONFIG-STOP

-- ASN1STOP

LogicalChannelConfig field descriptions

allowedSCS-List If present, UL MAC SDUs from this logical channel can only be mapped to the indicated numerology. Otherwise, UL MAC SDUs from this logical channel can be mapped to any configured numerology. Only the values 15/30/60 kHz (for FR1) and 60/120 kHz (for FR2) are applicable. Corresponds to 'allowedSCS-List' as specified in TS 38.321 [3].

allowedServingCells If present, UL MAC SDUs from this logical channel can only be mapped to the serving cells indicated in this list. Otherwise, UL MAC SDUs from this logical channel can be mapped to any configured serving cell of this cell group. Corresponds to 'allowedServingCells' in TS 38.321 [3].

bitRateQueryProhibitTimer The timer is used for bit rate recommendation query in TS 38.321 [3], in seconds. Value s0 means 0 s, s0dot4 means 0.4 s and so on.

bucketSizeDuration Value in ms. ms5 corresponds to 5 ms, value ms10 corresponds to 10 ms, and so on.

configuredGrantType1Allowed If present, UL MAC SDUs from this logical channel can be transmitted on a configured grant type 1. Corresponds to 'configuredGrantType1Allowed' in TS 38.321 [3].

logicalChannelGroup ID of the logical channel group, as specified in TS 38.321 [3], which the logical channel belongs to.

logicalChannelSR-Mask Controls SR triggering when a configured uplink grant of type1 or type2 is configured. true indicates that SR masking is configured for this logical channel as specified in TS 38.321 [3].

logicalChannelSR-DelayTimerApplied Indicates whether to apply the delay timer for SR transmission for this logical channel. Set to false if logicalChannelSR-DelayTimer is not included in BSR-Config.

maxPUSCH-Duration If present, UL MAC SDUs from this logical channel can only be transmitted using uplink grants that result in a PUSCH duration shorter than or equal to the duration indicated by this field. Otherwise, UL MAC SDUs from this logical channel can be transmitted using an uplink grant resulting in any PUSCH duration. Corresponds to "maxPUSCH-Duration" in TS 38.321 [3]. The PUSCH duration is calculated based on the same length of all symbols, and the shortest length applies if the symbol lengths are different.

priority Logical channel priority, as specified in TS 38.321 [3].

prioritisedBitRate Value in kiloBytes/s. Value kBps0 corresponds to 0 kiloBytes/s, value kBps8 corresponds to 8 kiloBytes/s, value kBps16 corresponds to 16 kiloBytes/s, and so on. For SRBs, the value can only be set to infinity.

schedulingRequestId If present, it indicates the scheduling request configuration applicable for this logical channel, as specified in TS 38.321 [3].

| Conditional Presence | Explanation |
| --- | --- |
| PDCP-CADuplication | The field is mandatory present if the DRB/SRB associated with this logical channel is configured with PDCP CA duplication in UL (i.e. the PDCP entity is associated with multiple RLC entities belonging to the same cell group). Otherwise the field is optionally present, need R. |
| UL | The field is mandatory present for a logical channel with uplink if it serves DRB. It is optionally present, Need R, for a logical channel with uplink if it serves an SRB. Otherwise it is absent. |

#### –	LogicalChannelIdentity

The IE LogicalChannelIdentity is used to identify one logical channel (LogicalChannelConfig) and the corresponding RLC bearer (RLC-BearerConfig).

LogicalChannelIdentity information element

-- ASN1START

-- TAG-LOGICALCHANNELIDENTITY-START

LogicalChannelIdentity ::=          INTEGER (1..maxLC-ID)

-- TAG-LOGICALCHANNELIDENTITY-STOP

-- ASN1STOP

#### –	MAC-CellGroupConfig

The IE MAC-CellGroupConfig is used to configure MAC parameters for a cell group, including DRX.

MAC-CellGroupConfig information element

-- ASN1START

-- TAG-MAC-CELLGROUPCONFIG-START

MAC-CellGroupConfig ::=             SEQUENCE {

drx-Config                          SetupRelease { DRX-Config }                                     OPTIONAL,   -- Need M

schedulingRequestConfig             SchedulingRequestConfig                                         OPTIONAL,   -- Need M

bsr-Config                          BSR-Config                                                      OPTIONAL,   -- Need M

tag-Config                          TAG-Config                                                      OPTIONAL,   -- Need M

phr-Config                          SetupRelease { PHR-Config }                                     OPTIONAL,   -- Need M

skipUplinkTxDynamic                 BOOLEAN,

...,

[[

csi-Mask                                BOOLEAN                                                     OPTIONAL,   -- Need M

dataInactivityTimer                     SetupRelease { DataInactivityTimer }                        OPTIONAL    -- Cond MCG-Only

]]

}

DataInactivityTimer ::=         ENUMERATED {s1, s2, s3, s5, s7, s10, s15, s20, s40, s50, s60, s80, s100, s120, s150, s180}

-- TAG-MAC-CELLGROUPCONFIG-STOP

-- ASN1STOP

MAC-CellGroupConfig field descriptions

csi-Mask If set to true, the UE limits CSI reports to the on-duration period of the DRX cycle, see TS 38.321 [3].

dataInactivityTimer Releases the RRC connection upon data inactivity as specified in clause 5.3.8.5 and in TS 38.321 [3]. Value s1 corresponds to 1 second, value s2 corresponds to 2 seconds, and so on.

drx-Config Used to configure DRX as specified in TS 38.321 [3].

skipUplinkTxDynamic If set to true, the UE skips UL transmissions as described in TS 38.321 [3].

| Conditional Presence | Explanation |
| --- | --- |
| MCG-Only | This field is optionally present, Need M, for the MAC-CellGroupConfig of the MCG. It is absent otherwise. |

#### –	MeasConfig

The IE MeasConfig specifies measurements to be performed by the UE, and covers intra-frequency, inter-frequency and inter-RAT mobility as well as configuration of measurement gaps.

MeasConfig information element

-- ASN1START

-- TAG-MEASCONFIG-START

MeasConfig ::=                      SEQUENCE {

measObjectToRemoveList              MeasObjectToRemoveList                                              OPTIONAL,   -- Need N

measObjectToAddModList              MeasObjectToAddModList                                              OPTIONAL,   -- Need N

reportConfigToRemoveList            ReportConfigToRemoveList                                            OPTIONAL,   -- Need N

reportConfigToAddModList            ReportConfigToAddModList                                            OPTIONAL,   -- Need N

measIdToRemoveList                  MeasIdToRemoveList                                                  OPTIONAL,   -- Need N

measIdToAddModList                  MeasIdToAddModList                                                  OPTIONAL,   -- Need N

s-MeasureConfig                     CHOICE {

ssb-RSRP                            RSRP-Range,

csi-RSRP                            RSRP-Range

}                                                                                                       OPTIONAL,   -- Need M

quantityConfig                      QuantityConfig                                                      OPTIONAL,   -- Need M

measGapConfig                       MeasGapConfig                                                       OPTIONAL,   -- Need M

measGapSharingConfig                MeasGapSharingConfig                                                OPTIONAL,   -- Need M

...

}

MeasObjectToRemoveList ::=              SEQUENCE (SIZE (1..maxNrofObjectId)) OF MeasObjectId

MeasIdToRemoveList ::=                  SEQUENCE (SIZE (1..maxNrofMeasId)) OF MeasId

ReportConfigToRemoveList ::=            SEQUENCE (SIZE (1..maxReportConfigId)) OF ReportConfigId

-- TAG-MEASCONFIG-STOP

-- ASN1STOP

MeasConfig field descriptions

measGapConfig Used to setup and release measurement gaps in NR.

measIdToAddModList List of measurement identities to add and/or modify.

measIdToRemoveList List of measurement identities to remove.

measObjectToAddModList List of measurement objects to add and/or modify.

measObjectToRemoveList List of measurement objects to remove.

reportConfigToAddModList List of measurement reporting configurations to add and/or modify.

reportConfigToRemoveList List of measurement reporting configurations to remove.

s-MeasureConfig Threshold for NR SpCell RSRP measurement controlling when the UE is required to perform measurements on non-serving cells. Choice of ssb-RSRP corresponds to cell RSRP based on SS/PBCH block and choice of csi-RSRP corresponds to cell RSRP of CSI-RS.

measGapSharingConfig Specifies the measurement gap sharing scheme and controls setup/ release of measurement gap sharing.

#### –	MeasGapConfig

The IE MeasGapConfig specifies the measurement gap configuration and controls setup/release of measurement gaps.

MeasGapConfig information element

-- ASN1START

-- TAG-MEASGAPCONFIG-START

MeasGapConfig ::=                   SEQUENCE {

gapFR2                              SetupRelease { GapConfig }                                              OPTIONAL,   -- Need M

...,

[[

gapFR1                              SetupRelease { GapConfig }                                              OPTIONAL,   -- Need M

gapUE                               SetupRelease { GapConfig }                                              OPTIONAL    -- Need M

]]

}

GapConfig ::=                       SEQUENCE {

gapOffset                           INTEGER (0..159),

mgl                                 ENUMERATED {ms1dot5, ms3, ms3dot5, ms4, ms5dot5, ms6},

mgrp                                ENUMERATED {ms20, ms40, ms80, ms160},

mgta                                ENUMERATED {ms0, ms0dot25, ms0dot5},

...,

[[

refServCellIndicator                ENUMERATED {pCell, pSCell, mcg-FR2}                                         OPTIONAL   -- Cond NEDCorNRDC

]]

}

-- TAG-MEASGAPCONFIG-STOP

-- ASN1STOP

MeasGapConfig field descriptions

gapFR1 Indicates measurement gap configuration that applies to FR1 only. In (NG)EN-DC, gapFR1 cannot be set up by NR RRC (i.e. only LTE RRC can configure FR1 measurement gap). In NE-DC, gapFR1 can only be set up by NR RRC (i.e. LTE RRC cannot configure FR1 gap). In NR-DC, gapFR1 can only be set up in the measConfig associated with MCG. gapFR1 can not be configured together with gapUE. The applicability of the FR1 measurement gap is according to Table 9.1.2-2 and Table 9.1.2-3 in TS 38.133 [14].

gapFR2 Indicates measurement gap configuration applies to FR2 only. In (NG)EN-DC or NE-DC, gapFR2 can only be set up by NR RRC (i.e. LTE RRC cannot configure FR2 gap). In NR-DC, gapFR2 can only be set up in the measConfig associated with MCG. gapFR2 cannot be configured together with gapUE. The applicability of the FR2 measurement gap is according to Table 9.1.2-2 and Table 9.1.2-3 in TS 38.133 [14].

gapUE Indicates measurement gap configuration that applies to all frequencies (FR1 and FR2). In (NG)EN-DC, gapUE cannot be set up by NR RRC (i.e. only LTE RRC can configure per UE measurement gap). In NE-DC, gapUE can only be set up by NR RRC (i.e. LTE RRC cannot configure per UE gap). In NR-DC, gapUE can only be set up in the measConfig associated with MCG. If gapUE is configured, then neither gapFR1 nor gapFR2 can be configured. The applicability of the per UE measurement gap is according to Table 9.1.2-2 and Table 9.1.2-3 in TS 38.133 [14].

gapOffset Value gapOffset is the gap offset of the gap pattern with MGRP indicated in the field mgrp. The value range is from 0 to mgrp-1.

mgl Value mgl is the measurement gap length in ms of the measurement gap. The measurement gap length is according to in Table 9.1.2-1 in TS 38.133 [14]. Value ms1dot5 corresponds to 1.5 ms, ms3 corresponds to 3 ms and so on.

mgrp Value mgrp is measurement gap repetition period in (ms) of the measurement gap. The measurement gap repetition period is according to Table 9.1.2-1 in TS 38.133 [14].

mgta Value mgta is the measurement gap timing advance in ms. The applicability of the measurement gap timing advance is according to clause 9.1.2 of TS 38.133 [14]. Value ms0 corresponds to 0 ms, ms0dot25 corresponds to 0.25 ms and ms0dot5 corresponds to 0.5 ms. For FR2, the network only configures 0 ms and 0.25 ms.

refServCellIndicator Indicates the serving cell whose SFN and subframe are used for gap calculation for this gap pattern. Value pCell corresponds to the PCell, pSCell corresponds to the PSCell, and mcg-FR2 corresponds to a serving cell on FR2 frequency in MCG.

| Conditional Presence | Explanation |
| --- | --- |
| NEDCorNRDC | This field is mandatory present when configuring gap pattern to UE in NE-DC or NR-DC. In case the gap pattern to UE in NE-DC and NR-DC is already configured, then the field is absent, need M. Otherwise, it is absent. |

#### –	MeasGapSharingConfig

The IE MeasGapSharingConfig specifies the measurement gap sharing scheme and controls setup/ release of measurement gap sharing.

MeasGapSharingConfig information element

-- ASN1START

-- TAG-MEASGAPSHARINGCONFIG-START

MeasGapSharingConfig ::=        SEQUENCE {

gapSharingFR2                   SetupRelease { MeasGapSharingScheme }       OPTIONAL,   -- Need M

...,

[[

gapSharingFR1                   SetupRelease { MeasGapSharingScheme }       OPTIONAL,   --Need M

gapSharingUE                    SetupRelease { MeasGapSharingScheme }       OPTIONAL    --Need M

]]

}

MeasGapSharingScheme::=         ENUMERATED {scheme00, scheme01, scheme10, scheme11}

-- TAG-MEASGAPSHARINGCONFIG-STOP

-- ASN1STOP

MeasGapSharingConfig field descriptions

gapSharingFR1 Indicates the measurement gap sharing scheme that applies to the gap set for FR1 only. In (NG)EN-DC, gapSharingFR1 cannot be set up by NR RRC (i.e. only LTE RRC can configure FR1 gap sharing). In NE-DC, gapSharingFR1 can only be set up by NR RRC (i.e. LTE RRC cannot configure FR1 gap sharing). In NR-DC, gapSharingFR1 can only be set up in the measConfig associated with MCG. gapSharingFR1 can not be configured together with gapSharingUE. For the applicability of the different gap sharing schemes, see TS 38.133 [14]. Value scheme00 corresponds to scheme "00", value scheme01 corresponds to scheme "01", and so on.

gapSharingFR2 Indicates the measurement gap sharing scheme that applies to the gap set for FR2 only. In (NG)EN-DC or NE-DC, gapSharingFR2 can only be set up by NR RRC (i.e. LTE RRC cannot configure FR2 gap sharing). In NR-DC, gapSharingFR2 can only be set up by MCG in the measConfig associated with MCG. gapSharingFR2 cannot be configured together with gapSharingUE. For applicability of the different gap sharing schemes, see TS 38.133 [14]. Value scheme00 corresponds to scheme "00", value scheme01 corresponds to scheme "01", and so on.

gapSharingUE Indicates the measurement gap sharing scheme that applies to the gap set per UE. In (NG)EN-DC, gapSharingUE cannot be set up by NR RRC (i.e. only LTE RRC can configure per UE gap sharing). In NE-DC, gapSharingUE can only be set up by NR RRC (i.e. LTE RRC cannot configure per UE gap sharing). In NR-DC, gapSharingUE can only be set up in the measConfig associated with MCG. If gapSharingUE is configured, then neither gapSharingFR1 nor gapSharingFR2 can be configured. For the applicability of the different gap sharing schemes, see TS 38.133 [14]. Value scheme00 corresponds to scheme "00", value scheme01 corresponds to scheme "01", and so on.

#### –	MeasId

The IE MeasId is used to identify a measurement configuration, i.e., linking of a measurement object and a reporting configuration.

MeasId information element

-- ASN1START

-- TAG-MEASID-START

MeasId ::=                          INTEGER (1..maxNrofMeasId)

-- TAG-MEASID-STOP

-- ASN1STOP

#### –	MeasIdToAddModList

The IE MeasIdToAddModList concerns a list of measurement identities to add or modify, with for each entry the measId, the associated measObjectId and the associated reportConfigId.

MeasIdToAddModList information element

-- ASN1START

-- TAG-MEASIDTOADDMODLIST-START

MeasIdToAddModList ::=              SEQUENCE (SIZE (1..maxNrofMeasId)) OF MeasIdToAddMod

MeasIdToAddMod ::=                  SEQUENCE {

measId                              MeasId,

measObjectId                        MeasObjectId,

reportConfigId                      ReportConfigId

}

-- TAG-MEASIDTOADDMODLIST-STOP

-- ASN1STOP

#### –	MeasObjectEUTRA

The IE MeasObjectEUTRA specifies information applicable for E-UTRA cells.

MeasObjectEUTRA information element

-- ASN1START

-- TAG-MEASOBJECTEUTRA-START

MeasObjectEUTRA::=                          SEQUENCE {

carrierFreq                                 ARFCN-ValueEUTRA,

allowedMeasBandwidth                        EUTRA-AllowedMeasBandwidth,

cellsToRemoveListEUTRAN                     EUTRA-CellIndexList                                         OPTIONAL,    -- Need N

cellsToAddModListEUTRAN                     SEQUENCE (SIZE (1..maxCellMeasEUTRA)) OF EUTRA-Cell         OPTIONAL,    -- Need N

blackCellsToRemoveListEUTRAN                EUTRA-CellIndexList                                         OPTIONAL,    -- Need N

blackCellsToAddModListEUTRAN                SEQUENCE (SIZE (1..maxCellMeasEUTRA)) OF EUTRA-BlackCell    OPTIONAL,    -- Need N

eutra-PresenceAntennaPort1                  EUTRA-PresenceAntennaPort1,

eutra-Q-OffsetRange                         EUTRA-Q-OffsetRange                                         OPTIONAL,    -- Need R

widebandRSRQ-Meas                           BOOLEAN,

...

}

EUTRA-CellIndexList ::=                     SEQUENCE (SIZE (1..maxCellMeasEUTRA)) OF EUTRA-CellIndex

EUTRA-CellIndex ::=                         INTEGER (1..maxCellMeasEUTRA)

EUTRA-Cell ::=                              SEQUENCE {

cellIndexEUTRA                              EUTRA-CellIndex,

physCellId                                  EUTRA-PhysCellId,

cellIndividualOffset                        EUTRA-Q-OffsetRange

}

EUTRA-BlackCell ::=                         SEQUENCE {

cellIndexEUTRA                              EUTRA-CellIndex,

physCellIdRange                             EUTRA-PhysCellIdRange

}

-- TAG-MEASOBJECTEUTRA-STOP

-- ASN1STOP

EUTRAN-BlackCell field descriptions

cellIndexEUTRA Entry index in the cell list.

physicalCellIdRange Physical cell identity or a range of physical cell identities.

EUTRAN-Cell field descriptions

physicalCellId Physical cell identity of a cell in the cell list.

cellIndividualOffset Cell individual offset applicable to a specific cell. Value dB-24 corresponds to -24 dB, dB-22 corresponds to -22 dB and so on.

MeasObjectEUTRA field descriptions

allowedMeasBandwidth The maximum allowed measurement bandwidth on a carrier frequency as defined by the parameter Transmission Bandwidth Configuration "NRB" TS 36.104 [33].

blackCellsToAddModListEUTRAN List of cells to add/ modify in the black list of cells.

blackCellsToRemoveListEUTRAN List of cells to remove from the black list of cells.

carrierFreq Identifies E-UTRA carrier frequency for which this configuration is valid. Network does not configure more than one MeasObjectEUTRA for the same physical frequency, regardless of the E-ARFCN used to indicate this.

cellsToAddModListEUTRAN List of cells to add/ modify in the cell list.

cellsToRemoveListEUTRAN List of cells to remove from the cell list.

eutra-PresenceAntennaPort1 When set to true, the UE may assume that at least two cell-specific antenna ports are used in all neighbouring cells.

eutra-Q-OffsetRange Used to indicate a cell, or frequency specific offset to be applied when evaluating triggering conditions for measurement reporting. The value is in dB. Value dB-24 corresponds to -24 dB, value dB-22 corresponds to -22 dB and so on.

widebandRSRQ-Meas If set to true, the UE shall, when performing RSRQ measurements, use a wider bandwidth in accordance with TS 36.133 [40]. The network may set the field to true if the measurement bandwidth indicated by allowedMeasBandwidth is 50 resource blocks or larger; otherwise the network sets this field to false.

#### –	MeasObjectId

The IE MeasObjectId used to identify a measurement object configuration.

MeasObjectId information element

-- ASN1START

-- TAG-MEASOBJECTID-START

MeasObjectId ::=                    INTEGER (1..maxNrofObjectId)

-- TAG-MEASOBJECTID-STOP

-- ASN1STOP

#### –	MeasObjectNR

The IE MeasObjectNR specifies information applicable for SS/PBCH block(s) intra/inter-frequency measurements and/or CSI-RS intra/inter-frequency measurements.

MeasObjectNR information element

-- ASN1START

-- TAG-MEASOBJECTNR-START

MeasObjectNR ::=                    SEQUENCE {

ssbFrequency                        ARFCN-ValueNR                                           OPTIONAL,   -- Cond SSBorAssociatedSSB

ssbSubcarrierSpacing                SubcarrierSpacing                                       OPTIONAL,   -- Cond SSBorAssociatedSSB

smtc1                               SSB-MTC                                                 OPTIONAL,   -- Cond SSBorAssociatedSSB

smtc2                               SSB-MTC2                                                OPTIONAL,   -- Cond IntraFreqConnected

refFreqCSI-RS                       ARFCN-ValueNR                                           OPTIONAL,   -- Cond CSI-RS

referenceSignalConfig               ReferenceSignalConfig,

absThreshSS-BlocksConsolidation     ThresholdNR                                                     OPTIONAL,   -- Need R

absThreshCSI-RS-Consolidation       ThresholdNR                                                     OPTIONAL,   -- Need R

nrofSS-BlocksToAverage              INTEGER (2..maxNrofSS-BlocksToAverage)                          OPTIONAL,   -- Need R

nrofCSI-RS-ResourcesToAverage       INTEGER (2..maxNrofCSI-RS-ResourcesToAverage)                   OPTIONAL,   -- Need R

quantityConfigIndex                 INTEGER (1..maxNrofQuantityConfig),

offsetMO                            Q-OffsetRangeList,

cellsToRemoveList                   PCI-List                                                        OPTIONAL,   -- Need N

cellsToAddModList                   CellsToAddModList                                               OPTIONAL,   -- Need N

blackCellsToRemoveList              PCI-RangeIndexList                                              OPTIONAL,   -- Need N

blackCellsToAddModList              SEQUENCE (SIZE (1..maxNrofPCI-Ranges)) OF PCI-RangeElement      OPTIONAL,   -- Need N

whiteCellsToRemoveList              PCI-RangeIndexList                                              OPTIONAL,   -- Need N

whiteCellsToAddModList              SEQUENCE (SIZE (1..maxNrofPCI-Ranges)) OF PCI-RangeElement      OPTIONAL,   -- Need N

...,

[[

freqBandIndicatorNR                 FreqBandIndicatorNR                                             OPTIONAL,   -- Need R

measCycleSCell                      ENUMERATED {sf160, sf256, sf320, sf512, sf640, sf1024, sf1280}  OPTIONAL    -- Need R

]]

}

ReferenceSignalConfig::=            SEQUENCE {

ssb-ConfigMobility                  SSB-ConfigMobility                                              OPTIONAL,   -- Need M

csi-rs-ResourceConfigMobility       SetupRelease { CSI-RS-ResourceConfigMobility }                  OPTIONAL    -- Need M

}

SSB-ConfigMobility::=               SEQUENCE {

ssb-ToMeasure                           SetupRelease { SSB-ToMeasure }                              OPTIONAL,   -- Need M

deriveSSB-IndexFromCell             BOOLEAN,

ss-RSSI-Measurement                         SS-RSSI-Measurement                                     OPTIONAL,   -- Need M

...

}

Q-OffsetRangeList ::=               SEQUENCE {

rsrpOffsetSSB                       Q-OffsetRange               DEFAULT dB0,

rsrqOffsetSSB                       Q-OffsetRange               DEFAULT dB0,

sinrOffsetSSB                       Q-OffsetRange               DEFAULT dB0,

rsrpOffsetCSI-RS                    Q-OffsetRange               DEFAULT dB0,

rsrqOffsetCSI-RS                    Q-OffsetRange               DEFAULT dB0,

sinrOffsetCSI-RS                    Q-OffsetRange               DEFAULT dB0

}

ThresholdNR ::=                     SEQUENCE{

thresholdRSRP                       RSRP-Range                                                      OPTIONAL,   -- Need R

thresholdRSRQ                       RSRQ-Range                                                      OPTIONAL,   -- Need R

thresholdSINR                       SINR-Range                                                      OPTIONAL    -- Need R

}

CellsToAddModList ::=               SEQUENCE (SIZE (1..maxNrofCellMeas)) OF CellsToAddMod

CellsToAddMod ::=                   SEQUENCE {

physCellId                          PhysCellId,

cellIndividualOffset                Q-OffsetRangeList

}

-- TAG-MEASOBJECTNR-STOP

-- ASN1STOP

CellsToAddMod field descriptions

cellIndividualOffset Cell individual offsets applicable to a specific cell.

physCellId Physical cell identity of a cell in the cell list.

MeasObjectNR field descriptions

absThreshCSI-RS-Consolidation Absolute threshold for the consolidation of measurement results per CSI-RS resource(s) from L1 filter(s). The field is used for the derivation of cell measurement results as described in 5.5.3.3 and the reporting of beam measurement information per CSI-RS resource as described in 5.5.5.2.

absThreshSS-BlocksConsolidation Absolute threshold for the consolidation of measurement results per SS/PBCH block(s) from L1 filter(s). The field is used for the derivation of cell measurement results as described in 5.5.3.3 and the reporting of beam measurement information per SS/PBCH block index as described in 5.5.5.2.

blackCellsToAddModList List of cells to add/modify in the black list of cells. It applies only to SSB resources.

blackCellsToRemoveList List of cells to remove from the black list of cells.

cellsToAddModList List of cells to add/modify in the cell list.

cellsToRemoveList List of cells to remove from the cell list.

freqBandIndicatorNR The frequency band in which the SSB and/or CSI-RS indicated in this MeasObjectNR are located and according to which the UE shall perform the RRM measurements. This field is always provided when the network configures measurements with this MeasObjectNR.

measCycleSCell The parameter is used only when an SCell is configured on the frequency indicated by the measObjectNR and is in deactivated state, see TS 38.133 [14]. gNB configures the parameter whenever an SCell is configured on the frequency indicated by the measObjectNR, but the field may also be signalled when an SCell is not configured. Value sf160 corresponds to 160 sub-frames, value sf256 corresponds to 256 sub-frames and so on.

nrofCSInrofCSI-RS-ResourcesToAverage Indicates the maximum number of measurement results per beam based on CSI-RS resources to be averaged. The same value applies for each detected cell associated with this MeasObjectNR.

nrofSS-BlocksToAverage Indicates the maximum number of measurement results per beam based on SS/PBCH blocks to be averaged. The same value applies for each detected cell associated with this MeasObject.

offsetMO Offset values applicable to all measured cells with reference signal(s) indicated in this MeasObjectNR.

quantityConfigIndex Indicates the n-th element of quantityConfigNR-List provided in MeasConfig.

referenceSignalConfig RS configuration for SS/PBCH block and CSI-RS.

refFreqCSI-RS Point A which is used for mapping of CSI-RS to physical resources according to TS 38.211 [16] clause 7.4.1.5.3.

smtc1 Primary measurement timing configuration. (see clause 5.5.2.10).

smtc2 Secondary measurement timing configuration for SS corresponding to this MeasObjectNR with PCI listed in pci-List. For these SS, the periodicity is indicated by periodicity in smtc2 and the timing offset is equal to the offset indicated in periodicityAndOffset modulo periodicity. periodicity in smtc2 can only be set to a value strictly shorter than the periodicity indicated by periodicityAndOffset in smtc1 (e.g. if periodicityAndOffset indicates sf10, periodicity can only be set of sf5, if periodicityAndOffset indicates sf5, smtc2 cannot be configured).

ssbFrequency Indicates the frequency of the SS associated to this MeasObjectNR.

ssbSubcarrierSpacing Subcarrier spacing of SSB. Only the values 15 kHz or 30 kHz (FR1), and 120 kHz or 240 kHz (FR2) are applicable.

whiteCellsToAddModList List of cells to add/modify in the white list of cells. It applies only to SSB resources.

whiteCellsToRemoveList List of cells to remove from the white list of cells.

ReferenceSignalConfig field descriptions

csi-rs-ResourceConfigMobility CSI-RS resources to be used for CSI-RS based RRM measurements.

ssb-ConfigMobility SSB configuration for mobility (nominal SSBs, timing configuration).

SSB-ConfigMobility field descriptions

deriveSSB-IndexFromCell If this field is set to true, UE assumes SFN and frame boundary alignment across cells on the same frequency carrier as specified in TS 38.133 [14]. Hence, if the UE is configured with a serving cell for which (absoluteFrequencySSB, subcarrierSpacing) in ServingCellConfigCommon is equal to (ssbFrequency, ssbSubcarrierSpacing) in this MeasObjectNR, this field indicates whether the UE can utilize the timing of this serving cell to derive the index of SS block transmitted by neighbour cell. Otherwise, this field indicates whether the UE may use the timing of any detected cell on that target frequency to derive the SSB index of all neighbour cells on that frequency.

ssb-ToMeasure The set of SS blocks to be measured within the SMTC measurement duration. The first/leftmost bit corresponds to SS/PBCH block index 0, the second bit corresponds to SS/PBCH block index 1, and so on. Value 0 in the bitmap indicates that the corresponding SS/PBCH block is not to be measured while value 1 indicates that the corresponding SS/PBCH block is to be measured (see TS 38.215 [9]). When the field is not configured the UE measures on all SS blocks. Regardless of the value of this field, SS/PBCH blocks outside of the applicable smtc are not to be measured. See TS 38.215 [9] clause 5.1.1.

| Conditional Presence | Explanation |
| --- | --- |
| CSI-RS | This field is mandatory present if csi-rs-ResourceConfigMobility is configured, otherwise, it is absent. |
| SSBorAssociatedSSB | This field is mandatory present if ssb-ConfigMobility is configured or associatedSSB is configured in at least one cell. Otherwise, it is absent, Need R. |
| IntraFreqConnected | This field is optionally present, Need R if the UE is configured with a serving cell for which (absoluteFrequencySSB, subcarrierSpacing) in ServingCellConfigCommon is equal to (ssbFrequency, ssbSubcarrierSpacing) in this MeasObjectNR, otherwise, it is absent. |

#### –	MeasObjectToAddModList

The IE MeasObjectToAddModList concerns a list of measurement objects to add or modify.

MeasObjectToAddModList information element

-- ASN1START

-- TAG-MEASOBJECTTOADDMODLIST-START

MeasObjectToAddModList ::=                  SEQUENCE (SIZE (1..maxNrofObjectId)) OF MeasObjectToAddMod

MeasObjectToAddMod ::=                      SEQUENCE {

measObjectId                                MeasObjectId,

measObject                                  CHOICE {

measObjectNR                                MeasObjectNR,

...,

measObjectEUTRA                             MeasObjectEUTRA

}

}

-- TAG-MEASOBJECTTOADDMODLIST-STOP

-- ASN1STOP

#### –	MeasResultCellListSFTD-NR

The IE MeasResultCellListSFTD-NR consists of SFN and radio frame boundary difference between the PCell and an NR cell as specified in TS 38.215 [9] and TS 38.133 [14].

MeasResultCellListSFTD-NR information element

-- ASN1START

-- TAG-MEASRESULTCELLLISTSFTD-NR-START

MeasResultCellListSFTD-NR ::=          SEQUENCE (SIZE (1..maxCellSFTD)) OF MeasResultCellSFTD-NR

MeasResultCellSFTD-NR ::=              SEQUENCE {

physCellId                            PhysCellId,

sfn-OffsetResult                      INTEGER (0..1023),

frameBoundaryOffsetResult             INTEGER (-30720..30719),

rsrp-Result                           RSRP-Range                      OPTIONAL

}

-- TAG-MEASRESULTCELLLISTSFTD-NR-STOP

-- ASN1STOP

MeasResultCellSFTD-NR field descriptions

sfn-OffsetResult Indicates the SFN difference between the PCell and the NR cell as an integer value according to TS 38.215 [9].

frameBoundaryOffsetResult Indicates the frame boundary difference between the PCell and the NR cell as an integer value according to TS 38.215 [9].

#### –	MeasResultCellListSFTD-EUTRA

The IE MeasResultCellListSFTD-EUTRA consists of SFN and radio frame boundary difference between the PCell and an E-UTRA PSCell.

MeasResultCellListSFTD-EUTRA information element

-- ASN1START

-- TAG-MEASRESULTCELLLISTSFTD-EUTRA-START

MeasResultCellListSFTD-EUTRA ::=          SEQUENCE (SIZE (1..maxCellSFTD)) OF MeasResultSFTD-EUTRA

MeasResultSFTD-EUTRA ::=           SEQUENCE {

eutra-PhysCellId                    EUTRA-PhysCellId,

sfn-OffsetResult                    INTEGER (0..1023),

frameBoundaryOffsetResult           INTEGER (-30720..30719),

rsrp-Result                         RSRP-Range                      OPTIONAL

}

-- TAG-MEASRESULTCELLLISTSFTD-EUTRA-STOP

-- ASN1STOP

MeasResultSFTD-EUTRA field descriptions

eutra-PhysCellId Identifies the physical cell identity of the E-UTRA cell for which the reporting is being performed.

sfn-OffsetResult Indicates the SFN difference between the PCell and the E-UTRA cell as an integer value according to TS 38.215 [9].

frameBoundaryOffsetResult Indicates the frame boundary difference between the PCell and the E-UTRA cell as an integer value according to TS 38.215 [9].

#### –	MeasResults

The IE MeasResults covers measured results for intra-frequency, inter-frequency, and inter-RAT mobility.

MeasResults information element

-- ASN1START

-- TAG-MEASRESULTS-START

MeasResults ::=                         SEQUENCE {

measId                                  MeasId,

measResultServingMOList                 MeasResultServMOList,

measResultNeighCells                    CHOICE {

measResultListNR                        MeasResultListNR,

...,

measResultListEUTRA                     MeasResultListEUTRA

}                                                                                                                   OPTIONAL,

...,

[[

measResultServFreqListEUTRA-SCG         MeasResultServFreqListEUTRA-SCG                                             OPTIONAL,

measResultServFreqListNR-SCG            MeasResultServFreqListNR-SCG                                                OPTIONAL,

measResultSFTD-EUTRA                    MeasResultSFTD-EUTRA                                                        OPTIONAL,

measResultSFTD-NR                       MeasResultCellSFTD-NR                                                       OPTIONAL

]],

[[

measResultCellListSFTD-NR                       MeasResultCellListSFTD-NR                                                       OPTIONAL

]]

}

MeasResultServMOList ::=                SEQUENCE (SIZE (1..maxNrofServingCells)) OF MeasResultServMO

MeasResultServMO ::=                    SEQUENCE {

servCellId                              ServCellIndex,

measResultServingCell                   MeasResultNR,

measResultBestNeighCell                 MeasResultNR                                                                OPTIONAL,

...

}

MeasResultListNR ::=                    SEQUENCE (SIZE (1..maxCellReport)) OF MeasResultNR

MeasResultNR ::=                        SEQUENCE {

physCellId                              PhysCellId                                                                  OPTIONAL,

measResult                              SEQUENCE {

cellResults                             SEQUENCE{

resultsSSB-Cell                         MeasQuantityResults                                                 OPTIONAL,

resultsCSI-RS-Cell                      MeasQuantityResults                                                 OPTIONAL

},

rsIndexResults                          SEQUENCE{

resultsSSB-Indexes                      ResultsPerSSB-IndexList                                             OPTIONAL,

resultsCSI-RS-Indexes                   ResultsPerCSI-RS-IndexList                                          OPTIONAL

}                                                                                                               OPTIONAL

},

...,

[[

cgi-Info                                CGI-InfoNR                                                                  OPTIONAL

]]

}

MeasResultListEUTRA ::=                 SEQUENCE (SIZE (1..maxCellReport)) OF MeasResultEUTRA

MeasResultEUTRA ::=                     SEQUENCE {

eutra-PhysCellId                        PhysCellId,

measResult                              MeasQuantityResultsEUTRA,

cgi-Info                                CGI-InfoEUTRA                                                               OPTIONAL,

...

}

MultiBandInfoListEUTRA ::=              SEQUENCE (SIZE (1..maxMultiBands)) OF FreqBandIndicatorEUTRA

MeasQuantityResults ::=                 SEQUENCE {

rsrp                                    RSRP-Range                                                                  OPTIONAL,

rsrq                                    RSRQ-Range                                                                  OPTIONAL,

sinr                                    SINR-Range                                                                  OPTIONAL

}

MeasQuantityResultsEUTRA ::=            SEQUENCE {

rsrp                                    RSRP-RangeEUTRA                                                             OPTIONAL,

rsrq                                    RSRQ-RangeEUTRA                                                             OPTIONAL,

sinr                                    SINR-RangeEUTRA                                                             OPTIONAL

}

ResultsPerSSB-IndexList::=              SEQUENCE (SIZE (1..maxNrofIndexesToReport2)) OF ResultsPerSSB-Index

ResultsPerSSB-Index ::=                 SEQUENCE {

ssb-Index                               SSB-Index,

ssb-Results                             MeasQuantityResults                                                         OPTIONAL

}

ResultsPerCSI-RS-IndexList::=           SEQUENCE (SIZE (1..maxNrofIndexesToReport2)) OF ResultsPerCSI-RS-Index

ResultsPerCSI-RS-Index ::=              SEQUENCE {

csi-RS-Index                            CSI-RS-Index,

csi-RS-Results                          MeasQuantityResults                                                         OPTIONAL

}

MeasResultServFreqListEUTRA-SCG ::= SEQUENCE (SIZE (1..maxNrofServingCellsEUTRA)) OF MeasResult2EUTRA

MeasResultServFreqListNR-SCG ::= SEQUENCE (SIZE (1..maxNrofServingCells)) OF MeasResult2NR

-- TAG-MEASRESULTS-STOP

-- ASN1STOP

MeasResultEUTRA field descriptions

eutra-PhysCellId Identifies the physical cell identity of the E-UTRA cell for which the reporting is being performed. The UE reports a value in the range 0..503, other values are reserved.

MeasResultNR field descriptions

cellResults Cell level measurement results.

physCellId The physical cell identity of the NR cell for which the reporting is being performed.

resultsSSB-Cell Cell level measurement results based on SS/PBCH related measurements.

resultsSSB-Indexes Beam level measurement results based on SS/PBCH related measurements.

resultsCSI-RS-Cell Cell level measurement results based on CSI-RS related measurements.

resultsCSI-RS-Indexes Beam level measurement results based on CSI-RS related measurements.

rsIndexResults Beam level measurement results.

MeasResults field descriptions

measId Identifies the measurement identity for which the reporting is being performed.

measResultCellListSFTD-NR SFTD measurement results between the PCell and the NR neighbour cell(s) in NR standalone.

measResultEUTRA Measured results of an E-UTRA cell.

measResultListEUTRA List of measured results for the maximum number of reported best cells for an E-UTRA measurement identity.

measResultListNR List of measured results for the maximum number of reported best cells for an NR measurement identity.

measResultNR Measured results of an NR cell.

measResultServFreqListEUTRA-SCG Measured results of the E-UTRA SCG serving frequencies: the measurement result of PSCell and each SCell, if any, and of the best neighbouring cell on each E-UTRA SCG serving frequency.

measResultServFreqListNR-SCG Measured results of the NR SCG serving frequencies: the measurement result of PSCell and each SCell, if any, and of the best neighbouring cell on each NR SCG serving frequency.

measResultServingMOList Measured results of measured cells with reference signals indicated in the serving cell measurement objects including measurement results of SpCell, configured SCell(s) and best neighbouring cell within measured cells with reference signals indicated in on each serving cell measurement object.

measResultSFTD-EUTRA SFTD measurement results between the PCell and the E-UTRA PScell in NE-DC.

measResultSFTD-NR SFTD measurement results between the PCell and the NR PScell in NR-DC.

#### –	MeasResult2EUTRA

The IE MeasResult2EUTRA contains measurements on E-UTRA frequencies.

MeasResult2EUTRA information element

-- ASN1START

-- TAG-MEASRESULT2EUTRA-START

MeasResult2EUTRA ::=       SEQUENCE {

carrierFreq                         ARFCN-ValueEUTRA,

measResultServingCell               MeasResultEUTRA                 OPTIONAL,

measResultBestNeighCell             MeasResultEUTRA                 OPTIONAL,

...

}

-- TAG-MEASRESULT2EUTRA-STOP

-- ASN1STOP

#### –	MeasResult2NR

The IE MeasResult2NR contains measurements on NR frequencies.

MeasResult2NR information element

-- ASN1START

-- TAG-MEASRESULT2NR-START

MeasResult2NR ::=                   SEQUENCE {

ssbFrequency                        ARFCN-ValueNR                           OPTIONAL,

refFreqCSI-RS                       ARFCN-ValueNR                           OPTIONAL,

measResultServingCell               MeasResultNR                            OPTIONAL,

measResultNeighCellListNR           MeasResultListNR                        OPTIONAL,

...

}

-- TAG-MEASRESULT2NR-STOP

-- ASN1STOP

#### –	MeasResultSCG-Failure

The IE MeasResultSCG-Failure is used to provide information regarding failures detected by the UE in EN-DC.

MeasResultSCG-Failure information element

-- ASN1START

-- TAG-MEASRESULTSCG-FAILURE-START

MeasResultSCG-Failure ::=           SEQUENCE {

measResultPerMOList                 MeasResultList2NR,

...

}

MeasResultList2NR ::=               SEQUENCE (SIZE (1..maxFreq)) OF MeasResult2NR

-- TAG-MEASRESULTSCG-FAILURE-STOP

-- ASN1STOP

#### –	MeasTriggerQuantityEUTRA

The IE MeasTriggerQuantityEUTRA is used to configure the trigger quantity and reporting range for E-UTRA measurements. The RSRP, RSRQ and SINR ranges correspond to RSRP-Range, RSRQ-Range and RS-SINR-Range in TS 36.331 [10], respectively.

MeasTriggerQuantityEUTRA information element

-- ASN1START

-- TAG-MEASTRIGGERQUANTITYEUTRA-START

MeasTriggerQuantityEUTRA::=                 CHOICE {

rsrp                                        RSRP-RangeEUTRA,

rsrq                                        RSRQ-RangeEUTRA,

sinr                                        SINR-RangeEUTRA

}

RSRP-RangeEUTRA ::=                 INTEGER (0..97)

RSRQ-RangeEUTRA ::=                 INTEGER (0..34)

SINR-RangeEUTRA ::=                 INTEGER (0..127)

-- TAG-MEASTRIGGERQUANTITYEUTRA-STOP

-- ASN1STOP

#### –	MobilityStateParameters

The IE MobilityStateParameters contains parameters to determine UE mobility state.

MobilityStateParameters information element

-- ASN1START

-- TAG-MOBILITYSTATEPARAMETERS-START

MobilityStateParameters ::=         SEQUENCE{

t-Evaluation                        ENUMERATED {

s30, s60, s120, s180, s240, spare3, spare2, spare1},

t-HystNormal                        ENUMERATED {

s30, s60, s120, s180, s240, spare3, spare2, spare1},

n-CellChangeMedium                  INTEGER (1..16),

n-CellChangeHigh                    INTEGER (1..16)

}

-- TAG-MOBILITYSTATEPARAMETERS-STOP

-- ASN1STOP

MobilityStateParameters field descriptions

n-CellChangeHigh The number of cell changes to enter high mobility state. Corresponds to NCR_H in TS 38.304 [20].

n-CellChangeMedium The number of cell changes to enter medium mobility state. Corresponds to NCR_M in TS 38.304 [20].

t-Evaluation The duration for evaluating criteria to enter mobility states. Corresponds to TCRmax in TS 38.304 [20]. Value in seconds, s30 corresponds to 30 s and so on.

t-HystNormal The additional duration for evaluating criteria to enter normal mobility state. Corresponds to TCRmaxHyst in TS 38.304 [20]. Value in seconds, value s30 corresponds to 30 seconds and so on.

#### –	MultiFrequencyBandListNR

The IE MultiFrequencyBandListNR is used to configure a list of one or multiple NR frequency bands.

MultiFrequencyBandListNR information element

-- ASN1START

-- TAG-MULTIFREQUENCYBANDLISTNR-START

MultiFrequencyBandListNR ::=        SEQUENCE (SIZE (1..maxNrofMultiBands)) OF FreqBandIndicatorNR

-- TAG-MULTIFREQUENCYBANDLISTNR-STOP

-- ASN1STOP

#### –	MultiFrequencyBandListNR-SIB

The IE MultiFrequencyBandListNR-SIB indicates the list of frequency bands, for which cell (re-)selection parameters are common, and a list of additionalPmax and additionalSpectrumEmission.

MultiFrequencyBandListNR-SIB information element

-- ASN1START

-- TAG-MULTIFREQUENCYBANDLISTNR-SIB-START

MultiFrequencyBandListNR-SIB ::=            SEQUENCE (SIZE (1.. maxNrofMultiBands)) OF NR-MultiBandInfo

NR-MultiBandInfo ::=                        SEQUENCE {

freqBandIndicatorNR                         FreqBandIndicatorNR         OPTIONAL,   -- Cond OptULNotSIB2

nr-NS-PmaxList                              NR-NS-PmaxList              OPTIONAL    -- Need S

}

-- TAG-MULTIFREQUENCYBANDLISTNR-SIB-STOP

-- ASN1STOP

NR-MultiBandInfo field descriptions

freqBandIndicatorNR Provides an NR frequency band number as defined in TS 38.101-1 [15] and TS 38.101-2 [39], table 5.2-1.

nr-NS-PmaxList Provides a list of additionalPmax and additionalSpectrumEmission values. If the field is absent the UE uses value 0 for the additionalSpectrumEmission (see TS 38.101-1 [15] table 6.2.3.1-1A , and TS 38.101-2 [39], table 6.2.3.1-2).

| Conditional Presence | Explanation |
| --- | --- |
| OptULNotSIB2 | The field is absent for SIB2 and is mandatory present in SIB4 and frequencyInfoDL-SIB. Otherwise, if the field is absent in frequencyInfoUL-SIB in UplinkConfigCommonSIB, the UE will use the frequency band indicated in frequencyInfoDL-SIB in DownlinkConfigCommonSIB. |

#### –	NextHopChainingCount

The IE NextHopChainingCount is used to update the KgNB key and corresponds to parameter NCC: See TS 33.501 [11].

NextHopChainingCount information element

-- ASN1START

-- TAG-NEXTHOPCHAININGCOUNT-START

NextHopChainingCount ::=                    INTEGER (0..7)

-- TAG-NEXTHOPCHAININGCOUNT-STOP

-- ASN1STOP

#### –	NG-5G-S-TMSI

The IE NG-5G-S-TMSI contains a 5G S-Temporary Mobile Subscription Identifier (5G-S-TMSI), a temporary UE identity provided by the 5GC which uniquely identifies the UE within the tracking area, see TS 23.003 [21].

NG-5G-S-TMSI information element

-- ASN1START

-- TAG-NG-5G-S-TMSI-START

NG-5G-S-TMSI ::=                         BIT STRING (SIZE (48))

-- TAG-NG-5G-S-TMSI-STOP

-- ASN1STOP

#### –	NR-NS-PmaxList

The IE NR-NS-PmaxList is used to configure a list of additionalPmax and additionalSpectrumEmission, as defined in TS 38.101-1 [15], table 6.2.3.1-1A, and TS 38.101-2 [39], table 6.2.3.1-2, for a given frequency band.

NR-NS-PmaxList information element

-- ASN1START

-- TAG-NR-NS-PMAXLIST-START

NR-NS-PmaxList ::=                          SEQUENCE (SIZE (1..maxNR-NS-Pmax)) OF NR-NS-PmaxValue

NR-NS-PmaxValue ::=                         SEQUENCE {

additionalPmax                              P-Max                               OPTIONAL,   -- Need N

additionalSpectrumEmission                  AdditionalSpectrumEmission

}

-- TAG-NR-NS-PMAXLIST-STOP

-- ASN1STOP

#### –	NZP-CSI-RS-Resource

The IE NZP-CSI-RS-Resource is used to configure Non-Zero-Power (NZP) CSI-RS transmitted in the cell where the IE is included, which the UE may be configured to measure on (see TS 38.214 [19], clause 5.2.2.3.1). A change of configuration between periodic, semi-persistent or aperiodic for an NZP-CSI-RS-Resource is not supported without a release and add.

NZP-CSI-RS-Resource information element

-- ASN1START

-- TAG-NZP-CSI-RS-RESOURCE-START

NZP-CSI-RS-Resource ::=             SEQUENCE {

nzp-CSI-RS-ResourceId               NZP-CSI-RS-ResourceId,

resourceMapping                     CSI-RS-ResourceMapping,

powerControlOffset                  INTEGER (-8..15),

powerControlOffsetSS                ENUMERATED{db-3, db0, db3, db6}                 OPTIONAL,   -- Need R

scramblingID                        ScramblingId,

periodicityAndOffset                CSI-ResourcePeriodicityAndOffset                OPTIONAL,   -- Cond PeriodicOrSemiPersistent

qcl-InfoPeriodicCSI-RS              TCI-StateId                                     OPTIONAL,   -- Cond Periodic

...

}

-- TAG-NZP-CSI-RS-RESOURCE-STOP

-- ASN1STOP

NZP-CSI-RS-Resource field descriptions

periodicityAndOffset Periodicity and slot offset sl1 corresponds to a periodicity of 1 slot, sl2 to a periodicity of two slots, and so on. The corresponding offset is also given in number of slots (see TS 38.214 [19], clause 5.2.2.3.1). Network always configures the UE with a value for this field for periodic and semi-persistent NZP-CSI-RS-Resource (as indicated in CSI-ResourceConfig).

powerControlOffset Power offset of PDSCH RE to NZP CSI-RS RE. Value in dB (see TS 38.214 [19], clauses 5.2.2.3.1 and 4.1).

powerControlOffsetSS Power offset of NZP CSI-RS RE to SSS RE. Value in dB (see TS 38.214 [19], clause 5.2.2.3.1).

qcl-InfoPeriodicCSI-RS For a target periodic CSI-RS, contains a reference to one TCI-State in TCI-States for providing the QCL source and QCL type. For periodic CSI-RS, the source can be SSB or another periodic-CSI-RS. Refers to the TCI-State which has this value for tci-StateId and is defined in tci-StatesToAddModList in the PDSCH-Config included in the BWP-Downlink corresponding to the serving cell and to the DL BWP to which the resource belongs to (see TS 38.214 [19], clause 5.2.2.3.1).

resourceMapping OFDM symbol location(s) in a slot and subcarrier occupancy in a PRB of the CSI-RS resource.

scramblingID Scrambling ID (see TS 38.214 [19], clause 5.2.2.3.1).

| Conditional Presence | Explanation |
| --- | --- |
| Periodic | The field is optionally present, Need M, for periodic NZP-CSI-RS-Resources (as indicated in CSI-ResourceConfig). The field is absent otherwise. |
| PeriodicOrSemiPersistent | The field is optionally present, Need M, for periodic and semi-persistent NZP-CSI-RS-Resources (as indicated in CSI-ResourceConfig). The field is absent otherwise. |

#### –	NZP-CSI-RS-ResourceId

The IE NZP-CSI-RS-ResourceId is used to identify one NZP-CSI-RS-Resource.

NZP-CSI-RS-ResourceId information element

-- ASN1START

-- TAG-NZP-CSI-RS-RESOURCEID-START

NZP-CSI-RS-ResourceId ::=           INTEGER (0..maxNrofNZP-CSI-RS-Resources-1)

-- TAG-NZP-CSI-RS-RESOURCEID-STOP

-- ASN1STOP

#### –	NZP-CSI-RS-ResourceSet

The IE NZP-CSI-RS-ResourceSet is a set of Non-Zero-Power (NZP) CSI-RS resources (their IDs) and set-specific parameters.

NZP-CSI-RS-ResourceSet information element

-- ASN1START

-- TAG-NZP-CSI-RS-RESOURCESET-START

NZP-CSI-RS-ResourceSet ::=          SEQUENCE {

nzp-CSI-ResourceSetId               NZP-CSI-RS-ResourceSetId,

nzp-CSI-RS-Resources                SEQUENCE (SIZE (1..maxNrofNZP-CSI-RS-ResourcesPerSet)) OF NZP-CSI-RS-ResourceId,

repetition                          ENUMERATED { on, off }                                                          OPTIONAL,   -- Need S

aperiodicTriggeringOffset           INTEGER(0..6)                                                                   OPTIONAL,   -- Need S

trs-Info                            ENUMERATED {true}                                                               OPTIONAL,   -- Need R

...

}

-- TAG-NZP-CSI-RS-RESOURCESET-STOP

-- ASN1STOP

NZP-CSI-RS-ResourceSet field descriptions

aperiodicTriggeringOffset Offset X between the slot containing the DCI that triggers a set of aperiodic NZP CSI-RS resources and the slot in which the CSI-RS resource set is transmitted. The value 0 corresponds to 0 slots, value 1 corresponds to 1 slot, value 2 corresponds to 2 slots, value 3 corresponds to 3 slots, value 4 corresponds to 4 slots, value 5 corresponds to 16 slots, value 6 corresponds to 24 slots. When the field is absent the UE applies the value 0.

nzp-CSI-RS-Resources NZP-CSI-RS-Resources associated with this NZP-CSI-RS resource set (see TS 38.214 [19], clause 5.2). For CSI, there are at most 8 NZP CSI RS resources per resource set.

repetition Indicates whether repetition is on/off. If the field is set to off or if the field is absent, the UE may not assume that the NZP-CSI-RS resources within the resource set are transmitted with the same downlink spatial domain transmission filter (see TS 38.214 [19], clauses 5.2.2.3.1 and 5.1.6.1.2). Can only be configured for CSI-RS resource sets which are associated with CSI-ReportConfig with report of L1 RSRP or "no report".

trs-Info Indicates that the antenna port for all NZP-CSI-RS resources in the CSI-RS resource set is same. If the field is absent or released the UE applies the value false (see TS 38.214 [19], clause 5.2.2.3.1).

#### –	NZP-CSI-RS-ResourceSetId

The IE NZP-CSI-RS-ResourceSetId is used to identify one NZP-CSI-RS-ResourceSet.

NZP-CSI-RS-ResourceSetId information element

-- ASN1START

-- TAG-NZP-CSI-RS-RESOURCESETID-START

NZP-CSI-RS-ResourceSetId ::=        INTEGER (0..maxNrofNZP-CSI-RS-ResourceSets-1)

-- TAG-NZP-CSI-RS-RESOURCESETID-STOP

-- ASN1STOP

#### –	P-Max

The IE P-Max is used to limit the UE's uplink transmission power on a carrier frequency, in TS 38.101-1 [15] and is used to calculate the parameter Pcompensation defined in TS 38.304 [20].

P-Max information element

-- ASN1START

-- TAG-P-MAX-START

P-Max ::=                           INTEGER (-30..33)

-- TAG-P-MAX-STOP

-- ASN1STOP

#### –	PCI-List

The IE PCI-List concerns a list of physical cell identities, which may be used for different purposes.

PCI-List information element

-- ASN1START

-- TAG-PCI-LIST-START

PCI-List ::=                        SEQUENCE (SIZE (1..maxNrofCellMeas)) OF PhysCellId

-- TAG-PCI-LIST-STOP

-- ASN1STOP

#### –	PCI-Range

The IE PCI-Range is used to encode either a single or a range of physical cell identities. The range is encoded by using a start value and by indicating the number of consecutive physical cell identities (including start) in the range. For fields comprising multiple occurrences of PCI-Range, the Network may configure overlapping ranges of physical cell identities.

PCI-Range information element

-- ASN1START

-- TAG-PCI-RANGE-START

PCI-Range ::=                       SEQUENCE {

start                               PhysCellId,

range                               ENUMERATED {n4, n8, n12, n16, n24, n32, n48, n64, n84,

n96, n128, n168, n252, n504, n1008,spare1}                  OPTIONAL    -- Need S

}

-- TAG-PCI-RANGE-STOP

-- ASN1STOP

PCI-Range field descriptions

range Indicates the number of physical cell identities in the range (including start). Value n4 corresponds with 4, value n8 corresponds with 8 and so on. The UE shall apply value 1 in case the field is absent, in which case only the physical cell identity value indicated by start applies.

start Indicates the lowest physical cell identity in the range.

#### –	PCI-RangeElement

The IE PCI-RangeElement is used to define a PCI-Range as part of a list (e.g. AddMod list).

PCI-RangeElement information element

-- ASN1START

-- TAG-PCI-RANGEELEMENT-START

PCI-RangeElement ::=                SEQUENCE {

pci-RangeIndex                      PCI-RangeIndex,

pci-Range                           PCI-Range

}

-- TAG-PCI-RANGEELEMENT-STOP

-- ASN1STOP

PCI-RangeElement field descriptions

pci-Range Physical cell identity or a range of physical cell identities.

#### –	PCI-RangeIndex

The IE PCI-RangeIndex identifies a physical cell id range, which may be used for different purposes.

PCI-RangeIndex information element

-- ASN1START

-- TAG-PCI-RANGEINDEX-START

PCI-RangeIndex ::=                  INTEGER (1..maxNrofPCI-Ranges)

-- TAG-PCI-RANGEINDEX-STOP

-- ASN1STOP

#### –	PCI-RangeIndexList

The IE PCI-RangeIndexList concerns a list of indexes of physical cell id ranges, which may be used for different purposes.

PCI-RangeIndexList information element

-- ASN1START

-- TAG-PCI-RANGEINDEXLIST-START

PCI-RangeIndexList ::=              SEQUENCE (SIZE (1..maxNrofPCI-Ranges)) OF PCI-RangeIndex

-- TAG-PCI-RANGEINDEXLIST-STOP

-- ASN1STOP

#### –	PDCCH-Config

The IE PDCCH-Config is used to configure UE specific PDCCH parameters such as control resource sets (CORESET), search spaces and additional parameters for acquiring the PDCCH. If this IE is used for the scheduled cell in case of cross carrier scheduling, the fields other than searchSpacesToAddModList and searchSpacesToReleaseList are absent.

PDCCH-Config information element

-- ASN1START

-- TAG-PDCCH-CONFIG-START

PDCCH-Config ::=                    SEQUENCE {

controlResourceSetToAddModList      SEQUENCE(SIZE (1..3)) OF ControlResourceSet                 OPTIONAL,   -- Need N

controlResourceSetToReleaseList     SEQUENCE(SIZE (1..3)) OF ControlResourceSetId               OPTIONAL,   -- Need N

searchSpacesToAddModList            SEQUENCE(SIZE (1..10)) OF SearchSpace                       OPTIONAL,   -- Need N

searchSpacesToReleaseList           SEQUENCE(SIZE (1..10)) OF SearchSpaceId                     OPTIONAL,   -- Need N

downlinkPreemption                  SetupRelease { DownlinkPreemption }                         OPTIONAL,   -- Need M

tpc-PUSCH                           SetupRelease { PUSCH-TPC-CommandConfig }                    OPTIONAL,   -- Need M

tpc-PUCCH                           SetupRelease { PUCCH-TPC-CommandConfig }                    OPTIONAL,   -- Need M

tpc-SRS                             SetupRelease { SRS-TPC-CommandConfig}                       OPTIONAL,   -- Need M

...

}

-- TAG-PDCCH-CONFIG-STOP

-- ASN1STOP

PDCCH-Config field descriptions

controlResourceSetToAddModList List of UE specifically configured Control Resource Sets (CORESETs) to be used by the UE. The network configures at most 3 CORESETs per BWP per cell (including UE-specific and common CORESETs). In case network reconfigures control resource set with the same ControlResourceSetId as used for commonControlResourceSet configured via PDCCH-ConfigCommon, the configuration from PDCCH-Config always takes precedence and should not be updated by the UE based on servingCellConfigCommon.

controlResourceSetToReleaseList List of UE specifically configured Control Resource Sets (CORESETs) to be released by the UE. This field only applies to CORESETs configured by controlResourceSetToAddModList and does not release the field commonControlResourceSet configured by PDCCH-ConfigCommon.

downlinkPreemption Configuration of downlink preemption indications to be monitored in this cell (see TS 38.213 [13], clause 11.2).

searchSpacesToAddModList List of UE specifically configured Search Spaces. The network configures at most 10 Search Spaces per BWP per cell (including UE-specific and common Search Spaces).

tpc-PUCCH Enable and configure reception of group TPC commands for PUCCH.

tpc-PUSCH Enable and configure reception of group TPC commands for PUSCH.

tpc-SRS Enable and configure reception of group TPC commands for SRS.

#### –	PDCCH-ConfigCommon

The IE PDCCH-ConfigCommon is used to configure cell specific PDCCH parameters provided in SIB as well as in dedicated signalling.

PDCCH-ConfigCommon information element

-- ASN1START

-- TAG-PDCCH-CONFIGCOMMON-START

PDCCH-ConfigCommon ::=              SEQUENCE {

controlResourceSetZero              ControlResourceSetZero                                  OPTIONAL,   -- Cond InitialBWP-Only

commonControlResourceSet            ControlResourceSet                                      OPTIONAL,   -- Need R

searchSpaceZero                     SearchSpaceZero                                         OPTIONAL,   -- Cond InitialBWP-Only

commonSearchSpaceList               SEQUENCE (SIZE(1..4)) OF SearchSpace                    OPTIONAL,   -- Need R

searchSpaceSIB1                     SearchSpaceId                                           OPTIONAL,   -- Need S

searchSpaceOtherSystemInformation   SearchSpaceId                                           OPTIONAL,   -- Need S

pagingSearchSpace                   SearchSpaceId                                           OPTIONAL,   -- Need S

ra-SearchSpace                      SearchSpaceId                                           OPTIONAL,   -- Need S

...,

[[

firstPDCCH-MonitoringOccasionOfPO   CHOICE {

sCS15KHZoneT                                                             SEQUENCE (SIZE (1..maxPO-perPF)) OF INTEGER (0..139),

sCS30KHZoneT-SCS15KHZhalfT                                               SEQUENCE (SIZE (1..maxPO-perPF)) OF INTEGER (0..279),

sCS60KHZoneT-SCS30KHZhalfT-SCS15KHZquarterT                              SEQUENCE (SIZE (1..maxPO-perPF)) OF INTEGER (0..559),

sCS120KHZoneT-SCS60KHZhalfT-SCS30KHZquarterT-SCS15KHZoneEighthT          SEQUENCE (SIZE (1..maxPO-perPF)) OF INTEGER (0..1119),

sCS120KHZhalfT-SCS60KHZquarterT-SCS30KHZoneEighthT-SCS15KHZoneSixteenthT SEQUENCE (SIZE (1..maxPO-perPF)) OF INTEGER (0..2239),

sCS120KHZquarterT-SCS60KHZoneEighthT-SCS30KHZoneSixteenthT               SEQUENCE (SIZE (1..maxPO-perPF)) OF INTEGER (0..4479),

sCS120KHZoneEighthT-SCS60KHZoneSixteenthT                                SEQUENCE (SIZE (1..maxPO-perPF)) OF INTEGER (0..8959),

sCS120KHZoneSixteenthT                                                   SEQUENCE (SIZE (1..maxPO-perPF)) OF INTEGER (0..17919)

}                                                                                                                OPTIONAL       -- Cond OtherBWP

]]

}

-- TAG-PDCCH-CONFIGCOMMON-STOP

-- ASN1STOP

PDCCH-ConfigCommon field descriptions

commonControlResourceSet An additional common control resource set which may be configured and used for any common or UE-specific search space. If the network configures this field, it uses a ControlResourceSetId other than 0 for this ControlResourceSet. The network configures the commonControlResourceSet in SIB1 so that it is contained in the bandwidth of CORESET#0.

commonSearchSpaceList A list of additional common search spaces. If the network configures this field, it uses the SearchSpaceIds other than 0. If the field is included, it replaces any previous list, i.e. all the entries of the list are replaced and each of the SearchSpace entries is considered to be newly created and the conditions and Need codes for setup of the entry apply.

controlResourceSetZero Parameters of the common CORESET#0 which can be used in any common or UE-specific search spaces. The values are interpreted like the corresponding bits in MIB pdcch-ConfigSIB1. Even though this field is only configured in the initial BWP (BWP#0) controlResourceSetZero can be used in search spaces configured in other DL BWP(s) than the initial DL BWP if the conditions defined in TS 38.213 [13], clause 10 are satisfied.

firstPDCCH-MonitoringOccasionOfPO Indicates the first PDCCH monitoring occasion of each PO of the PF on this BWP, see TS 38.304 [20].

pagingSearchSpace ID of the Search space for paging (see TS 38.213 [13], clause 10.1). If the field is absent, the UE does not receive paging in this BWP (see TS 38.213 [13], clause 10).

ra-SearchSpace ID of the Search space for random access procedure (see TS 38.213 [13], clause 10.1). If the field is absent, the UE does not receive RAR in this BWP. This field is mandatory present in the DL BWP(s) if the conditions described in TS 38.321 [3], subclause 5.15 are met.

searchSpaceOtherSystemInformation ID of the Search space for other system information, i.e., SIB2 and beyond (see TS 38.213 [13], clause 10.1) If the field is absent, the UE does not receive other system information in this BWP.

searchSpaceSIB1 ID of the search space for SIB1 message. In the initial DL BWP of the UE′s PCell, the network sets this field to 0. If the field is absent, the UE does not receive SIB1 in this BWP. (see TS 38.213 [13], clause 10)

searchSpaceZero Parameters of the common SearchSpace#0. The values are interpreted like the corresponding bits in MIB pdcch-ConfigSIB1. Even though this field is only configured in the initial BWP (BWP#0), searchSpaceZero can be used in search spaces configured in other DL BWP(s) than the initial DL BWP if the conditions described in TS 38.213 [13], clause 10, are satisfied.

| Conditional Presence | Explanation |
| --- | --- |
| InitialBWP-Only | If SIB1 is broadcast the field is mandatory present in the PDCCH-ConfigCommon of the initial BWP (BWP#0) in ServingCellConfigCommon; it is absent in other BWPs and when sent in system information. If SIB1 is not broadcast and there is an SSB associated to the cell, the field is optionally present, Need M, in the PDCCH-ConfigCommon of the initial BWP (BWP#0) in ServingCellConfigCommon (still with the same setting for all UEs). In other cases, the field is absent. |
| OtherBWP | This field is optionally present, Need R, if this BWP is not the initial DL BWP and pagingSearchSpace is configured in this BWP. Otherwise this field is absent. |

#### –	PDCCH-ConfigSIB1

The IE PDCCH-ConfigSIB1 is used to configure CORESET#0 and search space#0.

PDCCH-ConfigSIB1 information element

-- ASN1START

-- TAG-PDCCH-CONFIGSIB1-START

PDCCH-ConfigSIB1 ::=                SEQUENCE {

controlResourceSetZero              ControlResourceSetZero,

searchSpaceZero                     SearchSpaceZero

}

-- TAG-PDCCH-CONFIGSIB1-STOP

-- ASN1STOP

PDCCH-ConfigSIB1 field descriptions

controlResourceSetZero Determines a common ControlResourceSet (CORESET) with ID #0, see TS 38.213 [13], clause 13.

searchSpaceZero Determines a common search space with ID #0, see TS 38.213 [13], clause 13.

#### –	PDCCH-ServingCellConfig

The IE PDCCH-ServingCellConfig is used to configure UE specific PDCCH parameters applicable across all bandwidth parts of a serving cell.

PDCCH-ServingCellConfig information element

-- ASN1START

-- TAG-PDCCH-SERVINGCELLCONFIG-START

PDCCH-ServingCellConfig ::=         SEQUENCE {

slotFormatIndicator                 SetupRelease { SlotFormatIndicator }                                OPTIONAL,   -- Need M

...

}

-- TAG-PDCCH-SERVINGCELLCONFIG-STOP

-- ASN1STOP

PDCCH-ServingCellConfig field descriptions

slotFormatIndicator Configuration of Slot-Format-Indicators to be monitored in the correspondingly configured PDCCHs of this serving cell.

#### –	PDCP-Config

The IE PDCP-Config is used to set the configurable PDCP parameters for signalling and data radio bearers.

PDCP-Config information element

-- ASN1START

-- TAG-PDCP-CONFIG-START

PDCP-Config ::=         SEQUENCE {

drb                     SEQUENCE {

discardTimer            ENUMERATED {ms10, ms20, ms30, ms40, ms50, ms60, ms75, ms100, ms150, ms200,

ms250, ms300, ms500, ms750, ms1500, infinity}       OPTIONAL, -- Cond Setup

pdcp-SN-SizeUL          ENUMERATED {len12bits, len18bits}                               OPTIONAL, -- Cond Setup2

pdcp-SN-SizeDL          ENUMERATED {len12bits, len18bits}                               OPTIONAL, -- Cond Setup2

headerCompression       CHOICE {

notUsed                 NULL,

rohc                    SEQUENCE {

maxCID                  INTEGER (1..16383)                                      DEFAULT 15,

profiles                SEQUENCE {

profile0x0001           BOOLEAN,

profile0x0002           BOOLEAN,

profile0x0003           BOOLEAN,

profile0x0004           BOOLEAN,

profile0x0006           BOOLEAN,

profile0x0101           BOOLEAN,

profile0x0102           BOOLEAN,

profile0x0103           BOOLEAN,

profile0x0104           BOOLEAN

},

drb-ContinueROHC            ENUMERATED { true }                                 OPTIONAL    -- Need N

},

uplinkOnlyROHC          SEQUENCE {

maxCID                  INTEGER (1..16383)                                      DEFAULT 15,

profiles                SEQUENCE {

profile0x0006           BOOLEAN

},

drb-ContinueROHC            ENUMERATED { true }                                 OPTIONAL    -- Need N

},

...

},

integrityProtection     ENUMERATED { enabled }                                          OPTIONAL,   -- Cond ConnectedTo5GC1

statusReportRequired    ENUMERATED { true }                                             OPTIONAL,   -- Cond Rlc-AM

outOfOrderDelivery      ENUMERATED { true }                                             OPTIONAL    -- Need R

}                                                                                           OPTIONAL,   -- Cond DRB

moreThanOneRLC          SEQUENCE {

primaryPath             SEQUENCE {

cellGroup               CellGroupId                                                 OPTIONAL,   -- Need R

logicalChannel          LogicalChannelIdentity                                      OPTIONAL    -- Need R

},

ul-DataSplitThreshold   UL-DataSplitThreshold                                           OPTIONAL, -- Cond SplitBearer

pdcp-Duplication            BOOLEAN                                                     OPTIONAL    -- Need R

}                                                                                           OPTIONAL, -- Cond MoreThanOneRLC

t-Reordering                ENUMERATED {

ms0, ms1, ms2, ms4, ms5, ms8, ms10, ms15, ms20, ms30, ms40,

ms50, ms60, ms80, ms100, ms120, ms140, ms160, ms180, ms200, ms220,

ms240, ms260, ms280, ms300, ms500, ms750, ms1000, ms1250,

ms1500, ms1750, ms2000, ms2250, ms2500, ms2750,

ms3000, spare28, spare27, spare26, spare25, spare24,

spare23, spare22, spare21, spare20,

spare19, spare18, spare17, spare16, spare15, spare14,

spare13, spare12, spare11, spare10, spare09,

spare08, spare07, spare06, spare05, spare04, spare03,

spare02, spare01 }                                          OPTIONAL, -- Need S

...,

[[

cipheringDisabled       ENUMERATED {true}                                                   OPTIONAL    -- Cond ConnectedTo5GC

]]

}

UL-DataSplitThreshold ::= ENUMERATED {

b0, b100, b200, b400, b800, b1600, b3200, b6400, b12800, b25600, b51200, b102400, b204800,

b409600, b819200, b1228800, b1638400, b2457600, b3276800, b4096000, b4915200, b5734400,

b6553600, infinity, spare8, spare7, spare6, spare5, spare4, spare3, spare2, spare1}

-- TAG-PDCP-CONFIG-STOP

-- ASN1STOP

PDCP-Config field descriptions

cipheringDisabled If included, ciphering is disabled for this DRB regardless of which ciphering algorithm is configured for the SRB/DRBs. The field may only be included if the UE is connected to 5GC. Otherwise the field is absent. The network configures all DRBs with the same PDU-session ID with same value for this field. The value for this field cannot be changed after the DRB is set up.

discardTimer Value in ms of discardTimer specified in TS 38.323 [5]. Value ms10 corresponds to 10 ms, value ms20 corresponds to 20 ms and so on.

drb-ContinueROHC Indicates whether the PDCP entity continues or resets the ROHC header compression protocol during PDCP re-establishment, as specified in TS 38.323 [5]. This field is configured only in case of resuming an RRC connection or reconfiguration with sync, where the PDCP termination point is not changed and the fullConfig is not indicated.

headerCompression If rohc is configured, the UE shall apply the configured ROHC profile(s) in both uplink and downlink. If uplinkOnlyROHC is configured, the UE shall apply the configured ROHC profile(s) in uplink (there is no header compression in downlink). ROHC can be configured for any bearer type. The network reconfigures headerCompression only upon reconfiguration involving PDCP re-establishment. Network configures headerCompression to notUsed when outOfOrderDelivery is configured.

integrityProtection Indicates whether or not integrity protection is configured for this radio bearer. The network configures all DRBs with the same PDU-session ID with same value for this field. The value for this field cannot be changed after the DRB is set up.

maxCID Indicates the value of the MAX_CID parameter as specified in TS 38.323 [5]. The total value of MAX_CIDs across all bearers for the UE should be less than or equal to the value of maxNumberROHC-ContextSessions parameter as indicated by the UE.

moreThanOneRLC This field configures UL data transmission when more than one RLC entity is associated with the PDCP entity.

outOfOrderDelivery Indicates whether or not outOfOrderDelivery specified in TS 38.323 [5] is configured. This field should be either always present or always absent, after the radio bearer is established.

pdcp-Duplication Indicates whether or not uplink duplication status at the time of receiving this IE is configured and activated as specified in TS 38.323 [5]. The presence of this field indicates that duplication is configured. PDCP duplication is not configured for CA packet duplication of LTE RLC bearer. The value of this field, when the field is present, indicates the state of the duplication at the time of receiving this IE. If set to true, duplication is activated. The value of this field is always true, when configured for a SRB.

pdcp-SN-SizeDL PDCP sequence number size for downlink, 12 or 18 bits, as specified in TS 38.323 [5]. For SRBs only the value len12bits is applicable.

pdcp-SN-SizeUL PDCP sequence number size for uplink, 12 or 18 bits, as specified in TS 38.323 [5]. For SRBs only the value len12bits is applicable.

primaryPath Indicates the cell group ID and LCID of the primary RLC entity as specified in TS 38.323 [5], clause 5.2.1 for UL data transmission when more than one RLC entity is associated with the PDCP entity. In this version of the specification, only cell group ID corresponding to MCG is supported for SRBs. The NW indicates cellGroup for split bearers using logical channels in different cell groups. The NW indicates logicalChannel for CA based PDCP duplication, i.e., if both logical channels terminate in the same cell group.

statusReportRequired For AM DRBs, indicates whether the DRB is configured to send a PDCP status report in the uplink, as specified in TS 38.323 [5].

t-Reordering Value in ms of t-Reordering specified in TS 38.323 [5]. Value ms0 corresponds to 0 ms, value ms20 corresponds to 20 ms, value ms40 corresponds to 40 ms, and so on. When the field is absent the UE applies the value infinity.

ul-DataSplitThreshold Parameter specified in TS 38.323 [5]. Value b0 corresponds to 0 bytes, value b100 corresponds to 100 bytes, value b200 corresponds to 200 bytes, and so on. The network sets this field to infinity for UEs not supporting splitDRB-withUL-Both-MCG-SCG. If the field is absent when the split bearer is configured for the radio bearer first time, then the default value infinity is applied.

| Conditional presence | Explanation |
| --- | --- |
| DRB | This field is mandatory present when the corresponding DRB is being set up, absent for SRBs. Otherwise this field is optionally present, need M. |
| MoreThanOneRLC | This field is mandatory present upon RRC reconfiguration with setup of a PDCP entity for a radio bearer with more than one associated logical channel and upon RRC reconfiguration with the association of an additional logical channel to the PDCP entity. Upon RRC reconfiguration when a PDCP entity is associated with multiple logical channels, this field is optionally present need M. Otherwise, this field is absent. Need R. |
| Rlc-AM | For RLC AM, the field is optionally present, need R. Otherwise, the field is absent. |
| Setup | The field is mandatory present in case of radio bearer setup. Otherwise the field is optionally present, need M. |
| SplitBearer | The field is absent for SRBs. Otherwise, the field is optional present, need M, in case of radio bearer with more than one associated RLC mapped to different cell groups. |
| ConnectedTo5GC | The field is optionally present, need R, if the UE is connected to 5GC. Otherwise the field is absent. |
| ConnectedTo5GC1 | The field is optionally present, need R, if the UE is connected to NR/5GC. Otherwise the field is absent. |
| Setup2 | This field is mandatory present in case for radio bearer setup for RLC-AM and RLC-UM. Otherwise, this field is absent, Need M. |

#### –	PDSCH-Config

The PDSCH-Config IE is used to configure the UE specific PDSCH parameters.

PDSCH-Config information element

-- ASN1START

-- TAG-PDSCH-CONFIG-START

PDSCH-Config ::=                        SEQUENCE {

dataScramblingIdentityPDSCH             INTEGER (0..1023)                                                   OPTIONAL,   -- Need S

dmrs-DownlinkForPDSCH-MappingTypeA      SetupRelease { DMRS-DownlinkConfig }                                OPTIONAL,   -- Need M

dmrs-DownlinkForPDSCH-MappingTypeB      SetupRelease { DMRS-DownlinkConfig }                                OPTIONAL,   -- Need M

tci-StatesToAddModList                  SEQUENCE (SIZE(1..maxNrofTCI-States)) OF TCI-State                  OPTIONAL,   -- Need N

tci-StatesToReleaseList                 SEQUENCE (SIZE(1..maxNrofTCI-States)) OF TCI-StateId                OPTIONAL,   -- Need N

vrb-ToPRB-Interleaver                   ENUMERATED {n2, n4}                                                 OPTIONAL,   -- Need S

resourceAllocation                      ENUMERATED { resourceAllocationType0, resourceAllocationType1, dynamicSwitch},

pdsch-TimeDomainAllocationList          SetupRelease { PDSCH-TimeDomainResourceAllocationList }             OPTIONAL,   -- Need M

pdsch-AggregationFactor                 ENUMERATED { n2, n4, n8 }                                           OPTIONAL,   -- Need S

rateMatchPatternToAddModList            SEQUENCE (SIZE (1..maxNrofRateMatchPatterns)) OF RateMatchPattern   OPTIONAL,   -- Need N

rateMatchPatternToReleaseList           SEQUENCE (SIZE (1..maxNrofRateMatchPatterns)) OF RateMatchPatternId OPTIONAL,   -- Need N

rateMatchPatternGroup1                  RateMatchPatternGroup                                               OPTIONAL,   -- Need R

rateMatchPatternGroup2                  RateMatchPatternGroup                                               OPTIONAL,   -- Need R

rbg-Size                                ENUMERATED {config1, config2},

mcs-Table                               ENUMERATED {qam256, qam64LowSE}                                     OPTIONAL,   -- Need S

maxNrofCodeWordsScheduledByDCI          ENUMERATED {n1, n2}                                                 OPTIONAL,   -- Need R

prb-BundlingType                        CHOICE {

staticBundling                          SEQUENCE {

bundleSize                              ENUMERATED { n4, wideband }                                 OPTIONAL    -- Need S

},

dynamicBundling                     SEQUENCE {

bundleSizeSet1                      ENUMERATED { n4, wideband, n2-wideband, n4-wideband }           OPTIONAL,   -- Need S

bundleSizeSet2                      ENUMERATED { n4, wideband }                                     OPTIONAL    -- Need S

}

},

zp-CSI-RS-ResourceToAddModList                  SEQUENCE (SIZE (1..maxNrofZP-CSI-RS-Resources)) OF ZP-CSI-RS-Resource

OPTIONAL,   -- Need N

zp-CSI-RS-ResourceToReleaseList                 SEQUENCE (SIZE (1..maxNrofZP-CSI-RS-Resources)) OF ZP-CSI-RS-ResourceId

OPTIONAL,   -- Need N

aperiodic-ZP-CSI-RS-ResourceSetsToAddModList    SEQUENCE (SIZE (1..maxNrofZP-CSI-RS-ResourceSets)) OF ZP-CSI-RS-ResourceSet

OPTIONAL,   -- Need N

aperiodic-ZP-CSI-RS-ResourceSetsToReleaseList SEQUENCE (SIZE (1..maxNrofZP-CSI-RS-ResourceSets)) OF ZP-CSI-RS-ResourceSetId

OPTIONAL,   -- Need N

sp-ZP-CSI-RS-ResourceSetsToAddModList   SEQUENCE (SIZE (1..maxNrofZP-CSI-RS-ResourceSets)) OF ZP-CSI-RS-ResourceSet

OPTIONAL,   -- Need N

sp-ZP-CSI-RS-ResourceSetsToReleaseList  SEQUENCE (SIZE (1..maxNrofZP-CSI-RS-ResourceSets)) OF ZP-CSI-RS-ResourceSetId

OPTIONAL,   -- Need N

p-ZP-CSI-RS-ResourceSet                 SetupRelease { ZP-CSI-RS-ResourceSet }

OPTIONAL,   -- Need M

...

}

RateMatchPatternGroup ::=               SEQUENCE (SIZE (1..maxNrofRateMatchPatternsPerGroup)) OF CHOICE {

cellLevel                               RateMatchPatternId,

bwpLevel                                RateMatchPatternId

}

-- TAG-PDSCH-CONFIG-STOP

-- ASN1STOP

PDSCH-Config field descriptions

aperiodic-ZP-CSI-RS-ResourceSetsToAddModList AddMod/Release lists for configuring aperiodically triggered zero-power CSI-RS resource sets. Each set contains a ZP-CSI-RS-ResourceSetId and the IDs of one or more ZP-CSI-RS-Resources (the actual resources are defined in the zp-CSI-RS-ResourceToAddModList). The network configures the UE with at most 3 aperiodic ZP-CSI-RS-ResourceSets and it uses only the ZP-CSI-RS-ResourceSetId 1 to 3. The network triggers a set by indicating its ZP-CSI-RS-ResourceSetId in the DCI payload. The DCI codepoint '01' triggers the resource set with ZP-CSI-RS-ResourceSetId 1, the DCI codepoint '10' triggers the resource set with ZP-CSI-RS-ResourceSetId 2, and the DCI codepoint '11' triggers the resource set with ZP-CSI-RS-ResourceSetId 3 (see TS 38.214 [19], clause 5.1.4.2)

dataScramblingIdentityPDSCH Identifier used to initialize data scrambling (c_init) for PDSCH. If the field is absent, the UE applies the physical cell ID. (see TS 38.211 [16], clause 7.3.1.1).

dmrs-DownlinkForPDSCH-MappingTypeA DMRS configuration for PDSCH transmissions using PDSCH mapping type A (chosen dynamically via PDSCH-TimeDomainResourceAllocation). Only the fields dmrs-Type, dmrs-AdditionalPosition and maxLength may be set differently for mapping type A and B.

dmrs-DownlinkForPDSCH-MappingTypeB DMRS configuration for PDSCH transmissions using PDSCH mapping type B (chosen dynamically via PDSCH-TimeDomainResourceAllocation). Only the fields dmrs-Type, dmrs-AdditionalPosition and maxLength may be set differently for mapping type A and B.

maxNrofCodeWordsScheduledByDCI Maximum number of code words that a single DCI may schedule. This changes the number of MCS/RV/NDI bits in the DCI message from 1 to 2.

mcs-Table Indicates which MCS table the UE shall use for PDSCH. (see TS 38.214 [19], clause 5.1.3.1). If the field is absent the UE applies the value 64QAM.

pdsch-AggregationFactor Number of repetitions for data (see TS 38.214 [19], clause 5.1.2.1). When the field is absent the UE applies the value 1.

pdsch-TimeDomainAllocationList List of time-domain configurations for timing of DL assignment to DL data (see table 5.1.2.1.1-1 in TS 38.214 [19]).

prb-BundlingType Indicates the PRB bundle type and bundle size(s) (see TS 38.214 [19], clause 5.1.2.3). If dynamic is chosen, the actual bundleSizeSet1 or bundleSizeSet2 to use is indicated via DCI. Constraints on bundleSize(Set) setting depending on vrb-ToPRB-Interleaver and rbg-Size settings are described in TS 38.214 [19], clause 5.1.2.3. If a bundleSize(Set) value is absent, the UE applies the value n2.

p-ZP-CSI-RS-ResourceSet A set of periodically occurring ZP-CSI-RS-Resources (the actual resources are defined in the zp-CSI-RS-ResourceToAddModList). The network uses the ZP-CSI-RS-ResourceSetId=0 for this set.

rateMatchPatternGroup1 The IDs of a first group of RateMatchPatterns defined in PDSCH-Config->rateMatchPatternToAddModList (BWP level) or in ServingCellConfig ->rateMatchPatternToAddModList (cell level). These patterns can be activated dynamically by DCI (see TS 38.214 [19], clause 5.1.4.1).

rateMatchPatternGroup2 The IDs of a second group of RateMatchPatterns defined in PDSCH-Config->rateMatchPatternToAddModList (BWP level) or in ServingCellConfig ->rateMatchPatternToAddModList (cell level). These patterns can be activated dynamically by DCI (see TS 38.214 [19], clause 5.1.4.1).

rateMatchPatternToAddModList Resources patterns which the UE should rate match PDSCH around. The UE rate matches around the union of all resources indicated in the rate match patterns (see TS 38.214 [19], clause 5.1.4.1).

rbg-Size Selection between config 1 and config 2 for RBG size for PDSCH. The UE ignores this field if resourceAllocation is set to resourceAllocationType1 (see TS 38.214 [19], clause 5.1.2.2.1).

resourceAllocation Configuration of resource allocation type 0 and resource allocation type 1 for non-fallback DCI (see TS 38.214 [19], clause 5.1.2.2).

sp-ZP-CSI-RS-ResourceSetsToAddModList AddMod/Release lists for configuring semi-persistent zero-power CSI-RS resource sets. Each set contains a ZP-CSI-RS-ResourceSetId and the IDs of one or more ZP-CSI-RS-Resources (the actual resources are defined in the zp-CSI-RS-ResourceToAddModList) (see TS 38.214 [19], clause 5.1.4.2).

tci-StatesToAddModList A list of Transmission Configuration Indicator (TCI) states indicating a transmission configuration which includes QCL-relationships between the DL RSs in one RS set and the PDSCH DMRS ports (see TS 38.214 [19], clause 5.1.5).

vrb-ToPRB-Interleaver Interleaving unit configurable between 2 and 4 PRBs (see TS 38.211 [16], clause 7.3.1.6). When the field is absent, the UE performs non-interleaved VRB-to-PRB mapping.

zp-CSI-RS-ResourceToAddModList A list of Zero-Power (ZP) CSI-RS resources used for PDSCH rate-matching. Each resource in this list may be referred to from only one type of resource set, i.e., aperiodic, semi-persistent or periodic (see TS 38.214 [19]).

#### –	PDSCH-ConfigCommon

The IE PDSCH-ConfigCommon is used to configure cell specific PDSCH parameters.

PDSCH-ConfigCommon information element

-- ASN1START

-- TAG-PDSCH-CONFIGCOMMON-START

PDSCH-ConfigCommon ::=                  SEQUENCE {

pdsch-TimeDomainAllocationList                  PDSCH-TimeDomainResourceAllocationList          OPTIONAL,   -- Need R

...

}

-- TAG-PDSCH-CONFIGCOMMON-STOP

-- ASN1STOP

PDSCH-ConfigCommon field descriptions

pdsch-TimeDomainAllocationList List of time-domain configurations for timing of DL assignment to DL data (see table 5.1.2.1.1-1 in TS 38.214 [19]).

#### –	PDSCH-ServingCellConfig

The IE PDSCH-ServingCellConfig is used to configure UE specific PDSCH parameters that are common across the UE's BWPs of one serving cell.

PDSCH-ServingCellConfig information element

-- ASN1START

-- TAG-PDSCH-SERVINGCELLCONFIG-START

PDSCH-ServingCellConfig ::=             SEQUENCE {

codeBlockGroupTransmission              SetupRelease { PDSCH-CodeBlockGroupTransmission }       OPTIONAL,   -- Need M

xOverhead                               ENUMERATED { xOh6, xOh12, xOh18 }                       OPTIONAL,   -- Need S

nrofHARQ-ProcessesForPDSCH              ENUMERATED {n2, n4, n6, n10, n12, n16}                  OPTIONAL,   -- Need S

pucch-Cell                              ServCellIndex                                           OPTIONAL,   -- Cond SCellAddOnly

...,

[[

maxMIMO-Layers                          INTEGER (1..8)                                          OPTIONAL,  -- Need M

processingType2Enabled                  BOOLEAN                                                 OPTIONAL   -- Need M

]]

}

PDSCH-CodeBlockGroupTransmission ::=    SEQUENCE {

maxCodeBlockGroupsPerTransportBlock     ENUMERATED {n2, n4, n6, n8},

codeBlockGroupFlushIndicator            BOOLEAN,

...

}

-- TAG-PDSCH-SERVINGCELLCONFIG-STOP

-- ASN1STOP

PDSCH-CodeBlockGroupTransmission field descriptions

codeBlockGroupFlushIndicator Indicates whether CBGFI for CBG based (re)transmission in DL is enabled (true). (see TS 38.212 [17], clause 7.3.1.2.2).

maxCodeBlockGroupsPerTransportBlock Maximum number of code-block-groups (CBGs) per TB. In case of multiple CW, the maximum CBG is 4 (see TS 38.213 [13], clause 9.1.1).

PDSCH-ServingCellConfig field descriptions

codeBlockGroupTransmission Enables and configures code-block-group (CBG) based transmission (see TS 38.213 [13], clause 9.1.1).

maxMIMO-Layers Indicates the maximum number of MIMO layers to be used for PDSCH in all BWPs of this serving cell. (see TS 38.212 [17], clause 5.4.2.1).

nrofHARQ-ProcessesForPDSCH The number of HARQ processes to be used on the PDSCH of a serving cell. Value n2 corresponds to 2 HARQ processes, value n4 to 4 HARQ processes, and so on. If the field is absent, the UE uses 8 HARQ processes (see TS 38.214 [19], clause 5.1).

processingType2Enabled Enables configuration of advanced processing time capability 2 for PDSCH (see 38.214 [19], clause 5.3).

pucch-Cell The ID of the serving cell (of the same cell group) to use for PUCCH. If the field is absent, the UE sends the HARQ feedback on the PUCCH of the SpCell of this cell group, or on this serving cell if it is a PUCCH SCell.

xOverhead Accounts for overhead from CSI-RS, CORESET, etc. If the field is absent, the UE applies value xOh0 (see TS 38.214 [19], clause 5.1.3.2).

| Conditional Presence | Explanation |
| --- | --- |
| SCellAddOnly | It is optionally present, Need S, for (non-PUCCH) SCells when adding a new SCell. The field is absent, Need M, when reconfiguring SCells. The field is also absent for the SpCells as well as for a PUCCH SCell. |

#### –	PDSCH-TimeDomainResourceAllocationList

The IE PDSCH-TimeDomainResourceAllocation is used to configure a time domain relation between PDCCH and PDSCH. The PDSCH-TimeDomainResourceAllocationList contains one or more of such PDSCH-TimeDomainResourceAllocations. The network indicates in the DL assignment which of the configured time domain allocations the UE shall apply for that DL assignment. The UE determines the bit width of the DCI field based on the number of entries in the PDSCH-TimeDomainResourceAllocationList. Value 0 in the DCI field refers to the first element in this list, value 1 in the DCI field refers to the second element in this list, and so on.

PDSCH-TimeDomainResourceAllocationList information element

-- ASN1START

-- TAG-PDSCH-TIMEDOMAINRESOURCEALLOCATIONLIST-START

PDSCH-TimeDomainResourceAllocationList ::=  SEQUENCE (SIZE(1..maxNrofDL-Allocations)) OF PDSCH-TimeDomainResourceAllocation

PDSCH-TimeDomainResourceAllocation ::=   SEQUENCE {

k0                                      INTEGER(0..32)                                                     OPTIONAL,   -- Need S

mappingType                             ENUMERATED {typeA, typeB},

startSymbolAndLength                    INTEGER (0..127)

}

-- TAG-PDSCH-TIMEDOMAINRESOURCEALLOCATIONLIST-STOP

-- ASN1STOP

PDSCH-TimeDomainResourceAllocation field descriptions

k0 Slot offset between DCI and its scheduled PDSCH (see TS 38.214 [19], clause 5.1.2.1) When the field is absent the UE applies the value 0.

mappingType PDSCH mapping type. (see TS 38.214 [19], clause 5.3).

startSymbolAndLength An index giving valid combinations of start symbol and length (jointly encoded) as start and length indicator (SLIV). The network configures the field so that the allocation does not cross the slot boundary (see TS 38.214 [19], clause 5.1.2.1).

#### –	PHR-Config

The IE PHR-Config is used to configure parameters for power headroom reporting.

PHR-Config information element

-- ASN1START

-- TAG-PHR-CONFIG-START

PHR-Config ::=                      SEQUENCE {

phr-PeriodicTimer                   ENUMERATED {sf10, sf20, sf50, sf100, sf200,sf500, sf1000, infinity},

phr-ProhibitTimer                   ENUMERATED {sf0, sf10, sf20, sf50, sf100,sf200, sf500, sf1000},

phr-Tx-PowerFactorChange            ENUMERATED {dB1, dB3, dB6, infinity},

multiplePHR                         BOOLEAN,

dummy                               BOOLEAN,

phr-Type2OtherCell                  BOOLEAN,

phr-ModeOtherCG                     ENUMERATED {real, virtual},

...

}

-- TAG-PHR-CONFIG-STOP

-- ASN1STOP

PHR-Config field descriptions

dummy This field is not used in this version of the specification and the UE ignores the received value.

multiplePHR Indicates if power headroom shall be reported using the Single Entry PHR MAC control element or Multiple Entry PHR MAC control element defined in TS 38.321 [3]. True means to use Multiple Entry PHR MAC control element and False means to use the Single Entry PHR MAC control element defined in TS 38.321 [3]. The network configures this field to true for MR-DC and UL CA for NR, and to false in all other cases.

phr-ModeOtherCG Indicates the mode (i.e. real or virtual) used for the PHR of the activated cells that are part of the other Cell Group (i.e. MCG or SCG), when DC is configured. If the UE is configured with only one cell group (no DC), it ignores the field.

phr-PeriodicTimer Value in number of subframes for PHR reporting as specified in TS 38.321 [3]. Value sf10 corresponds to 10 subframes, value sf20 corresponds to 20 subframes, and so on.

phr-ProhibitTimer Value in number of subframes for PHR reporting as specified in TS 38.321 [3]. Value sf0 corresponds to 0 subframe, value sf10 corresponds to 10 subframes, value sf20 corresponds to 20 subframes, and so on.

phr-Tx-PowerFactorChange Value in dB for PHR reporting as specified in TS 38.321 [3]. Value dB1 corresponds to 1 dB, dB3 corresponds to 3 dB and so on. The same value applies for each serving cell (although the associated functionality is performed independently for each cell).

phr-Type2OtherCell If set to true, the UE shall report a PHR type 2 for the SpCell of the other MAC entity. See TS 38.321 [3], clause 5.4.6. Network sets this field to false if the UE is not configured with an E-UTRA MAC entity.

#### –	PhysCellId

The PhysCellId identifies the physical cell identity (PCI).

PhysCellId information element

-- ASN1START

-- TAG-PHYSCELLID-START

PhysCellId ::=                      INTEGER (0..1007)

-- TAG-PHYSCELLID-STOP

-- ASN1STOP

#### –	PhysicalCellGroupConfig

The IE PhysicalCellGroupConfig is used to configure cell-group specific L1 parameters.

PhysicalCellGroupConfig information element

-- ASN1START

-- TAG-PHYSICALCELLGROUPCONFIG-START

PhysicalCellGroupConfig ::=         SEQUENCE {

harq-ACK-SpatialBundlingPUCCH       ENUMERATED {true}                                               OPTIONAL,   -- Need S

harq-ACK-SpatialBundlingPUSCH       ENUMERATED {true}                                               OPTIONAL,   -- Need S

p-NR-FR1                            P-Max                                                           OPTIONAL,   -- Need R

pdsch-HARQ-ACK-Codebook             ENUMERATED {semiStatic, dynamic},

tpc-SRS-RNTI                        RNTI-Value                                                      OPTIONAL,   -- Need R

tpc-PUCCH-RNTI                      RNTI-Value                                                      OPTIONAL,   -- Need R

tpc-PUSCH-RNTI                      RNTI-Value                                                      OPTIONAL,   -- Need R

sp-CSI-RNTI                         RNTI-Value                                                      OPTIONAL,   -- Need R

cs-RNTI                             SetupRelease { RNTI-Value }                                     OPTIONAL,   -- Need M

...,

[[

mcs-C-RNTI                          RNTI-Value                                                      OPTIONAL,   -- Need R

p-UE-FR1                            P-Max                                                           OPTIONAL    -- Cond MCG-Only

]],

[[

xScale                              ENUMERATED {dB0, dB6, spare2, spare1}                           OPTIONAL    -- Cond SCG-Only

]],

[[

pdcch-BlindDetection                SetupRelease { PDCCH-BlindDetection }                           OPTIONAL    -- Need M

]]

}

PDCCH-BlindDetection ::=                INTEGER (1..15)

-- TAG-PHYSICALCELLGROUPCONFIG-STOP

-- ASN1STOP

PhysicalCellGroupConfig field descriptions

cs-RNTI RNTI value for downlink SPS (see SPS-Config) and uplink configured grant (see ConfiguredGrantConfig).

harq-ACK-SpatialBundlingPUCCH Enables spatial bundling of HARQ ACKs. It is configured per cell group (i.e. for all the cells within the cell group) for PUCCH reporting of HARQ-ACK. It is only applicable when more than 4 layers are possible to schedule. When the field is absent, the spatial bundling is disabled (see TS 38.213 [13], clause 9.1.2.1).

harq-ACK-SpatialBundlingPUSCH Enables spatial bundling of HARQ ACKs. It is configured per cell group (i.e. for all the cells within the cell group) for PUSCH reporting of HARQ-ACK. It is only applicable when more than 4 layers are possible to schedule. When the field is absent, the spatial bundling is disabled (see TS 38.213 [13], clauses 9.1.2.2 and 9.1.3.2).

mcs-C-RNTI RNTI to indicate use of qam64LowSE for grant-based transmissions. When the mcs-C-RNTI is configured, RNTI scrambling of DCI CRC is used to choose the corresponding MCS table.

pdcch-BlindDetection Indicates the reference number of cells for PDCCH blind detection for the CG. Network configures the field for each CG when the UE is in NR DC and sets the value in accordance with the constraints specified in TS 38.213 [13]. The network configures pdcch-BlindDetection only if the UE is in NR-DC.

p-NR-FR1 The maximum total transmit power to be used by the UE in this NR cell group across all serving cells in frequency range 1 (FR1). The maximum transmit power that the UE may use may be additionally limited by p-Max (configured in FrequencyInfoUL) and by p-UE-FR1 (configured total for all serving cells operating on FR1).

p-UE-FR1 The maximum total transmit power to be used by the UE across all serving cells in frequency range 1 (FR1) across all cell groups. The maximum transmit power that the UE may use may be additionally limited by p-Max (configured in FrequencyInfoUL) and by p-NR-FR1 (configured for the cell group).

pdsch-HARQ-ACK-Codebook The PDSCH HARQ-ACK codebook is either semi-static or dynamic. This is applicable to both CA and none CA operation (see TS 38.213 [13], clauses 9.1.2 and 9.1.3).

sp-CSI-RNTI RNTI for Semi-Persistent CSI reporting on PUSCH (see CSI-ReportConfig) (see TS 38.214 [19], clause 5.2.1.5.2). Network always configures the UE with a value for this field when at least one CSI-ReportConfig with reportConfigType set to semiPersistentOnPUSCH is configured.

tpc-PUCCH-RNTI RNTI used for PUCCH TPC commands on DCI (see TS 38.213 [13], clause 10.1).

tpc-PUSCH-RNTI RNTI used for PUSCH TPC commands on DCI (see TS 38.213 [13], clause 10.1).

tpc-SRS-RNTI RNTI used for SRS TPC commands on DCI (see TS 38.213 [13], clause 10.1).

xScale The UE is allowed to drop NR only if the power scaling applied to NR results in a difference between scaled and unscaled NR UL of more than xScale dB (see TS 38.213 [13]). If the value is not configured for dynamic power sharing, the UE assumes default value of 6 dB.

| Conditional Presence | Explanation |
| --- | --- |
| MCG-Only | This field is optionally present, Need R, in the PhysicalCellGroupConfig of the MCG. It is absent otherwise. |
| SCG-Only | This field is optionally present, Need S, in the PhysicalCellGroupConfig of the SCG in (NG)EN-DC as defined in TS 38.213 [13]. It is absent otherwise. |

#### –	PLMN-Identity

The IE PLMN-Identity identifies a Public Land Mobile Network. Further information regarding how to set the IE is specified in TS 23.003 [21].

PLMN-Identity information element

-- ASN1START

-- TAG-PLMN-IDENTITY-START

PLMN-Identity ::=                   SEQUENCE {

mcc                                 MCC                 OPTIONAL,                   -- Cond MCC

mnc                                 MNC

}

MCC ::=                             SEQUENCE (SIZE (3)) OF MCC-MNC-Digit

MNC ::=                             SEQUENCE (SIZE (2..3)) OF MCC-MNC-Digit

MCC-MNC-Digit ::=                   INTEGER (0..9)

-- TAG-PLMN-IDENTITY-STOP

-- ASN1STOP

PLMN-Identity field descriptions

mcc The first element contains the first MCC digit, the second element the second MCC digit and so on. If the field is absent, it takes the same value as the mcc of the immediately preceding IE PLMN-Identity. See TS 23.003 [21].

mnc The first element contains the first MNC digit, the second element the second MNC digit and so on. See TS 23.003 [21].

| Conditional Presence | Explanation |
| --- | --- |
| MCC | This field is mandatory present when PLMN-Identity is not used in a list or if it is the first entry of PLMN-Identity in a list. Otherwise it is optionally present, Need S. |

#### –	PLMN-IdentityInfoList

The IE PLMN-IdentityInfoList includes a list of PLMN identity information.

PLMN-IdentityInfoList information element

-- ASN1START

-- TAG-PLMN-IDENTITYINFOLIST-START

PLMN-IdentityInfoList ::=               SEQUENCE (SIZE (1..maxPLMN)) OF PLMN-IdentityInfo

PLMN-IdentityInfo ::=                   SEQUENCE {

plmn-IdentityList                       SEQUENCE (SIZE (1..maxPLMN)) OF PLMN-Identity,

trackingAreaCode                        TrackingAreaCode                                            OPTIONAL,       -- Need R

ranac                                   RAN-AreaCode                                                OPTIONAL,       -- Need R

cellIdentity                            CellIdentity,

cellReservedForOperatorUse              ENUMERATED {reserved, notReserved},

...

}

-- TAG-PLMN-IDENTITYINFOLIST-STOP

-- ASN1STOP

PLMN-IdentityInfo field descriptions

cellReservedForOperatorUse Indicates whether the cell is reserved for operator use (per PLMN), as defined in TS 38.304 [20].

trackingAreaCode Indicates Tracking Area Code to which the cell indicated by cellIdentity field belongs. The absence of the field indicates that the cell only supports PSCell/SCell functionality (per PLMN).

#### –	PRB-Id

The IE PRB-Id identifies a Physical Resource Block (PRB) position within a carrier.

PRB-Id information element

-- ASN1START

-- TAG-PRB-ID-START

PRB-Id ::=                          INTEGER (0..maxNrofPhysicalResourceBlocks-1)

-- TAG-PRB-ID-STOP

-- ASN1STOP

#### –	PTRS-DownlinkConfig

The IE PTRS-DownlinkConfig is used to configure downlink phase tracking reference signals (PTRS) (see TS 38.214 [19] clause 5.1.6.3)

PTRS-DownlinkConfig information element

-- ASN1START

-- TAG-PTRS-DOWNLINKCONFIG-START

PTRS-DownlinkConfig ::=             SEQUENCE {

frequencyDensity                    SEQUENCE (SIZE (2)) OF INTEGER (1..276)                                 OPTIONAL,   -- Need S

timeDensity                         SEQUENCE (SIZE (3)) OF INTEGER (0..29)                                  OPTIONAL,   -- Need S

epre-Ratio                          INTEGER (0..3)                                                          OPTIONAL,   -- Need S

resourceElementOffset               ENUMERATED { offset01, offset10, offset11 }                             OPTIONAL,   -- Need S

...

}

-- TAG-PTRS-DOWNLINKCONFIG-STOP

-- ASN1STOP

PTRS-DownlinkConfig field descriptions

epre-Ratio EPRE ratio between PTRS and PDSCH. Value 0 corresponds to the codepoint "00" in table 4.1-2. Value 1 corresponds to codepoint "01", and so on. If the field is not provided, the UE applies value 0 (see TS 38.214 [19], clause 4.1).

frequencyDensity Presence and frequency density of DL PT-RS as a function of Scheduled BW. If the field is absent, the UE uses K_PT-RS = 2 (see TS 38.214 [19], clause 5.1.6.3, table 5.1.6.3-2).

resourceElementOffset Indicates the subcarrier offset for DL PTRS. If the field is absent, the UE applies the value offset00 (see TS 38.214 [19], clause 6.4.1.2.2.1).

timeDensity Presence and time density of DL PT-RS as a function of MCS. The value 29 is only applicable for MCS Table 5.1.3.1-1 (TS 38.214 [19]). If the field is absent, the UE uses L_PT-RS = 1 (see TS 38.214 [19], clause 5.1.6.3, table 5.1.6.3-1).

#### –	PTRS-UplinkConfig

The IE PTRS-UplinkConfig is used to configure uplink Phase-Tracking-Reference-Signals (PTRS).

PTRS-UplinkConfig information element

-- ASN1START

-- TAG-PTRS-UPLINKCONFIG-START

PTRS-UplinkConfig ::=                   SEQUENCE {

transformPrecoderDisabled               SEQUENCE {

frequencyDensity                    SEQUENCE (SIZE (2)) OF INTEGER (1..276)                 OPTIONAL,   -- Need S

timeDensity                         SEQUENCE (SIZE (3)) OF INTEGER (0..29)                  OPTIONAL,   -- Need S

maxNrofPorts                        ENUMERATED {n1, n2},

resourceElementOffset               ENUMERATED {offset01, offset10, offset11 }              OPTIONAL,   -- Need S

ptrs-Power                          ENUMERATED {p00, p01, p10, p11}

}                                                                                               OPTIONAL,   -- Need R

transformPrecoderEnabled                SEQUENCE {

sampleDensity                           SEQUENCE (SIZE (5)) OF INTEGER (1..276),

timeDensityTransformPrecoding           ENUMERATED {d2}                                     OPTIONAL    -- Need S

}                                                                                               OPTIONAL,   -- Need R

...

}

-- TAG-PTRS-UPLINKCONFIG-STOP

-- ASN1STOP

PTRS-UplinkConfig field descriptions

frequencyDensity Presence and frequency density of UL PT-RS for CP-OFDM waveform as a function of scheduled BW If the field is absent, the UE uses K_PT-RS = 2 (see TS 38.214 [19], clause 6.1).

maxNrofPorts The maximum number of UL PTRS ports for CP-OFDM (see TS 38.214 [19], clause 6.2.3.1).

ptrs-Power UL PTRS power boosting factor per PTRS port (see TS 38.214 [19], clause 6.1, table 6.2.3.1.3).

resourceElementOffset Indicates the subcarrier offset for UL PTRS for CP-OFDM. If the field is absent, the UE applies the value offset00 (see TS 38.211 [16], clause 6.4.1.2.2).

sampleDensity Sample density of PT-RS for DFT-s-OFDM, pre-DFT, indicating a set of thresholds T={NRBn, n=0,1,2,3,4}, that indicates dependency between presence of PT-RS and scheduled BW and the values of X and K the UE should use depending on the scheduled BW, see TS 38.214 [19], clause 6.1, table 6.2.3.2-1.

timeDensity Presence and time density of UL PT-RS for CP-OFDM waveform as a function of MCS If the field is absent, the UE uses L_PT-RS = 1 (see TS 38.214 [19], clause 6.1).

timeDensityTransformPrecoding Time density (OFDM symbol level) of PT-RS for DFT-s-OFDM. If the field is absent, the UE applies value d1 (see TS 38.214 [19], clause 6.1).

transformPrecoderDisabled Configuration of UL PTRS without transform precoder (with CP-OFDM).

transformPrecoderEnabled Configuration of UL PTRS with transform precoder (DFT-S-OFDM).

#### –	PUCCH-Config

The IE PUCCH-Config is used to configure UE specific PUCCH parameters (per BWP).

PUCCH-Config information element

-- ASN1START

-- TAG-PUCCH-CONFIG-START

PUCCH-Config ::=                        SEQUENCE {

resourceSetToAddModList                 SEQUENCE (SIZE (1..maxNrofPUCCH-ResourceSets)) OF PUCCH-ResourceSet   OPTIONAL, -- Need N

resourceSetToReleaseList                SEQUENCE (SIZE (1..maxNrofPUCCH-ResourceSets)) OF PUCCH-ResourceSetId OPTIONAL, -- Need N

resourceToAddModList                    SEQUENCE (SIZE (1..maxNrofPUCCH-Resources)) OF PUCCH-Resource         OPTIONAL, -- Need N

resourceToReleaseList                   SEQUENCE (SIZE (1..maxNrofPUCCH-Resources)) OF PUCCH-ResourceId       OPTIONAL, -- Need N

format1                                 SetupRelease { PUCCH-FormatConfig }                                   OPTIONAL, -- Need M

format2                                 SetupRelease { PUCCH-FormatConfig }                                   OPTIONAL, -- Need M

format3                                 SetupRelease { PUCCH-FormatConfig }                                   OPTIONAL, -- Need M

format4                                 SetupRelease { PUCCH-FormatConfig }                                   OPTIONAL, -- Need M

schedulingRequestResourceToAddModList   SEQUENCE (SIZE (1..maxNrofSR-Resources)) OF SchedulingRequestResourceConfig

OPTIONAL, -- Need N

schedulingRequestResourceToReleaseList  SEQUENCE (SIZE (1..maxNrofSR-Resources)) OF SchedulingRequestResourceId

OPTIONAL, -- Need N

multi-CSI-PUCCH-ResourceList            SEQUENCE (SIZE (1..2)) OF PUCCH-ResourceId                            OPTIONAL, -- Need M

dl-DataToUL-ACK                         SEQUENCE (SIZE (1..8)) OF INTEGER (0..15)                             OPTIONAL, -- Need M

spatialRelationInfoToAddModList         SEQUENCE (SIZE (1..maxNrofSpatialRelationInfos)) OF PUCCH-SpatialRelationInfo

OPTIONAL, -- Need N

spatialRelationInfoToReleaseList        SEQUENCE (SIZE (1..maxNrofSpatialRelationInfos)) OF PUCCH-SpatialRelationInfoId

OPTIONAL, -- Need N

pucch-PowerControl                      PUCCH-PowerControl                                                    OPTIONAL, -- Need M

...

}

PUCCH-FormatConfig ::=                  SEQUENCE {

interslotFrequencyHopping               ENUMERATED {enabled}                                                  OPTIONAL, -- Need R

additionalDMRS                          ENUMERATED {true}                                                     OPTIONAL, -- Need R

maxCodeRate                             PUCCH-MaxCodeRate                                                     OPTIONAL, -- Need R

nrofSlots                               ENUMERATED {n2,n4,n8}                                                 OPTIONAL, -- Need S

pi2BPSK                                 ENUMERATED {enabled}                                                  OPTIONAL, -- Need R

simultaneousHARQ-ACK-CSI                ENUMERATED {true}                                                     OPTIONAL  -- Need R

}

PUCCH-MaxCodeRate ::=                   ENUMERATED {zeroDot08, zeroDot15, zeroDot25, zeroDot35, zeroDot45, zeroDot60, zeroDot80}

-- A set with one or more PUCCH resources

PUCCH-ResourceSet ::=                   SEQUENCE {

pucch-ResourceSetId                     PUCCH-ResourceSetId,

resourceList                            SEQUENCE (SIZE (1..maxNrofPUCCH-ResourcesPerSet)) OF PUCCH-ResourceId,

maxPayloadSize                          INTEGER (4..256)                                                      OPTIONAL  -- Need R

}

PUCCH-ResourceSetId ::=                 INTEGER (0..maxNrofPUCCH-ResourceSets-1)

PUCCH-Resource ::=                      SEQUENCE {

pucch-ResourceId                        PUCCH-ResourceId,

startingPRB                             PRB-Id,

intraSlotFrequencyHopping               ENUMERATED { enabled }                                                OPTIONAL, -- Need R

secondHopPRB                            PRB-Id                                                                OPTIONAL, -- Need R

format                                  CHOICE {

format0                                 PUCCH-format0,

format1                                 PUCCH-format1,

format2                                 PUCCH-format2,

format3                                 PUCCH-format3,

format4                                 PUCCH-format4

}

}

PUCCH-ResourceId ::=                    INTEGER (0..maxNrofPUCCH-Resources-1)

PUCCH-format0 ::=                               SEQUENCE {

initialCyclicShift                              INTEGER(0..11),

nrofSymbols                                     INTEGER (1..2),

startingSymbolIndex                             INTEGER(0..13)

}

PUCCH-format1 ::=                               SEQUENCE {

initialCyclicShift                              INTEGER(0..11),

nrofSymbols                                     INTEGER (4..14),

startingSymbolIndex                             INTEGER(0..10),

timeDomainOCC                                   INTEGER(0..6)

}

PUCCH-format2 ::=                               SEQUENCE {

nrofPRBs                                        INTEGER (1..16),

nrofSymbols                                     INTEGER (1..2),

startingSymbolIndex                             INTEGER(0..13)

}

PUCCH-format3 ::=                               SEQUENCE {

nrofPRBs                                        INTEGER (1..16),

nrofSymbols                                     INTEGER (4..14),

startingSymbolIndex                             INTEGER(0..10)

}

PUCCH-format4 ::=                               SEQUENCE {

nrofSymbols                                     INTEGER (4..14),

occ-Length                                      ENUMERATED {n2,n4},

occ-Index                                       ENUMERATED {n0,n1,n2,n3},

startingSymbolIndex                             INTEGER(0..10)

}

-- TAG-PUCCH-CONFIG-STOP

-- ASN1STOP

PUCCH-Config field descriptions

dl-DataToUL-ACK List of timing for given PDSCH to the DL ACK (see TS 38.213 [13], clause 9.1.2).

format1 Parameters that are common for all PUCCH resources of format 1.

format2 Parameters that are common for all PUCCH resources of format 2.

format3 Parameters that are common for all PUCCH resources of format 3.

format4. Parameters that are common for all PUCCH resources of format 4

resourceSetToAddModList, resourceSetToReleaseList Lists for adding and releasing PUCCH resource sets (see TS 38.213 [13], clause 9.2).

resourceToAddModList, resourceToReleaseList Lists for adding and releasing PUCCH resources applicable for the UL BWP and serving cell in which the PUCCH-Config is defined. The resources defined herein are referred to from other parts of the configuration to determine which resource the UE shall use for which report.

spatialRelationInfoToAddModList Configuration of the spatial relation between a reference RS and PUCCH. Reference RS can be SSB/CSI-RS/SRS. If the list has more than one element, MAC-CE selects a single element (see TS 38.321 [3], clause 5.18.8 and TS 38.213 [13], clause 9.2.2).

PUCCH-format3 field descriptions

nrofPRBs The supported values are 1,2,3,4,5,6,8,9,10,12,15 and 16.

PUCCH-FormatConfig field descriptions

additionalDMRS If the field is present, the UE enables 2 DMRS symbols per hop of a PUCCH Format 3 or 4 if both hops are more than X symbols when FH is enabled (X=4). And it enables 4 DMRS symbols for a PUCCH Format 3 or 4 with more than 2X+1 symbols when FH is disabled (X=4). The field is not applicable for format 1 and 2. See TS 38.213 [13], clause 9.2.2.

interslotFrequencyHopping If the field is present, the UE enables inter-slot frequency hopping when PUCCH Format 1, 3 or 4 is repeated over multiple slots. For long PUCCH over multiple slots, the intra and inter slot frequency hopping cannot be enabled at the same time for a UE. The field is not applicable for format 2. See TS 38.213 [13], clause 9.2.6.

maxCodeRate Max coding rate to determine how to feedback UCI on PUCCH for format 2, 3 or 4. The field is not applicable for format 1. See TS 38.213 [13], clause 9.2.5.

nrofSlots Number of slots with the same PUCCH F1, F3 or F4. When the field is absent the UE applies the value n1. The field is not applicable for format 2. See TS 38.213 [13], clause 9.2.6.

pi2BPSK If the field is present, the UE uses pi/2 BPSK for UCI symbols instead of QPSK for PUCCH. The field is not applicable for format 1 and 2. See TS 38.213 [13], clause 9.2.5.

simultaneousHARQ-ACK-CSI If the field is present, the UE uses simultaneous transmission of CSI and HARQ-ACK feedback with or without SR with PUCCH Format 2, 3 or 4. See TS 38.213 [13], clause 9.2.5. When the field is absent the UE applies the value off. The field is not applicable for format 1.

PUCCH-Resource field descriptions

format Selection of the PUCCH format (format 0 – 4) and format-specific parameters, see TS 38.213 [13], clause 9.2. format0 and format1 are only allowed for a resource in a first PUCCH resource set. format2, format3 and format4 are only allowed for a resource in non-first PUCCH resource set.

intraSlotFrequencyHopping Enabling intra-slot frequency hopping, applicable for all types of PUCCH formats. For long PUCCH over multiple slots, the intra and inter slot frequency hopping cannot be enabled at the same time for a UE. See TS 38.213 [13], clause 9.2.1.

pucch-ResourceId Identifier of the PUCCH resource.

secondHopPRB Index of first PRB after frequency hopping of PUCCH. This value is applicable for intra-slot frequency hopping (see TS 38.213 [13], clause 9.2.1) or inter-slot frequency hopping (see TS 38.213 [13], clause 9.2.6).

PUCCH-ResourceSet field descriptions

maxPayloadSize Maximum number of UCI information bits that the UE may transmit using this PUCCH resource set (see TS 38.213 [13], clause 9.2.1). In a PUCCH occurrence, the UE chooses the first of its PUCCH-ResourceSet which supports the number of bits that the UE wants to transmit. The field is absent in the first set (Set0) and in the last configured set since the UE derives the maximum number of UCI information bits as specified in TS 38.213 [13], clause 9.2.1. This field can take integer values that are multiples of 4.

resourceList PUCCH resources of format0 and format1 are only allowed in the first PUCCH resource set, i.e., in a PUCCH-ResourceSet with pucch-ResourceSetId = 0. This set may contain between 1 and 32 resources. PUCCH resources of format2, format3 and format4 are only allowed in a PUCCH-ResourceSet with pucch-ResourceSetId > 0. If present, these sets contain between 1 and 8 resources each. The UE chooses a PUCCH-Resource from this list as specified in TS 38.213 [13], clause 9.2.3. Note that this list contains only a list of resource IDs. The actual resources are configured in PUCCH-Config.

#### –	PUCCH-ConfigCommon

The IE PUCCH-ConfigCommon is used to configure the cell specific PUCCH parameters.

PUCCH-ConfigCommon information element

-- ASN1START

-- TAG-PUCCH-CONFIGCOMMON-START

PUCCH-ConfigCommon ::=              SEQUENCE {

pucch-ResourceCommon                INTEGER (0..15)                                      OPTIONAL,   -- Cond InitialBWP-Only

pucch-GroupHopping                  ENUMERATED { neither, enable, disable },

hoppingId                           INTEGER (0..1023)                                    OPTIONAL,   -- Need R

p0-nominal                          INTEGER (-202..24)                                   OPTIONAL,   -- Need R

...

}

-- TAG-PUCCH-CONFIGCOMMON-STOP

-- ASN1STOP

PUCCH-ConfigCommon field descriptions

hoppingId Cell-specific scrambling ID for group hopping and sequence hopping if enabled, see TS 38.211 [16], clause 6.3.2.2.

p0-nominal Power control parameter P0 for PUCCH transmissions. Value in dBm. Only even values (step size 2) allowed (see TS 38.213 [13], clause 7.2).

pucch-GroupHopping Configuration of group- and sequence hopping for all the PUCCH formats 0, 1, 3 and 4. Value neither implies neither group or sequence hopping is enabled. Value enable enables group hopping and disables sequence hopping. Value disable disables group hopping and enables sequence hopping (see TS 38.211 [16], clause 6.3.2.2).

pucch-ResourceCommon An entry into a 16-row table where each row configures a set of cell-specific PUCCH resources/parameters. The UE uses those PUCCH resources until it is provided with a dedicated PUCCH-Config (e.g. during initial access) on the initial uplink BWP. Once the network provides a dedicated PUCCH-Config for that bandwidth part the UE applies that one instead of the one provided in this field (see TS 38.213 [13], clause 9.2).

| Conditional Presence | Explanation |
| --- | --- |
| InitialBWP-Only | The field is mandatory present in the PUCCH-ConfigCommon of the initial BWP (BWP#0) in SIB1. It is absent in other BWPs. |

#### –	PUCCH-PathlossReferenceRS-Id

The IE PUCCH-PathlossReferenceRS-Id is an ID for a reference signal (RS) configured as PUCCH pathloss reference (see TS 38.213 [13], clause 7.2).

PUCCH-PathlossReferenceRS-Id information element

-- ASN1START

-- TAG-PUCCH-PATHLOSSREFERENCERS-ID-START

PUCCH-PathlossReferenceRS-Id ::=            INTEGER (0..maxNrofPUCCH-PathlossReferenceRSs-1)

-- TAG-PUCCH-PATHLOSSREFERENCERS-ID-STOP

-- ASN1STOP

#### –	PUCCH-PowerControl

The IE PUCCH-PowerControl is used to configure UE-specific parameters for the power control of PUCCH.

PUCCH-PowerControl information element

-- ASN1START

-- TAG-PUCCH-POWERCONTROL-START

PUCCH-PowerControl ::=              SEQUENCE {

deltaF-PUCCH-f0                     INTEGER (-16..15)                                                       OPTIONAL, -- Need R

deltaF-PUCCH-f1                     INTEGER (-16..15)                                                       OPTIONAL, -- Need R

deltaF-PUCCH-f2                     INTEGER (-16..15)                                                       OPTIONAL, -- Need R

deltaF-PUCCH-f3                     INTEGER (-16..15)                                                       OPTIONAL, -- Need R

deltaF-PUCCH-f4                     INTEGER (-16..15)                                                       OPTIONAL, -- Need R

p0-Set                              SEQUENCE (SIZE (1..maxNrofPUCCH-P0-PerSet)) OF P0-PUCCH                 OPTIONAL, -- Need M

pathlossReferenceRSs                SEQUENCE (SIZE (1..maxNrofPUCCH-PathlossReferenceRSs)) OF PUCCH-PathlossReferenceRS

OPTIONAL, -- Need M

twoPUCCH-PC-AdjustmentStates        ENUMERATED {twoStates}                                                  OPTIONAL, -- Need S

...

}

P0-PUCCH ::=                            SEQUENCE {

p0-PUCCH-Id                             P0-PUCCH-Id,

p0-PUCCH-Value                          INTEGER (-16..15)

}

P0-PUCCH-Id ::=                         INTEGER (1..8)

PUCCH-PathlossReferenceRS ::=                   SEQUENCE {

pucch-PathlossReferenceRS-Id                PUCCH-PathlossReferenceRS-Id,

referenceSignal                             CHOICE {

ssb-Index                                   SSB-Index,

csi-RS-Index                                NZP-CSI-RS-ResourceId

}

}

-- TAG-PUCCH-POWERCONTROL-STOP

-- ASN1STOP

P0-PUCCH field descriptions

p0-PUCCH-Value P0 value for PUCCH with 1dB step size.

PUCCH-PowerControl field descriptions

deltaF-PUCCH-f0 deltaF for PUCCH format 0 with 1dB step size (see TS 38.213 [13], clause 7.2).

deltaF-PUCCH-f1 deltaF for PUCCH format 1 with 1dB step size (see TS 38.213 [13], clause 7.2).

deltaF-PUCCH-f2 deltaF for PUCCH format 2 with 1dB step size (see TS 38.213 [13], clause 7.2).

deltaF-PUCCH-f3 deltaF for PUCCH format 3 with 1dB step size (see TS 38.213 [13], clause 7.2).

deltaF-PUCCH-f4 deltaF for PUCCH format 4 with 1dB step size (see TS 38.213 [13], clause 7.2).

p0-Set A set with dedicated P0 values for PUCCH, i.e., {P01, P02,... } (see TS 38.213 [13], clause 7.2).

pathlossReferenceRSs A set of Reference Signals (e.g. a CSI-RS config or a SS block) to be used for PUCCH pathloss estimation. Up to maxNrofPUCCH-PathlossReference-RSs may be configured. When the field is absent, the UE uses the SSB as reference signal (see TS 38.213 [13], clause 7.2).

twoPUCCH-PC-AdjustmentStates Number of PUCCH power control adjustment states maintained by the UE (i.e., g(i)). If the field is present (n2) the UE maintains two power control states (i.e., g(i,0) and g(i,1)). If the field is absent, it maintains one power control state (i.e., g(i,0)) (see TS 38.213 [13], clause 7.2).

#### –	PUCCH-SpatialRelationInfo

The IE PUCCH-SpatialRelationInfo is used to configure the spatial setting for PUCCH transmission and the parameters for PUCCH power control, see TS 38.213, [13], clause 9.2.2.

PUCCH-SpatialRelationInfo information element

-- ASN1START

-- TAG-PUCCH-SPATIALRELATIONINFO-START

PUCCH-SpatialRelationInfo ::=           SEQUENCE {

pucch-SpatialRelationInfoId         PUCCH-SpatialRelationInfoId,

servingCellId                           ServCellIndex                                                    OPTIONAL,   -- Need S

referenceSignal                         CHOICE {

ssb-Index                               SSB-Index,

csi-RS-Index                            NZP-CSI-RS-ResourceId,

srs                                     SEQUENCE {

resource                            SRS-ResourceId,

uplinkBWP                           BWP-Id

}

},

pucch-PathlossReferenceRS-Id            PUCCH-PathlossReferenceRS-Id,

p0-PUCCH-Id                             P0-PUCCH-Id,

closedLoopIndex                         ENUMERATED { i0, i1 }

}

PUCCH-SpatialRelationInfoId ::=         INTEGER (1..maxNrofSpatialRelationInfos)

-- TAG-PUCCH-SPATIALRELATIONINFO-STOP

-- ASN1STOP

PUCCH-SpatialRelationInfo field descriptions

servingCellId If the field is absent, the UE applies the ServCellId of the serving cell in which this PUCCH-SpatialRelationInfo is configured

#### –	PUCCH-TPC-CommandConfig

The IE PUCCH-TPC-CommandConfig is used to configure the UE for extracting TPC commands for PUCCH from a group-TPC messages on DCI.

PUCCH-TPC-CommandConfig information element

-- ASN1START

-- TAG-PUCCH-TPC-COMMANDCONFIG-START

PUCCH-TPC-CommandConfig ::=             SEQUENCE {

tpc-IndexPCell                          INTEGER (1..15)                         OPTIONAL,   -- Cond PDCCH-OfSpcell

tpc-IndexPUCCH-SCell                    INTEGER (1..15)                         OPTIONAL,   -- Cond PDCCH-ofSpCellOrPUCCH-SCell

...

}

-- TAG-PUCCH-TPC-COMMANDCONFIG-STOP

-- ASN1STOP

PUCCH-TPC-CommandConfig field descriptions

tpc-IndexPCell An index determining the position of the first bit of TPC command (applicable to the SpCell) inside the DCI format 2-2 payload.

tpc-IndexPUCCH-SCell An index determining the position of the first bit of TPC command (applicable to the PUCCH SCell) inside the DCI format 2-2 payload.

| Conditional Presence | Explanation |
| --- | --- |
| PDCCH-OfSpcell | The field is mandatory present if the PUCCH-TPC-CommandConfig is provided in the PDCCH-Config for the SpCell. Otherwise, the field is absent, Need R. |
| PDCCH-ofSpCellOrPUCCH-SCell | The field is mandatory present if the PUCCH-TPC-CommandConfig is provided in the PDCCH-Config for the PUCCH-SCell. The field is optionally present, need R, if the UE is configured with a PUCCH SCell in this cell group and if the PUCCH-TPC-CommandConfig is provided in the PDCCH-Config for the SpCell. Otherwise, the field is absent, Need R. |

#### –	PUSCH-Config

The IE PUSCH-Config is used to configure the UE specific PUSCH parameters applicable to a particular BWP.

PUSCH-Config information element

-- ASN1START

-- TAG-PUSCH-CONFIG-START

PUSCH-Config ::=                        SEQUENCE {

dataScramblingIdentityPUSCH             INTEGER (0..1023)                                                   OPTIONAL,   -- Need S

txConfig                                ENUMERATED {codebook, nonCodebook}                                  OPTIONAL,   -- Need S

dmrs-UplinkForPUSCH-MappingTypeA        SetupRelease { DMRS-UplinkConfig }                                  OPTIONAL,   -- Need M

dmrs-UplinkForPUSCH-MappingTypeB        SetupRelease { DMRS-UplinkConfig }                                  OPTIONAL,   -- Need M

pusch-PowerControl                      PUSCH-PowerControl                                                  OPTIONAL,   -- Need M

frequencyHopping                        ENUMERATED {intraSlot, interSlot}                                   OPTIONAL,   -- Need S

frequencyHoppingOffsetLists             SEQUENCE (SIZE (1..4)) OF INTEGER (1.. maxNrofPhysicalResourceBlocks-1)

OPTIONAL,   -- Need M

resourceAllocation                      ENUMERATED { resourceAllocationType0, resourceAllocationType1, dynamicSwitch},

pusch-TimeDomainAllocationList          SetupRelease { PUSCH-TimeDomainResourceAllocationList }             OPTIONAL,   -- Need M

pusch-AggregationFactor                 ENUMERATED { n2, n4, n8 }                                           OPTIONAL,   -- Need S

mcs-Table                               ENUMERATED {qam256, qam64LowSE}                                     OPTIONAL,   -- Need S

mcs-TableTransformPrecoder              ENUMERATED {qam256, qam64LowSE}                                     OPTIONAL,   -- Need S

transformPrecoder                       ENUMERATED {enabled, disabled}                                      OPTIONAL,   -- Need S

codebookSubset                          ENUMERATED {fullyAndPartialAndNonCoherent, partialAndNonCoherent,nonCoherent}

OPTIONAL, -- Cond codebookBased

maxRank                                 INTEGER (1..4)                                            OPTIONAL, -- Cond codebookBased

rbg-Size                                ENUMERATED { config2}                                     OPTIONAL, -- Need S

uci-OnPUSCH                             SetupRelease { UCI-OnPUSCH}                               OPTIONAL, -- Need M

tp-pi2BPSK                              ENUMERATED {enabled}                                      OPTIONAL, -- Need S

...

}

UCI-OnPUSCH ::=                         SEQUENCE {

betaOffsets                             CHOICE {

dynamic                             SEQUENCE (SIZE (4)) OF BetaOffsets,

semiStatic                          BetaOffsets

}                                                                                                             OPTIONAL, -- Need M

scaling                                 ENUMERATED { f0p5, f0p65, f0p8, f1 }

}

-- TAG-PUSCH-CONFIG-STOP

-- ASN1STOP

PUSCH-Config field descriptions

codebookSubset Subset of PMIs addressed by TPMI, where PMIs are those supported by UEs with maximum coherence capabilities (see TS 38.214 [19], clause 6.1.1.1).

dataScramblingIdentityPUSCH Identifier used to initalite data scrambling (c_init) for PUSCH. If the field is absent, the UE applies the physical cell ID. (see TS 38.211 [16], clause 6.3.1.1).

dmrs-UplinkForPUSCH-MappingTypeA DMRS configuration for PUSCH transmissions using PUSCH mapping type A (chosen dynamically via PUSCH-TimeDomainResourceAllocation). Only the fields dmrs-Type, dmrs-AdditionalPosition and maxLength may be set differently for mapping type A and B.

dmrs-UplinkForPUSCH-MappingTypeB DMRS configuration for PUSCH transmissions using PUSCH mapping type B (chosen dynamically via PUSCH-TimeDomainResourceAllocation). Only the fields dmrs-Type, dmrs-AdditionalPosition and maxLength may be set differently for mapping type A and B.

frequencyHopping The value intraSlot enables 'Intra-slot frequency hopping' and the value interSlot enables 'Inter-slot frequency hopping'. If the field is absent, frequency hopping is not configured (see TS 38.214 [19], clause 6.3).

frequencyHoppingOffsetLists Set of frequency hopping offsets used when frequency hopping is enabled for granted transmission (not msg3) and type 2 configured grant activation (see TS 38.214 [19], clause 6.3).

maxRank Subset of PMIs addressed by TRIs from 1 to ULmaxRank (see TS 38.214 [19], clause 6.1.1.1).

mcs-Table Indicates which MCS table the UE shall use for PUSCH without transform precoder (see TS 38.214 [19], clause 6.1.4.1). If the field is absent the UE applies the value 64QAM

mcs-TableTransformPrecoder Indicates which MCS table the UE shall use for PUSCH with transform precoding (see TS 38.214 [19], clause 6.1.4.1) If the field is absent the UE applies the value 64QAM

pusch-AggregationFactor Number of repetitions for data (see TS 38.214 [19], clause 6.1.2.1). If the field is absent the UE applies the value 1.

pusch-TimeDomainAllocationList List of time domain allocations for timing of UL assignment to UL data (see TS 38.214 [19], table 6.1.2.1.1-1).

rbg-Size Selection between configuration 1 and configuration 2 for RBG size for PUSCH. The UE does not apply this field if resourceAllocation is set to resourceAllocationType1. Otherwise, the UE applies the value config1 when the field is absent (see TS 38.214 [19], clause 6.1.2.2.1).

resourceAllocation Configuration of resource allocation type 0 and resource allocation type 1 for non-fallback DCI (see TS 38.214 [19], clause 6.1.2).

tp-pi2BPSK Enables pi/2-BPSK modulation with transform precoding if the field is present and disables it otherwise.

transformPrecoder The UE specific selection of transformer precoder for PUSCH (see TS 38.214 [19], clause 6.1.3). When the field is absent the UE applies the value of the field msg3-transformPrecoder.

txConfig Whether UE uses codebook based or non-codebook based transmission (see TS 38.214 [19], clause 6.1.1). If the field is absent, the UE transmits PUSCH on one antenna port, see TS 38.214 [19], clause 6.1.1.

UCI-OnPUSCH field descriptions

betaOffsets Selection between and configuration of dynamic and semi-static beta-offset. If the field is not configured, the UE applies the value 'semiStatic' (see TS 38.213 [13], clause 9.3).

scaling Indicates a scaling factor to limit the number of resource elements assigned to UCI on PUSCH. Value f0p5 corresponds to 0.5, value f0p65 corresponds to 0.65, and so on. The value configured herein is applicable for PUSCH with configured grant (see TS 38.212 [17], clause 6.3).

| Conditional Presence | Explanation |
| --- | --- |
| codebookBased | The field is mandatory present if txConfig is set to codebook and absent otherwise. |

#### –	PUSCH-ConfigCommon

The IE PUSCH-ConfigCommon is used to configure the cell specific PUSCH parameters.

PUSCH-ConfigCommon information element

-- ASN1START

-- TAG-PUSCH-CONFIGCOMMON-START

PUSCH-ConfigCommon ::=                  SEQUENCE {

groupHoppingEnabledTransformPrecoding   ENUMERATED {enabled}                                                OPTIONAL,   -- Need R

pusch-TimeDomainAllocationList          PUSCH-TimeDomainResourceAllocationList                              OPTIONAL,   -- Need R

msg3-DeltaPreamble                      INTEGER (-1..6)                                                     OPTIONAL,   -- Need R

p0-NominalWithGrant                     INTEGER (-202..24)                                                  OPTIONAL,   -- Need R

...

}

-- TAG-PUSCH-CONFIGCOMMON-STOP

-- ASN1STOP

PUSCH-ConfigCommon field descriptions

groupHoppingEnabledTransformPrecoding For DMRS transmission with transform precoder, the NW may configure group hopping by this cell-specific parameter, see TS 38.211 [16], clause 6.4.1.1.1.2.

msg3-DeltaPreamble Power offset between msg3 and RACH preamble transmission. Actual value = field value * 2 [dB] (see TS 38.213 [13], clause 7.1)

p0-NominalWithGrant P0 value for PUSCH with grant (except msg3). Value in dBm. Only even values (step size 2) allowed (see TS 38.213 [13], clause 7.1) This field is cell specific

pusch-TimeDomainAllocationList List of time domain allocations for timing of UL assignment to UL data (see TS 38.214 [19], table 6.1.2.1.1-1).

#### –	PUSCH-PowerControl

The IE PUSCH-PowerControl is used to configure UE specific power control parameter for PUSCH.

PUSCH-PowerControl information element

-- ASN1START

-- TAG-PUSCH-POWERCONTROL-START

PUSCH-PowerControl ::=              SEQUENCE {

tpc-Accumulation                    ENUMERATED { disabled }                                                 OPTIONAL, -- Need S

msg3-Alpha                          Alpha                                                                   OPTIONAL, -- Need S

p0-NominalWithoutGrant              INTEGER (-202..24)                                                      OPTIONAL, -- Need M

p0-AlphaSets                        SEQUENCE (SIZE (1..maxNrofP0-PUSCH-AlphaSets)) OF P0-PUSCH-AlphaSet     OPTIONAL, -- Need M

pathlossReferenceRSToAddModList     SEQUENCE (SIZE (1..maxNrofPUSCH-PathlossReferenceRSs)) OF PUSCH-PathlossReferenceRS

OPTIONAL, -- Need N

pathlossReferenceRSToReleaseList    SEQUENCE (SIZE (1..maxNrofPUSCH-PathlossReferenceRSs)) OF PUSCH-PathlossReferenceRS-Id

OPTIONAL,  -- Need N

twoPUSCH-PC-AdjustmentStates        ENUMERATED {twoStates}                                                  OPTIONAL, -- Need S

deltaMCS                            ENUMERATED {enabled}                                                    OPTIONAL, -- Need S

sri-PUSCH-MappingToAddModList       SEQUENCE (SIZE (1..maxNrofSRI-PUSCH-Mappings)) OF SRI-PUSCH-PowerControl

OPTIONAL, -- Need N

sri-PUSCH-MappingToReleaseList      SEQUENCE (SIZE (1..maxNrofSRI-PUSCH-Mappings)) OF SRI-PUSCH-PowerControlId

OPTIONAL  -- Need N

}

P0-PUSCH-AlphaSet ::=               SEQUENCE {

p0-PUSCH-AlphaSetId                 P0-PUSCH-AlphaSetId,

p0                                  INTEGER (-16..15)                                                       OPTIONAL, -- Need S

alpha                               Alpha                                                                   OPTIONAL  -- Need S

}

P0-PUSCH-AlphaSetId ::=             INTEGER (0..maxNrofP0-PUSCH-AlphaSets-1)

PUSCH-PathlossReferenceRS ::=       SEQUENCE {

pusch-PathlossReferenceRS-Id        PUSCH-PathlossReferenceRS-Id,

referenceSignal                     CHOICE {

ssb-Index                           SSB-Index,

csi-RS-Index                        NZP-CSI-RS-ResourceId

}

}

PUSCH-PathlossReferenceRS-Id ::=    INTEGER (0..maxNrofPUSCH-PathlossReferenceRSs-1)

SRI-PUSCH-PowerControl ::=          SEQUENCE {

sri-PUSCH-PowerControlId            SRI-PUSCH-PowerControlId,

sri-PUSCH-PathlossReferenceRS-Id    PUSCH-PathlossReferenceRS-Id,

sri-P0-PUSCH-AlphaSetId             P0-PUSCH-AlphaSetId,

sri-PUSCH-ClosedLoopIndex           ENUMERATED { i0, i1 }

}

SRI-PUSCH-PowerControlId ::=        INTEGER (0..maxNrofSRI-PUSCH-Mappings-1)

-- TAG-PUSCH-POWERCONTROL-STOP

-- ASN1STOP

P0-PUSCH-AlphaSet field descriptions

alpha alpha value for PUSCH with grant (except msg3) (see TS 38.213 [13], clause 7.1). When the field is absent the UE applies the value 1.

p0 P0 value for PUSCH with grant (except msg3) in steps of 1dB (see TS 38.213 [13], clause 7.1). When the field is absent the UE applies the value 0.

PUSCH-PowerControl field descriptions

deltaMCS Indicates whether to apply delta MCS. When the field is absent, the UE applies Ks = 0 in delta_TFC formula for PUSCH (see TS 38.213 [13], clause 7.1).

msg3-Alpha Dedicated alpha value for msg3 PUSCH (see TS 38.213 [13], clause 7.1). When the field is absent the UE applies the value 1.

p0-AlphaSets configuration {p0-pusch, alpha} sets for PUSCH (except msg3), i.e., { {p0,alpha,index1}, {p0,alpha,index2},...} (see TS 38.213 [13], clause 7.1). When no set is configured, the UE uses the P0-nominal for msg3 PUSCH, P0-UE is set to 0 and alpha is set according to msg3-Alpha configured for msg3 PUSCH.

p0-NominalWithoutGrant P0 value for UL grant-free/SPS based PUSCH. Value in dBm. Only even values (step size 2) allowed (see TS 38.213 [13], clause 7.1).

pathlossReferenceRSToAddModList A set of Reference Signals (e.g. a CSI-RS config or a SS block) to be used for PUSCH path loss estimation. Up to maxNrofPUSCH-PathlossReferenceRSs may be configured (see TS 38.213 [13], clause 7.1).

sri-PUSCH-MappingToAddModList A list of SRI-PUSCH-PowerControl elements among which one is selected by the SRI field in DCI (see TS 38.213 [13], clause 7.1).

tpc-Accumulation If enabled, UE applies TPC commands via accumulation. If not enabled, UE applies the TPC command without accumulation. If the field is absent, TPC accumulation is enabled (see TS 38.213 [13], clause 7.1).

twoPUSCH-PC-AdjustmentStates Number of PUSCH power control adjustment states maintained by the UE (i.e., fc(i)). If the field is present (n2) the UE maintains two power control states (i.e., fc(i,0) and fc(i,1)). If the field is absent, it maintains one power control state (i.e., fc(i,0)) (see TS 38.213 [13], clause 7.1).

SRI-PUSCH-PowerControl field descriptions

sri-P0-PUSCH-AlphaSetId The ID of a P0-PUSCH-AlphaSet as configured in p0-AlphaSets in PUSCH-PowerControl.

sri-PUSCH-ClosedLoopIndex The index of the closed power control loop associated with this SRI-PUSCH-PowerControl.

sri-PUSCH-PathlossReferenceRS-Id The ID of PUSCH-PathlossReferenceRS as configured in the pathlossReferenceRSToAddModList in PUSCH-PowerControl.

sri-PUSCH-PowerControlId The ID of this SRI-PUSCH-PowerControl configuration. It is used as the codepoint (payload) in the SRI DCI field.

#### –	PUSCH-ServingCellConfig

The IE PUSCH-ServingCellConfig is used to configure UE specific PUSCH parameters that are common across the UE's BWPs of one serving cell.

PUSCH-ServingCellConfig information element

-- ASN1START

-- TAG-PUSCH-SERVINGCELLCONFIG-START

PUSCH-ServingCellConfig ::=             SEQUENCE {

codeBlockGroupTransmission              SetupRelease { PUSCH-CodeBlockGroupTransmission }       OPTIONAL,   -- Need M

rateMatching                            ENUMERATED {limitedBufferRM}                            OPTIONAL,   -- Need S

xOverhead                               ENUMERATED {xoh6, xoh12, xoh18}                         OPTIONAL,   -- Need S

...,

[[

maxMIMO-Layers                          INTEGER (1..4)                                          OPTIONAL,   -- Need M

processingType2Enabled                  BOOLEAN                                                 OPTIONAL    -- Need M

]]

}

PUSCH-CodeBlockGroupTransmission ::=    SEQUENCE {

maxCodeBlockGroupsPerTransportBlock     ENUMERATED {n2, n4, n6, n8},

...

}

-- TAG-PUSCH-SERVINGCELLCONFIG-STOP

-- ASN1STOP

PUSCH-CodeBlockGroupTransmission field descriptions

maxCodeBlockGroupsPerTransportBlock Maximum number of code-block-groups (CBGs) per TB (see TS 38.213 [13], clause 9.1).

PUSCH-ServingCellConfig field descriptions

codeBlockGroupTransmission Enables and configures code-block-group (CBG) based transmission (see TS 38.214 [19], clause 5.1.5).

maxMIMO-Layers Indicates the maximum MIMO layer to be used for PUSCH in all BWPs of the normal UL of this serving cell (see TS 38.212 [17], clause 5.4.2.1). If present, the network sets maxRank to the same value. For SUL, the maximum number of MIMO layers is always 1, and network does not configure this field.

processingType2Enabled Enables configuration of advanced processing time capability 2 for PUSCH (see 38.214 [19], clause 6.4).

rateMatching Enables LBRM (Limited buffer rate-matching). When the field is absent the UE applies FBRM (Full buffer rate-matchingLBRM) (see TS 38.212 [17], clause 5.4.2).

xOverhead If the field is absent, the UE applies the value 'xoh0' (see TS 38.214 [19], clause 5.1.3.2).

#### –	PUSCH-TimeDomainResourceAllocationList

The IE PUSCH-TimeDomainResourceAllocation is used to configure a time domain relation between PDCCH and PUSCH. PUSCH-TimeDomainResourceAllocationList contains one or more of such PUSCH-TimeDomainResourceAllocations. The network indicates in the UL grant which of the configured time domain allocations the UE shall apply for that UL grant. The UE determines the bit width of the DCI field based on the number of entries in the PUSCH-TimeDomainResourceAllocationList. Value 0 in the DCI field refers to the first element in this list, value 1 in the DCI field refers to the second element in this list, and so on.

PUSCH-TimeDomainResourceAllocation information element

-- ASN1START

-- TAG-PUSCH-TIMEDOMAINRESOURCEALLOCATIONLIST-START

PUSCH-TimeDomainResourceAllocationList ::=  SEQUENCE (SIZE(1..maxNrofUL-Allocations)) OF PUSCH-TimeDomainResourceAllocation

PUSCH-TimeDomainResourceAllocation ::=  SEQUENCE {

k2                                      INTEGER(0..32)          OPTIONAL,   -- Need S

mappingType                             ENUMERATED {typeA, typeB},

startSymbolAndLength                    INTEGER (0..127)

}

-- TAG-PUSCH-TIMEDOMAINRESOURCEALLOCATIONLIST-STOP

-- ASN1STOP

PUSCH-TimeDomainResourceAllocationList field descriptions

k2 Corresponds to L1 parameter 'K2' (see TS 38.214 [19], clause 6.1.2.1) When the field is absent the UE applies the value 1 when PUSCH SCS is 15/30 kHz; the value 2 when PUSCH SCS is 60 kHz, and the value 3 when PUSCH SCS is 120KHz.

mappingType Mapping type (see TS 38.214 [19], clause 6.1.2.1).

startSymbolAndLength An index giving valid combinations of start symbol and length (jointly encoded) as start and length indicator (SLIV). The network configures the field so that the allocation does not cross the slot boundary. (see TS 38.214 [19], clause 6.1.2.1).

#### –	PUSCH-TPC-CommandConfig

The IE PUSCH-TPC-CommandConfig is used to configure the UE for extracting TPC commands for PUSCH from a group-TPC messages on DCI.

PUSCH-TPC-CommandConfig information element

-- ASN1START

-- TAG-PUSCH-TPC-COMMANDCONFIG-START

PUSCH-TPC-CommandConfig ::=         SEQUENCE {

tpc-Index                           INTEGER (1..15)                                                 OPTIONAL,   -- Cond SUL

tpc-IndexSUL                        INTEGER (1..15)                                                 OPTIONAL,   -- Cond SUL-Only

targetCell                          ServCellIndex                                                   OPTIONAL,   -- Need S

...

}

-- TAG-PUSCH-TPC-COMMANDCONFIG-STOP

-- ASN1STOP

PUSCH-TPC-CommandConfig field descriptions

targetCell The serving cell to which the acquired power control commands are applicable. If the value is absent, the UE applies the TPC commands to the serving cell on which the command has been received.

tpc-Index An index determining the position of the first bit of TPC command inside the DCI format 2-2 payload.

tpc-IndexSUL An index determining the position of the first bit of TPC command inside the DCI format 2-2 payload.

| Conditional Presence | Explanation |
| --- | --- |
| SUL-Only | The field is optionally present, Need R, if this serving cell is configured with a supplementary uplink (SUL). It is absent otherwise. |
| SUL | The field is optionally present, Need R, if this serving cell is configured with a supplementary uplink (SUL). It is mandatory present otherwise. |

#### –	Q-OffsetRange

The IE Q-OffsetRange is used to indicate a cell, beam or measurement object specific offset to be applied when evaluating candidates for cell re-selection or when evaluating triggering conditions for measurement reporting. The value is in dB. Value dB-24 corresponds to -24 dB, dB-22 corresponds to -22 dB and so on.

Q-OffsetRange information element

-- ASN1START

-- TAG-Q-OFFSETRANGE-START

Q-OffsetRange ::=                   ENUMERATED {

dB-24, dB-22, dB-20, dB-18, dB-16, dB-14,

dB-12, dB-10, dB-8, dB-6, dB-5, dB-4, dB-3,

dB-2, dB-1, dB0, dB1, dB2, dB3, dB4, dB5,

dB6, dB8, dB10, dB12, dB14, dB16, dB18,

dB20, dB22, dB24}

-- TAG-Q-OFFSETRANGE-STOP

-- ASN1STOP

#### –	Q-QualMin

The IE Q-QualMin is used to indicate for cell selection/ re-selection the required minimum received RSRQ level in the (NR) cell. Corresponds to parameter Qqualmin in TS 38.304 [20]. Actual value Qqualmin = field value [dB].

Q-QualMin information element

-- ASN1START

-- TAG-Q-QUALMIN-START

Q-QualMin ::=                       INTEGER (-43..-12)

-- TAG-Q-QUALMIN-STOP

-- ASN1STOP

#### –	Q-RxLevMin

The IE Q-RxLevMin is used to indicate for cell selection/ re-selection the required minimum received RSRP level in the (NR) cell. Corresponds to parameter Qrxlevmin in TS 38.304 [20]. Actual value Qrxlevmin = field value * 2 [dBm].

Q-RxLevMin information element

-- ASN1START

-- TAG-Q-RXLEVMIN-START

Q-RxLevMin ::=                      INTEGER (-70..-22)

-- TAG-Q-RXLEVMIN-STOP

-- ASN1STOP

#### –	QuantityConfig

The IE QuantityConfig specifies the measurement quantities and layer 3 filtering coefficients for NR and inter-RAT measurements.

QuantityConfig information element

-- ASN1START

-- TAG-QUANTITYCONFIG-START

QuantityConfig ::=                  SEQUENCE {

quantityConfigNR-List               SEQUENCE (SIZE (1..maxNrofQuantityConfig)) OF QuantityConfigNR          OPTIONAL,   -- Need M

...,

[[

quantityConfigEUTRA                 FilterConfig                                                            OPTIONAL    -- Need M

]]

}

QuantityConfigNR::=                 SEQUENCE {

quantityConfigCell                  QuantityConfigRS,

quantityConfigRS-Index              QuantityConfigRS                                                        OPTIONAL    -- Need M

}

QuantityConfigRS ::=                SEQUENCE {

ssb-FilterConfig                    FilterConfig,

csi-RS-FilterConfig                 FilterConfig

}

FilterConfig ::=                    SEQUENCE {

filterCoefficientRSRP               FilterCoefficient                                       DEFAULT fc4,

filterCoefficientRSRQ               FilterCoefficient                                       DEFAULT fc4,

filterCoefficientRS-SINR            FilterCoefficient                                       DEFAULT fc4

}

-- TAG-QUANTITYCONFIG-STOP

-- ASN1STOP

QuantityConfigNR field descriptions

quantityConfigCell Specifies L3 filter configurations for cell measurement results for the configurable RS Types (e.g. SS/PBCH block and CSI-RS) and the configurable measurement quantities (e.g. RSRP, RSRQ and SINR).

quantityConfigRS-Index Specifies L3 filter configurations for measurement results per RS index for the configurable RS Types (e.g. SS/PBCH block and CSI-RS) and the configurable measurement quantities (e.g. RSRP, RSRQ and SINR).

QuantityConfigRS field descriptions

csi-RS-FilterConfig CSI-RS based L3 filter configurations: Specifies L3 filter configurations for CSI-RSRP, CSI-RSRQ and CSI-SINR measurement results from the L1 filter(s), as defined in TS 38.215 [9].

ssb-FilterConfig SS Block based L3 filter configurations: Specifies L3 filter configurations for SS-RSRP, SS-RSRQ and SS-SINR measurement results from the L1 filter(s), as defined in TS 38.215 [9].

#### –	RACH-ConfigCommon

The IE RACH-ConfigCommon is used to specify the cell specific random-access parameters.

RACH-ConfigCommon information element

-- ASN1START

-- TAG-RACH-CONFIGCOMMON-START

RACH-ConfigCommon ::=               SEQUENCE {

rach-ConfigGeneric                  RACH-ConfigGeneric,

totalNumberOfRA-Preambles           INTEGER (1..63)                                                     OPTIONAL,   -- Need S

ssb-perRACH-OccasionAndCB-PreamblesPerSSB   CHOICE {

oneEighth                                   ENUMERATED {n4,n8,n12,n16,n20,n24,n28,n32,n36,n40,n44,n48,n52,n56,n60,n64},

oneFourth                                   ENUMERATED {n4,n8,n12,n16,n20,n24,n28,n32,n36,n40,n44,n48,n52,n56,n60,n64},

oneHalf                                     ENUMERATED {n4,n8,n12,n16,n20,n24,n28,n32,n36,n40,n44,n48,n52,n56,n60,n64},

one                                         ENUMERATED {n4,n8,n12,n16,n20,n24,n28,n32,n36,n40,n44,n48,n52,n56,n60,n64},

two                                         ENUMERATED {n4,n8,n12,n16,n20,n24,n28,n32},

four                                        INTEGER (1..16),

eight                                       INTEGER (1..8),

sixteen                                     INTEGER (1..4)

}                                                                                                       OPTIONAL,   -- Need M

groupBconfigured                    SEQUENCE {

ra-Msg3SizeGroupA                   ENUMERATED {b56, b144, b208, b256, b282, b480, b640,

b800, b1000, b72, spare6, spare5,spare4, spare3, spare2, spare1},

messagePowerOffsetGroupB            ENUMERATED { minusinfinity, dB0, dB5, dB8, dB10, dB12, dB15, dB18},

numberOfRA-PreamblesGroupA          INTEGER (1..64)

}                                                                                                       OPTIONAL,   -- Need R

ra-ContentionResolutionTimer            ENUMERATED { sf8, sf16, sf24, sf32, sf40, sf48, sf56, sf64},

rsrp-ThresholdSSB                       RSRP-Range                                                      OPTIONAL,   -- Need R

rsrp-ThresholdSSB-SUL                   RSRP-Range                                                      OPTIONAL,   -- Cond SUL

prach-RootSequenceIndex                 CHOICE {

l839                                    INTEGER (0..837),

l139                                    INTEGER (0..137)

},

msg1-SubcarrierSpacing                  SubcarrierSpacing                                               OPTIONAL,   -- Cond L139

restrictedSetConfig                     ENUMERATED {unrestrictedSet, restrictedSetTypeA, restrictedSetTypeB},

msg3-transformPrecoder                  ENUMERATED {enabled}                                            OPTIONAL,   -- Need R

...

}

-- TAG-RACH-CONFIGCOMMON-STOP

-- ASN1STOP

RACH-ConfigCommon field descriptions

messagePowerOffsetGroupB Threshold for preamble selection. Value is in dB. Value minusinfinity corresponds to –infinity. Value dB0 corresponds to 0 dB, dB5 corresponds to 5 dB and so on. (see TS 38.321 [3], clause 5.1.2)

msg1-SubcarrierSpacing Subcarrier spacing of PRACH (see TS 38.211 [16], clause 5.3.2). Only the values 15 or 30 kHz (FR1), and 60 or 120 kHz (FR2) are applicable. If absent, the UE applies the SCS as derived from the prach-ConfigurationIndex in RACH-ConfigGeneric (see tables Table 6.3.3.1-1 and Table 6.3.3.2-2, TS 38.211 [16]). The value also applies to contention free random access (RACH-ConfigDedicated), to SI-request and to contention-based beam failure recovery (CB-BFR). But it does not apply for contention free beam failure recovery (CF-BFR) (see BeamFailureRecoveryConfig).

msg3-transformPrecoder Enables the transform precoder for Msg3 transmission according to clause 6.1.3 of TS 38.214 [19]. If the field is absent, the UE disables the transformer precoder (see TS 38.213 [13], clause 8.3).

numberOfRA-PreamblesGroupA The number of CB preambles per SSB in group A. This determines implicitly the number of CB preambles per SSB available in group B. (see TS 38.321 [3], clause 5.1.1). The setting should be consistent with the setting of ssb-perRACH-OccasionAndCB-PreamblesPerSSB.

prach-RootSequenceIndex PRACH root sequence index (see TS 38.211 [16], clause 6.3.3.1). The value range depends on whether L=839 or L=139. The short/long preamble format indicated in this IE should be consistent with the one indicated in prach-ConfigurationIndex in the RACH-ConfigDedicated (if configured).

ra-ContentionResolutionTimer The initial value for the contention resolution timer (see TS 38.321 [3], clause 5.1.5). Value sf8 corresponds to 8 subframes, value sf16 corresponds to 16 subframes, and so on.

ra-Msg3SizeGroupA Transport Blocks size threshold in bits below which the UE shall use a contention-based RA preamble of group A. (see TS 38.321 [3], clause 5.1.2).

rach-ConfigGeneric RACH parameters for both regular random access and beam failure recovery.

restrictedSetConfig Configuration of an unrestricted set or one of two types of restricted sets, see TS 38.211 [16], clause 6.3.3.1.

rsrp-ThresholdSSB UE may select the SS block and corresponding PRACH resource for path-loss estimation and (re)transmission based on SS blocks that satisfy the threshold (see TS 38.213 [13]).

rsrp-ThresholdSSB-SUL The UE selects SUL carrier to perform random access based on this threshold (see TS 38.321 [3], clause 5.1.1). The value applies to all the BWPs.

ssb-perRACH-OccasionAndCB-PreamblesPerSSB The meaning of this field is twofold: the CHOICE conveys the information about the number of SSBs per RACH occasion. Value oneEighth corresponds to one SSB associated with 8 RACH occasions, value oneFourth corresponds to one SSB associated with 4 RACH occasions, and so on. The ENUMERATED part indicates the number of Contention Based preambles per SSB. Value n4 corresponds to 4 Contention Based preambles per SSB, value n8 corresponds to 8 Contention Based preambles per SSB, and so on. The total number of CB preambles in a RACH occasion is given by CB-preambles-per-SSB * max(1, SSB-per-rach-occasion). See TS 38.213 [13].

totalNumberOfRA-Preambles Total number of preambles used for contention based and contention free random access in the RACH resources defined in RACH-ConfigCommon, excluding preambles used for other purposes (e.g. for SI request). If the field is absent, all 64 preambles are available for RA. The setting should be consistent with the setting of ssb-perRACH-OccasionAndCB-PreamblesPerSSB, i.e. it should be a multiple of the number of SSBs per RACH occasion.

| Conditional Presence | Explanation |
| --- | --- |
| L139 | The field is mandatory present if prach-RootSequenceIndex L=139, otherwise the field is absent, Need S. |
| SUL | The field is mandatory present in initialUplinkBWP in supplementaryUplink; otherwise, the field is absent. |

#### –	RACH-ConfigDedicated

The IE RACH-ConfigDedicated is used to specify the dedicated random access parameters.

RACH-ConfigDedicated information element

-- ASN1START

-- TAG-RACH-CONFIGDEDICATED-START

RACH-ConfigDedicated ::=        SEQUENCE {

cfra                            CFRA                                                                    OPTIONAL, -- Need S

ra-Prioritization               RA-Prioritization                                                       OPTIONAL, -- Need N

...

}

CFRA ::=                    SEQUENCE {

occasions                       SEQUENCE {

rach-ConfigGeneric              RACH-ConfigGeneric,

ssb-perRACH-Occasion            ENUMERATED {oneEighth, oneFourth, oneHalf, one, two, four, eight, sixteen}

OPTIONAL  -- Cond Mandatory

}                                                                                                       OPTIONAL, -- Need S

resources                       CHOICE {

ssb                             SEQUENCE {

ssb-ResourceList                SEQUENCE (SIZE(1..maxRA-SSB-Resources)) OF CFRA-SSB-Resource,

ra-ssb-OccasionMaskIndex        INTEGER (0..15)

},

csirs                           SEQUENCE {

csirs-ResourceList              SEQUENCE (SIZE(1..maxRA-CSIRS-Resources)) OF CFRA-CSIRS-Resource,

rsrp-ThresholdCSI-RS            RSRP-Range

}

},

...,

[[

totalNumberOfRA-Preambles INTEGER (1..63)                                                         OPTIONAL -- Cond Occasions

]]

}

CFRA-SSB-Resource ::=           SEQUENCE {

ssb                             SSB-Index,

ra-PreambleIndex                INTEGER (0..63),

...

}

CFRA-CSIRS-Resource ::=         SEQUENCE {

csi-RS                          CSI-RS-Index,

ra-OccasionList                 SEQUENCE (SIZE(1..maxRA-OccasionsPerCSIRS)) OF INTEGER (0..maxRA-Occasions-1),

ra-PreambleIndex                INTEGER (0..63),

...

}

-- TAG-RACH-CONFIGDEDICATED-STOP

-- ASN1STOP

CFRA-CSIRS-Resource field descriptions

csi-RS The ID of a CSI-RS resource defined in the measurement object associated with this serving cell.

ra-OccasionList RA occasions that the UE shall use when performing CF-RA upon selecting the candidate beam identified by this CSI-RS. The network ensures that the RA occasion indexes provided herein are also configured by prach-ConfigurationIndex and msg1-FDM. Each RACH occasion is sequentially numbered, first, in increasing order of frequency resource indexes for frequency multiplexed PRACH occasions; second, in increasing order of time resource indexes for time multiplexed PRACH occasions within a PRACH slot and Third, in increasing order of indexes for PRACH slots.

ra-PreambleIndex The RA preamble index to use in the RA occasions associated with this CSI-RS.

CFRA field descriptions

occasions RA occasions for contention free random access. If the field is absent, the UE uses the RA occasions configured in RACH-ConfigCommon in the first active UL BWP.

ra-ssb-OccasionMaskIndex Explicitly signalled PRACH Mask Index for RA Resource selection in TS 38.321 [3]. The mask is valid for all SSB resources signalled in ssb-ResourceList.

rach-ConfigGeneric Configuration of contention free random access occasions for CFRA. The UE shall ignore preambleReceivedTargetPower, preambleTransMax, powerRampingStep, ra-ResponseWindow signaled within this field and use the corresponding values provided in RACH-ConfigCommon.

ssb-perRACH-Occasion Number of SSBs per RACH occasion.

totalNumberOfRA-Preambles Total number of preambles used for contention free random access in the RACH resources defined in CFRA, excluding preambles used for other purposes (e.g. for SI request). If the field is absent but the field occasions is present, the UE may assume all the 64 preambles are for RA. The setting should be consistent with the setting of ssb-perRACH-Occasion, if present, i.e. it should be a multiple of the number of SSBs per RACH occasion.

CFRA-SSB-Resource field descriptions

ra-PreambleIndex The preamble index that the UE shall use when performing CF-RA upon selecting the candidate beams identified by this SSB.

ssb The ID of an SSB transmitted by this serving cell.

RACH-ConfigDedicated field descriptions

cfra Parameters for contention free random access to a given target cell. If the field is absent, the UE performs contention based random access.

ra-prioritization Parameters which apply for prioritized random access procedure to a given target cell (see TS 38.321 [3], clause 5.1.1).

| Conditional Presence | Explanation |
| --- | --- |
| Mandatory | The field is mandatory present. |
| Occasions | The field is optionally present, Need S, if the field occasions is present, otherwise it is absent. |

#### –	RACH-ConfigGeneric

The IE RACH-ConfigGeneric is used to specify the random-access parameters both for regular random access as well as for beam failure recovery.

RACH-ConfigGeneric information element

-- ASN1START

-- TAG-RACH-CONFIGGENERIC-START

RACH-ConfigGeneric ::=              SEQUENCE {

prach-ConfigurationIndex            INTEGER (0..255),

msg1-FDM                            ENUMERATED {one, two, four, eight},

msg1-FrequencyStart                 INTEGER (0..maxNrofPhysicalResourceBlocks-1),

zeroCorrelationZoneConfig           INTEGER(0..15),

preambleReceivedTargetPower         INTEGER (-202..-60),

preambleTransMax                    ENUMERATED {n3, n4, n5, n6, n7, n8, n10, n20, n50, n100, n200},

powerRampingStep                    ENUMERATED {dB0, dB2, dB4, dB6},

ra-ResponseWindow                   ENUMERATED {sl1, sl2, sl4, sl8, sl10, sl20, sl40, sl80},

...

}

-- TAG-RACH-CONFIGGENERIC-STOP

-- ASN1STOP

RACH-ConfigGeneric field descriptions

msg1-FDM The number of PRACH transmission occasions FDMed in one time instance. (see TS 38.211 [16], clause 6.3.3.2).

msg1-FrequencyStart Offset of lowest PRACH transmission occasion in frequency domain with respective to PRB 0. The value is configured so that the corresponding RACH resource is entirely within the bandwidth of the UL BWP. (see TS 38.211 [16], clause 6.3.3.2).

powerRampingStep Power ramping steps for PRACH (see TS 38.321 [3],5.1.3).

prach-ConfigurationIndex PRACH configuration index. For prach-ConfigurationIndex configured under beamFailureRecovery-Config, the prach-ConfigurationIndex can only correspond to the short preamble format, (see TS 38.211 [16], clause 6.3.3.2).

preambleReceivedTargetPower The target power level at the network receiver side (see TS 38.213 [13], clause 7.4, TS 38.321 [3], clauses 5.1.2, 5.1.3). Only multiples of 2 dBm may be chosen (e.g. -202, -200, -198, ...).

preambleTransMax Max number of RA preamble transmission performed before declaring a failure (see TS 38.321 [3], clauses 5.1.4, 5.1.5).

ra-ResponseWindow Msg2 (RAR) window length in number of slots. The network configures a value lower than or equal to 10 ms (see TS 38.321 [3], clause 5.1.4). UE ignores the field if included in SCellConfig.

zeroCorrelationZoneConfig N-CS configuration, see Table 6.3.3.1-5 in TS 38.211 [16].

#### –	RA-Prioritization

The IE RA-Prioritization is used to configure prioritized random access.

RA-Prioritization information element

-- ASN1START

-- TAG-RA-PRIORITIZATION-START

RA-Prioritization ::=           SEQUENCE {

powerRampingStepHighPriority    ENUMERATED {dB0, dB2, dB4, dB6},

scalingFactorBI                 ENUMERATED {zero, dot25, dot5, dot75}                               OPTIONAL,   -- Need R

...

}

-- TAG-RA-PRIORITIZATION-STOP

-- ASN1STOP

RA-Prioritization field descriptions

powerRampingStepHighPrioritiy Power ramping step applied for prioritized random access procedure.

scalingFactorBI Scaling factor for the backoff indicator (BI) for the prioritized random access procedure. (see TS 38.321 [3], clause 5.1.4). Value zero corresponds to 0, value dot25 corresponds to 0.25 and so on.

#### –	RadioBearerConfig

The IE RadioBearerConfig is used to add, modify and release signalling and/or data radio bearers. Specifically, this IE carries the parameters for PDCP and, if applicable, SDAP entities for the radio bearers.

RadioBearerConfig information element

-- ASN1START

-- TAG-RADIOBEARERCONFIG-START

RadioBearerConfig ::=                   SEQUENCE {

srb-ToAddModList                        SRB-ToAddModList                                        OPTIONAL,   -- Cond HO-Conn

srb3-ToRelease                          ENUMERATED{true}                                        OPTIONAL,   -- Need N

drb-ToAddModList                        DRB-ToAddModList                                        OPTIONAL,   -- Cond HO-toNR

drb-ToReleaseList                       DRB-ToReleaseList                                       OPTIONAL,   -- Need N

securityConfig                          SecurityConfig                                          OPTIONAL,   -- Need M

...

}

SRB-ToAddModList ::=                    SEQUENCE (SIZE (1..2)) OF SRB-ToAddMod

SRB-ToAddMod ::=                        SEQUENCE {

srb-Identity                            SRB-Identity,

reestablishPDCP                         ENUMERATED{true}                                        OPTIONAL,   -- Need N

discardOnPDCP                           ENUMERATED{true}                                        OPTIONAL,   -- Need N

pdcp-Config                             PDCP-Config                                             OPTIONAL,   -- Cond PDCP

...

}

DRB-ToAddModList ::=                    SEQUENCE (SIZE (1..maxDRB)) OF DRB-ToAddMod

DRB-ToAddMod ::=                        SEQUENCE {

cnAssociation                           CHOICE {

eps-BearerIdentity                      INTEGER (0..15),

sdap-Config                             SDAP-Config

}                                                                                               OPTIONAL,   -- Cond DRBSetup

drb-Identity                            DRB-Identity,

reestablishPDCP                         ENUMERATED{true}                                        OPTIONAL,   -- Need N

recoverPDCP                             ENUMERATED{true}                                        OPTIONAL,   -- Need N

pdcp-Config                             PDCP-Config                                             OPTIONAL,   -- Cond PDCP

...

}

DRB-ToReleaseList ::=                   SEQUENCE (SIZE (1..maxDRB)) OF DRB-Identity

SecurityConfig ::=                      SEQUENCE {

securityAlgorithmConfig                 SecurityAlgorithmConfig                                 OPTIONAL,   -- Cond RBTermChange1

keyToUse                                ENUMERATED{master, secondary}                           OPTIONAL,   -- Cond RBTermChange

...

}

-- TAG-RADIOBEARERCONFIG-STOP

-- ASN1STOP

DRB-ToAddMod field descriptions

cnAssociation Indicates if the bearer is associated with the eps-bearerIdentity (when connected to EPC) or sdap-Config (when connected to 5GC).

drb-Identity In case of DC, the DRB identity is unique within the scope of the UE, i.e. an MCG DRB cannot use the same value as a split DRB. For a split DRB the same identity is used for the MCG and SCG parts of the configuration.

eps-BearerIdentity The EPS bearer ID determines the EPS bearer.

reestablishPDCP Indicates that PDCP should be re-established. Network sets this to true whenever the security key used for this radio bearer changes. Key change could for example be due to termination point change for the bearer, reconfiguration with sync, resuming an RRC connection, or the first reconfiguration after reestablishment. It is also applicable for LTE procedures when NR PDCP is configured.

recoverPDCP Indicates that PDCP should perform recovery according to TS 38.323 [5].

sdap-Config The SDAP configuration determines how to map QoS flows to DRBs when NR or E-UTRA connects to the 5GC and presence/absence of UL/DL SDAP headers.

RadioBearerConfig field descriptions

securityConfig Indicates the security algorithm and key to use for the signalling and data radio bearers configured with the list in this IE RadioBearerConfig. When the field is not included after AS security has been activated, the UE shall continue to use the currently configured keyToUse and security algorithm for the radio bearers reconfigured with the lists in this IE RadioBearerConfig. The field is not included when configuring SRB1 before AS security is activated.

srb3-ToRelease Release SRB3. SRB3 release can only be done over SRB1 and only at SCG release and reconfiguration with sync.

SecurityConfig field descriptions

keyToUse Indicates if the bearers configured with the list in this IE RadioBearerConfig are using the master key or the secondary key for deriving ciphering and/or integrity protection keys. For MR-DC, network should not configure SRB1 and SRB2 with secondary key and SRB3 with the master key. When the field is not included, the UE shall continue to use the currently configured keyToUse for the radio bearers reconfigured with the lists in this IE RadioBearerConfig.

securityAlgorithmConfig Indicates the security algorithm for the signalling and data radio bearers configured with the list in this IE RadioBearerConfig. When the field is not included, the UE shall continue to use the currently configured security algorithm for the radio bearers reconfigured with the lists in this IE RadioBearerConfig.

SRB-ToAddMod field descriptions

discardOnPDCP Indicates that PDCP should discard stored SDU and PDU according to TS 38.323 [5].

reestablishPDCP Indicates that PDCP should be re-established. Network sets this to true whenever the security key used for this radio bearer changes. Key change could for example be due to reconfiguration with sync, for SRB2 when resuming an RRC connection, or at the first reconfiguration after RRC connection reestablishment in NR. For LTE SRBs using NR PDCP, it could be for handover, RRC connection reestablishment or resume.

srb-Identity Value 1 is applicable for SRB1 only. Value 2 is applicable for SRB2 only. Value 3 is applicable for SRB3 only.

| Conditional Presence | Explanation |
| --- | --- |
| RBTermChange | The field is mandatory present in case of set up of signalling and data radio bearer and change of termination point for the radio bearer between MN and SN. It is optionally present otherwise, Need S. |
| RBTermChange1 | The field is mandatory present in case of: - set up of signalling and data radio bearer, - change of termination point for the radio bearer between MN and SN, - handover from E-UTRA/EPC or E-UTRA/5GC to NR, - handover from NR or E-UTRA/EPC to E-UTRA/5GC if the UE supports NGEN-DC. It is optionally present otherwise, Need S. |
| PDCP | The field is mandatory present if the corresponding DRB is being setup or corresponding DRB is reconfigured with NR PDCP or corresponding SRB associated with two RLC entities is being setup or if the number of RLC bearers associated with the DRB or SRB is changed. The field is optionally present, Need S, if the corresponding SRB associated with one RLC entity is being setup or corresponding SRB is reconfigured with NR PDCP; otherwise the field is optionally present, need M. |
| DRBSetup | The field is mandatory present if the corresponding DRB is being setup; otherwise the field is optionally present, need M. |
| HO-Conn | The field is mandatory present - in case of inter-system handover from E-UTRA/EPC to E-UTRA/5GC or NR, - or when the fullConfig is included in the RRCReconfiguration message and NE-DC/NR-DC is not configured, - or in case of RRCSetup. Otherwise the field is optionally present, need N. Upon RRCSetup, only SRB1 can be present. |
| HO-toNR | The field is mandatory present - in case of inter-system handover from E-UTRA/EPC to E-UTRA/5GC or NR, - or when the fullConfig is included in the RRCReconfiguration message and NE-DC/NR-DC is not configured. In case of RRCSetup, the field is absent; otherwise the field is optionally present, need N. |

#### –	RadioLinkMonitoringConfig

The IE RadioLinkMonitoringConfig is used to configure radio link monitoring for detection of beam- and/or cell radio link failure. See also TS 38.321 [3], clause 5.1.1.

RadioLinkMonitoringConfig information element

-- ASN1START

-- TAG-RADIOLINKMONITORINGCONFIG-START

RadioLinkMonitoringConfig ::=       SEQUENCE {

failureDetectionResourcesToAddModList   SEQUENCE (SIZE(1..maxNrofFailureDetectionResources)) OF RadioLinkMonitoringRS

OPTIONAL, -- Need N

failureDetectionResourcesToReleaseList  SEQUENCE (SIZE(1..maxNrofFailureDetectionResources)) OF RadioLinkMonitoringRS-Id

OPTIONAL, -- Need N

beamFailureInstanceMaxCount             ENUMERATED {n1, n2, n3, n4, n5, n6, n8, n10}                          OPTIONAL, -- Need R

beamFailureDetectionTimer               ENUMERATED {pbfd1, pbfd2, pbfd3, pbfd4, pbfd5, pbfd6, pbfd8, pbfd10}  OPTIONAL, -- Need R

...

}

RadioLinkMonitoringRS ::=           SEQUENCE {

radioLinkMonitoringRS-Id            RadioLinkMonitoringRS-Id,

purpose                             ENUMERATED {beamFailure, rlf, both},

detectionResource                   CHOICE {

ssb-Index                           SSB-Index,

csi-RS-Index                        NZP-CSI-RS-ResourceId

},

...

}

-- TAG-RADIOLINKMONITORINGCONFIG-STOP

-- ASN1STOP

RadioLinkMonitoringConfig field descriptions

beamFailureDetectionTimer Timer for beam failure detection (see TS 38.321 [3], clause 5.17). See also the BeamFailureRecoveryConfig IE. Value in number of "Qout,LR reporting periods of Beam Failure Detection" Reference Signal (see TS 38.213 [13], clause 6). Value pbfd1 corresponds to 1 Qout,LR reporting period of Beam Failure Detection Reference Signal, value pbfd2 corresponds to 2 Qout,LR reporting periods of Beam Failure Detection Reference Signal and so on.

beamFailureInstanceMaxCount This field determines after how many beam failure events the UE triggers beam failure recovery (see TS 38.321 [3], clause 5.17). Value n1 corresponds to 1 beam failure instance, value n2 corresponds to 2 beam failure instances and so on.

failureDetectionResourcesToAddModList A list of reference signals for detecting beam failure and/or cell level radio link failure (RLF). The limits of the reference signals that the network can configure are specified in TS 38.213 [13], table 5-1. The network configures at most two detectionResources per BWP for the purpose beamFailure or both. If no RSs are provided for the purpose of beam failure detection, the UE performs beam monitoring based on the activated TCI-State for PDCCH as described in TS 38.213 [13], clause 6. If no RSs are provided in this list for the purpose of RLF detection, the UE performs Cell-RLM based on the activated TCI-State of PDCCH as described in TS 38.213 [13], clause 5. The network ensures that the UE has a suitable set of reference signals for performing cell-RLM.

RadioLinkMonitoringRS field descriptions

detectionResource A reference signal that the UE shall use for radio link monitoring or beam failure detection (depending on the indicated purpose).

purpose Determines whether the UE shall monitor the associated reference signal for the purpose of cell- and/or beam failure detection.

#### –	RadioLinkMonitoringRS-Id

The IE RadioLinkMonitoringRS-Id is used to identify one RadioLinkMonitoringRS.

RadioLinkMonitoringRS-Id information element

-- ASN1START

-- TAG-RADIOLINKMONITORINGRS-ID-START

RadioLinkMonitoringRS-Id ::=            INTEGER (0..maxNrofFailureDetectionResources-1)

-- TAG-RADIOLINKMONITORINGRS-ID-STOP

-- ASN1STOP

#### –	RAN-AreaCode

The IE RAN-AreaCode is used to identify a RAN area within the scope of a tracking area.

RAN-AreaCode information element

-- ASN1START

-- TAG-RAN-AREACODE-START

RAN-AreaCode ::=                INTEGER (0..255)

-- TAG-RAN-AREACODE-STOP

-- ASN1STOP

#### –	RateMatchPattern

The IE RateMatchPattern is used to configure one rate matching pattern for PDSCH, see TS 38.214 [19], clause 5.1.4.1.

RateMatchPattern information element

-- ASN1START

-- TAG-RATEMATCHPATTERN-START

RateMatchPattern ::=                SEQUENCE {

rateMatchPatternId                  RateMatchPatternId,

patternType                         CHOICE {

bitmaps                             SEQUENCE {

resourceBlocks                      BIT STRING (SIZE (275)),

symbolsInResourceBlock              CHOICE {

oneSlot                             BIT STRING (SIZE (14)),

twoSlots                            BIT STRING (SIZE (28))

},

periodicityAndPattern               CHOICE {

n2                                  BIT STRING (SIZE (2)),

n4                                  BIT STRING (SIZE (4)),

n5                                  BIT STRING (SIZE (5)),

n8                                  BIT STRING (SIZE (8)),

n10                                 BIT STRING (SIZE (10)),

n20                                 BIT STRING (SIZE (20)),

n40                                 BIT STRING (SIZE (40))

}                                                                                           OPTIONAL,   -- Need S

...

},

controlResourceSet                  ControlResourceSetId

},

subcarrierSpacing                   SubcarrierSpacing                                               OPTIONAL,   -- Cond CellLevel

dummy                               ENUMERATED { dynamic, semiStatic },

...

}

-- TAG-RATEMATCHPATTERN-STOP

-- ASN1STOP

RateMatchPattern field descriptions

bitmaps Indicates rate matching pattern by a pair of bitmaps resourceBlocks and symbolsInResourceBlock to define the rate match pattern within one or two slots, and a third bitmap periodicityAndPattern to define the repetition pattern with which the pattern defined by the above bitmap pair occurs.

controlResourceSet This ControlResourceSet is used as a PDSCH rate matching pattern, i.e., PDSCH reception rate matches around it. In frequency domain, the resource is determined by the frequency domain resource of the CORESET with the corresponding CORESET ID. Time domain resource is determined by the parameters of the associated search space of the CORESET.

periodicityAndPattern A time domain repetition pattern at which the pattern defined by symbolsInResourceBlock and resourceBlocks recurs. This slot pattern repeats itself continuously. Absence of this field indicates the value n1, i.e., the symbolsInResourceBlock recurs every 14 symbols (see TS 38.214 [19], clause 5.1.4.1).

resourceBlocks A resource block level bitmap in the frequency domain. A bit in the bitmap set to 1 indicates that the UE shall apply rate matching in the corresponding resource block in accordance with the symbolsInResourceBlock bitmap. If used as cell-level rate matching pattern, the bitmap identifies "common resource blocks (CRB)". If used as BWP-level rate matching pattern, the bitmap identifies "physical resource blocks" inside the BWP. The first/ leftmost bit corresponds to resource block 0, and so on (see TS 38.214 [19], clause 5.1.4.1).

subcarrierSpacing The SubcarrierSpacing for this resource pattern. If the field is absent, the UE applies the SCS of the associated BWP. The value kHz15 corresponds to µ=0, the value kHz30 corresponds to µ=1, and so on. Only the values 15 kHz, 30 kHz or 60 kHz (FR1), and 60 kHz or 120 kHz (FR2) are applicable (see TS 38.214 [19], clause 5.1.4.1).

symbolsInResourceBlock A symbol level bitmap in time domain. It indicates with a bit set to true that the UE shall rate match around the corresponding symbol. This pattern recurs (in time domain) with the configured periodicityAndPattern (see TS 38.214 [19], clause 5.1.4.1). For oneSlot, if ECP is configured, the first 12 bits represent the symbols within the slot and the last two bits within the bitstring are ignored by the UE; Otherwise, the 14 bits represent the symbols within the slot. For twoSlots, if ECP is configured, the first 12 bits represent the symbols within the first slot and the next 12 bits represent the symbols in the second slot and the last four bits within the bit string are ignored by the UE; Otherwise, the first 14 bits represent the symbols within the first slot and the next 14 bits represent the symbols in the second slot. For the bits representing symbols in a slot, the most significant bit of the bit string represents the first symbol in the slot and the second most significant bit represents the second symbol in the slot and so on.

| Conditional Presence | Explanation |
| --- | --- |
| CellLevel | The field is mandatory present if the RateMatchPattern is defined on cell level. The field is absent when the RateMatchPattern is defined on BWP level. If the RateMatchPattern is defined on BWP level, the UE applies the SCS of the BWP. |

#### –	RateMatchPatternId

The IE RateMatchPatternId identifies one RateMatchMattern (see TS 38.214 [19], clause 5.1.4.2).

RateMatchPatternId information element

-- ASN1START

-- TAG-RATEMATCHPATTERNID-START

RateMatchPatternId ::=              INTEGER (0..maxNrofRateMatchPatterns-1)

-- TAG-RATEMATCHPATTERNID-STOP

-- ASN1STOP

#### –	RateMatchPatternLTE-CRS

The IE RateMatchPatternLTE-CRS is used to configure a pattern to rate match around LTE CRS. See TS 38.214 [19], clause 5.1.4.2.

RateMatchPatternLTE-CRS information element

-- ASN1START

-- TAG-RATEMATCHPATTERNLTE-CRS-START

RateMatchPatternLTE-CRS ::=         SEQUENCE {

carrierFreqDL                       INTEGER (0..16383),

carrierBandwidthDL                  ENUMERATED {n6, n15, n25, n50, n75, n100, spare2, spare1},

mbsfn-SubframeConfigList            EUTRA-MBSFN-SubframeConfigList                                          OPTIONAL,   -- Need M

nrofCRS-Ports                       ENUMERATED {n1, n2, n4},

v-Shift                             ENUMERATED {n0, n1, n2, n3, n4, n5}

}

-- TAG-RATEMATCHPATTERNLTE-CRS-STOP

-- ASN1STOP

RateMatchPatternLTE-CRS field descriptions

carrierBandwidthDL BW of the LTE carrier in number of PRBs (see TS 38.214 [19], clause 5.1.4.2).

carrierFreqDL Center of the LTE carrier (see TS 38.214 [19], clause 5.1.4.2).

mbsfn-SubframeConfigList LTE MBSFN subframe configuration (see TS 38.214 [19], clause 5.1.4.2).

nrofCRS-Ports Number of LTE CRS antenna port to rate-match around (see TS 38.214 [19], clause 5.1.4.2).

v-Shift Shifting value v-shift in LTE to rate match around LTE CRS (see TS 38.214 [19], clause 5.1.4.2).

#### –	RejectWaitTime

The IE RejectWaitTime is used to provide the value in seconds for timer T302.

RejectWaitTime information element

-- ASN1START

-- TAG-REJECTWAITTIME-START

RejectWaitTime ::=                  INTEGER (1..16)

-- TAG-REJECTWAITTIME-STOP

-- ASN1STOP

#### –	ReportConfigId

The IE ReportConfigId is used to identify a measurement reporting configuration.

ReportConfigId information element

-- ASN1START

-- TAG-REPORTCONFIGID-START

ReportConfigId ::=                          INTEGER (1..maxReportConfigId)

-- TAG-REPORTCONFIGID-STOP

-- ASN1STOP

#### –	ReportConfigInterRAT

The IE ReportConfigInterRAT specifies criteria for triggering of an inter-RAT measurement reporting event. The inter-RAT measurement reporting events for E-UTRA are labelled BN with N equal to 1, 2 and so on.

Event B1:	Neighbour becomes better than absolute threshold;

Event B2:	PCell becomes worse than absolute threshold1 AND Neighbour becomes better than another absolute threshold2;

ReportConfigInterRAT information element

-- ASN1START

-- TAG-REPORTCONFIGINTERRAT-START

ReportConfigInterRAT ::=                    SEQUENCE {

reportType                                  CHOICE {

periodical                                  PeriodicalReportConfigInterRAT,

eventTriggered                              EventTriggerConfigInterRAT,

reportCGI                                   ReportCGI-EUTRA,

...,

reportSFTD                                  ReportSFTD-EUTRA

}

}

ReportCGI-EUTRA ::=                         SEQUENCE {

cellForWhichToReportCGI         EUTRA-PhysCellId,

...

}

ReportSFTD-EUTRA ::=                     SEQUENCE {

reportSFTD-Meas                            BOOLEAN,

reportRSRP                                 BOOLEAN,

...

}

EventTriggerConfigInterRAT ::=              SEQUENCE {

eventId                                     CHOICE {

eventB1                                     SEQUENCE {

b1-ThresholdEUTRA                           MeasTriggerQuantityEUTRA,

reportOnLeave                               BOOLEAN,

hysteresis                                  Hysteresis,

timeToTrigger                               TimeToTrigger,

...

},

eventB2                                     SEQUENCE {

b2-Threshold1                               MeasTriggerQuantity,

b2-Threshold2EUTRA                          MeasTriggerQuantityEUTRA,

reportOnLeave                               BOOLEAN,

hysteresis                                  Hysteresis,

timeToTrigger                               TimeToTrigger,

...

},

...

},

rsType                              NR-RS-Type,

reportInterval                      ReportInterval,

reportAmount                        ENUMERATED {r1, r2, r4, r8, r16, r32, r64, infinity},

reportQuantity                      MeasReportQuantity,

maxReportCells                      INTEGER (1..maxCellReport),

...

}

PeriodicalReportConfigInterRAT ::=              SEQUENCE {

reportInterval                                  ReportInterval,

reportAmount                                    ENUMERATED {r1, r2, r4, r8, r16, r32, r64, infinity},

reportQuantity                                  MeasReportQuantity,

maxReportCells                                  INTEGER (1..maxCellReport),

...

}

-- TAG-REPORTCONFIGINTERRAT-STOP

-- ASN1STOP

ReportConfigInterRAT field descriptions

reportType Type of the configured measurement report. In EN-DC, network does not configure report of type ReportCGI-EUTRA.

EventTriggerConfigInterRAT field descriptions

b2-Threshold1 NR threshold to be used in inter RAT measurement report triggering condition for event B2.

bN-ThresholdEUTRA E-UTRA threshold value associated with the selected trigger quantity (RSRP, RSRQ, SINR) to be used in inter RAT measurement report triggering condition for event number bN. In the same eventB2, the network configures the same CHOICE name (rsrp, rsrq or sinr) for the MeasTriggerQuantity of the b2-Threshold1 and for the MeasTriggerQuantityEUTRA of the b2-Threshold2EUTRA.

eventId Choice of inter RAT event triggered reporting criteria.

maxReportCells Max number of non-serving cells to include in the measurement report.

reportAmount Number of measurement reports applicable for eventTriggered as well as for periodical report types

reportOnLeave Indicates whether or not the UE shall initiate the measurement reporting procedure when the leaving condition is met for a cell in cellsTriggeredList, as specified in 5.5.4.1.

reportQuantity The cell measurement quantities to be included in the measurement report.

timeToTrigger Time during which specific criteria for the event needs to be met in order to trigger a measurement report.

PeriodicalReportConfigInterRAT field descriptions

maxReportCells Max number of non-serving cells to include in the measurement report.

reportAmount Number of measurement reports applicable for eventTriggered as well as for periodical report types

reportQuantity The cell measurement quantities to be included in the measurement report.

#### –	ReportConfigNR

The IE ReportConfigNR specifies criteria for triggering of an NR measurement reporting event. Measurement reporting events are based on cell measurement results, which can either be derived based on SS/PBCH block or CSI-RS. These events are labelled AN with N equal to 1, 2 and so on.

Event A1:	Serving becomes better than absolute threshold;

Event A2:	Serving becomes worse than absolute threshold;

Event A3:	Neighbour becomes amount of offset better than PCell/PSCell;

Event A4:	Neighbour becomes better than absolute threshold;

Event A5:	PCell/PSCell becomes worse than absolute threshold1 AND Neighbour/SCell becomes better than another absolute threshold2;

Event A6:	Neighbour becomes amount of offset better than SCell.

ReportConfigNR information element

-- ASN1START

-- TAG-REPORTCONFIGNR-START

ReportConfigNR ::=                          SEQUENCE {

reportType                                  CHOICE {

periodical                                  PeriodicalReportConfig,

eventTriggered                              EventTriggerConfig,

...,

reportCGI                                   ReportCGI,

reportSFTD                                  ReportSFTD-NR

}

}

ReportCGI ::=                     SEQUENCE {

cellForWhichToReportCGI          PhysCellId,

...

}

ReportSFTD-NR ::=                 SEQUENCE {

reportSFTD-Meas                  BOOLEAN,

reportRSRP                       BOOLEAN,

...,

[[

reportSFTD-NeighMeas             ENUMERATED {true}                                OPTIONAL,   -- Need R

drx-SFTD-NeighMeas               ENUMERATED {true}                                OPTIONAL,   -- Need R

cellsForWhichToReportSFTD        SEQUENCE (SIZE (1..maxCellSFTD)) OF PhysCellId   OPTIONAL    -- Need R

]]

}

EventTriggerConfig::=                       SEQUENCE {

eventId                                     CHOICE {

eventA1                                     SEQUENCE {

a1-Threshold                                MeasTriggerQuantity,

reportOnLeave                               BOOLEAN,

hysteresis                                  Hysteresis,

timeToTrigger                               TimeToTrigger

},

eventA2                                     SEQUENCE {

a2-Threshold                                MeasTriggerQuantity,

reportOnLeave                               BOOLEAN,

hysteresis                                  Hysteresis,

timeToTrigger                               TimeToTrigger

},

eventA3                                     SEQUENCE {

a3-Offset                                   MeasTriggerQuantityOffset,

reportOnLeave                               BOOLEAN,

hysteresis                                  Hysteresis,

timeToTrigger                               TimeToTrigger,

useWhiteCellList                            BOOLEAN

},

eventA4                                     SEQUENCE {

a4-Threshold                                MeasTriggerQuantity,

reportOnLeave                               BOOLEAN,

hysteresis                                  Hysteresis,

timeToTrigger                               TimeToTrigger,

useWhiteCellList                            BOOLEAN

},

eventA5                                     SEQUENCE {

a5-Threshold1                               MeasTriggerQuantity,

a5-Threshold2                               MeasTriggerQuantity,

reportOnLeave                               BOOLEAN,

hysteresis                                  Hysteresis,

timeToTrigger                               TimeToTrigger,

useWhiteCellList                            BOOLEAN

},

eventA6                                     SEQUENCE {

a6-Offset                                   MeasTriggerQuantityOffset,

reportOnLeave                               BOOLEAN,

hysteresis                                  Hysteresis,

timeToTrigger                               TimeToTrigger,

useWhiteCellList                            BOOLEAN

},

...

},

rsType                                      NR-RS-Type,

reportInterval                              ReportInterval,

reportAmount                                ENUMERATED {r1, r2, r4, r8, r16, r32, r64, infinity},

reportQuantityCell                          MeasReportQuantity,

maxReportCells                              INTEGER (1..maxCellReport),

reportQuantityRS-Indexes                     MeasReportQuantity                                            OPTIONAL,   -- Need R

maxNrofRS-IndexesToReport                   INTEGER (1..maxNrofIndexesToReport)                            OPTIONAL,   -- Need R

includeBeamMeasurements                     BOOLEAN,

reportAddNeighMeas                          ENUMERATED {setup}                                             OPTIONAL,   -- Need R

...

}

PeriodicalReportConfig ::=                  SEQUENCE {

rsType                                      NR-RS-Type,

reportInterval                              ReportInterval,

reportAmount                                ENUMERATED {r1, r2, r4, r8, r16, r32, r64, infinity},

reportQuantityCell                          MeasReportQuantity,

maxReportCells                              INTEGER (1..maxCellReport),

reportQuantityRS-Indexes                    MeasReportQuantity                                             OPTIONAL,   -- Need R

maxNrofRS-IndexesToReport                   INTEGER (1..maxNrofIndexesToReport)                            OPTIONAL,   -- Need R

includeBeamMeasurements                     BOOLEAN,

useWhiteCellList                            BOOLEAN,

...

}

NR-RS-Type ::=                              ENUMERATED {ssb, csi-rs}

MeasTriggerQuantity ::=                     CHOICE {

rsrp                                        RSRP-Range,

rsrq                                        RSRQ-Range,

sinr                                        SINR-Range

}

MeasTriggerQuantityOffset ::=               CHOICE {

rsrp                                        INTEGER (-30..30),

rsrq                                        INTEGER (-30..30),

sinr                                        INTEGER (-30..30)

}

MeasReportQuantity ::=                      SEQUENCE {

rsrp                                        BOOLEAN,

rsrq                                        BOOLEAN,

sinr                                        BOOLEAN

}

-- TAG-REPORTCONFIGNR-STOP

-- ASN1STOP

ReportConfigNR field descriptions

reportType Type of the configured measurement report. In EN-DC, network does not configure report of type reportCGI using SRB3.

EventTriggerConfig field descriptions

a3-Offset/a6-Offset Offset value(s) to be used in NR measurement report triggering condition for event a3/a6. The actual value is field value * 0.5 dB.

aN-ThresholdM Threshold value associated to the selected trigger quantity (e.g. RSRP, RSRQ, SINR) per RS Type (e.g. SS/PBCH block, CSI-RS) to be used in NR measurement report triggering condition for event number aN. If multiple thresholds are defined for event number aN, the thresholds are differentiated by M. The network configures aN-Threshold1 only for events A1, A2, A4, A5 and a5-Threshold2 only for event A5. In the same eventA5, the network configures the same quantity for the MeasTriggerQuantity of the a5-Threshold1 and for the MeasTriggerQuantity of the a5-Threshold2.

eventId Choice of NR event triggered reporting criteria.

maxNrofRS-IndexesToReport Max number of RS indexes to include in the measurement report for A1-A6 events.

maxReportCells Max number of non-serving cells to include in the measurement report.

reportAddNeighMeas Indicates that the UE shall include the best neighbour cells per serving frequency.

reportAmount Number of measurement reports applicable for eventTriggered as well as for periodical report types.

reportOnLeave Indicates whether or not the UE shall initiate the measurement reporting procedure when the leaving condition is met for a cell in cellsTriggeredList, as specified in 5.5.4.1.

reportQuantityCell The cell measurement quantities to be included in the measurement report.

reportQuantityRS-Indexes Indicates which measurement information per RS index the UE shall include in the measurement report.

timeToTrigger Time during which specific criteria for the event needs to be met in order to trigger a measurement report.

useWhiteCellList Indicates whether only the cells included in the white-list of the associated measObject are applicable as specified in 5.5.4.1.

PeriodicalReportConfig field descriptions

maxNrofRS-IndexesToReport Max number of RS indexes to include in the measurement report.

maxReportCells Max number of non-serving cells to include in the measurement report.

reportAmount Number of measurement reports applicable for eventTriggered as well as for periodical report types

reportQuantityCell The cell measurement quantities to be included in the measurement report.

reportQuantityRS-Indexes Indicates which measurement information per RS index the UE shall include in the measurement report.

useWhiteCellList Indicates whether only the cells included in the white-list of the associated measObject are applicable as specified in 5.5.4.1.

ReportSFTD-NR field descriptions

cellForWhichToReportSFTD Indicates the target NR neighbour cells for SFTD measurement between PCell and NR neighbour cells.

drx-SFTD-NeighMeas Indicates that the UE shall use available idle periods (i.e. DRX off periods) for the SFTD measurement in NR standalone. The network only includes drx-SFTD-NeighMeas field when reprtSFTD-NeighMeas is set to true.

reportSFTD-Meas Indicates whether UE is required to perform SFTD measurement between PCell and NR PSCell in NR-DC.

reportSFTD-NeighMeas Indicates whether UE is required to perform SFTD measurement between PCell and NR neighbour cells in NR standalone. The network does not include this field if reportSFTD-Meas is set to true.

reportRSRP Indicates whether UE is required to include RSRP result of NR PSCell or NR neighbour cells in SFTD measurement result, derived based on SSB. If it is set to true, the network should ensure that ssb-ConfigMobility is included in the measurement object for NR PSCell or NR neighbour cells.

#### –	ReportConfigToAddModList

The IE ReportConfigToAddModList concerns a list of reporting configurations to add or modify.

ReportConfigToAddModList information element

-- ASN1START

-- TAG-REPORTCONFIGTOADDMODLIST-START

ReportConfigToAddModList ::=        SEQUENCE (SIZE (1..maxReportConfigId)) OF ReportConfigToAddMod

ReportConfigToAddMod ::=            SEQUENCE {

reportConfigId                      ReportConfigId,

reportConfig                        CHOICE {

reportConfigNR                      ReportConfigNR,

...,

reportConfigInterRAT                ReportConfigInterRAT

}

}

-- TAG-REPORTCONFIGTOADDMODLIST-STOP

-- ASN1STOP

#### –	ReportInterval

The IE ReportInterval indicates the interval between periodical reports. The ReportInterval is applicable if the UE performs periodical reporting (i.e. when reportAmount exceeds 1), for triggerTypeevent as well as for triggerTypeperiodical. Value ms120 corresponds to 120 ms, value ms240 corresponds to 240 ms and so on, while value min1 corresponds to 1 min, min6 corresponds to 6 min and so on.

ReportInterval information element

-- ASN1START

-- TAG-REPORTINTERVAL-START

ReportInterval ::=                  ENUMERATED {ms120, ms240, ms480, ms640, ms1024, ms2048, ms5120, ms10240, ms20480, ms40960,

min1,min6, min12, min30 }

-- TAG-REPORTINTERVAL-STOP

-- ASN1STOP

#### –	ReselectionThreshold

The IE ReselectionThreshold is used to indicate an Rx level threshold for cell reselection. Actual value of threshold = field value * 2 [dB].

ReselectionThreshold information element

-- ASN1START

-- TAG-RESELECTIONTHRESHOLD-START

ReselectionThreshold ::=                INTEGER (0..31)

-- TAG-RESELECTIONTHRESHOLD-STOP

-- ASN1STOP

#### –	ReselectionThresholdQ

The IE ReselectionThresholdQ is used to indicate a quality level threshold for cell reselection. Actual value of threshold = field value [dB].

ReselectionThresholdQ information element

-- ASN1START

-- TAG-RESELECTIONTHRESHOLDQ-START

ReselectionThresholdQ ::=           INTEGER (0..31)

-- TAG-RESELECTIONTHRESHOLDQ-STOP

-- ASN1STOP

#### –	ResumeCause

The IE ResumeCause is used to indicate the resume cause in RRCResumeRequest and RRCResumeRequest1.

ResumeCause information element

-- ASN1START

-- TAG-RESUMECAUSE-START

ResumeCause ::=             ENUMERATED {emergency, highPriorityAccess, mt-Access, mo-Signalling,

mo-Data, mo-VoiceCall, mo-VideoCall, mo-SMS, rna-Update, mps-PriorityAccess,

mcs-PriorityAccess, spare1, spare2, spare3, spare4, spare5 }

-- TAG-RESUMECAUSE-STOP

-- ASN1STOP

#### –	RLC-BearerConfig

The IE RLC-BearerConfig is used to configure an RLC entity, a corresponding logical channel in MAC and the linking to a PDCP entity (served radio bearer).

RLC-BearerConfig information element

-- ASN1START

-- TAG-RLC-BEARERCONFIG-START

RLC-BearerConfig ::=                        SEQUENCE {

logicalChannelIdentity                      LogicalChannelIdentity,

servedRadioBearer                           CHOICE {

srb-Identity                                SRB-Identity,

drb-Identity                                DRB-Identity

}                                                                                               OPTIONAL,   -- Cond LCH-SetupOnly

reestablishRLC                              ENUMERATED {true}                                   OPTIONAL,   -- Need N

rlc-Config                                  RLC-Config                                          OPTIONAL,   -- Cond LCH-Setup

mac-LogicalChannelConfig                    LogicalChannelConfig                                OPTIONAL,   -- Cond LCH-Setup

...

}

-- TAG-RLC-BEARERCONFIG-STOP

-- ASN1STOP

RLC-BearerConfig field descriptions

logicalChannelIdentity ID used commonly for the MAC logical channel and for the RLC bearer.

reestablishRLC Indicates that RLC should be re-established. Network sets this to true at least whenever the security key used for the radio bearer associated with this RLC entity changes. For SRB2 and DRBs, it is also set to true during the resumption of the RRC connection or the first reconfiguration after reestablishment.

rlc-Config Determines the RLC mode (UM, AM) and provides corresponding parameters. RLC mode reconfiguration can only be performed by DRB release/addition or full configuration.

servedRadioBearer Associates the RLC Bearer with an SRB or a DRB. The UE shall deliver DL RLC SDUs received via the RLC entity of this RLC bearer to the PDCP entity of the servedRadioBearer. Furthermore, the UE shall advertise and deliver uplink PDCP PDUs of the uplink PDCP entity of the servedRadioBearer to the uplink RLC entity of this RLC bearer unless the uplink scheduling restrictions (moreThanOneRLC in PDCP-Config and the restrictions in LogicalChannelConfig) forbid it to do so.

| Conditional Presence | Explanation |
| --- | --- |
| LCH-Setup | This field is mandatory present upon creation of a new logical channel for a DRB. This field is optionally present, Need S, upon creation of a new logical channel for an SRB. It is optionally present, Need M, otherwise. |
| LCH-SetupOnly | This field is mandatory present upon creation of a new logical channel. It is absent, Need M otherwise. |

#### –	RLC-Config

The IE RLC-Config is used to specify the RLC configuration of SRBs and DRBs.

RLC-Config information element

-- ASN1START

-- TAG-RLC-CONFIG-START

RLC-Config ::=                      CHOICE {

am                                  SEQUENCE {

ul-AM-RLC                           UL-AM-RLC,

dl-AM-RLC                           DL-AM-RLC

},

um-Bi-Directional                   SEQUENCE {

ul-UM-RLC                           UL-UM-RLC,

dl-UM-RLC                           DL-UM-RLC

},

um-Uni-Directional-UL               SEQUENCE {

ul-UM-RLC                           UL-UM-RLC

},

um-Uni-Directional-DL               SEQUENCE {

dl-UM-RLC                           DL-UM-RLC

},

...

}

UL-AM-RLC ::=                       SEQUENCE {

sn-FieldLength                      SN-FieldLengthAM                                    OPTIONAL,   -- Cond Reestab

t-PollRetransmit                    T-PollRetransmit,

pollPDU                             PollPDU,

pollByte                            PollByte,

maxRetxThreshold                    ENUMERATED { t1, t2, t3, t4, t6, t8, t16, t32 }

}

DL-AM-RLC ::=                       SEQUENCE {

sn-FieldLength                      SN-FieldLengthAM                                    OPTIONAL,   -- Cond Reestab

t-Reassembly                        T-Reassembly,

t-StatusProhibit                    T-StatusProhibit

}

UL-UM-RLC ::=                       SEQUENCE {

sn-FieldLength                      SN-FieldLengthUM                                    OPTIONAL    -- Cond Reestab

}

DL-UM-RLC ::=                       SEQUENCE {

sn-FieldLength                      SN-FieldLengthUM                                    OPTIONAL,   -- Cond Reestab

t-Reassembly                        T-Reassembly

}

T-PollRetransmit ::=                ENUMERATED {

ms5, ms10, ms15, ms20, ms25, ms30, ms35,

ms40, ms45, ms50, ms55, ms60, ms65, ms70,

ms75, ms80, ms85, ms90, ms95, ms100, ms105,

ms110, ms115, ms120, ms125, ms130, ms135,

ms140, ms145, ms150, ms155, ms160, ms165,

ms170, ms175, ms180, ms185, ms190, ms195,

ms200, ms205, ms210, ms215, ms220, ms225,

ms230, ms235, ms240, ms245, ms250, ms300,

ms350, ms400, ms450, ms500, ms800, ms1000,

ms2000, ms4000, spare5, spare4, spare3,

spare2, spare1}

PollPDU ::=                         ENUMERATED {

p4, p8, p16, p32, p64, p128, p256, p512, p1024, p2048, p4096, p6144, p8192, p12288, p16384,p20480,

p24576, p28672, p32768, p40960, p49152, p57344, p65536, infinity, spare8, spare7, spare6, spare5, spare4,

spare3, spare2, spare1}

PollByte ::=                        ENUMERATED {

kB1, kB2, kB5, kB8, kB10, kB15, kB25, kB50, kB75,

kB100, kB125, kB250, kB375, kB500, kB750, kB1000,

kB1250, kB1500, kB2000, kB3000, kB4000, kB4500,

kB5000, kB5500, kB6000, kB6500, kB7000, kB7500,

mB8, mB9, mB10, mB11, mB12, mB13, mB14, mB15,

mB16, mB17, mB18, mB20, mB25, mB30, mB40, infinity,

spare20, spare19, spare18, spare17, spare16,

spare15, spare14, spare13, spare12, spare11,

spare10, spare9, spare8, spare7, spare6, spare5,

spare4, spare3, spare2, spare1}

T-Reassembly ::=                    ENUMERATED {

ms0, ms5, ms10, ms15, ms20, ms25, ms30, ms35,

ms40, ms45, ms50, ms55, ms60, ms65, ms70,

ms75, ms80, ms85, ms90, ms95, ms100, ms110,

ms120, ms130, ms140, ms150, ms160, ms170,

ms180, ms190, ms200, spare1}

T-StatusProhibit ::=                ENUMERATED {

ms0, ms5, ms10, ms15, ms20, ms25, ms30, ms35,

ms40, ms45, ms50, ms55, ms60, ms65, ms70,

ms75, ms80, ms85, ms90, ms95, ms100, ms105,

ms110, ms115, ms120, ms125, ms130, ms135,

ms140, ms145, ms150, ms155, ms160, ms165,

ms170, ms175, ms180, ms185, ms190, ms195,

ms200, ms205, ms210, ms215, ms220, ms225,

ms230, ms235, ms240, ms245, ms250, ms300,

ms350, ms400, ms450, ms500, ms800, ms1000,

ms1200, ms1600, ms2000, ms2400, spare2, spare1}

SN-FieldLengthUM ::=                ENUMERATED {size6, size12}

SN-FieldLengthAM ::=                ENUMERATED {size12, size18}

-- TAG-RLC-CONFIG-STOP

-- ASN1STOP

RLC-Config field descriptions

maxRetxThreshold Parameter for RLC AM in TS 38.322 [4]. Value t1 corresponds to 1 retransmission, value t2 corresponds to 2 retransmissions and so on.

pollByte Parameter for RLC AM in TS 38.322 [4]. Value kB25 corresponds to 25 kBytes, value kB50 corresponds to 50 kBytes and so on. infinity corresponds to an infinite amount of kBytes.

pollPDU Parameter for RLC AM in TS 38.322 [4]. Value p4 corresponds to 4 PDUs, value p8 corresponds to 8 PDUs and so on. infinity corresponds to an infinite number of PDUs.

sn-FieldLength Indicates the RLC SN field size, see TS 38.322 [4], in bits. Value size6 means 6 bits, value size12 means 12 bits, value size18 means 18 bits. The value of sn-FieldLength for a DRB shall be changed only using reconfiguration with sync. The network configures only value size12 in SN-FieldLengthAM for SRB.

t-PollRetransmit Timer for RLC AM in TS 38.322 [4], in milliseconds. Value ms5 means 5 ms, value ms10 means 10 ms and so on.

t-Reassembly Timer for reassembly in TS 38.322 [4], in milliseconds. Value ms0 means 0 ms, value ms5 means 5 ms and so on.

t-StatusProhibit Timer for status reporting in TS 38.322 [4], in milliseconds. Value ms0 means 0 ms, value ms5 means 5 ms and so on.

| Conditional Presence | Explanation |
| --- | --- |
| Reestab | The field is mandatory present at bearer setup. It is optionally present, need M, at RLC re-establishment. Otherwise it is absent. Need M. |

#### –	RLF-TimersAndConstants

The IE RLF-TimersAndConstants is used to configure UE specific timers and constants.

RLF-TimersAndConstants information element

-- ASN1START

-- TAG-RLF-TIMERSANDCONSTANTS-START

RLF-TimersAndConstants ::=          SEQUENCE {

t310                                ENUMERATED {ms0, ms50, ms100, ms200, ms500, ms1000, ms2000, ms4000, ms6000},

n310                                ENUMERATED {n1, n2, n3, n4, n6, n8, n10, n20},

n311                                ENUMERATED {n1, n2, n3, n4, n5, n6, n8, n10},

...,

[[

t311                                ENUMERATED {ms1000, ms3000, ms5000, ms10000, ms15000, ms20000, ms30000}

]]

}

-- TAG-RLF-TIMERSANDCONSTANTS-STOP

-- ASN1STOP

RLF-TimersAndConstants field descriptions

n3xy Constants are described in clause 7.3. Value n1 corresponds to 1, value n2 corresponds to 2 and so on.

t3xy Timers are described in clause 7.1. Value ms0 corresponds to 0 ms, value ms50 corresponds to 50 ms and so on.

#### –	RNTI-Value

The IE RNTI-Value represents a Radio Network Temporary Identity.

RNTI-Value information element

-- ASN1START

-- TAG-RNTI-VALUE-START

RNTI-Value ::=                      INTEGER (0..65535)

-- TAG-RNTI-VALUE-STOP

-- ASN1STOP

#### –	RSRP-Range

The IE RSRP-Range specifies the value range used in RSRP measurements and thresholds. For measurements, integer value for RSRP measurements is according to Table 10.1.6.1-1 in TS 38.133 [14]. For thresholds, the actual value is (IE value – 156) dBm, except for the IE value 127, in which case the actual value is infinity.

RSRP-Range information element

-- ASN1START

-- TAG-RSRP-RANGE-START

RSRP-Range ::=                      INTEGER(0..127)

-- TAG-RSRP-RANGE-STOP

-- ASN1STOP

#### –	RSRQ-Range

The IE RSRQ-Range specifies the value range used in RSRQ measurements and thresholds. For measurements, integer value for RSRQ measurements is according to Table 10.1.11.1-1 in TS 38.133 [14]. For thresholds, the actual value is (IE value – 87) / 2 dB.

RSRQ-Range information element

-- ASN1START

-- TAG-RSRQ-RANGE-START

RSRQ-Range ::=                      INTEGER(0..127)

-- TAG-RSRQ-RANGE-STOP

-- ASN1STOP

#### –	SCellIndex

The IE SCellIndex concerns a short identity, used to identify an SCell or PSCell. The value range is shared across the Cell Groups.

SCellIndex information element

-- ASN1START

-- TAG-SCELLINDEX-START

SCellIndex ::=                      INTEGER (1..31)

-- TAG-SCELLINDEX-STOP

-- ASN1STOP

#### –	SchedulingRequestConfig

The IE SchedulingRequestConfig is used to configure the parameters, for the dedicated scheduling request (SR) resources.

SchedulingRequestConfig information element

-- ASN1START

-- TAG-SCHEDULINGREQUESTCONFIG-START

SchedulingRequestConfig ::=         SEQUENCE {

schedulingRequestToAddModList       SEQUENCE (SIZE (1..maxNrofSR-ConfigPerCellGroup)) OF SchedulingRequestToAddMod

OPTIONAL, -- Need N

schedulingRequestToReleaseList      SEQUENCE (SIZE (1..maxNrofSR-ConfigPerCellGroup)) OF SchedulingRequestId

OPTIONAL  -- Need N

}

SchedulingRequestToAddMod ::=       SEQUENCE {

schedulingRequestId                 SchedulingRequestId,

sr-ProhibitTimer                    ENUMERATED {ms1, ms2, ms4, ms8, ms16, ms32, ms64, ms128}          OPTIONAL, -- Need S

sr-TransMax                         ENUMERATED { n4, n8, n16, n32, n64, spare3, spare2, spare1}

}

-- TAG-SCHEDULINGREQUESTCONFIG-STOP

-- ASN1STOP

SchedulingRequestConfig field descriptions

schedulingRequestToAddModList List of Scheduling Request configurations to add or modify.

schedulingRequestToReleaseList List of Scheduling Request configurations to release.

SchedulingRequestToAddMod field descriptions

schedulingRequestId Used to modify a SR configuration and to indicate, in LogicalChannelConfig, the SR configuration to which a logical channel is mapped and to indicate, in SchedulingRequestresourceConfig, the SR configuration for which a scheduling request resource is used.

sr-ProhibitTimer Timer for SR transmission on PUCCH in TS 38.321 [3]. Value is in ms. Value ms1 corresponds to 1ms, value ms2 corresponds to 2ms, and so on. When the field is absent, the UE applies the value 0.

sr-TransMax Maximum number of SR transmissions as described in TS 38.321 [3]. Value n4 corresponds to 4, value n8 corresponds to 8, and so on.

#### –	SchedulingRequestId

The IE SchedulingRequestId is used to identify a Scheduling Request instance in the MAC layer.

SchedulingRequestId information element

-- ASN1START

-- TAG-SCHEDULINGREQUESTID-START

SchedulingRequestId ::=             INTEGER (0..7)

-- TAG-SCHEDULINGREQUESTID-STOP

-- ASN1STOP

#### –	SchedulingRequestResourceConfig

The IE SchedulingRequestResourceConfig determines physical layer resources on PUCCH where the UE may send the dedicated scheduling request (D-SR) (see TS 38.213 [13], clause 9.2.4).

SchedulingRequestResourceConfig information element

-- ASN1START

-- TAG-SCHEDULINGREQUESTRESOURCECONFIG-START

SchedulingRequestResourceConfig ::=     SEQUENCE {

schedulingRequestResourceId             SchedulingRequestResourceId,

schedulingRequestID                     SchedulingRequestId,

periodicityAndOffset                    CHOICE {

sym2                                    NULL,

sym6or7                                 NULL,

sl1                                     NULL,                       -- Recurs in every slot

sl2                                     INTEGER (0..1),

sl4                                     INTEGER (0..3),

sl5                                     INTEGER (0..4),

sl8                                     INTEGER (0..7),

sl10                                    INTEGER (0..9),

sl16                                    INTEGER (0..15),

sl20                                    INTEGER (0..19),

sl40                                    INTEGER (0..39),

sl80                                    INTEGER (0..79),

sl160                                   INTEGER (0..159),

sl320                                   INTEGER (0..319),

sl640                                   INTEGER (0..639)

}                                                                                                       OPTIONAL,   -- Need M

resource                                PUCCH-ResourceId                                                OPTIONAL    -- Need M

}

-- TAG-SCHEDULINGREQUESTRESOURCECONFIG-STOP

-- ASN1STOP

SchedulingRequestResourceConfig field descriptions

periodicityAndOffset SR periodicity and offset in number of symbols or slots (see TS 38.213 [13], clause 9.2.4) The following periodicities may be configured depending on the chosen subcarrier spacing: SCS = 15 kHz: 2sym, 7sym, 1sl, 2sl, 4sl, 5sl, 8sl, 10sl, 16sl, 20sl, 40sl, 80sl SCS = 30 kHz: 2sym, 7sym, 1sl, 2sl, 4sl, 8sl, 10sl, 16sl, 20sl, 40sl, 80sl, 160sl SCS = 60 kHz: 2sym, 7sym/6sym, 1sl, 2sl, 4sl, 8sl, 16sl, 20sl, 40sl, 80sl, 160sl, 320sl SCS = 120 kHz: 2sym, 7sym, 1sl, 2sl, 4sl, 8sl, 16sl, 40sl, 80sl, 160sl, 320sl, 640sl sym6or7 corresponds to 6 symbols if extended cyclic prefix and a SCS of 60 kHz are configured, otherwise it corresponds to 7 symbols. For periodicities 2sym, 7sym and sl1 the UE assumes an offset of 0 slots.

resource ID of the PUCCH resource in which the UE shall send the scheduling request. The actual PUCCH-Resource is configured in PUCCH-Config of the same UL BWP and serving cell as this SchedulingRequestResourceConfig. The network configures a PUCCH-Resource of PUCCH-format0 or PUCCH-format1 (other formats not supported) (see TS 38.213 [13], clause 9.2.4)

schedulingRequestID The ID of the SchedulingRequestConfig that uses this scheduling request resource.

#### –	SchedulingRequestResourceId

The IE SchedulingRequestResourceId is used to identify scheduling request resources on PUCCH.

SchedulingRequestResourceId information element

-- ASN1START

-- TAG-SCHEDULINGREQUESTRESOURCEID-START

SchedulingRequestResourceId ::=     INTEGER (1..maxNrofSR-Resources)

-- TAG-SCHEDULINGREQUESTRESOURCEID-STOP

-- ASN1STOP

#### –	ScramblingId

The IE ScramblingID is used for scrambling channels and reference signals.

ScramblingId information element

-- ASN1START

-- TAG-SCRAMBLINGID-START

ScramblingId ::=                    INTEGER(0..1023)

-- TAG-SCRAMBLINGID-STOP

-- ASN1STOP

#### –	SCS-SpecificCarrier

The IE SCS-SpecificCarrier provides parameters determining the location and width of the actual carrier or the carrier bandwidth. It is defined specifically for a numerology (subcarrier spacing (SCS)) and in relation (frequency offset) to Point A.

SCS-SpecificCarrier information element

-- ASN1START

-- TAG-SCS-SPECIFICCARRIER-START

SCS-SpecificCarrier ::=             SEQUENCE {

offsetToCarrier                     INTEGER (0..2199),

subcarrierSpacing                   SubcarrierSpacing,

carrierBandwidth                    INTEGER (1..maxNrofPhysicalResourceBlocks),

...,

[[

txDirectCurrentLocation         INTEGER (0..4095)                                       OPTIONAL            -- Need S

]]

}

-- TAG-SCS-SPECIFICCARRIER-STOP

-- ASN1STOP

SCS-SpecificCarrier field descriptions

carrierBandwidth Width of this carrier in number of PRBs (using the subcarrierSpacing defined for this carrier) (see TS 38.211 [16], clause 4.4.2).

offsetToCarrier Offset in frequency domain between Point A (lowest subcarrier of common RB 0) and the lowest usable subcarrier on this carrier in number of PRBs (using the subcarrierSpacing defined for this carrier). The maximum value corresponds to 275*8-1. See TS 38.211 [16], clause 4.4.2.

txDirectCurrentLocation Indicates the downlink Tx Direct Current location for the carrier. A value in the range 0..3299 indicates the subcarrier index within the carrier. The values in the value range 3301..4095 are reserved and ignored by the UE. If this field is absent for downlink within ServingCellConfigCommon and ServingCellConfigCommonSIB, the UE assumes the default value of 3300 (i.e. "Outside the carrier"). (see TS 38.211 [16], clause 4.4.2). Network does not configure this field via ServingCellConfig or for uplink carriers.

subcarrierSpacing Subcarrier spacing of this carrier. It is used to convert the offsetToCarrier into an actual frequency. Only the values 15 kHz, 30 kHz or 60 kHz (FR1), and 60 kHz or 120 kHz (FR2) are applicable.

#### –	SDAP-Config

The IE SDAP-Config is used to set the configurable SDAP parameters for a data radio bearer. All configured instances of SDAP-Config with the same value of pdu-Session correspond to the same SDAP entity as specified in TS 37.324 [24].

SDAP-Config information element

-- ASN1START

-- TAG-SDAP-CONFIG-START

SDAP-Config ::=                     SEQUENCE {

pdu-Session                         PDU-SessionID,

sdap-HeaderDL                       ENUMERATED {present, absent},

sdap-HeaderUL                       ENUMERATED {present, absent},

defaultDRB                          BOOLEAN,

mappedQoS-FlowsToAdd                SEQUENCE (SIZE (1..maxNrofQFIs)) OF QFI                                 OPTIONAL, -- Need N

mappedQoS-FlowsToRelease            SEQUENCE (SIZE (1..maxNrofQFIs)) OF QFI                                 OPTIONAL, -- Need N

...

}

QFI ::=                             INTEGER (0..maxQFI)

PDU-SessionID ::=                   INTEGER (0..255)

-- TAG-SDAP-CONFIG-STOP

-- ASN1STOP

SDAP-Config field descriptions

defaultDRB Indicates whether or not this is the default DRB for this PDU session. Among all configured instances of SDAP-Config with the same value of pdu-Session, this field shall be set to true in at most one instance of SDAP-Config and to false in all other instances.

mappedQoS-FlowsToAdd Indicates the list of QFIs of UL QoS flows of the PDU session to be additionally mapped to this DRB. A QFI value can be included at most once in all configured instances of SDAP-Config with the same value of pdu-Session. For QoS flow remapping, the QFI value of the remapped QoS flow is only included in mappedQoS-FlowsToAdd in sdap-Config corresponding to the new DRB and not included in mappedQoS-FlowsToRelease in sdap-Config corresponding to the old DRB.

mappedQoS-FlowsToRelease Indicates the list of QFIs of QoS flows of the PDU session to be released from existing QoS flow to DRB mapping of this DRB.

pdu-Session Identity of the PDU session whose QoS flows are mapped to the DRB.

sdap-HeaderUL Indicates whether or not a SDAP header is present for UL data on this DRB. The field cannot be changed after a DRB is established. The network sets this field to present if the field defaultDRB is set to true.

sdap-HeaderDL Indicates whether or not a SDAP header is present for DL data on this DRB. The field cannot be changed after a DRB is established.

#### –	SearchSpace

The IE SearchSpace defines how/where to search for PDCCH candidates. Each search space is associated with one ControlResourceSet. For a scheduled cell in the case of cross carrier scheduling, except for nrofCandidates, all the optional fields are absent.

SearchSpace information element

-- ASN1START

-- TAG-SEARCHSPACE-START

SearchSpace ::=                         SEQUENCE {

searchSpaceId                           SearchSpaceId,

controlResourceSetId                    ControlResourceSetId                                        OPTIONAL,   -- Cond SetupOnly

monitoringSlotPeriodicityAndOffset      CHOICE {

sl1                                     NULL,

sl2                                     INTEGER (0..1),

sl4                                     INTEGER (0..3),

sl5                                     INTEGER (0..4),

sl8                                     INTEGER (0..7),

sl10                                    INTEGER (0..9),

sl16                                    INTEGER (0..15),

sl20                                    INTEGER (0..19),

sl40                                    INTEGER (0..39),

sl80                                    INTEGER (0..79),

sl160                                   INTEGER (0..159),

sl320                                   INTEGER (0..319),

sl640                                   INTEGER (0..639),

sl1280                                  INTEGER (0..1279),

sl2560                                  INTEGER (0..2559)

}                                                                                                   OPTIONAL,   -- Cond Setup

duration                                INTEGER (2..2559)                                           OPTIONAL,   -- Need R

monitoringSymbolsWithinSlot             BIT STRING (SIZE (14))                                      OPTIONAL,   -- Cond Setup

nrofCandidates                          SEQUENCE {

aggregationLevel1                       ENUMERATED {n0, n1, n2, n3, n4, n5, n6, n8},

aggregationLevel2                       ENUMERATED {n0, n1, n2, n3, n4, n5, n6, n8},

aggregationLevel4                       ENUMERATED {n0, n1, n2, n3, n4, n5, n6, n8},

aggregationLevel8                       ENUMERATED {n0, n1, n2, n3, n4, n5, n6, n8},

aggregationLevel16                      ENUMERATED {n0, n1, n2, n3, n4, n5, n6, n8}

}                                                                                                   OPTIONAL,   -- Cond Setup

searchSpaceType                         CHOICE {

common                                  SEQUENCE {

dci-Format0-0-AndFormat1-0              SEQUENCE {

...

}                                                                                           OPTIONAL,   -- Need R

dci-Format2-0                           SEQUENCE {

nrofCandidates-SFI                      SEQUENCE {

aggregationLevel1                       ENUMERATED {n1, n2}                         OPTIONAL,   -- Need R

aggregationLevel2                       ENUMERATED {n1, n2}                         OPTIONAL,   -- Need R

aggregationLevel4                       ENUMERATED {n1, n2}                         OPTIONAL,   -- Need R

aggregationLevel8                       ENUMERATED {n1, n2}                         OPTIONAL,   -- Need R

aggregationLevel16                      ENUMERATED {n1, n2}                         OPTIONAL    -- Need R

},

...

}                                                                                           OPTIONAL,   -- Need R

dci-Format2-1                           SEQUENCE {

...

}                                                                                           OPTIONAL,   -- Need R

dci-Format2-2                           SEQUENCE {

...

}                                                                                           OPTIONAL,   -- Need R

dci-Format2-3                           SEQUENCE {

dummy1                                  ENUMERATED {sl1, sl2, sl4, sl5, sl8, sl10, sl16, sl20}  OPTIONAL,   -- Cond Setup

dummy2                                  ENUMERATED {n1, n2},

...

}                                                                                           OPTIONAL    -- Need R

},

ue-Specific                                 SEQUENCE {

dci-Formats                                 ENUMERATED {formats0-0-And-1-0, formats0-1-And-1-1},

...

}

}                                                                                                   OPTIONAL    -- Cond Setup

}

-- TAG-SEARCHSPACE-STOP

-- ASN1STOP

SearchSpace field descriptions

common Configures this search space as common search space (CSS) and DCI formats to monitor.

controlResourceSetId The CORESET applicable for this SearchSpace. Value 0 identifies the common CORESET#0 configured in MIB and in ServingCellConfigCommon. Values 1..maxNrofControlResourceSets-1 identify CORESETs configured in System Information or by dedicated signalling. The CORESETs with non-zero controlResourceSetId are configured in the same BWP as this SearchSpace.

dummy1, dummy2 This field is not used in the specification. If received it shall be ignored by the UE.

dci-Format0-0-AndFormat1-0 If configured, the UE monitors the DCI formats 0_0 and 1_0 according to TS 38.213 [13], clause 10.1.

dci-Format2-0 If configured, UE monitors the DCI format 2_0 according to TS 38.213 [13], clause 10.1, 11.1.1.

dci-Format2-1 If configured, UE monitors the DCI format 2_1 according to TS 38.213 [13], clause 10.1, 11.2.

dci-Format2-2 If configured, UE monitors the DCI format 2_2 according to TS 38.213 [13], clause 10.1, 11.3.

dci-Format2-3 If configured, UE monitors the DCI format 2_3 according to TS 38.213 [13], clause 10.1, 11.4

dci-Formats Indicates whether the UE monitors in this USS for DCI formats 0-0 and 1-0 or for formats 0-1 and 1-1.

duration Number of consecutive slots that a SearchSpace lasts in every occasion, i.e., upon every period as given in the periodicityAndOffset. If the field is absent, the UE applies the value 1 slot, except for DCI format 2_0. The UE ignores this field for DCI format 2_0. The maximum valid duration is periodicity-1 (periodicity as given in the monitoringSlotPeriodicityAndOffset).

monitoringSlotPeriodicityAndOffset Slots for PDCCH Monitoring configured as periodicity and offset. If the UE is configured to monitor DCI format 2_1, only the values 'sl1', 'sl2' or 'sl4' are applicable. If the UE is configured to monitor DCI format 2_0, only the values ′sl1′, ′sl2′, ′sl4′, ′sl5′, ′sl8′, ′sl10′, ′sl16′, and ′sl20′ are applicable (see TS 38.213 [13], clause 10).

monitoringSymbolsWithinSlot The first symbol(s) for PDCCH monitoring in the slots configured for PDCCH monitoring (see monitoringSlotPeriodicityAndOffset and duration). The most significant (left) bit represents the first OFDM in a slot, and the second most significant (left) bit represents the second OFDM symbol in a slot and so on. The bit(s) set to one identify the first OFDM symbol(s) of the control resource set within a slot. If the cyclic prefix of the BWP is set to extended CP, the last two bits within the bit string shall be ignored by the UE . For DCI format 2_0, the first one symbol applies if the duration of CORESET (in the IE ControlResourceSet) identified by controlResourceSetId indicates 3 symbols, the first two symbols apply if the duration of CORESET identified by controlResourceSetId indicates 2 symbols, and the first three symbols apply if the duration of CORESET identified by controlResourceSetId indicates 1 symbol. See TS 38.213 [13], clause 10.

nrofCandidates-SFI The number of PDCCH candidates specifically for format 2-0 for the configured aggregation level. If an aggregation level is absent, the UE does not search for any candidates with that aggregation level. The network configures only one aggregationLevel and the corresponding number of candidates (see TS 38.213 [13], clause 11.1.1).

nrofCandidates Number of PDCCH candidates per aggregation level. The number of candidates and aggregation levels configured here applies to all formats unless a particular value is specified or a format-specific value is provided (see inside searchSpaceType). If configured in the SearchSpace of a cross carrier scheduled cell, this field determines the number of candidates and aggregation levels to be used on the linked scheduling cell (see TS 38.213 [13], clause 10).

searchSpaceId Identity of the search space. SearchSpaceId = 0 identifies the searchSpaceZero configured via PBCH (MIB) or ServingCellConfigCommon and may hence not be used in the SearchSpace IE. The searchSpaceId is unique among the BWPs of a Serving Cell. In case of cross carrier scheduling, search spaces with the same searchSpaceId in scheduled cell and scheduling cell are linked to each other. The UE applies the search space for the scheduled cell only if the DL BWPs in which the linked search spaces are configured in scheduling cell and scheduled cell are both active.

searchSpaceType Indicates whether this is a common search space (present) or a UE specific search space as well as DCI formats to monitor for.

ue-Specific Configures this search space as UE specific search space (USS). The UE monitors the DCI format with CRC scrambled by C-RNTI, CS-RNTI (if configured), and SP-CSI-RNTI (if configured)

| Conditional Presence | Explanation |
| --- | --- |
| Setup | This field is mandatory present upon creation of a new SearchSpace. It is optionally present, Need M, otherwise. |
| SetupOnly | This field is mandatory present upon creation of a new SearchSpace. It is absent, Need M, otherwise. |

#### –	SearchSpaceId

The IE SearchSpaceId is used to identify Search Spaces. The ID space is used across the BWPs of a Serving Cell. The search space with the SearchSpaceId = 0 identifies the search space configured via PBCH (MIB) and in ServingCellConfigCommon (searchSpaceZero). The number of Search Spaces per BWP is limited to 10 including the common and UE specific Search Spaces.

SearchSpaceId information element

-- ASN1START

-- TAG-SEARCHSPACEID-START

SearchSpaceId ::=                   INTEGER (0..maxNrofSearchSpaces-1)

-- TAG-SEARCHSPACEID-STOP

-- ASN1STOP

#### –	SearchSpaceZero

The IE SearchSpaceZero is used to configure SearchSpace#0 of the initial BWP (see TS 38.213 [13], clause 13).

SearchSpaceZero information element

-- ASN1START

-- TAG-SEARCHSPACEZERO-START

SearchSpaceZero ::=                 INTEGER (0..15)

-- TAG-SEARCHSPACEZERO-STOP

-- ASN1STOP

#### –	SecurityAlgorithmConfig

The IE SecurityAlgorithmConfig is used to configure AS integrity protection algorithm and AS ciphering algorithm for SRBs and DRBs.

SecurityAlgorithmConfig information element

-- ASN1START

-- TAG-SECURITYALGORITHMCONFIG-START

SecurityAlgorithmConfig ::=         SEQUENCE {

cipheringAlgorithm                  CipheringAlgorithm,

integrityProtAlgorithm              IntegrityProtAlgorithm          OPTIONAL,   -- Need R

...

}

IntegrityProtAlgorithm ::=          ENUMERATED {

nia0, nia1, nia2, nia3, spare4, spare3,

spare2, spare1, ...}

CipheringAlgorithm ::=              ENUMERATED {

nea0, nea1, nea2, nea3, spare4, spare3,

spare2, spare1, ...}

-- TAG-SECURITYALGORITHMCONFIG-STOP

-- ASN1STOP

SecurityAlgorithmConfig field descriptions

cipheringAlgorithm Indicates the ciphering algorithm to be used for SRBs and DRBs, as specified in TS 33.501 [11]. The algorithms nea0-nea3 are identical to the LTE algorithms eea0-3. The algorithms configured for all bearers using master key shall be the same, and the algorithms configured for all bearers using secondary key, if any, shall be the same. If UE is connected to E-UTRA/EPC, this field indicates the ciphering algorithm to be used for RBs configured with NR PDCP, as specified in TS 33.501 [11].

integrityProtAlgorithm Indicates the integrity protection algorithm to be used for SRBs and DRBs, as specified in TS 33.501 [11]. The algorithms nia0-nia3 are identical to the E-UTRA algorithms eia0-3. The algorithms configured for all bearers using master key shall be the same and the algorithms configured for all bearers using secondary key, if any, shall be the same. The network does not configure nia0 except for unauthenticated emergency sessions for unauthenticated UEs in LSM (limited service mode). If UE is connected to E-UTRA/EPC, this field indicates the integrity protection algorithm to be used for SRBs configured with NR PDCP, as specified in TS 33.501 [11]. The network does not configure nia0 for SRB3.

#### –	ServCellIndex

The IE ServCellIndex concerns a short identity, used to identify a serving cell (i.e. the PCell, the PSCell or an SCell). Value 0 applies for the PCell, while the SCellIndex that has previously been assigned applies for SCells.

ServCellIndex information element

-- ASN1START

-- TAG-SERVCELLINDEX-START

ServCellIndex ::=                   INTEGER (0..maxNrofServingCells-1)

-- TAG-SERVCELLINDEX-STOP

-- ASN1STOP

#### –	ServingCellConfig

The IE ServingCellConfig is used to configure (add or modify) the UE with a serving cell, which may be the SpCell or an SCell of an MCG or SCG. The parameters herein are mostly UE specific but partly also cell specific (e.g. in additionally configured bandwidth parts). Reconfiguration between a PUCCH and PUCCHless SCell is only supported using an SCell release and add.

ServingCellConfig information element

-- ASN1START

-- TAG-SERVINGCELLCONFIG-START

ServingCellConfig ::=               SEQUENCE {

tdd-UL-DL-ConfigurationDedicated    TDD-UL-DL-ConfigDedicated                                   OPTIONAL,   -- Cond TDD

initialDownlinkBWP                  BWP-DownlinkDedicated                                       OPTIONAL,   -- Need M

downlinkBWP-ToReleaseList           SEQUENCE (SIZE (1..maxNrofBWPs)) OF BWP-Id                  OPTIONAL,   -- Need N

downlinkBWP-ToAddModList            SEQUENCE (SIZE (1..maxNrofBWPs)) OF BWP-Downlink            OPTIONAL,   -- Need N

firstActiveDownlinkBWP-Id           BWP-Id                                                      OPTIONAL,   -- Cond SyncAndCellAdd

bwp-InactivityTimer                 ENUMERATED {ms2, ms3, ms4, ms5, ms6, ms8, ms10, ms20, ms30,

ms40,ms50, ms60, ms80,ms100, ms200,ms300, ms500,

ms750, ms1280, ms1920, ms2560, spare10, spare9, spare8,

spare7, spare6, spare5, spare4, spare3, spare2, spare1 }    OPTIONAL,   --Need R

defaultDownlinkBWP-Id               BWP-Id                                                                  OPTIONAL,   -- Need S

uplinkConfig                        UplinkConfig                                                            OPTIONAL,   -- Need M

supplementaryUplink                 UplinkConfig                                                            OPTIONAL,   -- Need M

pdcch-ServingCellConfig             SetupRelease { PDCCH-ServingCellConfig }                                OPTIONAL,   -- Need M

pdsch-ServingCellConfig             SetupRelease { PDSCH-ServingCellConfig }                                OPTIONAL,   -- Need M

csi-MeasConfig                      SetupRelease { CSI-MeasConfig }                                         OPTIONAL,   -- Need M

sCellDeactivationTimer              ENUMERATED {ms20, ms40, ms80, ms160, ms200, ms240,

ms320, ms400, ms480, ms520, ms640, ms720,

ms840, ms1280, spare2,spare1}       OPTIONAL,   -- Cond ServingCellWithoutPUCCH

crossCarrierSchedulingConfig        CrossCarrierSchedulingConfig                                    OPTIONAL,   -- Need M

tag-Id                              TAG-Id,

dummy                               ENUMERATED {enabled}                                            OPTIONAL,   -- Need R

pathlossReferenceLinking            ENUMERATED {spCell, sCell}                                       OPTIONAL,   -- Cond SCellOnly

servingCellMO                       MeasObjectId                                                    OPTIONAL,   -- Cond MeasObject

...,

[[

lte-CRS-ToMatchAround               SetupRelease { RateMatchPatternLTE-CRS }                                OPTIONAL,   -- Need M

rateMatchPatternToAddModList        SEQUENCE (SIZE (1..maxNrofRateMatchPatterns)) OF RateMatchPattern       OPTIONAL,   -- Need N

rateMatchPatternToReleaseList       SEQUENCE (SIZE (1..maxNrofRateMatchPatterns)) OF RateMatchPatternId     OPTIONAL,   -- Need N

downlinkChannelBW-PerSCS-List       SEQUENCE (SIZE (1..maxSCSs)) OF SCS-SpecificCarrier                     OPTIONAL    -- Need S

]]

}

UplinkConfig ::=                    SEQUENCE {

initialUplinkBWP                    BWP-UplinkDedicated                                         OPTIONAL,   -- Need M

uplinkBWP-ToReleaseList             SEQUENCE (SIZE (1..maxNrofBWPs)) OF BWP-Id                  OPTIONAL,   -- Need N

uplinkBWP-ToAddModList              SEQUENCE (SIZE (1..maxNrofBWPs)) OF BWP-Uplink              OPTIONAL,   -- Need N

firstActiveUplinkBWP-Id             BWP-Id                                                      OPTIONAL,   -- Cond SyncAndCellAdd

pusch-ServingCellConfig             SetupRelease { PUSCH-ServingCellConfig }                    OPTIONAL,   -- Need M

carrierSwitching                    SetupRelease { SRS-CarrierSwitching }                       OPTIONAL,   -- Need M

...,

[[

powerBoostPi2BPSK                   BOOLEAN                                                     OPTIONAL,   -- Need M

uplinkChannelBW-PerSCS-List         SEQUENCE (SIZE (1..maxSCSs)) OF SCS-SpecificCarrier         OPTIONAL    -- Need S

]]

}

-- TAG-SERVINGCELLCONFIG-STOP

-- ASN1STOP

ServingCellConfig field descriptions

bwp-InactivityTimer The duration in ms after which the UE falls back to the default Bandwidth Part (see TS 38.321 [3], clause 5.15). When the network releases the timer configuration, the UE stops the timer without switching to the default BWP.

crossCarrierSchedulingConfig Indicates whether this serving cell is cross-carrier scheduled by another serving cell or whether it cross-carrier schedules another serving cell.

defaultDownlinkBWP-Id The initial bandwidth part is referred to by BWP-Id = 0. ID of the downlink bandwidth part to be used upon expiry of the BWP inactivity timer. This field is UE specific. When the field is absent the UE uses the initial BWP as default BWP. (see TS 38.213 [13], clause 12 and TS 38.321 [3], clause 5.15).

downlinkBWP-ToAddModList List of additional downlink bandwidth parts to be added or modified. (see TS 38.213 [13], clause 12).

downlinkBWP-ToReleaseList List of additional downlink bandwidth parts to be released. (see TS 38.213 [13], clause 12).

downlinkChannelBW-PerSCS-List A set of UE specific channel bandwidth and location configurations for different subcarrier spacings (numerologies). Defined in relation to Point A. The UE uses the configuration provided in this field only for the purpose of channel bandwidth and location determination. If absent, UE uses the configuration indicated in scs-SpecificCarrierList in DownlinkConfigCommon / DownlinkConfigCommonSIB. Network only configures channel bandwidth that corresponds to the channel bandwidth values defined in TS 38.101-1 [15] and TS 38.101-2 [39].

firstActiveDownlinkBWP-Id If configured for an SpCell, this field contains the ID of the DL BWP to be activated upon performing the RRC (re-)configuration. If the field is absent, the RRC (re-)configuration does not impose a BWP switch. If configured for an SCell, this field contains the ID of the downlink bandwidth part to be used upon MAC-activation of an SCell. The initial bandwidth part is referred to by BWP-Id = 0. Upon PCell change and PSCell addition/change, the network sets the firstActiveDownlinkBWP-Id and firstActiveUplinkBWP-Id to the same value.

initialDownlinkBWP The dedicated (UE-specific) configuration for the initial downlink bandwidth-part (i.e. DL BWP#0). If any of the optional IEs are configured within this IE, the UE considers the BWP#0 to be an RRC configured BWP (from UE capability viewpoint). Otherwise, the UE does not consider the BWP#0 as an RRC configured BWP (from UE capability viewpoint). Network always configures the UE with a value for this field if no other BWPs are configured. NOTE1

lte-CRS-ToMatchAround Parameters to determine an LTE CRS pattern that the UE shall rate match around.

pathlossReferenceLinking Indicates whether UE shall apply as pathloss reference either the downlink of SpCell (PCell for MCG or PSCell for SCG) or of SCell that corresponds with this uplink (see TS 38.213 [13], clause 7).

pdsch-ServingCellConfig PDSCH related parameters that are not BWP-specific.

rateMatchPatternToAddModList Resources patterns which the UE should rate match PDSCH around. The UE rate matches around the union of all resources indicated in the rate match patterns. Rate match patterns defined here on cell level apply only to PDSCH of the same numerology. See TS 38.214 [19], clause 5.1.2.2.3.

sCellDeactivationTimer SCell deactivation timer in TS 38.321 [3]. If the field is absent, the UE applies the value infinity.

servingCellMO measObjectId of the MeasObjectNR in MeasConfig which is associated to the serving cell. For this MeasObjectNR, the following relationship applies between this MeasObjectNR and frequencyInfoDL in ServingCellConfigCommon of the serving cell: if ssbFrequency is configured, its value is the same as the absoluteFrequencySSB and if csi-rs-ResourceConfigMobility is configured, the value of its subcarrierSpacing is present in one entry of the scs-SpecificCarrierList, csi-RS-CellListMobility includes an entry corresponding to the serving cell (with cellId equal to physCellId in ServingCellConfigCommon) and the frequency range indicated by the csi-rs-MeasurementBW of the entry in csi-RS-CellListMobility is included in the frequency range indicated by in the entry of the scs-SpecificCarrierList.

supplementaryUplink Network may configure this field only when supplementaryUplinkConfig is configured in ServingCellConfigCommon or ServingCellConfigCommonSIB.

tag-Id Timing Advance Group ID, as specified in TS 38.321 [3], which this cell belongs to.

uplinkConfig Network may configure this field only when uplinkConfigCommon is configured in ServingCellConfigCommon or ServingCellConfigCommonSIB. Addition or release of this field can only be done upon SCell addition or release (respectively).

UplinkConfig field descriptions

carrierSwitching Includes parameters for configuration of carrier based SRS switching (see TS 38.214 [19], clause 6.2.1.3.

firstActiveUplinkBWP-Id If configured for an SpCell, this field contains the ID of the UL BWP to be activated upon performing the RRC (re-)configuration. If the field is absent, the RRC (re-)configuration does not impose a BWP switch. If configured for an SCell, this field contains the ID of the uplink bandwidth part to be used upon MAC-activation of an SCell. The initial bandwidth part is referred to by BandiwdthPartId = 0.

initialUplinkBWP The dedicated (UE-specific) configuration for the initial uplink bandwidth-part (i.e. UL BWP#0). If any of the optional IEs are configured within this IE as part of the IE uplinkConfig, the UE considers the BWP#0 to be an RRC configured BWP (from UE capability viewpoint). Otherwise, the UE does not consider the BWP#0 as an RRC configured BWP (from UE capability viewpoint). Network always configures the UE with a value for this field if no other BWPs are configured. NOTE1

powerBoostPi2BPSK If this field is set to true, the UE determines the maximum output power for PUCCH/PUSCH transmissions that use pi/2 BPSK modulation according to TS 38.101-1 [15], clause 6.2.4.

pusch-ServingCellConfig PUSCH related parameters that are not BWP-specific.

uplinkBWP-ToAddModList The additional bandwidth parts for uplink to be added or modified. In case of TDD uplink- and downlink BWP with the same bandwidthPartId are considered as a BWP pair and must have the same center frequency.

uplinkBWP-ToReleaseList The additional bandwidth parts for uplink to be released.

uplinkChannelBW-PerSCS-List A set of UE specific channel bandwidth and location configurations for different subcarrier spacings (numerologies). Defined in relation to Point A. The UE uses the configuration provided in this field only for the purpose of channel bandwidth and location determination. If absent, UE uses the configuration indicated in scs-SpecificCarrierList in UplinkConfigCommon / UplinkConfigCommonSIB. Network only configures channel bandwidth that corresponds to the channel bandwidth values defined in TS 38.101-1 [15] and TS 38.101-2 [39].

NOTE 1:	If the dedicated part of initial UL/DL BWP configuration is absent, the initial BWP can be used but with some limitations. For example, changing to another BWP requires RRCReconfiguration since DCI format 1_0 doesn't support DCI-based switching.

| Conditional Presence | Explanation |
| --- | --- |
| MeasObject | This field is mandatory present for the SpCell if the UE has a measConfig, and it is optionally present, Need M, for SCells. |
| SCellOnly | This field is optionally present, Need R, for SCells. It is absent otherwise. |
| ServingCellWithoutPUCCH | This field is optionally present, Need S, for SCells except PUCCH SCells. It is absent otherwise. |
| SyncAndCellAdd | This field is mandatory present for a SpCell upon PCell change and PSCell addition/change and upon RRCSetup/RRCResume. The field is mandatory present for an SCell upon addition. For SpCell, the field is optionally present, Need N, upon reconfiguration without reconfigurationWithSync. In all other cases the field is absent. |
| TDD | This field is optionally present, Need R, for TDD cells. It is absent otherwise. |

#### –	ServingCellConfigCommon

The IE ServingCellConfigCommon is used to configure cell specific parameters of a UE's serving cell. The IE contains parameters which a UE would typically acquire from SSB, MIB or SIBs when accessing the cell from IDLE. With this IE, the network provides this information in dedicated signalling when configuring a UE with a SCells or with an additional cell group (SCG). It also provides it for SpCells (MCG and SCG) upon reconfiguration with sync.

ServingCellConfigCommon information element

-- ASN1START

-- TAG-SERVINGCELLCONFIGCOMMON-START

ServingCellConfigCommon ::=         SEQUENCE {

physCellId                          PhysCellId                                                          OPTIONAL,   -- Cond HOAndServCellAdd,

downlinkConfigCommon                DownlinkConfigCommon                                                OPTIONAL,   -- Cond HOAndServCellAdd

uplinkConfigCommon                  UplinkConfigCommon                                                  OPTIONAL,   -- Need M

supplementaryUplinkConfig           UplinkConfigCommon                                                  OPTIONAL,   -- Need S

n-TimingAdvanceOffset               ENUMERATED { n0, n25600, n39936 }                                   OPTIONAL,   -- Need S

ssb-PositionsInBurst                CHOICE {

shortBitmap                         BIT STRING (SIZE (4)),

mediumBitmap                        BIT STRING (SIZE (8)),

longBitmap                          BIT STRING (SIZE (64))

}                                                                                                       OPTIONAL, -- Cond AbsFreqSSB

ssb-periodicityServingCell          ENUMERATED { ms5, ms10, ms20, ms40, ms80, ms160, spare2, spare1 }   OPTIONAL, -- Need S

dmrs-TypeA-Position                 ENUMERATED {pos2, pos3},

lte-CRS-ToMatchAround               SetupRelease { RateMatchPatternLTE-CRS }                            OPTIONAL, -- Need M

rateMatchPatternToAddModList        SEQUENCE (SIZE (1..maxNrofRateMatchPatterns)) OF RateMatchPattern   OPTIONAL, -- Need N

rateMatchPatternToReleaseList       SEQUENCE (SIZE (1..maxNrofRateMatchPatterns)) OF RateMatchPatternId OPTIONAL, -- Need N

ssbSubcarrierSpacing                SubcarrierSpacing                                                   OPTIONAL, -- Cond HOAndServCellWithSSB

tdd-UL-DL-ConfigurationCommon       TDD-UL-DL-ConfigCommon                                              OPTIONAL, -- Cond TDD

ss-PBCH-BlockPower                  INTEGER (-60..50),

...

}

-- TAG-SERVINGCELLCONFIGCOMMON-STOP

-- ASN1STOP

ServingCellConfigCommon field descriptions

dmrs-TypeA-Position Position of (first) DM-RS for downlink (see TS 38.211 [16], clause 7.4.1.1.1) and uplink (TS 38.211 [16], clause 6.4.1.1.3).

downlinkConfigCommon The common downlink configuration of the serving cell, including the frequency information configuration and the initial downlink BWP common configuration. The parameters provided herein should match the parameters configured by MIB and SIB1 (if provided) of the serving cell, with the exception of controlResourceSetZero and searchSpaceZero which can be configured in ServingCellConfigCommon even if MIB indicates that they are absent.

longBitmap Bitmap when maximum number of SS/PBCH blocks per half frame equals to 64 as defined in TS 38.213 [13], clause 4.1.

lte-CRS-ToMatchAround Parameters to determine an LTE CRS pattern that the UE shall rate match around.

mediumBitmap Bitmap when maximum number of SS/PBCH blocks per half frame equals to 8 as defined in TS 38.213 [13], clause 4.1.

n-TimingAdvanceOffset The N_TA-Offset to be applied for all uplink transmissions on this serving cell. If the field is absent, the UE applies the value defined for the duplex mode and frequency range of this serving cell. See TS 38.133 [14], table 7.1.2-2.

rateMatchPatternToAddModList Resources patterns which the UE should rate match PDSCH around. The UE rate matches around the union of all resources indicated in the rate match patterns. Rate match patterns defined here on cell level apply only to PDSCH of the same numerology (see TS 38.214 [19], clause 5.1.4,1).

shortBitmap Bitmap when maximum number of SS/PBCH blocks per half frame equals to 4 as defined in TS 38.213 [13], clause 4.1.

ss-PBCH-BlockPower Average EPRE of the resources elements that carry secondary synchronization signals in dBm that the NW used for SSB transmission, see TS 38.213 [13], clause 7.

ssb-periodicityServingCell The SSB periodicity in ms for the rate matching purpose. If the field is absent, the UE applies the value ms5. (see TS 38.213 [13], clause 4.1)

ssb-PositionsInBurst Indicates the time domain positions of the transmitted SS-blocks in a half frame with SS/PBCH blocks as defined in TS 38.213 [13], clause 4.1. The first/ leftmost bit corresponds to SS/PBCH block index 0, the second bit corresponds to SS/PBCH block index 1, and so on. Value 0 in the bitmap indicates that the corresponding SS/PBCH block is not transmitted while value 1 indicates that the corresponding SS/PBCH block is transmitted. The network configures the same pattern in this field as in the corresponding field in ServingCellConfigCommonSIB.

ssbSubcarrierSpacing Subcarrier spacing of SSB. Only the values 15 kHz or 30 kHz (FR1), and 120 kHz or 240 kHz (FR2) are applicable.

supplementaryUplinkConfig The network configures this field only if uplinkConfigCommon is configured. If this field is absent, the UE shall release the supplementaryUplinkConfig and the supplementaryUplink configured in ServingCellConfig of this serving cell, if configured.

tdd-UL-DL-ConfigurationCommon A cell-specific TDD UL/DL configuration, see TS 38.213 [13], clause 11.1.

| Conditional Presence | Explanation |
| --- | --- |
| AbsFreqSSB | The field is absent when absoluteFrequencySSB in frequencyInfoDL is absent, otherwise the field is mandatory present. |
| HOAndServCellAdd | This field is mandatory present upon SpCell change and upon serving cell (PSCell/SCell) addition. Otherwise, the field is absent. |
| HOAndServCellWithSSB | This field is mandatory present upon SpCell change and upon serving cell (SCell with SSB or PSCell) addition. Otherwise, the field is absent. |
| TDD | The field is optionally present, Need R, for TDD cells; otherwise it is absent. |

#### –	ServingCellConfigCommonSIB

The IE ServingCellConfigCommonSIB is used to configure cell specific parameters of a UE's serving cell in SIB1.

ServingCellConfigCommonSIB information element

-- ASN1START

-- TAG-SERVINGCELLCONFIGCOMMONSIB-START

ServingCellConfigCommonSIB ::=      SEQUENCE {

downlinkConfigCommon                DownlinkConfigCommonSIB,

uplinkConfigCommon                  UplinkConfigCommonSIB                                       OPTIONAL, -- Need R

supplementaryUplink                 UplinkConfigCommonSIB                                       OPTIONAL, -- Need R

n-TimingAdvanceOffset               ENUMERATED { n0, n25600, n39936 }                           OPTIONAL, -- Need S

ssb-PositionsInBurst                SEQUENCE {

inOneGroup                          BIT STRING (SIZE (8)),

groupPresence                       BIT STRING (SIZE (8))                                   OPTIONAL  -- Cond FR2-Only

},

ssb-PeriodicityServingCell          ENUMERATED {ms5, ms10, ms20, ms40, ms80, ms160},

tdd-UL-DL-ConfigurationCommon       TDD-UL-DL-ConfigCommon                                      OPTIONAL, -- Cond TDD

ss-PBCH-BlockPower                  INTEGER (-60..50),

...

}

-- TAG-SERVINGCELLCONFIGCOMMONSIB-STOP

-- ASN1STOP

ServingCellConfigCommonSIB field descriptions

groupPresence This field is present when maximum number of SS/PBCH blocks per half frame equals to 64 as defined in TS 38.213 [13], clause 4.1. The first/leftmost bit corresponds to the SS/PBCH index 0-7, the second bit corresponds to SS/PBCH block 8-15, and so on. Value 0 in the bitmap indicates that the SSBs according to inOneGroup are absent. Value 1 indicates that the SS/PBCH blocks are transmitted in accordance with inOneGroup.

inOneGroup When maximum number of SS/PBCH blocks per half frame equals to 4 as defined in TS 38.213 [13], clause 4.1, only the 4 leftmost bits are valid; the UE ignores the 4 rightmost bits. When maximum number of SS/PBCH blocks per half frame equals to 8 as defined in TS 38.213 [13], clause 4.1, all 8 bits are valid. The first/ leftmost bit corresponds to SS/PBCH block index 0, the second bit corresponds to SS/PBCH block index 1, and so on. When maximum number of SS/PBCH blocks per half frame equals to 64 as defined in TS 38.213 [13], clause 4.1, all 8 bit are valid; The first/ leftmost bit corresponds to the first SS/PBCH block index in the group (i.e., to SSB index 0, 8, and so on); the second bit corresponds to the second SS/PBCH block index in the group (i.e., to SSB index 1, 9, and so on), and so on. Value 0 in the bitmap indicates that the corresponding SS/PBCH block is not transmitted while value 1 indicates that the corresponding SS/PBCH block is transmitted.

n-TimingAdvanceOffset The N_TA-Offset to be applied for random access on this serving cell. If the field is absent, the UE applies the value defined for the duplex mode and frequency range of this serving cell. See TS 38.133 [14], table 7.1.2-2.

ssb-PositionsInBurst Time domain positions of the transmitted SS-blocks in an SS-burst as defined in TS 38.213 [13], clause 4.1.

ss-PBCH-BlockPower Average EPRE of the resources elements that carry secondary synchronization signals in dBm that the NW used for SSB transmission, see TS 38.213 [13], clause 7.

| Conditional Presence | Explanation |
| --- | --- |
| FR2-Only | This field is mandatory present for an FR2 carrier frequency. It is absent otherwise and UE releases any configured value. |
| TDD | The field is optionally present, Need R, for TDD cells; otherwise it is absent. |

#### –	ShortI-RNTI-Value

The IE ShortI-RNTI-Value is used to identify the suspended UE context of a UE in RRC_INACTIVE using fewer bits compared to I-RNTI-Value.

ShortI-RNTI-Value information element

-- ASN1START

-- TAG-SHORTI-RNTI-VALUE-START

ShortI-RNTI-Value ::=   BIT STRING (SIZE(24))

-- TAG-SHORTI-RNTI-VALUE-STOP

-- ASN1STOP

#### –	ShortMAC-I

The IE ShortMAC-I is used to identify and verify the UE at RRC connection re-establishment. The 16 least significant bits of the MAC-I calculated using the AS security configuration of the source PCell, as specified in 5.3.7.4.

ShortMAC-I information element

-- ASN1START

-- TAG-SHORTMAC-I-START

ShortMAC-I ::=                      BIT STRING (SIZE (16))

-- TAG-SHORTMAC-I-STOP

-- ASN1STOP

#### –	SINR-Range

The IE SINR-Range specifies the value range used in SINR measurements and thresholds. For measurements, integer value for SINR measurements is according to Table 10.1.16.1-1 in TS 38.133 [14]. For thresholds, the actual value is (IE value – 46) / 2 dB.

SINR-Range information element

-- ASN1START

-- TAG-SINR-RANGE-START

SINR-Range ::=                      INTEGER(0..127)

-- TAG-SINR-RANGE-STOP

-- ASN1STOP

#### –	SI-SchedulingInfo

The IE SI-SchedulingInfo contains information needed for acquisition of SI messages.

SI-SchedulingInfo information element

-- ASN1START

-- TAG–SI-SCHEDULINGINFO-START

SI-SchedulingInfo ::=               SEQUENCE {

schedulingInfoList                  SEQUENCE (SIZE (1..maxSI-Message)) OF SchedulingInfo,

si-WindowLength                     ENUMERATED {s5, s10, s20, s40, s80, s160, s320, s640, s1280},

si-RequestConfig                    SI-RequestConfig                                                OPTIONAL,  -- Cond MSG-1

si-RequestConfigSUL                 SI-RequestConfig                                                OPTIONAL,  -- Cond SUL-MSG-1

systemInformationAreaID             BIT STRING (SIZE (24))                                          OPTIONAL,   -- Need R

...

}

SchedulingInfo ::=                  SEQUENCE {

si-BroadcastStatus                  ENUMERATED {broadcasting, notBroadcasting},

si-Periodicity                      ENUMERATED {rf8, rf16, rf32, rf64, rf128, rf256, rf512},

sib-MappingInfo                     SIB-Mapping

}

SIB-Mapping ::=                     SEQUENCE (SIZE (1..maxSIB)) OF SIB-TypeInfo

SIB-TypeInfo ::=                    SEQUENCE {

type                                ENUMERATED {sibType2, sibType3, sibType4, sibType5, sibType6, sibType7, sibType8, sibType9,

spare8, spare7, spare6, spare5, spare4, spare3, spare2, spare1,... },

valueTag                            INTEGER (0..31)                                                  OPTIONAL, -- Cond SIB-TYPE

areaScope                           ENUMERATED {true}                                                OPTIONAL -- Need S

}

-- Configuration for Msg1 based SI Request

SI-RequestConfig::=                 SEQUENCE {

rach-OccasionsSI                    SEQUENCE {

rach-ConfigSI                       RACH-ConfigGeneric,

ssb-perRACH-Occasion                ENUMERATED {oneEighth, oneFourth, oneHalf, one, two, four, eight, sixteen}

}                                                                                                       OPTIONAL,   -- Need R

si-RequestPeriod                    ENUMERATED {one, two, four, six, eight, ten, twelve, sixteen}       OPTIONAL,   -- Need R

si-RequestResources                 SEQUENCE (SIZE (1..maxSI-Message)) OF SI-RequestResources

}

SI-RequestResources ::=             SEQUENCE {

ra-PreambleStartIndex               INTEGER (0..63),

ra-AssociationPeriodIndex           INTEGER (0..15)                                                     OPTIONAL,   -- Need R

ra-ssb-OccasionMaskIndex            INTEGER (0..15)                                                     OPTIONAL    -- Need R

}

-- TAG-SI-SCHEDULINGINFO-STOP

-- ASN1STOP

SchedulingInfo field descriptions

areaScope Indicates that a SIB is area specific. If the field is absent, the SIB is cell specific.

si-BroadcastStatus Indicates if the SI message is being broadcasted or not. Change of si-BroadcastStatus should not result in system information change notifications in Short Message transmitted with P-RNTI over DCI (see clause 6.5). The value of the indication is valid until the end of the BCCH modification period when set to broadcasting.

si-Periodicity Periodicity of the SI-message in radio frames. Value rf8 corresponds to 8 radio frames, value rf16 corresponds to 16 radio frames, and so on.

SI-RequestConfig field descriptions

rach-OccasionsSI Configuration of dedicated RACH Occassions for SI. If the field is absent, the UE uses the corresponding parameters configured in rach-ConfigCommon of the initial uplink BWP.

si-RequestPeriod Periodicity of the SI-Request configuration in number of association periods.

si-RequestResources If there is only one entry in the list, the configuration is used for all SI messages for which si-BroadcastStatus is set to notBroadcasting. Otherwise the 1st entry in the list corresponds to the first SI message in schedulingInfoList for which si-BroadcastStatus is set to notBroadcasting, 2nd entry in the list corresponds to the second SI message in schedulingInfoList for which si-BroadcastStatus is set to notBroadcasting and so on. Change of si-RequestResources should not result in system information change notification.

SI-RequestResources field descriptions

ra-AssociationPeriodIndex Index of the association period in the si-RequestPeriod in which the UE can send the SI request for SI message(s) corresponding to this SI-RequestResources, using the preambles indicated by ra-PreambleStartIndex and rach occasions indicated by ra-ssb-OccasionMaskIndex.

ra-PreambleStartIndex If N SSBs are associated with a RACH occasion, where N > = 1, for the i-th SSB (i=0, …, N-1) the preamble with preamble index = ra-PreambleStartIndex + i is used for SI request; For N < 1, the preamble with preamble index = ra-PreambleStartIndex is used for SI request.

SI-SchedulingInfo field descriptions

si-RequestConfig Configuration of Msg1 resources that the UE uses for requesting SI-messages for which si-BroadcastStatus is set to notBroadcasting.

si-RequestConfigSUL Configuration of Msg1 resources that the UE uses for requesting SI-messages for which si-BroadcastStatus is set to notBroadcasting.

si-WindowLength The length of the SI scheduling window. Value s5 corresponds to 5 slots, value s10 corresponds to 10 slots and so on. The network always configures si-WindowLength to be shorter than or equal to the si-Periodicity.

systemInformationAreaID Indicates the system information area that the cell belongs to, if any. Any SIB with areaScope within the SI is considered to belong to this systemInformationAreaID. The systemInformationAreaID is unique within a PLMN.

| Conditional presence | Explanation |
| --- | --- |
| MSG-1 | The field is optionally present, Need R, if si-BroadcastStatus is set to notBroadcasting for any SI-message included in SchedulingInfo. It is absent otherwise. |
| SIB-TYPE | The field is mandatory present if the SIB type is different from SIB6, SIB7 or SIB8. For SIB6, SIB7 and SIB8 it is absent. |
| SUL-MSG-1 | The field is optionally present, Need R, if this serving cell is configured with a supplementary uplink and if si-BroadcastStatus is set to notBroadcasting for any SI-message included in SchedulingInfo. It is absent otherwise. |

#### –	SK-Counter

The IE SK-Counter is a counter used upon initial configuration of SN security for NR-DC and NE-DC, as well as upon refresh of S-KgNB or S-KeNB based on the current or newly derived KgNB during RRC Resume or RRC Reconfiguration, as defined in TS 33.501 [11].

-- ASN1START

-- TAG-SKCOUNTER-START

SK-Counter ::=  INTEGER (0..65535)

-- TAG-SKCOUNTER-STOP

-- ASN1STOP

#### –	SlotFormatCombinationsPerCell

The IE SlotFormatCombinationsPerCell is used to configure the SlotFormatCombinations applicable for one serving cell (see TS 38.213 [13], clause 11.1.1).

SlotFormatCombinationsPerCell information element

-- ASN1START

-- TAG-SLOTFORMATCOMBINATIONSPERCELL-START

SlotFormatCombinationsPerCell ::=   SEQUENCE {

servingCellId                       ServCellIndex,

subcarrierSpacing                   SubcarrierSpacing,

subcarrierSpacing2                  SubcarrierSpacing                                                         OPTIONAL, -- Need R

slotFormatCombinations              SEQUENCE (SIZE (1..maxNrofSlotFormatCombinationsPerSet)) OF SlotFormatCombination

OPTIONAL, -- Need M

positionInDCI                       INTEGER(0..maxSFI-DCI-PayloadSize-1)                                      OPTIONAL, -- Need M

...

}

SlotFormatCombination ::=           SEQUENCE {

slotFormatCombinationId             SlotFormatCombinationId,

slotFormats                         SEQUENCE (SIZE (1..maxNrofSlotFormatsPerCombination)) OF INTEGER (0..255)

}

SlotFormatCombinationId ::=         INTEGER (0..maxNrofSlotFormatCombinationsPerSet-1)

-- TAG-SLOTFORMATCOMBINATIONSPERCELL-STOP

-- ASN1STOP

SlotFormatCombination field descriptions

slotFormatCombinationId This ID is used in the DCI payload to dynamically select this SlotFormatCombination, see TS 38.213 [13], clause 11.1.1.

slotFormats Slot formats that occur in consecutive slots in time domain order as listed here (see TS 38.213 [13], clause 11.1.1).

SlotFormatCombinationsPerCell field descriptions

positionInDCI The (starting) position (bit) of the slotFormatCombinationId (SFI-Index) for this serving cell (servingCellId) within the DCI payload (see TS 38.213 [13], clause 11.1.1).

servingCellId The ID of the serving cell for which the slotFormatCombinations are applicable.

slotFormatCombinations A list with SlotFormatCombinations. Each SlotFormatCombination comprises of one or more SlotFormats (see TS 38.211 [16], clause 4.3.2). The total number of slotFormats in the slotFormatCombinations list does not exceed 512.

subcarrierSpacing2 Reference subcarrier spacing for a Slot Format Combination on an FDD or SUL cell (see TS 38.213 [13], clause 11.1.1). For FDD, subcarrierSpacing (SFI-scs) is the reference SCS for DL BWP and subcarrierSpacing2 (SFI-scs2) is the reference SCS for UL BWP. For SUL, subcarrierSpacing (SFI-scs) is the reference SCS for non-SUL carrier and subcarrierSpacing2 (SFI-scs2) is the reference SCS for SUL carrier. The network configures a value that is smaller than or equal to any SCS of configured BWPs of the serving cell that the command applies to. And the network configures a value that is smaller than or equal to the SCS of the serving cell which the UE monitors for SFI indications.

subcarrierSpacing Reference subcarrier spacing for this Slot Format Combination. The network configures a value that is smaller than or equal to any SCS of configured BWPs of the serving cell that the command applies to. And the network configures a value that is smaller than or equal to the SCS of the serving cell which the UE monitors for SFI indications (see TS 38.213 [13], clause 11.1.1).

#### –	SlotFormatIndicator

The IE SlotFormatIndicator is used to configure monitoring a Group-Common-PDCCH for Slot-Format-Indicators (SFI).

SlotFormatIndicator information element

-- ASN1START

-- TAG-SLOTFORMATINDICATOR-START

SlotFormatIndicator ::=     SEQUENCE {

sfi-RNTI                    RNTI-Value,

dci-PayloadSize             INTEGER (1..maxSFI-DCI-PayloadSize),

slotFormatCombToAddModList  SEQUENCE (SIZE(1..maxNrofAggregatedCellsPerCellGroup)) OF SlotFormatCombinationsPerCell

OPTIONAL, -- Need N

slotFormatCombToReleaseList SEQUENCE (SIZE(1..maxNrofAggregatedCellsPerCellGroup)) OF ServCellIndex                 OPTIONAL, -- Need N

...

}

-- TAG-SLOTFORMATINDICATOR-STOP

-- ASN1STOP

SlotFormatIndicator field descriptions

dci-PayloadSize Total length of the DCI payload scrambled with SFI-RNTI (see TS 38.213 [13], clause 11.1.1).

sfi-RNTI RNTI used for SFI on the given cell (see TS 38.213 [13], clause 11.1.1).

slotFormatCombToAddModList A list of SlotFormatCombinations for the UE's serving cells (see TS 38.213 [13], clause 11.1.1).

#### –	S-NSSAI

The IE S-NSSAI (Single Network Slice Selection Assistance Information) identifies a Network Slice end to end and comprises a slice/service type and a slice differentiator, see TS 23.003 [21].

S-NSSAI information element

-- ASN1START

-- TAG-S-NSSAI-START

S-NSSAI  ::=                        CHOICE{

sst                                 BIT STRING (SIZE (8)),

sst-SD                              BIT STRING (SIZE (32))

}

-- TAG-S-NSSAI-STOP

-- ASN1STOP

S-NSSAI field descriptions

sst Indicates the S-NSSAI consisting of Slice/Service Type, see TS 23.003 [21].

sst-SD Indicates the S-NSSAI consisting of Slice/Service Type and Slice Differentiator, see TS 23.003 [21].

#### –	SpeedStateScaleFactors

The IE SpeedStateScaleFactors concerns factors, to be applied when the UE is in medium or high speed state, used for scaling a mobility control related parameter.

SpeedStateScaleFactors information element

-- ASN1START

-- TAG-SPEEDSTATESCALEFACTORS-START

SpeedStateScaleFactors ::=          SEQUENCE {

sf-Medium                           ENUMERATED {oDot25, oDot5, oDot75, lDot0},

sf-High                             ENUMERATED {oDot25, oDot5, oDot75, lDot0}

}

-- TAG-SPEEDSTATESCALEFACTORS-STOP

-- ASN1STOP

SpeedStateScaleFactors field descriptions

sf-High The concerned mobility control related parameter is multiplied with this factor if the UE is in High Mobility state as defined in TS 38.304 [20]. Value oDot25 corresponds to 0.25, value oDot5 corresponds to 0.5, oDot75 corresponds to 0.75 and so on.

sf-Medium The concerned mobility control related parameter is multiplied with this factor if the UE is in Medium Mobility state as defined in TS 38.304 [20]. Value oDot25 corresponds to 0.25, value oDot5 corresponds to 0.5, value oDot75 corresponds to 0.75, and so on.

#### –	SPS-Config

The IE SPS-Config is used to configure downlink semi-persistent transmission. Downlink SPS may be configured on the SpCell as well as on SCells. The network ensures SPS-Config is configured for at most one cell in a cell group.

SPS-Config information element

-- ASN1START

-- TAG-SPS-CONFIG-START

SPS-Config ::=                          SEQUENCE {

periodicity                             ENUMERATED {ms10, ms20, ms32, ms40, ms64, ms80, ms128, ms160, ms320, ms640,

spare6, spare5, spare4, spare3, spare2, spare1},

nrofHARQ-Processes                      INTEGER (1..8),

n1PUCCH-AN                              PUCCH-ResourceId                                                            OPTIONAL,   -- Need M

mcs-Table                               ENUMERATED {qam64LowSE}                                                     OPTIONAL,   -- Need S

...

}

-- TAG-SPS-CONFIG-STOP

-- ASN1STOP

SPS-Config field descriptions

mcs-Table Indicates the MCS table the UE shall use for DL SPS (see TS 38.214 [19],clause 5.1.3.1. If present, the UE shall use the MCS table of low-SE 64QAM table indicated in Table 5.1.3.1-3 of TS 38.214 [19]. If this field is absent and field mcs-table in PDSCH-Config is set to 'qam256' and the activating DCI is of format 1_1, the UE applies the 256QAM table indicated in Table 5.1.3.1-2 of TS 38.214 [19]. Otherwise, the UE applies the non-low-SE 64QAM table indicated in Table 5.1.3.1-1 of TS 38.214 [19].

n1PUCCH-AN HARQ resource for PUCCH for DL SPS. The network configures the resource either as format0 or format1. The actual PUCCH-Resource is configured in PUCCH-Config and referred to by its ID. See TS 38.213 [13], clause 9.2.3.

nrofHARQ-Processes Number of configured HARQ processes for SPS DL (see TS 38.321 [3], clause 5.8.1).

periodicity Periodicity for DL SPS (see TS 38.214 [19] and TS 38.321 [3], clause 5.8.1).

#### –	SRB-Identity

The IE SRB-Identity is used to identify a Signalling Radio Bearer (SRB) used by a UE.

SRB-Identity information element

-- ASN1START

-- TAG-SRB-IDENTITY-START

SRB-Identity ::=                    INTEGER (1..3)

-- TAG-SRB-IDENTITY-STOP

-- ASN1STOP

#### –	SRS-CarrierSwitching

The IE SRS-CarrierSwitching is used to configure for SRS carrier switching when PUSCH is not configured and independent SRS power control from that of PUSCH.

SRS-CarrierSwitching information element

-- ASN1START

-- TAG-SRS-CARRIERSWITCHING-START

SRS-CarrierSwitching ::=            SEQUENCE {

srs-SwitchFromServCellIndex         INTEGER (0..31)                                                         OPTIONAL,   -- Need M

srs-SwitchFromCarrier               ENUMERATED {sUL, nUL},

srs-TPC-PDCCH-Group                 CHOICE {

typeA                               SEQUENCE (SIZE (1..32)) OF SRS-TPC-PDCCH-Config,

typeB                               SRS-TPC-PDCCH-Config

}                                                                                                           OPTIONAL,   -- Need M

monitoringCells                     SEQUENCE (SIZE (1..maxNrofServingCells)) OF ServCellIndex               OPTIONAL,   -- Need M

...

}

SRS-TPC-PDCCH-Config ::=            SEQUENCE {

srs-CC-SetIndexlist                 SEQUENCE (SIZE(1..4)) OF SRS-CC-SetIndex                                OPTIONAL    -- Need M

}

SRS-CC-SetIndex ::=                 SEQUENCE {

cc-SetIndex                         INTEGER (0..3)                                                          OPTIONAL,   -- Need M

cc-IndexInOneCC-Set                 INTEGER (0..7)                                                          OPTIONAL    -- Need M

}

-- TAG-SRS-CARRIERSWITCHING-STOP

-- ASN1STOP

SRS-CC-SetIndex field descriptions

cc-IndexInOneCC-Set Indicates the CC index in one CC set for Type A (see TS 38.212 [17], TS 38.213 [13], clause 7.3.1, 11.4). The network always includes this field when the srs-TPC-PDCCH-Group is set to typeA.

cc-SetIndex Indicates the CC set index for Type A associated (see TS 38.212 [17], TS 38.213 [13], clause 7.3.1, 11.4). The network always includes this field when the srs-TPC-PDCCH-Group is set to typeA. The network does not configure this field to 3 in this release of specification.

SRS-CarrierSwitching field descriptions

monitoringCells A set of serving cells for monitoring PDCCH conveying SRS DCI format with CRC scrambled by TPC-SRS-RNTI (see TS 38.212 [17], TS 38.213 [13], clause 7.3.1, 11.3).

srs-SwitchFromServCellIndex Indicates the serving cell whose UL transmission may be interrupted during SRS transmission on a PUSCH-less SCell. During SRS transmission on a PUSCH-less SCell, the UE may temporarily suspend the UL transmission on a serving cell with PUSCH in the same CG to allow the PUSCH-less SCell to transmit SRS. (see TS 38.214 [19], clause 6.2.1.3).

srs-TPC-PDCCH-Group Network configures the UE with either typeA-SRS-TPC-PDCCH-Group or typeB-SRS-TPC-PDCCH-Group, if any.

typeA Type A trigger configuration for SRS transmission on a PUSCH-less SCell (see TS 38.213 [13], clause 11.4). In this release, the network configures at most one entry (the first entry) of typeA, and the first entry corresponds to the uplink carrier in which the SRS-CarrierSwitching field is configured.

typeB Type B trigger configuration for SRS transmission on a PUSCH-less SCell (see TS 38.213 [13], clause 11.4).

SRS-TPC-PDCCH-Config field descriptions

srs-CC-SetIndexlist A list of pairs of [cc-SetIndex; cc-IndexInOneCC-Set] (see TS 38.212 [17], TS 38.213 [13], clause 7.3.1, 11.4). The network does not configure this field for typeB.

#### –	SRS-Config

The IE SRS-Config is used to configure sounding reference signal transmissions. The configuration defines a list of SRS-Resources and a list of SRS-ResourceSets. Each resource set defines a set of SRS-Resources. The network triggers the transmission of the set of SRS-Resources using a configured aperiodicSRS-ResourceTrigger (L1 DCI).

SRS-Config information element

-- ASN1START

-- TAG-SRS-CONFIG-START

SRS-Config ::=                          SEQUENCE {

srs-ResourceSetToReleaseList            SEQUENCE (SIZE(1..maxNrofSRS-ResourceSets)) OF SRS-ResourceSetId    OPTIONAL,   -- Need N

srs-ResourceSetToAddModList             SEQUENCE (SIZE(1..maxNrofSRS-ResourceSets)) OF SRS-ResourceSet      OPTIONAL,   -- Need N

srs-ResourceToReleaseList               SEQUENCE (SIZE(1..maxNrofSRS-Resources)) OF SRS-ResourceId          OPTIONAL,   -- Need N

srs-ResourceToAddModList                SEQUENCE (SIZE(1..maxNrofSRS-Resources)) OF SRS-Resource            OPTIONAL,   -- Need N

tpc-Accumulation                        ENUMERATED {disabled}                                               OPTIONAL,   -- Need S

...

}

SRS-ResourceSet ::=                     SEQUENCE {

srs-ResourceSetId                       SRS-ResourceSetId,

srs-ResourceIdList                      SEQUENCE (SIZE(1..maxNrofSRS-ResourcesPerSet)) OF SRS-ResourceId    OPTIONAL, -- Cond Setup

resourceType                            CHOICE {

aperiodic                               SEQUENCE {

aperiodicSRS-ResourceTrigger            INTEGER (1..maxNrofSRS-TriggerStates-1),

csi-RS                                  NZP-CSI-RS-ResourceId                               OPTIONAL, -- Cond NonCodebook

slotOffset                              INTEGER (1..32)                                     OPTIONAL, -- Need S

...,

[[

aperiodicSRS-ResourceTriggerList            SEQUENCE (SIZE(1..maxNrofSRS-TriggerStates-2))

OF INTEGER (1..maxNrofSRS-TriggerStates-1)  OPTIONAL  -- Need M

]]

},

semi-persistent                         SEQUENCE {

associatedCSI-RS                        NZP-CSI-RS-ResourceId                               OPTIONAL, -- Cond NonCodebook

...

},

periodic                                SEQUENCE {

associatedCSI-RS                        NZP-CSI-RS-ResourceId                               OPTIONAL, -- Cond NonCodebook

...

}

},

usage                                   ENUMERATED {beamManagement, codebook, nonCodebook, antennaSwitching},

alpha                                   Alpha                                                       OPTIONAL, -- Need S

p0                                      INTEGER (-202..24)                                          OPTIONAL, -- Cond Setup

pathlossReferenceRS                     CHOICE {

ssb-Index                               SSB-Index,

csi-RS-Index                            NZP-CSI-RS-ResourceId

}                                                                                                   OPTIONAL, -- Need M

srs-PowerControlAdjustmentStates        ENUMERATED { sameAsFci2, separateClosedLoop}                OPTIONAL, -- Need S

...

}

SRS-ResourceSetId ::=                   INTEGER (0..maxNrofSRS-ResourceSets-1)

SRS-Resource ::=                        SEQUENCE {

srs-ResourceId                          SRS-ResourceId,

nrofSRS-Ports                           ENUMERATED {port1, ports2, ports4},

ptrs-PortIndex                          ENUMERATED {n0, n1 }                                       OPTIONAL,   -- Need R

transmissionComb                        CHOICE {

n2                                      SEQUENCE {

combOffset-n2                           INTEGER (0..1),

cyclicShift-n2                          INTEGER (0..7)

},

n4                                      SEQUENCE {

combOffset-n4                           INTEGER (0..3),

cyclicShift-n4                          INTEGER (0..11)

}

},

resourceMapping                         SEQUENCE {

startPosition                           INTEGER (0..5),

nrofSymbols                             ENUMERATED {n1, n2, n4},

repetitionFactor                        ENUMERATED {n1, n2, n4}

},

freqDomainPosition                      INTEGER (0..67),

freqDomainShift                         INTEGER (0..268),

freqHopping                             SEQUENCE {

c-SRS                                   INTEGER (0..63),

b-SRS                                   INTEGER (0..3),

b-hop                                   INTEGER (0..3)

},

groupOrSequenceHopping                  ENUMERATED { neither, groupHopping, sequenceHopping },

resourceType                            CHOICE {

aperiodic                               SEQUENCE {

...

},

semi-persistent                         SEQUENCE {

periodicityAndOffset-sp                     SRS-PeriodicityAndOffset,

...

},

periodic                                SEQUENCE {

periodicityAndOffset-p                      SRS-PeriodicityAndOffset,

...

}

},

sequenceId                              INTEGER (0..1023),

spatialRelationInfo                     SRS-SpatialRelationInfo                                 OPTIONAL,   -- Need R

...

}

SRS-SpatialRelationInfo ::=     SEQUENCE {

servingCellId                       ServCellIndex                                               OPTIONAL,   -- Need S

referenceSignal                     CHOICE {

ssb-Index                           SSB-Index,

csi-RS-Index                        NZP-CSI-RS-ResourceId,

srs                                 SEQUENCE {

resourceId                          SRS-ResourceId,

uplinkBWP                           BWP-Id

}

}

}

SRS-ResourceId ::=                      INTEGER (0..maxNrofSRS-Resources-1)

SRS-PeriodicityAndOffset ::=            CHOICE {

sl1                                     NULL,

sl2                                     INTEGER(0..1),

sl4                                     INTEGER(0..3),

sl5                                     INTEGER(0..4),

sl8                                     INTEGER(0..7),

sl10                                    INTEGER(0..9),

sl16                                    INTEGER(0..15),

sl20                                    INTEGER(0..19),

sl32                                    INTEGER(0..31),

sl40                                    INTEGER(0..39),

sl64                                    INTEGER(0..63),

sl80                                    INTEGER(0..79),

sl160                                   INTEGER(0..159),

sl320                                   INTEGER(0..319),

sl640                                   INTEGER(0..639),

sl1280                                  INTEGER(0..1279),

sl2560                                  INTEGER(0..2559)

}

-- TAG-SRS-CONFIG-STOP

-- ASN1STOP

SRS-Config field descriptions

tpc-Accumulation If the field is absent, UE applies TPC commands via accumulation. If disabled, UE applies the TPC command without accumulation (this applies to SRS when a separate closed loop is configured for SRS) (see TS 38.213 [13], clause 7.3).

SRS-Resource field descriptions

cyclicShift-n2 Cyclic shift configuration (see TS 38.214 [19], clause 6.2.1).

cyclicShift-n4 Cyclic shift configuration (see TS 38.214 [19], clause 6.2.1).

freqHopping Includes parameters capturing SRS frequency hopping (see TS 38.214 [19], clause 6.2.1).

groupOrSequenceHopping Parameter(s) for configuring group or sequence hopping (see TS 38.211 [16], clause 6.4.1.4.2).

periodicityAndOffset-p Periodicity and slot offset for this SRS resource. All values are in "number of slots". Value sl1 corresponds to a periodicity of 1 slot, value sl2 corresponds to a periodicity of 2 slots, and so on. For each periodicity the corresponding offset is given in number of slots. For periodicity sl1 the offset is 0 slots (see TS 38.214 [19], clause 6.2.1).

periodicityAndOffset-sp Periodicity and slot offset for this SRS resource. All values are in "number of slots". Value sl1 corresponds to a periodicity of 1 slot, value sl2 corresponds to a periodicity of 2 slots, and so on. For each periodicity the corresponding offset is given in number of slots. For periodicity sl1 the offset is 0 slots (see TS 38.214 [19], clause 6.2.1).

ptrs-PortIndex The PTRS port index for this SRS resource for non-codebook based UL MIMO. This is only applicable when the corresponding PTRS-UplinkConfig is set to CP-OFDM. The ptrs-PortIndex configured here must be smaller than the maxNrofPorts configured in the PTRS-UplinkConfig (see TS 38.214 [19], clause 6.2.3.1).

resourceMapping OFDM symbol location of the SRS resource within a slot including nrofSymbols (number of OFDM symbols), startPosition (value 0 refers to the last symbol, value 1 refers to the second last symbol, and so on) and repetitionFactor (see TS 38.214 [19], clause 6.2.1 and TS 38.211 [16], clause 6.4.1.4). The configured SRS resource does not exceed the slot boundary.

resourceType Periodicity and offset for semi-persistent and periodic SRS resource (see TS 38.214 [19], clause 6.2.1).

sequenceId Sequence ID used to initialize pseudo random group and sequence hopping (see TS 38.214 [19], clause 6.2.1).

spatialRelationInfo Configuration of the spatial relation between a reference RS and the target SRS. Reference RS can be SSB/CSI-RS/SRS (see TS 38.214 [19], clause 6.2.1).

transmissionComb Comb value (2 or 4) and comb offset (0..combValue-1) (see TS 38.214 [19], clause 6.2.1).

SRS-ResourceSet field descriptions

alpha alpha value for SRS power control (see TS 38.213 [13], clause 7.3). When the field is absent the UE applies the value 1.

aperiodicSRS-ResourceTriggerList An additional list of DCI "code points" upon which the UE shall transmit SRS according to this SRS resource set configuration (see TS 38.214 [19], clause 6.1.1.2). When the field is not included during a reconfiguration of SRS-ResourceSet of resourceType set to aperiodic, UE maintains this value based on the Need M; that is, this list is not considered as an extension of aperiodicSRS-ResourceTrigger for purpose of applying the general rule for extended list in clause 6.1.3.

aperiodicSRS-ResourceTrigger The DCI "code point" upon which the UE shall transmit SRS according to this SRS resource set configuration (see TS 38.214 [19], clause 6.1.1.2).

associatedCSI-RS ID of CSI-RS resource associated with this SRS resource set in non-codebook based operation (see TS 38.214 [19], clause 6.1.1.2).

csi-RS ID of CSI-RS resource associated with this SRS resource set. (see TS 38.214 [19], clause 6.1.1.2).

p0 P0 value for SRS power control. The value is in dBm. Only even values (step size 2) are allowed (see TS 38.213 [13], clause 7.3).

pathlossReferenceRS A reference signal (e.g. a CSI-RS config or a SS block) to be used for SRS path loss estimation (see TS 38.213 [13], clause 7.3).

resourceType Time domain behavior of SRS resource configuration, see TS 38.214 [19], clause 6.2.1. The network configures SRS resources in the same resource set with the same time domain behavior on periodic, aperiodic and semi-persistent SRS.

slotOffset An offset in number of slots between the triggering DCI and the actual transmission of this SRS-ResourceSet. If the field is absent the UE applies no offset (value 0).

srs-PowerControlAdjustmentStates Indicates whether hsrs,c(i) = fc(i,1) or hsrs,c(i) = fc(i,2) (if twoPUSCH-PC-AdjustmentStates are configured) or separate close loop is configured for SRS. This parameter is applicable only for Uls on which UE also transmits PUSCH. If absent or release, the UE applies the value sameAs-Fci1 (see TS 38.213 [13], clause 7.3).

srs-ResourceIdList The IDs of the SRS-Resources used in this SRS-ResourceSet. If this SRS-ResourceSet is configured with usage set to codebook, the srs-ResourceIdList contains at most 2 entries. If this SRS-ResourceSet is configured with usage set to nonCodebook, the srs-ResourceIdList contains at most 4 entries.

srs-ResourceSetId The ID of this resource set. It is unique in the context of the BWP in which the parent SRS-Config is defined.

usage Indicates if the SRS resource set is used for beam management, codebook based or non-codebook based transmission or antenna switching. See TS 38.214 [19], clause 6.2.1. Reconfiguration between codebook based and non-codebook based transmission is not supported.

| Conditional Presence | Explanation |
| --- | --- |
| Setup | This field is mandatory present upon configuration of SRS-ResourceSet or SRS-Resource and optionally present, Need M, otherwise. |
| NonCodebook | This field is optionally present, Need M, in case of non-codebook based transmission, otherwise the field is absent. |

#### –	SRS-TPC-CommandConfig

The IE SRS-TPC-CommandConfig is used to configure the UE for extracting TPC commands for SRS from a group-TPC messages on DCI

SRS-TPC-CommandConfig information element

-- ASN1START

-- TAG-SRS-TPC-COMMANDCONFIG-START

SRS-TPC-CommandConfig ::=               SEQUENCE {

startingBitOfFormat2-3                  INTEGER (1..31)                                                     OPTIONAL,   -- Need R

fieldTypeFormat2-3                      INTEGER (0..1)                                                      OPTIONAL,   -- Need R

...,

[[

startingBitOfFormat2-3SUL           INTEGER (1..31)                                                         OPTIONAL    -- Need R

]]

}

-- TAG-SRS-TPC-COMMANDCONFIG-STOP

-- ASN1STOP

SRS-TPC-CommandConfig field descriptions

fieldTypeFormat2-3 The type of a field within the group DCI with SRS request fields (optional), which indicates how many bits in the field are for SRS request (0 or 2). Note that for Type A, there is a common SRS request field for all SCells in the set, but each SCell has its own TPC command bits. See TS 38.212 [17] clause 7.3.1 and , TS 38.213 [13], clause 11.3.

startingBitOfFormat2-3 The starting bit position of a block within the group DCI with SRS request fields (optional) and TPC commands. The value 1 of the field corresponds to the first/left most bit of format2-3. The value 2 of the field corresponds to the second bit format2-3, and so on (see TS 38.212 [17], clause 7.3.1 and TS 38.213 [13], clause 11.3).

startingBitOfFormat2-3SUL The starting bit position of a block within the group DCI with SRS request fields (optional) and TPC commands for SUL carrier (see TS 38.212 [17], clause 7.3.1 and TS 38.213 [13], clause 11.3).

#### –	SSB-Index

The IE SSB-Index identifies an SS-Block within an SS-Burst. See TS 38.213 [13], clause 4.1.

SSB-Index information element

-- ASN1START

-- TAG-SSB-INDEX-START

SSB-Index ::=                       INTEGER (0..maxNrofSSBs-1)

-- TAG-SSB-INDEX-STOP

-- ASN1STOP

#### –	SSB-MTC

The IE SSB-MTC is used to configure measurement timing configurations, i.e., timing occasions at which the UE measures SSBs.

SSB-MTC information element

-- ASN1START

-- TAG-SSB-MTC-START

SSB-MTC ::=                             SEQUENCE {

periodicityAndOffset                    CHOICE {

sf5                                 INTEGER (0..4),

sf10                                    INTEGER (0..9),

sf20                                    INTEGER (0..19),

sf40                                    INTEGER (0..39),

sf80                                    INTEGER (0..79),

sf160                                   INTEGER (0..159)

},

duration                                ENUMERATED { sf1, sf2, sf3, sf4, sf5 }

}

SSB-MTC2 ::=                        SEQUENCE {

pci-List                            SEQUENCE (SIZE (1..maxNrofPCIsPerSMTC)) OF PhysCellId                   OPTIONAL,   -- Need M

periodicity                         ENUMERATED {sf5, sf10, sf20, sf40, sf80, spare3, spare2, spare1}

}

-- TAG-SSB-MTC-STOP

-- ASN1STOP

SSB-MTC field descriptions

duration Duration of the measurement window in which to receive SS/PBCH blocks. It is given in number of subframes (see TS 38.213 [13], clause 4.1).

periodicityAndOffset Periodicity and offset of the measurement window in which to receive SS/PBCH blocks, see 5.5.2.10. Periodicity and offset are given in number of subframes.

SSB-MTC2 field descriptions

pci-List PCIs that are known to follow this SMTC.

#### –	SSB-ToMeasure

The IE SSB-ToMeasure is used to configure a pattern of SSBs.

SSB-ToMeasure information element

-- ASN1START

-- TAG-SSB-TOMEASURE-START

SSB-ToMeasure ::=                   CHOICE {

shortBitmap                         BIT STRING (SIZE (4)),

mediumBitmap                        BIT STRING (SIZE (8)),

longBitmap                          BIT STRING (SIZE (64))

}

-- TAG-SSB-TOMEASURE-STOP

-- ASN1STOP

SSB-ToMeasure field descriptions

longBitmap Bitmap when maximum number of SS/PBCH blocks per half frame equals to 64 as defined in TS 38.213 [13], clause 4.1.

mediumBitmap Bitmap when maximum number of SS/PBCH blocks per half frame equals to 8 as defined in TS 38.213 [13], clause 4.1.

shortBitmap Bitmap when maximum number of SS/PBCH blocks per half frame equals to 4 as defined in TS 38.213 [13], clause 4.1.

#### –	SS-RSSI-Measurement

The IE SS-RSSI-Measurement is used to configure RSSI measurements based on synchronization reference signals.

SS-RSSI-Measurement information element

-- ASN1START

-- TAG-SS-RSSI-MEASUREMENT-START

SS-RSSI-Measurement ::=             SEQUENCE {

measurementSlots                    BIT STRING (SIZE (1..80)),

endSymbol                           INTEGER(0..3)

}

-- TAG-SS-RSSI-MEASUREMENT-STOP

-- ASN1STOP

SS-RSSI-Measurement field descriptions

endSymbol Within a slot that is configured for RSSI measurements (see measurementSlots) the UE measures the RSSI from symbol 0 to symbol endSymbol. This field identifies the entry in Table 5.1.3-1 in TS 38.215 [9], which determines the actual end symbol.

measurementSlots Indicates the slots in which the UE can perform RSSI measurements. The length of the BIT STRING is equal to the number of slots in the configured SMTC window (determined by the duration and by the subcarrierSpacing). The first (left-most / most significant) bit in the bitmap corresponds to the first slot in the SMTC window, the second bit in the bitmap corresponds to the second slot in the SMTC window, and so on. The UE measures in slots for which the corresponding bit in the bitmap is set to 1.

#### –	SubcarrierSpacing

The IE SubcarrierSpacing determines the subcarrier spacing. Restrictions applicable for certain frequencies, channels or signals are clarified in the fields that use this IE.

SubcarrierSpacing information element

-- ASN1START

-- TAG-SUBCARRIERSPACING-START

SubcarrierSpacing ::=               ENUMERATED {kHz15, kHz30, kHz60, kHz120, kHz240, spare3, spare2, spare1}

-- TAG-SUBCARRIERSPACING-STOP

-- ASN1STOP

#### –	TAG-Config

The IE TAG-Config is used to configure parameters for a time-alignment group.

TAG-Config information element

-- ASN1START

-- TAG-TAG-CONFIG-START

TAG-Config ::=                      SEQUENCE {

tag-ToReleaseList                   SEQUENCE (SIZE (1..maxNrofTAGs)) OF TAG-Id                          OPTIONAL,   -- Need N

tag-ToAddModList                    SEQUENCE (SIZE (1..maxNrofTAGs)) OF TAG                             OPTIONAL    -- Need N

}

TAG ::=                             SEQUENCE {

tag-Id                              TAG-Id,

timeAlignmentTimer                  TimeAlignmentTimer,

...

}

TAG-Id ::=                          INTEGER (0..maxNrofTAGs-1)

TimeAlignmentTimer ::=              ENUMERATED {ms500, ms750, ms1280, ms1920, ms2560, ms5120, ms10240, infinity}

-- TAG-TAG-CONFIG-STOP

-- ASN1STOP

TAG field descriptions

tag-Id Indicates the TAG of the SpCell or an SCell, see TS 38.321 [3]. Uniquely identifies the TAG within the scope of a Cell Group (i.e. MCG or SCG).

timeAlignmentTimer Value in ms of the timeAlignmentTimer for TAG with ID tag-Id, as specified in TS 38.321 [3].

#### –	TCI-State

The IE TCI-State associates one or two DL reference signals with a corresponding quasi-colocation (QCL) type.

TCI-State information element

-- ASN1START

-- TAG-TCI-STATE-START

TCI-State ::=                       SEQUENCE {

tci-StateId                         TCI-StateId,

qcl-Type1                           QCL-Info,

qcl-Type2                           QCL-Info                                                    OPTIONAL,   -- Need R

...

}

QCL-Info ::=                        SEQUENCE {

cell                                ServCellIndex                                               OPTIONAL,   -- Need R

bwp-Id                              BWP-Id                                                      OPTIONAL, -- Cond CSI-RS-Indicated

referenceSignal                     CHOICE {

csi-rs                              NZP-CSI-RS-ResourceId,

ssb                                 SSB-Index

},

qcl-Type                            ENUMERATED {typeA, typeB, typeC, typeD},

...

}

-- TAG-TCI-STATE-STOP

-- ASN1STOP

QCL-Info field descriptions

bwp-Id The DL BWP which the RS is located in.

cell The UE's serving cell in which the referenceSignal is configured. If the field is absent, it applies to the serving cell in which the TCI-State is configured. The RS can be located on a serving cell other than the serving cell in which the TCI-State is configured only if the qcl-Type is configured as typeC or typeD. See TS 38.214 [19] clause 5.1.5.

referenceSignal Reference signal with which quasi-collocation information is provided as specified in TS 38.214 [19] subclause 5.1.5.

qcl-Type QCL type as specified in TS 38.214 [19] subclause 5.1.5.

| Conditional Presence | Explanation |
| --- | --- |
| CSI-RS-Indicated | This field is mandatory present if csi-rs is included, absent otherwise |

#### –	TCI-StateId

The IE TCI-StateId is used to identify one TCI-State configuration.

TCI-StateId information element

-- ASN1START

-- TAG-TCI-STATEID-START

TCI-StateId ::=                     INTEGER (0..maxNrofTCI-States-1)

-- TAG-TCI-STATEID-STOP

-- ASN1STOP

#### –	TDD-UL-DL-ConfigCommon

The IE TDD-UL-DL-ConfigCommon determines the cell specific Uplink/Downlink TDD configuration.

TDD-UL-DL-ConfigCommon information element

-- ASN1START

-- TAG-TDD-UL-DL-CONFIGCOMMON-START

TDD-UL-DL-ConfigCommon ::=          SEQUENCE {

referenceSubcarrierSpacing          SubcarrierSpacing,

pattern1                            TDD-UL-DL-Pattern,

pattern2                            TDD-UL-DL-Pattern                                                       OPTIONAL, -- Need R

...

}

TDD-UL-DL-Pattern ::=               SEQUENCE {

dl-UL-TransmissionPeriodicity       ENUMERATED {ms0p5, ms0p625, ms1, ms1p25, ms2, ms2p5, ms5, ms10},

nrofDownlinkSlots                   INTEGER (0..maxNrofSlots),

nrofDownlinkSymbols                 INTEGER (0..maxNrofSymbols-1),

nrofUplinkSlots                     INTEGER (0..maxNrofSlots),

nrofUplinkSymbols                   INTEGER (0..maxNrofSymbols-1),

...,

[[

dl-UL-TransmissionPeriodicity-v1530     ENUMERATED {ms3, ms4}                                               OPTIONAL -- Need R

]]

}

-- TAG-TDD-UL-DL-CONFIGCOMMON-STOP

-- ASN1STOP

TDD-UL-DL-ConfigCommon field descriptions

referenceSubcarrierSpacing Reference SCS used to determine the time domain boundaries in the UL-DL pattern which must be common across all subcarrier specific carriers, i.e., independent of the actual subcarrier spacing using for data transmission. Only the values 15, 30 or 60 kHz (FR1), and 60 or 120 kHz (FR2) are applicable. The network configures a not larger than any SCS of configured BWPs for the serving cell. See TS 38.213 [13], clause 11.1.

TDD-UL-DL-Pattern field descriptions

dl-UL-TransmissionPeriodicity Periodicity of the DL-UL pattern, see TS 38.213 [13], clause 11.1. If the dl-UL-TransmissionPeriodicity-v1530 is signalled, UE shall ignore the dl-UL-TransmissionPeriodicity (without suffix).

nrofDownlinkSlots Number of consecutive full DL slots at the beginning of each DL-UL pattern, see TS 38.213 [13], clause 11.1. In this release, the maximum value for this field is 80.

nrofDownlinkSymbols Number of consecutive DL symbols in the beginning of the slot following the last full DL slot (as derived from nrofDownlinkSlots). The value 0 indicates that there is no partial-downlink slot. (see TS 38.213 [13], clause 11.1).

nrofUplinkSlots Number of consecutive full UL slots at the end of each DL-UL pattern, see TS 38.213 [13], clause 11.1. In this release, the maximum value for this field is 80.

nrofUplinkSymbols Number of consecutive UL symbols in the end of the slot preceding the first full UL slot (as derived from nrofUplinkSlots). The value 0 indicates that there is no partial-uplink slot. (see TS 38.213 [13], clause 11.1).

#### –	TDD-UL-DL-ConfigDedicated

The IE TDD-UL-DL-ConfigDedicated determines the UE-specific Uplink/Downlink TDD configuration.

TDD-UL-DL-ConfigDedicated information element

-- ASN1START

-- TAG-TDD-UL-DL-CONFIGDEDICATED-START

TDD-UL-DL-ConfigDedicated ::=       SEQUENCE {

slotSpecificConfigurationsToAddModList      SEQUENCE (SIZE (1..maxNrofSlots)) OF TDD-UL-DL-SlotConfig       OPTIONAL, -- Need N

slotSpecificConfigurationsToReleaseList     SEQUENCE (SIZE (1..maxNrofSlots)) OF TDD-UL-DL-SlotIndex        OPTIONAL, -- Need N

...

}

TDD-UL-DL-SlotConfig ::=            SEQUENCE {

slotIndex                           TDD-UL-DL-SlotIndex,

symbols                             CHOICE {

allDownlink                         NULL,

allUplink                           NULL,

explicit                            SEQUENCE {

nrofDownlinkSymbols                 INTEGER (1..maxNrofSymbols-1)                                   OPTIONAL, -- Need S

nrofUplinkSymbols                   INTEGER (1..maxNrofSymbols-1)                                   OPTIONAL  -- Need S

}

}

}

TDD-UL-DL-SlotIndex ::=             INTEGER (0..maxNrofSlots-1)

-- TAG-TDD-UL-DL-CONFIGDEDICATED-STOP

-- ASN1STOP

TDD-UL-DL-ConfigDedicated field descriptions

slotSpecificConfigurationsToAddModList The slotSpecificConfigurationToAddModList allows overriding UL/DL allocations provided in tdd-UL-DL-configurationCommon, see TS 38.213 [13], clause 11.1.

TDD-UL-DL-SlotConfig field descriptions

nrofDownlinkSymbols Number of consecutive DL symbols in the beginning of the slot identified by slotIndex. If the field is absent the UE assumes that there are no leading DL symbols. (see TS 38.213 [13], clause 11.1).

nrofUplinkSymbols Number of consecutive UL symbols in the end of the slot identified by slotIndex. If the field is absent the UE assumes that there are no trailing UL symbols. (see TS 38.213 [13], clause 11.1).

slotIndex Identifies a slot within a slot configuration period given in tdd-UL-DL-configurationCommon, see TS 38.213 [13], clause 11.1.

symbols The direction (downlink or uplink) for the symbols in this slot. Value allDownlink indicates that all symbols in this slot are used for downlink; value allUplink indicates that all symbols in this slot are used for uplink; value explicit indicates explicitly how many symbols in the beginning and end of this slot are allocated to downlink and uplink, respectively.

#### –	TrackingAreaCode

The IE TrackingAreaCode is used to identify a tracking area within the scope of a PLMN, see TS 24.501 [23].

TrackingAreaCode information element

-- ASN1START

-- TAG-TRACKINGAREACODE-START

TrackingAreaCode ::= BIT STRING (SIZE (24))

-- TAG-TRACKINGAREACODE-STOP

-- ASN1STOP

#### –	T-Reselection

The IE T-Reselection concerns the cell reselection timer TreselectionRAT for NR and E-UTRA Value in seconds. For value 0, behaviour as specified in 7.1.2 applies.

T-Reselectioninformation element

-- ASN1START

-- TAG-TRESELECTION-START

T-Reselection ::=                   INTEGER (0..7)

-- TAG-TRESELECTION-STOP

-- ASN1STOP

#### –	TimeToTrigger

The IE TimeToTrigger specifies the value range used for time to trigger parameter, which concerns the time during which specific criteria for the event needs to be met in order to trigger a measurement report. Value ms0 corresponds to 0 ms and behaviour as specified in 7.1.2 applies, value ms40 corresponds to 40 ms, and so on.

TimeToTrigger information element

-- ASN1START

-- TAG-TIMETOTRIGGER-START

TimeToTrigger ::=                   ENUMERATED {

ms0, ms40, ms64, ms80, ms100, ms128, ms160, ms256,

ms320, ms480, ms512, ms640, ms1024, ms1280, ms2560,

ms5120}

-- TAG-TIMETOTRIGGER-STOP

-- ASN1STOP

#### –	UAC-BarringInfoSetIndex

The IE UAC-BarringInfoSetIndex provides the index of the entry in uac-BarringInfoSetList. Value 1 corresponds to the first entry in uac-BarringInfoSetList, value 2 corresponds to the second entry in this list and so on. An index value referring to an entry not included in uac-BarringInfoSetList indicates no barring.

UAC-BarringInfoSetIndex information element

-- ASN1START

-- TAG-UAC-BARRINGINFOSETINDEX-START

UAC-BarringInfoSetIndex ::=                INTEGER (1..maxBarringInfoSet)

-- TAG-UAC-BARRINGINFOSETINDEX-STOP

-- ASN1STOP

#### –	UAC-BarringInfoSetList

The IE UAC-BarringInfoSetList provides a list of access control parameter sets. An access category can be configured with access parameters according to one of the sets.

UAC-BarringInfoSetList information element

-- ASN1START

-- TAG-UAC-BARRINGINFOSETLIST-START

UAC-BarringInfoSetList ::=          SEQUENCE (SIZE(1..maxBarringInfoSet)) OF UAC-BarringInfoSet

UAC-BarringInfoSet ::=              SEQUENCE {

uac-BarringFactor                   ENUMERATED {p00, p05, p10, p15, p20, p25, p30, p40,

p50, p60, p70, p75, p80, p85, p90, p95},

uac-BarringTime                     ENUMERATED {s4, s8, s16, s32, s64, s128, s256, s512},

uac-BarringForAccessIdentity        BIT STRING (SIZE(7))

}

-- TAG-UAC-BARRINGINFOSETLIST-STOP

-- ASN1STOP

UAC-BarringInfoSetList field descriptions

uac-BarringInfoSetList List of access control parameter sets. Each access category can be configured with access parameters corresponding to a particular set by uac-barringInfoSetIndex. Association of an access category with an index that has no corresponding entry in the uac-BarringInfoSetList is valid configuration and indicates no barring.

uac-BarringForAccessIdentity Indicates whether access attempt is allowed for each Access Identity. The leftmost bit, bit 0 in the bit string corresponds to Access Identity 1, bit 1 in the bit string corresponds to Access Identity 2, bit 2 in the bit string corresponds to Access Identity 11, bit 3 in the bit string corresponds to Access Identity 12, bit 4 in the bit string corresponds to Access Identity 13, bit 5 in the bit string corresponds to Access Identity 14, and bit 6 in the bit string corresponds to Access Identity 15. Value 0 means that access attempt is allowed for the corresponding access identity.

uac-BarringFactor Represents the probability that access attempt would be allowed during access barring check.

uac-BarringTime The minimum time in seconds before a new access attempt is to be performed after an access attempt was barred at access barring check for the same access category.

#### –	UAC-BarringPerCatList

The IE UAC-BarringPerCatList provides access control parameters for a list of access categories.

UAC-BarringPerCatList information element

-- ASN1START

-- TAG-UAC-BARRINGPERCATLIST-START

UAC-BarringPerCatList ::=           SEQUENCE (SIZE (1..maxAccessCat-1)) OF UAC-BarringPerCat

UAC-BarringPerCat ::=               SEQUENCE {

accessCategory                       INTEGER (1..maxAccessCat-1),

uac-barringInfoSetIndex              UAC-BarringInfoSetIndex

}

-- TAG-UAC-BARRINGPERCATLIST-STOP

-- ASN1STOP

UAC-BarringPerCatList field descriptions

accessCategory The Access Category according to TS 22.261 [25].

#### –	UAC-BarringPerPLMN-List

The IE UAC-BarringPerPLMN-List provides access category specific access control parameters, which are configured per PLMN.

UAC-BarringPerPLMN-List information element

-- ASN1START

-- TAG-UAC-BARRINGPERPLMN-LIST-START

UAC-BarringPerPLMN-List ::=         SEQUENCE (SIZE (1.. maxPLMN)) OF UAC-BarringPerPLMN

UAC-BarringPerPLMN ::=              SEQUENCE {

plmn-IdentityIndex                  INTEGER (1..maxPLMN),

uac-ACBarringListType               CHOICE{

uac-ImplicitACBarringList           SEQUENCE (SIZE(maxAccessCat-1)) OF UAC-BarringInfoSetIndex,

uac-ExplicitACBarringList           UAC-BarringPerCatList

}                                                                                                     OPTIONAL     -- Need S

}

-- TAG-UAC-BARRINGPERPLMN-LIST-STOP

-- ASN1STOP

UAC-BarringPerPLMN-List field descriptions

uac-ACBarringListType Access control parameters for each access category valid only for a specific PLMN. UE behaviour upon absence of this field is specified in clause 5.3.14.2.

plmn-IdentityIndex Index of the PLMN across the plmn-IdentityList fields included in SIB1.

#### –	UE-TimersAndConstants

The IE UE-TimersAndConstants contains timers and constants used by the UE in RRC_CONNECTED, RRC_INACTIVE and RRC_IDLE.

UE-TimersAndConstants information element

-- ASN1START

-- TAG-UE-TIMERSANDCONSTANTS-START

UE-TimersAndConstants ::=           SEQUENCE {

t300                                ENUMERATED {ms100, ms200, ms300, ms400, ms600, ms1000, ms1500, ms2000},

t301                                ENUMERATED {ms100, ms200, ms300, ms400, ms600, ms1000, ms1500, ms2000},

t310                                ENUMERATED {ms0, ms50, ms100, ms200, ms500, ms1000, ms2000},

n310                                ENUMERATED {n1, n2, n3, n4, n6, n8, n10, n20},

t311                                ENUMERATED {ms1000, ms3000, ms5000, ms10000, ms15000, ms20000, ms30000},

n311                                ENUMERATED {n1, n2, n3, n4, n5, n6, n8, n10},

t319                                ENUMERATED {ms100, ms200, ms300, ms400, ms600, ms1000, ms1500, ms2000},

...

}

-- TAG-UE-TIMERSANDCONSTANTS-STOP

-- ASN1STOP

#### –	UplinkConfigCommon

The IE UplinkConfigCommon provides common uplink parameters of a cell.

UplinkConfigCommon information element

-- ASN1START

-- TAG-UPLINKCONFIGCOMMON-START

UplinkConfigCommon ::=              SEQUENCE {

frequencyInfoUL                     FrequencyInfoUL                                 OPTIONAL,   -- Cond InterFreqHOAndServCellAdd

initialUplinkBWP                    BWP-UplinkCommon                                OPTIONAL,   -- Cond ServCellAdd

dummy                               TimeAlignmentTimer

}

-- TAG-UPLINKCONFIGCOMMON-STOP

-- ASN1STOP

UplinkConfigCommon field descriptions

frequencyInfoUL Absolute uplink frequency configuration and subcarrier specific virtual carriers.

initialUplinkBWP The initial uplink BWP configuration for a serving cell (see TS 38.213 [13], clause 12).

| Conditional Presence | Explanation |
| --- | --- |
| InterFreqHOAndServCellAdd | This field is mandatory present for inter-frequency handover and upon serving cell (PSCell/SCell) addition. Otherwise, the field is optionally present, Need M. |
| ServCellAdd | This field is mandatory present upon serving cell addition (for PSCell and SCell) and upon handover from E-UTRA to NR. It is optionally present, Need M otherwise. |

#### –	UplinkConfigCommonSIB

The IE UplinkConfigCommonSIB provides common uplink parameters of a cell.

UplinkConfigCommonSIB information element

-- ASN1START

-- TAG-UPLINKCONFIGCOMMONSIB-START

UplinkConfigCommonSIB ::=               SEQUENCE {

frequencyInfoUL                         FrequencyInfoUL-SIB,

initialUplinkBWP                        BWP-UplinkCommon,

timeAlignmentTimerCommon                TimeAlignmentTimer

}

-- TAG-UPLINKCONFIGCOMMONSIB-STOP

-- ASN1STOP

UplinkConfigCommonSIB field descriptions

frequencyInfoUL Absolute uplink frequency configuration and subcarrier specific virtual carriers.

InitialUplinkBWP The initial uplink BWP configuration for a PCell (see TS 38.213 [13], clause 12).

#### –	UplinkTxDirectCurrentList

The IE UplinkTxDirectCurrentList indicates the Tx Direct Current locations per serving cell for each configured UL BWP in the serving cell, based on the BWP numerology and the associated carrier bandwidth.

UplinkTxDirectCurrentList information element

-- ASN1START

-- TAG-UPLINKTXDIRECTCURRENTLIST-START

UplinkTxDirectCurrentList ::=           SEQUENCE (SIZE (1..maxNrofServingCells)) OF UplinkTxDirectCurrentCell

UplinkTxDirectCurrentCell ::=           SEQUENCE {

servCellIndex                           ServCellIndex,

uplinkDirectCurrentBWP                  SEQUENCE (SIZE (1..maxNrofBWPs)) OF UplinkTxDirectCurrentBWP,

...,

[[

uplinkDirectCurrentBWP-SUL              SEQUENCE (SIZE (1..maxNrofBWPs)) OF UplinkTxDirectCurrentBWP               OPTIONAL

]]

}

UplinkTxDirectCurrentBWP ::=            SEQUENCE {

bwp-Id                                  BWP-Id,

shift7dot5kHz                           BOOLEAN,

txDirectCurrentLocation                 INTEGER (0..3301)

}

-- TAG-UPLINKTXDIRECTCURRENTLIST-STOP

-- ASN1STOP

UplinkTxDirectCurrentBWP field descriptions

bwp-Id The BWP-Id of the corresponding uplink BWP.

shift7dot5kHz Indicates whether there is 7.5 kHz shift or not. 7.5 kHz shift is applied if the field is set to true. Otherwise 7.5 kHz shift is not applied.

txDirectCurrentLocation The uplink Tx Direct Current location for the carrier. Only values in the value range of this field between 0 and 3299, which indicate the subcarrier index within the carrier corresponding to the numerology of the corresponding uplink BWP and value 3300, which indicates "Outside the carrier" and value 3301, which indicates "Undetermined position within the carrier" are used in this version of the specification.

UplinkTxDirectCurrentCell field descriptions

servCellIndex The serving cell ID of the serving cell corresponding to the uplinkDirectCurrentBWP.

uplinkDirectCurrentBWP The Tx Direct Current locations for all the uplink BWPs configured at the corresponding serving cell.

uplinkDirectCurrentBWP-SUL The Tx Direct Current locations for all the supplementary uplink BWPs configured at the corresponding serving cell.

#### –	ZP-CSI-RS-Resource

The IE ZP-CSI-RS-Resource is used to configure a Zero-Power (ZP) CSI-RS resource (see TS 38.214 [19], clause 5.1.4.2). Reconfiguration of a ZP-CSI-RS-Resource between periodic or semi-persistent and aperiodic is not supported.

ZP-CSI-RS-Resource information element

-- ASN1START

-- TAG-ZP-CSI-RS-RESOURCE-START

ZP-CSI-RS-Resource ::=              SEQUENCE {

zp-CSI-RS-ResourceId                ZP-CSI-RS-ResourceId,

resourceMapping                     CSI-RS-ResourceMapping,

periodicityAndOffset                CSI-ResourcePeriodicityAndOffset                OPTIONAL, --Cond PeriodicOrSemiPersistent

...

}

ZP-CSI-RS-ResourceId ::=            INTEGER (0..maxNrofZP-CSI-RS-Resources-1)

-- TAG-ZP-CSI-RS-RESOURCE-STOP

-- ASN1STOP

ZP-CSI-RS-Resource field descriptions

periodicityAndOffset Periodicity and slot offset for periodic/semi-persistent ZP-CSI-RS (see TS 38.214 [19], clause 5.1.4.2). Network always configures the UE with a value for this field for periodic and semi-persistent ZP-CSI-RS resource (as indicated in PDSCH-Config).

resourceMapping OFDM symbol and subcarrier occupancy of the ZP-CSI-RS resource within a slot.

zp-CSI-RS-ResourceId ZP CSI-RS resource configuration ID (see TS 38.214 [19], clause 5.1.4.2).

| Conditional Presence | Explanation |
| --- | --- |
| PeriodicOrSemiPersistent | The field is optionally present, Need M, for periodic and semi-persistent ZP-CSI-RS-Resources (as indicated in PDSCH-Config). The field is absent otherwise. |

#### –	ZP-CSI-RS-ResourceSet

The IE ZP-CSI-RS-ResourceSet refers to a set of ZP-CSI-RS-Resources using their ZP-CSI-RS-ResourceIds.

ZP-CSI-RS-ResourceSet information element

-- ASN1START

-- TAG-ZP-CSI-RS-RESOURCESET-START

ZP-CSI-RS-ResourceSet ::=           SEQUENCE {

zp-CSI-RS-ResourceSetId             ZP-CSI-RS-ResourceSetId,

zp-CSI-RS-ResourceIdList            SEQUENCE (SIZE(1..maxNrofZP-CSI-RS-ResourcesPerSet)) OF ZP-CSI-RS-ResourceId,

...

}

-- TAG-ZP-CSI-RS-RESOURCESET-STOP

-- ASN1STOP

ZP-CSI-RS-ResourceSet field descriptions

zp-CSI-RS-ResourceIdList The list of ZP-CSI-RS-ResourceId identifying the ZP-CSI-RS-Resource elements belonging to this set.

#### –	ZP-CSI-RS-ResourceSetId

The IE ZP-CSI-RS-ResourceSetId identifies a ZP-CSI-RS-ResourceSet.

ZP-CSI-RS-ResourceSetId information element

-- ASN1START

-- TAG-ZP-CSI-RS-RESOURCESETID-START

ZP-CSI-RS-ResourceSetId ::=                     INTEGER (0..maxNrofZP-CSI-RS-ResourceSets-1)

-- TAG-ZP-CSI-RS-RESOURCESETID-STOP

-- ASN1STOP

### 6.3.3	UE capability information elements

#### –	AccessStratumRelease

The IE AccessStratumRelease indicates the release supported by the UE.

AccessStratumRelease information element

-- ASN1START

-- TAG-ACCESSSTRATUMRELEASE-START

AccessStratumRelease ::= ENUMERATED {

rel15, spare7, spare6, spare5, spare4, spare3, spare2, spare1, ... }

-- TAG-ACCESSSTRATUMRELEASE-STOP

-- ASN1STOP

#### –	BandCombinationList

The IE BandCombinationList contains a list of NR CA and/or MR-DC band combinations (also including DL only or UL only band).

BandCombinationList information element

-- ASN1START

-- TAG-BANDCOMBINATIONLIST-START

BandCombinationList ::=             SEQUENCE (SIZE (1..maxBandComb)) OF BandCombination

BandCombinationList-v1540 ::=       SEQUENCE (SIZE (1..maxBandComb)) OF BandCombination-v1540

BandCombinationList-v1550 ::=       SEQUENCE (SIZE (1..maxBandComb)) OF BandCombination-v1550

BandCombinationList-v1560 ::=       SEQUENCE (SIZE (1..maxBandComb)) OF BandCombination-v1560

BandCombinationList-v1570 ::=       SEQUENCE (SIZE (1..maxBandComb)) OF BandCombination-v1570

BandCombinationList-v1580 ::=       SEQUENCE (SIZE (1..maxBandComb)) OF BandCombination-v1580

BandCombinationList-v1590 ::=       SEQUENCE (SIZE (1..maxBandComb)) OF BandCombination-v1590

BandCombination ::=                 SEQUENCE {

bandList                            SEQUENCE (SIZE (1..maxSimultaneousBands)) OF BandParameters,

featureSetCombination               FeatureSetCombinationId,

ca-ParametersEUTRA                  CA-ParametersEUTRA                          OPTIONAL,

ca-ParametersNR                     CA-ParametersNR                             OPTIONAL,

mrdc-Parameters                     MRDC-Parameters                             OPTIONAL,

supportedBandwidthCombinationSet    BIT STRING (SIZE (1..32))                   OPTIONAL,

powerClass-v1530                    ENUMERATED {pc2}                            OPTIONAL

}

BandCombination-v1540::=            SEQUENCE {

bandList-v1540                      SEQUENCE (SIZE (1..maxSimultaneousBands)) OF BandParameters-v1540,

ca-ParametersNR-v1540               CA-ParametersNR-v1540                       OPTIONAL

}

BandCombination-v1550 ::=           SEQUENCE {

ca-ParametersNR-v1550               CA-ParametersNR-v1550

}

BandCombination-v1560::=            SEQUENCE {

ne-DC-BC                                ENUMERATED {supported}                 OPTIONAL,

ca-ParametersNRDC                       CA-ParametersNRDC                      OPTIONAL,

ca-ParametersEUTRA-v1560                CA-ParametersEUTRA-v1560               OPTIONAL,

ca-ParametersNR-v1560                   CA-ParametersNR-v1560                  OPTIONAL

}

BandCombination-v1570 ::=           SEQUENCE {

ca-ParametersEUTRA-v1570            CA-ParametersEUTRA-v1570

}

BandCombination-v1580 ::=           SEQUENCE {

mrdc-Parameters-v1580               MRDC-Parameters-v1580

}

BandCombination-v1590::=            SEQUENCE {

supportedBandwidthCombinationSetIntraENDC  BIT STRING (SIZE (1..32))           OPTIONAL,

mrdc-Parameters-v1590                      MRDC-Parameters-v1590

}

BandParameters ::=                      CHOICE {

eutra                               SEQUENCE {

bandEUTRA                           FreqBandIndicatorEUTRA,

ca-BandwidthClassDL-EUTRA           CA-BandwidthClassEUTRA                 OPTIONAL,

ca-BandwidthClassUL-EUTRA           CA-BandwidthClassEUTRA                 OPTIONAL

},

nr                                  SEQUENCE {

bandNR                              FreqBandIndicatorNR,

ca-BandwidthClassDL-NR              CA-BandwidthClassNR                    OPTIONAL,

ca-BandwidthClassUL-NR              CA-BandwidthClassNR                    OPTIONAL

}

}

BandParameters-v1540 ::=            SEQUENCE {

srs-CarrierSwitch                   CHOICE {

nr                                  SEQUENCE {

srs-SwitchingTimesListNR            SEQUENCE (SIZE (1..maxSimultaneousBands)) OF SRS-SwitchingTimeNR

},

eutra                               SEQUENCE {

srs-SwitchingTimesListEUTRA         SEQUENCE (SIZE (1..maxSimultaneousBands)) OF SRS-SwitchingTimeEUTRA

}

}                                                                              OPTIONAL,

srs-TxSwitch                    SEQUENCE {

supportedSRS-TxPortSwitch       ENUMERATED {t1r2, t1r4, t2r4, t1r4-t2r4, t1r1, t2r2, t4r4, notSupported},

txSwitchImpactToRx              INTEGER (1..32)                            OPTIONAL,

txSwitchWithAnotherBand         INTEGER (1..32)                            OPTIONAL

}                                                                              OPTIONAL

}

-- TAG-BANDCOMBINATIONLIST-STOP

-- ASN1STOP

BandCombination field descriptions

BandCombinationList-v1540, BandCombinationList-v1550, BandCombinationList-v1560, BandCombinationList-v1570, BandCombinationList-v1580, BandCombinationList-v1590 The UE shall include the same number of entries, and listed in the same order, as in BandCombinationList (without suffix). If the field is included in supportedBandCombinationListNEDC-Only-v15a0, the UE shall include the same number of entries, and listed in the same order, as in BandCombinationList (without suffix) of supportedBandCombinationListNEDC-Only (without suffix) field.

ca-ParametersNRDC If the field is included for a band combination in the NR capability container, the field indicates support of NR-DC. Otherwise, the field is absent.

ne-DC-BC If the field is included for a band combination in the MR-DC capability container, the field indicates support of NE-DC. Otherwise, the field is absent.

srs-SwitchingTimesListNR Indicates, for a particular pair of NR bands, the RF retuning time when switching between a NR carrier corresponding to this band entry and another (PUSCH-less) NR carrier corresponding to the band entry in the order indicated below: - For the first NR band, the UE shall include the same number of entries for NR bands as in bandList, i.e. first entry corresponds to first NR band in bandList and so on, - For the second NR band, the UE shall include one entry less, i.e. first entry corresponds to the second NR band in bandList and so on - And so on

srs-SwitchingTimesListEUTRA Indicates, for a particular pair of E-UTRA bands, the RF retuning time when switching between an E-UTRA carrier corresponding to this band entry and another (PUSCH-less) E-UTRA carrier corresponding to the band entry in the order indicated below: - For the first E-UTRA band, the UE shall include the same number of entries for E-UTRA bands as in bandList, i.e. first entry corresponds to first E-UTRA band in bandList and so on, - For the second E-UTRA band, the UE shall include one entry less, i.e. first entry corresponds to the second E-UTRA band in bandList and so on - And so on

srs-TxSwitch Indicates supported SRS antenna switch capability for the associated band. If the UE indicates support of SRS-SwitchingTimeNR, the UE is allowed to set this field for a band with associated FeatureSetUplinkId set to 0 for SRS carrier switching.

#### –	CA-BandwidthClassEUTRA

The IE CA-BandwidthClassEUTRA indicates the E-UTRA CA bandwidth class as defined in TS 36.101 [22], table 5.6A-1.

CA-BandwidthClassEUTRA information element

-- ASN1START

-- TAG-CA-BANDWIDTHCLASSEUTRA-START

CA-BandwidthClassEUTRA ::=          ENUMERATED {a, b, c, d, e, f, ...}

-- TAG-CA-BANDWIDTHCLASSEUTRA-STOP

-- ASN1STOP

#### –	CA-BandwidthClassNR

The IE CA-BandwidthClassNR indicates the NR CA bandwidth class as defined in TS 38.101-1 [15], table 5.3A.5-1 and TS 38.101-2 [39], table 5.3A.4-1.

CA-BandwidthClassNR information element

-- ASN1START

-- TAG-CA-BANDWIDTHCLASSNR-START

CA-BandwidthClassNR ::=             ENUMERATED {a, b, c, d, e, f, g, h, i, j, k, l, m, n, o, p, q, ...}

-- TAG-CA-BANDWIDTHCLASSNR-STOP

-- ASN1STOP

#### –	CA-ParametersEUTRA

The IE CA-ParametersEUTRA contains the E-UTRA part of band combination parameters for a given MR-DC band combination.

NOTE:	If additional E-UTRA band combination parameters are defined in TS 36.331 [10], which are supported for MR-DC, they will be defined here as well.

CA-ParametersEUTRA information element

-- ASN1START

-- TAG-CA-PARAMETERSEUTRA-START

CA-ParametersEUTRA ::=                          SEQUENCE {

multipleTimingAdvance                           ENUMERATED {supported}                          OPTIONAL,

simultaneousRx-Tx                               ENUMERATED {supported}                          OPTIONAL,

supportedNAICS-2CRS-AP                          BIT STRING (SIZE (1..8))                        OPTIONAL,

additionalRx-Tx-PerformanceReq                  ENUMERATED {supported}                          OPTIONAL,

ue-CA-PowerClass-N                              ENUMERATED {class2}                             OPTIONAL,

supportedBandwidthCombinationSetEUTRA-v1530     BIT STRING (SIZE (1..32))                       OPTIONAL,

...

}

CA-ParametersEUTRA-v1560 ::=                    SEQUENCE {

fd-MIMO-TotalWeightedLayers                     INTEGER (2..128)                                OPTIONAL

}

CA-ParametersEUTRA-v1570 ::=                    SEQUENCE {

dl-1024QAM-TotalWeightedLayers                  INTEGER (0..10)                                 OPTIONAL

}

-- TAG-CA-PARAMETERSEUTRA-STOP

-- ASN1STOP

#### –	CA-ParametersNR

The IE CA-ParametersNR contains carrier aggregation related capabilities that are defined per band combination.

CA-ParametersNR information element

-- ASN1START

-- TAG-CA-PARAMETERSNR-START

CA-ParametersNR ::=                 SEQUENCE {

dummy                                         ENUMERATED {supported}      OPTIONAL,

parallelTxSRS-PUCCH-PUSCH                     ENUMERATED {supported}      OPTIONAL,

parallelTxPRACH-SRS-PUCCH-PUSCH               ENUMERATED {supported}      OPTIONAL,

simultaneousRxTxInterBandCA                   ENUMERATED {supported}      OPTIONAL,

simultaneousRxTxSUL                           ENUMERATED {supported}      OPTIONAL,

diffNumerologyAcrossPUCCH-Group               ENUMERATED {supported}      OPTIONAL,

diffNumerologyWithinPUCCH-GroupSmallerSCS     ENUMERATED {supported}      OPTIONAL,

supportedNumberTAG                            ENUMERATED {n2, n3, n4}     OPTIONAL,

...

}

CA-ParametersNR-v1540 ::=           SEQUENCE {

simultaneousSRS-AssocCSI-RS-AllCC                       INTEGER (5..32)         OPTIONAL,

csi-RS-IM-ReceptionForFeedbackPerBandComb               SEQUENCE {

maxNumberSimultaneousNZP-CSI-RS-ActBWP-AllCC            INTEGER (1..64)         OPTIONAL,

totalNumberPortsSimultaneousNZP-CSI-RS-ActBWP-AllCC     INTEGER (2..256)        OPTIONAL

}                                                                               OPTIONAL,

simultaneousCSI-ReportsAllCC                            INTEGER (5..32)         OPTIONAL,

dualPA-Architecture                                     ENUMERATED {supported}  OPTIONAL

}

CA-ParametersNR-v1550 ::=           SEQUENCE {

dummy                               ENUMERATED {supported}      OPTIONAL

}

CA-ParametersNR-v1560 ::=           SEQUENCE {

diffNumerologyWithinPUCCH-GroupLargerSCS      ENUMERATED {supported}            OPTIONAL

}

-- TAG-CA-PARAMETERSNR-STOP

-- ASN1STOP

#### –	CA-ParametersNRDC

The IE CA-ParametersNRDC contains dual connectivity related capabilities that are defined per band combination.

CA-ParametersNRDC information element

-- ASN1START

-- TAG-CA-PARAMETERS-NRDC-START

CA-ParametersNRDC ::=					SEQUENCE {

ca-ParametersNR-ForDC				    CA-ParametersNR							OPTIONAL,

ca-ParametersNR-ForDC-v1540			    CA-ParametersNR-v1540					OPTIONAL,

ca-ParametersNR-ForDC-v1550			    CA-ParametersNR-v1550					OPTIONAL,

ca-ParametersNR-ForDC-v1560			    CA-ParametersNR-v1560					OPTIONAL,

featureSetCombinationDC				    FeatureSetCombinationId					OPTIONAL

}

-- TAG-CA-PARAMETERS-NRDC-STOP

-- ASN1STOP

CA-ParametersNRDC field descriptions

ca-ParametersNR-forDC (with and without suffix) If this field is present for a band combination, it reports the UE capabilities when NR-DC is configured with the band combination. If no version of this field (i.e., with and without suffix) is present for a band combination, the ca-ParametersNR field versions (with and without suffix) in BandCombination are applicable to the UE configured with NR-DC for the band combination.

featureSetCombinationDC If this field is present for a band combination, it reports the feature set combination supported for the band combination when NR-DC is configured. If this field is absent for a band combination, the featureSetCombination in BandCombination (without suffix) is applicable to the UE configured with NR-DC for the band combination.

#### –	CarrierAggregationVariant

The IE CarrierAggregationVariant informs the network about supported “placement” of the SpCell in an NR cell group.

CarrierAggregationVariant information element

-- ASN1START

-- TAG-CARRIERAGGREGATIONVARIANT-START

CarrierAggregationVariant ::=           SEQUENCE {

fr1fdd-FR1TDD-CA-SpCellOnFR1FDD         ENUMERATED {supported}                      OPTIONAL,

fr1fdd-FR1TDD-CA-SpCellOnFR1TDD         ENUMERATED {supported}                      OPTIONAL,

fr1fdd-FR2TDD-CA-SpCellOnFR1FDD         ENUMERATED {supported}                      OPTIONAL,

fr1fdd-FR2TDD-CA-SpCellOnFR2TDD         ENUMERATED {supported}                      OPTIONAL,

fr1tdd-FR2TDD-CA-SpCellOnFR1TDD         ENUMERATED {supported}                      OPTIONAL,

fr1tdd-FR2TDD-CA-SpCellOnFR2TDD         ENUMERATED {supported}                      OPTIONAL,

fr1fdd-FR1TDD-FR2TDD-CA-SpCellOnFR1FDD  ENUMERATED {supported}                      OPTIONAL,

fr1fdd-FR1TDD-FR2TDD-CA-SpCellOnFR1TDD  ENUMERATED {supported}                      OPTIONAL,

fr1fdd-FR1TDD-FR2TDD-CA-SpCellOnFR2TDD  ENUMERATED {supported}                      OPTIONAL

}

-- TAG-CARRIERAGGREGATIONVARIANT-STOP

-- ASN1STOP

#### –	CodebookParameters

The IE CodebookParameters is used to convey codebook related parameters.

CodebookParameters information element

-- ASN1START

-- TAG-CODEBOOKPARAMETERS-START

CodebookParameters ::=             SEQUENCE {

type1                                  SEQUENCE {

singlePanel                           SEQUENCE {

supportedCSI-RS-ResourceList      SEQUENCE (SIZE (1.. maxNrofCSI-RS-Resources)) OF SupportedCSI-RS-Resource,

modes                                  ENUMERATED {mode1, mode1andMode2},

maxNumberCSI-RS-PerResourceSet    INTEGER (1..8)

},

multiPanel                            SEQUENCE {

supportedCSI-RS-ResourceList      SEQUENCE (SIZE (1.. maxNrofCSI-RS-Resources)) OF SupportedCSI-RS-Resource,

modes                                  ENUMERATED {mode1, mode2, both},

nrofPanels                            ENUMERATED {n2, n4},

maxNumberCSI-RS-PerResourceSet    INTEGER (1..8)

}                                                                                                               OPTIONAL

},

type2                                  SEQUENCE {

supportedCSI-RS-ResourceList      SEQUENCE (SIZE (1.. maxNrofCSI-RS-Resources)) OF SupportedCSI-RS-Resource,

parameterLx                           INTEGER (2..4),

amplitudeScalingType                ENUMERATED {wideband, widebandAndSubband},

amplitudeSubsetRestriction         ENUMERATED {supported}              OPTIONAL

}                                                                                                                   OPTIONAL,

type2-PortSelection                 SEQUENCE {

supportedCSI-RS-ResourceList      SEQUENCE (SIZE (1.. maxNrofCSI-RS-Resources)) OF SupportedCSI-RS-Resource,

parameterLx                              INTEGER (2..4),

amplitudeScalingType                   ENUMERATED {wideband, widebandAndSubband}

}                                                                                                                   OPTIONAL

}

SupportedCSI-RS-Resource ::=     SEQUENCE {

maxNumberTxPortsPerResource      ENUMERATED {p2, p4, p8, p12, p16, p24, p32},

maxNumberResourcesPerBand        INTEGER (1..64),

totalNumberTxPortsPerBand        INTEGER (2..256)

}

-- TAG-CODEBOOKPARAMETERS-STOP

-- ASN1STOP

#### –	FeatureSetCombination

The IE FeatureSetCombination is a two-dimensional matrix of FeatureSet entries.

Each FeatureSetsPerBand contains a list of feature sets applicable to the carrier(s) of one band entry of the associated band combination. Across the associated bands, the UE shall support the combination of FeatureSets at the same position in the FeatureSetsPerBand. All FeatureSetsPerBand in one FeatureSetCombination must have the same number of entries.

The number of FeatureSetsPerBand in the FeatureSetCombination must be equal to the number of band entries in an associated band combination. The first FeatureSetPerBand applies to the first band entry of the band combination, and so on.

Each FeatureSet contains either a pair of NR or E-UTRA feature set IDs for UL and DL.

In case of NR, the actual feature sets for UL and DL are defined in the FeatureSets IE and referred to from here by their ID, i.e., their position in the featureSetsUplink / featureSetsDownlink list in the FeatureSet IE.

In case of E-UTRA, the feature sets referred to from this list are defined in TS 36.331 [10] and conveyed as part of the UE-EUTRA-Capability container.

The FeatureSetUplink and FeatureSetDownlink referred to from the FeatureSet comprise, among other information, a set of FeatureSetUplinkPerCC-Id:s and FeatureSetDownlinkPerCC-Id:s. The number of these per-CC IDs determines the number of carriers that the UE is able to aggregate contiguously in frequency domain in the corresponding band. The number of carriers supported by the UE is also restricted by the bandwidth class indicated in the associated BandCombination, if present.

In feature set combinations the UE shall exclude entries for fallback combinations with same capabilities, since the network may anyway assume that the UE supports those.

NOTE 1:	The UE may advertise fallback band-combinations in which it supports additional functionality explicitly in two ways: Either by setting FeatureSet IDs to zero (inter-band and intra-band non-contiguous fallback) and by reducing the number of FeatureSet-PerCC Ids in a Feature Set (intra-band contiguous fallback). Or by separate BandCombination entries with associated FeatureSetCombinations.

NOTE 2:	The UE may advertise a FeatureSetCombination containing only fallback band combinations. That means, in a FeatureSetCombination, each group of FeatureSets across the bands may contain at least one pair of FeatureSetUplinkId and FeatureSetDownlinkId which is set to 0/0.

NOTE 3:	The Network configures serving cell(s) and BWP(s) configuration to comply with capabilities derived from the combination of FeatureSets at the same position in the FeatureSetsPerBand, regardless of activated/deactivated serving cell(s) and BWP(s).

FeatureSetCombination information element

-- ASN1START

-- TAG-FEATURESETCOMBINATION-START

FeatureSetCombination ::=       SEQUENCE (SIZE (1..maxSimultaneousBands)) OF FeatureSetsPerBand

FeatureSetsPerBand ::=          SEQUENCE (SIZE (1..maxFeatureSetsPerBand)) OF FeatureSet

FeatureSet ::=                  CHOICE {

eutra                           SEQUENCE {

downlinkSetEUTRA                FeatureSetEUTRA-DownlinkId,

uplinkSetEUTRA                  FeatureSetEUTRA-UplinkId

},

nr                              SEQUENCE {

downlinkSetNR                   FeatureSetDownlinkId,

uplinkSetNR                     FeatureSetUplinkId

}

}

-- TAG-FEATURESETCOMBINATION-STOP

-- ASN1STOP

#### –	FeatureSetCombinationId

The IE FeatureSetCombinationId identifies a FeatureSetCombination. The FeatureSetCombinationId of a FeatureSetCombination is the position of the FeatureSetCombination in the featureSetCombinations list (in UE-NR-Capability or UE-MRDC-Capability). The FeatureSetCombinationId = 0 refers to the first entry in the featureSetCombinations list (in UE-NR-Capability or UE-MRDC-Capability).

NOTE:	The FeatureSetCombinationId = 1024 is not used due to the maximum entry number of featureSetCombinations.

FeatureSetCombinationId information element

-- ASN1START

-- TAG-FEATURESETCOMBINATIONID-START

FeatureSetCombinationId ::=         INTEGER (0.. maxFeatureSetCombinations)

-- TAG-FEATURESETCOMBINATIONID-STOP

-- ASN1STOP

#### –	FeatureSetDownlink

The IE FeatureSetDownlink indicates a set of features that the UE supports on the carriers corresponding to one band entry in a band combination.

FeatureSetDownlink information element

-- ASN1START

-- TAG-FEATURESETDOWNLINK-START

FeatureSetDownlink ::=                  SEQUENCE {

featureSetListPerDownlinkCC             SEQUENCE (SIZE (1..maxNrofServingCells)) OF FeatureSetDownlinkPerCC-Id,

intraBandFreqSeparationDL               FreqSeparationClass                                                     OPTIONAL,

scalingFactor                           ENUMERATED {f0p4, f0p75, f0p8}                                          OPTIONAL,

crossCarrierScheduling-OtherSCS         ENUMERATED {supported}                                                  OPTIONAL,

scellWithoutSSB                         ENUMERATED {supported}                                                  OPTIONAL,

csi-RS-MeasSCellWithoutSSB              ENUMERATED {supported}                                                  OPTIONAL,

dummy1                                  ENUMERATED {supported}                                                  OPTIONAL,

type1-3-CSS                             ENUMERATED {supported}                                                  OPTIONAL,

pdcch-MonitoringAnyOccasions            ENUMERATED {withoutDCI-Gap, withDCI-Gap}                                OPTIONAL,

dummy2                                  ENUMERATED {supported}                                                  OPTIONAL,

ue-SpecificUL-DL-Assignment             ENUMERATED {supported}                                                  OPTIONAL,

searchSpaceSharingCA-DL                 ENUMERATED {supported}                                                  OPTIONAL,

timeDurationForQCL                      SEQUENCE {

scs-60kHz                           ENUMERATED {s7, s14, s28}                                                   OPTIONAL,

scs-120kHz                          ENUMERATED {s14, s28}                                                       OPTIONAL

}                                                                                                               OPTIONAL,

pdsch-ProcessingType1-DifferentTB-PerSlot SEQUENCE {

scs-15kHz                               ENUMERATED {upto2, upto4, upto7}                                        OPTIONAL,

scs-30kHz                               ENUMERATED {upto2, upto4, upto7}                                        OPTIONAL,

scs-60kHz                               ENUMERATED {upto2, upto4, upto7}                                        OPTIONAL,

scs-120kHz                              ENUMERATED {upto2, upto4, upto7}                                        OPTIONAL

}                                                                                                               OPTIONAL,

dummy3                                  DummyA                                                                  OPTIONAL,

dummy4                                  SEQUENCE (SIZE (1.. maxNrofCodebooks)) OF DummyB                        OPTIONAL,

dummy5                                  SEQUENCE (SIZE (1.. maxNrofCodebooks)) OF DummyC                        OPTIONAL,

dummy6                                  SEQUENCE (SIZE (1.. maxNrofCodebooks)) OF DummyD                        OPTIONAL,

dummy7                                  SEQUENCE (SIZE (1.. maxNrofCodebooks)) OF DummyE                        OPTIONAL

}

FeatureSetDownlink-v1540 ::= SEQUENCE {

oneFL-DMRS-TwoAdditionalDMRS-DL         ENUMERATED {supported}                       OPTIONAL,

additionalDMRS-DL-Alt                   ENUMERATED {supported}                       OPTIONAL,

twoFL-DMRS-TwoAdditionalDMRS-DL         ENUMERATED {supported}                       OPTIONAL,

oneFL-DMRS-ThreeAdditionalDMRS-DL       ENUMERATED {supported}                       OPTIONAL,

pdcch-MonitoringAnyOccasionsWithSpanGap SEQUENCE {

scs-15kHz                               ENUMERATED {set1, set2, set3}                OPTIONAL,

scs-30kHz                               ENUMERATED {set1, set2, set3}                OPTIONAL,

scs-60kHz                               ENUMERATED {set1, set2, set3}                OPTIONAL,

scs-120kHz                              ENUMERATED {set1, set2, set3}                OPTIONAL

}                                                                                    OPTIONAL,

pdsch-SeparationWithGap                 ENUMERATED {supported}                       OPTIONAL,

pdsch-ProcessingType2                   SEQUENCE {

scs-15kHz                               ProcessingParameters                         OPTIONAL,

scs-30kHz                               ProcessingParameters                         OPTIONAL,

scs-60kHz                               ProcessingParameters                         OPTIONAL

} OPTIONAL,

pdsch-ProcessingType2-Limited           SEQUENCE {

differentTB-PerSlot-SCS-30kHz           ENUMERATED {upto1, upto2, upto4, upto7}

} OPTIONAL,

dl-MCS-TableAlt-DynamicIndication       ENUMERATED {supported}                       OPTIONAL

}

FeatureSetDownlink-v15a0 ::= SEQUENCE {

supportedSRS-Resources              SRS-Resources                                    OPTIONAL

}

DummyA ::=      SEQUENCE {

maxNumberNZP-CSI-RS-PerCC                   INTEGER (1..32),

maxNumberPortsAcrossNZP-CSI-RS-PerCC        ENUMERATED {p2, p4, p8, p12, p16, p24, p32, p40, p48, p56, p64, p72, p80,

p88, p96, p104, p112, p120, p128, p136, p144, p152, p160, p168,

p176, p184, p192, p200, p208, p216, p224, p232, p240, p248, p256},

maxNumberCS-IM-PerCC                        ENUMERATED {n1, n2, n4, n8, n16, n32},

maxNumberSimultaneousCSI-RS-ActBWP-AllCC    ENUMERATED {n5, n6, n7, n8, n9, n10, n12, n14, n16, n18, n20, n22, n24, n26,

n28, n30, n32, n34, n36, n38, n40, n42, n44, n46, n48, n50, n52,

n54, n56, n58, n60, n62, n64},

totalNumberPortsSimultaneousCSI-RS-ActBWP-AllCC ENUMERATED {p8, p12, p16, p24, p32, p40, p48, p56, p64, p72, p80,

p88, p96, p104, p112, p120, p128, p136, p144, p152, p160, p168,

p176, p184, p192, p200, p208, p216, p224, p232, p240, p248, p256}

}

DummyB ::=       SEQUENCE {

maxNumberTxPortsPerResource         ENUMERATED {p2, p4, p8, p12, p16, p24, p32},

maxNumberResources                  INTEGER (1..64),

totalNumberTxPorts                  INTEGER (2..256),

supportedCodebookMode               ENUMERATED {mode1, mode1AndMode2},

maxNumberCSI-RS-PerResourceSet      INTEGER (1..8)

}

DummyC ::=        SEQUENCE {

maxNumberTxPortsPerResource         ENUMERATED {p8, p16, p32},

maxNumberResources                  INTEGER (1..64),

totalNumberTxPorts                  INTEGER (2..256),

supportedCodebookMode               ENUMERATED {mode1, mode2, both},

supportedNumberPanels               ENUMERATED {n2, n4},

maxNumberCSI-RS-PerResourceSet      INTEGER (1..8)

}

DummyD ::=                 SEQUENCE {

maxNumberTxPortsPerResource         ENUMERATED {p4, p8, p12, p16, p24, p32},

maxNumberResources                  INTEGER (1..64),

totalNumberTxPorts                  INTEGER (2..256),

parameterLx                         INTEGER (2..4),

amplitudeScalingType                ENUMERATED {wideband, widebandAndSubband},

amplitudeSubsetRestriction          ENUMERATED {supported}                          OPTIONAL,

maxNumberCSI-RS-PerResourceSet      INTEGER (1..8)

}

DummyE ::=    SEQUENCE {

maxNumberTxPortsPerResource         ENUMERATED {p4, p8, p12, p16, p24, p32},

maxNumberResources                  INTEGER (1..64),

totalNumberTxPorts                  INTEGER (2..256),

parameterLx                         INTEGER (2..4),

amplitudeScalingType                ENUMERATED {wideband, widebandAndSubband},

maxNumberCSI-RS-PerResourceSet      INTEGER (1..8)

}

-- TAG-FEATURESETDOWNLINK-STOP

-- ASN1STOP

FeatureSetDownlink field descriptions

crossCarrierScheduling-OtherSCS The UE shall set this field to the same value as crossCarrierScheduling-OtherSCS in the associated FeatureSetUplink (if present).

featureSetListPerDownlinkCC Indicates which features the UE supports on the individual DL carriers of the feature set (and hence of a band entry that refer to the feature set). The UE shall hence include at least as many FeatureSetDownlinkPerCC-Id in this list as the number of carriers it supports according to the ca-BandwidthClassDL, except if indicating additional functionality by reducing the number of FeatureSetDownlinkPerCC-Id in the feature set (see NOTE 1 in FeatureSetCombination IE description). The order of the elements in this list is not relevant, i.e., the network may configure any of the carriers in accordance with any of the FeatureSetDownlinkPerCC-Id in this list.

supportedSRS-Resources Indicates supported SRS resources for SRS carrier switching to the band associated with this FeatureSetDownlink. The UE is only allowed to set this field for a band with associated FeatureSetUplinkId set to 0.

#### –	FeatureSetDownlinkId

The IE FeatureSetDownlinkId identifies a downlink feature set. The FeatureSetDownlinkId of a FeatureSetDownlink is the index position of the FeatureSetDownlink in the featureSetsDownlink list in the FeatureSets IE. The first element in that list is referred to by FeatureSetDownlinkId = 1. The FeatureSetDownlinkId=0 is not used by an actual FeatureSetDownlink but means that the UE does not support a carrier in this band of a band combination.

FeatureSetDownlinkId information element

-- ASN1START

-- TAG-FEATURESETDOWNLINKID-START

FeatureSetDownlinkId ::=            INTEGER (0..maxDownlinkFeatureSets)

-- TAG-FEATURESETDOWNLINKID-STOP

-- ASN1STOP

#### –	FeatureSetDownlinkPerCC

The IE FeatureSetDownlinkPerCC indicates a set of features that the UE supports on the corresponding carrier of one band entry of a band combination.

FeatureSetDownlinkPerCC information element

-- ASN1START

-- TAG-FEATURESETDOWNLINKPERCC-START

FeatureSetDownlinkPerCC ::=         SEQUENCE {

supportedSubcarrierSpacingDL        SubcarrierSpacing,

supportedBandwidthDL                SupportedBandwidth,

channelBW-90mhz                     ENUMERATED {supported}                                                  OPTIONAL,

maxNumberMIMO-LayersPDSCH           MIMO-LayersDL                                                           OPTIONAL,

supportedModulationOrderDL          ModulationOrder                                                         OPTIONAL

}

-- TAG-FEATURESETDOWNLINKPERCC-STOP

-- ASN1STOP

#### –	FeatureSetDownlinkPerCC-Id

The IE FeatureSetDownlinkPerCC-Id identifies a set of features applicable to one carrier of a feature set. The FeatureSetDownlinkPerCC-Id of a FeatureSetDownlinkPerCC is the index position of the FeatureSetDownlinkPerCC in the featureSetsDownlinkPerCC. The first element in the list is referred to by FeatureSetDownlinkPerCC-Id = 1, and so on.

FeatureSetDownlinkPerCC-Id information element

-- ASN1START

-- TAG-FEATURESETDOWNLINKPERCC-ID-START

FeatureSetDownlinkPerCC-Id ::=      INTEGER (1..maxPerCC-FeatureSets)

-- TAG-FEATURESETDOWNLINKPERCC-ID-STOP

-- ASN1STOP

#### –	FeatureSetEUTRA-DownlinkId

The IE FeatureSetEUTRA-DownlinkId identifies a downlink feature set in E-UTRA list (see TS 36.331 [10]. The first element in that list is referred to by FeatureSetEUTRA-DownlinkId = 1. The FeatureSetEUTRA-DownlinkId=0 is used when the UE does not support a carrier in this band of a band combination.

FeatureSetEUTRA-DownlinkId information element

-- ASN1START

-- TAG-FEATURESETEUTRADOWNLINKID-START

FeatureSetEUTRA-DownlinkId ::=      INTEGER (0..maxEUTRA-DL-FeatureSets)

-- TAG-FEATURESETEUTRADOWNLINKID-STOP

-- ASN1STOP

#### –	FeatureSetEUTRA-UplinkId

The IE FeatureSetEUTRA-UplinkId identifies an uplink feature set in E-UTRA list (see TS 36.331 [10]. The first element in that list is referred to by FeatureSetEUTRA-UplinkId = 1. The FeatureSetEUTRA-UplinkId =0 is used when the UE does not support a carrier in this band of a band combination.

FeatureSetEUTRA-UplinkId information element

-- ASN1START

-- TAG-FEATURESETEUTRAUPLINKID-START

FeatureSetEUTRA-UplinkId ::=                    INTEGER (0..maxEUTRA-UL-FeatureSets)

-- TAG-FEATURESETEUTRAUPLINKID-STOP

-- ASN1STOP

#### –	FeatureSets

The IE FeatureSets is used to provide pools of downlink and uplink features sets. A FeatureSetCombination refers to the IDs of the feature set(s) that the UE supports in that FeatureSetCombination. The BandCombination entries in the BandCombinationList then indicate the ID of the FeatureSetCombination that the UE supports for that band combination.

The entries in the lists in this IE are identified by their index position. For example, the FeatureSetUplinkPerCC-Id = 4 identifies the 4th element in the featureSetsUplinkPerCC list.

NOTE:	When feature sets (per CC) IEs require extension in future versions of the specification, new versions of the FeatureSetDownlink, FeatureSetUplink, FeatureSets, FeatureSetDownlinkPerCC and/or FeatureSetUplinkPerCC will be created and instantiated in corresponding new lists in the FeatureSets IE. For example, if new capability bits are to be added to the FeatureSetDownlink, they will instead be defined in a new FeatureSetDownlink-rxy which will be instantiated in a new featureSetDownlinkList-rxy list. If a UE indicates in a FeatureSetCombination that it supports the FeatureSetDownlink with ID #5, it implies that it supports both the features in FeatureSetDownlink #5 and FeatureSetDownlink-rxy #5 (if present). The number of entries in the new list(s) shall be the same as in the original list(s).

FeatureSets information element

-- ASN1START

-- TAG-FEATURESETS-START

FeatureSets ::=    SEQUENCE {

featureSetsDownlink                 SEQUENCE (SIZE (1..maxDownlinkFeatureSets)) OF FeatureSetDownlink               OPTIONAL,

featureSetsDownlinkPerCC            SEQUENCE (SIZE (1..maxPerCC-FeatureSets)) OF FeatureSetDownlinkPerCC            OPTIONAL,

featureSetsUplink                   SEQUENCE (SIZE (1..maxUplinkFeatureSets)) OF FeatureSetUplink                   OPTIONAL,

featureSetsUplinkPerCC              SEQUENCE (SIZE (1..maxPerCC-FeatureSets)) OF FeatureSetUplinkPerCC              OPTIONAL,

...,

[[

featureSetsDownlink-v1540           SEQUENCE (SIZE (1..maxDownlinkFeatureSets)) OF FeatureSetDownlink-v1540         OPTIONAL,

featureSetsUplink-v1540             SEQUENCE (SIZE (1..maxUplinkFeatureSets)) OF FeatureSetUplink-v1540             OPTIONAL,

featureSetsUplinkPerCC-v1540        SEQUENCE (SIZE (1..maxPerCC-FeatureSets)) OF FeatureSetUplinkPerCC-v1540        OPTIONAL

]],

[[

featureSetsDownlink-v15a0           SEQUENCE (SIZE (1..maxDownlinkFeatureSets)) OF FeatureSetDownlink-v15a0         OPTIONAL

]]

}

-- TAG-FEATURESETS-STOP

-- ASN1STOP

#### –	FeatureSetUplink

The IE FeatureSetUplink is used to indicate the features that the UE supports on the carriers corresponding to one band entry in a band combination.

FeatureSetUplink information element

-- ASN1START

-- TAG-FEATURESETUPLINK-START

FeatureSetUplink ::=                SEQUENCE {

featureSetListPerUplinkCC           SEQUENCE (SIZE (1.. maxNrofServingCells)) OF FeatureSetUplinkPerCC-Id,

scalingFactor                       ENUMERATED {f0p4, f0p75, f0p8}                                          OPTIONAL,

crossCarrierScheduling-OtherSCS     ENUMERATED {supported}                                                  OPTIONAL,

intraBandFreqSeparationUL           FreqSeparationClass                                                     OPTIONAL,

searchSpaceSharingCA-UL             ENUMERATED {supported}                                                  OPTIONAL,

dummy1                              DummyI                                                                  OPTIONAL,

supportedSRS-Resources              SRS-Resources                                                           OPTIONAL,

twoPUCCH-Group                      ENUMERATED {supported}                                                  OPTIONAL,

dynamicSwitchSUL                    ENUMERATED {supported}                                                  OPTIONAL,

simultaneousTxSUL-NonSUL            ENUMERATED {supported}                                                  OPTIONAL,

pusch-ProcessingType1-DifferentTB-PerSlot SEQUENCE {

scs-15kHz                                 ENUMERATED {upto2, upto4, upto7}                                  OPTIONAL,

scs-30kHz                                 ENUMERATED {upto2, upto4, upto7}                                  OPTIONAL,

scs-60kHz                                 ENUMERATED {upto2, upto4, upto7}                                  OPTIONAL,

scs-120kHz                                ENUMERATED {upto2, upto4, upto7}                                  OPTIONAL

}                                                                                                           OPTIONAL,

dummy2                               DummyF                                                                 OPTIONAL

}

FeatureSetUplink-v1540 ::=           SEQUENCE {

zeroSlotOffsetAperiodicSRS           ENUMERATED {supported}                     OPTIONAL,

pa-PhaseDiscontinuityImpacts         ENUMERATED {supported}                     OPTIONAL,

pusch-SeparationWithGap              ENUMERATED {supported}                     OPTIONAL,

pusch-ProcessingType2                SEQUENCE {

scs-15kHz                            ProcessingParameters                       OPTIONAL,

scs-30kHz                            ProcessingParameters                       OPTIONAL,

scs-60kHz                            ProcessingParameters                       OPTIONAL

}                                                                               OPTIONAL,

ul-MCS-TableAlt-DynamicIndication    ENUMERATED {supported}                     OPTIONAL

}

SRS-Resources ::=                           SEQUENCE {

maxNumberAperiodicSRS-PerBWP                ENUMERATED {n1, n2, n4, n8, n16},

maxNumberAperiodicSRS-PerBWP-PerSlot        INTEGER (1..6),

maxNumberPeriodicSRS-PerBWP                 ENUMERATED {n1, n2, n4, n8, n16},

maxNumberPeriodicSRS-PerBWP-PerSlot         INTEGER (1..6),

maxNumberSemiPersistentSRS-PerBWP           ENUMERATED {n1, n2, n4, n8, n16},

maxNumberSemiPersistentSRS-PerBWP-PerSlot   INTEGER (1..6),

maxNumberSRS-Ports-PerResource              ENUMERATED {n1, n2, n4}

}

DummyF ::=                                  SEQUENCE {

maxNumberPeriodicCSI-ReportPerBWP           INTEGER (1..4),

maxNumberAperiodicCSI-ReportPerBWP          INTEGER (1..4),

maxNumberSemiPersistentCSI-ReportPerBWP     INTEGER (0..4),

simultaneousCSI-ReportsAllCC                INTEGER (5..32)

}

-- TAG-FEATURESETUPLINK-STOP

-- ASN1STOP

FeatureSetUplink field descriptions

crossCarrierScheduling-OtherSCS The UE shall set this field to the same value as crossCarrierScheduling-OtherSCS in the associated FeatureSetDownlink (if present).

featureSetListPerUplinkCC Indicates which features the UE supports on the individual UL carriers of the feature set (and hence of a band entry that refers to the feature set). The UE shall hence include at least as many FeatureSetUplinkPerCC-Id in this list as the number of carriers it supports according to the ca-BandwidthClassUL, except if indicating additional functionality by reducing the number of FeatureSetUplinkPerCC-Id in the feature set (see NOTE 1 in FeatureSetCombination IE description). The order of the elements in this list is not relevant, i.e., the network may configure any of the carriers in accordance with any of the FeatureSetUplinkPerCC-Id in this list.

#### –	FeatureSetUplinkId

The IE FeatureSetUplinkId identifies an uplink feature set. The FeatureSetUplinkId of a FeatureSetUplink is the index position of the FeatureSetUplink in the featureSetsUplink list in the FeatureSets IE. The first element in the list is referred to by FeatureSetUplinkId = 1, and so on. The FeatureSetUplinkId =0 is not used by an actual FeatureSetUplink but means that the UE does not support a carrier in this band of a band combination.

FeatureSetUplinkId information element

-- ASN1START

-- TAG-FEATURESETUPLINKID-START

FeatureSetUplinkId ::=                  INTEGER (0..maxUplinkFeatureSets)

-- TAG-FEATURESETUPLINKID-STOP

-- ASN1STOP

#### –	FeatureSetUplinkPerCC

The IE FeatureSetUplinkPerCC indicates a set of features that the UE supports on the corresponding carrier of one band entry of a band combination.

FeatureSetUplinkPerCC information element

-- ASN1START

-- TAG-FEATURESETUPLINKPERCC-START

FeatureSetUplinkPerCC ::=               SEQUENCE {

supportedSubcarrierSpacingUL            SubcarrierSpacing,

supportedBandwidthUL                    SupportedBandwidth,

channelBW-90mhz                         ENUMERATED {supported}                      OPTIONAL,

mimo-CB-PUSCH                           SEQUENCE {

maxNumberMIMO-LayersCB-PUSCH            MIMO-LayersUL                               OPTIONAL,

maxNumberSRS-ResourcePerSet             INTEGER (1..2)

}                                                                                   OPTIONAL,

maxNumberMIMO-LayersNonCB-PUSCH         MIMO-LayersUL                               OPTIONAL,

supportedModulationOrderUL              ModulationOrder                             OPTIONAL

}

FeatureSetUplinkPerCC-v1540 ::=       SEQUENCE {

mimo-NonCB-PUSCH                      SEQUENCE {

maxNumberSRS-ResourcePerSet           INTEGER (1..4),

maxNumberSimultaneousSRS-ResourceTx   INTEGER (1..4)

} OPTIONAL

}

-- TAG-FEATURESETUPLINKPERCC-STOP

-- ASN1STOP

#### –	FeatureSetUplinkPerCC-Id

The IE FeatureSetUplinkPerCC-Id identifies a set of features applicable to one carrier of a feature set. The FeatureSetUplinkPerCC-Id of a FeatureSetUplinkPerCC is the index position of the FeatureSetUplinkPerCC in the featureSetsUplinkPerCC. The first element in the list is referred to by FeatureSetUplinkPerCC-Id = 1, and so on.

FeatureSetUplinkPerCC-Id information element

-- ASN1START

-- TAG-FEATURESETUPLINKPERCC-ID-START

FeatureSetUplinkPerCC-Id ::=            INTEGER (1..maxPerCC-FeatureSets)

-- TAG-FEATURESETUPLINKPERCC-ID-STOP

-- ASN1STOP

#### –	FreqBandIndicatorEUTRA

-- ASN1START

-- TAG-FREQBANDINDICATOREUTRA-START

FreqBandIndicatorEUTRA ::=  INTEGER (1..maxBandsEUTRA)

-- TAG-FREQBANDINDICATOREUTRA-STOP

-- ASN1STOP

#### –	FreqBandList

The IE FreqBandList is used by the network to request NR CA and/or MR-DC band combinations for specific NR and/or E-UTRA frequency bands and/or up to a specific number of carriers and/or up to specific aggregated bandwidth. This is also used to request feature sets (for NR) and feature set combinations (for NR and MR-DC).

FreqBandList information element

-- ASN1START

-- TAG-FREQBANDLIST-START

FreqBandList ::=                SEQUENCE (SIZE (1..maxBandsMRDC)) OF FreqBandInformation

FreqBandInformation ::=         CHOICE {

bandInformationEUTRA            FreqBandInformationEUTRA,

bandInformationNR               FreqBandInformationNR

}

FreqBandInformationEUTRA ::=    SEQUENCE {

bandEUTRA                       FreqBandIndicatorEUTRA,

ca-BandwidthClassDL-EUTRA       CA-BandwidthClassEUTRA                  OPTIONAL,   -- Need N

ca-BandwidthClassUL-EUTRA       CA-BandwidthClassEUTRA                  OPTIONAL    -- Need N

}

FreqBandInformationNR ::=       SEQUENCE {

bandNR                          FreqBandIndicatorNR,

maxBandwidthRequestedDL         AggregatedBandwidth                     OPTIONAL,   -- Need N

maxBandwidthRequestedUL         AggregatedBandwidth                     OPTIONAL,   -- Need N

maxCarriersRequestedDL          INTEGER (1..maxNrofServingCells)        OPTIONAL,   -- Need N

maxCarriersRequestedUL          INTEGER (1..maxNrofServingCells)        OPTIONAL    -- Need N

}

AggregatedBandwidth ::=         ENUMERATED {mhz50, mhz100, mhz150, mhz200, mhz250, mhz300, mhz350,

mhz400, mhz450, mhz500, mhz550, mhz600, mhz650, mhz700, mhz750, mhz800}

-- TAG-FREQBANDLIST-STOP

-- ASN1STOP

#### –	FreqSeparationClass

The IE FreqSeparationClass is used for an intra-band non-contiguous CA band combination to indicate frequency separation between lower edge of lowest CC and upper edge of highest CC in a frequency band.

FreqSeparationClass information element

-- ASN1START

-- TAG-FREQSEPARATIONCLASS-START

FreqSeparationClass ::= ENUMERATED {c1, c2, c3, ...}

-- TAG-FREQSEPARATIONCLASS-STOP

-- ASN1STOP

#### –	IMS-Parameters

The IE IMS-Parameters is used to convery capabilities related to IMS.

IMS-Parameters information element

-- ASN1START

-- TAG-IMS-PARAMETERS-START

IMS-Parameters ::=         SEQUENCE {

ims-ParametersCommon       IMS-ParametersCommon                  OPTIONAL,

ims-ParametersFRX-Diff     IMS-ParametersFRX-Diff                OPTIONAL,

...

}

IMS-ParametersCommon ::=   SEQUENCE {

voiceOverEUTRA-5GC         ENUMERATED {supported}                OPTIONAL,

...,

[[

voiceOverSCG-BearerEUTRA-5GC       ENUMERATED {supported}        OPTIONAL

]]

}

IMS-ParametersFRX-Diff ::= SEQUENCE {

voiceOverNR                ENUMERATED {supported}                OPTIONAL,

...

}

-- TAG-IMS-PARAMETERS-STOP

-- ASN1STOP

#### –	InterRAT-Parameters

The IE InterRAT-Parameters is used convey UE capabilities related to the other RATs.

InterRAT-Parameters information element

-- ASN1START

-- TAG-INTERRAT-PARAMETERS-START

InterRAT-Parameters ::=             SEQUENCE {

eutra                               EUTRA-Parameters                OPTIONAL,

...

}

EUTRA-Parameters ::=                SEQUENCE {

supportedBandListEUTRA          SEQUENCE (SIZE (1..maxBandsEUTRA)) OF FreqBandIndicatorEUTRA,

eutra-ParametersCommon              EUTRA-ParametersCommon                                      OPTIONAL,

eutra-ParametersXDD-Diff            EUTRA-ParametersXDD-Diff                                    OPTIONAL,

...

}

EUTRA-ParametersCommon ::=      SEQUENCE {

mfbi-EUTRA                          ENUMERATED {supported}          OPTIONAL,

modifiedMPR-BehaviorEUTRA           BIT STRING (SIZE (32))          OPTIONAL,

multiNS-Pmax-EUTRA                  ENUMERATED {supported}          OPTIONAL,

rs-SINR-MeasEUTRA                   ENUMERATED {supported}          OPTIONAL,

...,

[[

ne-DC                               ENUMERATED {supported}          OPTIONAL

]]

}

EUTRA-ParametersXDD-Diff ::=        SEQUENCE {

rsrqMeasWidebandEUTRA               ENUMERATED {supported}          OPTIONAL,

...

}

-- TAG-INTERRAT-PARAMETERS-STOP

-- ASN1STOP

#### –	MAC-Parameters

The IE MAC-Parameters is used to convey capabilities related to MAC.

MAC-Parameters information element

-- ASN1START

-- TAG-MAC-PARAMETERS-START

MAC-Parameters ::= SEQUENCE {

mac-ParametersCommon            MAC-ParametersCommon        OPTIONAL,

mac-ParametersXDD-Diff          MAC-ParametersXDD-Diff      OPTIONAL

}

MAC-ParametersCommon ::=    SEQUENCE {

lcp-Restriction                 ENUMERATED {supported}      OPTIONAL,

dummy                           ENUMERATED {supported}      OPTIONAL,

lch-ToSCellRestriction          ENUMERATED {supported}      OPTIONAL,

...,

[[

recommendedBitRate              ENUMERATED {supported}      OPTIONAL,

recommendedBitRateQuery         ENUMERATED {supported}      OPTIONAL

]]

}

MAC-ParametersXDD-Diff ::=  SEQUENCE {

skipUplinkTxDynamic             ENUMERATED {supported}     OPTIONAL,

logicalChannelSR-DelayTimer     ENUMERATED {supported}     OPTIONAL,

longDRX-Cycle                   ENUMERATED {supported}     OPTIONAL,

shortDRX-Cycle                  ENUMERATED {supported}     OPTIONAL,

multipleSR-Configurations       ENUMERATED {supported}     OPTIONAL,

multipleConfiguredGrants    ENUMERATED {supported}         OPTIONAL,

...

}

-- TAG-MAC-PARAMETERS-STOP

-- ASN1STOP

#### –	MeasAndMobParameters

The IE MeasAndMobParameters is used to convey UE capabilities related to measurements for radio resource management (RRM), radio link monitoring (RLM) and mobility (e.g. handover).

MeasAndMobParameters information element

-- ASN1START

-- TAG-MEASANDMOBPARAMETERS-START

MeasAndMobParameters ::=                    SEQUENCE {

measAndMobParametersCommon              MeasAndMobParametersCommon              OPTIONAL,

measAndMobParametersXDD-Diff                MeasAndMobParametersXDD-Diff        OPTIONAL,

measAndMobParametersFRX-Diff                MeasAndMobParametersFRX-Diff        OPTIONAL

}

MeasAndMobParametersCommon ::=          SEQUENCE {

supportedGapPattern                     BIT STRING (SIZE (22))                  OPTIONAL,

ssb-RLM                                 ENUMERATED {supported}                  OPTIONAL,

ssb-AndCSI-RS-RLM                       ENUMERATED {supported}                  OPTIONAL,

...,

[[

eventB-MeasAndReport                    ENUMERATED {supported}                  OPTIONAL,

handoverFDD-TDD                         ENUMERATED {supported}                  OPTIONAL,

eutra-CGI-Reporting                     ENUMERATED {supported}                  OPTIONAL,

nr-CGI-Reporting                        ENUMERATED {supported}                  OPTIONAL

]],

[[

independentGapConfig                    ENUMERATED {supported}                  OPTIONAL,

periodicEUTRA-MeasAndReport             ENUMERATED {supported}                  OPTIONAL,

handoverFR1-FR2                         ENUMERATED {supported}                  OPTIONAL,

maxNumberCSI-RS-RRM-RS-SINR             ENUMERATED {n4, n8, n16, n32, n64, n96} OPTIONAL

]],

[[

nr-CGI-Reporting-ENDC                   ENUMERATED {supported}                  OPTIONAL

]],

[[

eutra-CGI-Reporting-NEDC                ENUMERATED {supported}                  OPTIONAL,

eutra-CGI-Reporting-NRDC                ENUMERATED {supported}                  OPTIONAL,

nr-CGI-Reporting-NEDC                   ENUMERATED {supported}                  OPTIONAL,

nr-CGI-Reporting-NRDC                   ENUMERATED {supported}                  OPTIONAL

]]

}

MeasAndMobParametersXDD-Diff ::=            SEQUENCE {

intraAndInterF-MeasAndReport        ENUMERATED {supported}                      OPTIONAL,

eventA-MeasAndReport                ENUMERATED {supported}                      OPTIONAL,

...,

[[

handoverInterF                      ENUMERATED {supported}                      OPTIONAL,

handoverLTE-EPC                     ENUMERATED {supported}                      OPTIONAL,

handoverLTE-5GC                     ENUMERATED {supported}                      OPTIONAL

]],

[[

sftd-MeasNR-Neigh                   ENUMERATED {supported}                      OPTIONAL,

sftd-MeasNR-Neigh-DRX               ENUMERATED {supported}                      OPTIONAL

]]

}

MeasAndMobParametersFRX-Diff ::=            SEQUENCE {

ss-SINR-Meas                                ENUMERATED {supported}              OPTIONAL,

csi-RSRP-AndRSRQ-MeasWithSSB                ENUMERATED {supported}              OPTIONAL,

csi-RSRP-AndRSRQ-MeasWithoutSSB             ENUMERATED {supported}              OPTIONAL,

csi-SINR-Meas                               ENUMERATED {supported}              OPTIONAL,

csi-RS-RLM                                  ENUMERATED {supported}              OPTIONAL,

...,

[[

handoverInterF                              ENUMERATED {supported}              OPTIONAL,

handoverLTE-EPC                             ENUMERATED {supported}              OPTIONAL,

handoverLTE-5GC                             ENUMERATED {supported}              OPTIONAL

]],

[[

maxNumberResource-CSI-RS-RLM                ENUMERATED {n2, n4, n6, n8}         OPTIONAL

]],

[[

simultaneousRxDataSSB-DiffNumerology        ENUMERATED {supported}              OPTIONAL

]]

}

-- TAG-MEASANDMOBPARAMETERS-STOP

-- ASN1STOP

#### –	MeasAndMobParametersMRDC

The IE MeasAndMobParametersMRDC is used to convey capability parameters related to RRM measurements and RRC mobility.

MeasAndMobParametersMRDC information element

-- ASN1START

-- TAG-MEASANDMOBPARAMETERSMRDC-START

MeasAndMobParametersMRDC ::=            SEQUENCE {

measAndMobParametersMRDC-Common         MeasAndMobParametersMRDC-Common                 OPTIONAL,

measAndMobParametersMRDC-XDD-Diff       MeasAndMobParametersMRDC-XDD-Diff               OPTIONAL,

measAndMobParametersMRDC-FRX-Diff       MeasAndMobParametersMRDC-FRX-Diff               OPTIONAL

}

MeasAndMobParametersMRDC-v1560 ::=      SEQUENCE {

measAndMobParametersMRDC-XDD-Diff-v1560    MeasAndMobParametersMRDC-XDD-Diff-v1560     OPTIONAL

}

MeasAndMobParametersMRDC-Common ::=     SEQUENCE {

independentGapConfig                    ENUMERATED {supported}                         OPTIONAL

}

MeasAndMobParametersMRDC-XDD-Diff ::=   SEQUENCE {

sftd-MeasPSCell                         ENUMERATED {supported}                         OPTIONAL,

sftd-MeasNR-Cell                        ENUMERATED {supported}                         OPTIONAL

}

MeasAndMobParametersMRDC-XDD-Diff-v1560 ::=    SEQUENCE {

sftd-MeasPSCell-NEDC                           ENUMERATED {supported}                  OPTIONAL

}

MeasAndMobParametersMRDC-FRX-Diff ::=          SEQUENCE {

simultaneousRxDataSSB-DiffNumerology           ENUMERATED {supported}                  OPTIONAL

}

-- TAG-MEASANDMOBPARAMETERSMRDC-STOP

-- ASN1STOP

#### –	MIMO-Layers

The IE MIMO-Layers is used to convey the number of supported MIMO layers.

MIMO-Layers information element

-- ASN1START

-- TAG-MIMO-LAYERS-START

MIMO-LayersDL ::=   ENUMERATED {twoLayers, fourLayers, eightLayers}

MIMO-LayersUL ::=   ENUMERATED {oneLayer, twoLayers, fourLayers}

-- TAG-MIMO-LAYERS-STOP

-- ASN1STOP

#### –	MIMO-ParametersPerBand

The IE MIMO-ParametersPerBand is used to convey MIMO related parameters specific for a certain band (not per feature set or band combination).

MIMO-ParametersPerBand information element

-- ASN1START

-- TAG-MIMO-PARAMETERSPERBAND-START

MIMO-ParametersPerBand ::=          SEQUENCE {

tci-StatePDSCH                      SEQUENCE {

maxNumberConfiguredTCIstatesPerCC   ENUMERATED {n4, n8, n16, n32, n64, n128}                                   OPTIONAL,

maxNumberActiveTCI-PerBWP           ENUMERATED {n1, n2, n4, n8}                                                OPTIONAL

}                                                                                                              OPTIONAL,

additionalActiveTCI-StatePDCCH              ENUMERATED {supported}                                             OPTIONAL,

pusch-TransCoherence                        ENUMERATED {nonCoherent, partialCoherent, fullCoherent}            OPTIONAL,

beamCorrespondenceWithoutUL-BeamSweeping    ENUMERATED {supported}                                             OPTIONAL,

periodicBeamReport                          ENUMERATED {supported}                                             OPTIONAL,

aperiodicBeamReport                         ENUMERATED {supported}                                             OPTIONAL,

sp-BeamReportPUCCH                          ENUMERATED {supported}                                             OPTIONAL,

sp-BeamReportPUSCH                          ENUMERATED {supported}                                             OPTIONAL,

dummy1                                      DummyG                                                             OPTIONAL,

maxNumberRxBeam                             INTEGER (2..8)                                                     OPTIONAL,

maxNumberRxTxBeamSwitchDL                   SEQUENCE {

scs-15kHz                                   ENUMERATED {n4, n7, n14}                                           OPTIONAL,

scs-30kHz                                   ENUMERATED {n4, n7, n14}                                           OPTIONAL,

scs-60kHz                                   ENUMERATED {n4, n7, n14}                                           OPTIONAL,

scs-120kHz                                  ENUMERATED {n4, n7, n14}                                           OPTIONAL,

scs-240kHz                                  ENUMERATED {n4, n7, n14}                                           OPTIONAL

}                                                                                                              OPTIONAL,

maxNumberNonGroupBeamReporting              ENUMERATED {n1, n2, n4}                                            OPTIONAL,

groupBeamReporting                          ENUMERATED {supported}                                             OPTIONAL,

uplinkBeamManagement                        SEQUENCE {

maxNumberSRS-ResourcePerSet-BM              ENUMERATED {n2, n4, n8, n16},

maxNumberSRS-ResourceSet                    INTEGER (1..8)

}                                                                                                              OPTIONAL,

maxNumberCSI-RS-BFD                 INTEGER (1..64)                                                            OPTIONAL,

maxNumberSSB-BFD                    INTEGER (1..64)                                                            OPTIONAL,

maxNumberCSI-RS-SSB-CBD             INTEGER (1..256)                                                           OPTIONAL,

dummy2                              ENUMERATED {supported}                                                     OPTIONAL,

twoPortsPTRS-UL                     ENUMERATED {supported}                                                     OPTIONAL,

dummy5                              SRS-Resources                                                              OPTIONAL,

dummy3                              INTEGER (1..4)                                                             OPTIONAL,

beamReportTiming                    SEQUENCE {

scs-15kHz                           ENUMERATED {sym2, sym4, sym8}                                              OPTIONAL,

scs-30kHz                           ENUMERATED {sym4, sym8, sym14, sym28}                                      OPTIONAL,

scs-60kHz                           ENUMERATED {sym8, sym14, sym28}                                            OPTIONAL,

scs-120kHz                          ENUMERATED {sym14, sym28, sym56}                                           OPTIONAL

}                                                                                                              OPTIONAL,

ptrs-DensityRecommendationSetDL     SEQUENCE {

scs-15kHz                           PTRS-DensityRecommendationDL                                               OPTIONAL,

scs-30kHz                           PTRS-DensityRecommendationDL                                               OPTIONAL,

scs-60kHz                           PTRS-DensityRecommendationDL                                               OPTIONAL,

scs-120kHz                          PTRS-DensityRecommendationDL                                               OPTIONAL

}                                                                                                              OPTIONAL,

ptrs-DensityRecommendationSetUL     SEQUENCE {

scs-15kHz                           PTRS-DensityRecommendationUL                                               OPTIONAL,

scs-30kHz                           PTRS-DensityRecommendationUL                                               OPTIONAL,

scs-60kHz                           PTRS-DensityRecommendationUL                                               OPTIONAL,

scs-120kHz                          PTRS-DensityRecommendationUL                                               OPTIONAL

}                                                                                                              OPTIONAL,

dummy4                              DummyH                                                                     OPTIONAL,

aperiodicTRS                        ENUMERATED {supported}                                                     OPTIONAL,

...,

[[

dummy6                              ENUMERATED {true}                                                          OPTIONAL,

beamManagementSSB-CSI-RS            BeamManagementSSB-CSI-RS                                                   OPTIONAL,

beamSwitchTiming                    SEQUENCE {

scs-60kHz                           ENUMERATED {sym14, sym28, sym48, sym224, sym336}                           OPTIONAL,

scs-120kHz                          ENUMERATED {sym14, sym28, sym48, sym224, sym336}                           OPTIONAL

}                                                                                                              OPTIONAL,

codebookParameters                  CodebookParameters                                                         OPTIONAL,

csi-RS-IM-ReceptionForFeedback      CSI-RS-IM-ReceptionForFeedback                                             OPTIONAL,

csi-RS-ProcFrameworkForSRS          CSI-RS-ProcFrameworkForSRS                                                 OPTIONAL,

csi-ReportFramework                 CSI-ReportFramework                                                        OPTIONAL,

csi-RS-ForTracking                  CSI-RS-ForTracking                                                         OPTIONAL,

srs-AssocCSI-RS                     SEQUENCE (SIZE (1.. maxNrofCSI-RS-Resources)) OF SupportedCSI-RS-Resource  OPTIONAL,

spatialRelations                    SpatialRelations                                                           OPTIONAL

]]

}

DummyG ::=                          SEQUENCE {

maxNumberSSB-CSI-RS-ResourceOneTx   ENUMERATED {n8, n16, n32, n64},

maxNumberSSB-CSI-RS-ResourceTwoTx   ENUMERATED {n0, n4, n8, n16, n32, n64},

supportedCSI-RS-Density             ENUMERATED {one, three, oneAndThree}

}

BeamManagementSSB-CSI-RS ::=        SEQUENCE {

maxNumberSSB-CSI-RS-ResourceOneTx   ENUMERATED {n0, n8, n16, n32, n64},

maxNumberCSI-RS-Resource            ENUMERATED {n0, n4, n8, n16, n32, n64},

maxNumberCSI-RS-ResourceTwoTx       ENUMERATED {n0, n4, n8, n16, n32, n64},

supportedCSI-RS-Density             ENUMERATED {one, three, oneAndThree}                                       OPTIONAL,

maxNumberAperiodicCSI-RS-Resource   ENUMERATED {n0, n1, n4, n8, n16, n32, n64}

}

DummyH ::=                          SEQUENCE {

burstLength                         INTEGER (1..2),

maxSimultaneousResourceSetsPerCC    INTEGER (1..8),

maxConfiguredResourceSetsPerCC      INTEGER (1..64),

maxConfiguredResourceSetsAllCC      INTEGER (1..128)

}

CSI-RS-ForTracking ::=              SEQUENCE {

maxBurstLength                      INTEGER (1..2),

maxSimultaneousResourceSetsPerCC    INTEGER (1..8),

maxConfiguredResourceSetsPerCC      INTEGER (1..64),

maxConfiguredResourceSetsAllCC      INTEGER (1..256)

}

CSI-RS-IM-ReceptionForFeedback ::=              SEQUENCE {

maxConfigNumberNZP-CSI-RS-PerCC                 INTEGER (1..64),

maxConfigNumberPortsAcrossNZP-CSI-RS-PerCC      INTEGER (2..256),

maxConfigNumberCSI-IM-PerCC                     ENUMERATED {n1, n2, n4, n8, n16, n32},

maxNumberSimultaneousNZP-CSI-RS-PerCC           INTEGER (1..64),

totalNumberPortsSimultaneousNZP-CSI-RS-PerCC    INTEGER (2..256)

}

CSI-RS-ProcFrameworkForSRS ::=                  SEQUENCE {

maxNumberPeriodicSRS-AssocCSI-RS-PerBWP         INTEGER (1..4),

maxNumberAperiodicSRS-AssocCSI-RS-PerBWP        INTEGER (1..4),

maxNumberSP-SRS-AssocCSI-RS-PerBWP              INTEGER (0..4),

simultaneousSRS-AssocCSI-RS-PerCC               INTEGER (1..8)

}

CSI-ReportFramework ::=                         SEQUENCE {

maxNumberPeriodicCSI-PerBWP-ForCSI-Report       INTEGER (1..4),

maxNumberAperiodicCSI-PerBWP-ForCSI-Report      INTEGER (1..4),

maxNumberSemiPersistentCSI-PerBWP-ForCSI-Report INTEGER (0..4),

maxNumberPeriodicCSI-PerBWP-ForBeamReport       INTEGER (1..4),

maxNumberAperiodicCSI-PerBWP-ForBeamReport      INTEGER (1..4),

maxNumberAperiodicCSI-triggeringStatePerCC      ENUMERATED {n3, n7, n15, n31, n63, n128},

maxNumberSemiPersistentCSI-PerBWP-ForBeamReport INTEGER (0..4),

simultaneousCSI-ReportsPerCC                    INTEGER (1..8)

}

PTRS-DensityRecommendationDL ::=    SEQUENCE {

frequencyDensity1                   INTEGER (1..276),

frequencyDensity2                   INTEGER (1..276),

timeDensity1                        INTEGER (0..29),

timeDensity2                        INTEGER (0..29),

timeDensity3                        INTEGER (0..29)

}

PTRS-DensityRecommendationUL ::=    SEQUENCE {

frequencyDensity1                   INTEGER (1..276),

frequencyDensity2                   INTEGER (1..276),

timeDensity1                        INTEGER (0..29),

timeDensity2                        INTEGER (0..29),

timeDensity3                        INTEGER (0..29),

sampleDensity1                      INTEGER (1..276),

sampleDensity2                      INTEGER (1..276),

sampleDensity3                      INTEGER (1..276),

sampleDensity4                      INTEGER (1..276),

sampleDensity5                      INTEGER (1..276)

}

SpatialRelations ::=                    SEQUENCE {

maxNumberConfiguredSpatialRelations     ENUMERATED {n4, n8, n16, n32, n64, n96},

maxNumberActiveSpatialRelations         ENUMERATED {n1, n2, n4, n8, n14},

additionalActiveSpatialRelationPUCCH    ENUMERATED {supported}                              OPTIONAL,

maxNumberDL-RS-QCL-TypeD                ENUMERATED {n1, n2, n4, n8, n14}

}

DummyI ::=               SEQUENCE {

supportedSRS-TxPortSwitch           ENUMERATED {t1r2, t1r4, t2r4, t1r4-t2r4, tr-equal},

txSwitchImpactToRx                  ENUMERATED {true}                                       OPTIONAL

}

-- TAG-MIMO-PARAMETERSPERBAND-STOP

-- ASN1STOP

MIMO-ParametersPerBand field description

csi-RS-IM-ReceptionForFeedback/ csi-RS-ProcFrameworkForSRS/ csi-ReportFramework CSI related capabilities which the UE supports on each of the carriers operated on this band. If the network configures the UE with serving cells on both FR1 and FR2 bands, these values may be further limited by the corresponding fields in fr1-fr2-Add-UE-NR-Capabilities.

#### –	ModulationOrder

The IE ModulationOrder is used to convey the maximum supported modulation order.

ModulationOrder information element

-- ASN1START

-- TAG-MODULATIONORDER-START

ModulationOrder ::= ENUMERATED {bpsk-halfpi, bpsk, qpsk, qam16, qam64, qam256}

-- TAG-MODULATIONORDER-STOP

-- ASN1STOP

#### –	MRDC-Parameters

The IE MRDC-Parameters contains the band combination parameters specific to MR-DC for a given MR-DC band combination.

MRDC-Parameters information element

-- ASN1START

-- TAG-MRDC-PARAMETERS-START

MRDC-Parameters ::= SEQUENCE {

singleUL-Transmission               ENUMERATED {supported}              OPTIONAL,

dynamicPowerSharingENDC             ENUMERATED {supported}              OPTIONAL,

tdm-Pattern                         ENUMERATED {supported}              OPTIONAL,

ul-SharingEUTRA-NR                  ENUMERATED {tdm, fdm, both}         OPTIONAL,

ul-SwitchingTimeEUTRA-NR            ENUMERATED {type1, type2}           OPTIONAL,

simultaneousRxTxInterBandENDC       ENUMERATED {supported}              OPTIONAL,

asyncIntraBandENDC                  ENUMERATED {supported}              OPTIONAL,

...,

[[

dualPA-Architecture                 ENUMERATED {supported}              OPTIONAL,

intraBandENDC-Support               ENUMERATED {non-contiguous, both}   OPTIONAL,

ul-TimingAlignmentEUTRA-NR          ENUMERATED {required}               OPTIONAL

]]

}

MRDC-Parameters-v1580 ::= SEQUENCE {

dynamicPowerSharingNEDC             ENUMERATED {supported}              OPTIONAL

}

MRDC-Parameters-v1590 ::=   SEQUENCE {

interBandContiguousMRDC             ENUMERATED {supported}              OPTIONAL

}

-- TAG-MRDC-PARAMETERS-STOP

-- ASN1STOP

#### –	NRDC-Parameters

The IE NRDC-Parameters contains parameters specific to NR-DC, i.e., which are not applicable to NR SA.

NRDC-Parameters information element

-- ASN1START

-- TAG-NRDC-PARAMETERS-START

NRDC-Parameters ::=                 SEQUENCE {

measAndMobParametersNRDC            MeasAndMobParametersMRDC                    OPTIONAL,

generalParametersNRDC               GeneralParametersMRDC-XDD-Diff              OPTIONAL,

fdd-Add-UE-NRDC-Capabilities        UE-MRDC-CapabilityAddXDD-Mode               OPTIONAL,

tdd-Add-UE-NRDC-Capabilities        UE-MRDC-CapabilityAddXDD-Mode               OPTIONAL,

fr1-Add-UE-NRDC-Capabilities        UE-MRDC-CapabilityAddFRX-Mode               OPTIONAL,

fr2-Add-UE-NRDC-Capabilities        UE-MRDC-CapabilityAddFRX-Mode               OPTIONAL,

lateNonCriticalExtension            OCTET STRING                                OPTIONAL,

dummy                               SEQUENCE {}                                 OPTIONAL

}

NRDC-Parameters-v1570 ::=           SEQUENCE {

sfn-SyncNRDC                        ENUMERATED {supported}                      OPTIONAL

}

-- TAG-NRDC-PARAMETERS-STOP

-- ASN1STOP

#### –	PDCP-Parameters

The IE PDCP-Parameters is used to convey capabilities related to PDCP.

PDCP-Parameters information element

-- ASN1START

-- TAG-PDCP-PARAMETERS-START

PDCP-Parameters ::=         SEQUENCE {

supportedROHC-Profiles      SEQUENCE {

profile0x0000               BOOLEAN,

profile0x0001               BOOLEAN,

profile0x0002               BOOLEAN,

profile0x0003               BOOLEAN,

profile0x0004               BOOLEAN,

profile0x0006               BOOLEAN,

profile0x0101               BOOLEAN,

profile0x0102               BOOLEAN,

profile0x0103               BOOLEAN,

profile0x0104               BOOLEAN

},

maxNumberROHC-ContextSessions       ENUMERATED {cs2, cs4, cs8, cs12, cs16, cs24, cs32, cs48, cs64,

cs128, cs256, cs512, cs1024, cs16384, spare2, spare1},

uplinkOnlyROHC-Profiles             ENUMERATED {supported}      OPTIONAL,

continueROHC-Context                ENUMERATED {supported}      OPTIONAL,

outOfOrderDelivery                  ENUMERATED {supported}      OPTIONAL,

shortSN                             ENUMERATED {supported}      OPTIONAL,

pdcp-DuplicationSRB                 ENUMERATED {supported}      OPTIONAL,

pdcp-DuplicationMCG-OrSCG-DRB       ENUMERATED {supported}      OPTIONAL,

...

}

-- TAG-PDCP-PARAMETERS-STOP

-- ASN1STOP

#### –	PDCP-ParametersMRDC

The IE PDCP-ParametersMRDC is used to convey PDCP related capabilities for MR-DC.

PDCP-ParametersMRDC information element

-- ASN1START

-- TAG-PDCP-PARAMETERSMRDC-START

PDCP-ParametersMRDC ::=                 SEQUENCE {

pdcp-DuplicationSplitSRB                ENUMERATED {supported}      OPTIONAL,

pdcp-DuplicationSplitDRB                ENUMERATED {supported}      OPTIONAL

}

-- TAG-PDCP-PARAMETERSMRDC-STOP

-- ASN1STOP

#### –	Phy-Parameters

The IE Phy-Parameters is used to convey the physical layer capabilities.

Phy-Parameters information element

-- ASN1START

-- TAG-PHY-PARAMETERS-START

Phy-Parameters ::=                  SEQUENCE {

phy-ParametersCommon                Phy-ParametersCommon                        OPTIONAL,

phy-ParametersXDD-Diff              Phy-ParametersXDD-Diff                      OPTIONAL,

phy-ParametersFRX-Diff              Phy-ParametersFRX-Diff                      OPTIONAL,

phy-ParametersFR1                   Phy-ParametersFR1                           OPTIONAL,

phy-ParametersFR2                   Phy-ParametersFR2                           OPTIONAL

}

Phy-ParametersCommon ::=            SEQUENCE {

csi-RS-CFRA-ForHO                   ENUMERATED {supported}                      OPTIONAL,

dynamicPRB-BundlingDL               ENUMERATED {supported}                      OPTIONAL,

sp-CSI-ReportPUCCH                  ENUMERATED {supported}                      OPTIONAL,

sp-CSI-ReportPUSCH                  ENUMERATED {supported}                      OPTIONAL,

nzp-CSI-RS-IntefMgmt                ENUMERATED {supported}                      OPTIONAL,

type2-SP-CSI-Feedback-LongPUCCH     ENUMERATED {supported}                      OPTIONAL,

precoderGranularityCORESET          ENUMERATED {supported}                      OPTIONAL,

dynamicHARQ-ACK-Codebook            ENUMERATED {supported}                      OPTIONAL,

semiStaticHARQ-ACK-Codebook         ENUMERATED {supported}                      OPTIONAL,

spatialBundlingHARQ-ACK             ENUMERATED {supported}                      OPTIONAL,

dynamicBetaOffsetInd-HARQ-ACK-CSI   ENUMERATED {supported}                      OPTIONAL,

pucch-Repetition-F1-3-4             ENUMERATED {supported}                      OPTIONAL,

ra-Type0-PUSCH                      ENUMERATED {supported}                      OPTIONAL,

dynamicSwitchRA-Type0-1-PDSCH       ENUMERATED {supported}                      OPTIONAL,

dynamicSwitchRA-Type0-1-PUSCH       ENUMERATED {supported}                      OPTIONAL,

pdsch-MappingTypeA                  ENUMERATED {supported}                      OPTIONAL,

pdsch-MappingTypeB                  ENUMERATED {supported}                      OPTIONAL,

interleavingVRB-ToPRB-PDSCH         ENUMERATED {supported}                      OPTIONAL,

interSlotFreqHopping-PUSCH          ENUMERATED {supported}                      OPTIONAL,

type1-PUSCH-RepetitionMultiSlots    ENUMERATED {supported}                      OPTIONAL,

type2-PUSCH-RepetitionMultiSlots    ENUMERATED {supported}                      OPTIONAL,

pusch-RepetitionMultiSlots          ENUMERATED {supported}                      OPTIONAL,

pdsch-RepetitionMultiSlots          ENUMERATED {supported}                      OPTIONAL,

downlinkSPS                         ENUMERATED {supported}                      OPTIONAL,

configuredUL-GrantType1             ENUMERATED {supported}                      OPTIONAL,

configuredUL-GrantType2             ENUMERATED {supported}                      OPTIONAL,

pre-EmptIndication-DL               ENUMERATED {supported}                      OPTIONAL,

cbg-TransIndication-DL              ENUMERATED {supported}                      OPTIONAL,

cbg-TransIndication-UL              ENUMERATED {supported}                      OPTIONAL,

cbg-FlushIndication-DL              ENUMERATED {supported}                      OPTIONAL,

dynamicHARQ-ACK-CodeB-CBG-Retx-DL   ENUMERATED {supported}                      OPTIONAL,

rateMatchingResrcSetSemi-Static     ENUMERATED {supported}                      OPTIONAL,

rateMatchingResrcSetDynamic         ENUMERATED {supported}                      OPTIONAL,

bwp-SwitchingDelay                  ENUMERATED {type1, type2}                   OPTIONAL,

...,

[[

dummy                               ENUMERATED {supported}                      OPTIONAL

]],

[[

maxNumberSearchSpaces               ENUMERATED {n10}                            OPTIONAL,

rateMatchingCtrlResrcSetDynamic     ENUMERATED {supported}                      OPTIONAL,

maxLayersMIMO-Indication            ENUMERATED {supported}                      OPTIONAL

]],

[[

spCellPlacement                     CarrierAggregationVariant                   OPTIONAL

]]

}

Phy-ParametersXDD-Diff ::=          SEQUENCE {

dynamicSFI                          ENUMERATED {supported}                      OPTIONAL,

twoPUCCH-F0-2-ConsecSymbols         ENUMERATED {supported}                      OPTIONAL,

twoDifferentTPC-Loop-PUSCH          ENUMERATED {supported}                      OPTIONAL,

twoDifferentTPC-Loop-PUCCH          ENUMERATED {supported}                      OPTIONAL,

...,

[[

dl-SchedulingOffset-PDSCH-TypeA     ENUMERATED {supported}                      OPTIONAL,

dl-SchedulingOffset-PDSCH-TypeB     ENUMERATED {supported}                      OPTIONAL,

ul-SchedulingOffset                 ENUMERATED {supported}                      OPTIONAL

]]

}

Phy-ParametersFRX-Diff ::=                  SEQUENCE {

dynamicSFI                                  ENUMERATED {supported}                      OPTIONAL,

dummy1                                      BIT STRING (SIZE (2))                       OPTIONAL,

twoFL-DMRS                                  BIT STRING (SIZE (2))                       OPTIONAL,

dummy2                                      BIT STRING (SIZE (2))                       OPTIONAL,

dummy3                                      BIT STRING (SIZE (2))                       OPTIONAL,

supportedDMRS-TypeDL                        ENUMERATED {type1, type1And2}               OPTIONAL,

supportedDMRS-TypeUL                        ENUMERATED {type1, type1And2}               OPTIONAL,

semiOpenLoopCSI                             ENUMERATED {supported}                      OPTIONAL,

csi-ReportWithoutPMI                        ENUMERATED {supported}                      OPTIONAL,

csi-ReportWithoutCQI                        ENUMERATED {supported}                      OPTIONAL,

onePortsPTRS                                BIT STRING (SIZE (2))                       OPTIONAL,

twoPUCCH-F0-2-ConsecSymbols                 ENUMERATED {supported}                      OPTIONAL,

pucch-F2-WithFH                             ENUMERATED {supported}                      OPTIONAL,

pucch-F3-WithFH                             ENUMERATED {supported}                      OPTIONAL,

pucch-F4-WithFH                             ENUMERATED {supported}                      OPTIONAL,

pucch-F0-2WithoutFH                         ENUMERATED {notSupported}                   OPTIONAL,

pucch-F1-3-4WithoutFH                       ENUMERATED {notSupported}                   OPTIONAL,

mux-SR-HARQ-ACK-CSI-PUCCH-MultiPerSlot      ENUMERATED {supported}                      OPTIONAL,

uci-CodeBlockSegmentation                   ENUMERATED {supported}                      OPTIONAL,

onePUCCH-LongAndShortFormat                 ENUMERATED {supported}                      OPTIONAL,

twoPUCCH-AnyOthersInSlot                    ENUMERATED {supported}                      OPTIONAL,

intraSlotFreqHopping-PUSCH                  ENUMERATED {supported}                      OPTIONAL,

pusch-LBRM                                  ENUMERATED {supported}                      OPTIONAL,

pdcch-BlindDetectionCA                      INTEGER (4..16)                             OPTIONAL,

tpc-PUSCH-RNTI                              ENUMERATED {supported}                      OPTIONAL,

tpc-PUCCH-RNTI                              ENUMERATED {supported}                      OPTIONAL,

tpc-SRS-RNTI                                ENUMERATED {supported}                      OPTIONAL,

absoluteTPC-Command                         ENUMERATED {supported}                      OPTIONAL,

twoDifferentTPC-Loop-PUSCH                  ENUMERATED {supported}                      OPTIONAL,

twoDifferentTPC-Loop-PUCCH                  ENUMERATED {supported}                      OPTIONAL,

pusch-HalfPi-BPSK                           ENUMERATED {supported}                      OPTIONAL,

pucch-F3-4-HalfPi-BPSK                      ENUMERATED {supported}                      OPTIONAL,

almostContiguousCP-OFDM-UL                  ENUMERATED {supported}                      OPTIONAL,

sp-CSI-RS                                   ENUMERATED {supported}                      OPTIONAL,

sp-CSI-IM                                   ENUMERATED {supported}                      OPTIONAL,

tdd-MultiDL-UL-SwitchPerSlot                ENUMERATED {supported}                      OPTIONAL,

multipleCORESET                             ENUMERATED {supported}                      OPTIONAL,

...,

[[

csi-RS-IM-ReceptionForFeedback              CSI-RS-IM-ReceptionForFeedback              OPTIONAL,

csi-RS-ProcFrameworkForSRS                  CSI-RS-ProcFrameworkForSRS                  OPTIONAL,

csi-ReportFramework                         CSI-ReportFramework                         OPTIONAL,

mux-SR-HARQ-ACK-CSI-PUCCH-OncePerSlot       SEQUENCE {

sameSymbol                                  ENUMERATED {supported}                      OPTIONAL,

diffSymbol                                  ENUMERATED {supported}                      OPTIONAL

}                                                                                       OPTIONAL,

mux-SR-HARQ-ACK-PUCCH                       ENUMERATED {supported}                      OPTIONAL,

mux-MultipleGroupCtrlCH-Overlap             ENUMERATED {supported}                      OPTIONAL,

dl-SchedulingOffset-PDSCH-TypeA             ENUMERATED {supported}                      OPTIONAL,

dl-SchedulingOffset-PDSCH-TypeB             ENUMERATED {supported}                      OPTIONAL,

ul-SchedulingOffset                         ENUMERATED {supported}                      OPTIONAL,

dl-64QAM-MCS-TableAlt                       ENUMERATED {supported}                      OPTIONAL,

ul-64QAM-MCS-TableAlt                       ENUMERATED {supported}                      OPTIONAL,

cqi-TableAlt                                ENUMERATED {supported}                      OPTIONAL,

oneFL-DMRS-TwoAdditionalDMRS-UL             ENUMERATED {supported}                      OPTIONAL,

twoFL-DMRS-TwoAdditionalDMRS-UL             ENUMERATED {supported}                      OPTIONAL,

oneFL-DMRS-ThreeAdditionalDMRS-UL           ENUMERATED {supported}                      OPTIONAL

]],

[[

pdcch-BlindDetectionNRDC                SEQUENCE {

pdcch-BlindDetectionMCG-UE              INTEGER (1..15),

pdcch-BlindDetectionSCG-UE              INTEGER (1..15)

}                                                                                       OPTIONAL,

mux-HARQ-ACK-PUSCH-DiffSymbol               ENUMERATED {supported}                      OPTIONAL

]]

}

Phy-ParametersFR1 ::=                       SEQUENCE {

pdcch-MonitoringSingleOccasion              ENUMERATED {supported}                      OPTIONAL,

scs-60kHz                                   ENUMERATED {supported}                      OPTIONAL,

pdsch-256QAM-FR1                            ENUMERATED {supported}                      OPTIONAL,

pdsch-RE-MappingFR1-PerSymbol               ENUMERATED {n10, n20}                       OPTIONAL,

...,

[[

pdsch-RE-MappingFR1-PerSlot                 ENUMERATED {n16, n32, n48, n64, n80, n96, n112, n128,

n144, n160, n176, n192, n208, n224, n240, n256}         OPTIONAL

]]

}

Phy-ParametersFR2 ::=                       SEQUENCE {

dummy                                       ENUMERATED {supported}                                  OPTIONAL,

pdsch-RE-MappingFR2-PerSymbol               ENUMERATED {n6, n20}                                    OPTIONAL,

...,

[[

pCell-FR2                                   ENUMERATED {supported}                                  OPTIONAL,

pdsch-RE-MappingFR2-PerSlot                 ENUMERATED {n16, n32, n48, n64, n80, n96, n112, n128,

n144, n160, n176, n192, n208, n224, n240, n256}     OPTIONAL

]]

}

-- TAG-PHY-PARAMETERS-STOP

-- ASN1STOP

Phy-ParametersFRX-Diff field description

csi-RS-IM-ReceptionForFeedback/ csi-RS-ProcFrameworkForSRS/ csi-ReportFramework These fields are optionally present in fr1-fr2-Add-UE-NR-Capabilities in UE-NR-Capability. They shall not be set in any other instance of the IE Phy-ParametersFRX-Diff. If the network configures the UE with serving cells on both FR1 and FR2 bands, these parameters, if present, limit the corresponding parameters in MIMO-ParametersPerBand.

#### –	Phy-ParametersMRDC

The IE Phy-ParametersMRDC is used to convey physical layer capabilities for MR-DC.

Phy-ParametersMRDC information element

-- ASN1START

-- TAG-PHY-PARAMETERSMRDC-START

Phy-ParametersMRDC ::=              SEQUENCE {

naics-Capability-List               SEQUENCE (SIZE (1..maxNrofNAICS-Entries)) OF NAICS-Capability-Entry         OPTIONAL,

...,

[[

spCellPlacement                     CarrierAggregationVariant                   OPTIONAL

]]

}

NAICS-Capability-Entry ::=          SEQUENCE {

numberOfNAICS-CapableCC             INTEGER(1..5),

numberOfAggregatedPRB               ENUMERATED {n50, n75, n100, n125, n150, n175, n200, n225,

n250, n275, n300, n350, n400, n450, n500, spare},

...

}

-- TAG-PHY-PARAMETERSMRDC-STOP

-- ASN1STOP

PHY-ParametersMRDC field descriptions

naics-Capability-List Indicates that UE in MR-DC supports NAICS as defined in TS 36.331 [10].

#### –	ProcessingParameters

The IE ProcessingParameters is used to indicate PDSCH/PUSCH processing capabilities supported by the UE.

ProcessingParameters information element

-- ASN1START

-- TAG-PROCESSINGPARAMETERS-START

ProcessingParameters ::=        SEQUENCE {

fallback                        ENUMERATED {sc, cap1-only},

differentTB-PerSlot              SEQUENCE {

upto1                          NumberOfCarriers                    OPTIONAL,

upto2                          NumberOfCarriers                    OPTIONAL,

upto4                          NumberOfCarriers                    OPTIONAL,

upto7                          NumberOfCarriers                    OPTIONAL

} OPTIONAL

}

NumberOfCarriers ::=    INTEGER (1..16)

-- TAG-PROCESSINGPARAMETERS-STOP

-- ASN1STOP

#### –	RAT-Type

The IE RAT-Type is used to indicate the radio access technology (RAT), including NR, of the requested/transferred UE capabilities.

RAT-Type information element

-- ASN1START

-- TAG-RAT-TYPE-START

RAT-Type ::= ENUMERATED {nr, eutra-nr, eutra, spare1, ...}

-- TAG-RAT-TYPE-STOP

-- ASN1STOP

#### –	RF-Parameters

The IE RF-Parameters is used to convey RF-related capabilities for NR operation.

RF-Parameters information element

-- ASN1START

-- TAG-RF-PARAMETERS-START

RF-Parameters ::=                   SEQUENCE {

supportedBandListNR                 SEQUENCE (SIZE (1..maxBands)) OF BandNR,

supportedBandCombinationList        BandCombinationList                         OPTIONAL,

appliedFreqBandListFilter           FreqBandList                                OPTIONAL,

...,

[[

supportedBandCombinationList-v1540  BandCombinationList-v1540                   OPTIONAL,

srs-SwitchingTimeRequested          ENUMERATED {true}                           OPTIONAL

]],

[[

supportedBandCombinationList-v1550  BandCombinationList-v1550                   OPTIONAL

]],

[[

supportedBandCombinationList-v1560  BandCombinationList-v1560                   OPTIONAL

]]

}

BandNR ::=                          SEQUENCE {

bandNR                              FreqBandIndicatorNR,

modifiedMPR-Behaviour               BIT STRING (SIZE (8))                           OPTIONAL,

mimo-ParametersPerBand              MIMO-ParametersPerBand                          OPTIONAL,

extendedCP                          ENUMERATED {supported}                          OPTIONAL,

multipleTCI                         ENUMERATED {supported}                          OPTIONAL,

bwp-WithoutRestriction              ENUMERATED {supported}                          OPTIONAL,

bwp-SameNumerology                  ENUMERATED {upto2, upto4}                       OPTIONAL,

bwp-DiffNumerology                  ENUMERATED {upto4}                              OPTIONAL,

crossCarrierScheduling-SameSCS      ENUMERATED {supported}                          OPTIONAL,

pdsch-256QAM-FR2                    ENUMERATED {supported}                          OPTIONAL,

pusch-256QAM                        ENUMERATED {supported}                          OPTIONAL,

ue-PowerClass                       ENUMERATED {pc1, pc2, pc3, pc4}                 OPTIONAL,

rateMatchingLTE-CRS                 ENUMERATED {supported}                          OPTIONAL,

channelBWs-DL                       CHOICE {

fr1                                 SEQUENCE {

scs-15kHz                           BIT STRING (SIZE (10))                      OPTIONAL,

scs-30kHz                           BIT STRING (SIZE (10))                      OPTIONAL,

scs-60kHz                           BIT STRING (SIZE (10))                      OPTIONAL

},

fr2                                 SEQUENCE {

scs-60kHz                           BIT STRING (SIZE (3))                       OPTIONAL,

scs-120kHz                          BIT STRING (SIZE (3))                       OPTIONAL

}

}                                                                                   OPTIONAL,

channelBWs-UL                       CHOICE {

fr1                                 SEQUENCE {

scs-15kHz                           BIT STRING (SIZE (10))                      OPTIONAL,

scs-30kHz                           BIT STRING (SIZE (10))                      OPTIONAL,

scs-60kHz                           BIT STRING (SIZE (10))                      OPTIONAL

},

fr2                                 SEQUENCE {

scs-60kHz                           BIT STRING (SIZE (3))                       OPTIONAL,

scs-120kHz                          BIT STRING (SIZE (3))                       OPTIONAL

}

}                                                                                   OPTIONAL,

...,

[[

maxUplinkDutyCycle-PC2-FR1                  ENUMERATED {n60, n70, n80, n90, n100}   OPTIONAL

]],

[[

pucch-SpatialRelInfoMAC-CE          ENUMERATED {supported}                          OPTIONAL,

powerBoosting-pi2BPSK               ENUMERATED {supported}                          OPTIONAL

]],

[[

maxUplinkDutyCycle-FR2          ENUMERATED {n15, n20, n25, n30, n40, n50, n60, n70, n80, n90, n100}     OPTIONAL

]],

[[

channelBWs-DL-v1590                 CHOICE {

fr1                                 SEQUENCE {

scs-15kHz                           BIT STRING (SIZE (16))              OPTIONAL,

scs-30kHz                           BIT STRING (SIZE (16))              OPTIONAL,

scs-60kHz                           BIT STRING (SIZE (16))              OPTIONAL

},

fr2                                 SEQUENCE {

scs-60kHz                           BIT STRING (SIZE (8))               OPTIONAL,

scs-120kHz                          BIT STRING (SIZE (8))               OPTIONAL

}

}                                                                               OPTIONAL,

channelBWs-UL-v1590                 CHOICE {

fr1                                 SEQUENCE {

scs-15kHz                           BIT STRING (SIZE (16))              OPTIONAL,

scs-30kHz                           BIT STRING (SIZE (16))              OPTIONAL,

scs-60kHz                           BIT STRING (SIZE (16))              OPTIONAL

},

fr2                                 SEQUENCE {

scs-60kHz                           BIT STRING (SIZE (8))               OPTIONAL,

scs-120kHz                          BIT STRING (SIZE (8))               OPTIONAL

}

}                                                                               OPTIONAL

]],

[[

asymmetricBandwidthCombinationSet-v15a0     BIT STRING (SIZE (1..32))           OPTIONAL

]]

}

-- TAG-RF-PARAMETERS-STOP

-- ASN1STOP

RF-Parameters field descriptions

appliedFreqBandListFilter In this field the UE mirrors the FreqBandList that the NW provided in the capability enquiry, if any. The UE filtered the band combinations in the supportedBandCombinationList in accordance with this appliedFreqBandListFilter. The UE does not include this field if the UE capability is requested by E-UTRAN and the network request includes the field eutra-nr-only [10].

supportedBandCombinationList A list of band combinations that the UE supports for NR (and NR-DC, if requested). The FeatureSetCombinationId:s in this list refer to the FeatureSetCombination entries in the featureSetCombinations list in the UE-NR-Capability IE. The UE does not include this field if the UE capability is requested by E-UTRAN and the network request includes the field eutra-nr-only [10].

#### –	RF-ParametersMRDC

The IE RF-ParametersMRDC is used to convey RF related capabilities for MR-DC.

RF-ParametersMRDC information element

-- ASN1START

-- TAG-RF-PARAMETERSMRDC-START

RF-ParametersMRDC ::=                   SEQUENCE {

supportedBandCombinationList            BandCombinationList                 OPTIONAL,

appliedFreqBandListFilter               FreqBandList                        OPTIONAL,

...,

[[

srs-SwitchingTimeRequested              ENUMERATED {true}                   OPTIONAL,

supportedBandCombinationList-v1540      BandCombinationList-v1540           OPTIONAL

]],

[[

supportedBandCombinationList-v1550      BandCombinationList-v1550           OPTIONAL

]],

[[

supportedBandCombinationList-v1560      BandCombinationList-v1560           OPTIONAL,

supportedBandCombinationListNEDC-Only   BandCombinationList                 OPTIONAL

]],

[[

supportedBandCombinationList-v1570      BandCombinationList-v1570           OPTIONAL

]],

[[

supportedBandCombinationList-v1580      BandCombinationList-v1580           OPTIONAL

]],

[[

supportedBandCombinationList-v1590      BandCombinationList-v1590           OPTIONAL

]],

[[

supportedBandCombinationListNEDC-Only-v15a0   SEQUENCE {

supportedBandCombinationList-v1540      BandCombinationList-v1540       OPTIONAL,

supportedBandCombinationList-v1560      BandCombinationList-v1560       OPTIONAL,

supportedBandCombinationList-v1570      BandCombinationList-v1570       OPTIONAL,

supportedBandCombinationList-v1580      BandCombinationList-v1580       OPTIONAL,

supportedBandCombinationList-v1590      BandCombinationList-v1590       OPTIONAL

}                                                                           OPTIONAL

]]

}}

-- TAG-RF-PARAMETERSMRDC-STOP

-- ASN1STOP

RF-ParametersMRDC field descriptions

appliedFreqBandListFilter In this field the UE mirrors the FreqBandList that the NW provided in the capability enquiry, if any. The UE filtered the band combinations in the supportedBandCombinationList in accordance with this appliedFreqBandListFilter.

supportedBandCombinationList A list of band combinations that the UE supports for (NG)EN-DC, or both (NG)EN-DC and NE-DC. The FeatureSetCombinationId:s in this list refer to the FeatureSetCombination entries in the featureSetCombinations list in the UE-MRDC-Capability IE.

supportedBandCombinationListNEDC-Only A list of band combinations that the UE supports only for NE-DC. The FeatureSetCombinationId:s in this list refer to the FeatureSetCombination entries in the featureSetCombinations list in the UE-MRDC-Capability IE.

#### –	RLC-Parameters

The IE RLC-Parameters is used to convey capabilities related to RLC.

RLC-Parameters information element

-- ASN1START

-- TAG-RLC-PARAMETERS-START

RLC-Parameters ::= SEQUENCE {

am-WithShortSN                  ENUMERATED {supported}  OPTIONAL,

um-WithShortSN                  ENUMERATED {supported}  OPTIONAL,

um-WithLongSN                   ENUMERATED {supported}  OPTIONAL,

...

}

-- TAG-RLC-PARAMETERS-STOP

-- ASN1STOP

#### –	SDAP-Parameters

The IE SDAP-Parameters is used to convey capabilities related to SDAP.

SDAP-Parameters information element

-- ASN1START

-- TAG-SDAP-PARAMETERS-START

SDAP-Parameters ::= SEQUENCE {

as-ReflectiveQoS                ENUMERATED {true}       OPTIONAL,

...

}

-- TAG-SDAP-PARAMETERS-STOP

-- ASN1STOP

#### –	SRS-SwitchingTimeNR

The IE SRS-SwitchingTimeNR is used to indicate the SRS carrier switching time supported by the UE for one NR band pair.

SRS-SwitchingTimeNR information element

-- ASN1START

-- TAG-SRS-SWITCHINGTIMENR-START

SRS-SwitchingTimeNR ::= SEQUENCE {

switchingTimeDL         ENUMERATED {n0us, n30us, n100us, n140us, n200us, n300us, n500us, n900us}  OPTIONAL,

switchingTimeUL         ENUMERATED {n0us, n30us, n100us, n140us, n200us, n300us, n500us, n900us}  OPTIONAL

}

-- TAG-SRS-SWITCHINGTIMENR-STOP

-- ASN1STOP

#### –	SRS-SwitchingTimeEUTRA

The IE SRS-SwitchingTimeEUTRA is used to indicate the SRS carrier switching time supported by the UE for one E-UTRA band pair.

SRS-SwitchingTimeEUTRA information element

-- ASN1START

-- TAG-SRS-SWITCHINGTIMEEUTRA-START

SRS-SwitchingTimeEUTRA ::= SEQUENCE {

switchingTimeDL            ENUMERATED {n0, n0dot5, n1, n1dot5, n2, n2dot5, n3, n3dot5, n4, n4dot5, n5, n5dot5, n6, n6dot5, n7}

OPTIONAL,

switchingTimeUL            ENUMERATED {n0, n0dot5, n1, n1dot5, n2, n2dot5, n3, n3dot5, n4, n4dot5, n5, n5dot5, n6, n6dot5, n7}

OPTIONAL

}

-- TAG-SRS-SWITCHINGTIMEEUTRA-STOP

-- ASN1STOP

#### –	SupportedBandwidth

The IE SupportedBandwidth is used to indicate the maximum channel bandwidth supported by the UE on one carrier of a band of a band combination.

SupportedBandwidth information element

-- ASN1START

-- TAG-SUPPORTEDBANDWIDTH-START

SupportedBandwidth ::=      CHOICE {

fr1                         ENUMERATED {mhz5, mhz10, mhz15, mhz20, mhz25, mhz30, mhz40, mhz50, mhz60, mhz80, mhz100},

fr2                         ENUMERATED {mhz50, mhz100, mhz200, mhz400}

}

-- TAG-SUPPORTEDBANDWIDTH-STOP

-- ASN1STOP

#### –	UE-CapabilityRAT-ContainerList

The IE UE-CapabilityRAT-ContainerList contains a list of radio access technology specific capability containers.

UE-CapabilityRAT-ContainerList information element

-- ASN1START

-- TAG-UE-CAPABILITYRAT-CONTAINERLIST-START

UE-CapabilityRAT-ContainerList ::=    SEQUENCE (SIZE (0..maxRAT-CapabilityContainers)) OF UE-CapabilityRAT-Container

UE-CapabilityRAT-Container ::=        SEQUENCE {

rat-Type                              RAT-Type,

ue-CapabilityRAT-Container            OCTET STRING

}

-- TAG-UE-CAPABILITYRAT-CONTAINERLIST-STOP

-- ASN1STOP

UE-CapabilityRAT-ContainerList field descriptions

ue-CapabilityRAT-Container Container for the UE capabilities of the indicated RAT. The encoding is defined in the specification of each RAT: For rat-Type set to nr: the encoding of UE capabilities is defined in UE-NR-Capability. For rat-Type set to eutra-nr: the encoding of UE capabilities is defined in UE-MRDC-Capability. For rat-Type set to eutra: the encoding of UE capabilities is defined in UE-EUTRA-Capability specified in TS 36.331 [10].

#### –	UE-CapabilityRAT-RequestList

The IE UE-CapabilityRAT-RequestList is used to request UE capabilities for one or more RATs from the UE.

UE-CapabilityRAT-RequestList information element

-- ASN1START

-- TAG-UE-CAPABILITYRAT-REQUESTLIST-START

UE-CapabilityRAT-RequestList ::=        SEQUENCE (SIZE (1..maxRAT-CapabilityContainers)) OF UE-CapabilityRAT-Request

UE-CapabilityRAT-Request ::=            SEQUENCE {

rat-Type                                RAT-Type,

capabilityRequestFilter                 OCTET STRING                    OPTIONAL,   -- Need N

...

}

-- TAG-UE-CAPABILITYRAT-REQUESTLIST-STOP

-- ASN1STOP

UE-CapabilityRAT-Request field descriptions

capabilityRequestFilter Information by which the network requests the UE to filter the UE capabilities. For rat-Type set to nr or eutra-nr: the encoding of the capabilityRequestFilter is defined in UE-CapabilityRequestFilterNR. For rat-Type set to eutra: the encoding of the capabilityRequestFilter is defined by UECapabilityEnquiry message defined in TS36.331 [10], in which RAT-Type in UE-CapabilityRequest includes only 'eutra'.

rat-Type The RAT type for which the NW requests UE capabilities.

#### –	UE-CapabilityRequestFilterCommon

The IE UE-CapabilityRequestFilterCommon is used to request filtered UE capabilities. The filter is common for all capability containers that are requested.

UE-CapabilityRequestFilterCommon information element

-- ASN1START

-- TAG-UE-CAPABILITYREQUESTFILTERCOMMON-START

UE-CapabilityRequestFilterCommon ::=            SEQUENCE {

mrdc-Request                                SEQUENCE {

omitEN-DC                                   ENUMERATED {true}                      OPTIONAL,    -- Need N

includeNR-DC                                ENUMERATED {true}                      OPTIONAL,    -- Need N

includeNE-DC                                ENUMERATED {true}                      OPTIONAL     -- Need N

}                                                                                  OPTIONAL,        -- Need N

...

}

-- TAG-UE-CAPABILITYREQUESTFILTERCOMMON-STOP

-- ASN1STOP

UE-CapabilityRequestFilterCommon field descriptions

includeNE-DC Only if this field is present, the UE supporting NE-DC shall indicate support for NE-DC in band combinations and include feature set combinations which are applicable to NE-DC. Band combinations supporting both NE-DC and (NG)EN-DC shall be included in supportedBandCombinationList, band combinations supporting only NE-DC shall be included in supportedBandCombinationListNEDC-Only.

includeNR-DC Only if this field is present, the UE supporting NR-DC shall indicate support for NR-DC in band combinations and include feature set combinations which are applicable to NR-DC.

omitEN-DC Only if this field is present, the UE shall omit band combinations and feature set combinations which are only applicable to (NG)EN-DC.

#### –	UE-CapabilityRequestFilterNR

The IE UE-CapabilityRequestFilterNR is used to request filtered UE capabilities.

UE-CapabilityRequestFilterNR information element

-- ASN1START

-- TAG-UE-CAPABILITYREQUESTFILTERNR-START

UE-CapabilityRequestFilterNR ::=            SEQUENCE {

frequencyBandListFilter                     FreqBandList                          OPTIONAL,   -- Need N

nonCriticalExtension                        UE-CapabilityRequestFilterNR-v1540    OPTIONAL

}

UE-CapabilityRequestFilterNR-v1540 ::=      SEQUENCE {

srs-SwitchingTimeRequest                    ENUMERATED {true}                     OPTIONAL,  -- Need N

nonCriticalExtension                        SEQUENCE {}                           OPTIONAL

}

-- TAG-UE-CAPABILITYREQUESTFILTERNR-STOP

-- ASN1STOP

#### –	UE-MRDC-Capability

The IE UE-MRDC-Capability is used to convey the UE Radio Access Capability Parameters for MR-DC, see TS 38.306 [26].

UE-MRDC-Capability information element

-- ASN1START

-- TAG-UE-MRDC-CAPABILITY-START

UE-MRDC-Capability ::=              SEQUENCE {

measAndMobParametersMRDC            MeasAndMobParametersMRDC                                                        OPTIONAL,

phy-ParametersMRDC-v1530            Phy-ParametersMRDC                                                              OPTIONAL,

rf-ParametersMRDC                   RF-ParametersMRDC,

generalParametersMRDC               GeneralParametersMRDC-XDD-Diff                                                  OPTIONAL,

fdd-Add-UE-MRDC-Capabilities        UE-MRDC-CapabilityAddXDD-Mode                                                   OPTIONAL,

tdd-Add-UE-MRDC-Capabilities        UE-MRDC-CapabilityAddXDD-Mode                                                   OPTIONAL,

fr1-Add-UE-MRDC-Capabilities        UE-MRDC-CapabilityAddFRX-Mode                                                   OPTIONAL,

fr2-Add-UE-MRDC-Capabilities        UE-MRDC-CapabilityAddFRX-Mode                                                   OPTIONAL,

featureSetCombinations              SEQUENCE (SIZE (1..maxFeatureSetCombinations)) OF FeatureSetCombination         OPTIONAL,

pdcp-ParametersMRDC-v1530           PDCP-ParametersMRDC                                                             OPTIONAL,

lateNonCriticalExtension            OCTET STRING                                                                    OPTIONAL,

nonCriticalExtension                UE-MRDC-Capability-v1560                                                        OPTIONAL

}

UE-MRDC-Capability-v1560 ::=        SEQUENCE {

receivedFilters                     OCTET STRING (CONTAINING UECapabilityEnquiry-v1560-IEs)                         OPTIONAL,

measAndMobParametersMRDC-v1560      MeasAndMobParametersMRDC-v1560                                                  OPTIONAL,

fdd-Add-UE-MRDC-Capabilities-v1560  UE-MRDC-CapabilityAddXDD-Mode-v1560                                             OPTIONAL,

tdd-Add-UE-MRDC-Capabilities-v1560  UE-MRDC-CapabilityAddXDD-Mode-v1560                                             OPTIONAL,

nonCriticalExtension                SEQUENCE {}                                                                     OPTIONAL

}

UE-MRDC-CapabilityAddXDD-Mode ::=   SEQUENCE {

measAndMobParametersMRDC-XDD-Diff       MeasAndMobParametersMRDC-XDD-Diff                                           OPTIONAL,

generalParametersMRDC-XDD-Diff          GeneralParametersMRDC-XDD-Diff                                              OPTIONAL

}

UE-MRDC-CapabilityAddXDD-Mode-v1560 ::=    SEQUENCE {

measAndMobParametersMRDC-XDD-Diff-v1560    MeasAndMobParametersMRDC-XDD-Diff-v1560                                  OPTIONAL

}

UE-MRDC-CapabilityAddFRX-Mode ::=   SEQUENCE {

measAndMobParametersMRDC-FRX-Diff       MeasAndMobParametersMRDC-FRX-Diff

}

GeneralParametersMRDC-XDD-Diff ::= SEQUENCE {

splitSRB-WithOneUL-Path             ENUMERATED {supported}                                                          OPTIONAL,

splitDRB-withUL-Both-MCG-SCG        ENUMERATED {supported}                                                          OPTIONAL,

srb3                                ENUMERATED {supported}                                                          OPTIONAL,

v2x-EUTRA                           ENUMERATED {supported}                                                          OPTIONAL,

...

}

-- TAG-UE-MRDC-CAPABILITY-STOP

-- ASN1STOP

UE-MRDC-Capability field descriptions

featureSetCombinations A list of FeatureSetCombination:s for supportedBandCombinationList and supportedBandCombinationListNEDC-Only in UE-MRDC-Capability. The FeatureSetDownlink:s and FeatureSetUplink:s referred to from these FeatureSetCombination:s are defined in the featureSets list in UE-NR-Capability.

#### –	UE-NR-Capability

The IE UE-NR-Capability is used to convey the NR UE Radio Access Capability Parameters, see TS 38.306 [26].

UE-NR-Capability information element

-- ASN1START

-- TAG-UE-NR-CAPABILITY-START

UE-NR-Capability ::=            SEQUENCE {

accessStratumRelease            AccessStratumRelease,

pdcp-Parameters                 PDCP-Parameters,

rlc-Parameters                  RLC-Parameters                                                              OPTIONAL,

mac-Parameters                  MAC-Parameters                                                              OPTIONAL,

phy-Parameters                  Phy-Parameters,

rf-Parameters                   RF-Parameters,

measAndMobParameters            MeasAndMobParameters                                                        OPTIONAL,

fdd-Add-UE-NR-Capabilities      UE-NR-CapabilityAddXDD-Mode                                                 OPTIONAL,

tdd-Add-UE-NR-Capabilities      UE-NR-CapabilityAddXDD-Mode                                                 OPTIONAL,

fr1-Add-UE-NR-Capabilities      UE-NR-CapabilityAddFRX-Mode                                                 OPTIONAL,

fr2-Add-UE-NR-Capabilities      UE-NR-CapabilityAddFRX-Mode                                                 OPTIONAL,

featureSets                     FeatureSets                                                                 OPTIONAL,

featureSetCombinations          SEQUENCE (SIZE (1..maxFeatureSetCombinations)) OF FeatureSetCombination     OPTIONAL,

lateNonCriticalExtension        OCTET STRING                                                                OPTIONAL,

nonCriticalExtension            UE-NR-Capability-v1530                                                      OPTIONAL

}

UE-NR-Capability-v1530 ::=               SEQUENCE {

fdd-Add-UE-NR-Capabilities-v1530         UE-NR-CapabilityAddXDD-Mode-v1530                                  OPTIONAL,

tdd-Add-UE-NR-Capabilities-v1530         UE-NR-CapabilityAddXDD-Mode-v1530                                  OPTIONAL,

dummy                                    ENUMERATED {supported}                                             OPTIONAL,

interRAT-Parameters                      InterRAT-Parameters                                                OPTIONAL,

inactiveState                            ENUMERATED {supported}                                             OPTIONAL,

delayBudgetReporting                     ENUMERATED {supported}                                             OPTIONAL,

nonCriticalExtension                     UE-NR-Capability-v1540                                             OPTIONAL

}

UE-NR-Capability-v1540 ::=              SEQUENCE {

sdap-Parameters                         SDAP-Parameters                                                     OPTIONAL,

overheatingInd                          ENUMERATED {supported}                                              OPTIONAL,

ims-Parameters                          IMS-Parameters                                                      OPTIONAL,

fr1-Add-UE-NR-Capabilities-v1540        UE-NR-CapabilityAddFRX-Mode-v1540                                   OPTIONAL,

fr2-Add-UE-NR-Capabilities-v1540        UE-NR-CapabilityAddFRX-Mode-v1540                                   OPTIONAL,

fr1-fr2-Add-UE-NR-Capabilities          UE-NR-CapabilityAddFRX-Mode                                         OPTIONAL,

nonCriticalExtension                    UE-NR-Capability-v1550                                              OPTIONAL

}

UE-NR-Capability-v1550 ::=               SEQUENCE {

reducedCP-Latency                        ENUMERATED {supported}                                             OPTIONAL,

nonCriticalExtension                     UE-NR-Capability-v1560                                             OPTIONAL

}

UE-NR-Capability-v1560 ::=               SEQUENCE {

nrdc-Parameters                         NRDC-Parameters                                                     OPTIONAL,

receivedFilters                         OCTET STRING (CONTAINING UECapabilityEnquiry-v1560-IEs)             OPTIONAL,

nonCriticalExtension                    UE-NR-Capability-v1570                                              OPTIONAL

}

UE-NR-Capability-v1570 ::=               SEQUENCE {

nrdc-Parameters-v1570                   NRDC-Parameters-v1570                                               OPTIONAL,

nonCriticalExtension                    SEQUENCE {}                                                         OPTIONAL

}

UE-NR-CapabilityAddXDD-Mode ::=         SEQUENCE {

phy-ParametersXDD-Diff                  Phy-ParametersXDD-Diff                                              OPTIONAL,

mac-ParametersXDD-Diff                  MAC-ParametersXDD-Diff                                              OPTIONAL,

measAndMobParametersXDD-Diff            MeasAndMobParametersXDD-Diff                                        OPTIONAL

}

UE-NR-CapabilityAddXDD-Mode-v1530 ::=    SEQUENCE {

eutra-ParametersXDD-Diff                 EUTRA-ParametersXDD-Diff

}

UE-NR-CapabilityAddFRX-Mode ::= SEQUENCE {

phy-ParametersFRX-Diff              Phy-ParametersFRX-Diff                                                  OPTIONAL,

measAndMobParametersFRX-Diff        MeasAndMobParametersFRX-Diff                                            OPTIONAL

}

UE-NR-CapabilityAddFRX-Mode-v1540 ::=    SEQUENCE {

ims-ParametersFRX-Diff                   IMS-ParametersFRX-Diff                                             OPTIONAL

}

-- TAG-UE-NR-CAPABILITY-STOP

-- ASN1STOP

UE-NR-Capability field descriptions

featureSetCombinations A list of FeatureSetCombination:s for supportedBandCombinationList in UE-NR-Capability. The FeatureSetDownlink:s and FeatureSetUplink:s referred to from these FeatureSetCombination:s are defined in the featureSets list in UE-NR-Capability.

UE-NR-Capability-v1540 field descriptions

fr1-fr2-Add-UE-NR-Capabilities This instance of UE-NR-CapabilityAddFRX-Mode does not include any other fields than csi-RS-IM-ReceptionForFeedback/ csi-RS-ProcFrameworkForSRS/ csi-ReportFramework.

### 6.3.4	Other information elements

#### –	EUTRA-AllowedMeasBandwidth

The IE EUTRA-AllowedMeasBandwidth is used to indicate the maximum allowed measurement bandwidth on a carrier frequency as defined by the parameter Transmission Bandwidth Configuration "NRB" in TS 36.104 [33]. The values mbw6, mbw15, mbw25, mbw50, mbw75, mbw100 indicate 6, 15, 25, 50, 75 and 100 resource blocks, respectively.

EUTRA-AllowedMeasBandwidth information element

-- ASN1START

-- TAG-EUTRA-ALLOWEDMEASBANDWIDTH-START

EUTRA-AllowedMeasBandwidth ::=              ENUMERATED {mbw6, mbw15, mbw25, mbw50, mbw75, mbw100}

-- TAG-EUTRA-ALLOWEDMEASBANDWIDTH-STOP

-- ASN1STOP

#### –	EUTRA-MBSFN-SubframeConfigList

The IE EUTRA-MBSFN-SubframeConfigList is used to define an E-UTRA MBSFN subframe pattern (for the purpose of NR rate matching).

EUTRA-MBSFN-SubframeConfigList information element

-- ASN1START

-- TAG-EUTRA-MBSFN-SUBFRAMECONFIGLIST-START

EUTRA-MBSFN-SubframeConfigList ::= SEQUENCE (SIZE (1..maxMBSFN-Allocations)) OF EUTRA-MBSFN-SubframeConfig

EUTRA-MBSFN-SubframeConfig ::=      SEQUENCE {

radioframeAllocationPeriod          ENUMERATED {n1, n2, n4, n8, n16, n32},

radioframeAllocationOffset          INTEGER (0..7),

subframeAllocation1                 CHOICE {

oneFrame                            BIT STRING (SIZE(6)),

fourFrames                          BIT STRING (SIZE(24))

},

subframeAllocation2                 CHOICE {

oneFrame                            BIT STRING (SIZE(2)),

fourFrames                          BIT STRING (SIZE(8))

}                                                                                                       OPTIONAL,   -- Need R

...

}

-- TAG-EUTRA-MBSFN-SUBFRAMECONFIGLIST-STOP

-- ASN1STOP

EUTRA-MBSFN-SubframeConfig field descriptions

radioframeAllocationOffset Field as defined in MBSFN-SubframeConfig in TS 36.331 [10].

radioframeAllocationPeriod Field as defined in MBSFN-SubframeConfig in TS 36.331 [10], where SFN refers to the SFN of the NR serving cell.

subframeAllocation1 Field as defined in MBSFN-SubframeConfig in TS 36.331 [10], where the UE assumes the duplex mode (FDD or TDD) of the NR cell for which the E-UTRA-MBSFN-SubframeConfig is provided.

subframeAllocation2 Field as defined in MBSFN-SubframeConfig-v1430 in TS 36.331 [10], where the UE assumes the duplex mode (FDD or TDD) of the NR cell for which the E-UTRA-MBSFN-SubframeConfig is provided.

#### –	EUTRA-MultiBandInfoList

The IE EUTRA-MultiBandInfoList indicates the list of frequency bands in addition to the band represented by CarrierFreq for which cell reselection parameters are common, and a list of additionalPmax and additionalSpectrumEmission.

EUTRA-MultiBandInfoList information element

-- ASN1START

-- TAG-EUTRA-MULTIBANDINFOLIST-START

EUTRA-MultiBandInfoList ::=     SEQUENCE (SIZE (1..maxMultiBands)) OF EUTRA-MultiBandInfo

EUTRA-MultiBandInfo ::=         SEQUENCE {

eutra-FreqBandIndicator         FreqBandIndicatorEUTRA,

eutra-NS-PmaxList               EUTRA-NS-PmaxList                           OPTIONAL    -- Need R

}

-- TAG-EUTRA-MULTIBANDINFOLIST-STOP

-- ASN1STOP

#### –	EUTRA-NS-PmaxList

The IE EUTRA-NS-PmaxList concerns a list of additionalPmax and additionalSpectrumEmission, as defined in TS 36.101 [22], table 6.2.4-1 for UEs neither in CE nor BL UEs and TS 36.101 [22], table 6.2.4E-1 for UEs in CE or BL UEs, for a given frequency band.

EUTRA-NS-PmaxList information element

-- ASN1START

-- TAG-EUTRA-NS-PMAXLIST-START

EUTRA-NS-PmaxList ::=               SEQUENCE (SIZE (1..maxEUTRA-NS-Pmax)) OF EUTRA-NS-PmaxValue

EUTRA-NS-PmaxValue ::=              SEQUENCE {

additionalPmax                      INTEGER (-30..33)                       OPTIONAL,   -- Need R

additionalSpectrumEmission          INTEGER (1..288)                        OPTIONAL    -- Need R

}

-- TAG-EUTRA-NS-PMAXLIST-STOP

-- ASN1STOP

#### –	EUTRA-PhysCellId

The IE EUTRA-PhysCellId is used to indicate the physical layer identity of the cell, as defined in TS 36.211 [31].

EUTRA-PhysCellId information element

-- ASN1START

-- TAG-EUTRA-PHYSCELLID-START

EUTRA-PhysCellId ::=                        INTEGER (0..503)

-- TAG-EUTRA-PHYSCELLID-STOP

-- ASN1STOP

#### –	EUTRA-PhysCellIdRange

The IE EUTRA-PhysCellIdRange is used to encode either a single or a range of physical cell identities. The range is encoded by using a start value and by indicating the number of consecutive physical cell identities (including start) in the range. For fields comprising multiple occurrences of EUTRA-PhysCellIdRange, NW may configure overlapping ranges of physical cell identities.

EUTRA-PhysCellIdRange information element

-- ASN1START

-- TAG-EUTRA-PHYSCELLIDRANGE-START

EUTRA-PhysCellIdRange ::=       SEQUENCE {

start                           EUTRA-PhysCellId,

range                           ENUMERATED {n4, n8, n12, n16, n24, n32, n48, n64, n84, n96,

n128, n168, n252, n504, spare2, spare1}                         OPTIONAL    -- Need N

}

-- TAG-EUTRA-PHYSCELLIDRANGE-STOP

-- ASN1STOP

#### –	EUTRA-PresenceAntennaPort1

The IE EUTRA-PresenceAntennaPort1 is used to indicate whether all the neighbouring cells use Antenna Port 1. When set to true, the UE may assume that at least two cell-specific antenna ports are used in all neighbouring cells.

EUTRA-PresenceAntennaPort1 information element

-- ASN1START

-- TAG-EUTRA-PRESENCEANTENNAPORT1-START

EUTRA-PresenceAntennaPort1 ::=              BOOLEAN

-- TAG-EUTRA-PRESENCEANTENNAPORT1-STOP

-- ASN1STOP

#### –	EUTRA-Q-OffsetRange

The IE EUTRA-Q-OffsetRange is used to indicate a cell, or frequency specific offset to be applied when evaluating triggering conditions for measurement reporting. The value in dB. Value dB-24 corresponds to -24 dB, value dB-22 corresponds to -22 dB and so on.

EUTRA-Q-OffsetRange information element

-- ASN1START

-- TAG-EUTRA-Q-OFFSETRANGE-START

EUTRA-Q-OffsetRange ::=                     ENUMERATED {

dB-24, dB-22, dB-20, dB-18, dB-16, dB-14,

dB-12, dB-10, dB-8, dB-6, dB-5, dB-4, dB-3,

dB-2, dB-1, dB0, dB1, dB2, dB3, dB4, dB5,

dB6, dB8, dB10, dB12, dB14, dB16, dB18,

dB20, dB22, dB24}

-- TAG-EUTRA-Q-OFFSETRANGE-STOP

-- ASN1STOP

#### –	OtherConfig

The IE OtherConfig contains configuration related to miscellaneous other configurations.

OtherConfig information element

-- ASN1START

-- TAG-OTHERCONFIG-START

OtherConfig ::=                 SEQUENCE {

delayBudgetReportingConfig  CHOICE{

release                 NULL,

setup                   SEQUENCE{

delayBudgetReportingProhibitTimer   ENUMERATED {s0, s0dot4, s0dot8, s1dot6, s3, s6, s12, s30}

}

}                                                                                                     OPTIONAL        -- Need M

}

OtherConfig-v1540 ::=           SEQUENCE {

overheatingAssistanceConfig     SetupRelease {OverheatingAssistanceConfig}                            OPTIONAL, -- Need M

...

}

OverheatingAssistanceConfig ::= SEQUENCE {

overheatingIndicationProhibitTimer    ENUMERATED {s0, s0dot5, s1, s2, s5, s10, s20, s30,

s60, s90, s120, s300, s600, spare3, spare2, spare1}

}

-- TAG-OTHERCONFIG-STOP

-- ASN1STOP

OtherConfig field descriptions

delayBudgetReportingProhibitTimer Prohibit timer for delay budget reporting. Value in seconds. Value s0 means prohibit timer is set to 0 seconds, value s0dot4 means prohibit timer is set to 0.4 seconds, and so on.

overheatingAssistanceConfig Configuration for the UE to report assistance information to inform the gNB about UE detected internal overheating.

overheatingIndicationProhibitTimer Prohibit timer for overheating assistance information reporting. Value in seconds. Value s0 means prohibit timer is set to 0 seconds, value s0dot5 means prohibit timer is set to 0.5 seconds, value s1 means prohibit timer is set to 1 second and so on.

#### –	RRC-TransactionIdentifier

The IE RRC-TransactionIdentifier is used, together with the message type, for the identification of an RRC procedure (transaction).

RRC-TransactionIdentifier information element

-- ASN1START

-- TAG-RRC-TRANSACTIONIDENTIFIER-START

RRC-TransactionIdentifier ::=       INTEGER (0..3)

-- TAG-RRC-TRANSACTIONIDENTIFIER-STOP

-- ASN1STOP

## 6.4	RRC multiplicity and type constraint values

### –	Multiplicity and type constraint definitions

-- ASN1START

-- TAG-MULTIPLICITY-AND-TYPE-CONSTRAINT-DEFINITIONS-START

maxBandComb                             INTEGER ::= 65536   -- Maximum number of DL band combinations

maxCellBlack                            INTEGER ::= 16      -- Maximum number of NR blacklisted cell ranges in SIB3, SIB4

maxCellInter                            INTEGER ::= 16      -- Maximum number of inter-Freq cells listed in SIB4

maxCellIntra                            INTEGER ::= 16      -- Maximum number of intra-Freq cells listed in SIB3

maxCellMeasEUTRA                        INTEGER ::= 32      -- Maximum number of cells in E-UTRAN

maxEARFCN                               INTEGER ::= 262143  -- Maximum value of E-UTRA carrier frequency

maxEUTRA-CellBlack                      INTEGER ::= 16      -- Maximum number of E-UTRA blacklisted physical cell identity ranges

-- in SIB5

maxEUTRA-NS-Pmax                        INTEGER ::= 8       -- Maximum number of NS and P-Max values per band

maxMultiBands                           INTEGER ::= 8       -- Maximum number of additional frequency bands that a cell belongs to

maxNARFCN                               INTEGER ::= 3279165 -- Maximum value of NR carrier frequency

maxNR-NS-Pmax                           INTEGER ::= 8       -- Maximum number of NS and P-Max values per band

maxNrofServingCells                     INTEGER ::= 32      -- Max number of serving cells (SpCells + SCells)

maxNrofServingCells-1                   INTEGER ::= 31      -- Max number of serving cells (SpCell + SCells) minus 1

maxNrofAggregatedCellsPerCellGroup      INTEGER ::= 16

maxNrofSCells                           INTEGER ::= 31      -- Max number of secondary serving cells per cell group

maxNrofCellMeas                         INTEGER ::= 32      -- Maximum number of entries in each of the cell lists in a measurement

-- object

maxNrofSS-BlocksToAverage               INTEGER ::= 16      -- Max number for the (max) number of SS blocks to average to determine cell

-- measurement

maxNrofCSI-RS-ResourcesToAverage        INTEGER ::= 16      -- Max number for the (max) number of CSI-RS to average to determine cell

-- measurement

maxNrofDL-Allocations                   INTEGER ::= 16      -- Maximum number of PDSCH time domain resource allocations

maxNrofSR-ConfigPerCellGroup            INTEGER ::= 8       -- Maximum number of SR configurations per cell group

maxLCG-ID                               INTEGER ::= 7       -- Maximum value of LCG ID

maxLC-ID                                INTEGER ::= 32      -- Maximum value of Logical Channel ID

maxNrofTAGs                             INTEGER ::= 4       -- Maximum number of Timing Advance Groups

maxNrofTAGs-1                           INTEGER ::= 3       -- Maximum number of Timing Advance Groups minus 1

maxNrofBWPs                             INTEGER ::= 4       -- Maximum number of BWPs per serving cell

maxNrofCombIDC                          INTEGER ::= 128     -- Maximum number of reported MR-DC combinations for IDC

maxNrofSymbols-1                        INTEGER ::= 13      -- Maximum index identifying a symbol within a slot (14 symbols, indexed

-- from 0..13)

maxNrofSlots                            INTEGER ::= 320     -- Maximum number of slots in a 10 ms period

maxNrofSlots-1                          INTEGER ::= 319     -- Maximum number of slots in a 10 ms period minus 1

maxNrofPhysicalResourceBlocks           INTEGER ::= 275     -- Maximum number of PRBs

maxNrofPhysicalResourceBlocks-1         INTEGER ::= 274     -- Maximum number of PRBs minus 1

maxNrofPhysicalResourceBlocksPlus1      INTEGER ::= 276     -- Maximum number of PRBs plus 1

maxNrofControlResourceSets-1            INTEGER ::= 11      -- Max number of CoReSets configurable on a serving cell minus 1

maxCoReSetDuration                      INTEGER ::= 3       -- Max number of OFDM symbols in a control resource set

maxNrofSearchSpaces-1                   INTEGER ::= 39      -- Max number of Search Spaces minus 1

maxSFI-DCI-PayloadSize                  INTEGER ::= 128     -- Max number payload of a DCI scrambled with SFI-RNTI

maxSFI-DCI-PayloadSize-1                INTEGER ::= 127     -- Max number payload of a DCI scrambled with SFI-RNTI minus 1

maxINT-DCI-PayloadSize                  INTEGER ::= 126     -- Max number payload of a DCI scrambled with INT-RNTI

maxINT-DCI-PayloadSize-1                INTEGER ::= 125     -- Max number payload of a DCI scrambled with INT-RNTI minus 1

maxNrofRateMatchPatterns                INTEGER ::= 4       -- Max number of rate matching patterns that may be configured

maxNrofRateMatchPatterns-1              INTEGER ::= 3       -- Max number of rate matching patterns that may be configured minus 1

maxNrofRateMatchPatternsPerGroup        INTEGER ::= 8       -- Max number of rate matching patterns that may be configured in one group

maxNrofCSI-ReportConfigurations         INTEGER ::= 48      -- Maximum number of report configurations

maxNrofCSI-ReportConfigurations-1       INTEGER ::= 47      -- Maximum number of report configurations minus 1

maxNrofCSI-ResourceConfigurations       INTEGER ::= 112     -- Maximum number of resource configurations

maxNrofCSI-ResourceConfigurations-1     INTEGER ::= 111     -- Maximum number of resource configurations minus 1

maxNrofAP-CSI-RS-ResourcesPerSet        INTEGER ::= 16

maxNrOfCSI-AperiodicTriggers            INTEGER ::= 128     -- Maximum number of triggers for aperiodic CSI reporting

maxNrofReportConfigPerAperiodicTrigger  INTEGER ::= 16      -- Maximum number of report configurations per trigger state for aperiodic

-- reporting

maxNrofNZP-CSI-RS-Resources             INTEGER ::= 192     -- Maximum number of Non-Zero-Power (NZP) CSI-RS resources

maxNrofNZP-CSI-RS-Resources-1           INTEGER ::= 191     -- Maximum number of Non-Zero-Power (NZP) CSI-RS resources minus 1

maxNrofNZP-CSI-RS-ResourcesPerSet       INTEGER ::= 64      -- Maximum number of NZP CSI-RS resources per resource set

maxNrofNZP-CSI-RS-ResourceSets          INTEGER ::= 64      -- Maximum number of NZP CSI-RS resources per cell

maxNrofNZP-CSI-RS-ResourceSets-1        INTEGER ::= 63      -- Maximum number of NZP CSI-RS resources per cell minus 1

maxNrofNZP-CSI-RS-ResourceSetsPerConfig INTEGER ::= 16      -- Maximum number of resource sets per resource configuration

maxNrofNZP-CSI-RS-ResourcesPerConfig    INTEGER ::= 128     -- Maximum number of resources per resource configuration

maxNrofZP-CSI-RS-Resources              INTEGER ::= 32      -- Maximum number of Zero-Power (ZP) CSI-RS resources

maxNrofZP-CSI-RS-Resources-1            INTEGER ::= 31      -- Maximum number of Zero-Power (ZP) CSI-RS resources minus 1

maxNrofZP-CSI-RS-ResourceSets-1         INTEGER ::= 15

maxNrofZP-CSI-RS-ResourcesPerSet        INTEGER ::= 16

maxNrofZP-CSI-RS-ResourceSets           INTEGER ::= 16

maxNrofCSI-IM-Resources                 INTEGER ::= 32      -- Maximum number of CSI-IM resources. See CSI-IM-ResourceMax in 38.214.

maxNrofCSI-IM-Resources-1               INTEGER ::= 31      -- Maximum number of CSI-IM resources minus 1. See CSI-IM-ResourceMax

-- in 38.214.

maxNrofCSI-IM-ResourcesPerSet           INTEGER ::= 8       -- Maximum number of CSI-IM resources per set. See CSI-IM-ResourcePerSetMax

-- in 38.214

maxNrofCSI-IM-ResourceSets              INTEGER ::= 64      -- Maximum number of NZP CSI-IM resources per cell

maxNrofCSI-IM-ResourceSets-1            INTEGER ::= 63      -- Maximum number of NZP CSI-IM resources per cell minus 1

maxNrofCSI-IM-ResourceSetsPerConfig     INTEGER ::= 16      -- Maximum number of CSI IM resource sets per resource configuration

maxNrofCSI-SSB-ResourcePerSet           INTEGER ::= 64      -- Maximum number of SSB resources in a resource set

maxNrofCSI-SSB-ResourceSets             INTEGER ::= 64      -- Maximum number of CSI SSB resource sets per cell

maxNrofCSI-SSB-ResourceSets-1           INTEGER ::= 63      -- Maximum number of CSI SSB resource sets per cell minus 1

maxNrofCSI-SSB-ResourceSetsPerConfig    INTEGER ::= 1       -- Maximum number of CSI SSB resource sets per resource configuration

maxNrofFailureDetectionResources        INTEGER ::= 10      -- Maximum number of failure detection resources

maxNrofFailureDetectionResources-1      INTEGER ::= 9       -- Maximum number of failure detection resources minus 1

maxNrofObjectId                         INTEGER ::= 64      -- Maximum number of measurement objects

maxNrofPageRec                          INTEGER ::= 32      -- Maximum number of page records

maxNrofPCI-Ranges                       INTEGER ::= 8       -- Maximum number of PCI ranges

maxPLMN                                 INTEGER ::= 12      -- Maximum number of PLMNs broadcast and reported by UE at establisghment

maxNrofCSI-RS-ResourcesRRM              INTEGER ::= 96      -- Maximum number of CSI-RS resources for an RRM measurement object

maxNrofCSI-RS-ResourcesRRM-1            INTEGER ::= 95      -- Maximum number of CSI-RS resources for an RRM measurement object minus 1

maxNrofMeasId                           INTEGER ::= 64      -- Maximum number of configured measurements

maxNrofQuantityConfig                   INTEGER ::= 2       -- Maximum number of quantity configurations

maxNrofCSI-RS-CellsRRM                  INTEGER ::= 96      -- Maximum number of cells with CSI-RS resources for an RRM measurement

-- object

maxNrofSRS-ResourceSets                 INTEGER ::= 16      -- Maximum number of SRS resource sets in a BWP.

maxNrofSRS-ResourceSets-1               INTEGER ::= 15      -- Maximum number of SRS resource sets in a BWP minus 1.

maxNrofSRS-Resources                    INTEGER ::= 64      -- Maximum number of SRS resources.

maxNrofSRS-Resources-1                  INTEGER ::= 63      -- Maximum number of SRS resources in an SRS resource set minus 1.

maxNrofSRS-ResourcesPerSet              INTEGER ::= 16      -- Maximum number of SRS resources in an SRS resource set

maxNrofSRS-TriggerStates-1              INTEGER ::= 3       -- Maximum number of SRS trigger states minus 1, i.e., the largest code

-- point.

maxNrofSRS-TriggerStates-2              INTEGER ::= 2       -- Maximum number of SRS trigger states minus 2.

maxRAT-CapabilityContainers             INTEGER ::= 8       -- Maximum number of interworking RAT containers (incl NR and MRDC)

maxSimultaneousBands                    INTEGER ::= 32      -- Maximum number of simultaneously aggregated bands

maxNrofSlotFormatCombinationsPerSet     INTEGER ::= 512     -- Maximum number of Slot Format Combinations in a SF-Set.

maxNrofSlotFormatCombinationsPerSet-1   INTEGER ::= 511     -- Maximum number of Slot Format Combinations in a SF-Set minus 1.

maxNrofPUCCH-Resources                  INTEGER ::= 128

maxNrofPUCCH-Resources-1                INTEGER ::= 127

maxNrofPUCCH-ResourceSets               INTEGER ::= 4       -- Maximum number of PUCCH Resource Sets

maxNrofPUCCH-ResourceSets-1             INTEGER ::= 3       -- Maximum number of PUCCH Resource Sets minus 1.

maxNrofPUCCH-ResourcesPerSet            INTEGER ::= 32      -- Maximum number of PUCCH Resources per PUCCH-ResourceSet

maxNrofPUCCH-P0-PerSet                  INTEGER ::= 8       -- Maximum number of P0-pucch present in a p0-pucch set

maxNrofPUCCH-PathlossReferenceRSs       INTEGER ::= 4       -- Maximum number of RSs used as pathloss reference for PUCCH power control.

maxNrofPUCCH-PathlossReferenceRSs-1     INTEGER ::= 3       -- Maximum number of RSs used as pathloss reference for PUCCH power

-- control minus 1.

maxNrofP0-PUSCH-AlphaSets               INTEGER ::= 30      -- Maximum number of P0-pusch-alpha-sets (see 38,213, clause 7.1)

maxNrofP0-PUSCH-AlphaSets-1             INTEGER ::= 29      -- Maximum number of P0-pusch-alpha-sets minus 1 (see 38,213, clause 7.1)

maxNrofPUSCH-PathlossReferenceRSs       INTEGER ::= 4       -- Maximum number of RSs used as pathloss reference for PUSCH power control.

maxNrofPUSCH-PathlossReferenceRSs-1     INTEGER ::= 3       -- Maximum number of RSs used as pathloss reference for PUSCH power

-- control minus 1.

maxNrofNAICS-Entries                    INTEGER ::= 8       -- Maximum number of supported NAICS capability set

maxBands                                INTEGER ::= 1024    -- Maximum number of supported bands in UE capability.

maxBandsMRDC                            INTEGER ::= 1280

maxBandsEUTRA                           INTEGER ::= 256

maxCellReport                           INTEGER ::= 8

maxDRB                                  INTEGER ::= 29      -- Maximum number of DRBs (that can be added in DRB-ToAddModLIst).

maxFreq                                 INTEGER ::= 8       -- Max number of frequencies.

maxFreqIDC-MRDC                         INTEGER ::= 32      -- Maximum number of candidate NR frequencies for MR-DC IDC indication

maxNrofCandidateBeams                   INTEGER ::= 16      -- Max number of PRACH-ResourceDedicatedBFR that in BFR config.

maxNrofPCIsPerSMTC                      INTEGER ::= 64      -- Maximun number of PCIs per SMTC.

maxNrofQFIs                             INTEGER ::= 64

maxNrOfSemiPersistentPUSCH-Triggers     INTEGER ::= 64      -- Maximum number of triggers for semi persistent reporting on PUSCH

maxNrofSR-Resources                     INTEGER ::= 8       -- Maximum number of SR resources per BWP in a cell.

maxNrofSlotFormatsPerCombination        INTEGER ::= 256

maxNrofSpatialRelationInfos             INTEGER ::= 8

maxNrofIndexesToReport                  INTEGER ::= 32

maxNrofIndexesToReport2                 INTEGER ::= 64

maxNrofSSBs-1                           INTEGER ::= 63      -- Maximum number of SSB resources in a resource set minus 1.

maxNrofS-NSSAI                          INTEGER ::= 8       -- Maximum number of S-NSSAI.

maxNrofTCI-StatesPDCCH                  INTEGER ::= 64

maxNrofTCI-States                       INTEGER ::= 128     -- Maximum number of TCI states.

maxNrofTCI-States-1                     INTEGER ::= 127     -- Maximum number of TCI states minus 1.

maxNrofUL-Allocations                   INTEGER ::= 16      -- Maximum number of PUSCH time domain resource allocations.

maxQFI                                  INTEGER ::= 63

maxRA-CSIRS-Resources                   INTEGER ::= 96

maxRA-OccasionsPerCSIRS                 INTEGER ::= 64      -- Maximum number of RA occasions for one CSI-RS

maxRA-Occasions-1                       INTEGER ::= 511     -- Maximum number of RA occasions in the system

maxRA-SSB-Resources                     INTEGER ::= 64

maxSCSs                                 INTEGER ::= 5

maxSecondaryCellGroups                  INTEGER ::= 3

maxNrofServingCellsEUTRA                INTEGER ::= 32

maxMBSFN-Allocations                    INTEGER ::= 8

maxNrofMultiBands                       INTEGER ::= 8

maxCellSFTD                             INTEGER ::= 3       -- Maximum number of cells for SFTD reporting

maxReportConfigId                       INTEGER ::= 64

maxNrofCodebooks                        INTEGER ::= 16      -- Maximum number of codebooks suppoted by the UE

maxNrofCSI-RS-Resources                 INTEGER ::= 7       -- Maximum number of codebook resources supported by the UE

maxNrofSRI-PUSCH-Mappings               INTEGER ::= 16

maxNrofSRI-PUSCH-Mappings-1             INTEGER ::= 15

maxSIB                                  INTEGER::= 32       -- Maximum number of SIBs

maxSI-Message                           INTEGER::= 32       -- Maximum number of SI messages

maxPO-perPF                             INTEGER ::= 4       -- Maximum number of paging occasion per paging frame

maxAccessCat-1                          INTEGER ::= 63      -- Maximum number of Access Categories minus 1

maxBarringInfoSet                       INTEGER ::= 8       -- Maximum number of Access Categories

maxCellEUTRA                            INTEGER ::= 8       -- Maximum number of E-UTRA cells in SIB list

maxEUTRA-Carrier                        INTEGER ::= 8       -- Maximum number of E-UTRA carriers in SIB list

maxPLMNIdentities                       INTEGER ::= 8       -- Maximum number of PLMN identites in RAN area configurations

maxDownlinkFeatureSets                  INTEGER ::= 1024    -- (for NR DL) Total number of FeatureSets (size of the pool)

maxUplinkFeatureSets                    INTEGER ::= 1024    -- (for NR UL) Total number of FeatureSets (size of the pool)

maxEUTRA-DL-FeatureSets                 INTEGER ::= 256     -- (for E-UTRA) Total number of FeatureSets (size of the pool)

maxEUTRA-UL-FeatureSets                 INTEGER ::= 256     -- (for E-UTRA) Total number of FeatureSets (size of the pool)

maxFeatureSetsPerBand                   INTEGER ::= 128     -- (for NR) The number of feature sets associated with one band.

maxPerCC-FeatureSets                    INTEGER ::= 1024    -- (for NR) Total number of CC-specific FeatureSets (size of the pool)

maxFeatureSetCombinations               INTEGER ::= 1024    -- (for MR-DC/NR)Total number of Feature set combinations (size of the

-- pool)

maxInterRAT-RSTD-Freq                   INTEGER ::= 3

-- TAG-MULTIPLICITY-AND-TYPE-CONSTRAINT-DEFINITIONS-STOP

-- ASN1STOP

### –	End of NR-RRC-Definitions

-- ASN1START

END

-- ASN1STOP

## 6.5	Short Message

Short Messages can be transmitted on PDCCH using P-RNTI with or without associated Paging message using Short Message field in DCI format 1_0 (see TS 38.212 [17], clause 7.3.1.2.1).

Table 6.5-1 defines Short Messages. Bit 1 is the most significant bit.

Table 6.5-1: Short Messages

| Bit | Short Message |
| --- | --- |
| 1 | systemInfoModification If set to 1: indication of a BCCH modification other than SIB6, SIB7 and SIB8. |
| 2 | etwsAndCmasIndication If set to 1: indication of an ETWS primary notification and/or an ETWS secondary notification and/or a CMAS notification. |
| 3 – 8 | Not used in this release of the specification, and shall be ignored by UE if received. |

# 7	Variables and constants

## 7.1	Timers

### 7.1.1	Timers (Informative)

| Timer | Start | Stop | At expiry |
| --- | --- | --- | --- |
| T300 | Upon transmission of RRCSetupRequest. | Upon reception of RRCSetup or RRCReject message, cell re-selection and upon abortion of connection establishment by upper layers. | Perform the actions as specified in 5.3.3.7. |
| T301 | Upon transmission of RRCReestabilshmentRequest | Upon reception of RRCReestablishment or RRCSetup message as well as when the selected cell becomes unsuitable | Go to RRC_IDLE |
| T302 | Upon reception of RRCReject while performing RRC connection establishment or resume, upon reception of RRCRelease with waitTime. | Upon entering RRC_CONNECTED or RRC_IDLE, upon cell re-selection and upon reception of RRCReject message. | Inform upper layers about barring alleviation as specified in 5.3.14.4 |
| T304 | Upon reception of RRCReconfiguration message including reconfigurationWithSync | Upon successful completion of random access on the corresponding SpCell For T304 of SCG, upon SCG release | For T304 of MCG, in case of the handover from NR or intra-NR handover, initiate the RRC re-establishment procedure; In case of handover to NR, perform the actions defined in the specifications applicable for the source RAT. For T304 of SCG, inform network about the reconfiguration with sync failure by initiating the SCG failure information procedure as specified in 5.7.3. |
| T310 | Upon detecting physical layer problems for the SpCell i.e. upon receiving N310 consecutive out-of-sync indications from lower layers. | Upon receiving N311 consecutive in-sync indications from lower layers for the SpCell, upon receiving RRCReconfiguration with reconfigurationWithSync for that cell group, upon reception of MobilityFromNRCommand, and upon initiating the connection re-establishment procedure. Upon SCG release, if the T310 is kept in SCG. | If the T310 is kept in MCG: If AS security is not activated: go to RRC_IDLE else: initiate the connection re-establishment procedure. If the T310 is kept in SCG, Inform E-UTRAN/NR about the SCG radio link failure by initiating the SCG failure information procedure as specified in 5.7.3. |
| T311 | Upon initiating the RRC connection re-establishment procedure | Upon selection of a suitable NR cell or a cell using another RAT. | Enter RRC_IDLE |
| T319 | Upon transmission of RRCResumeRequest or RRCResumeRequest1. | Upon reception of RRCResume, RRCSetup, RRCRelease, RRCRelease with suspendConfig or RRCReject message, cell re-selection and upon abortion of connection establishment by upper layers. | Perform the actions as specified in 5.3.13.5. |
| T320 | Upon reception of t320 or upon cell (re)selection to NR from another RAT with validity time configured for dedicated priorities (in which case the remaining validity time is applied). | Upon entering RRC_CONNECTED, upon reception of RRCRelease, when PLMN selection is performed on request by NAS, when the UE enters RRC_IDLE from RRC_INACTIVE, or upon cell (re)selection to another RAT (in which case the timer is carried on to the other RAT). | Discard the cell reselection priority information provided by dedicated signalling. |
| T321 | Upon receiving measConfig including a reportConfig with the purpose set to reportCGI | Upon acquiring the information needed to set all fields of cgi-info, upon receiving measConfig that includes removal of the reportConfig with the purpose set to reportCGI and upon detecting that a cell is not broadcasting SIB1. | Initiate the measurement reporting procedure, stop performing the related measurements. |
| T322 | Upon receving measConfig including reportConfigNR with the purpose set to reportSFTD and drx-SFTD-NeighMeas is set to true. | Upon acquiring the SFTD measurement results, upon receiving measConfig that includes removal of the reportConfig with the purpose set to reportSFTD. | Initiate the measurement reporting procedure, stop performing the related measurements. |
| T325 | Upon reception of RRCRelease message with deprioritisationTimer. |  | Stop deprioritisation of all frequencies or NR signalled by RRCRelease. |
| T342 | Upon transmitting UEAssistanceInformation message with DelayBudgetReport. | Upon initiating the connection re-establishment/resume procedures, and upon receiving delayBudgetReportingConfig set to release. | No action. |
| T345 | Upon transmitting UEAssistanceInformation message with overheatingAssistance | Upon initiating the connection re-establishment procedure and upon initiating the connection resumption procedure | No action. |
| T380 | Upon reception of t380 in RRCRelease. | Upon reception of RRCResume, RRCSetup or RRCRelease. | Perform the actions as specified in 5.3.13. |
| T390 | When access attempt is barred at access barring check for an Access Category. The UE maintains one instance of this timer per Access Category. | Upon cell (re)selection, upon entering RRC_CONNECTED, upon reception of RRCReconfiguration including reconfigurationWithSync, upon change of PCell while in RRC_CONNECTED, upon reception of MobilityFromNRCommand, or upon reception of RRCRelease. | Perform the actions as specified in 5.3.14.4. |

### 7.1.2	Timer handling

When the UE applies zero value for a timer, the timer shall be started and immediately expire unless explicitly stated otherwise.

## 7.2	Counters

| Counter | Reset | Incremented | When reaching max value |
| --- | --- | --- | --- |
| N310 | Upon reception of "in-sync" indication from lower layers; upon receiving RRCReconfiguration with reconfigurationWithSync for that cell group; upon initiating the connection re-establishment procedure. | Upon reception of "out-of-sync" from lower layer while the timer T310 is stopped. | Start timer T310 |
| N311 | Upon reception of "out-of-sync" indication from lower layers; upon receiving RRCReconfiguration with reconfigurationWithSync for that cell group; upon initiating the connection re-establishment procedure. | Upon reception of the "in-sync" from lower layer while the timer T310 is running. | Stop the timer T310. |

## 7.3	Constants

| Constant | Usage |
| --- | --- |
| N310 | Maximum number of consecutive "out-of-sync" indications for the SpCell received from lower layers |
| N311 | Maximum number of consecutive "in-sync" indications for the SpCell received from lower layers |

## 7.4	UE variables

NOTE:	To facilitate the specification of the UE behavioural requirements, UE variables are represented using ASN.1. Unless explicitly specified otherwise, it is however up to UE implementation how to store the variables. The optionality of the IEs in ASN.1 is used only to indicate that the values may not always be available.

#### –	NR-UE-Variables

This ASN.1 segment is the start of the NR UE variable definitions.

-- ASN1START

-- NR-UE-VARIABLES-START

NR-UE-Variables DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

IMPORTS

CellIdentity,

EUTRA-PhysCellId,

MeasId,

MeasIdToAddModList,

MeasObjectToAddModList,

PhysCellId,

RNTI-Value,

ReportConfigToAddModList,

RSRP-Range,

QuantityConfig,

maxNrofCellMeas,

maxNrofMeasId

FROM NR-RRC-Definitions;

-- NR-UE-VARIABLES-STOP

-- ASN1STOP

#### –	VarPendingRNA-Update

The UE variable VarPendingRNA-Update indicates whether there is a pending RNA update procedure or not. The setting of this BOOLEAN variable to true means that there is a pending RNA Update procedure.

VarPendingRNA-Update UE variable

-- ASN1START

-- TAG-VARPENDINGRNA-UPDATE-START

VarPendingRNA-Update ::=                    SEQUENCE {

pendingRNA-Update                           BOOLEAN                             OPTIONAL

}

-- TAG-VARPENDINGRNA-UPDATE-STOP

-- ASN1STOP

#### –	VarMeasConfig

The UE variable VarMeasConfig includes the accumulated configuration of the measurements to be performed by the UE, covering intra-frequency, inter-frequency and inter-RAT mobility related measurements.

VarMeasConfig UE variable

-- ASN1START

-- TAG-VARMEASCONFIG-START

VarMeasConfig ::=                   SEQUENCE {

-- Measurement identities

measIdList                          MeasIdToAddModList                  OPTIONAL,

-- Measurement objects

measObjectList                      MeasObjectToAddModList              OPTIONAL,

-- Reporting configurations

reportConfigList                    ReportConfigToAddModList            OPTIONAL,

-- Other parameters

quantityConfig                      QuantityConfig                      OPTIONAL,

s-MeasureConfig                         CHOICE {

ssb-RSRP                                RSRP-Range,

csi-RSRP                                RSRP-Range

}                                                                       OPTIONAL

}

-- TAG-VARMEASCONFIG-STOP

-- ASN1STOP

#### –	VarMeasReportList

The UE variable VarMeasReportList includes information about the measurements for which the triggering conditions have been met.

VarMeasReportList UE variable

-- ASN1START

-- TAG-VARMEASREPORTLIST-START

VarMeasReportList ::=               SEQUENCE (SIZE (1..maxNrofMeasId)) OF VarMeasReport

VarMeasReport ::=                   SEQUENCE {

-- List of measurement that have been triggered

measId                              MeasId,

cellsTriggeredList                  CellsTriggeredList              OPTIONAL,

numberOfReportsSent                 INTEGER

}

CellsTriggeredList ::=              SEQUENCE (SIZE (1..maxNrofCellMeas)) OF CHOICE {

physCellId                          PhysCellId,

physCellIdEUTRA                     EUTRA-PhysCellId

}

-- TAG-VARMEASREPORTLIST-STOP

-- ASN1STOP

#### –	VarResumeMAC-Input

The UE variable VarResumeMAC-Input specifies the input used to generate the resumeMAC-I during RRC Connection Resume procedure.

VarResumeMAC-Input variable

-- ASN1START

-- TAG-VARRESUMEMAC-INPUT-START

VarResumeMAC-Input  ::=     SEQUENCE {

sourcePhysCellId                        PhysCellId,

targetCellIdentity                      CellIdentity,

source-c-RNTI                           RNTI-Value

}

-- TAG-VARRESUMEMAC-INPUT-STOP

-- ASN1STOP

VarResumeMAC-Input field descriptions

targetCellIdentity An input variable used to calculate the resumeMAC-I. Set to the cellIdentity of the first PLMN-Identity included in the PLMN-IdentityInfoList broadcasted in SIB1 of the target cell i.e. the cell the UE is trying to resume.

source-c-RNTI Set to C-RNTI that the UE had in the PCell it was connected to prior to suspension of the RRC connection.

sourcePhysCellId Set to the physical cell identity of the PCell the UE was connected to prior to suspension of the RRC connection.

#### –	VarShortMAC-Input

The UE variable VarShortMAC-Input specifies the input used to generate the shortMAC-I during RRC Connection Reestablishment procedure.

VarShortMAC-Input variable

-- ASN1START

-- TAG-VARSHORTMAC-INPUT-START

VarShortMAC-Input   ::=                 SEQUENCE {

sourcePhysCellId                        PhysCellId,

targetCellIdentity                      CellIdentity,

source-c-RNTI                           RNTI-Value

}

-- TAG-VARSHORTMAC-INPUT-STOP

-- ASN1STOP

VarShortMAC-Input field descriptions

targetCellIdentity An input variable used to calculate the shortMAC-I. Set to the cellIdentity of the first PLMN-Identity in the PLMN-IdentityInfoList broadcasted in SIB1 of the target cell i.e. the cell the UE is trying to reestablish the connection.

source-c-RNTI Set to C-RNTI that the UE had in the PCell it was connected to prior to the reestablishment.

sourcePhysCellId Set to the physical cell identity of the PCell the UE was connected to prior to the reestablishment.

#### –	End of NR-UE-Variables

-- ASN1START

END

-- ASN1STOP

# 8	Protocol data unit abstract syntax

## 8.1	General

The RRC PDU contents in clause 6 and clause 10 are described using abstract syntax notation one (ASN.1) as specified in ITU-T Rec. X.680 [6] and X.681 [7]. Transfer syntax for RRC PDUs is derived from their ASN.1 definitions by use of Packed Encoding Rules, unaligned as specified in ITU-T Rec. X.691 [8].

The following encoding rules apply in addition to what has been specified in X.691:

-	When a bit string value is placed in a bit-field as specified in 15.6 to 15.11 in X.691, the leading bit of the bit string value shall be placed in the leading bit of the bit-field, and the trailing bit of the bit string value shall be placed in the trailing bit of the bit-field;

NOTE:	The terms 'leading bit' and 'trailing bit' are defined in ITU-T Rec. X.680. When using the 'bstring' notation, the leading bit of the bit string value is on the left, and the trailing bit of the bit string value is on the right.

-	When decoding types constrained with the ASN.1 Contents Constraint ("CONTAINING"), automatic decoding of the contained type should not be performed because errors in the decoding of the contained type should not cause the decoding of the entire RRC message PDU to fail. It is recommended that the decoder first decodes the outer PDU type that contains the OCTET STRING or BIT STRING with the Contents Constraint, and then decodes the contained type that is nested within the OCTET STRING or BIT STRING as a separate step;

-	When decoding a) RRC message PDUs, b) BIT STRING constrained with a Contents Constraint, or c) OCTET STRING constrained with a Contents Constraint, PER decoders are required to never report an error if there are extraneous zero or non-zero bits at the end of the encoded RRC message PDU, BIT STRING or OCTET STRING.

## 8.2	Structure of encoded RRC messages

An RRC PDU, which is the bit string that is exchanged between peer entities/across the radio interface contains the basic production as defined in X.691.

RRC PDUs shall be mapped to and from PDCP SDUs (in case of DCCH) or RLC SDUs (in case of PCCH, BCCH or CCCH) upon transmission and reception as follows:

-	when delivering an RRC PDU as an PDCP SDU to the PDCP layer for transmission, the first bit of the RRC PDU shall be represented as the first bit in the PDCP SDU and onwards; and

-	when delivering an RRC PDU as an RLC SDU to the RLC layer for transmission, the first bit of the RRC PDU shall be represented as the first bit in the RLC SDU and onwards; and

-	upon reception of an PDCP SDU from the PDCP layer, the first bit of the PDCP SDU shall represent the first bit of the RRC PDU and onwards; and

-	upon reception of an RLC SDU from the RLC layer, the first bit of the RLC SDU shall represent the first bit of the RRC PDU and onwards.

## 8.3	Basic production

The 'basic production' is obtained by applying UNALIGNED PER to the abstract syntax value (the ASN.1 description) as specified in X.691. It always contains a multiple of 8 bits.

## 8.4	Extension

The following rules apply with respect to the use of protocol extensions:

-	A transmitter compliant with this version of the specification shall, unless explicitly indicated otherwise on a PDU type basis, set the extension part empty. Transmitters compliant with a later version may send non-empty extensions;

-	A transmitter compliant with this version of the specification shall set spare bits to zero.

## 8.5	Padding

If the encoded RRC message does not fill a transport block, the RRC layer shall add padding bits. This applies to PCCH and BCCH.

Padding bits shall be set to 0 and the number of padding bits is a multiple of 8.

Figure 8.5-1: RRC level padding

# 9	Specified and default radio configurations

Specified and default configurations are configurations of which the details are specified in the standard. Specified configurations are fixed while default configurations can be modified using dedicated signalling. The default value for the parameters not listed in following subclauses shall be set such as the corresponding features are not configured, i.e. release or false unless explicitly stated otherwise.

NOTE:	The UE applies the default values specified in the field description of ASN.1 parameters only when the parent IE is present. Hence, the UE does not apply all default values in field descriptions when it applies the "default radio configuration" in accordance with this clause.

## 9.1	Specified configurations

### 9.1.1	Logical channel configurations

#### 9.1.1.1	BCCH configuration

Parameters

| Name | Value | Semantics description | Ver |
| --- | --- | --- | --- |
| SDAP configuration | Not used |  |  |
| PDCP configuration | Not used |  |  |
| RLC configuration | TM |  |  |
| Logical channel configuration | Not used |  |  |

NOTE:	RRC will perform padding, if required due to the granularity of the TF signalling, as defined in 8.5.

#### 9.1.1.2	CCCH configuration

Parameters

| Name | Value | Semantics description | Ver |
| --- | --- | --- | --- |
| SDAP configuration | Not used |  |  |
| PDCP configuration | Not used |  |  |
| RLC configuration | TM |  |  |
| Logical channel configuration |  |  |  |
| >priority | 1 | Highest priority |  |
| >prioritisedBitRate | infinity |  |  |
| >bucketSizeDuration | ms1000 |  |  |
| >logicalChannelGroup | 0 |  |  |

#### 9.1.1.3	PCCH configuration

Parameters

| Name | Value | Semantics description | Ver |
| --- | --- | --- | --- |
| SDAP configuration | Not used |  |  |
| PDCP configuration | Not used |  |  |
| RLC configuration | TM |  |  |
| Logical channel configuration | Not used |  |  |

NOTE:	RRC will perform padding, if required due to the granularity of the TF signalling, as defined in 8.5.

### 9.1.2	Void

## 9.2	Default radio configurations

The following clauses only list default values for REL-15 parameters included in protocol version v15.3.0. For all fields introduced in a later protocol version, the default value is "released" or "false" unless explicitly specified otherwise. If the UE is to apply default configuration while it is configured with some critically extended fields, the UE shall apply the original version of those fields with only default values.

NOTE 1:	In general, the signalling should preferably support a "release" option for fields introduced after v15.3.0. The "value not applicable" should be used restrictively, mainly limited to for fields which value is relevant only if another field is set to a value other than its default.

NOTE 2:	For parameters in ServingCellConfig, the default values are specified in the corresponding specification.

### 9.2.1	Default SRB configurations

Parameters

| Name | Value |  |  |  | Semantics description | Ver |
| --- | --- | --- | --- | --- | --- | --- |
|  | SRB1 | SRB2 |  | SRB3 |  |  |
| PDCP-Config >t-Reordering | infinity |  |  |  |  |  |
| RLC-Config CHOICE | Am |  |  |  |  |  |
| ul-RLC-Config >sn-FieldLength >t-PollRetransmit >pollPDU >pollByte >maxRetxThreshold | size12 ms45 infinity infinity t8 |  |  |  |  |  |
| dl-RLC-Config >sn-FieldLength >t-Reassembly >t-StatusProhibit | size12 ms35 ms0 |  |  |  |  |  |
| logicalChannelIdentity | 1 | 2 | 3 |  |  |  |
| LogicalChannelConfig |  |  |  |  |  |  |
| >priority | 1 | 3 | 1 |  |  |  |
| >prioritisedBitRate | infinity |  |  |  |  |  |
| >logicalChannelGroup | 0 |  |  |  |  |  |

### 9.2.2	Default MAC Cell Group configuration

Parameters

| Name | Value | Semantics description | Ver |
| --- | --- | --- | --- |
| MAC Cell Group configuration |  |  |  |
| bsr-Config |  |  |  |
| >periodicBSR-Timer | sf10 |  |  |
| >retxBSR-Timer | sf80 |  |  |
| phr-Config |  |  |  |
| >phr-PeriodicTimer | sf10 |  |  |
| >phr-ProhibitTimer | sf10 |  |  |
| >phr-Tx-PowerFactorChange | dB1 |  |  |

### 9.2.3	Default values timers and constants

Parameters

| Name | Value | Semantics description | Ver |
| --- | --- | --- | --- |
| t310 | ms1000 |  |  |
| n310 | n1 |  |  |
| t311 | ms30000 |  |  |
| n311 | n1 |  |  |

# 10	Generic error handling

## 10.1	General

The generic error handling defined in the subsequent sub-clauses applies unless explicitly specified otherwise e.g. within the procedure specific error handling.

The UE shall consider a value as not comprehended when it is set:

-	to an extended value that is not defined in the version of the transfer syntax supported by the UE;

-	to a spare or reserved value unless the specification defines specific behaviour that the UE shall apply upon receiving the concerned spare/reserved value.

The UE shall consider a field as not comprehended when it is defined:

-	as spare or reserved unless the specification defines specific behaviour that the UE shall apply upon receiving the concerned spare/reserved field.

## 10.2	ASN.1 violation or encoding error

The UE shall:

1>	when receiving an RRC message on the BCCH, CCCH or PCCH for which the abstract syntax is invalid [6]:

2>	ignore the message.

NOTE:	This clause applies in case one or more fields is set to a value, other than a spare, reserved or extended value, not defined in this version of the transfer syntax. E.g. in the case the UE receives value 12 for a field defined as INTEGER (1..11). In cases like this, it may not be possible to reliably detect which field is in the error hence the error handling is at the message level.

## 10.3	Field set to a not comprehended value

The UE shall, when receiving an RRC message on any logical channel:

1>	if the message includes a field that has a value that the UE does not comprehend:

2>	if a default value is defined for this field:

3>	treat the message while using the default value defined for this field;

2>	else if the concerned field is optional:

3>	treat the message as if the field were absent and in accordance with the need code for absence of the concerned field;

2>	else:

3>	treat the message as if the field were absent and in accordance with sub-clause 10.4.

## 10.4	Mandatory field missing

The UE shall:

1>	if the message includes a field that is mandatory to include in the message (e.g. because conditions for mandatory presence are fulfilled) and that field is absent or treated as absent:

2>	if the RRC message was not received on DCCH or CCCH:

3>	if the field concerns a (sub-field of) an entry of a list (i.e. a SEQUENCE OF):

4>	treat the list as if the entry including the missing or not comprehended field was absent;

3>	else if the field concerns a sub-field of another field, referred to as the 'parent' field i.e. the field that is one nesting level up compared to the erroneous field:

4>	consider the 'parent' field to be set to a not comprehended value;

4>	apply the generic error handling to the subsequent 'parent' field(s), until reaching the top nesting level i.e. the message level;

3>	else (field at message level):

4>	ignore the message.

NOTE 1:	The error handling defined in these sub-clauses implies that the UE ignores a message with the message type or version set to a not comprehended value.

NOTE 2:	The nested error handling for messages received on logical channels other than DCCH and CCCH applies for errors in extensions also, even for errors that can be regarded as invalid network operation e.g. the network not observing conditional presence.

NOTE 3:	UE behaviour on receipt of an RRC message on DCCH or CCCH that does not include a field that is mandatory (e.g. because conditions for mandatory presence are fulfilled) is unspecified.

The following ASN.1 further clarifies the levels applicable in case of nested error handling for errors in extension fields.

-- /example/ ASN1START

-- Example with extension addition group

ItemInfoList ::=                    SEQUENCE (SIZE (1..max)) OFItemInfo

ItemInfo ::=                        SEQUENCE {

itemIdentity                        INTEGER (1..max),

field1                              Field1,

field2                              Field2                  OPTIONAL,           -- Need N

...

[[

field3-r9                       Field3-r9               OPTIONAL,              -- Cond Cond1

field4-r9                       Field4-r9               OPTIONAL               -- Need N

]]

}

-- Example with traditional non-critical extension (empty sequence)

BroadcastInfoBlock1 ::=             SEQUENCE {

itemIdentity                        INTEGER (1..max),

field1                              Field1,

field2                              Field2                  OPTIONAL,           -- Need N

nonCriticalExtension                BroadcastInfoBlock1-v940-IEs    OPTIONAL

}

BroadcastInfoBlock1-v940-IEs::=	SEQUENCE {

field3-r9                           Field3-r9               OPTIONAL,           -- Cond Cond1

field4-r9                           Field4-r9               OPTIONAL,           -- Need N

nonCriticalExtension                SEQUENCE {}             OPTIONAL            -- Need S

}

-- ASN1STOP

The UE shall, apply the following principles regarding the levels applicable in case of nested error handling:

-	an extension additon group is not regarded as a level on its own. E.g. in the ASN.1 extract in the previous, a error regarding the conditionality of field3 would result in the entire itemInfo entry to be ignored (rather than just the extension addition group containing field3 and field4);

-	a traditional nonCriticalExtension is not regarded as a level on its own. E.g. in the ASN.1 extract in the previous, an error regarding the conditionality of field3 would result in the entire BroadcastInfoBlock1 to be ignored (rather than just the non-critical extension containing field3 and field4).

## 10.5	Not comprehended field

The UE shall, when receiving an RRC message on any logical channel:

1>	if the message includes a field that the UE does not comprehend:

2>	treat the rest of the message as if the field was absent.

NOTE:	This clause does not apply to the case of an extension to the value range of a field. Such cases are addressed instead by the requirements in clause 10.3.

# 11	Radio information related interactions between network nodes

## 11.1	General

This clause specifies RRC messages that are transferred between network nodes. These RRC messages may be transferred to or from the UE via another Radio Access Technology. Consequently, these messages have similar characteristics as the RRC messages that are transferred across the NR radio interface, i.e. the same transfer syntax and protocol extension mechanisms apply.

## 11.2	Inter-node RRC messages

### 11.2.1	General

This clause specifies RRC messages that are sent either across the X2-, Xn- or the NG-interface, either to or from the gNB, i.e. a single 'logical channel' is used for all RRC messages transferred across network nodes. The information could originate from or be destined for another RAT.

-- ASN1START

-- TAG-NR-INTER-NODE-DEFINITIONS-START

NR-InterNodeDefinitions DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

IMPORTS

ARFCN-ValueNR,

ARFCN-ValueEUTRA,

CellIdentity,

CGI-InfoEUTRA,

CGI-InfoNR,

CSI-RS-Index,

DRX-Config,

EUTRA-PhysCellId,

FreqBandIndicatorNR,

GapConfig,

maxBandComb,

maxBands,

maxCellSFTD,

maxFeatureSetsPerBand,

maxFreqIDC-MRDC,

maxNrofCombIDC,

maxNrofSCells,

maxNrofServingCells,

maxNrofServingCells-1,

maxNrofServingCellsEUTRA,

maxNrofIndexesToReport,

maxSimultaneousBands,

MeasQuantityResults,

MeasResultCellListSFTD-EUTRA,

MeasResultCellListSFTD-NR,

MeasResultList2NR,

MeasResultSCG-Failure,

MeasResultServFreqListEUTRA-SCG,

P-Max,

PhysCellId,

RadioBearerConfig,

RAN-NotificationAreaInfo,

RRCReconfiguration,

ServCellIndex,

SetupRelease,

SSB-Index,

SSB-MTC,

SSB-ToMeasure,

SS-RSSI-Measurement,

ShortMAC-I,

SubcarrierSpacing,

UEAssistanceInformation,

UE-CapabilityRAT-ContainerList

FROM NR-RRC-Definitions;

-- TAG-NR-INTER-NODE-DEFINITIONS-STOP

-- ASN1STOP

### 11.2.2	Message definitions

#### –	HandoverCommand

This message is used to transfer the handover command as generated by the target gNB.

Direction: target gNB to source gNB/source RAN.

HandoverCommand message

-- ASN1START

-- TAG-HANDOVER-COMMAND-START

HandoverCommand ::=                 SEQUENCE {

criticalExtensions                  CHOICE {

c1                                  CHOICE{

handoverCommand                     HandoverCommand-IEs,

spare3 NULL, spare2 NULL, spare1 NULL

},

criticalExtensionsFuture            SEQUENCE {}

}

}

HandoverCommand-IEs ::=             SEQUENCE {

handoverCommandMessage              OCTET STRING (CONTAINING RRCReconfiguration),

nonCriticalExtension                SEQUENCE {}                                        OPTIONAL

}

-- TAG-HANDOVER-COMMAND-STOP

-- ASN1STOP

HandoverCommand field descriptions

handoverCommandMessage Contains the RRCReconfiguration message used to perform handover within NR or handover to NR, as generated (entirely) by the target gNB.

#### –	HandoverPreparationInformation

This message is used to transfer the NR RRC information used by the target gNB during handover preparation or UE context retrieval, e.g. in case of resume or re-establishment, including UE capability information. This message is also used for transferring the information between the CU and DU.

Direction: source gNB/source RAN to target gNB or CU to DU.

HandoverPreparationInformation message

-- ASN1START

-- TAG-HANDOVER-PREPARATION-INFORMATION-START

HandoverPreparationInformation ::=      SEQUENCE {

criticalExtensions                      CHOICE {

c1                                      CHOICE{

handoverPreparationInformation          HandoverPreparationInformation-IEs,

spare3 NULL, spare2 NULL, spare1 NULL

},

criticalExtensionsFuture            SEQUENCE {}

}

}

HandoverPreparationInformation-IEs ::=  SEQUENCE {

ue-CapabilityRAT-List                   UE-CapabilityRAT-ContainerList,

sourceConfig                            AS-Config                                       OPTIONAL, -- Cond HO

rrm-Config                              RRM-Config                                      OPTIONAL,

as-Context                              AS-Context                                      OPTIONAL,

nonCriticalExtension                    SEQUENCE {}                                     OPTIONAL

}

AS-Config ::=                           SEQUENCE {

rrcReconfiguration                      OCTET STRING (CONTAINING RRCReconfiguration),

...,

[[

sourceRB-SN-Config                      OCTET STRING (CONTAINING RadioBearerConfig)     OPTIONAL,

sourceSCG-NR-Config                     OCTET STRING (CONTAINING RRCReconfiguration)    OPTIONAL,

sourceSCG-EUTRA-Config                  OCTET STRING                                    OPTIONAL

]],

[[

sourceSCG-Configured                    ENUMERATED {true}                               OPTIONAL

]]

}

AS-Context ::=                          SEQUENCE {

reestablishmentInfo                     ReestablishmentInfo                             OPTIONAL,

configRestrictInfo                      ConfigRestrictInfoSCG                           OPTIONAL,

...,

[[  ran-NotificationAreaInfo            RAN-NotificationAreaInfo                        OPTIONAL

]],

[[  ueAssistanceInformation             OCTET STRING (CONTAINING UEAssistanceInformation)  OPTIONAL   -- Cond HO2

]],

[[

selectedBandCombinationSN               BandCombinationInfoSN                           OPTIONAL

]]

}

ReestablishmentInfo ::=             SEQUENCE {

sourcePhysCellId                        PhysCellId,

targetCellShortMAC-I                    ShortMAC-I,

additionalReestabInfoList               ReestabNCellInfoList                            OPTIONAL

}

ReestabNCellInfoList ::=        SEQUENCE ( SIZE (1..maxCellPrep) ) OF ReestabNCellInfo

ReestabNCellInfo::= SEQUENCE{

cellIdentity                            CellIdentity,

key-gNodeB-Star                         BIT STRING (SIZE (256)),

shortMAC-I                              ShortMAC-I

}

RRM-Config ::=              SEQUENCE {

ue-InactiveTime             ENUMERATED {

s1, s2, s3, s5, s7, s10, s15, s20,

s25, s30, s40, s50, min1, min1s20, min1s40,

min2, min2s30, min3, min3s30, min4, min5, min6,

min7, min8, min9, min10, min12, min14, min17, min20,

min24, min28, min33, min38, min44, min50, hr1,

hr1min30, hr2, hr2min30, hr3, hr3min30, hr4, hr5, hr6,

hr8, hr10, hr13, hr16, hr20, day1, day1hr12, day2,

day2hr12, day3, day4, day5, day7, day10, day14, day19,

day24, day30, dayMoreThan30}                            OPTIONAL,

candidateCellInfoList       MeasResultList2NR                                           OPTIONAL,

...,

[[

candidateCellInfoListSN-EUTRA      MeasResultServFreqListEUTRA-SCG                      OPTIONAL

]]

}

-- TAG-HANDOVER-PREPARATION-INFORMATION-STOP

-- ASN1STOP

HandoverPreparationInformation field descriptions

as-Context Local RAN context required by the target gNB or DU.

rrm-Config Local RAN context used mainly for RRM purposes.

sourceConfig The radio resource configuration as used in the source cell.

ue-CapabilityRAT-List The UE radio access related capabilities concerning RATs supported by the UE. A gNB that retrieves MRDC related capability containers ensures that the set of included MRDC containers is consistent w.r.t. the feature set related information.

ue-InactiveTime Duration while UE has not received or transmitted any user data. Thus the timer is still running in case e.g., UE measures the neighbour cells for the HO purpose. Value s1 corresponds to 1 second, s2 corresponds to 2 seconds and so on. Value min1 corresponds to 1 minute, value min1s20 corresponds to 1 minute and 20 seconds, value min1s40 corresponds to 1 minute and 40 seconds and so on. Value hr1 corresponds to 1 hour, hr1min30 corresponds to 1 hour and 30 minutes and so on.

AS-Config field descriptions

rrcReconfiguration Contains the RRCReconfiguration configuration as generated entirely by the MN.

sourceRB-SN-Config Contains the IE RadioBearerConfig as generated entirely by the SN. This field is only used when the UE is configured with SN terminated RB(s).

sourceSCG-Configured Value true indicates that the UE is configured with NR or EUTRA SCG in source configuration. The field is only used in NR-DC and NE-DC and is included only if the fields sourceSCG-NR-Config and sourceSCG-EUTRA-Config are absent.

sourceSCG-EUTRA-Config Contains the current dedicated SCG configuration in RRCConnectionReconfiguration message as specified in TS 36.331 [10] and generated entirely by the SN. In this version of the specification, the E-UTRA RRCConnectionReconfiguration message can only include the field scg-Configuration . This field is only used in NE-DC.

sourceSCG-NR-Config Contains the current dedicated SCG configuration in RRCReconfiguration message as generated entirely by the SN. In this version of the specification, the RRCReconfiguration message can only include fields secondaryCellGroup and measConfig. This field is only used in NR-DC.

AS-Context field descriptions

selectedBandCombinationSN Indicates the band combination selected by SN in (NG)EN-DC, NE-DC, and NR-DC.

ueAssistanceInformation Includes for each UE assistance feature the information last reported by the UE, if any.

RRM-Config field descriptions

candidateCellInfoList A list of the best cells on each frequency for which measurement information was available

candidateCellInfoListSN-EUTRA A list of EUTRA cells including serving cells and best neighbour cells on each serving frequency, for which measurement results were available. This field is only used in NE-DC.

| Conditional Presence | Explanation |
| --- | --- |
| HO | The field is mandatory present in case of handover within NR or UE context retrieval, e.g. in case of resume or re-establishment. The field is optionally present in case of handover from E-UTRA/5GC. Otherwise the field is absent. |
| HO2 | The field is optionally present in case of handover within NR; otherwise the field is absent. |

NOTE 1:	The following table indicates per source RAT whether RAT capabilities are included or not.

| Source RAT | NR capabilites | E-UTRA capabilities | MR-DC capabilities |
| --- | --- | --- | --- |
| NR | Included | May be included | May be included |
| E-UTRAN | Included | May be included | May be included |

NOTE 2:	The following table indicates, in case of inter-RAT handover from E-UTRA, which additional IEs are included or not:

| Source system | sourceConfig | rrm-Config | as-Context |
| --- | --- | --- | --- |
| E-UTRA/EPC | Not included | May be included | Not included |
| E-UTRA/5GC | May be included, but only radioBearerConfig is included in the RRCReconfiguration. | May be included | Not included |

#### –	CG-Config

This message is used to transfer the SCG radio configuration as generated by the SgNB or SeNB. It can also be used by a CU to request a DU to perform certain actions, e.g. to request the DU to perform a new lower layer configuration.

Direction: Secondary gNB or eNB to master gNB or eNB, alternatively CU to DU.

CG-Config message

-- ASN1START

-- TAG-CG-CONFIG-START

CG-Config ::=                   SEQUENCE {

criticalExtensions                  CHOICE {

c1                                  CHOICE{

cg-Config                           CG-Config-IEs,

spare3 NULL, spare2 NULL, spare1 NULL

},

criticalExtensionsFuture            SEQUENCE {}

}

}

CG-Config-IEs ::=                   SEQUENCE {

scg-CellGroupConfig                 OCTET STRING (CONTAINING RRCReconfiguration)    OPTIONAL,

scg-RB-Config                       OCTET STRING (CONTAINING RadioBearerConfig)     OPTIONAL,

configRestrictModReq                ConfigRestrictModReqSCG                         OPTIONAL,

drx-InfoSCG                         DRX-Info                                        OPTIONAL,

candidateCellInfoListSN             OCTET STRING (CONTAINING MeasResultList2NR)     OPTIONAL,

measConfigSN                        MeasConfigSN                                    OPTIONAL,

selectedBandCombination             BandCombinationInfoSN                           OPTIONAL,

fr-InfoListSCG                      FR-InfoList                                     OPTIONAL,

candidateServingFreqListNR          CandidateServingFreqListNR                      OPTIONAL,

nonCriticalExtension                CG-Config-v1540-IEs                             OPTIONAL

}

CG-Config-v1540-IEs ::=             SEQUENCE {

pSCellFrequency                     ARFCN-ValueNR                                   OPTIONAL,

reportCGI-RequestNR                 SEQUENCE {

requestedCellInfo                   SEQUENCE {

ssbFrequency                        ARFCN-ValueNR,

cellForWhichToReportCGI             PhysCellId

}                                                                               OPTIONAL

}                                                                                   OPTIONAL,

ph-InfoSCG                          PH-TypeListSCG                                  OPTIONAL,

nonCriticalExtension                CG-Config-v1560-IEs                             OPTIONAL

}

CG-Config-v1560-IEs ::=             SEQUENCE {

pSCellFrequencyEUTRA                ARFCN-ValueEUTRA                                OPTIONAL,

scg-CellGroupConfigEUTRA            OCTET STRING                                    OPTIONAL,

candidateCellInfoListSN-EUTRA       OCTET STRING                                    OPTIONAL,

candidateServingFreqListEUTRA       CandidateServingFreqListEUTRA                   OPTIONAL,

needForGaps                         ENUMERATED {true}                               OPTIONAL,

drx-ConfigSCG                       DRX-Config                                      OPTIONAL,

reportCGI-RequestEUTRA              SEQUENCE {

requestedCellInfoEUTRA          SEQUENCE {

eutraFrequency                             ARFCN-ValueEUTRA,

cellForWhichToReportCGI-EUTRA              EUTRA-PhysCellId

}                                                                               OPTIONAL

}                                                                                   OPTIONAL,

nonCriticalExtension                CG-Config-v1590-IEs                             OPTIONAL

}

CG-Config-v1590-IEs ::=             SEQUENCE {

scellFrequenciesSN-NR               SEQUENCE (SIZE (1.. maxNrofServingCells-1)) OF  ARFCN-ValueNR      OPTIONAL,

scellFrequenciesSN-EUTRA            SEQUENCE (SIZE (1.. maxNrofServingCells-1)) OF  ARFCN-ValueEUTRA   OPTIONAL,

nonCriticalExtension                SEQUENCE {}                                                        OPTIONAL

}

PH-TypeListSCG ::=                  SEQUENCE (SIZE (1..maxNrofServingCells)) OF PH-InfoSCG

PH-InfoSCG ::=                      SEQUENCE {

servCellIndex                       ServCellIndex,

ph-Uplink                           PH-UplinkCarrierSCG,

ph-SupplementaryUplink              PH-UplinkCarrierSCG                             OPTIONAL,

...

}

PH-UplinkCarrierSCG ::=             SEQUENCE{

ph-Type1or3                         ENUMERATED {type1, type3},

...

}

MeasConfigSN ::=                    SEQUENCE {

measuredFrequenciesSN               SEQUENCE (SIZE (1..maxMeasFreqsSN)) OF NR-FreqInfo  OPTIONAL,

...

}

NR-FreqInfo ::=                     SEQUENCE {

measuredFrequency                   ARFCN-ValueNR                                       OPTIONAL,

...

}

ConfigRestrictModReqSCG ::=         SEQUENCE {

requestedBC-MRDC                    BandCombinationInfoSN                               OPTIONAL,

requestedP-MaxFR1                   P-Max                                               OPTIONAL,

...,

[[

requestedPDCCH-BlindDetectionSCG    INTEGER (1..15)                                     OPTIONAL,

requestedP-MaxEUTRA                 P-Max                                               OPTIONAL

]]

}

BandCombinationIndex ::= INTEGER (1..maxBandComb)

BandCombinationInfoSN ::=           SEQUENCE {

bandCombinationIndex                BandCombinationIndex,

requestedFeatureSets                FeatureSetEntryIndex

}

FR-InfoList ::= SEQUENCE (SIZE (1..maxNrofServingCells-1)) OF FR-Info

FR-Info ::= SEQUENCE {

servCellIndex       ServCellIndex,

fr-Type             ENUMERATED {fr1, fr2}

}

CandidateServingFreqListNR ::= SEQUENCE (SIZE (1.. maxFreqIDC-MRDC)) OF ARFCN-ValueNR

CandidateServingFreqListEUTRA ::= SEQUENCE (SIZE (1.. maxFreqIDC-MRDC)) OF ARFCN-ValueEUTRA

-- TAG-CG-CONFIG-STOP

-- ASN1STOP

CG-Config field descriptions

candidateCellInfoListSN Contains information regarding cells that the source secondary node suggests the target secondary gNB to consider configuring.

candidateCellInfoListSN-EUTRA Includes the MeasResultList3EUTRA as specified in TS 36.331 [10]. Contains information regarding cells that the source secondary node suggests the target secondary eNB to consider configuring. This field is only used in NE-DC.

candidateServingFreqListNR, candidateServingFreqListEUTRA Indicates frequencies of candidate serving cells for In-Device Co-existence Indication (see TS 36.331 [10]).

configRestrictModReq Used by SN to request changes to SCG configuration restrictions previously set by MN to ensure UE capabilities are respected. E.g. can be used to request configuring an NR band combination whose use MN has previously forbidden.

drx-ConfigSCG This field contains the complete DRX configuration of the SCG. This field is only used in NR-DC.

drx-InfoSCG This field contains the DRX long and short cycle configuration of the SCG. This field is used in (NG)EN-DC and NE-DC.

fr-InfoListSCG Contains information of FR information of serving cells that include PScell and SCells configured in SCG.

measuredFrequenciesSN Used by SN to indicate a list of frequencies measured by the UE.

needForGaps In NE-DC, indicates wheter the SN requests gNB to configure measurements gaps.

ph-InfoSCG Power headroom information in SCG that is needed in the reception of PHR MAC CE of MCG

ph-SupplementaryUplink Power headroom information for supplementary uplink. In the case of (NG)EN-DC and NR-DC, this field is only present when two UL carriers are configued for a serving cell and one UL carrier reports type1 PH while the other reports type 3 PH.

ph-Type1or3 Type of power headroom for a certain serving cell in SCG (PSCell and activated SCells). Value type1 refers to type 1 power headroom, value type3 refers to type 3 power headroom. (See TS 38.321 [3]).

ph-Uplink Power headroom information for uplink.

pSCellFrequency, pSCellFrequencyEUTRA Indicates the frequency of PSCell in NR (i.e., pSCellFrequency) or E-UTRA (i.e., pSCellFrequencyEUTRA). In this version of the specification, pSCellFrequency is not used in NE-DC whereas pSCellFrequencyEUTRA is only used in NE-DC.

reportCGI-RequestNR, reportCGI-RequestEUTRA Used by SN to indicate to MN about configuring reportCGI procedure. The request may optionally contain information about the cell for which SN intends to configure reportCGI procedure. In this version of the specification, the reportCGI-RequestNR is used in (NG)EN-DC and NR-DC whereas reportCGI-RequestEUTRA is used only for NE-DC.

requestedBC-MRDC Used to request configuring a band combination and corresponding feature sets which are forbidden to use by MN (i.e. outside of the allowedBC-ListMRDC) to allow re-negotiation of the UE capabilities for SCG configuration.

requestedPDCCH-BlindDetectionSCG Requested value of the reference number of cells for PDCCH blind detection allowed to be configured for the SCG.

requestedP-MaxEUTRA Requested value for the maximum power for the serving cells the UE can use in E-UTRA SCG. This field is only used in NE-DC.

requestedP-MaxFR1 Requested value for the maximum power for the serving cells on frequency range 1 (FR1) in this secondary cell group (see TS 38.104 [12]) the UE can use in NR SCG.

scellFrequenciesSN-EUTRA, scellFrequenciesSN-NR Indicates the frequency of all SCells configured in SCG. The field scellFrequenciesSN-EUTRA is used in NE-DC; the field scellFrequenciesSN-NR is used in (NG)EN-DC and NR-DC. In (NG)EN-DC, the field is optionally provided to the MN.

scg-CellGroupConfig Contains the RRCReconfiguration message (containing only secondaryCellGroup and/or measConfig): - to be sent to the UE, used upon SCG establishment or modification, as generated (entirely) by the (target) SgNB. In this case, the SN sets the RRCReconfiguration message in accordance with clause 6 e.g. regarding the "Need" or "Cond" statements. or - including the current SCG configuration of the UE, when provided in response to a query from MN, or in SN triggered SN change in order to enable delta signaling by the target SN. In this case, the SN sets the RRCReconfiguration message in accordance with clause 11.2.3. The field is absent if neither SCG (re)configuration nor SCG configuration query nor SN triggered SN change is performed, e.g. at inter-node capability/configuration coordination which does not result in SCG (re)configuration towards the UE. This field is not applicable in NE-DC.

scg-CellGroupConfigEUTRA Includes the E-UTRA RRCConnectionReconfiguration message as specified in TS 36.331 [10]. In this version of the specification, the E-UTRA RRC message can only include the field scg-Configuration. Used to (re-)configure the SCG configuration upon SCG establishment or modification, as generated (entirely) by the (target) SeNB. This field is only used in NE-DC.

scg-RB-Config Contains the IE RadioBearerConfig: - to be sent to the UE, used to (re-)configure the SCG RB configuration upon SCG establishment or modification, as generated (entirely) by the (target) SgNB or SeNB. In this case, the SN sets the RadioBearerConfig in accordance with clause 6, e.g. regarding the "Need" or "Cond" statements. or - including the current SCG RB configuration of the UE, when provided in response to a query from MN or in SN triggered SN change or bearer type change between SN terminated bearer to MN terminated bearer in order to enable delta signaling by the MN or target SN. In this case, the SN sets the RadioBearerConfig in accordance with clause 11.2.3. The field is absent if neither SCG (re)configuration nor SCG configuration query nor SN triggered SN change is performed, e.g. at inter-node capability/configuration coordination which does not result in SCG RB (re)configuration.

selectedBandCombination Indicates the band combination selected by SN in (NG)EN-DC, NE-DC, and NR-DC. The SN should inform the MN with this field whenever the band combination and/or feature set it selected for the SCG changes (i.e. even if the new selection concerns a band combination and/or feature set that is allowed by the allowedBC-ListMRDC)

BandCombinationInfoSN field descriptions

bandCombinationIndex In case of (NG)EN-DC and NR-DC, this field indicates the position of a band combination in the supportedBandCombinationList. In case of NE-DC, this field indicates the position of a band combination in the supportedBandCombinationList and/or supportedBandCombinationListNEDC-Only. Band combination entries in supportedBandCombinationList are referred by an index which corresponds to the position of a band combination in the supportedBandCombinationList. Band combination entries in supportedBandCombinationListNEDC-Only are referred by an index which corresponds to the position of a band combination in the supportedBandCombinationListNEDC-Only increased by the number of entries in supportedBandCombinationList.

requestedFeatureSets The position in the FeatureSetCombination which identifies one FeatureSetUplink/Downlink for each band entry in the associated band combination

#### –	CG-ConfigInfo

This message is used by master eNB or gNB to request the SgNB or SeNB to perform certain actions e.g. to establish, modify or release an SCG. The message may include additional information e.g. to assist the SgNB or SeNB to set the SCG configuration. It can also be used by a CU to request a DU to perform certain actions, e.g. to establish, or modify an MCG or SCG.

Direction: Master eNB or gNB to secondary gNB or eNB, alternatively CU to DU.

CG-ConfigInfo message

-- ASN1START

-- TAG-CG-CONFIG-INFO-START

CG-ConfigInfo ::=               SEQUENCE {

criticalExtensions              CHOICE {

c1                              CHOICE{

cg-ConfigInfo               CG-ConfigInfo-IEs,

spare3 NULL, spare2 NULL, spare1 NULL

},

criticalExtensionsFuture        SEQUENCE {}

}

}

CG-ConfigInfo-IEs ::=           SEQUENCE {

ue-CapabilityInfo               OCTET STRING (CONTAINING UE-CapabilityRAT-ContainerList)          OPTIONAL,-- Cond SN-AddMod

candidateCellInfoListMN         MeasResultList2NR                                                 OPTIONAL,

candidateCellInfoListSN         OCTET STRING (CONTAINING MeasResultList2NR)                       OPTIONAL,

measResultCellListSFTD-NR       MeasResultCellListSFTD-NR                                         OPTIONAL,

scgFailureInfo                  SEQUENCE {

failureType                     ENUMERATED { t310-Expiry, randomAccessProblem,

rlc-MaxNumRetx, synchReconfigFailure-SCG,

scg-reconfigFailure,

srb3-IntegrityFailure},

measResultSCG                   OCTET STRING (CONTAINING MeasResultSCG-Failure)

}                                                                                                 OPTIONAL,

configRestrictInfo              ConfigRestrictInfoSCG                                             OPTIONAL,

drx-InfoMCG                     DRX-Info                                                          OPTIONAL,

measConfigMN                    MeasConfigMN                                                      OPTIONAL,

sourceConfigSCG                 OCTET STRING (CONTAINING RRCReconfiguration)                      OPTIONAL,

scg-RB-Config                   OCTET STRING (CONTAINING RadioBearerConfig)                       OPTIONAL,

mcg-RB-Config                   OCTET STRING (CONTAINING RadioBearerConfig)                       OPTIONAL,

mrdc-AssistanceInfo             MRDC-AssistanceInfo                                               OPTIONAL,

nonCriticalExtension            CG-ConfigInfo-v1540-IEs                                           OPTIONAL

}

CG-ConfigInfo-v1540-IEs ::=     SEQUENCE {

ph-InfoMCG                      PH-TypeListMCG                                                    OPTIONAL,

measResultReportCGI             SEQUENCE {

ssbFrequency                    ARFCN-ValueNR,

cellForWhichToReportCGI         PhysCellId,

cgi-Info                        CGI-InfoNR

}                                                                                                 OPTIONAL,

nonCriticalExtension            CG-ConfigInfo-v1560-IEs                                           OPTIONAL

}

CG-ConfigInfo-v1560-IEs ::=	 SEQUENCE {

candidateCellInfoListMN-EUTRA       OCTET STRING                                              OPTIONAL,

candidateCellInfoListSN-EUTRA       OCTET STRING                                              OPTIONAL,

sourceConfigSCG-EUTRA               OCTET STRING                                              OPTIONAL,

scgFailureInfoEUTRA                 SEQUENCE {

failureTypeEUTRA                    ENUMERATED { t313-Expiry, randomAccessProblem,

rlc-MaxNumRetx, scg-ChangeFailure},

measResultSCG-EUTRA                 OCTET STRING

}                                                                                             OPTIONAL,

drx-ConfigMCG                       DRX-Config                                                OPTIONAL,

measResultReportCGI-EUTRA               SEQUENCE {

eutraFrequency                      ARFCN-ValueEUTRA,

cellForWhichToReportCGI-EUTRA           EUTRA-PhysCellId,

cgi-InfoEUTRA                           CGI-InfoEUTRA

}                                                                                             OPTIONAL,

measResultCellListSFTD-EUTRA        MeasResultCellListSFTD-EUTRA                              OPTIONAL,

fr-InfoListMCG                      FR-InfoList                                               OPTIONAL,

nonCriticalExtension                CG-ConfigInfo-v1570-IEs                                   OPTIONAL

}

CG-ConfigInfo-v1570-IEs ::=  SEQUENCE {

sftdFrequencyList-NR                SFTD-FrequencyList-NR                                     OPTIONAL,

sftdFrequencyList-EUTRA             SFTD-FrequencyList-EUTRA                                  OPTIONAL,

nonCriticalExtension                CG-ConfigInfo-v1590-IEs                                   OPTIONAL

}

CG-ConfigInfo-v1590-IEs ::=  SEQUENCE {

servFrequenciesMN-NR            SEQUENCE (SIZE (1.. maxNrofServingCells-1)) OF  ARFCN-ValueNR     OPTIONAL,

nonCriticalExtension            SEQUENCE {}                                                       OPTIONAL

}

SFTD-FrequencyList-NR ::=               SEQUENCE (SIZE (1..maxCellSFTD)) OF ARFCN-ValueNR

SFTD-FrequencyList-EUTRA ::=            SEQUENCE (SIZE (1..maxCellSFTD)) OF ARFCN-ValueEUTRA

ConfigRestrictInfoSCG ::=       SEQUENCE {

allowedBC-ListMRDC              BandCombinationInfoList                                       OPTIONAL,

powerCoordination-FR1               SEQUENCE {

p-maxNR-FR1                     P-Max                                                     OPTIONAL,

p-maxEUTRA                      P-Max                                                     OPTIONAL,

p-maxUE-FR1                     P-Max                                                     OPTIONAL

}                                                                                             OPTIONAL,

servCellIndexRangeSCG           SEQUENCE {

lowBound                        ServCellIndex,

upBound                         ServCellIndex

}                                                                                             OPTIONAL,   -- Cond SN-AddMod

maxMeasFreqsSCG                     INTEGER(1..maxMeasFreqsMN)                                OPTIONAL,

dummy                               INTEGER(1..maxMeasIdentitiesMN)                           OPTIONAL,

...,

[[

selectedBandEntriesMNList        SEQUENCE (SIZE (1..maxBandComb)) OF SelectedBandEntriesMN    OPTIONAL,

pdcch-BlindDetectionSCG          INTEGER (1..15)                                              OPTIONAL,

maxNumberROHC-ContextSessionsSN  INTEGER(0.. 16384)                                           OPTIONAL

]],

[[

maxIntraFreqMeasIdentitiesSCG     INTEGER(1..maxMeasIdentitiesMN)                             OPTIONAL,

maxInterFreqMeasIdentitiesSCG     INTEGER(1..maxMeasIdentitiesMN)                             OPTIONAL

]]

}

SelectedBandEntriesMN ::=       SEQUENCE (SIZE (1..maxSimultaneousBands)) OF BandEntryIndex

BandEntryIndex ::=              INTEGER (0.. maxNrofServingCells)

PH-TypeListMCG ::=              SEQUENCE (SIZE (1..maxNrofServingCells)) OF PH-InfoMCG

PH-InfoMCG ::=                  SEQUENCE {

servCellIndex                       ServCellIndex,

ph-Uplink                           PH-UplinkCarrierMCG,

ph-SupplementaryUplink              PH-UplinkCarrierMCG                                       OPTIONAL,

...

}

PH-UplinkCarrierMCG ::=         SEQUENCE{

ph-Type1or3                         ENUMERATED {type1, type3},

...

}

BandCombinationInfoList ::=     SEQUENCE (SIZE (1..maxBandComb)) OF BandCombinationInfo

BandCombinationInfo ::=         SEQUENCE {

bandCombinationIndex            BandCombinationIndex,

allowedFeatureSetsList          SEQUENCE (SIZE (1..maxFeatureSetsPerBand)) OF FeatureSetEntryIndex

}

FeatureSetEntryIndex ::=        INTEGER (1.. maxFeatureSetsPerBand)

DRX-Info ::=                    SEQUENCE {

drx-LongCycleStartOffset        CHOICE {

ms10                            INTEGER(0..9),

ms20                            INTEGER(0..19),

ms32                            INTEGER(0..31),

ms40                            INTEGER(0..39),

ms60                            INTEGER(0..59),

ms64                            INTEGER(0..63),

ms70                            INTEGER(0..69),

ms80                            INTEGER(0..79),

ms128                           INTEGER(0..127),

ms160                           INTEGER(0..159),

ms256                           INTEGER(0..255),

ms320                           INTEGER(0..319),

ms512                           INTEGER(0..511),

ms640                           INTEGER(0..639),

ms1024                          INTEGER(0..1023),

ms1280                          INTEGER(0..1279),

ms2048                          INTEGER(0..2047),

ms2560                          INTEGER(0..2559),

ms5120                          INTEGER(0..5119),

ms10240                         INTEGER(0..10239)

},

shortDRX                            SEQUENCE {

drx-ShortCycle                      ENUMERATED  {

ms2, ms3, ms4, ms5, ms6, ms7, ms8, ms10, ms14, ms16, ms20, ms30, ms32,

ms35, ms40, ms64, ms80, ms128, ms160, ms256, ms320, ms512, ms640, spare9,

spare8, spare7, spare6, spare5, spare4, spare3, spare2, spare1 },

drx-ShortCycleTimer                 INTEGER (1..16)

}                                                                                             OPTIONAL

}

MeasConfigMN ::= SEQUENCE {

measuredFrequenciesMN               SEQUENCE (SIZE (1..maxMeasFreqsMN)) OF NR-FreqInfo        OPTIONAL,

measGapConfig                       SetupRelease { GapConfig }                                OPTIONAL,

gapPurpose                          ENUMERATED {perUE, perFR1}                                OPTIONAL,

...,

[[ measGapConfigFR2                 SetupRelease { GapConfig }                                OPTIONAL

]]

}

MRDC-AssistanceInfo ::= SEQUENCE {

affectedCarrierFreqCombInfoListMRDC     SEQUENCE (SIZE (1..maxNrofCombIDC)) OF AffectedCarrierFreqCombInfoMRDC,

...

}

AffectedCarrierFreqCombInfoMRDC ::= SEQUENCE {

victimSystemType                    VictimSystemType,

interferenceDirectionMRDC           ENUMERATED {eutra-nr, nr, other, utra-nr-other, nr-other, spare3, spare2, spare1},

affectedCarrierFreqCombMRDC         SEQUENCE    {

affectedCarrierFreqCombEUTRA        AffectedCarrierFreqCombEUTRA                      OPTIONAL,

affectedCarrierFreqCombNR           AffectedCarrierFreqCombNR

}       OPTIONAL

}

VictimSystemType ::= SEQUENCE {

gps                         ENUMERATED {true}               OPTIONAL,

glonass                     ENUMERATED {true}               OPTIONAL,

bds                         ENUMERATED {true}               OPTIONAL,

galileo                     ENUMERATED {true}               OPTIONAL,

wlan                        ENUMERATED {true}               OPTIONAL,

bluetooth                   ENUMERATED {true}               OPTIONAL

}

AffectedCarrierFreqCombEUTRA ::= SEQUENCE (SIZE (1..maxNrofServingCellsEUTRA)) OF ARFCN-ValueEUTRA

AffectedCarrierFreqCombNR ::= SEQUENCE (SIZE (1..maxNrofServingCells)) OF ARFCN-ValueNR

-- TAG-CG-CONFIG-INFO-STOP

-- ASN1STOP

CG-ConfigInfo field descriptions

allowedBC-ListMRDC A list of indices referring to band combinations in MR-DC capabilities from which SN is allowed to select the SCG band combination. Each entry refers to: - a band combination numbered according to supportedBandCombinationList in the UE-MRDC-Capability (in case of (NG)EN-DC), or according to supportedBandCombinationList and supportedBandCombinationListNEDC-Only in the UE-MRDC-Capability (in case of NE-DC), or according to supportedBandCombinationList in the UE-NR-Capability (in case of NR-DC), - and the Feature Sets allowed for each band entry. All MR-DC band combinations indicated by this field comprise the MCG band combination, which is a superset of the MCG band(s) selected by MN.

candidateCellInfoListMN, candidateCellInfoListSN Contains information regarding cells that the master node or the source node suggests the target gNB or DU to consider configuring. For (NG)EN-DC, including CSI-RS measurement results in candidateCellInfoListMN is not supported in this version of the specification. For NR-DC, including SSB and/or CSI-RS measurement results in candidateCellInfoListMN is supported.

candidateCellInfoListMN-EUTRA, candidateCellInfoListSN-EUTRA Includes the MeasResultList3EUTRA as specified in TS 36.331 [10]. Contains information regarding cells that the master node or the source node suggests the target secondary eNB to consider configuring. These fields are only used in NE-DC.

configRestrictInfo Includes fields for which SgNB is explictly indicated to observe a configuration restriction.

drx-ConfigMCG This field contains the complete DRX configuration of the MCG. This field is only used in NR-DC.

drx-InfoMCG This field contains the DRX long and short cycle configuration of the MCG. This field is used in (NG)EN-DC and NE-DC.

fr-InfoListMCG Contains information of FR information of serving cells that include PCell and SCell(s) configured in MCG.

maxMeasFreqsSCG Indicates the maximum number of NR inter-frequency carriers the SN is allowed to configure with PSCell for measurements.

dummy This field is not used in the specification and SN ignores the received value.

maxInterFreqMeasIdentitiesSCG Indicates the maximum number of allowed measurement identities that the SCG is allowed to configure for inter-frequency measurement. The maximum value for this field is 10. If the field is absent, the SCG is allowed to configure inter-frequency measurements up to the maximum value. This field is only used in NR-DC.

maxIntraFreqMeasIdentitiesSCG Indicates the maximum number of allowed measurement identities that the SCG is allowed to configure for intra-frequency measurement on each serving frequency. The maximum value for this field is 9 (in case of (NG)EN-DC or NR-DC) or 10 (in case of NE-DC). If the field is absent, the SCG is allowed to configure intra-frequency measurements up to the maximum value on each serving frequency.

maxNumberROHC-ContextSessionsSN Indicates the maximum number of context sessions allowed to SN terminated bearer, excluding context sessions that leave all headers uncompressed.

measuredFrequenciesMN Used by MN to indicate a list of frequencies measured by the UE.

measGapConfig Indicates the FR1 and perUE measurement gap configuration configured by MN.

measGapConfigFR2 Indicates the FR2 measurement gap configuration configured by MN.

mcg-RB-Config Contains all of the fields in the IE RadioBearerConfig used in MCG, used by the SN to support delta configuration to UE, for bearer type change between MN terminated bearer with NR PDCP to SN terminated bearer. It is also used to indicate the PDCP duplication related information for MN terminated split bearer (whether duplication is configured and if so, whether it is initially activated) in SN Addition/Modification procedure. Otherwise, this field is absent.

measResultReportCGI, measResultReportCGI-EUTRA Used by MN to provide SN with CGI-Info for the cell as per SN′s request. In this version of the specification, the measResultReportCGI is used for (NG)EN-DC and NR-DC and the measResultReportCGI-EUTRA is used only for NE-DC.

measResultSCG-EUTRA This field includes the MeasResultSCG-FailureMRDC IE as specified in TS 36.331 [10]. This field is only used in NE-DC.

measResultSFTD-EUTRA SFTD measurement results between the PCell and the E-UTRA PScell in NE-DC. This field is only used in NE-DC.

mrdc-AssistanceInfo Contains the IDC assistance information for MR-DC reported by the UE (see TS 36.331 [10]).

p-maxEUTRA Indicates the maximum total transmit power to be used by the UE in the E-UTRA cell group (see TS 36.104 [33]). This field is used in (NG)EN-DC and NE-DC.

p-maxNR-FR1 Indicates the maximum total transmit power to be used by the UE in the NR cell group across all serving cells in frequency range 1 (FR1) (see TS 38.104 [12]). The field is used in (NG)EN-DC and NE-DC.

p-maxUE-FR1 Indicates the maximum total transmit power to be used by the UE across all serving cells in frequency range 1 (FR1).

pdcch-BlindDetectionSCG Indicates the maximum value of the reference number of cells for PDCCH blind detection allowed to be configured for the SCG.

ph-InfoMCG Power headroom information in MCG that is needed in the reception of PHR MAC CE in SCG.

ph-SupplementaryUplink Power headroom information for supplementary uplink. For UE in (NG)EN-DC, this field is absent.

ph-Type1or3 Type of power headroom for a serving cell in MCG (PCell and activated SCells). type1 refers to type 1 power headroom, type3 refers to type 3 power headroom. (See TS 38.321 [3]).

ph-Uplink Power headroom information for uplink.

powerCoordination-FR1 Indicates the maximum power that the UE can use in FR1.

scgFailureInfo Contains SCG failure type and measurement results. In case the sender has no measurement results available, the sender may include one empty entry (i.e. without any optional fields present) in measResultPerMOList. This field is used in (NG)EN-DC and NR-DC.

scgFailureInfoEUTRA Contains SCG failure type and measurement results of the EUTRA secondary cell group. This field is only used in NE-DC.

scg-RB-Config Contains all of the fields in the IE RadioBearerConfig used in SCG, used to allow the target SN to use delta configuration to the UE, e.g. during SN change. The field is signalled upon change of SN. Otherwise, the field is absent. This field is also absent when master eNB uses full configuration option.

selectedBandEntriesMNList A list of indices referring to the position of a band entry selected by the MN, in each band combination entry in allowedBC-ListMRDC IE. BandEntryIndex 0 identifies the first band in the bandList of the BandCombination, BandEntryIndex 1 identifies the second band in the bandList of the BandCombination, and so on. This selectedBandEntriesMNList includes the same number of entries, and listed in the same order as in allowedBC-ListMRDC. The SN uses this information to determine which bands out of the NR band combinations in allowedBC-ListMRDC it can configure in SCG. This field is only used in NR-DC.

servCellIndexRangeSCG Range of serving cell indices that SN is allowed to configure for SCG serving cells.

servFrequenciesMN-NR Indicates the frequency of all serving cells that include PCell and SCell(s) configured in MCG. This field is only used in NR-DC.

sftdFrequencyList-NR Includes a list of SSB frequencies. Each entry identifies the SSB frequency of a PSCell, which corresponds to one MeasResultCellSFTD-NR entry in the MeasResultCellListSFTD-NR.

sftdFrequencyList-EUTRA Includes a list of E-UTRA frequencies. Each entry identifies the carrier frequency of a PSCell, which corresponds to one MeasResultSFTD-EUTRA entry in the MeasResultCellListSFTD-EUTRA.

sourceConfigSCG Includes all of the current SCG configurations used by the target SN to build delta configuration to be sent to UE, e.g. during SN change. The field contains the RRCReconfiguration message, i.e. including secondaryCellGroup and measConfig. The field is signalled upon change of SN, unless MN uses full configuration option. Otherwise, the field is absent.

sourceConfigSCG-EUTRA Includes the E-UTRA RRCConnectionReconfiguration message as specified in TS 36.331 [10]. In this version of the specification, the E-UTRA RRC message can only include the field scg-Configuration. In this version of the specification, this field is absent when master gNB uses full configuration option. This field is only used in NE-DC.

ue-CapabilityInfo Contains the IE UE-CapabilityRAT-ContainerList supported by the UE (see NOTE 3). A gNB that retrieves MRDC related capability containers ensures that the set of included MRDC containers is consistent w.r.t. the feature set related information.

BandCombinationInfo field descriptions

allowedFeatureSetsList Defines a subset of the entries in a FeatureSetCombination. Each index identifies a position in the FeatureSetCombination, which corresponds to one FeatureSetUplink/Downlink for each band entry in the associated band combination.

bandCombinationIndex In case of (NG)EN-DC and NR-DC, this field indicates the position of a band combination in the supportedBandCombinationList. In case of NE-DC, this field indicates the position of a band combination in the supportedBandCombinationList and/or supportedBandCombinationListNEDC-Only. Band combination entries in supportedBandCombinationList are referred by an index which corresponds to the position of a band combination in the supportedBandCombinationList. Band combination entries in supportedBandCombinationListNEDC-Only are referred by an index which corresponds to the position of a band combination in the supportedBandCombinationListNEDC-Only increased by the number of entries in supportedBandCombinationList.

| Conditional Presence | Explanation |
| --- | --- |
| SN-AddMod | The field is mandatory present upon SN addition and SN change. It is optionally present upon SN modification and inter-MN handover without SN change. Otherwise, the field is absent. |

NOTE 3:	The following table indicates per source RAT whether RAT capabilities are included or not in ue-CapabilityInfo.

| Source RAT | NR capabilities | E-UTRA capabilities | MR-DC capabilities |
| --- | --- | --- | --- |
| E-UTRA | Included | Not included | Included |

#### –	MeasurementTimingConfiguration

The MeasurementTimingConfiguration message is used to convey assistance information for measurement timing.

Direction: en-gNB to eNB, eNB to en-gNB, gNB to gNB, ng-eNB to gNB, gNB to ng-eNB, ng-eNB to ng-eNB, gNB DU to gNB CU, and gNB CU to gNB DU.

MeasurementTimingConfiguration message

-- ASN1START

-- TAG-MEASUREMENT-TIMING-CONFIGURATION-START

MeasurementTimingConfiguration ::=      SEQUENCE {

criticalExtensions                      CHOICE {

c1                                      CHOICE{

measTimingConf                          MeasurementTimingConfiguration-IEs,

spare3 NULL, spare2 NULL, spare1 NULL

},

criticalExtensionsFuture        SEQUENCE {}

}

}

MeasurementTimingConfiguration-IEs ::=  SEQUENCE {

measTiming                              MeasTimingList                              OPTIONAL,

nonCriticalExtension                    MeasurementTimingConfiguration-v1550-IEs    OPTIONAL

}

MeasurementTimingConfiguration-v1550-IEs ::= SEQUENCE {

campOnFirstSSB                               BOOLEAN,

psCellOnlyOnFirstSSB                         BOOLEAN,

nonCriticalExtension                         SEQUENCE {}                            OPTIONAL

}

MeasTimingList ::= SEQUENCE (SIZE (1..maxMeasFreqsMN)) OF MeasTiming

MeasTiming ::= SEQUENCE {

frequencyAndTiming                      SEQUENCE {

carrierFreq                             ARFCN-ValueNR,

ssbSubcarrierSpacing                    SubcarrierSpacing,

ssb-MeasurementTimingConfiguration      SSB-MTC,

ss-RSSI-Measurement                     SS-RSSI-Measurement                         OPTIONAL

}                                                                                   OPTIONAL,

...,

[[

ssb-ToMeasure                           SSB-ToMeasure                               OPTIONAL,

physCellId                              PhysCellId                                  OPTIONAL

]]

}

-- TAG-MEASUREMENT-TIMING-CONFIGURATION-STOP

-- ASN1STOP

MeasTiming field descriptions

carrierFreq, ssbSubcarrierSpacing Indicates the frequency and subcarrier spacing of the SS block of the cell for which this message is included, or of other SS blocks within the same carrier.

ssb-MeasurementTimingConfiguration Indicates the SMTC which can be used to search for SSB of the cell for which the message is included. When the message is included in "Served NR Cell Information" (see TS 36.423 [37]), "Served Cell Information NR" (see TS 38.423 [35]), or "Served Cell Information" (see TS 38.473 [36]), the timing is based on the cell for which the message is included. When the message is included in "NR Neighbour Information" (see TS 36.423 [37]), or "Served Cell Information" (see TS 38.423 [35]), the timing is based on the cell indicated in the "Served NR Cell Information" or "Served Cell Information NR" with which the "NR Neighbour Information" or "Neighbour Information NR" is provided. When the message is included in "CU to DU RRC Information", the timing is based on the cell indicated by SpCell ID with which the message is included.

ss-RSSI-Measurement Provides the configuration which can be used for RSSI measurements of the cell for which the message is included.

MeasurementTimingConfiguration field descriptions

campOnFirstSSB Value true indicates that the SSB indicated in the first instance of MeasTiming in the measTiming list can be used for camping and for a PCell configuration (i.e. in spCellConfigCommon of the masterCellGroup).

measTiming A list of SMTC information, SSB RSSI measurement information and associated NR frequency exchanged via EN-DC X2 Setup, EN-DC Configuration Update, Xn Setup and NG-RAN Node Configuration Update procedures, or F1 messages between gNB DU and gNB CU.

physCellId Physical Cell Identity of the SSB on the ARFCN indicated by carrierFreq.

psCellOnlyOnFirstSSB Value true indicates that only the SSB indicated in the first instance of MeasTiming in the measTiming list can be used for a PSCell configuration (i.e. in spCellConfigCommon of the secondaryCellGroup).

ssb-ToMeasure The set of SS blocks to be measured within the SMTC measurement duration (see TS 38.215 [9]).

#### –	UERadioPagingInformation

This message is used to transfer radio paging information, covering both upload to and download from the 5GC, and between gNBs.

Direction: gNB to/ from 5GC and gNB to/from gNB

UERadioPagingInformation message

-- ASN1START

-- TAG-UE-RADIO-PAGING-INFORMATION-START

UERadioPagingInformation ::= SEQUENCE {

criticalExtensions                  CHOICE {

c1                                  CHOICE{

ueRadioPagingInformation            UERadioPagingInformation-IEs,

spare7 NULL,

spare6 NULL, spare5 NULL, spare4 NULL,

spare3 NULL, spare2 NULL, spare1 NULL

},

criticalExtensionsFuture            SEQUENCE {}

}

}

UERadioPagingInformation-IEs ::=    SEQUENCE {

supportedBandListNRForPaging        SEQUENCE (SIZE (1..maxBands)) OF FreqBandIndicatorNR    OPTIONAL,

nonCriticalExtension                SEQUENCE {}                                             OPTIONAL

}

-- TAG-UE-RADIO-PAGING-INFORMATION-STOP

-- ASN1STOP

UERadioPagingInformation field descriptions

supportedBandListNRForPaging Indicates the UE supported NR frequency bands which are derived by the gNB from UE-NR-Capability.

#### –	UERadioAccessCapabilityInformation

This message is used to transfer UE radio access capability information, covering both upload to and download from the 5GC.

Direction: ng-eNB or gNB to/ from 5GC

UERadioAccessCapabilityInformation message

-- ASN1START

-- TAG-UE-RADIO-ACCESS-CAPABILITY-INFORMATION-START

UERadioAccessCapabilityInformation ::= SEQUENCE {

criticalExtensions                  CHOICE {

c1                                  CHOICE{

ueRadioAccessCapabilityInformation    UERadioAccessCapabilityInformation-IEs,

spare7 NULL,

spare6 NULL, spare5 NULL, spare4 NULL,

spare3 NULL, spare2 NULL, spare1 NULL

},

criticalExtensionsFuture            SEQUENCE {}

}

}

UERadioAccessCapabilityInformation-IEs ::= SEQUENCE {

ue-RadioAccessCapabilityInfo               OCTET STRING (CONTAINING UE-CapabilityRAT-ContainerList),

nonCriticalExtension                       SEQUENCE {}                                                   OPTIONAL

}

-- TAG-UE-RADIO-ACCESS-CAPABILITY-INFORMATION-STOP

-- ASN1STOP

UERadioAccessCapabilityInformation-IEs field descriptions

ue-RadioAccessCapabilityInfo Including NR, MR-DC, E-UTRA radio access capabilities. A gNB that retrieves MRDC related capability containers ensures that the set of included MRDC containers is consistent w.r.t. the feature set related information.

### 11.2.3	Mandatory information in inter-node RRC messages

For the AS-Config transferred within the HandoverPreparationInformation:

-	The source node shall include all fields necessary to reflect the AS configuration of the UE, except for the fields sourceSCG-NR-Config, sourceSCG-EUTRA-Config and sourceRB-SN-Config, which can be omitted in case the source MN did not receive the latest configuration from the source SN. For RRCReconfiguration included in the field rrcReconfiguration, ReconfigurationWithSync is included with only the mandatory subfields (e.g. newUE-Identity and t304) and ServingCellConfigCommon;

-	Need codes or conditions specified for subfields according to IEs defined in clause 6 do not apply. I.e. some fields shall be included regardless of the "Need" or "Cond" e.g. discardTimer;

-	Based on the received AS configuration, the target node can indicate the delta (difference) to the UE's AS configuration (as included in HandoverCommand). The fields newUE-Identity and t304 included in ReconfigurationWithSync are not used for delta configuration purpose.

The candidateCellInfoListSN(-EUTRA) in CG-Config and the candidateCellInfoListMN(-EUTRA)/candidateCellInfoListSN(-EUTRA) in CG-ConfigInfo need not be included in procedures that do not involve a change of node.

For a field that conveys the UE configuration in CG-Config (SN initiated change of SN configuration, or SCG configuration query) and in CG-ConfigInfo upon change of SN (i.e. mcg-RB-Config, scg-RB-Config and sourceConfigSCG):

-	The source node shall include all fields necessary to reflect the AS configuration of the UE, unless stated otherwise in the field description or in this sub-clause. For RRCReconfiguration included in the field scg-CellGroupConfig in CG-Config, ReconfigurationWithSync is included with only the mandatory subfields (e.g. newUE-Identity and t304) and ServingCellConfigCommon;

-	Need codes or conditions specified for subfields according to IEs defined in clause 6 do not apply;

-	Based on the received AS configuration, the target node can indicate the delta (difference) to the UE's AS configuration (as included in CG-Config). The fields newUE-Identity and t304 included in ReconfigurationWithSync are not used for delta configuration purpose.

For the other fields in CG-Config and CG-ConfigInfo, the sender shall always signal the appropriate value even if same as indicated in the previous RRC INM, unless explicitly stated otherwise. As an exception to this general rule, the absence of the below listed fields means that the receiver maintains the values informed via the previous message. Note that every time there is a change in the configuration covered by a listed field, the MN shall include the field and it shall provide the full configuration provided by that field. Otherwise, if there is no change, the field can be omitted:

-	configRestrictInfo;

-	gapPurpose;

-	measGapConfig (for which delta signaling applies);

-	measGapConfigFR2 (for which delta signaling applies);

-	measResultCellListSFTD;

-	measResultSFTD-EUTRA;

-	sftdFrequencyList-EUTRA;

-	sftdFrequencyList-NR;

-	ue-CapabilityInfo;

-	servFrequenciesMN-NR;

-	scellFrequenciesSN-EUTRA;

-	scellFrequenciesSN-NR.

## 11.3	Inter-node RRC information element definitions

-

## 11.4	Inter-node RRC multiplicity and type constraint values

#### –	Multiplicity and type constraints definitions

-- ASN1START

-- TAG-NR-MULTIPLICITY-AND-CONSTRAINTS-START

maxMeasFreqsMN              INTEGER ::= 32  -- Maximum number of MN-configured measurement frequencies

maxMeasFreqsSN              INTEGER ::= 32  -- Maximum number of SN-configured measurement frequencies

maxMeasIdentitiesMN         INTEGER ::= 62  -- Maximum number of measurement identities that a UE can be configured with

maxCellPrep                 INTEGER ::= 32  -- Maximum number of cells prepared for handover

-- TAG-NR-MULTIPLICITY-AND-CONSTRAINTS-STOP

-- ASN1STOP

#### –	End of NR-InterNodeDefinitions

-- ASN1START

-- TAG-NR-INTER-NODE-DEFINITIONS-END-START

END

-- TAG-NR-INTER-NODE-DEFINITIONS-END-STOP

-- ASN1STOP

# 12	Processing delay requirements for RRC procedures

The UE performance requirements for RRC procedures are specified in the following tables. The performance requirement is expressed as the time in [ms] from the end of reception of the network -> UE message on the UE physical layer up to when the UE shall be ready for the reception of uplink grant for the UE -> network response message with no access delay other than the TTI-alignment (e.g. excluding delays caused by scheduling, the random access procedure or physical layer synchronisation). In case the RRC procedure triggers BWP switching, the RRC procedure delay is the value defined in the following table plus the BWP switching delay defined in TS 38.133 [14], clause 8.6.3.

Figure 12.1-1: Illustration of RRC procedure delay

Table 12.1-1: UE performance requirements for RRC procedures for UEs

| Procedure title: | Network -> UE | UE -> Network | Value [ms] | Notes |
| --- | --- | --- | --- | --- |
| RRC Connection Control Procedures |  |  |  |  |
| RRC reconfiguration | RRCReconfiguration | RRCReconfigurationComplete | 10 |  |
| RRC reconfiguration (scell addition/release) | RRCReconfiguration | RRCReconfigurationComplete | 16 |  |
| RRC reconfiguration (SCG establishment/ modification/ release) | RRCReconfiguration | RRCReconfigurationComplete | 16 |  |
| RRC setup | RRCSetup | RRCSetupComplete | 10 |  |
| RRC Release | RRCRelease |  | NA |  |
| RRC re-establishment | RRCReestablishment | RRCReestablishmentComplete | 10 |  |
| RRC resume | RRCResume | RRCResumeComplete | 6 or 10 | Value=6 applies for a UE supporting reduced CP latency for the case of RRCResume message only including MAC and PHY configuration, and no DRX, SPS, configured grant, CA or MIMO re-configuration will be triggered by this message. Further, the UL grant for transmission of RRCResumeComplete and the data is transmitted over common search space with DCI format 0_0. In this scenario, the RRC procedure delay [ms] can extend beyond the reception of the UL grant, up to 7 ms. For other cases, Value = 10 applies. |
| RRC resume (scell addition) | RRCResume | RRCResumeComplete | 16 |  |
| Initial AS security activation | SecurityModeCommand | SecurityModeComplete/SecurityModeFailure | 5 |  |
| Other procedures |  |  |  |  |
| UE assistance information |  | UEAssistanceInformation | NA |  |
| UE capability transfer | UECapabilityEnquiry | UECapabilityInformation | 80 |  |
| Counter check | CounterCheck | CounterCheckResponse | 5 |  |

Annex A (informative):	Guidelines, mainly on use of ASN.1

# A.1	Introduction

The following clauses contain guidelines for the specification of RRC protocol data units (PDUs) with ASN.1.

# A.2	Procedural specification

## A.2.1	General principles

The procedural specification provides an overall high level description regarding the UE behaviour in a particular scenario.

It should be noted that most of the UE behaviour associated with the reception of a particular field is covered by the applicable parts of the PDU specification. The procedural specification may also include specific details of the UE behaviour upon reception of a field, but typically this should be done only for cases that are not easy to capture in the PDU clause e.g. general actions, more complicated actions depending on the value of multiple fields.

Likewise, the procedural specification need not specify the UE requirements regarding the setting of fields within the messages that are sent to the network i.e. this may also be covered by the PDU specification.

## A.2.2	More detailed aspects

The following more detailed conventions should be used:

-	Bullets:

-	Capitals should be used in the same manner as in other parts of the procedural text i.e. in most cases no capital applies since the bullets are part of the sentence starting with 'The UE shall:'

-	All bullets, including the last one in a sub-clause, should end with a semi-colon i.e. an ';.

-	Conditions:

-	Whenever multiple conditions apply, a semi-colon should be used at the end of each conditions with the exception of the last one, i.e. as in 'if cond1, or cond2.

# A.3	PDU specification

## A.3.1	General principles

### A.3.1.1	ASN.1 sections

The RRC PDU contents are formally and completely described using abstract syntax notation (ASN.1), see X.680 [6], X.681 [7].

The complete ASN.1 code is divided into a number of ASN.1 sections in the specifications. In order to facilitate the extraction of the complete ASN.1 code from the specification, each ASN.1 section begins with the following:

-	a first text paragraph consisting entirely of an ASN.1 start tag, which consists of a double hyphen followed by a single space and the text string "ASN1START" (in all upper case letters);

-	a second text paragraph consisting entirely of a block start tag is included, which consists of a double hyphen followed by a single space and the text string "TAG-NAME-START" (in all upper case letters), where the "NAME" refers to the main name of the paragraph (in all upper-case letters).

Similarly, each ASN.1 section ends with the following:

-	a first text paragraph consisting entirely of a blockstop tag, which consists of a double hyphen followed by a single space and the text string "TAG-NAME-STOP" (in all upper-case letters), where the "NAME" refers to the main name of the paragraph (in all upper-case letters);

-	a second text paragraph consisting entirely of an ASN.1 stop tag, which consists of a double hyphen followed by a singlespace and the text "ASN1STOP" (in all upper case letters).

This results in the following tags:

-- ASN1START

-- TAG-NAME-START

-- TAG-NAME-STOP

-- ASN1STOP

The text paragraphs containing either of the start and stop tags should not contain any ASN.1 code significant for the complete description of the RRC PDU contents. The complete ASN.1 code may be extracted by copying all the text paragraphs between an ASN.1 start tag and the following ASN.1 stop tag in the order they appear, throughout the specification.

NOTE:	A typical procedure for extraction of the complete ASN.1 code consists of a first step where the entire RRC PDU contents description (ultimately the entire specification) is saved into a plain text (ASCII) file format, followed by a second step where the actual extraction takes place, based on the occurrence of the ASN.1 start and stop tags.

### A.3.1.2	ASN.1 identifier naming conventions

The naming of identifiers (i.e., the ASN.1 field and type identifiers) should be based on the following guidelines:

-	Message (PDU) identifiers should be ordinary mixed case without hyphenation. These identifiers, e.g., the RRCConnectionModificationCommand, should be used for reference in the procedure text. Abbreviations should be avoided in these identifiers and abbreviated forms of these identifiers should not be used.

-	Type identifiers other than PDU identifiers should be ordinary mixed case, with hyphenation used to set off acronyms only where an adjacent letter is a capital, e.g., EstablishmentCause, SelectedPLMN (not Selected-PLMN, since the "d" in "Selected" is lowercase), InitialUE-Identity and MeasSFN-SFN-TimeDifference.

-	Field identifiers shall start with a lowercase letter and use mixed case thereafter, e.g., establishmentCause. If a field identifier begins with an acronym (which would normally be in upper case), the entire acronym is lowercase (plmn-Identity, not pLMN-Identity). The acronym is set off with a hyphen (ue-Identity, not ueIdentity), in order to facilitate a consistent search pattern with corresponding type identifiers.

-	Identifiers should convey the meaning of the identifier and should avoid adding unnecessary postfixes (e.g. abstractions like 'Info') for the name.

-	Identifiers that are likely to be keywords of some language, especially widely used languages, such as C++ or Java, should be avoided to the extent possible.

-	Identifiers, other than PDU identifiers, longer than 25 characters should be avoided where possible. It is recommended to use abbreviations, which should be done in a consistent manner i.e. use 'Meas' instead of 'Measurement' for all occurrences. Examples of typical abbreviations are given in table A.3.1.2.1-1 below.

-	For future extension: When an extension is introduced a suffix is added to the identifier of the concerned ASN.1 field and/or type. A suffix of the form "-rX" is used, with X indicating the release, for ASN.1 fields or types introduced in a later release (i.e. a release later than the original/first release of the protocol) as well as for ASN.1 fields or types for which a revision is introduced in a later release replacing a previous version, e.g., Foo-r9 for the Rel-9 version of the ASN.1 type Foo. A suffix of the form "-rXb" is used for the first revision of a field that it appears in the same release (X) as the original version of the field, "-rXc" for a second intra-release revision and so on. A suffix of the form "-vXYZ" is used for ASN.1 fields or types that only are an extension of a corresponding earlier field or type (see sub-clause A.4), e.g., AnElement-v10b0 for the extension of the ASN.1 type AnElement introduced in version 10.11.0 of the specification. A number 0...9, 10, 11, etc. is used to represent the first part of the version number, indicating the release of the protocol. Lower case letters a, b, c, etc. are used to represent the second (and third) part of the version number if they are greater than 9. In the procedural specification, in field descriptions as well as in headings suffices are not used, unless there is a clear need to distinguish the extension from the original field.

-	More generally, in case there is a need to distinguish different variants of an ASN.1 field or IE, a suffix should be added at the end of the identifiers e.g. MeasObjectUTRA, ConfigCommon. When there is no particular need to distinguish the fields (e.g. because the field is included in different IEs), a common field identifier name may be used. This may be attractive e.g. in case the procedural specification is the same for the different variants.

-	It should be avoided to use field identifiers with the same name within the elements of a CHOICE, including using a CHOICE inside a SEQUENCE (to avoid certain compiler errors).

Table A.3.1.2-1: Examples of typical abbreviations used in ASN.1 identifiers

| Abbreviation | Abbreviated word |
| --- | --- |
| Config | Configuration |
| DL | Downlink |
| Ext | Extension |
| Freq | Frequency |
| Id | Identity |
| Ind | Indication |
| Meas | Measurement |
| MIB | MasterInformationBlock |
| Neigh | Neighbour(ing) |
| Param(s) | Parameter(s) |
| Phys | Physical |
| PCI | Physical Cell Id |
| Proc | Process |
| Reconfig | Reconfiguration |
| Reest | Re-establishment |
| Req | Request |
| Rx | Reception |
| Sched | Scheduling |
| SIB | SystemInformationBlock |
| Sync | Synchronisation |
| Thr | Threshold |
| Tx | Transmission |
| UL | Uplink |

NOTE:	The table A.3.1.2.1-1 is not exhaustive. Additional abbreviations may be used in ASN.1 identifiers when needed.

### A.3.1.3	Text references using ASN.1 identifiers

A text reference into the RRC PDU contents description from other parts of the specification is made using the ASN.1 field identifier of the referenced type. The ASN.1 field and type identifiers used in text references should be in the italic font style. The "do not check spelling and grammar" attribute in Word should be set. Quotation marks (i.e., "") should not be used around the ASN.1 field or type identifier.

A reference to an RRC PDU should be made using the corresponding ASN.1 field identifier followed by the word "message", e.g., a reference to the RRCRelease message.

A reference to a specific part of an RRC PDU, or to a specific part of any other ASN.1 type, should be made using the corresponding ASN.1 field identifier followed by the word "field", e.g., a reference to the prioritisedBitRate field in the example below.

-- /example/ ASN1START

LogicalChannelConfig ::=            SEQUENCE {

ul-SpecificParameters               SEQUENCE {

priority                            Priority,

prioritisedBitRate                  PrioritisedBitRate,

bucketSizeDuration                  BucketSizeDuration,

logicalChannelGroup                 INTEGER (0..3)

}       OPTIONAL

}

-- ASN1STOP

NOTE:	All the ASN.1 start tags in the ASN.1 sections, used as examples in this annex to the specification, are deliberately distorted, in order not to include them when the ASN.1 description of the RRC PDU contents is extracted from the specification.

A reference to a specific type of information element should be made using the corresponding ASN.1 type identifier preceded by the acronym "IE", e.g., a reference to the IE LogicalChannelConfig in the example above.

References to a specific type of information element should only be used when those are generic, i.e., without regard to the particular context wherein the specific type of information element is used. If the reference is related to a particular context, e.g., an RRC PDU type (message) wherein the information element is used, the corresponding field identifier in that context should be used in the text reference.

A reference to a specific value of an ASN.1 field should be made using the corresponding ASN.1 value without using quotation marks around the ASN.1 value, e.g., 'if the status field is set to value true'.

## A.3.2	High-level message structure

Within each logical channel type, the associated RRC PDU (message) types are alternatives within a CHOICE, as shown in the example below.

-- /example/ ASN1START

DL-DCCH-Message ::= SEQUENCE {

message                 DL-DCCH-MessageType

}

DL-DCCH-MessageType ::= CHOICE {

c1                      CHOICE {

dlInformationTransfer                   DLInformationTransfer,

handoverFromEUTRAPreparationRequest     HandoverFromEUTRAPreparationRequest,

mobilityFromEUTRACommand                MobilityFromEUTRACommand,

rrcConnectionReconfiguration            RRCConnectionReconfiguration,

rrcConnectionRelease                    RRCConnectionRelease,

securityModeCommand                     SecurityModeCommand,

ueCapabilityEnquiry                     UECapabilityEnquiry,

spare1 NULL

},

messageClassExtension   SEQUENCE {}

}

-- ASN1STOP

A nested two-level CHOICE structure is used, where the alternative PDU types are alternatives within the inner level c1 CHOICE.

Spare alternatives (i.e., spare1 in this case) may be included within the c1 CHOICE to facilitate future extension. The number of such spare alternatives should not extend the total number of alternatives beyond an integer-power-of-two number of alternatives (i.e., eight in this case).

Further extension of the number of alternative PDU types is facilitated using the messageClassExtension alternative in the outer level CHOICE.

## A.3.3	Message definition

Each PDU (message) type is specified in an ASN.1 section similar to the one shown in the example below.

-- /example/ ASN1START

RRCConnectionReconfiguration ::=    SEQUENCE {

rrc-TransactionIdentifier           RRC-TransactionIdentifier,

criticalExtensions                  CHOICE {

c1                                  CHOICE{

rrcConnectionReconfiguration-r8     RRCConnectionReconfiguration-r8-IEs,

spare3 NULL, spare2 NULL, spare1 NULL

},

criticalExtensionsFuture            SEQUENCE {}

}

}

RRCConnectionReconfiguration-r8-IEs ::= SEQUENCE {

-- Enter the IEs here.

...

}

-- ASN1STOP

Hooks for critical and non-critical extension should normally be included in the PDU type specification. How these hooks are used is further described in sub-clause A.4.

Critical extensions are characterised by a redefinition of the PDU contents and need to be governed by a mechanism for protocol version agreement between the encoder and the decoder of the PDU, such that the encoder is prevented from sending a critically extended version of the PDU type, which is not comprehended by the decoder.

Critical extension of a PDU type is facilitated by a two-level CHOICE structure, where the alternative PDU contents are alternatives within the inner level c1 CHOICE. Spare alternatives (i.e., spare3 down to spare1 in this case) may be included within the c1 CHOICE. The number of spare alternatives to be included in the original PDU specification should be decided case by case, based on the expected rate of critical extension in the future releases of the protocol.

Further critical extension, when the spare alternatives from the original specifications are used up, is facilitated using the criticalExtensionsFuture in the outer level CHOICE.

In PDU types where critical extension is not expected in the future releases of the protocol, the inner level c1 CHOICE and the spare alternatives may be excluded, as shown in the example below.

-- /example/ ASN1START

RRCConnectionReconfigurationComplete ::= SEQUENCE {

rrc-TransactionIdentifier           RRC-TransactionIdentifier,

criticalExtensions                  CHOICE {

rrcConnectionReconfigurationComplete-r8

RRCConnectionReconfigurationComplete-r8-IEs,

criticalExtensionsFuture            SEQUENCE {}

}

}

RRCConnectionReconfigurationComplete-r8-IEs ::= SEQUENCE {

-- Enter the fields here.

...

}

-- ASN1STOP

Non-critical extensions are characterised by the addition of new information to the original specification of the PDU type. If not comprehended, a non-critical extension may be skipped by the decoder, whilst the decoder is still able to complete the decoding of the comprehended parts of the PDU contents.

Non-critical extensions at locations other than the end of the message or other than at the end of a field contained in a BIT or OCTET STRING are facilitated by use of the ASN.1 extension marker "...". The original specification of a PDU type should normally include the extension marker at the end of the sequence of information elements contained.

Non-critical extensions at the end of the message or at the end of a field that is contained in a BIT or OCTET STRING may be facilitated by use of an empty sequence that is marked OPTIONAL e.g. as shown in the following example:

-- /example/ ASN1START

RRCMessage-r8-IEs ::=                   SEQUENCE {

field1                                  InformationElement1,

field2                                  InformationElement2,

nonCriticalExtension                    SEQUENCE {}                     OPTIONAL

}

-- ASN1STOP

The ASN.1 section specifying the contents of a PDU type may be followed by a field description table where a further description of, e.g., the semantic properties of the fields may be included. The general format of this table is shown in the example below. The field description table is absent in case there are no fields for which further description needs to be provided e.g. because the PDU does not include any fields, or because an IE is defined for each field while there is nothing specific regarding the use of this IE that needs to be specified.

%PDU-TypeIdentifier% field descriptions

%field identifier% Field description.

%field identifier% Field description.

The field description table has one column. The header row shall contain the ASN.1 type identifier of the PDU type.

The following rows are used to provide field descriptions. Each row shall include a first paragraph with a field identifier (in bold and italic font style) referring to the part of the PDU to which it applies. The following paragraphs at the same row may include (in regular font style), e.g., semantic description, references to other specifications and/or specification of value units, which are relevant for the particular part of the PDU.

The parts of the PDU contents that do not require a field description shall be omitted from the field description table.

## A.3.4	Information elements

Each IE (information element) type is specified in an ASN.1 section similar to the one shown in the example below.

-- /example/ ASN1START

PRACH-ConfigSIB ::=                 SEQUENCE {

rootSequenceIndex                   INTEGER (0..1023),

prach-ConfigInfo                    PRACH-ConfigInfo

}

PRACH-Config ::=                    SEQUENCE {

rootSequenceIndex                   INTEGER (0..1023),

prach-ConfigInfo                    PRACH-ConfigInfo                    OPTIONAL    -- Need N

}

PRACH-ConfigInfo ::=                SEQUENCE {

prach-ConfigIndex                   ENUMERATED {ffs},

highSpeedFlag                       ENUMERATED {ffs},

zeroCorrelationZoneConfig           ENUMERATED {ffs}

}

-- ASN1STOP

IEs should be introduced whenever there are multiple fields for which the same set of values apply. IEs may also be defined for other reasons e.g. to break down a ASN.1 definition in to smaller pieces.

A group of closely related IE type definitions, like the IEs PRACH-ConfigSIB and PRACH-Config in this example, are preferably placed together in a common ASN.1 section. The IE type identifiers should in this case have a common base, defined as the generic type identifier. It may be complemented by a suffix to distinguish the different variants. The "PRACH-Config" is the generic type identifier in this example, and the "SIB" suffix is added to distinguish the variant. The sub-clause heading and generic references to a group of closely related IEs defined in this way should use the generic type identifier.

The same principle should apply if a new version, or an extension version, of an existing IE is created for critical or non-critical extension of the protocol (see sub-clause A.4). The new version, or the extension version, of the IE is included in the same ASN.1 section defining the original. A suffix is added to the type identifier, using the naming conventions defined in sub-clause A.3.1.2, indicating the release or version of the where the new version, or extension version, was introduced.

Local IE type definitions, like the IE PRACH-ConfigInfo in the example above, may be included in the ASN.1 section and be referenced in the other IE types defined in the same ASN.1 section. The use of locally defined IE types should be encouraged, as a tool to break up large and complex IE type definitions. It can improve the readability of the code. There may also be a benefit for the software implementation of the protocol end-points, as these IE types are typically provided by the ASN.1 compiler as independent data elements, to be used in the software implementation.

An IE type defined in a local context, like the IE PRACH-ConfigInfo, should not be referenced directly from other ASN.1 sections in the RRC specification. An IE type which is referenced in more than one ASN.1 section should be defined in a separate sub-clause, with a separate heading and a separate ASN.1 section (possibly as one in a set of closely related IE types, like the IEs PRACH-ConfigSIB and PRACH-Config in the example above). Such IE types are also referred to as 'global IEs'.

NOTE:	Referring to an IE type, that is defined as a local IE type in the context of another ASN.1 section, does not generate an ASN.1 compilation error. Nevertheless, using a locally defined IE type in that way makes the IE type definition difficult to find, as it would not be visible at an outline level of the specification. It should be avoided.

The ASN.1 section specifying the contents of one or more IE types, like in the example above, may be followed by a field description table, where a further description of, e.g., the semantic properties of the fields of the information elements may be included. This table may be absent, similar as indicated in sub-clause A.3.3 for the specification of the PDU type. The general format of the field description table is the same as shown in sub-clause A.3.3 for the specification of the PDU type.

## A.3.5	Fields with optional presence

A field with optional presence may be declared with the keyword DEFAULT. It identifies a default value to be assumed, if the sender does not include a value for that field in the encoding:

-- /example/ ASN1START

PreambleInfo ::=                    SEQUENCE {

numberOfRA-Preambles                INTEGER (1..64)                     DEFAULT 1,

...

}

-- ASN1STOP

Alternatively, a field with optional presence may be declared with the keyword OPTIONAL. It identifies a field for which a value can be omitted. The omission carries semantics, which is different from any normal value of the field:

-- /example/ ASN1START

PRACH-Config ::=                    SEQUENCE {

rootSequenceIndex                   INTEGER (0..1023),

prach-ConfigInfo                    PRACH-ConfigInfo                    OPTIONAL    -- Need N

}

-- ASN1STOP

The semantics of an optionally present field, in the case it is omitted, should be indicated at the end of the paragraph including the keyword OPTIONAL, using a short comment text with a need code. The need code includes the keyword "Need", followed by one of the predefined semantics tags (S, M, N or R) defined in sub-clause 6.1. If the semantics tag S is used, the semantics of the absent field are further specified either in the field description table following the ASN.1 section, or in procedure text.

The addition of OPTIONAL keywords for capability groups is based on the following guideline. If there is more than one field in the lower level IE, then OPTIONAL keyword is added at the group level. If there is only one field in the lower level IE, OPTIONAL keyword is not added at the group level.

## A.3.6	Fields with conditional presence

A field with conditional presence is declared with the keyword OPTIONAL. In addition, a short comment text shall be included at the end of the paragraph including the keyword OPTIONAL. The comment text includes the keyword "Cond", followed by a condition tag associated with the field ("UL" in this example):

-- /example/ ASN1START

LogicalChannelConfig ::=            SEQUENCE {

ul-SpecificParameters               SEQUENCE {

priority                            INTEGER (0),

...

}       OPTIONAL                                                                    -- Cond UL

}

-- ASN1STOP

When conditionally present fields are included in an ASN.1 section, the field description table after the ASN.1 section shall be followed by a conditional presence table. The conditional presence table specifies the conditions for including the fields with conditional presence in the particular ASN.1 section.

| Conditional presence | Explanation |
| --- | --- |
| UL | Specification of the conditions for including the field associated with the condition tag = "UL". Semantics in case of optional presence under certain conditions may also be specified. |

The conditional presence table has two columns. The first column (heading: "Conditional presence") contains the condition tag (in italic font style), which links the fields with a condition tag in the ASN.1 section to an entry in the table. The second column (heading: "Explanation") contains a text specification of the conditions and requirements for the presence of the field. The second column may also include semantics, in case of an optional presence of the field, under certain conditions i.e. using the same predefined tags as defined for optional fields in A.3.5.

Conditional presence should primarily be used when presence of a field depends on the presence and/or value of other fields within the same message. If the presence of a field depends on whether another feature/function has been configured, while this function can be configured independently e.g. by another message and/or at another point in time, the relation is best reflected by means of a statement in the field description table.

If the ASN.1 section does not include any fields with conditional presence, the conditional presence table shall not be included.

Whenever a field is only applicable in specific cases e.g. TDD, use of conditional presence should be considered.

## A.3.7	Guidelines on use of lists with elements of SEQUENCE type

Where an information element has the form of a list (the SEQUENCE OF construct in ASN.1) with the type of the list elements being a SEQUENCE data type, an information element shall be defined for the list elements even if it would not otherwise be needed.

For example, a list of PLMN identities with reservation flags is defined as in the following example:

-- /example/ ASN1START

PLMN-IdentityInfoList ::=           SEQUENCE (SIZE (1..6)) OF PLMN-IdentityInfo

PLMN-IdentityInfo ::=               SEQUENCE {

plmn-Identity                       PLMN-Identity,

cellReservedForOperatorUse          ENUMERATED {reserved, notReserved}

}

-- ASN1STOP

rather than as in the following (bad) example, which may cause generated code to contain types with unpredictable names:

-- /bad example/ ASN1START

PLMN-IdentityList ::=                   SEQUENCE (SIZE (1..6)) OF SEQUENCE {

plmn-Identity                           PLMN-Identity,

cellReservedForOperatorUse              ENUMERATED {reserved, notReserved}

}

-- ASN1STOP

## A.3.8	Guidelines on use of parameterised SetupRelease type

The usage of the parameterised SetupRelease type is like a function call in programming languages where the element type parameter is passed as a parameter. The parameterised type only implies a textual change in abstract syntax where all references to the parameterised type are replaced by the compiler with the release/setup choice. Two examples of the usage are shown below:

-- /example/ ASN1START

RRCMessage-rX-IEs ::= SEQUENCE {

field-rX               SetupRelease { IE-rX }                   OPTIONAL,      --  Need M

...

}

RRCMessage-rX-IEs ::= SEQUENCE {

field-rX               SetupRelease { Element-rX }

}                                                                   OPTIONAL,       -- Need M

Element-rX ::= SEQUENCE {

field1-rX                  IE1-rX,

field2-rX                  IE2-rX                               OPTIONAL        -- Need N

}                                                                       OPTIONAL,   -- Need M

-- /example/ ASN1STOP

The SetupRelease is always be used with only named IEs, i.e. the example below is not allowed:

-- /example/ ASN1START

RRCMessage-rX-IEs ::= SEQUENCE {

field-rX       SetupRelease { SEQUENCE {   -- Unnamed SEQUENCEs are not allowed!

field1-rX                  IE1-rX,

field2-rX                  IE2-rX                         OPTIONAL        -- Need N

}

}                                                                     OPTIONAL,   -- Need M

}

-- /example/ ASN1STOP

If a field defined using the parameterized SetupRelease type requires procedural text, the field is referred to using the values defined for the type itself, namely, "setup" and "release". For example, procedural text for field-rX above could be as follows:

1> if field-rX is set to "setup":

2> do something;

1> else (field-rX is set to "release"):

2> release field-rX (if appropriate).

## A.3.9	Guidelines on use of ToAddModList and ToReleaseList

In order to benefit from delta signalling when modifying lists with many and/or large elements, so-called add/mod- and release- lists should be used. Instead of a single list containing all elements of the list, the ASN.1 provides two lists. One list is used to convey the actual elements that are to be added to the list or modified in the list. The second list conveys only the identities (IDs) of the list elements that are to be released from the list. In other words, the ASN.1 defines only means to signal modifications to a list maintained in the receiver (typically the UE). An example is provided below:

-- /example/ ASN1START

AnExampleIE ::=         SEQUENCE {

elementsToAddModList    SEQUENCE (SIZE (1..maxNrofElements)) OF Element                                     OPTIONAL,   --  Need N

elementsToReleaseList   SEQUENCE (SIZE (1..maxNrofElements)) OF ElementId                                   OPTIONAL,   --  Need N

...

}

Element ::=             SEQUENCE {

elementId               ElementId,

aField                  INTEG ER (0..16777215),

anotherField            OCTET STRING,

...

}

ElementId ::=           INTEGER (0..maxNrofElements-1)

maxNrofElements         INTEGER ::= 50

maxNrofElements-1       INTEGER ::= 49

-- /example/ ASN1STOP

As can be seen, the elements of the list must contain an identity (INTEGER) that identifies the elements unambiguously upon addition, modification and removal. It is recommended to define an IE for that identifier (here ElementId) so that it can be used both for a field inside the element as well as in the elementsToReleaseList.

Both lists should be made OPTIONAL and flagged as "Need N". The need code reflects that the UE does not maintain the received lists as such but rather updates its configuration using the information therein. In other words, it is not possible to provide via delta signalling an update to a previously signalled elementsToAddModList or elementsToReleaseList (which Need M would imply). The update is always in relation to the UE's internal configuration.

Note that the release of parent field also releases all of the child fields, regardless of whether they have been added via AddModList or as normal fields.

If no procedural text is provided for a set of ToAddModList and ToReleaseList, the following generic procedure applies:

The UE shall:

1>	for each ElementId in the elementsToReleaseList,:

2>	if the current UE configuration includes an Element with the given ElementId:

3>	release the Element from the current UE configuration;

1>	for each Element in the elementsToAddModList:

2>	if the current UE configuration includes an Element with the given ElementId:

3>	modify the configured Element in accordance with the received Element;

2>	else:

3>	add received Element to the UE configuration.

## A.3.10	Guidelines on use of of lists (without ToAddModList and ToReleaseList)

As per subclause 6.1.3, when using lists without the ToAddModList and ToReleaseList structure, the contents of the lists are always replaced. To illustrate this, an example is provided below:

-- /example/ ASN1START

-- TAG_EXAMPLE_LISTS_START

AnExampleIE ::=         SEQUENCE {

elementList             SEQUENCE (SIZE (1..maxNrofElements)) OF Element                                     OPTIONAL,   --  Need M

...,

[[

elementListExt-v2030    SEQUENCE (SIZE (1..maxNrofElementsExt)) OF Element                                  OPTIONAL,   --  Need M

]]

}

Element ::=         SEQUENCE {

useFeatureX         BOOLEAN,

aField              INTEGER (0..127)                                                                        OPTIONAL,   --  Need M

anotherField        INTEGER (0..127)                                                                        OPTIONAL,   --  Need R

...

}

maxNrofElements         INTEGER ::= 8

maxNrofElements-1       INTEGER ::= 7

maxNrofElementsExt      INTEGER ::= 8

maxNrofElementsExt-1    INTEGER ::= 7

-- TAG_EXAMPLE_LISTS_STOP

-- /example/ ASN1STOP

As can be seen, the elementList list itself uses Need M, but each list entry Element contains mandatory, Need M and Need R fields. If the list is first signalled to UE with 3 entries, and subsequently again with 2 entries, UE shall retain only the latter list, i.e. the list with 2 elements will completely replace the list with 3 elements. That also means that the field aField will be treated as if it was newly created, i.e. network must include it if it wishes UE to utilize the field even if it was previously signalled. This also implies that the Need M field (aField) will be treated in the same way as the Need R field (anotherField), i.e. delta signalling is not applied and the network has to signal the field to ensure UE does not release the value (which is why Need M should not normally be used in the entries of these lists).

# A.4	Extension of the PDU specifications

## A.4.1	General principles to ensure compatibility

It is essential that extension of the protocol does not affect interoperability i.e. it is essential that implementations based on different versions of the RRC protocol are able to interoperate. In particular, this requirement applies for the following kind of protocol extensions:

-	Introduction of new PDU types (i.e. these should not cause unexpected behaviour or damage).

-	Introduction of additional fields in an extensible PDUs (i.e. it should be possible to ignore uncomprehended extensions without affecting the handling of the other parts of the message).

-	Introduction of additional values of an extensible field of PDUs. If used, the behaviour upon reception of an uncomprehended value should be defined.

It should be noted that the PDU extension mechanism may depend on the logical channel used to transfer the message e.g. for some PDUs an implementation may be aware of the protocol version of the peer in which case selective ignoring of extensions may not be required.

The non-critical extension mechanism is the primary mechanism for introducing protocol extensions i.e. the critical extension mechanism is used merely when there is a need to introduce a 'clean' message version. Such a need appears when the last message version includes a large number of non-critical extensions, which results in issues like readability, overhead associated with the extension markers. The critical extension mechanism may also be considered when it is complicated to accommodate the extensions by means of non-critical extension mechanisms.

## A.4.2	Critical extension of messages and fields

The mechanisms to critically extend a message are defined in A.3.3. There are both "outer branch" and "inner branch" mechanisms available. The "outer branch" consists of a CHOICE having the name criticalExtensions, with two values, c1 and criticalExtensionsFuture. The criticalExtensionsFuture branch consists of an empty SEQUENCE, while the c1 branch contains the "inner branch" mechanism.

The "inner branch" structure is a CHOICE with values of the form "MessageName-rX-IEs" (e.g., "RRCConnectionReconfiguration-r8-IEs") or "spareX", with the spare values having type NULL. The "-rX-IEs" structures contain the complete structure of the message IEs for the appropriate release; i.e., the critical extension branch for the Rel-10 version of a message includes all Rel-8 and Rel-9 fields (that are not obviated in the later version), rather than containing only the additional Rel-10 fields.

The following guidelines may be used when deciding which mechanism to introduce for a particular message, i.e. only an 'outer branch', or an 'outer branch' in combination with an 'inner branch' including a certain number of spares:

-	For certain messages, e.g. initial uplink messages, messages transmitted on a broadcast channel, critical extension may not be applicable.

-	An outer branch may be sufficient for messages not including any fields.

-	The number of spares within inner branch should reflect the likelihood that the message will be critically extended in future releases (since each release with a critical extension for the message consumes one of the spare values). The estimation of the critical extension likelihood may be based on the number, size and changeability of the fields included in the message.

-	In messages where an inner branch extension mechanism is available, all spare values of the inner branch should be used before any critical extensions are added using the outer branch.

The following example illustrates the use of the critical extension mechanism by showing the ASN.1 of the original and of a later release

-- /example/ ASN1START                  -- Original release

RRCMessage ::=                          SEQUENCE {

rrc-TransactionIdentifier               RRC-TransactionIdentifier,

criticalExtensions                      CHOICE {

c1                                      CHOICE{

rrcMessage-r8                           RRCMessage-r8-IEs,

spare3 NULL, spare2 NULL, spare1 NULL

},

criticalExtensionsFuture                SEQUENCE {}

}

}

-- ASN1STOP

-- /example/ ASN1START                  -- Later release

RRCMessage ::=                          SEQUENCE {

rrc-TransactionIdentifier               RRC-TransactionIdentifier,

criticalExtensions                          CHOICE {

c1                                          CHOICE{

rrcMessage-r8                               RRCMessage-r8-IEs,

rrcMessage-r10                              RRCMessage-r10-IEs,

rrcMessage-r11                              RRCMessage-r11-IEs,

rrcMessage-r14                              RRCMessage-r14-IEs

},

later                                      CHOICE {

c2                                         CHOICE{

rrcMessage-r16                             RRCMessage-r16-IEs,

spare7 NULL, spare6 NULL, spare5 NULL, spare4 NULL,

spare3 NULL, spare2 NULL, spare1 NULL

},

criticalExtensionsFuture                SEQUENCE {}

}

}

}

-- ASN1STOP

It is important to note that critical extensions may also be used at the level of individual fields i.e. a field may be replaced by a critically extended version. When sending the extended version, the original version may also be included (e.g. original field is mandatory, E-UTRAN is unaware if UE supports the extended version). In such cases, a UE supporting both versions may be required to ignore the original field. The following example illustrates the use of the critical extension mechanism by showing the ASN.1 of the original and of a later release.

-- /example/ ASN1START                  -- Original release

RRCMessage ::=                          SEQUENCE {

rrc-TransactionIdentifier               RRC-TransactionIdentifier,

criticalExtensions                      CHOICE {

c1                                      CHOICE{

rrcMessage-r8                           RRCMessage-r8-IEs,

spare3 NULL, spare2 NULL, spare1 NULL

},

criticalExtensionsFuture            SEQUENCE {}

}

}

RRCMessage-rN-IEs ::= SEQUENCE {

field1-rN                           ENUMERATED {

value1, value2, value3, value4}     OPTIONAL,   -- Need N

field2-rN                           InformationElement2-rN                  OPTIONAL,   -- Need N

nonCriticalExtension                RRCConnectionReconfiguration-vMxy-IEs   OPTIONAL

}

RRCConnectionReconfiguration-vMxy-IEs ::= SEQUENCE {

field2-rM                                 InformationElement2-rM            OPTIONAL, -- Cond NoField2rN

nonCriticalExtension                      SEQUENCE {}                       OPTIONAL

}

-- ASN1STOP

| Conditional presence | Explanation |
| --- | --- |
| NoField2rN | The field is optionally present, need N, if field2-rN is absent. Otherwise the field is absent |

Finally, it is noted that a critical extension may be introduced in the same release as the one in which the original field was introduced e.g. to correct an essential ASN.1 error. In such cases a UE capability may be introduced, to assist the network in deciding whether or not to use the critical extension.

## A.4.3	Non-critical extension of messages

### A.4.3.1	General principles

The mechanisms to extend a message in a non-critical manner are defined in A.3.3. W.r.t. the use of extension markers, the following additional guidelines apply:

-	When further non-critical extensions are added to a message that has been critically extended, the inclusion of these non-critical extensions in earlier critical branches of the message should be avoided when possible.

-	The extension marker ("...") is the primary non-critical extension mechanism that is used but empty sequences may be used if length determinant is not required. Examples of cases where a length determinant is not required:

-	at the end of a message;

-	at the end of a structure contained in a BIT STRING or OCTET STRING.

-	When an extension marker is available, non-critical extensions are preferably placed at the location (e.g. the IE) where the concerned parameter belongs from a logical/ functional perspective (referred to as the 'default extension location').

-	It is desirable to aggregate extensions of the same release or version of the specification into a group, which should be placed at the lowest possible level.

-	In specific cases it may be preferable to place extensions elsewhere (referred to as the 'actual extension location') e.g. when it is possible to aggregate several extensions in a group. In such a case, the group should be placed at the lowest suitable level in the message. <TBD: ref to separate example>

-	In case placement at the default extension location affects earlier critical branches of the message, locating the extension at a following higher level in the message should be considered.

-	In case an extension is not placed at the default extension location, an IE should be defined. The IE's ASN.1 definition should be placed in the same ASN.1 section as the default extension location. In case there are intermediate levels in-between the actual and the default extension location, an IE may be defined for each level. Intermediate levels are primarily introduced for readability and overview. Hence intermediate levels need not always be introduced e.g. they may not be needed when the default and the actual extension location are within the same ASN.1 section. <TBD: ref to separate example>

### A.4.3.2	Further guidelines

Further to the general principles defined in the previous section, the following additional guidelines apply regarding the use of extension markers:

-	Extension markers within SEQUENCE:

-	Extension markers are primarily, but not exclusively, introduced at the higher nesting levels.

-	Extension markers are introduced for a SEQUENCE comprising several fields as well as for information elements whose extension would result in complex structures without it (e.g. re-introducing another list).

-	Extension markers are introduced to make it possible to maintain important information structures e.g. parameters relevant for one particular RAT.

-	Extension markers are also used for size critical messages (i.e. messages on BCCH, BR-BCCH, PCCH and CCCH), although introduced somewhat more carefully.

-	The extension fields introduced (or frozen) in a specific version of the specification are grouped together using double brackets.

-	Extension markers within ENUMERATED:

-	Spare values may be used until the number of values reaches the next power of 2, while the extension marker caters for extension beyond that limit, given that the use of spare values in a later Release is possible without any error cases.

-	A suffix of the form "vXYZ" is used for the identifier of each new value, e.g. "value-vXYZ".

-	Extension markers within CHOICE:

-	Extension markers are introduced when extension is foreseen and when comprehension is not required by the receiver i.e. behaviour is defined for the case where the receiver cannot comprehend the extended value (e.g. ignoring an optional CHOICE field). It should be noted that defining the behaviour of a receiver upon receiving a not comprehended choice value is not required if the sender is aware whether or not the receiver supports the extended value.

-	A suffix of the form "vXYZ" is used for the identifier of each new choice value, e.g. "choice-vXYZ".

Non-critical extensions at the end of a message/ of a field contained in an OCTET or BIT STRING:

-	When a nonCriticalExtension is actually used, a "Need" code should not be provided for the field, which always is a group including at least one extension and a field facilitating further possible extensions. For simplicity, it is recommended not to provide a "Need" code when the field is not actually used either.

Further, more general, guidelines:

-	In case a need code is not provided for a group, a "Need" code is provided for all individual extension fields within the group i.e. including for fields that are not marked as OPTIONAL. The latter is to clarify the action upon absence of the whole group.

### A.4.3.3	Typical example of evolution of IE with local extensions

The following example illustrates the use of the extension marker for a number of elementary cases (sequence, enumerated, choice). The example also illustrates how the IE may be revised in case the critical extension mechanism is used.

NOTE	In case there is a need to support further extensions of release n while the ASN.1 of release (n+1) has been frozen, without requiring the release n receiver to support decoding of release (n+1) extensions, more advanced mechanisms are needed e.g. including multiple extension markers.

-- /example/ ASN1START

InformationElement1 ::=             SEQUENCE {

field1                              ENUMERATED {

value1, value2, value3, value4-v880,

..., value5-v960 },

field2                              CHOICE {

field2a                             BOOLEAN,

field2b                             InformationElement2b,

...,

field2c-v960                        InformationElement2c-r9

},

...,

[[

field3-r9                           InformationElement3-r9      OPTIONAL        -- Need R

]],

[[

field3-v9a0                         InformationElement3-v9a0    OPTIONAL,       -- Need R

field4-r9                           InformationElement4         OPTIONAL        -- Need R

]]

}

InformationElement1-r10 ::=         SEQUENCE {

field1                              ENUMERATED {

value1, value2, value3, value4-v880,

value5-v960, value6-v1170, spare2, spare1, ... },

field2                              CHOICE {

field2a                             BOOLEAN,

field2b                             InformationElement2b,

field2c-v960                        InformationElement2c-r9,

...,

field2d-v12b0                       INTEGER (0..63)

},

field3-r9                           InformationElement3-r10         OPTIONAL,   -- Need R

field4-r9                           InformationElement4             OPTIONAL,   -- Need R

field5-r10                          BOOLEAN,

field6-r10                          InformationElement6-r10         OPTIONAL,   -- Need R

...,

[[

field3-v1170                        InformationElement3-v1170       OPTIONAL    -- Need R

]]

}

-- ASN1STOP

Some remarks regarding the extensions of InformationElement1 as shown in the above example:

–	The InformationElement1 is initially extended with a number of non-critical extensions. In release 10 however, a critical extension is introduced for the message using this IE. Consequently, a new version of the IE InformationElement1 (i.e. InformationElement1-r10) is defined in which the earlier non-critical extensions are incorporated by means of a revision of the original field.

–	The value4-v880 is replacing a spare value defined in the original protocol version for field1. Likewise value6-v1170 replaces spare3 that was originally defined in the r10 version of field1.

–	Within the critically extended release 10 version of InformationElement1, the names of the original fields/IEs are not changed, unless there is a real need to distinguish them from other fields/IEs. E.g. the field1 and InformationElement4 were defined in the original protocol version (release 8) and hence not tagged. Moreover, the field3-r9 is introduced in release 9 and not re-tagged; although, the InformationElement3 is also critically extended and therefore tagged InformationElement3-r10 in the release 10 version of InformationElement1.

### A.4.3.4	Typical examples of non critical extension at the end of a message

The following example illustrates the use of non-critical extensions at the end of the message or at the end of a field that is contained in a BIT or OCTET STRING i.e. when an empty sequence is used.

-- /example/ ASN1START

RRCMessage-r8-IEs ::=           SEQUENCE {

field1                          InformationElement1,

field2                          InformationElement2,

field3                          InformationElement3                 OPTIONAL,   -- Need N

nonCriticalExtension            RRCMessage-v860-IEs                 OPTIONAL

}

RRCMessage-v860-IEs ::=         SEQUENCE {

field4-v860                     InformationElement4                 OPTIONAL,   -- Need S

field5-v860                     BOOLEAN                             OPTIONAL,   -- Cond C54

nonCriticalExtension            RRCMessage-v940-IEs                 OPTIONAL

}

RRCMessage-v940-IEs ::=         SEQUENCE {

field6-v940                     InformationElement6-r9              OPTIONAL,   -- Need R

nonCriticalExtensions           SEQUENCE {}                         OPTIONAL

}

-- ASN1STOP

Some remarks regarding the extensions shown in the above example:

–	The InformationElement4 is introduced in the original version of the protocol (release 8) and hence no suffix is used.

### A.4.3.5	Examples of non-critical extensions not placed at the default extension location

The following example illustrates the use of non-critical extensions in case an extension is not placed at the default extension location.

#### –	ParentIE-WithEM

The IE ParentIE-WithEMis an example of a high level IE including the extension marker (EM). The root encoding of this IE includes two lower level IEs ChildIE1-WithoutEM and ChildIE2-WithoutEM which not include the extension marker. Consequently, non-critical extensions of the Child-IEs have to be included at the level of the Parent-IE.

The example illustrates how the two extension IEs ChildIE1-WithoutEM-vNx0 and ChildIE2-WithoutEM-vNx0 (both in release N) are used to connect non-critical extensions with a default extension location in the lower level IEs to the actual extension location in this IE.

ParentIE-WithEM information element

-- /example/ ASN1START

ParentIE-WithEM ::=                 SEQUENCE {

-- Root encoding, including:

childIE1-WithoutEM                  ChildIE1-WithoutEM              OPTIONAL,       -- Need N

childIE2-WithoutEM                  ChildIE2-WithoutEM              OPTIONAL,       -- Need N

...,

[[

childIE1-WithoutEM-vNx0             ChildIE1-WithoutEM-vNx0     OPTIONAL,       -- Need N

childIE2-WithoutEM-vNx0             ChildIE2-WithoutEM-vNx0     OPTIONAL        -- Need N

]]

}

-- ASN1STOP

Some remarks regarding the extensions shown in the above example:

–	The fields childIEx-WithoutEM-vNx0 may not really need to be optional (depends on what is defined at the next lower level).

–	In general, especially when there are several nesting levels, fields should be marked as optional only when there is a clear reason.

#### –	ChildIE1-WithoutEM

The IE ChildIE1-WithoutEM is an example of a lower level IE, used to control certain radio configurations including a configurable feature which can be setup or released using the local IE ChIE1-ConfigurableFeature. The example illustrates how the new field chIE1-NewField is added in release N to the configuration of the configurable feature. The example is based on the following assumptions:

–	When initially configuring as well as when modifying the new field, the original fields of the configurable feature have to be provided also i.e. as if the extended ones were present within the setup branch of this feature.

–	When the configurable feature is released, the new field should be released also.

–	When omitting the original fields of the configurable feature the UE continues using the existing values (which is used to optimise the signalling for features that typically continue unchanged upon handover).

–	When omitting the new field of the configurable feature the UE releases the existing values and discontinues the associated functionality (which may be used to support release of unsupported functionality upon handover to an eNB supporting an earlier protocol version).

The above assumptions, which affect the use of conditions and need codes, may not always apply. Hence, the example should not be re-used blindly.

ChildIE1-WithoutEM information element

-- /example/ ASN1START

ChildIE1-WithoutEM ::=              SEQUENCE {

-- Root encoding, including:

chIE1-ConfigurableFeature           ChIE1-ConfigurableFeature       OPTIONAL        -- Need N

}

ChildIE1-WithoutEM-vNx0 ::=     SEQUENCE {

chIE1-ConfigurableFeature-vNx0      ChIE1-ConfigurableFeature-vNx0  OPTIONAL    -- Cond ConfigF

}

ChIE1-ConfigurableFeature ::=       CHOICE {

release                             NULL,

setup                               SEQUENCE {

-- Root encoding

}

}

ChIE1-ConfigurableFeature-vNx0 ::=  SEQUENCE {

chIE1-NewField-rN                   INTEGER (0..31)

}

-- ASN1STOP

| Conditional presence | Explanation |
| --- | --- |
| ConfigF | The field is optional present, need R, in case of chIE1-ConfigurableFeature is included and set to "setup"; otherwise the field is absent and the UE shall delete any existing value for this field. |

#### –	ChildIE2-WithoutEM

The IE ChildIE2-WithoutEM is an example of a lower level IE, typically used to control certain radio configurations. The example illustrates how the new field chIE1-NewField is added in release N to the configuration of the configurable feature.

ChildIE2-WithoutEM information element

-- /example/ ASN1START

ChildIE2-WithoutEM ::=              CHOICE {

release                             NULL,

setup                               SEQUENCE {

-- Root encoding

}

}

ChildIE2-WithoutEM-vNx0 ::=         SEQUENCE {

chIE2-NewField-rN                   INTEGER (0..31)                 OPTIONAL    -- Cond ConfigF

}

-- ASN1STOP

| Conditional presence | Explanation |
| --- | --- |
| ConfigF | The field is optional present, need R, in case of chIE2-ConfigurableFeature is included and set to "setup"; otherwise the field is absent and the UE shall delete any existing value for this field. |

# A.5	Guidelines regarding inclusion of transaction identifiers in RRC messages

The following rules provide guidance on which messages should include a Transaction identifier

1:	DL messages on CCCH that move UE to RRC-Idle should not include the RRC transaction identifier.

2:	All network initiated DL messages by default should include the RRC transaction identifier.

3:	All UL messages that are direct response to a DL message with an RRC Transaction identifier should include the RRC Transaction identifier.

4:	All UL messages that require a direct DL response message should include an RRC transaction identifier.

5:	All UL messages that are not in response to a DL message nor require a corresponding response from the network should not include the RRC Transaction identifier.

# A.6	Guidelines regarding use of need codes

The following rule provides guidance for determining need codes for optional downlink fields:

- if the field needs to be stored by the UE (i.e. maintained) when absent:

- use Need M (=Maintain);

- else, if the field needs to be released by the UE when absent:

- use Need R (=Release);

- else, if UE shall take no action when the field is absent (i.e. UE does not even need to maintain any existing value of the field):

- use Need N (=None);

- else (UE behaviour upon absence does not fit any of the above conditions):

- use Need S (=Specified);

- specify the UE behaviour upon absence of the field in the procedural text or in the field description table.

# A.7	Guidelines regarding use of conditions

Conditions are primarily used to specify network restrictions, for which the following types can be distinguished:

-	Message Contents related constraints e.g. that a field B is mandatory present if the same message includes field A and when it is set value X.

-	Configuration Constraints e.g. that a field D can only be signalled if field C is configured and set to value Y. (i.e. regardless of whether field C is present in the same message or previously configured).

The use of these conditions is illustrated by an example.

-- /example/ ASN1START

RRCMessage-IEs ::= SEQUENCE {

fieldA                          FieldA                  OPTIONAL,   -- Need M

fieldB                          FieldB                  OPTIONAL,   -- Cond FieldAsetToX

fieldC                          FieldC                  OPTIONAL,   -- Need M

fieldD                          FieldD                  OPTIONAL,   -- Cond FieldCsetToY

nonCriticalExtension            SEQUENCE {}             OPTIONAL

}

-- /example/ ASN1STOP

| Conditional presence | Explanation |
| --- | --- |
| FieldAsetToX | The field is mandatory present if fieldA is included and set to valueX. Otherwise the field is optionally present, need R. |
| FieldCsetToY | The field is optionally present, need M, if fieldC is configured and set to valueY. Otherwise the field is absent and the UE does not maintain the value |

# A.8	Miscellaneous

The following miscellaneous convention should be used:

-	UE capabilities: TS 38.306 [26] specifies that the network should in general respect the UE's capabilities. Hence there is no need to include statement clarifying that the network, when setting the value of a certain configuration field, shall respect the related UE capabilities unless there is a particular need e.g. particularly complicated cases.

Annex B (informative):	RRC Information

# B.1	Protection of RRC messages

The following list provides information which messages can be sent (unprotected) prior to AS security activation and which messages can be sent unprotected after AS security activation. Those messages indicated "-" in "P" column should never be sent unprotected by gNB or UE. Further requirements are defined in the procedural text.

P…Messages that can be sent (unprotected) prior to AS security activation

A – I…Messages that can be sent without integrity protection after AS security activation

A – C…Messages that can be sent unciphered after AS security activation

NA… Message can never be sent after AS security activation

| Message | P | A-I | A-C | Comment |
| --- | --- | --- | --- | --- |
| CounterCheck | - | - | - |  |
| CounterCheckResponse | - | - | - |  |
| DLInformationTransfer | + | - | - |  |
| FailureInformation | - | - | - |  |
| LocationMeasurementIndication | - | - | - |  |
| MIB | + | + | + |  |
| MeasurementReport | - | - | - | Measurement configuration may be sent prior to AS security activation. But: In order to protect privacy of UEs, MeasurementReport is only sent from the UE after successful AS security activation. |
| MobilityFromNRCommand | - | - | - |  |
| Paging | + | + | + |  |
| RRCReconfiguration | + | - | - | The message shall not be sent unprotected before AS security activation if it is used to perform handover or to establish SRB2 and DRBs. |
| RRCReconfigurationComplete | + | - | - | Unprotected, if sent as response to RRCReconfiguration which was sent before AS security activation. |
| RRCReestablishment | - | - | + | Integrity protection applied, but no ciphering. |
| RRCReestablishmentComplete | - | - | - |  |
| RRCReestablishmentRequest | - | - | + | This message is not protected by PDCP operation. However, a shortMAC-I is included. |
| RRCReject | + | + | + | Justification for A-I and A-C: the message can be sent in SRB0 in RRC_INACTIVE state, after the AS security is activated. |
| RRCRelease | + | - | - | Justification for P: If the RRC connection only for signalling not requiring DRBs or ciphered messages, or the signalling connection has to be released prematurely, this message is sent as unprotected. RRCRelease message sent before AS security activation cannot include deprioritisationReq, suspendConfig, redirectedCarrierInfo, cellReselectionPriorities information fields. |
| RRCResume | - | - | - |  |
| RRCResumeComplete | - | - | - |  |
| RRCResumeRequest | - | - | + | This message is not protected by PDCP operation. However, a resumeMAC-I is included. |
| RRCResumeRequest1 | - | - | + | This message is not protected by PDCP operation. However, a resumeMAC-I is included. |
| RRCSetup | + | + | + | Justification for A-I and A-C: the message can be sent in SRB0 in RRC_INACTIVE or RRC_CONNECTED states, after the AS security is activated. |
| RRCSetupComplete | + | NA | NA |  |
| RRCSetupRequest | + | NA | NA |  |
| RRCSystemInfoRequest | + | + | + | Justification for A-I and A-C: the message can be sent in SRB0 in RRC_INACTIVE state, after the AS security is activated. |
| SIB1 | + | + | + |  |
| SCGFailureInformation | - | - | - |  |
| SCGFailureInformationEUTRA | - | - | - |  |
| SecurityModeCommand | + | NA | NA | Integrity protection applied, but no ciphering (integrity verification done after the message received by RRC). |
| SecurityModeComplete | - | - | + | The message is sent after AS security activation. Integrity protection applied, but no ciphering. Ciphering is applied after completing the procedure. |
| SecurityModeFailure | + | NA | NA | Neither integrity protection nor ciphering applied. |
| SystemInformation | + | + | + |  |
| UEAssistanceInformation | - | - | - |  |
| UECapabilityEnquiry | + | - | - | The network should retrieve UE capabilities only after AS security activation. |
| UECapabilityInformation | + | - | - |  |
| ULInformationTransfer | + | - | - |  |
| ULInformationTransferMRDC | - | - | - |  |

# B.2	Description of BWP configuration options

There are two possible ways to configure BWP#0 (i.e. the initial BWP) for a UE:

1)	Configure BWP-DownlinkCommon and BWP-UplinkCommon in ServingCellConfigCommon, but do not configure dedicated configurations in BWP-DownlinkDedicated or BWP-UplinkDedicated in ServingCellConfig.

2)	Configure both BWP-DownlinkCommon and BWP-UplinkCommon in ServingCellConfigCommon and configure dedicated configurations in at least one of BWP-DownlinkDedicated or BWP-UplinkDedicated in ServingCellConfig.

The same way of configuration is used for UL BWP#0 and DL BWP#0 if both are configured.

With the first option (illustrated by figure B2-1 below), the BWP#0 is not considered to be an RRC-configured BWP, i.e. UE only supporting one BWP can still be configured with BWP#1 in addition to BWP#0 when using this configuration. The BWP#0 can still be used even if it does not have the dedicated configuration, albeit in a more limited manner since only the SIB1-defined configurations are available. For example, only DCI format 1_0 can be used with BWP#0 without dedicated configuration, so changing to another BWP requires RRCReconfiguration since DCI format 1_0 doesn't support DCI-based switching.

Figure B2-1: BWP#0 configuration without dedicated configuration

With the second option (illustrated by figure B2-2 below), the BWP#0 is considered to be an RRC-configured BWP, i.e. UE only supporting one BWP cannot be configured with BWP#1 in addition to BWP#0 when using this configuration. However, UE supporting more than one BWP can still switch to and from BWP#0 e.g. via DCI normally, and there are no explicit limitations to using the BWP#0 (compared to the first option).

Figure B2-2: BWP#0 configuration with dedicated configuration

For BWP#0, the BWP-DownlinkCommon and BWP-UplinkCommon in ServingCellConfigCommon should match the parameters configured by MIB and SIB1 (if provided) in the corresponding serving cell.

Annex C (informative):
Change history

| Change history |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 04/2017 | RAN2#97bis | R2-1703395 |  |  |  |  | 0.0.1 |
| 04/2017 | RAN2#97bis | R2-1703922 |  |  |  |  | 0.0.2 |
| 05/2017 | RAN2#98 | R2-1705815 |  |  |  |  | 0.0.3 |
| 06/2017 | RAN2#NR2 | R2-1707187 |  |  |  |  | 0.0.4 |
| 08/2017 | RAN2#99 | R2-1708468 |  |  |  |  | 0.0.5 |
| 09/2017 | RAN2#99bis | R2-1710557 |  |  |  |  | 0.1.0 |
| 11/2017 | RAN2#100 | R2-1713629 |  |  |  |  | 0.2.0 |
| 11/2017 | RAN2#100 | R2-1714126 |  |  |  |  | 0.3.0 |
| 12/2017 | RAN2#100 | R2-1714259 |  |  |  |  | 0.4.0 |
| 12/2017 | RP#78 | RP-172570 |  |  |  | Submitted for Approval in RAN#78 | 1.0.0 |
| 12/2017 | RP#78 |  |  |  |  | Upgraded to Rel-15 (MCC) | 15.0.0 |
| 03/2018 | RP#79 | RP-180479 | 0008 | 1 | F | Corrections for EN-DC (Note: the clause numbering between 15.0.0 and 15.1.0 has changed in some cases). | 15.1.0 |
| 06/2018 | RP-80 | RP-181326 | 0042 | 7 | F | Miscellaneous EN-DC corrections | 15.2.0 |
|  | RP-80 |  |  |  |  | Correction: Duplicate Foreword clause removed & ASN.1 clauses touched up | 15.2.1 |
| 09/2018 | RP-81 | RP-181942 | 0100 | 4 | F | Introduction of SA | 15.3.0 |
| 12/2018 | RP-82 | RP-182656 | 0179 | 3 | F | Handling of Resume Failure | 15.4.0 |
|  | RP-82 | RP-182651 | 0187 | 1 | F | Clarification on the presence of ra-ResponseWindow | 15.4.0 |
|  | RP-82 | RP-182656 | 0188 | 3 | F | Addition of RAN specific Access Category | 15.4.0 |
|  | RP-82 | RP-182653 | 0199 | 2 | F | CR for TS38.331 on MIB | 15.4.0 |
|  | RP-82 | RP-182653 | 0200 | 1 | F | CR for TS38.331 on PDCCH-ConfigSIB | 15.4.0 |
|  | RP-82 | RP-182661 | 0202 | 2 | F | Handling Cell Reselection during SI Request | 15.4.0 |
|  | RP-82 | RP-182649 | 0213 | 2 | F | Corrections on security field descriptions | 15.4.0 |
|  | RP-82 | RP-182649 | 0216 | 2 | F | Remain issue for T302 | 15.4.0 |
|  | RP-82 | RP-182649 | 0219 | 1 | F | [C204] Handling of timer T380 | 15.4.0 |
|  | RP-82 | RP-182655 | 0229 | 2 | F | Clarification on configured grant timer in 38.331 | 15.4.0 |
|  | RP-82 | RP-182663 | 0232 | 2 | F | CR for ServingCellConfigCommon in 38.331 | 15.4.0 |
|  | RP-82 | RP-182659 | 0234 | 3 | F | Introduction of cell level rate matching parameters in ServingCellConfig | 15.4.0 |
|  | RP-82 | RP-182650 | 0235 | 2 | F | CR for introducing PSCell frequency in CG-Config | 15.4.0 |
|  | RP-82 | RP-182650 | 0236 | 2 | F | CR for security handling for eLTE in 38.331 | 15.4.0 |
|  | RP-82 | RP-182650 | 0237 | 1 | F | Handling on simultaneously triggered NAS&AS events (I770) | 15.4.0 |
|  | RP-82 | RP-182650 | 0238 | 2 | F | Handling on security keys for resume procedure (I774) | 15.4.0 |
|  | RP-82 | RP-182664 | 0239 | 5 | F | RIL I556, I557, I558 on RB handling when resuming | 15.4.0 |
|  | RP-82 | RP-182650 | 0242 | 2 | F | Security for RRC connection release | 15.4.0 |
|  | RP-82 | RP-182650 | 0243 | 4 | F | Corrections on reestablishment and security procedures | 15.4.0 |
|  | RP-82 | RP-182650 | 0244 | 1 | F | RIL I118 on release case to upper layers for CN paging for a UE in RRC_INACTIVE | 15.4.0 |
|  | RP-82 | RP-182650 | 0246 | 2 | F | CR on SI request procedure in TS38.331 | 15.4.0 |
|  | RP-82 | RP-182650 | 0248 | 2 | F | CR to 38331 on ul-DataSplitThreshold for SRB | 15.4.0 |
|  | RP-82 | RP-182652 | 0249 | 2 | F | Clarification of guami-Type | 15.4.0 |
|  | RP-82 | RP-182652 | 0252 | 1 | F | CR to 38.331 on Protection of RRC messages Table | 15.4.0 |
|  | RP-82 | RP-182663 | 0254 | 2 | F | Access barring check after handover | 15.4.0 |
|  | RP-82 | RP-182663 | 0259 | 3 | F | Stop of T390 and related UE actions | 15.4.0 |
|  | RP-82 | RP-182657 | 0260 | 4 | F | Corrections for handover between NR and E-UTRA | 15.4.0 |
|  | RP-82 | RP-182738 | 0267 | 3 | F | CR on ssb-ToMeasure in MeasurementTimingConfiguration | 15.4.0 |
|  | RP-82 | RP-182659 | 0269 | 3 | F | Clarification of the applicability of 38.331 to EN-DC | 15.4.0 |
|  | RP-82 | RP-182654 | 0270 | 3 | F | Clarification on the smtc signalled for intra-NR handover, PSCell change or SCell addition | 15.4.0 |
|  | RP-82 | RP-182654 | 0273 | 3 | F | CR on fallback to the setup procedure | 15.4.0 |
|  | RP-82 | RP-182654 | 0275 | 1 | F | Correction on cell sorting for periodical measurement reporting | 15.4.0 |
|  | RP-82 | RP-182660 | 0277 | 2 | F | Measurement related actions upon re-establishment | 15.4.0 |
|  | RP-82 | RP-182654 | 0278 | 1 | F | CR on threshold description for cell quality derivation | 15.4.0 |
|  | RP-82 | RP-182654 | 0282 | 1 | F | CR to avoid unnecessary L3 filtered beam measurements | 15.4.0 |
|  | RP-82 | RP-182660 | 0283 | 2 | F | CR on CGI reporting | 15.4.0 |
|  | RP-82 | RP-182660 | 0291 | 3 | F | Additional UE capabilities for NR standalone | 15.4.0 |
|  | RP-82 | RP-182667 | 0294 | 4 | F | NR RRC Processing Time | 15.4.0 |
|  | RP-82 | RP-182812 | 0295 | 5 | F | Update of L1/RF capabilities | 15.4.0 |
|  | RP-82 | RP-182651 | 0296 | 2 | F | UE configuration on re-establishment procedure | 15.4.0 |
|  | RP-82 | RP-182651 | 0298 | 2 | F | SIB size limitation [M201] | 15.4.0 |
|  | RP-82 | RP-182651 | 0299 | 2 | F | Correction on SRS-TPC-CommandConfig | 15.4.0 |
|  | RP-82 | RP-182651 | 0302 | 2 | F | Clarification on counter check procedure | 15.4.0 |
|  | RP-82 | RP-182666 | 0307 | 4 | F | CR on the Clarification for the Support of the Delay Budget Report in NR | 15.4.0 |
|  | RP-82 | RP-182666 | 0320 | 3 | F | ssb-PositionsInBurst correction | 15.4.0 |
|  | RP-82 | RP-182666 | 0325 | 3 | F | Barring behaviour when SIB1 reception fails | 15.4.0 |
|  | RP-82 | RP-182666 | 0329 | 5 | F | System Information Storing and Validity Clarifications and Corrections | 15.4.0 |
|  | RP-82 | RP-182666 | 0330 | 3 | F | SIBs required before initiating connection | 15.4.0 |
|  | RP-82 | RP-182652 | 0333 | 1 | F | On contens of measObjectEUTRA | 15.4.0 |
|  | RP-82 | RP-182654 | 0335 | 2 | F | A3 and A5 corrections – neighbouring cell definition | 15.4.0 |
|  | RP-82 | RP-182650 | 0339 | 2 | F | SI reception in RRC Connected mode (RIL#II611) | 15.4.0 |
|  | RP-82 | RP-182650 | 0340 | 2 | F | Miscellaneous corrections on SI procedures | 15.4.0 |
|  | RP-82 | RP-182652 | 0342 | 1 | F | On RRM measurements related procedual text corrections | 15.4.0 |
|  | RP-82 | RP-182651 | 0344 | 1 | F | Clarification for absense of nr-NS-PmaxList IE | 15.4.0 |
|  | RP-82 | RP-182652 | 0345 | 2 | F | Clarification on paging in connected mode | 15.4.0 |
|  | RP-82 | RP-182651 | 0350 | 2 | F | ASN.1 correction to fr-InfoListSCG in CG-Config | 15.4.0 |
|  | RP-82 | RP-182661 | 0355 | 2 | F | Update of L2 capability parameters | 15.4.0 |
|  | RP-82 | RP-182651 | 0364 | 1 | F | Procedures for full config at RRCResume | 15.4.0 |
|  | RP-82 | RP-182651 | 0365 | 2 | F | Clarification of PDCP-Config field descriptions | 15.4.0 |
|  | RP-82 | RP-182653 | 0368 | 2 | F | UE Context handling during handover to NR | 15.4.0 |
|  | RP-82 | RP-182652 | 0371 | 2 | F | [E255] CR to 38.331 on corrections related to CGI reporting timer T321 | 15.4.0 |
|  | RP-82 | RP-182655 | 0375 | 3 | F | R2-1817981 CR to 38.331 on pendingRnaUpdate setting | 15.4.0 |
|  | RP-82 | RP-182653 | 0376 | 2 | F | Introducing procedure for reporting RLC failures | 15.4.0 |
|  | RP-82 | RP-182654 | 0379 | 2 | F | Correction of frequency band indication in MeasObjectNR | 15.4.0 |
|  | RP-82 | RP-182654 | 0382 | 2 | F | RRC connection release triggered by upper layers | 15.4.0 |
|  | RP-82 | RP-182660 | 0384 | 3 | F | Correction to configuration of measurement object | 15.4.0 |
|  | RP-82 | RP-182665 | 0388 | 3 | F | Correction to 38331 in SRS-Config | 15.4.0 |
|  | RP-82 | RP-182657 | 0391 | 2 | F | Correction for PowerControl-related issues | 15.4.0 |
|  | RP-82 | RP-182668 | 0395 | 4 | F | Inter-band EN-DC Configured Output Power requirements | 15.4.0 |
|  | RP-82 | RP-182655 | 0396 | 2 | F | E573 Configuration of SRB1 during Resume | 15.4.0 |
|  | RP-82 | RP-182655 | 0402 | 1 | F | Triggers for abortion of RRC establishment | 15.4.0 |
|  | RP-82 | RP-182656 | 0406 | 2 | F | Correction on CN type indication for Redirection from NR to E-UTRA | 15.4.0 |
|  | RP-82 | RP-182664 | 0409 | 4 | F | Miscellaneous minor corrections | 15.4.0 |
|  | RP-82 | RP-182654 | 0410 | 1 | F | Invalidation of L1 parameter nrofCQIsPerReport | 15.4.0 |
|  | RP-82 | RP-182654 | 0411 | 1 | F | Clarifications on RNA update and CN registration (N023) | 15.4.0 |
|  | RP-82 | RP-182666 | 0412 | 3 | F | Missing optionality bit in CG-ConfigInfo | 15.4.0 |
|  | RP-82 | RP-182662 | 0414 | 2 | F | Clarification for the implementation of UE feature list item 6-1 (BWP op1) | 15.4.0 |
|  | RP-82 | RP-182654 | 0417 | 1 | F | Clarification on ssb-PositionsInBurst | 15.4.0 |
|  | RP-82 | RP-182667 | 0418 | 3 | F | Correction to commonControlResourceSet | 15.4.0 |
|  | RP-82 | RP-182667 | 0420 | 2 | F | Correction to TDD configuration in SIB1 | 15.4.0 |
|  | RP-82 | RP-182668 | 0421 | 5 | F | Clarification on handling of default parameters | 15.4.0 |
|  | RP-82 | RP-182663 | 0429 | 2 | F | SRB3 integrity protection failure handling | 15.4.0 |
|  | RP-82 | RP-182653 | 0431 | 2 | F | Corrections to the field decriptions of System Information | 15.4.0 |
|  | RP-82 | RP-182653 | 0434 | 1 | F | Correction to SI provision in connected mode | 15.4.0 |
|  | RP-82 | RP-182661 | 0436 | 3 | F | PDCCH Monitoring Occasions in SI Window | 15.4.0 |
|  | RP-82 | RP-182655 | 0438 | 3 | F | CR on SI Message Acquisition | 15.4.0 |
|  | RP-82 | RP-182652 | 0439 | 1 | F | Update of nas-SecurityParamFromNR according to LS from SA3 | 15.4.0 |
|  | RP-82 | RP-182652 | 0445 | 2 | F | Correction to Default MAC Cell Group configuration | 15.4.0 |
|  | RP-82 | RP-182652 | 0447 | 1 | F | Correction to missing field descriptions of PLMN Identity | 15.4.0 |
|  | RP-82 | RP-182657 | 0448 | 2 | F | Introducing PDCP suspend procedure | 15.4.0 |
|  | RP-82 | RP-182657 | 0449 | 2 | F | Correction to PDCP statusReportRequired | 15.4.0 |
|  | RP-82 | RP-182664 | 0454 | 3 | F | CR to 38.331 on the ambiguity of targetCellIdentity in Resume/Reestablishment MAC-I calculation | 15.4.0 |
|  | RP-82 | RP-182655 | 0457 | 1 | F | Corrections on P-Max description | 15.4.0 |
|  | RP-82 | RP-182651 | 0460 | 2 | F | Clarification on Configuration of multiplePHR for EN-DC and NR-CA | 15.4.0 |
|  | RP-82 | RP-182656 | 0469 | 2 | F | Correction on conditional presence of PCellOnly | 15.4.0 |
|  | RP-82 | RP-182657 | 0474 | 4 | F | Introduction of power boosting indicator for pi2BPSK waveform | 15.4.0 |
|  | RP-82 | RP-182655 | 0475 | 1 | F | Correction on the allowedBC-ListMRDC | 15.4.0 |
|  | RP-82 | RP-182649 | 0476 | 2 | F | Removal of restriction on RB removal and addition | 15.4.0 |
|  | RP-82 | RP-182649 | 0482 | 2 | F | Correction to full configuration | 15.4.0 |
|  | RP-82 | RP-182661 | 0492 | 3 | F | CR on MeasurementTimingConfiguration | 15.4.0 |
|  | RP-82 | RP-182654 | 0502 | 1 | F | Bandwidth configuration for initial BWP | 15.4.0 |
|  | RP-82 | RP-182664 | 0503 | 4 | F | CORESET#0 configuration when SIB1 is not broadcast | 15.4.0 |
|  | RP-82 | RP-182663 | 0506 | 1 | F | Correction on the behaviors with cell reselection while T302 is running | 15.4.0 |
|  | RP-82 | RP-182661 | 0509 | 2 | F | Correction on SDAP reconfiguration handling | 15.4.0 |
|  | RP-82 | RP-182663 | 0510 | 1 | F | Clarification for the UE behaviour in camped normally and camped on any cell states | 15.4.0 |
|  | RP-82 | RP-182663 | 0514 | 2 | F | Correction to description of parameter Ns nAndPagingFrameOffset | 15.4.0 |
|  | RP-82 | RP-182649 | 0515 | - | F | Correction to description of parameter Ns | 15.4.0 |
|  | RP-82 | RP-182661 | 0516 | 1 | F | CR on UE behaviour after SI Acquisition Failure | 15.4.0 |
|  | RP-82 | RP-182662 | 0518 | 1 | F | CR on PUCCH-ConfigCommon | 15.4.0 |
|  | RP-82 | RP-182662 | 0520 | 1 | F | Clarifications on receiving RRCReject without wait timer | 15.4.0 |
|  | RP-82 | RP-182665 | 0522 | 1 | F | CR on powerControlOffset | 15.4.0 |
|  | RP-82 | RP-182664 | 0524 | 2 | F | Correction to configuration of firstPDCCH-MonitoringOccasionOfPO | 15.4.0 |
|  | RP-82 | RP-182660 | 0539 | 1 | F | R on PCCH-Config | 15.4.0 |
|  | RP-82 | RP-182649 | 0541 | - | F | Clarification to no barring configuration for Implicit UAC | 15.4.0 |
|  | RP-82 | RP-182649 | 0542 | - | F | Correction to Access Category and barring config determination for implicit access barring | 15.4.0 |
|  | RP-82 | RP-182664 | 0543 | 2 | F | Per serving cell MIMO layer configuration | 15.4.0 |
|  | RP-82 | RP-182661 | 0545 | 1 | F | Correction to reconfiguration with sync | 15.4.0 |
|  | RP-82 | RP-182659 | 0552 | 1 | F | Correction for SSB power | 15.4.0 |
|  | RP-82 | RP-182659 | 0554 | 1 | F | Corrections on SearchSpace configuration | 15.4.0 |
|  | RP-82 | RP-182665 | 0558 | 1 | F | Correction for TCI state in ControlResourceSet | 15.4.0 |
|  | RP-82 | RP-182663 | 0560 | 1 | F | CR for the optional configuration of subbandSize | 15.4.0 |
|  | RP-82 | RP-182650 | 0562 | - | F | Correction on ShortMAC-I description in 38.331 | 15.4.0 |
|  | RP-82 | RP-182661 | 0567 | 1 | F | CR to the field descriptions of System Information | 15.4.0 |
|  | RP-82 | RP-182650 | 0569 | - | F | Clarification on SRB3 release | 15.4.0 |
|  | RP-82 | RP-182650 | 0570 | - | F | Avoiding security risk for RLC UM bearers during termination point change | 15.4.0 |
|  | RP-82 | RP-182660 | 0571 | 1 | F | MO configuration with SSB SCS for a given SSB frequency | 15.4.0 |
|  | RP-82 | RP-182663 | 0572 | 1 | F | Barring alleviation for emergency service | 15.4.0 |
|  | RP-82 | RP-182664 | 0575 | 1 | F | Corrections for security configurations during setup of SRB1 | 15.4.0 |
|  | RP-82 | RP-182660 | 0577 | 1 | F | Clarification of UE behaviour when frequencyBandList is absent in SIB4 | 15.4.0 |
|  | RP-82 | RP-182661 | 0578 | 2 | F | Handling of missing fields in SIB1 | 15.4.0 |
|  | RP-82 | RP-182659 | 0580 | 1 | F | Correction to ControlResourceSetZero | 15.4.0 |
|  | RP-82 | RP-182667 | 0582 | 2 | F | Full configuration for inter-RAT handover | 15.4.0 |
|  | RP-82 | RP-182664 | 0587 | 1 | F | Corrections on number of RadioLinkMonitoringRS condifuration | 15.4.0 |
|  | RP-82 | RP-182659 | 0591 | 1 | F | Clarification on phr-Type2OtherCell | 15.4.0 |
|  | RP-82 | RP-182667 | 0594 | 2 | F | Addition of PCI in MeasTiming | 15.4.0 |
|  | RP-82 | RP-182667 | 0600 | 5 | F | Clarifications to SIBs requiring request procedure | 15.4.0 |
|  | RP-82 | RP-182659 | 0601 | 1 | F | Correction for support of initial downlink BWP | 15.4.0 |
|  | RP-82 | RP-182657 | 0602 | 1 | F | Miscellaneous corrections related to idle mode SIBs | 15.4.0 |
|  | RP-82 | RP-182657 | 0603 | 1 | F | Correction for missing fields in SIB2 and SIB4 | 15.4.0 |
|  | RP-82 | RP-182657 | 0604 | 2 | F | Correction to Q-QualMin value range | 15.4.0 |
|  | RP-82 | RP-182663 | 0616 | 1 | F | Clarification of cell reselection during resume procedure | 15.4.0 |
|  | RP-82 | RP-182663 | 0617 | 1 | F | Determination of Access Identities for RRC-triggered Access Attempts | 15.4.0 |
|  | RP-82 | RP-182663 | 0618 | 1 | F | CR to 38.331 on stopping of timer T390 upon reception of RRCRelease | 15.4.0 |
|  | RP-82 | RP-182840 | 0620 | 3 | F | CR on MN/SN coordination for report CGI procedure | 15.4.0 |
|  | RP-82 | RP-182666 | 0624 | 2 | F | CR to 38.331 on aligning I-RNTI terminology in paging and SuspendConfig (Alt.2) | 15.4.0 |
|  | RP-82 | RP-182665 | 0627 | 2 | F | CR to 38.331 on IRAT Cell reselection in RRC_INACTIVE | 15.4.0 |
|  | RP-82 | RP-182662 | 0638 | 1 | F | CR for pendingRnaUpdate set | 15.4.0 |
|  | RP-82 | RP-182665 | 0640 | 2 | F | Corrections on BWP ID | 15.4.0 |
|  | RP-82 | RP-182664 | 0643 | 1 | F | Inter-frequency handover capability | 15.4.0 |
|  | RP-82 | RP-182659 | 0646 | 1 | F | Search space configuration for DCI format 2_0 monitoring | 15.4.0 |
|  | RP-82 | RP-182739 | 0647 | 3 | F | Correction on power headroom configuration exchange | 15.4.0 |
|  | RP-82 | RP-182665 | 0649 | 2 | F | UE capability on PA architecture | 15.4.0 |
|  | RP-82 | RP-182662 | 0654 | 1 | F | CR on pdsch-TimeDomainAllocationList and pusch-TimeDomainAllocationList | 15.4.0 |
|  | RP-82 | RP-182664 | 0655 | 1 | F | Correction on the SSB based RACH configuration | 15.4.0 |
|  | RP-82 | RP-182659 | 0656 | 1 | F | CR on starting bit of Format 2-3 | 15.4.0 |
|  | RP-82 | RP-182663 | 0660 | 1 | C | CR on wait timer in RRC release | 15.4.0 |
|  | RP-82 | RP-182662 | 0664 | 1 | F | SCell release at RRC Reestablishment | 15.4.0 |
|  | RP-82 | RP-182663 | 0665 | 1 | F | Clean up of SRB1 terminology | 15.4.0 |
|  | RP-82 | RP-182662 | 0670 | 1 | F | Correction on the size of PUCCH resource ID | 15.4.0 |
|  | RP-82 | RP-182667 | 0673 | 3 | F | CR to 38.331 on Integrity Check failure at RRC Reestablishment | 15.4.0 |
|  | RP-82 | RP-182661 | 0680 | 1 | F | Correction on SI message acquisition timing | 15.4.0 |
|  | RP-82 | RP-182653 | 0682 | - | F | Add t-ReselectionNR-SF in SIB2 | 15.4.0 |
|  | RP-82 | RP-182654 | 0683 | - | F | freqBandIndicatorNR correction in MultiFrequencyBandListNR-SIB | 15.4.0 |
|  | RP-82 | RP-182658 | 0684 | 2 | F | Corrections to CellSelectionInfo in SIB1 and SIB4 | 15.4.0 |
|  | RP-82 | RP-182654 | 0686 | - | F | Correction on the field description of DRX timers | 15.4.0 |
|  | RP-82 | RP-182661 | 0687 | 1 | F | Correction on DC subcarrier usage in SetupComplete message | 15.4.0 |
|  | RP-82 | RP-182665 | 0688 | 3 | F | Various carrier frequency definiton corrections | 15.4.0 |
|  | RP-82 | RP-182661 | 0689 | 1 | F | CR on signaling contiguous and non-contiguous EN-DC capability | 15.4.0 |
|  | RP-82 | RP-182654 | 0692 | - | F | Update of the usage of QCL type-C | 15.4.0 |
|  | RP-82 | RP-182659 | 0694 | 1 | F | Cleanup of references to L1 specifications | 15.4.0 |
|  | RP-82 | RP-182660 | 0695 | 1 | F | Correction of MeasResultEUTRA | 15.4.0 |
|  | RP-82 | RP-182660 | 0696 | 1 | F | Missing need code for refFreqCSI-RS | 15.4.0 |
|  | RP-82 | RP-182661 | 0697 | 2 | F | Missing procedure text in RRC Reconfiguration | 15.4.0 |
|  | RP-82 | RP-182781 | 0700 | 3 | F | Correction to UE capability procedures in 38.331 | 15.4.0 |
|  | RP-82 | RP-182667 | 0701 | 1 | F | Correction to aperiodicTriggeringOffset | 15.4.0 |
|  | RP-82 | RP-182664 | 0709 | 1 | F | CR to 38.331 on including serving cell measurements | 15.4.0 |
|  | RP-82 | RP-182660 | 0711 | 1 | F | CR to 38.331 on associatedSSB | 15.4.0 |
|  | RP-82 | RP-182662 | 0714 | 1 | F | CR on 38.331 for RRCResumeRequest and RRCResumeRequest1 and protection of RRCResumeRequest1 | 15.4.0 |
|  | RP-82 | RP-182667 | 0715 | 2 | F | Correction for reporting of NR serving cell measurements when rsType is missing | 15.4.0 |
|  | RP-82 | RP-182656 | 0719 | 1 | F | Clarification of the values for RangeToBestCell | 15.4.0 |
|  | RP-82 | RP-182668 | 0721 | 2 | F | CR on handling of timer T380 | 15.4.0 |
|  | RP-82 | RP-182662 | 0723 | 2 | F | CR on supporting signalling only connection | 15.4.0 |
|  | RP-82 | RP-182838 | 0725 | 3 | F | Signalling introduction of SRS switching capability | 15.4.0 |
|  | RP-82 | RP-182667 | 0729 | 3 | B | CR on signalling introduction of UE overheating support in NR SA scenario | 15.4.0 |
|  | RP-82 | RP-182856 | 0730 | 4 | F | CR on SRS antenna switching | 15.4.0 |
|  | RP-82 | RP-182660 | 0731 | 1 | F | Correction to offsetToPointA | 15.4.0 |
|  | RP-82 | RP-182655 | 0732 | - | F | Correction to cell selection parameters | 15.4.0 |
|  | RP-82 | RP-182665 | 0746 | 2 | F | CR to 38.331 on stopping T302 and UE related actions | 15.4.0 |
|  | RP-82 | RP-182666 | 0750 | 2 | F | Correction on indication for user plane resource release | 15.4.0 |
|  | RP-82 | RP-182662 | 0767 | 1 | F | Correction on the terminology scg-ChangeFailure | 15.4.0 |
|  | RP-82 | RP-182661 | 0768 | 1 | F | Correction on default configuration | 15.4.0 |
|  | RP-82 | RP-182660 | 0772 | 1 | F | Clarification of measurement object for beam reporting for NR cells | 15.4.0 |
|  | RP-82 | RP-182667 | 0773 | 3 | F | CR to 38.331 on UE AS Context definition – Include suspendConfig | 15.4.0 |
|  | RP-82 | RP-182661 | 0778 | 1 | F | CR to 38.331 on HO support in Setup Procedure | 15.4.0 |
|  | RP-82 | RP-182656 | 0781 | - | F | CR on description of k0 | 15.4.0 |
|  | RP-82 | RP-182666 | 0783 | 2 | F | CR to 38.331 on removing FFS of locationInfo | 15.4.0 |
|  | RP-82 | RP-182661 | 0787 | - | F | Clarification on MIB Acquisition | 15.4.0 |
|  | RP-82 | RP-182662 | 0788 | - | F | CR to 38331 on release after completion of inter-RAT HO | 15.4.0 |
|  | RP-82 | RP-182662 | 0789 | - | F | CR to 38.331 on rbg-Size in PDSCH-Config, PUSCH-Config and ConfiguredGrantConfig | 15.4.0 |
|  | RP-82 | RP-182657 | 0790 | - | F | Advanced processing time configuration for PDSCH and PUSCH | 15.4.0 |
|  | RP-82 | RP-182896 | 0791 | 2 | F | UE specific channel bandwidth signaling | 15.4.0 |
| 03/2019 | RP-83 | RP-190541 | 0416 | 4 | F | Clarification on hopping parameters for PUSCH | 15.5.0 |
|  | RP-83 | RP-190541 | 0593 | 2 | F | Removal of creation of MCG MAC entity | 15.5.0 |
|  | RP-83 | RP-190633 | 0792 | 1 | F | Capability for aperiodic CSI-RS triggering with different numerology between PDCCH and CSI-RS | 15.5.0 |
|  | RP-83 | RP-190541 | 0796 | 2 | F | Correction on Mapping between SSBs and PDCCH Monitoring Occasions in SI Window | 15.5.0 |
|  | RP-83 | RP-190541 | 0797 | 2 | F | Correction to SI Reqeust Procedure | 15.5.0 |
|  | RP-83 | RP-190546 | 0798 | 2 | F | CR to 38.331 on clarification of reportCGI | 15.5.0 |
|  | RP-83 | RP-190545 | 0799 | 2 | F | Describing mandatory/optional information in inter-node RRC messages | 15.5.0 |
|  | RP-83 | RP-190541 | 0800 | 1 | F | Search space configuration for cross carrier scheduling | 15.5.0 |
|  | RP-83 | RP-190542 | 0803 | 1 | F | Clarification on FeatureSetCombinationId zero value | 15.5.0 |
|  | RP-83 | RP-190546 | 0805 | 2 | F | Clarification on UE Capability Request Filtering | 15.5.0 |
|  | RP-83 | RP-190545 | 0807 | 3 | F | Miscellaneous non-controversial corrections | 15.5.0 |
|  | RP-83 | RP-190541 | 0808 | 2 | F | CR to 38.331 on MAC configuration | 15.5.0 |
|  | RP-83 | RP-190543 | 0810 | 2 | F | Correction to SCG failiure | 15.5.0 |
|  | RP-83 | RP-190540 | 0811 | 1 | F | Clarifying handling of parent and child IE need nodes | 15.5.0 |
|  | RP-83 | RP-190544 | 0812 | 2 | F | Clarification to channel bandwidth signalling | 15.5.0 |
|  | RP-83 | RP-190541 | 0813 | 1 | F | Clarifications to BWP configuration options | 15.5.0 |
|  | RP-83 | RP-190543 | 0822 | 2 | F | Correction to EUTRA-MBSFN-SubframeConfig | 15.5.0 |
|  | RP-83 | RP-190545 | 0823 | 2 | F | Clarification on dedicated serving cell configuration in Re-establishment | 15.5.0 |
|  | RP-83 | RP-190541 | 0828 | 2 | F | Clarification on the BWP id configuration | 15.5.0 |
|  | RP-83 | RP-190541 | 0836 | 1 | F | Upon entering a new PLMN which is in the list of EPLMNs in RRC INACTIVE state | 15.5.0 |
|  | RP-83 | RP-190546 | 0843 | 1 | F | EUTRA UE capability filtering in NR UE capability enquiry | 15.5.0 |
|  | RP-83 | RP-190543 | 0847 | 2 | F | Correction to SIB1 transmission during handover | 15.5.0 |
|  | RP-83 | RP-190545 | 0850 | 2 | F | Clarification to monitoring occasion of PWS notification | 15.5.0 |
|  | RP-83 | RP-190541 | 0853 | 1 | F | HandoverPreparationInformation for CU/DU | 15.5.0 |
|  | RP-83 | RP-190542 | 0855 | 1 | F | CR to introduce simultaneousRxDataSSB-DiffNumerology for NR SA | 15.5.0 |
|  | RP-83 | RP-190550 | 0858 | - | F | Condition on integrity protection for DRB | 15.5.0 |
|  | RP-83 | RP-190544 | 0860 | 2 | F | Handling on UE Inactive AS context upon resume | 15.5.0 |
|  | RP-83 | RP-190542 | 0861 | 1 | F | Miscellaneous Corrections for INACTIVE | 15.5.0 |
|  | RP-83 | RP-190542 | 0864 | 1 | F | Correction on RRC processing delay | 15.5.0 |
|  | RP-83 | RP-190540 | 0865 | - | F | Dummify the ue-BeamLockFunction IE | 15.5.0 |
|  | RP-83 | RP-190545 | 0866 | 2 | F | Further update of Need codes | 15.5.0 |
|  | RP-83 | RP-190541 | 0867 | 1 | F | Corrections to reestablishment procedure | 15.5.0 |
|  | RP-83 | RP-190545 | 0868 | 2 | F | CR on use of positioning measurement gaps for subframe and slot timing detection towards E-UTRA | 15.5.0 |
|  | RP-83 | RP-190542 | 0876 | 2 | F | Barring alleviation when T302 or T390 is stopped | 15.5.0 |
|  | RP-83 | RP-190544 | 0877 | 2 | F | Correction on smtc configuration in NR SCell addition procedure | 15.5.0 |
|  | RP-83 | RP-190543 | 0884 | 2 | F | Correction on the configuration for transform preceding of PUSCH | 15.5.0 |
|  | RP-83 | RP-190543 | 0896 | 2 | F | Correction to Need Codes in system information | 15.5.0 |
|  | RP-83 | RP-190541 | 0897 | 1 | F | Corrections on drb-ContinueROHC | 15.5.0 |
|  | RP-83 | RP-190541 | 0898 | 1 | F | Correction on outOfOrderDelivery | 15.5.0 |
|  | RP-83 | RP-190542 | 0902 | 1 | F | Corrections on radio link failure related actions | 15.5.0 |
|  | RP-83 | RP-190541 | 0904 | 1 | F | Clarification for SIB validity | 15.5.0 |
|  | RP-83 | RP-190540 | 0905 | - | F | Corrections to MFBI | 15.5.0 |
|  | RP-83 | RP-190542 | 0912 | 1 | F | CR on clarification on the description of NIA0 | 15.5.0 |
|  | RP-83 | RP-190542 | 0913 | 1 | F | CR on the number of bits of downlink NAS COUNT value | 15.5.0 |
|  | RP-83 | RP-190541 | 0920 | 1 | F | CR to 38.331 for not supporting different quantities for thresholds in Event A5 and B2 | 15.5.0 |
|  | RP-83 | RP-190544 | 0922 | 2 | F | CR on SSB type indication | 15.5.0 |
|  | RP-83 | RP-190545 | 0923 | 2 | F | Correction for measurements of serving cells without SSB or without CSI-RS | 15.5.0 |
|  | RP-83 | RP-190540 | 0930 | - | F | CR on introduction of UE assistance information in inter-node message | 15.5.0 |
|  | RP-83 | RP-190540 | 0931 | - | F | CR on description of SRS carrier switching | 15.5.0 |
|  | RP-83 | RP-190542 | 0932 | 1 | F | Clarification on the relation between CA configuration and supported featureset combination_Option1 | 15.5.0 |
|  | RP-83 | RP-190545 | 0935 | 2 | F | Unification of EN-DC terminology | 15.5.0 |
|  | RP-83 | RP-190550 | 0938 | - | F | PDCP re-establishment during SRB modification for EUTRA/5GC | 15.5.0 |
|  | RP-83 | RP-190541 | 0939 | 1 | F | The support of drb-ContinueROHC | 15.5.0 |
|  | RP-83 | RP-190541 | 0948 | 1 | F | Correction on PTRS port index | 15.5.0 |
|  | RP-83 | RP-190541 | 0956 | 1 | F | CR on the supplementaryUplink and uplinkConfig | 15.5.0 |
|  | RP-83 | RP-190545 | 0963 | 2 | F | Correction on MIB acquisition upon Reconfiguration with Sync | 15.5.0 |
|  | RP-83 | RP-190543 | 0967 | 2 | F | Qoffset for inter-RAT cell reselection | 15.5.0 |
|  | RP-83 | RP-190541 | 0975 | 1 | F | Correction on SI scheduling | 15.5.0 |
|  | RP-83 | RP-190543 | 0976 | 2 | F | Correction of uac-AccessCategory1-SelectionAssistanceInfo field description | 15.5.0 |
|  | RP-83 | RP-190546 | 0978 | 3 | F | Correction on going to RRC_IDLE upon inter-RAT cell reselection in RRC_INACTIVE | 15.5.0 |
|  | RP-83 | RP-190543 | 0981 | 2 | F | Clarification on nrofSS-BlocksToAverage and absThreshSS-BlocksConsolidation | 15.5.0 |
|  | RP-83 | RP-190543 | 0984 | 3 | F | Correction on compilation of featureSets for NR container | 15.5.0 |
|  | RP-83 | RP-190540 | 0985 | - | F | Enable and disable of security at DRB setup | 15.5.0 |
|  | RP-83 | RP-190545 | 0986 | 2 | F | Clarification on TCI state ID | 15.5.0 |
|  | RP-83 | RP-190544 | 0987 | - | F | Clarification for random access on SUL | 15.5.0 |
|  | RP-83 | RP-190545 | 0988 | 1 | F | Correction on supportedBandwidthCombinationSetEUTRA-v1530 usage | 15.5.0 |
|  | RP-83 | RP-190544 | 0989 | - | F | CR on Processing delay requirements for RRC Resume procedures in TS 38.331 | 15.5.0 |
| 04/2019 | RP-83 |  |  |  |  | MCC: Formatting error correction (missing carriage return) in the end of clause 5.3.5.11. | 15.5.1 |
| 06/2019 | RP-84 | RP-191379 | 0906 | 5 | F | Reconfig with sync terminology | 15.6.0 |
|  | RP-84 | RP-191378 | 0916 | 5 | B | Introduction of late drop NGEN-DC, NE-DC and NR-DC | 15.6.0 |
|  | RP-84 | RP-191374 | 0996 | 2 | F | Correction to the need code of some fields in SIB2 | 15.6.0 |
|  | RP-84 | RP-191377 | 1003 | 3 | F | Clarification for handling of suspendConfig | 15.6.0 |
|  | RP-84 | RP-191377 | 1005 | 3 | F | Reporting of serving cell and best neighbour cell and sorting of beam | 15.6.0 |
|  | RP-84 | RP-191377 | 1011 | 1 | F | On T321 timer related informative text correction | 15.6.0 |
|  | RP-84 | RP-191377 | 1013 | 1 | C | CR to direct current report for UL and SUL | 15.6.0 |
|  | RP-84 | RP-191380 | 1014 | 1 | F | Correction on storing UE AS Inactive Context | 15.6.0 |
|  | RP-84 | RP-191373 | 1015 | - | F | Correction on ReconfigurationWithSync | 15.6.0 |
|  | RP-84 | RP-191380 | 1016 | 2 | F | Correction on Handover from NR to EUTRAN | 15.6.0 |
|  | RP-84 | RP-191373 | 1018 | - | F | Introduction of additional UE capability on HARQ-ACK multiplexing on PUSCH | 15.6.0 |
|  | RP-84 | RP-191378 | 1019 | 1 | F | Correction on bar indication of emergency service (access category 2) | 15.6.0 |
|  | RP-84 | RP-191373 | 1020 | - | F | Correction on UE configuration for RRC Resume procedure | 15.6.0 |
|  | RP-84 | RP-191373 | 1021 | - | F | RRC release with suspend configuration and inter-RAT redirection | 15.6.0 |
|  | RP-84 | RP-191373 | 1022 | - | F | RRC Reconfiguration via SRB3 in EN-DC | 15.6.0 |
|  | RP-84 | RP-191373 | 1023 | - | F | Corrections on RLC bearer setup | 15.6.0 |
|  | RP-84 | RP-191373 | 1024 | - | F | Clarification to Permitted MaxCID for ROHC and Uplink-Only ROHC | 15.6.0 |
|  | RP-84 | RP-191373 | 1025 | - | F | Coordination of ROHC capability for MR-DC | 15.6.0 |
|  | RP-84 | RP-191373 | 1026 | - | F | Correction on the rlmInSyncOutOfSyncThreshold | 15.6.0 |
|  | RP-84 | RP-191373 | 1027 | - | F | Correction on description of tci-PresentInDCI | 15.6.0 |
|  | RP-84 | RP-191374 | 1031 | - | F | RRC processing delay for UE capability transfer | 15.6.0 |
|  | RP-84 | RP-191377 | 1032 | 1 | F | Handling of SMTC configuration | 15.6.0 |
|  | RP-84 | RP-191378 | 1033 | 1 | F | Clarification on filters used to generate FeatureSets (38.331) | 15.6.0 |
|  | RP-84 | RP-191374 | 1034 | - | F | Correction of behavior for eutra-nr-only | 15.6.0 |
|  | RP-84 | RP-191377 | 1038 | 1 | F | Clarification on CSI-RS resource configuration in MO | 15.6.0 |
|  | RP-84 | RP-191378 | 1039 | 2 | F | Update on usage of Need codes | 15.6.0 |
|  | RP-84 | RP-191377 | 1040 | 1 | F | Ignore additional fields in RRC Release message before security activation | 15.6.0 |
|  | RP-84 | RP-191374 | 1041 | - | F | Correction on use of Null algorithm for DRBs during emergency calls in LSM | 15.6.0 |
|  | RP-84 | RP-191380 | 1042 | 2 | F | NR changes for FullConfig for Inter-RAT intra-system HO | 15.6.0 |
|  | RP-84 | RP-191376 | 1043 | 2 | F | Monitoring of short messages with multi-beams | 15.6.0 |
|  | RP-84 | RP-191377 | 1045 | 1 | F | Clarification of commonControlResourceSet frequency reference point | 15.6.0 |
|  | RP-84 | RP-191379 | 1046 | 2 | F | CR on capability of maxUplinkDutyCycle for FR2 | 15.6.0 |
|  | RP-84 | RP-191374 | 1049 | - | F | CR to subcarrierSpacing in RateMatchPattern and SCS-SpecificCarrier | 15.6.0 |
|  | RP-84 | RP-191377 | 1053 | 1 | F | CR on transferring common configuration during handover and SN change | 15.6.0 |
|  | RP-84 | RP-191381 | 1054 | 2 | F | Correction to barring alleviation | 15.6.0 |
|  | RP-84 | RP-191381 | 1055 | 3 | F | UE behaviour on the cell without TAC | 15.6.0 |
|  | RP-84 | RP-191379 | 1058 | 2 | F | Correction to RRC resume | 15.6.0 |
|  | RP-84 | RP-191376 | 1061 | 1 | F | Corrections to inter-node messages | 15.6.0 |
|  | RP-84 | RP-191378 | 1063 | 1 | F | Clarification on mandatory information in inter node RRC messages | 15.6.0 |
|  | RP-84 | RP-191374 | 1066 | - | F | Correction to PWS reception | 15.6.0 |
|  | RP-84 | RP-191377 | 1068 | 1 | F | Serving cell measurement handling with different rsType configuration scenarios | 15.6.0 |
|  | RP-84 | RP-191374 | 1069 | - | F | On CGI reporting contents | 15.6.0 |
|  | RP-84 | RP-191374 | 1071 | - | F | CR for 38.331 on security related corrections to UE and Network initiated RRC procedures to increase user's security and privacy | 15.6.0 |
|  | RP-84 | RP-191379 | 1072 | 3 | F | Correction on the issue with NCP and ECP for RateMatchPattern | 15.6.0 |
|  | RP-84 | RP-191377 | 1075 | 1 | F | Security protection of RRC messages | 15.6.0 |
|  | RP-84 | RP-191381 | 1076 | 1 | F | Introduction of a new NR band for LTE/NR spectrum sharing in Band 41/n41 | 15.6.0 |
|  | RP-84 | RP-191375 | 1077 | - | F | Stop of T302 and T390 at reception of RRCRelease with waitTime | 15.6.0 |
|  | RP-84 | RP-191375 | 1078 | - | F | Restriction of piggybacking of NAS PDUs | 15.6.0 |
|  | RP-84 | RP-191379 | 1079 | 3 | F | Correction on intra-band fallback behavior with FeatureSetsPerCC | 15.6.0 |
|  | RP-84 | RP-191375 | 1081 | - | F | Removal of spurious requirement on consistency of feature set combination IDs | 15.6.0 |
|  | RP-84 | RP-191381 | 1082 | 3 | F | Miscellaneous non-controversial corrections Set II | 15.6.0 |
|  | RP-84 | RP-191377 | 1083 | 1 | F | Correction to configuration of security in RadioBearerConfig | 15.6.0 |
|  | RP-84 | RP-191379 | 1086 | 2 | F | CR to 38.331 on MeasurementTimingConfiguration | 15.6.0 |
|  | RP-84 | RP-191375 | 1088 | - | F | Correction to the description of subcarrierspacing usage in ServingCellConfigCommon | 15.6.0 |
|  | RP-84 | RP-191375 | 1089 | - | F | 38.331 Clarfication on multiple TA capabilities | 15.6.0 |
|  | RP-84 | RP-191375 | 1091 | - | F | Set beamCorrespondenceCA dummy | 15.6.0 |
|  | RP-84 | RP-191377 | 1092 | 1 | F | Correction on Measurement Report Triggering for Periodical Report | 15.6.0 |
|  | RP-84 | RP-191375 | 1094 | - | F | Correction on PDCP duplication configuration | 15.6.0 |
|  | RP-84 | RP-191375 | 1095 | - | F | Correction on BWP configuration | 15.6.0 |
|  | RP-84 | RP-191377 | 1097 | 1 | F | Correction on configuration of pucch-ResourceCommon | 15.6.0 |
|  | RP-84 | RP-191376 | 1098 | 1 | F | Clarification of PUCCH reconfiguration on NR UL and SUL | 15.6.0 |
|  | RP-84 | RP-191375 | 1100 | 1 | F | Correction on initial BWP configuration in DownlinkConfigCommon and UplinkConfigCommon | 15.6.0 |
|  | RP-84 | RP-191377 | 1101 | 1 | F | Correction on PUCCH cell | 15.6.0 |
|  | RP-84 | RP-191377 | 1103 | 1 | F | Correction on the pdcp-Config | 15.6.0 |
|  | RP-84 | RP-191379 | 1104 | 2 | F | Correction on pathlossReferenceLinking | 15.6.0 |
|  | RP-84 | RP-191381 | 1106 | 2 | F | Clarification of dedicated priority handling from RRC_INACTIVE to RRC_IDLE | 15.6.0 |
|  | RP-84 | RP-191375 | 1110 | - | F | Clarification on sending condition for mcg-RB-Config | 15.6.0 |
|  | RP-84 | RP-191375 | 1111 | - | F | Clarification of timing reference for CSI-RS resources | 15.6.0 |
|  | RP-84 | RP-191376 | 1113 | - | F | Setting of resumeCause for NAS triggered event | 15.6.0 |
|  | RP-84 | RP-191376 | 1114 | - | F | UE capability signalling for FD-MIMO processing capabilities for EN-DC | 15.6.0 |
|  | RP-84 | RP-191376 | 1115 | - | F | Modified UE capability on different numerologies within the same PUCCH group | 15.6.0 |
|  | RP-84 | RP-191478 | 1116 | 2 | F | Clarification to commonSearchSpaceList in PDCCH-ConfigCommon | 15.6.0 |
|  | RP-84 | RP-191589 | 1117 | 1 | F | Removal of "Capability for aperiodic CSI-RS triggering with different numerology between PDCCH and CSI-RS" | 15.6.0 |
| 09/2019 | RP-85 | RP-192196 | 1120 | 1 | C | Additional capability signalling for 1024QAM support | 15.7.0 |
|  | RP-85 | RP-192191 | 1121 | 1 | F | Correction on TDD-UL-DL-Config | 15.7.0 |
|  | RP-85 | RP-192191 | 1122 | 1 | F | Correction of the secondHopPRB Parameter | 15.7.0 |
|  | RP-85 | RP-192190 | 1123 | - | F | RSRP reporting of SFTD measurement in NR-DC | 15.7.0 |
|  | RP-85 | RP-192191 | 1124 | 1 | F | Small Corrections for System Information | 15.7.0 |
|  | RP-85 | RP-192194 | 1125 | - | F | Corrections for Inter-node Messages | 15.7.0 |
|  | RP-85 | RP-192191 | 1126 | 1 | F | Clarification of Layer 3 Filtering for E-UTRA | 15.7.0 |
|  | RP-85 | RP-192190 | 1127 | - | F | Clarification on FailureInformation report for NE-DC | 15.7.0 |
|  | RP-85 | RP-192191 | 1136 | 1 | F | Clarification to fullConfig in NR | 15.7.0 |
|  | RP-85 | RP-192190 | 1137 | - | F | Updates for positioning measurement gaps for subframe and slot timing detection towards E-UTRA | 15.7.0 |
|  | RP-85 | RP-192191 | 1138 | 1 | F | Clarification for enabling of configured PUSCH frequency hopping | 15.7.0 |
|  | RP-85 | RP-192191 | 1139 | 1 | B | Introduction of SFTD measurement to neighbour cells for NR SA | 15.7.0 |
|  | RP-85 | RP-192192 | 1144 | 1 | F | Corrections to 38.331 on SI-schedulingInfo | 15.7.0 |
|  | RP-85 | RP-192193 | 1148 | 2 | F | Clarification on SRB2 and DRB configuration | 15.7.0 |
|  | RP-85 | RP-192191 | 1151 | 2 | F | PDU session release indication to upper layers during Full Configuration | 15.7.0 |
|  | RP-85 | RP-192191 | 1160 | 1 | F | Clarification on max payload of PUCCH-ResourceSet | 15.7.0 |
|  | RP-85 | RP-192191 | 1161 | 1 | F | Clarification on PUSCH configuration | 15.7.0 |
|  | RP-85 | RP-192192 | 1167 | 1 | F | Correction of condition HO-toNR and HO-Conn | 15.7.0 |
|  | RP-85 | RP-192192 | 1172 | 1 | F | Clarifying UE capability freqHoppingPUCCH-F0-2 and freqHoppingPUCCH-F1-3-4 | 15.7.0 |
|  | RP-85 | RP-192190 | 1173 | 1 | F | Clarification on selectedBandCombination | 15.7.0 |
|  | RP-85 | RP-192193 | 1174 | 2 | F | Clarifying handling of information elements on the F1 interface | 15.7.0 |
|  | RP-85 | RP-192192 | 1178 | 1 | F | Correction of field descriptions in UE-CapabilityRequestFilterCommon (38.331) | 15.7.0 |
|  | RP-85 | RP-192190 | 1179 | - | F | Clarification of ca-ParametersNR-forDC (38.331) | 15.7.0 |
|  | RP-85 | RP-192193 | 1183 | 2 | F | Correction on reestablishRLC | 15.7.0 |
|  | RP-85 | RP-192191 | 1185 | 1 | F | Correction on SFTD measurement configuration | 15.7.0 |
|  | RP-85 | RP-192193 | 1191 | 2 | F | Handling of embedded RRC message in RRCReconfiguration procedure | 15.7.0 |
|  | RP-85 | RP-192192 | 1201 | 2 | F | Clarification on definition of PUSCH-Less SCell | 15.7.0 |
|  | RP-85 | RP-192190 | 1204 | - | F | Correction on non-critical extension for NRDC-Parameters | 15.7.0 |
|  | RP-85 | RP-192191 | 1208 | 1 | F | Correction on UE actions upon going to RRC_IDLE | 15.7.0 |
|  | RP-85 | RP-192192 | 1211 | 1 | F | Correction on the acquisition of MIB and SIB1 for re-establishment | 15.7.0 |
|  | RP-85 | RP-192192 | 1212 | 1 | F | Correction on band selection in SIB1 | 15.7.0 |
|  | RP-85 | RP-192193 | 1213 | 2 | F | Correction on the actions uption reception of SIB2 and SIB4 | 15.7.0 |
|  | RP-85 | RP-192193 | 1219 | 3 | F | Miscellaneous non-controversial corrections Set III | 15.7.0 |
|  | RP-85 | RP-192194 | 1220 | 3 | F | Channel Bandwidth validation upon SIB1 acquisition | 15.7.0 |
|  | RP-85 | RP-192193 | 1224 | 1 | F | Correction of presence conditions for common PSCell parameters | 15.7.0 |
|  | RP-85 | RP-192190 | 1226 | - | F | Release of unnecessary power restrictions upon RRC connection re-establishment in NE-DC and NR-DC | 15.7.0 |
|  | RP-85 | RP-192193 | 1232 | 2 | F | Correction of field inclusion for inter-node message | 15.7.0 |
|  | RP-85 | RP-192192 | 1234 | 1 | F | SFTD measurement information in CG-ConfigInfo | 15.7.0 |
|  | RP-85 | RP-192192 | 1235 | 1 | F | Correction for UE context retrieval | 15.7.0 |
|  | RP-85 | RP-192190 | 1236 | - | F | Correction on CGI measurements | 15.7.0 |
|  | RP-85 | RP-192191 | 1237 | 1 | F | Corrections to SIB8 for CMAS geo-fencing | 15.7.0 |
|  | RP-85 | RP-192194 | 1242 | 3 | F | Corrections on the condition of RBTermChange | 15.7.0 |
|  | RP-85 | RP-192192 | 1243 | 1 | F | CR on clarification of aggregated bandwidth for overheating | 15.7.0 |
|  | RP-85 | RP-192190 | 1244 | - | F | Clarification on the selectedBandEntriesMN - Understanding 1 | 15.7.0 |
|  | RP-85 | RP-192194 | 1253 | 1 | F | Correction on RRC connection release indication after handover | 15.7.0 |
|  | RP-85 | RP-192193 | 1254 | 2 | F | Corrections on SIB1 configuration | 15.7.0 |
|  | RP-85 | RP-192191 | 1256 | 1 | F | Correction on inter-RAT cell re-selection when UE is in RRC_IDLE | 15.7.0 |
|  | RP-85 | RP-192193 | 1257 | 2 | F | maxMIMO-Layers for the normal uplink and the supplementary uplink_Option 1 | 15.7.0 |
|  | RP-85 | RP-192193 | 1261 | 1 | F | Correction on overheating indication | 15.7.0 |
|  | RP-85 | RP-192192 | 1262 | - | F | Handling lists other than AddMod | 15.7.0 |
|  | RP-85 | RP-192193 | 1263 | 1 | F | Releasing source cell ConfigCommon fields not present in target cell | 15.7.0 |
|  | RP-85 | RP-192347 | 1265 | - | C | Introduction of UE capability for NR-DC with SFN synchronization between PCell and PSCell | 15.7.0 |
| 12/2019 | RP-86 | RP-192934 | 1147 | 2 | C | Security requirement for UE capability enquiry for NR | 15.8.0 |
|  | RP-86 | RP-192934 | 1267 | 1 | F | Corrections on CG-Config | 15.8.0 |
|  | RP-86 | RP-192934 | 1273 | 1 | F | CR to introduce timer for DRX based SFTD measurement | 15.8.0 |
|  | RP-86 | RP-192934 | 1274 | 1 | F | Correction on absence of gapPurpose | 15.8.0 |
|  | RP-86 | RP-192934 | 1278 | 1 | F | Correction on field description of cellReselectionInfoCommon | 15.8.0 |
|  | RP-86 | RP-192935 | 1283 | 2 | F | Clarifying the alignment of capability filtering across LTE and NR in MR-DC | 15.8.0 |
|  | RP-86 | RP-192934 | 1292 | 1 | F | Correction for P-Max in FR2 | 15.8.0 |
|  | RP-86 | RP-192934 | 1296 | 2 | F | Correction on frequency indication in SIB1 and SIB2 | 15.8.0 |
|  | RP-86 | RP-192937 | 1300 | 2 | F | Handling of AS-Config in HandoverPreparationInformation | 15.8.0 |
|  | RP-86 | RP-192936 | 1301 | 2 | F | Corrections on scg-RB-Config in CG-Config | 15.8.0 |
|  | RP-86 | RP-192937 | 1305 | 2 | F | Correction on MCG measurements in SCGFailureInformation | 15.8.0 |
|  | RP-86 | RP-192937 | 1308 | 3 | F | Correction of SRB3 handling at full configuration (Alt2) | 15.8.0 |
|  | RP-86 | RP-192936 | 1309 | 2 | F | Correction to integrity protection in DRB addition and modification | 15.8.0 |
|  | RP-86 | RP-192938 | 1323 | 3 | F | Miscellaneous non-controversial corrections Set IV | 15.8.0 |
|  | RP-86 | RP-192935 | 1325 | 2 | F | Presence and absence of TAC in NR cell | 15.8.0 |
|  | RP-86 | RP-192935 | 1332 | 4 | F | Security Algorithms for Radio Bearers | 15.8.0 |
|  | RP-86 | RP-192934 | 1333 | 1 | F | Correction on the Msg3 based on demand system information | 15.8.0 |
|  | RP-86 | RP-192935 | 1335 | 2 | F | Clarification for aggregated bandwidth for overheating | 15.8.0 |
|  | RP-86 | RP-192935 | 1337 | 1 | F | Clarification on the feature set report in EUTRAN | 15.8.0 |
|  | RP-86 | RP-192934 | 1341 | 1 | F | CR to 38.331 on CGI information | 15.8.0 |
|  | RP-86 | RP-192936 | 1357 | 1 | F | Restoring SDAP and RoHC contexts during Resumption | 15.8.0 |
|  | RP-86 | RP-192934 | 1358 | - | F | Conditional presence on ue-CapabilityInfo and servCellIndexRangeSCG for inter-MN handover without SN change | 15.8.0 |
|  | RP-86 | RP-192934 | 1362 | - | F | Configuration limitation for RRCRelease message in R15 | 15.8.0 |
|  | RP-86 | RP-192937 | 1368 | 1 | F | Correction to AS security key update | 15.8.0 |
|  | RP-86 | RP-192936 | 1369 | 1 | F | Correction on the condition of RBTermChange | 15.8.0 |
|  | RP-86 | RP-192936 | 1375 | 1 | F | Correction on the configuration of split SRB | 15.8.0 |
|  | RP-86 | RP-192934 | 1378 | - | F | Correction on camping conditions | 15.8.0 |
|  | RP-86 | RP-192937 | 1379 | 1 | F | Correction on CORESET location | 15.8.0 |
|  | RP-86 | RP-192938 | 1381 | 2 | F | Correction to key derivation for the UE configured with sk-counter | 15.8.0 |
|  | RP-86 | RP-192936 | 1383 | 1 | F | Correction on the pre-condition for reconfiguration with sync of SCG | 15.8.0 |
|  | RP-86 | RP-192935 | 1385 | - | F | Correction on AS-Config | 15.8.0 |
|  | RP-86 | RP-192935 | 1387 | - | F | Correction on measurement reporting in NR-DC | 15.8.0 |
|  | RP-86 | RP-192936 | 1389 | 1 | F | Correction on SIB1 description | 15.8.0 |
|  | RP-86 | RP-192937 | 1402 | 1 | F | Correction to Feature Set Combination and Band combination list for NR-DC (38.331) | 15.8.0 |
|  | RP-86 | RP-192937 | 1403 | 2 | F | Security requirements for split PDU session (38.331) | 15.8.0 |
|  | RP-86 | RP-192938 | 1405 | 2 | F | Correction of UE assistance information | 15.8.0 |
|  | RP-86 | RP-192936 | 1406 | 1 | F | Clarification regarding inter-node transfer of UE capability containers | 15.8.0 |
|  | RP-86 | RP-192749 | 1421 | 1 | F | NE-DC dynamic power sharing capability | 15.8.0 |
| 03/2020 | RP-87 | RP-200335 | 1272 | 3 | F | Corrections on maxMeasIdentitiesSCG-NR in MR-DC | 15.9.0 |
|  | RP-87 | RP-200334 | 1409 | 2 | F | CR on BWCS for inter-ENDC BC with intra-ENDC BC (38.331) | 15.9.0 |
|  | RP-87 | RP-200334 | 1410 | 4 | F | CR to 38.331 on support of 70MHz channel bandwidth | 15.9.0 |
|  | RP-87 | RP-200335 | 1440 | 2 | F | Clarification on the PLMN-IdentityInfoList | 15.9.0 |
|  | RP-87 | RP-200334 | 1444 | 1 | F | Correction on removal of NR-DC and NE-DC band combinations when capabilityRequestFilterCommon is absent | 15.9.0 |
|  | RP-87 | RP-200334 | 1450 | 1 | F | Correction on reporting of uplink TX direct current | 15.9.0 |
|  | RP-87 | RP-200334 | 1454 | 1 | F | Corrections to the Location measurement indication procedure | 15.9.0 |
|  | RP-87 | RP-200334 | 1455 | - | F | Introduction of provisions for late non-critical extensions | 15.9.0 |
|  | RP-87 | RP-200334 | 1460 | 1 | F | Correction on p-maxNR-FR1 for NE-DC | 15.9.0 |
|  | RP-87 | RP-200334 | 1461 | - | F | Correction on SFTD frequency list in INM | 15.9.0 |
|  | RP-87 | RP-200335 | 1472 | 2 | F | Miscellaneous non-controversial corrections Set V | 15.9.0 |
|  | RP-87 | RP-200335 | 1475 | 1 | F | Capability coordination for NE-DC | 15.9.0 |
|  | RP-87 | RP-200335 | 1483 | 2 | F | CR on fallback BC reporting | 15.9.0 |
|  | RP-87 | RP-200334 | 1484 | 1 | F | CR on overheating assistance reporting in handover case | 15.9.0 |
|  | RP-87 | RP-200334 | 1496 | 1 | F | Correction on NZP-CSI-RS-ResourceSet | 15.9.0 |
|  | RP-87 | RP-200335 | 1501 | 1 | F | UE capability of intra-band requirements for inter-band EN-DC/NE-DC | 15.9.0 |
| 07/2020 | RP-88 | RP-201164 | 1445 | 3 | F | CR on introduction of extended capabilities for NR-DC only BCs | 15.10.0 |
|  | RP-88 | RP-201159 | 1449 | 2 | F | Clarification on the presence of ssb-perRACH-Occasion for the CSI-RS based CFRA | 15.10.0 |
|  | RP-88 | RP-201161 | 1518 | 2 | F | CR on SRS-CarrierSwitching | 15.10.0 |
|  | RP-88 | RP-201160 | 1534 | 2 | F | Clarification on pdcp-Duplication at RRC Reconfiguration | 15.10.0 |
|  | RP-88 | RP-201159 | 1539 | 2 | F | Avoiding security risk for RLC AM bearers during termination point change | 15.10.0 |
|  | RP-88 | RP-201161 | 1555 | 2 | F | Clarification on avoiding keystream repeat due to COUNT reuse | 15.10.0 |
|  | RP-88 | RP-201160 | 1559 | 2 | F | SRS Capability report for SRS only Scell | 15.10.0 |
|  | RP-88 | RP-201160 | 1561 | 1 | F | Correction to RequestedCapabilityCommon | 15.10.0 |
|  | RP-88 | RP-201159 | 1567 | 2 | F | Correction on PUCCH configuration | 15.10.0 |
|  | RP-88 | RP-201160 | 1571 | 2 | F | Correction on the need for reconfiguration with sync in (NG)EN-DC, NR-DC and NE-DC | 15.10.0 |
|  | RP-88 | RP-201160 | 1578 | 1 | F | 38331 CR(R15) on inter-RAT SFTD measurements | 15.10.0 |
|  | RP-88 | RP-201163 | 1623 | 1 | F | Clarification on the maxPUSCH-Duration for LCP Restriction | 15.10.0 |
|  | RP-88 | RP-201159 | 1630 | - | F | Clarification for SIB6, SIB7 and SIB8 acquisition during a measurement gap | 15.10.0 |
|  | RP-88 | RP-201164 | 1633 | 1 | F | Corrections to CORESET and PDCCH TCI state release | 15.10.0 |
|  | RP-88 | RP-201162 | 1643 | 1 | F | Clarification on release and addition of the uplink for SCell | 15.10.0 |
|  | RP-88 | RP-201159 | 1648 | - | F | Ambiguity in fr1-fr2-Add-UE-NR-Capabilities parameter | 15.10.0 |
|  | RP-88 | RP-201162 | 1655 | 1 | F | Correction to measurement coordination in MR-DC | 15.10.0 |
|  | RP-88 | RP-201164 | 1661 | 1 | F | T310 handling during MobilityFromNR | 15.10.0 |
|  | RP-88 | RP-201161 | 1667 | 2 | F | Miscellaneous non-controversial corrections Set V | 15.10.0 |
|  | RP-88 | RP-201163 | 1681 | 2 | F | Correction on SRS antenna capability for carrier switching | 15.10.0 |
|  | RP-88 | RP-201161 | 1684 | 1 | F | UE Capability Enhancement for FR1(TDD/FDD) / FR2 CA and DC | 15.10.0 |
|  | RP-88 | RP-201164 | 1688 | 1 | F | Correction to re-sending UEAssistanceInformation upon reconfiguration w/ sync | 15.10.0 |
|  | RP-88 | RP-201161 | 1695 | - | F | Corrections to SIB1 Processing | 15.10.0 |
|  | RP-88 | RP-201164 | 1706 | 1 | F | SMTC Configuration for PSCell Addition for NR-DC (Option 2) | 15.10.0 |
|  | RP-88 | RP-201187 | 1709 | - | B | CR on introduction of BCS to asymmetric channel bandwidths (38.331) | 15.10.0 |
|  | RP-88 | RP-201165 | 1712 | 1 | F | Introduction of CGI reporting capabilities | 15.10.0 |