3GPP TSG-RAN WG2 Meeting #101	                                     R2-1801837

Athens, Greece, 26th February - 2nd March 2018

Source:	CATT

Title:	Issues on Paging Occasion Design

Agenda Item:	10.4.5.6

Document for:	Discussion and Decision

Introduction

In RAN1 NR AdHoc #3 meeting, some agreements have been made on paging design [1], details are as followed:

- For paging, RAN1 to down-select from the following options

  - Option 1: Paging DCI followed by Paging Message

    - Note: These do not imply that they are consecutive

  - Option 2: Paging group indicator triggering UE feedback and Paging DCI followed by Paging Message

  - Option 3: Paging group indicator and Paging DCI followed by Paging Message

  - Option 4: Paging DCI indicates use of Option 1 or 2.

More agreements on paging have been made in RAN1#90bis meeting [2]:

- At least Option 1 (Paging scheduling DCI followed by Paging Message) is supported

  - From RAN1 perspective, paging scheduling DCI and Paging Message are sent at least in the same slot

  - From RAN1 perspective, NR supports LTE-like UE grouping where UE is specifically configured of its PO/slot. This is considered part of Option 1.

Details of UE grouping are up to RAN2

In this contribution, we’d like to focus on the design of paging occasion considering the above agreements as the baseline.

Discussion

In RAN2 #97 meeting, NR paging was discussed and the following agreements were reached:

Agreements

1:  UE in RRC_IDLE and RRC_INACTVE state monitors paging/notification every DRX cycle.

2:  UE monitors one paging occasion per DRX cycle. Paging occasion is the time interval over which a paging message is transmitted by gNB.

3:  The length of DRX cycle is configurable. A default DRX cycle length is provided in system information. Additionally, a UE specific DRX cycle length can also be provided to UE in dedicated signaling.

4:  The number of paging occasions in the DRX cycle is configurable and provided in system information.

5:  If multiple paging occasions are configured by network in the DRX cycle then UEs can be distributed to these paging occasions based on UE ID.

6:  RAN2 understanding is that paging can be transmitted at least using beam sweeping (content of paging may be a paging indicator or the paging message, FFS)

7:  Paging occasion can consists of multiple time slots (e.g. subframe or OFDM symbol). (Multiple time slots enable transmission of paging using a different set of DL TX beam(s) in each time slot, or could enable repetition - RAN1 decision).

8:  The number of time slots in a paging occasion is provided in system information.

According to the agreements above, paging can be transmitted at least using beam sweeping in NR, which means paging occasion can consists of multiple time slots (e.g. subframe or OFDM symbol). Before we discuss the motivation to redefine the design of paging occasion, we should talk about what degree the beam sweeping mechanism can change the length of one PO. At the moment, there is no agreement about how the paging DCI or paging message will be transmitted using beam sweeping in RAN1. However transmission mechanism for SS block can give us some related information. Some agreements have been made on SS block using beam sweeping in RAN1, which are listed in Table 1:

Table 1 the actual time length occupied by SS blocks in different NR case

| SCS Maximal SSB number | 15kHz | 30kHz | 60kHz | 120kHz | 240kHz |
| --- | --- | --- | --- | --- | --- |
| L=4 for up to 3GHz case | 2 subframe | 1 subframe | N/A | N/A | N/A |
| L=8 for 3GHz to 6GHz case | 4 subframe | 2 subframe | N/A | N/A | N/A |
| L=64 for 6GHz to 52.6GHz case | N/A | N/A | N/A | More than 4 subframe | More than 2 subframe |

We can conclude from Table 1 that the actual time length occupied by SS blocks is varied due to the usage of different carrier frequency and SCS. This is also applicable to the transmission of paging DCI or paging message when beam sweeping scenario is applied.

Observation 1: In NR, PO length is carrier frequency and SCS dependent.

Some agreements have been made for paging in RAN1#90bis meeting, which says from RAN1 perspective, paging scheduling DCI and paging message are sent at least in the same slot. In table 1, we can find the actual length occupied by SS blocks is no less than one subframe. Considering the information contained in one PO is usually much more than that in one SS burst, the actual time length occupied by one PO is larger than SS blocks in the same case. In our understanding, the length of one PO in NR is usually no less than one subframe if beam sweeping scenario is applied compared to SS blocks .

Observation 2: In NR, PO length is usually no less than one subframe if beam sweeping scenario is applied.

The definition of paging occasion in TS36.304 release8 is copied in Annex A. According to the table listed in Annex A, we can find some POs are continuous distribution for both FDD and TDD in LTE. Based on Observation 2, some POs overlap neighboring POs.

Observation 3: Some POs are continuous distribution for both FDD and TDD in LTE. Based on Observation 2, some POs overlap neighboring POs.

There has obvious contradiction between varied PO length and fixed paging frame if we reuse the LTE-like equation on paging frame definition in NR. For NR beam sweeping scenario, if we define the maximal number of PO in one PF based on the minimum length of PO, the neighboring POs still can overlap each other in some cases. If we define the maximal number of PO based on the maximal length of PO, many subframes in a paging frame will be wasted in minimum PO length case. Due to the PO length is SCS and carrier frequency dependent, if we define different PO pattern table in a paging frame for different PO length, it will be too complex and undesirable for forward compatibility if the type of PO length increases in the future.

In LTE, the paging mechanism is unified in the whole system. Different cell can adjust the paging related system parameters to achieve cell level flexible paging scheduling. Like LTE, a unified paging mechanism should be considered in NR.

Proposal 1: Like LTE, a unified paging mechanism should be considered in NR.

In LTE, PF is UE specific in which UE can find its PO. In NR, the function requirement of PF also exists. Like LTE, the concept of UE specific PF should be reused in NR.

Proposal 2: Like LTE, the concept of UE specific PF should be reused in NR.

In LTE, the PO location is static configured within one frame. Because some special subframes can’t be used for paging, the static PO location table mechanism can avoid the special subframe and the UL only subframe. But for NR, these restriction conditions are removed. Paging message can appear in any slot or any subframe, it’s all up to system scheduling. In NR, the minimum scheduling unit is OFDM symbol and the concept of frame is mainly used to achieve system synchronization. There is no strong view to limit one PO within one frame in NR.

Proposal 3: In NR, one PO can cross neighbouring frame.

Some POs may cross neighbouring frame while others may not, but for both of the two cases we think the PO calculation mechanism should be the same from UE perspective. Some concept enhancement is needed for UE specific PF.

Proposal 3a: If one PO crosses neighbouring frame, UE specific PF means UE specific PO starts from this frame.

In LTE, there are many subframes which are not used for paging. In NR, some slots/subframes between two neighbouring POs can be used for DL data transmission. We can’t assume all the slots/subframes in one DRX cycle are used for paging purpose. There is no direct relationship between the length of PO and DRX cycle.

Proposal 4: DRX cycle is not always multiple times of one PO length.

Proposal 4a: DRX cycle is not always multiple times of one beam sweeping duration.

In every DRX period, if we can find a fixed timer reference point and all the POs location are set based on this fixed time reference point, we can find a easy way to design a  PO location definition which will be applicable to all the PO length types. A periodic symbol SS block has been introduced in NR, which can be regarded as the fixed time reference point.

In NR, the periodicity of SS block can be {5, 10, 20, 40, 80, 160} ms. In RAN2 NR AdHoc 0118 meeting, we have agreed idle mode/inactive mode DRX cycle configuration in NR should take the default DRX cycle parameter in LTE as baseline [3]. In other words, the periodicity of paging message is integer multiples of SS periodicity

Proposal 5: The periodicity of paging message is integer multiples of SS periodicity.

A relatively fixed time reference point has been found in the beginning of each DRX cycle from the analysis above. An offset can be introduced to define the location of each PO within a DRX cycle, figure 1 illustrates such a PO location definition. The details of PO location definition is provided here as per request from other companies during last meeting discussion.

Figure 1: PO location definition based on SS burst

PF and PO can be separately derived from following formula:

PF:

PO:

The meaning of parameters in the formula above is listed below:

Table 2 the meaning of parameters in the formula

| Parameter | Meaning |
| --- | --- |
| T(ms) | T is determined by the shortest of the UE specific DRX value, if allocated by upper layers, and a default DRX value broadcast in system information. If UE specific DRX is not configured by upper layers, the default value is applied |
|  | UE specific PF, which means UE specific PO starts from this frame |
|  | The starting SFN number of any DRX cycle , which meets , k is the starting SFN number of the first DRX cycle in the system |
| UE_ID | SUPI (or 5G-S-TMSI or 5G-IMSI or truncated 5G-S-TMSI) mod 1024 |
|  | Physical layer parameter, which means the total slot number within one frame. This parameter is specific, where is the subcarrier spacing. |
| (slot) | The offset between two neighbouring POs within a DRX cycle, which is configurable by system information. =Z slot |
|  | The first slot number of UE specific PO within UE specific PF. |

From RAN1 agreements [4], there has QCL between SS Blocks, Paging DCIs and Paging message which means UE is not required to soft combine multiple Paging DCIs within one PO. So UE can know which slot/ slots in its specific PO should be monitored based on the calculated and the pre-acquired QCL information.

Proposal 6: An OffsetPO should be introduced to define the location of each PO within a DRX cycle.

Proposal 6a: OffsetPO is explicitly broadcast in the RMSI and the value of OffsetPO is configurable.

Proposal 7: NR should consider this SS burst based PO calculation mechanism.

Conclusion

According to the description above, we’d like to make a conclusion:

Observation 1: In NR, PO length is carrier frequency and SCS dependent.

Observation 2: In NR, PO length is usually no less than one subframe if beam sweeping scenario is applied.

Observation 3: Some POs are continuous distribution for both FDD and TDD in LTE. Based on Observation 2, some POs overlap neighboring POs.

Proposal 1: Like LTE, a unified paging mechanism should be considered in NR.

Proposal 2: Like LTE, the concept of UE specific PF should be reused in NR.

Proposal 3: In NR, one PO can cross neighbouring frame.

Proposal 3a: If one PO crosses neighbouring frame, UE specific PF means UE specific PO starts from this frame.

Proposal 4: DRX cycle is not always multiple times of one PO length.

Proposal 4a: DRX cycle is not always multiple times of one beam sweeping duration.

Proposal 5: The periodicity of paging message is integer multiples of SS periodicity.

Proposal 6: An OffsetPO should be introduced to define the location of each PO within a DRX cycle.

Proposal 6a: OffsetPO is explicitly broadcast in the RMSI and the value of OffsetPO is configurable.

Proposal 7: NR should consider this SS burst based PO calculation mechanism.

References

- RAN1 NR AdHoc #3 Chairman notes

- RAN1#90bis Chairman notes

- RAN2 NR AdHoc #0118 Chairman notes

- RAN1 NR AdHoc #1801 Chairman notes

- Annex

PF is given by following equation:

SFN mod T= (T div N)*(UE_ID mod N)

Index i_s pointing to PO from subframe pattern defined in 7.2 will be derived from following calculation:

i_s = floor(UE_ID/N) mod Ns

The following Parameters are used for the calculation of the PF and i_s:

-	T: DRX cycle of the UE. T is determined by the shortest of the UE specific DRX value, if allocated by upper layers, and a default DRX value broadcast in system information. If UE specific DRX is not configured by upper layers, the default value is applied.

-	nB: 4T, 2T, T, T/2, T/4, T/8, T/16, T/32.

-	N: min(T,nB)

-	Ns: max(1,nB/T)

-	UE_ID: IMSI mod 1024.

FDD:

| Ns | PO when i_s=0 | PO when i_s=1 | PO when i_s=2 | PO when i_s=3 |
| --- | --- | --- | --- | --- |
| 1 | 9 | N/A | N/A | N/A |
| 2 | 4 | 9 | N/A | N/A |
| 4 | 0 | 4 | 5 | 9 |

TDD (all UL/DL patterns):

| Ns | PO when i_s=0 | PO when i_s=1 | PO when i_s=2 | PO when i_s=3 |
| --- | --- | --- | --- | --- |
| 1 | 0 | N/A | N/A | N/A |
| 2 | 0 | 5 | N/A | N/A |
| 4 | 0 | 1 | 5 | 6 |