3GPP TSG RAN WG2 Meeting #101     	                                     R2-1803125

Athens, Greece, 26th February – 2nd March 2018	                (Revision of R2-1800986)

Agenda Item:	10.4.5.4

Source: 	Sony

Title:	Service based cell reselection

Document for:	Discussion

# Introduction

In 3GPP RAN2 NR-Adhoc#1 meeting, the following agreements on cell selection/reselection have been reached [1].

Agreements

1	As in LTE, UE can prioritise a frequency based on service. On the selected frequency the UE attempts to camp on the best cell.

2	Suitability criterion: Cell quality is above a threshold; Cell is not barred; Cell belongs to selected/R (E) PLMN. Other conditions (if any) are FFS.

3	Cell broadcasts (e.g. in minimum SI) the service(s) supported by it.

FFS for which services (e.g. MBMS, CSG, V2X, URLLC) we apply this mechanism.

FFS how this might apply for the case of network slices.

FFS whether a cell may also broadcast service(s) supported in neighbouring frequencies.

In 3GPP RAN2 NR-Adhoc#1801 meeting, the following agreements on service related cell selection/reselection have been reached [2].

Agreements

1	The idle UE considers the frequency to be the highest reselection priority if the idle UE prefers to receive its interested service while camping on a frequency on which it is provided, as in LTE. FFS which service is applied for this rule.

2	Prioritization of multiple services when the idle UE cannot receive all is up to UE implementation.

In this contribution, we further elaborate our perspectives on the service based cell reselection.

# Discussion

## System information to support service based cell reselection

According to the agreement, cell will broadcast the service(s) supported by it. The concerns on broadcasting such services related information in system information are mainly on

- It is not sure which services information should be provided in the system information.

- If services related information is provided in system information, how to deal with the forward compatibilty issue.

Although these concerns make sense to some extent, it is beneficial to broadcast service related information in SI in order to

- Save UE power during cell selection/reselection procedure as UE could prioritize the frequencies based on the provided services.

- Reduce the delay of the UE to access the expected services as UE won’t have to be redirected to another cell if its previously accessed cell can’t provide the desired service.

- Ease network load on redirecting the UE to a suitable cell once the UE finds that the cell can’t provide the desired services after access to it.

Therefore we support to broadcast service related information in SI. In order to address above concerns and take the trade-off between signalling overhead and forward compatibility into consideration, one indication can be introduced in system information e.g. in minimum SI to indicate whether the cell is a service dedicated cell or not.

With this indication, from network point of view,

- If the cell is a serivce dedicated cell e.g. can only provide one specific service like MBMS or CSG etc, it will set the indication as true in its SI.

- If the cell is not a service dedicated cell e.g. can support various services, it will set the indication as false in its SI.

And from UE point of view, after it camped on a suitable cell

- If the UE is a service dedicated UE e.g. is only interested in one specific service, it will read the indication in SI

  - If the indication states that the cell is a service dedicated cell

    - If the cell can provide the expected service, UE will prioritise this cell for cell reselection.

    - If the cell can not provide the expected service, UE may tune to another frequency depending on the setting of IFRI bit.

  - If the indication states that the cell is not a service dedicated cell, UE could further read related service information to decide whether prioritise this cell for cell reselection.

- If the UE is not a service dedicated UE, it may or may not read the indication in SI

On how to provide the supported services information in SI, several options can be adopted e.g. a bitmap in the minimum SI or be provided via on demand manner according to UE’s request which can be discussed later.

Proposal 1: One bit to indicate a cell is a service dedicated cell or not should be introduced in system information. Where to include the one bit indication can be discussed further.

## Services supported on neighbouring frequencies

## There was discussion on whether to broadcast service(s) supported in neighbouring frequencies. We think it is better to avoid broadcasting such information unless identified necessary, as it will increase the control signalling overhead, introduce the redundant signalling and is not a forward compatible method.

## Instead, if the UE is allowed to read service related system information from neighbour frequency cell, the UE could make proper service based cell reselection afterwards based on the acquired system information from neighbour inter-frequency cells.

Proposal 2: UE is allowed to read service related system information from neighbour frequency cells and then make service based cell reselection afterwards if necessary.

# Conclusion

Therefore we propose that

Proposal 1: One bit to indicate a cell is a service dedicated cell or not should be introduced in system information. Where to include the one bit indication can be discussed further.

Proposal 2: UE is allowed to read service related system information from neighbour frequency cells and then make service based cell reselection afterwards if necessary.

# References

- RAN2 NR Adhoc#1 Chairman Notes.

- RAN2 NR Adhoc#1801 Chairman Notes.