3GPP TSG-RAN WG2 Meeting#101 	R2-1803292

# Athens, Greece, 26th February  – 2nd March 2018

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.321 | CR | 1222 | rev | - | Current version: | 14.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarifications on number of SL processes for V2X communication |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Intel Corporation |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_V2X-Core |  |  |  |  |  |  | Date: |  | 2018-02-08 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-14 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | 1) The number of SL processes with the Sidelink HARQ Entity for V2X transmission is not clear. 2) The maximum number of SL processes for transmissions of multiple MAC PDUs without sensing is not clear. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Section 5.1.14.1.2.1: Added explicit indication of up to 8 SL processes with the Sidelink HARQ Entity supported for V2X transmission. - Section 5.1.14.1.2.1: Clarified that the maximum number of SL processes with the Sidelink HARQ Entity for transmissions of multiple MAC PDUs is 2 and removed reference to sensing. Impact analysis Impacted functionality: SL HARQ operation for V2X communication Inter-operability: UE not implementing this functionality may assume that the number of total SL processes for V2X transmission is greater than 8. UE not implementing this CR may assume that the maximum number of SL processes with the Sidelink HARQ Entity for transmissions of multiple MAC PDUs is more than 2 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Specification is not complete and the number of SL process for V2X transmissions is ambiguous. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.14.1.2.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | x | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

CHANGE START

#### 5.14.1.2	Sidelink HARQ operation

##### 5.14.1.2.1	Sidelink HARQ Entity

There is one Sidelink HARQ Entity at the MAC entity for transmission on SL-SCH, which maintains a number of parallel Sidelink processes.

The number of transmitting Sidelink processes associated with the Sidelink HARQ Entity is defined in [8]. Up to 8 Sidelink processes with the Sidelink HARQ Entity are supported for V2X sidelink transmission. A sidelink process may be configured for transmissions of multiple MAC PDUs. For transmissions of multiple MAC PDUs, the maximum number of transmitting Sidelink processes with the Sidelink HARQ Entity is 2.

A delivered and configured sidelink grant and its associated HARQ information are associated with a Sidelink process.

<Text Removed Here>

CHANGE END