3GPP TSG-RAN WG2 #101	R2-1803628

Athens, Greece, 26th February – 2nd March 2018

Agenda Item:	10.4.4.2

Source:	Qualcomm Incorporated

Title:	Avoiding bridges to nowhere

Document for:	Discussion, Decision

# Introduction

In this contribution, we explain the problematic state of UE capability discussions for NR, when we combine RAN2 and RAN4 views on this topic.

We propose multiple solutions that can solve the real problem, which is to minimize or manage the size of the UE categories, and its processing.

# Discussion

The LTE capabilities grew to exceed the current 8k limit of PDCP packet sizes for some configurations. With the addition of NR, the size of the UE capabilities is forecast to be a problem as well.

3GPP RAN2 has already agreed to introduce LTE mechanism (query of bands) in NR from the start, but has also taken the additional step of splitting the capabilities into baseband and RF capabilities.

The current RAN2 UE RF capability (e.g. CA band combination) and baseband capabilities (e.g. modulation scheme) are separated:

- Baseband capability is called Baseband Processing Capability (BPC)

  - BPC includes bandwidth class information (TBD), MIMO

  - Potentially numerology, and other capabilities that are not dependent on the RF band.

- RF capabilities are divided in two groups (RF Capability):

  - Per band capabilities (including MIMO)

  - Per band combination capability

To reconstruct the UE’s capability, the network has to bridge the BPC and the RF capability via the (still undefined) bandwidth class.

## NR Standalone

For NR standalone, a given BPC is applicable in `band combinations which consist of bands 1) with equal bandwidth class, and 2) equal or higher MIMO layer capability. This applicability rule also applies to fallback combinations which may not be signalled.

“Reduced” MIMO capability can be provided in BPC.

There is also an additional proposal to introduce MIMO restriction on a subset of bands, regardless whether there are additional bands in the configured band combination.

Figure 1 NR BPC and RF dependence

In Figure 1, we show the different inputs, and the derived bridges are the lines between the BPC and the RF capabilities.

## EN-DC BPC-RF dependency

For EN-DC EN-DC, the same concept is being proposed. So, we will have to define the UE’s baseband capabilities for LTE and perform the same exercise on combined LTE/NR baseband capabilities.

The same split is done for LTE part of MR-DC UE capability.

The dependency between LTE BPCs and NR BPCs is signaled as part of UE capability and comprehended by eNB and gNB.

## RAN4 status

In the meantime, RAN4 is still discussing the definition of the bandwidth class. Many proposals are attempting to inherit the UE definition of the LTE bandwidth class

# The problems

The types of problems we will face depend on the final agreements RAN2/RAN4 will make this week. This is a list of potential problems depending on the chosen solutions.

## Lack of forward compatibility of UEs

Without major change in direction in RAN2 and RAN4, we observe that:

- The UE cannot limit the expression of its configurations/capabilities, to match exactly the requested band combinations and configurations from the operators and OEMs.

- Depending on RAN4’s agreement on BCS, the UE may not be able to limit the allowed bandwidth configurations.

### Matching requested UE configurations

For example, assume the requirement is to support these configuration:

- CA combination 1

  - N1; DL-BCL=a,#DL Layers= 2, with UL-BCL=a, with “baseband-layers-numerology-etc”-bpc0

  - N2; DL-BCL=c, #DL Layers= 4, without uplink, with “baseband-layers-numerology-etc” -bpc1

- CA combination 2

  - N3; DL-BCL=a, #DL Layers=4, with UL-BCL=a, with “baseband-layers-numerology-etc” -bpc1

  - N4, DL-BCL=a, #DL Layers=2, without uplink, with “baseband-layers-numerology-etc” -bpc0

  - N5, DL-BCL=c, #DL Layers=2, without uplink, with “baseband-layers-numerology-etc” -bpc0

- Intra-band CA:

  - N5, DL-BCL=c, #DL Layers=4, with UL-BCL=a, with the support of “baseband-layers-numerology-etc” -bpc1

Based on the above, the current design of the BPC/RF split would require the following to be signalled:

- BPC

| “Virtual” Band 1 |  |  |  | “Virtual” Band 2 |  |  |  | “Virtual” Band 3 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DL-BCL | #DL Layers | UL-BCL | “baseband-layers-numerology-etc” | DL-BCL | #DL Layers | UL-BCL | “baseband-layers-numerology-etc” | DL-BCL | #DL Layers | UL-BCL | “baseband-layers-numerology-etc” |
| c | 4 | a | bpc1 |  |  |  | bpc0 |  |  |  |  |
| a | 2 | a | bpc0 | c | 4 |  | bpc1 |  |  |  |  |
| a | 4 | a | bpc1 | a | 2 |  | bpc0 | c | 2 |  | bpc0 |

Table 1 BPC table example

- RF per band, downlink support of maximum class/Layers

| Band | Class | DL-Layers |
| --- | --- | --- |
| n5 | c | 4 |
| n2 | c | 4 |
| n1 | a | 2 |
| n4 | a | 2 |
| n3 | a | 4 |

- RF per band combination

| RF Band 1 |  |  | RF Band 2 |  |  | RF Band 3 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | DL-BCL | UL-BCL | Band | DL-BCL | UL-BCL | Band | DL-BCL | UL-BCL |
| n5 | c | a |  |  |  |  |  |  |
| n1 | a | a | n2 | c |  |  |  |  |
| n3 | a | a | n4 | a |  | 2 | c |  |

Then we need to support all the original configurations, but also additionally the following two new ones. They are the result of the product of the RF and baseband capabilities:

- New config 1:

  - n1; DL-BCL=a; #DL Layers=2; UL-BCL=a; with “baseband-layers-numerology-etc”-bpc1

  - n2; DL-BCL=c; #DL Layers=2; no uplink, with“baseband-layers-numerology-etc”-bpc0

- New config 2:

  - N3; DL-BCL=a; #DL Layers=2; UL-BCL=a; with “baseband-layers-numerology-etc”-bpc0

  - N5; DL-BCL=c; #DL Layers=4; no uplink, with “baseband-layers-numerology-etc”-bpc1

Qualcomm has applied the proposed signalling to our existing LTE capabilities, and we found the following:

- Around ~25% of existing configurations would be over-reported. ie, we would need to test for configurations that the operator did not request, and where the network vendors may not support.

- An additional ~60% of new configurations would magically appear and would need to be supported, verified and tested.

This is a partial heatmap, showing visually how new unrequested configurations show up (in red), or are over dimensioned (in orange), while the green represents exact UE capabilities. The horizontal lines are RF capabilities, while the vertical lines are baseband capabilities. The cells are valid intersections based on matching bandwidth classes.

Figure 2 Partial heatmap of new configurations to test

Observation 1: The additional design and testing required by the current state of the specifications is not acceptable. No company will prioritize testing of configurations that nobody has asked for, over the continuous development work.

### Matching requested bandwidth configurations

RAN4 is considering the need to support the BCS concept (Bandwidth Combination Set, which defines subsets of configurations the UE needs to support for CA in a certain band or band combination). However, with or without BCS, we still have a problem derived from the previous analysis, when applied to the bandwidths.

First, we should note that in any proposal, RAN4 specifications limit the combinations that the UE is allowed to support, as it has been in LTE. In LTE, even if the UE supports bandwidth class c on a number of bands, RAN4 defines different CC bandwith combinations that the UE shall support for every one of those bands, and they can be different. Thus, in Figure 3, RAN2 signalling will be a superset of RAN4 defined test cases.

RAN4 will continuously define new band combinations subsets, which may be targeting new markets with different numerologies and layers, but, maybe the same bandwidth class.  To avoid having dynamic UE capabilities depending on the evolution of RAN4 specifications, we would either need to make RAN4 specifications release dependent (see next section), or undo the current agreement in RAN2 about linking the RF and BPC with bandwidth class.

This was not a problem in LTE, because of the last part of the second bullet above: LTE allowed the UE to express what baseband it supports per RF band, per RF band combination.

Allowing the UE to explicitly list the supported bandwidths per band configuration is the only workable solution for NR as well. That list of supported bandwidths is dependent on the number of layers and numerologies, in that particular bandwidth combination. Without such linking, we fall to a similar problem as in Figures 2 and 3.

Keeping the BCS concept by itself is not enough, because the same BCS0 for a legacy bandwidth combination, could tomorrow be mapped to a new required baseband class with a different numerology, for a new band combination. When that happens, the UE suddenly has to verify the application of the new numerology to all the “intersecting” RF bandwidths, as demonstrated in the example in 3.1.1.

The problem is visually expressed inFigure 3:

Figure 3 Forward compatibility danger

## Potential impact to RAN4

To address the problem above, we either have to make changes to RAN4 or to RAN2. Let us start by examining what RAN4 could do:

- RAN4 could increase the number of RF band combinations to be defined and studied:

  - With more bands defined, the introduction of any numerology in a certain combination, would require a lot of new studies for all other combinations that match the bandwidth classes combination.

  - This is deemed unfeasible by our RAN4 colleagues, as RAN4 has been historically backlogged.

- RAN4 specifications can become release dependent.

  - Breaking years of release independent specifications of GSM/UMTS/LTE, and

  - Making the network’s job very difficult in figuring out what are the actual UE capabilities. The network will have to refer to historical RAN4 specifications (not just the latest), to understand older UE capabilities.

  - UE vendors wanting to add a new band combination recently defined by RAN4, would have to guarantee the operation of any new baseband classes with all intersecting RF band combinations and their evolution.

- RAN4 can keep the use of bandwidth class and bandwidth combination set.

  - This still is problematic for UE vendors as mentioned in the previous section, where any new numerology will have to be checked to work in any existing matching combination of bandwidth classes.

Observation 2: RAN4 doesn’t have the time to define additional requirements, so the UE capabilities will need to be bound but what RAN4 has defined at the time of manufacturing. Other choices have negative impacts that are avoidable.

## Impact to UE vendors

Depending on which route RAN4 would take, UE vendors would still have to either:

- Additional testing that is not really required by any customer, or

- When trying to add compliance to a new release/specification date, the UE may have to pick up new configurations that it doesn’t need, in case we go with release dependent RAN4 specifications.

There is an added risk to EN-DC:

- The definition of EN-DC baseband capabilities would result into the same problem for LTE. This may become a  huge issue, if we need to nearly double the amount of additional testing, for the bands supporting EN-DC – when no IOT will be present.

Observation 3: There is a special risk for the LTE part of EN-DC, because of the BPC definition of LTE.

## Impact to network vendors

Impact to the network vendors:

  - Figuring out the UE’s capabilities now requires an exhaustive cross multiplication of the baseband and RF capabilities, potentially restricted by per band MIMO and layer restrictions, and potentially by RAN4 release dates (above).

  - In order to figure out all of the UE capabilities, the network would need to evaluate a very large number of combinations, which grows in the order of:

    - ~Cst* number of BPC * number of RF * FACT(Number of Carriers)*[(Average Number of L2 constraints) ^NCarriers * (Average Number of numerology constraints)]^NCarriers .

  - This requires multi-level recursion for full list (or lots of for loops), and very intricate optimizations to minimize the number of dead leaves.

  - Alternatively, the network could save the results once, per type of UE, per area, per requested set of bands. That’s an alternative solution, but still requires the additional development and constant maintenance of such databases.

Table 1 is an example shows how fast the number of iterations grows:

Table 2 Network iterations to figure out the UE’s capabilities

Observation 4: In any case, there is additional work in the network to manage and make sense of the new RAN2 capability structure.

# Proposals

Having gone through the problems with current situation, we do want to optimize the size of the UE capabilities, which is the real problem. In the process, we want to confirm that:

Proposal 1: Agree that 3GPP/RAN4 will only introduce requirements requested by operators. ie RAN2 signalling structure shall not result in work that no operator has requested.

Splitting the capabilities in baseband and RF was a tool to achieve the capability size savings, but that’s not the only tool. RAN2 should be open to revisit this decision based on the above extensive analysis with the large impact to testing and thus deployment schedules.

The following proposals achieve the same or better capability size savings:

Proposal 2: In order to ensure that the UE is built and tested for exactly what the operators have requested,  RAN2/RAN4 shall agree on 1 of these solutions:

- Option 1: RAN2 shall define signaling allowing the UE to signal exact capabilities meant to be deployed, and that is not dependent on the version of the RAN4 specifications.

  - The UE defines baseband capabilities per adjacent CC, per band (but not per band combination)

  - The RF capabilities are per band per band combination, and refer to a list of UE defined baseband capabilities per band above.

  - In this case, RAN4 could even avoid defining BCS.

- Option 2: RAN4 specifications shall keep the BCS- Bandwidth Combination Set for NR per RF Band combination (as in LTE) and add Layers and numerology considerations to the bandwidth class definition.

- Option 3: RAN4 specifications shall keep the BCS- Bandwidth Combination Set for NR per RF Band combination (as in LTE) and add Layers and numerology considerations to the Bandwidth combination set(BCS) definitions.

# Details of the proposals

All the proposals revolve around the idea that the triplet (Bandwidth, Numerology, Number of layers) is crucial in defining the UE’s envelop.

## Option 1: Explicit RAN2 signalling

This proposal “looks” similar to past proposals, but, it is different in important details. In this proposal, the UE defines “baseband capabilities for adjacent carriers”, and references these in the RF capabilities, per band, per band combination.

The UE lists a “list of bandwidths” {bwi) associated with a modulation and subcarrier spacing. Then, the UE can reference these building blocks from the RF capabilities.

### Signalling details

Fyi to RAN4, Also input to RAN1 discussion on capabilities.

- For RF band combinations,

  - Per band combination

    - Per band per DL band combination, including intra-band non-contiguous:

      - List of DL baseband configuration IDs (defined below)

      - Band number (or reference to a band combination per band combination)

      - Reference to a List of supported uplink band combinations

      - BCS

    - Per band per UL band combination:

      - Bandwidth Class

      - …

  - Per band, or group of bands (repeat entries allowed):

    - Downlink Band number(s)

    - Maximum Aggregate DL Bandwidth

    - Maximum Number of Layers on the downlink

  - Per band, or group of bands (repeat entries allowed):

    - Uplink Band number(s)

    - Maximum Aggregate UL Bandwidth

    - Maximum Number of Layers on the uplink (Do we need this?)

- For baseband (virtual) band,  (referenced for RF)

    - DL baseband configuration ID (implicit or explicit numbering)

    - Scaling factor (if we follow to the letter 38.306, but may be better below per carrier.)

    - Per Carrier per virtual band) (B)

      - Numerology

      - DL-Bandwidth

      - Maximum number of layers

      - Maximum modulation order

      - Scaling factor?

      - UL Bandwidths, Max number of layers, MIMO,…

      - tm?…

## Signalling size comparison

This solution is comparable in size to the current RAN2 solution, as instead of bandwidth class on both sides, we use the index of BPCs in the RF.

## Option 2: RAN4 to consider baseband in the bandwidth class

Additional granularity is required in the NR CA combination tables to capture baseband limitations when combining NR component carriers.

In this option, RAN4 defines the bandwidth classes in conjunction with baseband, but could still group multiple capabilities together. The important is to couple the triplet as discussed previously.

This triplet can still be used to link the BPC and RF capabilities as per the current RAN2 structures.

Figure 4 Option 2

## Option 3: RAN4 to consider baseband in the BCS

In this option, the bandwidth class is kept as is, so is the RAN2 signalling, but each CA Configuration defines its BCS in terms of layers and numerologies that are expected to be supported as shown in Figure.

Figure 5 Option 3

The CA table examples are repeated outside the figure for clarity:

| NR CA configuration / Bandwidth combination set |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA Configuration | Uplink CA configurations (NOTE 4) | NR Bands | 5 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 100 MHz | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
| n10A |  | N10 |  |  | 2L,15K 4L,30K |  |  | 2L,30K |  | 0 |
| … |  | … |  |  | … |  |  | … |  | … |
| CAN20B |  | N20 |  | 2L,15K |  |  |  |  |  | 0 |
|  |  |  |  |  |  | 2L,30K |  |  |  |  |
|  |  | N20 |  |  | 4L,30K |  |  |  |  |  |
|  |  |  |  |  | 4L,30K |  |  |  |  |  |

| CAN30A-30A |  | N30 |  |  |  |  |  | 2L,30K |  | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | N31 |  |  |  |  |  | 2L,30K |  |  |
|  |  | N30 |  |  |  |  |  | 4L,30K |  | 1 |
|  |  | N31 |  |  | 2L,30K |  |  |  |  |  |
| CAN30A-30A |  | N30 |  |  | 4L,30K |  |  |  |  | 0 |
|  |  | N31 |  |  |  |  |  | 2L,30K |  |  |

| CAn40A- n41A- n42A- n43A |  | N40 |  | 2L,15K | 2L,15K |  |  | 2L,30K |  | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | N41 |  | 2L,15K | 2L,15K |  |  | 2L,30K |  |  |
|  |  | N42 |  | 2L,15K | 2L,15K |  |  | 2L,30K |  | 1 |
|  |  | N43 |  | 2L,15K | 2L,15K |  |  | 2L,30K |  |  |

### Signalling in RAN2

As currently agreed in RAN2, with keeping the notion of BCS for the evolution of RAN4 requirements.

# Conclusion

We observe that:

Observation 1: The additional design and testing required by the current state of the specifications is not acceptable. No company will prioritize testing of configurations that nobody has asked for, over the continuous development work.

Observation 2: RAN4 doesn’t have the time to define additional requirements, so the UE capabilities will need to be bound but what RAN4 has defined at the time of manufacturing.

Observation 3: There is a special risk for the LTE part of EN-DC, because of the BPC definition of LTE.

Observation 4: In any case, there is additional work in the network to manage and make sense of the new RAN2 capability structure.

Proposal 1: Agree that 3GPP/RAN4 will only introduce requirements requested by operators. ie RAN2 signalling structure shall not result in work that no operator has requested.

Proposal 2: In order to ensure that the UE is built and tested for exactly what the operators have requested,  RAN2/RAN4 shall agree on 1 of these solutions:

- Option 1: RAN2 shall define signaling allowing the UE to signal exact capabilities meant to be deployed, and that is not dependent on the version of the RAN4 specifications.

  - The UE defines baseband capabilities per adjacent CC, per band (but not per band combination)

  - The RF capabilities are per band per band combination, and refer to a list of UE defined baseband capabilities per band above.

  - In this case, RAN4 could even avoid defining BCS.

- Option 2: RAN4 specifications shall keep the BCS- Bandwidth Combination Set for NR per RF Band combination (as in LTE) and add Layers and numerology considerations to the bandwidth class definition.

- Option 3: RAN4 specifications shall keep the BCS- Bandwidth Combination Set for NR per RF Band combination (as in LTE) and add Layers and numerology considerations to the Bandwidth combination set(BCS) definitions.