3GPP TSG-RAN WG2 #101	Tdoc R2-1803772

Athens, Greece, 26th February – 2nd March 2018

Agenda Item:	10.03.04.03

Source:	Ericsson

Title:	QoS Flow Remapping

Document for:	Discussion, Decision

# Introduction

RAN2#AH January 2018 agreed:

Agreements

=>	RAN should be able to move/remap a QoS flow from one DRB to another DRB.

=>	RAN should have an option to perform handover by preserving part of the DRB configuration and QoS flow to DRB mapping complemented by established/removed DRBs with corresponding changes in the QoS flow mapping.

Agreements

=>	In-order delivery should be ensured during flow re-mapping

This contribution discusses about in-order delivery impact on gNB and UE.

# Discussion

According to the agreements from the last RAN2 meeting, RAN should be able to move / remap from QoS Flows from DRB to another while maintaining order of the consecutive packets inside one QoS Flow.

When considering a case where Flow is mapped from source DRB 1 to target DRB 2, one main approach can be observed: target DRB buffers data until source DRB has end finishing data transmission.  Whether data should be buffered on transmitter or receiver is yet to be decided. In general, three options can be observed:

- Option 1: Buffer the data on transmitter SDAP entity

- Option 2: Buffer the data on transmitter DRB entity

- Option 2: Buffer the data on receivers DRB entity

- Option 3: Buffer the data on receivers SDAP entity

When taking previous agreements into account, the SDAP entity does not have the concept of queues or buffers. Additionally, the SDAP entity does not maintain any sequence number. Furthermore, as captured in stage 2 PDCP is not aware of QoS Flows as a result of establishing the SDAP protocol layer.

Figure 1: Packet Buffering options. Options 1 and 4 are not valid as the SDAP does not have buffers.

Figure 1 describes the buffering options and taking into consideration the agreed details. As it can be seen from the figure 1, the QoS Flow data can only be buffered on DRB level. When QoS Flow is mapped from source DRB to target DRB, the PDCP entity changes, which results that the sequence number state will be different between source and target node. As a result, the target PDCP is not able to guarantee in-sequence delivery on TX side in the case where target DRB has more than one QFI mapped into it. As the PDCP is not aware of the QoS Flows, the in-sequence delivery between two DRBs on QoS Flow level does not work between PDCP entities.

The source PDCP cannot deliver not-yet ACKed packets to target PDCP entity if there is more than one QFI mapped to the target DRB as the PDCP SN space on target is maintained by another entity.

PDCP does not have a concept of QoS Flows and therefore it cannot enforce in sequence delivery per QoS Flow level.

The only option to guarantee the in-sequence delivery for a QoS flow on a DRB is to buffer data on the target node until source node has finished the data transmission for all QoS flows. However, in the cases where packets are lost over the air, significant delay may be introduced in the flow remapping procedure as the source needs to handle retransmissions before it can notify target that the data has finished. I.e. transmission of a remapped QoS flow on a new DRB can begin only after:

- Delivered the PDCP PDUs

- Received Status Report that indicates the delivery of PDUs (either on PDCP or RLC level).

- Unblock the target DRB

Because of the default DRB concept, flow remapping may be commonly used in some network implementations. By doing so, this comes with requiring UE to support increased buffering capability. Requiring the system to consume more resources, such as memory and power budget, seems unreasonable.

Note that the motivating use case for creating a comprehensive mechanism builds on the basis of if QoS flow relocation is likely and commonly used. In our mind, the network typically would only group QoS flows within a single DRB where the packet forwarding treatment are equal or very similar, i.e the latency and error requirements can be met by the treatment of that DRB for all those QoS flows. Should the QoS treatment be different, the network would configure the QoS flow to DRB mapping thereafter.

As 5GC have N2/NG-C signaling to gNB when TFT/SDF filters are delivered to UE via N1 signaling, the gNB has readily information about all possible QFIs and associated QoS profiles known in a PDU session and can thus in all cases make an appropriate QoS to DRB mapping from start. I.e. the network can always establish multiple DRBs from, so that QoS flow handling using only a “default DRB” can be eliminated.

Requiring RX buffering on QoS Flow remapping from UE on DL heavy traffic seems to be an unreasonable requirement.

Scenarios requiring complex QoS flow reallocation schemes for in order delivery can be avoided through network implementation.

The receiver buffering with end-marker solution will require UE to have significant buffering for high data rate services. Additionally, the source SDAP entity must be aware of any over the air losses before it may declare the end marker. Otherwise the receiver may receive the end maker, open the target DRB for data transmission, resulting that packets not yet delivered would be out-of-order.

RX buffering and an end-marker solution may not guarantee out of order delivery in all cases.

It is important to consider that the Flow mapping mechanism should support both inter-cell flow mapping as well as intra-cell flow mapping. For instance, upon handover, the target would first copy the source DRB configuration before it remaps the QoS Flows to complete the handover. The remapping would be done with intra-cell flow remapping mechanism or by remapping the full DRB.

The QoS Flow remapping procedure should be used to remap flows after handover is finished.

In order delivery is guaranteed for QoS Flow mapping within one PDCP entity

If specification changes are needed, a DRB-level transmitter buffering solution is defined.

Exact transmitter buffering solution is left for implementation.

# Conclusion

In section 2 we made the following observations:

Observation 1	The source PDCP cannot deliver not-yet ACKed packets to target PDCP entity if there is more than one QFI mapped to the target DRB as the PDCP SN space on target is maintained by another entity.

Observation 2	PDCP does not have a concept of QoS Flows and therefore it cannot enforce in sequence delivery per QoS Flow level.

Observation 3	Requiring RX buffering on QoS Flow remapping from UE on DL heavy traffic seems to be an unreasonable requirement.

Observation 4	Scenarios requiring complex QoS flow reallocation schemes for in order delivery can be avoided through network implementation.

Observation 5	RX buffering and an end-marker solution may not guarantee out of order delivery in all cases.

Observation 6	The QoS Flow remapping procedure should be used to remap flows after handover is finished.

Based on the discussion in section 2 we propose the following:

Proposal 1	In order delivery is guaranteed for QoS Flow mapping within one PDCP entity

Proposal 2	If specification changes are needed, a DRB-level transmitter buffering solution is defined.

Proposal 3	Exact transmitter buffering solution is left for implementation.

# References

R2-1803404. Flow relocation. Qualcomm Incorporated. Nokia Shanghai Bell. Athens. Greece, 26th February - 2nd March 2018

R2-1802504. QoS Flow Remapping. Nokia, Huawei, HiSilicon, Nokia Shanghai Bell. Athens. Greece, 26th February - 2nd March 2018