3GPP TSG-RAN WG2 #101Bis	Tdoc R2-1805435

Sanya, P.R. of China, 16th – 20th April 2018

Agenda Item:	10.3.2.4

Source:	Ericsson

Title:	TX_NEXT in NR RLC

Document for:	Discussion, Decision

# Discussion

Currently TX_NEXT contains the latest assigned Sequence Number. When UEs do pre-processing and assign SN before a request from lower layers, the value which POLL_SN will have may be incorrect.

Further on, when TX_NEXT contains the latest assigned SN, there will be issues when RLC discard is applied to a packet for which a SN has been allocated. All packets which already had assigned SN (after packet discard), will need to have updated SN. However, this procedure cannot use the variable TX_NEXT as may point to the latest assigned SN and not submitted SN. Yet, the RLC transmitter must ensure that SN are consecutive, and no gaps are introduced.

Any solution should thus capture the following principles:

- Pre-processing should be supported; however, it should not be specified

  - This results in that when and how the UE calculates, updates, and sets the SN in preprocessing is up to the UE and should not be captured in the specs.

- RLC shall always deliver consecutive sequence numbers i.e. there shall be no SN gaps

Considering these two principles, the following would address those remaining issues:

- The specification does not indicate how and when the UE set the SN. This would be part of the UE internal processing. However, it should be clear that the RLC entity always shall deliver to lower layers PDUs with consecutive sequence numbers. The following text would need to be updated:

For each RLC SDU received from the upper layer, the AM RLC entity shall: - associate a SN with the RLC SDU equal to TX_Next and construct an AMD PDU by setting the SN of the AMD PDU to TX_Next; - increment TX_Next by one.

- Replace “TX_NEXT” by “the highest SN of the AMD PDU among the AMD PDUs submitted to lower layer” (except for the instances related to the SN setting). For example, the following text would need to be updated to:

When receiving a negative acknowledgement for an RLC SDU or an RLC SDU segment by a STATUS PDU from its peer AM RLC entity, the transmitting side of the AM RLC entity shall: - if the SN of the corresponding RLC SDU falls within the range TX_Next_Ack <= SN < the highest SN of the AMD PDU among the AMD PDUs submitted to lower layer: - consider the RLC SDU or the RLC SDU segment for which a negative acknowledgement was received for retransmission.

- Considering 1) and 2), including the principal of implementation aspects means TX_NEXT is of no real use any longer. Thus, the variable TX_NEXT should be removed from the specification.

Agree on 1), 2), and 3) as outlined above in this document

# Conclusion

Based on the discussion in section 1 we propose the following:

Proposal 1		Agree on 1), 2) and 3) as outlined above in this document

# References

R2-1803004, Correction on TX_Next - 1 for POLL_SN and the RLC SDU for retransmission

R2-1803163, Clarification of RLC poll handling for Dual Connectivity.