3GPP TSG-RAN WG2 #102	Tdoc R2-1806806

# Busan, South Korea, 21 – 25 May 2018

Agenda Item:	10.4.5.6

Source:	Ericsson

Title:	Paging in DCI only – Text Proposal to TS 38.331

Document for:	Discussion, Decision

# Introduction

RAN2 has agreed to support means for providing paging information in paging DCI through the following two agreements captured from the RAN2#101-bis meeting report.

Agreement

1:	Single bit is provided in paging message and in DCI (1 bit of the bit string provided by RAN1) to indicate system information change (i.e. no indication of SIB or SI message). As a consequence the UE reads SIB1 to determine what has changed. For idle mode case the UE also reads MIB.

Agreement

1	Single bit is provided in paging message and in DCI for warning messages (not a separate bit for CMAS and ETWS as previously agreed). UE immediately acquires warning messages after this indication.

This paper contains a text proposal to TS 38.331 based on the two RAN2 agreements above.

The DCI paging message is preferably defined in TS 38.331 according to the Direct Indication Information message definition in TS 36.331.

However, in order to better reflect the purpose of the message, a different name of the message is proposed, namely Paging Indication.

Paging information may be provided in the paging DCI transmitted on PDCCH. For this purpose, a Paging Indication message is defined in TS 38.331.

Since the number of reserved bits used for the Paging Indication message need to be defined in the DCI format used for paging (TS 38.212), RAN1 need to be informed of the Paging Indication message definition in TS 38.331. Therefore, a draft LS to RAN1 is proposed in [1].

# Text Proposal

## 6.x        Paging Indication

Paging Indication is transmitted on PDCCH using P-RNTI but without associated Paging message. Table 6.x-1 defines the Paging Indication, see TS 38.212 [xx, 7.3.1.2.1].

When bit n is set to 1, UE shall behave as if the corresponding field is set in the Paging message, see 5.3.2.3. Bit 1 is the least significant bit.

Table 6.x-1: Paging Indication

| Bit | Paging Indication |
| --- | --- |
| 1 | systemInfoModification |
| 2 | etwsAndCmasIndication |

# Conclusion

In this paper we propose the following:

Proposal 1	Paging information may be provided in the paging DCI transmitted on PDCCH. For this purpose, a Paging Indication message is defined in TS 38.331.

# References

- [1]	R2-1806809, [Draft] LS on paging information in DCI, RAN WG2 #102, Busan, South Korea, 21st – 25th May 2018.