3GPP TSG-RAN WG2 Meeting #102 	R2-187009

# Busan, Korea, 21th – 25th May 2018

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.321 | CR | 1280 | rev | - | Current version: | 15.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | PHR MAC CE for EN-DC |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  |  | Date: |  | 2018-05-10 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | It was agreed on RAN2#100 meeting that the MN provides the SN the range of SCellIndex to be used by signalling a start value and a stop value if the range. 32 SCellIndex range is used between the MN and the SN. The UE uses “Dual Connectivity PHR MAC Control Element supporting 32 serving cells with configured uplink” for PH reporting. But in the current TS36.321, it says “when the highest SCellIndex of SCell with configured uplink is less than 8, otherwise four octets are used.” This contradicts the agreement on RAN2#100. Hence, clarification is needed. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clarify that for EN-DC, always four octets CC bitmap are used. Impact analysis Impacted functionality: PHR operation. Inter-operability: - If the network is implemented according to the CR and the UE is not, the network cannot decode the PHR MAC CE successfully. - If the UE is implemented according to the CR and the network is not, the network also cannot decode the PHR MAC CE successfully. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The UE may use erroneous Dual Connectivity PHR MAC CE Control Element format. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.1.3.6b |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

Start of change

6.1.3.6b	Dual Connectivity Power Headroom Report MAC Control Element

The Dual Connectivity Power Headroom Report (PHR) MAC control element is identified by a MAC PDU subheader with LCID as specified in table 6.2.1-2. It has a variable size and is defined in Figure 6.1.3.6b-1 and Figure 6.1.3.6b-2. One octet with Ci fields is used for indicating the presence of PH per SCell when the highest SCellIndex of SCell with configured uplink is less than 8, otherwise four octets are used. For EN-DC, always four octets are used. When Type 2 PH is reported for the PCell, the octet containing the Type 2 PH field is included first after the octet(s) indicating the presence of PH per cell (PSCell and all SCells of all MAC entities) and followed by an octet containing the associated PCMAX,c field (if reported). Then after that, when Type 2 PH is reported for the PSCell, the octet containing the Type 2 PH field is included followed by an octet containing the associated PCMAX,c field (if reported). Then follows an octet with the Type 1 PH field and an octet with the associated PCMAX,c field (if reported), for the PCell. And then follows in ascending order based on the ServCellIndex [8] an octet with the Type x PH field, wherein x is equal to 3 when the ul-Configuration-r14 is configured for this serving cell, x is equal to 1 otherwise, and an octet with the associated PCMAX,c field (if reported), for all serving cells of all MAC entities indicated in the bitmap.

The Dual Connectivity PHR MAC Control Element is defined as follows:

-	Ci: this field indicates the presence of a PH field for the serving cell of any MAC entity, except the PCell, with SCellIndex i as specified in [8]. The Ci field set to "1" indicates that a PH field for the serving cell with SCellIndex i is reported. The Ci field set to "0" indicates that a PH field for the serving cell with SCellIndex i is not reported;

-	R: reserved bit, set to "0";

-	V: this field indicates if the PH value is based on a real transmission or a reference format. For Type 1 PH, V=0 indicates real transmission on PUSCH and V=1 indicates that a PUSCH reference format is used. For Type 2 PH, V=0 indicates real transmission on PUCCH and V=1 indicates that a PUCCH reference format is used. For Type 3 PH, V=0 indicates real transmission on SRS and V=1 indicates that an SRS reference format is used. Furthermore, for Type 1 ,Type 2 and Type 3 PH, V=0 indicates the presence of the octet containing the associated PCMAX,c field, and V=1 indicates that the octet containing the associated PCMAX,c field is omitted;

-	Power Headroom (PH): this field indicates the power headroom level. The length of the field is 6 bits. The reported PH and the corresponding power headroom levels are shown in Table 6.1.3.6-1 (the corresponding measured values in dB can be found in subclause 9.1.8.4 of [9]);

-	P: this field indicates whether power backoff due to power management is applied (as allowed by P-MPRc [10]). The MAC entity shall set P=1 if the corresponding PCMAX,c field would have had a different value if no power backoff due to power management had been applied;

-	PCMAX,c: if present, this field indicates the PCMAX,c or [2] used for calculation of the preceding PH field. The reported PCMAX,c and the corresponding nominal UE transmit power levels are shown in Table 6.1.3.6a-1 (the corresponding measured values in dBm can be found in subclause 9.6.1 of [9]).

Figure 6.1.3.6b-1: Dual Connectivity PHR MAC Control Element

Figure 6.1.3.6b-2: Dual Connectivity PHR MAC Control Element supporting 32 serving cells with configured uplink

End of change