3GPP TSG-RAN WG2 #102	R2-1807190

Busan, Republic of Korea, 21st – 25th May 2018			     Revision of R2-1805428

Agenda Item: 	9.7.5

Source:	Ericsson

Title:	Number of DRBs in E-UTRA connected to 5GC

Document for:	Discussion, Decision

# Introduction

In RAN Plenary, following was agreed [1]:

The work item objective is to extend RAN side protocols and interfaces to support up to 15 data radio bearers at the UE side. The intended changes are:

-  Extend the logical channel ID space by extending the MAC header structure (TS 36.321)

-  Extend the logical channel ID space in the RRC signalling and allow to signal up to 15 data radio bearers (TS 36.331)

-  Specify number of supported RLC AM and UM entities for extended number of LTE data bearers (TS 36.331)

-  Introduce the UE capability for the extended number of LTE data bearers (36.306)

Also during the Email discussion on adopting NR QoS aspects from NR to LTE connected to 5GC, number of DRB’s was identified as dependent on outcome of WI "extended number of bearers for LTE"

Proposal 3: For aspect 7, it is proposed to wait for the progress of the WI "extended number of bearers for LTE" (RP-172835). If the WI is finalized, LTE/5GC can also support up to 15 data radio bearers.

This contribution discusses number of DRBs supported when LTE is connected to 5GC.

# Discussion

When LTE UE is connected to 5GC, the UP and CP protocol stack is expected to be following:

ng-eNB

Figure 1	Illustration of UP and CP protocol stacks when Rel-15 E-UTRA UE is connected to 5GC

When UE is connected to 5GC, NR PDCP is used together with LTE RLC and LTE MAC. Therefore, UE is required to be able to configure NR PDCP with NR RRC configuration. RLC and MAC are configured with LTE RRC configuration. Having possibility to configure either NR or LTE RRC configuration elements, results that there is no limitation on number of DRBs from PDCP configuration perspective. The limitations are introduced by LTE RRC configuration, which only expands to DRB space of LTE RRC.

DRB configuration of E-UTRA – 5GC is limited by DRB space of LTE RRC configuration

Upper limit on number of DRBs supported on E-UTRA connected to 5GC is 15 for UE and Network.

# Conclusion

In section 2 we made the following observations:

Observation 1	DRB configuration of E-UTRA – 5GC is limited by DRB space of LTE RRC configuration

Based on the discussion in section 2 we propose the following:

Proposal 1	Upper limit on number of DRBs supported on E-UTRA connected to 5GC is 15 for UE and Network.

# References

RP-172835, RAN Plenary, Chennai