3GPP TSG-RAN WG2 #102	R2-1807938

Busan, South Korea, 21st – 25th May 2018

Agenda Item:	9.18.5

Source:	Ericsson

Title:	Aerial UE capability aspects

Document for:	Discussion and Decision

# 1	Introduction

In RAN#78, a new WI for enhanced aerial UE support was agreed [1]. In RAN2#101bis the following was agreed:

- Provide threshold(s) in meters at least through dedicated RRC signaling. Using system information is FFS. - Event of UE’s height is above threshold(s) can trigger report. The content of the report are FFS. It can be discussed in the running RRC CR email discussion. - Introduce the number of triggered cells for interference detection for UAV UE. FFS the sum of RSRP. - The flight path information provided from UE to eNB through RRC is supported. The trigger condition and content of the information is FFS.

This document discusses UE capabilities for the Rel-15 Aerial WI features. The discussion revolves around what (if any) aerial features should be mandatory for Rel-15 aerial UE and how to signal the capabilities of features that will be introduced.

# 2	Discussion

The following table lists the features that have been agreed upon as well as other features that have been discussed that could potentially require capability-signalling. In this table, we also provide our views on whether they are mandatory or optional for aerial UEs.

| Agreed features | Mandatory/Optional | Signalling |
| --- | --- | --- |
| Measurement reporting based on height [2] (i.e., event is fulfilled when the height is above a threshold) | Mandatory. This feature is essential for allowing network-control of aerial UEs, which directly ties to future regulations. | One bit |
| Support number of triggered cells in report configuration [3] (e.g., event is triggered and sent if multiple cells are above a threshold). | Mandatory. See above, also this is useful for interference detection which has been identified as one of the biggest problems related with aerial UEs. |  |
| Flight path plan [4] | Optional. The content of the flight path plan is still under discussion and depending on the outcome, it is not clear if all aerials UE have the flight path. Also the claimed benefit for handover relies on tight eNB coordination and it may not be feasible in some deployments. | One bit |
| Aerial power control (RAN1) [7] | TBC by RAN1 | TBC by RAN1 |
| Discussed features | Other comments | Signalling |
| Prohibit timer to limit measurement reporting [8] | This feature could be useful for non-aerial UEs. | One bit |
| ExtendedWaitTime | This feature was introduced for delay-tolerant-feature and can be introduced for aerial UEs too. If agreed, suggested to be mandatory. | One bit |

Table 1: Table of agreed and currently discussed features for Rel-15 aerial UEs.

There are features that will be crucial for the network to support aerial UEs while others are not. Measurement reporting based on height and number of triggered cells in report configuration fall into this category. In this case, only one bit is needed in the signalling.  However, the flight-path plan should be optional and is signaled by another bit. On RAN1 related issues, we can further discuss during the meeting.

RAN2 to decide what should be mandatory for Rel-15 aerial UEs.

Some features seem to be only related with aerials. One such feature is height-based reporting since other types of UEs do not in general have a large latitude change and a change of altitude does not need to notify eNB in this case.

However, some of the features introduced in this WI can also be useful for other types of UEs. For these features, it would be good if they can be separately indicated. Features such as number of triggered cells, prohibit timer, aerial power control can be useful for other UEs since they can be used in high interference scenarios which might not be unique in aerial UEs.

Not all Rel-15 aerial features are specific to Aerial UEs.

In [5-6], CRs are provided to reflect our views. In particular, we show UE-capability signalling for the case where height-based measurement and number of triggered cells in report configuration are signaled as one bit in one aerial-specific item. This item also includes an optional flight path plan indication. Number of triggered cells, prohibit timer and power control are signaled under general Rel-15 capabilities so that they can be supported by other Rel-15 UEs.

# Conclusion

In the previous sections we made the following observations:

Observation 1	Not all Rel-15 aerial features are specific to Aerial UEs.

Based on the discussion in the previous sections we propose the following:

Proposal 1	RAN2 to decide what should be mandatory for Rel-15 aerial UEs.

# References

RP-172826, New WID on Enhanced LTE Support for Aerial Vehicles, Ericsson, RAN#78, Lisbon, Portugal, Dec 2017

R2-1807258, Remaining aspects for height based triggering, Ericsson, RAN#102, Busan, South Korea, May 2018

R2-1805610, Measurement reporting enhancements for interference and flying status detection, Ericsson, RAN#101bis, Sanya, China, April 2018

R2-1802661, Introduction of flight path for Aerial Vehicles, Huawei, RAN#101, Athens, Greece, February 2018

R2-1807696, Draft CR 36.331 Aerial UE capability, Ericsson, RAN#102, Busan, South Korea, May 2018

R2-1807697, Draft CR 36.306 Aerial UE capability, Ericsson, RAN#102, Busan, South Korea, May 2018

R1-1805019, Discussion on UL power control enhancements for aerial vehicles, RAN#101bis, Sanya, China, April 2018

R2-1802789, Measurement reporting enhancement, RAN#101, Athens, Greece, February 2018