3GPP TSG-RAN WG2 #102	R2-1808017

# Busan, Republic of Korea, 21st – 25th May 2018

Agenda Item:	10.4.5.6

Source:	Ericsson

Title:	On the need to notify RAN1 of the PDSCH resource allocation requirements for paging

Document for:	Discussion, Decision

# 1	Introduction

Paging may be conveyed in the form of a separate PDCCH beam sweep followed by a separate PDSCH beam sweep. This results in extraordinary requirements on the PDSCH resource allocation mechanism in some scenarios and the need to inform RAN1 of this is highlighted in this contribution.

# 2	Discussion

RAN2 has defined a paging occasion (PO) as the time during which PDCCH transmissions may occur, which is the time a UE in RRC_IDLE or RRC_INACTIVE state, which has been allocated to this PO, has to monitor the PDCCH in order not to miss any pages. The RRC Paging message on the PDSCH may be transmitted outside the actual PO. As a consequence, when beam sweeping is used to convey the paging, one of the ways is to perform the PDCCH transmissions and the PDSCH transmissions as two separate beam sweeps, one after the other. An advantage of this approach is that the PDCCH transmissions can be concluded faster, i.e. the PO can be shorter, allowing a monitoring UE to go back to DRX sleep mode sooner, in case no page is transmitted.

However, with this transmission mode, there will be unusually long time between a PDCCH transmission and the corresponding PDSCH transmission, for which the PDCCH transmission allocates transmission resources, especially in the worst case with up to 64 beams and several UEs being paged simultaneously. The PDSCH resource allocation mechanisms currently specified by RAN1 do not support such long time periods between the PDCCH and the PDSCH. This highlights the need for RAN2 to notify RAN1 of the special requirements for PDSCH resource allocation for paging. Therefore, we propose to send an LS to RAN1, asking them to take these requirements into account when designing the PDSCH resource allocation mechanisms to be used for paging. A draft of such an LS is provided in our companion contribution R2-1808018 [1].

RAN2 should send an LS to RAN1, notifying RAN1 and asking them to take into account the extraordinary requirements paging (in the form of a separate PDCCH beam sweep followed by a PDSCH beam sweep) places on the PDSCH resource allocation mechanisms, in terms of time period between the PDCCH transmission and the corresponding PDSCH transmission, for which the PDCCH transmission allocates transmission resources. A draft LS is provided in R2-1808018 [1].

# Conclusion

Based on the discussion in the previous sections we propose the following:

Proposal 1	RAN2 should send a LS to RAN2, notifying RAN1 and asking them to take into account the extraordinary requirements paging (in the form of a separate PDCCH beam sweep followed by a PDSCH beam sweep) places on the PDSCH resource allocation mechanisms, in terms of time period between the PDCCH transmission and the corresponding PDSCH transmission, for which the PDCCH transmission allocates transmission resources.

# References

R2-1808018, [DRAFT] LS on resource allocation requirements for paging, Ericsson, RAN2 #102, 21 – 25 May 2018