3GPP TSG-RAN WG2#102 	R2-1808765

# Busan, Korea, 21th-25th May 2018

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.321 | CR | 183 | rev | - | Current version: | 15.1.0 |  |
| For HELPon using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | BFR on SCell |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  |  | Date: |  | 2018-05-16 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Capturing BFR support on SCell according to the following principles: - CFRA-only, no CBRA fallback - Both CFRA preamble and gNB response are on the SCell. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - In subclause 5.1.1, CFRA on SCell can be initialized by beam failure recovery request; - In subclause 5.1.2, UE can select CFRA-BFR resource on SCell if the SSB-RSRP/CSI-RSRP on SCell can satisfy the threshold condition. Otherwise, the RA procedure is considered unsuccessfully completed. - In subclause 5.1.4, UE receives BFR response on the same cell where CFRA-BFR was triggered. - In subclause 5.17, UE can perform BFR on Serving Cell. Impact analysis Impacted functionality: RACH and BFR. Inter-operability: - If the network is implemented according to the CR and the UE is not, BFR on SCell can’t be performed. - If the UE is implemented according to the CR and the network is not, BFR on SCell can’t be performed. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | 1) UE cannot perform BFR on SCell. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.1.1, 5.1.2, 5.1.4, 5.17 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  | This CR is implemented on top of rapporteur’s latest running CR 57 rev2 in R2-1806229 |  |  |  |  |  |  |  |  |

Start of 1st change

### 5.1.1	Random Access procedure initialization

The Random Access procedure described in this subclause is initiated by a PDCCH order, by the MAC entity itself, or by RRC for the events in accordance with TS 38.300 [2]. There is only one Random Access procedure ongoing at any point in time in a MAC entity. The Random Access procedure on an SCell shall only be initiated by a PDCCH order with ra-PreambleIndex different from 0b000000 or for beam failure recovery request (see subclause 5.17).

End of 1st change

Start of 2nd change

### 5.1.2	Random Access Resource selection

The MAC entity shall:

1>	if the Random Access procedure was initiated for beam failure recovery (as specified in subclause 5.17); and

1>	if the contention-free Random Access Resources for beam failure recovery request associated with any of the SSBs and/or CSI-RSs have been explicitly provided by RRC; and

1>	if at least one of the SSBs with SS-RSRP above rsrp-ThresholdSSB amongst the associated SSBs or the CSI-RSs with CSI-RSRP above cfra-csirs-DedicatedRACH-Threshold amongst the associated CSI-RSs is available:

2>	select an SSB with SS-RSRP above rsrp-ThresholdSSB amongst the associated SSBs or a CSI-RS with CSI-RSRP above cfra-csirs-DedicatedRACH-Threshold amongst the associated CSI-RSs;

2>	set the PREAMBLE_INDEX to a ra-PreambleIndex corresponding to the selected SSB or CSI-RS from the set of Random Access Preambles for beam failure recovery request.

1>	else if the ra-PreambleIndex has been explicitly provided by either PDCCH or RRC; and

1>	if the ra-PreambleIndex is not 0b000000; and

1>	if contention-free Random Access Resource associated with SSBs or CSI-RSs have not been explicitly provided by RRC:

2>	set the PREAMBLE_INDEX to the signalled ra-PreambleIndex.

1>	else if the contention-free Random Access Resources associated with SSBs have been explicitly provided by RRC and at least one SSB with SS-RSRP above rsrp-ThresholdSSB amongst the associated SSBs is available:

2>	select an SSB with SS-RSRP above rsrp-ThresholdSSB amongst the associated SSBs;

2>	set the PREAMBLE_INDEX to a ra-PreambleIndex corresponding to the selected SSB.

1>	else if the contention-free Random Access Resources associated with CSI-RSs have been explicitly provided by RRC and at least one CSI-RS with CSI-RSRP above cfra-csirs-DedicatedRACH-Threshold amongst the associated CSI-RSs is available:

2>	select a CSI-RS with CSI-RSRP above cfra-csirs-DedicatedRACH-Threshold amongst the associated CSI-RSs;

2>	set the PREAMBLE_INDEX to a ra-PreambleIndex corresponding to the selected CSI-RS.

1>	else if the Random Access procedure was initiated on SpCell:

2>	if at least one of the SSBs with SS-RSRP above rsrp-ThresholdSSB is available:

3>	select an SSB with SS-RSRP above rsrp-ThresholdSSB.

2>	else:

3>	select any SSB.

2>	if Msg3 has not yet been transmitted:

3>	if Random Access Preambles group B is configured; and

3>	if the potential Msg3 size (UL data available for transmission plus MAC header and, where required, MAC CEs) is greater than ra-Msg3SizeGroupA and the pathloss is less than PCMAX (of the Serving Cell performing the Random Access Procedure) – preambleReceivedTargetPower – msg3-DeltaPreamble – messagePowerOffsetGroupB:

4>	select the Random Access Preambles group B.

3>	else:

4>	select the Random Access Preambles group A.

2>	else (i.e. Msg3 is being retransmitted):

3>	select the same group of Random Access Preambles as was used for the Random Access Preamble transmission attempt corresponding to the first transmission of Msg3.

2>	if the association between Random Access Preambles and SSBs is configured:

3>	select a ra-PreambleIndex randomly with equal probability from the Random Access Preambles associated with the selected SSB and the selected Random Access Preambles group.

2>	else:

3>	select a ra-PreambleIndex randomly with equal probability from the Random Access Preambles within the selected Random Access Preambles group.

2>	set the PREAMBLE_INDEX to the selected ra-PreambleIndex.

1>	if an SSB is selected above and an association between PRACH occasions and SSBs is configured:

2>	determine the next available PRACH occasion from the PRACH occasions corresponding to the selected SSB permitted by the restrictions given by the ra-ssb-OccasionMaskIndex if configured (the MAC entity shall select a PRACH occasion randomly with equal probability amongst the PRACH occasions occurring simultaneously but on different subcarriers, corresponding to the selected SSB; the MAC entity may take into account the possible occurrence of measurement gaps when determining the next available PRACH occasion corresponding to the selected SSB).

1>	else if a CSI-RS is selected above and an association between PRACH occasions and CSI-RSs is configured:

2>	determine the next available PRACH occasion from the PRACH occasions in ra-OccasionList corresponding to the selected CSI-RS (the MAC entity shall select a PRACH occasion randomly with equal probability amongst the PRACH occasions occurring simultaneously but on different subcarriers, corresponding to the selected CSI-RS; the MAC entity may take into account the possible occurrence of measurement gaps when determining the next available PRACH occasion corresponding to the selected CSI-RS).

1>	else:

2>	determine the next available PRACH occasion (the MAC entity shall select a PRACH occasion randomly with equal probability amongst the PRACH occasions occurring simultaneously but on different subcarriers; the MAC entity may take into account the possible occurrence of measurement gaps when determining the next available PRACH occasion).

1>	perform the Random Access Preamble transmission procedure (see subclause 5.1.3).

End of 2nd change

Start of 3rd change

### 5.1.4	Random Access Response reception

Once the Random Access Preamble is transmitted and regardless of the possible occurrence of a measurement gap, the MAC entity shall:

1>	if the contention-free Random Access Preamble for beam failure recovery request was transmitted by the MAC entity:

2>	start the ra-ResponseWindow configured in BeamFailureRecoveryConfig at the first PDCCH occasion as specified in TS 38.213 [6] from the end of the Random Access Preamble transmission;

2>	monitor the PDCCH of the Serving Cell where was initiated the random access procedure for response to beam failure recovery request identified by the C-RNTI while ra-ResponseWindow is running.

1>	else:

2>	start the ra-ResponseWindow configured in RACH-ConfigCommon at the first PDCCH occasion as specified in TS 38.213 [6] from the end of the Random Access Preamble transmission;

2>	monitor the PDCCH of the SpCell for Random Access Response(s) identified by the RA-RNTI while the ra-ResponseWindow is running.

1>	if notification of a reception of a PDCCH transmission is received from lower layers; and

1>	if PDCCH transmission is addressed to the C-RNTI; and

1>	if the contention-free Random Access Preamble for beam failure recovery request was transmitted by the MAC entity:

2>	consider the Random Access procedure successfully completed.

1>	else if a downlink assignment has been received on the PDCCH for the RA-RNTI and the received TB is successfully decoded:

2>	if the Random Access Response contains a MAC subPDU with Backoff Indicator:

3>	set the PREAMBLE_BACKOFF to value of the BI field of the MAC subPDU using Table 7.2-1.

2>	else:

3>	set the PREAMBLE_BACKOFF to 0 ms.

End of 3rd change

Start of 4th change

## 5.17	Beam Failure Detection and Recovery procedure

The MAC entity may be configured by RRC with a beam failure recovery procedure which is used for indicating to the serving gNB of a new SSB or CSI-RS when beam failure is detected on the serving SSB(s)/CSI-RS(s). Beam failure is detected by counting beam failure instance indication from the lower layers to the MAC entity.

For each Serving Cell configured to support beam failure recovery, RRC configures the following parameters in the BeamFailureRecoveryConfig for the Beam Failure Detection and Recovery procedure:

-	beamFailureInstanceMaxCount for the beam failure detection;

-	beamFailureDetectionTimer for the beam failure detection;

-	candidateBeamThreshold: an RSRP threshold for the beam failure recovery;

-	preamblePowerRampingStep: preamblePowerRampingStep for the beam failure recovery;

-	preambleReceivedTargetPower: preambleReceivedTargetPower for the beam failure recovery;

-	preambleTransMax: preambleTransMax for the beam failure recovery;

-	ra-ResponseWindow: the time window to monitor response(s) for the beam failure recovery using contention-free Random Access Preamble;

-	prach-ConfigIndex: prach-ConfigIndex for the beam failure recovery;

-	ra-ssb-OccasionMaskIndex: ra-ssb-OccasionMaskIndex for the beam failure recovery;

-	ra-OccasionList: ra-OccasionList for the beam failure recovery.

The following UE variables are used for the beam failure detection procedure:

-	BFI_COUNTER: counter for beam failure instance indication which is initially set to 0.

The MAC entity shall for each activated serving cell for which BeamFailureRecoveryConfig is configured:

1>	if beam failure instance indication has been received from lower layers for this Serving Cell:

2>	start or restart the beamFailureDetectionTimer;

2>	increment BFI_COUNTER by 1;

2>	if BFI_COUNTER >= beamFailureInstanceMaxCount:

3>	initiate a Random Access procedure (see subclause 5.1) on this Serving Cell by applying the parameters configured in BeamFailureRecoveryConfig.

1>	if the beamFailureDetectionTimer expires:

2>	set BFI_COUNTER to 0.

1>	if the Random Access procedure is successfully completed (see subclause 5.1):

2>	consider the Beam Failure Recovery procedure successfully completed.

End of 4th change