3GPP TSG-RAN WG2 Meeting #103bis                                                  R2-1813738

Chengdu, China, 8th - 12th Oct 2018

Source:	CATT

Title:	QoS Management for NR V2X

Agenda Item:	11.4.5

Document for:	Discussion and Decision

Introduction

In RAN#80, the NR Rel-16 V2X SID [1] was agreed. One of its objectives is as below:

5: QoS management [RAN1, RAN2]: - Study technical solutions for QoS management of the radio interface (including both Uu and sidelink) used for V2X operations based on input from SA2

This paper analyses the feasibility of using current NR QoS framework for V2X services both in Uu and sidelink interface and proposes the potential enhancements.

SA2 V2X QoS requirement

To support 5G V2X services, 25 use cases and their performance requirements are studied in TR 22.286[2]. Five use case groups are further defined, which include vehicles platooning, advanced driving, remote driving, extended sensor and general aspects (interworking, communication-related requirements valid for all V2X scenarios). Performance requirements of each use case are characterized by:

- Payload (Bytes);

- Transmission rate (Message/Sec);

- Maximum end-to-end latency (ms);

- Reliability (%);

- Data rate (Mbps);

- Minimum communication range (meters).

Observation1: From the perspective of requirement, V2X QoS parameters include payload, transmission rate, maximum end-to-end latency, reliability, data rate and minimum communication range.

NR QoS framework

According to [3], 5G QoS parameters of QoS profile include 5QI, ARP, RQA, GFBR, MFBR, notification control and maximum packet loss rate. A QoS profile is provided by CN to the gNB over the N2 reference point or preconfigured in the access network. gNB will use these QoS parameters to perform admission control, DRB management/configuration and QoS guarantee.

The 5G QoS characteristics associated with 5QI are in terms of the following terms:

- Resource Type (GBR, delay critical GBR or Non-GBR)

- Priority level;

- Packet Delay Budget;

- Packet Error Rate;

- Averaging window;

- Maximum Data Burst Volume (for Delay-critical GBR resource type only).

The 5G QoS characteristics are bundled and indicated through the 5QI value as part of the QoS profile as shown in Table 1.   In Table 1, some 5QI values had been defined for V2X, such as 5QI=79 for V2X message, 5QI=82, 83 for ITS (Intelligent Transport Systems).   The 5G QoS characteristics are based on edge-to-edge requirements, which include the RAN and 5G core network.  The NR V2X QoS characteristic should follows the 5G QoS characteristics with more stringent performance requirements in Uu and SL links in order to satisfy edge-to-edge performance requirements.

| 5QI Value | Resource Type | Default Priority Level | Packet Delay Budget | Packet Error Rate | Default Maximum Data Burst Volume (NOTE 2) | Default Averaging Window | Example Services |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | GBR | 20 | 100 ms | 10-2 | N/A | 2000 ms | Conversational Voice |
| 2 | NOTE 1 | 40 | 150 ms | 10-3 | N/A | 2000 ms | Conversational Video (Live Streaming) |
| 3 |  | 30 | 50 ms | 10-3 | N/A | 2000 ms | Real Time Gaming, V2X messages Electricity distribution – medium voltage, Process automation - monitoring |
| 4 |  | 50 | 300 ms | 10-6 | N/A | 2000 ms | Non-Conversational Video (Buffered Streaming) |
| 65 |  | 7 | 75 ms | 10-2 | N/A | 2000 ms | Mission Critical user plane Push To Talk voice (e.g., MCPTT) |
| 66 |  | 20 | 100 ms | 10-2 | N/A | 2000 ms | Non-Mission-Critical user plane Push To Talk voice |
| 67 |  | 15 | 100 ms | 10-3 | N/A | 2000 ms | Mission Critical Video user plane |
| 75 |  | 25 | 50 ms | 10-2 | N/A | 2000 ms | V2X messages |
| 5 | Non-GBR | 10 | 100 ms | 10-6 | N/A | N/A | IMS Signalling |
| 6 | NOTE 1 | 60 | 300 ms | 10-6 | N/A | N/A | Video (Buffered Streaming) TCP-based (e.g., www, e-mail, chat, ftp, p2p file sharing, progressive video, etc.) |
| 7 |  | 70 | 100 ms | 10-3 | N/A | N/A | Voice, Video (Live Streaming) Interactive Gaming |
| 8 |  | 80 | 300 ms | 10-6 | N/A | N/A | Video (Buffered Streaming) TCP-based (e.g., www, e-mail, chat, ftp, p2p file sharing, progressive |
| 9 |  | 90 |  |  |  |  | video, etc.) |
| 69 |  | 5 | 60 ms | 10-6 | N/A | N/A | Mission Critical delay sensitive signalling (e.g., MC-PTT signalling) |
| 70 |  | 55 | 200 ms | 10-6 | N/A | N/A | Mission Critical Data (e.g. example services are the same as QCI 6/8/9) |
| 79 |  | 65 | 50 ms | 10-2 | N/A | N/A | V2X messages |
| 80 |  | 68 | 10 ms | 10-6 | N/A | N/A | Low Latency eMBB applications Augmented Reality |
| 81 | Delay Critical GBR | 11 | 5 ms | 10-5 | 160 B | 2000 ms | Remote control (see TS 22.261 [2]) |
| 82 |  | 12 | 10 ms NOTE 5 | 10-5 | 320 B | 2000 ms | Intelligent transport systems |
| 83 |  | 13 | 20 ms | 10-5 | 640 B | 2000 ms | Intelligent Transport Systems |
| 84 |  | 19 | 10 ms | 10-4 | 255 B | 2000 ms | Discrete Automation |
| 85 |  | 22 | 10 ms | 10-4 | 1358 B NOTE 3 | 2000 ms | Discrete Automation |
| NOTE 1: a packet which is delayed more than PDB is not counted as lost, thus not included in the PER. NOTE 2: it is required that default MDBV is supported by a PLMN supporting the related 5QIs. NOTE 3: This MDBV value is intended to avoid IP fragmentation on an IPv6 based, IPSec protected, GTP tunnel to the 5G-AN node. NOTE 4: A delay of 1 ms for the delay between a UPF terminating N6 and a 5G-AN should be subtracted from a given PDB to derive the packet delay budget that applies to the radio interface. NOTE 5: The jitter for this service is assumed to be 20 msec according to TS 22.261 [2]. |  |  |  |  |  |  |  |

Table 1: Standardized 5QI to QoS characteristics mapping

From the above content, some QoS parameters of QoS profile, i.e. ARP, RQA, notification control and maximum packet loss rate, have no mapping relationship with V2X service requirements. Table 2 gives a comparison between V2X requirements and 5G QoS parameter/characteristics in 5QI. It indicates that most of V2X service requirements are already covered by 5G QoS parameter/characteristic in 5QI except for transmission rate and minimum required communication range. It can be discussed whether transmission rate and minimum required communication range are necessary V2X QoS characteristic to be informed by CN. Relevant work regarding V2X QoS framework is ongoing in SA2. Potential RAN enhancements may be developed based on SA2’s progress.

Table 2 Comparison between V2X requirement and 5G QoS parameter/characteristic

| V2X service requirement | 5G QoS parameter/characteristic in 5QI |
| --- | --- |
| Payload | Maximum data burst volume |
| Transmission rate | N/A |
| Maximum end-to-end latency | Packet delay budget |
| Reliability | Packet error rate |
| Data rate | GFBR/MFBR |
| Minimum required communication range | N/A |

Observation2: Most of V2X service requirements are already covered by 5G QoS parameter/characteristic in 5QI.

Proposal1: 5G QoS framework can be used as baseline for V2X services over Uu interface and potential for SL interface.

NR V2X QoS management

In current 5G system, QoS profile is carried by N2 signaling from CN to access network before data packet arrival. The current signaling and procedure can be reused for V2X services over Uu interface. But for SL interface, R14/15 V2X uses the method of carrying PPPP and PPPR parameters in each packet, which can omit QoS parameter notification procedure before data packet arrival and reduce transmission delay.

If 5G QoS framework will be used for V2X over SL interface, there are following potential methods to get QoS parameters related to 5QI:

- Alt1: All necessary parameters carried in each packet;

- Alt2: Design or reuse signaling procedure for QoS parameter notification;

- Alt3: Build static/semi-static mapping between possible 5QI and related QoS parameters and carry 5QI in each packet;

Alt 1 is similar to R14/15 V2X mechanism and the demerit is higher overhead of each packet. Alt 2 is similar to Uu procedure and the demerit is higher latency. Alt 3 is similar to SA2 solution19 in [3] and can be further studied.

Proposal2: A unified QoS framework, e.g. 5QI parameter framework, can be used for NR SL UEs.

Conclusion

According to the analysis in the above sections, we have following observations and proposals:

Observation1: From the perspective of requirement, V2X QoS parameters include payload, transmission rate, maximum end-to-end latency, reliability, data rate and minimum communication range.

Observation2: Most of V2X service requirements are already covered by 5G QoS parameter/characteristic in 5QI.

Proposal1: 5G QoS framework can be used as baseline for V2X services over Uu interface and potential for SL interface.

Proposal2: A unified QoS framework, e.g. 5QI parameter framework, can be used for NR SL UEs.

References

- RP-181480, “New SID: Study on NR V2X”,  Vodafone

- TR22.886 v16.0.0, “Study on enhancement of 3GPP Support for 5G V2X Services”

- TS23.501v15.2.0, “System Architecture for the 5G System”

- TR23786v0.8.0, “Study on architecture enhancements for EPS and 5G System to support advanced V2X services(Release 16) ”