3GPP TSG-RAN WG2#103bis	R2-1814660

Chengdu, China, 8th – 12th October 2018

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.331 | CR | 0367 | rev | - | Current version: | 15.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR to 38.331 on SCell-RLF |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT_core |  |  |  |  |  |  | Date: |  | 2018-09-27 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | When a RLF is detected on a logical channel restricted to SCell(s) the UE should report the SCell-RLF to the network that then can configure the UE to take the necessary actions (e.g., reconfiguration, addition, or removal of the SCell). |  |  |  |  |  |  |  |  |
| Summary of change: |  | Once an RLF on the SCell used for CA level duplication is detected, the UE should deactivate CA duplication on the concerned SCell, and report the SCellRLFReport message to the MN if the SCell belongs to the MN and to the SN if the Scell belongs to the SN. Also, the UE includes in the SCellRLFRepost the SCell Index and latest available measurements. Impact analysis Impacted functionalities: Radio link failure in case of CA duplication Inter-operability: If the network implements the CR and the UE does not, the UE will start the RRC re-establishment procedure when understanding that the maximum number of retransmission has been reached while the network expect to recevie a failure information report. If the UE implements the CR and the network does not, the network will not be able to decode the failure information report sent by the UE and will most likely start the RRC re-establishment procedure. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | RRC specification will not be clear and potential erroneous behaviour on the UE and network side when an RLF happen on the SCell. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.3.10.3 Detection of radio link failure 6.2.2 Message definitions |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

START OF CHANGES

#### 5.3.10.3	Detection of radio link failure

The UE shall:

1>	upon T310 expiry in PCell; or

1>	upon random access problem indication from MCG MAC while neither T300, T301, T304 nor T311 is running; or

1>	upon indication from MCG RLC that the maximum number of retransmissions has been reached:

2>	consider radio link failure to be detected for the MCG i.e. RLF;

Editor’s Note: FFS: How to handle RLC failure in CA duplication for MCG DRB and SRB.

2>	if AS security has not been activated:

3>	perform the actions upon going to RRC_IDLE as specified in 5.3.11, with release cause 'other';

2>	else:

3>	initiate the connection re-establishment procedure as specified in 5.3.7.

The UE shall:

1>	upon T310 expiry in PSCell; or

1>	upon random access problem indication from SCG MAC; or

1>	upon indication from SCG RLC that the maximum number of retransmissions has been reached:

2> if the RLC is associated with an SCell that is being used for CA level duplication;

3> consider SCell radio link failure to be detected for the SCell i.e. SCell-RLF;

3> initiate the SCell RLF report procedure as specified in 5.7.x to report SCell failure;

2> else if the RLC is associated with an PSCell that is being used for CA level duplication;

3> consider SpCell radio link failure to be detected for the PSCell, i.e., SpCell-RLF;

3> initiate the SpCell RLF report procedure as specified in 5.7.x to report SpCell failure;

2> else

3> consider radio link failure to be detected for the SCG i.e. SCG-RLF;

3> initiate the SCG failure information procedure as specified in 5.7.3 to report SCG radio link failure;

END OF CHANGES

START OF CHANGES

#### 5.7.x	SCell RLF Report

#### 5.7.x.1	General

Figure 5.7.x.1-1: SCell RLF Report

The purpose of this procedure is to inform NR about an SCell failure the UE has experienced during CA level duplication, i.e., exceeding the maximum number of RLC retransmissions.

#### 5.7.x.2	Initiation

A UE initiates the procedure to report SCell RLF when SCell transmission is not suspended and when the following condition is met:

1> upon detecting that the maximum number of RLC retransmissions has been reached, in accordance with subclause 5.3.11.3, for an SCell that is being used for CA level duplication;

Upon initiating the procedure, the UE shall:

1>	deactivate the CA level duplication on the SCell where the RLF was detected;

1>	initiate transmission of the SCellRLFReport message as specified in 5.7.x.3;

#### 5.7.x.3	Actions related to transmission of SCellRLFReport message

The UE shall set the contents of the SCellRLFReport message as follows:

1>	if the UE initiates transmission of the SCellRLFReport message to provide SCell radio link failure information due to exceeding maximum of RLC retransmissions:

2> set the FailureReportSCell to include sCellIndex of the SCell where the RLF was detected;

The UE shall submit the SCellRLFReport message to lower layers for transmission.

END OF CHANGES

START OF CHANGES

#### 6.2.2	Message definitions

#### –	SCellRLFReport

The SCellRLFReport message is used to provide information regarding failures detected by the UE for an SCell in CA level duplication.

Signalling radio bearer: SRB1 or SRB3

RLC-SAP: AM

Logical channel: DCCH

Direction: UE to Network

SCellRLFReport message

-- ASN1START

SCellRLFReport-r15 ::=		SEQUENCE {

criticalExtensions					CHOICE {

c1									CHOICE {

scellRlfReport-r15 			SCellRLFReport-r15,

},

criticalExtensionsFuture			SEQUENCE {}

}

}

SCellRLFReport-r15 ::=	SEQUENCE {

failureReportSCell-r15				FailureReportSCell-r15

failureType                     ENUMERATED { t310-Expiry, randomAccessProblem,

rlc-MaxNumRetx, scg-ChangeFailure,

scg-reconfigFailure,

srb3-IntegrityFailure},

}

FailureReportSCell-r15 ::= 			SEQUENCE {

failedCellGroupID						CellGroupId

failedLogicalChannelID					LogicalChannelIdentity

--FFS: Which measurements to be included (e.g., serving SCG/MCG cells, best neighbor cell of the serving SCG/MCG frequency, non-serving frequency, and beam level measurements).

}

-- ASN1STOP

END OF CHANGES