3GPP TSG-RAN WG2#103bis	R2-1814679

Chengdu, China, 8th – 12th October 2018

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 37.340 | CR | 0074 | rev | - | Current version: | 15.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | MR-DC measurement principles |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT_core |  |  |  |  |  |  | Date: |  | 2018-09-27 |
| Category: | B |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Measurement gaps and gap coordination for NE-DC and NR-DC is not present in the specification |  |  |  |  |  |  |  |  |
| Summary of change: |  | 7.2 Measurements - Added a description about how the measurements gap and gap coordinaton is handled in NE-DC and NR-DC |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | If the CR is not implemented it will be not clear for the MN and SN how to behave when configuring gap and gap coordination in NE-DC and NR-DC. |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

START OF CHANGES

## 7.2	Measurements

If the measurement is configured to the UE in preparation for the Secondary Node Addition procedure described in sub-clause 10.2, the Master node should configure the measurement to the UE.

In case of the intra-secondary node mobility described in sub-clause 10.3, the SN should configure the measurement to the UE in coordination with the MN, if required.

The Secondary Node Change procedure described in sub-clause 10.5 can be triggered by both the MN (only for inter-frequency secondary node change) and the SN. For secondary node changes triggered by the SN, the RRM measurement configuration is maintained by the SN which also processes the measurement reporting, without providing the measurement results to the MN.

Measurements can be configured independently by the MN (for inter-RAT measurement) and by the SN (intra-RAT measurements on serving and non-serving frequencies). The MN indicates the maximum number of frequency layers and measurement identities that can be used in the SN to ensure that UE capabilities are not exceeded. If MN and SN both configure measurements on the same carrier frequency, then the configurations need to be consistent (if the network wants to ensure these are considered as a single measurement layer). Each node (MN and SN) can configure independently a threshold for the SpCell quality: when the PCell quality is above the threshold configured by the MN, the UE is still required to perform inter-RAT measurements configured by the MN on the SN RAT (while it's not required to perform intra-RAT measurements); when the PSCell quality is above the threshold configured by the SN, the UE is not required to perform measurements configured by the SN.

NOTE:	The SN cannot renegotiate the number of frequency layers allocated by the MN in this version of the protocol.

When SRB3 is not configured, reports for measurements configured by the SN are sent on SRB1. When SRB3 is configured, reports for measurements configured by the SN are sent on SRB3.

Measurement results related to the target SN can be provided by MN to target SN at MN initiated SN change procedure. Measurement results of target SN can be forwarded from source SN to target SN via MN at SN initiated SN change procedure. Measurement results related to the target SN can be provided by source MN to target MN at Inter-MN handover with/without SN change procedure.

In EN-DC, measurement results according to measurement configuration from the MN are encoded according to NR RRC when they are provided by MN to SN in SgNB Addition Request message. During SN initiated SN change procedure, measurement results according to measurement configuration from SN are encoded according to NR RRC when they are provided by MN to SN in SgNB Addition Request message.

Per-UE or per-FR measurement gaps can be configured, depending on UE capability and network preference. Per-UE gap applies to both FR1 (LTE and NR) and FR2 (NR) frequencies. For per-FR gap, two independent gap patterns (i.e. FR1 gap and FR2 gap) are configured for FR1 and FR2 respectively. The UE may also be configured with a per-UE gap sharing configuration (applying to per-UE gap) or with two separate gap sharing configurations (applying to FR1 and FR2 measurement gaps respectively) [8].

In EN-DC and NGEN-DC, if per-UE gap is used, the MN decides the gap pattern and the related gap sharing configuration. If per-FR gap is used, the MN decides the FR1 gap pattern and the related gap sharing configuration for FR1, while the SN decides the FR2 gap pattern and the related gap sharing configuration for FR2. The measurement gap configuration from the MN to the UE indicates if the configuration from the MN is a per-UE gap or an FR1 gap configuration. The MN also indicates the configured per-UE or FR1 measurement gap pattern and the gap purpose (per-UE or per-FR1) to the SN.

In NE-DC, if per-UE gap is used, the MN decides the gap pattern and the related gap sharing configuration. If per-FR gap is used, the MN decides the FR2 gap pattern and the related gap sharing configuration for FR1, while both the MN and SN can decide the FR1 gap pattern and the related gap sharing configuration for FR2.

In NR-DC, if per-UE gap is used, the MN decides the gap pattern and the related gap sharing configuration. If per-FR gap is used, the MN and SN may decide the FR2 or FR1 gap pattern and the related gap sharing configuration for FR1 or FR2.

Measurement gap configuration assistance information can be exchanged between the MN and the SN. In EN-DC and NGEN-DC, for the case of per-UE gap, the SN indicates to the MN the list of SN configured frequencies in FR1 and FR2 measured by the UE. For the per-FR gap case, the SN indicates to the MN the list of SN configured frequencies in FR1 measured by the UE and the MN indicates to the SN the list of MN configured frequencies in FR2 measured by the UE.

In NE-DC, for the case of per-UE gap, the SN indicates to the MN the list of SN configured frequencies in FR1. For the per-FR gap case, the SN indicates to the MN the list of SN configured frequencies in FR2 measured by the UE or the FR1 gap configuration and the MN indicates to the SN the list of MN configured frequencies in FR2 or FR1 measured by the UE.

In NR-DC, for the case of per-UE gap, the SN indicates to the MN the list of SN configured frequencies in FR1 or FR2. For the per-FR gap case, MN indicates to the SN the list of MN configured frequencies in FR2 or FR1 measured by the UE.

END OF CHANGES