3GPP TSG-RAN WG2 #103bis			     R2-1815153

Chengdu, China, 8th – 12th October, 2018

Agenda Item	: 11.2.1.1	(FS_NR_unlic)

Source	: LG Electronics Inc.

Title	: Enhanced RACH proceudre for NR-U

Document for	: Discussion and Decision

1.	Introduction

At RAN2 #103 and RAN1#94, the following agreements were made separately:

Agreements@ RAN2 #103

R2 assumes that RACH may be enhanced by additional opportunities, e.g. in time or frequency domain, FFS which messages the additional opportunities apply to.

Will study the model of single-RACH procedure. FFS multiple parallel procedure model

Will study impact to PREAMBLE_TRANSMISSION_COUNTER, PREAMBLE_POWER_RAMPING_COUNTER, ra-ResponseWindow, ra-ContentionResolutionTimer

It is FFS if LBT failure knowledge would be used in MAC (if available), e.g. to decide whether to increments counters PREAMBLE_POWER_RAMPING_COUNTER PREAMBLE_TRANSMISSION_COUNTER, or start stop of timers.

Agreement@ RAN1 #94:

If preamble transmissions are dropped due to LBT failure, then

- From a RAN1 perspective, it is recommended that preamble power ramping is not performed and that the preamble transmission counter is not incremented

Agreement@ RAN1 #94:

- In some scenarios it is beneficial for the maximum RAR window size to be extended beyond 10 ms to increase robustness to DL LBT failure

  - FFS: Value of maximum RAR window size

In this contribution, we’d like to propose the details to enhance the transmission opportunities of a UE within a single RACH procedure for unlicensed band.

2.	Discussion

According to the agreements in the last meeting, RAN2 assumes that RACH may be enhanced by additional opportunities, but which message to apply to is FFS. As we all know, a UE considers the RACH procedure unsuccessfully completed if the UE doesn’t receive MSG2 or MSG4 in response to successfully transmitted MSG1 or MSG3, or even if the UE unsuccessfully transmits MSG1 or MSG3 due to LBT failure in unlicensed band of LTE. Therefore, improving transmission/reception opportunities for all RACH related steps will help the success of the RACH procedure.

Observation 1. Improving transmission/reception opportunities for all RACH related steps will help the success of the RACH procedure of a UE.

In order for a UE to improve the reception opportunities for MSG2 or MSG4, the network/UE may have to configure the RAR window size or contention resolution timer to the longer value than the legacy RACH considering LBT failure of gNB. In addition, the network may have to allocate more UL resources in advance considering LBT failures of a UE. From this point of view, it seems to be obvious that the maximum size of ra-ResponseWindow and ra-ContentionResolutionTimer should be extended, and the network should provide additional resources for both MSG1 and MSG3. So we propose that the maximum size for both ra-ResponseWindow and ra-ContentionResolutionTimer should be extended beyond the current value in NR, and the network provides additional opportunities for both MSG1 and MSG3.

Proposal 1. The maximum size for both ra-ResponseWindow and ra-ContentionResolutionTimer should be extended beyond the current value in NR.

Proposal 2. The network should provide additional opportunities for both MSG1 and MSG3.

More specifically, in 4-step RACH procedure, the transmission opportunity for MSG3 is provided by a UL grant in a RAR, and the network transmits a RAR including only one UL grant for a successfully received preamble. In other words, the additional transmission opportunities for MSG3 depends on the number of UL grants received from the RAR message. The ways to provide the multiple UL grants by using the RAR message are two options as follows:

- Option 1. By receiving multiple UL grants in a RAR

- Option 2. By receiving multiple RARs

In option 1, since the UE receives a RAR with multiple UL grants, it is sufficient that the UE transmits one preamble only for the PRACH occasion in which LBT succeeds. An example for the option1 is shown in Figure 1. This means that MAC in option 1 may know the LBT result for each PRACH occasion, and it additionally requires the change of the current RAR format. Therefore, option1 not only may require additional modification, such as indication of LBT results, multiple UL grants in a RAR, but also has the limitation of flexibly providing additional opportunities for MSG3 due to the fixed RAR format.

Figure 1. Single RACH procedure with multiple UL grants in a RAR

On the other hand, a UE in option 2 should monitor the multiple RARs within a single RACH procedure. The simplest way to receive multiple RARs is that the UE transmits multiple preambles and monitors the RAR for each transmitted preamble. The number of transmitted MSG1 can be configured by the network, and the transmission opportunity of MSG3 can be improved through one or more RAR(s) received for the successfully transmitted preambles. For the flexibility of transmission opportunities and simple design, we propose that RAN2 allow for a UE to transmit multiple MSG1s and monitor multiple RARs for the transmitted MSG1s, irrespective of LBT results of MSG1.

Figure 2. Single RACH procedure with multiple RARs

Observation 2. It seems simpler to receive multiple RARs than to receive multiple UL grants in a RAR to improve the transmission opportunities for MSG3 with the current format.

Proposal 3. RAN2 should allow for a UE to transmit multiple MSG1s and monitor multiple RARs for the transmitted MSG1s.

According to the agreement in RAN1#94, RAN1 recommends that the preamble transmission counter is not incremented if preamble transmissions are dropped due to LBT failure. It may be beneficial to know the LBT failure in MAC because the UE may retry the RAP retransmission as soon as possible. But it may not timely perform RLF or RA retries because the UE cannot reach to the maximum value of the preamble transmission counter in the worst case where LBT fails continuously. That is, if a UE increments the preamble transmission counter only when receiving the LBT success, the RACH procedure can be stuck in RA resource selection step. In order to avoid this problem, we propose to define the maximum number (N) of transmission opportunities for MSG1 per RA attempt.

Observation 3. If a UE increments the preamble transmission counter only when receiving the LBT success, the RACH procedure can be stuck in RA resource selection step, especially in the worst case where LBT fails continuously.

Proposal 4. RAN2 should define the maximum number (N) of transmission opportunities for MSG1 per RA attempt.

The maximum number of LBT opportunities for MSG1 in a RA attempt doesn’t mean that MAC should know the LBT results. Rather, this is associated with the number of PRACH occasions selected by MAC. In other words, when MAC determines multiple PRACH occasions for the multiple preambles and instructs the related PRACH resources to the lower layer, the LBT result for each PRACH resource doesn’t need to be indicated to MAC from RAN2 point of view since the UE can monitor the RAR message for each preamble. If the UE doesn’t receive all RARs for the transmitted preambles, then the UE considers the RA procedure unsuccessfully completed and increments the preamble transmission counter.

Proposal 5. PREAMBLE_TRANSMISSION_COUNTER should be increment if the UE doesn’t receive all RARs for transmitted multiple preambles (N).

Proposal 6. MAC doesn’t need to know the LBT results for the transmitted MSG1s.

3.	Conclusion

In this contribution, we address several solutions to enhance the CBRA procedure in a single RA model, and our observations and proposals are as follows.

Observation 1. Improving transmission/reception opportunities for all RACH related steps will help the success of the RACH procedure of a UE.

Observation 2. It seems simpler to receive multiple RARs than to receive multiple UL grants in a RAR to improve the transmission opportunities for MSG3 with the current format.

Observation 3. If a UE increments the preamble transmission counter only when receiving the LBT success, the RACH procedure can be stuck in RA resource selection step, especially in the worst case where LBT fails continuously.

Proposal 1. The maximum size for both ra-ResponseWindow and ra-ContentionResolutionTimer should be extended beyond the current value in NR.

Proposal 2. The network should provide additional opportunities for both MSG1 and MSG3.

Proposal 3. RAN2 should allow for a UE to transmit multiple MSG1s and monitor multiple RARs for the transmitted MSG1s.

Proposal 4. RAN2 should define the maximum number (N) of transmission opportunities for MSG1 per RA attempt.

Proposal 5. PREAMBLE_TRANSMISSION_COUNTER should be increment if the UE doesn’t receive all RARs for transmitted multiple preambles (N).

Proposal 6. MAC doesn’t need to know the LBT results for the transmitted MSG1s.