3GPP TSG-RAN WG2 Meeting #103bis 	                           R2-1815946

Chengdu, China, 8th – 12th Aug 2018

Source: 			ZTE Corporation (Rapporteur)

Title: 	[Offline discussion 29] Common coreset for a NSA-only (P)Scell

Agenda item:		10.4.1.3.1.1

Document for: 	Discussion and Decision

# Introduction

This contribution discusses the possibility to configure controlResourceSetZero/searchSpaceZero for a NSA-only (P)Scell, where at least RA needs to be performed.

# Discussion

The following solutions are possible:

1) Allow the configuration of controlResourceSetZero/searchSpaceZero in pdcch-ConfigCommon within ServingCellConfigCommon, even if SIB1 is not broadcast (this does not remove the possibility to configure different commonControlResourceSets for the (P)Scell where RA needs to be performed).

This solution is already supported in TS 38.331 since the finalization of EN-DC (i.e. before SA was introduced) and does not require any ASN.1 changes.

After RAN2#103 a concern was raised about the possible mismatch between the configuration provided by dedicated signalling and the one provided in SIB1. But at the same time the need to provide a RA search space for a NSA-only (P)Scell was also highlighted. This resulted in the following conditions for the inclusion of controlResourceSetZero/ searchSpaceZero in pdcch-ConfigCommon within ServingCellConfigCommon, clearly allowing the configuration also in case SIB1 is not broadcast.

| Conditional Presence | Explanation |
| --- | --- |
| InitialBWP-Only | If SIB1 is broadcast the field is mandatory present in the PDCCH-ConfigCommon of the initial BWP (BWP#0) in dedicated signalling. It is absent in other BWPs and when sent in system information. In other cases, the field is optionally present. |

As discussed online, it is unclear whether configuring controlResourceSetZero/searchSpaceZero in pdcch-ConfigCommon when SIB1 is not broadcast actually breaks any established principles (because in this case SIB1 is not transmitted so no different configuration is provided with respect to the one obtained via dedicated signalling) . However, to remove the concern it seems possible to introduce some clarifications in TS 38.331, e.g.

- In the field description of downlinkConfigCommon in ServingCellConfigCommon (as suggested in [2]):

downlinkConfigCommon The common downlink configuration of the serving cell, including the frequency information configuration and the initial downlink BWP common configuration. The parameters provided herein should match the parameters that UE is required to receive via configured by MIB and SIB1 of the serving cell.

- In 5.2.2.4.1 (Actions upon reception of the MIB), adding a Note at the end, e.g.:

Upon receiving the MIB the UE shall:

1>	store the acquired MIB;

1>	if the UE is in RRC_IDLE or in RRC_INACTIVE or if the UE is in RRC_CONNECTED while T311 is running: [FFS]

2>	if the cellBarred in the acquired MIB is set to barred:

3>	consider the cell as barred in accordance with TS 38.304 [20];

3>	if intraFreqReselection is set to notAllowed:

4>	consider cell re-selection to other cells on the same frequency as the barred cell as not allowed, as specified in TS 38.304 [20].

3>	else:

4>	consider cell re-selection to other cells on the same frequency as the barred cell as allowed, as specified in TS 38.304 [20].

2>	else:

3>	apply the received pdcch-ConfigSIB1, subCarrierSpacingCommon, ssb-SubcarrierOffset and dmrs-TypeA-Position and acquire SIB1, if ssb-SubcarrierOffset indicates SIB1 is transmitted in the cell [13] and if SIB1 acquisition is required for the UE.

NOTE:	If the field ssb-SubcarrierOffset indicates that SIB1 is not transmitted in the cell, the UE shall not release the configuration for ControlResourceSetZero and searchSpaceZero for the same cell, if previously received via dedicated signalling.

2) Don't allow the configuration of controlResourceSetZero/searchSpaceZero in pdcch-ConfigCommon within ServingCellConfigCommon if SIB1 is not broadcast (and only rely on the configuration of different commonControlResourceSets (one for each DL BWP) for the (P)Scell where RA needs to be performed).

Configuring a different commonControlResourceSet for each DL BWP is already supported by current ASN.1, however disallowing the configuration of controlResourceSetZero/searchSpaceZero in pdcch-ConfigCommon if SIB1 is not broadcast requires a correction in the following condition:

| Conditional Presence | Explanation |
| --- | --- |
| InitialBWP-Only | If SIB1 is broadcast the field is mandatory present in the PDCCH-ConfigCommon of the initial BWP (BWP#0) in dedicated signalling. It is absent in other BWPs and when sent in system information. In other cases, the field is optionally present. If SIB1 is not broadcast the field is always absent. |

While this solution removes any possible concern about the possible mismatch between the configuration provided by dedicated signalling and the one provided in SIB1, the following drawbacks/consequences should be discussed:

- It can be argued that this is a not backwards compatible change for EN-DC

- This implies that all the concepts/properties of controlResourceSetZero/searchSpaceZero (developed also for EN-DC) cannot be used in EN-DC only cells (for instance the natural association between the SS/PBCH block and CORESET#0) and then the need to e.g. always configure TCI-states for a commonControlResourceSet. Furthermore it is not clear whether concepts like "SS/PBCH block and control resource set multiplexing patterns 1, 2 or 3" apply in case CORESET#0 is missing and a different commonControlResourceSet is considered. For instance, see the following excerpt from TS 38.213 sub-clause 13, linking the discussion on "SS/PBCH block and control resource set multiplexing patterns 2 and 3" to the case of controlResourceSetZero/searchSpaceZero (and then hinting that if those multiplexing patterns need to be used, then controlResourceSetZero/searchSpaceZero need to be configured):

For the SS/PBCH block and control resource set multiplexing patterns 2 and 3, a UE monitors PDCCH in the Type0-PDCCH common search space over one slot with Type0-PDCCH common search space periodicity equal to the periodicity of SS/PBCH block. For the SS/PBCH block and control resource set multiplexing patterns 2 and 3, if the active DL BWP is the initial DL BWP, the UE is expected to be able to perform radio link monitoring, as described in Subclause 5, and measurements for radio resource management [10, TS 38.133] using a SS/PBCH block that provides a control resource set for Type0-PDCCH common search space. For a SS/PBCH block with index , the UE determines the slot index  and  based on parameters provided by Tables 13-13 through 13-15.

- In case of multiple DL BWPs, multiple commonControlResourceSets need to be configured (one per DL BWP), even if they will likely use the same frequency domain resources: RA can only be performed in DL BWPs including the SSB, so all the involved BWPs need to overlap and it is reasonable to also have the option to reuse the same frequency domain resources for the coreset (as possible when relying on controlResourceSetZero/searchSpaceZero)

# Conclusion and proposals

After some offline discussion, there seems to be some consensus to go for solution 1. The following proposals are made:

Proposal 1: Keep the option to configure controlResourceSetZero/searchSpaceZero in pdcch-ConfigCommon within ServingCellConfigCommon, even if SIB1 is not broadcast (Note that this does not remove the possibility to configure different commonControlResourceSets (one for each DL BWP)).

Proposal 2: Agree a CR to TS38.331 clarifying that, if during MIB reception the UE detects that SIB1 is not transmitted in the cell, the UE shall not release the configuration for ControlResourceSetZero and searchSpaceZero for the same cell, if previously received via dedicated signalling.

A CR to TS38.331 reflecting proposal 2 is available in [3].

# References

[1] R2-1815486	Consideration on initial DL BWP and CORESET#0	ZTE Corporation, Sanechips

[2] R2-1813770	Corrections to BWP configuration	Nokia, Nokia Shanghai Bell

[3] R2-1815968	CORESET#0 configuration when SIB1 is not broadcast	ZTE Corporation, Sanechips