3GPP TSG-RAN WG2 Meeting #104 	R2-1816762

# Spokane, USA, 12 – 16 November 2018 											 Revision of R2-1813996

Agenda item:		10.4.1.3.11

Source:	Intel Corporation

Title:	RRC processing delay in NR

Document for:		Discussion and Decision

# Introduction

In RAN2#103bis meeting, RRC procedure delay in NR was discussed and the conclusion is “We aim that by the end of this quarter we agree the numbers for the RRC procedure performance table in 38.331. The focus should not be on IMT2020 requirements at this stage”.

In this contribution, we discuss RRC procedure delay in NR.

# Discussion

In 38.331 section 12, RRC procedure delay (as shown below) is defined as “the time in [ms] from the end of reception of the network -> UE message on the UE physical layer up to when the UE shall be ready for the reception of uplink grant for the UE -> network response message with no access delay other than the TTI-alignment (e.g. excluding delays caused by scheduling, the random access procedure or physical layer synchronisation)”. In LTE, RRC procedure delay varies for different RRC procedures, with typical values of 15 and 20 ms.

Figure 11.2-1: Illustration of RRC procedure delay

For RRC processing time applicable to all UEs, we agree with the analysis in [1] that LTE RRC processing delays are used as baseline for NR. The reasons are as follows:

- The increased UE processing capability for L1/L2 does not necessarily mean that RRC processing capability is increased accordingly.

- RRC processing time includes the processing time for various RF, PHY, MAC/RLC/PDCP/SDAP reconfigurations, and such reconfiguration time might not be improved over LTE.

- The latest TS 38.331 (v15.3.0) has 445 pages, which are roughly the same size as the latest Rel-12 TS 36.331 (v12.17.0) with 448 pages. The reason to choose Rel-12 of LTE RRC specification for comparison is that the supported features in NR Rel-15 are roughly similar to that of LTE Rel-12 (e.g. with dual connectivity introduced from LTE Rel-12). Note that LTE Rel-12 still contains many features not available in NR, e.g. MBMS, Sidelink, Relay, In-device coexistence, MDT, MTC, Home NB, which actually means that RRC specification is actually more complicated in NR compared with that in LTE. This also reflects the increased complexity of physical layer as RRC signalling is used for reconfiguration. Therefore even maintaining similar RRC processing time as LTE actually means that RRC processing time in NR is improved.

Proposal 1: LTE RRC procedure delays are used as baseline for NR.

Although RAN2#103bis agreed that "The focus should not be on IMT2020 requirements at this stage", and acknowledging therefore that this topic might be a secondary priority for RAN2#104, we also provide our views on how to handle the IMT2020 requirements.

We now derive the RRC delay requirement for NR UEs supporting reduced CP latency to satisfy IMT-2020 requirements. It is useful to refer to what has been defined in Rel-15 LTE. In RAN2#102 meeting, it was agreed that if UE supports reduced LTE CP latency to meet 20 ms IMT-2020 requirement, LTE RRC procedure delay is reduced to 3 ms. Following is captured in 36.331 CR R2-1809184.

N = 3 applies for the case of reception of RRCConnectionResume if reducedCP-LatencyEnabled is configured, the UE supports reduced CP latency, and the RRC message only includes MAC and PHY (re-)configurations and does not include (re-)configurations of DRX, SPS, SCells, and MIMO. Further, the UL grant is sent using PDCCH DCI format 0 in common search space. In this scenario, the RRC procedure delay can extend beyond the reception of the UL grant, up to 7 ms.

Given that both LTE and NR are submitted as RITs for IMT-2020, while the CP latency requirement is an optional requirement for Rel-15 LTE UEs, we prefer to apply the same principle to NR, i.e. supporting of reduced CP latency to satisfy IMT-2020 requirements is NR UE capability.

Proposal 2: The support of reduced CP latency to satisfy IMT-2020 requirements is optional UE capability in NR.

For the derivation of RRC procedure delay for reduced CP latency, there are different views on whether the target is 20 ms IMT-2020 requirement, or 10 ms target from section 7.4 of TR 38.913 [2] (“The target for control plane latency should be 10ms”). It should be noted that 10 ms target from TR 38.913 [2] is a target for NR system design, not a requirement for NR Rel-15 system. Similarly, in section 7.5 of TR 38.913 [2], “For URLLC, the target for user plane latency should be 0.5ms for UL, and 0.5ms for DL.” Such 0.5 ms user plane latency is also not used as a requirement in URLLC related evaluation. In summary, only 20 ms IMT-2020 CP latency requirement should be considered when deriving RRC procedure delay requirement for NR UEs supporting reduced CP latency.

Proposal 3: When deriving RRC procedure delay requirement for NR UEs supporting reduced CP latency, 20 ms IMT-2020 CP latency requirement is considered.

In contribution R2-1811689 [3] on NR CP latency evaluation, component 9 (RRC procedure delay and the procedure of UL grant) of message RRCResume is assumed to be 7 ms (aligned with the LTE CP latency CR R2-1809184), which was from the agreed value during LTE CP latency discussion. 3ms is considered as the processing time to reconfigure PHY layer before the UE receive UL grant and overall 7ms is considered to process RRCResume message.

The CP latency result in R2-1811689 [3] is as follows for 4 OFDM symbol PDSCH/PUSCH duration (the results in TR 37.910 [4] Table 5.7.2.1-2 and 5.7.2.1-5 are similar):

Table 1: Control plane latency evaluation results

| CP latency (ms) | Paired spectrum |  | Unpaired spectrum |  |
| --- | --- | --- | --- | --- |
|  | 15 kHz SCS | 30 kHz SCS | 15 kHz SCS | 30 kHz SCS |
|  | 13.8 | 12.3 | 14.3 | 12.4 |

It can be seen that if component 9 is relaxed from 7 ms to 12.7 ms, IMT-2020 CP latency requirement of 20 ms can be still satisfied for all cases. Component 9 includes both the RRC procedure delay and the L1 processing time of UL grant, which is assumed to be 10 and 12 OFDM symbols for 15 and 30 kHz SCS, respectively (according to Table 6.4-1 of TS 38.214 [5]). Therefore RRC procedure delay is 12.7 – 10/14 ≈ 12 ms. It is also proposed to apply similar restriction as in LTE regarding when the reduced CP latency can be applied.

Proposal 4: For NR UEs supporting reduced CP latency, NR RRC procedure delay requirement is set to 12 ms, if the following condition is satisfied: RRC message only includes MAC and PHY (re-)configurations and does not include (re-)configurations of DRX, Configured scheduling, SCells, and MIMO.

# Conclusion

In this contribution, we discuss RRC procedure delay in NR. We propose the following:

Proposal 1: LTE RRC procedure delays are used as baseline for NR.

Proposal 2: The support of reduced CP latency to satisfy IMT-2020 requirements is optional UE capability in NR.

Proposal 3: When deriving RRC procedure delay requirement for NR UEs supporting reduced CP latency, 20 ms IMT-2020 CP latency requirement is considered.

Proposal 4: For NR UEs supporting reduced CP latency, NR RRC procedure delay requirement is set to 12 ms, if the following condition is satisfied: RRC message only includes MAC and PHY (re-)configurations and does not include (re-)configurations of DRX, Configured scheduling, SCells, and MIMO.

Corresponding CR to TS 38.331 is in R2-1818235 [6].

# References

[1] R2-1812750, Qualcomm, "Q157, UE RRC Processing Delays in NR for EN-DC and SA"

[2] 3GPP TR 38.913, "Study on Scenarios and Requirements for Next Generation Access Technologies"

[3] R2-1811689, Intel, "Evaluation of NR CP latency"

[4] 3GPP TR 37.910 v1.0.0 (RP-182102), "Study on Self Evaluation towards IMT-2020 Submission"

[5] 3GPP TS 38.214: “NR; Physical layer procedures for data”

[6] R2-1818235, Intel, "Correction on RRC processing delay"