3GPP TSG-RAN WG2 Meeting #104	R2-1817544

Spokane, USA, 12 - 16 November 2018

Agenda item:	10.4.1.3.11

Source:	Nokia, Nokia Shanghai Bell

Title:	Wait timer in RRC Release

WID/SID:	NR_newRAT-Core - Release 15

Document for:	Discussion and Decision

# 1	Introduction

Wait time in RRC Release message has been discussed and RAN2 has sent an LS [1] to SA2 about the topic. SA2 reply LS [2] is now available. In this paper we discuss further the wait time in RRC Release message.

# 2	Discussion

According to LS [2] SA2 considers that the Wait Timer in the NR RRC release message is useful and SA2 has agreed that the required wait timer in REL-15 in RRC Release can be generic i.e. not slice specific wait timer. This has been now captured in TS 23.501 in [3]. Therefore, we think that RRC specification needs to be aligned with 23.501.

Proposal 1: The wait timer is introduced in RRC Release message

In 36.331 RRCConnectionRelease message can include extended wait time values for delay tolerant UEs or NB-IoT UEs. Since these device types are not supported in REL-15 NR it seems unnecessary to support extended values of the wait time. Therefore, we propose that wait time values in RRC Release message can be the same than in RRC Reject message i.e. up to 16 secs.

Proposal 2: Extended wait timer values are not supported in REL-15

Proposal 3: Wait timer values up to 16 secs are introduced in RRC Release message

# 3	Conclusion

Based on the discussion the following is proposed:

Proposal 1: The wait timer is introduced in RRC Release message

Proposal 2: Extended wait timer values are not supported in REL-15

Proposal 3: Wait timer values up to 16 secs are introduced in RRC Release message

CR capturing the wait time in RRC release message is provided in [4]

# References

- R2-1810964: "LS on Slice based Wait timer in NR RRC release ", RAN2.

- S2-1811520: “LS on Slice based Wait timer in NR RRC release”, SA2

- S2-1811401: “Clarification to the slice based congestion control handling at NG-RAN”, NEC

- R2-1817545: “Introduction of wait time in RRC Release message”, Nokia, Nokia Shanghai Bell