3GPP TSG-RAN WG2#104	R2-1817794

Spokane, Washington, USA, 12th – 16th November 2018

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.331 | CR | CRNum | rev | - | Current version: | 15.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | SCG failure in NE-DC |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  |  | Date: |  | 2018-11-12 |
| Category: | B |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | SCG Failure handling for EN-DC is missing. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Add procedure description of SCGFailureInformation reporting, including failure type determination and setting the contents of MeasResultSCG-Failure in 5.6.13.x1 and 5.6.13.x2. - Add message defination and corresponding asn.1 for MeasResultSCG-Failure IE. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | SCG Failure handling for EN-DC is not supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.6.13, 6.3.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  | TR 38.331 |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

START OF CHANGES

### 5.6.13	SCG failure information

#### 5.6.13.1	General

Figure 5.6.13.1-1: SCG failure information

The purpose of this procedure is to inform E-UTRAN about an SCG failure the UE has experienced i.e. SCG radio link failure, SCG change failure.

#### 5.6.13.2	Initiation

A UE initiates the procedure to report SCG failures when SCG transmission is not suspended and when one of the following conditions is met:

1>	upon detecting radio link failure for the SCG, in accordance with 5.3.11; or

1>	upon SCG change failure, in accordance with 5.3.5.7a; or

1>	upon stopping uplink transmission towards the PSCell due to exceeding the maximum uplink transmission timing difference when powerControlMode is configured to 1, in accordance with subclause 7.17.2 of TS 36.133 [29].

In case of DC, upon initiating the procedure, the UE shall:

1>	suspend all SCG DRBs and suspend SCG transmission for split DRBs;

1>	reset SCG-MAC;

1>	stop T307;

1>	if the UE is operating in NE-DC:

2>	initiate transmission of the SCGFailureInformationEUTRA message as specified in TS 38.331 [x.x.x.x];

1>	else

2> initiate transmission of the SCGFailureInformation message in accordance with 5.6.13.3;

END OF CHANGES

START OF CHANGES

#### 5.6.13. x1	Failure type determination in NE-DC

The UE shall set the contents of the SCGFailureInformationEUTRA message as follows:

1>	if the UE initiates transmission of the SCGFailureInformationEUTRA message due to T307 expiry:

2>	set the failureType as t307-Expiry;

1>	else if the UE initiates transmission of the SCGFailureInformationEUTRA message to provide SCG change failure information:

2>	set the failureType as scg-ChangeFailure;

1>	else if the UE initiates transmission of the SCGFailureInformationEUTRA message to provide random access problem indication from SCG MAC:

2>	set the failureType as randomAccessProblem;

1>	else if the UE initiates transmission of the SCGFailureInformationEUTRA message to provide indication from SCG RLC that the maximum number of retransmissions has been reached:

2>	set the failureType as rlc-MaxNumRetx;

#### 5.6.13. x2	Setting the contents of MeasResultSCG-Failure

The UE shall set the contents of the MeasResultSCG-Failure as follows:

1>	for each E-UTRA SCG serving frequency included in measResultServFreqList, include within measResultBestNeighCell the physCellId and the quantities of the best non-serving cell, based on RSRP, on the concerned serving frequency;

1>	set the measResultNeighCells to include the best measured cells on non-serving E-UTRA frequencies, ordered such that the best cell is listed first, and based on measurements collected up to the moment the UE detected the failure, and set its fields as follows;

2>	if the UE was configured to perform measurements for one or more non-serving EUTRA frequencies and measurement results are available, include the measResultListEUTRA;

2>	for each neighbour cell included, include the optional fields that are available;

NOTE 1:	The measured quantities are filtered by the L3 filter as configured in the mobility measurement configuration. The measurements are based on the time domain measurement resource restriction, if configured. Blacklisted cells are not required to be reported.

END OF CHANGES

START OF CHANGES

### 6.3.5	Measurement information elements

#### –	MeasResultSCG-Failure

The IE MeasResultSCG-Failure is used to provide information regarding failures detected by the UE in case of NE-DC.

MeasResultSCG-Failure information element

-- ASN1START

MeasResultSCG-Failure-r15 ::=			SEQUENCE {

measResultServFreqList-r15			MeasResultServFreqList-r10		OPTIONAL,

measResultNeighCells-r15			MeasResultList2EUTRA-r9			OPTIONAL,

...,

}

-- ASN1STOP

END OF CHANGES