# 3GPP TSG-RAN2 Meeting #104	R2-1817827

# Spokane, USA, 2018-11-12 to 2018-11-16

Title:	[DRAFT] Reply LS on network synchronization for handover purposes

Response to:	LS on network synchronization for handover purposes (R1-1812074/R2-1816222)

Release:	Release 15

Work Item:	NR_newRAT-Core

Source:	Ericsson [To be RAN WG2]

To:	RAN1

Cc:	-

Contact Person:

#### Name:	Henning Wiemann

Tel. Number:	+49-2407-575-0

E-mail Address:	Henning [dot] Wiemann [at] Ericsson [dot] com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:

1. Overall Description:

RAN2 would like to thank RAN1 for their LS.

The section 5.3.5.5.2 of 38.331 that RAN1 mentions in the LS is the so-called “Reconfiguration with sync” procedure. It is used for (but not limited to) any kind of handover of the PCell (MCG) as well as for addition or change of the PSCell (SCG). It is the only connected mode procedure by which the network can instruct the UE to change the PCell/PSCell, i.e., to leave the current (source) cell and connect to another (target) cell. The indicated target cell may be on the same carrier as the source cell or on a different one.

As RAN1 observed, the UE is required to acquire MIB of the target cell when instructed to perform the “reconfiguration with sync”, i.e., before performing the RA towards the indicated target cell.

RAN2 discussed briefly whether the UE has to acquire MIB necessarily during the “reconfiguration with sync” and whether it has to do it in all cases. RAN2 observed that a UE implementation might e.g. choose to acquire MIB of the (candidate) target cell(s) already when performing measurements. If it does that, it may already have the necessary information and may then omit the (re-)acquisition of MIB during the actual handover execution. RAN2 noticed that there may also be cases in which the content of the MIB (e.g. half-frame indicator, SFN, ...) is not necessary for performing the HO. In such cases, the UE implementation could also refrain from reading MIB during the actual HO. However, despite these possible exceptions, RAN2 agreed to write the specification so that upon performing the RA towards the target cell the UE has acquired all necessary information of the target cell’s MIB.

2. Actions:

To RAN1

ACTION: 	 RAN2 kindly asks RAN1 to take the information above into account.

3. Date of Next TSG-RAN WG2 Meetings:

TSG-RAN WG2 Meeting #105	2019-02-25 to 2019-03-01	Athens, Greece

TSG-RAN WG2 Meeting #105bis	2019-04-08 to 2019-04-12	China