3GPP TSG-RAN WG2 Meeting #105											R2-1900674

Athens, Greece, 25th February - 1st March 2019

Agenda item:	11.2.2.1

Source:	ZTE corporation, Sanechips

Title:	Considerations on cell selection/reselection for NR-U

Document for:	Discussion and Decision

# Introduction

The work item on NR-U has been agreed [1] and one of the objectives of the work item includes:

-	Inactive and Idle mode procedures: Rel-15 NR design is the baseline, including NR licensed measurement framework (cell and beam quality derivation for RSRP, RSRQ, and SINR, filtering and combining multiple beams). Specify necessary extensions for NR-U operation for cell selection/reselection, including enabling the possibility for the UE to camp on a non-best cell if the best cell does not belong to the registered PLMN (or E-PLMN), where the non-best cell would still be the best cell of the registered PLMN; increased paging opportunities per DRX cycle for the UE to receive the page; increased opportunities for SI delivery

This contribution will focus on the discussion about camping on the non-best cell and the other necessary extensions for NR-U operation for cell selection/reselection.

# Discussion

During the cell reselection process, UE shall perform detection, measurements and evaluation of the neighboring cells regularly. UE shall perform ranking of all candidate cells that fulfil the cell selection criterion S [2]. If a better cell is found, that cell is selected. And then, UE shall camp on a suitable cell, monitor control channel(s) of that cell so that the UE can receive system information, paging, or initiate transfer to connected mode.

On NR-U, since LBT mechanism used for obtaining the channel is introduced, the concern is how to select a suitable cell for UE on NR-U. Two solutions that had been discussed in SI stage are as below:

- Channel occupancy metric

- Enabling the UE to camp on a non-best cell

## Channel occupancy metric

In licensed spectrum, the factors determining the cell selection are the channel quality, such as RSRP and RSRQ. But, in unlicensed spectrum, the success/failure of the LBT plays a major role in the perceived QoS on the cell. A cell with good channel quality may suffer with intense contention and hence offer poor quality of service as a result.

Furthermore, in idle or inactive mode, UE needs to be able to receive transmissions such as system information and paging in a timely manner. It also needs to perform random access procedure efficiently. For these reasons, it is highly desirable that the UE does not camp on a cell for which the channel occupancy is excessive. To ensure this and to enable better load balance between UEs, the cell reselection evaluation needs to incorporate an estimate of the channel load of the cell (e.g. in cell quality metric) [3]. For example, an offset (such as QoffsetCO) can be applied to the cell-ranking criterion for neighboring cells as below, so that, the cells with the high load are deprioritized. If the measurement results for the channel load of a neighboring cell are high than a threshold, so that this cell is declared to be overload, Rn of this cell is lowered after subtracting a QoffsetCO. QoffsetCO is an offset parameter that is configured by the network and this will be applied when the channel occupancy on a given frequency is greater than a threshold configured by the network.

Rn = Qmeas,n -Qoffset - Qoffsettemp - QoffsetCO

In addition, the definition and the requirements of the channel occupancy metric are up to RAN1 and RAN4.

Proposal 1: The channel occupancy metric is beneficial to be introduced in the cell reselection procedure.

## 2.2 Non-best cell

In NR-U, due to the unplanned deployment, multiple operators may share a frequency, so that, the highest ranked cell on the cell (re)selection procedure is not suitable due to not belonging to the registered PLMN (or equivalent PLMN), which is common. Hence, UE should exclude those cells not belonging to the registered PLMN (or equivalent PLMN) for cell reselection.

However, the time that UE spends to decode SIB1 successfully is determined by the time that UE obtains downlink synchronization, frame timing, as well as the periodicity of SIB1. UE may need to read every MIB/SIB1 of the candidate cells until a cell belonging to the registered PLMN (or equivalent PLMN) is found. As a result, the requirement that UE obtains the PLMN information from multiple cells will result in a very high power consumption and/or a long cell reselection time at the UE.

Observation 1: The requirement that UE obtains the PLMN information from multiple cells will result in a very high power consumption and/or a very long cell reselection time for the UE.

Hence, some means to mitigate this impact are preferable. For instance, the serving cell can offer UE the PLMN information of the neighbour cells. Although this is feasible, in unlicensed deployments, it is not practical for the serving cell to get the PLMN information about all possible neighbour cells. Even if the serving cell can get those via ANR, the PLMN information is unreliable due to the PCI collision etc.

Observation 2: The PLMN information of neighboring cell indicated from the serving cell is unreliable.

In the conclusion, RAN2 should seek for a feasible power saving solution. One possibility is to lower the priority of this frequency during the cell reselection procedure. This solution can enable UE to select to the best cell in the other frequency with the equal or lower priority fast.

Proposal 2: If the cell does not belong to the registered PLMN (or E-PLMN), UE can lower the priority of the frequency of this cell during the cell reselection procedure, to balance the power saving and search for the best cell.

# Summary

Proposal 1: The channel occupancy metric is beneficial to be introduced in the cell reselection procedure.

Observation 1: The requirement that UE obtains the PLMN information from multiple cells will result in a very high power consumption and/or a very long cell reselection time for the UE.

Observation 2: The PLMN information of neighboring cell indicated from the serving cell is unreliable.

Proposal 2: If the cell does not belong to the registered PLMN (or E-PLMN), UE can lower the priority of the frequency of this cell during the cell reselection procedure, to balance the power saving and search for the best cell.

# References

- RP-182878, "New WID on NR-based Access to Unlicensed Spectrum".

- 3GPP TS 38.304: "NR; User Equipment (UE) procedures in Idle mode and RRC Inactive state".

- R2-1817904, “Use of channel occupancy metric for cell reselection in NR-U”, InterDigital Inc., Charter Communications, Inc., ASUSTeK, Ericsson, ZTE Corporation, OPPO