3GPP TSG-RAN2 Meeting #105	R2-1902194

# Athens, Greece, 25th Feb 2019 - 1st Mar 2019

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.321 | CR | 1435 | rev | - | Current version: | 15.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on Tx carrier reselection behavior in TS 36.321 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ITL |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_eV2X-Core |  |  |  |  |  | Date: |  |  | 2019-02-15 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | According to the current TS 36.321, when the Tx carrier reselection is triggered, the UE considers a CBR of carrier(s). If the CBR of a current carrier is below the threshold, the UE continues to use the current carrier and associated resource pool. Else if the CBR of the current carrier is above the threshold, the UE select other carrier and associated resource pool based on CBR results. However, else if the CBR of other carrier is above the threshold and there is no candidate carrier, the UE behaviour is not clear. Therefore, we suggest that if none of the configured carries meets CBR threshold, the UE can randomly select a configured carrier. |  |  |  |  |  |  |  |  |
| Summary of change: |  | In 5.14.1.5 in TS 36.321, add a condition to select select carrier(s) when the CBR of other carrier is above the threshold and there is no candidate carrier. Impact analysis Impacted functionality: Tx carrier (re)selection Inter-operability: If the network is implemented according to this CR while the UE is not, there is no inter-operability issue. If the UE is implemented according to this CR while the network is not, there is no inter-operability issue. If one UE is implemented according to this CR while the other is not, there is no inter-operability issue. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The Tx carrier (re)selection procedue can work properly. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.14.1.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

CHANGE START

#### 5.14.1.5	TX carrier (re-)selection for V2X sidelink communication

The MAC entity shall consider a CBR of a carrier to be one measured by lower layers according to TS 36.214 [6] if CBR measurement results are available, or the corresponding defaultTxConfigIndex configured by upper layers for the carrier if CBR measurement results are not available.

If the TX carrier (re-)selection is triggered for a Sidelink process according to sub-clause 5.14.1.1, the MAC entity shall:

-	if there is no configured sidelink grant on any carrier allowed for the sidelink logical channel where data is available as indicated by upper layers (TS 36.331 [8] and TS 24.386 [15]):

-	for each carrier configured by upper layers associated with the concerned sidelink logical channel:

-	if the CBR of the carrier is below threshCBR-FreqReselection associated with the priority of the sidelink logical channel:

-	consider the carrier as a candidate carrier for TX carrier (re-)selection for the concerned sidelink logical channel.

-	else:

-	for each sidelink logical channel, if any, where data is available and that are allowed on the carrier for which Tx carrier (re-)selection is triggered according to subclause 5.14.1.1:

-	if the CBR of the carrier is below threshCBR-FreqKeeping associated with priority of the sidelink logical channel:

-	select the carrier and the associated pool of resources.

-	else:

-	for each carrier configured by upper layers on which the sidelink logical channel is allowed, if the CBR of the carrier is below threshCBR-FreqReselection associated with the priority of the sidelink logical channel;

-	consider the carrier as a candidate carrier for TX carrier (re-)selection.

The MAC entity shall:

-	if one or more carriers are considered as the candidate carriers for TX carrier (re-)selection:

-	for each sidelink logical channel allowed on the carrier where data is available and Tx carrier (re-)selection is triggered:

-	select one or more carrier(s) and associated pool(s) of resources among the candidate carriers with increasing order of CBR from the lowest CBR.

-	else if the candidate carrier is not available for Tx carrier (re-)selection:

-	for each sidelink logical channel allowed on the carrier where data is available and Tx carrier (re-)selection is triggered:

-	randomly select one or more carrier(s) and associated pool(s) of resources.

NOTE 1:	It is left to UE implementation how many carriers to select based on UE capability.

NOTE 2:	It is left to UE implementation to determine the sidelink logical channels among the sidelink logical channels where data is available and that are allowed on the carrier for which Tx carrier (re-) selection is triggered.

NOTE 3:	If the MAC entity is configured by the upper layer to receive a sidelink grant dynamically on the PDCCH, it is left to UE implementation to determine which carriers configured by upper layer in sl-V2X-ConfigDedicated, as specified in TS 36.331 [8] are considered as selected carriers for the sidelink synchronization procedures in subclauses 5.10.7, 5.10.8 and 5.10.8a of TS 36.331 [8].

CHANGE END