3GPP TSG-RAN WG2 Meeting #105bis	R2-1905168

Xi'an, China, 8th April -12th April 2019

Agenda item:	11.2.2.1

Source:	Charter Communications

Title:	Views on LBT Impacts in MAC in NR-U Operation

WID/SID:	NR-U WID - Release 16

Document for:	Discussion and Decision

# 1	Introduction

Operation in unlicensed spectrum requires conformance to regulatory rules. Particularly, listen-before-talk (LBT) rule ensures that operating nodes do not transmit over each other. While LBT is very important for operation in unlicensed bands, it calls for a revision in some of the NR baseline procedures.

In NR-Unlicensed WID [1], the implications that LBT brings has been captured in a few of the objectives. Specifically, the WID lists the following:

-	Initial access: specify required NR modifications to increase the maximum number of candidate SS/PBCH block positions within the DRS transmission window; to handle reduced SS/PBCH block and RMSI transmission opportunities due to LBT failure; to determine frame timing and QCL assumptions from the detected SS/PBCH block; single SS/PBCH block numerology assumed per band for Pcells in unlicensed spectrum. (RAN1)

- 	Random access: specify required NR modifications to enhance RACH procedure in line with the agreements during the study phase, including 4-step RACH modifications to handle reduced Msg 1/2/3/4 transmission opportunities due to LBT failure (RAN1/RAN2); LBT for 2-step RACH and application of PRACH and PUSCH format improvements for NR-U to 2-step RACH. (RAN1

-	Scheduling request: specify required NR modifications due to LBT failure in line with agreements during the study phase. (RAN1/RAN2)

- 	RLM/RRM extensions for NR-U operation due to uncertain and reduced transmission opportunities for DL signals and channels due to LBT failure in line with agreements during the study phase (NR-U TR section 7.2.1.3.2), including configuring different DRS Measurement Time Configuration (DMTCs) for RRM and RLM respectively, identifying the set of RLM-RSs to measure, which set(s) are used for in-sync, out-of-sync evaluations, potential definition of a metric to accurately identify unsuccessful detection of RLM-RS. Support RSSI reporting. Define a metric to measure channel occupancy or medium contention and its corresponding reporting. (RAN1/RAN2).

- Potential enhancements to DRX considering channel access uncertainty. (RAN2)

In RAN2 #105 meeting [2], the following agreement was reached:

- Consistent LBT failures can lead to RLF, at least for UL transmissions, for which consistent failures can currently eventually lead to RLF

In this contribution, we express our views regarding potential issues that LBT could bring in various MAC procedures during NR-U operation.

# 2	Discussion

There are several MAC procedures that get affected due to possible LBT failure. Particularly, if LBT failures happen consistently, it would trigger timers or counters that eventually may lead to significant delay. The following provides a non-comprehensive list:

- Scheduling request (SR):

  - Transmission of SR, like transmission of any UL signal or channel, is subject to LBT. Initiation of SR is subject to transmission of any prior pending SR, which might have not been transmitted due to LBT failure. Consistent failure in SR transmission could ultimately trigger RLF procedure.

- Random access:

  - Transmission of PRACH is subject to LBT. Upon transmission of a PRACH, several counters and timers in MAC are maintained. If a PRACH is not transmitted due to LBT, maintenance of such timers and counters should be revisited.

- RLM/RRM:

  - Consecutives failures in SR transmission and in RA procedure would trigger RFL. In NR-U, such consecutives failures could occur more often due to LBT failures.

  - Transmission of SSB and CSI-RS is subject to LBT. Several consecutive LBT failures at gNB leads to out-of-syncs notifications from PHY to MAC at UE side, which triggers timers and eventually may lead to RLF.

In the following, each of above are discussed in details.

## 2.1 Scheduling Request

In the baseline specification, the SR counter and SR prohibit timer are maintained by MAC and depending on their status, an SR transmission may be initiated, prevented, or cancelled.

- Upon MAC layer indication to PHY to transmit a scheduling request in a PUCCH resource, the SR counter is increased by one. In NR-U operation, the actual transmission may be blocked if LBT is not completed successfully. Therefore, incrementing the SR counter as in the baseline spec, may result in a situation where PHY has not succeeded in transmission of the SR, and yet MAC is expecting a response from the gNB.

- Similarly, upon MAC layer indication to PHY to transmit an SR in a PUCCH resource, MAC entity starts the SR prohibit timer. If this timer has started, MAC entity would block transmission of another SR until the timer expires. Therefore, starting the SR prohibit timer as in the baseline spec may result in a situation where PHY has not succeeded in transmission of the SR (due to LBT failure), yet the MAC entity prevents transmission of subsequent SRs, in anticipation of a response from the gNB.

- While an occasional LBT failure may not cause significant problems in SR procedure, if LBT failure happens consistently it would lead to the SR counter to reach its maximum threshold, and subsequently triggering RLF.

The current SR procedure (SR initiation and prevention of subsequent SR transmission) are designed considering operation in licensed spectrum. However, for operation in unlicensed spectrum, it is vital to make MAC entity aware of whether PHY has been able to transmit the SR as instructed. Therefore, we believe that in SR procedure, MAC entity should be able to distinguish an LBT failure vs an unsuccessful transmission event. The consistent occurrence of each these events should indeed trigger a recovery effort, however if these events are not distinguished in MAC their occurrence gets aggregated and unnecessarily trigger a recovery effort.

Observation 1: In SR procedure, MAC entity should be able to distinguish an LBT failure vs an unsuccessful transmission event. The consistent occurrence of each of these events should indeed trigger a recovery effort, however if these events are not distinguished in MAC their occurrence may get aggregated and unnecessarily trigger a recovery effort.

## 2.2 Random Access

In the baseline specification, during random access procedure the preamble transmission counter, power ramping counter, RA response window and RA contention resolution timer are maintained by MAC and depending on their status, the random-access procedure progresses differently.

- Upon MAC layer indication to PHY to transmit a preamble in a PRACH resource, the preamble transmission counter is increased by one. In NR-U operation, the actual transmission may be block if LBT is not completed successfully. Therefore, incrementing this counter as in the baseline spec, may result in a situation where PHY has not succeeded in transmission of the preamble, and yet MAC is expecting the RA response from the gNB.

- Similarly, upon MAC layer indication to PHY to transmit a preamble in a PRACH resource, MAC entity starts the RA response window. If this timer has started, MAC entity expects the response unless the timer expires, in which case the preamble may be retransmitted. Therefore, starting the RA response timer as in the baseline spec may result in a situation where PHY has not succeeded in transmission of the preamble (due to LBT failure), yet the MAC entity expects a RA response from the gNB.

- With each preamble retransmission, the preamble power is ramped, unless a notification of suspending power ramping counter is received from PHY. With several LBT failures in a row, the preamble power may be ramped several times without the preamble even being transmitted. Hence a procedure that is designed to take care of varying propagation loss across UEs, malfunctions in light of LBT failures. While the notification of power ramping suspension from PHY can partially alleviate the LBT problem, it is only a solution that could take of the power raping counter and not the other RA-related counters and timers.

- Moreover, the RA contention resolution timer may have been started while the intended transmission has been blocked due to LBT failure.

- While an occasional LBT failure may not cause significant problems in RACH procedure, in a sequence of such LBT failures causes the above counters and timers reach their maximum thresholds, and subsequently triggering RLF.

The current RA procedure are designed considering operation in licensed spectrum. However, for operation in unlicensed spectrum, it is vital to make MAC entity aware of whether PHY has been able to transmit the RA preamble or Msg3 as instructed. Therefore, we believe that in RA procedure, MAC entity should be able to distinguish an LBT failure vs an unsuccessful transmission event that would occur due to propagation loss (e.g. higher path-loss than expected which the power ramping concept could take care of). The baseline RA spec takes care of the latter, while for operation in unlicensed spectrum the former should be addressed as well. If these events are not distinguished in MAC their occurrence gets accumulated, which unnecessarily trigger a recovery effort.

Observation 2: In RA procedure, MAC entity should be able to distinguish an LBT failure vs an unsuccessful transmission event that would occur due to propagation loss. The baseline RA spec takes care of the latter, while for operation in unlicensed spectrum the former should be addressed as well. If these events are not distinguished in MAC their occurrence may get accumulated, which unnecessarily trigger a recovery effort.

## 2.3 RLM and RRM

In the baseline specification, the management of radio resource in connected mode is covered under radio resource management (RRM) and radio link monitoring (RLM). The RLM is a mechanism defined so that a UE can perform fast recovery without going to the RRC Idle mode.

In the baseline RLM spec, if several out-of-syncs notifications is received from PHY a timer to observe the situation is triggered. To be precise, upon receiving N310 consecutive out-of-syncs from PHY, the timer T310 gets started until it is stopped by several receiving consecutive in-syncs from PHY (N311). Or, it may expire, in which case it leads to radio link failure (RLF). During operation in unlicensed spectrum, occurrence of several out-of-syncs may not be prevented due to successive LBT failures at the gNB.

As mentioned in previous sections, consecutive LBT failures in SR and RA procedures could lead to expiry of timers or counters reaching their maximum values which would lead to RLF. Particularly, RLF gets triggered if the preamble transmission counter exceeds the maximum threshold. This could possibly happen when consecutive SR retransmission happens (for a given SR configuration) and the SR counter, as discussed above, exceeds its maximum threshold.

A consistent LBT failures during performing SR or RA procedures could lead to trigger RLF. This could happen due to following: 1) congested unlicensed channel due to presence of other NR-U or other technologies nodes, 2) presence of interfering nodes in the vicinity of the UE but hidden to gNB. In other to more efficiently handle RLF, the underlying events should be distinguished. The propagation-related events is the underlying cause for many remedies considered in the baseline spec. However, in NR-U operation, consecutive LBT failures could lead to triggering of RLF. We believe it is best if the event of consecutive LBT failures is treated separately.

Observation 3: In other to more efficiently handle RLF, the underlying events that lead to RLF should be distinguished and treated separately. Particularly for NR-U operation, consecutive LBT failures is a new event that has not been considered in the baseline spec.

While it may not be impossible to use existing counters and timers to distinguish an event of consecutive LBT failures, it’d require a redo of many of the baseline procedures to detect occurrence of such event. We believe that it is more straightforward and systematic to have a new mechanism for systematic LBT failure in MAC without changing the baseline procedures.

Observation 4: It is more straightforward and systematic to have a new mechanism for systematic LBT failure in MAC without changing the baseline procedures.

Proposal 1: The occurrence of successive LBT failures as a new underlying event should be distinguished and treated separately.

Given MAC is enabled to distinctly recognize consecutive LBT failures, appropriate solutions can be considered to address the problem. For instance, the UE may be better off to switch BWP or to another cell etc. But even more timely, an average indication of such events can early on be reported to gNB for further optimization. Ultimately, if successive LBT failure metric reaches a threshold the UE may initiate RLF, but before that for a more timely reaction other measures should be considered.

Observation 5: Given MAC is enabled to distinctly recognize consecutive LBT failures, appropriate solutions can be considered to address the problem.

Proposal 2: Enable MAC to distinctly recognize consecutive LBT failures and take appropriate actions (FFS) such as LBT failure reporting.

## 2.4 Common LBT Status Indication

Considering above discussion, an LBT failure may happen in preparation to transmit in PUCCH or PRACH resources. While not part of the discussion of this document, LBT failure during PUSCH access should also be taken into account when considering consistent LBT failures. We recognize that occurrence of LBT failure also depends on the LBT category which should be considered appropriately.

An occurrence of LBT failure during access of any of above-listed physical channels should be considered in case a pattern of consistent LBT failures happen. While PHY layer would not be able to recognize such pattern, it can pass such indication to MAC. Therefore, we propose to enable PHY to send LBT status indication during accessing any physical channel to enable MAC to recognize consistent LBT failure patterns.

Proposal 3: Consider a common LBT status indication from PHY to MAC, during accessing any uplink physical channel, to enable MAC to recognize consistent LBT failure patterns.

# 3	Conclusions

In this contribution, we highlighted our concerns regarding consequences that LBT could bring in several MAC procedures. We have proposed the following:

Observation 1: In SR procedure, MAC entity should be able to distinguish an LBT failure vs an unsuccessful transmission event. The consistent occurrence of each of these events should indeed trigger a recovery effort, however if these events are not distinguished in MAC entity their occurrence may get accumulated and unnecessarily trigger a recovery effort.

Observation 2: In RA procedure, MAC entity should be able to distinguish an LBT failure vs an unsuccessful transmission event that would occur due to propagation loss. The baseline RA spec takes care of the latter, while for operation in unlicensed spectrum the former should be addressed as well. If these events are not distinguished in MAC their occurrence may get accumulated, which unnecessarily trigger a recovery effort.

Observation 3: In other to more efficiently handle RLF, the underlying events that lead to RLF should be distinguished and treated separately. Particularly for NR-U operation, consecutive LBT failures is a new event that has not been considered in the baseline spec.

Observation 4: It is more straightforward and systematic to have a new mechanism for systematic LBT failure in MAC without changing the baseline procedures.

Observation 5: Given MAC is enabled to distinctly recognize consecutive LBT failures, appropriate solutions can be considered to address the problem.

Proposal 1: The occurrence of successive LBT failures as a new underlying event should be distinguished and treated separately.

Proposal 2: Enable MAC to distinctly recognize consecutive LBT failures and take appropriate actions such as LBT failure reporting.

Proposal 3: Consider a common LBT status indication from PHY to MAC, during accessing any uplink physical channel, to enable MAC to recognize consistent LBT failure patterns.

# References

RP-182878, “New WID on NR-based Access to Unlicensed Spectrum,” Qualcomm, RAN#82, December 2018

RAN2 #105 Chairman notes, Athens, Greece, RAN2#105, February-March 2019