3GPP TSG RAN WG2 Meeting #106        				 R2-1905897
Reno, USA, 13th-17th May 2019                                              (Revision of R2-1903268)

Agenda Item:	12.3.2.1

Source: 	MediaTek Inc.

Title:  	Single/Dual RRC for Simultaneous Connectivity during HO

Document for:	Discussion and decision

# 1. Introduction

In RAN2#103bis meeting, simultaneous connectivity with both the source cell and the target cell during HO was discussed. Following FFS issues were identified for further discussion.

=>	Use the protocol stack comparison in this contribution as baseline for further discussions between the split bearer and non-split bearer solutions.

=>	We should discuss the security key aspects more when we discuss the details of the solutions.

=>	Consider how to do reordering in non-split case

=>	FFS whether single or dual RRC (and e.g. whether we have 1 or 2 S1-C connections) is considered (S1-C would affect also RAN3)

=>	FFS how duplication is considered (depending on solution details)

Up to now, there is no common understanding on what ‘dual RRC’ exactly means. So it should be clarified first before considering its necessity. In this contribution, we first clarify our understanding on dual RRC and then justify it by potential benefits and certain deployment scenarios.

# 2. Discussion

In current LTE, the UE only has one RRC connection with the network and C-RNTI is used as the unique identification for identifying the RRC connection.  Furthermore, there is only one S1-MME connection per UE. In LTE for UE in CONNECTED mode, one RRC connection and one S1-MME connection per UE is a basic principle applied in all the RRC procedures, no matter whether UE is configured with CA or DC, in normal reconfiguration procedure or in HO procedure.

Observation 1: For UE in CONNECTED mode, there is only one RRC connection and only one S1-MME connection per UE.

## 2.1 Single S1-C

In our understanding, the objective of the WI is to reduce the user data interruption during HO by RAN solutions. It is not intended to impact CN and S1 procedures and signalling.  There is no clear benefit to justify the necessity of dual S1-C and both RAN3 and CT1 needs to be involved for the evaluation. Furthermore, user data interruption during HO can be reduced even with single S1-C connection.

One benefit of single S1-C is that the existing path switching and BEARER modify procedure in the phase of HO completion can be reused. E.g. the target eNB sends a PATH SWITCH REQUEST message to MME to inform that the UE has changed cell. The MME sends a MODIFY BEARER REQUEST message to the Serving Gateway. The Serving Gateway switches the downlink data path to the target side. Then the Serving Gateway sends a MODIFY BEARER RESPONSE message to MME. the MME confirms the PATH SWITCH REQUEST message with the PATH SWITCH REQUEST ACKNOWLEDGE message. If there were two S1-C connections during HO, it will be very complicated on how to do the path switch.

Proposal 1: There is only one S1-C for simultaneous connectivity HO.

Proposal 2: The path switching and BEARER modify procedures in HO complete phase can be reused for simultaneous connectivity HO.

## 2.1 Dual RRC

Under the assumption that there is only one S1-C, the dual RRC is considered as a tentative RRC status from UE aspect when simultaneous connectivity with both the source cell and the target cell exists.  From RRC aspect, dual RRC starts when the connection with the target cell is established, i.e. upon transmission of RRCConnectionReconfigurationComplete and ends when the connection with source cell is released.

Observation 2: Dual RRC is a tentative status, existing in the simultaneous connectivity period with dual active protocol stacks. It begins when transmission of RRCConnectionReconfigurationComplete message to the target and ends when the connection with the source cell is released.

During the simultaneous connectivity period, UE keeps two sets of RRC configurations. One is for the source cell and the other one is for the target cell. Each set of the RRC configuration includes lower layer configurations, security configurations etc. to guarantee that data transmission/reception with both the source cell and the target cell can be performed simultaneously.

Observation 3:  In the period of simultaneous connectivity during HO, UE keeps two sets of RRC configurations for the source cell and the target cell simultaneously.

The necessary RRC functions and procedures required at the source cell during the period of tentative dual RRC also needs to be clarified.  One necessary function is RRC connection release. In dual RRC, it allow the possibility for the source cell to release the connection with the UE directly without involvement of the target cell. For example, the source cell can send the release message directly to the UE when all the buffered data before SN status transfer is transmitted completely. UE can also release the connection with the source cell when RLF on the source cell occurs or when the DataInactivityTimer for the source cell at the UE side expires.  So the RLM/RLF and RRC connection release can be performed at the source cell during the simultaneous connectivity period. Other RRC procedures, e.g. RRM measurement and RRC reconfiguration may not be necessary.

In order to support the RRC functions in source cell, e.g. RRC connection release, SRB1 needs to be kept with the source cell in dual RRC. However, the RRC connection with the target cell has already established and SRB1 is also required to support the RRC functions/procedures at the target side. In EN-DC, although there is only one RRC entity at the UE side, different SRB types are defined for MCG and SCG respectively, i.e. SRB1 and SRB3. In dual RRC, there is no differentiation of MCG or SCG and both SRB1 are needed to maintain the connection with the source and the target. How to differentiate the SRB1 from different cells need to be considered. One straightway is to have two RRC entities at the UE side in tentative dual RRC just as illustrated in figure 1.

Figure 1 CP Protocol for dual RRC

Observation 4: In tentative dual RRC, at least the RRC procedures of RLM/RLF and RRC connection release can be performed at the source cell.

Observation 5: In tentative dual RRC, there may be two SRB1 for the source and the target cell simultaneously.

Proposal 3: Dual RRC is characterised by the following aspects:

- UE has two RRC entities for the source cell and the target cell simultaneously;

- UE keeps two SRB1 for the RRC procedures performed with the source cell and the target cell simultaneously;

- At least the RRC procedures of RLM/RLF and RRC connection release for the source cell are performed at the RRC entity corresponding to the source cell.

## 2.2 Single RRC

Compared with dual RRC, single RRC is simple, which is the same as normal HO procedure. Even there is simultaneous connectivity during HO, RRC connection is switched from the source cell to the target cell upon transmission of the RRCReconfigurationComplete message. Although UE has only one RRC entity, UE also needs to keeps two sets of RRC configurations for the source cell and target cell for simultaneous transmission/reception. UE only has one SRB1 performing the RRC procedures with the target cell. In this case, if the release of the connection with the source cell is explicitly signalled, it will be the target eNB who will sends the dedicated RRC message. Different from dual RRC that the source cell can release UE connection by itself, certain coordination between the source eNB and the target eNB is required and then the connection with the source cell is released by the target side.

Observation 6: In single RRC, the release of the connection with source cell requires some coordination between the source eNB and the target eNB and the corresponding RRC message is transmitted by the target eNB.

For dual RRC, the complexity at both the UE and network side may be concerned, the compatibility in current LTE with single RRC entity needs to be considered. It will also introduce additional standardization effort. Considering single RRC is workable for both single active protocol stack and dual active protocol stacks, single RRC is supported as baseline.

Proposal 4: Single RRC is supported for simultaneous connectivity HO.

# 3 Conclusion

In this contribution, we clarify our understanding on dual RRC and justify its usage.

Observation 1: For UE in CONNECTED mode, there is only one RRC connection and only one S1-MME connection per UE.

Observation 2: Dual RRC is a tentative status, existing in the simultaneous connectivity period with dual active protocol stacks. It begins when transmission of RRCConnectionReconfigurationComplete message to the target and ends when the connection with the source cell is released.

Observation 3:  In the period of simultaneous connectivity during HO, UE keeps two sets of RRC configurations for the source cell and the target cell simultaneously.

Observation 4: In tentative dual RRC, at least the RRC procedures of RLM/RLF and RRC connection release can be performed at the source cell.

Observation 5: In tentative dual RRC, there may be two SRB1 for the source and the target cell simultaneously.

Observation 6: In single RRC, the release of the connection with source cell requires some coordination between the source eNB and the target eNB and the corresponding RRC message is transmitted by the target eNB.

Based on the observations, we propose:

Proposal 1: There is only one S1-C for simultaneous connectivity HO.

Proposal 2: The path switching and BEARER modify procedures in HO complete phase can be reused for simultaneous connectivity HO.

Proposal 3: Dual RRC is characterised by the following aspects:

- UE has two RRC entities for the source cell and the target cell simultaneously;

- UE keeps two SRB1 for the RRC procedures performed with the source cell and the target cell simultaneously;

- At least the RRC procedures of RLM/RLF and RRC connection release for the source cell are performed at the RRC entity corresponding to the source cell.

Proposal 4: Single RRC is supported for simultaneous connectivity HO.