3GPP TSG-RAN2 Meeting#106                                                 	R2-1907370

Reno, USA, May 13th – 17th 2019

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.331 | CR | draft | rev | - | Current version: | 15.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR for A new SIB to host posSIB scheduling information |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | n.a. (to be merged in to the running CR) |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_Pos |  |  |  |  |  | Date: |  |  | 2019-03-25 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | A SIB for hosting positioning schedulojg information has been added |  |  |  |  |  |  |  |  |
| Summary of change: |  | Section 5.2..4 has a new subsection describing the action upon reception of the new SIB. ASN.1 changes has been done in chapter 6 to define the new SIB. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | SIB1 size would be large if positioning scheduling information is added here. Further for future extenstions, it would be useful if a dedicated positiong SIB scheduling host is present. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2.2.4.X(new), 6.2.2,6.3.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This serves as a baseline but RAN2 should consider to reduce number of positioning related SIBs. |  |  |  |  |  |  |  |  |

Beginning of changes

#### 5.2.2.4	Actions upon receipt of System Information

##### 5.2.2.4.1	Actions upon reception of the MIB

<Skipped Unmodified Changes>

##### 5.2.2.4.X	Actions upon reception of the SIB10-Pos

No UE requirements related to the contents of this SIB10-Pos apply other than those specified elsewhere e.g. within procedures using the concerned system information, and/ or within the corresponding field descriptions.

Next change

### 6.2.2	Message definitions

#### –	CounterCheck

The CounterCheck message is used by the network to indicate the current COUNT MSB values associated to each DRB and to request the UE to compare these to its COUNT MSB values and to report the comparison results to the network.

Signalling radio bearer: SRB1

RLC-SAP: AM

Logical channel: DCCH

Direction: Network to UE

<Skipped Unmodified Changes>

#### –	SystemInformation

The SystemInformation message is used to convey one or more System Information Blocks. All the SIBs included are transmitted with the same periodicity.

Signalling radio bearer: N/A

RLC-SAP: TM

Logical channels: BCCH

Direction: Network to UE

SystemInformation message

-- ASN1START

-- TAG-SYSTEMINFORMATION-START

SystemInformation ::=               SEQUENCE {

criticalExtensions                  CHOICE {

systemInformation-r15               SystemInformation-IEs,

criticalExtensionsFuture            SEQUENCE {}

}

}

SystemInformation-IEs ::=           SEQUENCE {

sib-TypeAndInfo                     SEQUENCE (SIZE (1..maxSIB)) OF CHOICE {

sib2                                SIB2,

sib3                                SIB3,

sib4                                SIB4,

sib5                                SIB5,

sib6                                SIB6,

sib7                                SIB7,

sib8                                SIB8,

sib9                                SIB9,

...,

sib10-v16xy							SIB10-Pos-r16

},

lateNonCriticalExtension            OCTET STRING                        OPTIONAL,

nonCriticalExtension                SEQUENCE {}                         OPTIONAL

}

-- TAG-SYSTEMINFORMATION-STOP

-- ASN1STOP

Next change

### 6.3.1	System information blocks

#### –	SIB2

SIB2 contains cell re-selection information common for intra-frequency, inter-frequency and/or inter-RAT cell re-selection (i.e. applicable for more than one type of cell re-selection but not necessarily all) as well as intra-frequency cell re-selection information other than neighbouring cell related.

SIB2 information element

<Skipped Unmodified Changes>

#### –	SIB10-Pos

The SIB10 contains information needed for acquisition of positioning SI messages.

SIB10-Pos information element

-- ASN1START

-- TAG-OTHER-SI-INFO-START

SIB10-Pos-r16 ::=               SEQUENCE {

posSchedulingInfoList-r16   SEQUENCE (SIZE (1..maxSI-Message)) OF PosSchedulingInfo-r16

...

}

PosSchedulingInfo-r16 ::=	SEQUENCE {

posSI-Periodicity-r16		ENUMERATED {rf8, rf16, rf32, rf64, rf128, rf256, rf512},

posSIB-MappingInfo-r16		PosSIB-MappingInfo-r16

}

PosSIB-MappingInfo-r16 ::= SEQUENCE (SIZE (1..maxSIB)) OF PosSIB-Type-r16

PosSIB-Type-r16 ::= SEQUENCE {

encrypted-r16		ENUMERATED { true }				OPTIONAL,		-- Need OP

gnss-id-r16			GNSS-ID-r16						OPTIONAL,		-- Need OP

sbas-id-r16			SBAS-ID-r16						OPTIONAL,		-- Need OP

posSibType-r16		ENUMERATED { 	posSibType1-1,

posSibType1-2,

posSibType1-3,

posSibType1-4,

posSibType1-5,

posSibType1-6,

posSibType1-7,

posSibType2-1,

posSibType2-2,

posSibType2-3,

posSibType2-4,

posSibType2-5,

posSibType2-6,

posSibType2-7,

posSibType2-8,

posSibType2-9,

posSibType2-10,

posSibType2-11,

posSibType2-12,

posSibType2-13,

posSibType2-14,

posSibType2-15,

posSibType2-16,

posSibType2-17,

posSibType2-18,

posSibType2-19,

posSibType3-1,

...},

...

}

-- TAG-OTHER-SI-INFO-STOP

-- ASN1STOP

End of changes