3GPP TSG-RAN WG2 Meeting #107                                                 	R2-1908759

Prague, Czech Republic, 26th - 30th Aug 2019

Source:	CATT

Title:	Detection and Reporting of Out-of-coverage

Agenda Item:	11.12.2

Document for:	Discussion and Decision

Introduction

In RAN2#106 meeting, the issue about out-of-coverage has been discussed, and some agreements have been achieved:

Agreements:

1	The out-of-coverage detection and logging in NR can be configured to be independent of the periodical DL pilot strength logged measurements.

2	For MDT in NR, UE can be configured to only perform the logging for out-of-coverage detection, i.e., UE is not required to log the DL pilot strength measurements if the configuration is only out-of-coverage detection.

Agreement

1.	A new indicator is introduced per logged measurement information entry in order to informing of detection of any cell selection state as in Rel-15 LTE MDT.

In this contribution, we discuss the details about the recording of the out-of-coverage case.

Discussion

  - Periodical Out-of-coverage Logging

In previous LTE specification, the UE in RRC_IDLE performs logged MDT in camped normally state only i.e., UE shall be camping on serving cell in RRC_IDLE state. Considering the MDT feature is working for the network operators who want to know the specific status of coverage including out of service area, the ‘any cell selection’ state is added to the periodical stored logged MDT information in R15[1]. The logged MDT is started when UE enter the IDLE state, and when performing the logging of measurement results periodically, the UE sets this field if  there is no suitable cell or no acceptable cell found.

Observation: LTE only introduce the periodical ‘any cell selection’ state logging and the recording of out-of-coverage may not be accurate.

In NR, a new indicator of ‘any cell selection’ state is agreed to be added in the periodical stored logged MDT information similar as LTE. Since the solution of NR logged MDT is also depended on DL pilot strength measurements, the size of logged results will grow bigger and bigger. Therefore an event triggered periodical solution to log the out-of coverage independent of the periodical DL pilot strength logged measurements could be used.  To exactly identify the problem of e.g. coverage hole and minimum the logging size, the “out-of-coverage” could be a trigger to start the periodical logging.

It is mentioned in 38.133 that: “If the UE in RRC_IDLE has not found any new suitable cell based on searches and measurements using the intra-frequency, inter-frequency and inter-RAT information indicated in the system information for 10 s, the UE shall initiate cell selection procedures for the selected PLMN as defined in TS 38.304.” which means 10s is used for NR between the cell selection criteria not being met and UE entering ‘Any Cell selection’ state. Therefore for the “out-of-coverage” logging, the recording time could be defined as below:

- Logging of “Out-of-coverage” should be started if UE can not found any suitable cell or acceptable cell for 10 seconds;

- After the start, UE performs the logging of “Out-of-coverage” at regular time intervals, as defined by the e.g. loggingInterval;

- Logging of “Out-of-coverage” should be ended if UE found a suitable cell or acceptable cell again (return from out of coverage).

Here is an example in Figure1:

Figure1 UE entering and leaving coverage hole

Proposal 1: UE logs “out-of-coverage” following the principles below:

- Logging of “Out-of-coverage” should be started if UE can not found any suitable cell or acceptable cell for 10 seconds;

- After the start, UE performs the logging of “Out-of-coverage” at regular time intervals, as defined by the e.g. loggingInterval;

- Logging of “Out-of-coverage” should be ended if UE found a suitable cell or acceptable cell again (return from out of coverage).

  - Configuration of Event Triggered Periodical Out-of coverage Logging

In LTE, the “anyCellSelection” indicator is introduced by requesting of many operators [2], therefore it should be an SA5 requirement. Since it is reported in the periodical logged result in LTE, no indicator is needed in the MDT configuration. But in NR if the event triggered periodical out-of-coverage is introduced, whether to perform the periodical logging for DL pilot strength measurements or the event triggered periodical out-of-coverage needs to be indicated in the MDT configuration, no matter the MDT configuration is set for signaling based MDT or for area based MDT.

Proposal 2: Whether to enable the event triggered periodical “out-of-coverage” should be indicated by the network.

Since the Logged MDT configuration of signaling based MDT is transferred via AMF, the enable indicator of the event triggered periodical out-of-coverage should also be included.

Proposal 3: Send an LS to RAN3 for the enable indicator of the event triggered periodical “out-of-coverage”.

Since the periodical logging of out-of-coverage could use LTE as baseline, for event triggered periodical “out-of-coverage” in NR, it is suggest comparing and analyses whether the configuration of periodical logging in LTE could be reused, as in Table1:

Table1 Compare of the configuration of LTE periodical logging and NR event triggered periodical logging

| Configuration of LTE periodical logging | Configuration of NR event triggered “Out-of-coverage” logging |
| --- | --- |
| traceReference | Needed. Whether to enable this logging need to be indicated by the network, and the traceReference, the traceRecordingSessionRef and the tce-Id are necessary for a trace procedure. |
| traceRecordingSessionRef |  |
| tce-Id |  |
| absoluteTimeInfo | Needed. Each trace procedure need an absolute time. |
| areaConfiguration | Could be configured. If absent, the whole registered PLMN could be considered. |
| loggingDuration | Needed. Each trace procedure need a total time, but this could be configured more than 120mins. |
| loggingInterval | Needed. After the logging start, UE should perform logging at regular time intervals. |
| plmn-IdentityList | Could be configured. UE performs these actions when the RPLMN is part of this set of PLMNs. |
| targetMBSFN-AreaList | N/A, no MBSFN in NR |
| bt-NameList | N/A, do not consider in out-of coverage logging. |
| wlan-NameList | N/A, do not consider in out-of coverage logging. |

According to the analyses in the table, for event triggered periodical logging of out-of-coverage in NR, the configured parameters include trace and TCE related parameters, the absolute time, the logging Duration, the logging Interval, the area configuration and the PLMN ID list.

Proposal 4: The configuration parameters of “out-of-coverage” may include:

- The event triggered periodical out-of-coverage logging indicator;

- The trace and TCE related parameters of “traceReference”, “traceRecordingSessionRef” and “tce-Id”;

- absoluteTimeInfo;

- areaConfiguration;

- loggingDuration;

- loggingInterval;

- plmn-IdentityList.

  - Recording and Reporting of Event Triggered Periodical Out-of coverage Logging

Since the event triggered periodical logging of out-of-coverage could use LTE as baseline, in NR it is suggest comparing and analyses whether the periodical logging parameters in LTE could be reused, as in Table2:

Table2 Compare of LTE periodical logging and NR event triggered periodical logging

| LTE periodical logging | NR event triggered “Out-of-coverage” logging |
| --- | --- |
| absoluteTimeStamp | Needed. The relative time is based on the absolute time. |
| traceReference | Needed. Network could identify the relationship between the trace procedure and the reported results based on the traceReference, the traceRecordingSessionRef and the tce-Id. |
| traceRecordingSessionRef |  |
| tce-Id |  |
| locationInfo | Could be recorded, if any. It is an important parameter to indicate where the coverage hole place. |
| relativeTimeStamp | Needed. |
| servCellIdentity | Cannot be logged since UE cannot find a suitable cell or an acceptable cell to camp on. But UE could log the last cell just before UE enter “any cell selection” state together with the last measurement result; and the UE could log the cell ID just after UE successfully find a suitable/acceptable cell together with the related measurement result. |
| measResultServCell |  |
| measResultNeighCells | Not needed. It is only a “out-of-coverage” logging. |
| inDeviceCoexDetected | N/A, currently there is no IDC in NR. |
| logMeasResultListBT | N/A, do not consider in out-of coverage logging. |
| logMeasResultListWLAN | N/A, do not consider in out-of coverage logging. |

According to the analyses in the table, for event triggered periodical logging of out-of-coverage in NR, the location information, timestamp, the cell ID and measurement of last/current serving cell could be logged. Since the last/current serving cell only occur before/after a set of out-of-coverage logging, this field could be marked as optional and UE could record the cell related results if needed.

Proposal 5: Logged results of event triggered “out-of-coverage” may include:

- Absolute Time stamp;

- The trace and TCE related parameters of “traceReference”, “traceRecordingSessionRef” and “tce-Id”;

For each entry:

- Location information;

- Relative Time stamp;

- Cell ID and Measurement result of last/current serving cell, if needed.

Since the event triggered periodical logging of out-of-coverage may have separated configuration from the network, the reporting should also be separated from the logging of DL pilot strength measurements. A single available indicator could be introduced for the evented triggered periodical “out-of-coverage”.

Proposal 6: A single available indicator could be introduced for the evented triggered periodical “out-of-coverage”.

Conclusion

According to the analysis in section 2, we have:

Observation: LTE only introduce the periodical ‘any cell selection’ state logging and the recording of out-of-coverage may not be accurate.

And we propose:

Proposal 1: UE logs “out-of-coverage” following the principles below:

- Logging of “Out-of-coverage” should be started if UE can not found any suitable cell or acceptable cell for 10 seconds;

- After the start, UE performs the logging of “Out-of-coverage” at regular time intervals, as defined by the e.g. loggingInterval;

- Logging of “Out-of-coverage” should be ended if UE found a suitable cell or acceptable cell again (return from out of coverage).

Proposal 2: Whether to enable the event triggered periodical “out-of-coverage” should be indicated by the network.

Proposal 3: Send an LS to RAN3 for the enable indicator of the event triggered periodical “out-of-coverage”.

A draft LS is provide in [3].

Proposal 4: The configuration parameters of “out-of-coverage” may include:

- The event triggered periodical out-of-coverage logging indicator;

- The trace and TCE related parameters of “traceReference”, “traceRecordingSessionRef” and “tce-Id”;

- absoluteTimeInfo;

- areaConfiguration;

- loggingDuration;

- loggingInterval;

- plmn-IdentityList.

Proposal 5: Logged results of event triggered “out-of-coverage” may include:

- Absolute Time stamp;

- The trace and TCE related parameters of “traceReference”, “traceRecordingSessionRef” and “tce-Id”;

For each entry:

- Location information;

- Relative Time stamp;

- Cell ID and Measurement result of last/current serving cell, if needed.

Proposal 6: A single available indicator could be introduced for the evented triggered periodical “out-of-coverage”.

Reference

- R2-1808802 Support for Logging of 'Any cell selection' state, LG Electronics

- R2-105208 Summary of Offline Discussion MDT Logging in OOS Vodafone, Deutsche Telekom, CMCC, NTT DoCoMO,Telecom Italia,Nokia, ZTE, LG, Motorola, NEC, Kyocera, ET-Ericsson, Ericsson, Teliasonera

- R2-1908760 [Draft] LS on MDT Configuration of Event triggered Out-of-coverage, CATT

TP for 37.320

This TP is based on TS37.320-f00.

5.1.1.1.1	Configuration parameters

The logged measurement configuration consists of:

-	configuration of downlink pilot strength measurements logging.

-	configuration of MBSFN measurement logging for E-UTRA.

-	configuration of the triggering of logging events. Periodic measurement trigger is supported for UTRA, E-UTRA and NR, and event triggered periodic measurement of “Out-of-coverage” is only supported for NR.

-    configuration of the logging interval. The parameter specifies the periodicity for storing MDT measurement results. It should be configured in seconds in multiples of the applied IDLE mode DRX, i.e. multiples of 1.28s which is either a factor or multiple of the IDLE mode DRX. The UE behaviour is unspecified when the UE is configured with a DRX cycle larger than the logging interval.

-	configuration of the logging duration. This configuration parameter defines a timer activated at the moment of configuration, that continues independent of state changes, RAT or RPLMN change. When the timer expires the logging is stopped and the configuration is cleared (except for the parameters that are required for further reporting e.g. network absolute time stamp, trace reference, trace recording session reference and TCE Id).

-	network absolute time stamp to be used as a time reference to UE

-	Trace Reference parameter as indicated by the OAM configuration as specified in TS 32.422 [6]

-	Trace Recording Session Reference as indicated by the OAM configuration as specified in TS 32.422 [6]

-	TCE Id as indicated by the OAM configuration as specified in TS 32.422 [6]

-	MDT PLMN List, indicating the PLMNs where measurement collection and log reporting is allowed. It is either the Management Based MDT PLMN List or the Signalling Based MDT PLMN List, depending on how the Logged MDT task was initiated (see 5.1.3).

-	(optionally) configuration of a logging area. A UE will log measurements as long as it is within the configured logging area. The scope of the logging area may consist of one of:

-	a list of up to 32 global cell identities. If this list is configured, the UE will only log measurements when camping in any of these cells

-	a list of up to 8 TAs or 8 LAs or 8 RAs. If this list is configured, the UE will only log measurements when camping in any cell belonging to the preconfigured TA/LA/RAs.

-	The configured logging area can span PLMNs in the MDT PLMN List. If no area is configured, the UE will log measurements throughout the PLMNs of the MDT PLMN list.

-	(optionally) configuration of target MBSFN area(s) for MBSFN measurement logging. If target MBSFN area(s) is configured, UE applies it in addition to other restrictions such as the logging area. The UE will log measurements as long as it receives MBMS service from an indicated target MBSFN area and is within the configured logging area. The target MBSFN area(s) is defined by a list of up to 8 entries, where each entry indicates a carrier frequency and optionally indicates a specific MBSFN area on a carrier frequency.

-	(optionally) configuration of the WLAN measurements, indicating the UE to attempt to obtain WLAN measurements.

-	(optionally) configuration of the Bluetooth measurements, indicating the UE to attempt to obtain Bluetooth measurements.

5.1.1.1.2	Configuration effectiveness

The logged measurement configuration is provided in a cell by dedicated control while UE is in CONNECTED and implies:

-	logged measurement configuration for downlink pilot strength measurements logging is active

-	in IDLE UE state in E-UTRAN, or

-	in IDLE mode, CELL_PCH and URA_PCH states in UTRAN, or

-	in CELL_FACH state when second DRX cycle is used in UTRAN

-	until logging duration timer expires or stops

-	logged measurement configuration for MBSFN measurement logging is active

-	in IDLE and CONNECTED UE states in E-UTRAN

-	until logging duration timer expires or stops

-	logged measurement configuration for event triggered periodic measurement of “Out-of-coverage” logging is active

-	in IDLE and INACTIVE UE states in NR

-	if UE cannot find any suitable cell or acceptable cell for 10 seconds

-	until logging duration timer expires or stops, or until UE finds a suitable cell or acceptable cell again (return from out of coverage)

-	logged measurement configuration and logs are maintained when the UE is in any state as described above, despite multiple periods interrupted by UE state transitions, e.g. for downlink pilot strength measurements when the UE is in CONNECTED state for E-UTRAN and CELL_DCH, CELL_FACH state when second DRX cycle is not used in UTRAN

-	logged measurement configuration and logs are maintained when the UE is in any state as described above in that RAT, despite multiple periods interrupted by UE presence in another RAT

There is only one RAT-specific logged measurement configuration for Logged MDT in the UE. When the network provides a configuration, any previously configured logged measurement configuration will be entirely replaced by the new one. Moreover, logged measurements corresponding to the previous configuration will be cleared at the same time. It is left up to the network to retrieve any relevant data before providing a new configuration.

NOTE:	The network may have to do inter-RAT coordination.

When a logging area is configured, logged MDT measurements are performed as long as the UE is within this logging area. If no logging area is configured, logged MDT measurements are performed as long as the RPLMN is part of the MDT PLMN list. When the UE is not in the logging area or RPLMN is not part of the MDT PLMN list, the logging is suspended, i.e. the logged measurement configuration and the log are kept but measurement results are not logged. In addition, for MBSFN logged measurements, logged MDT measurements are performed in logging intervals when the UE receives MBMS service from a MBSFN area according to the target MBSFN area(s) configuration When the UE is not in the logging area or does not receive MBMS service from a MBSFN area that matches the target MBSFN area(s) configuration in the logging interval the logged measurement configuration and the log are kept but measurement results are not logged.

NOTE:	The logging duration timer continues.

In case the new PLMN that does not belong to the MDT PLMN list provides a logged measurement configuration any previously configured logged measurement configuration and corresponding log are cleared and overwritten without being retrieved.

5.1.1.2	Measurement collection

In "camp normally" state, a UE shall perform logging as per the logged measurement configuration. This state includes a period between cell selection criteria not being met and UE entering "any cell selection" state, i.e. 10 s for E-UTRA (See TS 36.133 [3]) or 12 s for UTRA (See TS 25.133 [2]).

In "any cell selection" state, a UE shall perform logging of available information (i.e. at least indicator 'anyCellSelectionDetected' for periodic measurement, or “out-of-coverage” for event triggered periodic measurement if configured, and time stamp). In "camped on any cell" state, the periodic logging stops. However, it should be noted that the duration timer is kept running. When the UE re-enters "camped normally" state and the duration timer has not expired, the periodic logging is restarted based on new DRX and logging resumes automatically (with a leap in time stamp).

When an E-UTRA UE detects an in-device coexistence problem that may affect the logged measurement results, the UE shall stop measurement logging, indicate in the log that an in-device coexistence problem has occurred, and keep the duration timer running. When the in-device coexistence problem is no longer present, and the duration timer has not expired, the logging resumes, with a leap in time stamp.

For E-UTRA MBSFN measurement logging, the UE shall perform MBSFN measurements only when receiving MBMS service, and measurement logging is performed only for logging intervals for which MBSFN measurements are available. The UE shall perform MBSFN measurements and MBSFN measurement logging in both IDLE and CONNECTED modes.

NOTE:	the UE is only required to perform MBSFN measurements when receiving MBMS service of the MBSFN area(s) targeted for logging.

For WLAN measurement logging and Bluetooth measurement logging, the UE shall perform WLAN and Bluetooth measurements, respectively, only when indicated in the corresponding configuration. The measurement logging is performed only for logging intervals for which WLAN and Bluetooth measurements are available, respectively.

The measurement quantities for downlink pilot strength measurement logging are fixed and consist of both RSRP and RSRQ for EUTRA, both RSCP and Ec/No for UTRA FDD, P-CCPCH RSCP for UTRA 1.28 Mcps TDD, Rxlev for GERAN, and Pilot Pn Phase and Pilot Strength for CDMA2000 if the serving cell is EUTRAN cell.

The measurement quantities for E-UTRA MBSFN measurement logging are fixed and consist of MBSFN RSRP, MBSFN RSRQ, BLER for signalling and BLER for data per MCH, in addition to the measurement quantities for downlink pilot strength measurements.

The measurement quantities for WLAN measurement logging are fixed and consist of BSSID, SSID, HESSID of WLAN APs. If configured by the network, optionally available RSSI and RTT can be included.

The measurement quantity for Bluetooth measurement logging is fixed and consists of MAC address of Bluetooth beacons. If configured by the network, optionally available RSSI can be included.

UE collects MDT measurements and continues logging according to the logged measurement configuration until UE memory reserved for MDT is full. In this case the UE stops logging, stops the log duration timer and starts the 48 hour timer.

5.1.1.3	Measurement reporting

5.1.1.3.1	Availability Indicator

A UE configured to perform Logged MDT downlink pilot strength measurements indicates the availability of Logged MDT measurements, by means of an indicator, in RRCConnectionSetupComplete message during connection establishment. Furthermore, the indicator (possibly updated) shall be provided within E-UTRAN handover and re-establishment, and UTRAN procedures involving the change of SRNC (SRNC relocation), CELL UPDATE, URA UPDATE messages as well as MEASUREMENT REPORT message in case of state transition to CELL_FACH without CELL UPDATE. The UE includes the indication in one of these messages at every transition to RRC Connected mode even though the logging period has not ended, upon connection to RAT which configured the UE to perform Logged MDT measurements and RPLMN which is equal to a PLMN in the MDT PLMN list.

A E-UTRA UE configured to perform Logged MDT MBSFN measurements indicates the availability of Logged MDT MBSFN measurements, by means of an indicator, in RRCConnectionSetupComplete message during connection establishment. The indicator (possibly updated) shall be provided within E-UTRAN also at handover and re-establishment, except when the logged measurement configuration is active in CONNECTED mode, i.e. except when the logging campaign is still ongoing.

A E-UTRA UE configured to perform Logged MDT WLAN measurements indicates the availability of Logged MDT WLAN measurements, by means of an indicator, in RRCConnectionSetupComplete message or RRCConnectionResumeComplete message during connection establishment. Furthermore, the indicator can be included in some uplink RRC messages, i.e., RRCConnectionReconfigurationComplete message, RRCConnectionReestablishmentComplete message, or UEInformationResponse message, at every transition to RRC Connected mode even though the logging period has not ended.

A E-UTRA UE configured to perform Logged MDT Bluetooth measurements indicates the availability of Logged MDT Bluetooth measurements, by means of an indicator, in RRCConnectionSetupComplete message or RRCConnectionResumeComplete message during connection establishment. Furthermore, the indicator can be included in some uplink RRC messages, i.e., RRCConnectionReconfigurationComplete message, RRCConnectionReestablishmentComplete message, or UEInformationResponse message, at every transition to RRC Connected mode even though the logging period has not ended.

An NR UE configured to perform event triggered periodic out-of-coverage measurements logging indicates the availability of Logged out-of-coverage measurements, by means of an indicator, in RRCSetupComplete message during connection establishment or RRCResumeComplete during connection resumption. Furthermore, the indicator shall be provided within NR handover and re-establishment. The UE includes the indication in one of these messages at every transition to RRC Connected mode even though the logging period has not ended, upon connection to RAT which configured the UE to perform Logged out-of-coverage measurements and RPLMN which is equal to a PLMN in the MDT PLMN list.

An indicator shall be also provided in UEInformationResponse message during MDT report retrieval in case the UE has not transferred the total log in one RRC message in order to indicate the remaining data availability.

The UE will not indicate the availability of MDT measurements in another RAT or in a PLMN that is not in the MDT PLMN list.

The network may decide to retrieve the logged measurements based on this indication. In case Logged MDT measurements are retrieved before the completion of the pre-defined logging duration, the reported measurement results are deleted, but MDT measurement logging will continue according to ongoing logged measurement configuration.

In case the network does not retrieve Logged MDT measurements, UE should store non-retrieved measurements for 48 hours from the moment the duration timer for logging expired. There is no requirement to store non-retrieved data beyond 48 hours. In addition, all logged measurement configuration and the log shall be removed by the UE at switch off or detach.

5.1.1.3.2	Report retrieval

For Logged MDT the measurement reporting is triggered by an on-demand mechanism, i.e. the UE is asked by the network to send the collected measurement logs via RRC signalling. UE Information procedure defined in TS 25.331 [4] and TS 36.331 [5] is used to request UE to send the collected measurement logs. The reporting may occur in different cells than which the logged measurement configuration is signalled.

Transport of Logged MDT reports in multiple RRC messages is supported. With every request, the network may receive a part of the total UE log. To indicate the reported data is a segment, the UE shall include data availability indicator as specified in 5.1.1.3.1. In multiple RRC transmissions for segmented Logged MDT reporting, FIFO order is followed, i.e. the UE should provide oldest available measurement entries in earliest message. There is no requirement specified on the size of particular reporting parts. However, each reported part should be "self-decodable", i.e. interpretable even in case all the other parts are not available.

The UE shall send an empty report when retrieval is attempted and the RPLMN is not in the MDT PLMN list.

5.1.1.3.3	Reporting parameters

For downlink pilot strength measurements, the logged measurement report consists of measurement results for the serving cell (the measurement quantity), available UE measurements performed in idle for intra-frequency/inter-frequency/inter-RAT, time stamp and location information.

For E-UTRA MBSFN measurements logging, the logged measurement report consists of MBSFN measurement results from target MBSFN area(s), if configured, and available downlink pilot strength measurement results. Inter-RAT downlink pilot strength measurements are not required to be logged.

For WLAN and Bluetooth measurement logging, the logged measurement reports consist of WLAN and Bluetooth measurement results, respectively.

For NR event triggered periodic out-of-coverage measurements logging, the logged measurement report consists of out-of-coverage measurement results.

The number of neighbouring cells to be logged is limited by a fixed upper limit per frequency for each category below. The UE should log the measurement results for the neighbouring cells, if available, up to:

-	6 for intra-frequency neighbouring cells;

-	3 for inter-frequency neighbouring cells;

-	3 for GERAN neighbouring cells;

-	3 for UTRAN (if non-serving) neighbouring cells;

-	3 for E-UTRAN (if non-serving) neighbouring cells;

-	3 for CDMA2000 (if serving is E-UTRA) neighbouring cells;

-	32 for WLAN APs;

-	32 for Bluetooth Beacons.

The measurement reports for neighbour cells consist of:

-	Physical cell identity of the logged cell;

-	Carrier frequency;

-	RSRP and RSRQ for EUTRA;

-	RSCP and Ec/No for UTRA FDD,

-	P-CCPCH RSCP for UTRA 1.28 Mcps TDD;

-	Rxlev for GERAN;

-	Pilot Pn Phase and Pilot Strength for CDMA2000;

-	RSSI and RTT for WLAN APs;

-	RSSI for Bluetooth Beacons.

For any logged cell (serving or neighbour), latest available measurement result made for cell reselection purposes is included in the log only if it has not already been reported.

While logging neighbour cells measurements, the UE shall determine a fixed number of best cells based on the measurement quantity used for ranking during cell reselection per frequency or RAT.

The MBSFN measurement results consist of, per MBSFN area where MBMS service is received:

-	MBSFN area identity;

-	Carrier frequency;

-	MBSFN RSRP;

-	MBSFN RSRQ;

-	MCH BLER for signalling;

-	MCH BLER for data, and related MCH index.

The WLAN measurement results consist of, per wireless network served by the WLAN AP:

-	BSSID, SSID and HESSID;

-	RSSI for WLAN;

-	RTT.

The Bluetooth measurement results consist of, per wireless network served by the Bluetooth beacon:

-	MAC address;

-	RSSI for Bluetooth.

The event triggered periodic out-of-coverage measurement results consist of, per out-of-coverage entry:

-	Location information;

-	Relative Time stamp;

-	Cell ID and Measurement result of last/current serving cell, if needed.

Measurements are performed in accordance with requirements defined in TS 25.133 [2] and TS 36.133 [3].

The measurement report is self contained, i.e. the RAN node is able to interpret the Logged MDT reporting results even if it does not have access to the logged measurement configuration. Each measurement report also contains the necessary parameters for the network to be able to route the reports to the correct TCE and for OAM to identify what is reported. The parameters are sent to the UE in the logged configuration message, see subclause 5.1.1.1.1.

For each MDT measurement the UE includes a relative time stamp. The base unit for time information in the Logged MDT reports is the second. In the log, the time stamp indicates the point in time when periodic logging timer expires. The time stamp is counted in seconds from the moment the logged measurement configuration is received at the UE, relative to the absolute time stamp received within the configuration. The absolute time stamp is the current network time at the point when Logged MDT is configured to the UE. The UE echoes back this absolute reference time. The time format for Logged MDT report is: YY-MM-DD HH:MM:SS.

Location information is based on available location information in the UE. Thus, the Logged MDT measurements are tagged by the UE with location data in the following manner:

-	ECGI or Cell-Id of the serving cell when the measurement was taken is always included in E-UTRAN or UTRAN respectively

-	Detailed location information (e.g. GNSS location information) is included if available in the UE when the measurement was taken. If detailed location information is available the reporting shall consist of latitude and longitude. Depending on availability, altitude, uncertainty and confidence may be also additionally included. UE tags available detailed location information only once with upcoming measurement sample, and then the detailed location information is discarded, i.e. the validity of detailed location information is implicitly assumed to be one logging interval.

NOTE:	The neighbour cell measurement information that is provided by the UE may be used to determine the UE location (RF fingerprint).

Depending on location information, measurement log/report consists of:

-	time information, RF measurements, RF fingerprints, or

-	time information, RF measurements, detailed location information (e.g. GNSS location information)