3GPP TSG RAN WG2 #107	 R2-1909324

Prague, Czech Republic, 26 – 30 August, 2019                                  (Revision of R2-1906200)

Agenda Item:	12.3.3.1 Execution Details of CHO

Source: 	Ericsson

Title:  	Suspend while monitoring CHO in LTE

Document for:	Discussion and Decision

# Introduction

In the work item for Further Enhanced Mobility Enhancements, one objective is to improve the robustness at handover. In RAN2#105bis it has been agreed to support Conditional handover (CHO) in LTE.

In this contribution we discuss some further stage-2 principles for Conditional Handover (CHO) in E-UTRAN.

# Discussion

## Conditional Handover

In conditional handover the network configures the UE with triggering conditions for when a handover should be executed. When the conditions are fulfilled, the UE executes the handover without any further order from the network. The advantage of the procedure is that the HO Command may be provided to the UE at an earlier stage before the radio conditions have become poor, which increases the chance of a successful transmission of the message. The basic signalling flow for conditional handover is shown in Figure 1.

Figure 1: Conditional handover

CHO Configuration to the UE

In legacy handovers in LTE, the source receives the configuration prepared by target and provides to the UE in an RRCConnectionReconfiguration message containing a mobilityControlInfo field containing instructions for the UE to access the target cell e.g. RACH parameters, cell identity, frequency, etc. It has been agreed in RAN2#105bis in Xi’an that for CHO, similar information needs to be provided to the UE for a target candidate. In addition, In RAN2#106 in Reno it was agreed that A3/A5 like triggering conditions will be set for the UE, so that the provided configuration is only applied if/when the condition is fulfilled.

CHO configuration consists of an RRC Reconfiguration (with reconfiguration With Sync) and associated trigger condition configuration (A3/A5 event) for a target cell candidate.

Then, from a stage-2 perspective, upon receiving the HO triggering condition(s) and dedicated RRC configuration(s), the UE starts to perform the monitoring of the trigger conditions (which involves the UE performing measurements) and, when at least one condition is fulfilled the UE performs a CHO handover execution.

Upon receiving CHO configuration(s) the UE monitors the configured CHO trigger conditions for one or multiple cells.

Source decides to suspend UE while UE monitors CHO conditions

In RAN2#105, the following has also been agreed for CHO in NR:

Agreements

. . .

e/ The baseline operation for Conditional HO assumes the source RAN remains responsible for RRC until UE successfully sends RRC Reconfiguration Complete message to target RAN.

. . .

It may happen that while the UE is monitoring CHO trigger conditions, the source eNB decides to suspend the UE i.e. move the UE to IDLE with stored AS context (or to INACTIVE state, if the UE is connected to 5GC). That may be done by the reception of an RRCRelease message with a suspend indication (or suspend configuration).

Hence, as the UE transitions to IDLE (with stored context) or INACTIVE (in case of 5GC), upon the reception of an RRCConnectionRelease message (with a suspend indication) while the UE is monitoring CHO trigger conditions, the UE stops the actions related to CHO monitoring, such as stopping performing associated measurements.

UE stops the actions related to CHO monitoring upon transitioning to IDLE with stored context.

One issue is what should be done with the CHO configurations upon the reception of an RRCConnectionRelease. That needs to be handled to avoid any sort of state mismatch e.g. if the network stores the CHO configurations as part of the UE context, while the UE deletes it (or vice versa).

A first alternative is clearly specifying in RRC that the UE suspends the actions associated to the monitoring of the CHO trigger conditions (e.g. suspend measurements) and stores CHO related configurations (e.g. in the AS context) upon transitioning to IDLE (with stored context or to INACTIVE). The advantage here is that these CHO configurations may be resumed in the RRC resume procedure and, possibly relying on delta signalling.

A second alternative is to stop the actions associated to the monitoring of the CHO trigger conditions (e.g. suspend measurements) and release the CHO related configurations. In that case the network could still configure the UE with CHO configurations in RRC Resume procedure (e.g. with CHO configurations within the RRCConnectionResume message or multiplexing an RRCConnectionConditionalResconfiguration with the RRCResume message).

Network may configure CHO configurations upon resume procedure.

Discuss the alternatives for the transition to IDLE (with stored context) while the UE monitors CHO conditions:

    - UE stores the CHO configurations (so they can be resume);

    - UE deletes the CHO configurations;

Source decides to release UE while UE monitors CHO conditions

In this case, releasing means moving the UE to IDLE without storing an AS context at the UE and at the network sides. As in the case of moving to IDLE with stored context, the UE also stops monitoring the CHO trigger conditions and perform clean up actions (e.g. stop timers, release measurements, etc.). And, since the network has no means to restore the context anyway, the first alternative of deleting the CHO configurations seems to be the most reasonable.

UE stops the actions related to CHO monitoring upon transitioning to IDLE without stored context.

For the transition to IDLE (without stored context) while the UE monitors CHO conditions, the UE deletes the CHO configurations.

In the same way we have discussed the possibility to provide a CHO configurations in RRCConnectionResume, we could also discuss the possibility to provide CHO configurations in RRCConnectionSetup. One difference that could be a potential issue is the security requirements when configuring CHO. In our view, as CHO configurations contain a dedicated RRC configuration per target candidate i.e. an RRCConnectionReconfiguration with mobilityControlInfo, and that requires security to be activated, in the CHO case that shall also require security to be activated. In other words, providing CHO in RRCConnectionResume seems to violate existing security requirements.

Providing CHO configuration(s) in RRCConnectionSetup seems to violate existing security requirements.

CHO configurations shall not be included in RRCConnectionSetup.

# Summary

Based on the discussions above, the following is proposed:

- UE stops the actions related to CHO monitoring upon transitioning to IDLE with stored context.

- Network may configure CHO configurations upon resume procedure.

- Discuss the alternatives for the transition to IDLE (with stored context) while the UE monitors CHO conditions:

    - UE stores the CHO configurations (so they can be resume);

    - UE deletes the CHO configurations;

- UE stops the actions related to CHO monitoring upon transitioning to IDLE without stored context.

- For the transition to IDLE (without stored context) while the UE monitors CHO conditions, the UE deletes the CHO configurations.

- CHO configurations shall not be included in RRCConnectionSetup.

# References

- RP-181433, Work Item on NR Mobility Enhancements, Intel, RAN#80, La Jolla, USA, May 21 – May 25 2018 September 2018.

- R2-1909321, Triggering of conditional handover in LTE, Ericsson

- R2-1909320, Configuration of conditional handover in LTE, Ericsson

- R2-1909322, Conditonal handover execution in LTE, Ericsson