3GPP TSG-RAN2 107                                                                                          R2-1909793

Prague, Czech Republic, 26th – 30th August 2019

Title:	[Draft] LS to clarify DRX configuration in ANR

Response to:

Release:	Release 15

Work Item:	NR_newRAT-Core

Source:	RAN WG2

To:	RAN WG4

Cc:

Contact Person:

Name: ZTE

E-mail Address:	liu dot jing30 (at) ZTE dot com dot cn

Attachments:

1. Overall Description:

RAN2 has discussed the DRX configuration for ANR reportCGI measurement of MR-DC UEs, to make sure the success of CGI reading and avoid data transmission impact as much as possible, RAN2 understands it is possible to reuse gap capability (i.e. per-UE gap, per-FR gap) to determine the responsible node (e.g. MN or SN) for providing the idle period (e.g. long DRX cycle) to UE to do CGI reading.

Take EN-DC as an example, no matter the reportCGI measurement is configured by MN or SN, the expected DRX configuration for reportCGI measurement can follow the guidance given in following table.

|  | Serving cells (EN-DC) | Target cell of reportCGI | Expected DRX configuration |  |
| --- | --- | --- | --- | --- |
|  |  |  | UE supports per-UE gap | UE supports per-FR gap |
| 1 | LTE + NR FR1 | NR FR1 | C | C |
| 2 | LTE + NR FR1 | NR FR2 | C | D |
| 3 | LTE + NR FR2 | NR FR1 | C | A |
| 4 | LTE + NR FR2 | NR FR2 | C | B |
| 5 | LTE + NR FR1&FR2 | NR FR1 | C | C |
| 6 | LTE + NR FR1&FR2 | NR FR2 | C | B |
| 7 | LTE + NR FR1 | 2G/3G/4G | C | C |
| 8 | LTE + NR FR2 | 2G/3G/4G | C | A |
| 9 | LTE + NR FR1&FR2 | 2G/3G/4G | C | C |
| NOTE: A: Only MN needs to provide long DRX cycle; B: Only SN needs to provide long DRX cycle; C: Both MN and SN need to provide long DRX cycle, and UE is suppose to read target CGI during the common idle period; D: None of MN or SN needs to provide long DRX cycle. |  |  |  |  |

Note that above table is produced based on Table 9.1.2-2 defined in TS38.133.

Given the above, RAN2 would like to know that for MR-DC UEs, whether UE’s gap capability (i.e. per-UE gap, per-FR gap) can be reused in DRX configuration judgement in reportCGI measurement?

2. Actions:

To RAN4 group.

ACTION: 	RAN2 respectfully asks RAN4 to provide feedback on the following questions:

- Whether UE’s gap capability (i.e. per-UE gap, per-FR gap) can be reused in DRX configuration judgement in reportCGI measurement for MR-DC UEs? E.g. Follow the principle defined in table 9.1.2-2 and table 9.1.2-3 in TS38.133.

3. Date of Next TSG-RAN2 Meetings:

3GPP RAN2#107bis		14 – 18 October 2019	Chongqing, China

3GPP RAN2#108                            18 – 22 November 2019	Reno, USA