3GPP TSG-RAN2 Meeting #107	R2-1911025

# Prague, Czech Republic, 26th – 30th August 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.306 | CR | 0159 | rev | - | Current version: | 15.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR on BWCS for inter-ENDC BC with intra-ENDC BC |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2019-08-26 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | RAN4 has confirmed that there is no separate BWCS for inter-band EN-DC, the BWCS of LTE and BWCS of NR are used. And there is BWCS defined for intra-band EN-DC. However, for an inter-band EN-DC combination which consists of an intra-band EN-DC combination, it is not clear which BWCS value should be used. For example, an inter-band EN-DC with intra-band EN-DC part: DC_2-66-(n)71, for one of the understanding, the inter-band EN-DC is divided into LTE part and NR part, BWCS of LTE part and BWCS of NR part are used (e.g. BWCS of CA_2-66-71 + BWCS of n71). However, according to the TR 37.863-02-01, it is not the RAN4 intention. The correct understanding would be that the inter-band EN-DC is divided into intra-band EN-DC part and residual LTE part, BWCS of intra-band EN-DC and BWCS of LTE are used (e.g. BWCS of CA_2-66 + BWCS of DC_(n)71). To determine the BWCS for an inter-band EN-DC with intra-band EN-DC part, BWCS of intra-band EN-DC should be identified. However, in some cases, the intra-band EN-DC BC won’t be reported since it is a fallback BC. To avoid this, the supportedBandwidthCombinationSet should be mandatory for an inter-band EN-DC with intra-band EN-DC part and it is used to indicate the supported bandwidth combination for the intra-band EN-DC part. |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1. Add the field description to clarifiy the supportedBandwidthCombinationSet is mandatory if the BC is an inter-band EN-DC combination with an intra-band EN-DC combination. 2. Add the field description to clarifiy the BWCS of intra-band EN-DC and BWCS of LTE are used for an inter-band EN-DC combination with an intra-band EN-DC combination. Impact Analysis Impacted 5G architecture options: EN-DC, NGEN-DC, NE-DC Impacted functionality: UE radio capability Inter-operability: 1. If the network is implemented according to the CR and the UE is not, there may be inter-operability problem, the understanding of UE and network would be misaligned. 2. If the UE is implemented according to the CR and the network is not, there may be inter-operability problem, the understanding of UE and network would be misaligned. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The understanding of UE and network would be misaligned, which may lead to configuration failure. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.2.7.1, 4.2.7.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

---------------------------------------------START OF CHANGE-------------------------------------------

4.2.7.1	BandCombinationList parameters

| Definitions for parameters | Per | M | FDD-TDD DIFF | FR1-FR2 DIFF |
| --- | --- | --- | --- | --- |
| bandEUTRA Defines supported EUTRA frequency band by NR frequency band number, as specified in TS 36.101. | Band | Yes | No | No |
| bandList Each entry of the list should include at least one bandwidth class for UL or DL. | BC | Yes | No | No |
| bandNR Defines supported NR frequency band by NR frequency band number, as specified in TS 38.101-1 [2] and TS 38.101-2 [3]. | Band | Yes | No | No |
| ca-BandwidthClassDL-EUTRA Defines for DL, the class defined by the aggregated transmission bandwidth configuration and maximum number of component carriers supported by the UE, as specified in TS 36.101. When all FeatureSetEUTRA-DownlinkId:s in the corresponding FeatureSetsPerBand are zero, this field is absent. | Band | No | No | No |
| ca-BandwidthClassDL-NR Defines for DL, the class defined by the aggregated transmission bandwidth configuration and maximum number of component carriers supported by the UE, as specified in TS 38.101-1 [2] and TS 38.101-2 [3]. When all FeatureSetDownlinkId:s in the corresponding FeatureSetsPerBand are zero, this field is absent. | Band | No | No | No |
| ca-BandwidthClassUL-EUTRA Defines for UL, the class defined by the aggregated transmission bandwidth configuration and maximum number of component carriers supported by the UE, as specified in TS 36.101. When all FeatureSetEUTRA-UplinkId:s in the corresponding FeatureSetsPerBand are zero, this field is absent. | Band | No | No | No |
| ca-BandwidthClassUL-NR Defines for UL, the class defined by the aggregated transmission bandwidth configuration and maximum number of component carriers supported by the UE, as specified in TS 38.101-1 [2] and TS 38.101-2 [3]. When all FeatureSetUplinkId:s in the corresponding FeatureSetsPerBand are zero, this field is absent. | Band | No | No | No |
| ca-ParametersEUTRA Contains the EUTRA part of band combination parameters for a given EN-DC band combination. | BC | No | No | No |
| ca-ParametersNR Contains the NR band combination parameters for a given EN-DC and/or NR CA band combination. | BC | No | No | No |
| featureSetCombination Indicates the feature set that the UE supports on the NR and/or MR-DC band combination by FeatureSetCombinationId. | BC | N/A | No | No |
| mrdc-Parameters Contains the band combination parameters for a given EN-DC band combination. | BC | No | No | No |
| ne-DC-BC Indicates whether the UE supports NE-DC for the band combination. | BC | Tbd | No | No |
| nr-DC-BC Indicates whether the UE supports NR-DC for the band combination. | BC | Tbd | No | No |
| srs-SwitchingTimeNR Indicates the interruption time on DL/UL reception within a NR band pair during the RF retuning for switching between a carrier on one band and another (PUSCH-less) carrier on the other band to transmit SRS. switchingTimeDL/ switchingTimeUL : n0 represents 0 us, n30us represents 30us, and so on. switchingTimeDL/ switchingTimeDL is mandatory present if switching between the NR band pair is supported, otherwise the field is absent. It is signalled per pair of bands per band combination. | FD | No | No | No |
| srs-SwitchingTimeEUTRA indicates the interruption time on DL/UL reception within a EUTRA band pair during the RF retuning for switching between a carrier on one band and another (PUSCH-less) carrier on the other band to transmit SRS. switchingTimeDL/ switchingTimeUL: n0 represents 0 OFDM symbols, n0dot5 represents 0.5 OFDM symbols, n1 represents 1 OFDM symbol and so on. switchingTimeDL/ switchingTimeUL is mandatory present if switching between the EUTRA band pair is supported, otherwise the field is absent. It is signalled per pair of bands per band combination. | FD | No | No | No |
| SRS-TxSwitch Defines whether UE supports SRS for DL CSI acquisition as defined in clause 6.2.1.2 of TS 38.214 [12]. The capability signalling comprises of the following parameters: - supportedSRS-TxPortSwitch indicates SRS Tx port switching pattern supported by the UE. The indicated UE antenna switching capability of ′xTyR′ corresponds to a UE, capable of SRS transmission on ′x′ antenna ports over total of ′y′ antennas, where ′y′ corresponds to all or subset of UE receive antennas, where 2T4R is two pairs of antennas; - txSwitchImpactToRx indicates the entry number of the first-listed band with UL in the band combination that affects this DL; - txSwitchWithAnotherBand indicates the entry number of the first-listed band with UL in the band combination that switches together with this UL. For txSwitchImpactToRx and txSwitchWithAnotherBand, value 1 means first entry, value 2 means second entry and so on. All DL and UL that switch together indicate the same entry number. The UE is restricted not to include fallback band combinations for the purpose of indicating different SRS antenna switching capabilities. | BC | Yes | No | No |
| supportedBandwidthCombinationSet Defines the supported bandwidth combination for the band combination set as defined in the TS 38.101-1 [2], TS 38.101-2 [3] and TS 38.101-3 [4]. Field encoded as a bit map, where bit N is set to "1" if UE support Bandwidth Combination Set N for this band combination as defined in the TS 38.101-1 [2], TS 38.101-2 [3] and TS 38.101-3 [4]. The leading / leftmost bit (bit 0) corresponds to the Bandwidth Combination Set 0, the next bit corresponds to the Bandwidth Combination Set 1 and so on. It is mandatory if the band combination has more than one NR carrier (at least one SCell in an NR cell group) or is an intra-band EN-DC combination or is an inter-band EN-DC combination with an intra-band EN-DC combination. If this field is included in an inter-band EN-DC combination with an intra-band EN-DC combination, this field is used to indicate the supported bandwidth combination for the intra-band EN-DC combination part. | BC | CY | No | No |

4.2.7.3	CA-ParametersEUTRA

| Definitions for parameters | Per | M | FDD-TDD DIFF | FR1-FR2 DIFF |
| --- | --- | --- | --- | --- |
| additionalRx-Tx-PerformanceReq additionalRx-Tx-PerformanceReq defined in 4.3.5.22, TS 36.306 [15]. | BC | No | No | No |
| multipleTimingAdvance multipleTimingAdvance defined in 4.3.5.3, TS 36.306 [15]. | BC | No | No | No |
| simultaneousRx-Tx simultaneousRx-Tx defined in 4.3.5.4, TS 36.306 [15]. | BC | No | No | No |
| supportedBandwidthCombinationSetEUTRA Indicates the set of supported bandwidth combinations for the LTE part for inter-band EN-DC. The field is encoded as a bit map, where bit N is set to "1" if UE support Bandwidth Combination Set N for this band combination. The leading / leftmost bit (bit 0) corresponds to the Bandwidth Combination Set 0, the next bit corresponds to the Bandwidth Combination Set 1 and so on. The UE shall neither include the field for a EN-DC combination which has only one LTE carrier, nor for a EN-DC combination which has more than one LTE carrier for which the UE only supports Bandwidth Combination Set 0 for the LTE part. If the inter-band EN-DC has more than one LTE carrier excluding the LTE carriers are in an intra-band EN-DC within an inter-band EN-DC combination, the UE shall support at least one bandwidth combination for the supported LTE part. If this field is included in an inter-band EN-DC with an intra-band EN-DC, this field is used to indicate the supported bandwidth combination for the LTE part with intra-band EN-DC part removed. | BC | CY | No | No |
| supportedNAICS-2CRS-AP supportedNAICS-2CRS-AP defined in 4.3.5.8, TS 36.306 [15]. | BC | No | No | No |
| totalWeightedLayers Indicates total number of weighted layers for the LTE part of the concerned EN-DC band combination the UE can process for FD-MIMO, as described in TS 36.306 [15] equation 4.3.28.13-1 and TS 36.331 [5] clause 6.3.6, NOTE 8 in UE-EUTRA-Capability field descriptions. For the EN-DC band combination for which this field is not included, totalWeightedLayers-r13 as described in TS 36.331 [5] applies, if included. | BC | No | No | No |
| ue-CA-PowerClass-N ue-CA-PowerClass-N defined in 4.3.5.1.3, TS 36.306 [15]. | BC | No | No | No |

---------------------------------------------END OF CHANGE---------------------------------------------