3GPP RAN WG2 Meeting #107bis		R2-1913974

Chongqing, China, 14 – 18th October 2019

Agenda item:	5.4.4

Source:	Apple Inc.

Title:	Handling of fallbacks for combined contiguous and non-contiguous CA in FR2

Document for:	Approval

# 1	Introduction

During the RAN4#92 meeting, an issue was raised in [1] that there are many combinations of intra-band contiguous and non-contiguous CA configurations, especially in FR2. Generating all possible fallbacks for these combinations adds a huge amount of new combinations, and many fallbacks for the existing combinations are missing in the spec. Thus, it was agreed further by RAN WG4 to add the corresponding clarification to Rel-15 TS 38.101-2 [2] and TS 38.101-3 [3] that "a UE is not required to support all possible fallback combinations but can directly fall back to a single FR2 carrier". In addition to it, RAN WG4 sent an LS [4] asking RAN WG2 to check signalling to avoid mandating support of all possible fallback combinations.

# 2	Fallbacks of contiguous/non-contiguous CA in FR2

## 2.1	Background

In the E-UTRA specs it was introduced that a UE needs to support all fallbacks for the supported CA combinations. The same approach has been generally used also for NR, however, there are many more complicated combinations in NR especially within FR2. Fallback groups have been defined to mitigate this a bit by not mandating fallbacks outside the fallback group. However, there are certain CA combinations in in FR2, which are a combination of contiguous and non-contiguous CA; and these CA combinations can also be part of EN-DC/NE-DC combinations. Examples are all the combinations which have dashes within the bandwidth class part or have multiple blocks of contiguous carriers like CA_n260(2O), CA_n260(H-I), CA_n260(2A-2O-Q). While we have fallback groups for contiguous CA, fallback groups or other limitations of fallbacks are missing for combinations of contiguous and non-contiguous fallbacks. Currently this sentence on fallbacks is in 38.101-3:

“A terminal which supports an inter-band EN-DC configuration with a certain UL configuration shall support the all lower order DL configurations of the lower order EN-DC combinations, which have this certain UL configuration and the fallbacks of this UL configuration.”

This sentence does not clearly say how the fallbacks are generated and which combination is considered a fallback. Therefore, it is unclear which fallbacks are to be supported and which not. Also, assuming all combinations as fallbacks that result from removing one carrier after the next results in a huge number of additional combinations, that would need to be supported in the spec and in the UE. In the current specs many of these fallbacks are missing.

One relatively simple example of such a combination is DC_2A_n261(H-I). However, already this simple example generates a fallback tree with 12 fallbacks, as shown in figure 1:

Figure 1: Fallback tree for DC_2A_n261(H-I)

There are much more complicated CA combinations that will create many more combinations like CA_n260(2A-2O-Q) and there are many of these combinations. For CA_n260(2A-2O-Q) figure 2 shows the fallback tree with 46 unique fallback combinations (all duplicates already removed). This combination is already in 38.101, however, most of these  fallbacks are missing. Figure 3 shows a visualization of what CA_n260(2A-2O-Q) means.

Figure 2: Fallback tree for CA_n260(2A-2O-Q)

Figure 3: Visualization of CA combination CA_n260(2A-2O-Q)

Usually, an operator requesting a particular band combination is just interested in the original combination. Of course, there should be a possibility not to use some of the carriers, and this can always be done by deactivating one or more carriers of a combination. Nevertheless, it seems that supporting all the fallback combinations as stand-alone combinations is not needed in real network deployments.

Observation 1a:	Especially FR2 combinations with combined contiguous and non-contiguous intra-band CA generate a huge amount of fallback combinations.

Observation 1b:	Each single of the additional fallback combinations add to the already huge number of CA/DC combinations in 3GPP adding a lot of work for the rapporteurs and needs to be implemented separately in the UE as well as tested for the UE type approval.

Observation 1c:	Fallback FR2 combinations with combined contiguous and non-contiguous intra-band CA seem to be not needed, as long as it is possible to deactivate some of the carriers.

Due to these observations, RAN WG4 discussed and agreed to modify fallback requirements in the specification in such a way that fallbacks of combined contiguous and non-contiguous FR2 intra-band combinations are allowed to directly fallback to bandwidth class "A". As said above, this does not mean that some carriers of the original combination cannot be deactivated, but the fallbacks cannot be used as a stand-alone combination. The corresponding CR were agreed in [2] and [3], in which the following clarification was captured.

"A terminal which supports CA or DC configurations, which include FR2 intra-band CA combinations with multiple subblocks, where at least one of the subblocks consists of a contiguous CA combination is not required to support all fallback combinations but can directly fall back to a single FR2 carrier."

With such change, for FR2 intra-band contiguous and non-contiguous band combinations, gNB should consider the UE only support the reported CA band combination and the one single carrier. In details, gNB can only configure the UE with the reported FR2 intra-band CA combination in RRCReconfiguration message. However, gNB can still perform Scell(s) deactivation to achieve lower order CA after the reconfiguration. It is also worth noting that the corresponding clarification was agreed to both Rel-15 and Rel-16.

Observation 2:	RAN WG4 agreement applies to both Rel-15 and Rel-16.

## 2.2	RAN WG2 specification impact

In the LS, which RAN WG4 sent to RAN WG2, it is asked to clarify whether there is any RRC signalling impact caused by the latest RAN WG4 agreement. Referring to the excerpt from TS 38.306 below, a UE typically signals only a particular "master" CA combination and all the fallbacks combinations are not signalled explicitly. In that sense, there is indeed no signalling impact to RAN WG2. At the same time, TS 38.306 also clarifies that all the fallback combinations shall be supported, which is not in line with the latest RAN WG4 agreements according to which a UE can fall back directly to the bandwidth class "A".

| supportedBandCombinationList Defines the supported NR and/or MR-DC band combinations by the UE. For each band combination the UE identifies the associated feature set combination by featureSetCombinations index referring to featureSetCombination. A fallback band combination resulting from the reported CA and MR-DC band combination is not signalled but the UE shall support it. For intra-band non-contiguous CA band combinations, the UE only includes one band combination, and exclude the others for which the presence of uplink CA bandwidth class in the band combination entry is different. One band combination entry can also indicate support of any other possible permutations in the presence of uplink CA bandwidth class where a paired downlink CA bandwidth class is the same or where the number of UL CCs is smaller than the one of paired DL CCs expressed by the CA bandwidth class, as specified in TS 36.306 [15]. For these band combinations not included in the capability, the supported feature set is the same as the ones for the band combination included in the UE capability. | UE | Yes | No | No |
| --- | --- | --- | --- | --- |

Observation 3a:	Since a UE does not have to signal explicitly fallback combinations, the latest RAN WG4 agreement does not have a direct impact on the capability signalling..

Observation 3b:	At the same time, it seems that further clarifications are needed (at least for TS 38.306) to clarify that a UE is not anticipated to support all the fallback combinations..

Corresponding CR to TS38.306 is aprovided in [5].

Proposal: Discuss which changes are needed in RAN2 specs.

# 3	Conclusions

In this contribution, we provide the background of the RAN4 discussion and agreement on handling of fallbacks for combined contiguous and non-contiguous CA in FR2.

Observation 1a:	Especially FR2 combinations with combined contiguous and non-contiguous intra-band CA generate a huge amount of fallback combinations.

Observation 1b:	Each single of the additional fallback combinations add to the already huge number of CA/DC combinations in 3GPP adding a lot of work for the rapporteurs and needs to be implemented separately in the UE as well as tested for the UE type approval.

Observation 1c:	Fallback FR2 combinations with combined contiguous and non-contiguous intra-band CA seem to be not needed, as long as it is possible to deactivate some of the carriers.

Observation 2:	RAN WG4 agreement applies to both Rel-15 and Rel-16.

Observation 3a:	Since a UE does not have to signal explicitly fallback combinations, the latest RAN WG4 agreement does not have a direct impact on the capability signalling..

Observation 3b:	At the same time, it seems that further clarifications are needed (at least for TS 38.306) to clarify that a UE is not anticipated to support all the fallback combinations..

Proposal: Discuss which changes are needed in RAN2 specs.

# 4	References

- R4-1908027, "Handling of fallbacks for combined contiguous and non-contiguous CA in FR2", Apple Inc.

- R4-1908028, "CR for Handling of fallbacks for combined contiguous and non-contiguous CA in FR2", Apple Inc., CR TS 38.101-3

- R4-1910238, "CR for Handling of fallbacks for combined contiguous and non-contiguous CA in FR2", Apple Inc., CR TS 38.101-2

- R4-1910239, "LS on handling of fallbacks for combined contiguous and non-contiguous CA in FR2", Apple Inc.

- R4-1913384, “CR to TS38.306 on FR2 intra band contiguous and non-contiguous CA fallback”