3GPP TSG-RAN WG2 Meeting #107bis	R2-1914009

# ChongQing, China, 14 – 18 October 2019

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.300 | CR | CRNum | rev | - | Current version: | 15.X.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Stage-2 running CR for support of NR Industrial IoT WI |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia (Rapporteur) |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_IIOT |  |  |  |  |  |  | Date: |  | 2019-10 |
| Category: | B |  |  |  |  |  |  | Release: |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | This CR introduces the enhancements specified as part of the Work Item on support of Industrial Internet of Things. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Ethernet header compression support is mentioned in sections 4.2 and 6.4.1. - New function of MAC to handle prioritization between overlapping resources of a UE is captured in section 6.2.1. - Description of downlink and uplink scheduling (sections 10.2, 10.3) are updated to account for support of multiple active SPS and CG configurations for a BWP of a UE. - Section 10.3 is updated to describe overlapping grants prioritization functionality. - A possibility of configuring restrictive mapping between an LCH and certain CG configurations as well as support for LCH restriction based on an indication in the DCI providing dynamic uplink grant is added in section 16.1.2. - PDCP duplication description (section 16.1.3) is updated to account for the possibility to confgure up to 3 secondary RLC entities and support for delivery of up to 4 copies of the same PDCP PDU. - Section 16.X is added to describe enhancements related to Time Sensitive Communications support, i.e. accurate reference time information provisioning, TSC Assistance Information provisioning. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Enhancements agreed as part of WI on support of NR Industrial IoT are not specified in stage-2 specifications. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.2, 6.2.1, 6.4.1, 10.2, 10.3, 16.1.2, 16.1.3, 16.X (new clause) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  |  | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

First Modified Subclause

## 4.2	Functional Split

The gNB and ng-eNB host the following functions:

-	Functions for Radio Resource Management: Radio Bearer Control, Radio Admission Control, Connection Mobility Control, Dynamic allocation of resources to UEs in both uplink and downlink (scheduling);

-	IP and Ethernet header compression, encryption and integrity protection of data;

-	Selection of an AMF at UE attachment when no routing to an AMF can be determined from the information provided by the UE;

-	Routing of User Plane data towards UPF(s);

-	Routing of Control Plane information towards AMF;

-	Connection setup and release;

-	Scheduling and transmission of paging messages;

-	Scheduling and transmission of system broadcast information (originated from the AMF or OAM);

-	Measurement and measurement reporting configuration for mobility and scheduling;

-	Transport level packet marking in the uplink;

-	Session Management;

-	Support of Network Slicing;

-	QoS Flow management and mapping to data radio bearers;

-	Support of UEs in RRC_INACTIVE state;

-	Distribution function for NAS messages;

-	Radio access network sharing;

-	Dual Connectivity;

-	Tight interworking between NR and E-UTRA.

Next Modified Subclause

## 6.2	MAC Sublayer

### 6.2.1	Services and Functions

The main services and functions of the MAC sublayer include:

-	Mapping between logical channels and transport channels;

-	Multiplexing/demultiplexing of MAC SDUs belonging to one or different logical channels into/from transport blocks (TB) delivered to/from the physical layer on transport channels;

-	Scheduling information reporting;

-	Error correction through HARQ (one HARQ entity per cell in case of CA);

-	Priority handling between UEs by means of dynamic scheduling;

-	Priority handling between logical channels of one UE by means of logical channel prioritisation;

-	Priority handling between overlapping resources of one UE;

-	Padding.

A single MAC entity can support multiple numerologies, transmission timings and cells. Mapping restrictions in logical channel prioritisation control which numerology(ies), cell(s), and transmission timing(s) a logical channel can use (see subclause 16.1.2).

Next Modified Subclause

## 6.4	PDCP Sublayer

### 6.4.1	Services and Functions

The main services and functions of the PDCP sublayer include:

-	Transfer of data (user plane or control plane);

-	Maintenance of PDCP SNs;

-	Header compression and decompression using the ROHC protocol;

-	Ethernet header compression and decompression;

-	Ciphering and deciphering;

-	Integrity protection and integrity verification;

-	Timer based SDU discard;

-	For split bearers, routing;

-	Duplication;

-	Reordering and in-order delivery;

-	Out-of-order delivery;

-	Duplicate discarding.

Since PDCP does not allow COUNT to wrap around in DL and UL, it is up to the network to prevent it from happening (e.g. by using a release and add of the corresponding radio bearer or a full configuration).

Next Modified Subclause

## 10.2	Downlink Scheduling

In the downlink, the gNB can dynamically allocate resources to UEs via the C-RNTI on PDCCH(s). A UE always monitors the PDCCH(s) in order to find possible assignments when its downlink reception is enabled (activity governed by DRX when configured). When CA is configured, the same C-RNTI applies to all serving cells.

The gNB may pre-empt an ongoing PDSCH transmission to one UE with a latency-critical transmission to another UE. The gNB can configure UEs to monitor interrupted transmission indications using INT-RNTI on a PDCCH. If a UE receives the interrupted transmission indication, the UE may assume that no useful information to that UE was carried by the resource elements included in the indication, even if some of those resource elements were already scheduled to this UE.

In addition, with Semi-Persistent Scheduling (SPS), the gNB can allocate downlink resources for the initial HARQ transmissions to UEs: RRC defines the periodicity of the configured downlink assignments while PDCCH addressed to CS-RNTI can either signal and activate the configured downlink assignment, or deactivate it; i.e. a PDCCH addressed to CS-RNTI indicates that the downlink assignment can be implicitly reused according to the periodicity defined by RRC, until deactivated.

NOTE:	when required, retransmissions are explicitly scheduled on PDCCH(s).

The dynamically allocated downlink reception overrides the configured downlink assignment in the same serving cell, if they overlap in time. Otherwise a downlink reception according to the configured downlink assignment is assumed, if activated.

The UE may be configured with up to 8 active configured downlink assignments for a given BWP of a serving cell. When more than one is configured:

-	The network decides which of these configured downlink assignments are active at a time (including all of them); and

-	Each configured downlink assignment is activated separately using a DCI command and deactivation of configured downlink assignments is done using a DCI command, which can either deactivate a single configured downlink assignment or multiple configured downlink assignments jointly.

Activation and deactivation of configured downlink assignments are independent among the serving cells.

Editor’s note: FFS whether there are other restrictions of how many SPS configurations are supported, e.g. per cell / per UE.

## 10.3	Uplink Scheduling

In the uplink, the gNB can dynamically allocate resources to UEs via the C-RNTI on PDCCH(s). A UE always monitors the PDCCH(s) in order to find possible grants for uplink transmission when its downlink reception is enabled (activity governed by DRX when configured). When CA is configured, the same C-RNTI applies to all serving cells.

In addition, with Configured Grants, the gNB can allocate uplink resources for the initial HARQ transmissions to UEs. Two types of configured uplink grants are defined:

-	With Type 1, RRC directly provides the configured uplink grant (including the periodicity).

-	With Type 2, RRC defines the periodicity of the configured uplink grant while PDCCH addressed to CS-RNTI can either signal and activate the configured uplink grant, or deactivate it; i.e. a PDCCH addressed to CS-RNTI indicates that the uplink grant can be implicitly reused according to the periodicity defined by RRC, until deactivated.

If the UE is not capable of enhanced intra-UE overlapping resources prioritization, the dynamically allocated uplink transmission overrides the configured uplink grant in the same serving cell, if they overlap in time. Otherwise an uplink transmission according to the configured uplink grant is assumed, if activated.

If the UE is capable of enhanced intra-UE overlapping resources prioritization, in case configured uplink transmission overlaps in time with dynamically allocated uplink transmission or with another configured uplink transmission in the same serving cell, the UE prioritizes the transmission with higher priority. If the prioritization takes place before MAC PDU generation for overlapping grants, the UE only generates MAC PDU for the prioritized transmission. Similarly, in case a configured grant or dynamically allocated uplink transmission overlaps in time with scheduling request occasion for a pending scheduling request, the UE prioritizes the transmission with higher priority. If the PUSCH transmission is deprioritized and the pending scheduling request and the PUSCH transmission cannot be transmitted simultaneously, the UE does not generate a MAC PDU for deprioritized PUSCH transmission, if not yet generated. In case the MAC PDU associated with a deprioritized transmission has already been generated, the UE shall store it to allow the gNB to schedule a retransmission.

Editor’s note: FFS how transmission with higher priority is determined exactly.

Editor’s note: FFS other ways of treating de-prioritized data from configured grants.

Retransmissions other than repetitions are explicitly allocated via PDCCH(s).

The UE may be configured with up to 12 active configured uplink grants for a given BWP of a serving cell. When more than one is configured, the network decides which of these configured uplink grants are active at a time (including all of them). Each configured uplink grant can either be of Type 1 or Type 2.

Editor’s note: FFS whether there are other restrictions of how many CG configurations are supported, e.g. per cell / per UE.

For Type 2, activation and deactivation of configured uplink grants are independent among the serving cells. When more than one Type 2 configured grant is configured, each configured grant is activated separately using a DCI command and deactivation of Type 2 configured grants is done using a DCI command, which can either deactivate a single configured grant configuration or multiple configured grant configurations jointly.

When SUL is configured, configured uplink grants can only be signalled for one of the 2 ULs of the cell.

Next Modified Subclause

### 16.1.2	LCP Restrictions

With LCP restrictions in MAC, RRC can restrict the mapping of a logical channel to a subset of the configured cells, numerologies, PUSCH transmission durations, configured grant configurations and control whether a logical channel can utilise the resources allocated by a Type 1 Configured Grant (see clause 10.3) or whether a logical channel can utilise dynamic grants containing a “specific indication”.

Editor’s note: Exact naming and mechanism for “specific indication” is FFS.

With such restrictions, it then becomes possible to reserve, for instance, the numerology with the largest subcarrier spacing and/or shortest PUSCH transmission duration for URLLC services. Furthermore, RRC can associate logical channels with different SR configurations, for instance, to provide more frequent SR opportunities to URLLC services.

Editor’s note: FFS how to enhance LCP restrictions for dynamic grant to take into account reliability.

### 16.1.3	Packet Duplication

When duplication is configured for a radio bearer by RRC, at least one secondary RLC entity is added to the radio bearer to handle the duplicated PDCP PDUs as depicted on Figure 16.1.3-1, where the logical channel corresponding to the primary RLC entity is referred to as the primary logical channel, and the logical channel corresponding to the secondary RLC entity(ies), the secondary logical channel(s). All RLC entities have the same RLC mode. Duplication at PDCP therefore consists in submitting the same PDCP PDUs multiple times: once to each activated RLC entity for the radio bearer. With independent transmission paths, packet duplication therefore increases reliability and reduces latency and is especially beneficial for URLLC services.

Figure 16.1.3-1: Packet Duplication

NOTE:	PDCP control PDUs are not duplicated and always submitted to the primary RLC entity.

When configuring duplication for a DRB, RRC also sets the initial state (either activated or deactivated). After the configuration, the states of the RLC entities configured for the DRB can then be dynamically controlled by means of a MAC control element and in DC, the UE applies the MAC CE commands regardless of their origin (MCG or SCG). When duplication is configured for an SRB the state is always active and cannot be dynamically controlled.

When activating duplication for a DRB, NG-RAN should ensure that at least one serving cell is activated for each logical channel associated to an activated RLC entity of the DRB; and when the deactivation of SCells leaves no serving cells activated for a logical channel of the DRB, NG-RAN should ensure that duplication is also deactivated.

When duplication is activated, the original PDCP PDU and the corresponding duplicate(s) shall not be transmitted on the same carrier. The primary and secondary logical channels can either belong to the same MAC entity (referred to as CA duplication) or to different ones (referred to as DC or DC+CA duplication). CA duplication can be configured together with DC duplication when duplication over more than two legs is configured in the UE. In CA duplication, logical channel mapping restrictions are used in MAC to ensure that the primary and secondary logical channels are not sent on the same carrier.

When duplication is deactivated for a DRB, the secondary RLC entity(ies) is/are not re-established, the HARQ buffers are not flushed, and the transmitting PDCP entity should indicate to the secondary RLC entity(ies) to discard all duplicated PDCP PDUs. In addition, in case of CA duplication, the logical channel mapping restrictions of the primary and secondary logical channels are lifted for as long as duplication remains deactivated.

When an RLC entity acknowledges the transmission of a PDCP PDU, the PDCP entity shall indicate to the other RLC entity(ies) to discard it. In addition, in case of CA duplication, when the RLC entity restricted to only SCell(s) reaches the maximum number of retransmissions for a PDCP PDU, the UE informs the gNB but does not trigger RLF.

Next Modified Subclause

## 16.X	Support for Time Sensitive Communications

Time Sensitive Communications (TSC), as defined in TS 23.501 [3], is a communication service that supports deterministic communication and/or isochronous communication with high reliability and availability. Examples of such services are the ones in the area of Industrial Internet of Things, e.g. related to cyber-physical control applications as described in TS 22.104 [REF].

To support strict synchronization accuracy requirements of TSC applications, the gNB may signal 5G system time reference information to the UE using unicast or broadcast RRC signalling with a granularity of 10 ns. Uncertainty parameter may be included in reference time information to indicate accuracy.

Editor’s note: FFS how the need for reference time information can be determined for any given connected UE.

Editor’s note: FFS whether more information should be captured in Stage-2.

The gNB may also receive TSC Assistance Information (TSCAI), see TS 23.501 [3], from the Core Network, e.g. during QoS flow establishment, or from another gNB during handover. TSCAI contains additional information about the traffic flow such as burst arrival time and burst periodicity. TSCAI knowledge may be leveraged in the gNB’s scheduler to more efficiently schedule periodic, deterministic traffic flows either via Configured Grants, Semi-Persistent Scheduling or with dynamic grants.

Editor’s Note: FFS (in SA2) whether burst size is also included in TSCAI.

End of changes

# Annex	- collection of RAN2 agreements on NR IIoT WI

Agreements from RAN2#105bis meeting (Xi’an, China, 08 – 12 April 2019):

| Accurate reference timing | Confirm that we use LTE rel-15 SIB and RRC unicast based methods for reference time delivery The reference time information shall correspond to a reference SFN, explicitly indicated in unicast signalling, FFS if inferred from the transmission of the SIB for SIB signalling. R2 assumes the UE shall use the end of the reference SFN value as the precise point in time to which the reference time corresponds. FFS whether the reference SFN refers to time in the future, past or whether this need to mandated one way or another. R2 assumes that some propagation delay compensation may be needed for distance > 200m. FFS what would be the method, e.g. based on current TA, and whether this can be left for UE implementation or something need to be specified. |
| --- | --- |
| Scheduling Enhancements | RAN2 think that knowledge of survival time is beneficial to gNB. FFS whether there would be any impact to AS specifications to make use of this, and such discussions would have lower priority, as it is not explicitly a WI objective. There are also concerns that QoS framework may be impacted due to survival time being provided explicitly. R2 assumes that the maximum number of active SPS configurations for a given BWP of a serving cell in the specification is 8 or 16 (FFS). R2 assumes short SPS/CG periodicities and/or multiple SPS/CG configurations and/or combination thereof could be used to mitigate the periodicity misalignment between the TSN periodicity and CG/SPS periodicity. Other solutions not precluded, e.g. to address resource consumption. Will support “short” SPS periodicities, at least down to 0.5ms Ask R1 on feasibility, and additionally the feasibility to go down to even lower values, e.g. 2 symb. R2 assumes that activation/deactivation is done by DCI. RAN1 should address activation/deactivation DCIs related with configured grant Type 2 and SPS in the case of multiple configurations When multiple UL CG or DL SPS configurations is configured, an offset for each configuration is needed for the calculation of the HARQ process ID |
| Ethernet Header Compression | IETF see no issues for 3GPP to develop and maintain a ROHC profile. Also, it seems feasible in the time frame of Rel-16. We develop Ethernet header compression 100% in 3GPP TS (not by extending ROHC) |

Agreements from RAN2#106 meeting (Reno, US, 13 – 17 May 2019):

| Accurate reference timing | SFN boundary at or immediately after the ending boundary of the SI-window in which SIB is transmitted is always used as a reference in case the time reference information is provided by broadcast signalling (as in LTE) The UE considers the frame indicated by the referenceSFN nearest to the frame where the time information is received, which can be either in the past or in future, in case the time reference information is provided by unicast signalling Signalling to support 10ns granularity. R2 assumes that either SIB9 or a new SIB is used for reference time information broadcast delivery, depending on R3 discussion outcome. “00:00:00 on Gregorian calendar date 6 January, 1980 (start of GPS time)” as the origin of the time reference information, at least for the baseline case where time info type is not present or used (as in LTE). The field used for reference time information delivery is excluded from estimation of changes in system information. Specify uncertainty parameter in the reference time information in NR, encoding FFS We will have the clock type field, similar to LTE. R2 considers that this have no relation to ongoing discussions in SA2 on TSC |
| --- | --- |
| Ethernet Header Compression | Ethernet Header Compression (EHC) is configured per DRB, separately for UL and DL. Use context ID concept such that compressor and decompressor associates a context ID with Ethernet header contents. Compression is done with following principle: - For Ethernet flow resulting in creation of new context, compressor transmits at least one packet with full header and context id (to establish context in decompressor). - After above, compressor starts transmits compressed packets. FFS if multiple transmissions and/or feedback is needed. EHC header format is designed to include following mandatory fields: Context ID, Indication of header format (i.e. full header and compressed header), FFS other field, e.g. profile ID |
| PDCP duplication enhancements | Intention is that Copies are sent on different legs Dynamic Network control of DRB duplication is by MAC CE By the MAC CE, Network to control which of the configured RLC entities that is/are active Support the case that no of copies = no of active RLC entities |
| Intra-UE prioritization / multiplexing | For de-prioritized PUSCH on dynamic grant, the UE should store the de-prioritized MAC PDU in the HARQ buffer, to allow gNB to schedule re-transmission using the same HARQ process. For de-prioritized PUSCH on configured grants, a) the UE could store the de-prioritized MAC PDU in the HARQ buffer, to allow gNB to schedule re-transmission. b) FFS if the UE could transmit it using the subsequent radio resources e.g. associated with the same HARQ process The above agreements are at least applicable for cases when MAC has already generated the de-prioritized MAC PDU |

Agreements from RAN2#107 meeting (Prague, Czechia, 26 – 30 August 2019):

| PDCP duplication enhancements | The number of copies generated is equal to the number of active RLC entities, i.e. one copy per leg/RLC entity, and active/inactive state is determined by MAC CE. The network provides in RRC only one LCH cell restriction configuration per LCH, like in Rel-15. Changes to LCH cell restriction configuration is only possible via RRC. At PDCP duplication, application of the configured cell restrictions are not dynamically changed upon activation or deactivation of PDCP duplication beyond Rel-15. (FFS the case of CA duplication) The MAC CE signaling structure is either: a. Per DRB signaling with the activation status of the associated RLC entities, or b. All DRBs with the activation status of the associated RLC entities for each DRB, or A new LCID is used for the Rel-16 MAC CE controlling PDCP duplication. |
| --- | --- |
| Intra-UE prioritization / multiplexing | same prioritization solution for CG vs CG conflict and CG vs DG conflict Extend LCP restrictions by allowing restrictive mapping between an LCH and certain CG configurations. LCP restriction enhancements for DG to take into account reliability is needed, details FFS. no need to define UE processing time in MAC The same UE prioritization behaviour should be applied for resource conflicts between new transmissions or a new transmission and a retransmission. RAN2 assumes that MAC PDU recovery method in grant prioritization could be reused for PUSCH vs SR conflict. The case of highest priorities of two conflicting grants are equal is handled according to the following: for CG DG conflict, DG is prioritized, other cases FFS to what extent to specify. For The case when no PDU has been generated at all yet, and there is two grants where one will be de-prioritized (and there is data available for both grants). One PDU is generated If PUCCH resource for an SR’s transmission occasion overlaps a UL-SCH resource, SR’s transmission is allowed based on a comparison of priority of the LCH that triggered the SR and a priority value for the UL-SCH resource, if the priority of the LCH that triggered the SR is “high” (FFS). Priority value of the UL-SCH resource is FFS If an SR was triggered before MAC PDU assembly and PUCCH resource for the SR’s transmission occasion conflicts with UL-SCH resource of the MAC PDU, and the UL-SCH transmission is deprioritized, a MAC PDU will not be generated. (conflict = they cannot both be transmitted) When a PUSCH transmission is deprioritized, desired PHY behaviour is for RAN1 to decide |

Agreements from RAN2#107bis meeting (Chongqing, China, 14 – 18 October 2019):

| Accurate reference timing | SIB9 is used for accurate reference timing delivery by broadcast. DLInformationTransfer message is used for serving cell’s accurate reference timing delivery by unicast. R2 assumes there will be no particular functionality to ensure accurate timing distribution at the moment of handover in Rel-16 The uncertainty of reference time info is unspecified, if the uncertainty field is absent. We send an LS: RAN2 asks SA2 to provide information on whether and how the need for reference time information can be determined for any given connected UE FFS if The referenceSFN field indicates the time at the ending boundary of the SFN indicated by referenceSFN of PCell. |
| --- | --- |
| Scheduling Enhancements | - Multiple SPS/CG configuration and new CG/SPS periodicities: R2 assumes to support 8 as the maximum number of simultaneously activated SPS configurations per BWP per serving cell. Introduce SPS/CG index to identify each SPS/CG among multiple SPS/CG configurations, i.e., as in Rel-15 LTE. The association between “state” (used in the joint release DCI) and the CG configuration(s) for type-2 CG is configured via RRC message. Each CG configuration is always configured independently, as in Rel-15 LTE. The association between “state” (used in the joint release DCI) and the SPS configuration(s) is configured via RRC message, if RAN1 working assumption for joint release for multiple SPS configuration is confirmed. Each SPS configuration is always configured independently, as in Rel-15 LTE. Support simultaneous Type 1 & 2 CG configurations in a BWP. CG periodicities of any integer-multiple of one slot (FFS if we go even lower, e.g. 2 symb, 7 symb) below a maximum value should be supported. FFS on the maximum value of integer N. SPS periodicities of any integer-multiple of one slot below a maximum value should be supported in Rel-16. FFS on the maximum value of integer N. R2 assumes that HARQ offset parameter is explicitly configured by the network for each CG/SPS configuration. For CG, HARQ Process ID = [floor(CURRENT_symbol/periodicity)] modulo nrofHARQ-Processes + harq-procID-offset. FFS (for checking) if For SPS, HARQ Process ID = [floor(CURRENT_slot/periodicity)] modulo nrofHARQ-Processes + harq-ProcID-offset, Where CURRENT_slot = [(SFN × numberOfSlotsPerFrame) + slot number in the frame]. Introduce a new confirmation MAC CE format in Rel-16, which reflects the confirmation of multiple configured grant configurations LCH restrictions related: A single LCH can be map to multiple CG configurations. Multiple LCHs can be map to a single CG configuration. R2 think it would be useful to introduce a new LCP restriction in the following way: The DCI that is scheduling PUSCH may include a specific indication. LCH configuration in RRC contains information on whether the LCH can utilize grant with this indication or not. R2 intends that this mechanism can be used to differentiate grants for traffic that requires high reliability. TSCAI related: Granularity of burst arrival time and periodicity signalled to RAN should be preferably 1 us. |
| Ethernet Header Compression | The EHC function is in PDCP The EHC header is located after the SDAP header, and it is ciphered The EHC can removes the following fields: SOURCE/DESTINATION ADDRESS, TYPE, and EHC do not support multiple formats FFS: Pad removal For context establishment the compressor send the full header and the context ID via PDCP data PDU ROHC and EHC are independent, e.g. from specification point of view they could both be configured for a DRB. FFS if for context establishment the explicit feedback is sent via PDCP control PDU. Baseline feedback mechanism, enhancements not precluded: For context establishment the de-compressor sends an explicit feedback to the compressor after the establishment of the context, i.e. when a full header packet is received with a context id. For context establishment the explicit feedback includes the “Context ID”. When the compressor receives the feedback it is confident that the context is successfully established, and from this time compressed header packets can be transmitted. FFS if EHC is allowed to be configured for a unidirectional link. |
| Intra-UE prioritization / multiplexing | - Deprioritized MAC PDU handling: We don’t do the solution where the UE indicate explicitly to the network that there is data for a deprioritized PDU There is support to have “UE autonomous retransmission in a CG resource”. Allow checking of complexity to next meeting. |
| PDCP duplication enhancements | The mechanism of primary path defined for Rel-15 PDCP duplication should be retained for Rel-16 (FFS if allowed to deactivate a primary path ie to not send data PDU). |