3GPP TSG RAN WG2#108	R2-1914325

Reno, USA, 18 – 22 November, 2019

3GPP TSG-RAN4 Meeting #92BIS 	R4-1911416

# Chongqing, China, 14 – 18 October, 2019

Title:	LS on CLI measurement reporting range

Response to:

Release:	Rel-16

Work Item:	NR_CLI_RIM-Core

Source:	RAN WG4

To:	RAN WG2

Cc:

Contact Person:

#### Name:	Jin-Yup Hwang

E-mail Address:	jinyup.hwang@lge.com

1. Overall Description:

RAN4 has discussed the reporting range for SRS-RSRP and CLI-RSSI. RAN4 would like to inform RAN2 the conclusion in RAN4 as follows:

- SRS-RSRP measurement report mapping

| Reported value | Measured quantity value | Unit |
| --- | --- | --- |
| SRS_RSRP_0 | SRS-RSRP<-140 | dBm |
| SRS_RSRP_1 | -140≤ SRS-RSRP<-139 | dBm |
| SRS_RSRP_2 | -139≤ SRS-RSRP<-138 | dBm |
| SRS_RSRP_3 | -138≤ SRS-RSRP<-137 | dBm |
| SRS_RSRP_4 | -137≤ SRS-RSRP<-136 | dBm |
| … | … | … |
| SRS_RSRP_95 | -46≤ SRS-RSRP<-45 | dBm |
| SRS_RSRP_96 | -45≤ SRS-RSRP<-44 | dBm |
| SRS_RSRP_97 | -44≤ SRS-RSRP | dBm |
| SRS_RSRP_98Note1 | Infinity |  |
| Note1 : ‘Infinity’ means that UE cannot detect SRS due to too strong signal to measure. |  |  |

- CLI-RSSI measurement report mapping

| Reported value | Measured quantity value | Unit |
| --- | --- | --- |
| CLI_RSSI_00 | CLI-RSSI -100 | dBm |
| CLI_RSSI_01 | -100 CLI-RSSI < -99 | dBm |
| CLI_RSSI_02 | -99 CLI-RSSI < -98 | dBm |
| … | … | … |
| CLI_RSSI_74 | -27 CLI-RSSI < -26 | dBm |
| CLI_RSSI_75 | -26 CLI-RSSI < -25 | dBm |
| CLI_RSSI_76 | -25 CLI-RSSI | dBm |

2. Actions:

To RAN2:

ACTION: RAN4 respectfully asks RAN2 to take the above RAN4 conclusion into consideration for their further work.

3. Date of Next TSG-RAN4 Meetings:

3GPP RAN4 #93		18 - 22 November 2019,		US

3GPP RAN4 #94		24 - 28 February 2020,		Greece