3GPP TSG-RAN WG2 Meeting #108		                           R2-1914937

Reno, Nevada, USA, 18th – 22th Nov, 2019                   (Revision of R2-1912269)

Source:	vivo

Title:	Power control for NR-DC

Agenda Item:	6.10.2

Document for:	Discussion and Decision

- Introduction

At the RAN#81 meeting, WI on Multi-RAT Dual-Connectivity and Carrier Aggregation enhancements was approved which includes following objective [1]:

- Support of asynchronous and synchronous NR-NR Dual Connectivity [RAN1, RAN2, RAN4] - UE power control [RAN1] - RRC signalling to support of enhanced NR-NR DC [RAN2] - Core requirements to support enhanced NR-NR DC [RAN4] Note: Synchronous DC enhancements in this WID considers only cases not covered in Rel-15 exception sheet for NR WI NR_newRAT-Core.

It was agreed in RAN1#98bis to consider the following Alt.1-2 and Alt.2 for semi-static power sharing with PMCG+PSCG≤PTotalNN- DC:

- Alt.1: For the UL transmission in MCG, the UE checks the semi-statically configured direction of the overlapping symbols of all serving cells of SCG, and vice versa.

  - If such overlapping with UL transmission on the SCG is possible (i.e. collides with semi-static ‘UL’ and ‘flexible’ symbols on some CCs of SCG), UE limits its actual transmission power pMCG in MCG such that pMCG≤PMCG;

  - Otherwise (i.e. collides with only semi-static ‘DL’ symbols on all CCs of SCG), pMCG can be up to PMCG' and pSCG can be up to PSCG'.

    - Alt.1-1: PMCG' and PSCG' are configured by RRC signalling.

    - Alt.1-2: PMCG' and PSCG' are determined by RAN4 requirement.

- Alt.2: For the uplink transmission in MCG and in SCG, UE limits its actual transmission power pMCG to be up to PMCG and pSCG to be up to PSCG.

For above two alternatives for semi-static power sharing in NR-DC, it is essential to configure the maximum transmit power used by the UE in each NR CG, i.e., PMCG and PSCG, in RAN2 signalling. In this contribution, we discuss how RRC signalling is used to support the NR-DC power sharing on FR1 and FR2.

- Discussion

2.1 Maximum transmit power configuration Procedure

- Maximum transmit power configuration Procedure in EN-DC:

In EN-DC, the procedure of the maximum transmit power configuration is shown in Figure 1.

Figure 1: The configuration procedure of the maximum transmit power for EN-DC

- MeNB sends the CG-ConfigInfo message to SgNB, which includes the following powerCoordination-FR1 field to indicate the maximum transmit power to be used by the UE in MCG (p-maxEUTRA), and the maximum transmit power to be used by the UE in SCG across all serving cells in FR1 (p-maxNR-FR1), and the maximum total transmit power to be used by the UE across all serving cells in FR1 (p-maxUE-FR1).

powerCoordination-FR1               SEQUENCE {

p-maxNR-FR1                     P-Max                                OPTIONAL,

p-maxEUTRA                      P-Max                                OPTIONAL,

p-maxUE-FR1                     P-Max                                OPTIONAL

}                                                                        OPTIONAL,

- SgNB configures the maximum transmit power used by the UE in the SCG across all serving cells in FR1 (p-NR-FR1) according to the fields in powerCoordination-FR1, and sets p-NR-FR1 into the PhysicalCellGroupConfig IE of the RRCReconfiguration message that is sent to the UE.

- SgNB sends the CG-Config message to the MeNB, in which the RRCReconfiguration message generated by the SgNB is contained.

- MeNB configures the maximum transmit power used by the UE in MCG (p-MaxEUTRA-r15) and the maximum total transmit power used by the UE in FR1 (p-MaxUE-FR1-r15). p-MaxEUTRA-r15 and p-NR-FR1 (in container nr-SecondaryCellGroupConfig-r15) are included in RRCConnectionReconfiguration-v1510 IE. p-MaxUE-FR1-r15 is included in RRCConnectionReconfiguration-v1530 IE.

- MeNB sends RRCConnectionReconfiguration message to the UE, in which RRCConnectionReconfiguration-v1510 and RRCConnectionReconfiguration-v1530 IEs are included.

Observation 1: In EN-DC, the maximum transmit power used by the UE in the NR SCG is configured by the SN.

- Maximum transmit power configuration Procedure in NR-DC:

In NR-DC for FR1, there are two potential alternatives that can be considered:

- Alt1: Similar to EN-DC, the maximum transmit power used by the UE in the SCG is configured by the SN;

- Alt2: MN configures the maximum transmit power used by the UE in the SCG.

For Alt1, SN anyway sends CG-Config message to the MN to indicate the configured maximum transmit power used by the UE in the SCG, after receiving CG-ConfigInfo message from the MN. However, for Alt2, MN configures the maximum transmit power used by the UE in the SCG and informs the SN this value. This value can be modified by the SN later via requestedP-MaxFR1 field in the CG-ConfigInfo message, if the SN needed. Thus, Alt2 is preferred due to signal overhead and complexity reduction resulting from power control coordination between MN and SN. The above discussion can be also applied to the case of NR-DC for FR2.

Observation 2: In NR-DC, MN can configure the maximum transmit power used by the UE in the SCG, in the perspective of both signal overhead and complexity reduction between MN and SN.

Thus, an example of the configuration procedure of the maximum transmit power for NR-DC as depicted in Figure 2.

Figure 2: The configuration procedure of the maximum transmit power for NR-DC

- MN configures the maximum transmit power used by the UE in the MCG, and the maximum transmit power used by the UE in the SCG as well as the maximum total transmit power used by the UE. Which information elements of the RRCReconfiguration message the MN may use to include these fields is further discussed in the Section 2.2.

- MN sends RRCReconfiguration message to the UE, which includes the above configured maximum transmit power parameters.

- MN may inform the SN about the configured maximum transmit power used by the UE in the SCG via CG-ConfigInfo message. Section 2.3 also discusses the issue about what information may be necessary to be informed to the SN.

Proposal 1: In NR-DC, MN configures the maximum transmit power used by the UE in the MCG, and in the SCG as well as the maximum total transmit power. MN informs the SN about the configured maximum transmit power used by the UE in the SCG via CG-ConfigInfo message.

2.2 Power configuration

In an email discussion after RAN1#98bis, there is a conclusion that p-NR-FR1 defined in Rel-15 can be reused to configure the maximum transmit power used by the UE in both MCG and SCG for NR-DC on FR1. Moreover, p-NR-FR2 is introduced for NR-DC on FR2.

p-NR-FR1 The maximum total transmit power to be used by the UE in this NR cell group across all serving cells in frequency range 1 (FR1). The maximum transmit power that the UE may use may be additionally limited by p-Max (configured in FrequencyInfoUL) and by p-UE-FR1 (configured total for all serving cells operating on FR1).

For NR-DC on FR1, p-NR-FR1 can be used to configure the maximum transmit power used by the UE in MCG and SCG, but in different IEs of the RRCReconfiguration message. p-NR-FR1 refers to the maximum transmit power in the MCG is in PhysicalCellGroupConfig IE of masterCellGroup IE of RRCReconfiguration-v1530-IEs, while p-NR-FR1 refers to the maximum transmit power in the SCG is in the PhysicalCellGroupConfig IE of secondaryCellGroup IE of RRCReconfiguration-IEs.

Observation 3: For NR-DC on FR1, p-NR-FR1 can be used to configure the maximum transmit power used by the UE in the MCG and SCG.

Besides, the existing field p-UE-FR1 in the PhysicalCellGroupConfig IE also can be reused to configure the maximum total transmit power used by the UE for NR-DC on FR1.

p-UE-FR1 The maximum total transmit power to be used by the UE across all serving cells in frequency range 1 (FR1) across all cell groups. The maximum transmit power that the UE may use may be additionally limited by p-Max (configured in FrequencyInfoUL) and by p-NR-FR1 (configured for the cell group).

Proposal 2: Reuse p-UE-FR1 field in the PhysicalCellGroupConfig IE to configure the maximum total transmit power used by the UE for NR-DC on FR1.

For the case of FR2, due to the introduction of p-NR-FR2 and, FR1 and FR2 max powers are differently defined, a new field p-UE-FR2 should also be introduced.

Proposal 3: Introduce a new field p-UE-FR2 to configure the maximum total transmit power used by the UE for NR-DC on FR2.

2.3 Power coordination between MN and SN

Based on the descriptions in the Section 2.1, for NR-DC, a new field powerCoordinationNRDC-FR1 can be added in CG-ConfigInfo massage to indicate the configured maximum transmit power used in the SCG in FR1. The field powerCoordinationNRDC-FR1 can be defined as follows:

powerCoordinationNRDC-FR1               SEQUENCE {

p-maxNR-MCG-FR1                     P-Max                                OPTIONAL,

p-ConfigMaxNR-SCG-FR1               P-Max                                OPTIONAL,

p-maxUE-nrdc-FR1                    P-Max                                OPTIONAL

}

p-maxNR-MCG-FR1 Indicates the maximum total transmit power to be used by the UE in the NR master cell group in frequency range 1 (FR1).

p-ConfigMaxNR-SCG-FR1 Indicates the configured maximum total transmit power used by the UE in the NR secondary cell group in frequency range 1 (FR1).

p-maxUE-nrdc-FR1 Indicates the maximum total transmit power to be used by the UE across all serving cells in frequency range 1 (FR1).

Proposal 4: A new field powerCoordinationNRDC-FR1 is added in CG-ConfigInfo massage for NR-DC in FR1 to indicate the configured maximum transmit power used in the SCG, in which p-maxNR-MCG-FR1, p-maxNR-SCG-FR1 and p-maxUE-nrdc-FR1 are defined for MCG, SCG and UE, respectively.

Proposal 5: RAN2 sends an Ls to inform RAN1 and RAN4 about RAN2 progress and feedback comments, if any.

Similarly, the above parameters should be also introduced for NR-DC on FR2, i.e.,

powerCoordinationNRDC-FR2               SEQUENCE {

p-maxNR-MCG-FR2                     P-Max                                OPTIONAL,

p-ConfigMaxNR-SCG-FR2               P-Max                                OPTIONAL,

p-maxUE-nrdc-FR2                    P-Max                                OPTIONAL

}

p-maxNR-MCG-FR2 Indicates the maximum total transmit power to be used by the UE in the NR master cell group in frequency range 1 (FR2).

p-ConfigMaxNR-SCG-FR2 Indicates the configured maximum total transmit power used by the UE in the NR secondary cell group in frequency range 1 (FR2).

p-maxUE-nrdc-FR2 Indicates the maximum total transmit power to be used by the UE across all serving cells in frequency range 1 (FR2).

Proposal 6: A new field powerCoordinationNRDC-FR2 is added in CG-ConfigInfo massage for NR-DC in FR2 to indicate the configured maximum transmit power used in the SCG, in which p-maxNR-MCG-FR2, p-maxNR-SCG-FR2 and p-maxUE-nrdc-FR2 are defined for MCG, SCG and UE, respectively.

- Conclusion

In this contribution, we discuss how RRC signalling is used to support the NR-DC for semi-static power sharing, and have the following observations and proposals:

Observation 1: In EN-DC, the maximum transmit power used by the UE in the NR SCG is configured by the SN.

Observation 2: In NR-DC, MN can configure the maximum transmit power used by the UE in the SCG, in the perspective of both signal overhead and complexity reduction between MN and SN.

Observation 3: For NR-DC on FR1, p-NR-FR1 can be used to configure the maximum transmit power used by the UE in the MCG and SCG.

Proposal 1: In NR-DC, MN configures the maximum transmit power used by the UE in the MCG, and in the SCG as well as the maximum total transmit power. MN informs the SN about the configured maximum transmit power used by the UE in the SCG via CG-ConfigInfo message.

Proposal 2: Reuse p-UE-FR1 field in the PhysicalCellGroupConfig IE to configure the maximum total transmit power used by the UE for NR-DC on FR1.

Proposal 3: Introduce a new field p-UE-FR2 to configure the maximum total transmit power used by the UE for NR-DC on FR2.

Proposal 4: A new field powerCoordinationNRDC-FR1 is added in CG-ConfigInfo massage for NR-DC in FR1 to indicate the configured maximum transmit power used in the SCG, in which p-maxNR-MCG-FR1, p-maxNR-SCG-FR1 and p-maxUE-nrdc-FR1 are defined for MCG, SCG and UE, respectively.

Proposal 5: RAN2 sends an Ls to inform RAN1 and RAN4 about RAN2 progress and feedback comments, if any.

Proposal 6: A new field powerCoordinationNRDC-FR2 is added in CG-ConfigInfo massage for NR-DC in FR2 to indicate the configured maximum transmit power used in the SCG, in which p-maxNR-MCG-FR2, p-maxNR-SCG-FR2 and p-maxUE-nrdc-FR2 are defined for MCG, SCG and UE, respectively.

- References

- RP-182076, ‘WID on Multi-RAT Dual-Connectivity and Carrier Aggregation enhancements (LTE_NR_DC_CA_enh_Core),’ Ericsson, Nokia, Nokia Shanghai Bell, Huawei