3GPP TSG-RAN2 Meeting #108	R2-1915355

# Reno, USA, 18th – 22nd November 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.331 | CR | 1367 | rev | - | Current version: | 15.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification on timer T304 in SCGFailureInformation |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2019-11-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | According to current RRC specification, when a failure is detected, and the SCG failure information procedure is initiated, the UE will stop timer T304. However, differently from LTE where we have separate timer for the MCG and SCG, in NR the timer T304 may be configured and started on both the MCG and SCG. Nevertheless, is not clear in the current specification which timer T304 should be stopped when the SCG failure information is initiated. In such case, it should be clear that the T304 that is stopped when the SCG failure information is triggered is the one of the SCG. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Section 5.7.3.2 - Clarify that the timer T304 that is stopped is the one referring to the SCG. Impact analysis Impacted 5G architecture options: NR-DC, (NG)EN-DC Impacted functionality: SCG failure information Inter-operability: If the UE implements the CR and the network does not, there is no interoperability issue. If the network implements the CR and the UE does not, there is inter-operability issue since the UE may stop the T304 referring to the MCG and when the T304 SCG expires, the UE will trigger again the SCG failure procedure (i.e., but the UE behavioir is not clear in this case). On top of this, there may be a wrong behaviour on the MCG. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | If the CR is not agreed, it will be not clear with timer T304 needs to be stopped. In case the UE stops the T304 referring to the MCG and if the T304 SCG expires, the UE will trigger again the SCG failure procedure (i.e., but the UE behavioir is not clear in this case). On top of this, there may be a wrong behaviour on the MCG. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.7.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

START OF CHANGE

### 5.7.3	SCG failure information

#### 5.7.3.1	General

Figure 5.7.3.1-1: SCG failure information

The purpose of this procedure is to inform E-UTRAN or NR MN about an SCG failure the UE has experienced i.e. SCG radio link failure, failure of SCG reconfiguration with sync, SCG configuration failure for RRC message on SRB3 and SCG integrity check failure.

#### 5.7.3.2	Initiation

A UE initiates the procedure to report SCG failures when SCG transmission is not suspended and when one of the following conditions is met:

1>	upon detecting radio link failure for the SCG, in accordance with subclause 5.3.10.3;

1>	upon reconfiguration with sync failure of the SCG, in accordance with subclause 5.3.5.8.3;

1>	upon SCG configuration failure, in accordance with subclause 5.3.5.8.2;

1>	upon integrity check failure indication from SCG lower layers concerning SRB3.

Upon initiating the procedure, the UE shall:

1>	suspend SCG transmission for all SRBs and DRBs;

1>	reset SCG MAC;

1>	stop T304 for the corresponding SCG, if running;

1>	if the UE is in (NG)EN-DC:

2>	initiate transmission of the SCGFailureInformationNR message as specified in TS 36.331 [10], clause 5.6.13a.

1>	else:

2>	initiate transmission of the SCGFailureInformation message in accordance with 5.7.3.5.

END OF CHANGE