3GPP TSG-RAN2 Meeting #108	R2-1915579

# Reno, US, 18 – 22 Nov 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.331 | CR | 1383 | rev | - | Current version: | 15.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on the pre-condition for reconfiguration with sync of SCG |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2019-11-18 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | In section 5.3.5.2, it is described that the reconfigurationWithSync is included in secondaryCellGroup only when at least one DRB is setup in SCG. However, the meaning of "DRB is setup in SCG" is unclear. There can be several differnet understanding for DRB setup. For example, the DRB can be either an MN terminated DRB or an SN terminated DRB, or it can be an MCG bearer, an SCG bearer or split beaer. And, it can be inferred that the network can send RRC reconfiguration with reconfigurationWithSync to the UE for SRB3 setup, even no DRB is setup according to the following description in section 5.3.1.2. when the UE is in NR-DC, the network may provide a UE configured with an SCG with an sk-Counter even when no DRB is setup using the secondary key (S-KgNB) in order to allow the configuration of SRB3. Therefore, the above SRB3 case is not aligned with the condition for reconfigurationWithSync in section 5.3.5.2. In addition, in the following ASN.1 condition, there is no special limitation relevant to SRB and DRB. ReconfWithSync; The field is mandatory present in case of SpCell change, PSCell addition, update of required SI for PSCell and AS security key change; otherwise it is optionally present, need M. The field is absent in RRCResume or RRCSetup messages.. Therefore, it is unclear for reconfigurationWithSync using in secondaryCellGroup. |
| Summary of change: |  | To remove the unnecessay limitation “the reconfigurationWithSync is included in secondaryCellGroup only when at least one DRB is setup in SCG;” from section 5.3.5.2. Impact Analysis Impacted 5G architecture options: (NG)EN-DC, NE-DC, NR-DC Impacted functionality: RRC reconfiguration procedure Inter-operability: 1. If the UE is implemented according to this CR but the network is not, there is no inter-operability issue (the UE does not consider any pre condition so the problem is solved). 2. If the network is implemented according to this CR but the UE is not, the consequences if not approved remain. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | reconfigurationWithSync on secondaryCellGroup may be rejected by the UE in some valid scenarios. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.3.5.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

--------------------------------------------------START OF CHANGE-------------------------------------------

#### 5.3.5.2	Initiation

The Network may initiate the RRC reconfiguration procedure to a UE in RRC_CONNECTED. The Network applies the procedure as follows:

-	the establishment of RBs (other than SRB1, that is established during RRC connection establishment) is performed only when AS security has been activated;

-	the addition of Secondary Cell Group and SCells is performed only when AS security has been activated;

-	the reconfigurationWithSync is included in masterCellGroup only when AS security has been activated, and SRB2 with at least one DRB are setup and not suspended.

-----------------------------------------------------END OF CHANGE-------------------------------------------